3GPP TSG RAN meeting #103									           RP-240091

Maastrich, The Netherlands, March 18-22, 2024

## Status Report to TSG

Title:			Status report for New WID: Non-Terrestrial Networks (NTN) for NR Phase 3; rapporteur: Thales, CATT

Agenda item:			9.3.2.3

| WI / SI Name | Rel-19 Non-Terrestrial Networks (NTN) for NR Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Ph3 |  |  |  |  |
| Unique ID | 1020097 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234078 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: N/A | Core part: Overall: 5% RAN1: 10% RAN2: 0% RAN3: 0% RAN4: 0% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Nicolas Chuberre |
|  | Company | Thales |
|  | Email | Nicolas.chuberre@thalesaleniaspace.com |
|  | Name | Jiancheng Sun |
|  | Company | CATT |
|  | Email | sunjiancheng@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

-

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.1.1 Decisions during RAN1#116

##### 2.1.1.1.1 NR-NTN downlink coverage enhancement

Agreement

For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1:

| LEO600km Set1-1 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size(Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set1-2 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |
| --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Agreement

Adopt the following phased array antenna parameters for LEO 600km in FR1:

| Satellite phased array antenna Characteristics | LEO-600 |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | [65] for H [65] for V |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | [400 elements (20 x 20)] |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | [4dB] |

Agreement

RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level:

At least:

- CDF of the received SINR

- The dwell time and revisit time interval for each beam illumination across the coverage

- Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio

Other metrics may be reported such as

- CDF of the cell throughput

- CDF of user perceived throughput (UPT)

- CDF of Latency

- Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889)

For system level study based on analytical evaluation:

- N1 beam footprints are in state “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints are also served the necessary information for cell discovery and initial access

- N1 + N2 + N3 = “Total number of beam footprints “

- N1, N2, N3, X are to be reported by companies.

- Resource utilization obtained under the assumptions above is to be reported by companies.

- Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Agreement

For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following:

- -5.5 dBi antenna gain is assumed

- at least 2Rx are considered at the UE

  - 4Rx can be optionally considered and reported

Note: Redcap device is not considered in the scope of DL coverage study

Agreement

The following traffic models are considered for system level evaluation of DL coverage:

- FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time

- FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time

- VoIP can be considered in the evaluation.

It is up to company report which traffic model is used among the discussed traffic models in their evaluations.

- Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in Rel-18 NTN CE. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

Agreement

For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused.

- Using other beam layouts is not precluded, and should be reported by companies

Agreement

For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed.

Values different from 0 can be optionally reported

Agreement

DL coverage is evaluated at link level with the following considerations:

- NGSO at LEO-600 operating in FR1 is considered in priority

- Additional satellite payload parameters defined for system level evaluation are used

- FFS: Antenna gain reduction due to steering loss can be considered

Agreement

For the evaluation of NTN downlink coverage at link level, reuse the target data rate from Rel-18 NTN Coverage enhancements:

- For VoIP: AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval

- For data rate service: both 3 kbps and 1Mbps can be considered

  - Companies can also use the data rates corresponding to the traffic types used for system level evaluations

Agreement

For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps.

Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821

Step 2: Required SNR of target service is evaluated by LLS

Step 3: The CNR and the required SNR are compared

Agreement

For link-level study, for NR NTN DL coverage enhancement, the following channels/signals can be considered for evaluations:

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB, e.g., SIB1, SIB 19

- PDSCH for paging

- PDCCH

- Broadcast PDCCH (e.g. PDCCH of Msg.2, paging)

- SSB

Note: RAN1 will aim to identify necessary link-level enhancements for these channels in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential link-level enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread:

  - Channel model as in Table 6.1.2-4 of TR38.821, NTN-TDL-C (LOS)

- Evaluation scenario:

  - Rural (LOS)

- Channel estimation: Realistic estimation:

  - Companies are encouraged to report channel estimation method.

- SCS:

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: TBD

- Frequency offset: 0.1 ppm

Agreement

For link budget calculation, parameters in the following table are assumed:

| Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz for DL (S-band) |
| Satellite altitude | 600 km |
| Target elevation angle | 30° (LEO) |
| Atmospheric loss | Equation (6.6-8) in [38.811] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| UE antenna gain | -5.5dBi |
| Free space path loss | Equation (6.6-2) in [38.811] |
| Polarization loss | 3dB |
| Outcome | CNR |

##### 2.1.1.1.2 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Agreement

Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN:

- Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report:

  - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission

  - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission

  - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission

  - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

  - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission

  - Case 6: Dynamic or semi-static DL collides with valid RO

  - Case 7: Collision due to direction switching

- At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs

  - Error cases in case 3 and case 4

  - SIB19 reception collides with UL transmission

  - Slot counting for UL repetition transmission colliding with SSB reception

  - Invalid symbol determination for PUSCH repetition type B

  - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling

  - CPU occupation due to omitted DL reception or UL transmission

Note: Both GSO and Non-GSO should be considered.

##### 2.1.1.1.3 NR-NTN uplink capacity/throughput enhancement

Agreement

Adopt the table below for assumptions for Evaluation parameters for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| Channel model | - NTN-TDL-C Rural, 30° elevation angle |
| Carrier frequency | - 2 GHz |
| Subcarrier spacing | - 15 kHz |
| UE speed | - 3 km/h |
| Frequency hopping | - No frequency hopping |
| PUSCH mapping type A with | - 14 OS- for OCC across slots including DMRS |
| HARQ configuration | - No HARQ |
| Channel coding | - LDPC |
| TBS | Reported by companies, e.g. - ≈184 bits payload @AMR 4.75kbps96 bits @Low data rate |
| DMRS configuration / port / bundling | 1 port per UE Reported by companies - DMRS positions for single-symbol DMRS and optional double-symbol DMRS for PUSCH mapping type A defined in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 respectively with ld=14, l0=2 and pos1 in [38.211]. - up to 8 DMRS Ports Optional DMRS Bundling |
| PRBs/MCS | Reported by companies, e.g. - 1 PRB, 2 PRBs - MCS in Table 6.1.4.1-2 in [TS 38.214] |
| Max repetition number | - Reported by companies – up to 20 for VoIP, up to 32 for low data rates |
| OCC length | Reported by companies, e.g. - Up to 8 |
| OCC sequence | Reported by companies, e.g. - Walsh sequences in Table 6.3.2.6.3-1 in TS38.211 - DFT sequence in Table 6.3.2.6.3-2 in TS38.211 |
| Antenna configuration at Satellite | - 1Rx |
| Antenna configuration at UE | - 1Tx |

Agreement

Adopt the table below for assumptions for modelling impairments for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| TO | Reported by companies - With TO: Uniform selection from [-0.94us, 0.94us], where 0.94us=29Ts - Optional without TO |
| FO | Reported by companies - Uniform selection from [-0.1 ppm, +0.1 ppm], Variation of frequency error is negligible. - Optional: with lower maximum residual FO, to be reported by companies |
| Timing drift | Optional |
| Receiver algorithm | To be reported by companies, e.g. - MMSE |
| Channel estimation | - Real channel estimation |

Agreement

Adopt the table below for assumptions for KPIs for link level evaluation in NR NTN UL capacity and throughput enhancements

| Parameter | Value |
| --- | --- |
| Number of code-division multiplexed users | Reported by companies (up to 8) |
| KPI – SNR for a target BLER per UE | As in Rel-18 (otherwise reported by companies) - VoIP: SNR @2% BLER - For other cases: SNR @10% BLER |
| KPI - Aggregated throughput | Reported by companies Total throughput according to number of code-division multiplexed users (up to 8) Note: companies should also report the throughput for the case without OCC |

#### 2.1.2	Remaining Open issues

Definition of necessary physical layer features enabling

- NR-NTN downlink coverage enhancement covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN

- Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

- NR-NTN uplink capacity/throughput enhancement

## 2.2	RAN2

#### 2.2.1	Agreements

Not started

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

Not started

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Not started

#### 2.4.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

## 4.1	RAN1

RAN1#116 meeting, Athens, Greece, February 26th – March 1st, 2024:

- R1-2401730	Work Plan	Work plan for NR_NTN_Ph3	THALES, CATT

- R1-2400843	Work Plan	Work plan for NR_NTN_Ph3	THALES

- R1-2400871	discussion	NR-NTN Downlink Coverage Enhancement	PANASONIC

- R1-2400875	discussion	Discussion on downlink coverage enhancement for NR NTN	Lenovo

- R1-2400837	discussion	Discussion on downlink coverage enhancements for NR NTN	CCU, NTPU

- R1-2400897	discussion	An operator view on Downlink Coverage Enhancements for FR2-NTN	Eutelsat Group

- R1-2400787	discussion	Discussion on NR-NTN downlink coverage enhancement	LG Electronics

- R1-2400749	discussion	Discussion on downlink coverage enhancement for NR-NTN	Samsung

- R1-2401299	discussion	Discussion on NR-NTN downlink coverage enhancement	MediaTek Inc.

- R1-2401342	discussion	On NR-NTN downlink coverage enhancement	Ericsson

- R1-2401237	discussion	Discussion on NR-NTN downlink coverage enhancement	ETRI

- R1-2401282	discussion	Link Level Enhancements for DL Coverage of NR NTN	CEWiT

- R1-2401059	discussion	Beam groups/patterns for NR-NTN downlink coverage enhancement	Sharp

- R1-2401079	discussion	Discussion on DL coverage enhancements for NR-NTN	NICT

- R1-2401083	discussion	Discussion on downlink coverage enhancement for NR NTN	Baicells, CMCC

- R1-2401029	discussion	Study on NR-NTN Downlink Coverage Enhancement	Apple

- R1-2400971	discussion	Downlink coverage enhancements for NR over NTN	Nokia, Nokia Shanghai Bell

- R1-2400132	discussion	Discussion on downlink coverage enhancements for NR NTN	Huawei, HiSilicon

- R1-2400071	discussion	Discussion on NR-NTN downlink coverage enhancement	Spreadtrum Communications

- R1-2400259	discussion	Discussion on NR-NTN downlink coverage enhancement	vivo

- R1-2400344	discussion	Discussion on NR-NTN DL coverage enhancement	CMCC

- R1-2400355	discussion	Discussion on DL coverage enhancement for NR NTN	ZTE

- R1-2400303	discussion	Discussion on NR NTN Downlink coverage enhancements	THALES

- R1-2400402	discussion	Downlink Coverage Enhancement for NR NTN	Google

- R1-2400424	discussion	Discussion on downlink coverage enhancement for NR NTN	CATT

- R1-2400499	discussion	NR-NTN downlink coverage enhancement	Fraunhofer IIS, Fraunhofer HHI

- R1-2400478	discussion	NR-NTN downlink coverage enhancement	NEC

- R1-2400516	discussion	Seamless NTN downlink coverage for high-availability services	Dell Technologies

- R1-2400528	discussion	Discussion on NR-NTN downlink coverage enhancement	Honor

- R1-2400549	discussion	Discussion on NR-NTN downlink coverage enhancement	xiaomi

- R1-2400576	discussion	NR-NTN downlink coverage enhancement	InterDigital, Inc.

- R1-2400602	discussion	Discussion on NR-NTN downlink coverage enhancement	OPPO

- R1-2401470	discussion	An operator view on Downlink Coverage Enhancements for FR2-NTN	Eutelsat Group

- R1-2401843	discussion	FL Summary #1: NR-NTN downlink coverage enhancements	Moderator (THALES)

- R1-2401845	discussion	FL Summary #3: NR-NTN downlink coverage enhancements	Moderator (THALES)

- R1-2401592	discussion	Discussion on downlink coverage enhancement for NR NTN	Baicells

- R1-2401844	discussion	FL Summary #2: NR-NTN downlink coverage enhancements	Moderator (THALES)

- R1-2401458	discussion	Downlink coverage enhancement for NR NTN	Qualcomm Incorporated

- R1-2401130	discussion	Discussion on DL coverage enhancement for NR-NTN	NTT DOCOMO, INC.

- R1-2401131	discussion	Discussion on support of RedCap and eRedCap UEs in FR1-NTN	NTT DOCOMO, INC.

- R1-2401459	discussion	Support of Redcap and eRedcap UEs in NR NTN	Qualcomm Incorporated

- R1-2401194	discussion	On HD-FDD Redcap UEs for NTN	Ericsson

- R1-2401847	discussion	Summary for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Moderator (CATT)

- R1-2401861	discussion	Final summary for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Moderator (CATT)

- R1-2400603	discussion	Discussion on supporting of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	OPPO

- R1-2400550	discussion	Discussion on the support of Redcap UE for NTN operating on FR1 bands	xiaomi

- R1-2400627	discussion	Discussion on half-duplex RedCap issues for NTN FR1 operation	InterDigital, Inc.

- R1-2400425	discussion	Discussion on the operation of RedCap and eRedCap UEs In NTN	CATT

- R1-2400356	discussion	Discussion on support of RedCap/eRedCap UEs for NR NTN	ZTE

- R1-2400345	discussion	Discussion on the collision issues of HD-FDD Redcap UE in FR1-NTN	CMCC

- R1-2400260	discussion	Discussion on support of RedCap and eRedCap UEs with NR-NTN	vivo

- R1-2400072	discussion	Discussion on support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Spreadtrum Communications

- R1-2400133	discussion	Discussion on HD-FDD RedCap UEs and eRedCap UEs for FR1-NTN	Huawei, HiSilicon

- R1-2400972	discussion	RedCap support for NR over NTN while operating in FR1-NTN bands	Nokia, Nokia Shanghai Bell

- R1-2401030	discussion	Discussion on support of RedCap UEs with NR NTN operation	Apple

- R1-2401238	discussion	Discussion on HD UEs with NR NTN	ETRI

- R1-2401300	discussion	Discussion on support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	MediaTek Inc.

- R1-2400750	discussion	Discussion on support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Samsung

- R1-2400788	discussion	Discussion on support of (e)RedCap UEs with NR-NTN operating in FR1-NTN bands	LG Electronics

- R1-2400789	discussion	Discussion on NR-NTN uplink capacity/throughput enhancement	LG Electronics

- R1-2400751	discussion	Discussion on uplink capacity/throughput enhancement for NR-NTN	Samsung

- R1-2400674	discussion	Discussion on NR-NTN uplink enhancement	China Telecom

- R1-2400819	discussion	Discussion on NR-NTN Uplink Capacity/Throughput Enhancement	Lenovo

- R1-2400824	discussion	Uplink capacity/throughput enhancement for NR-NTN	Panasonic

- R1-2401301	discussion	Discussion on NR-NTN uplink capacity and throughput	MediaTek Inc.

- R1-2401239	discussion	Discussion on NR-NTN uplink capacity/throughput enhancement	ETRI

- R1-2401031	discussion	Study on NR-NTN Uplink Capacity Enhancement	Apple

- R1-2400973	discussion	On NR-NTN uplink capacity/throughput enhancements	Nokia, Nokia Shanghai Bell

- R1-2400977	discussion	On uplink capacity/throughput enhancement for NR NTN	Ericsson

- R1-2401080	discussion	NR-NTN uplink capacity/throughput enhancement	NICT

- R1-2400134	discussion	Discussion on uplink capacity/throughput enhancement for FR1-NTN	Huawei, HiSilicon

- R1-2400073	discussion	Discussion on NR-NTN uplink capacity/throughput enhancement	Spreadtrum Communications

- R1-2400261	discussion	Discussion on NR-NTN uplink capacity enhancement	vivo

- R1-2400346	discussion	Discussion on the NR-NTN uplink capacity/throughput enhancements	CMCC

- R1-2400357	discussion	Discussion on UL capacity enhancement for NR NTN	ZTE

- R1-2400403	discussion	Uplink Capacity Enhancement for NR NTN	Google

- R1-2400426	discussion	Discussion on UL capacity enhancement for NR NTN	CATT

- R1-2400628	discussion	NR-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

- R1-2400551	discussion	Discussion on NR-NTN PUSCH capacity enhancement	xiaomi

- R1-2400604	discussion	Discussion on NR-NTN uplink capacity/throughput enhancement	OPPO

- R1-2400517	discussion	Disaggregated NTN uplink and downlink sessions	Dell Technologies

- R1-2400479	discussion	NR-NTN uplink capacity/throughput enhancement	NEC

- R1-2401484	discussion	Discussion on NR-NTN uplink capacity/throughput enhancement	LG Electronics

- R1-2401791	discussion	Feature lead summary #2 of AI 9.11.3 on NR-NTN uplink capacity and throughput	Moderator (MediaTek)

- R1-2401543	discussion	Feature lead summary #1 of AI 9.11.3 on NR-NTN uplink capacity and throughput	Moderator (MediaTek)

- R1-2401460	discussion	NR-NTN uplink capacity / throughput enhancement	Qualcomm Incorporated

- R1-2401132	discussion	Discussion on NR-NTN uplink capacity/throughput enhancement	NTT DOCOMO, INC.

- R1-2401133	discussion	NR-NTN uplink capacity/throughput enhancement	Sharp

## 4.2	RAN2

-

## 4.3	RAN3

-

## 4.4	RAN4

-

END