3GPP TSG RAN meeting #104										RP-240921

Shanghai, China, June 17-20, 2024

## Status Report to TSG

Agenda item:			9.4.2.4

| WI / SI Name | Rel-18 NR NTN (Non-Terrestrial Networks) enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Study Item: No | Study Item: No |  | Study Item: No |
| Acronym | NR-NTN-enh |  |  |  |  |
| Unique ID | 941006 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-232669 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 06/2024 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 100% RAN1: 100% RAN2: 100% RAN3: 100% RAN4: 100% | Performance Part: Overall: 100% RAN4: 100% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Nicolas Chuberre |
|  | Company | Thales |
|  | Email | Nicolas.chuberre@thalesaleniaspace.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

- RAN4#110bis meeting

Rel-18 NTN UE RF requirements

Issue 1-1: Maximum TRP

Online Agreement:

- For type 1 and type 4 with mechanical steering antenna (e.g. parabolic antenna)

  - 35dBm

- For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array)

  - 35dBm, 40dBm or 45dBm for further decision in this meeting.

Offline agreement:

For maximum TRP:

- For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array)

  - [43dBm] for further ACLR evaluation; if there is coexistence issue identified for 43dBm next meeting, then set 40dBm as maximum TRP power;

  - The maximum EIRP 76.2 is not changed, the antenna gain for other TRP values could further adjusted;

Offline agreement:

For the minimum peak EIRP requirement:

- To state in RAN4 spec to declare the supported elevation angles to comply the minimum peak EIRP requirement and leave the test for RAN5 discussion.

Issue 1-3: Transmit ON-OFF power

- Recommended for further discussion:

  - Option 1: reusing FR2 BS OFF power requirement  -36dBm/MHz

Online Agreement:

- Agree on Option 1.

Issue 1-4: Transmit ON-OFF time mask

Online Agreement:

- Transient period for FR2-NTN is 5us.

Issue 1-7:  SEM

Online Agreement:

- Agree on proposal 4.

  - FFS on definition of P_(rated,UE)

Offline agreement for  NS indication for regional requirements

Step 1: to make the regulatory requirement clear and applicable to which bands;

Step 2: to further discuss how to arrange the regulatory requirement in the specification next meeting;

Issue 1-8:  Power control requirement

Online Agreement:

- No requirement for power control for Ka band for NR-NTN UE

Issue 1-9:  MPR requirement for type 2 and type 5 to meet OFF-axis EIRP requirement

Offline agreement:

For type 2 an type 5,  the power reduction for OFF-axis EIRP requirement might be needed and the exact value is left for the implementation;

Issue 2-1  Minimum EIS requirement

Online Agreement:

- for type 3 UE, to specify minimum EIS as -115.6dBm for 50MHz, for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz;

- for type 1/2/4/5 UE, to specify minimum EIS as -122dBm for 50MHz, for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz

Issue 2-2: Maximum input power

Online Agreement:

- For type 3,

  - [-101]dBm as maximum input power with 64QAM

- For type 1/2/4/5:

  - -109.6dBm with 16QAM or [64QAM].

  - FFS for QPSK

- The exact MCS or coding rate for FRC of maximum input power need further discussion and confirmation.

Issue 2-5: Polarization characteristics

Online Agreement:

- Either RHCP or LHCP is supported by Ka band VSAT in Rel-18

Issue 1-7:  SEM

Online Agreement:

- Agree on proposal 4.

  - FFS on definition of P_(rated,UE)

  - FFS for the boundary for SEM and spurious emission requirements.

NR_NTN_enh_SAN_UE_demod

Issue 1-1: Doppler Spread for both UL and DL

- Agreement

  - 1200Hz

Issue 2-1: PDSCH demodulation requirements for above 10 GHz bands

- Agreement:

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLC5-1200 | MCS4 | Disabled HARQ (4/16) |
|  | MCS4 | 16 HARQ Proc |
| NTN-TDLC5-1200 | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

Issue 2-2: How to define requirements for GSO and NGSO for above 10 GHz bands

- Agreement

  - Define one set of performance requirements for both NGSO and GSO

    - Only consider K_offset = [8] that corresponding to 64 slots for 120kHz SCS

Issue 2-3: PDCCH demodulation requirements for above 10 GHz bands

- Agreement

- AL8 and AL16, with the following test configurations

| Case | Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleaver size | Shift index | DCI format | Payload (without CRC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | 120 | 60 | 1 | 8 | Interleaved | 2 | 3 | 0 | [1-0] | [40] |
| 2 | 200 | 120 | 60 | 2 | 16 | Interleaved | 2 | 3 | 0 | [1-1] | [56] |

Issue 2-4: Antenna configuration

- Agreement

  - 1Tx1Rx

Issue 2-5: Applicability rule

- Agreement

  - All demodulation performance requirements for NTN enhancements are applicable for Type 1 fixed and Type 2 mobile VSAT UEs

| UE feature/capability [TBD] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| NR NTN access (nonTerrestrialNetwork-r17) | FR2-NTN | PDSCH | Clause x (Testxx) |  |
|  |  | PDCCH | Clause y (Test 1, Test 2) |  |
| Increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17) | FR2-NTN | PDSCH | Clause x (Testxx) |  |
| Disabled HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17) | FR2-NTN | PDSCH | Clause x (Testxx) |  |

Issue 3-1-1: MCS

- Agreement

  - MCS 2 and [MCS 12]

Issue 3-1-2: PTRS configuration

- Agreement

  - Not configure PTRS for test with MCS 2 and [MCS12]

Issue 3-2-1: UCI info

- Agreement

  - The UCI information can include both ACK/NACK and CSI part 1 for specifying PUCCH UCI BLER performance requirement.

Issue 3-2-2: Additional DMRS configuration for PUCCH format 3/4

- Agreement

  - Define PUCCH format 3/4 requirements with additional DMRS configuration both enabled and disabled.

  - Reuse the applicability of requirements for different configurations in section 8.1.2.3.4 of TS 38.181

Issue 3-3-1: RB assignment

- Agreement

  - 6RBs in the middle of the test bandwidth for both 15KHz and 30KHz

Issue 3-3-2: PUSCH aggregation factor

- Agreement

  - n4 for 15kHz SCS

  - n8 for 30kHz SCS

Issue 3-3-3: pusch-TimeDomainWindowLength

- Agreement

  - 4 for 15kHz SCS

  - 8 for 30kHz SCS

NR_NTN_enh_Part3

Issue 1-1: NTN VSAT UE ACS

Agreement:

- The summary of co-existence study results of VSAT UE ACS will be capture in TR 38.863 Chapter 6. The suggested ACS value for UE could be different and captured in Chapter 8 with some notes elaborating reasons.

- NTN VSAT UE ACS value is 25dBc.

Issue 1-3: Notes 1&2 in R4-2403092

Agreement:

- Keep Note 1 and Note 2 only in TR 38.863 for 17(DL)/27(UL) GHz.

- The conclusion on the ACLR and ACS values for NTN SAN and VSAT UE in the table below are applicable for 17(DL)/27(UL) GHz only.

|  | SAN |  | UE |  |
| --- | --- | --- | --- | --- |
|  | GEO | LEO | Fixed | Mobile |
| ACLR (dBc) | 12 | 12 | 14 | 14 |
| ACS (dBc) | 18 | 24 | 251, 2, 3 | 251, 2, 3 |
| NOTE 1: At the time of this 3GPP co-existence study, there is no TN band defined or planned near 17 GHz. The parameters are derived based on 3GPP coexistence scenarios in which a TN system is simulated to be operating in the band directly adjacent to the proposed NTN system as well as technical assumptions that may or may not be applicable in practice. The results of the study are not intended to address coexistence issues from a regulatory standpoint. NOTE 2: There are existing non-3GPP VSAT UE operating in Ka band at present and will likely continue operating in the future, with ACS performance lower than the value. NOTE 3: Additional solutions could be further considered to address coexistence issues if and when TN is deployed in 17 GHz. |  |  |  |  |

RRM requirements for NR_NTN_enh

Issue 1-6A: Te_NTN for 60kHz and 120kHz in Case2

Agreement: (ad-hoc)

- Further check and come back in next meeting

Issue 1-6B: Te_NTN for 60kHz and 120kHz in Case3

Agreement: (ad-hoc)

- In Case-3, Te_NTN [Ts] for 120kHz UL SCS is:

  - [10]

<Note: No update was made compared RAN4#110 agreement>

Issue 3-2: Measurement period and accuracy requirements on RTD

Agreement (online):

- Remove factors related to multiple positioning frequency layers, multiple RX TEGs and Rx beam sweeping from the Rx-Tx requirements.

- Remove the below applicability rules from UE RX-TX time difference measurements:

  - FFS: When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

  - If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1C.2 during the UE Rx-Tx measurement period, then:

    - UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1C.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

- Supporting Nsample = 1 is a component of NW verified location (FG 44-2), and it does not require UE to support reduced sample number for TN positioning measurement (FG 27-3-1).

  - HW to draft the LS to RAN1/2 about the information

Issue 3-5: Other impact on RRM

Agreement (online):

- When UE switches to a new satellite switch with same PCI (through HO, CHO or satellite switch with re-sync for both hard and soft satellite switch), select one option from:

  - Option 1a: UE stops the PRS measurement after t_service and restart the UE Rx-Tx time difference measurement after switch is complete.

    - The PRS configuration of the two satellites with the same PCI follows RAN1/2 specficiaton, and no further restriction for gNB/LMF to be defined in RAN4.

    - The UE measurement accuracy requiremet does not apply if the PRS transmission from two satelliates overlap in time/frequency domain. Further clarify the definition of “overlapping” offline.

  - Option 1b: UE stops the PRS measurement after t_servicestart and restart the UE Rx-Tx time difference measurement after switch is complete.

    - The PRS configuration of the two satellites with the same PCI follows RAN1/2 specficiaton, and no further restriction for gNB/LMF to be defined in RAN4.

  - Option 2: UE stops the PRS measurement after t_service and until LMF triggers new measurement.

Issue 5-2-S: Soft’ Satellite switch (5-2-S1 and -S2 from RAN4#110 are merged)

Agreement (online):

- Scheduling restrictions over [t-ServiceStart ~ t-Service] for UE incapable of parallelMeasurementWithoutRestriction-r17 and/or [differentSCS between SSB and data]

  - Define scheduling restriction during soft satellite switch from UE perspective, i.e. scheduling restriction are allowed only during SSB occasions of the target satellite (same as 9.2C.5.3)

    - For the scheduling restriction: For RSRP measurement, 1 additional symbol before and after SSB block. For RSRQ measurement, 1 additional symbol before and after RSSI symbols.

Agreement (online):

- Optimization on measurements

  - UE is allowed to skip measurements other cells and satellites than the target satellite and source satellite from T-serviceStart to the satellite switch completion.

Agreement (online):

- Impact on inter-satellite neighbour cell measurements

  - No optimization for the scenario of ‘(both hard and soft) satellite switch with re-sync’ on inter-satellite neighbor cell measurements

Issue 6-2: Above 10 GHz, Test case list

Agreement: (ad-hoc)

- Define test cases for the below cases for both Type 1 and Type 2 UEs. Detailed configurations are to be discussed separately.

  - UL timing accuracy

  - Mobility in RRC Idle/Inactive mode

  - Mobility in RRC Connected mode

    - Intra-satellite

    - Inter-satellite (blind mobility)

  - RLM

  - L1-RSRP

  - L3 measurements

  - Measurement accuracy

    - If measurement accuracy requirements specified in Rel-18

Issue 6-2-2: Above 10 GHz, Rx beam gain

Agreement: (ad-hoc)

- FFS whether the RF margin for different RX beams in existing TN FR2-1 intra-frequency relative accuracy requirements can be removed or  not

- Existing absolute measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz.

- Further discuss the minimum SSB_RP condition on accuracy requirements

Issue 6-2-3: Above 10 GHz, UL timing accuracy

Agreement: (ad-hoc)

- Define UL timing test cases at least for following configuration:

  - UL SCS 120kHz with DL SSB SCS: 120kHz

- FFS whether any other configuration need to be considered or not

- For UL SCS is 120kHz,

  - Transmit Timing Test covering

    - Case 1,2,3

  - Timing advance adjustment accuracy covering

    - Case 1,2,3

- Note:

  - Case-1: Stationary UE for GSO

  - Case-2: Stationary UE for LEO

  - Case-3: Mobile UE for GSO

Issue 6-3-1: Below 10 GHz, Test set-up and applicability rule

Agreement: (ad-hoc)

- NTN-TN inter-frequency cell reselection (Intra-RAT to NR TN and Inter-RAT to LTE TN)

  - For earth-moving cell, time-/location-based measurement initiation for cell reselection:

    - No test case

  - For earth fixed cell:

    - Define test cases

- NTN to NTN time-based measurement initiation for cell reselection in earth-moving cell

  - Not define test case

- NTN to NTN location-based measurement initiation for cell reselection in earth-moving cell

  - Not define test case

Agreement: (online)

- Define two TCs:

  - RACH-less for soft satellite switch

  - RACH-based for hard satellite switch

- RAN4#111 meeting

Rel-18 NTN UE RF requirements

Issue 1-0: Maximum EIRP

Agreement:

- Note: Maximum EIRP is defined using 13 PRBs allocation with 120KHz SCS.

Issue 1-1: Maximum TRP

Agreement:

- 43dBm for type 2 and type 5, and max EIRP and off-axis EIRP requirements CANNOT be relaxed

- 43dBm for type 3 if max EIRP requirements CANNOT be relaxed

Issue 1-2: Power backoff for type 2 and type 5 with phase array to meet OFF-axis requirement for GSO orbit

Agreement:

- Add the NOTE in the VSAT class table

- Note X: 	UE may need power reduction for meeting OFF-axis EIRP requirement defined in clause 9.2.2. Value is implementation dependent.

Issue 1-3:  TRPmax

Agreement:

- To update TRPmax in the emission requirements with TRPrated which  is the declared rated output power lower than TRPmax specified in clause 9.2.1.

Issue 1-4:  Number of VSAT simultaneously transmitting, N

Agreement:

- Clarify that the number of VSAT simultaneously transmitting N shall be declared by the VSAT manufacturer.

Issue 1-5: the clarification of  “uncoordinated” from the additional Off-axis EIRP density requirements for protection of fixed services (sub-clause 9.2.2.3.3)

Agreement:

- remove the uncoordinated in the specification

Issue 1-6:  NS value for FR2 NTN bands

Agreement:

- For  band n512,  to use the NS approach to define the regional requirements;

- For band n511 and n510,  not to define the NS values;

Issue 1-7: Beam steering range and test direction to fulfill minimum EIRP requirement.

Agreement:

- [For measurement metric for min peak EIRP , the min peak EIRP should be verified between the declared supported lowest transmitter elevation angles (as shown in Figure 1-3). ]

- Figure 1-3 example measurement grid for min peak EIRP of the supported elevation angle

Issue 1-8: On-axis cross polarization isolation requirements VS Pointing accuracy requirements in clause 9.6.1

Agreement:

- For the type 3 VSAT,  linear polarization requirement is not applicable

- For the type 1/2/4/5,  both linear and circular polarization requirement are applicable.

Issue 1-9: Off-Axis EIRP

Agreement:

- ”Off-axis EIRP should” be ”Off-axis EIRP emission density limit within the operating band”

Issue 1-10: Fixed VSAT Off-axis EIRP requirements for n512

- Proposal 1 (R4-2409758):

  - For co-polarized transmissions, the requirements specified in table 9.2.2.3.1-1 apply to Fixed VSAT type 1 or 2 when transmitting towards GSO. [THALES, Ericsson]

  - For cross-polarized transmissions, the requirements specified in table 9.2.2.3.1-2 apply to Fixed VSAT type 1 or 2 when transmitting towards GSO. [THALES, Ericsson]

Agreement:

- Proposal 1 is agreeable.

Issue 2-1  Minimum EIS requirement

Agreement:

### 10.3.2 Minimum requirement The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as [specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 10.3.2-1. And EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT]. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle). Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT NTN VSAT channel bandwidth (MHz); UL/DL RB allocation; OTA reference sensitivity level, EISREFSENS (dBm). 50, 100, 200, 400; Full RB allocation NRB as specified in clause 5.3.2; EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1). NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz.. Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT Operating band; NTN VSAT class; NTN VSAT type; The range of EISREFSENS_50MHz (dBm). n512, n511; Fixed VSAT; 1, 2; ≤ -122. 3; ≤ -115.6. n512, n511, n510; Mobile VSAT; 4, 5; ≤ -122.

Issue 2-2: Maximum input power

Agreement:

  - For type 1/2/3,

- -101dBm as maximum input power with 64QAM

  - For type 4/5:

- -109.6dBm with 16QAM and [64QAM ].

  - For MCS for 64QAM

- Please check the MCS in R4-2409618.

Issue 2-3: ACS

Agreement:

For type 1/2/4/5/3,  to define the following requirements;

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| PInterferer for band n512, n511, n510 | dBm | (EISREFSENS_50M + 28.7.)+ 10log10(NRB x SCS x 12 / factor) |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern as described in Annex A.3.2 and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. [NOTE 4: The transmitter shall be set to same as the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Clause 10.3.] NOTE 5: The “factor” represents the normalized factor to scale wanted signal and interference level for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

The wanted signal and interference signal is supposed to come from the same directions

Issue 2-4: Beam steering range and test direction to fulfill minimum EIS requirement

Agreement:

- NTN VSAT need to declare its supported lowest elevation angle from receiver perspective regardless of mechanical steering antenna or electronic steering antenna or hybrid

- For measurement metric for min EIS , the EIS should be verified between the declared supported receiver lowest elevation angles (as shown in Figure 1-3).

- Figure 1-3 example measurement grid for min EIS of the supported elevation angle

  - Offline agreement:

  - The minimum elevation angle for both transmitter and receiver is up to the declaration by vendor..

Issue 2-5: Receiver antenna off-axis performance

- Proposal 1: to consider receiver antenna off-axis performance as in R4-2408702

Agreement:

- Proposal 1 is agreeable.

NR_NTN_enh_SAN_UE_demod

Issue 1-1-1: Power class and NTN UE Types

- Agreement

  - Not include UE power class and UE Type 1~5 for the FR2-NTN demod requirements explicitly, the demod requirements are applicable to all UE types by default.

Issue 1-1-2: Clarification on TS 38.101-5 structure for radiated requirements

- Agreement

  - Follow the specification structure of section 8.1 for FR1-NTN in TS 38.101-5 for FR2-NTN performance requirements

Issue 1-2-1: K_offset for GSO and NGSO for above 10 GHz bands

- Agreement

  - K_offset for 120kHz SCS

    - 8 for NGSO

    - 258 for GSO

Issue 1-2-2: PTRS configuration

- Agreement

  - No need further ideal simulation results alignment among companies with and without PT-RS configuration in the simulation

  - Not configure PTRS for all NTN PDSCH demodulation requirements with MCS 4 and MCS 13 for above 10 GHz

Issue 1-3-1: CORESET RB

- Agreement

  - Change the CORESET RB from 60 to 132 with consideration 200MHz channel bandwidth

Issue 1-3-2: Payload size for DCI format 1_0 case

- Agreement

  - Change the payload size from 40 to 42 with consideration of 200MHz channel bandwidth

Issue 2-1-1: The number of demodulation branches

- Agreement

  - Keep the previous agreement, i.e. both 1Rx and 2Rx, with the test applicability rule for support of different number of demodulation branches based on declaration.

Issue 2-2-1: MCS

- Agreement

  - MCS 2 and MCS 12

Issue 2-2-2: PTRS configuration

- Agreement

  - Not configure PTRS for test with MCS 2 and MCS12

Issue 2-4-1: PUCCH format 0

- Agreement

  - Keep case with first symbol is 12 with 2 symbols.

Issue 2-4-2: PUCCH format 3

- Agreement

  - Keep case#1 with first symbol is 0 with 14 symbols.

Issue 2-5-1: Test metric

- Agreement

  - The false alarm probability shall be less than or equal to 0.1%.

  - The probability of detection shall be equal to or exceed 99%

RRM requirements for NR_NTN_enh

Issue 1-6A: Te_NTN for 60kHz and 120kHz in Case2

Agreement: (online)

- In Case-2, remove the below side condition for requirement applicability.

  - The requirements are applicable only if the ephemeris information be refreshed (i.e. update rate of ephemeris information in SIB19) at least every [7] seconds.

Issue 1-11: Additional enhancements (for Case-3)

Note

- Based on the existing gradual timing adjustment UE requirement, i.e. UL timing is supposed to be gradually adjusted (subject to Tp and Tq) upon GNSS location update, the issue is closed. No further discussion.

Issue 2-4: RRC Re-establishment

Agreement: (online)

- For Type 2 UE, RRC re-establishment requirements do not apply when the cause for the RRC re-establishment is an inter-satellite HO failure.

Issue 3-2: Measurement period and accuracy requirements on RTD

Agreement: (online - Friday)

- Remove following applicability rule for UE Rx-Tx measurement requirements:

  - When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

Issue 3-4: Measurement accuracy requirements on UL timing drift

Agreement: (online - Friday)

- No new applicability condition for UE Rx-Tx measurement requirements related to amount of variation in the applied TA during measurement period.

Issue 3-5: Other impact on RRM

Agreement: (online)

- When UE switches to a new cell with different PCI, UE stops the PRS measurement for the source cell after HO occurs and starts new PRS measurement for the target cell after SRS reconfiguration on the target cell is complete.

  - Further discuss how to capture “starts new measurement” in the CR.

- When UE switches to a new cell with same PCI through hard and soft satellite switch with re-sync, UE stops the PRS measurement at whichever point occurs earlier between t-Service and t-ServiceStart and starts new measurement for the UE Rx-Tx time difference after switch is complete.

  - Further discuss how to capture “starts new measurement” in the CR.

Issue 4-1: TN to NTN cell reselection

Agreement: (online)

- It is a common understanding that location-based measurement triggering parts are not applicable for cell reselection from TN to NTN. Whether/How to implement this in RAN4 spec is left to CR.

- If both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode,

  - For NTN capable UE, the cell reselection requirements (i.e., the TN to NTN reselection requirement agreed in RAN4 #110) are applied to both TN and NTN target cells/carriers.

  - FFS: For NTN incapable UE, the existing TN-to-TN cell reselection requirements are applied.

    - Further check does UE know whether it is a TN carrier or NTN carrier

Issue 4-2: NTN to TN cell reselection

Agreement: (online)

- Clarify the requirements related to TN measurement skipping as follows:

  - UE shall perform TN measurement if its estimated distance to tn-ReferenceLocation is smaller than tn-DistanceRadius. The requirements apply provided that the actual distance between UE to tn-ReferenceLocation is smaller than tn-DistanceRadius – 50m.

Issue 5-1: NTN to NTN RACH-less (C)HO

Agreement: (online - Friday)

- For RACH-less HO/CHO, TIU is the interruption uncertainty in acquiring the first UL transmission resource, which can be a configured grant based PUSCH or dynamic grant based PUSCH according to NW configuration and scheduling.

Issue 5-2: NTN to NTN Satellite switching without PCI change

Agreement: (online)

- In response to RAN2 LS (R4-2407009_R2-2403771), RAN4 to confirm that it is feasible to adopt the gNB as the reference point of ssb-TimeOffset for both soft and hard satellite switch scenarios.

Issue 5-2-S: Soft’ Satellite switch (5-2-S1 and -S2 from RAN4#110 are merged)

Agreement: (online)

- Do not define known case for soft satellite switch.

  - It does not mean RAN4 will define the “known” or “unknown” case in the spec for soft satellite switching.

Issue 6-2-1: (FR2-NTN) Test set-up and applicability rule

Note

- Based on UE capability and VSAT class defined in Table 9.2.1.0-1 of TS38.101-5, the below proposed applicability rue is evident. Therefore, no separate agreement is pursued.

- Proposal: Mobile UE does not need to pass the TCs with NGSO. Fixed UE needs to pass the TCs with either GSO or NGSO depending on UE capability.

Issue 6-2-2: (FR2-NTN) Rx beam gain

Agreement: (ad-hoc + online)

- The existing absolute measurement accuracy requirement and relative measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with [1]dB relaxation.

  - Note: Companies are encouraged to further evaluate the performance loss due to single polarization assumption on FR2 Ka band VSAT UE. It’s not precluded to further update the tentative relaxation value in future RAN4 meeting.

- Remove an RF margin for different RX beams in the relative accuracy.

- Define the minimum SSB_RP condition for accuracy requirement of five UE types specified in table 9.2.1.0-1 of TS38.101-5. Minimum SSB_RP_NTN_FR2 for Rx Beam Peak angle of arrival = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n, where,

  - Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5.

  - PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32;

  - SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

  - SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

  - ∆MBP,n is 0.

- For the minimum SSB_RP condition,

  - The gain difference between fine and rough beams is YdB:

    - For mechanical steering antenna, Y=0

    - For electronic steering antenna, FFS Y

- Do not define separate spherical coverage unless spherical coverage is introduced in RF session.

- FFS: Gmin and Gmax

- Note: If anything above inconsistent with RF requirement is identified, RAN4 to make updates to those aspects accordingly.

Issue 6-2-3: (FR2-NTN) UL timing accuracy

Agreement: (ad-hoc)

- The value for TGNSS_margin for mobile and fixed UEs shall be introduced for uplink timing error requirements for FR2 NTN (Ka band introduced in Rel-18 )

  - Further discuss the exact values:

    - Option 1: X = [(Te_NTN-Te)/2]

    - Other options not precluded

- In the test case of UE transmission timing accuracy for Case-3 (120kHz SCS) (Mobile UE for GSO):

  - UE mobility is not considered before the testability issue is resolved. And without consideration of UE mobility during test, UE test requirement for Case-3 will be further discussed considering necessary margin.

  - It’s not precluded to further update test case including test requirements for case -3 if testability issue on UE mobility resolved in future release.

Agreement: (ad-hoc)

- Only define test case with UL SCS 120kHz and DL SSB SCS 120kHz

Issue 6-2-4: (FR2-NTN) Mobility

Note

- The below proposal is expected to be reflected in relevant test cases as per the agreed core requirements without further agreements.

- Proposal: For mobility test cases involving UEs with mechanically steered beams,

  - make a note that the timing requirement for the completion of the mobility procedure depends on the setup of the test case, considering UE “minimum steering speed” of 22 degrees/s.

Issue 6-2-7: (FR2-NTN) Measurement accuracy

- Note: Covered in Issue 6-2-2: (FR2-NTN) Rx beam gain

Issue 6-2-8: (FR2-NTN) Test case details

Note

- The RMC configurations:

  - Option 1: Use the TN table as baseline, and only specify the different ones (delta approach).

  - Option 2: Copy the TN table, and update value which is different with TN configuration.

Issue 6-2-9: (FR2-NTN) AoA setup

Agreement: (online)

- Use the following AoA setup for test cases:

  - Transmission timing accuracy: single AoA in Rx beam direction, if defined and applicable.

  - Inter-satellite mobility: 2-AoA setup

    - Offset of relative angles of 2 AoAs from UE perspective is 30 degrees in Rel-18 test.

    - The AoA is set up with the corresponding UE RF requirements fulfiled.

    - The AoA and the epherimis information of two satellites are aligned.

  - The rest test cases: single AoA in Rx beam direction, if defined and applicable

Issue 6-3-2: (FR1-NTN) Idle mode mobility, NTN-TN inter-frequency cell reselection

Agreement: (ad-hoc)

- In NTN-TN cell reselection test case,

  - Set the distance between the UE and tn-ReferenceLocation as tn-DistanceRadius - 50m.

  - Only one TN neighbour cell is configured.

  - Note: Detailed test configurations and procedures are confirmed by the review and endorsement of relevant test case CR.

  - Note: TN measurement skipping is not verified.

Issue 6-3-3: (FR1-NTN) Connected mode mobility, Satellite switch

Agreement: (ad-hoc)

- In the TCs for satellite switch,

  - GSO test configuration is not applicable.

  - (t-Service - t-serviceStart) > (Tsearch + T∆ + Tmargin)

- Note: Reflect above agreements into draft CRs

Issue 6-3-5: (FR1-NTN) Connected mode mobility, NTN to NTN RACH-less HO

Agreement: (ad-hoc)

- NTN to NTN RACH-less HO test case is defined, and the detailed test configurations and procedures are confirmed by the review and endorsement of relevant test case CR.

Issue 6-3-6: (FR1-NTN) Measurement procedure and accuracy, Network verified UE location

Agreement: (ad-hoc)

- Do not define test case for network verified UE location.

#### 2.4.2	Remaining Open issues

No remaining open issues.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

-

## 4.2	RAN2

-

## 4.3	RAN3

-

## 4.4	RAN4

RAN4#110bis meeting, Changsha, China, April 15 - 19, 2024:

- R4-2405315	discussion	Discussions on Fixed and Mobile VSAT def and abbr	Samsung

- R4-2405320	draftCR	draft CR for TS 38.101-5 Chapter 3	Samsung

- R4-2405340	other	Doppler shift issues for guard band and transmission bandwidth configuration	Huawei, HiSilicon

- R4-2405976	CR	CR for TR 38.863 regulatory update after WRC-23	THALES

- R4-2405994	draftCR	CR for TR 38.863 regulatory update after WRC-23	THALES

- R4-2405923	draftCR	Draft CR to TS 38.108: Correction of the NTN frequency ranges, Rel-18	Huawei, HiSilicon

- R4-2405705	draftCR	draft CR to 38.108: Correction of applicability table for SAN	NEC

- R4-2405993	draftCR	draft CR for TS 38.101-5 Chapter 3	Samsung, Cybercore

- R4-2405393	other	Views on coexistence study between TN and NTN above 10GHz bands	Qualcomm Incorporated

- R4-2405972	discussion	On the ACS requirement issue for VSAT UE in above 10 GHz	THALES, Magister Solutions Ltd, Eutelsat Group, ESA, Inmarsat, Viasat, Novamint, EchoStar, Amazon

- R4-2405337	other	Discussion on Ka band NTN UE ACS	Huawei, HiSilicon

- R4-2405084	draftCR	draf CR on TR38.863 Addition of coeixstence results in above 10GHz	Samsung Electronics

- R4-2405086	discussion	Discussion on NTN VSAT UE ACS	Samsung Electronics

- R4-2404938	other	NTN enhancement - coexistence: further discussion on NTN UE ACS	Ericsson

- R4-2404439	other	Further discussion on NTN UE ACS for above 10GHz bands	CATT

- R4-2405977	CR	CR for correction of SAN ACS value in TS 38.108	THALES

- R4-2405924	draftCR	Draft CR to TS 38.108: Correction of the OOBB requirement, Rel-18	Huawei, HiSilicon

- R4-2405925	draftCR	Draft CR to TS 38.108: missing references, Rel-18	Huawei, HiSilicon

- R4-2406113	draftCR	CR for correction of SAN ACS value in TS 38.108	THALES

- R4-2405996	draftCR	Draft CR to TS 38.108: missing references, Rel-18	Huawei, HiSilicon

- R4-2405999	draftCR	Draft CR for TS 38.181: FR2 intro in clause 4.1 MUs	Ericsson, Thales

- R4-2406000	draftCR	Draft CR for TS 38.181: FR2 intro in clause 4.7 test config	Ericsson, Thales

- R4-2406001	draftCR	Draft CR for TS 38.181: FR2 intro in clause 9.4 output power dynamics	Ericsson, Thales

- R4-2405928	discussion	Discussion on the Extreme testing conditions for NTN SAN (Ka band)	Huawei, HiSilicon

- R4-2405929	discussion	Discussion on the MU values for Ka band	Huawei, HiSilicon

- R4-2405888	draftCR	NTN enhancement - Running draft CR to TS 38.181	Ericsson

- R4-2405997	draftCR	Draft CR for TS 38.181, On RF channels and test models in clause 4.9 for Ka-band NTN	CATT

- R4-2405998	draftCR	Draft CR for TS 38.181, On radiated receiver characteristic in clauses10.1 and 10.2 for Ka-band NTN	CATT

- R4-2406076	draftCR	Draft CR for TS 38.181, On radiated receiver characteristic in clauses10.3 for Ka-band NTN	CATT

- R4-2405555	draftCR	Draft CR for TS 38.181: FR2 intro in clause 4.1 MUs	Ericsson, Thales

- R4-2405556	draftCR	Draft CR for TS 38.181: FR2 intro in clause 4.7 test config	Ericsson, Thales

- R4-2405557	draftCR	Draft CR for TS 38.181: FR2 intro in clause 9.4 output power dynamics	Ericsson, Thales

- R4-2405558	draftCR	Draft CR for TS 38.181: FR2 intro in clause 9.7.5 spurious emissions	Ericsson, Thales

- R4-2404440	draftCR	Draft CR for TS 38.181, On applicability of requirements in clause 4.8 for Ka-band NTN	CATT

- R4-2404441	draftCR	Draft CR for TS 38.181, On RF channels and test models in clause 4.9 for Ka-band NTN	CATT

- R4-2404442	draftCR	Draft CR for TS 38.181, On radiated receiver characteristic in clauses10.1 and 10.2 for Ka-band NTN	CATT

- R4-2404443	draftCR	Draft CR for TS 38.181, On radiated receiver characteristic in clauses10.3 for Ka-band NTN	CATT

- R4-2404617	draftCR	Draft CR for TS 38.181, On manufacturer declarations in clause 4.6 for Ka-band NTN	CATT

- R4-2405085	draftCR	draft Big CR to TS 38.101-5	Samsung Electronics

- R4-2405274	other	Topic summary for [110bis][121] NR_NTN_enh_UERF_R18	Moderator(ZTE)

- R4-2405341	draftCR	Draft CR for TR 38.863 to introduce some technical background for R18 NTN VSAT UE Tx requirements	Huawei, HiSilicon

- R4-2405318	draftCR	draft CR for TS 38.101-5 9.2.1	Samsung

- R4-2405296	discussion	Discussion on NTN UE RF Tx requirements	Samsung

- R4-2405176	other	Discussion on the minimum output power of NTN UE	OPPO

- R4-2404939	other	NTN enhancement - NTN UE TRP requirement and Tx antenna performance	Ericsson

- R4-2404941	draftCR	NTN enhancement: draft CR to TS 38.101-5 NTN Ka-band - additional Tx updates to the running CR	Ericsson, Thales

- R4-2405640	other	Further discussion on Tx RF requirements for NTN in Ka-band	ZTE Corporation

- R4-2405641	draftCR	Draft CR to TS 38.101-5 Clause 9.3 Output power dynamics	ZTE Corporation

- R4-2405339	other	Discussion on Tx requirement for Ka band NTN UE	Huawei, HiSilicon

- R4-2406602	draftCR	NTN enhancement: draft CR to TS 38.101-5 NTN Ka-band - additional Tx updates to the running CR	Ericsson, Thales

- R4-2405889	other	VSAT type 2 and 5 MPR to meet OFF-axis EIRP	Qualcomm Incorporated

- R4-2406605	draftCR	Draft CR to TS 38.101-5 Clause 9.3 Output power dynamics	ZTE Corporation

- R4-2406604	draftCR	Draft CR for TR 38.863 to introduce some technical background for R18 NTN VSAT UE Tx requirements	Huawei, HiSilicon

- R4-2406607	draftCR	Draft CR for TR 38.863 to introduce some technical background for R18 NTN VSAT UE Rx requirements	Huawei, HiSilicon

- R4-2406606	draftCR	Draft CR for 38.101-5 to introduce clause 10.1~10.3	Huawei, HiSilicon, Ericsson, Thales

- R4-2406608	draftCR	Draft CR to TS 38.101-5 Clause 10.4 Maximum input power requirement	ZTE Corporation

- R4-2405974	discussion	THALES, Magister Solutions Ltd, Eutelsat Group, ESA, Inmarsat, Viasat, Novamint, EchoStar, Amazon	THALES

- R4-2405642	other	Further discussion on Rx RF requirements for NTN in Ka-band	ZTE Corporation

- R4-2405643	draftCR	Draft CR to TS 38.101-5 Clause 10.4 Maximum input power requirement	ZTE Corporation

- R4-2405644	draftCR	Draft CR to TS 38.101-5 Clause 10.6 Blocking characteristics	ZTE Corporation

- R4-2405645	draftCR	Draft CR to TS 38.101-5 Annex NTN VSAT related FRC	ZTE Corporation

- R4-2404942	draftCR	NTN enhancement: draft CR to TS 38.101-5 NTN Ka-band - additional Rx updates to the running CR	Ericsson, Thales

- R4-2404940	other	NTN enhancement - NTN UE Rx antenna performance	Ericsson

- R4-2405314	discussion	Discussions on NTN UE RF Rx requirements	Samsung

- R4-2405342	other	Discussion on Rx requirement for Ka band NTN UE	Huawei, HiSilicon

- R4-2405343	draftCR	Draft CR for TR 38.863 to introduce some technical background for R18 NTN VSAT UE Rx requirements	Huawei, HiSilicon

- R4-2405338	draftCR	Draft CR for 38.101-5 to introduce clause 10.1~10.3	Huawei, HiSilicon

- R4-2405197	draftCR	Draft CR_Cell Re-selection for NR UE for Satellite in IDLE state	ZTE Corporation

- R4-2405198	draftCR	Draft CR_Cell Re-selection for NR UE for Satellite in Incative state	ZTE Corporation

- R4-2406515	draftCR	Draft Big CR to TS 38.133 on RRM core requirements for NR NTN enhancement	Qualcomm

- R4-2406476	draftCR	Draft CR_Cell Re-selection for NR UE for Satellite in IDLE state	ZTE Corporation

- R4-2406477	draftCR	Draft CR_Cell Re-selection for NR UE for Satellite in Incative state	ZTE Corporation

- R4-2406478	draftCR	Draft CR on VSAT UE timing requirements for NTN in above 10GHz	Samsung

- R4-2405594	discussion	Discussion on remaining issues for NTN in Ka band	Huawei, HiSilicon

- R4-2405595	draftCR	draftCR on measurement requirements for NTN in Ka band	Huawei, HiSilicon

- R4-2405355	draftCR	Draft CR on VSAT UE timing requirements for NTN in above 10GHz	Samsung

- R4-2405749	discussion	Considerations on Timing Adjustments for FR2-NTN	Nokia

- R4-2405191	other	Discussion on NR-NTN deployment in above 10GHz bands	ZTE Corporation

- R4-2405103	draftCR	Draft CR to TS 38.133: Removing brackets around agreed side condition.	Ericsson

- R4-2405013	discussion	Core requirements for NR-NTN RRM requirements in above 10 GHz bands	Ericsson

- R4-2404405	draftCR	DraftCR on core requirements maintenance for NTN in above 10GHz bands	CATT

- R4-2404360	discussion	On NR-NTN RRM requirements in above 10 GHz bands	Apple

- R4-2405014	discussion	Core requirements for Network verified UE location	Ericsson

- R4-2405750	discussion	Maintenance aspects of network verified UE location	Nokia

- R4-2405596	LS out	Discussion on RRM requirements for NW verified location	Huawei, HiSilicon

- R4-2405597	draftCR	draftCR on Rx-Tx measurement requirements	Huawei, HiSilicon

- R4-2406482	draftCR	draftCR on Rx-Tx measurement requirements	Huawei, HiSilicon

- R4-2406483	draftCR	draftCR on requirements for satellite switch with re-sync	Huawei, HiSilicon

- R4-2406481	draftCR	draft CR on RRC_CONNECTED state mobility for NTN	vivo

- R4-2405598	discussion	Discussion on mobility enhancements in NTN	Huawei, HiSilicon

- R4-2405599	draftCR	draftCR on requirements for satellite switch with re-sync	Huawei, HiSilicon

- R4-2405751	discussion	Discussion on Satellite switching with resynchronization	Nokia

- R4-2406479	draftCR	Draft CR for 38.133 on handover for NTN	Ericsson

- R4-2406480	draftCR	Draft CR for 38.133 on satellite switch	Ericsson

- R4-2406485	draftCR	(NR_NTN_enh-Core) draft CR on core maintenance for R18 NTN mobility and service continuity	Apple

- R4-2405015	discussion	Core requirements for NTN-TN and NTN-NTN mobility and service continuity enhancements	Ericsson

- R4-2405019	draftCR	Draft CR for 38.133 on handover for NTN	Ericsson

- R4-2405020	draftCR	Draft CR for 38.133 on satellite switch	Ericsson

- R4-2405110	discussion	Discussion on maintenance issues for NTN mobility enhancements	vivo

- R4-2405192	other	Discussion on NTN-TN and TN-NTN mobility	ZTE Corporation

- R4-2405114	draftCR	draft CR on RRC_CONNECTED state mobility for NTN	vivo

- R4-2404361	discussion	On mobility and service continuity for eNTN	Apple

- R4-2404362	draftCR	(NR_NTN_enh-Core) draft CR on core maintenance for R18 NTN mobility and service continuity	Apple

- R4-2404683	discussion	(NR_NTN_enh-Core) Discussion on the maintenance issue for NR NTN enhancement	CMCC

- R4-2404564	discussion	Discussion on NTN-TN and NTN-NTN mobility and service continuity enhancements	Xiaomi

- R4-2404562	discussion	Discussion on NR-NTN RRM performance requirements in above 10 GHz bands	Xiaomi

- R4-2404637	discussion	Discussion on NTN performance requirements in Ka band	Samsung

- R4-2404363	discussion	On NR-NTN accuracy requirements in above 10 GHz bands	Apple

- R4-2404406	discussion	Discussion on performance requirements for NTN in above 10GHz bands	CATT

- R4-2405111	discussion	Discussion on test cases desgin for NTN bands above 10GHz	vivo

- R4-2405016	discussion	RRM performance requirements for above 10GHz	Ericsson

- R4-2405752	discussion	On the configuration of test cases in FR2-NTN	Nokia

- R4-2405600	discussion	Discussion on performance requirements for Rel-18 NTN	Huawei, HiSilicon

- R4-2405435	discussion	Discussion on NR-NTN RRM performance requirements in above 10 GHz bands	MediaTek inc.

- R4-2405017	discussion	RRM performance requirements for Network verified UE location Network verified UE location	Ericsson

- R4-2405112	discussion	Discussion on performance requirements for NW verified UE location	vivo

- R4-2405113	discussion	Discussion on test cases desgin for NTN mobility enhancements	vivo

- R4-2405018	discussion	RRM performance requirements on NTN-TN and NTN-NTN mobility and service continuity enhancements	Ericsson

- R4-2404407	discussion	Discussion on performance requirements for NTN-TN and NTN-NTN mobility and service continuity enhancements	CATT

- R4-2404638	discussion	Discussion on RRM performance part for NTN mobility enhancements	Samsung

- R4-2404684	discussion	(NR_NTN_enh-Perf) Discussion on the test cases for NR NTN bands below 10GHz	CMCC

- R4-2404563	discussion	Discussion on NR-NTN RRM performance requirements in below 10 GHz bands	Xiaomi

- R4-2405753	discussion	Performance Aspects of Satellite Switch with resync	Nokia

- R4-2405542	discussion	Discussion on NR NTN SAN demodulation requirement	Ericsson

- R4-2405543	other	Simulation results on NR NTN SAN demodulation requirement	Ericsson

- R4-2405869	discussion	Discussion and simulation results for NTN enhancement	Samsung

- R4-2404444	other	Further discussion on SAN demodulation requirements for above 10 GHz bands	CATT

- R4-2404445	other	Simulation results for SAN demodulation requirements for above 10 GHz bands	CATT

- R4-2404136	discussion	Discussion on NR NTN SAN Demodulation	Nokia

- R4-2405142	discussion	Discussion on SAN demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2405143	other	Simulation results on SAN demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2405144	discussion	Discussion on UE demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2405145	other	Simulation results on UE demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2404137	discussion	Discussion on NR NTN UE Demodulation	Nokia

- R4-2404297	discussion	On UE demod requirements for NR NTN enhancement	Apple

- R4-2404298	discussion	Simulation results for FR2 NTN	Apple

- R4-2405931	discussion	[NR_NTN_enh-Perf] Discussion on the performance requirements for NR NTN enhancements	Qualcomm India Pvt Ltd

- R4-2405932	discussion	[NR_NTN_enh-Perf] Simulation resutls for NR NTN enhancements	Qualcomm India Pvt Ltd

- R4-2405481	discussion	On general and UE demodulation requirement for NTN enhancement	Ericsson

- R4-2405482	other	Simulation results for NR NTN enhancement UE demodulation	Ericsson

- R4-2405844	other	Topic summary for [110bis][325] NR_NTN_enh_SAN_UE_demod	Moderator (Huawei)

- R4-2405825	other	Topic summary for [110bis][305] NR_NTN_enh_Part1	Moderator (Thales)

- R4-2405826	other	Topic summary for [110bis][306] NR_NTN_enh_Part2	Moderator (Ericsson)

- R4-2405827	other	Topic summary for [110bis][307] NR_NTN_enh_Part3	Moderator (Samsung)

- R4-2406292	other	Ad-hoc minutes on RRM requirements for NR_NTN_enh	Samsung

- R4-2405995	other	Way Forward for [110bis][305] NR_NTN_enh_Part1 Doppler precompensation into the guard band	Huawei, Thales

- R4-2406002	other	Way Forward for [110bis][306] NR_NTN_enh_Part2 Extreme conditions and MU	Huawei

- R4-2406021	other	Ad-hoc meeting minutes for [110bis][325] NR_NTN_enh_SAN_UE_demod	Huawei

- R4-2406129	other	Ad-hoc meeting minutes for [110bis][307] NR_NTN_enh_Part3	Samsung

- R4-2406130	other	Way Forward for [110bis][307] NR_NTN_enh_Part3	Samsung

- R4-2406024	other	Way Forward for [110bis][325] NR_NTN_enh_SAN_UE_demod	Huawei

- R4-2406429	LS out	LS on UE capability for NW verified location	Qualcomm

- R4-2406430	other	WF on RRM requirements for NR_NTN_enh	Qualcomm

- R4-2406609	other	WF on Rel-18 NTN UE RF requirements	ZTE, Samsung

- R4-2405146	other	Simulation assumption on demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2405147	other	Simulation results summary on demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2404832	other	Topic summary for [110bis][221] NR_NTN_enh	Moderator (Qualcomm)

- R4-2406134	other	Way Forward for [110bis][307] NR_NTN_enh_Part3	Samsung

- R4-2406496	LS out	LS on UE capability for NW verified location	Qualcomm

RAN4#111 meeting, Fukuoka city, Japan, May 20 - 24, 2024:

- R4-2407323	other	VSAT performance requirements and testability	Eutelsat Group

- R4-2408194	CR	CR to 38.108: Correction of applicability table for SAN	NEC

- R4-2409324	draftCR	Draft CR for 38.101-5 to fix the channel raster wordings for FR2-NTN	Huawei, HiSilicon

- R4-2409323	other	Discussion on conclusion of Ka band coexistence study	Huawei, HiSilicon

- R4-2408414	CR	CR on TR 38.863 Addition of simulation results in above 10GHz	Samsung

- R4-2408415	CR	CR on TR38.863 Summary of coeixstence results in above 10GHz	Samsung

- R4-2407509	other	Discussion on note of the NTN UE ACS table	CATT

- R4-2407471	other	Discussion on ACS interferer frequency offset for SAN type 2-O	CATT

- R4-2407472	CR	(NR_NTN_enh-Core)CR for TS 38.108, Correction on ACS interferer frequency offset for SAN type 2-O	CATT

- R4-2408697	CR	NTN enhancement - Running CR to TS 38.108	Ericsson, Thales, CATT, Huawei, ZTE, NEC

- R4-2409553	CR	Updated running CR to TS 38.108 NTN Ka-band	Huawei, HiSilicon

- R4-2409541	CR	CR to TS 38.108: Band-agnostic OBUE requirement	Huawei, HiSilicon

- R4-2409542	CR	CR to TS 38.181: Band-agnostic OBUE requirement	Huawei, HiSilicon

- R4-2409556	discussion	Further discussion on test conditions for NTN SAN (Ka band)	Huawei, HiSilicon

- R4-2409561	draftCR	Updated running draft CR to TS 38.181: missing SAN type 2-O corrections	Huawei, HiSilicon

- R4-2409444	draftCR	DraftCR to 38.181: Additional correction on FR2-NTN introduction MU table	Keysight Technologies UK Ltd

- R4-2409117	draftCR	Draft CR for TS 38.181: FR2 intro in clause 9.7.5 spurious emissions	Ericsson, Thales

- R4-2408696	CR	NTN enhancement - Running CR to TS 38.181	Ericsson, CATT, Thales

- R4-2407473	draftCR	Draft CR for TS 38.181, On manufacturer declarations in clause 4.6 for Ka-band NTN	CATT

- R4-2407474	other	Discussion on SAN extreme testing conditions for NTN SAN (Ka band)	CATT

- R4-2407475	CR	(NR_NTN_enh-Perf)CR for TS 38.181, Add Declaration identifier for SAN extreme testing condition declaration	CATT

- R4-2408195	discussion	Discussion on extreme conditions for SAN	NEC

- R4-2408196	CR	CR to 38.181: Removal of extreme conditions from SAN conformance specifications	NEC

- R4-2409674	draftCR	DraftCR for TS 38.181: FR2-NTN introduction in Clause 9.6 OTA transmitted signal quality	THALES

- R4-2409619	draftCR	DraftCR for TS 38.181: Clause 10.5 OTA in-band selectivity and blocking	ZTE Corporation, Sanechips

- R4-2409620	draftCR	DraftCR for TS 38.181: Clause 10.6 OTA out-of-band blocking	ZTE Corporation, Sanechips

- R4-2409621	draftCR	DraftCR for TS 38.181: Clause 10.9 OTA in-channel selectivity	ZTE Corporation, Sanechips

- R4-2409821	draftCR	DraftCR to 38.181: Additional correction on FR2-NTN introduction MU table	Keysight Technologies UK Ltd

- R4-2409668	draftCR	DraftCR for TS 38.181: FR2-NTN introduction in Clause 9.2 Radiated transmit power	THALES, Ericsson

- R4-2409670	draftCR	DraftCR for TS 38.181: FR2-NTN introduction in Clause 9.3 OTA SAN output power	THALES, Ericsson

- R4-2409737	draftCR	DraftCR for TS 38.181: FR2-NTN introduction in Clauses 9.7.1 General 9.7.2 OTA occupied bandwidth 9.7.3 ACLR	THALES

- R4-2409735	draftCR	DraftCR for TS 38.181: FR2-NTN introduction in Clause 9.7.4 OTA out-of-band emissions	THALES

- R4-2408416	CR	Big CR to TS38.101-5	Samsung R&D Institute UK

- R4-2408698	other	NTN enhancement - NTN UE Tx requirements	Ericsson

- R4-2408700	draftCR	NTN enhancement: draft CR to TS 38.101-5 NTN Ka-band - additional Tx updates to the running CR - subclause 9.6	Ericsson, Thales

- R4-2408701	draftCR	NTN enhancement: draft CR to TS 38.101-5 NTN Ka-band - additional Tx updates to the running CR	Ericsson

- R4-2409044	other	On NTN UE RF Tx requirements	Samsung

- R4-2409047	draftCR	draft CR for TS 38.101-5 Chapter 9.2.1	Samsung

- R4-2409325	draftCR	Draft CR for 38.101-5 to clarify the polarization charactertistic for general Tx requirements	Huawei, HiSilicon

- R4-2409329	other	Discussion on Tx requirement for Ka band NTN UE	Huawei, HiSilicon

- R4-2409331	CR	CR for TR 38.863 to introduce some technical background for R18 NTN VSAT UE Tx requirements	Huawei, HiSilicon

- R4-2407462	draftCR	Draft CR for R4-2405085 to add OFF-axis AMPR	Qualcomm Incorporated

- R4-2409758	draftCR	Tx Corrections to TS 38.101-5	THALES, Ericsson

- R4-2409777	draftCR	Corrections to EIRPmax in TS 38.101-5	THALES

- R4-2409616	other	Further discussion on Tx RF requirements for NTN in Ka-band	ZTE Corporation, Sanechips

- R4-2409617	other	Further discussion on Rx RF requirements for NTN in Ka-band	ZTE Corporation, Sanechips

- R4-2409618	draftCR	Draft CR to TS 38.101-5 Clause 10.4, 10.6, 10.8 and Annex	ZTE Corporation, Sanechips

- R4-2409332	CR	CR for TR 38.863 to introduce some technical background for R18 NTN VSAT UE Rx requirements	Huawei, HiSilicon

- R4-2409330	other	Discussion on Rx requirement for Ka band NTN UE	Huawei, HiSilicon

- R4-2409326	draftCR	Draft CR for 38.101-5 to introduce clause 10.1~10.3	Huawei, HiSilicon

- R4-2409048	draftCR	draft CR for TS 38.101-5 Chapter 10.3	Samsung

- R4-2409045	other	On NTN UE RF Rx requirements	Samsung

- R4-2408702	draftCR	NTN enhancement: draft CR to TS 38.101-5 NTN Ka-band - additional Rx updates to the running CR	Ericsson

- R4-2408699	other	NTN enhancement - NTN UE Rx requirements	Ericsson

- R4-2409327	draftCR	Draft CR for 38.101-5 to introduce NS for regional regulatory requirements	Huawei, HiSilicon

- R4-2409328	other	Discussion on how to organize the regulation requirements for Ka band	Huawei, HiSilicon

- R4-2407460	draftCR	(NR_NTN_enh-Core) Baseline-Big CR for RAN4#111 draft CRs to TS 38.133 on RRM requirements for NR NTN enhancement_v0	Qualcomm Incorporated

- R4-2407305	discussion	On RRM requirements in bands above 10GHz	Apple

- R4-2407841	draftCR	draftCR on L3-RSRP measurement requirements maintenance in above 10 GHz scenario	Xiaomi

- R4-2407677	draftCR	Draft CR on VSAT UE timing requirements for NTN in above 10GHz	Samsung

- R4-2409284	discussion	Discussion on remaining issues for NTN in Ka band	Huawei, HiSilicon

- R4-2409285	draftCR	draftCR on measurement requirements for NTN in Ka band	Huawei, HiSilicon

- R4-2409056	other	On side condition for timing requirements	Ericsson

- R4-2409058	draftCR	Draft CR to TS 38.133: Removing side condition.	Ericsson

- R4-2409059	LS out	LS reply on reference point for SSB-TimeOffset	Ericsson

- R4-2408868	discussion	Discussion on maintenance issues for NTN bands above 10GHz	vivo

- R4-2408419	discussion	Discussion on RRM core maintenance for NTN in above 10GHz	Samsung

- R4-2408510	discussion	Discussion on Side Conditions for UE transmit timing requirements	Nokia

- R4-2408511	discussion	Discussion on UE RX-TX Time difference	Nokia

- R4-2408605	discussion	Core requirements for Network verified UE location	Ericsson

- R4-2408869	discussion	Discussion on maintenance issues for NW verified UE location	vivo

- R4-2409286	discussion	Discussion on RRM requirements for NW verified location	Huawei, HiSilicon

- R4-2409287	draftCR	draftCR on Rx-Tx measurement requirements	Huawei, HiSilicon

- R4-2409288	LS out	Discussion on mobility enhancements in NTN	Huawei, HiSilicon

- R4-2409289	draftCR	draftCR on requirements for satellite switch with re-sync	Huawei, HiSilicon

- R4-2408867	discussion	Discussion on maintenance issues for NTN mobility enhancements	vivo

- R4-2408606	discussion	Core requirements for NTN-TN and NTN-NTN mobility and service continuity enhancements	Ericsson

- R4-2408512	CR	CR to TS 38.133 on applicability of soft satellite switching requirements	Nokia

- R4-2408513	CR	CR to TS 38.133 on RLM and measurements during satellite switching with resynchronization	Nokia

- R4-2408514	discussion	Considerations on Soft Satellite Switch Capability	Nokia

- R4-2407840	discussion	Discussion on NTN-TN and NTN-NTN mobility and service continuity enhancements	Xiaomi

- R4-2407932	discussion	(NR_NTN_enh-Core) Discussion on the maintenance issue and the LS from RAN2 for NR NTN enhancement	CMCC

- R4-2407306	discussion	On mobility and service continuity for eNTN	Apple

- R4-2407307	LS out	Reply LS on reference point for SSB-TimeOffset	Apple

- R4-2408608	draftCR	Draft CR for 38.133 on Radio Link Monitoring test for NTN	Ericsson

- R4-2408870	discussion	Discussion on test cases design for NTN bands above 10GHz	vivo

- R4-2408864	draftCR	Draft CR on test case for NTN RLM requirements in bands above 10GHz	vivo

- R4-2408515	discussion	Dicussion on Testability of Case 3 UEs in FR2-NTN	Nokia

- R4-2408420	discussion	Discussion on NTN performance requirements in Ka band	Samsung

- R4-2409290	discussion	Discussion on performance requirements for NTN in Ka band	Huawei, HiSilicon

- R4-2409291	draftCR	draftCR on TC for inter-satellite HO for FR2-NTN	Huawei, HiSilicon

- R4-2407308	discussion	On NR-NTN accuracy requirements in above 10 GHz bands	Apple

- R4-2407461	draftCR	(NR_NTN_enh-Perf) draft CR on TC for Connected mode mobility in FR2-NTN	Qualcomm Incorporated

- R4-2407364	draftCR	(NR_NTN_enh-Perf) Test case of SSB based L1-RSRP measurement for NTN above 10GHz	Apple

- R4-2407842	draftCR	draftCR on test case for L3-RSRP measurement without gap under non-DRX with SSB index reading in above 10 GHz scenario	Xiaomi

- R4-2407678	draftCR	Draft CR on test cases of VSAT UE timing requirements for NTN in above 10GHz	Samsung

- R4-2409292	draftCR	draftCR on UE Rx-Tx time difference accuracy requirements	Huawei, HiSilicon

- R4-2409293	discussion	Discussion on RRM tests for mobility enhancement in NTN	Huawei, HiSilicon

- R4-2408421	discussion	Discussion on RRM performance part for NTN mobility enhancements	Samsung

- R4-2408607	discussion	RRM performance requirements for NTN enhancements	Ericsson

- R4-2407933	draftCR	(NR_NTN_enh-Perf) draftCR to TS 38.133 Introduction of satellite switch test cases for NTN enh	CMCC

- R4-2407964	draftCR	Draft CR on TC for NTN-NTN time-based trigger CHO enhancements for NR NTN	LG Electronics Inc.

- R4-2407195	discussion	Discussion on the performance requirement for NR NTN enhancement below 10GHz	MediaTek inc.

- R4-2407196	draftCR	Introduce the test for NTN to NTN RACH-less HO	MediaTek inc.

- R4-2409691	draftCR	Draft CR _Cell Reselection for NR UE for Satellite in IDLE state	ZTECorporation,Sanechips

- R4-2409692	draftCR	Draft CR _Cell Reselection for NR UE for Satellite in INACTIVE state	ZTECorporation,Sanechips

- R4-2409695	draftCR	Draft CR for test case on NTN to LTE TN	ZTECorporation,Sanechips

- R4-2409696	draftCR	Draft CR for test case on NTN to NR TN	ZTECorporation,Sanechips

- R4-2409867	other	Simulation assumption on demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2408977	other	Simulation assumption on demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2408978	other	Simulation results summary on demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2408981	draftCR	Draft CR on NTN radiated performance requirements for PUSCH (TS38.108, Rel-18)	Huawei,HiSilicon

- R4-2408982	draftCR	Draft CR on MU, manufacturer declarations and applicability rules for NTN (TS38.181, Rel-18)	Huawei,HiSilicon

- R4-2408983	draftCR	Draft CR on NTN OTA performance requirements for PUCCH (TS38.181, Rel-18)	Huawei,HiSilicon

- R4-2408973	discussion	Discussion on SAN demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2408974	other	Simulation results on SAN demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2409482	discussion	Discussion and simulation results for NTN enhancement	Samsung

- R4-2409483	draftCR	Draft CR on performance requirements for PUSCH with DM-RS bundling	Samsung

- R4-2409484	draftCR	Draft CR on OTA performance requirements for PUSCH	Samsung

- R4-2408339	discussion	Discussion on NR NTN enhancement SAN demodulation requirements	Ericsson

- R4-2408340	other	Simulation results for NR NTN enhancement SAN demodulation requirements	Ericsson

- R4-2408341	draftCR	(NR_NTN_enh-Perf) Draft CR for 38.181 on SAN demodulation requirements	Ericsson

- R4-2408342	draftCR	(NR_NTN_enh-Perf) Draft big CR for 38.181 on SAN demodulation requirements	Ericsson

- R4-2407142	discussion	Discussion on NR NTN SAN Demodulation	Nokia

- R4-2407145	draftCR	[NR_NTN_enh-Perf] draftCR on performance requirements for 38.101-5	Nokia

- R4-2407510	draftCR	Draft CR for TS 38.181, Introduction on OTA performance requirement for PRACH	CATT

- R4-2407476	other	Simulation results for SAN demodulation requirements for above 10 GHz bands	CATT

- R4-2407477	draftCR	Draft CR for TS 38.108, On Performance requirements for PRACH in clause 11.4 for Ka-band NTN	CATT

- R4-2409869	draftCR	Draft CR on NTN radiated performance requirements for PUSCH (TS38.108, Rel-18)	Huawei,HiSilicon

- R4-2409870	draftCR	Draft CR on NTN OTA performance requirements for PUCCH (TS38.181, Rel-18)	Huawei,HiSilicon

- R4-2409865	draftCR	(NR_NTN_enh-Perf) Draft CR for 38.181 on SAN demodulation requirements	Ericsson

- R4-2409858	draftCR	[NR_NTN_enh-Perf] draftCR on performance requirements for 38.101-5	Nokia

- R4-2409909	CR	(NR_NTN_enh-Perf) Big CR for 38.181 on SAN demodulation requirements	Ericsson

- R4-2409864	draftCR	Draft CR for TS 38.181, Introduction on OTA performance requirement for PRACH	CATT

- R4-2409847	other	Simulation results for SAN demodulation requirements for above 10 GHz bands	CATT

- R4-2409863	draftCR	Draft CR for TS 38.108, On Performance requirements for PRACH in clause 11.4 for Ka-band NTN	CATT

- R4-2409871	draftCR	Draft CR on performance requirements for PUSCH with DM-RS bundling	Samsung

- R4-2409872	draftCR	Draft CR on OTA performance requirements for PUSCH	Samsung

- R4-2409862	draftCR	Draft CR to 38.101-5 on eNTN demod requirements for PDCCH	Apple

- R4-2409859	draftCR	[NR_NTN_enh-Perf] draftCR on PUCCH performance requirements for 38.108	Nokia

- R4-2409860	draftCR	[NR_NTN_enh-Perf] draftCR on propagation conditions and channels for 38.108	Nokia

- R4-2409861	draftCR	[NR_NTN_enh-Perf] draftCR on PUSCH demodulation requirements for 38.181	Nokia

- R4-2409868	draftCR	Draft CR on NTN PDSCH demodulation requirements (TS38.101-5, Rel-18)	Huawei,HiSilicon

- R4-2409866	draftCR	[NR_NTN_enh-Perf] Draft CR to 38.101-5 Reference measurement channel for PDCCH requirements and channel model for NR NTN enhancements	Qualcomm India Pvt Ltd

- R4-2407357	draftCR	Draft CR to 38.101-5 for updates to Annex C	Apple

- R4-2407146	draftCR	[NR_NTN_enh-Perf] draftCR on PUCCH performance requirements for 38.108	Nokia

- R4-2407147	draftCR	[NR_NTN_enh-Perf] draftCR on propagation conditions and channels for 38.108	Nokia

- R4-2407148	draftCR	[NR_NTN_enh-Perf] draftCR on PUSCH demodulation requirements for 38.181	Nokia

- R4-2407149	CR	[NR_NTN_enh-Perf] bigCR for 38.108, NR_NTN Demodulation requirements	Nokia

- R4-2407143	other	Supporting Simulations for NR NTN UE Demodulation	Nokia

- R4-2407144	discussion	Discussion on NR NTN UE Demodulation	Nokia

- R4-2407251	discussion	Simulation results for FR2 NTN	Apple

- R4-2407252	draftCR	Draft CR to 38.101-5 on eNTN demod requirements for PDCCH	Apple

- R4-2408641	draftCR	[NR_NTN_enh-Perf] Draft CR to 38.101-5 Reference measurement channel for PDCCH requirements and channel model for NR NTN enhancements	Qualcomm India Pvt Ltd

- R4-2408678	discussion	[NR_NTN_enh-Perf] Discussion on the performance requirements for NR NTN enhancements	Qualcomm India Pvt Ltd

- R4-2408679	discussion	[NR_NTN_enh-Perf] Simulation results for NR NTN enhancements	Qualcomm India Pvt Ltd

- R4-2408745	draftCR	Draft CR to 38.101-5 FRC for PDSCH performance requirement	Ericsson

- R4-2408746	discussion	On UE demodulation requirement for NTN enhancement	Ericsson

- R4-2408747	other	Simulation results for NR NTN enhancement UE demodulation	Ericsson

- R4-2408975	discussion	Discussion on UE demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2408976	other	Simulation results on UE demodulation requirements for NR NTN enhancements	Huawei,HiSilicon

- R4-2408979	CR	Big CR on NTN demodulation requirements (TS38.101-5, Rel-18)	Huawei,HiSilicon

- R4-2408980	draftCR	Draft CR on NTN PDSCH demodulation requirements (TS38.101-5, Rel-18)	Huawei,HiSilicon

- R4-2408931	other	Topic summary for [111][120] NR_NTN_enh_UERF_R18	Moderator(ZTE)

- R4-2408016	other	Topic summary for [111][219] NR_NTN_enh	Moderator (Qualcomm)

- R4-2409853	other	Ad-hoc meeting minutes for [111][325] NR_NTN_enh_SAN_UE_demod	Huawei

- R4-2410118	other	Topic summary for [111][325] NR_NTN_enh_SAN_UE_demod	Moderator (Huawei)

- R4-2410101	other	Topic summary for [111][306] NR_NTN_enh_Part2	Moderator (Ericsson)

- R4-2410102	other	Topic summary for [111][307] NR_NTN_enh_Part3	Moderator (Samsung)

- R4-2410137	other	Ad-hoc minutes for NR_NTN_enh WI	Samsung

- R4-2409876	other	Way Forward for [111][325] NR_NTN_enh_SAN_UE_demod	Huawei

- R4-2409857	other	Ad-hoc meeting minutes for [111][325] NR_NTN_enh_SAN_UE_demod	Huawei

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template