3GPP TSG RAN meeting #110									           RP-253138

Baltimore, USA, December 15-18th, 2025						rev of RP-251934

## Status Report to TSG

Title:			Status report for SID: Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation; rapporteur: Thales

Agenda item:			9.2.2

| WI / SI Name | Rel-20 Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_NTN_GNSS_resilient |  |  |  |  |
| Unique ID | 1080071 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251933 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2026 | Core part: N/A | Performance part: N/A | Testing part: |  |
| Overall Completion level | Study Item: Overall: 10% RAN1: 50% RAN4: 0% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Mohamed El Jaafari |
|  | Company | Thales |
|  | Email | Mohamed.el-jaafari@thalesaleniaspace.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Note that the updated work plan can be found in R1-2508470Work plan for NR NTN Phase 4  THALES

#### 2.1.1.1 Decisions during RAN1#122bis

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions:

- For scenario 1 and 2

  - The following time/frequency differences should be evaluated:

    - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area.

      - For UL performance evaluation 2 times differential one-way delay should be considered.

    - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area.

      - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported.

Agreement:

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration) - 2 Te  Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or (scaling factor*(Differential Doppler) +2* fe×fc) ≥

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

Agreement:

RAN1 to assess the impacts on connected mode during NR NTN GNSS-resilient operation, including at least:

- Impact on signalling overhead.

Agreement:

In RAN1#123, based on the following alternatives representing two calculation methodologies, select a calculation methodology with potential adjustments, including correction of equations:

Alt 1:

For the calculation of the differential one-way delay and the differential one-way Doppler/frequency offset the following could be considered:

Geometry model and equations could be considered. For the variables in the equations, refer to Figure below.

For non-nadir beam

|  | ??1=??-2RcRE |
| --- | --- |
|  | drefnear=RE+hue2+RE+hs2-2RE+hueRE+hscos??1 |
|  | ?21=sin-1RE+huedrefnearsin??1 |
|  | ?3=cos-1RE+hsdrefnearsin??1 |
|  | fdrefnear=vssin?21+vuecos?3cfc |
|  | a??t=d-drefdfar-dnearc |
|  | a??fd=fdref-fdestfdfar-fdnear For nadir beam dnear=hs-hue ??=RcRE dfar=RE+hue2+RE+hs2-2RE+hueRE+hscos?? ?2=sin-1RE+huedfarsin?? a??t=dfar-dnearc a??fd=2fdfar |

Note 1: UE is at same height hue as the reference point.

Note 2: For the purpose of evaluation, fixed size of uncertainty area is used within the whole satellite coverage footprint is a baseline. Other size of uncertainty area is not precluded.

FFS : Whether the minimum elevation is at farthest w.r.t satellite point of the uncertainty area or at the center of the uncertainty area

FFS: How to take into account the UE altitude uncertainty.

Alt 2:

For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered. For the variables in the equations, refer to Figure-X.

Considering satellite location at s=0,0,RE+h

One-way delay between a point u and satellite:
done-wayu=|u-s|2c

One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along  with speed vue:

dopplerone-wayu=vcfc<u-s,x>done-wayu+vuecfc<s-u,k>done-wayu

Let R be the location uncertainty area.

Differential one-way delay = maxu∈R done-wayu-minu∈Rdone-wayu

Differential one-way doppler = maxu∈Rdopplerone-wayu-minua∈Rdopplerone-way(u)

For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ?? from the orbital plane (x-y plane).

Note: Assume satellite and UE moves parallel to the x-y plane. <⋅,⋅> is the vector inner product, ||.|​2 is the vector magnitude.

#### 2.1.1.2 Decisions during RAN1#123

Agreement:

For the calculation of one-way differential delay and one-way differential Doppler, adopt Alt2 in RAN1#112bis agreement with the following correction on the one-way Doppler:

dopplerone-way(u)=vc2fc<u-s,x>done-way(u)+vc2fc<u-s,k>done-way(u)

Note: Earth’s rotational motion is neglected for LEO.

Agreement:

The inequation for the limit on differential roundtrip delay in the RAN#122bis Agreement (copied below), is not applicable for unrestricted set.

For unrestricted set PRACH tolerance is exceeded if

min (Ncp, Sequence duration) - 2 Te ≤ Differential RTT.

Agreement (RAN1#122bis):

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration×maxNcsL) -2Te ≤ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) ≥ 1+γΔfRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

Agreement:

For PRACH performance evaluation,  Te  used for the limit on differential roundtrip delay determination is to be set according to TS 38.133, Table 7.1.2-1.

Working assumption:

For UL performance evaluation, UL differential Doppler/frequency offset is calculated based on scaled DL one-way differential Doppler/frequency offset with the following scaling factor:

scaling factor=1+fc,ul/fc,dl

Scaling factor = 2*fc,ul / fc,dl

fc,ul and fc,dl  are the uplink and downlink carrier frequency respectively.

Agreement

For the study of GNSS resilient operation, for evaluation purposes, it is assumed that the UE pre-compensates RTT and Doppler on the service link for UL transmission. This may include, but not limited to:

- At least for Scenario 1, based on assistance information provided by the Network.

- For Scenario 2, based on at least the last known UE location.

Working assumption

Capture in the TR 38.742 the following tables of differential one-way delay reflecting results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1: within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.3 |  |  | 357.4 |
|  | 30 | 143.3 | 257.8 |  | 57.6 | 115.1 |  |
|  | 90 | 1.9 | 3.3 | 4.8 | 0.3 | 0.7 | 0.9 |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1451.9 |  |  | 906.6 |
|  | 30 | 255.3 | 535.3 |  | 143.1 | 257.1 |  |
|  | 90 | 6.1 | 14.9 | 15.5 | 1.9 | 3.3 | 6.0 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.2 |  |  | 357.4 |
|  | 30 | 142.9 | 257.7 |  | 57.5 | 114.9 |  |
|  | 90 | 1.9 | 3.3 |  | 0.3 | 0.7 |  |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1451.8 |  |  | 906.6 |
|  | 30 | 255.3 | 535.2 |  | 143.0 | 257.0 |  |
|  | 90 | 6.1 | 14.9 |  | 1.9 | 3.3 |  |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.515 | 32.554 | 65.106 | 162.779 |
|  | 30 | 5.8 | 28.8 | 57.6 | 143.2 | 5.8 | 28.8 | 57.646 | 143.603 |  |  |  |  |
|  | 90 | 0.0 | 0.1 | 0.3 | 1.8 | 0.0 | 0.0 | 0.1 | 0.9 | 0.0 | 0.0 | 0.0 | 0.2 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.5 | 32.5 | 65.1 | 162.8 |
|  | 30 | 5.8 | 28.8 | 57.5 | 142.9 | 5.8 | 28.8 | 57.6 | 143.3 |  |  |  |  |
|  | 90 | 0.003 | 0.1 | 0.3 | 1.9 | 0.0 | 0.0 | 0.2 | 1.0 | 0.0 | 0.0 | 0.0 | 0.2 |

Note: For the above results, UE altitude of 0 km is assumed.

Note: These values do not take into account UE altitude uncertainty.

Note: For the above results for Set1/2 parameters, it is assumed that the area served by the cell or beam (corresponding to the nadir beam size as defined in 38.821) is fixed within the satellite coverage.

Working assumption

Capture in the TR 38.742 the following tables of differential one-way Doppler reflecting results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1: within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.011 |  |  | 0.005 |
|  | 30 | 0.371 | 0.390 |  | 0.146 | 0.168 |  |
|  | 90 | 2.112 | 1.843 | 0.046 | 0.848 | 0.767 | 0.020 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.023 |  |  | 0.012 |
|  | 30 | 0.754 | 0.921 |  | 0.410 | 0.425 |  |
|  | 90 | 3.790 | 3.861 | 0.102 | 2.118 | 1.843 | 0.052 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.041 |  |  | 0.024 |
|  | 30 | 1.212 | 1.125 |  | 0.497 | 0.514 |  |
|  | 90 | 2.100 | 1.805 | 0.044 | 0.855 | 0.821 | 0.024 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.070 |  |  | 0.054 |
|  | 30 | 2.191 | 2.386 |  | 1.224 | 1.136 |  |
|  | 90 | 3.776 | 3.799 | 0.079 | 2.138 | 1.849 | 0.062 |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.000 | 0.000 | 0.001 | 0.002 |
|  | 30 | 0.015 | 0.075 | 0.150 | 0.387 | 0.009 | 0.043 | 0.086 | 0.219 |  |  |  |  |
|  | 90 | 0.085 | 0.407 | 0.861 | 2.034 | 0.041 | 0.208 | 0.415 | 1.038 | 0.000 | 0.002 | 0.004 | 0.009 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.001 | 0.002 | 0.003 | 0.007 |
|  | 30 | 0.047 | 0.235 | 0.472 | 1.189 | 0.024 | 0.121 | 0.243 | 0.610 |  |  |  |  |
|  | 90 | 0.084 | 0.421 | 0.841 | 2.102 | 0.040 | 0.202 | 0.404 | 1.010 |  |  |  |  |

Note: The above results do not take into account the Earth rotation.

Note: These values are for UE speed of 3 km/h.

Note: For the above results, UE altitude of 0 km is assumed.

Note: These values do not take into account UE altitude uncertainty.

Note: For the above results for Set1/2 parameters, it is assumed that the area served by the cell or beam (corresponding to the nadir beam size as defined in 38.821) is fixed within the satellite coverage.

Working assumption

Differential roundtrip delay limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Unrestricted | 839 | 103,13 | 419 | 0,39 | 800 | 102,34 |
| 0 | type A | 839 | 103,13 | 237 | 0,39 | 800 | 102,34 |
| 0 | type B | 839 | 103,13 | 137 | 0,39 | 800 | 102,34 |
| 1 | unrestricted | 839 | 684,38 | 419 | 0,39 | 800 | 683,59 |
| 1 | type A | 839 | 684,38 | 237 | 0,39 | 800 | 225,20 |
| 1 | type B | 839 | 684,38 | 137 | 0,39 | 800 | 129,85 |
| 2 | unrestricted | 839 | 152,60 | 419 | 0,39 | 800 | 151,82 |
| 2 | type A | 839 | 152,60 | 237 | 0,39 | 800 | 151,82 |
| 2 | type B | 839 | 152,60 | 137 | 0,39 | 800 | 129,85 |
| 3 | unrestricted | 839 | 103,13 | 419 | 0,39 | 200 | 102,34 |
| 3 | type A | 839 | 103,13 | 237 | 0,39 | 200 | 55,71 |
| 3 | type B | 839 | 103,13 | 137 | 0,39 | 200 | 31,88 |

Differential roundtrip delay limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 15 | 139 | 9.38 | 69 | 0.39 | 66.67 | 8.59 |
| A1 | 30 | 139 | 4.69 | 69 | 0.33 | 33.33 | 4.04 |
| A1 | 60 | 139 | 2.34 | 69 | 0.11 | 16.67 | 2.12 |
| A1 | 120 | 139 | 1.17 | 69 | 0.11 | 8.33 | 0.94 |
| A2 | 15 | 139 | 18.75 | 69 | 0.39 | 66.67 | 17.97 |
| A2 | 30 | 139 | 9.38 | 69 | 0.33 | 33.33 | 8.72 |
| A2 | 60 | 139 | 4.69 | 69 | 0.11 | 16.67 | 4.46 |
| A2 | 120 | 139 | 2.34 | 69 | 0.11 | 8.33 | 2.12 |
| A3 | 15 | 139 | 28.13 | 69 | 0.39 | 66.67 | 27.34 |
| A3 | 30 | 139 | 14.06 | 69 | 0.33 | 33.33 | 13.41 |
| A3 | 60 | 139 | 7.03 | 69 | 0.11 | 16.67 | 6.80 |
| A3 | 120 | 139 | 3.52 | 69 | 0.11 | 8.33 | 3.29 |
| B1 | 15 | 139 | 7.03 | 69 | 0.39 | 66.67 | 6.25 |
| B1 | 30 | 139 | 3.52 | 69 | 0.33 | 33.33 | 2.86 |
| B1 | 60 | 139 | 1.76 | 69 | 0.11 | 16.67 | 1.53 |
| B1 | 120 | 139 | 0.88 | 69 | 0.11 | 8.33 | 0.65 |
| B2 | 15 | 139 | 11.72 | 69 | 0.39 | 66.67 | 10.94 |
| B2 | 30 | 139 | 5.86 | 69 | 0.33 | 33.33 | 5.21 |
| B2 | 60 | 139 | 2.93 | 69 | 0.11 | 16.67 | 2.70 |
| B2 | 120 | 139 | 1.46 | 69 | 0.11 | 8.33 | 1.24 |
| B3 | 15 | 139 | 16.41 | 69 | 0.39 | 66.67 | 15.63 |
| B3 | 30 | 139 | 8.20 | 69 | 0.33 | 33.33 | 7.55 |
| B3 | 60 | 139 | 4.10 | 69 | 0.11 | 16.67 | 3.87 |
| B3 | 120 | 139 | 2.05 | 69 | 0.11 | 8.33 | 1.82 |
| B4 | 15 | 139 | 30.47 | 69 | 0.39 | 66.67 | 29.69 |
| B4 | 30 | 139 | 15.23 | 69 | 0.33 | 33.33 | 14.58 |
| B4 | 60 | 139 | 7.62 | 69 | 0.11 | 16.67 | 7.39 |
| B4 | 120 | 139 | 3.81 | 69 | 0.11 | 8.33 | 3.58 |
| C0 | 15 | 139 | 40.36 | 69 | 0.39 | 66.67 | 39.58 |
| C0 | 30 | 139 | 20.18 | 69 | 0.33 | 33.33 | 19.52 |
| C0 | 60 | 139 | 10.09 | 69 | 0.11 | 16.67 | 9.87 |
| C0 | 120 | 139 | 5.05 | 69 | 0.11 | 8.33 | 4.82 |
| C2 | 15 | 139 | 66.67 | 69 | 0.39 | 66.67 | 65.89 |
| C2 | 30 | 139 | 33.33 | 69 | 0.33 | 33.33 | 32.67 |
| C2 | 60 | 139 | 16.67 | 69 | 0.11 | 16.67 | 16.45 |
| C2 | 120 | 139 | 8.33 | 69 | 0.11 | 8.33 | 8.11 |

Working assumption

Theoretical differential Doppler limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0,1,2 | unrestricted | 1250 | 0 | 0.1 | 2 | 2 | 2.0 | 0.21 | 0.85 |
| 0,1,2 | type A | 1250 | 1 | 0.1 | 2 | 2 | 2.0 | 0.53 | 2.1 |
| 0,1,2 | type A | 1250 | 2 | 0.1 | 2 | 2 | 2.0 | 0.84 | 3.35 |
| 0,1,2 | type B | 1250 | 3 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 0,1,2 | type B | 1250 | 4 | 0.1 | 2 | 2 | 2.0 | 1.46 | 5.85 |
| 3 | unrestricted | 5000 | 0 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 3 | type A | 5000 | 1 | 0.1 | 2 | 2 | 2.0 | 2.40 | 9.6 |
| 3 | type A | 5000 | 2 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
| 3 | type B | 5000 | 3 | 0.1 | 2 | 2 | 2.0 | 4.90 | 19.6 |
| 3 | type B | 5000 | 4 | 0.1 | 2 | 2 | 2.0 | 6.15 | 24.6 |

Theoretical differential Doppler limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
|  | 30 | 30000 | 0 | 0.1 | 2 | 2 | 2.0 | 7.40 | 29.6 |
|  | 60 | 60000 | 0 | 0.1 | 30 | 20 | 2.5 | 1.08 | 54 |
|  | 120 | 120000 | 0 | 0.1 | 30 | 20 | 2.5 | 2.28 | 114 |

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following solutions at least for initial access procedure at least to increase PRACH tolerance and/or reducing time/frequency uncertainty:

- Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

- Solution 1B: New PRACH restricted sets.

- Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in single/multiple RAR MAC PDU scheduling multiple msg3 with different TA/FA.

- Solution 1D: Signalling enhancements for Msg2/Msg4 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command).

- Solution 1E: configuration/selection of a set of existing PRACH root for a Single PRACH transmission.

- Solution 1F: PRACH transmission using TA margin indicated by Network and RO masking.

- Solution 1G: Adaptation of PRACH configuration.

- Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

- Solution 2B: Multiple random access attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

- Solution 2C: Solutions based on broadcasting DL timestamp(s).

- Solution 2D: UE side time/frequency pre-compensation based on reference location or TA/Doppler compensation information provided by gNB.

- Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

- Solution 2F: UE side time/frequency pre-compensation based on reference signal.

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

Other Solutions are not precluded.

As part of the evaluation of the solutions above, at least the following aspects should be assessed:

- specification impact when relevant (including whether the solutions remain within the scope of the SID)

- performance,

- applicability to different scenarios,

- complexity,

- Coexistence with legacy UEs

- the signalling overhead.

Agreement:

With respect solution 2A, for the evaluation of DL-only UE-based positioning method, using DL-TDOA, consider the following evaluation assumptions and parameters:

| Parameter | Description/Value |
| --- | --- |
| Satellite Orbit | As per RAN1#122 agreement |
| Satellite parameters | As per RAN1#122 agreement |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | As per RAN1#122 agreement |
| BW | To be reported by companies. Baseline: 10 MHz for FR1 and 50 MHz for FR2 |
| Subcarrier spacing | 15 kHz for FR1-NTN, 120 kHz for FR2-NTN |
| Number of satellites in view | Case 1: 1 Satellite Case 2: At least 2 Satellites |
| UE type | As per RAN1#122 agreement |
| Positioning signals | DL-PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL: Comb-2 |
| Number of symbols used per occasion | 6 |
| Number of DL-PRS occasions | To be reported. |
| Time window for measurement collection | To be reported. |
| UE speed | As per RAN1#122 agreement |
| UE minimum elevation angle | For Case 1, as per RAN1#122 agreement. For Case 2, to be reported. |
| Ue clock drift/error | To be reported |
| Performance metrics | UE 2D as baseline. 3D can be reported by companies. Residual UL timing and frequency error after applying pre-compensation based on positioning |
| UE position estimation algorithme | To be reported. |

Agreement:

The skeleton for TR 38.742 proposed in R1-2508479 is endorsed in R1-2509611 with the following updates in red:

# 4 Background and motivation ## 4.1 Review of previous NR-NTN releases ## 4.2 Needs for GNSS resilience NR NTN operation ## 4.3 Use cases and deployment scenarios # 5 Assumptions and study parameters ## 5.1 GNSS availability scenarios ## 5.2 System assumptions ## 5.3 Evaluation scenarios and parameters # 6 Evaluation of impacts of GNSS temporary unavailability ## 6.1 Impact on synchronization in initial access ## 6.2 Impact on synchronization in connected mode # 7 Evaluation of solutions for GNSS resilient NR-NTN ## 7.1 Potential solutions for initial access ## 7.1.1 Evaluation of solutions ## 7.2 Potential solutions for RRC connected mode ## 7.2.1 Evaluation of solutions

# 8 Conclusions and recommendations ## 8.1 Summary of findings ## 8.2 Open issues and future work ## 8.3 Recommendations for normative work

Agreement:

To assess the impacts on connected mode during NR-NTN GNSS-resilient operation, evaluate the signalling overhead for a UE in RRC Connected mode under the following assumptions:

| Parameter | Description/Value |
| --- | --- |
| Satellite Orbit | As per RAN1#122 agreement |
| Satellite parameters | As per RAN1#122 agreement |
| Cell type | Quasi-Earth Fixed cell - Moving cell may be considered and reported. |
| Range of elevation angle | Baseline: From 90 degree to minimum elevation angle. Other values outside orbital plane to be reported. In this case, starting point is the highest elevation angle, and ending at the minimum elevation angle. |
| UE pre-compensation of RTT/Doppler on the service link | Case 1: At least for Scenario 1, based on assistance information provided by the Network Case 2: For Scenario 2, based on at least the last known UE location Details to be reported. Other pre-compensation methods can be reported by companies. |
| Duty cycle | Baseline: Continuous UL transmission every slot Other approaches if used, to be reported. |
| UE speed | Baseline: 3 km/h To be reported |
| Timing tolerance when connected TtolTUL (Note 1) | To be reported |
| Frequency tolerance when connected Ftol (Note 1) | To be reported |
| Initial timing error | Baseline: 0 To be reported |
| Initial frequency error | Baseline: 0 To be reported |
| Other | To be reported Note: Details of deriving UL timing and frequency error at gNB to be reported. |
| Performance metrics | Minimum period between two consecutive adjustment commands (in seconds) Maximum period between two consecutive adjustment commands (in seconds) Average number of adjustment commands per second Average number of times frequency error exceeds tolerance limit per second Average number of times timing error exceeds tolerance limit per second Other metrics may be considered and reported. |
| Note 1: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively. |  |

Conclusion:

A Post-meeting Email discussion for Rel-20 NR-NTN is assigned for the following tasks:

- Collection of companies evaluation results for PRACH performance.

  - Note: The spreadsheet for PRACH performance evaluation embedded in R1-2509609 is used to collect companies’ results.

- Endorsement of a template for collection of results for Connected mode.

Duration of Email discussion: 2nd to 4th of February 2026.

Moderator: THALES (Mohamed)

#### 2.1.2	Remaining Open issues

Consolidate evaluation results.

Evaluate potential solutions for NR-NTN resilience to GNSS unavailability and degradation

Identify impact on physical layer procedures

Identify and assess backward compatible issues with legacy NTN capable UEs

## 2.2	RAN4

#### 2.2.1	Agreements

None

#### 2.2.1.1 Decisions during RAN4#116bis

None

#### 2.2.1.2 Decisions during RAN4#117

None

#### 2.2.2	Remaining Open issues

Assess potential impact on RRM specifications related to initial access and connected mode procedures for NR-NTN.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention. 
-

## 4.	References

## 4.1	RAN1

RAN1#122bis meeting, Prague, Czech Republik, October 13-17th, 2025:

- R1-2506850	Work Plan	Work plan for NR NTN Phase 4	THALES

- R1-2508153	discussion	Session Notes of AI 10.6	Ad-Hoc Chair (Ericsson)

- R1-2507952	discussion	Compilation of company proposals regarding NR-NTN GNSS resilience	Moderator (Thales)

- R1-2507784	discussion	Discussion of NR-NTN GNSS Resilience	Johns Hopkins University APL

- R1-2507782	discussion	Discussion on GNSS-resilient NR-NTN operation	ISSDU, NYCU

- R1-2508178	discussion	FL Summary #5 - NR-NTN GNSS resilience	Moderator (Thales)

- R1-2506842	discussion	Considerations on NR-NTN Resilience to GNSS Unavailability and Degradation	THALES

- R1-2506917	discussion	Discussion on NR-NTN GNSS resilience	ZTE Corporation, Sanechips

- R1-2506785	discussion	On NR-NTN GNSS resilience	Ericsson

- R1-2506846	discussion	FL Summary #1: Study on GNSS resilient NR-NTN operation	THALES

- R1-2506847	discussion	FL Summary #2: Study on GNSS resilient NR-NTN operation	THALES

- R1-2506848	discussion	FL Summary #3: Study on GNSS resilient NR-NTN operation	THALES

- R1-2506849	discussion	FL Summary #4: Study on GNSS resilient NR-NTN operation	THALES

- R1-2506987	discussion	Discussion on NR-NTN GNSS resilience	Xiaomi

- R1-2507012	discussion	Discussion on NR-NTN GNSS resilience	CMCC

- R1-2506941	discussion	GNSS resilience for NR-NTN	Huawei, HiSilicon

- R1-2506812	discussion	Discussion on NR NTN GNSS resilience	Spreadtrum, UNISOC

- R1-2506896	discussion	Discussion on NR-NTN GNSS resilience	vivo

- R1-2506749	discussion	Discussion on Rel-20 GNSS resilient NR NTN operation	FUTUREWEI

- R1-2507174	discussion	Discussion on NR-NTN GNSS resilience	OPPO

- R1-2507211	discussion	Discussion on NR-NTN GNSS resilience	HONOR

- R1-2507117	discussion	Discussion on NR-NTN GNSS resilience	CATT

- R1-2507129	discussion	Discussion on the GNSS resilient NR-NTN operation	TCL

- R1-2507130	discussion	Discussion on NR-NTN GNSS resilience	SageRAN

- R1-2507321	discussion	Discussion on NR-NTN GNSS resilience	NEC

- R1-2507333	discussion	Discussion on NR-NTN GNSS resilience	China Telecom

- R1-2507251	discussion	Discussion on NR-NTN GNSS resilience	Samsung

- R1-2507217	discussion	NR-NTN GNSS resilience	InterDigital, Inc.

- R1-2507202	discussion	GNSS Resilient NR-NTN Operation -NR-NTN Phase 4	Tejas Network Limited

- R1-2507642	discussion	Discussion on GNSS resilient NR-NTN operation	Toyota ITC

- R1-2507643	discussion	Preamble transmission and detection in NR NTN GNSS resilient operation	Sharp

- R1-2507624	discussion	GNSS resilient operations in NR NTN	MediaTek Inc.

- R1-2507586	discussion	Discussion on NR-NTN GNSS resilience	CCU

- R1-2507761	discussion	Discussion on Rel-20 GNSS resilient NR NTN operation	ESA, Airbus, Thales

- R1-2507845	discussion	Discussion on NR-NTN GNSS resilience	CAICT

- R1-2507811	discussion	Discussion on NR-NTN GNSS resilience	NTT DOCOMO, INC.

- R1-2507901	discussion	Discussion on GNSS Resilient Operation for NR NTN	CEWiT

- R1-2507857	discussion	Discussion on NR-NTN GNSS resilience	Google Korea LLC

- R1-2507850	discussion	Discussion on GNSS resilience for NR-NTN	CSCN

- R1-2507937	discussion	Considerations for GNSS-resilient NR-NTN operation	ST Engineering iDirect

- R1-2507594	discussion	Discussion on GNSS resilient NR-NTN operation	Sony

- R1-2507560	discussion	Discussion on GNSS resilient operation for NR over NTN	Nokia

- R1-2507557	discussion	Discussion on NR-NTN GNSS resilience	Lenovo

- R1-2507550	discussion	Discussion on GNSS resilient NR-NTN	Panasonic

- R1-2507541	discussion	Discussion on NR-NTN GNSS resilience	Fraunhofer IIS, Fraunhofer HHI

- R1-2507675	discussion	Considerations of NR-NTN GNSS Resilient Operations	Apple

- R1-2507719	discussion	NR-NTN GNSS resilience	Qualcomm Incorporated

- R1-2507504	discussion	Discussion on NR-NTN GNSS resilient operations	ETRI

- R1-2507379	discussion	Discussion on GNSS resilient NR-NTN operation	LG Electronics

- R1-2507429	discussion	Ku band parameters and inter-SAN handover with GNSS unavailable	Eutelsat Group

- R1-2507465	discussion	Discussion on NR-NTN GNSS resilience	Ofinno

RAN1#123 meeting, Dallas, USA, November 17-21st, 2025:

- R1-2508479	draft TR	TR 38.742 skeleton for study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation	Rapporteur (Thales)

- R1-2508470	Work Plan	Work plan for NR NTN Phase 4	Rapporteur (Thales)

- R1-2509446	other	Session Notes of AI 10.6	Ad-Hoc Chair (Ericsson)

- R1-2509434	discussion	Considerations on timing error budget under GNSS resilience in NR-NTN	ISSDU

- R1-2509016	discussion	NR-NTN GNSS resilience	InterDigital, Inc.

- R1-2509017	discussion	Discussion on NR-NTN GNSS resilience	IMU

- R1-2509018	discussion	Discussion on GNSS resilient NR-NTN operation	Lenovo

- R1-2509476	discussion	Compilation of company proposals on NR-NTN GNSS resilience	Moderator (Thales)

- R1-2509007	discussion	Discussion on GNSS resilient operation for NR over NTN	Nokia

- R1-2509347	discussion	Discussion on GNSS Resilient Operation for NR NTN	CEWiT

- R1-2509336	discussion	Discussion on GNSS resilience for NR-NTN	CSCN

- R1-2509185	discussion	Discussion on NR-NTN GNSS resilience	China Telecom

- R1-2509304	discussion	Discussion on GNSS resilient NR-NTN operation	LG Electronics

- R1-2509298	discussion	Discussion on NR-NTN GNSS resilience	Google Korea LLC

- R1-2509301	discussion	Discussion of NR-NTN GNSS Resilience	Johns Hopkins University APL

- R1-2509296	discussion	Discussion on GNSS resilient NR-NTN	Panasonic

- R1-2509107	discussion	Discussion on NR-NTN GNSS resilience	Apple

- R1-2509071	discussion	On NR-NTN GNSS resilience approaches	Sony

- R1-2508984	discussion	Discussion on the GNSS resilient NR-NTN operation	TCL

- R1-2508970	discussion	Discussion on NR-NTN GNSS resilient operations	ETRI

- R1-2508992	discussion	Discussion on NR-NTN GNSS resilience	HONOR

- R1-2509131	discussion	Discussion on NR-NTN GNSS resilience	Ofinno

- R1-2509137	discussion	Discussion on GNSS resilient NR-NTN operation	TOYOTA Info Technology Center

- R1-2509178	discussion	Discussion on NR-NTN GNSS resilient operation using synchronization signals	ViaSat Satellite Holdings Ltd, Inmarsat

- R1-2509163	discussion	GNSS resilient operations in NR NTN	MediaTek Inc.

- R1-2509277	discussion	Discussion on NR-NTN GNSS resilience	NTT DOCOMO, INC.

- R1-2509228	discussion	NR-NTN GNSS resilience	Qualcomm Incorporated

- R1-2508465	discussion	Considerations on NR-NTN Resilience to GNSS Unavailability and Degradation	THALES

- R1-2508466	discussion	FL Summary #1: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2508467	discussion	FL Summary #2: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2508468	discussion	FL Summary #3: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2508469	discussion	FL Summary #4: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2508481	discussion	Considerations for GNSS-resilient NR-NTN operation	ST Engineering iDirect

- R1-2508489	discussion	GNSS resilience for NR-NTN	Huawei, HiSilicon

- R1-2508594	discussion	Discussion on NR-NTN GNSS resilience	CATT

- R1-2508452	discussion	Discussion on NR-NTN GNSS resilience	CMCC

- R1-2508360	discussion	PRACH with GNSS unavailable	Eutelsat Group

- R1-2508429	discussion	Discussion on NR-NTN GNSS resilience	vivo

- R1-2508385	discussion	Discussion on NR NTN GNSS resilience	Spreadtrum, UNISOC

- R1-2508333	discussion	Discussion on Rel-20 GNSS resilient NR NTN operation	FUTUREWEI

- R1-2508889	discussion	PRACH transmission and detection in NR NTN GNSS resilient operation	Sharp

- R1-2508843	discussion	On NR-NTN GNSS resilience	Ericsson

- R1-2508854	discussion	Discussion on NR-NTN GNSS resilience	ZTE Corporation, Sanechips

- R1-2508872	discussion	Discussion on NR-NTN GNSS resilience	Airbus

- R1-2508875	discussion	Discussion on NR NTN GNSS Resilience	Amazon Web Services

- R1-2508799	discussion	Discussion on NR-NTN GNSS resilience	Samsung

- R1-2508760	discussion	Discussion on GNSS Resilient NR-NTN Operation	Tejas Network Limited

- R1-2508754	discussion	Discussion on NR-NTN GNSS resilience	Fraunhofer IIS, Fraunhofer HHI

- R1-2508724	discussion	Discussion on NR-NTN GNSS resilience	OPPO

- R1-2508626	discussion	Considerations for GNSS-resilient NR-NTN operation	ESA

- R1-2508555	discussion	Discussion on NR-NTN GNSS resilience	NEC

- R1-2508681	discussion	Discussion on NR-NTN GNSS resilience	Xiaomi

- R1-2509498	discussion	NR-NTN GNSS resilience	Qualcomm Incorporated

- R1-2509609	discussion	FL Summary #5: Study on GNSS resilient NR-NTN operation	Moderator (Thales)

- R1-2509610	discussion	FL Summary #6: Study on GNSS resilient NR-NTN operation	Moderator (Thales)

- R1-2509611	draft TR	TR 38.742 skeleton for study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation	Rapporteur (Thales)

- R1-2509456	discussion	Discussion on NR-NTN GNSS resilience	CATT

## 4.2	RAN4

-

END