3GPP TSG RAN meeting #111									           RP-260064

Fukuoka, Japan, March 9th – 13th, 2026							rev of RP-253138

## Status Report to TSG

Title:			Status report for SID: Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation; rapporteur: Thales

Agenda item:			9.2.2

| WI / SI Name | Rel-20 Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_NTN_GNSS_resilient |  |  |  |  |
| Unique ID | 1080071 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251933 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2026 | Core part: N/A | Performance part: N/A | Testing part: |  |
| Overall Completion level | Study Item: Overall: 70% RAN1: 70% RAN4: 30% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Mohamed El Jaafari |
|  | Company | Thales |
|  | Email | Mohamed.el-jaafari@thalesaleniaspace.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Note that the updated work plan can be found in R1-2600253Work plan for NR NTN Phase 4  THALES

#### 2.1.1.1 Decisions during RAN1#124

Conclusion:

- Companies’ inputs for PRACH performance evaluation results are collected in Excel sheet “PRACH performance evaluation v022” that is embedded in R1-2601483.

- The template for collection of Connected mode results in Excel sheet “Template-Results for Connected mode_v004,” that is embedded in R1-2601483, is endorsed.

Agreement:

Confirm the following working assumption.

Working assumption: For UL performance evaluation, UL differential Doppler/frequency offset is calculated based on scaled DL one-way differential Doppler/frequency offset with the following scaling factor: Scaling factor = 2*fc,ul / fc,dl fc,ul and fc,dl are the uplink and downlink carrier frequency respectively.

Agreement:

Capture the following formula in the TR 38.742:

dopplerone-wayu=vsatc2fcu-s,xdone-wayu+vUEc2fcu-s,kdone-wayu

Where vsat is the speed of satellite and vUE is the speed of the UE.

Agreement:

Confirm the working assumption on theoretical differential Doppler limits as follows:

Theoretical differential Doppler limits of each PRACH preamble with long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0,1,2 | unrestricted | 1250 | 0 | 0.1 | 2 | 2 | 2.0 | 0.21 | 0.85 |
| 0,1,2 | type A | 1250 | 1 | 0.1 | 2 | 2 | 2.0 | 0.53 | 2.1 |
| 0,1,2 | type A | 1250 | 2 | 0.1 | 2 | 2 | 2.0 | 0.84 | 3.35 |
| 0,1,2 | type B | 1250 | 3 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 0,1,2 | type B | 1250 | 4 | 0.1 | 2 | 2 | 2.0 | 1.46 | 5.85 |
| 3 | unrestricted | 5000 | 0 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 3 | type A | 5000 | 1 | 0.1 | 2 | 2 | 2.0 | 2.40 | 9.6 |
| 3 | type A | 5000 | 2 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
| 3 | type B | 5000 | 3 | 0.1 | 2 | 2 | 2.0 | 4.90 | 19.6 |
| 3 | type B | 5000 | 4 | 0.1 | 2 | 2 | 2.0 | 6.15 | 24.6 |

Theoretical differential Doppler limits of each PRACH preamble with for  short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
|  | 30 | 30000 | 0 | 0.1 | 2 | 2 | 2.0 | 7.40 | 29.6 |
|  | 60 | 60000 | 0 | 0.1 | 30 | 20 | 3.0 | 0.90 | 54 |
|  | 120 | 120000 | 0 | 0.1 | 30 | 20 | 3.0 | 1.90 | 114 |

Note: These limits are assumed to represent theoretical upper bounds.

Agreement:

Confirm the RAN1#123 working assumptions on differential roundtrip delay limits for PRACH preamble formats considered for the study.

Observation:

Based on the evaluation results of the study on GNSS resilient operation:

For scenario 1; where the UE cannot rely on its GNSS for timing and frequency compensation on the service link,

- There is no single PRACH preamble with long PRACH format that could tolerate the differential delay and Doppler simultaneously across all elevation angles in LEO-600 S-band, LEO-1200 S-band and GEO in S-band in case a, where the location uncertainty is the area served by beam.

- There is no single PRACH preamble with short PRACH format that could tolerate the differential delay and Doppler simultaneously across all elevation angles in LEO-600 S-band, LEO-1200 S-band and GEO in S-band and Ku/Ka band, in case a, where the location uncertainty is the area served by beam.

For scenario 2; where there is a previously acquired GNSS based position, the workable PRACH formats (a PRACH format is workable if the PRACH format can tolerate the differential delay and Doppler) across all angles are provided in the tables below for different PRACH formats, uncertainty area X (km) and operating bands:

- Long PRACH Formats | S band

| Orbit/X | Workable PRACH formats across all angles |
| --- | --- |
| LEO 600 km, X=1 km | All formats (0–3; Unrestricted/Type A/Type B; all γ) |
| LEO 600 km, X=5 km | Format 0-2 Type A/B all γ; Format 3 Unrestricted |
| LEO 600 km, X=10 km | Format 1 type B, format 2 type B |
| LEO 600 km, X=25 km | None |
| LEO 1200 km, X=1 km | All formats (0–3; Unrestricted/Type A/Type B; all γ) |
| LEO 1200 km, X=5 km | All formats, except Format 3 Type A/B |
| LEO 1200 km, X=10 km | Format 1 type B, format 2 type B |
| LEO 1200 km, X=25 km | None |
| GEO, X=1 km | All formats (0–3; Unrestricted/Type A/Type B; all γ) |
| GEO, X=5 km | All formats, except Format 3 Type A/B |
| GEO, X=10 km | Format 1 Unrestricted/Type A, format 2 Unrestricted/Type A |
| GEO, X=25 km | Format 1 Unrestricted |

- Short PRACH Formats | S band

| Orbit/X | Workable PRACH formats across all angles |
| --- | --- |
| LEO 600 km, X=1 km | A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30 |
| LEO 600 km, X=5 km | C2-15 only |
| LEO 600 km, X=10 km | None |
| LEO 600 km, X=25 km | None |
| LEO 1200 km, X=1 km | A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30 |
| LEO 1200 km, X=5 km | C2-15 only |
| LEO 1200 km, X=10 km | None |
| LEO 1200 km, X=25 km | None |
| GEO, X=1 km | A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30 |
| GEO, X=5 km | C2-15 only |
| GEO, X=10 km | None |
| GEO, X=25 km | None |

- Short PRACH Formats | Ku/Ka band

| Orbit/X | Workable PRACH formats across all angles |
| --- | --- |
| LEO 600 km, X=1 km | Ku: A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30/60 Ka: C2-60 |
| LEO 600 km, X=5 km | None |
| LEO 600 km, X=10 km | None |
| LEO 600 km, X=25 km | None |
| LEO 1200 km, X=1 km | Ku: A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30/60 Ka: C2-60 |
| LEO 1200 km, X=5 km | Ku: C2-15 Ka: None |
| LEO 1200 km, X=10 km | None |
| GEO, X=1 km | Ku: A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30/60 Ka: C2-60 |
| GEO, X=5 km | Ku: C2-15 Ka: None |
| GEO, X=10 km | None |
| GEO, X=25 km | None |

Note 1: The tolerance gaps for non-workable PRACH formats and tolerance margin for workable PRACH can be found in “PRACH performance evaluation v022,” embedded in R1-2601483.

Note 2: These results have been collected from different sources under the following assumptions:

- UE altitude of 0 km is assumed

- UE altitude uncertainty is not taken into account.

- For Set1/2 parameters, it is assumed that the area served by the cell or beam (corresponding to the nadir beam size as defined in 38.821) is fixed within the satellite coverage.

Note 3: Some sources have reported larger TO/FO tolerance gaps than the reported values in R1-2601483 in case of elongated non-nadir beam at edge of the satellite coverage (larger than the nadir beam size as defined in 38.821).

Agreement:

Confirm the working assumptions from RAN1#123 regarding differential one-way delay and Doppler. With the following note:

Note: the difference of one-way delay between cross/along orbit is due to averaging from multiple sources

Agreement:

For the evaluation of GNSS resilient NR-NTN operation at least for initial access, the companies are encouraged to further study the following candidate solutions

- Solution 2D: UE side time/frequency pre-compensation based on reference location or TA/Doppler compensation information provided by gNB.

- Solution 1D: Signalling enhancements for Msg2/Msg4 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command).

- Solution 2E: service link time/frequency UE side pre-compensation based on last acquired GNSS position

- Solution 1A: Multiple PRACH transmissions (e.g. with different roots or cyclic shifts or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

- Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

- Solution 2B: Multiple random access attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

- Solution 1G: Adaptation of PRACH configuration.

- Solution 2C: Solutions based on broadcasting DL timestamp(s).

To ensure a good consolidation of the results, companies are encouraged to continue evaluating each solution and their combinations in more detail, in particular with respect to the specification impact where relevant (including whether the solutions remain within the scope of the SID), performance, applicability to different scenarios, complexity, coexistence with legacy UEs, and signalling overhead.

Observation

For the study on GNSS resilient NR-NTN operation, the following candidate solutions for Connected mode are listed based on inputs/discussions in RAN1#124

- Open‑loop time and frequency pre‑compensation:

  - At least for scenario 1: Common TA on service link and/or reference location and ephemeris‑based open‑loop pre‑compensation.

  - For scenario 2: UE uses last GNSS location or network‑provided reference location and satellite ephemeris.

- TA control:

  - Reuse of legacy closed‑loop TA, possibly with extended range

  - Reuse of legacy absolute TAC indication; with possible enhancements such as negative TA

  - TA drift

- New explicit FAC for connected mode:

  - Instantaneous/relative FO value

  - Optional drift parameters.

- Overhead and robustness optimization:

  - Group/common adjustment command

  - Combined TAC and FAC

- New explicit PAC (position adjustment command) for connected mode:

  - PAC with adjustment of UE reference location.

Other solutions are not precluded.

As part of the evaluation and assessment of the solutions above, at least the following aspects should be assessed:

- specification impact when relevant (including whether the solutions remain within the scope of the SID)

- performance,

- applicability to different scenarios,

- complexity,

- the signalling overhead.

Observation

Based on the evaluation results of the study on GNSS resilient operation in Connected Mode, it is observed that:

Depending on their assumptions on

- UE pre-compensation of RTT/Doppler on the service link (w/ e.g Case 1 or Case 2 or w/o)

- Timing and frequency tolerance limits assumed for issuing a correction (Ttol and Ftol): different values are reported by companies (e.g. Ttol= 1/2CP , 1/2CP-Te, 19 TS etc..)

- UE speed (Most companies considered 3 km/h, few companies evaluated 120km/h or 1500km)

- Initial timing/frequency error

- The assumed candidate solutions

companies reported the following typical average number (Ncmd) of times timing error or frequency error exceed tolerance limit per second.

Note: The above observation is made under the assumption of ideal conditions for estimation of the required timing offset and Doppler offset adjustment (always on UL signals).

Detailed evaluation results for Connected Mode can be found in the Excel Spreadsheet attached to R1-2600252

Timing control - case a:

| Source | Typical Ncmd range (cmd/s) | Notes |
| --- | --- | --- |
| vivo | S: ~1.2–5.4 , Ka: ~2–9.6 , GEO: ~0.1–0.2 | 3 km/h, Case 1 |
| Qualcomm | S: ~0.25–2 , Ka: ~1–6 | Ttol= 1/2CP ¦ (3–1500 km/h) |
| ST Eng. iDirect | S, LEO600, µ0: 13.26 S, LEO600, µ1: 25.33 Ka, LEO600, µ3: 80.20 S, LEO1200, µ0: 18.96 Ka, LEO1200, µ3: 53.15 | Using existing TA command, considering UE speed of 3 km/h, and using Timing threshold of CP/2. Averaged over the full pass (elevation 30 to nadir to elevation 30). |
| Huawei / HiSilicon | S: ~0.6–2.8, Ka: ~2.8–6.7 | Ttol= 1/2CP-Te, case 1, 3km/h, LEO600/LEO1200 |
| ZTE | S: ~0.46–1.94, Ka: ~1.6–4.0 | Ttol= 1/2CP (3 km/h, Case 1) |
| Toyota ITC | S: ~2.2–5, Ka: ~3.7–9.3 | 120km/h, Time error tolerance limit is 1/2CP |
| Ericsson | S: 0.01–0.83 (with Case1) , ~23 (no pre-comp) | T_tol= 19 TS. 3 km/h. Shows impact of TAC design |
| Thales | S: ~1.01–3.9, Ka: ~3.2–7.95 | 3 km/h, Ttol= 1/2CP |
| CMCC | S: ~0.18-1.7, Ka: ~1-8.4 | Ttol= 1/2CP, 3 km/h, Case 1 |
| Futurewei | LEO-600 S: 11.97 – 21.99, Ka: 43.98 – 87.21 LEO-1200: S: 9.45 – 18.94, Ka: 37.88 – 75.13 | Ttol=CP2, 3 km/h, without UE pre-compensation RTT/Doppler on the service link |
| OPPO | S band: ~0.47-2 | 3km/h, case 1, Ttol= 1/2CP-Te, LEO600/LEO1200 |
| CSCN | LEO-600 S set1:10.9~21.9 LEO-600 Ka set1:87.8 | Ttol=CP4, 3 km/h, without UE pre-compensation RTT/Doppler on the service link |
| CATT | S band：case1：0.375-0.379；W/O：11.33-13.17 Ka band：case1：1.96-2.17；W/O：123.6-143.7 | Ttol= CP/2-Te |

Timing control - case b:

| Source | Ncmd (cmd/s) by X (approx ranges) | Notes |
| --- | --- | --- |
| vivo | S band :X=1: 0.1–0.1, X=5: 0.1–0.6, X=10: 0.3–1.2 , X=25: 0.6–3.0 Ka-band: X=1: 0.4, X=5: 1.9 , X=10: 3.8 , X=25: 9.6 | 3 km/h, Case 1 |
| Qualcomm | S-band : X=1: 0.004–0.12, X=5: 0.026–0.18, X=10: 0.05–0.30 , X=25: 0.07–0.38 Ka-band : X=1: 0.05–0.43 , X=5: 0.25–0.67 , X=10: 0.49–1.09 , X=25: 0.61–1.34 | UE speed 3 km/h, Ttol=half CP,ftol=±0.1 ppm, no additional timing/frequency error at UE is assumed, with time-freq precompensation |
| Huawei / HiSilicon | S-band : X=1: 0.01–0.06 , X=5: 0.06–0.28 , X=10: 0.12–0.56 , X=25: 0.31–1.38 Ka-band : X=1: 0.34–0.47 , X=5: 0.88–1.52 , X=10: 1.55–2.81 , X=25: 3.60–6.71 | Ttol= 1/2CP-Te, case 2, 3km/h, LEO600/LEO1200 |
| ZTE | S-band : X=1: 0.016–0.033 , X=5: 0.05–0.20 , X=10: 0.10–0.39 , X=25: 0.25–1.00 Ka-band : X=1: 0.08–0.15 , X=5: 0.41–0.80 , X=10: 0.82–1.59 , X=25: 2.05–4.0 | Ttol= 1/2CP (3 km/h, Case 1), LEO600/LEO1200 |
| Futurewei | LEO-600: S: 11.97 – 21.99, Ka: 43.98 – 87.21 LEO-1200: S: 9.45 – 18.94, Ka: 37.88 – 75.13 | Ttol=CP2, 3 km/h, without UE pre-compensation RTT/Doppler on the service link |
| CATT | S band： X=1：0.0097-0.0177；X=5：0.0484-0.0887；X=10：0.0972-0.1786；X=25：0.2457-0.455； Ka band： X=1：0.1055-0.1928；X=5：0.5309-0.9724；X=10：1.0708-1.9662；X=25：2.7465-5.0829； | Ttol= CP/2-Te |

Frequency control - case a

| Source | Typical Ncmd range (cmd/s) | Notes (e.g. Ftol, UE speed, precompensation…) |
| --- | --- | --- |
| vivo | LEO-600: S:0.4-0.7, LEO1200: 0.1-0.2 LEO-600: Ka:0.3-0.9, LEO1200: 0.0-0.1, GEO:0. | Precomp: with case 1, 3km/h, F_tol S:0.2kHz, F_tol Ka: 3kHz. Initial frequency error 0kHz |
| Qualcomm | LEO-600: S: 0.03-0.08, Ka: 0.01-0.03 | UE speed 3 km/h, Ttol=half CP,ftol=±0.1 ppm, no additional timing/frequency error at UE is assumed, initial time frequency error: unif(-CP/2,CP/2), unif (-0.1,0.1) ppm, with time-freq precompensation |
| ST Eng. iDirect | S, LEO600, µ0 and µ1: 1.73 Ka, LEO600, µ3: 1.73 S, LEO1200, µ0 and µ1: 0.98 Ka, LEO1200, µ3: 0.98 | Using FA command similar to existing TA command, considering UE speed of 3 km/h, and using Frequency threshold of 0.1ppm. Averaged over the full pass. |
| Huawei / HiSilicon | S-band: 0.06 – 0.24, Ka-band: 0.02-0.13 | Ftol= 0.1ppm, case 1, 3km/h, LEO600/LEO1200 |
| ZTE | S-band, Ftol=SCS/2: 0 S-band, Ftol=0.1ppm: 0.07 -0.28 Ka-band, Ftol=SCS/2: 0 Ka-band, Ftol=0.1ppm: 0.03 - 0.15 | 3 km/h, Case 1, LEO600/LEO1200 |
| Toyota ITC | S, LEO600: ~0.2 - 0.5, Ka: ~0.1 – 0.3 | 120km/h, Frequency error tolerance limit is 0.1ppm |
| Ericsson | S, LEO600: 0.03-0.08 (with Case1), 0.12-2.16 (no pre comp), <0.01 (PAC) | F_tol= 0.08 ppm. 3 km/h. Shows impact of FAC/PAC design |
| Futurewei | LEO-600: S: 0.065 – 0.129, Ka: 0.2418 – 0.4837 LEO-1200: S: 0.029 – 0.059, Ka: 0.011 – 0.220 | Ftol=SCS3, 3 km/h, without UE pre-compensation RTT/Doppler on the service link |
| OPPO | S band: ~0.05-0.22 | 3km/h, case 1, Ftol= 0.1ppm, LEO600/LEO1200 |
| CSCN | LEO-600 S: 1.1~2.1 LEO-600 Ka: 0.3 | Ftol=0.01SCS, 3 km/h, without UE pre-compensation RTT/Doppler on the service link |

Frequency control - case b

| Source | Typical Ncmd range (cmd/s) | Notes (e.g. Ftol, UE speed, precompensation…) |
| --- | --- | --- |
| vivo | LEO S band : X=1: 0 , X=5: 0–0.0 , X=10: 0.0-0.2 , X=25: 0.1-0.4 Ka band : X=1:0-0: X=5:0-0.X=10:0-0.2 , X=25: 0.1-0.5. GEO: S:X=1-25:0-0 | with case 2, 3km/h, Ftol: 0.2kHz¦ LEO Ka:3 kHz, Initial frequency error: 0kHz, |
| Qualcomm | LEO-600: S: 0.001-0.03 (UE speed 3 km/s), LEO-600 S: 0.15 (UE speed 1500 km/s) | UE speed 3, 1500 km/h, Ttol=half CP, ftol=±0.1 ppm, no additional timing/frequency error at UE is assumed, with time-freq precompensation |
| Huawei / HiSilicon | S-band : X=1: 0 , X=5: 0–0.02 , X=10: 0.01–0.05 , X=25: 0.03-0.13 Ka-band : X=1: 0 , X=5: 0–0.02 , X=10: 0.01-0.05 , X=25: 0.03-0.13 | Ftol= 0.1ppm, case 2, 3km/h, LEO600/LEO1200 |
| ZTE | S-band, Ftol=SCS/2: 0 S-band, Ftol=0.1ppm: X=1: 0 , X=5: 0–0.02 , X=10: 0.01–0.06 , X=25: 0.04–0.15 Ka-band, Ftol=SCS/2: 0 Ka-band, Ftol=0.1ppm: X=1: 0 , X=5: 0–0.02 , X=10: 0.01–0.06 , X=25: 0.04–0.15 | 3 km/h, Case 1, LEO600/LEO1200 |
| Futurewei | LEO-600 S: 0.065 – 0.129, Ka: 0.2418 – 0.4837 LEO-1200: S: 0.029 – 0.059, Ka: 0.011 – 0.220 | Ftol=SCS3, 3 km/h, without UE pre-compensation RTT/Doppler on the service link |

Agreement:

The draft TR38.742 v010 in R1-2601708 is endorsed in principle. The final TR in R1-2601709 is endorsed in principle for information.

#### 2.1.2	Remaining Open issues

Consolidate evaluation results.

Evaluate potential solutions for NR-NTN resilience to GNSS unavailability and degradation

Identify impact on physical layer procedures

Identify and assess backward compatible issues with legacy NTN capable UEs

## 2.2	RAN4

#### 2.2.1	Agreements

The followings have been agreed:

Issue 1-1-1: Discussion with respect to WID

This study will initially focus on 1/ (where GNSS information is temporarily unavailable). In alignment with RAN1 agreements, RAN4 shall consider the both scenarios 1 and 2 concluded in RAN1.

Issue 1-1-2: Discussion with respect to potential solutions

RAN4 to consider RAN1 assumptions/set of potential solutions for future work.

Issue 1-1-3: Satellite constellation type

RAN4 shall consider both GSO and NGSO.

Issue 1-1-4: Satellite payload type

Priority for transparent payload, regenerative payload not precluded.

Issue 1-1-5: Frequency range/Numerology

Both FR1-NTN and FR2-NTN scenario.

Issue 1-1-6: UE type

Normal UEs and VSAT UEs should be considered for the study, RedCap UEs not precluded.

Issue 1-1-8: PRACH type

Pending a RAN1 decision, RAN4 might consider also other types of PRACH formats (e.g. FR1-NTN like PRACH formats for FR2-NTN usage).

Issue 1-2-2:  RF demod requirements

RAN4 to consider re-evaluation of demodulation performance requirements for initial access (PRACH) when RAN1 decides the enhanced initial access methodology for Study_NR_NTN_GNSS_resilient.

Issue 1-3-1:  Differentiation between initial access and other channels

Issue 1-3-2:  Timing error requirement

RAN4 to study on the impact of temporary GNSS unavailability or degraded accuracy on timing pre-compensation. RAN4 to revise the transmit timing error in GNSS resilient scenarios.

Different PRACH formats can be considered. UL timing requirement shall be evaluated.

Issue 1-3-4:  Gradual Timing Adjustment, etc.

RAN4 to discuss whether the current gradual uplink timing adjustment requirements remain adequate for GNSS-resilient NR-NTN scenarios where timing errors accumulate due to uncompensated drift. Clarification may be needed on how Tq_NTN and Tp_NTN should be interpreted when uplink timing error evolution is dominated by continuous accumulation rather than slowly varying residuals.

Issue 1-3-5:  Potential scheduling issues

RAN4 to study on the impact of network scheduling issue caused by GNSS unavailable on RRM spec.

Issue 1-3-6:  Impact on UE mobility management

Issue 1-3-7:  Conditional HO

RAN4 to study on the impact of GNSS unavailability on UE mobility management: RRC_Connected state.

RAN4 to study the impacts of GNSS resilience to location-based CHO requirement. Evaluate the applicability of the location-based triggers when UEs are operating in GNSS resilient scenarios. Potential impact on CHO delay requirements may be considered.

Issue 1-3-10:  Measurement delay and accuracy

Measurement impact due to GNSS not being available:

- Unavailable GNSS information will cause longer measurement delay and UE will not be able to report the measurement results within the specified time period.

- Unavailable GNSS information will have an impact on the RRM measurement accuracy.

Issue 1-4-1:  Prioritize issues for the work plan of FS_NR_NTN_GNSS_resilient at RAN4#118

- RAN4 to prepare simulation results and proposals for new requirements considering RAN4 impairments, according to discussed priorities/identified issues.

- Decide simulations to be done and respective KPIs (e.g. probability of detection, false alarm detection, SNR target, etc. per a given timing error).

- Simulation under AWGN to calibrate.

Work plan for next meetings:

| Meeting | Prioritized activity |
| --- | --- |
| RAN4#118bis | Evaluate potential PRACH degradation in frequency/time; Evaluate potential relaxation of timing error requirements; Evaluate potential performance degradation for CHO; If time allows and depending on the progress, evaluate the degradation for other UL channels. |
| RAN4#119 | Evaluate potential PRACH degradation in frequency/time; Evaluate potential relaxation of timing error requirements; Further evaluate potential performance degradation for CHO; If time allows and depending on the progress, evaluate the degradation for other UL channels; Prepare a RAN4 contribution for a dedicated RAN4 section under TR 38.742 (RAN1-lead TR) with respect to RAN4 potential impacts on existent requirements. |

#### 2.2.2	Remaining Open issues

Assess potential impact on RRM specifications related to initial access and connected mode procedures for NR-NTN.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention. 
None

## 4.	References

## 4.1	RAN1

RAN1#124 meeting, Gothenburg, Sweden, February 9-13th , 2026:

- R1-2600253	Work Plan	Work plan for NR NTN Phase 4	Moderator (THALES)

- R1-2601708	discussion	Draft TR 38.742 v010: Study on GNSS resilient NR-NTN operation	Rapporteur (Thales)

- R1-2601709	draft TR	TR 38.742 v010: Study on GNSS resilient NR-NTN operation	Rapporteur (Thales)

- R1-2601710	discussion	Summary #1 Draft TR 38.742	Rapporteur (Thales)

- R1-2601321	draft TR	Draft TR 38.742 Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation	Rapporteur (Thales)

- R1-2601510	other	Session Notes of AI 9.6	Ad-Hoc Chair (Ericsson)

- R1-2601483	discussion	Summary on [Post-123- Rel-20 NR-NTN GNSS resilience]	Moderator (THALES)

- R1-2601487	discussion	Compilation of company proposals regarding NR-NTN GNSS resilience	Moderator (THALES)

- R1-2601547	discussion	RAN1 agreements regarding NR-NTN GNSS resilience up to RAN1#123	Rapporteur (Thales)

- R1-2601322	discussion	Discussion on NR-NTN GNSS resilience	CAICT

- R1-2601318	discussion	Discussion on NR-NTN GNSS resilience	Google Korea LLC

- R1-2601410	discussion	Discussion on GNSS resilience for NR-NTN	CSCN

- R1-2601391	discussion	Discussion on GNSS Resilient Operation for NR NTN	CEWiT

- R1-2601704	discussion	FL Summary #5: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2601705	discussion	FL Summary #6: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2601461	discussion	GNSS resilience for NR-NTN	Huawei, HiSilicon

- R1-2601529	discussion	Discussion on NR-NTN GNSS resilience	MediaTek Inc.

- R1-2601538	discussion	GNSS resilience for NR-NTN	Huawei, HiSilicon

- R1-2600332	discussion	Discussion on NR-NTN GNSS resilience	CATT

- R1-2600341	discussion	GNSS Resilient operation for NR-NTN - 5G Adv	Tejas Network Limited

- R1-2600382	discussion	Discussion on NR-NTN GNSS resilience	CMCC

- R1-2600106	discussion	Discussion on NR-NTN GNSS resilience	Spreadtrum, UNISOC

- R1-2600184	discussion	Discussion on NR-NTN GNSS resilience	OPPO

- R1-2600248	discussion	Considerations on GNSS resilient NR-NTN operation	THALES

- R1-2600249	discussion	FL Summary #1: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2600250	discussion	FL Summary #2: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2600251	discussion	FL Summary #3: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2600252	discussion	FL Summary #4: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

- R1-2600259	discussion	Discussion on NR-NTN GNSS resilience	ZTE Corporation, Sanechips

- R1-2600237	discussion	On NR-NTN GNSS resilience	Ericsson

- R1-2601172	discussion	Discussion on NR-NTN GNSS resilience	NTT DOCOMO, INC.

- R1-2601231	discussion	Discussion of Using DL Timestamps for Initial Access	Johns Hopkins University APL

- R1-2601265	discussion	NR NTN GNSS resilience	Qualcomm Incorporated

- R1-2600962	discussion	Discussion on GNSS resilient NR-NTN operation	LG Electronics

- R1-2600971	discussion	Discussion on GNSS resilient NR-NTN	Panasonic

- R1-2600996	discussion	Discussion on NR-NTN GNSS resilient operations	ETRI

- R1-2601041	discussion	Location uncertainty reduction and mitigation techniques	Eutelsat Group

- R1-2601058	discussion	Discussion on GNSS resilient NR-NTN operation	Lenovo

- R1-2601060	discussion	Discussion on GNSS resilient operation for NR over NTN	Nokia

- R1-2601077	discussion	Discussion on NR-NTN GNSS resilience	Airbus

- R1-2601126	discussion	On Solutions for NR-NTN GNSS Resilience	Sony

- R1-2601090	discussion	Discussion on NR-NTN GNSS resilience	Ofinno

- R1-2600524	discussion	Discussion on NR-NTN GNSS resilience	Fraunhofer IIS, Fraunhofer HHI

- R1-2600449	discussion	Discussion on the GNSS resilient NR-NTN operation	TCL

- R1-2600496	discussion	Discussions on NR-NTN GNSS resilience	vivo

- R1-2600421	discussion	Further discussion on NR NTN GNSS resilience	Xiaomi

- R1-2600677	discussion	Discussion on NR-NTN GNSS resilience	NEC

- R1-2600690	discussion	Discussion on NR-NTN GNSS resilience	China Telecom

- R1-2600619	discussion	Discussion on GNSS resilient NR-NTN operation	TOYOTA Info Technology Center

- R1-2600581	discussion	NR-NTN GNSS resilience	InterDigital, Inc.

- R1-2600577	discussion	IMU Views on NR-NTN GNSS Resilience	IMU

- R1-2600077	discussion	GNSS resilience for NR-NTN	Huawei, HiSilicon

- R1-2600065	discussion	Discussion on Rel-20 GNSS resilient NR NTN operation	FUTUREWEI

- R1-2600903	discussion	Discussion on NR-NTN GNSS resilience	MediaTek Inc.

- R1-2600943	discussion	Discussion on NR-NTN GNSS resilience	HONOR

- R1-2600858	discussion	NR NTN GNSS resilient operation	Sharp

- R1-2600820	discussion	Discussion on NR-NTN GNSS resilience	Apple

- R1-2600794	discussion	Considerations for GNSS-resilient NR-NTN operation	ESA, Thales

- R1-2600806	discussion	Discussion on NR NTN GNSS Resilience	Amazon Web Services

- R1-2600784	discussion	GNSS-resilient NR-NTN operation	ST Engineering iDirect

- R1-2600748	discussion	Discussion on NR-NTN GNSS resilience	Samsung

## 4.2	RAN4

RAN4#118 meeting, Gothenburg, Sweden, February 9-13th , 2026:

- R4-2602118	other	Topic summary for [118][335] Study_NR_NTN_GNSS_resilient	Moderator (Thales)

- R4-2602971	other	Way Forward for [118][335] Study_NR_NTN_GNSS_resilient	Thales, Nokia

- R4-2600186	discussion	Discussion on GNSS resilient NR-NTN operation	LG Electronics Inc.

- R4-2600227	discussion	Discussion on RRM requirements for GNSS resilient NR-NTN operation	CATT

- R4-2600452	discussion	Discussion on RRM impact of GNSS resilient NR NTN operation	Xiaomi

- R4-2600552	discussion	On NTN RRM with GNSS resilience	Apple

- R4-2600833	discussion	Discussion on GNSS resilient NTN operation impact on RAN4 RRM	CMCC

- R4-2601101	discussion	Discussion on RRM requirements for GNSS resilience	Ericsson

- R4-2601358	other	Discussion on core parts on GNSS resilient	ZTECorporation,Sanechips

- R4-2601482	discussion	General aspect on GNSS resilient NR-NTN operation	vivo

- R4-2601561	discussion	Discussion on RAN4 study for NTN GNSS resilience	Huawei, HiSilicon

- R4-2601659	discussion	(FS_NR_NTN_GNSS_resilient)Discussion on GNSS resilient NR-NTN operation	Samsung

- R4-2601927	other	General aspects for RAN4 in GNSS resilient scenarios for NTN	Nokia

- R4-2602110	discussion	On the GNSS resilient NR-NTN operation	THALES, EUTELSAT

- R4-2602121	discussion	Discussion on NR-NTN GNSS resilience	Samsung

- R4-2602989	other	Way Forward for [118][335] Study_NR_NTN_GNSS_resilient	Thales, Nokia

- R4-2602991	other	Way Forward for [118][335] Study_NR_NTN_GNSS_resilient	Thales, Nokia

- R4-2602992	other	Way Forward for [118][335] Study_NR_NTN_GNSS_resilient	Thales, Nokia

END