3GPP TSG-RAN WG2 #131-bis	R2-2507686

# Prague, Czech Republic, October 13 – 17, 2025

# 38331 ASN.1 Comments file

| File version | Added RILs | Commented RILs |
| --- | --- | --- |
| V021 | H155, H156, H157, H158 for MOB |  |
| V020 | S060 for SONMDT |  |
| V019 | S059 for SONMDT |  |
| V018 | H153, H154 for MOB | Z166, Z167 |
| V017 | OF001 for MIMO | S017 |
| V016 | C032 C033 for LPWUS |  |
| V015 | Z166 Z167 for MOB | E054 |
| v014 | E056 for NTN |  |
| v013 |  | E054 |
| v012 | B002 for RedirToNTN B003 for SBFD |  |
| v011 |  | Updated status on NTN RILs by WI rapporteur after RAN1#131bis. |
| v010 | V052 V053 V054 for XR V080 V081 for MIMO | LP-WUS WI CR rapporteur updated the status of LPWUS RILs after RAN1#131bis. |
| v007 |  |  |
| v006 | E052 AIML |  |
|  | AIML WI CR rapporteur updated the status of AIML RILs after RAN1#131bis. |  |
| v005 | RRC Rapp fixed some RIL Id typos, RIL table duplicates etc. |  |
| v004 |  |  |
| v003 |  |  |
| v002 |  |  |
| v001 | S058 |  |
| v000 | E051 SONMDT |  |
|  | # Merged Comments files: # AIML Comments file v031 # LPWUS Comments file v017 # MIMO Comments file v011 # Mobility Comments file v025 # NES Comments file v036 # NR NTN Comments file v023 # SBFD Comments file v011 # SLRelay Comments file v021 # SONMDT Comments file v014 # UECap Comments file v003 XR Comments file v12 |  |

Template:

# E000

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E000 |  |  |  |  |  |  |  | ToDo |

[Description]:

[Proposed Change]:

[Comments]:

- Copy the template RIL comments fields above (including Heading)

- Paste the RIL comments fields at its position while respecting the spec order, i.e. order of the RILs in the Review file (such that the RIL has correct neighbours).

  - We might later split this file and Review File into smaller parts, then good if a RIL gets located in “correct” part.

- Fill in the fields, see R19 ASN.1 Guideline.

- Fill in the RIL ID twice, both as Heading and in the Table

- Companies may comment whether they agree or disagree. Can use Word Tracked changes. Do not delete text added by other companies.

- Fill in the table above with Added and Commented RILs

# N031

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N031 | AIML | 1 | Circular definition of applicable AI/ML configuration | N/A | Jerediah Fevold |  | vnnn | ToDo |

[Description]: The definition of applicable AI/ML configuration is circular and does not provide insight into the purpose. We also have not defined the term “functionality”, so it should not be used here.

[Proposed Change]:

Applicable AI/ML configuration: AI/ML-enabled configuration which has been determined to be executable by the UE, as defined in TS 38.300 [2].

[Comments]:

[WI CR rapporteur-v022]: Deleting this definition altogether is an alternative which has been raised as a class 0 issue (in “NR Rel-19 ASN1 Editorials”), since 38.331 does not use this term. We think it should be discussed within the scope of this RIL.

# E050

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E050 | NR_SL_relay_multihop-Core | 1 | Wrong/inaccurate definition for Last U2N Relay UE |  | Ericsson - Min |  | V018 | PropAgree |

[Description]:

Last U2N Relay UE: A U2N Relay UE having both Uu connection to the network and PC5 connection to a child UE for serving a U2N Remote UE in case of L2 U2N Relay communication. The child UE is the U2N Remote UE in case of single-hop L2 U2N Relay communication.

Based on the above definition, a Rel-20 last U2N relay UE may serve both single hop L2 U2N relay communication and multi-hop L2 U2N relay communication. In this case, the last U2N relay UE operates dual roles.

With respect to the below IE

#### –	SL-RelayUE-ConfigMH

The IE SL-RelayUE-ConfigMH specifies the threshold configuration information for NR sidelink Last U2N Relay UE or Intermediate U2N Relay UE or First U2N Relay UE.

SL-RelayUE-ConfigMH information element

-- ASN1START

-- TAG-SL-RELAYUE-CONFIGMH-START

SL-RelayUE-ConfigMH-r19::=           SEQUENCE {

sd-RSRP-ThreshDiscConfigMH-r19       SL-RSRP-Range-r16,

sd-hystMaxRelayMH-r19                Hysteresis

}

-- TAG-SL-RELAYUE-CONFIGMH-STOP

-- ASN1STOP

The last U2N Relay UE may apply the same configuration SL-RelayUE-ConfigMH for both single hop U2N relay communication and multi-hop U2N relay communication. This would be wrong, since the IE was agreed only for multi-hop U2N relay in Rel-19.

[Proposed Change]:

Therefore, correction is needed. Suggested corrections

Option 1 – update definition for the term Last U2N relay UE by removing “single hop”

Last U2N Relay UE: A U2N Relay UE having both Uu connection to the network and PC5 connection to a child UE for serving a U2N Remote UE in case of L2 U2N Relay communication. The child UE is the U2N Remote UE in case of single-hop L2 U2N Relay communication.

Option 2 – update the description for the IE SL-RelayUE-ConfigMH

#### –	SL-RelayUE-ConfigMH

The IE SL-RelayUE-ConfigMH specifies the threshold configuration information for NR sidelink Last U2N Relay UE (in case of multi-hop L2 U2N Relay communication) or Intermediate U2N Relay UE or First U2N Relay UE.

[Comments]:

[Rapporteur]: Agree to add “in case of multi-hop L2 U2N Relay communication” as clarified in the Option 2. Have changed the status from “ToDo” to “PropAgree”.

# E031

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E031 | MIMO | 1 | CJT definition is missing |  | Ericsson (Lian) |  | V006 | PropAgree |

[Description]: Abbreviation for CJT is missing and it is used in field descriptions e.g.:

csi-ReportCJTC

Configures parameters used for CJT calibration.

[Proposed Change]:

Add abbreviation of CJT (coherent joint transmission) in abbreviation clause.

[Comments]:

# H455

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H455 | NR_SL_relay_multihop-Core | 1 | Indication in the measurement report whether the candidate Relay UE is on a single-hop or multi-hop target path, or to provide the hop count information to avoid inter gNB path switch failures. |  | Huawei (Jagdeep) |  | V12 | ToDo |

[Description]: In RAN 2 # 131 meeting there were discussions whether RAN2 should consider adding an indication in the measurement report to specify whether the candidate Relay UE is on a single-hop or multi-hop target path, or to provide the hop count information to avoid inter gNB path switch failures. During the meeting it was agreed that -

No enhancement is added now to allow indicating the hop count of a candidate target relay UE to the gNB.  It can be discussed in maintenance if something is broken with the case where the target relay UE is in idle/inactive.

If no indication of the hop count of a candidate target relay UE is provided to the source gNB there will be frequent failures and unnecerry signalling between the gNBs during the path switch procedures.

[Proposed Change]:

Add an indication in the measurement report to indicate to the source gNB whether the candidate Relay UE is on a single-hop or multi-hop target path and help the source gNB to take a informed decision whether or not to initiate the inter gNB path switch procedure and avoid any unnecessary failures.

[Comments]:

[Rapporteur]: The need to add an indication in the measurement report to indicate to the source gNB whether the candidate Relay UE is on a single-hop or multi-hop target path can be discussed further considering different scenarios. Companies are are invited to discuss this issue in the contribution. The Status of this RIL is set to “ToDo”.

# S054

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S054 | AIML | 1 | Interaction with NR-U |  | Aby K Abraham |  | v026 | ToDo |

[Description]:

RAN2 to discuss if AI/ML for Phy is applicable for NR-U. If it is not explicitly disallowed, Channel Access Priority Class (CAPC) needs to be configured for SRBx

[Proposed Change]:

Discuss if AI/ML for Phy is applicable for NR-U. If it is applicable, below changes are needed.

### 4.2.2	Signalling radio bearers

<change 1>

For operation with shared spectrum channel access in FR1, SRB0, SRB1 and SRB3 are assigned with the highest priority Channel Access Priority Class (CAPC), (i.e. CAPC = 1) while CAPC for SRB2,SRBx is configurable.

<change 2>

LogicalChannelConfig field descriptions

channelAccessPriority Indicates the Channel Access Priority Class (CAPC), as specified in TS 38.300 [2], to be used on uplink transmissions for operation with shared spectrum channel access in FR1. The network configures this field only for SRB2,SRBx and DRBs.

[Comments]:

# V200

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V200 | NTN | 1 | Having a valid version of SIB2 when the CONNECTED UE is configured with location information reporting for assisted SMTC configuration | Yes, R2-250xxxx | vivo (Stephen) |  | v005 | Rejected |

[Description]: When the CONNECTED UE is configured with location information reporting for assisted SMTC configuration, the network may not configure refLocList-r19 via dedicated RRC message (for overhead saving), the UE shall have a valid version of SIB2. Otherwise, the UE may not be able to report the nearest location in the RRC complete message.

[Proposed Change]: Clarify that the CONNECTED UE is configured with location information reporting for assisted SMTC configuration shall have a valid version of SIB2.

5.2.2.1	General UE requirements

….

The UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20, regardless of the RRC state the UE is in.

The UE configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state shall ensure having a valid version of SIB2.

The UE shall ensure having a valid version of the posSIB requested by upper layers.

[Comments]:

[Rapp] RRC specifies in section 5.2.2.2.1 that “The UE shall apply the SI acquisition procedure […] whenever the UE does not have a valid version of a stored SIB”. In addition, section 5.2.2.1 states that “The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5”. Therefore, a UE starting from RRC_IDLE/INACTIVE should already have a valid SIB2 version, while a UE already in RRC_CONNECTED applies the SI acquisition procedure if its version of SIB2 is determined invalid or receives a SI update notification.

# V500

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V500 | NES | 1 | OD-SIB1 related text for reception of SI change or PWS notification |  | vivo (Jianhui) |  | V015 | PropReject |

[Description]:

For the following procedural text, the RRC state description of ‘4>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or’ and ‘if the UE is in RRC_CONNECTED while T311 is running:’ should be ahead of reception of SI change or PWS notification, as a UE in general RRC_CONNECTED mode may also receive SI change or PWS notification and apply the same procedure ‘6>	acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13];’

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: [RIL]: V500, NES

3>	if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.2x:

4>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

4>	if the UE is in RRC_CONNECTED while T311 is running:

5>	if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification:

6>	acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13];

5>	else:

6>	perform the actions as specified in clause 5.2.2.3.3x;

3>	else:

4>	perform the actions as specified in clause 5.2.2.5.

[Proposed Change]:

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: [RIL]: V500, NES

3>	if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.2x:

4> if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification:

5> acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13];

4> else:

5>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

5>	if the UE is in RRC_CONNECTED while T311 is running:

6>	perform the actions as specified in clause 5.2.2.3.3x;

3>	else:

4>	perform the actions as specified in clause 5.2.2.5.

[Comments]:

[Apple] Agree withi change.

[Rapporteur] Considering that RAN2 has agreed the following:

•	NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

The proposed change is not needed.

# V201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V201 | NTN | 1 | PDCCH repetition impacts on SI acquisition | Yes, R2-250xxxxx | vivo (Stephen) |  | v005 | Rejected |

[Description]: With common PDCCH repetition, the UE not only monitors searchSpaceOtherSystemInformation, but also monitors searchSpace linked with searchSpaceOtherSystemInformation. Clarification is needed in sub-clause 5.2.2.3.2

[Proposed Change]: Clarify that PDCCH monitoring occasions for SI message are determined based on search space(s) indicated by searchSpaceOtherSystemInformation and its linked searchSpace (if any) in sub-clause 5.2.2.3.2.

5.2.2.3.2	Acquisition of an SI message

For SI message acquisition PDCCH monitoring occasion(s) are determined according to searchSpaceOtherSystemInformation and linked searchSpace (if any). If searchSpaceOtherSystemInformation or linked searchSpace is set to zero, PDCCH monitoring occasions for SI message reception in SI-window are same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchSpaceOtherSystemInformation or linked searchSpace (if any) is not set to zero, PDCCH monitoring occasions for SI message are determined based on search space(s) indicated by searchSpaceOtherSystemInformation and its linked searchSpace (if any). PDCCH monitoring occasions for SI message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the SI window. The [x×N+K]th PDCCH monitoring occasion (s) for SI message in SI-window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in SI-window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation.

[Comments]:

[Qualcomm] Shouldn’t we use “linked searchSpace (if any)” consistently everywhere?

# X203

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X203 | NES | 1 | Using NUL/SUL for OD-SIB1 request | R2-25xxxxx | Xiaomi (Haitao) |  | V003 | PropReject |

[Description]: Description on OD-SIB1 request for NUL and SUL repeat quite much and make spec messy.

[Proposed Change]: merge into a single procedure text for OD-SIB1 request on NUL and SUL.

[Comments]:Nokia: OK to merge but It would be good to see the proposal for this. IT does not seem to be trivial to do the merge

[vivo] sib1-RequestConfigSUL is optionally configured and thus it’s better not to merge them even it’s a little bit wordy. There are far more wordy procedural texts in RACH and Sidelink operation for instance...

[Ericsson] The implementation of this comes from Samsung and it is indeed for supporting SUL. If there is a need to simplify, please provide complete option that is error free.

[Xiaomi] the simplified procedure text is given as below.

[Samsung]: The current modelling is similar to OSI request and is very clear. There is no need/motivation to change.

##### 5.2.2.3.3x	Request for on-demand SIB1

The UE shall, while SDT procedure is not ongoing:

1>	if od-SIB1-Config for this cell in stored valid version of SIBxx includes sib1-RequestConfig and/or sib1-RequestConfigSUL:

2> trigger the lower layer to initiate the Random Access procedure on the selected uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfig (if normal uplink is selected) or in sib1-RequestConfigSUL (if supplementary uplink is selected) included in od-sib1-Config for this cell in stored valid version of SIBxx;

3>	if indication that maximum number of PRACH attempts as configured in sib1-RequestConfig (if normal uplink is selected) or in sib1-RequestConfigSUL (if supplementary uplink is selected) is reached is received from lower layers as defined in TS 38.321 [3]:

4>	perform the actions as specified in clause 5.2.2.5.

3>	if acknowledgement for SIB1 request is received from lower layers:

4>	acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

1>

[Apple] We understand it just mimics existing OD-SIB procedure text. If we modify it, do we still need to also change legacy OD-SIB procedure text? So, prefer to keep as it is.

[Rapporteur] Nothing seems to be broken with the current formulation. It is clear and correct whereas the proposed text is a bit convoluted and thus may lead to misinterpretation. The proposed change is not essential.

# C002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C002 | NTN | 1 | Missed geographical area coordinates in procedure of SIB7 reception | N | CATT (Da Wang) |  | v008 | Agreed |

[Description]:.

[Proposed Change]: Add geographical area coordinates as below.

1>	else if all segments of a warning message and geographical area coordinates (if any) have been received:

2>	assemble the warning message from the received warningMessageSegment(s);

2>	assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any);

2>	forward the received complete warning message, messageIdentifier, serialNumber, dataCodingScheme and geographical area coordinates (if any) to upper layers;

2>	stop reception of SIB7;

2>	discard the current values of messageIdentifier and serialNumber for SIB7;

[Comments]:

# Z451

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z451 | NR_SL_relay_multihop-Core | 1 | Single hop and multi-hop type differentiation |  | ZTE (Weiqiang Du) |  | V009 | PropAgree |

[Description]: For below description, differentiation of single hop and multi-hop message is needed.

4>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

4>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-MH-Relay is included in SIB12; or

[Proposed Change]: Adopt below change for all related conditions, will submit the RIL TP to show the necessary places if needed:

4>	if the UE is configured by upper layers to transmit NR sidelink L2 single hop U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

4>	if the UE is configured by upper layers to transmit NR sidelink L2 MH U2N relay discovery messages and sl-L2U2N-MH-Relay is included in SIB12; or

[Comments]:

[Rapporteur]: Agree to add the clarification with the change as below. Have changed the status from “ToDo” to “PropAgree”.

4>	if the UE is configured by upper layers to transmit NR sidelink L2 single hop U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

4>	if the UE is configured by upper layers to transmit NR sidelink L2 multi hop U2N relay discovery messages and sl-L2U2N-MH-Relay is included in SIB12; or

# C181

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C181 | NES | 1 | Cell selection case is missing in section 5.2.2.4.2x Actions upon reception of SIBxx |  | Rui (CATT) |  | V008 | PropReject |

[Description]: In section 5.2.2.4.2x	Actions upon reception of SIBxx, Cell selection case is missing.

[Proposed Change]:

Upon receiving SIBxx, the UE shall:

1>	store the SIBxx;

1>	SIB1 request configuration in the SIBxx is valid for acquiring OD-SIB1 of this cell in accordance with clause 5.2.2.3.1;

1>	SIB1 request configuration of another cell in this stored SIBxx is valid for acquiring OD-SIB during (re)selection to that cell, and after (re)selection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1:

[Comments]: Nokia: looks valid proposal

[vivo] It’s not necessary. Both cell selection and cell reselection case has been embodied in 5.2.2.3.1, as the UE relies on kssb value to determine how it acquires SIB1, and thus ‘1>	SIB1 request configuration in the SIBxx is valid for acquiring OD-SIB1 of this cell in accordance with clause 5.2.2.3.1;’ is sufficient.

[Ericsson] Either way is ok. Maybe not necessary but may not harm either.

[Apple] We agree with this change because cell selection may be triggered in OD-SIB1 (i.e. RRC re-establishment). It is different from legacy OD-SIB which can’t trigger cell selection.

[Rapporteur]: The proposed change does not seem to be essential.

[Samsung]: Support this change. The change seems essential. Without this change, SIBxx validity for the RRC re-establishment case is unclear.

# C003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C003 | NTN | 1 | clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned | Yes, R2-250xxxxx | CATT (Da Wang) |  | v008 | ToDo |

[Description]: We prefer to clarify how UE determines the service area if the Target Service Area in the USD or the ISA in SIBXX are not totally aligned. Since the UE is not expect to receive MBS service outside the service area, it would be better for the UE to get a precise service area scope.

[Proposed Change]: Add the following NOTE to clarify the relationship between the Target Service Area in the USD and the ISA in SIBXX.

NOTE: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD

[Comments]:

[Rapp] Even thought it is unlikely that both ways of delivering the information may be configured simultaneously, RAN2 should further discuss which one is prioritized (if needed).

[Qualcomm] Which ever area UE wants to use should work pointing to (may be not exactly same) almost same region. Otherwise this should be error case.

# J055

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J055 | NR_SL_relay_multihop-Core | 1 | Wasteful behaviour of an intermediate relay UE |  | Tsuboi (Sharp) |  | v019 | PropReject |

[Description]: RAN2 agreed to support the monitoring of paging over the Uu interface by an intermediate UE in coverage on behalf of a child UE. However, this part does not include the behavior of the intermediate UE. Considering the agreement and this behavior, the intermediate UE can only monitor paging for the child UE but cannot notify that information. This results in a redundant operation for the intermediate UE.

1>	if the UE is acting as a L2 U2N Relay UE in case of single hop or L2 Last U2N Relay UE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord in the Paging message matches the UE identity in sl-PagingIdentityRemoteUE included in sl-PagingInfo-RemoteUE received in RemoteUEInformationSidelink message from a L2 U2N Remote UE or from a child L2 U2N Relay UE:

3>	inititate the Uu Message transfer in sidelink to that UE as specified in 5.8.9.9;

[Proposed Change]: There are two alternatives:

- Revert the sentence (yellow part) to “a L2 U2N Relay UE” to include the behaviour of an intermediate relay UE.

- Revert the agreement and restrict paging monitoring by the intermediate UE on behalf of the child UE.

[Comments]:

[Rapporteur]: The monitoring of the paging by the intermediate relay UE when is coverage is similar to a Remote UE being able to monitor paging when in coverage in R17. This is left to the Remote UE implementation in R17. Similarly monitoring paging when is coverage is left to Intermediate Relay UE implementation in R19. The primary path of receiving the paging is via the parent relay and on Uu is a secondary path which depends on the UE. Hence no change is needed in the specification. . Hence propose the status “PropReject” for this RIL.

# O500

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O500 | NR_SL_relay_multihop-Core | 1 | Last relay UE monitors PO based on sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List | R2-25xxxxx | OPPO (Bingxue Leng) |  | V004 | PropAgree |

[Description]: Last relay UE should monitor PO for the downstream remote UEs based on the paging information in sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List

[Proposed Change]:

1>	if the UE is acting as a L2 U2N Relay UE in case of single hop or L2 Last U2N Relay UE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord in the Paging message matches the UE identity in sl-PagingIdentityRemoteUE included in sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List received in RemoteUEInformationSidelink message from a L2 U2N Remote UE or from a child L2 U2N Relay UE:

3>	inititate the Uu Message transfer in sidelink to that UE as specified in 5.8.9.9;

[Comments]:

[Rapporteur]: Agree to add sl-PagingInfo-RemoteUE-List as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# H450

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H450 | NR_SL_relay_multihop-Core | 1 | The action of intermediate Relay UE when performing as a remote UE |  | Huawei (Jagdeep) |  | V12 | PropAgree |

[Description]: The intermediate relay UE will act as a remote UE when receiving RRC Setup message for itself. However in the running CR in 5.3.3.4 there is a description below that describes UE to perform the L2 U2N Remote UE or L2 Intermediate U2N Relay UE configuration procedure for the UE which will be confusing in relation to configuration for L2 Intermediate U2N Relay UE. Hence the  L2 Intermediate U2N Relay UE should be removed to avoid any confusion.

- perform the L2 U2N Remote UE or L2 Intermediate U2N Relay UE configuration procedure in accordance with the received sl-L2RemoteUE-Config

[Proposed Change]:

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

<omitted>

1>	perform the L2 U2N Remote UE configuration procedure in accordance with the received sl-L2RemoteUE-Config as specified in 5.3.5.16;

1>	perform the sidelink dedicated configuration procedure in accordance with the received sl-ConfigDedicatedNR as specified in 5.3.5.14;

[Comments]:

[Rapporteur]: Agree to remove “or L2 Intermediate U2N Relay UE”  description in 5.3.3.4 to avoid any confusion as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# E047

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E047 | SONMDT | 1 | Issues when failedPCellId is same with ltm-RecoveryCellId |  | Ali Parichehreh |  | V013 | Agreed |

[Description]: The cell chosen for LTM recovery maybe the same cell of the failedPcellId, e.g with a different beam for connection. In 5.3.3.4, when setting timeUntilReconnection, this same should be clarified.

[Proposed Change]:

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

2>	if reconnectCellId in VarRLF-Report is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	if the UE supports RLF-Report for conditional handover and if choCellId in VarRLF-Report is set; or

3> if the UE supports RLF-Report for MCG LTM cell switch and if ltm-RecoveryCellId in VarRLF-Report is set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the radio link failure or the first reconfiguration with sync failure experienced in the failedPCellId stored in VarRLF-Report;

3>	else:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or reconfiguration with sync failure;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

[Comments]:

# J056

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J056 | NR_SL_relay_multihop-Core | 1 | Unify |  | Tsuboi (Sharp) |  | v019 | PropReject |

[Description]: Clarify that the yellow parts are same meaning. Otherwise, the change should be NBC change.

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release with its child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) or to the child UE(s) in accordance with 5.8.9.10.

[Proposed Change]: Use the same wording as below:

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release with its child UE(s)) or sends NotificationMessageSidelink message to its child UE(s) in accordance with 5.8.9.10.

[Comments]:

[Rapporteur]: “the connected L2 U2N Remote UE(s)” is needed for the Single hop scenario and is the legacy R17 text in the specification.  Hence propose to set the status of this RIL to “PropReject”.

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release with its child UE(s)) or sends NotificationMessageSidelink message to the connected L2 U2N Remote UE(s) or to the child UE(s) in accordance with 5.8.9.10

# N086

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N086 | NTN | 1 | Missing smtc5list in measConfig association to CG check | No | Nokia (Jakob) |  | v022 | Agreed |

[Description]: to ensure that, in the measConfig associated with a CG, the network checks whether any smtc list is included in a measurement object with the same ssbFrequency, but the new smtc5list is missing

[Proposed Change]: Add smtc5list to the check for ssbFrequency

#### 5.5.2.1	General

The network applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig associated with a CG, it includes a measObject for the SpCell and for each NR SCell of the CG to be measured;

-	to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportCGI;

-	to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-DelayValueConfig;

-	to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-ExcessDelayConfig;

-	to ensure that, in the measConfig associated with a CG:

-	for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

-	an smtc1 included in any measurement object with the same ssbFrequency has the same value and that an smtc2 included in any measurement object with the same ssbFrequency has the same value and that an smtc3list included in any measurement object with the same ssbFrequency has the same value and that an smtc4list included in any measurement object with the same ssbFrequency has the same value and that an smtc5list included in any measurement object with the same ssbFrequency has the same value;

[Comments]:

# O501

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O501 | NR_SL_relay_multihop-Core | 1 | Unnecessary differtiation of First U2N Relay and Intermediate U2N Relay |  | OPPO (Bingxue Leng) |  | V004 | PropAgree |

[Description]: Based on the definition, First U2N Relay UE is also an Intermediate U2N Relay UE, so “the L2 First U2N Relay UE” can be removed to avoid misunderstanding.

[Proposed Change]:

The L2 U2N Remote UE or L2 Intermediate U2N Relay UE indicates to upper layers to trigger PC5 unicast link release with its connected parent L2 U2N Relay UE.

[Comments]:

[Rapporteur]: Agree to remove First U2N Relay UE as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# J050

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J050 | MOB | 1 | Clarify an RRCReconfiguration can be triggered by Conditional LTM |  | Takaki (Sharp) |  | V018 | Rejected |

[Description]: In the current specification, CLTM is excluded from the case of “reception of an RRCReconfiguration.” Considering that other conditional mobility cases are explicitly stated, it would be more natural to explicitly mention CLTM as well.

[Proposed Change]: add “(C)” before “LTM” as below:

5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an (C)LTM cell switch:

[Comments]:

[Rapporteur] The term "LTM cell switch" cover both LTM and CLTM as the procedure that the UE performs is the same. Therefore, I don't think that any new change is needed

[Huawei] For CLTM, the procedure that is called is called "LTM cell switch execution procedure", not "(C)LTM cell switch excecution procedure" so prefer not to add "(C").

# C150

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C150 | MOB | 1 | Upon inter-CU MCG LTM execution, SN key update is also needed for the case that SN terminated bearer configured with MCG RLC leg only. |  | Rui (CATT) |  | V005 | Agreed |

[Description]: Upon inter-CU MCG LTM execution, SN key update is also needed for the case that SN terminated bearer configured with MCG RLC leg only. However, in this case, the LTM-Candidate IE indicated by lower layers includes an mrdc-SecondaryCellGroupConfig set to release.Thus.the current spec is not correct.

[Proposed Change]:

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure which requires an update of the master security key, according to clause 5.3.5.18.6:

3>	if the LTM-Candidate IE indicated by lower layers does not include an mrdc-SecondaryCellGroupConfig set to release:

43>	perform security key update procedure as specified in 5.3.5.7;

[Comments]:

[OPPO]

Agree with the change. In current specification, when the target cell configuration includes mrdc-SecondaryCellGroupConfig setting to relase, both the cases that the SN terminated bearers are kept and released can be supported. It can be up to NW to ensure that the sk-counter is configured properly, i.e., if sk-counter is included in target cell configuration, it implies that there are existing SN terminated bearers in the target cell and the UE can directly use the sk-counter without checking whether mrdc-SecondaryCellGroupConfig is set to release.

[Huawei]

Agree with the change

# C051

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C051 | SONMDT | 1 | MCG LTM |  | Tangxun |  | V002 | Agreed |

[Description]: in R19 SONMDT, RAN2 only focuses on MCG LTM, and we notice that in the corresponding changes sometimes “MCG LTM” is used and sometimes “LTM” is used. We suggest to keep all relevant changes aligned, i.e., to use “MCG LTM” consistently.

[Proposed Change]: add “MCG” in front of “LTM”.

[Comments]:

Rapporture agrees with the RIL and implemented che change in the drafted CR according to the RIL.

# H002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H002 | AIML | 1 | Retaining logged measurements during LTM | R2-25xxxx | Dawid |  | vnnn | Rejected |

[Description]:

RAN2 made the following agreement:

- RAN2 confirm that the solution agreed in RAN2#130 is applicable to regular HO and CHO (i.e. 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration is provided).

LTM is not part of the agreement, but it seems there is nothing preventing the network from adding retainLoggedMeasurements also to the LTM candidate cells which also reuse RRCReconfiguraiton containers.

[Proposed Change]: No change is needed, but RAN2 is requested to confirm that reatinLoggedMeasurements can also be used for LTM candidate configurations.

[Comments]:

[WI CR rapporteur-v022]: In our understanding, the intention of the proposal was to not do enhancements specific to LTM, while also not explicitly exclude it. However, for alignment, we suggest that companies discuss this RIL in a Tdoc.

# Q510

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q510 | AIML | 1 | csi-LogMeasAvailable in the RRCReconfigurationComplete | N/A | Rajeev Kumar |  | V018 | Rejected |

[Description]:

The csi-LogMeasAvailable indication can be sent by the UE in any RRCReconfigurationComplete for MCG, not only for “RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG”

[Proposed Change]:

- set the content of the RRCReconfigurationComplete message as follows:

/*omitted

2>	if the RRCReconfiguration includes the masterCellGroup:

3>	if the UE has logged measurement entries available in VarCSI-LogMeasReport: [RIL]: Q510, AIML

4>	include csi-LogMeasAvailable in the RRCReconfigurationComplete message;

[Comments]:

[WI CR rapporteur-v022]: We think the following procedural text from clause 5.3.5.3 implemets correctly the relevant RAN2#129 agreement (“6. UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).”):

1>	set the content of the RRCReconfigurationComplete message as follows:

<...>

2>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG:

<...>

3>	if the RRCReconfiguration includes retainLoggedMeasurements:

4>	if the UE has logged measurement entries available in VarCSI-LogMeasReport:

5>	include csi-LogMeasAvailable in the RRCReconfigurationComplete message;

# C052

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C052 | SONMDT | 1 | Duplicate successHO-InfoAvailable indication |  | Tangxun |  | V002 | Agreed |

[Description]: according to current wording, it means UE will report successHO-InfoAvailable every time when it receives a RRCReconfiguration message via SRB1. In our understanding, this is not the original intention, since this resolution is only for rach-less scenario when the UE can’t report this indication in Msg3.

[Proposed Change]: add further restriction, e.g., if UE hasn’t reported successHO-InfoAvailable since successful handover information becomes available in VarSuccessHO-Report, to make sure successHO-InfoAvailable is reported only once.

[Comments]:

[Samsung] Agree.

I remember there was this condition in the previous endorsed CR which seems missing.

3> if the UE has not previously sent successHO-InfoAvailable for the current content of VarSuccessHO-Report since the UE entered the serving cell in RRC_CONNECTED state:

Rapporteur agrees and captures the change in the draft CR.

# H300

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H300 | SONMDT | 1 | SHR indicator for LTM |  | Jun Chen |  | V004 | Rejected |

[Description]: In section 5.3.5.3, we wonder about the need of the following addition.

3> if the UE supports successful handover report for MCG LTM cell switch and if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

3>	if the UE supports successful handover report for MCG LTM cell switch and if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

4>	include successHO-InfoAvailable in the RRCReconfigurationComplete message;

Currently there is the following text, and we think it has covered any of handover types, and then it seems no need to add the above text.

2> if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

2> if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

3> include successHO-InfoAvailable in the RRCSetupComplete message;

[Proposed Change]: Suggest to remove the UE behaviour of including successHO-InfoAvailable in the RRCReconfigurationComplete message for LTM handover.

[Comments]:

[Samsung] Our understanding is this is based on agreement to handle case where availability cannot determined at the time of reconfigurationcomplet for ReconfigurationWithSync.

[Rapporteur]: Agree with Samsung analysis, during the review we have convered to the point that the check is needed outside the following bullet 2, to enable availability indication for the SHR which are generated based on RACH less LTM procedure and detemination of SHR is not possible within the same RRC complete message, e.g., next RRC message to be used for the SHR availability indication.

2>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG:

Therefore RIl is rejected.

# E015

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E015 | SONMDT | 1 | Misalignment on the naming of the newly introduced UE capabilities with TS 38.306 | R2-25xxxx | Ali Parichehreh |  | V004 | Agreed |

[Description]: UE capability naming alignment for the newly introduced UE capabilities.

In chapter 5.3.5.3, the capability of MRO for SHR in LTM is named as “successful handover report for MCG LTM cell switch”, whereas in TS38.306, it is “SON enhancements for MCG LTM”.

In chapter 5.3.10.5, the capability for MRO for time- and location-based CHO is named as “RLF-Report for conditional handover with time-based or location-based trigger condition”, whereas in TS38.306, it is “RLF Report for time-/location-based CHO”.

[Proposed Change]: Suggest using the same name of the UE capabilities in TS 38.331 and TS 38.306

[Comments]:

Rapporteur thinks the issue requires further discussion on UE capability bits before harmonizing the text.

[Rapporteur]: agreed in the meeting RAN2#131bis, but the impact is captured in TS 38.306

# O600

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O600 | MOB | 1 | Unncessary condition for including the selectedSK-Counter in RRCReconfigurationComplete |  | OPPO (Xue Lin) |  | V013 | Rejected |

[Description]: In current spec, when the UE determines whether to include the selectedSK-Counter in RRCReconfigurationComplete, it needs to check how the target cell configruation is received as yellow-hilighted text. While if a sk-counter has been selected due to LTM execution as in its sub-condition, it already represents that it is an inter-CU SCG LTM procedure and the target configuration can be only provided via SRB1 but not within the nr-SCG within mrdc-SecondaryCellGroup, i.e., ltm-ConfigNRDC. Therefore, the yellow-hilighted condition is unnecessary for checking whether to include SelectedSK-Counter.

[Proposed Change]:

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

3>	if a new sk-Counter value has been selected due to the LTM cell switch execution procedure as specified in 5.3.5.18.6:

5>	include selectedSK-Counter and set its value to the selected sk-Counter value;

[Comments]:

[Rapporteur] Current procedural text seems correct and anyway not broken. Therefore, I prefer to keep it as it is.

# S024

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S024 | NTN | 1 | UE optionally reports N closest reference locations via the RRCReconfigurationComplete message. | Yes, R2-250xxxxx | Samsung (Shiyang) |  | v011 | Agreed |

[Description]: RAN2 agreement is that

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

“UE can ..” in our understanding means UE optionally reports in RRCReconfigurationComplete message. If we implement in the current way below, it enforces UE always reports in RRCReconfigurationComplete message, which obviously is not aligned with the agreement!

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport;.

[Proposed Change]: Add “if available” at the end.

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport, if available;

[Comments]:

[vivo]: The proposed change is fine to us

[Rapp] In our understanding of the agreement, the “can” indicates an alternative way of reporting. Let’s clarify this in the next meeting. We are unsure what is the concern and why “if available” would be applicable. A UE needs to know its accurate location to initiate and maintain its connection to the network.

# H252

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H252 | NTN | 1 | Closest RL reporting in RRCConfigurationComplete | R2-25xxxxx | Huawei (Lili) |  | V014 | Agreed |

[Description]: According to the current CR, closest reference location can be reported in both RRCConfigurationComplete and UAI. The reported information is the same (i.e. referenceLocationReport), but the number of reference locations to be reported is only configured in OtherConfig, not in RRCConfiguration (while outside of OtherConfig).

In RRCConfigurationComplete:

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport;

In UAI:

1>	if transmission of the UEAssistanceInformation message is initiated to provide location information for assisted SMTC configuration in RRC_CONNECTED state according to 5.7.4.2;

2>	include the referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport;

[Proposed Change]:

Option1: Clarify that the referenceLocationReport in RRCConfigurationComplete only reports the single closest reference location, it cannot report multiple reference locations.

Option2: Add closestLocsToReport to RRCConfiguration (outside of OtherConfig)

Option3: Remove referenceLocationReport from RRCConfigurationComplete (in our understanding, this reporting is quite duplicated with reporting in UAI, it is less useful than having it in RRCResume)

[Comments]:

[Nokia] We agree to discussion either option, but would like to consider also that the UE may not support more than 2 parallel SMTCs, as noted in N085

[Rapp] To configure location information for assisted SMTC configuration, the RRCReconfiguration message contains OtherConfig, that is why in the procedure of the report it is captured “the UE is configured in this RRCReconfiguration message”. It is unclear why it would matter where in the hierarchy of the RRC message the closestLocsToReport is included, the message is fully processed by the UE before sending the response.

[Qualcomm] Configuration of SMTCs based on UE’s capability is what matters. We are also not sure what is the issue here. UAI is triggered only when UE determines new reference locations need to be updated to network, otherwise UAI is never needed.

# A104

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A104 | AIML | 1 | Simplification of procedure text on applicability reporting in Section 5.3.5.3/5.3.13.4/5.7.4.3 |  | Peng Cheng (Apple) |  | V024 | ToDo |

[Description]: In existing applicability reporting procedure text (see below example text from of Section 5.3.5.3):

- including csi-InferencePrediction is used to describle inference configuration of CSI prediction.

- including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19 is used to describle inference configuration of beam management.

The description of beam management is complex. However, according to current IE structure of predictionConfiguration-r19, configurationForChannelPredictio is now only used for inference configuration of beam management. So, the description of beam management inference can be simplified to “including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19 configurationForChannelPrediction”.

===========related copy from Section 5.3.5.3=======================================

2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or [RIL]: A104, AIML

2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation); or [RIL]: A104, AIML

2>	if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation):

3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): [RIL]: A104, AIML

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status[RIL]: V100, AIML of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

[Proposed Change]: We provide trackable changes as follows:

Section 5.3.5.3

2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including configurationForChannelPrediction; or [RIL]: A104, AIML

2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForChannelPrediction, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation); or [RIL]: A104, AIML

2>	if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation):

3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForChannelPrediction, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): [RIL]: A104, AIML

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status[RIL]: V100, AIML of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

4>	for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable[RIL]: Z001, AIML:

7>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

Section 5.3.13.4

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForChannelPrediction: [RIL]: A104, AIML

3>	for each such serving cell, include an entry in the applicabilityReportList and set the content as follows:

4>		set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForChannelPrediction: [RIL]: A104, AIML

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

Section 5.7.4.3

1>	if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations subject to applicability determination procedure according to 5.7.4.2:

2>	include applicabilityReportList in this UEAssistanceInformation message;

2>	for each serving cell:

3>	if the cell is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction , or including configurationForChannelPrediction, for which the applicability status has changed; or [RIL]: A104, AIML

3>	if the associated serving cell index was included in an entry in applicabilityConfigList within applicabilityReportConfig and the applicability status for at least one of the associated entries in applicabilitySetConfigList has changed: [RIL]: N034 AIML

4>	include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows:

5>	set the applicabilityCellId to the serving cell index of the cell;

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including configurationForChannelPrediction, for which the applicability status has changed[RIL]: O300 AIML: [RIL]: A104, AIML

6>	include an entry in the applicabilityReportConfigIdList[RIL]: C076, AIML and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityReportConfigId to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

[Comments]:

ZTE: Agree to have this modification which make the text procedure more readable and aligned the description method for both AIML CSI-Prediction and AIML based BM

[WI CR rapporteur-v031]: We agree that the procedural text could be simplified as suggested. However, since the ASN.1 structure is still under discussion in N021/H003, we suggest to come back to A104 after resolving N021/H003.

# S049

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S049 | AIML | 1 | Applicability reporting needs to consider RRCResumeComplete |  | Aby K Abraham |  | v026 | Agreed |

[Description]:

UE reports change in applicability in RRCResumeComplete. UE nees to consider the applicability while reporting the change in applicability in RRCReconfigurationComplete or UAI.

[Proposed Change]:

- While reporting UAI,

1>	if configured to report assistance information about the applicability of configurations subject to the applicability determination procedure:

2>	if the applicability status of configurations subject to the applicability determination procedure has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or in UEAssistanceInformation or RRCResumeComplete):

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report assistance information about the applicability of configurations subject to the applicability determination procedure;

- While reporting ReconfigurationComplete:

- 2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or

- 2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

- 2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or UEAssistanceInformation or RRCResumeComplete); or

- 2>	if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or UEAssistanceInformation or RRCResumeComplete):

- 3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

- 4>	set the applicabilityCellId to the serving cell index of the cell;

- 4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (RRCReconfigurationComplete or UEAssistanceInformation or RRCResumeComplete):

- 5>	include an entry in the applicabilityInfoReportList and set the content as follows:

- 6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

- 6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

- 6>	if the applicabilityStatus is set to inapplicable:

- 7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

- 4>	for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (RRCReconfigurationComplete or UEAssistanceInformation or RRCResumeComplete):

- 5>	include an entry in the applicabilityInfoReportList and set the content as follows:

- 6>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

- 6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

- 6>	if the applicabilityStatus is set to inapplicable:

- 7>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

[Comments]:

[WI CR rapporteur-v031]: We changed the status from “ToDo” to “PropAgree”.

# V100

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V100 | AIML | 1 | Missing applicability status | - | Boubacar |  | V009 | Rejected |

[Description]: The UE action of determining the applicability status is missing. Maybe need to add the UE action somewhere.

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

----------------------skip--------------------

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

The UE action of determining the applicability status is missing. Maybe need to add the UE action somewhere.

[Proposed Change]:

[Comments]:

[WI CR rapporteur-v022] We assume that the “UE action” means how the UE determines applicability. We do not think it’s needed to add in 38.331 how the UE determines the applicability, since that is anyway up to UE implementation. Thus, it’s sufficient if it’s captured in stage 2.

# Z001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z001 | AIML | 1 | Preference suggestion to the functionality that is reported as inapplicable in RRCReconfiguration | R2-25xxxx | Fei |  | V015 | Rejected |

[Description]:

Currently, for the applicability reporting via RRCReconfigurationComplete, it has been defined as below:

4>	for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

For the functionality that is provided in the OtherConfig whose applicability has been changed, there is no need for UE to provide the information of the releaseConfigurationPrefernece in the RRCReconfigurationComplete as there is no hurt to keep it still in the otherConfig.

[Proposed Change]:

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

/omit for short/

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured which has not been successfully transmitted since entering RRC_CONNECTED state:

3>	include measConfigReportAppLayerAvailable;

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or

2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation); or

2>	if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation):

3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

4>	for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

[Comments]:

[WI CR rapporteur-v022]: We think this issue should be discussed based on Tdocs, since there are changes in the UE behaviour for applicability reporting in RRCReconfigurationComplete versus UAI.

# Z151

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z151 | MOB | 1 | Wrong removal of report configuration associated with L3 based CLTM upon reconfiguration with sync |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: According to the current spec, upon reconfiguration with sync, the UE shall remove the report configuration if the reportConfigId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included. However, the reportConfig which has a reportType set to condTriggerConfig may also be associated with the execution condition for L3 based CLTM. In order to support subsequent CLTM, such report configuration should not be released upon reconfiguration with sync.

[Proposed Change]: To add a condition to exclude the report configuration and measurement object configuration associated with L3 based CLTM. And also fix the wrong indentation. For example:

3>	for each measId of the MCG measConfig, if configured, and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	if the reportConfigId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included; and

4>	if the reportConfigId is not associated with any measId indicated by the LTM-ExecutionCondition in an entry of LTM-ExecutionConditionList:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig; and

4>	if the measObjectId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included; and

4>	if the measObjectId is not associated with any measId indicated by the LTM-ExecutionCondition in an entry of LTM-ExecutionConditionList:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

[Comments]:

[Huawei] In the TP, it is unclear which LTM-ExecutionConditionList to use. In general, we believe current procedure text and ASN.1 are unclear on which the execution conditions in use are

# S050

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S050 | AIML | 1 | UAI retransmission after HO/CHO/LTM cell switch |  | Aby K Abraham |  | v026 | ToDo |

[Description]:

If the UE has send a UAI and within 1 second a handover has occurred or if the UE has send a UAI and a CHO or LTM cell switch has occurred, UE is required to resend the UAI. This is to avoid the transmission of UAI from source to target cell or to prepare CHO/LTM candidate cells, which can interrupt ongoing mobility For applicability reporting, based on the RRC specification, UE will send the new applicability after handover in RRCReconfigurationComplete. Thus, there is no need to retransmit the UAI.

[Proposed Change]: Add suggested check:

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4> if the UE assistance information was not initiated to report assistance information about the applicability of configurations subject to the applicability determination procedure:

5>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

5>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

4>	start or restart the leave without response timer (if exists) or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

[Comments]:

# Z152

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z152 | MOB, Sidelink | 1 | The coexistsnce of (C)LTM and sidelink |  | ZTE (Mengjie Zhang) |  | V009 | Rejected |

[Description]: Based on the current spec, it is unclear whether (C)LTM can be coexisted with NR sidelink.

[Proposed Change]: RAN2 to clarify whether the coexistence of (C)LTM and NR sidelink can be supported or not.

[Comments]:

[Rapporteur] No tdoc expected on this as we can simply discuss online. Please note that this is not a Rel-19 issue but rather a Rel-18. My preference would be to not work on this coexistance, meaning that whatever situation we have in Rel-18 it applies also to Rel-19.

# Z153

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z153 | MOB, QoE | 1 | The coexistsnce of (C)LTM and QoE |  | ZTE (Mengjie Zhang) |  | V009 | Rejected |

[Description]: Based on the current spec, it is unclear whether (C)LTM can be coexisted with QoE.

[Proposed Change]: RAN2 to clarify whether the coexistence of (C)LTM and QoE can be supported or not.

[Comments]:

[Rapporteur] No tdoc expected on this as we can simply discuss online. Please note that this is not a Rel-19 issue but rather a Rel-18. My preference would be to not work on this coexistance, meaning that whatever situation we have in Rel-18 it applies also to Rel-19.

# Z154

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z154 | MOB, MBS | 1 | The coexistsnce of (C)LTM and MBS |  | ZTE (Mengjie Zhang) |  | V009 | Rejected |

[Description]: Based on the current spec, it is unclear whether (C)LTM can be coexisted with MBS.

[Proposed Change]: RAN2 to clarify whether the coexistence of (C)LTM and MBS can be supported or not.

[Comments]:

[Rapporteur] No tdoc expected on this as we can simply discuss online. Please note that this is not a Rel-19 issue but rather a Rel-18. My preference would be to not work on this coexistance, meaning that whatever situation we have in Rel-18 it applies also to Rel-19.

# C151

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C151 | MOB | 1 | It is not clear on whether the UE should stop the LTM conditions evaluation based on L1 measurements and/or based on L3 measurements. |  | Rui (CATT) |  | V005 | Agreed |

[Description]:.

[Proposed Change]:

2>	else:

3> if the target SpCell is different from current SpCell:

4>	stop the LTM conditions evaluation, if any, for all the LTM candidate configurations;

4>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

5>	request lower layers to stop the LTM cell switch conditions evaluation for all LTM candidate configurations;

4>	if UE is performing LTM cell switch conditions evaluation based on L1 measurements:

5>	request lower layers to stop the LTM conditions evaluation based on L1 measurements for all the LTM candidate configurations;

4>	if UE is performing LTM cell switch conditions evaluation based on L3 measurements:

5>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations;

3>	start synchronising to the DL of the target SpCell;

[Comments]:

# O502

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O502 | NR_SL_relay_multihop-Core | 1 | Clarification on the relationship between L2 U2N Relay UE and L2 Last U2N Relay UE |  | OPPO (Bingxue Leng) |  | V004 | PropAgree |

[Description]: In the current RRC specification, for the procedures applicable to both single-hop U2N Relay UE and the Last Relay UE, sometimes it uses “L2 U2N Relay UE in case of single hop or the L2 Last U2N Relay UE”, sometimes it uses “L2 U2N Relay UE or the L2 Last U2N Relay UE”. The description should be aligned to avoid confusion

[Proposed Change]:

L2 U2N Relay UE in case of single hop or L2 Last U2N Relay UE

[Comments]:

[Rapporteur]: Agree to clarify on the relationship between L2 U2N Relay UE and L2 Last U2N Relay UE as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# V000

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V000 | LPWUS | 1 | lpwus-OffsetPreferenceConfig should be captured as per cell group configuration |  | Vivo(Chenli) |  | V005 | Agreed |

[Description]: As lpwus-OffsetPreferenceConfig is configured per CG and the offset preference is reported per CG, while the current specification didn’t reflect it in several places.

[Proposed Change]: The corresponding clarification in 5.3.5.9, 5.3.5.10, 5.3.7.2, 5.3.7.3, 5.3.13.2 should be added.

[Comments]:

# H250

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H250 | NTN | 1 | Descriptions of UAI | R2-25xxxxx | Huawei (Lili) |  | V006 | Agreed |

[Description]: The closest reference location information in the UAI can be used for both SMTC configuration and gap configuration. Besides, it is preferred that we use “reference location information reporting” because it is different from directly reporting UE location. Similar changes (referring to gap configuration) need to be made to multiple other places.

[Proposed Change]:

2>	if the assisted-SSB-MTC-Config is set to setup:

3>	consider itself to be configured to provide closest reference location information for assisting SMTC and measurement gap configuration in RRC_CONNECTED state in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide closest reference location information for assisting SMTC and measurement gap configuration in RRC_CONNECTED state.

[Comments]:

[Rapp] Regarding the first change, we consider it is incorrect since the UE does not provide a “reference location” but an index/bitmap (whatever RAN2 eventually decides). Measurement gap considerations need to be addressed if inter-frequency is agreed for SMTC enhancements.

# V202

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V202 | NTN | 1 | Reference to 5.3.5.3 | N | vivo (Stephen) |  | v005 | Agreed |

[Description]: The OtherConfig setting up location information reporting also impacts sub-clause 5.3.5.3. A reference to sub-clause 5.3.5.3 should be added in sub-clause 5.3.5.9.

[Proposed Change]:

1>	if the received otherConfig includes the assisted-SSB-MTC-Config:

2>	if the assisted-SSB-MTC-Config is set to setup:

3>	consider itself to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state in accordance with 5.3.5.3 and 5.7.4;

2>	else:

3>	consider itself not to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state.

[Comments]:

# N081

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N081 | NTN | 1 | Considerations on no UE requirements for GNSS | Yes, R2-25xxxxx | Nokia (Jakob) |  | v016 | Rejected |

[Description]: There are no requirements for the UE to obtain GNSS location at specific intervals, nor should there be a requirement for the UE to obtain a GNSS location simply for the location information reporting. However, currently, receiving otherConfig including assisted-SSB-MTC-Config implies that the UE is configured to do so.

[Proposed Change]: Add a note to section 5.3.5.9 related to other config to state that this does not imply a requirement to obtain location.

1>	if the received otherConfig includes the assisted-SSB-MTC-Config:

2>	if the assisted-SSB-MTC-Config is set to setup:

3>	consider itself to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state in accordance with 5.7.4;

NOTE 2a:	The UE is requested to attempt to have valid detailed location information available whenever sending location information for assisted SMTC. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding how to determine the location information is up to UE implementation.

2>	else:

3>	consider itself not to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state.

[Comments]:

[Rapp] There are (RAN4) GNSS requirements for the UE to initiate and maintain a connection to an NTN payload. Without accurate GNSS, it is likely that the UE cannot communicate with the network since its messages would arrive outside the cyclic prefix.

# C072

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C072 | AIML | 1 | “UE-side data collection” wording |  | Tangxun |  | V003 | Agreed |

[Description]: “UE data collection” should be changed to “UE-side data collection” for unified wording.

[Proposed Change]: update the procedural text as below:

2>	if dataCollectionPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on being configured with radio measurement resources for UE-side data collection in accordance with 5.7.4;

[Comments]:

[WI CR rapporteur-v020]: We changed the status from “ToDo” to “PropAgree”.

# B200

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B200 | AIML | 1 | Missing of crossing reference to 5.7.4 |  | Congchi Zhang |  | V011 | Rejected |

[Description]: The reference to 5.7.4 “not to be configured to report applicability info….. in accordance with 5.7.4;” is missing in some procedure description.

1>	if the received otherConfig includes applicabilityReportConfig:

…

2>	else:

3>	consider itself not to be configured to report applicability information of configurations subject to the applicability determination procedure in accordance with 5.7.4;

1>	if the received otherConfig includes dataCollectionPreferenceConfig:

…

2>	else:

3>	consider itself not to be configured to provide its preference on being configured with radio measurement resources for UE data collection [Missing];

…

2>	else:

3>	consider itself not to be configured to report assistance information related to logging of radio measurements for network-side data collection [Missing].

[Proposed Change]:

1>	if the received otherConfig includes dataCollectionPreferenceConfig:

…

2>	else:

3>	consider itself not to be configured to provide its preference on being configured with radio measurement resources for UE data collection in accordance with 5.7.4;

…

2>	else:

3>	consider itself not to be configured to report assistance information related to logging of radio measurements for network-side data collection in accordance with 5.7.4.

[Comments]:

[WI CR rapporteur-v022]: The proposed changes are not needed and not aligned with the procedural text for legacy configurations. The text “in accordance with 5.7.4” is typically used in legacy only for the branches that start with “consider itself to be configured” and not for the branches that start with “consider itself not to be configured”. The reason to make an exception for “3> consider itself not to be configured to report applicability information of configurations subject to the applicability determination procedure in accordance with 5.7.4;” is that the UE will still report applicability information in RRCReconfigurationComplete, but not in the UAI according to 5.7.4. For the other two cases the reporting can only be done in UAI anyway, so it’s not needed to mention the clause number.

# J051

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J051 | MOB | 1 | Wrong IE name |  | Takaki (Sharp) |  | V018 | Agreed |

[Description]: same issue with Z158

[Proposed Change]: Change the IE name to ltm-SK-CounterConfigToReleaseList as below:

1>	for each securityCellSetId value included in the sk-CounterConfigToReleaseList that is part of the current sk-CounterConfigToAddModList in VarConditionalReconfig:

[Comments]:

# J011

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J011 | NR_SL_relay_multihop-Core | 1 | Add intermedia relay UE behavior for SRB1 in Remote UE Addition procedure |  | Sharp (LIU Lei) |  | V006 | ToDo |

[Description]: The SRAP spec has described how to determine egress RLC channel for SRB1, but RRC spec only includes the description applied for single hop relay UE and last relay UE. The behaviour of intermedia relay UE for SRB1 in Remote UE Addition procedure should be added.

[Proposed Change]:

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay or sl-SRAP-ConfigRelay-ToAddModList if applicable;

2> for L2 U2N Relay UE in case of single hop or L2 Last U2N Relay UE:

3>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured:

4>	release SL-RLC1, if established;

4>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1;

3>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured)

4>	if SL-RLC1 is not established:

5>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

2> for L2 Intermediate U2N Relay UE:

3>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-Channel-DL is configured:

4>	release SL-RLC1, if established;

4>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-Channel-DL with SRB1;

3>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-Channel-DL is not configured)

4>	if SL-RLC1 is not established:

5>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

[Comments]:

[Rapporteur]: The field description of the sl-EgressRLC-Channel-DL mentions that it Indicates the egress RLC channel on PC5 Hop for downlink transmissions at the L2 Intermediate U2N Relay UE. We can discuss if such clarification is needed in the procedural text and companies . I will recommend "ToDo" status for this RIL.

# C152

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C152 | MOB | 1 | Ambiguity on whether the IE ltm-Config can be the one included in the RRCReconfiguration message contained in nr-SCG . |  | Rui (CATT) |  | V005 | Rejected |

[Description]: Ambiguity on whether the IE ltm-Config can be the one included in the RRCReconfiguration message contained in nr-SCG.

[Proposed Change]:

5.3.5.18.1	LTM configuration

……

An ltm-Config included within an RRCReconfiguration message not included in nr-SCG received via SRB1 is for LTM on the MCG. It may include an SCG configuration and/or ltm-ServingCellNoSecurityChangeID.

An ltm-Config included within an RRCReconfiguration message either received via SRB3, or embedded nr-SCG in in an RRCReconfiguration message received via SRB1 is for LTM on the SCG. It does not include any MCG configuration and does not include ltm-ServingCellNoSecurityChangeID.

[Comments]:

[MediaTek (Pasi)]

Agree for the first change. The second change seems unnecessary, because "... RRCReconfiguration message ... embedded in an RRCReconfiguration message received via SRB1" looks unambiguous. However, if the second change is applied it should be "in nr-SCG" instead of "nr-SCG in".

[Rapporteur] ltm-Config is not an IE but is a field (LTM-Config is the IE). The current text seems fine and it also uses a terminology that is widely used in RRC spec.

[Huawei]

Agree with MediaTek's comments.

# Z155

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z155 | MOB | 1 | The missing description for VarLTM-ServingCellNoSecurityChange |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: In the current spec, we have specified how to handle the ltm-Config/ltm-ConfigNRDC and several related UE variables in NR-DC, but the description for VarLTM-ServingCellNoSecurityChange is missing here. It may cause some confusion whether one or two VarLTM-ServingCellNoSecurityChange shall be maintained in NR-DC.

[Proposed Change]: To add the description for VarLTM-ServingCellNoSecurityChange, for example:

In this case, the following principles apply:

-	the UE maintains independently the two ltm-Config, or the ltm-Config and the ltm-ConfigNRDC;

-	the UE maintains two independent VarLTM-ServingCellNoResetID, one associated with each ltm-Config, or one associated with the ltm-Config and one associated with the ltm-ConfigNRDC;

-	the UE maintains two independent VarLTM-ServingCellUE-MeasuredTA-ID, one associated with each ltm-Config, or one associated with the ltm-Config and one associated with the ltm-ConfigNRDC;

-	the UE maintains only one VarLTM-ServingCellNoSecurityChange, associated with either the ltm-Config or the ltm-ConfigNRDC;

-	the UE independently performs all the procedures in clause 5.3.5.18 for each ltm-Config, or an ltm-Config and an ltm-ConfigNRDC, and the associated VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID, and VarLTM-ServingCellNoSecurityChange, unless explicitly stated otherwise.

[Comments]:

[Rapporteur] This was already discussed during the RRC implementation and that text was deleted because there is no scenarios where inter-CU MCG LTM and inter-CU SCG LTM are configured at the same time. Therefore, no scenarios where UE keeps two that that variables.

[Huawei]

It should be specified that, if ltm-Config and ltm-ConfigNRDC are both configured, ltm-NoSecurityChangeID can either be configured in ltm-Config or in ltm-ConfigNRDC but not in both, and then VarLTM-ServingCellNoSecurityChange is associated with the one from ltm-Config and ltm-ConfigNRDC in which ltm-NoSecurityChangeID is configured.

# Z156

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z156 | MOB | 1 | The handling order of the IEs in the received LTM-Config |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: Since we have specified some specific handling on some IEs received in the LTM-Config, the reconfiguration of all other fields in the received LTM-Config IE should be put at the end of the procedure.

[Proposed Change]: Move “1>	reconfigure the UE according to all other fields of the received LTM-Config IE” to the end of the procedure.

[Comments]:

# Z166

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z166 | MOB | 1 | Clarification on ltm-ServingCellExecutionCondition set to release |  | ZTE (Mengjie Zhang) |  | V015 | ToDo |

[Description]: In RRC CR agreed in the last meeting (R2-2507729), it was agreed to have a new procedure for the handling of the execution condition for LTM. However, according to the latest CR, it’s only specified the UE behaviour when the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup, but there is no handling on the field ltm-ServingCellExecutionCondition set to release. If the field ltm-ServingCellExecutionCondition is set to release, the UE should clear the entry in VarLTM-ExecutionConditionList and stop performing LTM cell switch conditions evaluation. Such operation can also be covered by the handling specified in the new clause 5.3.5.18.1a LTM cell switch execution conditions modification (as highlighted by yellow below). Thus, if the received LTM-Config includes the field ltm-ServingCellExecutionCondition, the UE can perform LTM cell switch execution conditions modification procedure, regardless of whether the field is set to setup or release.

#### 5.3.5.18 LTM configuration and execution ##### 5.3.5.18.1 LTM configuration … 1> if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup: 2> perform LTM cell switch execution conditions modification as specified in 5.3.5.18.1a; 1> reconfigure the UE according to all other fields of the received LTM-Config IE. ##### 5.3.5.18.1a LTM cell switch execution conditions modification The UE shall: 1> clear the entry in VarLTM-ExecutionConditionList; 1> if the UE is performing LTM cell switch conditions evaluation based on L1 measurements: 2> request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations; 1> if the UE is performing LTM cell switch conditions evaluation based on L3 measurements: 2> stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8; 1> if this procedure is triggered by LTM cell switch execution as specified in 5.3.5.18.6 and if ltm-ExecutionCondition is configured in the LTM-Candidate IE to which LTM cell switch is performed: 2> store ltm-ExecutionCondition in VarLTM-ExecutionConditionList; 1> else if this procedure is triggered by LTM configuration as specified in 5.3.5.18.1 and if ltm-ServingCellExecutionCondition is set to setup: 2> store ltm-ServingCellExecutionCondition in VarLTM-ExecutionConditionList; 1> for each LTM-ExecutionCondition in VarLTM-ExecutionConditions: 2> if l3-Conditions is included in the LTM-ExecutionCondition: 3> perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the LTM-ExecutionCondition; 2> else if l1-Conditions is included in the LTM-ExecutionCondition: 3> request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the LTM-ExecutionCondition.

[Proposed Change]: Remove “set to setup”, as below:

1>	if the received LTM-Config includes the field ltm-ServingCellExecutionCondition:

2>	perform LTM cell switch execution conditions modification as specified in 5.3.5.18.1a;

1>	reconfigure the UE according to all other fields of the received LTM-Config IE.

[Comments]:

[Huawei] Agree.

# X150

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X150 | MOB | 1 | Avoid stop CLTM evaluation for new configured CLTM conditions. | R2-25xxxxx | Xiaomi (Yi Xiong) |  | V006 | Agreed |

[Description]: In LTM configuration section 5.3.5.18.1, for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition, the UE stop CLTM evaluation for all the LTM candidate configurations and then UE perform CLTM evaluation for associated candidate cell, as shown below.

1>	if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup:

2>	for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition:

3>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

4>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

3>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

4>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.x;

3>	if l3-Conditions is included within ltm-ServingCellExecutionCondition:

4>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.x according to the received ltm-ServingCellExecutionCondition;

3>	else if l1-Conditions is included within ltm-ServingCellExecutionCondition:

4>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ServingCellExecutionCondition;

Based on the current spec, for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition, the UE shall perform the blue part once. The blue part performed during the next “if” LOOP will cause the UE stop the CLTM evaluation which is triggered in previous “if” LOOP(s). The final result is that the UE will only perform CLTM evaluation for the conditions included in the last LTM-ExecutionCondition of the LOOP. This is incorrect and the UE shall perform evaluation based on the all new configured CLTM conditions

Hence, we suggest to move the procedure of “2>	for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition:” after the UE stop CLTM execution for all CLTM candidate.

[Proposed Change]: Based on the above description, we suggest the following change:

1>	if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup:

2>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

3>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

2>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

3>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.x;

2>	for each LTM-ExecutionCondition included within ltm-ServingCellExecutionCondition:

3>	if l3-Conditions is included within ltm-ServingCellExecutionCondition:

4>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.x according to the received ltm-ServingCellExecutionCondition;

3>	else if l1-Conditions is included within ltm-ServingCellExecutionCondition:

4>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ServingCellExecutionCondition;

[Comments]:

# C153

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C153 | MOB | 1 | UE should stop the corresponding LTM conditions evaluation before release the ltm-ServingCellExecutionCondition. |  | Rui (CATT) |  | V005 | Agreed |

[Description]: When ltm-ServingCellExecutionCondition is set to release, UE should stop the corresponding LTM conditions evaluation before release the ltm-ServingCellExecutionCondition..

[Proposed Change]:

5.3.5.18.1	LTM configuration

……

- else (ltm-ServingCellExecutionCondition set to release):

2> Stop the corresponding LTM conditions evaluation,

2>	release the ltm-ServingCellExecutionCondition

[Comments]:

[Xiaomi/Yi Xiong]

We also think UE needs to stop CLTM evaluation, but we think UE needs to stop CLTM conditions evaluation for all the LTM candidate configurations.

Because subsequent CLTM is supported, after the initial CLTM, the UE may perform CLTM evaluation based on the conditions configured by ltm-ExecutionCondition configured within the LTM-Candidate. Hence, just stopping the corresponding LTM conditions evaluation does not cause the UE to stop CLTM evaluation based on the conditions configured by ltm-ExecutionCondition. And when the current serving cell set ltm-ServingCellExecutionCondition to release, it means the current serving cell wants UE to suspend CLTM.

Based on the above description, we suggest the following change:

1>	else (ltm-ServingCellExecutionCondition set to release):

2>	release the ltm-ServingCellExecutionCondition;

2>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

3>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

2>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

3>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.x.

[Huawei] The execution condition that are being evaluated might be from LTM-Candidate, not from ltm-ServingCellExecutionCondition. In this case, it is rather strange that the release of ltm-ServingCellExecutionCondition is stopping the execution of those conditions.

# X151

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X151 | MOB | 1 | Correction on conditional LTM cell switch execution for only one triggered LTM candidate configuration | R2-25xxxxx | Xiaomi (Yi Xiong) |  | V006 | Agreed |

[Description]: In LTM cell switch conditions evalution based on L3 measurements section 5.3.5.18.x, the target candidate cell, whose condition is met, is not considered as the selected cell.

In LTM cell switch execution section 5.3.5.18.6:

For L3 based CLTM (LTM cell switch triggered upon the fulfilment of LTM cell switch execution conditions), the LTM cell switch execution is based on the selected cell., as shown below:

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.5.18.x or 5.3.7.3) according to clause 5.3.5.3;

However, the selected cell is determined by the bullet 2>, which is only applicable when ‘more than one LTM candidate configuration has triggered’, as shown below:

1>	if this procedure is triggered due to fulfilment of LTM cell switch execution conditions:

2> if more than one LTM candidate configuration has triggered this procedure:

3>	select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution;

Therefore, if only one LTM candidate configuration has triggered, there would be no ‘selected cell’ according to the current spec. LTM cell switch execution for L3 CLTM can not be executed.

The similar issue of CHO has been discussed in RAN2, and the CR “R2- 2202835 Correction on conditional reconfiguraiton execution for only one triggered cell” has been agreed in RAN2#117-e meeting for the similar issue of CHO.

[Proposed Change]: Based on the above description, we suggest the following change:

1>	if this procedure is triggered due to fulfilment of LTM cell switch execution conditions:

2> if more than one LTM candidate configuration has triggered this procedure:

3>	select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution;

2>	else:

3> consider the triggered LTM candidate configurations as the selected cell for the LTM cell switch execution;

[Comments]:

[MediaTek (Pasi)]

Agree with Xiaomi's proposal, except we think the new 3> should have "candidate configuration", not "candidate configurations".

# M200

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M200 | MOB | 1 | Ambiguity in handling of LTM UE variables in LTM cell switch |  | Pasi (MediaTek) |  | V008 | Agreed |

[Description]:

1>	if the LTM cell switch is triggered on the MCG; or

1>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID, and VarLTM-ServingCellNoSecurityChange;

-	the ltm-Config and ltm-ConfigNRDC (if configured);

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

3>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

-	the ServingCellConfigCommon of the PCell;

In above spec clip, the yellow text refers to the UE variables associated with ltm-Config for LTM on the MCG.

However, since this spec part is executed by the UE also when LTM cell switch is triggered on the SCG based on ltm-ConfigNRDC, it is ambiguous which UE variables, those associated with ltm-Config for MCG or those associated with ltm-ConfigNRDC, are meant here.

Note that even if we have "2> release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:", which intends to say "MCG related configurations", there still is ambiguity as the UE variables are not received via any SRB.

[Proposed Change]:

1>	if the LTM cell switch is triggered on the MCG; or

1>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID associated with the ltm-Config for LTM on the MCG (if configured);

-	the UE variable VarLTM-ServingCellNoSecurityChange;

-	the ltm-Config and ltm-ConfigNRDC (if configured);

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

3>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

-	the ServingCellConfigCommon of the PCell;

[Comments]:

[Huawei] In Rel-18, it is already possible to have two instances of VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID. If there is any ambiguity because UE variables are not received from an SRB, that ambiguity already exists in Rel-18, then a Rel-18 correction should be considered.

On the TP: for SCG LTM cell switch, shouldn't the variables associated with ltm-ConfigNRDC also be kept? In general, the variables for MCG LTM should not be cleared until the whole MCG LTM configuration is released, and the variables for SCG LTM should be cleared if the whole SCG LTM configurations is released, or if the SCG is released.

# Z157

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z157 | MOB | 1 | The release of the current SCG configuration in case of MCG LTM with SCG configuration |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: According to the current spec, the UE shall release/clear the MCG part of the current UE configuration if the LTM cell switch is triggered on the MCG, but no handling on the SCG part of the current UE configuration. It means that the SCG part of the current UE configuration shall be maintained during LTM cell switch. However, for MCG LTM with SCG configuration, the applied candidate configuration shall include both MCG and SCG part. Thus, the UE should also remove the SCG part of the current configuration, to avoid any configuration mismatch.

[Proposed Change]: To clarify that the UE shall release the current SCG configuration if the LTM cell switch is triggered on the MCG and the LTM candidate configuration to be applied includes mrdc-SecondaryCellGroup. For example:

1>	if the LTM cell switch is triggered on the SCG; and

1>	if the LTM cell switch is triggered on the MCG and the LTM candidate configuration to be applied includes mrdc-SecondaryCellGroup:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

[Comments]:

[Rapporteur] We agreed that the field mrdc-ReleaseAndAdd is always included if there is an SCG. So whether the UE release the SCG or not is enforced by the ASN.1 signalling (and network).

[Huawei] Disagree, because mrdc-ReleaseAndAdd is always included if there is an SCG in the target MCG LTM configuration.

# J052

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J052 | MOB | 1 | Ambiguity on selecting a sk-Conter |  | Takaki (Sharp) |  | V018 | Rejected |

[Description]: There is ambiguity regarding which sk-Counter should be selected. In the current wording, an sk-Counter included in ltm-SK-Counters, which is not directly related, is selected. While this is technically correct, it is not clear for implementers. The wording needs to be clarified by referring to the SCPAC text.

[Proposed Change]: Clarify which sk-CounterList to be used to get the first sk-Counter as below:

2> else if the LTM cell switch is triggered on the SCG:

3>	consider the first sk-Counter value in the sk-CounterList associated with the field ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange as the selected sk-Counter value, and update the secondary key by performing security key update procedure as specified in 5.3.5.7;

[Comments]:

[Rapporteur] There seems to be no ambiguity as the UE selects the sk-counter which is related to the value of "ltm-NoSecurityChangeID" as written in the procedural text. Therefore, current text seems to be fine.

[Huawei] Agree with the TP.

# C154

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C154 | MOB | 1 | Ambiguity on removing the the selected sk-Counter value from which entry in ltm-SK-Counters. |  | Rui (CATT) |  | V005 | Rejected |

[Description]: it is not clear UE should remove the selected sk-Counter value from which entry in ltm-SK-Counters, as ltm-SK-Counters includes multiple entries of SK-CounterConfigLTM as follows,

VarLTM-ServingCellNoSecurityChange-r19 ::=     SEQUENCE {

ltm-ServingCellNoSecurityChangeID-r19           INTEGER (1..maxNrofLTM-Configs-plus1-r18)                                 OPTIONAL,

ltm-SK-Counters-r19                             SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfigLTM-r19    OPTIONAL

}

.

[Proposed Change]:

5.3.5.18.6	LTM cell switch execution

……

2> else if the LTM cell switch is triggered on the SCG:

3>	consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange associated to the the field ltm-NoSecurityChangeID as the selected sk-Counter value, and update the secondary key by performing security key update procedure as specified in 5.3.5.7;

3>	remove the selected sk-Counter value from the entry associated to the the field ltm-NoSecurityChangeID in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange;

[Comments]:

[Rapporteur] Current text seems fine. There is little ambiguity as the UE variable has only two fields.

# M201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M201 | MOB | 1 | PDCP data recovery at the end of LTM cell switch |  | Pasi (MediaTek) |  | V008 | Rejected |

[Description]:

2>	at the end of the procedure, for each drb-Identity value that is part of the current UE configuration:

3>	if the LTM cell switch is triggered on the MCG; or

3>	if the LTM cell switch is triggered on the SCG and this DRB is using the secondary key; or

3>	if the LTM cell switch is triggered on the SCG and the keyToUse for this DRB is changed:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KgNB) or secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

4>	if drb-ContinueROHC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueROHC is configured;

4>	if drb-ContinueEHC-DL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-DL is configured;

4>	if drb-ContinueEHC-UL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-UL is configured;

4>	if drb-ContinueUDC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueUDC is configured;

4>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

3>	else if LTM cell switch is triggered on the SCG and this DRB is using the master key:

4>	if the RLC entity of an RLC bearer associated with this DRB is re-established or released during LTM cell switch execution:

5>	if this DRB is an AM DRB:

6>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

In above spec clip, the yellow text seems fully unnecessary, because there is already corresponding green text in 2>.

[Proposed Change]:

2>	at the end of the procedure, for each drb-Identity value that is part of the current UE configuration:

3>	if the LTM cell switch is triggered on the MCG; or

3>	if the LTM cell switch is triggered on the SCG and this DRB is using the secondary key; or

3>	if the LTM cell switch is triggered on the SCG and the keyToUse for this DRB is changed:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KgNB) or secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

4>	if drb-ContinueROHC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueROHC is configured;

4>	if drb-ContinueEHC-DL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-DL is configured;

4>	if drb-ContinueEHC-UL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-UL is configured;

4>	if drb-ContinueUDC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueUDC is configured;

4>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

3>	else if LTM cell switch is triggered on the SCG and this DRB is using the master key:

4>	if the RLC entity of an RLC bearer associated with this DRB is re-established or released during LTM cell switch execution:

5>	if this DRB is an AM DRB:

6>	trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5];

[Comments]:

[Rapporteur] True, it seems there is a repetition, but the procedural text does not seems to be wrong. I prefer to keep it as it is unless we find some problem (since part of this text is there also for Rel-18).

[Huawei] Agree.

# C155

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C155 | MOB | 1 | Upon LTM execution, UE behaivor is missing on performing PDCP discard for SRBs according to the Rel-19 ID. |  | Rui (CATT) |  | V005 | Agreed |

[Description]: Upon LTM execution, UE behaivor is missing on performing PDCP discard for SRBs according to the Rel-19 ID.This is needed according to the RAN2 agreements as follows,

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

[Proposed Change]:

[Comments]:

[Huawei] For intra-CU LTM, we agreed that the network will include discardOnPDCP. So perhaps we rely on this here?

# E055

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E055 | MOB | 1 | Using existing PDCP discard flag for SRBs |  | Tony (Ericsson) |  | V009 | ToDo |

[Description]: In RRC CR agreed in the last meeting (R2-2507729) it was agreed to add the following sentence because of an agreement taken during the WI which clarified that for SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3:

Section 5.3.5.18.6:

3>	if the LTM cell switch is triggered on the SCG and the SRB is using the secondary key:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KgNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

3>	else:

4>	trigger the PDCP entity of SRB to perform SDU discard as specified in TS 38.323 [5];

However, in Rel-18 we have agreed that the handling for the SRBs about e.g., PDCP discarding was according to the existing flag discardOnPDCP. According to this, would be good to maintain the alignment with Rel-18.

[Proposed Change]:

Section 5.3.5.18.6:

3>	if the LTM cell switch is triggered on the SCG and the SRB is using the secondary key:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KgNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

[Comments]:

# C156

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C156 | MOB | 1 | Duplicated check for the different R19 ID case |  | Rui (CATT) |  | V005 | Agreed |

[Description]: it is duplicated with,

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

[Proposed Change]:

5.3.5.18.6	LTM cell switch execution

……

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

……

2>	if the value of field ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChange

23>	replace the value of ltm-ServingCellNoSecurityChangeID in VarLTM-ServingCellNoSecurityChange with the value of ltm-NoSecurityChangeID in the LTM-Candidate in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1>	else if the field ltm-NoSecurityChangeID is not configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

[Comments]:

[Xiaomi/Yi Xiong]

We also think the above procedures are duplicated. We support the change.

[MediaTek (Pasi)]

Agree with CATT and Xiaomi.

[Huawei] Agree.

# X152

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X152 | MOB | 1 | UE behaviours when ltm-NoSecurityChangeID = ltm-ServingCellNoSecurityChangeID is missing. | R2-25xxxxx | Xiaomi (Yi Xiong) |  | V006 | Agreed |

[Description]: In LTM cell switch execution section 5.3.5.18.6:

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

…………………unused part, skip…………………

1>	else if the field ltm-NoSecurityChangeID is not configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

…………………unused part, skip…………………

Based on the above spec, the UE behaviours when the value of ltm-NoSecurityChangeID is not equal to the value of ltm-ServingCellNoSecurityChange and the UE behaviours when the ltm-NoSecurityChangeID is not configured and ltm-ServingCellNoSecurityChangeID is not stored have been captured in the spec.

But the UE behaviours when the value of ltm-NoSecurityChangeID is equal to the value of ltm-ServingCellNoSecurityChange is missing, suggest to add associated wording.

[Proposed Change]: Based on the above description, we suggest the following change:

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

…………………unused part, skip…………………

1>	else if the field ltm-NoSecurityChangeID is not configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

…………………unused part, skip…………………

[Comments]:

[MediaTek (Pasi)]

Agree that this condition should be added. However, if we add that, we can replace both of these conditions with simple "1> else:" (assuming also X153 is agreed).

[Huawei] The two tests should not be there, what should be there is the text from Rel-18.

# X153

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X153 | MOB | 1 | Incorrect UE behaviours when the value of ltm-NoResetID is equal to the value of ltm-ServingCellNoResetID. | R2-25xxxxx | Xiaomi (Yi Xiong) |  | V006 | Agreed |

[Description]: In LTM cell switch execution section 5.3.5.18.6:

1>	else if the field ltm-NoSecurityChangeID is not configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

1>	if the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.x or 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.x or 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

Based on the above procedure, when the ltm-NoSecurityChangeID is not configured and ltm-ServingCellNoSecurityChangeID is not stored, the UE shall perform RLC re-establishment and PDCP data recovery (AM DRB). In other words, for this case, even if the value of ltm-NoResetID is equal to the value of ltm-ServingCellNoResetID, the UE also needs to perform RLC re-establishment and PDCP data recovery (AM DRB), which is inconsistent with the principle of Rel-18 LTM.

Hence, we think the wording for whether to perform RLC re-establishment and PDCP data recovery (AM DRB) based on the Rel-18 ID (ltm-NoResetID and ltm-ServingCellNoResetID) shall be the next bullet of “1>	else if the field ltm-NoSecurityChangeID …… VarLTM-ServingCellNoSecurityChangeID; or”, rather than in the same layer bullet.

In addition, whether the Rel-19 IDs are configured or not and whether the Rel-19 ID(s) are same or different, the Rel-18 ID of serving cell (ltm-ServingCellNoResetID) shall be the value of Rel-18 ID in candidate configuration associated with current serving cell (target cell). Hence, the wording for the update of Rel-18 ID of serving cell shall be the first bullet.

[Proposed Change]: Based on the above description, we suggest the following change:

1>	else if the field ltm-NoSecurityChangeID is not configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID:

2>	if the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.x or 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.x or 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

4>	if servedRadioBearer is set to drb-Identity:

5>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

3>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

3>	for each drb-Identity value that is part of the current UE configuration:

4>	if this DRB is an AM DRB:

5>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.5.18.x or 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

[Comments]:

[MediaTek (Pasi)]

Agree with Xiaomi.

[Rapporteur] Some work seems to be needed and is fine to handle this via a contribution

# X154

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X154 | MOB | 1 | Incorrect accordance section of the selected cell for CLTM |  | Xiaomi (Yi Xiong) |  | V006 | Agreed |

[Description]: In LTM cell switch conditions evalution based on L3 measurements section 5.3.5.18.x, the triggered candidate cell, whose condition is met, is not considered as the selected cell.

In LTM cell switch execution section 5.3.5.18.6, the selected cell is determined by the bullet 2> , as shown below:

1>	if this procedure is triggered due to fulfilment of LTM cell switch execution conditions:

2> if more than one LTM candidate configuration has triggered this procedure:

3>	select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution;

Hence, in section 5.3.5.18.6, the UE can consider one of the LTM candidate configurations as the selected cell for the LTM cell switch execution. Hence, we suggest to change the accordance section from “5.3.5.18.x” to “5.3.5.18.6”

[Proposed Change]: Based on the above description, we suggest the following change:

the selected cell in accordance with 5.3.5.18.6 or 5.3.7.3

[Comments]:

[Huawei] "select as the selected cell" looks ugly.

In addition, 5.3.5.18.y says " perform the LTM cell switch procedure for the LTM candidate configuration associated to the ltm-CandidateId according to the actions specified in 5.3.5.18.6." so "selecting a cell" in 5.3.5.18.6 contradicts with this.

Suggestion: remove this, add a note that if 5.3.5.18.6 is triggered for more than one candidate cell, the UE executes it only for one of them and it is up to the UE to choose one.

# E005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E005 | MOB | 2 | Handling of radio bearers during LTM cell switch | R2-25xxxxx | Tony (Ericsson) |  | V004 | Agreed |

[Description]: Current specification assumes that when the target configuration prepares the LTM candidate configuration, it needs to prepare a radio bearer configuration which is according to the bearer configuration the UE is using in its current source cell. However, there are no means at the moment for the target cell to know what bearer configuration the UE is using in the source cell.

[Proposed Change]: The issue is rather complex and we plan to bring a contribution to the next meeting where we explain the problem and also the possible solutions.

[Comments]:

[Rapporteur] Suggest to discuss this based the company contribution

# X155

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X155 | MOB | 1 | Perform the subsequent CLTM evaluation after LTM Cell switch triggered based on the indication from lower layers. | R2-25xxxxx | Xiaomi (Yi Xiong) |  | V006 | Rejected |

[Description]: In LTM cell switch execution section 5.3.5.18.6, the UE only perform subsequent CLTM evaluation for the selected LTM candidate configuration, as shown below:

1>	if ltm-ExecutionCondition is configured within the LTM-Candidate IE for the selected LTM candidate configuration:

2>	if the field l3-Conditions is included within ltm-ExecutionCondition:

3>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.x according to the received ltm-ExecutionCondition once this procedure is completed;

2>	else if the field l1-Conditions is included within ltm-ExecutionCondition:

3>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ExecutionCondition once this procedure is completed.

RAN2 has agreed “Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change)”. So, after the CLTM candidate configuration is triggered by LTM Cell Switch Command MAC CE, the UE shall also perform subsequent CLTM evaluation. But in current spec, the LTM candidate configuration indicated by lower layers has not been included in the first bullet.

[Proposed Change]: Based on the above description, we suggest the following change:

1>	if ltm-ExecutionCondition is configured within the LTM-Candidate IE for the LTM candidate configuration either indicated by lower layers or for the selected cell:

2>	if the field l3-Conditions is included within ltm-ExecutionCondition:

3>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.x according to the received ltm-ExecutionCondition once this procedure is completed;

2>	else if the field l1-Conditions is included within ltm-ExecutionCondition:

3>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the received field ltm-ExecutionCondition once this procedure is completed.

[Comments]:

[Rapporteur] For the case when an LTM cell switch MAC CE is sent to a LTM candidate configuration which has execution condition, when the UE applies the LTM candidate configuration it will also apply the execution condition within LTM-Candidate IE and it will restart already the execution condition evaluation. So this seems to be already the case.

[Huawei] At this point of the procedure, it could be "for the LTM candidate configuration that was applied in this procedure".

# Z167

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z167 | MOB | 1 | Clarification on stopping LTM cell switch conditions evaluation in LTM cell switch execution conditions modification procedure |  | ZTE (Mengjie Zhang) |  | V015 | ToDo |

[Description]: In RRC CR agreed in the last meeting (R2-2507729), it was agreed to have a new procedure for the handling of the execution condition for LTM. A new section 5.3.5.18.1a LTM cell switch execution conditions modification is introduced to handle the LTM execution condition update upon LTM cell switch execution or receiving the LTM execution condition (re)-configuration, as below:

##### 5.3.5.18.1a LTM cell switch execution conditions modification The UE shall: 1> clear the entry in VarLTM-ExecutionConditionList; 1> if the UE is performing LTM cell switch conditions evaluation based on L1 measurements: 2> request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations; 1> if the UE is performing LTM cell switch conditions evaluation based on L3 measurements: 2> stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8; 1> if this procedure is triggered by LTM cell switch execution as specified in 5.3.5.18.6 and if ltm-ExecutionCondition is configured in the LTM-Candidate IE to which LTM cell switch is performed: 2> store ltm-ExecutionCondition in VarLTM-ExecutionConditionList; 1> else if this procedure is triggered by LTM configuration as specified in 5.3.5.18.1 and if ltm-ServingCellExecutionCondition is set to setup: 2> store ltm-ServingCellExecutionCondition in VarLTM-ExecutionConditionList; 1> for each LTM-ExecutionCondition in VarLTM-ExecutionConditions: 2> if l3-Conditions is included in the LTM-ExecutionCondition: 3> perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the LTM-ExecutionCondition; 2> else if l1-Conditions is included in the LTM-ExecutionCondition: 3> request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the LTM-ExecutionCondition.

In this procedure, the UE shall stop the LTM cell switch conditions evaluation based on L1/L3 measurements for all the LTM candidate configurations (as highlighted by yellow above). However, in case that this procedure is triggered by LTM cell switch execution, the UE has stopped the LTM cell switch conditions evaluation when applying the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE for the target cell and performing the reconfiguration with sync in clause 5.3.5.5.2 (as highlighted by green below).

##### 5.3.5.18.6 LTM cell switch execution … 1> if the LTM cell switch is triggered by an indication from lower layers: 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3; 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.8)): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with this procedure or clause 5.3.7.3) according to clause 5.3.5.3; 1> if the LTM cell switch is triggered on the MCG: 2> release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with this procedure or clause 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete); 1> else, if the LTM cell switch is triggered on the SCG: 2> release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.5.18.8 or 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete); 1> if VarLTM-ExecutionConditionList is present and is not empty: 2> perform LTM cell switch execution conditions modification as specified in 5.3.5.18.1a. ##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. … 2> if this procedure is performed due to an LTM cell switch execution: 3> if UE is performing LTM cell switch conditions evaluation based on L1 measurements: 4> request lower layers to stop the LTM conditions evaluation based on L1 measurements for all the LTM candidate configurations; 3> if UE is performing LTM cell switch conditions evaluation based on L3 measurements: 4> stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations; 3> start synchronising to the DL of the indicated LTM candidate cell, if no DL synchronization for the indicated LTM candidate cell has been already acquired; 2> else: 3> if the target SpCell is different from current SpCell: 4> if UE is performing LTM cell switch conditions evaluation based on L1 measurements: 5> request lower layers to stop the LTM conditions evaluation based on L1 measurements for all the LTM candidate configurations; 4> if UE is performing LTM cell switch conditions evaluation based on L3 measurements: 5> stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations;

Thus, the UE is not required to stop the LTM cell switch conditions evaluation redundantly upon performing LTM cell switch execution conditions modification procedure, if this procedure is triggered by LTM cell switch execution.

[Proposed Change]: Clarify that the UE shall stop the LTM cell switch conditions evaluation if the procedure is triggered by LTM configuration as specified in 5.3.5.18.1. For example:

##### 5.3.5.18.1a	LTM cell switch execution conditions modification

The UE shall:

1>	clear the entry in VarLTM-ExecutionConditionList;

1>	if this procedure is triggered by LTM cell switch execution as specified in 5.3.5.18.6 and if ltm-ExecutionCondition is configured in the LTM-Candidate IE to which LTM cell switch is performed:

2>	store ltm-ExecutionCondition in VarLTM-ExecutionConditionList;

1>	else if this procedure is triggered by LTM configuration as specified in 5.3.5.18.1:

2>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

3>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

2>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

3>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8;

2>	if ltm-ServingCellExecutionCondition is set to setup:

3>	store ltm-ServingCellExecutionCondition in VarLTM-ExecutionConditionList;

1>	for each LTM-ExecutionCondition in VarLTM-ExecutionConditions:

2>	if l3-Conditions is included in the LTM-ExecutionCondition:

3>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the LTM-ExecutionCondition;

2>	else if l1-Conditions is included in the LTM-ExecutionCondition:

3>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the LTM-ExecutionCondition.

[Comments]:

[Huawei] Thanks for raising that "stop..." is also invoked in 5.3.5.5.2.

However, we note that:

- in 5.3.5.5.2: for the case of non-LTM switch, it looks wrong to stop the evaluation of conditions but not clear the corresponding UE variable

- the following alternative would address your concern (not a functional problem though), fix the issue of 5.3.5.5.2 and make the specification shorter:

5.3.5.5.2:

2>	if this procedure is performed due to an LTM cell switch execution:

3>	perform the LTM cell switch execution conditions modification as specified in 5.3.5.18.1a

3>	start synchronising to the DL of the indicated LTM candidate cell, if no DL synchronization for the indicated LTM candidate cell has been already acquired;

2>	else:

3>	if the target SpCell is different from current SpCell:

4>	if the UE has a non-empty VarLTM-ExecutionConditionList

:

5>	perform the LTM cell switch execution conditions modification as specified in 5.3.5.18.1a;

##### 5.3.5.18.1a	LTM cell switch execution conditions modification

The UE shall:

1>	clear the entry in VarLTM-ExecutionConditionList;

1>	if the UE is performing LTM cell switch conditions evaluation based on L1 measurements:

2>	request lower layers to stop the LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

1>	if the UE is performing LTM cell switch conditions evaluation based on L3 measurements:

2>	stop the LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8;

1>	if this procedure is triggered by reconfiguration with sync due to an LTM cell switch execution as specified in 5.3.5.5.2 and if ltm-ExecutionCondition is configured in the LTM-Candidate IE to which LTM cell switch is performed:

2>	store ltm-ExecutionCondition in VarLTM-ExecutionConditionList;

1>	else if this procedure is triggered by LTM configuration as specified in 5.3.5.18.1 and if ltm-ServingCellExecutionCondition is set to setup:

2>	store ltm-ServingCellExecutionCondition in VarLTM-ExecutionConditionList;

1>	for each LTM-ExecutionCondition in VarLTM-ExecutionConditions:

2>	if l3-Conditions is included in the LTM-ExecutionCondition:

3>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the LTM-ExecutionCondition;

2>	else if l1-Conditions is included in the LTM-ExecutionCondition:

3>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the LTM-ExecutionCondition.

5.3.5.3.18.6:

# J053

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J053 | MOB | 1 | Ambiguity on replaceing a sk-ConterList |  | Takaki (Sharp) |  | V018 | Rejected |

[Description]:

5.3.5.18.y	LTM sk-Counter configuration addition/modification

The UE shall:

1>	for each ltm-NoSecurityChangeID received in ltm-SK-CounterConfigToAddModList:

2>	if an entry with the matching ltm-NoSecurityChangeID exists in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange:

3>	replace the sk-CounterList within the VarLTM-ServingCellNoSecurityChange with the sk-CounterList according to the received ltm-NoSecurityChangeID;

2>	else:

3>	add a new entry for this ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange.

[Proposed Change]: Clarify what information element UE is replaced as below:

5.3.5.18.y	LTM sk-Counter configuration addition/modification

The UE shall:

1>	for each ltm-NoSecurityChangeID received in ltm-SK-CounterConfigToAddModList:

2>	if an entry with the matching ltm-NoSecurityChangeID exists in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange:

3>	replace the sk-CounterList within the VarLTM-ServingCellNoSecurityChange with the received sk-CounterList associated to the ltm-NoSecurityChangeID;

2>	else:

3>	add a new entry for this ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange.

[Comments]:

[Rapporteur] Current procedural text already clarify which IE should be replaced. Prooosed text does not seems to add anything new.

[Huawei] Agree.

# Z158

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z158 | MOB | 1 | The wrong IE name |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: According to the ASN.1 signaling, the IE name should be ltm-SK-CounterConfigToReleaseList, instead of ltm-SK-CounterConfigToRemoveList.

LTM-ConfigNRDC information element

-- ASN1START

-- TAG-LTM-CONFIGNRDC-START

LTM-ConfigNRDC-r19 ::=       SEQUENCE {

ltm-ConfigurationSCG-r19                   LTM-Config-r18                                                               OPTIONAL,    -- Need M

ltm-SK-CounterConfigToAddModList-r19       SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfigLTM-r19       OPTIONAL,    -- Need N

ltm-SK-CounterConfigToReleaseList-r19      SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF LTM-NoSecurityChangeId-r19    OPTIONAL,    -- Need N

...

}

-- TAG-LTM-CONFIGNRDC-STOP

-- ASN1STOP

[Proposed Change]: Change the IE name to ltm-SK-CounterConfigToReleaseList.

[Comments]:

# Q511

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q511 | AIML | 1 | Release CSI-LoggedMeasurementConfig in RRC Re-establishment | N/A | Rajeev Kumar |  | V018 | Rejected |

[Description]:

CSI logged measurement configuration should be release upon RRC reestablishment together with logged data (based on RAN2 agreement).

[Proposed Change]:

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

/*omitted

- release successPSCell-Config configured by the PCell, if configured;

- release CSI-LoggedMeasurementConfig, if configured;

[Comments]:

[WI CR rapporteur-v022]: We agree that the CSI logged measurement configuration should be released upon RRC reestablishment. Please note that this is implicitly captured in the legacy procedural text from clause 5.3.7.2 below, by releasing spCellConfig, which contains CSI-MeasConfig within which the CSI logging configuration is sent. Thus, we do not think that the proposed changes are needed.

1>	if UE is not configured with attemptLTM-Switch:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

# Z159

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z159 | MOB | 1 | Removing the wrong reference clause |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: The 5.3.5.18.x is for LTM cell switch conditions evalution based on L3 measurements, i.e. CLTM based on L3 condition, which is not triggered by the indication from lower layers. Besides, the LTM candidate with ltm-NoSecurityChangeID is not applicable to CLTM, i.e. only intra-CU CLTM is supported. Thus, only LTM triggered by the indication from lower layers needs to be considered here.

[Proposed Change]: Remove the “5.3.5.18.x”. For example:

2>	the cell selection is triggered by detecting re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6 and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-NoSecurityChangeID configured within the LTM candidate configuration for which the re-configuration with sync failure is detected:

3>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

[Comments]:

[Rapporteur] I guess that is still good to clarify that this procedure applies also to CLTM, so I am plannig to clarify this.

[Huawei] Agree. About "I guess that is still good to clarify that this procedure applies also to CLTM": all what is refered to in 5.7.3.7 is the LTM cell switch procedure, that is also applicable to CLTM, so this is already clear.

# E051

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E051 | SONMDT | 1 | Remove editor’s note on logging of ltm-RecoveryCellId |  | Ali Parichehreh |  |  | Agreed |

[Description]: in 5.3.7.3, there is an editor’s note left, which needs to be handled. The ltm-RecoveryCellId should only be set when there is an LTM recovery procedure, therefore, when there is no LTM recovery procedure due to security issue in inter-CU LTM, ltm-RecoveryCellId should not be set as in the current implementation.

[Proposed Change]:

Remove the editor’s note:

Editor’s note:      It need to be verified that above SON/MDT statement is in correct position .

[Comments]:

Agreed in RAN2#131bis

# Q512

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q512 | AIML | 1 | Release CSI-LoggedMeasurementConfig following cell selection while T311 is running | N/A | Rajeev Kumar |  | V018 | Rejected |

[Description]:

CSI logged measurement configuration should be release following cell selection while T311 is running.

[Proposed Change]:

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

/* omitted

1>	else:

2>	if UE is configured with attemptCondReconfig; or

2>	if UE is configured with attemptLTM-Switch:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

3>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

3>	release musim-GapPriorityAssistanceConfig, if configured;

3>	release musim-LeaveAssistanceConfig, if configured;

3>	release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running;

3>	release propDelayDiffReportConfig, if configured;

3>	release ul-GapFR2-PreferenceConfig, if configured;

3>	release rrm-MeasRelaxationReportingConfig, if configured;

3>	release maxBW-PreferenceConfigFR2-2, if configured;

3>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

3>	release minSchedulingOffsetPreferenceConfigExt, if configured;

3>	release aerial-FlightPathAvailabilityConfig, if configured;

3>	release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running;

3>	release loggedDataCollectionAssistanceConfig, if configured;

3>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

3>	release applicabilityReportConfig, if configured;

3>	release dataCollectionPreferenceConfig, if configured;

3> release CSI-LoggedMeasurementConfig, if configured;

[Comments]:

[WI CR rapporteur-v022]: As also commented for RIL Q512, we do not think the proposed change is needed, since the UE releases the CSI logged configurations (sent within CSI-MeasConfig), when it releases the cell configurations. Copied from the text above:

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

# C073

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C073 | AIML | 1 | CSI logged measurement configuration |  | Tangxun |  | V003 | Agreed |

[Description]: “CSI-LoggedMeasurementConfig” is an IE name, but not the configuration to release.

[Proposed Change]: update the procedural text as below:

2>	release CSI logged measurement configuration, if configured;

[Comments]:

[Lenovo-Congchi-v011]: no strong view, but it should be “release any CSI logged measurement configuration, if configured” since there could be multiple.

[Ericsson-v022]: We agree with the issue, and we propose further small changes for clarity.

2>	release any CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList, if configured;

[Samsung-Beom-v028]: csi-LoggedMeasurementConfigToAddModList is the field name for NW-side data collection configuration. A field name of a certain configuration could be referred upon reception of the configuration. However, it should not be reffered in the other cases, as UE does not have to store the field name as UE configuration. We understand that is the principle RRC spec generally follows. There are several spots to refer to “csi-LoggedMeasurementConfigToAddModList”. We are okay to keep the text in 5.3.5.5, 5.5x.1 (i.e., upon reception of configuration), but we suggest not refering “csi-LoggedMeasurementConfigToAddModList” but using “CSI logged measurement configuration” simply in other places (i.e., 5.5.4, 5.5x.3.2)

And, we support C073 or Lenovo’s update.

[WI CR rapporteur-v031]: We changed the status from “ToDo” to “PropAgree” with the intention to capture the original proposed changed combined with Lenovo’s proposal, as also suggested by Samsung.

# S042

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S042 | AIML | 1 | Release of NSDC configuration upon transition to RRC_INACTIVE |  | Seung-Beom |  | v026 | ToDo |

[Description]: According to current text, UE stores NSDC (NW-side data collection) configuration and corresponding UAI configuration in UE Inactive AS Context, before they are released. So, they would be restored during RRCResume procedure.

[Proposed Change]: We suggest to follow the legacy principle. i.e., UE stores both configurations in UE Inactive AS Context, and releasese upon RRC resume initiation.

#### 5.3.8.3 Reception of the RRCRelease by the UE 2> if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1: 2> else: 3> store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the ncr-FwdConfig (if configured), the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for: - parameters within ReconfigurationWithSync of the PCell; - parameters within ReconfigurationWithSync of the NR PSCell, if configured; - parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured; - servingCellConfigCommonSIB; - sl-L2RelayUE-Config, if configured; - sl-L2RemoteUE-Config, if configured; - aerial-Config, if configured; - cellDTX-DRX-Config, if configured; 2> release CSI-LoggedMeasurementConfig, if configured; 2> release loggedDataCollectionAssistanceConfig, if configured; 2> discard the logged measurement entries included in VarCSI-LogMeasReport, if any; ### 5.3.13 RRC connection resume #### 5.3.13.2 Initiation Upon initiation of the procedure, the UE shall: 1> if the UE is in NE-DC or NR-DC: 2> if the UE does not support maintaining SCG configuration upon connection resumption: 3> release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored; 1> if the UE does not support maintaining the MCG SCell configurations upon connection resumption: 2> release the MCG SCell(s) from the UE Inactive AS context, if stored; <…> 1> release delayBudgetReportingConfig from the UE Inactive AS context, if stored; - stop timer T342, if running; <…> 1> release CSI-LoggedMeasurementConfig, from the UE Inactive AS context, if stored; - release loggedDataCollectionAssistanceConfig, from the UE Inactive AS context, if stored; - discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

[Comments]:

# C074

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C074 | AIML | 1 | Duplicate release/discard UE behaviour |  | Tangxun |  | V003 | ToDo |

[Description]: in case of MCG RLF, UE will initiate RRC re-establishment procedure, and the CSI logged measurement configuration will also be released due to “2>	release spCellConfig, if configured;”, as it’s a part of spCellConfig. For the similar reason, it’s also unnecessary to add duplicate description of “release loggedDataCollectionAssistanceConfig,” and “discard the logged measurement entries”, since the same contents have been added in RRC re-establishment procedure.

[Proposed Change]: update the procedural text as below:

3>	else:

4>	consider radio link failure to be detected for the MCG, i.e. MCG RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

[Comments]:

[Huawei-Dawid-v004] Agree with CATT’s comment and proposal. During the CR review, rapporteur mentioned MCG failure case. However, this case results in MCG recovery procedure being triggered which can result in the following outcome:

- Connection release  configuration and data discard is already covered in a dedicated section

- Handover  already covered in a dedicated section

- Re-establishment  already covered in a dedicated section

[Lenovo-Congchi-v011]: Also agree with CATT

[Xiaomi-Xing-v012]: We also suppor to delete these parts. If UE is configured with CHO, UE may perform CHO recovery successfully. Therefore, UE should not release UAI and data logging configuration upon RLF.

[Samsung-Beom-v028]: Agree

# J057

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J057 | NR_SL_relay_multihop-Core | 1 | L3 multi-hop relay behaviour is restricted. |  | Tsuboi (Sharp) |  | v019 | PropAgree |

[Description]: SA/CT support L3 multi-hop U2N relay operation. Thus, L3 relay UE in multi-hop should indicate it to upper layer or child UE(s) if an L3 relay UE detects Uu RLF.

A L2/L3 U2N Relay UE in case of single hop or the L2 Last U2N Relay UE shall:

[Proposed Change]:

A L3 U2N Relay UE, L2 U2N Relay UE in case of single hop or the L2 Last U2N Relay UE shall:

[Comments]:

[Rapporteur]: Agree to add the clarification that both single and multi hop L3 U2N Relay UE shall perform RLF handling. Have changed the status from “ToDo” to “PropAgree”.

# H312

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H312 | SONMDT | 0 | Some editorial issues |  | Jun Chen |  | V007 | Agreed |

[Description]: In section 5.3.10.5, some editorial issues have been identified, e.g.:

1>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the UE was configured with condExecutionCond and condExecutionCondPSCell;

2>	if the UE does not support RLF-Report for fast MCG recovery procedure as specified in TS 38.306 [26] or if T316 is not configured:

3>	set pSCellId to the the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change);

if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition and if one entry of choConfig concerns condEventD2;

[Proposed Change]: The following changes are proposed:

[Rapporteur]: thanks, fixed!

# Z301

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z301 | SONMDT | 1 | Alignment on terms of CHO with candidate SCG(s). | R2-25xxxx | QI Tao |  | V012 | Agreed |

[Description]: both “Conditional handover with candidate SCG” and “CHO with candidate SCG” are widely used. It is better to be aligned to “CHO with candidate SCG(s)” as in the Mobility Working item.

[Proposed Change]: Suggest update both “Conditional handover with candidate SCG” and “CHO with candidate SCG” to “CHO with candidate SCG(s)” as in legacy Rel-18 mobility terms.

[Comments]: [Rapporteur] Rapporture agrees with the RIL and implemented the change in the drafted CR according to the RIL.

# C053

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C053 | SONMDT | 1 | PSCell may not be available when RLF happens |  | Tangxun |  | V002 | Agreed |

[Description]: In case a RLF happens, even if the UE was configured with condExecutionCond and condExecutionCondPSCell, there may be no PSCell at UE side.

[Proposed Change]: add “if available” as below:

PSCell (if available, in case of no PSCell change)

[Comments]:

Rapporteur aress and captures the change in the drafted CR

# Z302

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z302 | SONMDT | 1 | Unnecessary reference | R2-25xxxx | QI Tao |  | V012 | Agreed |

[Description]: No need to refer to 38.306 about UE capability each time.

[Proposed Change]: Suggest removing the reference:

2>	if the UE does not support RLF-Report for fast MCG recovery procedure or if T316 is not configured:

3>	set pSCellId to the the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change);

[Comments]:

[Rapporteur] Rapporture agrees with the RIL and implemented the change in the drafted CR according to the RIL.

# X550

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X550 | SONMDT | 1 | Incomplete procedure text |  | Xiaomi (Shuai) |  | V001 | Rejected |

[Description]: Procedure in clause 5.3.10.5 only consider the case for earth-moving cell, the case that NTN cell is (quasi-)Earth fixed cell is missed.

[Proposed Change]: To complete the below procedure texts when considering the reference location of (quasi-)Earth fixed cell is fixed.

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. including the RPLMN);

1>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell (in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the UE was configured with condExecutionCond and condExecutionCondPSCell;

2>	set the measResultLastServPSCell to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected the failure;

2>	if the UE does not support RLF-Report for fast MCG recovery procedure as specified in TS 38.306 [26] or if T316 is not configured:

3>	set pSCellId to the the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change);

1>	if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition:

2>	if one entry of choConfig concerns condEventD1:

3>	set distanceFromReference1 to the measured distance between the UE and the serving cell fixed reference location;

2>	else if one entry of choConfig concerns condEventD2:

3>	set distanceFromReference1 to the measured distance between the UE and the serving cell moving reference location;

Omitted….

2>	for each neighbour cell, if any, included in measResultListNR in measResultNeighCells:

3>	if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure and if the related MCG VarConditionalReconfig only concerns measurement-based trigger condition; or

3>	if the UE supports RLF-Report for conditional handover with time-based and location-based trigger conditions in NTN and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure; or

3>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure:

4>	set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig;

4>	if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or

4>	if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure:

5>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time;

5>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in choConfig that was fulfilled first in time, and the point in time of fulfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fulfilled;

4>	if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition:

5>	if one entry of choConfig concerns condEventD1:

6>	set distanceFromReference2 to the measured distance between the UE and the fixed reference location of the neighbour cell, at the moment of handover failure, or radio link failure;

5>	else if one entry of choConfig concerns condEventD2:

6>	set distanceFromReference2 to the measured distance between the UE and the moving reference location of the neighbour cell, at the moment of handover failure, or radio link failure;

[Comments]:

[Samsung] Agreements in RAN2/RAN3 were only about condEventD2. We also note that the since timeSinceFailure is included in the RLF report, network can calculate the distance, so condEventD1 changes are not critical. So there is no need to extend the agreement.

[Rapporteur]: RAN2 agreement on letting UE report the measured distances is only for earth moving scenario, and there is no enhancement for (quasi-) Earth fixed scenario according to RAN3 agreements from RAN3#125bis: For earth fixed case, there is no need to report the measured distance from the UE.

# H301

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H301 | SONMDT | 1 | CHO trigger condition |  | Jun Chen |  | V004 | Rejected |

[Description]: In section 5.3.10.5, for "time-based or location-based", we understand that the terminology has been used in TS 38.300, but they have not been used in TS 38.331. Insteadly, we have defined some trigger events. In this case, we suggest to clarify the trigger condition a bit.

if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition and if one entry of choConfig concerns condEventD2;

[Proposed Change]: if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition (e.g. condEventD1, condEventD2 or condEventT1) and if one entry of choConfig concerns condEventD2;

[Comments]:

[Samsung]Are we supporting anything based on condEventD1? Anyways, if anything needs to be clarified, it is better to do in capability CR 38.306 so that it will be in a single place, rather than adding it in RRC CR in multiple places. These examples may make the maintenance difficult, as we will have to change later for each conditional event added.

[Rapporteur] Agree with Samsung analysis. Better to fix it in 38.306 if needed. And RAN3 LS only considered moving eath scenario and we don’t need to support fixed earth events so the RIL is rejected.

# N041

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N041, N042 | SONMDT | 1 | RLF-Report for conditional handover with time-based or location-based trigger condition |  | Nokia (Mani) |  | V005 | Rejected |

[Description]: The reporting of distanceFromReference1 and distanceFromReference2 applies only to MRO enhancement for CHO for NTN as per the LS received in RAN2#129 from RAN3. But this is not very clear in the current specification text.

[Proposed Change]: Change as follows in 5.3.10.5. Note that there are two instances in this clause where this change needs to be applied:

“1>	if the UE supports RLF-Report for conditional handover for NTN with time-based or location-based trigger condition and if one entry of choConfig concerns condEventD2”

[Comments]:

[Samsung] This condition should be according to description TS 38.306. So if some changes are needed, they need to be done in TS 38.306 first. Also suggest to add a reference to TS 38.306

[Rapporteur]: the wording is aligned with the 38.306. if any change is needed it should be done in that spec first. RIL is rejected.

# J031

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J031 | SONMDT | 1 | Place distanceFromReference1 after cho-config |  | Chang Ningjuan |  | V008 | Agreed |

[Description]: as per 5.1.2(General requirements) in RRC spec, UE executes steps in procedure in the specified order. In RLF report for location-based CHO in NTN, UE sets distanceFromReference1 if one entry of choConfig concerns condEventD2. However, in current procedure, choConfig has not been included in RLF report yet when UE sets distanceFromReference1. Thus the setting of distanceFromReference1 should be placed after the setting of choConfig.

[Proposed Change]:

Move the setting of distanceFromReference1 after the setting of choConfig in the procedural text of RLF report.

[Comments]:

[Rapporteur]: agree with the proposal. Procedural text for distanceFromReference1 is moved to fulfil the general requirements.

# Z303

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z303 | SONMDT | 1 | The timing to record the distance information and also the usage of puncturation marks (comma and semicolon) | R2-25xxxx | QI Tao |  | V012 | Agreed |

[Description]: the coma at the end of the procedure should be a semicolon.

[Proposed Change]: add the timing of record, and update the punctuation marks:

1>	if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition and if one entry of choConfig concerns condEventD2;

2>	set distanceFromReference1 to the measured distance between the UE and the serving cell moving reference location, at the moment of handover failure, or radio link failure;

[Comments]:

[Rapporteur] Rapporture agrees with the RIL and implemented the change in the drafted CR according to the RIL.

# C054

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C054 | SONMDT | 1 | Duplicate condition |  | Tangxun |  | V002 | Agreed |

[Description]: the condition “if the UE was configured with condExecutionCond and condExecutionCondPSCell” in bullet 3 is duplicate, as it already appears in the bullet 2.

[Proposed Change]: remove the duplicate condition as below:

3>	set the rsIndexResults in measResultLastServPSCell to include all the available measurement quantities of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) , ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

[Comments]:

[Rapporteur]: Rapporteur agrees and captured the change in the draft CR

# H313

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H313 | SONMDT | 0 | Missing a space in CHO with candidate SCG |  | Jun Chen |  | V007 | Agreed |

[Description]: In section 5.3.10.5, it is missing a comma between SCG and ordered below:

1>	for each of the configured measObjectNR in which measurements are available or in which the associated reportConfigNR is configured as conditional handover with time-based or location-based trigger condition:

2>	if the SS/PBCH block-based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), and other than the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell and if the UE supports RLF-Report for conditional handover with candidate SCG ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

[Proposed Change]: In the following sentence in section 5.3.10.5, add a comma between SCG and ordered.

if the UE was configured with condExecutionCond and condExecutionCondPSCell and if the UE supports RLF-Report for conditional handover with candidate SCG ordered such that the

[Comments]:

# J033

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J033 | SONMDT | 1 | Neighbour cell identity when measurement quantity is not available in RLF report |  | Chang Ningjuan |  | V008 | Agreed |

[Description]: similar to other places, UE should first check its capability to include the neighbour cell identity when measurement quantity is not avalible in RLF repot.

[Proposed Change]:

2>	if measurement quantities are not available and if UE supports RLF report for distance-based CHO:

3>	set physCellId in measResultListNR in measResultNeighCells to include the physical cell identity of the neighbour cells that are candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

[Comments]:

[Rapporteur]: agree with the sprit of the change, but we think it should be the following and hamonised with the rest of the text

if the UE supports RLF-Report for conditional handover with time-based and location-based trigger conditions in NTN

# C055

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C055 | SONMDT | 1 | Time based CHO is not relevant |  | Tangxun |  | V002 | Rejected |

[Description]: distanceFromReference2 is only related to location based CHO, so it only needs UE to support RLF-Report for conditional handover with location-based trigger condition.

[Proposed Change]: remove “time-based or” as below:

4>	if the UE supports RLF-Report for conditional handover with location-based trigger condition and if one entry of choConfig concerns condEventD2;

[Comments]:

[Samsung]Isn’t this based on terminoglogy used in capability CR? May need to update in TS 38.306 and add a reference here.

[Rapporteur]: Rapporteur also thinks the UE capability naming should be aligned with TS38.306, thus suggests using the same name that is defined in TS38.306

# C056

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C056 | SONMDT | 1 | UE capability check is missing |  | Tangxun |  | V002 | Agreed |

[Description]: before setting the values in RLF report for conditional handover with candidate SCG, UE capability check should be done.

[Proposed Change]: add “if the UE supports RLF-Report for conditional handover with candidate SCG” as below:

1>	if the UE supports RLF-Report for conditional handover with candidate SCG, for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in cho-WithCandidateSCGInfoList and set the values as follows:

[Comments]:

[Rapporteur]: Rapporteur agrees and captured it in the draft CR.

# Z304

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z304 | SONMDT | 1 | All “cpc” should also include the case of “cpa” | R2-25xxxx | QI Tao |  | V012 | Agreed |

[Description]: using cpc could be misleading that cpa is excluded.

[Proposed Change]: suggest update “cpc” to “cpac” in related IEs, e.g., firstFulfilledConfig.

[Comments]:

[Rapporteur] Rapporture agrees with the RIL and implemented the change in the drafted CR according to the RIL.

# J034

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J034 | SONMDT | 1 | Naming of timeBetweenLastFulfillmentAndEvent-r19 |  | Chang Ningjuan |  | V008 | Agreed |

[Description]: timeBetweenLastFulfillmentAndEvent-r19 is used for the time between the last fulfilled event and RLF/SCG failure for UE configured with CHO with candidate SCG. We think the name of the IE can be improved to align with its purpose.

[Proposed Change]:

Rename the timeBetweenLastFulfillmentAndEvent-r19 to timeBetweenLastFulfillmentAndFailure-r19

[Comments]:

[Rapporteur] we think the event is more generic and could be reused for other purposes in the future but we are fine with the change. Captured in the draft CR.

# N064

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N064 | SONMDT | 1 | RLF, SHR logging when CHO only HO is performed | R2-250nnnn | Gyorgy Wolfner |  | V011 | Rejected |

[Description]: There are a couple of issues with the current wording on when the UE logs the additional parameters when CHO-only handover is performed even if the is also configured with CHO with candidate SCGs:

- The current wording logs the parameters independently of the type of handover was intended to be performed, e.g., it logs the parameters even if the CHO with candidates SCGs was failed.

- The current wording limits the logging of the parameters for the case when the CHO configuration is received after the CHO with candidate SCG configuration.

- The current wording only applies when there is an RLF before the handover, but it does not cover the case when RLF happen shortly after the HO.

- The current wording it does not consider that this is an optional feature.

[Proposed Change]: It is proposed

- To log CHO with candidate SCGs related information only when CHO-only was executed instead of CHO with candidate SCGs;

- Not to limit the logging for the case when the CHO configuration is received after the CHO with candidate SCG configuration;

- To cover the RLF shortly after HO

- To consider that this is an optional feature

A wording changes will be provided in a separate tdoc.

[Comments]:

[Rapporteur]: rejected at the meeting RAN2#131bis

# J030

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J030 | SONMDT | 1 | Condition for fulfilledConfigWhenChoOnly |  | Chang Ningjuan |  | V008 | Agreed |

[Description]: in current condition for setting fulfilledConfigWhenChoOnly “if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration including condRRCReconfig for the same target candidate PCell as set in pCellId”, the yellow highlied part aims for a CHO-only configuration, but the wording “a conditional handover configuration” may also be a CHO with candidate SCG configuration, e.g. UE may receive a CHO with candidate SCG configuration(PCell 1+PSCell 1) first, then receives another CHO with candidate SCG configuration(PCell 1+PSCell 2) later. But in this case, fulfilledConfigWhenChoOnly should not be set. Thus such case should be excluded.

[Proposed Change]:

2>	if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration including condRRCReconfig not associated with condExecutionCondPSCell for the same target candidate PCell as set in pCellId:

3>	set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

[Comments]:

[Rapporteur] Agree with clarification. Proposed change in RIL is captured in the draft CR.

# Z305

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z305 | SONMDT | 1 | Which condRRCReconfig should be more specific. | R2-25xxxx | QI Tao |  | V012 | ToDo |

[Description]: network might isues mutiple condRRCReconfig update, the one we are concerning about is the latest one.

[Proposed Change]: adding a condition to specific which conditional handover it is:

2>	if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration including condRRCReconfig for the same target candidate PCell as set in pCellId and the conditional handover was the last one UE received before the failure:

3>	set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

[Comments]:

[Rapporteur]: Rapporteur tends to agree but in rapporteurs understanding we have not discussed/agreed which conditional handover configuration to be taken as reference point, so I suggest to bring a contribution on this issue, so we discuss this in online session.

# E016

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E016 | SONMDT | 1 | Conditions to log the L1 RSRP in RLF report |  | Ali Parichehreh |  | V004 | Agreed |

[Description]: In 5.3.10.5, the condition to log the L1-RSRP for each neighboring cell should be aligned with the case when logging the L1-RSRP for the serving cell.

[Proposed Change]:

1>	if the UE supports RLF-Report for MCG LTM cell switch, and if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell (in case of HO failure) or PCell (in case of RLF), for each neighbour MCG LTM candidate cell:

[Comments]:

# E017

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E017 | SONMDT | 1 | Incorrect condition check to decide lastHO-Type |  | Ali Parichehreh |  | V004 | Agreed |

[Description]: in 5.3.10.5, when setting the lastHo-Type, the procedure text used is “if …  else if… if” which is not correct.

[Proposed Change]:

2>	if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

3>	set lastHO-Type to cho;

2>	else if the UE supports RLF-Report for MCG LTM cell switch and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning an LTM cell switch:

3>	set lastHO-Type to ltm;

2>	else if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning conditional handover with candidate SCG:

3>	set lastHO-Type to choWithCandidateSCG;

<omitted>

4>	if the UE supports RLF-Report for DAPS handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a DAPS handover:

5>	set lastHO-Type to daps;

4>	else if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

5>	set lastHO-Type to cho;

4>	else if the UE supports RLF-Report for MCG LTM cell switch and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning an LTM cell switch:

5>	set lastHO-Type to ltm;

4>	else if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning conditional handover with candidate SCG:

5>	set lastHO-Type to choWithCandidateSCG;

[Comments]:

[Sharp] firstly, would like to clarify whether a conditional handover with candidate SCG can also be considered as a conditional handover? If no, the proposed change is good. If yes, for a executed CHO with candidate SCG, UE will set the lastHO-Type to cho instead of choWithCandidateSCG according to the proposed change, then the change is not correct/needed.

[Rapporteur]: in rapporteur understanding the CHO with Candidate SCG should not be mixed with CHO. I captured the RIL in the draft CR. If Sharp has concern on this, please feel free to bring a contribution addressing your concern.

# Z003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z003 | AIML | 1 | The UE behaviour for NW side data collection configuration when going to RRC-Idle. |  | Fei |  | V015 | Agreed |

[Description]:

### 5.3.11	UE actions upon going to RRC_IDLE

The UE shall:

/omit for short/

1>	except if going to RRC_IDLE was triggered by inter-RAT cell reselection while the UE is in RRC_INACTIVE or RRC_IDLE or when selecting an inter-RAT cell while T311 was running or when selecting an E-UTRA cell for EPS fallback for IMS voice as specified in 5.4.3.5:

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_IDLE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_IDLE and perform cell selection as specified in TS 38.304 [20];

1>	release CSI-LoggedMeasurementConfig, if configured;

1>	release loggedDataCollectionAssistanceConfig, if configured;

1>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

As the NW side data collection only can work in RRC-Connected state, there is no need to explicitly describe the releasing of the CSI-LoggedMeasurementConfig, and loggedDataCollectionAssistanceConfig in the UE actions going to RRC_IDLE, anyway all those configurations will be released.

[Proposed Change]:

Remove yellow highlighted part from the specification directly..

[Comments]:

[WI CR Rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# Z452

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z452 | NR_SL_relay_multihop-Core | 1 | RRC connection setup/resume initiation | R2-25xxxxx | ZTE (Weiqiang Du) |  | V009 | PropAgree |

[Description]: As legacy, UE is allowed initiate RRC setup or resume to transmit L2 multihop relay discovery message if sl-L2U2N-MH-Relay is included in in SIB12..

[Proposed Change]: RAN2 is to agree that UE can initiate RRC Setup and Resume if network indicate support of MH, but corresponding pool is not configured, capture new condition in 5.3.3.1a and 5.3.13.1a as below:

if the UE is configured by upper layers to transmit NR sidelink L2 U2N MH relay discovery messages and sl-L2U2N-MH-Relay is included in SIB12

[Comments]:

[Rapporteur]: Agree to capture new condition in 5.3.3.1a and 5.3.13.1a as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# E037

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E037 | LPWUS | 1 | Clarify when the UE is allowed to use LR-RSRP instead of RSRP | R2-25xxx | Ericsson (Martin) |  | V006 | Rejected |

# H056

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H056 | LPWUS | 1 | Missing “release lpwus-OffsetPreferenceConfig, if configured, and stop timer T346xx, if running;” from Section 5.3.7.3 | R2-25xxxxx | Rama Kumar Mopidevi (Huawei) |  | V010 | Agreed |

[Description]: Section 5.3.7.3 should also include “release lpwus-OffsetPreferenceConfig, if configured, and stop timer T346xx, if running;” As the section is not present in the Review file, I added RIL just after section 5.3.13.2

[Proposed Change]: Add “release lpwus-OffsetPreferenceConfig, if configured, and stop timer T346xx, if running;” to Section 5.3.7.3.

[Comments]:

# H057

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H057 | LPWUS | 1 | Merge sentence “stop timer T346xx, if running” with “release lpwus-OffsetPreferenceConfig from the UE Inactive AS context, if stored” | R2-25xxxxx | Rama Kumar Mopidevi (Huawei) |  | V010 | Rejected |

[Description]: Following the existing description, merge the two sentences.

[Proposed Change]: “release lpwus-OffsetPreferenceConfig from the UE Inactive AS context, if stored, and stop timer T346xx, if running.”

[Comments]:

[WI Rapp]: There is no difference. Current wording is similar as the legacy text.

# J001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J001 | NES | 1 | Align smtc between MCG and SCG |  | Sharp (LIU Lei) |  | V006 | ToDo |

[Description]: When OD-SSB is activated, smtcx is used to setup SMTC and when adapt SSB is activated, smtcy is used to setup SMTC, i.e., smtc1 is replaced. Thus smtcx/smtcy configured by the MCG should be aligned with smtc1 configured by SCG for the same ssbFrequency.

[Proposed Change]: The text proposal is as below:

-	to ensure that, if a measurement object associated with the MCG has the same ssbFrequency as a measurement object associated with the SCG:

-	for that ssbFrequency, the measurement window according to the smtc1/smtcx/smtcy configured by the MCG includes the measurement window according to the smtc1 configured by the SCG, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14].

[Comments]:

[Apple] Agree with the intention because we have the case that smtcx/smtcy replace with smtc1. So, the restriction on smtc1 should be also applied to smtcx/smtcy.

[Rapporteur]: We suggest discussing this together with the other stmc related changes.

# C009

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C009 | NTN | 1 | clarify how UE determines the periodicity of smtc5list in MO | Yes, R2-250xxxxx | CATT (Da Wang) |  | v019 | Agreed |

[Description]: We think the periodicity of smtc5list can be optional present. For an entry, if its periodicity is absent, it uses the periodicity from the smtc1.

[Proposed Change]:

If smtc5list is present, for cells indicated in the pci-List parameter in each SSB-MTC5 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity (if present) and offset parameter in each SSB-MTC5 configuration and use the duration parameter from the smtc1 configuration. If the periodicity parameter is absent for an SSB-MTC5 configuration, the UE uses the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR serving cell meeting the above condition.

[Comments]:

[Rapp] In the MO, the network can configure a maximum of 2/4 SMTC offsets (depending on the UE capabilities). Therefore, legacy smtc4list can be used for the case where the periodicity is equal to smtc1. Smtc5list was introduced with the purpose of enabling a new periodicity.

[Qualcomm] If UE does not support two periodicities, SMTC configuration in connected mode is exactly same as Rel-17.

# X200

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X200 | NES | 1 | SMTC handling for OD-SSB | R2-25xxxxx | Xiaomi (Li Zhao) |  | V003 | ToDo |

[Description]: The existing text in 5.5.2.10 still needs to be further polished to reflect the mapping between the SMTC and the OD-SSB periodicity

[Proposed Change]: RAN2 to discuss and agree the following text.

“If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as the first OD-SSB periodicity value of od-ssb-Periodicity-r19; the UE shall setup SMTC according to the second configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as the second OD-SSB periodicity value of od-ssb-Periodicity and so on”

[Comments]: [OPPO] It is not super clear to us how to handle this. Specifically: 1)	If there is a periodicity for which there is no corresponding OD-SSB periodicity, following this rule, the SMTC has to be configured within smtcxlist (e.g., if there are OD-SSB configuration for ms5 and ms20, but there is no OD-SSB configuration for ms10), leading to unnecessary signaling overhead?

[Sharp]: Agree with the comments from OPPO. We think for simplicity, configured SMTC number in smtcxlist can be same as configured OD-SSB periodicity value number. For example, 4 OD-SSB configurations with 5ms OD-SSB periodicity, and 4 OD-SSB configurations with 20ms OD-SSB periodicity, then there are 2 OD-SSB periodicity values and 2 SMTC in smtcxlist. The potential change is as below:

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as the first value among OD-SSB periodicity value(s) in OD-SSB configuration(s) for the serving cell

[Xiaomi]: We think both options can work, considering there are only six candidate periodicities, then six candidate SMTC, the signalling overhead is not a big issue? Not sure if the option proposed by Sharp can describe the intention clearly.

[ER] Agree with Oppo. The current text, or the proposed correction are not clear. It seems rather convoluted approach to try to map values in a list to something configured elsewhere. Typically this kind of associations are done via configIDs not by comparing field values.

Furthe, see N002 and other and ER response therein. It means also offset may vary with MAC CE and periodicity is not enough to map these. Further, offset is coded in different way in SSB-MTC and in OD-SSB.

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

OD-SSB-r19 ::=      SEQUENCE {

od-ssb-SFN-Offset-r19                   INTEGER (0..15)                                                    OPTIONAL,      -- Cond ODssbAOssb [RIL]: O007, NES[RIL]: L203, NES

od-ssb-halfFrameIndex-r19               ENUMERATED {zero, one}                                             OPTIONAL,      -- Cond ODssbAOssb [RIL]: O007, NES

N002, NES

[Xiaomi]: I think the mapping between SMTC and periodicity is RAN2 agreement, whether the other parameters may also vary or not, e.g., offset, is another issue. As long as the OD-SSB periodicity is determined, the mapped SMTC is determined.

[Samsung]: Not sure why we need to have this strict first, second and so on mapping between smtcxlist and od-ssb-Periodicity. Suggest to reword as

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to an SMTC in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, which has the same SS/PBCH block reception periodicity (derived from parameter periodicityAndOffset) as the SS/PBCH block reception periodicity (od-ssb-Periodicity) in the configuration of activated OD-SSB of the serving cell.

[Apple] First, we agree with Xiaomi that mapping between SMTC and OD-SSB periodicity is RAN2 agreement. SFN-offset and HalfFrameIndex are parameters of OD-SSB (rathe than SMTC), which is indepdent and should not be mixed with SMTC operation. Our understanding on the procedure (same procedure between OD-SSB and SSB adaptation):

- The UE first determines SMTC according to periodicity of OD-SSB indicated by MAC-CE.

- Then, within the SMTC window in 1), the UE applies the SNF-offset and HalfFrameIndex indicated by MAC-CE to identify the location of OD-SSB for seriving cell measurement.

Only 1) needs to be implemented in clause 5.5.2.10 of 38.331 while details of 2) are captured in RAN1 spec.

Based on above, we think Xiaomi’s procedure text is correct, but some companies still have some concerns:

- Reduce signaling overhead to allow not configure all possible SSB periodicity (e.g. OPPO and Sharp)

- Need to simplify it and align with SSB adaptation (e.g. N001, Samsung and Sharp)

  - We think the Samsung suggested text has some issue that it assumes SMTC periodicity is always same as SSB periroicity, which is not necessary.

To address above 2 issues, we provide a suggestion on how to similify the procedure and align OD-SSB and SSB adaptation in A103.

[Rapporteur] This requires further discussion in the next meeting.

[Huawei] With [N002], the maximum number of OD-SSB specific SMTC should be extended to 12, and there is a one-to-one mapping between the SMTCs and the OD-SSB configurations. This mapping relationship is already provide in the existing text in 5.5.2.10, so we think no further change to procedure text is needed.

# L201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L201 | NES | 1 | SMTC configuration for OD-SSB |  | LGE (Han Cha) |  | V016 | PropReject |

[Description]: According to the section 5.5.2.10, UE setups a new SMTC for OD-SSB instead of maintaining the legacy SMTC. It means that UE performs measurements on the intra-frequency neighbour cell based on the new SMTC as well as on OD-SSB of the serving cell. This implementation violates RAN2 and RAN4 agreements. Related RAN2 and RAN4 agreements are listed as follows:

RAN2#130 - (modified) The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. RAN4#116 - For SCell measurement, UE shall follow the OD-SSB specific SMTC when OD-SSB is activated; For neighbor cell measurement, UE follows legacy SMTC regardless of status of OD-SSB.

According to the above agreements, UE shall setup an additional SMTC to perform measurements on OD-SSB and maintain the legacy SMTC to perform measurements on intra-frequency neighbour cell.

[Proposed Change]: SMTC for OD-SSB shall be an addtioanl SMTC. The text proposal is as below:

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup an additional SMTC according to the first configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first OD-SSB configuration for the serving cell; the UE shall setup an additional SMTC according to the second SMTC in smtcx-list for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in od-ssb-Periodicity and so on.

[Comments]:

[Ericsson] This is for corresponding MO. The original servingcell MO and SMTC therein is used for neighborcell measurements and OD-SSB specific MO and or smtc is only used for this serving cell SSB measurements.

[Apple] Agree with Ericsson. In addition, case 1 (no AO-SSB) doesn’t use legacy SMTC. So, we don’t see the need to add “an additional”.

[Rapporteur]: The proposed change does not seem to be essential.

# A103

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A103 | NES | 1 | Simplification/polishment of SMTC handling for OD-SSB and SSB adaptation | Yes | Apple (Peng Cheng) |  | V030 | ToDo |

[Description]: In X200 discussion, there are some concerns about procedure text on handling smtcx/smtcy for OD-SSB and SSB adaptation:

- Need to reduce signaling overhead to allow not configure all possible values of SSB periodicities (e.g. 6 values for OD-SSB and 2 values for SSB adaptation).

- Need to simplify the procedure text on SMTC setup and align the wording between OD-SSB and SSB adaptation (e.g. suggestion from N001).

To address above 2 issues, we provide the following suggestion on how to similify the procedure and align OD-SSB and SSB adaptation:

- In ASN.1, modify SSB-MTCxList-r19 and SSB-MTC-SSBAdapt-r19 to use same IE type (e.g. SSB-MTCx-r19) which explicitly includes the mapped SSB periodicity.

- In Section 5.5.2.10, modify the procedure of OD-SSB and SSB adaptation with aligned wording that the SMTC with same mapped SSB periodicity is selected.

[Proposed Change]:

ASN.1:

SSB-MTCxList-r19 ::=           SEQUENCE (SIZE(1..6maxAdditionalSMTC-ODSSB-r19)) OF SSB-MTCSSB-MTCx-r19

SSB-MTC-SSBAdapt-r19 ::=  SEQUENCE (SIZE(1..2maxAdditionalSMTC-SSBAdapt-r19)) OF SSB-MTCSSB-MTCx-r19

maxAdditionalSMTC-ODSSB-r19      INTEGER ::= 6  -- Maximum number of additiona SMTC for OD-SSB.

maxAdditionalSMTC-SSBAdapt-r19  INTEGER ::= 2-- Maximum number of additiona SMTC for SSB adaptation.

SSB-MTCx-r19 ::=             SEQUENCE {

ssb-Periodicity-r19                 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1},

SMTC-r19 						      SSB-MTC

}

Section 5.5.2.10:

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field an SMTC in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first OD-SSB configuration for the serving cell which has the same SS/PBCH block reception periodicity (ssb-Periodicity within SSB-MTCx) as the one in the configuration of activated OD-SSB of the serving cell; the UE shall setup SMTC according to the second SMTC in smtcx-list for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in od-ssb-Periodicity and so on.

If smtcy-SSBAdapt is present, the UE shall setup SS/PBCH block measurement timing configuration (SMTC) according to smtc1 for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI is not received or the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by ssb-periodicityServingCell; Otherwise, the UE shall setup SMTC according to the first an SMTC in smtcy-SSBAdapt for serving cell measurements on the corresponding MeasObjectNR, which has the same SS/PBCH block reception periodicity (ssb-Periodicity within SSB-MTCx) as the one indicated by if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the first adaptive SSB periodicity in adap-SSB-BurstPeriodicityList; the UE shall setup SMTC according to the second SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the second adaptive SSB periodicity in adap-SSB-BurstPeriodicityList.

[Rapporteur] This requires further discussion in the next meeting.

# N001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N001 | NES | 1 | Smtcy and smtcx description sseems to differ unnecessarily much. |  | Jarkko Koskela (Nokia) |  | V009 | ToDo |

[Description]: We should aim to align descriptions of smtcx/smtcy. For smtcy likely we do not need to talk  how it is activated – that would simplify the text a lot. It seems obvious from elsewhere that DCI is used – no need to repeat that multiple times in the text.

[Proposed Change]: For example something like this (but we could contribute more proper TP to the meeting as this is not purely asn.1 problem):

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first  OD-SSB configuration for the serving cell; the UE shall setup SMTC according to the second SMTC in smtcx-list for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in od-ssb-Periodicity and so on [RIL]: X200, NES.

If smtcy-SSBAdapt is present, when SSB adaptation is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtcy-SSBAdapt  for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first  SSB adapatation configuration for the serving cell; the UE shall setup SMTC according to the second SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in adap-SSB-BurstPeriodicityList

[Comments]:

[Ericsson] For smtcx, since N002 seems valid, it is not enough to map per periodicity and further changes are needed.

[Apple] Agree with the intention of Nokia (i.e. align descriptions of smtcx/smtcy) and it can achieved by N002 (i.e. moving SFN offsert and half frame index under od-ssc-config). And we also agree that smtcy (SSB adaptation) doesn’t need to mention “after activated”.

We provide a suggestion on how to similify the procedure and align OD-SSB and SSB adaptation in A103.

[Rapporteur] This requires further discussion in the next meeting.

# L202

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L202 | NES | 1 | SMTC configuration for adapted SSB |  | LGE (Han Cha) |  | V016 | PropReject |

[Description]: According to the section 5.5.2.10, UE setups a new SMTC for adapted SSB instead of maintaining the legacy SMTC. It means that the UE performs measurements on the intra-frequency neighbour cell based on the new SMTC as well as on adapted SSB of the serving cell. This implementation violates RAN2 and RAN4 agreements. Related RAN2 and RAN4 agreements are listed as follows:

RAN2#129 - RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. RAN4#114bis - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply.

According to the above agreements, UE shall setup an additional SMTC to perform measurements on adapted SSB and maintain the legacy SMTC to perform measurements on intra-frequency neighbour cell.

[Proposed Change]: SMTC for adapted SSB shall be an addtioanl SMTC. The text proposal is as below:

If smtcy-SSBAdapt is present, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) according to smtc1 for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by ssb-periodicityServingCell; the UE shall setup an additional SMTC according to the first SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the first adaptive SSB periodicity in adap-SSB-BurstPeriodicityList; the UE shall setup an additional SMTC according to the second SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the second adaptive SSB periodicity in adap-SSB-BurstPeriodicityList.

[Comments]:

[Ericsson] See comment to L201 and note that the text says for servingcell measurements.

[Apple] Agree with Ericsson. We don’t see the need to add “an additional”.

[Rapporteur]: Similar to the RIL above, the proposed change does not seem to be essential.

# X201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X201 | NES | 1 | Serving cell MO handling for OD-SSB | R2-25xxxxx | Xiaomi (Li Zhao) |  | V003 | ToDo |

[Description]: The existing text in 5.5.3.1 still needs to be further polished to make it more readable and solve the conflict between the level 2 “absoluteFrequencySSB is not configured” and level 3 “and absoluteFrequencySSB is configured in ServingCellConfigCommon”

[Proposed Change]: RAN2 to discuss and agree the following text.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the OD-SSB-Config is not configured:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO, and absoluteFrequencySSB is configured in ServingCellConfigCommon:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is not configured, or:

2>	if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated, or:

2>	if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is not activated:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

2>	else if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is activated:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO-OD:

4> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4> derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the OD-SSB-Config is not configured:

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO, and absoluteFrequencySSB is configured in ServingCellConfigCommon:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is not configured, or:

2>	if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated, or:

2>	if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is not activated:

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

2>	else if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is activated:

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO-OD:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>  if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO-OD:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

[Comments]: [OPPO] if “and absoluteFrequencySSB is configured in ServingCellConfigCommon” is the main concern, it seems we should not relocate the bullets for “absoluteFrequencySSB are configured”, but just the one of “if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated” should be relocated, but that should rely on servingCellMO-OD so should combine with the branch of servingCellMO-OD? [OPPO2] After offline with Xiaomi, it seems the intention was to mandate using servingCellMO (rather than servingCellMO-OD) for the SSB-less case, i.e., does not pursue the case where the ssbFrequency in servingCellMO is different from od-ssb-absoluteFrequency. If so, OK to rely on servingCellMO for all SSB-less case, but good to clarify it using R2 conclusion to make it more explicit.

[Sharp]: “absoluteFrequencySSB is configured in ServingCellConfigCommon” and other conditions can be moved to the lower level to avoid too many duplicate texts:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO, and:

4> if the OD-SSB-Config is not configured and absoluteFrequencySSB is configured in ServingCellConfigCommon, or:

4>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is not configured, or:

4>	if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated, or:

4>	if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is not activated:

<Legacy behavior>

[Nokia] It seems procedural text does not work very well. So we would think we should have contributions from companies for coming meeting to consider how the procedural text should be written. It looks like the current text is missing the scenario when serving cell has only CSI-RS measurement in the MO and then we have also OD-SSB measurements.

[Xiaomi]: Regarding OPPO’s comments, I think either relocate the bullets for “absoluteFrequencySSB are configured” or not can work, no strong view. Regarding the proposed change by Sharp, I think the conditions to utilize servingcellMO should be in an upper level instead of a lower level, otherwise, servingcellMO will always be utilized.

[Ericsson] Agree with Nokia. See also E023, E024, we need also RAN2 conlcusions related to Case1. Is it not so that od-ssb MO or od-ssb smtc is used only when od-ssb is activated? Would it not be simpler to have those configured in od-ssb and activated when od-ssb is active and that’s it? Which case is missing then?

[Xiaomi] “od-ssb smtc is used only when od-ssb is activated” is not always the case. For case 2 (AO-SSB and OD-SSB) when OD-SSB and AO-SSB having the same frequency, no matter if OD-SSB is activated or not, servingcellMO is used (legacy SMTC), for case 2 when OD-SSB and AO-SSB having different frequency, od-ssb smtc is used only when od-ssb is activated.

[Apple] We agree with this change. We understand that the intention is that the following 3 cases can’t share legacy behavior under “absoluteFrequencySSB is configured in ServingCellConfigCommon” (which was introduced in Rel-18 SSB-less SCell to handle the scenario that ServingCellMO is configured for an SCell without associated with SSB):

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is not configured (i.e. AO-SSB and OD-SSB in same frequency), or:

2>	if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated (OD-SSB case 1, i.e. without AO-SSB), or:

2>	if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is not activated (i.e. AO-SSB and OD-SSB in different frequency)

We also think it is necessary to clarify the following aspects:

- “SSB-less SCell” need to be updated to: the serving cell is associated with neither AO-SSB nor OD-SSB.

- Clarify that ServingCellMO is mandatory present for OD-SSB case 1 (i.e. ssbFrequency is configured, and its value is the same as either the absoluteFrequencySSB or od-ssb-absoluteFrequency-r19 in SCellConfig).

[Rapporteur] This requires further discussion in the next meeting.

[Huawei] Below is our understanding of whether/which servingCellMO is used for deriving serving cell measurement results. For clarity, we provide our suggested TP in [H131].

| absoluteFrequencySSB | OD-SSB-r19 |  | Which servingCellMO is used for serving cell measurements |
| --- | --- | --- | --- |
| absent | absent | SSB-less | None |
| absent | present | SSB-less + on-demand SSB | Legacy servingCellMO (Even though there is only one servingCellMO, OD-SSB is still measured only when it is transmitted) |
| present | absent | AO-SSB | Legacy servingCellMO |
| present | present | OD-SSB + AO-SSB | When od-ssb-absoluteFrequency is configured (i.e., AO-SSB and OD-SSB are inter-frequency): Legacy servingCellMO when OD-SSB is not activated, servingCellMO-OD when OD-SSB is activated When od-ssb-absoluteFrequency is not configured (i.e., AO-SSB and OD-SSB are intra-frequency): Legacy servingCellMO |

# S029

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S029 | NES | 2 | neighbor cell measurements | R2-25xxxxx | Anil Agiwal (Samsung) |  | V027 | PropReject |

[Description]:

For SCell change, measurement object for frequency of a configured/activated SCell and measurement report for event triggered reporting for event A6 can be configured. A measurement configuration identified by a measurement identity is configured for this measurement object and measurement report. For this measurement configuration, UE measures the SCell corresponding to frequency in the measurement object; UE also detects and measures neighboring cells on the frequency in the measurement object. SCell measurements and neighbor cell measurements are then used to evaluate whether event A6 is met or not.

The issue is that if SCell is not configured with always on SSBs and is configured with on demand SSB, SCell measurements cannot be performed until on demand SSB is activated but the UE continues to measure the neighbor cells on the frequency of this SCell as measurement configuration include measurement object corresponding to frequency of this SCell. This leads to unnecessary energy consumption in the UE.

[Proposed Change]: for a measObject (in the measurement identity list), if a SCell corresponding to the frequency of the measObject is configured with on demand SSB and always on/periodic SSB is not configured for the SCell and on demand SSB is not activated and this measObject is associated only with a measurement reporting of a type/event which needs measurements of this SCell: UE ignore the measObject and do not measure neighbor cells according to measObject.

[Comments]:

[Rapporteur] This is not an ASN.1 review related discussion, it is an optimization that requires a separate discussion driven by a Tdoc.

[Samsung]: This is an RRC issue related to procedural text and all RRC issues are discussed as part of ASN.1 review. There is no separate discussion on RRC issues which does not impact ASN.1. We will submit Tdoc.

# Z101

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z101 | NES | 1 | Clarification on the description of servingCellMO-OD |  | ZTE (Gao Yuan) |  | V028 | ToDo |

[Description]: In clause 5.5.3.1, Level 1 only covers performing SINR, RSRP and RSRQ measurements for each serving cell configured with servingCellMO; however, servingCellMO-OD shall also be included.

[Proposed Change]: Suggest to:

- whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO or servingCellMO-OD is configured as follows:

1>	for each serving cell for which servingCellMO or servingCellMO-OD is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

[Comments]:

[Apple] It is not correct. The measurement on servingCellMO-OD is not mandatory but only when switching to use it (in case of different frequency between AO-SSB and OD-SSB).

[Rapporteur] This requires further discussion in the next meeting.

# H131

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H131 | NES | 1 | Serving cell measurements based on legacy servingCellMO/ new servingCellMO-OD | R2-25xxxxx | Huawei (Lili) |  | V035 |  |

[Description]:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the OD-SSB-Config is not configured, or:  // SSB-less SCell, or AO-SSB based SCell

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is not configured, or:  // OD-SSB + AO-SSB based SCell, intra-frequency case

2>	if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated, or:  // OD-SSB + SSB-less SCell (i.e., no AO-SSB), and OD-SSB is activated

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is configured and OD-SSB transmission is not activated:  // OD-SSB + AO-SSB based SCell, inter-frequency case, and OD-SSB is not transmitted

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO, and absoluteFrequencySSB is configured in ServingCellConfigCommon: // for SSB-less SCell, servingCellMO (if configured) is only for neighbour cell measurements, not for serving cell measurements in the following procedures

The current text is a two-layer structure:

- The 4 cases with indentation “2>” are to control whether legacy servingCellMO can be configured. (But the mentioning of whether OD-SSB is activated has something to do with whether servingCellMO can be used for serving cell measurements)

- The paragraph highlighted in yellow is to control whether the configured servingCellMO can be used for serving cell measurements

Without the introduction of OD-SSB, the SSB-less SCell has no SSB-based serving cell measurements. Now that OD-SSB is introduced, the question is: if AO-SSB is not configured (otherwise it is not an “SSB-less SCell”), whether legacy servingCellMO is used for OD-SSB measurements or the new servingCellMO-OD is used for serving cell measurements.

| absoluteFrequencySSB | OD-SSB-r19 |  | Which servingCellMO is used for serving cell measurements |
| --- | --- | --- | --- |
| absent | absent | SSB-less | None |
| absent | present | SSB-less + on-demand SSB | Legacy servingCellMO (Even though there is only one servingCellMO, OD-SSB is still measured only when it is transmitted) |
| present | absent | AO-SSB | Legacy servingCellMO |
| present | present | OD-SSB + AO-SSB | When od-ssb-absoluteFrequency is configured (i.e., AO-SSB and OD-SSB are inter-frequency): Legacy servingCellMO when OD-SSB is not activated, servingCellMO-OD when OD-SSB is activated When od-ssb-absoluteFrequency is not configured (i.e., AO-SSB and OD-SSB are intra-frequency): Legacy servingCellMO |

Without the introduction of OD-SSB, the SSB-less SCell has no SSB-based serving cell measurements. Now that OD-SSB is introduced, the question is: if AO-SSB is not configured (otherwise it is not an “SSB-less SCell”), whether legacy servingCellMO is used for OD-SSB measurements or the new servingCellMO-OD is used for serving cell measurements.

In our understanding, the simplest way is to clearly indicate in which cases legacy servingCellMO can be used for serving cell measurements, i.e., moving “and absoluteFrequencySSB is configured in ServingCellConfigCommon” to indentation “2>”. Besides, it’s better to use od-ssb rather than OD-SSB-Config because OD-SSB-r19 is the parent IE (corresponding to the field od-ssb-r19) including all the OD-SSB configurations, or we can directly say OD-SSB (referring to the concept, not the field). The proposed changes are as follows:

[Proposed Change]:

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured, or:

2>	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency, or:

2>	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is not activated, or:

2>	if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and OD-SSB transmission is activated:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

2>	else if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is activated, or:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured, or:

2>	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is not configured with od-ssb-absoluteFrequency, or:

2>	if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is not activated, or:

2>	if absoluteFrequencySSB is not configured in ServingCellConfigCommon and od-ssb is configured and OD-SSB transmission is activated:

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

2>	else if absoluteFrequencySSB is configured in ServingCellConfigCommon and od-ssb is configured with od-ssb-absoluteFrequency and OD-SSB transmission is activated, or:

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO-OD:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3

[Comments]:

# Z102

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z102 | NES | 1 | Clarification on the description of rsType |  | ZTE (Gao Yuan) |  | V028 | ToDo |

[Description]: According to the specs, when rsType is set to ssb or csi-rs, the judgment criteria between the Level 3 “if ... contains an rsType set to ssb” and the Level 4 “if ... contains an rsType set to ssb” are duplicated.

[Proposed Change]: Suggest to:

2>	if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is configured and OD-SSB transmission is activated:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4> derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

[Comments]:

[Rapporteur] This requires further discussion in the next meeting.

# V101

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V101 | AIML | 1 | Change to “configuration” to “entry” |  | Boubacar |  | V009 | Agreed |

[Description]: It is better to change to “configuration” to “entry”

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event, or hysteresis as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event, or threshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

[Proposed Change]:

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event, or hysteresis as defined within csi-LoggedMeasurementEventTriggerConfig in an entry configuration in csi-LoggedMeasurementConfigToAddModList for this event).

[Comments]:

[WI CR rapporteur-v022] We changed the status from “ToDo” to “PropAgree”.

# V102

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V102 | AIML | 1 | Change “threshold” to “aboveThreshold” |  | Boubacar |  | V009 | Duplicate |

[Description]: It it better to “threshold” change to “aboveThreshold”

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event, or hysteresis as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event, or threshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

[Proposed Change]:

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event, or hysteresis as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event, or aboveThreshold threshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

[Comments]:

[WI CR rapporteur-v022]: This is a duplicate of N032.

# V103

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V103 | AIML | 1 | Change “configuration” to “entry” |  | Boubacar |  | V009 | Agreed |

[Description]: It is better to change to “configuration” to “entry”

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event, or hysteresis as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event, or threshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

[Proposed Change]:

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event, or hysteresis as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event, or threshold as defined within csi-LoggedMeasurementEventTriggerConfig in an entry configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# N032

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N032 | AIML | 1 | Incorrect mapping of thresh in Section 5.5.4.2 and 5.5.4.3, Event A1 and Event A2, respectively, to the threshold in csi-LoggedMeasurementEventTriggerConfig | N/A | Jerediah Fevold |  | vnnn | Rejected |

[Description]: The mapping of threshold is not complete. The mapping should be to aboveThreshold-r19 since threshold-r19 includes above and below, but both do not apply.

[Proposed Change]:

5.5.4.2

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event, or aboveThreshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

5.5.4.3

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event, or belowThreshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Related ASN.1

threshold-r19                     CHOICE {

aboveThreshold-r19               MeasTriggerQuantity,

belowThreshold-r19               MeasTriggerQuantity

},

[Comments]:

[WI CR rapporteur-v022]: We think both the current and proposed solutions work fine. We can include the proposed change and we changed the status from “ToDo” to “PropAgree”.

[WI CR rapporteur, 2025-10-24]: Status changed to “Rejected” since this change is not needed anymore, after adopting changes based on N028.

# V104

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V104 | AIML | 1 | Change “threshold ” to “belowThreshold” |  | Boubacar |  | V009 | Duplicate |

[Description]: It is better to change “threshold ” to “belowThreshold”

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event, or threshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

[Proposed Change]:

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event, or belowThreshold threshold as defined within csi-LoggedMeasurementEventTriggerConfig in a configuration in csi-LoggedMeasurementConfigToAddModList for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

[Comments]:

[WI CR rapporteur-v022]: This is a duplicate of N032.

# H007

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H007 | AIML | 1 | Logged measurement configuration modification and release | R2-25xxxx | Dawid |  | vnnn | Rejected |

[Description]:

Currently it is possible for the network to provide an updated logged measurement configuration (i.e. reusing the same csi-LoggedMeasurementConfigId as already included in the UE conifguration). This may lead to ambiguity in the collected data, i.e. once it is reported it will be unclear to which configuration this data referred to.

Similarly, the UE currently does not discard the collected data for a certain logging conifguration even when this conifguraiton is relased. If the UE is subsequently configured with a new clogging configuration later on, reusing the ID of the previously released configuration, similar ambiguity exists. Such situation should be avoided.

[Proposed Change]:

It is proposed to clarify that when the UE receives a modified logging conifguration or releases a logging configuration, the discards the logged data related to the modified/released logging configuration. This way the ambiguity mentioned above can be avoided.

#### 5.5x.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	discard any logged measurement entries included in VarCSI-LogMeasReport for this csi-LoggedMeasurementConfigId;

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

2>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

3>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

3>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCN and PCI of the serving cell;

2>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

2>	perform measurements logging as specified in 5.5x.3.2.

### 5.5x.2	Release of Network-Side Logged Measurement Configuration

#### 5.5x.2.1	General

The purpose of this procedure is to release the logged measurement configuration for network-side data collection.

#### 5.5x.2.2	Initiation

Upon receiving csi-LoggedMeasurementConfigToReleaseList, the UE shall:

1>	for each csi-LoggedMeasurementConfigId included in csi-LoggedMeasurementConfigToReleaseList associated with a serving cell:

2>	if the current UE configuration for the associated serving cell includes a CSI logged measurement configuration with the associated csi-LoggedMeasurementConfigId:

3>	discard any logged measurement entries included in VarCSI-LogMeasReport for this csi-LoggedMeasurementConfigId;

3>	release the concerned CSI logged measurement configuration.

[Comments]:

[WI CR rapporteur-v022] We suggest that companies discuss this RIL in Tdocs, since the UE behaviour would be somewhat modified.

# Z004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z004 | AIML | Class 1 | The timing point of logging the cell Id and configuration Id for the data collection | R2-25xxxx | Fei |  | V015 | Agreed |

[Description]:

In the current description, the logging of cell Id information and configuration Id information at the timing point when UE receiving the configuration, please see below:

#### 5.5x.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

2>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

3>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

3>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCN, AIML and PCI of the serving cell;

2>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

2>	perform measurements logging as specified in 5.5x.3.2.

It is not workable for the case that UE receiving the data logging configuration when AS buffer has been full since, at that moment, UE cannot establish any entries in the AS buffer.

[Proposed Change]:

It is proposed that UE to add new entries when logging the first piece of entry instead of receiving the logging configuration , that means, this can be captured in 5.5x.3.2,please see below:

#### 5.5x.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

2>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

3>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

3>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCN and PCI of the serving cell;

2>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

2>	perform measurements logging as specified in 5.5x.3.2.

### 5.5x.2	Release of Network-Side Logged Measurement Configuration

#### 5.5x.2.1	General

The purpose of this procedure is to release the logged measurement configuration for network-side data collection.

#### 5.5x.2.2	Initiation

Upon receiving csi-LoggedMeasurementConfigToReleaseList, the UE shall:

1>	for each CSI logged measurement configuration ID included in csi-LoggedMeasurementConfigToReleaseList associated with a serving cell:

2>	if the current UE configuration for the associated serving cell includes a CSI logged measurement configuration with the associated CSI logged measurement configuration ID:

3>	release the CSI logged measurement configuration.

### 5.5x.3	Measurements logging

#### 5.5x.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_CONNECTED that has a logged measurement configuration for network-side data collection.

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer for network-side data collection is not full:

3>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-			LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

4>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

4>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if 				available. If the CGI is not available for that cell, set cellId to the ARFCN and PCI of the serving cell;

3>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-							LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

[Comments]:

[WI CR rapporteur-v022]: It seems the proposed change is addressing bad network implementations, where the network would send a logging configuration although it knows that the UE’s buffer is full, since the UE reports this to the NW. In our view, both the current procedural text and the proposed change work, so we propose that this is discussed in a Tdoc, especially since it may also have some relation to J008/J009.

# C075

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C075 | AIML | 1 | ARFCN and PCI |  | Tangxun |  | V003 | Agreed |

[Description]: “ARFCN and PCI” should be replaced by “physical cell identity and carrier frequency”.

[Proposed Change]: update the procedural text as below:

3>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the physical cell identity and carrier frequency of the serving cell;

[Comments]:

[Ericsson-v022]: We agree with the intention of the proposal and we think also “CGI” should be updated, to be aligned with legacy cases, as follows:

3>	set cellId to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received;

[WI CR rapporteur-v031]: We changed the status from “ToDo” to “PropAgree”.

# H009

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H009 | AIML | 1 | Interactions with PHY for NW-side data collection | R2-25xxxx | Dawid |  | V017 | ToDo |

[Description]:

This issue is related to an LS which RAN2 sent to RAN1 in R2-2506545. In particular, it is currently unclear whether any PHY-RRC ionteractions need to be captured in RRC and in RAN1 specifications. Currenlty there are two possibilities for NW-side data collection configurations:

- Periodic logging

- L3 event-based logging (either based on A1 or A2 event, i.e. L3-cell RSRP>threshold or L3-cell RSRP<threshold

In our understanding, the responsibilities between the layers should be as follows:

- PHY layer specifications should capture that the UE performs measurements and generates L1-RSRP/CRI based on data collection configuration in CSI-LoggedMeasurementConfig

- PHY layer provides the generated measurement results to the RRC based on the configured periodicity

  - For periodic logging, PHY layer should perform measurements as soon as the conifguration is provided to the UE and should provide the results to RRC according to the configured periodicity

  - For event-based logging, PHY layer should only perform measurements and provide them to RRC when event conditions are met

- RRC should be responsible for logging the measurements as provided by PHY layers and then for providing them to the network etc.

In my understandning, the RRC part of the procedure should be modified as below. PHY layer specs should then capture that the UE peforms measurements and provides them to upper layers, upon being instructed by upper layers to do so (which is pending RAN1 discussion and actions based on RAN2 LS).

[Proposed Change]:

### 5.5x.3	Measurements logging

#### 5.5x.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_CONNECTED that has a logged measurement configuration for network-side data collection.

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer for network-side data collection is not full:

3>	instruct lower layers to perform the measurements at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	perform logging of the measurement results provided by lower layers;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	instruct lower layers to perform measurements at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

4>	perform logging of the measurement received from lower layers;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	instruct lower layers not to perform measurements for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

[Comments]:

[WI CR rapporteur-v022]: The same issue is raised in RIL Z005 with slightly different proposed changes, so we suggest to discuss this issue in a Tdoc, together with Z005.

# B201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B201 | AIML | 1 | Terminlogy buffer vs. memory |  | Congchi Zhang |  | V011 | Agreed |

[Description]: Mixed use of “memory” and “buffer”

As raised also over email, we have been using "buffer" during our WI discussion. On the other hand, when it comes to spec terminology w.r.t logging, we notice the term "memory" is actually used in MDT/QoE description. In legacy, “buffer” is normally used when it’s relevant to a protocol layer operation.

Using “access stratum buffer” is another option, but since we already have “memory” in legacy spec..

[Proposed Change]:

Overall, to be precise and consistent with legacy spec wording, suggest to use “memory” instead of “buffer” when it’s relevant to data collection, although it will impact quite many places.

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# Z005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z005 | AIML | Class 1 | Start/stop performing L1 measurement in Logged NW side data collection | R2-25xxxx | Fei |  | V015 | ToDo |

[Description]:

In the current description of text procedure for performing logging, it only describes UE behaviour regrading logging or not logging , please see below:

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer, AIML for network-side data collection is not full:

3>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

Actually, this procedure need have the inter-layer interaction when UE performing the logging or stop logging: for example, to indicate the lower layers to stop the L1 measurement, or start the L1 measurement, otherwise, PHY layer need always perform the Layer 1 measurement no matter whether higher layer to log the data or not, this is really power-consuming.

[Proposed Change]:

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer, AIML for network-side data collection is not full:

3> instruct lower layer to start the L1 measurement in accordance with the corresponding CSI logged measurement configuration as specified in TS 38.214 [19];

3>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4> instruct lower layer to start the L1 measurement in accordance with the corresponding CSI logged measurement configuration as specified in TS 38.214 [19];

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4> instruct lower layer to stop the L1 measurement in accordance with the corresponding CSI logged measurement configuration as specified in TS 38.214 [19];

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, instruct lower layer to stop the L1 measurement as specified in TS 38.214 [19],stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, instruct lower layer to start the L1 measurement as specified in TS 38.214 [19], resume logging.

[Comments]:

[WI CR rapporteur-v022]: We agree that the proposed changes are a possible solution to allow the UE to start/stop measurements and not only start/stop logging. However, we think this issue should be discussed in Tdocs, since it may have implications in the UE behaviour and we may also need to wait for RAN1 input.

# S043

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S043 | AIML | 1 | Correction on NSDC resource measurement and logging |  | Seung-Beom |  | v026 | Agreed |

[Description]: NSDC (NW-side data collection) configuration indicates one CSI-ResourceConfig.

#### – CSI-ResourceConfig The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet. CSI-ResourceConfig information element -- ASN1START -- TAG-CSI-RESOURCECONFIG-START CSI-ResourceConfig ::= SEQUENCE { csi-ResourceConfigId CSI-ResourceConfigId, csi-RS-ResourceSetList CHOICE { nzp-CSI-RS-SSB SEQUENCE { nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId OPTIONAL -- Need R }, csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId }, bwp-Id BWP-Id, resourceType ENUMERATED { aperiodic, semiPersistent, periodic }, ..., [[ csi-SSB-ResourceSetListExt-r17 CSI-SSB-ResourceSetId OPTIONAL -- Need R ]] } -- TAG-CSI-RESOURCECONFIG-STOP -- ASN1STOP

It could include “both” or “either” SSB and CSI-RS resource set. However, according to the current text, UE always sets “both” csi-RS-MeasResultList and SSB-MeasResultList. If CSI-ResourceConfig is includes “either” SSB or CSI-RS, UE should set “either” csi-RS-MeasResultList and SSB-MeasResultList.

[Proposed Change]:

### 5.5x.3 Measurements logging #### 5.5x.3.2 Initiation The UE shall: <…> 2> when performing the logging: 3> for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport: 4> set the csi-RS-MeasResultList and/or SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities if received from the lower layers;

[Comments]:

[WI CR rapporteur-v031]: We changed the status from “ToDo” to “PropAgree”.

# Z011

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z011 | AIML | 1 | The scenario where TimeGap indication presence |  | Fei |  | v027 | Rejected |

[Description]:

In the current version of specification, the time gap indication agreed to have for the BM case 2, please see below agreements

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

In this agreement, the time gap indication is just needed for the BM case 2 as the continuous data collection is critical for training the NW side model of BM case 2. However, in the current text procedure for data logging, the time gap indication is mandatory for both use cases which seems not necessary:

#### 5.5x.3.2 Initiation The UE shall: 1> for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList: 2> if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer[for network-side data collection is not full: 3> perform the logging at regular time intervals,[ according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present; 2> if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full: 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger: 4> perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present; 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger: 4> stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList; 2> when performing the logging: 3> for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport: 4> set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers; 4> if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured): 5> set the timeGap to true; 2> when the memory reserved for the logged measurement information for data collection becomes full, stop logging; 2> when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

[Proposed Change]:

#### –	CSI-LoggedMeasurementConfig

The IE CSI-LoggedMeasurementConfig is used to configure a CSI logged measurement configuration. It defines a group of one or more CSI resources for which the UE logs the associated L1 radio measurements.

CSI-LoggedMeasurementConfig information element

-- ASN1START

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-START

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

loggingPeriodicity-r19                    ENUMERATED {n2, n3, n4, n5, spare4, spare3, spare2, spare1}            OPTIONAL,  -- Need M[RIL]: B203, AIML

csi-LoggedMeasurementEventTriggerConfig-r19         CSI-LoggedMeasurementEventTriggerConfig-r19                OPTIONAL,  -- Need R

enableTimeGap-r19                               ENUMERATED {True}                                                OPTIONAL,  -- Need R

...

}

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

threshold-r19                     CHOICE {

aboveThreshold-r19               MeasTriggerQuantity,

belowThreshold-r19               MeasTriggerQuantity

},

hysteresis                        Hysteresis,

timeToTrigger                     TimeToTrigger,

...

}

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-STOP

-- ASN1STOP

CSI-LoggedMeasurementConfig field descriptions

csi-LoggedMeasurementConfigId This field indicates the instance of CSI-LoggedMeasurementConfig.

csi-LoggedResourceConfig Resources in which the UE performs channel measurement whose associated measurement results are logged by the UE. The csi-LoggedResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources.

csi-LoggedMeasurementEventTriggerConfigThis field is used to configure the UE with event-triggered measurement logging. If this field is included and threshold is set to aboveThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is included and threshold is set to belowThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. If this field is not included, the UE starts the measurement logging according to csi-LoggedResourceConfig upon reception.

loggingPeriodicity The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2nd occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3rd occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer[for network-side data collection is not full:

3>	perform the logging at regular time intervals,[ according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the enableTimeGap is set to true in the associated csi-LoggedMeasurementConfig and the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

[Comments]:

[WI CR rapporteur-v031]: We changed the status from “ToDo” to “PropReject”. In the rapporteur’s view, this is a misinterpretation of the agreement quited by ZTE above. The agreement was not intended only for BM case 2. Instead, “For temporal domain” was intended to refer to the moments in time when the UE performs logging. Please see also the other related RAN2 agreements, where there is not restriction on the BM use case (spatial or temporal).

RAN2#130: “Data is collected on per data logging configuration basis and UE indicates data logging configuration ID.    An indication of the “gap” is needed.  “Gap” is time interval larger than the configured logging periodicity.    FFS if timestamp and relative time stamp for each group is needed per “group”. ”

RAN2#131: “The UE will indicate the presence of a gap (i.e. there will be no indication on the length of gap or time instance, etc).  Rapporteur will suggest a way to implemented as part of the RRC review”

# L002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L002 | AIML | 1 | Redundancy in Buffer Full Handling for CSI Logged Measurement |  | Soo Kim |  | V020 | ToDo |

[Description]:

Looking at this, the clause “if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full” already implies that logging will resume once the buffer full condition is resolved, so the very last part (explicit resume operation) seems redundant.

[Proposed Change]:

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer for network-side data collection is not full:

3>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

[Comments]:

# X551

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X551 | SONMDT | 1 | Incomplete procedure text |  | Xiaomi (Shuai) |  | V001 | Rejected |

[Description]: In clause 5.5a.3.2, the procedure is not clear due to there are two scearios will leading to measurement logging suspending, which needs to be clarified to make the procedure clear.

[Proposed Change]: As below changes, the first “if..else” judgment sentence is restricted to IDC scenario, so that when UE cannot get the location information, UE will go to second “if..else” judgment sentence.

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended due to IDC problems:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if AreaConfigurationNTN-List is included in VarLogMeasConfig:

2> if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List; or

2>	if location information is not available:

3> skip the execution of the remainder of clause 5.5a.3.2 for the current logging interval (i.e. do not perform measurement logging for this interval);

[Comments]:

[Samsung] Not sure if this change is needed. We do suspend only due to IDC problems, and in case location information is not available, it is “skip” for an interval.

[Rapporteur]: agree with Samsung view, suspension/resume happens only in case of IDC, so the change is rejected.

[Rapporteur]: This RIL has been discussed again in onlie session in RAN2#131bis and has been rejected given the lack of support from other companies.

# E046

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E046 | SONMDT | 1 | Missing UE capability check |  | Ali Parichehreh |  | V013 | Agreed |

[Description]: UE capability checking is missed in geographical area scope checking.

In chapter 5.5a.3.2, UE capability should be checked before performing MDT based on geographical area scope.

[Proposed Change]:

1>	if AreaConfigurationNTN-List is included in VarLogMeasConfig and if the UE supports geographic area scope checking for logged MDT:

2> if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List; or

2>	if location information is not available:

3> skip the execution of the remainder of clause 5.5a.3.2 for the current logging interval (i.e. do not perform measurement logging for this interval);

[Comments]:

Rapporture agrees with the RIL and implemented the change in the drafted CR according to the RIL.

# H302

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H302 | SONMDT | 1 | Logged MDT for NTN |  | Jun Chen |  | V004 | Rejected |

[Description]: In section 5.5a.3.2, the UE checks the logging internal when doing logging, and the interval can be put in the condition rather than the UE behaviours.

1>	if AreaConfigurationNTN-List is included in VarLogMeasConfig:

2> if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List; or

2>	if location information is not available:

3> skip the execution of the remainder of clause 5.5a.3.2 for the current logging interval (i.e. do not perform measurement logging for this interval);

[Proposed Change]: Suggested changes as shown as below:

1>	if AreaConfigurationNTN-List is included in VarLogMeasConfig when performing the logging on a logging interval:

for

2> if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List; or

2>	if location information is not available:

3> skip the execution of the remainder of clause 5.5a.3.2 for the current logging interval (i.e. do not perform measurement logging for this the logging interval);

[Comments]:

[Rapporteur]: In rapporteur’s understanding in a logging session for which the UE received a logging configuration, the UE either received AreaConfigurationNTN-List or not. The first change implies that the AreaConfigurationNTN-List might exist in one logging interval and might be absent in the next logging interval. Hence we think the legacy text is ok and change is not needed.

# S019

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S019 | SONMDT | 1 | (Pre)Condition for logging NSAG information need to be updated. |  | Samsung (Aby) |  | V006 | Agreed |

[Description]: In section 5.5a.3.2

- “If the UE was configured with slice based cell reselection” is ambiguous. For e.g. does it mean that UE received NSAG priority and slice list for cell reselection from NAS? Or does it mean that UE received NSAG priority and slice list for cell reselection from NAS and had sliceInfoList from SIB16 or sliiceInfoListDedicated from RRC? TS 38.304, doesn’t specify when the UE is configured with slice base cell reselection, i.e. the above two cases clearly.

“if the UE has performed cell reselection using reselection priorities for slice-based cell reselection” may be used instead of “If the UE was configured with slice based cell reselection”

- UE was not able to “select” a suitable cell according to TS 38.304 is not correct. Cell selection is a different process in TS 38.304 and the NSAG information is about cell reselection. UE needs to log the NSAG information if the UE has not reselected to a cell supporting highest priority NSAG except when the UE is already in the cell which supports highest priority NSAG. “Camp on” a suitable cell may be used instead of select a suitable cell, which cover these cases.

[Proposed Change]: Suggested changes as shown as below:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the UE has performed cell reselection using reselection priorities for slice-based cell reselection and was not able to camp on a suitable cell that supports the NSAG ID with the highest priority  (as specified in TS 38.304 [20]) during the last logging interval:

<unchanged>

4>	if the UE has performed cell reselection using reselection priorities for slice-based cell reselection  and was not able to camp on a suitable cell that supports the NSAG ID with the highest priority (as specified in TS 38.304 [20]) during the last logging interval:

[Comments]:

[Rapporteur]: agree with the RIL. Captured in the draft CR, thanks!

# S020

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S020 | SONMDT | 1 | “highest priority” is not clear | R2-25xxxx | Samsung (Aby) |  | V006 | Rejected |

[Description]: In section 5.5a.3.2

- It is not clear what is the NSAGID with the highest priority in the statement “and was not able to camp on a suitable cell that supports the NSAG ID with the highest priority  (as specified in TS 38.304 [20])”.  Does the UE log NSAG information if there is no frequency which supports the highest priority NSAG? Or Do we consider the frequencies which support highest priority NSAG which is also configured by RRC (either SIB16 or RRCRelease). From RAN2 agreement “does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG”, NSAG configured by NAS and RRC seems more suitable. But we think this needs to be confirmed.

- Similar to above, it needs to be clarified what is the highest priority NSAG in the following also:  set the nsag-ID to the NSAG ID with the highest priority. Based on the discussions in RAN2 where companies explained that this reporting is to get information which is not available with the UE, our understaning is that it is highest priority NSAG configured by NAS.

- Since the NSAG priorities from the NAS are received only during registeration, UE logs the same value every time UE logs the highest priority NSAG. We see there is no reason to repeatedly log the same information many many times in the report.. So we suggest to log NSAG information if it is not logged in the report.

[Proposed Change]:

We Suggest one of the below based on common understanding:

Option 1:

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority received from the NAS  (as specified in TS 38.304 [20]) during the last logging interval:

5>	if the NSAG ID with the highest priority received from the NAS is not included in any LogMeasInfo in the logged measurement report:

6>	set the nsag-ID to the NSAG ID with the highest priority received from the NAS ;

6>	set the reselectedCellId to the cell UE reselected after failure in attempt to select a suitable cell that support the NSAG ID with the highest priority before transition to any cell selection state;

<unchanged part>

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority received from the NAS (as specified in TS 38.304 [20]) during the last logging interval:

5>	if the NSAG ID with the highest priority received from the NAS is not included in any LogMeasInfo in the logged measurement report:

6>	set the nsag-ID to the NSAG ID with the highest priority received from the NAS;

Option 2

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority received from the NAS and present in the used FreqPriorityListDedicatedSlicing or FreqPriorityListSlicing (as specified in TS 38.304 [20]) during the last logging interval:

5>	if the NSAG ID with the highest priority received from the NAS is not included in any LogMeasInfo in the logged measurement report:

6>	set the nsag-ID to the NSAG ID with the highest priority received from the NAS ;

6>	set the reselectedCellId to the cell UE reselected after failure in attempt to select a suitable cell that support the NSAG ID with the highest priority before transition to any cell selection state;

<unchanged part>

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority received from the NAS and present in the used FreqPriorityListDedicatedSlicing or FreqPriorityListSlicing (as specified in TS 38.304 [20]) during the last logging interval:

5>	if the NSAG ID with the highest priority received from the NAS is not included in any LogMeasInfo in the logged measurement report:

6>	set the nsag-ID to the NSAG ID with the highest priority received from the NAS;

[Comments]:

[Rapporteur]: Rapporteur thinks better to discuss this RIL in online session. Please bring a contribution so we discuss it in the next meeting.

[Rapporteur]: This was not agreed in the online session in RAN2#131bis, so the RIL is rejected.

# X552

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X552 | SONMDT | 1 | Incomplete procedure text and field description |  | Xiaomi (Shuai) |  | V001 | Agreed |

[Description]: In clause 5.5a.3.2, NSAG ID is used but not even defined or referred, thus need to add the reference for it.

[Proposed Change]: As below changes, add the NAS reference for NSAG ID in procedure text and field description part.

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority (as specified in TS 38.304 [20]) during the last logging interval:

5>	set the nsag-ID to the NSAG ID with the highest priority provided by upper layer (see TS 24.501 [23]);

5>	set the reselectedCellId to the cell UE reselected after failure in attempt to select a suitable cell that support the NSAG ID with the highest priority before transition to any cell selection state;

4>	if the reportType is set to eventTriggered in the VarLogMeasConfig; and

4>	if the RPLMN at the time of entering the any cell selection state is included in plmn-IdentityList stored in VarLogMeasReport or if the registered SNPN identity at the time of entering the any cell selection state is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the last suitable cell that the UE was camping on is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last suitable cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last suitable cell the UE was camping on;

4>	else if the reportType is set to periodical in the VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority (as specified in TS 38.304 [20]) during the last logging interval:

5>	set the nsag-ID to the NSAG ID with the highest priority provided by upper layer (see TS 24.501 [23]);

5>	set the reselectedCellId to the cell UE reselected after failure in attempt to select a suitable cell that support the NSAG ID with the highest priority, if it is different from servCellIdentity;

LogMeasReport field descriptions

nsag-ID Indicates the NSAG ID with the highest priority provided by upper layer (see TS 24.501 [23]), for the UE that was configured with slice-based cell reselection and was not able to perform a cell reselection to a cell asscoiated with the NSAG with highest priority (as specified in TS 38.304 [20]).

relativeTimeStamp Indicates the time of logging measurement results, measured relative to the absoluteTimeStamp. Value in seconds.

reselectedCellId Indicates the cell that does not support the NSAG ID with highest priority provided by upper layer (see TS 24.501 [23]), for the UE that was configured with slice-based cell reselection and was not able to perform a cell reselection to a cell asscoiated with NSAG with the highest priority (as specified in TS 38.304 [20]).

[Comments]:

[Samsung] We also have a RIL with respect to this and think this needs to be discussed.

[Rapporteur] agree with the proposed change, it is captured in the draft CR. The RIL is closed.

# C057

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C057 | SONMDT | 1 | reselectedCellId doesn’t exist in any cell selection state | R2-25xxxx | Tangxun |  | V002 | Rejected |

[Description]: according to RAN2 agreement, in case of transition to any cell selection state, there is no reselected cell to log

- RAN2 includes the following information along with NSAG information in the logged MDT report: Cell information of the cell that the UE was in when it was not able to find its highest prio NSAG (if available, it may not be available if the UE goes to any cell selection state).

[Proposed Change]: remove this bullet 5 as below:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority (as specified in TS 38.304 [20]) during the last logging interval:

5>	set the nsag-ID to the NSAG ID with the highest priority ;

[Comments]:

[Rapporteur]: Rapporteur believes this issue requires discussion as there were different views during the past review phase. The reason is that the UE may fails to camp on a suitable cell according to the NSAG priority and camp on a differet suitable cell and then at the time of logging the MDT sampe goes to any cell selection state; thus, the reselectedCellId is still available to log.

After discussion in RAN2131bis we concluded the following which means UE logs the reselected cell for the last failure occured pior to logging the current MDT sample.

The UE shall log the cell the UE ended up camping on for the last failure occurrence.

So the solution in this RIL is not accepted and hence the RIL is rejected.

# S021

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S021 | SONMDT | 1 | Logging of reselectid | R2-25xxxx | Samsung (Aby) |  | V006 | Rejected |

[Description]: In section 5.5a.3.2

- Similar to NSAG id, use “camped-on” instead of select in “failure in attempt to select a cell”

- UE may contine to camp on the same cell in the logging interval and  move to RRC_CONNECTED state from the camped on cell, and reselection cell id will not be available and need not be logged in this case. In the case of any cell selection state, UE may move to any cell selection state directly from the camped on cell. So we think that cell reselection id needs to be logged if the UE has performed cell reselection in the last logging interval (normal case) ir before performing cell reselection (in the any cell state).Other cases, cellreselectionid will not be available.

- “if it is different from servCellIdentity” is not required. How does the UE reselect to a cell with same servCellIdentity.

[Proposed Change]: Suggested changes as shown as below:

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority (as specified in TS 38.304 [20]) during the last logging interval:

5>	set the nsag-ID to the NSAG ID with the highest priority ;

5>If the UE has performed cell reselection after failure in attempt to select a suitable cell that support the NSAG ID with the highest priority before transition to any cell selection state:

6>	set the reselectedCellId to the cell UE reselected;

<unchanged>

4>	if the UE was configured with slice-based cell reselection and was not able to select a suitable cell that supports the NSAG ID with the highest priority (as specified in TS 38.304 [20]) during the last logging interval:

5>	set the nsag-ID to the NSAG ID with the highest priority ;

5>If the UE has performed cell reselection during the last logging interval after failure in attempt to select a suitable cell that support the NSAG ID with the highest priority :

6>	set the reselectedCellId to the cell UE;

[Comments]:

[Rapporteur]: please bring a contribution so we discuss this in online session.

[Rapporteur]: We discussed this online and the following parts are agreed. Rest of the proposed changes are rejected or captured with a different language. I change the status of this RIL to rejected, to avoid confusion in the future.

=> Use “camped-on” instead of “select” in “failure in attempt to select a cell” and remove “if it is different from servCellIdentity” in the condition for logging reselectedCellId.

=> ReselectedCellId is logged only for Intra-NR reselection.

# Z306

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z306 | SONMDT | 0 | Extra space. | R2-25xxxx | QI Tao |  | V012 | Agreed |

[Description]: extra space found.

[Proposed Change]: remove the double space.

# S018

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S018 | SONMDT | 1 | Logging PCell and PSCell in SCGFailureInformation |  | Samsung (Aby) |  | V006 | Agreed |

[Description]: In section 5.7.3	SCG failure information PCell and PSCell need to be logged when all triggering events of any of condExecutionCond and condExecutionCondPSCell are fulfilled. The else if should be change as if as the previous two conditions also will be true in this case

[Proposed Change]: Suggested changes as shown as below:

1>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in cho-WithCandidateSCGInfoList and set the values as follows:

2>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first;

3>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

2>	else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList is fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled;

3>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the SCG failure;

2>	if all triggering events of any of condExecutionCond and condExecutionCondPSCell are fulfilled:

3>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

3>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

[ZTE]: wondering why the condition of “if all triggering events of any of condExecutionCond and condExecutionCondPSCell are fulfilled” is not reflected in the rlf-report.

[Rapporteur]: agree with the RIL. It is captured in the draft CR.

# C058

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C058 | SONMDT | 1 | SCG RLF | R2-25xxxx | Tangxun |  | V002 | Agreed |

[Description]: “SCG RLF” should be used here instead of “SCG failure”, as SCG failure has a wider coverage, e.g., “failure of SCG reconfiguration with sync, SCG configuration failure for RRC message on SRB3, SCG integrity check failure”

[Proposed Change]: replace “SCG failure” by “SCG RLF”

[Comments]:

[Rapporteur]: Rapporteur beleives the proposed change requires discussion. Replacing with SCG RLF would exclude the SCG failure during PSCell change or addition. An alternative proposal is to add explanation in braces, e.g., (radio link failure at PSCell or PSCell change or addition failure)

[Rapporteur]: After discussion in RAN2#131bis this RIL is agreed.

# E048

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E048 | SONMDT | 1 | MRO for subsequenct CPAC |  | Ali Parichehreh |  | V013 | Agreed |

[Description]: In section 5.7.3, where the SCGFailureInformation message is populated, capability checks for support for SCG Failure for subsequent CPAC are miussing. Further, in the SCGFailureInformation field descriptions in section 6.2.2, the subsequent CPA case is missing from the field description of previousPSCellId, and the subsequent CPAC failure case is missing from the field description of the failedPSCellId.

[Proposed Change]:

Section 5.7.3:

1>	if the UE supports SCG failure for mobility robustness optimization or if the UE supports SCG failure for mobility robustness optimization for subsequent CPAC:

2>	if the failureType is set to synchReconfigFailureSCG; or

2>	if the failureType is set to randomAccessProblem and the SCG failure was declared while T304 was running:

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5.

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the target PSCell of the failed PSCell change or failed PSCell addition;

3>	if the failure occurred during a subsequent CPC:

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

3>	else:

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

3>	set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure;

2>	else:

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the PSCell in which the SCG failure was declared;

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

4>	set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure;

4>	if the failure occurred after a subsequent CPC and if the UE supports SCG failure for mobility robustness optimization for subsequent CPAC:

5>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

Section 6.2.2:

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.

previousPSCellId This field indicates the physical cell id and carrier frequency of the cell that is the source PSCell of the last PSCell change. In case of PSCell addition failure or subsequent CPA, this field is absent. In case of subsequent CPC, this field indicates the physical cell identity and carrier frequency of the source PSCell associated to the last executed CPC.

failedPSCellId This field indicates the physical cell id and carrier frequency of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change or failed PSCell addition or failed subsequent CPAC.

timeSCGFailure This field is used to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

[Comments]:

# C059

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C059 | SONMDT | 1 | CPC execution |  | Tangxun |  | V002 | Agreed |

[Description]: in bullet 3, the “failure” only refers to CPC execution failure, but not other failures.

[Proposed Change]: add “execution” as below:

3>	if the failure occurred during a subsequent CPC execution:

[Comments]:

[Rapporteur]: agree

# Z251

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z251 | NTN | 1 | - Descriptions of UAI | No | ZTE (Zhihong) |  | v012 | Duplicate H250 |

[Description]: UE reported information is actually reference location information instead of location information

[Proposed Change]: Update the description as below:

-	the information of the relay UE(s) with which it connects via a non-3GPP connection for MP; or

-	configured grant assistance information for NR sidelink positioning.

-	reference location information for assisted SMTC configuration in RRC_CONNECTED state.

[Comments]:

# X001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X001 | AIML | 1 | Applicability report via UAI |  | Xing |  | V012 | Agreed |

[Description]:

The initial applicability report should be done via RRCReconfigutaionComplete rather than UAI. After the initial report, if applicability doesn’t change, UE shall not trigger UAI upon being configured to report assistance information of applicability.

[Proposed Change]:

#### 5.7.4.2	Initiation

…

A UE capable of providing assistance information related to the applicability of configurations subject to the applicability determination procedure may initiate upon change of the applicability of the configurations subject to the applicability determination procedure. A UE capable of providing assistance information related to the applicability of configurations subject to the applicability determination procedure shall initiate the procedure if it was configured to do so, upon determining that the applicability of a configuration subject to the applicability determination procedure changed from applicable to inapplicable.

[Comments]:

[Ericsson-v022]: We agree with the intention of the change and we think it can be combine with further changes in this paragraph, to also align better with the agreed capabilities. We propose the following alternative change:

A UE capable of applicability reporting and/or its updates (via RRCReconfigurationComplete or via UEAssistanceInformation message may initiate the procedure if it was configured to report the applicability in UEAssistanceInformation message, upon change of the applicability of the configurations subject to the applicability determination procedure. A UE capable of applicability reporting and/or its updates (via RRCReconfigurationComplete or via UEAssistanceInformation message  shall initiate the procedure if it was configured to report the applicability in UEAssistanceInformation message, upon determining that the applicability of a configuration subject to the applicability determination procedure changed from applicable to inapplicable.

[WI CR rapporteur-v031]: We changed the status from “ToDo” to “PropAgree”.

# B202

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B202 | AIML | 1 | Trigger UAI upon applicability change since last report |  | Congchi Zhang |  | V011 | Rejected |

[Description]:

The UAI is trigger if the applicability changes since the last report.

[Proposed Change]:

A UE capable of providing assistance information related to the applicability of configurations subject to the applicability determination procedure may initiate the procedure in several cases, including upon being configured to report assistance information about the applicability of configurations subject to the applicability determination procedure and upon change of the applicability of the configurations subject to the applicability determination procedure since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation). A UE capable of providing assistance information related to the applicability of configurations subject to the applicability determination procedure shall initiate the procedure if it was configured to do so, upon determining that the applicability of a configuration subject to the applicability determination procedure changed from applicable to inapplicable since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation).

[Comments]:

[WI CR rapporteur-v022]: We think the proposed change is not needed, since the same text as the one suggested to be added is captured later in the same clause (5.7.4.2):

1>	if configured to report assistance information about the applicability of configurations subject to the applicability determination procedure:

2>	if the applicability status of configurations subject to the applicability determination procedure has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation):

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report assistance information about the applicability of configurations subject to the applicability determination procedure;

# E039

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E039 | AIML | 1 | Update UAI procedural text for UE-side data collection in 5.7.4.2 to match corresponding capability |  | Ericsson (Andra) |  | V022 | ToDo |

[Description]: The following procedural text in clause 5.7.4.2 for UE-side data collection should be aligned with the capability description for UE-side data collection, once this is finalized.

“A UE capable of providing its preference to be configured with or stop being configured with radio resources to perform UE-side data collection may initiate the procedure if it was configured to do so, upon determining that it would like to perform UE-side data collection, or upon determining a list of preferred radio resource configurations for UE-side data collection, or upon determining a list of radio resource configurations for which it prefers to stop UE-side data collection.”

[Proposed Change]: There is no concrete change yet. This needs to be checked again once the capability for UE-side data collection is finalized.

[Comments]:

# X002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X002 | AIML | 1 | UAI trigger |  | Xing |  | V012 | Agreed |

[Description]:

We understand UE shall trigger UAI upon low power, full buffer or buffer threshold reaced. The report trigger is not up to UE implementation.

[Proposed Change]:

#### 5.7.4.2	Initiation

…

A UE capable of logging measurements for network-side data collection shall initiate the procedure if it was configured to do so, upon determining that it has entered a low power state, or upon determining that the buffer reserved for the logging of radio measurements for network-side data collection has become full.

A UE capable of logging measurements for network-side data collection and of providing a data availability indication based on a buffer threshold shall initiate the procedure if it was configured to do so, upon determining that the amount of logged data related to radio measurements for network-side data collection reached a configured buffer threshold.

[Comments]:

[Ericsson-v022]: We agree with the proposed changes, since it is important that the network receives these indications from the UE and they are not a preference of the UE.

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# C005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C005 | NTN | 1 | Clarify the meaning of “can” in the following agreement: - The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations. - The UE can report the N closest reference locations via the RRCReconfigurationComplete message. | Yes, R2-250xxxxx | CATT (Da Wang) |  | v008 | Duplicate S024 |

[Description]: From our perspective, “can” means the UE does not have to report the reference location in the RRCReconfigurationComplete message. However, the following text implies that the UE shall report the N closest reference locations in the RRCReconfigurationComplete message for the first time.

1>	set the content of the RRCReconfigurationComplete message as follows:

[unreleated part is ommitted]

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport;.

We have two options to solve this issue:

- Option 1: restrict the UE have to report the reference locations in the RRCReconfigurationComplete message for the first time, and remove the “upon being configured to do so” triggered condition under the UEAssistanceInformation message.

- Option 2: make the UE report the reference locations in the RRCReconfigurationComplete message as an optional behaviour, and complete the UE behaviour of “upon being configured to do so” triggered condition under the UEAssistanceInformation message.

For simplicity, we can go with the option 1, i.e., restrict that the UE have to report the N closest reference locations in the RRCReconfigurationComplete message for the first time and the UEAssistanceInformation message can be used for the subsequent reports. The spec can be modified as following.

[Proposed Change]: Remove the “upon being configured to do so” for the condition of reporting N closest reference locations in the UEAssistanceInformation message.

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon determining that the closest reference location(s) have changed compared with the last reported values.

[Comments]:

[Samsung]: we share the same view on the issue, as we already mentioned in the CR review but not addressed by the Rapporteur. In our understanding of the agreement, we think Option 2 should be the corresponding UE behaviour, i.e., reporting reference location in RRCReconfigurationComplete message is optional. We propose a change in RIL S024.

[vivo]: we also share a similar view on this issue. If the network may not configure refLocList-r19 via RRC configuration message, the UE may take some additional time to acquire the SIB2 meesage containing reference location list. To aviod delaying the transmission of the RRC reconfiguration complete message, the UE should be able to omit the  location report in the RRC RRC reconfiguration complete message.

[Rapp] As commented above, the UE should already have a SIB2 valid version available, and the UE should know its own location to initiate/maintain connection. Thus, Option1 makes more sense and it captures the idea with the agreement (sparing unnecessary additional messages). Marked as duplicate of S024, please consider the options listed in C005 and Z252 in your contribution.

# Z252

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z252 | NTN | 1 | Triggering condition for reference location report in UAI | Yes, R2-250xxxxx | ZTE (Zhihong) |  | v012 | Duplicate S024 |

[Description]: When discussing the procedure, it is preferred by companies to report first report in RRCReconfigurationComplete message to save one more RRC message for reporting. However there could be the case when refLocList configuration is not provided in RRCReconfiguration but instead UE shall acquire this information from SIB2, which will cause additional delays for UE to compute the closest referenceLocation and put it in RRCReconfigurationComplete message. It is not desired to mandate such beahvior and delay the transmission of RRCReconfigurationComplete message, a compromised way could be leave it to UE implementation whether to report this information in RRCReconfigruationComplete message, if it is the common understanding then the trigger for reference location for UAI report would be update to when configured to do so and UE has not yet report, and upon determining that the closesr reference location(s) have changed...

[Proposed Change]: Update the description as below:

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon being configured to do so and the first report has not been sent, and upon determining that the closest reference location(s) have changed compared with the last reported values

[Comments]:

Similar issue as raised by S024. It needs to be further discussed in the meeting. Thanks for proposing a compromise solution.

# N082

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N082 | NTN | 1 | Considerations on no UE requirements for GNSS | Yes, R2-25xxxxx | Nokia (Jakob) |  | v016 | Rejected |

[Description]: Similar to N081, there are no requirements for the UE to obtain GNSS location at specific intervals, nor should there be a requirement for the UE to obtain a GNSS location simply to continuously test whether the location information has changed. However, current spec may indicate that this is required.

[Proposed Change]: Add a note to state that it is up to UE whether/how to test the change of location in section 5.7.4.2.

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon being configured to do so, and upon determining that the closest reference location(s) have changed compared with the last reported values.

NOTE x:	Further details, e.g. regarding how to determine the location information has changed is up to UE implementation since a UE may not be able to continuously obtain a valid GNSS location e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage.

[Comments]:

[Rapp] Please see answer to N081.

# S038

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S038 | XR | 1 | Checking T346o running status |  | Vinay (Samsung) |  | V07 | PropRej |

[Description]: The timer not running condition should be addressed in bullet 1>, as it is a general condition and it is also consistent description as in the legacy spec

#### 5.7.4.2 Initiation …. 1> if configured to provide its preference for gap occasion cancellation ratio: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.

[Proposed Change]: Revise to have T346o not running condition check as general and aligned with legacy.

#### 5.7.4.2 Initiation …. 1> if configured to provide its preference for gap occasion cancellation ratio and T346o is not running: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.

[Comments]: [Rapp] The first condition, highlighted in yellow above, is that the UAI has not been transmitted before. Then is it still necessary to add the T346o is running condition for a case where the timer is not possible to be running?

# N091

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N091 | XR | 1 | UAI for measurement gap cancellation preference | R2-25xxxxx | Chunli WU (Nokia) |  | V04 | ToDo |

[Description]: Current conditions for UAI triggering allow the UE to send the UAI if it has not sent before or otherwise when the prohibit timer is not running. Considering now that we agreed single rohibit timer while the the UAI can include preference for mutliple measurement gaps, it makes sense to allow UAI to be sent for a newly configured measurement gao without delay even if UAI has been sent for other MG before.

[Proposed Change]: Revise the first condition to allow such case or add the condition to allow UAI for a newly configured measurement gap with preference becomes available:

1> if configured to provide its preference for gap occasion cancellation ratio: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the preference for gap occasion cancellation ratio becomes available for at least one measurement gap configuration for which preference for gap occasion cancellation ratio has not been included in any UEAssistanceInformation message with gapOccasionCancelRatio previously transmitted by the UE; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.

[Comments]:[Rapp] Since this issue has not been discussed before, it could be discussed by the discussion paper submitted to the next R2 meeting.

# E009

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E009 | LPWUS | 1 | Empty UAI message for LP-WUS time offset (RRC-5, MAX-X3) |  | Ericsson (Martin) |  | V002 | Agreed |

[Description]: The legacy rules should apply for the preferred time offset signalled via UAI. This means that when timeOffset-r19 is absent in LPWUS-OffsetPreference-r19, aka the UE sends an “empty” UAI message, that the UE does not have a preference for the LP-WUS time offset. PS: when the UE does not include LPWUS-OffsetPreference-r19 this means that the previous signalled preferred LP-WUS time offset remains valid:

UEAssistanceInformation-v19xx-IEs ::= SEQUENCE {

lpwus-OffsetPreference-r19               LPWUS-OffsetPreference-r19                    OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                      OPTIONAL

}

LPWUS-OffsetPreference-r19 ::=           SEQUENCE {

timeOffset-r19              ENUMERATED {ms5, ms13, ms37}                             OPTIONAL

}

[Proposed Change]: The brackets can be removed, i.e. when the UE does not have a preference for the LP-WUS time offset, then that is also considered a preference, and this preference can be different from an actual preferred LP-WUS time offset previously:

2>	if the UE has a preference on time offset for LP-WUS monitoring of the cell group and the UE did not transmit a UEAssistanceInformation message with lpwus-OffsetPreference for the cell group since it was configured to provide its preference on time offset for LP-WUS monitoring of the cell group for power saving; or

[Comments]:

# Z051

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z051 | LPWUS | 1 | timer T346xx per cell group | R2-25xxxxx | Tao Qi (ZTE) |  | V013 | Agreed |

[Description]: It has agreed that LP-WUS for RRC_CONNECTED state can be configured per cell group, and the lpwus-OffsetPreference can be reported per cell group, the prohibit timer for lpwus-OffsetPreference can should also be per cell group.

[Proposed Change]: Clarify that the prohibit timer for lpwus-OffsetPreference is per cell group, such as:

#### 5.7.4.2	Initiation

//OMITTED THE UNRELATED PART

1>	if configured to provide its preference on time offset for LP-WUS monitoring of a cell group:

2>	if [the UE has a preference on time offset for LP-WUS monitoring of the cell group and] [RIL]: E009 LPWUS the UE did not transmit a UEAssistanceInformation message with lpwus-OffsetPreference for the cell group since it was configured to provide its preference on time offset for LP-WUS monitoring of the cell group for power saving; or

2>	if the current lpwus-OffsetPreference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including lpwus-OffsetPreference for the cell group and timer T346xx associated with the cell group is not running:

3>	start the timer T346xx with the timer value for the cell group set to the lpwus-OffsetPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current lpwus-OffsetPreference.

[Comments]:

# S048

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S048 | AIML | 1 | UAI initiation |  | Youn Heo |  | v026 | Agreed |

[Description]: Preference for data collection should be also initiated when UE want to stop for a configured data collection configuration.

[Proposed Change]:

1> if configured to provide its preference to be configured with radio measurement resources for UE-side data collection: 2> if the UE has a preference to be configured with radio measurement resources to perform UE-side data collection or to stop a configured data collection configuration and did not transmit a UEAssistanceInformation message with dataCollectionPreference since it was configured to provide its preference to be configured with radio measurement resources to perform UE-side data collection; or 2> if the preference to be configured with radio measurement resources to perform UE-side data collection has changed since the last transmission of the UEAssistanceInformation message including dataCollectionPreference: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report the UE preference to be configured with radio measurement resources for UE-side data collection;

[Comments]:

[WI CR rapporteur-v031]: We changed the status from “ToDo” to “PropAgree”.

# S025

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S025 | NTN | 1 | UE first time reports N closest reference locations using UAI. | Yes, R2-250xxxxx | Samsung (Shiyang) |  | v011 | Agreed |

[Description]: As agreed “The UE can report the N closest reference locations via the RRCReconfigurationComplete message.”, if UE has not reported in RRCReconfigurationComplete message or in UAI since it is configured to do so, UE shall report reference location via UAI for the first time. The condition for the first-time report is missing currently.

[Proposed Change]: add the condition for the first-time report.

1>	if configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

2>	if the UE did not include referenceLocationReport in a RRCReconfigurationComplete message or in a UEAssistanceInformation message since it has been configured to provide location information; or

2>	if the current closest reference locations are different from the ones indicated in the last transmission including referenceLocationReport:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide location information for assisted SMTC configuration;

[Comments]:

[vivo]: The proposed change is fine to us.

[Nokia]: Since we add the clause, we would still suggest to also consider “since last entering connected mode” just to cover all cases.

# V001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V001 | LPWUS | 1 | Empty UAI on offset for LP-WUS monitoring (RRC-5) | R2-25xxx | Vivo(Chenli) |  | V005 | Agreed |

[Description]: ]: We don’t have the conclusion on the open issue [RRC-5], i.e. whether empty UAI on offset for LP-WUS monitoring is allowed. The legacy rules should apply for the preferred time offset signalled via UAI, similar as DRX-Preference.

[Proposed Change]: In section 5.7.4.3, add the scenario that when UE initiates the UEAssistanceInformation message and UE has no preference on offset for LP-WUS monitoring of the cell group, the UE doesn’t include offset as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide lpwus-OffsetPreference of a cell group according to 5.7.4.2 or 5.3.5.3:

2>	include lpwus-OffsetPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on time offset for LP-WUS monitoring:

3>	set the timeOffset to the preferred offset value.

2>	else (if the UE has no preference on offset for LP-WUS monitoring of the cell group):

3>	do not include offset in the Offset-Preference IE;

[Comments]:

# N034

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N034 | AIML | 2 | Incorrect field referenced in applicability reporting procedure | N/A | Jerediah Fevold |  | vnnn | Agreed |

[Description]: Incorrect field names are referenced in the applicability reporting procedure.

[Proposed Change]:

3>	if the associated serving cell index was included in an entry in applicabilityConfigList within applicabilityReportConfig and the applicability status for at least one of the associated entries in applicabilitySetConfigList has changed:

4>	include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows:

5>	set the applicabilityCellId to the serving cell index of the cell;

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed:

6>	include an entry in the applicabilityInfoReportList and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityInfoReport to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

5>	for each entry within applicabilitySetConfigList that changed applicability status, associated with the concerned serving cell:

6>	include an entry in the applicabilityInfoReportList and set the content as follows:

7>	set the applicabilitySetId within applicabilityReportConfigId to the corresponding applicabilitySetConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReport;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# O300

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O300 | AIML | 1 | Incomplete applicability info may be transferred during HO | R2-25xxxx | OPPO(Jiangsheng Fan) |  | V23 | Rejected |

[Description]:

2>	for each serving cell:

3>	if the cell is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction , or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed; or

3>	if the associated serving cell index was included in an entry in applicabilityConfigList within applicabilityReportConfig and the applicability status for at least one of the associated entries in applicabilitySetConfigList has changed: [RIL]: N034 AIML

4>	include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows:

5>	set the applicabilityCellId to the serving cell index of the cell;

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed [RIL]: O300 AIML:

6>	include an entry in the applicabilityReportConfigIdList[RIL]: C076, AIML and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityReportConfigId to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

5>	for each entry within applicabilitySetConfigList that changed applicability status [RIL]: O300 AIML, associated with the concerned serving cell:

6>	include an entry in the applicabilityReportConfigIdList and set the content as follows:

7>	set the applicabilitySetId within applicabilityReportConfigId [RIL]: C077, AIML to the corresponding applicabilitySetConfigId;

OPPO: The condition highlighted yellow means only the ID associated to an entry that the applicability status has changed can be reported, i.e. delta reporting is used in UAI for applicability change case. But this limitation may cause incomplete applicability info transferred during HO, because based on current spec, the content reported via UAI will be forwarded from source cell to target cell without modification in HO preparation message, which means only the change part of applicability info is forwarded to target cell. Rapp can decide whether Tdoc is needed or not to address this issue.

[Proposed Change]:

2>	for each serving cell:

3>	if the cell is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction , or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed; or

3>	if the associated serving cell index was included in an entry in applicabilityConfigList within applicabilityReportConfig and the applicability status for at least one of the associated entries in applicabilitySetConfigList has changed: [RIL]: N034 AIML

4>	include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows:

5>	set the applicabilityCellId to the serving cell index of the cell;

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19 [RIL]: O300 AIML:

6>	include an entry in the applicabilityReportConfigIdList[RIL]: C076, AIML and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityReportConfigId to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

5>	for each entry within applicabilitySetConfigList [RIL]: O300 AIML, associated with the concerned serving cell:

6>	include an entry in the applicabilityReportConfigIdList and set the content as follows:

7>	set the applicabilitySetId within applicabilityReportConfigId [RIL]: C077, AIML to the corresponding applicabilitySetConfigId;

[Comments]:

[WI CR rapporteur-v031]: The proposed change is not compliant with the RAN2#129 agreements below:

“Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.”

“Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause”

However, since this RIL is related to B206, it is recommended that both RILs B206 and O300 are discussed together in a Tdoc.

# C076

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C076 | AIML | 1 | Wrong field name |  | Tangxun |  | V003 | Duplicate |

[Description]: “applicabilityReportConfigIdList” should be replaced by “applicabilityInfoReportList”.

[Proposed Change]: update the procedural text as below:

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed:

6>	include an entry in the applicabilityInfoReportList and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityReportConfigId to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

5>	for each entry within applicabilitySetConfigList that changed applicability status, associated with the concerned serving cell:

6>	include an entry in the applicabilityInfoReportList and set the content as follows:

[Comments]:

[WI CR rapporteur-v022]: Duplicate of RIL N034.

# C077

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C077 | AIML | 1 | Undefined applicabilityReportConfigId |  | Tangxun |  | V003 | Agreed |

[Description]: “applicabilityReportConfigId” has been used in 5 places, but this parameter is not defined. Actually it should be replaced by “applicabilityInfoReportId”.

[Proposed Change]: update the procedural text as below (also in other places):

7>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# Z002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z002 | AIML | 1 | Preference suggestion to the functionality that is reported as inapplicable in UAI | R2-25xxxx | Fei |  | V015 | Rejected |

[Description]:

Currently, for the applicability reporting via UAI, it has been defined as below:

5>	for each entry within applicabilitySetConfigList that changed applicability status, associated with the concerned serving cell:

6>	include an entry in the applicabilityReportConfigIdList and set the content as follows:

7>	set the applicabilitySetId within applicabilityReportConfigId to the corresponding applicabilitySetConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

For the functionality that is provided in the OtherConfig whose applicability has been changed, there is no need for UE to provide the information of the releaseConfigurationPrefernece in the UAI as there is no hurt to keep it still in the otherConfig.

[Proposed Change]:

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

/omit for short/

1>	if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations subject to applicability determination procedure according to 5.7.4.2:

2>	include applicabilityReportList in this UEAssistanceInformation message;

2>	for each serving cell:

3>	if the cell is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction , or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed; or

3>	if the associated serving cell index was included in an entry in applicabilityConfigList within applicabilityReportConfig and the applicability status for at least one of the associated entries in applicabilitySetConfigList has changed:

4>	include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows:

5>	set the applicabilityCellId to the serving cell index of the cell;

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed:

6>	include an entry in the applicabilityReportConfigIdList and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityReportConfigId to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

5>	for each entry within applicabilitySetConfigList that changed applicability status, associated with the concerned serving cell:

6>	include an entry in the applicabilityReportConfigIdList and set the content as follows:

7>	set the applicabilitySetId within applicabilityReportConfigId, AIML to the corresponding applicabilitySetConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

[Comments]:

[WI CR rapporteur-v022]: As for RIL Z001, we suggest to discuss this in Tdocs.

# S045

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S045 | AIML | 2 | Correction on dataCollectionSart | R2-25xxxx | Seung-Beom |  | v026 | Rejected |

[Description]: We have the following agreement:

3 On stop/start indication - The UE can send start indication (without a preferred list) to indicate preference to start data collection - The UE can send preferred list implying that it would like to start data collection on those configuration - The UE can send stop indication for all or a given actual CSI report config ID. - Rapporteur will determine best way of signaling. This doesn’t preclude merging 1 and 2.

In our understanding, UE can indicate its preference for the following three cases via UAI:

Case 1) Absence of Start indication: UE has no preference to start data collection

Case 2) Presnece of Start indication “without” preferred list: UE has preference to start data collection, but none of given candidates is preferred

Case 3) Presnece of Start indication “with” preferred list: UE has preference to start data collection, and the indicated list among given candidates is preferred

Here, we want to clarify UE could have different preference for each serving cell. For example,

- For serving cell 1, UE has no preference to start data collection (i.e., Case 1), leading to absence of start indication

- For serving cell 2, UE has preference to start data collection (i.e., Case 2 or 3), leading to presence of start indication

However, according to the current ASN.1, the start indication is defined commonly to all serving cells. Therefore, there is no way for UE to indicate different start indication (i.e., absence vs. presence) for different cells.

[Proposed Change]:

Start indication is defined per serving cell, and corresponding TP could be also updated as follows:

UEAssistanceInformation-v19xy-IEs ::= SEQUENCE { applicabilityReportList-r19 ApplicabilityReportList-r19 OPTIONAL, dataCollectionPreference-r19 DataCollectionPreference-r19 OPTIONAL, loggedDataCollectionAssistance-r19 LoggedDataCollectionAssistance-r19 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } DataCollectionPreference-r19 ::= SEQUENCE { dataCollectionStart-r19 ENUMERATED {start} OPTIONAL, dataCollectionPreferredConfigurationList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19 OPTIONAL, dataCollectionStopConfigurationList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19 OPTIONAL, ... } DataCollectionCandidateList-r19 ::= SEQUENCE { dataCollectionServCellIndex-r19 ServCellIndex, dataCollectionStart-r19 ENUMERATED {start} OPTIONAL, dataCollectionCandidateIdList-r19 SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigId-r19 OPTIONAL } DataCollectionList-r19 ::= SEQUENCE { dataCollectionStopServCellIndex-r19 ServCellIndex, dataCollectionIdList-r19 SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId OPTIONAL }

#### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message The UE shall set the contents of the UEAssistanceInformation message as follows: <…> 1> if transmission of the UEAssistanceInformation message is initiated to report the UE preference to be configured with radio resources to perform UE-side data collection according to 5.7.4.2: 2> include dataCollectionPreference in this UEAssistanceInformation message; 2> if the UE prefers to be configured with radio resources to perform data collection: 3> for each serving cell for which UE prefers to be configured with radio resources to perform data collection: 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> set dataCollectionStart to start; 5> if dataCollectionCandidateConfigParameterList is configured for the serving cell and the UE has one or more preferred radio resource configuration(s) among the dataCollectionCandidateConfigParameterList: 6> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList; 3> set dataCollectionStart to start; 3> for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s): 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;

[Comments]: Nokia v030: We disagree with this change to the start indication. In our issue N114 we propose to remove the start indication all together since it doesn’t have an obvious use. If the NW has data collection configurations available for a UE, then it will provide a list. No further information is required.

[WI CR rapporteur-v031]: We suggest to discuss this in a Tdoc, together with RILs O301/N114.

# X003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X003 | AIML | 1 | Remove the dataCollectionStart | R2-25xxx | Xing |  | V012 | Rejected |

[Description]:

The presence of dataCollectionCandidateConfigList can implicitly indicate UE is about to collect data. There is no need to intorduce separate indication. On the other hand, start indication is useless without dataCollectionCandidateConfigList, since NW is not aware of the required configuration. If UE has already obtained the required configuration for data collection, UE doesn’t need to indicate start indication alone. We propose to remove the dataCollectionStart.

[Proposed Change]:

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

…

1>	if transmission of the UEAssistanceInformation message is initiated to report the UE preference to be configured with radio resources to perform UE-side data collection according to 5.7.4.2:

2>	include dataCollectionPreference in this UEAssistanceInformation message;

2>	if the UE prefers to be configured with radio resources to perform data collection:

3>	for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s):

4>	include an entry in dataCollectionPreferredConfigurationList and set the content as follows:

5>	set the dataCollectionServCellIndex to the serving cell index of the cell;

5>	include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionPreferredConfigurationList-r19     SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19      OPTIONAL,

dataCollectionStopConfigurationList-r19          SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19               OPTIONAL,

...

}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

[Comments]:

[WI CR rapporteur-v020]: We agree that at the moment it is unclear when the start indication should be included by the UE, without including a list of preferred candidates. Several interpretations were discussed in the past, e.g. (i) the UE sends just a start indication and then it expects to receive the list of candidates from the network, or (ii) the UE sends just a start indication with the meaning that it can accept any data collection configuration that the NW may decide to provide (that assumes the list of candidates is optional on top of the start/stop procedure). In any case, we agree that this topic needs further discussion based on Tdocs.

# E040

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E040 | AIML | 2 | Ensure that UE does not request a candidate UE-side data collection configuration for which it already has a UE-side data collection configuration | R2-25xxxx | Ericsson (Andra) |  | V022 | Rejected |

[Description]: According to the current procedural text in 5.7.4.3, the UE may request a candidate UE-side data collection configuration in UAI (received in otherConfig), even if it already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig) for that candidate configuration. This is not a desired behaviour and it should be fixed.

[Proposed Change]: The issue and some possible solutions will be discussed in more detail in a Tdoc. Some possible solutions are:

- change only the procedural text to make it clear that the UE does not request candidate configurations for which it already has a corresponding UE-side data collection configuration;

- change the ASN.1 (and corresponding procedures) so the network can indicate which candidate configuration a real UE-side data collection configuration corresponds to.

[Comments]:

# N084

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N084 | NTN | 1 | Determining the number of locations to report | No | Nokia (Jakob) |  | v016 | Rejected |

[Description]: Some UEs may only support up to 2 SMTCs, as indicated by parallelSMTC-r17, thus the number of SMTCs to report should be related also to the number of SMTCs supportes

[Proposed Change]: Add UE to consider also number of parallel SMTCs to consider in section 5.7.4.3;

1>	if transmission of the UEAssistanceInformation message is initiated to provide location information for assisted SMTC configuration in RRC_CONNECTED state according to 5.7.4.2;

2>	include the referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport or parallelSMTC-r17, which ever is the lowest;

[Comments]:

[Rapp] Provided this is a connected mode feature, the network already knows UE capabilities and can configure the UE accordingly with the number N of closets reference locations based on UE limitations.

[Qualcomm] In the end what should matter is number of SMTCs configured based on UE capability.

# N061

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N061 | SONMDT | 1 | timeSinceSHR missing in SHR for intra-NR mobility and timeSinceSPR not existing in SPR |  | Gyorgy Wolfner |  | V011 | Rejected |

[Description]: In RAN2#131 it was decided that SHR and SPR may be correlated in the case of CHO with candidate SCGs based on C-RNTI in the target cell. The undelying assumption in the community was that timeSinceSHR and timeSinceSPR may be used in addition. The issue is that timeSinceSHR was only defined for inter-RAT HO (execution of the last MobilityFromNRCommand towards the target EUTRA cell) and a similar field is missing completely from SPR.

[Proposed Change]: The following change is proposed in 5.7.10.3:

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessHO-Report:

2>	if the successHO-Report in the VarSuccessHO-Report concerns a DAPS handover and if a PDCP PDU has been received from the source cell of the concerned HO and a non-duplicated PDCP PDU has been received from the target cell of the concerned HO:

3>	set upInterruptionTimeAtHO in VarSuccessHO-Report to include the time elapsed between the time of arrival of the last PDCP PDU received from the source cell of the concerned handover and the time of arrival of the first non-duplicate PDCP PDU received from the target cell of the concerned handover, as measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell;

2>	if the successHO-Report in the VarSuccessHO-Report concerns a mobilityFromNRCommand:

3>	set timeSinceSHR in VarSuccessHO-Report to the time that elapsed since the execution of the MobilityFromNRCommand associated to the successful handover report in the VarSuccessHO-Report;

2>	if the successHO-Report in the VarSuccessHO-Report concerns a CHO with candidate SCG handover:

3>	set timeSinceSHR in VarSuccessHO-Report to the time that elapsed since the execution of the PCell handover associated to the successful handover report in the VarSuccessHO-Report;

2>	if the successPSCell-Report in the VarSuccessPSCell-Report concerns a CHO with candidate SCG handover:

3>	set timeSinceSPR in VarSuccessPSCell-Report to the time that elapsed since the execution of the PSCell handover associated to the successful handover report in the VarSuccessPSCell-Report;

2>	set the successHO-Report in the UEInformationResponse message to the value of successHO-Report in the VarSuccessHO-Report, if available;

2>	discard the VarSuccessHO-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[Comments]:

[Rapporteur] The agreement is “We add the C-RNTI for the PCell as a correlation indication for SHR and SPR reports. UE adds this unconditionally, and no other correlation info will be added.”

Given that there is no agreement support for it, for the time being, I reject this RIL, but feel free to bring a contribution to discuss this further.

# N033

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N033 | AIML | 2 | NW-side DC log request should not be datatype specific. | N/A | Jerediah Fevold |  | vnnn | ToDo |

[Description]:  Related to N024, N025, and N026. The field for requesting the NW-side data collection log should not be specific to a datatype as the buffer is generic. It is reasonable to keep the logs separated by datatype as long as the request and the indication of more data are generic. The consequence of leaving the requests, responses, and indications of further data specific to a datatype is that it will be unclear how to handle the generic buffer behavior. Right now, we have a single indication for buffer full, buffer and threshold reached. When we have more datatypes for collection, it could easily be that each individually do not reach the threshold, but the two together do. Then, the gNB would start to empty the buffer, perhaps for the first datatype. Then, the buffer threshold would no longer be met and the message containing the first datatype would not have the proper indicator to indicate that more data is available since it would be datatype specific.

[Proposed Change]:

1>	if the nw-DC-LogMeasReportReq is present:

2>	if VarCSI-LogMeasReport includes one or more logged measurement entries, set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows:

3>	include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first;

3>	if the VarCSI-LogMeasReport includes one or more additional logged measurement entries that are not included within the UEInformationResponse message:

4>	include the csi-MoreLogMeasAvailable;

<cut for brevity>

1>	else if nw-DC-LogMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRBX;

2>	discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[Comments]:

[Lenovo-Congchi-v011]: we don’t agree with Nokia. We believe the current implementation is fine. It is also future proof considering we may have new UE variable defined in Rel20 for AI mobility. We can easily add similar flag to support new measurement types. Besides, DC usually refers to dual connectivity.

[Huawei-Dawid-v017]: We agree with Lenovo and prefer keeping the current name. As commented during the RRC CR review, we believe the current implementation is intentional and aligned with RAN2 agreements

[Ericsson-v022]: We agree with Lenovo and Huawei.

[Nokia-v030]: We can agree that DC implies dual connectivity, but there has been no agreement that the gNB can retrieve the buffer per datatype. We have agreed on one buffer that will work for all use cases.

# S044

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S044 | AIML | 1 | Correction on inclusion of NSDC data | R2-25xxxx | Seung-Beom |  | v026 | Rejected |

[Description]: According to current text, UE may report NSDC (NW-side data collection) data without measured data (i.e., L1-RSRP and resource ID). For example, assume UE is configured with two NSDC configurations with ID 1 and 2.

- Upon reception of two NSDC (NW-side data collection) configuration (i.e., in 5.5x.1.3), UE includes csi-LogMeasInfoCellList in VarCSI-LogMeasReport to set refCSI-LoggedMeasurementConfigId (e.g., 1 and 2) and cellId.

- UE starts measurement and logging for configuration 1 (i.e., periodic logging), while adding measured data (i.e., L1-RSRP and resource ID) in VarCSI-LogMeasReport for refCSI-LoggedMeasurementConfigId 1.

- However, UE may not start measurement and logging for configuration 2 (i.e., event-based logging). i.e., No measured data for for refCSI-LoggedMeasurementConfigId 2 in VarCSI-LogMeasReport.

- Upon reception of UEInformationRequest, UE copies and pastes csi-LogMeasInfoCellList from VarCSI-LogMeasReport to UEInformationResponse. It means UE would include refCSI-LoggedMeasurementConfigId 2 (and corresponding cellId) without any measured data (i.e., L1-RSRP and resource ID) in UEInformationResponse, which is not useful information to NW-side model training at all.

[Proposed Change]:

UE should include in UEInformationResponse the entries which include SSB and/or CSI-RS measurement results.

#### 5.7.10.3 Reception of the UEInformationRequest message Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation: 1> if the csi-LogMeasReportReq is present: 2> if VarCSI-LogMeasReport includes one or more logged measurement entries, set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows: 3> include the csi-LogMeasInfoCellList and set it to include one or more entries which include SSB and/or CSI-RS measurement results from the VarCSI-LogMeasReport starting from the entries logged first; 3> if the VarCSI-LogMeasReport includes one or more additional logged measurement entries that are not included within the UEInformationResponse message: 4> include the csi-MoreLogMeasAvailable;

[Comments]:

[WI CR rapporteur-v031]: We suggest to discuss this in a Tdoc together with RILs J008/J009/Z004, since they are related.

# J008

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J008 | AIML | 1 | Setting csi-LogMeasInfoList in UEInformationResponse | R2-25xxxx | Sharp (LIU Lei) |  | V009 | Rejected |

[Description]: All the logged measurements in a cell may not be able to include in a single UEInformationResponse message, thus how to set csi-LogMeasInfoList in UEInformationResponse message should be clarified.

[Proposed Change]:

1>	if the csi-LogMeasReportReq is present:

2>	if VarCSI-LogMeasReport includes one or more logged measurement entries, set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows:

3>	include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first;

3> for each entry in csi-LogMeasInfoCellList, if the corresponding logged measurement entries are available in VarCSI-LogMeasReport, include the csi-LogMeasInfoList and set it to include one or more logged measurement entries associated with that cell, starting from the logged measurement entries logged first.

[Comments]:

[WI CR rapporteur-v022]: We suggest to discuss this issue in Tdocs, together with J009.

# J009

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J009 | AIML | 1 | Discard entries in csi-LogMeasInfoCellList after sending UEInformationResponse | R2-25xxxx | Sharp (LIU Lei) |  | V009 | Agreed |

[Description]: After sending UEInformationResponse message, the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport are discard. If csi-LogMeasInfoList is empty, the entries included in csi-LogMeasInfoCellList should be discard.

[Proposed Change]:

1> if the csi-LogMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRBX;

2>	discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport and discard the entries in csi-LogMeasInfoCellList from VarCSI-LogMeasReport if the corresponding csi-LogMeasInfoList is empty upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[Comments]:

[WI CR rapporteur-v022]: We suggest to discuss this issue in Tdocs, together with J008.

# V105

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V105 | AIML | 1 | Logged measurement entries included in the csi-LogMeasInfoList |  | Boubacar |  | V009 | Rejected |

[Description]: I think if both logMeasReport and csi-LogMeasReport are included, the “logged measurement entries included in the csi-LogMeasInfoList” is not discarded.

### 5.7.10	UE Information

1>	if the csi-LogMeasReportReq is present:

2>	if VarCSI-LogMeasReport includes one or more logged measurement entries, set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows:

3>	include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first;

3>	if the VarCSI-LogMeasReport includes one or more additional logged measurement entries that are not included within the UEInformationResponse message:

4>	include the csi-MoreLogMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[Proposed Change]:

### 5.7.10	UE Information

1>	if the csi-LogMeasReportReq is present:

2>	if VarCSI-LogMeasReport includes one or more logged measurement entries, set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows:

3>	include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first;

3>	if the VarCSI-LogMeasReport includes one or more additional logged measurement entries that are not included within the UEInformationResponse message:

4>	include the csi-MoreLogMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else if csi-LogMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRBX;

2>	discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[Comments]:

[WI CR rapporteur-v022]: The proposed change is already included in the agreed CR from the previous meeting (and also in the AIML review file).

# Z307

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z307 | SONMDT | 1 | CG-SDT information in RA report |  | QI Tao |  | V010 | Rejected |

[Description]: As the specs, sdt-FailureCause was agreed to include cg-SDT-TimeAlignTimer and configuredGrantTimer, but the description of the purpose in the RA report was not captured correctly, i.e., excluding other types of SDT.

[Proposed Change]:

Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed SDT operation as specified in clause 5.3.13.5, the UE shall:

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report; or

1>	if the registered SNPN identity or the SNPN identity selected by upper layers (see TS24.501 [23]) from the list of SNPN(s) included in the NPN-IdentityInfoList in SIB1is not included in snpn-IdentityList stored in a non-empty VarRA-Report:

2>	clear the information included in VarRA-Report;

// skipped.

The UE may discard the random access report information, i.e. release the UE variable VarRA-Report, 48 hours after the last successful random access procedure or the failed or successfully completed on-demand system information acquisition procedure or the failed or successfully completed SDT procedure related information is added to the VarRA-Report.

[Comments]:

[Rapporteur] Rapporteur notea that the RA report is logged only when RA SDT is performed, and if there was a CG SDT prior to this RA SDT, some information will be logged. So the change make the functionality unclear/ambegious. Hence the RIL is rejected.

# Z308

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z308 | SONMDT | 1 | Corrections on sdt-FailureCause information in RA report |  | QI Tao |  | V012 | Rejected |

[Description]: As the specs, the setting of sdt-FailureCause is in the scenario where SDT initiates the random access process, but the cause for setting cg-SDT-TimeAlignTimer and configuredGrantTimer is for CG-SDT rather than RA-SDT. According to our understanding, if SDT initiates the random access process, the cause for SDT failure will never include information about CG-SDT. Suggest clarifying the description in the specs that sdt-FailureCause is set to cg-SDT-TimeAlignTimer and configuredGrantTimer.

[Proposed Change]:

4>	if the SDT transmission was failed as defined in TS 38.300 [2]:

5>	include the sdt-Failed;

5>	if the random-access procedure is initiated for SDT:

6>	set the sdt-FailureCause to the cause of RA-SDT failure;

5>	else:

6>	set the sdt-FailureCause to the cause of CG-SDT failure;

4>	if the conditions to initiate MO-SDT were evaluated and not fullfilled according to TS 38.321 [3]:

5>	set the sdt-DL-RsrpInfo to the RSRP value measured at the time of SDT evaluation as specified in TS 38.321 [3];

5>	set the sdt-UL-DataVolume to the UL data volume at the time of SDT evaluation as specified in TS 38.321 [3];

<unchanged part>

5>	if the corresponding random-access procedure was performed on PSCell:

6>	if the cellId is not set to the global cell identity of the PSCell, set the spCellId to the global cell identity of the PCell;

4>	if the SDT transmission was failed as defined in TS 38.300 [2]:

5>	include the sdt-Failed;

5>	if the random-access procedure is initiated for SDT:

6>	set the sdt-FailureCause to the cause of RA-SDT failure;

5>	else:

6>	set the sdt-FailureCause to the cause of CG-SDT failure;

sdt-FailureCause

This field is included when the SDT procedure failed. Otherwise, the field is absent. This field indicates the SDT failure cause.

The field is set to t319a-expiry upon expiration of T319a timer. If the UE upper layers receive maxRetxThreshold reached indication from RLC while SDT procedure is ongoing, this field is set to maxRetxThreshold. It is set to preambleTransMax upon the UE upper layer receiving indication of reaching preambleTransMax from the MAC layer. Upon expiration of cg-SDT-TimeAlignmentTimer from the MAC, the field is set to cg-SDT-TimeAlignmentTimer. The field is set to configuredGrantTimer upon reception of indication that configuration grant timer has been expired from the MAC. The field is set to cellReselection upon SDT failure due to UE’s cell re-selection.

[Comments]:

[Rapporteur] Rapporteur notea that the RA report is logged only when RA SDT is performed, and if there was a CG SDT prior to this RA SDT, some information will be logged in the RA report. So the change makes the functionality unclear/ambegious. Hence the RIL is rejected.

# E020

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E020 | SONMDT | 1 | SHR report to both HO and LTM |  | Ali Parichehreh |  | V004 | Agreed |

[Description]: In 5.7.10.6, the terminology “handover” should be change to “reconfiguration with sync” to cover both handover and LTM cell switch.

[Proposed Change]:

3>	for intra-NR reconfiguration with sync, set the c-RNTI to the C-RNTI assigned by the target PCell of the reconfiguration with sync;

[Comments]:

[Rapporteur]: Rapporteur agrees with the change.

# E028

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E028 | SONMDT | 1 | Simplifying the if condition to check if the UE was configured with CHO with candidate SCG configuration |  | Ali Parichehreh |  | V004 | Agreed |

[Description]: Simplify the text “3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if the UE was configured with condExecutionCond and condExecutionCondPScell, for the source PSCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied” and harmonize with SPR section.

[Proposed Change]:

3>	if the procedure is triggered due to successful completion of CHO with candidate SCG ;

[Rapoorteur]: agree to simplify the text.

# S022

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S022 | SONMDT | 1 | Clarify as MCG LTM in SHR |  | Samsung (Aby) |  | V006 | Agreed |

[Description]:

While logging the L1 meausrmenets for the target cell, claify that L1 measurements are logged if it is MCG LTM candidate cell.

[Proposed Change]: Suggest below change.

4>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with MCG ltm-Config and LTM-CSI-ReportConfig associated with the target PCell when connected to the source PCell:

5>	set the resultsSSB-Indexes in targetCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement results of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

[Comments]:

[Rapporteur]: RIL is agreed, the change is applied in other places with the same formulation.

# C060

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C060 | SONMDT | 1 | Not a R19 change | R2-25xxxx | Tangxun |  | V002 | Rejected |

[Description]: an “applied” is added here, but our understanding is that this change is not related a R19 feature. If this change is deemed needed, we should use a correction CR to make this change from R17.

[Proposed Change]: remove “applied” as below:

5>	set the timeSinceCHO-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PCell and the reception of the last conditionalReconfiguration including the condRRCReconfig of the target PCell in the source PCell;

[Comments]:

[Samsung] We think the R19 spec is correct, as the changes are related to CHO with Candidate SCG(s) and SCPAC. R17/R18 is also fine as it didn’t support MRO for these features. So no need to change.

[Rapporteur]: Rapporteur tends to agree with the RIL but since Samsung has a different view, maybe we can discuss this in the online session. Please bring contribution for this RIL.

[Rapporteur]: This is a Rel-18 change and in RAN2#131bis it is agreed to produce a proper Rel-18 CR as noted by session chair (Session chair says we can wait to the next meeting and can see a proper CR). Therefore, this RIL is rejected as there is no more impact on the release 19 spec.

# E018

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E019 | SONMDT | 1 | Logging L1 measurements for LTM candidate cells in RLF report |  | Ali Parichehreh |  | V009 | Agreed |

[Description]: the SS/PBCH block-based L1-RSRP measurement results is logged one (in RLF report) with separate list of LTM candidate cells per SSB frequency. as of now there is a loop over each neighbour MCG LTM candidate cell, which makes the logging of candidate cells ina a loop which is wrong. Ina ddition the cells should be ordered per SSB frequency. In addition we think SS/PBCH block-based L1-RSRP measurement results should be performed based on LTM-CSI-ReportConfig

[Proposed Change]:

3>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with MCG ltm-Config including LTM-CSI-ReportConfig associated with the MCG when connected to the source PCell:

4>	if SS/PBCH block-based L1-RSRP measurement results performed based on LTM-CSI-ReportConfig are available:

5>	set the neighCellsMeasL1ListNR to include all the available SS/PBCH block-based L1-RSRP measurement results of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block-based L1-RSRP (of all SS/PBCH block-based L1-RSRP measurement results for the cell) is listed first per each SSB frequency, based on the available SS/PBCH block-based L1-RSRP measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

[Comments]:

# J032

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J032 | SONMDT | 1 | L1 measurement result for neighbour cells in SHR |  | Chang Ningjuan |  | V008 | Rejected |

[Description]: the procedure text for log L1 measurement results for each neighbour MCG LTM candidate cell is not clear. As it is executed for each LTM candidate cell, thus the wording“of the best measured cells, other than the source PCell or target PCell” is not needed.

[Proposed Change]:

3>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config including LTM-CSI-ReportConfig associated with the MCG when connected to the source PCell:

4>	for each neighbour MCG LTM candidate cell:

5>	if SS/PBCH block-based L1-RSRP measurement results are available:

6>	set the neighCellsMeasL1ListNR to include all the available SS/PBCH block-based L1-RSRP measurement results, ordered such that the cell with highest SS/PBCH block-based L1-RSRP (of all SS/PBCH block-based L1-RSRP measurement results for the cell) is listed first, based on the available SS/PBCH block-based L1-RSRP measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

[Comments]:

[Rapporteur] In rapporteur inderstanding the L1 measurement for the source and target PCell measurmeents are logged separately, so it should not be deleted. The first part needs some changes e.g., “of the MCG LTM candidate cells”, I have updated the draft CR.

# N062

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N062 | SONMDT | 1 | Simplification of SHR determation section for CHO with candidate SCGs |  | Gyorgy Wolfner |  | V011 | Rejected |

[Description]: The actions for logging the chronological fulfillment of the two execution conditions which is logged in the new IE choWithCandidateSCGInfoList should be different for SHR determination compared to RLF report determination, since for successful completion of CHO with candidate SCG (CwcS) both criteria are always fulfilled and, therefore, only the order is to be checked. Checking a condition which is per-se fulfilled is unnecessary.

[Proposed Change]: The following change is proposed in 5.7.10.6:

3>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in cho-WithCandidateSCGInfoList and set the values as follows:

4>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first in time;

4>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

4>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

4>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

4>	if after receiving this CHO with candidate SCG configuration, the UE received a conditional handover configuration for the same target candidate PCell as set in pCellId:

5>	set fulfilledConfigWhenChoOnly to cho if condExecutionCond was fulfilled at the time of receiving the conditional handover configuration or cpc if condExecutionCondPSCell was fulfilled at the time of receiving the conditional handover configuration, otherwise set fulfilledConfigWhenChoOnly to neither;

[Comments]:

[Rapporteur] In rapporteurs understanding both conditions are fulfilled for the PCell-PSCell where the UE performs handover. However, there can be upto 8 CHO with candidate SCG configurations and for some of them both conditions might not be fulfilled. If this change is implemented, there would be ambiguity in UE behavior in those cases. Therefore this RIL is rejected.

# N063

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N063 | SONMDT | 1 | Simplification of SPR determation section for CHO with candidate SCGs |  | Gyorgy Wolfner |  | V011 | Rejected |

[Description]: The actions for logging the chronological fulfillment of the two execution conditions which is logged in the new IE choWithCandidateSCGInfoList should be different for SPR determination compared to RLF report determination, since for successful completion of CHO with candidate SCG (CwcS) both criteria are always fulfilled and, therefore, only the order is to be checked. Checking a condition which is per-se fulfilled is unnecessary.

[Proposed Change]: The following change is proposed in 5.7.10.7:

3>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in cho-WithCandidateSCGInfoList and set the values as follows:

4>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first in time;

4>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

4>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

4>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

[Comments]:

[Rapporteur] In rapporteurs understanding both conditions are fulfilled for the PCell-PSCell where the UE performs handover. However, there can be upto 8 CHO with candidate SCG configurations and for some of them both conditions might not be fulfilled. If this change is implemented, there would be ambiguity in UE behavior in those cases. Therefore, this RIL is rejected.

# N065

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N065 | SONMDT | 1 | SHR logging when CHO only HO is performed | R2-250nnnn | Gyorgy Wolfner |  | V011 | Rejected |

[Description]: It was introduced to log parameters in SHR report when CHO only handover is performed even if CHO with candidate SCG was also configured. As no new SHR trigger is introduced, SHR is only generated when the UE faces one of near failure cases, e.g. by passing a configured thresholdPercentageT310. The most typical case when CHO only handover is normally performed instead of the CHO with candidate SCGs will not result in generating any SHR, and thus the network will not be informed about the problem.

[Proposed Change]: The successHO-Config is to be amended to configure the UE with the new criterion, which indicates to check preferred usage of CHO with candidate SCG, when the UE is operating in DC mode and being configured with other legacy HO options, e.g. with a complementary CHO-only.

The new criterion checks if a CHO only is executed while UE is operating in DC and is configured for CHO with candidate SCGs. This results in providing SHR including the choWithCandidateSCGInfoList.

We will provide the details of the solution in a separate tdoc.

[Comments]:

[Rapporteur]: RIL is rejected according to the discussion in RAN2#131bis

# Z453

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z453 | NR_SL_relay_multihop-Core | 1 | SUI initiation for MH |  | ZTE (Weiqiang Du) |  | V009 | PropAgree |

[Description]: As legacy, My understanding is that for each condition for sl-L2U2N-Relay, there should be a corresponding new condition for sl-L2U2N-MH-Relay. Will submit the CR to show all the necessary changes, if needed.

[Proposed Change]:In clause 5.8.3.3, review all conditions of  sl-L2U2N-Relay, and create new conditions for sl-L2U2N-MH-Relay correspondingly, if necessary, for example:

2>	if configured by upper layer to receive NR sidelink L2 U2N single hop relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay;if configured by upper layer to receive NR sidelink L2 U2N MH relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-MH-Relay or if configured by upper layer to receive NR sidelink L3 U2N relay discovery messages on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery

[Comments]:

[Rapporteur]: Agree to new condition for sl-L2U2N-MH-Relay as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# O503

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O503 | NR_SL_relay_multihop-Core | 1 | SUI initiation for discovery transmission resource requesting | R2-25xxxxx | OPPO (Bingxue Leng) |  | V004 | ToDo |

[Description]: In legacy, the SUI for discovery transmission resource requesting is initiated if discovery transmission condition is met (as specified in clause 5.8.13.3), i.e., the threshold condition, this should be followed in multo-hop U2N Relay. For MH-U2N Relay, the following condition is defined:

For the last relay UE, 1) Uu lower bound is defined (same value as single-hop U2N Relay UE); 2) PC5 threshold is defined for Model-B respond message transmission if there is no PC5 connection with the child node;

For the intermediate relay UE, 1) Uu upper bound is defined (same value as single-hop U2N Remote UE); 2) PC5 threshold is defined for Model-B solicitation message transmission.

[Proposed Change]:

4>	if the UE is capable of U2N Relay UE in case of single hop, and if SIB12 includes sl-RelayUE-ConfigCommon, and if the U2N Relay UE UE threshold conditions as specified in 5.8.14.2 are met; or

4>	if the UE is capable of Last U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon, and if the Last U2N Relay UE UE threshold condition as specified in 5.8.14.2 are met when the UE has the PC5 connection with the Child UE; Or if the UE is capable of Last U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon and sl-RelayUE-ConfigCommonMH, and if the Last U2N Relay UE UE threshold condition as specified in 5.8.14.2 and 5.8.XX.2 are met when the UE has no PC5 connection with the Child UE; or

4>	if the UE is capable of Intermediate U2N Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommon, and if the U2N Remote UE threshold conditions as specified in 5.8.15.2 are met when the UE has the PC5 connection with the Parent UE; Or if the UE is capable of Intermediate U2N Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommon and sl-RelayUE-ConfigCommonMH, and if the U2N Remote UE threshold conditions as specified in 5.8.15.2 and Intermediate Relay UE threshold as specified in 5.8.XX.2 are both met when the UE has no PC5 connection with the Parent UE; or

[Comments]:

[Rapporteur]: In Rapporteur view the intermediate relay UE does not need to check U2N Remote UE threshold conditions for transmitting the discovery message when the UE has no PC5 connection with the Parent UE. We can discuss this further and contributions from companies are invited on this aspect. RIL Status is set to “ToDo”

# X500

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X500 | SLRelay | 1 | U2N Relay UE needs clarification |  | Xiaomi (Shuai) |  | V005 | ToDo |

[Description]: Should add “in case of single hop” to avoid confusion. Due to “U2N Relay UE” is only referred to single-hop case, and Last U2N Relay UE is only referred to multi-hop case.

[Proposed Change]: See below change.

4>	if the UE is capable of U2N Relay UE or of Last U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon, and if the U2N Relay UE in case of single hop or if the Last U2N Relay UE threshold conditions as specified in 5.8.14.2 are met; or

[Comments]:

[Rapporteur]: Rapporteur agrees that such clarification will be helpful and recommends "ToDo" status for this RIL as it can be discussed together with other changed porposed in O503

# Z454

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z454 | NR_SL_relay_multihop-Core | 1 | Source L2 ID report | R2-25xxxxx | ZTE (Weiqiang Du) |  | V009 | ToDo |

[Description]: In legacy, for D2I path switch, remote UE will report it’s source L2 ID to network, so that network can send SRAP configuration to target relay UE before remote UE switch to target path.

[Proposed Change]: Suggest RAN2 to agree MH remote UE will report it’s own SRC L2 ID to network and capture below text:

3>	if SIB12 includes sl-L2U2N-Relay or sl-L2U2N-MH-Relay and the UE is capable of L2 U2N remote UE:

4>	include sl-SourceIdentityRemoteUE corresponding to the upstream direction and set it to the source identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

[Comments]:

[Rapporteur]: First change is ok however the second change is not needed as it is clear that it is the sl-SourceIdentityRemoteUE of the remote UE. If agreeable, the status can be changed from from “ToDo” to “PropAgree” and the first change can be included in the CR.

# Z456

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z456 | NR_SL_relay_multihop-Core | 1 | UE type in SUI message | R2-25xxxxx | ZTE (Weiqiang Du) |  | V009 | ToDo |

[Description]: For intermediate relay UE, it has both relay UE and remote UE functionality. In SUI message, UE should report the requirements of discovery message and set the UE type to remote UE or relay UE. How intermediate relay UE set the UE type in sl-TxResourceReqListDisc is not clear.

4>	include sl-TxResourceReqListDisc and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink discovery messages resource:

****

5>	if the UE is acting as L2/L3 U2U Relay UE:

6>	include ue-TypeU2U and set it to relayUE;

5>	if the UE is acting as L2/L3 U2U Remote UE:

6>	include ue-TypeU2U and set it to remoteUE;

[Proposed Change]:

Solution1: Clarify that intermediate relay UE will use different L2 ID for remote UE discovery and relay UE discovery, which may need double check with SA2.

Solution2: If it is possible that UE use same L2 ID for remote UE and relay UE discovery, a new UE type is needed.

[Comments]:

[Rapporteur]: Rapporteur recommends "ToDo" status for this RIL. Since there might be alternative solutions, companies can discuss it in their contributions along with the text proposal.

# W500

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W500 | NR_SL_relay_multihop-Core |  | SUI enhancement for Intermediate Relay UE | R2-xxxxxxx | NEC(Boyuan Zhang) |  | V13 | PropReject |

[Description]: According to the current SUI, it is only applicable for single hop U2N relay, for the case of multi-hop U2N, when Last Relay sends sl-TxResourceReqL2U2N-Relay towards network, the local ID shall be set for Intermediate Relay UE rather than L2 U2N remote UE.

[Proposed Change]:

#### 5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

<….>

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE is acting as L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-LocalID-Request to request local ID for L2 U2N Remote UE or L2 Intermediate U2N Relay UE transiting to RRC_CONNECTED or in RRC_CONNECTED state;

5>	set sl-PagingIdentityRemoteUE to the paging UE ID received from peer L2 U2N Remote UE, if it is not released as in 5.8.9.8.3;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE;

[Comments]:

[Rapporteur]: The existing SUI procedure can be extended for multi hop scenario with the understanding that the Intermediate Relay UE is also a Remote UE. So no enhancements are needed.   " PropReject " status for this RIL

# W501

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W501 | NR_SL_relay_multihop-Core |  | SUI enhancement for Intermediate Relay UE | R2-xxxxxxx | NEC(Boyuan Zhang) |  | V13 | PropReject |

[Description]: According to the current SUI, it is only applicable for single hop U2N relay, for the case of multi-hop U2N, when Last Relay sends sl-TxResourceReqL2U2N-Relay towards network, the paging UE ID is received from peer L2 Intermediate U2N Relay UE rather than L2 U2N Remote UE.

[Proposed Change]:

#### 5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

<….>

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE is acting as L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-LocalID-Request to request local ID for L2 U2N Remote UE transiting to RRC_CONNECTED or in RRC_CONNECTED state;

5>	set sl-PagingIdentityRemoteUE to the paging UE ID received from peer L2 U2N Remote UE or L2 Intermeidate U2N Relay UE, if it is not released as in 5.8.9.8.3;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE;

[Comments]:

[Rapporteur]: Same as W500 - The existing SUI procedure can be extended for multi hop scenario with the understanding that the Intermediate Relay UE is also a Remote UE. So no enhancements are needed. " PropReject " status for this RIL

# Z455

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z455 | NR_SL_relay_multihop-Core | 1 | Paging ID report | R2-25xxxxx | ZTE (Weiqiang Du) |  | V009 | ToDo |

[Description]: In legacy, relay UE will report paging ID information of it’s connected remote UE, each paging ID is associated to L2 ID of connected remote UE. For MH relay, relay UE may also receives the more than one unconnected child UE’s paging information from it’s connected child UE, all these paging information will be associated to connected child UE’s L2 ID. In last RAN2 meeting, we have agreed to introduce a paging ID list in PC5 remote UE information message to solve this issue. We think same rule shall be applied in SUI message.

[Proposed Change]: RAN2 is to agree to introduce a paging ID list in SL-TxResourceReqL2U2N-Relay-r17.

[Comments]:

[Rapporteur]: In Rapporteur view the change is not essential. However we can discuss this further and contributions from companies are invited on this aspect. The status is “ToDo” for this RIL

# K001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K001 | NR_SL_relay_multihop-Core | 1 | PC5 Relay RLC channel release by Intermediate U2N Relay UE |  | ASUSTeK (Richard Kuo) |  | V007 | PropReject |

[Description]: In our understanding, an Intermediate U2N Relay UE should provide PC5 Relay RLC channel configuration(s) configured by the gNB to its parent UE for upstream transmission. Given this, when SL RLF with its child UE is detected, the Intermediate Relay UE should inform its parent UE to release a PC5 Relay RLC channel if there is no other SLRB associated with this PC5 Relay RLC channel.

[Proposed Change]:

##### 5.8.9.1.2   Actions related to transmission of RRCReconfigurationSidelink message

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

…

1> if the UE is acting as L2 U2U Remote UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and the procedure is initiated to release the first hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Relay UE (i.e. Rx UE) according to clause 5.8.9.7.1; or

1> if the UE is acting as L2 U2U Relay UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and the procedure is initiated to release the second hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Remote UE (i.e. Rx UE) according to clause 5.8.9.7.1; or

1> if the UE is acting as L2 Intermediate U2N Relay UE (i.e. Tx UE) and the procedure is initiated to release the PC5 Relay RLC channel to the connected UE (i.e. Rx UE) according to clause 5.8.9.7.1:

2> set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5;

…

#### 5.8.9.3   Sidelink radio link failure related actions

The UE shall:

1> upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1> upon T400 expiry for a specific destination; or

1> upon indication from MAC entity that HARQ-based Sidelink RLF for a specific destination has been detected; or

1> upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination; or

1> upon indication of consistent sidelink LBT failures for all RB sets for a specific destination from MAC entity:

2> consider sidelink radio link failure to be detected for this destination;

2> release the DRBs (if any) of this destination, according to clause 5.8.9.1a.1;

2> release the SRBs of this destination, according to clause 5.8.9.1a.3;

2> release the PC5 Relay RLC channels of this destination if configured, in according to clause 5.8.9.7.1;

2> discard the NR sidelink communication related configuration of this destination;

2> reset the sidelink specific MAC of this destination, except for end-to-end PC5 connection in L2 U2U Relay operation;

2> consider the PC5-RRC connection is released for the destination;

2> indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2> if UE is in RRC_CONNECTED:

3> if the UE is acting as L2 U2N Remote UE for the destination:

4> if MP is configured, and neither MCG transmission nor indirect path transmission is suspended:

5> initiate the indirect path failure information procedure as specified in 5.7.3c;

4> else (i.e., MP is not configured, or MP is configured and MCG transmission or indirect path transmission is suspended):

5> initiate the RRC connection re-establishment procedure as specified in 5.3.7;

3> else:

4> perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

4> if the UE is acting as L2 Intermediate U2N Relay UE for the destination and the destination is a child UE:

5> perform the PC5 Relay RLC channel release according to 5.8.9.7.1, if there is no other SLRB associated with this PC5 Relay RLC channel;

…

##### 5.8.9.7.1              PC5 Relay RLC channel release

The UE shall:

1> if the PC5 Relay RLC channel release was triggered after the reception of the RRCReconfigurationSidelink message; or

1> after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel release was triggered due to the configuration received within the sl-ConfigDedicatedNR or due to sidelink radio link failure detected by an L2 Intermediate U2N Relay UE as specified in 5.8.9.3:

2> for each SL-RLC-ChannelID in sl-RLC-ChannelToReleaseList received in sl-ConfigDedicatedNR within RRCReconfiguration, or for each SL-RLC-ChannelID included in the received sl-RLC-ChannelToReleaseListPC5 that is part of the current UE sidelink configuration:

3> release the RLC entity and the corresponding logical channel associated with the SL-RLC-ChannelID;

…

[Comments]:

[Rapporteur]: There is no need for the Intermediate U2N Relay UE to provide PC5 Relay RLC channel configuration(s) configured by the gNB to its parent UE as the parent relay UE will also be configured with the proper PC5 Relay RLC channel configuration.

When PC5 RLF happens the remote UEs will perform RRCReestablishment and the gNB will eventually release the allocated resources in all the relays on the disrupted path for these remote UEs via the RRC Reconfiguration procedure. Hence hence Rapporteur recommends " PropReject " status for this RIL as such mechanism are not needed..

[Apple] Simialr view as Rapp. I think the intermediate relay UE via trigger SUI procedure and the NW will update the relay UE with new set of PC5 Relay RLC channel configurations and new SRAP mapping, based on those updated configuiraiton, the UE will be triggered to release PC5 relay RLC channel. So the proposed change is not needed.

# A500

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A500 | SLRelay | 1 | Clarificaiton of SIB, PosSIB, and Paging triggering conditions |  | Apple (Zhibin Wu) |  | V015 | ToDo |

[Description]: In clause 5.8.9.8.2, the triggering conditions for the transmission of RemoteUEInformaitonSL largely resuse single-hop relay case and is outdated, there are new triggering conditions are not well specified and cannot be simply covered by just adding “intermediate relay UE”.

Also, forwading posSIB or SFN-DFN offset in multi-hop case is not discussed yet, so this should be separately triggered only for single-hop case .

Thirdly, the “sl-PagingInfo-RemoteUE-List “ is not only used by adding received paging request from children, but also the case when a child is disconnected. So, we need change "received” to “updated”.

Finally. the exmnaple in (e.g, ….) for paging case only represents a typical single-hop case and will be misleading. So, we suggest to remove this “(e.g…..)”

[Proposed Change]: See below change.

##### 5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

When L2 U2N remote UE entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink for remote UE’s own SIB/posSIB/Paging or SFN-DFN offset request while in RRC_IDLE or RRC_INACTIVE;

When RRC_IDLE or RRC_INACTIVE L2 Intermediate U2N Relay UE receives new or updated SIB/Paging request from one or more child UE(s), or PC5 link to a Child UE is no longer viable (e.g, due to SL RLF), the UE shall:

1>	if the UE has SIB request information to provide e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before) :

2>	include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

1>	if the UE has not stored a valid version, in accordance with clause 5.2.2.2.1, of one or several posSIB(s) that the UE requires for a positioning operation, and the requested posSIB has not been indicated in RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before, and the connected parent L2 U2N relay UE set posSIB-ForwardingSupported to supported:

2>	include sl-RequestedPosSIB-List in the RemoteUEInformationSidelink to indicate the requested posSIB(s);

1>	if the UE needs the SFN-DFN offset based on the request from upper layers and the connected L2 U2N relay UE set sfn-DFN-OffsetSupported to supported:

2>	set sl-SFN-DFN-OffsetRequested to true;

1>	if the UE has paging related information to provide, set sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List as follows:

2>	if the L2 U2N Remote UE is in RRC_IDLE:

3>	include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer;

2>	else if the L2 U2N Remote UE is in RRC_INACTIVE:

3>	include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer,

4>	set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC);

3>	else:

4>	set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC;

2>	if any paging information is received from the Child UE or a Child UE is no longer connected to the L2 U2N intermediate Relay UE:

3>	include the updated paging information for Child UE(s) in the sl-PagingInfo-RemoteUE-List;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

[Comments]:

[Rapporteur]: The procedure introduction clearly specifies that “request the SFN-DFN offset from the connected L2 U2N Relay UE in case of single hop”. Furthermore there is a sentence dedicated to this “This procedure is used by the L2 U2N Remote UE in RRC_CONNECTED to request the SFN-DFN offset from the connected L2 U2N Relay UE in case of single hop”. Hence it does not need any seprate triggering condions.

PC5 RLF will only happen in the when the UEs are in RRC_CONNECTED State it will not happen when the UEs are in RRC_IDLE or inactive. So the second trigger condition is not valid.

We can discuss the need for other changes based on contribution .

# X501

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X501 | SLRelay | 1 | SI request determination |  | Xiaomi (Xing Yang) |  | V005 | PropReject |

[Description]:

The intermediate relay UE shall also request the SIBs requested by child UE.

[Proposed Change]:

##### 5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE or L2 Intermediate U2N Relay UE shall:

1>	if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 or SIB(s) requested by child UE and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before):

[Comments]:

[Rapporteur]: The proposed addition is not essential as the intermediate relay UE will generally have child UEs connected to it hence rapporteur recommends " PropReject " status for this RIL.

[Apple] We support this change because there is a need to differentiation its own SIB requiest and SIB request relayed on behalf its children.

[Rapporteur]: After offline discussion with Xiaomi the status is changed from " PropReject " to “Todo in V021 . Companies can discuss this further based on contibutions.

# J058

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J058 | NR_SL_relay_multihop-Core | 1 | Missing sl-PagingInfo-RemoteUE-List |  | Tsuboi (Sharp) |  | v019 | PropAgree |

[Description]: sl-PagingInfo-RemoteUE-List should be needed in the sentence.

1>	if the UE has paging related information to provide (e.g. the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before), set sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List as follows:

[Proposed Change]:

1>	if the UE has paging related information to provide (e.g. the UE has not sent sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List in the RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before), set sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List as follows:

[Comments]:

[Rapporteur]: Agree to add sl-PagingInfo-RemoteUE-List  as proposed above . The status of this RIL is set to “PropAgree”.

# X502

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X502 | SLRelay | 1 | Definition of Paging information is not clear |  | Xiaomi (Shuai) |  | V005 | PropReject |

[Description]: In clause 5.8.9.8.2, legacy paging information usually indicates paging message, while paging information here means paging monitoring parameter or paging information request, thus need to clarify.

[Proposed Change]: See below change.

2>	if any sl-PagingInfo-RemoteUE-List or sl-PagingInfo-RemoteUE is received from the Child UE:

3>	include the received paging information in the sl-PagingInfo-RemoteUE-List;

[Comments]:

[Rapporteur]: The proposed clarification is not essential as it is generallly understood that paging information is included in  sl-PagingInfo-RemoteUE-List or sl-PagingInfo-RemoteUE hence rapporteur recommends " PropReject " status for this RIL.

# K003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K003 | NR_SL_relay_multihop-Core | 1 | Release of sl-PagingInfo-RemoteUE-List |  | ASUSTeK (Richard Kuo) |  | V007 | PropReject |

[Description]: New IE sl-PagingInfo-RemoteUE-List is added in RemoteUEInformationSidelink message. However, release of sl-PagingInfo-RemoteUE-List is missing in 5.8.9.8.2 when the L2 Intermediate U2N Relay UE enters RRC_CONNECTED.

[Proposed Change]:

##### 5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE or L2 Intermediate U2N Relay UE shall:

…

When entering RRC_CONNECTED, if L2 U2N remote UE or L2 Intermediate U2N Relay UE had sent sl-RequestedSIB-List, sl-RequestedPosSIB-List, and/or sl-PagingInfo-RemoteUE, the L2 U2N Remote UE or L2 Intermediate U2N Relay UE shall:

1> set the sl-RequestedSIB-List to the value release if requested before;

1> set the sl-RequestedPosSIB-List to the value release if requested before;

1> set the sl-PagingInfo-RemoteUE/sl-PagingInfo-RemoteUE-List to the value release if sent before;

1> submit the RemoteUEInformationSidelink message to lower layers for transmission;

…

[Comments]:

[Rapporteur]: There might be INACTVE or IDLE remote UEs connected to this intermediate downstream hence the intermediate relay UE will set the sl-PagingInfo-RemoteUE only for itself in 5.8.9.8.. hence rapporteur recommends " PropReject " status for this RIL

[Apple] I think whether RRC_CONNECTED intermediate relay UE will monitor paging for all its children (i.e., release sl-PagingInfo-RemoteUE-List) can be discussed in maintenance stage…so this issue is still FFS and can be discussed based on company input.

[Rapporteur]: It should be noted that the intermediate relay UE is also a remote UE and if the intermediate relay UE transitions to RRC_CONNECTED state for having its own service it should not release the the paging for its child UEs. If there are other secanrios it can be discussed based on company contributions.

# K004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K004 | NR_SL_relay_multihop-Core | 1 | Behavior of Intermediate U2N Relay UE in response to reception of RemoteUEInformationSidelink |  | ASUSTeK (Richard Kuo) |  | V007 | PropReject |

[Description]: The behavior of the Intermediate U2N Relay UE to send the RemoteUEInformationSidelink message to its parent UE upon reception of an RemoteUEInformationSidelink message from its child UE while in RRC_IDLE or RRC_INACTIVE is missing in 5.8.9.8.3. Alternatively, a similar subclause as 5.8.9.9.X could be added to describe the Intermediate U2N Relay UE behavior so as to align the Intermediate U2N Relay UE behaviors in different procedures.

[Proposed Change]:

[Comments]:

[Rapporteur]: Since the intermediate relay UEs behaviour will be covered by L2 U2N Relay UE’s behaviour that also includes intermediate relay UE based on the definition hence rapporteur recommends " PropReject " status for this RIL.

# J059

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J059 | NR_SL_relay_multihop-Core | 1 | Missing sl-PagingInfo-RemoteUE-List |  | Tsuboi (Sharp) |  | v019 |  |

[Description]: sl-PagingInfo-RemoteUE-List is needed in the sentence.

1>	if the RemoteUEInformationSidelink includes the sl-PagingInfo-RemoteUE:

[Proposed Change]:

1>	if the RemoteUEInformationSidelink includes the sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List:

[Comments]:

# Z457

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z457 | NR_SL_relay_multihop-Core | 1 | Paging info list reception handling |  | ZTE (Weiqiang Du) |  | V009 | PropAgree |

[Description]:operation related to sl-PagingInfo-RemoteUE-List is missing in clause 5.8.9.8.3

[Proposed Change]:

2>	if the UE is in RRC_IDLE or RRC_INACTIVE:

3>	if the sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List is set to setup:

4>	monitor the Paging message at the L2 U2N Remote UE's paging occasion calculated according to sl-PagingIdentityRemoteUE and sl-PagingCycleRemoteUE included in sl-PagingInfo-RemoteUE;

3>	else (the sl-PagingInfo-RemoteUEor sl-PagingInfo-RemoteUE-List is set to release):

4>	stop monitoring the Paging message at the L2 U2N Remote UE's paging occasion;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

2>	else:

3>	if the sl-PagingInfo-RemoteUEor sl-PagingInfo-RemoteUE-List is set to setup:

4>	include the received sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message and perform Sidelink UE information transmission in accordance with 5.8.3;

3>	else (the sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List is set to release):

4>	initiate transmission of the SidelinkUEInformationNR message to release the sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message in accordance with 5.8.3;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

[Comments]:

[Rapporteur]: Agree to add the clarification with the change as below. Have changed the status from “ToDo” to “PropAgree”.

# J060

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J060 | NR_SL_relay_multihop-Core | 2 | The issue on signalling overhead for releasing paging information. |  | Tsuboi (Sharp) |  | v019 | PropReject |

[Description]: Only one paging information can be released by a message even if multiple paging information should be released, e.g., all relays are in IDLE and a remote UE want to be in CONNECTED. It will increase signalling overhead.

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	stop monitoring the Paging message at the L2 U2N Remote UE's paging occasion;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

[Proposed Change]: It is suggested to introduce a paging information release list.

[Comments]:

[Rapporteur]: The time when each of the parent relay UEs in the multihop chain will move to RRC CONNECTED and release the paging information for itself will be different t. Consequently there is no need for having paging information release list. hence rapporteur recommends " PropReject " status for this RIL

# H451

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H451 | NR_SL_relay_multihop-Core | 1 | SIB/paging request information release at the relay UE when the connected child UE triggers the PC5 link release. |  | Huawei (Jagdeep) |  | V12 | PropAgree |

[Description]:Clarify in section 5.8.9.8.3 through a note that if the child UE triggers PC5 Link Release with its parent UE, the parent UE needs to need to release the related SIB request information and paging request information of the directly/indirectly connected child UE(s).

[Proposed Change]: Add a note in Clause 5.8.9.8.3.

NOTE X:	If a connected child UE triggers PC5 link release with its parent UE, the parent UE need to release the related SIB request information and paging request information of the directly or indirectly connected child UE(s) via this link.

[Comments]:

[Rapporteur]: Agree to add a note in in 5.8.9.8.3 to clarify the PC5 Link Release triggered by the Child UE as suggested above . Have changed the status from “ToDo” to “PropAgree”.

[Apple]: We do not agree to add a NOTE for this. If this parent is the Last U2N relay UE, then this is same behaviour as Rel-17, then we either change from Rel-17 or do nothing. If this parent is intermediate relay UE, then the intermediate relay UE is mandated to trigger RemoteUEInformaitonSidelink message and some normative text change is required in 5.8.9.8.2.

# O504

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O504 | NR_SL_relay_multihop-Core | 1 | For Intermediate relay UE, the Paging/SIB associated with the downstream remote UEs may comes from Parent instead of the network | R2-25xxxxx | OPPO (Bingxue Leng) |  | V004 | PropAgree |

[Description]: The Paging/SIB/posSIB acquisition at the Intermediate Relay UE, it may from the Parent UE instead of the network.

[Proposed Change]:

1>	upon receiving Paging message related to the connected L2 U2N Remote UE or the Child UE from network (including Paging message within RRCReconfiguration message) or Parent UE;

1>	upon acquisition of the SIB(s) requested by the connected L2 U2N Remote UE or by the Child UE (as indicated in sl-RequestedSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated SIB(s) from network or Parent UE which has been requested by the connected L2 U2N Remote UE or by the Child UE;

1>	upon acquisition of the posSIB(s) requested by the connected L2 U2N Remote UE or by the Child UE (as indicated in sl-RequestedPosSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated posSIB(s) from network or Parent UE which have been requested by the connected L2 U2N Remote UE or by the Child UE;

1>	upon unsolicited SIB1 forwarding to the connected L2 U2N Remote UE or by the Child UE or upon receiving the updated SIB1 from network or Parent UE;

For each associated L2 U2N Remote UE or for each associated Child UE, the L2 U2N Relay UE shall set the contents of UuMessageTransferSidelink message as follows:

1>	include sl-PagingDelivery if the Paging message received from network or Parent UE containing the ue-Identity of the L2 U2N Remote UE;

[Comments]:

[Rapporteur]: Agree to add the clarification with the change as below. Have changed the status from “ToDo” to “PropAgree”.

[Apple]: To be consistent, we need use “Parent relay UE’

# O505

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O505 | NR_SL_relay_multihop-Core | 1 | Whether sl-PagingDelivery in multi-hop case needs to be a list for multipe child UEs | R2-25xxxxx | OPPO (Bingxue Leng) |  | V004 | ToDo |

[Description]: It is agreed the Paging request can be a list for signalling efficiency. The same logic can be followd for the Paging delivery in UuMessageTransfer.

[Proposed Change]:

1>	include sl-PagingDelivery/ sl-PagingDelivery-List if the Paging message(s) received from network or Parent UE containing the ue-Identity of the L2 U2N Remote UE(s);

[Comments]:

[Rapporteur]: Rapporteur recommends "ToDo" status for this RIL as this is a signalling optimisation. Companies can discuss it whether this is essential or not in their contributions.

[Apple] We support O005. We are fine to furthet discuss this based on company controbition.

# E029

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E029 | NR_SL_relay_multihop-Core | 1 | Unclear text on Reception of the UuMessageTransferSidelink by the L2 Intermediate U2N Relay UE |  | Ericsson - Min |  | V009 | PropAgree |

[Description]:

##### 5.8.9.9.X	Reception of the UuMessageTransferSidelink by the L2 Intermediate U2N Relay UE

Upon receiving the UuMessageTransferSidelink message from the connected L2 U2N Parent Relay UE, the L2 Intermediate U2N Relay UE shall:

1>	if sl-PagingDelivery contains the ue-Identity of the child UEs:

2>		consider the paging message of the child UE is acquired;

1>	if sl-SystemInformationDelivery requested by the child UEs and/or sl-SIB1-Delivery is included:

2>		consider the SIB requested by the child UE is acquired;

Bullet 2) highlighted is not correct, since SIB1 may or may not be requested by child Ues,

[Proposed Change]:

in this case. We shall reformulate “2>		consider the SIB requested by the child UE is acquired;”

2>	consider the SIBs valid for child UEs is acquired;

[Comments]:

[Rapporteur]: Agree to clarify the it with slightly different wording as suggested above . Have changed the status from “ToDo” to “PropAgree”.

2>		consider the SIBs (other than SIB1) requested by the child UE is acquired

# X503

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X503 | SLRelay | 1 | Figure needs to be revised |  | Xiaomi (Shuai) |  | V005 | PropReject |

[Description]: Figure 5.8.9.8.1-1 should be revised to align with the below text “This procedure is used by a U2N Relay UE to send notification to the connected U2N Remote UE or to the connected child UE”.

[Proposed Change]: See below change.

Figure 5.8.9.8.1-1: Notification message in sidelink

This procedure is used by a U2N Relay UE to send notification to the connected U2N Remote UE or to the connected child UE, or used by a L2 U2U Relay UE to send notification to the L2 U2U Remote UE for an end-to-end PC5 connection when condition(s) as specified in 5.8.9.10.2 is met for the hop between the L2 U2U Relay UE and the peer L2 U2U Remote UE.

[Comments]:

[Rapporteur]: The proposed change to the figure is not needed as figure are maintained as in the previous release and the text below the figure is update extend the procedure to the multihop relay hence rapporteur recommends " PropReject " status for this RIL.

# J061

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J061 | NR_SL_relay_multihop-Core | 1 | Avoid Using ‘Connected Child UE’ to Prevent Confusion |  | Tsuboi (Sharp) |  | V019 |  |

[Description]: Connected is not needed since the definition of child UE is “Child UE: A U2N Relay UE’s next hop in downstream direction for serving a U2N Remote UE in U2N Relay communication.” If the term “connected child UE” is included in the specification, it should have a different meaning from “child UE.” However, the “connected child UE” described here does not carry such a special meaning and only causes unnecessary confusion.

“This procedure is used by a U2N Relay UE to send notification to the connected U2N Remote UE or to the connected child UE, or used by a L2 U2U Relay UE to send notification to the L2 U2U Remote UE for an end-to-end PC5 connection when condition(s) as specified in 5.8.9.10.2 is met for the hop between the L2 U2U Relay UE and the peer L2 U2U Remote UE.”

[Proposed Change]: It is suggested to remve “connected” from “connected child UE”.

[Comments]:

# O506

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O506 | NR_SL_relay_multihop-Core | 1 | Notification trigger at the Intermediate Relay UE | R2-25xxxxx | OPPO (Bingxue Leng) |  | V004 | PropReject |

[Description]: RRC connection failure case is also applicable to Intermediate Relay UE. Consideirng many trigger conditions for single hop can be reused for multihop, we can capture the trigger conditions together instead of duplicated capturing of the trigger conditions.

[Proposed Change]:

##### 5.8.9.10.2	Initiation

The Relay UE may initiate the procedure when one of the following conditions is met:

1>	if the UE is acting as U2N Relay UE:

2>	upon Uu RLF as specified in 5.3.10;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon cell reselection;

2>	upon L2 U2N Relay UE's or Last U2N Relay UE’s RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

2>	upon relay reselection;

2>	upon cell selection;

2>	upon PC5 RLF with its parent relay UE;

2>	upon reception of an NotificationMessageSidelink from the parent while in RRC_CONNECTED;

1>	if the UE is acting as L2 U2U Relay UE:

2>	upon detection of PC5 RLF for the hop between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.3;

2>	upon PC5-RRC connection release for the per-hop link between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.5;

Note 1: The Notification Message may not be sent by an Intermediate U2N relay UE in RRC_IDLE or RRC_INACTIVE to its child UEs if the relay reselection or cell selection does not cause the change of the serving cell.

##### 5.8.9.10.3	Actions related to transmission of NotificationMessageSidelink message

The Relay UE shall set the indication type as follows:

1>	if the UE is acting as U2N Relay UE:

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF:

3>	set the indicationType as relayUE-Uu-RLF;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync:

3>	set the indicationType as relayUE-HO;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to cell reselection:

3>	set the indicationType as relayUE-CellReselection;

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RRC connection establishment/Resume failure:

3>	set the indicationType as relayUE-Uu-RRC-Failure;

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to relay reselection:

3>	set the indicationType as relayUE-RelayReselection;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to cell selection:

3>	set the indicationType as relayUE-CellSelection;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to PC5 RLF with its parent Relay UE:

3>	set the indicationType as relayUE-PC5-RLF;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message upon reception of the NotificationMessageSidelink message from the parent relay UE:

3>	set the indicationType as received from the parent relay UE;

2>	submit the NotificationMessageSidelink message to lower layers for transmission.

[Comments]:

[Rapporteur]: Combining the trigger conditions and actions for the Last and intermediatie relay UE will be confusing hence Rapporteur recommends " PropReject " status for this RIL and suggests to keep them separate.

# B100

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B100 | NR_SL_relay_multihop-Core | 1 | One agreed case for intermediate relay in idle/inactive is missing | R2-25xxxxx | Lenovo (Lianhai Wu) |  | V011 | ToDo |

[Description]: It was agreed in RAN2#131 meeting that ‘Notification by an intermediate relay UE at least in idle/inactive, when caused by an upstream reselection/RLF/link release, occurs upon the intermediate relay UE’s handling after receiving the notification or release message (e.g., relay reselection or cell selection), but not triggered by the reception of the upstream notification itself.’ That means the intermediate relay UE at least in idle/inactive can trigger notification message after receiving release message from upstream link. This agreed case is not captured in RRC CR for SL relay.

[Proposed Change]: at least 5.8.9.10.2, 5.8.9.10.3 and the value of indicationtype IE should be updated. Below shows the update for 5.8.9.10.2. We will submit a contribution to show more changes.

5.8.9.10.2	Initiation

……

1>	if the UE is acting as Intermediate U2N Relay UE:

2>	upon relay reselection;

2>	upon cell selection;

2>	upon PC5 RLF with its parent relay UE;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon reception of an NotificationMessageSidelink from the parent while in RRC_CONNECTED;

2> upon PC5 unicast link release indicated by upper layer at Intermediate U2N Relay UE while in RRC_IDLE or RRC_INACTIVE;

[Comments]:

[Rapporteur]: This can be handled by the upper layer at the intermediate Relay UE which in turn can release PC5 inicast link release with its child UEs rather than sending a notification message and mixing higher layer and AS layers notification. However this can be further discussed in the next meeting based on the contribution from the proponent. Rapporteur recommends "ToDo" status for this RIL.

# B101

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B101 | NR_SL_relay_multihop-Core | 1 | Intermediate relay in connected state transmits notification message after receiving release message | R2-25xxxxx | Lenovo (Lianhai Wu) |  | V011 | ToDo |

[Description]: It was agreed in RAN2#131 meeting that ntification by an intermediate relay UE in idle/inactive can be triggered after receiving the release message. WE also need to discuss whether notification message by an intermediate relay UE in connected state can be triggered after receiving the release message.

[Proposed Change]: 5.8.9.10.2, 5.8.9.10.3 and the value of indicationtype IE should be updated. Below shows the change for 5.8.9.10.2. We will submit a contribution to show more changes.

5.8.9.10.2	Initiation

……

1>	if the UE is acting as Intermediate U2N Relay UE:

2>	upon relay reselection;

2>	upon cell selection;

2>	upon PC5 RLF with its parent relay UE;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon reception of an NotificationMessageSidelink from the parent while in RRC_CONNECTED;

2> upon PC5 unicast link release indicated by upper layer at Intermediate U2N Relay UE;

[Comments]:

[Rapporteur]: Similar to B100. This can be handled by the upper layer at the intermediate Relay UE which in turn can release PC5 inicast link release with its child UEs rather than sending a notification message and mixing higher layer and AS layers notification. However this can be further discussed in the next meeting based on the contribution from the proponent. Rapporteur recommends "ToDo" status for this RIL.

# B102

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B102 | NR_SL_relay_multihop-Core | 1 | The intermediate relay UE is triggered to transmit notification messaage due to connection failure | R2-25xxxxx | Lenovo (Lianhai Wu) |  | V011 | ToDo |

[Description]: In legacy single hop relay operation, the relay UE may initiate the procedure for Notification Message upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5. The same case will occur in the intermedidate relay UE. Therefore, we need to discuss this case.

[Proposed Change]:

Both 5.8.9.10.2 and 5.8.9.10.3 shoul be updated. Below shows the change for 5.8.9.10.2. We will submit a contribution for this.

5.8.9.10.2	Initiation

……

1>	if the UE is acting as Intermediate U2N Relay UE:

2>	upon relay reselection;

2>	upon cell selection;

2>	upon PC5 RLF with its parent relay UE;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon reception of an NotificationMessageSidelink from the parent while in RRC_CONNECTED;

2> upon Intermediate U2N Relay UEs RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

[Comments]:

[Rapporteur]: The network will usually not reject the RRC connection or Resuming of the RRC Connection for the remote UEs as the will be indirectly connected to the network hence these cases are only relavent for the U2N Relay UE in single hop case an Last Relay UE in multi hop case. However the companies are invited to discuss this issue in the contribution.     .

# O507

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O507 | NR_SL_relay_multihop-Core | 1 | Missing Notification trigger at the intermediate relay UE | R2-25xxxxx | OPPO (Bingxue Leng) |  | V004 | PropReject |

[Description]:PC5 link release case is missed, and for the Notification reception from the parent, seems no reason to only restrict to CONNECTED case.

[Proposed Change]:

##### 5.8.9.10.2	Initiation

The Relay UE may initiate the procedure when one of the following conditions is met:

1>	if the UE is acting as U2N Relay UE or Last U2N Relay UE:

2>	upon Uu RLF as specified in 5.3.10;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon cell reselection;

2>	upon L2 U2N Relay UE's or Last U2N Relay UE’s RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

1>	if the UE is acting as Intermediate U2N Relay UE:

2>	upon relay reselection;

2>	upon cell selection;

2>	upon PC5 RLF or PC5-RRC connection release with its parent relay UE;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon reception of an NotificationMessageSidelink from the parent;

1>	if the UE is acting as L2 U2U Relay UE:

2>	upon detection of PC5 RLF for the hop between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.3;

2>	upon PC5-RRC connection release for the per-hop link between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.5;

Note 1: The Notification Message may not be sent by an Intermediate U2N relay UE in RRC_IDLE or RRC_INACTIVE to its child UEs if the relay reselection or cell selection does not cause the change of the serving cell.

##### 5.8.9.10.3	Actions related to transmission of NotificationMessageSidelink message

The Relay UE shall set the indication type as follows:

1>	if the UE is acting as U2N Relay UE or Last U2N Relay UE:

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF:

3>	set the indicationType as relayUE-Uu-RLF;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync:

3>	set the indicationType as relayUE-HO;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to cell reselection:

3>	set the indicationType as relayUE-CellReselection;

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RRC connection establishment/Resume failure:

3>	set the indicationType as relayUE-Uu-RRC-Failure;

1>	if the UE is acting as Intermediate U2N Relay UE:

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to relay reselection:

3>	set the indicationType as relayUE-RelayReselection;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to cell selection:

3>	set the indicationType as relayUE-CellSelection;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to PC5 RLF or PC5-RRC connection release with its parent Relay UE:

3>	set the indicationType as relayUE-PC5-RLF;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync:

3>	set the indicationType as relayUE-HO;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message upon reception of the NotificationMessageSidelink message from the parent relay UE:

3>	set the indicationType as received from the parent relay UE;

2>	submit the NotificationMessageSidelink message to lower layers for transmission.

[Comments]:

[Rapporteur]: PC5 link release will be handled by the upper layers hence we don’t need to capture it explicitly in the specs and for the restricting sending of the ntification message for CONNECTED case is needed otherwise it will result in double notification for the IDLE or INACTVE intermediate relay UE. hence Rapporteur recommends " PropReject " status for this RIL.

# X504

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X504 | SLRelay | 1 | Missed words |  | Xiaomi (Shuai) |  | V005 | PropAgree |

[Description]: In clause 5.8.9.10.2, “relay UE” is missed after “parent”.

[Proposed Change]: See below change.

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon reception of an NotificationMessageSidelink from the parent relay UE while in RRC_CONNECTED;

[Comments]:

[Rapporteur]: Agree to add “relay UE” after “parent”.. Have changed the status from “ToDo” to “PropAgree”.

# H454

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H454 | NR_SL_relay_multihop-Core | 1 | Whether the intermediate relay UE in RRC_IDLE/INACTIVE can omit sending the notification message should be limited to the case where the hop count does not exceed the maximum permited value. |  | Huawei (Jagdeep) |  | V12 | ToDo |

[Description]: RAN2 agreed the intermediate relay UE in RRC_IDLE/INACTIVE may omit sending the notification message if the relay reselection or cell selection (from multihop to direct path for the intermediate relay) does not cause the change of the serving cell, and does not preclude the implementation considering other parameters such as hop count if they are made visible to AS layer by implementation. Consequently we had captured a note in section 5.8.9.10.2

Note 1: The Notification Message may not be sent by an Intermediate U2N relay UE in RRC_IDLE or RRC_INACTIVE to its child UEs if the relay reselection or cell selection does not cause the change of the serving cell.

However this omition should ensure that the hop count has not increased . In some cases the hop count on performing relay reselection might increase and may exceeds the maximum allowed limit for directly or indirectly connected child UEs. Hence it is suggested to modify the the note.

[Proposed Change]:

##### 5.8.9.10.2	Initiation

The Relay UE may initiate the procedure when one of the following conditions is met:

1>	if the UE is acting as U2N Relay UE or Last U2N Relay UE:

2>	upon Uu RLF as specified in 5.3.10;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon cell reselection;

2>	upon L2 U2N Relay UE's or Last U2N Relay UE’s RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

1>	if the UE is acting as Intermediate U2N Relay UE:

2>	upon relay reselection;

2>	upon cell selection;

2>	upon PC5 RLF with its parent relay UE;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon reception of an NotificationMessageSidelink from the parent while in RRC_CONNECTED;

1>	if the UE is acting as L2 U2U Relay UE:

2>	upon detection of PC5 RLF for the hop between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.3;

2>	upon PC5-RRC connection release for the per-hop link between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.5;

Note 1: The Notification Message may not be sent by an Intermediate U2N relay UE in RRC_IDLE or RRC_INACTIVE to its child UEs if the relay reselection or cell selection does not cause the change of the serving cell and does not result in increase of the hop count for the connected child UEs.

[Comments]:

[Rapporteur]: The need for additional clarification about the hop count as suggested in the note above can be discussed further considering different scenarios. Companies are are invited to discuss this issue in the contribution. The Status of this RIL is set to “ToDo”.

# K005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K005 | NR_SL_relay_multihop-Core | 1 | Behavior of Intermediate U2N Relay UE in response to reception of NotificationMessageSidelink |  | ASUSTeK (Richard Kuo) |  | V007 | PropReject |

[Description]: The behavior of the Intermediate U2N Relay UE to send the NotificationMessageSidelink message to its child UE upon reception of a NotificationMessageSidelink message from its parent UE while in RRC_CONNECTED is missing in 5.8.9.10.4. Alternatively, a similar subclause as 5.8.9.9.X could be added to describe the Intermediate U2N Relay UE behavior so as to align the Intermediate U2N Relay UE behaviors in different procedures.

[Proposed Change]:

[Comments]:

[Rapporteur]: In 5.8.9.10.4 this is not needed as the intermediate relay UE needs to set the set the indicationType as received from the parent relay UE as described in 5.8.9.10.3 and the child UE on reciving the notification message shall initiate the RRC connection re-establishment procedure as described in 5.8.9.10.4. hence rapporteur recommends " PropReject " status for this RIL

# O508

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O508 | NR_SL_relay_multihop-Core | 1 | Discovery transmission condition | R2-25xxxxx | OPPO (Bingxue Leng) |  | V004 | ToDo |

[Description]: In legacy, for discovery transmission, the threshold condition (for both Uu and PC5) are checked for each discovery transmission. For MH-U2N Relay, the following condition is defined and should be captured in the specification properly:

For the last relay UE, 1) Uu lower bound is defined (same value as single-hop U2N Relay UE); 2) PC5 threshold is defined for Model-B respond message transmission if there is no PC5 connection with the child node;

For the intermediate relay UE, 1) Uu upper bound is defined (same value as single-hop U2N Remote UE); 2) PC5 threshold is defined for Model-B solicitation message transmission.

[Proposed Change]:

A UE capable of NR sidelink discovery that is configured by upper layer to transmit NR sidelink discovery message shall:

1>	if the frequency used for NR sidelink discovery is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message; or if the frequency used for NR sidelink discovery is included in sl-FreqInfoList within SIB12:

2>	if the UE is in RRC_CONNECTED and uses the frequency included in sl-ConfigDedicatedNR within RRCReconfiguration message:

3>	if the UE is acting as NR sidelink U2N Relay UE and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2N Relay UE conditions as specified in 5.8.14.2 are met based on sl-RelayUE-Config; or

3>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE/ configured with measurement object associated to L2 U2N Relay UEs in both single hop or multi hop case and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-Config; or

3>	if the UE is acting as Last U2N Relay UE and sl-DiscConfig is included in RRCReconfiguration, and if the Last U2N Relay UE conditions as specified in 5.8.14.2 are met based on sl-RelayUE-Config when the UE has the PC5 connection with the Child UE; Or if the UE acting as Last U2N Relay UE is sending Discovery Response message with Model B as specified in TS 23.304 [65], and if sl-DiscConfig is included in RRCReconfiguration, and if the Last U2N Relay UE UE threshold condition as specified in 5.8.14.2 and 5.8.XX.2 are met based on sl-RelayUE-ConfigCommon and sl-RelayUE-ConfigCommonMH when the UE has no PC5 connection with the Child UE; or

3>		if the UE is acting as Intermediate U2N Relay UE and sl-DiscConfig is included in RRCReconfiguration, and if the U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-ConfigCommon when the UE has the PC5 connection with the Parent UE; Or if the UE acting as Intermediate U2N Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304 [65] and sl-DiscConfig is included in RRCReconfiguration, and if the U2N Remote UE threshold conditions as specified in 5.8.15 are met based on sl-RemoteUE-ConfigCommon and the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-RelayUE-ConfigMH; or

…

2>	else if the cell chosen for NR sidelink discovery transmission provides SIB12:

3>	if the UE is acting as NR sidelink U2N Relay UE and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2N Relay UE threshold conditions as specified in 5.8.14.2 are met based on sl-RelayUE-ConfigCommon in SIB12; or

3>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE in both single hop or multi hop case and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-ConfigCommon in SIB12; or

3>	if the UE is acting as Last U2N Relay UE and sl-DiscConfigCommon is included in SIB12, and if the Last U2N Relay UE conditions as specified in 5.8.14.2 are met based on sl-RelayUE-Config when the UE has the PC5 connection with the Child UE; Or if the UE acting as Last U2N Relay UE is sending Discovery Response message with Model B as specified in TS 23.304 [65], and if sl-DiscConfig is included in RRCReconfiguration, and if the Last U2N Relay UE UE threshold condition as specified in 5.8.14.2 and 5.8.XX.2 are met based on sl-RelayUE-ConfigCommon and sl-RelayUE-ConfigCommonMH when the UE has no PC5 connection with the Child UE; or

3>	if the UE acting as Intermediate U2N Relay UE has an established PC5 link with the selected parent U2N Relay UE, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-ConfigCommon in SIB12; or

3>	if the UE acting as Intermediate U2N Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304 [65] and sl-DiscConfigCommon is included in SIB12, and if the U2N Remote UE threshold conditions as specified in 5.8.15 are met based on sl-RemoteUE-ConfigCommon and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-RelayUE-ConfigCommonMH; or

…

1>	else if out of coverage on the concerned frequency for NR sidelink discovery:

2>	if the UE is acting as L3 U2N Relay UE; or

2>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE in both single hop or multi hop case and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-PreconfigDiscConfig in SidelinkPreconfigNR; or

2>	if the UE acting as Intermediate U2N Relay UE has an established PC5 link with the selected parent U2N Relay UE, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-PreconfigDiscConfig in SidelinkPreconfigNR; or

2>	if the UE acting as Intermediate U2N Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304 [65] and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-PreconfigDiscConfig in SidelinkPreconfigNR and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-PreconfigDiscConfig in SidelinkPreconfigNR; or

[Comments]:

[Rapporteur]: In Rapporteur view it is unclear why the Last Relay UE needs to chek if the UE has the PC5 connection with the Child UE.  Similarly for the intermediate relay UE does not need to check U2N Remote UE threshold conditions for transmitting the discovery message when the UE has no PC5 connection with the Parent UE. We can discuss this further and contributions from companies are invited on this aspect. The RIL Status is set to “ToDo”

# Z459

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z459 | NR_SL_relay_multihop-Core | 1 | Prioritization between last relay UE and intermediate relay UE | R2-25xxxxx | ZTE (Weiqiang Du) |  | V009 | PropReject |

[Description]: In case the Uu threshold for intermediate relay UE and last relay UE is not configured or only Uu threshold for last relay UE is configured, we think UE should prioritize to operate as a last relay UE

[Proposed Change]: RAN2 is suggested to agrees that Capture in normative text to say that if both conditions for last relay UE operation and intermediate relay UE operation are met, UE shall prioritize to act as a last relay UE.

[Comments]: For approach 1 these conditions will not be met at the same time hence Rapporteur recommends " PropReject " status for this RIL

# H452

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H452 | NR_SL_relay_multihop-Core | 1 | The condition of discovery transmission |  | Huawei (Jagdeep) |  | V12 | ToDo |

[Description]:

U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s). Up to three L2 U2N Relay UEs (i.e. one Last U2N Relay UE and up to two Intermediate U2N Relay UEs including one First U2N Relay UE) can be configured for serving a L2 U2N Remote UE in multi-hop L2 U2N Relay communication in this release.

We believe that the UE need to consider the hop count limit when performing the discovery message, which is four in this release (one Last U2N Relay UE and up to two Intermediate U2N Relay UEs).

For example, in model A, if the hop count of the current relay UE is already four (the maximum number specified in the current version of the specification) and performs the discovery, a remote UE may choose the path of through this relay UE. However, in this case, the hop count will already exceed the maximum value and the remote UE will not be allowed to access the network through this relay UE.

Similarly in model B, a candidate relay UE may receive the solicitation message from another relay UE (named it relay UE1) which is forwarding the Solicitation message of a remote UE. If the sum of the hop count of the candidate relay UE and the hop count between the relay UE1 and the remote UE exceeds the maximum value, the remote UE will not be able to access the network either.

For the two cases above the discovery messages will be transmitted unnecessarily even though the maximum permitted hop count has been exceeded. Hence it is necessary that the UE checks the hop limit before performing discovery message transmission.

[Proposed Change]:

#### 5.8.13.3	NR sidelink discovery transmission

A UE capable of NR sidelink discovery that is configured by upper layer to transmit NR sidelink discovery message shall:

1>	if the frequency used for NR sidelink discovery is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message; or if the frequency used for NR sidelink discovery is included in sl-FreqInfoList within SIB12:

- 2>	if the UE is in RRC_CONNECTED and uses the frequency included in sl-ConfigDedicatedNR within RRCReconfiguration message:

3>	if the UE is acting as NR sidelink U2N Relay UE or Last U2N Relay UE and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2N Relay UE or Last U2N Relay UE threshold conditions as specified in 5.8.14.2 are met based on sl-RelayUE-Config; or3>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE/ configured with measurement object associated to L2 U2N Relay UEs in both single hop or multi hop case and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-Config; or

3>	if the UE acting as Last U2N Relay UE is sending Discovery Response message with Model B as specified in TS 23.304 [65] and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-RelayUE-ConfigMH and if the sum of the hop count of the UE and the hop count information in the solicitation message from the sending UE is less than the maximum hop limit; or

3>	if the UE acting as Intermediate U2N Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304 [65] and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-RelayUE-ConfigMH and if the sum of the hop count of the UE and the hop count information in the solicitation message from the sending UE is less than the maximum hop limit; or

3>	if the UE acting as first U2N Relay UE is sending Announcement message with Model A as specified in TS 23.304 [65] and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-RelayUE-ConfigMH and if the hop count of the UE is less than the maximum hop limit; or

3>	if the UE is selecting NR sidelink U2U Relay UE / has a selected NR sidelink U2U Relay UE and sl-DiscConfig is included in RRCReconfiguration, and if the NR sidelink U2U Remote UE threshold conditions associated with the peer NR Sidelink U2U Remote UE as specified in 5.8.17.2 are met based on sl-RemoteUE-ConfigU2U; or

<omitted>

- 2>	else if the cell chosen for NR sidelink discovery transmission provides SIB12:

3>	if the UE is acting as NR sidelink U2N Relay UE or Last U2N Relay UE and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2N Relay UE or Last U2N Relay UE threshold conditions as specified in 5.8.14.2 are met based on sl-RelayUE-ConfigCommon in SIB12; or

3>	if the UE is selecting NR sidelink U2N Relay UE / has a selected NR sidelink U2N Relay UE in both single hop or multi hop case and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-ConfigCommon in SIB12; or

3>	if the UE acting as Intermediate U2N Relay UE has an established PC5 link with the selected parent U2N Relay UE, and if the NR sidelink U2N Remote UE threshold conditions as specified in 5.8.15.2 are met based on sl-RemoteUE-ConfigCommon in SIB12; or

3>	if the UE acting as Intermediate U2N Relay UE is sending Discovery Solicitation message with Model B as specified in TS 23.304 [65] and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-RelayUE-ConfigCommonMH and if the sum of the hop count of the UE and the hop count information in the solicitation message from the sending UE is less than the maximum hop limit; or

3>	if the UE acting as Last U2N Relay UE is sending Discovery Response message with Model B as specified in TS 23.304 [65] and sl-DiscConfigCommon is included in SIB12, and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-RelayUE-ConfigCommonMH and if the sum of the hop count of the UE and the hop count information in the solicitation message from the sending UE is less than the maximum hop limit; or

3>	if the UE acting as first U2N Relay UE is sending Announcement message with Model A as specified in TS 23.304 [65] and if the NR sidelink multi-hop relay threshold conditions as specified in 5.8.x.2 are met based on sl-RelayUE-ConfigMH and if the hop count of the UE is less than the maximum hop limit; or

[Comments]:

[Rapporteur]: Limiting of the discovery message transmission based on the maximum permited hop count in this release can be discussed further as it will improve system efficiency. Companies are are invited to discuss this issue in the contribution. The Status of this RIL is set to “ToDo”.

# E044

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E044 | NR_SL_relay_multihop-Core | 1 | The capturing of the note is not aligned with RAN2 agreements |  | Ericsson - Min |  | V012 | ToDo |

[Description]:

NOTE X:	The L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on any information available in the discovery message including the RRC State information. The RRC State information in the discovery message RRC container reflects the state of the UE that sends the discovery message.

The capturing of the above note is not aligned with RAN2 agreements

- It needs to capture that RRC state may be considered by remote UE in multihop relay scenario.

- RAN2 didn’t agree that remote UE can consider any other information in discovery, other than RRC state.

[Proposed Change]:

Suggest rewording the note as

NOTE X:	In case of multi-hop L2 U2N Relay communication, the L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on the RRC state information any information available in the discovery message including the RRC State information . The RRC State information in the discovery message RRC container reflects the state of the UE that sends the discovery message.

[Comments]:

[Rapporteur]: The note was modified based on the following agreement in RAN2 131 meeting.

Capture in a revision of the existing note in RRC running CR (relay reselection section 5.8.15.3) that the UE may prioritise relays based on any information available in the discovery message for the applicable model.

However it can be discussed if the note can be modified as suggested above. Rapporteur recommends "ToDo" status for this RIL.

# E046

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E046 | NR_SL_relay_multihop-Core | 1 | relayUE-RRCState is referred nowhere |  | Ericsson - Min |  | V012 | PropAgree |

[Description]:

relayUE-RRCState-r19 is not referred in the below note

NOTE X:	The L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on any information available in the discovery message including the RRC State information . The RRC State information in the discovery message RRC container reflects the state of the UE that sends the discovery message.

[Proposed Change]:

Suggest to update the note as

NOTE X:	The L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE based on any information available in the discovery message including the RRC State information relayUE-RRCState. The RRC State information in the discovery message RRC container reflects the state of the UE that sends the discovery message.

[Comments]:

[Rapporteur]: Agree to add relayUE-RRCState in the note as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# E019

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E019 | SONMDT | 1 | Incomplete parameter |  | Ali Parichehreh |  | V004 | Agreed |

[Description]: in 5.5a.3.2, there is a lack of clear description of the parameter AreaConfigurationNTN-List.

[Proposed Change]:

1>	if AreaConfigurationNTN-List is included in VarLogMeasConfig:

2> if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List in the VarLogMeasConfig; or

2>	if location information is not available:

[Comments]:

# O509

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O509 | NR_SL_relay_multihop-Core | 1 | No Discovery Solicitation Response message type |  | OPPO (Bingxue Leng) |  | V004 | ToDo |

[Description]: No Discovery Solicitation Response message type, so “Response” should be removed

[Proposed Change]:

1>	if the threshold conditions for sending the Discovery Solicitation message with Model B Discovery specified in this clause were previously not met:

[Comments]:

[Rapporteur]: Is the suggested change is to remove the word “Solicitation” and call it Discovery Response message as indicated belwo. If so the RIL status can be changed to “PropAgree”

1>	if the threshold conditions for sending the Discovery Response message with Model B Discovery specified in this clause were previously not met:

# Z458

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z458 | NR_SL_relay_multihop-Core | 1 | Upper bound for intermediate relay UE |  | ZTE (Weiqiang Du) |  | V009 | PropReject |

[Description]: RAN2 agreed that “The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.”. I believe this should be a new separate Uu threshold for intermediate relay UE, but corresponding new threshold is missing.

[Proposed Change]: Introduce a new separate Uu threshold for intermediate relay UE.

[Comments]:

[Rapporteur]: No seprate Uu threshold is needed for for intermediate relay UE as it can use the threshold condition for the remote UE to connect with the parent UE and to access the netwok. This agreement was made when both Approach 1 and Approach 2 were being discussed but is no longer needed with for Approach 1. Hence Rapporteur recommends " PropReject " status for this RIL

# E011

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E011 | NTN | 1 | Establishment/Release of MRBs following the ISA |  | Ericsson (Ignacio) |  | v001 | ToDo |

[Description]: Following RAN2 agreements, a UE may initiate the establishment or release when it enters/leaves the ISA of the MBS service in question. However, the text captured in 5.9.3.2 is not sufficient to ensure these limitations. For instance, there are some clauses such as “upon start of the MBS session” that would allow a UE to acquire establish the MRBs even outside the ISA..

[Proposed Change]: It is simpler to include the geofencing limitation in the general configuration of broadcast MRBs (section 5.9.3.1) so that it applies both to initial establishment/release but also to updates. Here an example:

#### 5.9.3.1	General

The broadcast MRB configuration procedure is used by the UE to configure PDCP, RLC, MAC and the physical layer upon starting and/or stopping to receive a broadcast MRB transmitted on MTCH, or upon modification of a configuration of a broadcast MRB received by the UE. The procedure applies to MBS capable UEs that are interested to receive or that are receiving an MBS broadcast service that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMTCH or searchSpaceMCCH and are located within the Intended Service Area associated with the MBS service, if any.

[Comments]:

[Qualcomm]We suggest this to change to ToDO. This should not be applicable to legacy UEs. It should be applicable to only UEs capable of supporting ISA and this is not clear to us.

# E014

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E014 | NTN | 1 | Clarificatory NOTE for the use of ISA in both SIB and USD to establish MRBs |  | Ericsson (Ignacio) |  | v001 | Agreed |

[Description]: Last meeting, RAN2 agreed to consider the Target Service Area for the purpose of geofencing MBS broadcast services in NTN. Following previous agreements related to the ISA, this information in USD can also be used to establish/release MRBs depending on whether the UE is location within or outside the Target Service Area.

[Proposed Change]: Include a general NOTE so that the UE can consider both sources of information to establish/release MRB(s).

[Comments]: RAN2 to consider the following TP:

NOTE 2:	It is up to UE implementation to use either the Target Service Area in the USD or the ISA(s) in SIBXX, if provided, for broadcast MRB configuration in NTN.

[vivo] We think the TSA is only needed for MCCH acquisition. The benefit of considering both resources for MRB management is unclear.

[Ericsson] In our understanding, the concept of ISA/TSA applies to MRB establishment. In case only TSA is provided via UDS, the UE should take this information into account.

# Z253

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z253 | NTN | 2 | Add SIBxx as on demand SI in DedicatedSIBRequest | No | ZTE (Zhihong) |  | v012 | Agreed |

[Description]: SIB20/21 can be on demand requested by connected UEs in DedicatedSIBRequest message, since ISAs in SIBxx is also essential for a UE capable MBS in NTN to acquire broadcast service, it is suggested to also allows UE to request this SIB in dedicatedSIBRequest message.

[Proposed Change]: Include SIBxx in DedicatedSIBRequest as below:

SIB-ReqInfo-r16 ::=                   ENUMERATED { sib12, sib13, sib14, sib20-v1700, sib21-v1700, sib23-v1810, sibxx-V1900, spare1 }

[Qualcomm] Shouldn’t we add SIB25 as well?

[Comments]:

# X553

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X553 | SONMDT | 1 | Incomplete field description |  | Xiaomi (Shuai) |  | V001 | Agreed |

[Description]: The definition of circleArea, distanceRadius IE is missed in field description part.

[Proposed Change]: Add the missed field description.

LoggedMeasurementConfiguration field descriptions

circleArea Used to describe a circle-shaped geographical area, in which include reference location and radius parameter.

distanceRadius Distance from the NTN coverage area reference location. Each step represents 50m.

[Comments]:

[Samsung] Field description needs to be added if it provides additional information which isnot clear from procedural text. So distanceRadius is ok, as the step is not provided. But we may need to double check if the step of 50m (total 3000km) is apt or not. For circleArea, the field description doesn’t add anything.

[Rapporteur] agree with the RIL. It is captured in the draft CR. Please note that in the online discussion in RAN2#131bis we have made some changes in the name of the IEs.

# E021

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E021 | SONMDT | 1 | Missing or wrong release version within AreaConfigurationNTN and MeasResult3NR |  | Ali Parichehreh |  | V004 | Agreed |

[Description]: Within information element AreaConfigurationNTN-r19 and MeasResult3NR-r19, the -r19 should be used.

[Proposed Change]:

AreaConfigurationNTN-r19 ::=   SEQUENCE {

areaCoordinates-r19             CHOICE {

polygonArea-r19                OCTET STRING,

circleArea-r19                 SEQUENCE {

referenceLocation-r19       ReferenceLocation-r17,

distanceRadius-r19          INTEGER (0..65535)

}

}

}

MeasResult3NR-r19 ::=                SEQUENCE {

ssbFrequency-r19                     ARFCN-ValueNR                                           OPTIONAL,

l1-MeasResultList-r19                L1-MeasResultList-r19                                   OPTIONAL,

...

}

[Comments]:

[Rapporteur]: Rapporteur agrees with the changes.

# X554

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X554 | SONMDT | 1 | Incomplete field description |  | Xiaomi (Shuai) |  | V001 | Rejected |

[Description]: The definition of referenceLocation IE in LoggedMeasurementConfiguration is missed in field description part, and the reference location should be restricted to (quasi-)Earth fixed cell. There are two reasons for this retriction: Firstly, reference location of earth-moving cell will change dynamicly and it can only be used when configured with epochTime. Secondly, the area scope checking concerning dynamic refeference location seems not reasonable due to NTN cells that serves the UE will change frequently.

[Proposed Change]: Add the missed field description.

LoggedMeasurementConfiguration field descriptions

referenceLocation Indicates the NTN coverage area reference location and is used in logged measurement for NTN (quasi-)Earth fixed cell.

[Comments]:

[Rapporteur]: RAN2 agreement on letting UE report the measured distances is only for earth moving scenario, and there is no enhancement for (quasi-) Earth fixed scenario according to RAN3 agreements from RAN3#125bis: For earth fixed case, there is no need to report the measured distance from the UE. However rapporteur agrees to have a field description for the sake of thoroughness without ((quasi-)Earth fixed cell).

As per offline discussion with the company provding this RIL, the status is changed from PropReject to ToDo, so the company is invited to bring contribution for this RIL for the online discussions.

[Rapporteur]: This RIL is rejected based on the online discussions in RAN2#131bis. As noted below we agreed to change the name of the geographical area scope fields to avoid confusions.

We rename “referenceLocation” to “circularAreaReferenceLocation” and “radius” to “circularAreaRadius”.

# N043

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N043 | SONMDT | 2 | Value range for distanceRadius-r19 in AreaConfigurationNTN-r19 |  | Nokia (Mani) |  | V005 | Agreed |

[Description]: The value range for distanceRadius-r19 in AreaConfigurationNTN-r19 in TS 38.331 is defined as INTEGER (0..65535). This is not aligned with RAN3 definition of “Distance Radius” in IE Geographical Area in TS 38.413. RAN3 had defined it as INTEGER(1..65535).

[Proposed Change]: Align 38.331 with 38.413 and change the min value of distanceRadius-r19 in AreaConfigurationNTN-r19 to “1”.

[Comments]:

[Rapporteur]: captured in the draft CR. RIL is considered closed.

# H303

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H303 | SONMDT | 1 | NTN deployment |  | Jun Chen |  | V004 | Rejected |

[Description]: For the following field description, the wording "NTN deployment" is confusing, and "NTN cells" is better.

areaConfigurationNTN-List

Used to restrict the geographic area in which the UE performs measurement logging for NTN deployment. The network does not configure areaConfiguration together with areaConfigurationNTN-List.

[Proposed Change]: In the field desription of areaConfigurationNTN-List, suggest to change "NTN deployment" into "NTN cells".

[Comments]:

[Rapporteur]: NTN deployment is already used in the RRC spec, while NTN cells is not used at all. We don’t have strong view/preference, but as of now we think change is not needed. Please bring contribution if you think this change is essential.

# H304

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H304 | SONMDT | 1 | areaConfigurationNTN-List |  | Jun Chen |  | V004 | Agreed |

[Description]: In section 6.2.2, for the following field description. We think the UE behaviours may also need to be added into the Note 1 below:

areaConfigurationNTN-List

Used to restrict the geographic area in which the UE performs measurement logging for NTN deployment. The network does not configure areaConfiguration together with areaConfigurationNTN-List.

NOTE 1:	The UE should perform measurement logging based on the following area configuration limitations:

-	If the areaConfiguration-r16/areaConfiguration-r17 is present, and the cag-ConfigList is absent, the UE should perform logging in both PN and PNI-NPN based on areaConfiguration-r16/areaConfiguration-r17, if any;

-	If the areaConfiguration-r17 and the cag-ConfigList are present simultaneously, the UE should perform logging in PN within the areaConfig-r16/areaConfig-r17 and perform logging in PNI-NPN within cag-ConfigList;

-	If the snpn-ConfigList is present, the UE should perform logging only in SNPN based on snpn-ConfigList. The snpn-ConfigList should not be configured together with PN or PNI-NPN area configurations.

[Proposed Change]: Our suggestion is:

NOTE 1:	The UE should perform measurement logging based on the following area configuration limitations:

-	If the areaConfiguration-r16/areaConfiguration-r17 is present, and the cag-ConfigList is absent, the UE should perform logging in both PN and PNI-NPN based on areaConfiguration-r16/areaConfiguration-r17, if any;

-	If the areaConfiguration-r17 and the cag-ConfigList are present simultaneously, the UE should perform logging in PN within the areaConfig-r16/areaConfig-r17 and perform logging in PNI-NPN within cag-ConfigList;

-	If the snpn-ConfigList is present, the UE should perform logging only in SNPN based on snpn-ConfigList. The snpn-ConfigList should not be configured together with PN or PNI-NPN area configurations.

-	If the areaConfigurationNTN-List is present, the UE should perform logging only in this area configuration. The areaConfigurationNTN-List should not be configured together with areaConfiguration.

[Comments]:

[Rapporteur]: Agree with the RIL and captured in the draft CR.

# C071

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C071 | AIML | 2 | Cond Sync |  | Tangxun |  | V003 | Agreed |

[Description]: “retainLoggedMeasurements-r19” can only be configured for UE in case of handover. So a conditional presence should be added.

[Proposed Change]: add conditional presence “Cond Sync” for “retainLoggedMeasurements-r19” as below:

RRCReconfiguration-v19xy-IEs ::=        SEQUENCE {

otherConfig-v19xy                       OtherConfig-v19xy                                                  OPTIONAL, -- Need M

retainLoggedMeasurements-r19            ENUMERATED {true}                                                  OPTIONAL, -- Cond Sync

nonCriticalExtension                    SEQUENCE {}                                                        OPTIONAL

}

| Conditional Presence | Explanation |
| --- | --- |
| Sync | The field is optionally present, Need N, upon reconfiguration with reconfigurationWithSync. It is absent otherwise. |

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# O510

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O510 | NR_SL_relay_multihop-Core | 1 | Clarification on how to understand the Paging message included in dedicatedPagingDelivery | R2-25xxxxx | OPPO (Bingxue Leng) |  | V004 | PropAgree |

[Description]: It is described in the field description of dedicatedPagingDelivery: This field is used to transfer Paging message for the associated L2 U2N Remote UE or for the associated child UE to the L2 U2N Relay UE or to L2 Last U2N Relay UE in RRC_CONNECTED. While it is not clear about how to understand the Paging message included in dedicatedPagingDelivery:

1/If only the directly connected remote UE’s Paging message can be include, then new IE for delivering of the indirectly connected remote UE’s paging is needed;

2/If both directly and indirectly connected remote UE’s Paging message can be included, the “or for the associated child UE” can be removed and rely on associated L2 U2N Remote UE to cover both directly and indirectly connected remote UEs

[Proposed Change]: The second option is preferred:

dedicatedPagingDelivery

This field is used to transfer Paging message for the associated L2 U2N Remote UE to the L2 U2N Relay UE in case of single hop or to L2 Last U2N Relay UE in RRC_CONNECTED.

[Comments]:

[Rapporteur]: Agree to gowith option 2 as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# Z254

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z254 | NTN | 1 | Missing on demand SIBXX in dedicatedSystemInformationDelivery | No | ZTE (Zhihong) |  | v012 | Agreed |

[Description]: Missing on demand SIBxx carrying ISA(s) in the field description of dedicatedSystemInformationDelivery in RRCReconfiguration message

[Proposed Change]: Update the description as below:

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8, SIB19, SIB20, SIB21, SIB25, SIBxx to the UE with an active BWP with no common search space configured or the L2 U2N Remote UE in RRC_CONNECTED. For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is also used to transfer the SIBs requested on-demand.

[Comments]:

# Z160

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z160 | MOB | 1 | Clarification on the sk-counter contained in the ltm-CandidateConfig for SCG LTM |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: For the SCG LTM configuration provided in LTM-ConfigNRDC, the sk-counter is provided via SK-CounterConfigLTM, so the RRCReconfiguration contained in the ltm-CandidateConfig should not include the field sk-counter, similar to subsequent CPAC.

[Proposed Change]: To clarify that the sk-counter is absent for the RRCReconfiguration message contained in the ltm-CandidateConfig within ltm-ConfigNRDC.

sk-Counter

A counter used upon initial configuration of S-KgNB or S-KeNB, as well as upon refresh of S-KgNB or S-KeNB. This field is always included either upon initial configuration of an NR SCG or upon configuration of the first RB with keyToUse set to secondary, whichever happens first. This field is absent if there is neither any NR SCG nor any RB with keyToUse set to secondary, or if the RRCReconfiguration message is contained in condRRCReconfig for subsequent CPAC, or if the RRCReconfiguration message is contained in ltm-CandidateConfig within ltm-ConfigNRDC.

[Comments]:

[Huawei] Ok.

# N024

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N024 | AIML | 1 | NW-side buffer is implied to be datatype specific but it is supposed to be general. | N/A | Jerediah Fevold |  | vnnn | ToDo |

[Description]: The field name implies that the buffer can be reported or emptied at the granularity of the datatype. However, that has never been agreed.

[Proposed Change]:

RRCReconfigurationComplete-v19xy-IEs ::=    SEQUENCE {

applicabilityReportList-r19                 ApplicabilityReportList-r19                                             OPTIONAL,

nw-DC-LogMeasAvailable-r19                    ENUMERATED {true}                                                       OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                             OPTIONAL

}

[Comments]:

[Huawei-Dawid-v017]: Disagree with Nokia comment. There was a RAN2 agreement: “To define field names and IE based on the content of the logged data rather than the specific use case”

[Ericsson-v022]: Agree with Huawei’s comment.

[Nokia-v030]: NW-side DC isn’t a use case. The point being made here is that only a signle buffer was agreed. Emptying the buffer by datatype doesn’t make sense. The gNB is aware of all the data collection it configures and we agreed on use-case and datatype agnostic reporting of buffer status. Therefore, we do not agree that csi-LogMeasAvailable and the associated indication of additional data availability, and the request for data are use-case and datatype agnostic in this implementation.

# V208

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V208 | NTN | 2 | Confirm that bitmap is used for UE reference location report | Yes, R2-250xxxx | vivo (Stephen) |  | v007 | Agreed |

[Description]: In the post email discussion, there is an argument on whether a bitmap or a reference location index should be used for the UE reference location report. With index indicating, the network can identify which one location is the nearest one or second-nearest one, based on the listed order in the reporting list. However, we don’t see index is beneficical. The network can configure the parameter N to control the number of locations that can be reported. Such information is sufficient for configuring N SMTCs for UE. The network doesn’t require which location is the nearest one.

[Proposed Change]: RAN2 confirms that bitmap of 6 bit is used for UE reference location report.

[Comments]:

ZTE: We still prefer to use index, and the reason is that even when NW configured UE to report 4, it is possible that NW can still have the possibility to configure UE with less SMTCs, e.g., 2 SMTCs. And report the reference Location in order in such scenarios is beneficial.

[Ericsson] We agree with vivo’s proposal.

[Rapp] The final reporting format be discussed in the next meeting. Please bring contributions addressing signaling such as message size and structure.

# Z006

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z006 | AIML | Class 1 | The field description of applicabilityReportList in RRCReconfigurationComplete |  | Fei |  | V015 | ToDo |

[Description]:

In the current field description of applicabilityReportList in RRCReconfigurationComplete

applicabilityReportList The applicability reports related to prediction configurations and sets of parameters for prediction configurations.

The applicability report list in RRCReconfigurationComplete only includes the applicability information, not include any parameter for prediction configurations...

#### –	ApplicabilityReportList

The IE ApplicabilityReportList comprises information that the UE reports to gNB related to the applicability of configurations subject to the applicability determination procedure.

ApplicabilityReportList information element

-- ASN1START

-- TAG-APPLICABILITYREPORTLIST-START

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..maxNrofServingCells) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19                  ServCellIndex,

applicabilityInfoReportList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports-r19)) OF ApplicabilityInfoReport-r19     OPTIONAL,

...

}

ApplicabilityInfoReport-r19 ::=    SEQUENCE {

applicabilityInfoReportId-r19          CHOICE {

csi-ReportConfigId-r19                          CSI-ReportConfigId,

applicabilitySetId-r19                     ApplicabilitySetConfigId-r19,

spare2                                     NULL,

spare1                                     NULL

},

applicabilityStatus-r19                        ENUMERATED {applicable, inapplicable},

releaseConfigurationPreference-r19             ENUMERATED {true}                                                          OPTIONAL,

...

}

-- TAG-APPLICABILITYREPORTLIST-STOP

-- ASN1STOP

[Proposed Change]:

The yellow highlighted wording shall be removed from the field description

[Comments]: Nokia. We do not need to update this as the following was agreed in RAN2#130:

(RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

[WI CR rapporteur-v022]: We agree with Nokia that we should not delete the highlighted part, which is intended to cover option B. However, to address the concern raised by ZTE and avoid confusion when reading the field description, we propose the following change instead.

applicabilityReportList The applicability reports related to prediction configurations and related to sets of parameters for prediction configurations.

# S058

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S058 | TEI19 (NR_TN_NTN_redir) | 1 | SMTC handling for NR TN to NR NTN redirection | R2-2507289 | Jonas (Samsung) | Related to LTE RIL X500 | V001 | ToDo |

[Description]:

As discussed in LTE TN to NR NTN for LTE RIL X500, how to handle the SMTC when redirecting to NR NTN is not clear. First of all, since it is not considered to be supported to measure an NR NTN frequency when the UE is connected to NR TN, then there is a question of how to handle the smtc, if the smtc is not configured in the CarrerInfo-v1900. In anticipation of potential future support of measuring NTN in TN, we believe that for this release, the field can be mandatory to signal.

Furthermore, the offset should be clarified to be based on the actual propagation delay.

[Proposed Change]: To follow the proposed change from X500 (or otherwise follow the approach from LTE TN to NR NTN):

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. In this release the field is mandatory present in CarrierFreqInfoNR-r19 and the corresponding offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE’s propagation delay difference between serving cell and neighbour cells equals to 0 ms, and the UE adjust the offset based on the actual propagation delay.

[Comments]:

# Z161

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z161 | MOB | 1 | Clarification on the selectedSK-Counter for SCG LTM |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: Upon the execution of SCG LTM, the UE may also include the selectedSK-Counter in the RRCReconfigurationComplete message. The SCG LTM case is missing in the current field description.

[Proposed Change]: To add the execution of SCG LTM.

selectedSK-Counter

This field includes the selected sk-counter value for security key update upon the execution of subsequent CPAC or SCG LTM.

[Comments]:

[Huawei] Ok.

# E010

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E010 | NTN | 2 | Add possibility for referenceLocationReport in the RRCResumeComplete message | R2-25xxxxx | Ericsson (Philipp) |  | v001 | ToDo |

[Description]: The current solution for UE-assisted SMTC selection in RRC_CONNECTED mode requires two RRC reconfigurations of the UE, for each UE upon each transition to RRC_CONNECTED mode. This may cause significant signaling overhead and pose a scalability issue for NTN, where cells are large and radio resources are extremely scarce. The additional, second RRC reconfiguration is needed, because at the time when the usual, first RRC reconfiguration is performed, the network does not yet know which are the relevant SMTCs to be configured for the UE.

[Proposed Change]: The above problem can be avoided for UEs transitioning from RRC_INACTIVE to RRC_CONNECTED mode by allowing the UEs to report the N closest reference locations, i.e., by allowing them to add referenceLocationReport, in the RRCResumeComplete message based on prior UE configuration. For UEs transitioning from RRC_INACTIVE to RRC_CONNECTED mode, AS security is enabled after reception of the RRCResumeRequest message by the network. Hence, the RRCResume and RRCResumeComplete message are subject to the same protection (i.e., cyphering and integrity protection) as the RRCReconfiguration and RRCReconfigurationComplete message.

This change enables the network to efficiently resume RRC connections of UEs (without RRC reconfiguration).

[Comments]:

[Qualcomm] This probably can be discussed to do it properly as the network may need to add new reference location list in RRCResume. Otherwise after being inactive so long, just reporting outdated reference location is not useful. Please change to ToDO.

# H305

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H305 | SONMDT | 1 | subsequent CPC |  | Jun Chen |  | V004 | Rejected |

[Description]: In section 6.2.2, the wording "subsequent CPC" has not been used in this TS 38.331, and instead "subsequent CPAC" is used.

previousPSCellId

This field indicates the physical cell id and carrier frequency of the cell that is the source PSCell of the last PSCell change. In case of PSCell addition failure, this field is absent. In case of subsequent CPC, this field indicates the physical cell identity and carrier frequency of the source PSCell associated to the last executed CPC.

[Proposed Change]: In the field description of previousPSCellId, it is suggested to change "subsequent CPC" into "subsequent CPAC".

[Comments]:

[Rapporteur] Here the subject matter is the CPC and not the CPA, so current phrasing would lower the risk for confusion.

Using the term CPAC would make the case of CPA failure fall into both the statement “In case of PSCell addition failure, this field is absent.” and the proposed statement “In case of subsequent CPAC …”. We therefore propose to keep the current phrasing, “in case of subsequent CPC”.

Note that we are using the term “Subsequent CPC” in the procedural text in 5.7.3.5.

So the RIL is rejected

# H050

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H050 | LPWUS | 1 | UAI for disabling LP-WUS | R2-25xxxxx | Kuang Yiru (Huawei) |  | V003 | ToDo |

[Description]: The NW only knows the MR measurement results based on existing RRC measurement report. Sometimes even when the MR measurement result is good, the LR can be bad due to the weaker tolerance for adjacent-channel interference. To avoid the LP-WUS missing and data loss in this case, it is beneficial to assist network for proper configuration since the UE is aware of the situation of LR. The UE can inform the network to stop using LP-WUS, or indicate whether the LP-WUS can be used again. It was discussed and postponed in the last RAN2 meeting.

[Proposed Change]: UE can send UAI to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again. The field of “preference on disabling LP-WUS” is added in UE assistance information message, e.g., use BOOLEAN. If the UE has a preference for disabling LP-WUS, set the field to true; otherwise (UE does not have a preference for disabling LP-WUS anymore), set the field to false.

[Comments]:

# N071

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N071 | AIML | 1 | Field applicabilityReportList description missing |  | Sakira Hassan |  | V015 | Agreed |

[Description]: applicabilityReportList description is missing from the UEAssistanceInformation description box. Adding the description clarifies that both inference configurations and inference related parameters can be reported in UAI.

[Proposed Change]: update the field description as below:

applicabilityReportList The applicability reports related to prediction configurations and sets of parameters for prediction configurations.

[Comments]:

[WI CR rapporteur-v022]: We do not think the proposed change is needed, since applicabilityReportList is described for RRCReconfigurationComplete.

# X251

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X251 | NTN | 2 | Providing the closest reference location in a list instead of bitmap | R2-25XXXX | Xiaomi (Xiaowei Jiang) |  | v010 | Duplicate V208 |

[Description]: The current ASN.1 design of UE reported assisted closest reference locations (i.e. referenceLocationReport-r19) is a bitmap. It doesn't allow network to differentiate the reference locations in the order of closeness. As a result, network has to always configure all the SMTCs for all the reported reference locations.

[Proposed Change]: Provide closest reference locations in the form of a list, the order of the reference location indicates the closeness/UE preference.

[Comments]:

[Rapp] To be discussed in the meeting.

# S045

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S045 | AIML | 2 | Correction on dataCollectionSart | R2-25xxxx | Seung-Beom |  | v026 | Rejected |

[Description]: We have the following agreement:

3 On stop/start indication - The UE can send start indication (without a preferred list) to indicate preference to start data collection - The UE can send preferred list implying that it would like to start data collection on those configuration - The UE can send stop indication for all or a given actual CSI report config ID. - Rapporteur will determine best way of signaling. This doesn’t preclude merging 1 and 2.

In our understanding, UE can indicate its preference for the following three cases via UAI:

Case 1) Absence of Start indication: UE has no preference to start data collection

Case 2) Presnece of Start indication “without” preferred list: UE has preference to start data collection, but none of given candidates is preferred

Case 3) Presnece of Start indication “with” preferred list: UE has preference to start data collection, and the indicated list among given candidates is preferred

Here, we want to clarify UE could have different preference for each serving cell. For example,

- For serving cell 1, UE has no preference to start data collection (i.e., Case 1), leading to absence of start indication

- For serving cell 2, UE has preference to start data collection (i.e., Case 2 or 3), leading to presence of start indication

However, according to the current ASN.1, the start indication is defined commonly to all serving cells. Therefore, there is no way for UE to indicate different start indication (i.e., absence vs. presence) for different cells.

[Proposed Change]:

Start indication is defined per serving cell, and corresponding TP could be also updated as follows:

UEAssistanceInformation-v19xy-IEs ::= SEQUENCE { applicabilityReportList-r19 ApplicabilityReportList-r19 OPTIONAL, dataCollectionPreference-r19 DataCollectionPreference-r19 OPTIONAL, loggedDataCollectionAssistance-r19 LoggedDataCollectionAssistance-r19 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } DataCollectionPreference-r19 ::= SEQUENCE { dataCollectionStart-r19 ENUMERATED {start} OPTIONAL, dataCollectionPreferredConfigurationList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19 OPTIONAL, dataCollectionStopConfigurationList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19 OPTIONAL, ... } DataCollectionCandidateList-r19 ::= SEQUENCE { dataCollectionServCellIndex-r19 ServCellIndex, dataCollectionStart-r19 ENUMERATED {start} OPTIONAL, dataCollectionCandidateIdList-r19 SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigId-r19 OPTIONAL } DataCollectionList-r19 ::= SEQUENCE { dataCollectionStopServCellIndex-r19 ServCellIndex, dataCollectionIdList-r19 SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId OPTIONAL }

#### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message The UE shall set the contents of the UEAssistanceInformation message as follows: <…> 1> if transmission of the UEAssistanceInformation message is initiated to report the UE preference to be configured with radio resources to perform UE-side data collection according to 5.7.4.2: 2> include dataCollectionPreference in this UEAssistanceInformation message; 2> if the UE prefers to be configured with radio resources to perform data collection: 3> for each serving cell for which UE prefers to be configured with radio resources to perform data collection: 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> set dataCollectionStart to start; 5> if dataCollectionCandidateConfigParameterList is configured for the serving cell and the UE has one or more preferred radio resource configuration(s) among the dataCollectionCandidateConfigParameterList: 6> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList; 3> set dataCollectionStart to start; 3> for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s): 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;

[Comments]: Nokia v030: We disagree with this change to the start indication. In our issue N114 we propose to remove the start indication all together since it doesn’t have an obvious use. If the NW has data collection configurations available for a UE, then it will provide a list. No further information is required.

[WI CR rapporteur-v031]: We suggest to discuss this in a Tdoc, together with RILs O301/N114.

# O301

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O301 | AIML | 2 | Missing purpose for UE-side data collection request | R2-250x | OPPO(Jiangsheng Fan) |  | V23 | Rejected |

[Description]: The ‘start’ indicator in UAI is just an overall info. In R19 for UE sided model, we have two use cases, i.e. BM and CSI prediction, and we also agree to introduce candidate configuration negotiation proceudure for both cases. The contents of candidate configuration for BM and CSI prediction are not the same (even we still check with RAN1 for CSI prediciton), at least, assocated ID info is not needed for CSI predicition. In this sense, only ‘start’ indicator does not make sense, as the NW has no idea which use case this ‘start’ indicator refers to.

[Proposed Change]:

Option 1: Define ‘start’ indicator per use case.

Option 2: replace ‘start’ indicator parameter by ‘intended data collection purpose’ parameter.

[Comments]:

[WI CR rapporteur-v031]: It is fine to discuss this in a Tdoc, as suggested by OPPO. Furthermore, it is suggested to discuss this together with S045/N114.

# A105

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A105 | AIML | 1 | Separate monitoring configuration for beam management and CSI prediction | R2-25xxxx | Peng Cheng (Apple) |  | V024 | Agreed |

[Description]: In existing implementation, BM and CSI prediction share the same monitoring configuration (configurationForChannelMonitoring-r19). However, we understand that separate parameters are needed for them:

- Only refToPredictionConfig-r19 and timeInstanceFor-SGCS-r19 are used for CSI prediction

- refToPredictionConfig-r19, nrofBestBeamForMonitoring-r19, nrofTransmissionOccasion-r19, timeInstanceFor-RS-PAI-r19 and mappingToResourcesForChannelPrediction-r19 are used for beam management.

The existing mixed structure between BM and CSI are difficulty to read. To improve the readness, we propose to have seperate monitoring configuration for beam management and CSI prediction.

configurationForChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId, [RIL]: C078, AIML

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}                                OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {n1, n2, n8, spare1}                             OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet))    OPTIONAL,   -- Need R

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}                     OPTIONAL,   -- Need R

...

}[RIL]: A105, AIML

[Proposed Change]: We provide suggested changes as follows:

predictionConfiguration-r19         CHOICE {

csi-InferencePrediction-r19         ENUMERATED {true},

configurationForChannelPrediction-r19   SEQUENCE {

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

nrofReportedPredicted-RS-r19                ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                                        OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL,   -- Need R

...

},

configurationForBMChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}                                OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {n1, n2, n8, spare1}                             OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet))    OPTIONAL,   -- Need R

...

}

configurationForCSIChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}                     OPTIONAL,   -- Need R

...

}

}

[Comments]:

ZTE: Agree to have this modification, make the usage in the choice structure more clear.

[WI CR rapporteur-v031]: We suggest to discuss this issue in a Tdoc, together with RILs N021/H003/S047.

# N114

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N114 | AIML | 2 | Remove dataCollectionStart and dataCollectionStopConfigurationList | R2-2507181 | Jerediah Fevold |  | v030 | Rejected |

[Description]: The purpose of dataCollectionStart is unclear. When it was first discussed, no other parameters were discussed. But then we introduced the list of candidate configurations and the list of preferred configurations. We think that by providing a list of preferences, it is implied that the UE wants to start data collection. Without indicating a preference, it isn’t clear with what the NW should configure the UE with regard to data collection. Then, the dataCollectionStopConfigurationList is also redundant because the UE can implicitly request to stop a configuration by sending the list of preferred configurations again, excluding the configuration to stop. If the UE can associate a CSI-ReportConfigId with a dataCollectionCandidateConfigId, the gNB can, too.

[Proposed Change]: Remove the fields dataCollectionStart and dataCollectionStopConfigurationList from DataCollectionPreference-r19.

[Comments]:

[WI CR rapporteur-v031]: We suggest to discuss this in a Tdoc, together with RILs O301/S045.

# N115

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N115 | AIML | 2 | Remove a level of hierarchy from DataCollectionPreference-r19 | R2-2507181 | Jerediah Fevold |  | v030 | ToDo |

[Description]: The SCell ID is signaled by storing dataCollectionCandidateConfigIds inside of SCell-specific lists. These data collection candidate configurations correspond to eventual CSI-ReportConfigs, which configure a field called carrier to store the SCellIndex. We think the same method can be used here.

[Proposed Change]: Signal the SCellIndex of a dataCollectionCandidateConfig inside the configuration itself and remove a level of hierarchy, signaling all the dataCollectionCandidateConfigs in a single list in DataCollectionPreference-r19.

[Comments]:

# O700

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O700 | LPWUS | 1 | The preference LP-WUS offset |  | Haocheng Wang (OPPO) |  | V007 | Agreed |

[Description]: The field description of lpwus-OffsetPreference should be for timeOffest instead of lpwus-OffsetPreference.

[Proposed Change]: Change the field name “lpwus-OffsetPreference” to “timeOffest”.

timeOffset

Indicates the UE's preferred time offset for PDCCH monitoring after LP-WUS monitoring. Value in ms (milliSecond). ms5 corresponds to 5 ms, ms13 corresponds to 13 ms, and ms37 corresponds to 37 ms. The reported preferred time offset value is equal to or longer than the minimum time gap reported by UE capability for a UE. If the field is absent, it is interpreted as the UE having no preference for the time offset for LP-WUS monitoring.

[Comments]:

# C031

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C031 | LPWUS | 2 | Empty UAI report for LP-WUS time offset | R2-25xxxxx | Da Wang (CATT) |  | V014 | Duplicate |

[Description]: We have no conclusion on the open issue [RRC-5], i.e. whether empty UAI on offset for LP-WUS monitoring is allowed. This should be discussed and corresponding procedures should be captured.

[Proposed Change]:

Change 1:

1>	if transmission of the UEAssistanceInformation message is initiated to provide lpwus-OffsetPreference of a cell group according to 5.7.4.2 or 5.3.5.3:

2>	include lpwus-OffsetPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on time offset for LP-WUS monitoring:

3>	set the timeOffset to the preferred offset value.

2>	else if the UE has no preference on time offset for LP-WUS monitoring of the cell group:

3>	do not include timeOffset in the lpwus-OffsetPreference;

Change 2:

timeOffset Indicates the UE's preferred time offset for PDCCH monitoring after LP-WUS monitoring. Value in ms (milliSecond). ms5 corresponds to 5 ms, ms13 corresponds to 13 ms, and ms37 corresponds to 37 ms. The value of timeOffset is equal to or longer than the minimum time gap reported by UE capability for a UE. If the field is absent, it is interpreted as the UE having no preference for the time offset for LP-WUS monitoring.

[Comments]:

# C028

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C028 | LPWUS | 1 | UAI reporting for UE’s preferred time offset for LP-WUS monitoring | No | Da Wang (CATT) |  | V012 | Agreed |

[Description]: UAI reporting for UE’s preferred time offset for LP-WUS monitoring is actually used to report the preferred time offset for RRC_CONNECTED. But this is not reflected in RRC.

[Proposed Change]:

lpwus-OffsetPreference Indicates the UE's preferred time offset for PDCCH monitoring after LP-WUS monitoring for RRC_CONNECTED mode. Value in ms (milliSecond). ms5 corresponds to 5 ms, ms13 corresponds to 13 ms, and ms37 corresponds to 37 ms. The reported preferred time offset value is equal to or longer than the minimum time gap reported by UE capability for a UE. If the field is absent, it is interpreted as the UE having no preference for the time offset for LP-WUS monitoring [RIL]: O701, LPWUS.

[Comments]:

# W201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W201 | LPWUS | 1 | UAI report for LP-WUS time offset | R2-25xxxxx | Rao Shi (NEC) |  | V015 | Rejected |

[Description]: Even though RAN2 has agreed that “If configured, the UE can signal a preferred time offset via UAI signalling” in RAN2#129bis, during the historical discussion, it is not clear whether this report is applied to both LP-WUS option 1-1 and option 1-2. In our understanding, this UAI report can be applied to both, or at least should be clarified.

[Proposed Change]:

lpwus-OffsetPreference Indicates the UE's preferred time offset for PDCCH monitoring after LP-WUS monitoring. The preferred time offset is applicable to both LP-WUS operation option 1-1 and 1-2. Value in ms (milliSecond). ms5 corresponds to 5 ms, ms13 corresponds to 13 ms, and ms37 corresponds to 37 ms. The reported preferred time offset value is equal to or longer than the minimum time gap reported by UE capability for a UE. If the field is absent, it is interpreted as the UE having no preference for the time offset for LP-WUS monitoring.

[Comments]:

As for the definition of option 1-1 and 1-2, it can be further discussed whether to replace the name based on RAN2 further progress.

[WI Rapp]: As LP-WUS operation option 1-1 and option 1-2 can’t be configured simultaneously, so the lpwus-OffsetPreference is applicable to either option 1-1 or option 1-2.

# O701

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O701 | LPWUS | 1 | Whether UE can send report an empty preference offset is still FFS. | R2-25xxxxx | Haocheng Wang (OPPO) |  | V007 | Rejected |

[Description]: Since there is no conclusion on whether UE can report an empty preference offset, we should delete the description on timeoffset is absent. We can add the make the corresponding change upon we have the clear conclusion.

[Proposed Change]: Delete the sentence of “If the field is absent, it is interpreted as the UE having no preference for the time offset for LP-WUS monitoring.” at least now.

lpwus-OffsetPreference

Indicates the UE's preferred time offset for PDCCH monitoring after LP-WUS monitoring. Value in ms (milliSecond). ms5 corresponds to 5 ms, ms13 corresponds to 13 ms, and ms37 corresponds to 37 ms. The reported preferred time offset value is equal to or longer than the minimum time gap reported by UE capability for a UE.

[Comments]:

# Z052

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z052 | LPWUS | 1 | Correction or empty UAI for LP-WUS time offset | R2-25xxxxx | Tao Qi (ZTE) |  | V013 | Rejected |

[Description]: Based on the current field description, the absence of the lpwus-OffsetPreference field will be interpreted as the UE having no preference for the time offset for LP-WUS monitoring, it is not correct.

UEAssistanceInformation may be used for other assistance information reporting except LP-WUS time offset, in which case lpwus-OffsetPreference need not be included.

One understanding is, lpwus-OffsetPreference field is included, but timeOffset-r19 field is absent, it is interpreted as the UE having no preference for the time offset for LP-WUS monitoring, i.e., as a release indication of previous preference.

One alternative is to add a separate field description of timeOffset to indicate network understanding of its absence, similar to DRX-Preference and IEs inside DRX-Preference.

[Proposed Change]:

UEAssistanceInformation field descriptions

...

lpwus-OffsetPreference Indicates the UE's preferred time offset for PDCCH monitoring after LP-WUS monitoring. Value in ms (milliSecond). ms5 corresponds to 5 ms, ms13 corresponds to 13 ms, and ms37 corresponds to 37 ms. The reported preferred time offset value is equal to or longer than the minimum time gap reported by UE capability for a UE. If the field timeOffset-r19 is absent from lpwus-OffsetPreference-r19, it is interpreted as the UE having no preference for the time offset for LP-WUS monitoring.

[Comments]:

# N025

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N025 | AIML | 1 | NW-side buffer is implied to be datatype specific but it is supposed to be general. | N/A | Jerediah Fevold |  | vnnn | ToDo |

[Description]: The field name implies that the buffer can be reported or emptied at the granularity of the datatype. However, that has never been agreed.

[Proposed Change]:

UEInformationRequest-v19xy-IEs ::= SEQUENCE {

nw-DC-LogMeasReportReq-r19         ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             SEQUENCE {}                              OPTIONAL

}

[Comments]:

[Huawei-Dawid-v017]: Disagree with Nokia comment. There was a RAN2 agreement: “To define field names and IE based on the content of the logged data rather than the specific use case”

[Ericsson-v022]: Agree with Huawei’s comment.

# Z007

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z007 | AIML | Class 1 | The field description of CSI-LogMeasReportReq | R2-25xxxx | Fei |  | V017 | ToDo |

[Description]:

According to RAN2 agreements achieved in RAN2#131bis meeting

- Multiplexing of legacy SON/MDT report and AIML logged data is not supported in the same UE information response message.  Up to the network to ensure that data is not requested at the same time

But there is nowhere to capture this agreement in the RRC spec so that the data from SON/MDT and NW side data collection can be retrieved simultaneously from specification perspective

In our understanding, it shall be captured in the field description of CSI-LogMeasReportConfig.

[Proposed Change]:

csi-LogMeasReportReq

This field is used to indicate whether the UE shall report information about CSI radio measurements logged in RRC_CONNECTED for network-side data collection. This information element shall be absent if the logMeasReportReq is present.

[Comments]:

[WI CR rapporteur-v022]: We suggest to discuss this issue in a Tdoc. There were discussions in the past on whether there should be clear restrictions on the NW, by adding them in the field descriptions but some companies were not in favour of it. In the current specification, not allowing multiplexing is captured in the procedural text from clause 5.7.10.3 below, by using “else if”, which excludes the case of having both SON/MDT reports and AIML reports in the same UEInformationResponse message. It can be discussed further if this procedural text is sufficient or not to capture the RAN2 agreement.

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else if csi-LogMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRBX;

2>	discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[Samsung-Beom-v028]: We think the procedure text is not enough. It could be interpretated NW is allowed to request both data and then UE inlcudes both data and transmits them via SRB2. So, we also suggest capture some text in field description. We are okay with Z007, but can be reworded in accordance to the agreement:

csi-LogMeasReportReq

This field is used to indicate whether the UE shall report information about CSI radio measurements logged in RRC_CONNECTED for network-side data collection. Network shall not include both csi-LogMeasReportReq and logMeasReportReq in the same message.

# N026

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N026 | AIML | 2 | NW-side buffer is implied to be datatype specific but it is supposed to be general. | N/A | Jerediah Fevold |  | vnnn | ToDo |

[Description]: In the future, other use cases could share the same buffer. If we use separate IEs to report each datatype, then the indication of further data availability will need to be signaled in each type for which more data is available. This would have limited utility, though, since the gNB presumably wants all of the data which it configured for collection. Therefore, a higher-level report IE needs to store the different types as a choice (Option 1), or the data availability indicator needs to be moved outside of the report IE (Option 2).

[Proposed Change]:

UEInformationResponse-v19xy-IEs ::=  SEQUENCE {

nw-DC-LogMeasReport-r19              NW-DC-LogMeasReport-r19             OPTIONAL, (Option 1)

nw-DC-logMeasAvailable-r19           ENUMERATED {true}                   OPTIONAL, (Option 2)

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

NW-DC-LogMeasReport-r19 ::=  SEQUENCE {

logMeasReportType-r19 CHOICE {

csi-LogMeasReport-r19    CSI-LogMeasReport-r19,

spare3                   NULL,

spare2                   NULL,

spare1                   NULL

},

nw-DC-logMeasAvailable-r19           ENUMERATED {true}                   OPTIONAL,

}

[Comments]:

[Huawei-Dawid-v017]: Disagree with Nokia comment. There was a RAN2 agreement: “To define field names and IE based on the content of the logged data rather than the specific use case”. We can discuss how to implement other use cases once these use cases get specified in Rel-20

[Ericsson-v022]: Agree with Huawei’s comment.

# Z008

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z008 | AIML | Class 2 | Size optimization for measurement result list for logged data reporting |  | Fei |  | V017 | Agreed |

[Description]:

In the current measurement report list in UEInformationResponse, it has been defined in as below:

CSI-LogMeasInfoCell-r19 ::=          SEQUENCE {

cellId-r19                              CHOICE {

cellGlobalId-r19                        CGI-Info-Logging-r16,

pci-arfcn-r19                           PCI-ARFCN-NR-r16

},

csi-LogMeasInfoList-r19                    SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF CSI-LogMeasInfo-r19,

...

}

CSI-LogMeasInfo-r19 ::=              SEQUENCE {

refCSI-LoggedMeasurementConfigId-r19    CSI-LoggedMeasurementConfigId-r19,

csi-RS-MeasResultList-r19               SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-RS-MeasResult-r19    OPTIONAL,

ssb-MeasResultList-r19                  SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-MeasResult-r19                       OPTIONAL,

timeGap-r19                          ENUMERATED {true}                                       OPTIONAL,    ...

}

For each instance of CSI-LogMeasInfo, the refCSI-LoggedMeasurementConfigId-r19 shall be reported which dramatically increases the signaling overhead. It is suggested to report only one refCSI-LoggedMeasurementConfigId with a number of measurement results from this refCSI-LoggedMeasurementConfigId.

[Proposed Change]:

CSI-LogMeasInfoCell-r19 ::=          SEQUENCE {

cellId-r19                              CHOICE {

cellGlobalId-r19                        CGI-Info-Logging-r16,

pci-arfcn-r19                           PCI-ARFCN-NR-r16

},

csi-LogMeasInfoMeasConfigList-r19                    SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LogMeasInfoMeasConfig-r19,

...

}

CSI-LogMeasInfoMeasConfig-r19 ::=              SEQUENCE {

refCSI-LoggedMeasurementConfigId-r19    CSI-LoggedMeasurementConfigId-r19,

csi-LogMeasInfoList-r19                    SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF CSI-LogMeasInfo-r19,

...

}

CSI-LogMeasInfo-r19 ::=              SEQUENCE {

refCSI-LoggedMeasurementConfigId-r19    CSI-LoggedMeasurementConfigId-r19,

csi-RS-MeasResultList-r19               SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-RS-MeasResult-r19    OPTIONAL,

ssb-MeasResultList-r19                  SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-MeasResult-r19                       OPTIONAL,

timeGap-r19                          ENUMERATED {true}                                       OPTIONAL,    ...

}

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”. Please note that there are other updates to the proposed changes that we will need to make as rapporteurs on top of the proposed ones, e.g. to align the procedural text with the new ASN.1.

# S046

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S046 | AIML | 2 | Content of NSDC data |  | Seung-Beom |  | v026 | ToDo |

[Description]: According to current ASN.1 structure,

- Issue 1. UE includes refCSI-LoggedMeasurementConfigId-r19 redundantly (i.e., per every CSI-LogMeasInfo-r19)

- Issue 2. UE includes resourceId-r19 redundantly (i.e., per every l1-RSRP-r19)

[Proposed Change]: As the issue 1 has been addressed in Z008, we propose further update on Z008’s proposed text to resolve issue 2.

CSI-LogMeasInfoCell-r19 ::=          SEQUENCE {

cellId-r19                              CHOICE {

cellGlobalId-r19                        CGI-Info-Logging-r16,

pci-arfcn-r19                           PCI-ARFCN-NR-r16

},

csi-LogMeasInfoMeasConfigList-r19                    SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LogMeasInfoMeasConfig-r19,

...

}

CSI-LogMeasInfoMeasConfig-r19 ::=              SEQUENCE {

refCSI-LoggedMeasurementConfigId-r19    CSI-LoggedMeasurementConfigId-r19,

csi-RS-MeasResultList-r19               SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-RS-MeasResult-r19    OPTIONAL,

ssb-MeasResultList-r19                  SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-MeasResult-r19                       OPTIONAL,

...

}

CSI-RS-MeasResult-r19 ::=            SEQUENCE {

resourceId-r19                       NZP-CSI-RS-ResourceId,

rsrp-Results-r19					SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF RSRP-Result-r19

}

SSB-MeasResult-r19 ::=               SEQUENCE {

ssb-Id-r19                           SSB-Index,

rsrp-Results-r19 					SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF RSRP-Result-r19

}

RSRP-Result-r19 ::=            SEQUENCE {

l1-RSRP-r19 						RSRP-Range

timeGap-r19                         ENUMERATED {true}                                       OPTIONAL,    ...

}

[Comments]:

# S023

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S023 | SONMDT | 1 | Clarify ra-purpose for reconfigurationWithSync |  | Samsung (Aby) |  | V006 | Agreed |

[Description]:

The indicator reconfigurationWithSync is used if the UE executes a reconfiguration with sync in the field description need to exclude MCG LTM.

[Proposed Change]: Updated field description as below to exclude MCG LTM. SCG LTM RA-Report will still use the reconfigurationWithSync till MRO for SCG LTM is specified

raPurpose

This field is used to indicate the RA scenario for which the RA report entry is triggered. The RA accesses associated to Initial access from RRC_IDLE, RRC re-establishment procedure, transition from RRC-INACTIVE. The indicator beamFailureRecovery is used in case of successful beam failure recovery related RA procedure in the SpCell [3]. The indicator reconfigurationWithSync is used if the UE executes a reconfiguration with sync except MCG LTM cell switch. The indicator ltm is used if the UE executes a RACH-based LTM cell switch. The indicator ulUnSynchronized is used if the random access procedure is initiated in a SpCell by DL or UL data arrival during RRC_CONNECTED when the timeAlignmentTimer is not running in the PTAG or if the RA procedure is initiated in a serving cell by a PDCCH order [3]. The indicator schedulingRequestFailure is used in case of SR failures [3]. The indicator noPUCCHResourceAvailable is used when the UE has no valid SR PUCCH resources configured [3]. The indicator requestForOtherSI is used for MSG1 based on demand SI request. The indicator msg3RequestForOtherSI is used in case of MSG3 based SI request. The indication lbtFailure is used when the UE initiates RACH in SpCell due to consistent uplink LBT failures [3]. The field can also be used for the SCG-related RA-Report when the raPurpose is set to beamFailureRecovery, reconfigurationWithSync, ulUnSynchronized, schedulingRequestFailure, noPUCCHResourceAvailable and lbtFailure.

[Comments]:

# H306

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H306 | SONMDT | 1 | SDT failure cause |  | Jun Chen |  | V004 | Agreed |

[Description]: In section 6.2.2, the field description can be improved, and normally the purpose should be indicated in the beginning.

sdt-FailureCause

This field is included when the RA report entry is included because of SDT and if the SDT procedure failed. Otherwise, the field is absent. This field indicates the SDT failure cause.

[Proposed Change]: Proposed change is shown as below:

sdt-FailureCause

This field indicates the SDT failure cause and it is included when the RA report entry is included because of SDT and if the SDT procedure failed. Otherwise, the field is absent. This field indicates the SDT failure cause.

[Comments]:

[Rapporteur] agree and captured in the draft CR.

# C061

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C061 | SONMDT | 1 | Wrong field description |  | Tangxun |  | V002 | Agreed |

[Description]: the field description of “sdt-UL-DataVolume” seems to be a copy of sdt-FailureCause.

[Proposed Change]: update the field description of “sdt-UL-DataVolume” as below:

sdt-UL-DataVolume

This field logs the buffered data volume in the UE for the radio bearer configured for the SDT during evaluation of SDT procedure. This field is included when the RA report entry is included because of SDT initiation failure. Otherwise, the field is absent. Value in bytes, the maximum value 96000 means 96000 bytes or larger.

[Comments]:

[Samsung] Agree. There is also a small change needed from above text “radio bearers configured” instead of radio bearer configured.

[Rapporteur]: Agree, but there seems to be a copy-paste error which is fixed as well while accommodating this change.

# H307

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H307 | SONMDT | 1 | SDT UL data volume |  | Jun Chen |  | V004 | Agreed |

[Description]: In section 6.2.2, the text is duplicated from the field description of the IE sdt-FailureCause, and it should be updated.

sdt-UL-DataVolume

The field is set to t319a-expiry upon expiration of T319a timer. If the UE upper layers receive maxRetxThreshold reached indication from RLC while SDT procedure is ongoing, this field is set to maxRetxThreshold. It is set to preambleTransMax upon the UE upper layer receiving indication of reaching preambleTransMax from the MAC layer. Upon expiration of cg-SDT-TimeAlignmentTimer from the MAC, the field is set to cg-SDT-TimeAlignmentTimer. The field is set to configuredGrantTimer upon reception of indication that configuration grant timer has been expired from the MAC. The field is set to cellReselection upon SDT failure due to UE’s cell re-selection.

[Proposed Change]: Suggest to:

sdt-UL-DataVolume

The field is set to indicate the UL data volume at the time of SDT evaluation as specified in TS 38.321 [3].

[Comments]:

[Samsung] We understand that this is an editing mistake and needs to be fixed according to C061 considering the endorsed CR and agreements.

[Rapporteur] Yes this happened when adjusting the width of the table (which in. Word required copy-paste from the row above it). I have now fixed it and addressed the C061 as well.

# H308

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H308 | SONMDT | 1 | Time since SDT execution |  | Jun Chen |  | V004 | Agreed |

[Description]: In section 6.2.2, the actual value may exceed the maximum one, so it should be clarified in the description below.

timeSinceSdt-Execution

This field logs the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds.

[Proposed Change]: Suggest to:

timeSinceSdt-Execution

This field logs the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds. If the time exceeds 172800 seconds, the UE shall set it to 172800 seconds.

[Comments]:

[Rapporteur] Rapporteur agrees with the proposal. Change is captured in the draft CR.

# X555

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X555 | SONMDT | 1 | Incomplete field description |  | Xiaomi (Shuai) |  | V001 | Rejected |

[Description]: distanceFromReference1, distanceFromReference2 IE only consider the case for earth-moving cell, the case that NTN cell is (quasi-)Earth fixed cell is missed.

[Proposed Change]: To complete the below field description when considering the reference location of (quasi-)Earth fixed cell is fixed.

RLF-Report field descriptions

distanceFromReference1 This field indicates the measured distance between UE and the fixed reference location of the serving cell if the conditional handover is based on condEventD1. This field indicates the measured distance between UE and the moving reference locations of the serving cell if the conditional handover is based on condEventD2. Each step represents 50m. The actual distance shall be rounded down to the nearest step value (i.e., FLOOR(actual distance[m] / 50)). The maximum value is 65535, which indicates a distance equal to or greater than 65535 multiplied by 50m.

MeasResultNR field descriptions

distanceFromReference2 This field indicates the measured distance between UE and the fixed reference location of the serving cell if the conditional handover is based on condEventD1. This field indicates the measured distance between UE and the moving reference locations of associated neighbour cell if the conditional handover is based on condEventD2. Each step represents 50m. The actual distance shall be rounded down to the nearest step value (i.e., FLOOR(actual distance[m] / 50)). The maximum value is 65535, which indicates a distance equal to or greater than 65535 multiplied by 50m.

[Comments]:

# N044

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N044 | SONMDT | 1 | Definition of distanceFromReference1 in UEInformationResponse message |  | Nokia (Mani) |  | V005 | Agreed |

[Description]: The definition of distanceFromReference1 in UEInformationResponse message is not accurate.

[Proposed Change]:

distanceFromReference1

This field indicates the distance between UE and the moving reference locations of the serving cell if the conditional handover for NTN is based on condEventD2. Each step represents 50m. The actual measured distance shall be rounded down to the nearest lower step value. If the UE is within 50m of the moving reference location, the UE shall report a value of 0. The maximum value is 65535, which indicates a distance equal to or greater than 65535 multiplied by 50m.

[Comments]:

[Samsung] We think that current text is better. Floor function is applicable for all real numbers. So it should cover the case within 50m also (though it is not practical for NTN).

[Rapporteur]: I have tried to improve the text based on the RIL and the comment from Samsung. If you are not happy with the current version, feel free to bring a contribution so we discuss it further. As of now I consider the RIL agreed given that I have made some changes according to the RIL.

# C062

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C062 | SONMDT | 1 | Misalignment with procedural text |  | Tangxun |  | V002 | Agreed |

[Description]: according to procedural text, the field “pSCellId” can be set to “source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change)”. And it’s not only applicable to CHO with candidate SCG procedure failure scenario.

[Proposed Change]: update the field description of “pSCellId” to align with procedural text as below:

This field is used to indicate the PSCell in which the UE failed to perform fast MCG recovery procedure or the UE successfully performed fast MCG recovery procedure, or the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell.

[Comments]:

[Rapporteur]: agree with the changes, captured in the drafted CR.

# C063

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C063 | SONMDT | 1 | Co-existence of LTM and DAPS is not supported |  | Tangxun |  | V002 | Agreed |

[Description]: according to the following RAN2 agreement made in R18 Mobility WI, we don’t need to consider the co-existence scenario of LTM and DAPS

The coexistence of LTM and DAPS HO is not supported in Rel-18.

[Proposed Change]: update the field description of “timeSinceFailure” as below:

timeSinceFailure

This field is used to indicate the time that elapsed since the connection (radio link, handover or LTM cell switch) failure. Value in seconds. The maximum value 172800 means 172800s or longer. In the case of failure(s) (either at source or at target or at both) associated to DAPS handover, this field indicates the time elapsed since the latest connection (radio link or handover) failure.

[Comments]:

# E022

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E022 | SONMDT | 1 | Field description in RLF-Report and SuccessHO-Report |  | Ali Parichehreh |  | V004 | Agreed |

[Description]: In the field description for some of the elements within RLF-Report and SuccessHO-Report, the terminology “handover” should be change to “reconfiguration with sync” to cover both handover and LTM cell switch.

[Proposed Change]:

Some examples of the proposed change is listed below.

In RLF-Report:

failedPCellId

This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed reconfiguration with sync. For intra-NR reconfiguration with sync nrFailedPCellId is included and for the handover from NR to EUTRA eutraFailedPCellId is included. The UE sets the ARFCN according to the frequency band used for transmission/ reception when the failure occurred.

In SHR:

sourcePCellId

This field is used to indicate the source PCell of a reconfiguration with sync in which the successful handover triggers the SuccessHO-Report

[Comments]:

# H309

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H309 | SONMDT | 1 | pCellId |  | Jun Chen |  | V004 | Agreed |

[Description]: In section 6.2.2, the wording "Alternatively" is confusing, and it can be improvded.

pCellId

This field is used to indicate the PCell to which the UE was connected when the successful PSCell change or addition triggers the SuccessPSCell-Report. Alternatively this field indicates the source PCell to which the UE was connected to before executing CHO with candidate SCG in which the SuccessPSCell-Report was triggered.

[Proposed Change]: Suggest to:

pCellId

This field is used to indicate the PCell to which the UE was connected when the successful PSCell change or addition triggers the SuccessPSCell-Report. In addition, Alternatively this field indicates the source PCell to which the UE was connected to before executing CHO with candidate SCG in which the SuccessPSCell-Report was triggered.

[Comments]:

[Rapporteure]: the change does not seem necessary, but we are fine to capture it. RIL is agreed

# V106

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V106 | AIML | 1 | Better field description of cellid |  | Boubacar |  | V009 | Agreed |

[Description]: The field description of cellid is be update for better readability.

### 6.2.2

cellId

This field indicates the CGI of the cell in which the logging of the measurements included within csi-LogMeasInfoList was performed. If the CGI is not available, this field indicates the PCI-ARFCN-NR.

[Proposed Change]:

cellId

This field indicates the CGI of the cell in which the logging of the measurements included within csi-LogMeasInfoList was performed. If the CGI is not available, this field indicates the PCI-ARFCN-NR of the cell.

[Comments]:

[WI CR rapporteur-v020]: We changed the status from “ToDo” to “PropAgree”.

# H251

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H251 | NTN | 1 | SMTC for serving cell | R2-25xxxxx | Huawei (Lili) |  | V006 | ToDo |

[Description]: It was agreed to have 7 SMTCs altogether on a single frequency. Serving cell does not require a reference location, and in this case the legacy smtc is used for the serving cell measurement. However, this understanding is a bit different from legacy releases because smtc is now changed to a cell-specific SMTC rather than a frequency-specific SMTC. Also, the field description of smtc related to SMTC adjustment based on PDD needs to revised so that UE does not need to consider neighbour cell propagation delay.

[Proposed Change]:  Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequency cells. If the field is broadcast by an NTN cell and smtc5list is not configured, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. If the field is broadcast by an NTN cell and smtc5list is configured, smtc is for serving cell measurements and the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference equals to 0 ms.

[Comments]:

[Rapp] In our understanding, even if smtc5list is configured, there could be neighbours to be detected within legacy smtc. Thus, legacy text still applies.

# Q200

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q200 | NTN | 1 | RefLocList-r19 in SIB2 | Yes | Qualcomm (Bharat) |  | v022 | Agreed |

[Description]: Currently only fixed cell is supported as no moving reference location is defined. Either clarification should be added or we need to agree this is not supported in moving cell.

[Proposed Change]: in FD, Add clarification that for moving cell the reference location is associated with serving cell epoch time.

[Comments]:

# E013

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E013 | NTN | 2 | Maximum amount of reference locations for location-based SMTC selection | R2-25XXXX | Ericsson (Ignacio) |  | v001 | Agreed |

[Description]: Last meeting, RAN2 took the following agreement: “The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs)”. In our understanding, the overall sentiment in the last RAN2 meeting is that the network will configure 6 potential neighbour SMTCs and SMTC1 is used for the serving cell. Therefore, the network only needs to configure 6 reference locations, i.e., the serving cell does not need a reference location. RAN2 needs to decide whether a reference location for the serving cell is needed for the purpose of location-based SMTC selection feature.

[Proposed Change]: The maximum number of reference locations for location-based SMTC selection is 6. The serving cell, i.e., SMTC1, is excluded from SMTC selection.

[Comments]: We understand that the UE needs SMTC1 to keep track of the serving cell which always needs to be measured.

[vivo] We agree with 6 as the max size of the reference location list. In addition, the scenario where there are 7 detectable neighboring cells is not a common case in TN. And we believe this is even less common for the NTN scenario.

[Qualcomm] Sorry what is the purpose of “PropAgree” here? Current implementation in CR is max 6 neighbors, that’s already confirmed. An this RIL is for double confirmation?

# X250

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X250 | NTN | 2 | Broadcast of distance threshold together with reference location for neighbor cells in SIB2 | R2-25XXXX | Xiaomi (Xiaowei Jiang) |  | v010 | Rejected |

[Description]: As the cell size of neighbor cells can be different according to TR38.821, UE cannot decide which SMTC to select purely based on the closeness of neighbour cells, as UE may be out of coverage of the closest neighbour cell if it happens to be with a small cell size.

[Proposed Change]: Broadcast a distance threshold together with the reference location.

[Comments]:

[Rapp] Cells of different sizes has not yet been considered in NTN deployments, but it may be a reality in multi-orbit scenarios. To be discussed next meeting.

# V209

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V209 | NTN | 2 | Radius is also needed for UE-based SMTC selection | Yes, Re-250xxxx | vivo (Stephen) |  | v013 | Duplicate X250 |

[Description]: We think a radius field is needed for SMTC selection as it indicates the coverage area of the neighbouring cell..

[Proposed Change]: Introduce a radius field asspcaited with refLocList-r19.

[Comments]:

# A200

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A200 | NTN | 1 | Reference location and SMTC5list | Yes | Apple (Yuqin) |  | v017 | ToDo |

[Description]: The current assumption from CR rapporteur is SMTC in SIB2 is associated with reference location in SIB19, and it is used for serving cell measurement. We think it is not always the case. Note that when NW provides smtc in SIB2, it should associate with the largest SSB periodicity of serving and neighbor cell. For example, if serving cell has a SSB periodicity of 160ms, and neighbor cell has a SSB periodicity of 20ms, the smtc in SIB2 has to be with 160ms periodicity. That is, it is actually for neighbor cell measurement. And smtc5list can provide another periodicity of 20ms for serving cell measurement.

Therefore, we would like to clarify that there should be an explicit reference location associated with legacy smtc configured in SIB2.

[Proposed Change]: Allow smtc in SIB2 to associate with a reference location in RefLocList-r19.

[Comments]:

[Rapp] In our understanding, smtc is used for the serving cell. However, this needs to be clarified in RAN2 first.

# E035

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E035 | LPWUS | 2 | TimeToTrigger (TTT) for RRM relaxation entry | R2-25xxx | Ericsson (Martin) |  | V006 | Rejected |

[Description]: Similar as for SintraSearch/SnonIntraSearch there is no exit condition for Rel-19 RRM relaxation. There is a risk for ping-pong behavior which could increase the power consumption. Similar as for cell reselection, a TimeToTrigger (TTT) should be introduced to avoid this.

[Proposed Change]: Add the possibility to configure a TTT for the entry condition for Rel-19 RRM relaxation, similar as TreselectionRAT for cell reselection. For example a TTT to the RRM relaxation thresholds in SIB2.

SIB2 ::=                            SEQUENCE {

…

]],

[[

relaxedMeasurementForServingAndNeighboringCell-r19                  SEQUENCE {

…

cellEdgeEvaluationOnLR-ForLR-OnSSB-r19   SEQUENCE {

rsrpThresholdLR2-r19               ThresholdP-LR-r19,

rsrqThresholdLR2-r19               ThresholdQ-LR-r19                            OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

TimeToTrigger-r19                      T-Reselection                                OPTIONAL        -- Need R

}                                                                                       OPTIONAL,        -- Need R

offloadMeasurementForServingCell-r19                  SEQUENCE {

cellEdgeEvaluationOnMR-ForLR-OnSSB-r19   SEQUENCE {

s-SearchThresholdP5-r19                 ReselectionThreshold,

s-SearchThresholdQ5-r19                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                  OPTIONAL,    -- Cond SupportLR-OnSSB

…

}                                                                                    OPTIONAL    -- Cond SupportLR-OnSSB

}                                                                                       OPTIONAL        -- Need R

]]

}

Further details/update are provided in Tdoc. The value range may require further discussion.

[Comments]:

# N083

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N083 | NTN | 1 | Reference location mapping to SMTC configuration IE | R2-25xxxxx | Nokia (Jakob) |  | v016 | Rejected |

[Description]: refLocList indicates a reference location associated to an SMTC configuration in smtc5list, and it includes the same number of entries as smtc5list. However, the network might choose to cover different areas, with distinct traffic needs, with cells that are active more frequently than the others. These two cells can have SSBs with 160 ms periodicity for example, while the other cells might have 20 ms periodicity. If these cells belong to the same satellite, the UE can use the same SMTC to measure both of them.

[Proposed Change]: Define the refloc as a field in SMTC5 list. In case the SMTC is associated with different cells, the refloc contains multiple entries, and each entry maps to the neighbor cells in the order the PCI Is listed in the SMTC5 list.

[Comments]:

[Rapp] Thanks for the suggestion. This was one of the signalling designs proposed and considered for smtc5list. However, it does not allow a clear reporting using a bitmap since it would refer to an entry within a field in the entry of a list. Please note that the fact that two SMTC have the same periodicity does not imply that they can be measured with the same SMTC since that depends on the offset of the SSB transmission.

# O702

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O702 | LPWUS | 1 | Align the name of low power wake-up receiver. |  | Haocheng Wang (OPPO) |  | V007 | Agreed |

[Description]: we define the LR/LP-WUS for low power wake-up receiver. But in some field description, we use the low power reveiver. To avoid the confusion, align the name low power wake-up receiver.

[Proposed Change]: change the “low power receiver” to “LP-WUR” or “low power wake-up receiver”

cellEdgeEvaluationOnLR-ForLR-OnLPSS Indicates the criteria for a UE to detect that it is not at cell edge based on the serving cell measurement on LR for OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS, in order to relax serving cell and neighboring cell measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.x.2), or to offload serving cell measurement toLP-WUR (see TS 38.304 [20], clause 5.2.4.x.4).

cellEdgeEvaluationOnLR-ForLR-OnSSB Indicates the criteria for a UE to detect that it is not at cell edge based on the serving cell measurement on LR for OFDM based LP-WUR measuring on SSB, in order to relax serving cell and neighboring cell measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.x.2), or to offload serving cell measurement to LP-WUR (see TS 38.304 [20], clause 5.2.4.x.4).

cellEdgeEvaluationOnMR-ForLR-OnLPSS Indicates the criteria for a UE to detect that it is not at cell edge based on the serving cell measurement on main radio for OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS, in order to relax serving cell and neighboring cell measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.x.2), or to offload serving cell measurement to LP-WUR (see TS 38.304 [20], clause 5.2.4.x.4).

cellEdgeEvaluationOnLR-ForLR-OnLPSS-Exit Indicates the exit criteria for serving cell measurement offloading for a UE to detect that it is not at cell edge based on the serving cell measurement on LR for OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS. This field is optional present for the cell supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS.

cellEdgeEvaluationOnMR-ForLR-OnSSB Indicates the criteria for a UE to detect that it is not at cell edge based on the serving cell measurement on main radio for OFDM based LP-WUR measuring on SSB, in order to relax serving cell cell and neighboring cell measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.x.2), or to offload serving cell measurement to LP-WUR (see TS 38.304 [20], clause 5.2.4.x.4).

[Comments]:

# V203

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V203 | NTN | 1 | Refine the mapping between reference location and smtc4 and smtc5 | No | vivo (Stephen) |  | v007 | Rejected |

[Description]: Currently, the refLocList can only be associated with smtc5. In our understanding, the reference location should be allowed to be associated with smtc4. For example, the network may configure 3 smtc4 and 3 smtc5 of different periodicity, with 6 reference locations. For Rel-19 UE, the UE should know the detailed association between smtc4/smtc5 and the reference location.

[Proposed Change]: Refine the mapping between reference location and smtc4 and smtc5

refLocList

Indicates a reference location associated to an SMTC configuration in smtc5list. If present, it includes the same number of entries as smtc5list. The first entry in this list corresponds to the first entry across smtc4list and smtc5list, the second entry corresponds to the seccond entry across smtc4list and smtc5list, and so on.

[Comments]:

[Rapp] Following last meeting discussions, the solution adopted was the one proposed by Samsung since it was determined less complicated. The location list is associated with smtc5list, if an entry is absent, then the entry in smtc4list applies. In this way, the network can configure, for instance, 3 SMTC in smtc4list and configure its reference location in refLocList which would make the first three entries of stmc5list to be absent.

[Qualcomm] We suggest changing it to “ToDO”. We would like to discuss this issue. It should be possible that the RefLocList is configured and smtc5list is not configured.

# E008

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E008 | LPWUS | 1 | Restrictions to configure s-SearchThresholdP5 and P6. |  | Ericsson (Martin) |  | V002 | ToDo |

[Description]: In case the NW does not configure Rel-19 RRM relaxation with LP-WUS, then the NW should be allowed to configure MR serving cell offloading in the complete LP-WUS coverage area. Currently it says: The network configures both s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-IntraSearchP and s-NonIntraSearchP. See R2-2505857 for more details.

[Proposed Change]:

s-SearchThresholdP, s-SearchThresholdP2, s-SearchThresholdP3, s-SearchThresholdP4, s-SearchThresholdP5, s-SearchThresholdP6 Parameters "SSearchThresholdP", "SSearchThresholdP2", "SSearchThresholdP3", "SSearchThresholdP4", "SSearchThresholdP5", and "SSearchThresholdP6" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP. If the network configures both RRM serving cell relaxation (relaxedMeasurementForServingAndNeighboringCell) and RRM serving cell offloading (offloadMeasurementForServingCell) then the network configures both s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-IntraSearchP and s-NonIntraSearchP, if there is such configuration(s). The network configures s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-SearchThresholdP3 and s-SearchThresholdP4, respectively, if there is such configuration(s).

[Comments]:

This is a critical issue, i.e. if the NW only configures RRM offloading without RRM relaxation, then the UE can only use LP-WUS when it is above SintraSearch. This means that the use of LP-WUS is (artificially) restricted to less than at least half of the cell. This is not acceptable and there is also no technical motivation for such restriction. The serving cell restrictions were introduced because similar restrictions were captured for Rel-16 RRM relaxation. However the Rel-16 RRM relaxation is only for neighbour cell measurements, and here we talk about serving cell measurements.

As a compromise we can agree to keep the restriction, but it is only applicable when both RRM serving cell offloading and relaxation are configured, see above.

[WI Rapp] This has been discussed and concluded, current specification is captured based on the agreements. So I marked it as PropRejct before (now changed as ToDo), while proponent companies could raise proposals if really have concern. But I don’t want to reopen too much discussion unless critical issue identified.

# V002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V002 | LPWUS | 1 | The relationship on MR based entry conditions between RRM relaxation and LP-WUS monitoring | R2-25xxx | Vivo(Chenli) |  | V005 | Rejected |

[Description]: The MR-based threshold for RRM relaxation condition should be lower than or equal to the threshold of entry condition for LP-WUS monitoring. Otherwise, there is no power saving gain for LP-WUS monitoring.

[Proposed Change]: This restriction should be captured in the field description of RRM relaxation condition as below:

s-SearchThresholdP, s-SearchThresholdP2, s-SearchThresholdP3, s-SearchThresholdP4, s-SearchThresholdP5, s-SearchThresholdP6 Parameters "SSearchThresholdP", "SSearchThresholdP2", "SSearchThresholdP3", "SSearchThresholdP4", "SSearchThresholdP5", and "SSearchThresholdP6" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP. The network configures both s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-IntraSearchP and s-NonIntraSearchP, if there is such configuration(s). [RIL]: E008 LPWUS The network configures s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-SearchThresholdP3 and s-SearchThresholdP4, respectively, if there is such configuration(s). The network configures s-SearchThresholdP3 and s-SearchThresholdP4 to be smaller than or equal to thresholdP2 and thresholdP1, respectively, if there is such configuration(s).

s-SearchThresholdQ, s-SearchThresholdQ2, s-SearchThresholdQ3, s-SearchThresholdQ4, s-SearchThresholdQ5, s-SearchThresholdQ6 Parameters "SSearchThresholdQ" "SSearchThresholdQ2", "SSearchThresholdQ3", "SSearchThresholdQ4", "SSearchThresholdQ5", and "SSearchThresholdQ6" in TS 38.304 [20]. The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ. The network configures both s-SearchThresholdQ5 and s-SearchThresholdQ6 to be larger than or equal to s-IntraSearchQ and s-NonIntraSearchQ, if there is such configuration(s). The network configures s-SearchThresholdQ5 and s-SearchThresholdQ6 to be larger than or equal to s-SearchThresholdQ3 and s-SearchThresholdQ4, respectively, if there is such configuration(s). The network configures s-SearchThresholdQ3 and s-SearchThresholdQ4 to be smaller than or equal to thresholdQ2 and thresholdQ1, respectively, if there is such configuration(s).

[Comments]:

# V004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V004 | LPWUS | 1 | The relationship on MR based entry conditions between RRM offloading and LP-WUS monitoring | R2-25xxx | Vivo(Chenli) |  | V005 | Rejected |

[Description]: Considering the entry conditions of LP-WUS monitoring include at least serving cell quality via MR, serving cell measurement via MR cannot be offloaded to LR before UE starts the LP-WUS monitoring. Thus, the threshold of the entry condition for serving cell RRM offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

[Proposed Change]: This restriction should be captured in the field description of RRM offloading condition as below:

s-SearchThresholdP, s-SearchThresholdP2, s-SearchThresholdP3, s-SearchThresholdP4, s-SearchThresholdP5, s-SearchThresholdP6 Parameters "SSearchThresholdP", "SSearchThresholdP2", "SSearchThresholdP3", "SSearchThresholdP4", "SSearchThresholdP5", and "SSearchThresholdP6" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP. The network configures both s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-IntraSearchP and s-NonIntraSearchP, if there is such configuration(s). [RIL]: E008 LPWUS The network configures s-SearchThresholdP5 and s-SearchThresholdP6 to be larger than or equal to s-SearchThresholdP3 and s-SearchThresholdP4, respectively, if there is such configuration(s). The network configures s-SearchThresholdP5and s-SearchThresholdP6 to be larger than or equal to thresholdP2 and thresholdP1, respectively, if there is such configuration(s).

s-SearchThresholdQ, s-SearchThresholdQ2, s-SearchThresholdQ3, s-SearchThresholdQ4, s-SearchThresholdQ5, s-SearchThresholdQ6 Parameters "SSearchThresholdQ" "SSearchThresholdQ2", "SSearchThresholdQ3", "SSearchThresholdQ4", "SSearchThresholdQ5", and "SSearchThresholdQ6" in TS 38.304 [20]. The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ. The network configures both s-SearchThresholdQ5 and s-SearchThresholdQ6 to be larger than or equal to s-IntraSearchQ and s-NonIntraSearchQ, if there is such configuration(s). The network configures s-SearchThresholdQ5 and s-SearchThresholdQ6 to be larger than or equal to s-SearchThresholdQ3 and s-SearchThresholdQ4, respectively, if there is such configuration(s). The network configures s-SearchThresholdQ5 and s-SearchThresholdQ6 to be larger than or equal to thresholdQ2 and thresholdQ1, respectively, if there is such configuration(s).

[Comments]:

# V003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V003 | LPWUS | 1 | The relationship on LR based entry conditions between RRM relaxation and LP-WUS monitoring | R2-25xxx | Vivo(Chenli) |  | V005 | Rejected |

[Description]: The LR-based threshold for RRM relaxation condition should be lower than or equal to the threshold of entry condition for LP-WUS monitoring. Otherwise, there is no power saving gain for LP-WUS monitoring.

[Proposed Change]: This restriction should be captured in the field description of RRM relaxation condition as below:

rsrpThresholdLR, rsrpThresholdLR2, rsrpThresholdLR3, rsrpThresholdLR4, rsrpThresholdLR5, rsrpThresholdLR6 Parameters "SRSRPThresholdLR", "SRSRPThresholdLR2", "SRSRPThresholdLR3", "SRSRPThresholdLR4", "SRSRPThresholdLR5", and "SRSRPThresholdLR6" in TS 38.304 [20]. The network configures rsrpThresholdLR3 and rsrpThresholdLR4 to be larger than or equal to rsrpThresholdLR and rsrpThresholdLR2, respectively, if there is such configuration(s). The network configures rsrpThresholdLR and rsrpThresholdLR2 to be smaller than or equal to thresholdP3-LR and thresholdP1-LR, respectively, if there is such configuration(s).

rsrqThresholdLR, rsrqThresholdLR2, rsrThresholdLR3, rsrqThresholdLR4, rsrqThresholdLR5, rsrqThresholdLR6 Parameters "SRSRQThresholdLR", "SRSRQThresholdLR2", "SRSRQThresholdLR3", "SRSRQThresholdLR4", "SRSRQThresholdLR5", and "SRSRQThresholdLR6" in TS 38.304 [20]. The network configures rsrqThresholdLR3 and rsrqThresholdLR4 to be larger than or equal to rsrqThresholdLR and rsrqThresholdLR2, respectively, if there is such configuration(s). The network configures rsrqThresholdLR and rsrqThresholdLR2 to be smaller than or equal to thresholdQ3-LR and thresholdQ1-LR, respectively, if there is such configuration(s).

[Comments]:

# V005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V005 | LPWUS | 1 | The relationship on LR based entry conditions between RRM offloading and LP-WUS monitoring | R2-25xxx | Vivo(Chenli) |  | V005 | Rejected |

[Description]: Considering the entry conditions of LP-WUS monitoring include at least serving cell quality via LR, serving cell measurement via MR cannot be offloaded to LR before UE starts the LP-WUS monitoring. Thus, the threshold of the entry condition for serving cell RRM offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

[Proposed Change]: This restriction should be captured in the field description of RRM offloading condition as below:

rsrpThresholdLR, rsrpThresholdLR2, rsrpThresholdLR3, rsrpThresholdLR4, rsrpThresholdLR5, rsrpThresholdLR6 Parameters "SRSRPThresholdLR", "SRSRPThresholdLR2", "SRSRPThresholdLR3", "SRSRPThresholdLR4", "SRSRPThresholdLR5", and "SRSRPThresholdLR6" in TS 38.304 [20]. The network configures rsrpThresholdLR3 and rsrpThresholdLR4 to be larger than or equal to rsrpThresholdLR and rsrpThresholdLR2, respectively, if there is such configuration(s). The network configures rsrpThresholdLR3 and rsrpThresholdLR4 to be larger than or equal to thresholdP3-LR and thresholdP1-LR, respectively, if there is such configuration(s).

rsrqThresholdLR, rsrqThresholdLR2, rsrThresholdLR3, rsrqThresholdLR4, rsrqThresholdLR5, rsrqThresholdLR6 Parameters "SRSRQThresholdLR", "SRSRQThresholdLR2", "SRSRQThresholdLR3", "SRSRQThresholdLR4", "SRSRQThresholdLR5", and "SRSRQThresholdLR6" in TS 38.304 [20]. The network configures rsrqThresholdLR3 and rsrqThresholdLR4 to be larger than or equal to rsrqThresholdLR and rsrqThresholdLR2, respectively, if there is such configuration(s). The network configures rsrqThresholdLR3 and rsrqThresholdLR4 to be larger than or equal to thresholdQ3-LR and thresholdQ1-LR, respectively, if there is such configuration(s).

[Comments]:

# N085

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N085 | NTN | 1 | Selecting the number of SMTCs to consider | No | Nokia (Jakob) |  | v016 | ToDo |

[Description]: A UE may not only support less SMTCs than signalled in SMTC4 and SMTC5, but also only a single periodicity.

[Proposed Change]: Use “or” instead of and for SMTC4 and SMTC5 to be supported

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports only a single periodicity or less SMTCs than what is included in smtc4list or smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

[Comments]:

[Rapp] A UE that only supports one periodicity can safely ignore smtc5list in SIB2. In the MO, provided network has UE capabilities, this will not be configured. We agree with the intention that this should be somewhat captured.

# C006

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C006 | NTN | 1 | Clarify whether the R19 DL CE capable UEs perform measurement as configured in the SMTC4 | Yes, R2-250xxxxx | CATT (Da Wang) |  | v008 | Agreed |

[Description]: Based on the field description of smtc4list and smtc5list under SIB2, it is unclear whether the R19 DL CE capable UEs use the SMTC4 perform measurement as configured in the SMTC4.

We agreed that “The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs)”. The yellow highlight text means the R19 DL CE capable UE takes into account all the SMTCs in smtc4list and smtc5list. While the green highlight text means the SMTC5 explict configure SMTCs not configured in SMTC4 and implicit configure SMTCs configured in SMTC4, that the UE only take the entries in SMTC5 into account. There is a contradiction here and the total number of configurable SMTCs across smtc4list and smtc5list may exceed 6.

For instance, SMTC4={a,b,c} SMTC5={-,d,e,f,g,h}, the total number of configurable SMTCs across smtc4list and smtc5list is 9, which is against with the yellow highlight part.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

[Proposed Change]: remove the limitation of “The total number of configurable SMTCs across smtc4list and smtc5list is 6”. The maximum number configured SMTCs for idle/inactive is 7 can be restricted by the sequence length of SMTC5 naturally.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

[Comments]:

[Samsung]: we share the same view that the current sentence “The total number of configurable SMTCs across smtc4list and smtc5list is 6.” is not correct and should be removed.

[Rapp] It is uncertain why the sentence is not correct as it follows a RAN2 agreement. Please explain further.

Another reason why this change cannot be implemented like this relates to the refLocList, please see my reply to V203.

# C008

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C008 | NTN | 1 | Corrections on the smtc5list | Yes, R2-250xxxxx | CATT (Da Wang) |  | v019 | Agreed |

[Description]: Same as RIL C006.

[Proposed Change]: Modify “The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2.” as “The total number of different SMTC periodicities across smtc and smtc5list is 2.”. Illustrate that “If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.”.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present. If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.

[Comments]:

[Rapp] The first change is fine. In our view, the second change is incorrect. RAN2 has agreed delta signalling with smtc4list to precisely avoid configuring two lists (smct4list and smtc5list) with the same information (pci-list and offset).

[Qualcomm] The design here is to apply delta configuration so current text is correct that total number does not exceed, may be should say 7. If smtc5 is present, the smtc4 shall be present. But if smtc4 is present, it is not necessary smtc5 to be included.

# V204

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V204 | NTN | 2 | SMTC5 and the reference location list can be configured for the inter-frequency case | Yes, R2-250xxxx | vivo (Stephen) |  | v005 | Duplicate H250 |

[Description]: There are use cases to include SMTC5 and reference location list in SIB4 for the inter-frequency case.

[Proposed Change]: Add refLocList and smtc5list in SIB4.

[Comments]:

[xiaomi] We agree with the proposal. For idle/inactive mode, there is no measurement gap. So there is no UE capability issue related to inter frequency measurement.

[Samsung] share same view

# H253

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H253 | NTN | 1 | Need code of warningAreaCoordinates-r19 | R2-25xxxxx | Huawei (Lili) |  | V014 | Rejected |

[Description]: The need code of warningAreaCoordinates-r19 is Need R, which means the UE releases this information if not included. However, this geographical area information corresponds to the warning message segments, how the UE handle the case where some of the segments are provided with the geographical area while other segments are not provided with such information?

SIB7 ::=                            SEQUENCE {

messageIdentifier                   BIT STRING (SIZE (16)),

serialNumber                        BIT STRING (SIZE (16)),

warningMessageSegmentType           ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber         INTEGER (0..63),

warningMessageSegment               OCTET STRING,

dataCodingScheme                    OCTET STRING (SIZE (1))                     OPTIONAL,   -- Cond Segment1

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

warningAreaCoordinatesSegment-r19   OCTET STRING                                OPTIONAL    -- Need R

]]

}

Based on the description in the procedure text, we think UE should store the geographical area for other segments as well (for instance, segment1 includes geographical area information while segment2 does not include such information, then the geographical area information from segement1 should not be released, it should be used for segment2 as well).

[Proposed Change]:

Change “Need R” to “Need S”, and clarify in the field description that if the field is missing, warningAreaCoordinatesSegment for other segments of the same warning message applies.

[Comments]:

[Rapp] Need R is present for warningAreaCoordinates in CMAS (SIB8) since its introduction. If this were a problem, it needs to be fixed first for earlier releases.

# O710

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O710 | NTN | 1 | Adding “warning area coordinates segment” to field description | R2-25xxxx | OPPO (Haocheng) |  | v015 | Agreed |

[Description]: in current spec, the warningAreaCoordinatesSegment is included in SIB7 without introducing the new segment type and segment number parameters for warning area coordinates segment. In our understanding, the current parameters of warningMessageSegmentType and warningMessageSegmentNumber will be reused. So in the field description, we need also specify these two parameters are applicable to warning area coordinates segment.

Besides, if our understanding is correct, then there is a problem on how to set the value of warningMessageSegmentType in case of the segment number of warning message and warning area coordinates is different. maybe we need further discussion. For example, we can introduce a new segment type parameter for warning area coordinate segment or let the NW to guarantee the segment number is same.

[Proposed Change]:

warningMessageSegmentNumber Segment number of the ETWS warning message segment and warning area coordinates segment contained in the SIB. A segment number of zero corresponds to the first segment, A segment number of one corresponds to the second segment, and so on.

warningMessageSegmentType Indicates whether the included ETWS warning message segment and warning area coordinates segment is the last segment or not.

[Comments]:

[Rapp] Agree with proposed change, but we do not understand well the issue. The signalling design should be copied from CMAS. A similar formulation can be used instead. A single message number and segment type is used for both the warning message and the area coordinates.

[Qualcomm] Agree with Rapp. Perhaps it would be better to say ‘and warning area coordinates segment if any”.

# V502

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V502 | NES | 1 | mandatory IEs in OD-SIB1-config |  | vivo (Jianhui) |  | V026 | Duplicate |

[Description]:

Similar to the comments from H101-H103, and S030-S031, we think there are some parameters in OD-SIB1-config that should be mandatory. Otherwise, the OD-SIB1 request operation cannot work with the absence of such parameters.

[Proposed Change]:

For the following parameters, some are from RAN1 agreement which is agreed to be mandatory, and the others are those we think it’s necessary to be present for OD-SIB1 request operation. We propose that these parameters should be mandatory, i.e. Remove   OPTIONAL, -- Need R

Parameters that RAN1 agreed to be mandatory:

- od-sib1-WindowDuration-r19

- locationAndBandwidth-r19

Parameters that we also assume essential for OD-SIB1 request operation:

- carrierBandwidth-r19

- ss-PBCH-BlockPower-r19

- ssb-PositionsInBurst-r19

- sib1-RequestPeriod-r19

- msg1-SubcarrierSpacing-r19

- prach-RootSequenceIndex-r19

- sib1-rsrp-ThresholdSSB-r19

- ul-SubCarrierSpacing-r19

- sib1-restrictedSetConfig-r19

[Comments]:

[Rapporteur] This is a duplicated issue. Please see H101 - H103, and S030 - S031.

# A101

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A101 | NES | 1 | Type of carrierBandwidth-r19 |  | Apple (Peng Cheng) |  | V029 | PropAgree |

[Description]: Currnently, it is captured as “OPTIONAL –Need R”. However, this IE is needed for the UE to derive the frequency resource to send Msg1. So, it should be mandatory.

carrierBandwidth-r19                     INTEGER (1..maxNrofPhysicalResourceBlocks)  OPTIONAL, -- Need R

[Proposed Change]: Suggest to change this IE to mandatory.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# H100

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H100 | NES | 2 | frequencyBandList-r19 field |  | Huawei (Marcin) |  | V013 | PropAgree |

[Description]:

This field should be mandatory for TDD based on Agreement (RAN1#121):

The frequencyBandList is mandatorily present in WUS configuration for TDD system, which refers to the IE within FrequencyInfoDL-SIB.

[Proposed Change]:

Add conditional mandatory presence for TDD.

[Comments]:

[Apple] Agree.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# A102

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A102 | NES | 1 | Type of ss-PBCH-BlockPower-r19 |  | Apple (Peng Cheng) |  | V029 | PropAgree |

[Description]: Currnently, it is captured as “OPTIONAL –Need R”. However, this IE is needed for the UE to derive the transmit power to send Msg1. So, it should be mandatory.

ss-PBCH-BlockPower-r19                   INTEGER (-60..50).       OPTIONAL, -- Need R

[Proposed Change]: Suggest to change this IE to mandatory.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# H101

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H101 | NES | 2 | od-sib1-WindowDuration-r19 field |  | Huawei (Marcin) |  | V013 | PropAgree |

[Description]:

this should be mandatory based on R1-2506622 and Agreement (RAN1#120bis)

From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings:

-	PhysCellId and ARFCN-ValueNR are mandatory

-	frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification)

-	K_SSB is mandatory

-	searchSpaceZero and controlResourceSetZero are mandatory

-	ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory

[Proposed Change]:

Remove   OPTIONAL, -- Need R

[Comments]:

[Samsung]: Agree

[Apple] Agree.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# Z103

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z103 | NES | 1 | Correction on the SIBxx |  | ZTE (Gao Yuan) |  | V028 | PropAgree |

[Description]: According to clause 7.4.3.1 of TS 38.211, the range of k-ssb-r19 shall be (0..23) rather than (1..23). Furthermore, the IE ul-SubCarrierSpacing-r19 in the IE SIBxx has seven values, and there should be a spare value. So this spare value can be added, and sib1-restrictedSetConfig-r19 has the same issue..

[Proposed Change]: Suggest to:

OD-SIB1-Config-r19  ::=       SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellIdList-r19                       SEQUENCE (SIZE(1.. maxPCI-OD-SIB1-r19)) OF PhysCellId,

sib1-TDD-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                     OPTIONAL, -- Cond TDD

offsetToCarrier-r19                      INTEGER (0..2199),

carrierBandwidth-r19                     INTEGER (1..maxNrofPhysicalResourceBlocks)                                 OPTIONAL, -- Need R

offsetToPointA-r19                       INTEGER (0..2199)                                                          OPTIONAL, -- Cond TDD

frequencyBandList-r19                    MultiFrequencyBandListNR-SIB                                              OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                   INTEGER (-60..50)                                                          OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                 SEQUENCE {

inOneGroup                             BIT STRING (SIZE (8)),

groupPresence                          BIT STRING (SIZE (8))                                                      OPTIONAL  -- Cond FR2-Only

},

ssb-Periodicity-r19                      ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2}            OPTIONAL, -- Need R

od-sib1-WindowDuration-r19               ENUMERATED {ms20, ms40, ms80, ms160, ms320, spare1, spare2, spare3}        OPTIONAL, -- Need R

od-sib1-windowStartOffset-r19            ENUMERATED {sl0, sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

k-ssb-r19                                INTEGER (0..23),

controlResourceSetZero-r19               ControlResourceSetZero,

searchSpaceZero-r19                      SearchSpaceZero,

sib1-RequestConfig-r19                   SIB1-RequestConfig-r19,

sib1-RequestConfigSUL-r19                SIB1-RequestConfig-r19                                                     OPTIONAL -- Need R

}

SIB1-RequestConfig-r19 ::=                 SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                  SEQUENCE {

ssb-perRACH-Occasion-r19                  ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

},

sib1-RequestPeriod-r19                  ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}                  OPTIONAL, -- Need R

sib1-RequestResources-r19               SIB1-RequestResources-r19,

sib1-PDCCH-RestrictionToPRACH-r19       ENUMERATED {true},

rach-ConfigSIB1-r19                     RACH-ConfigGeneric,

ra-SearchSpace-r19                      SearchSpace                                                                    OPTIONAL, -- Need S

n-TimingAdvanceOffset-r19               ENUMERATED { n0, n25600, n39936, spare1 }                                      OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19              ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}      OPTIONAL, -- Need R

prach-RootSequenceIndex-r19             CHOICE {

l839                                       INTEGER (0..837),

l139                                       INTEGER (0..137)

}                                                                                                                    OPTIONAL, -- Need R

sib1-rsrp-ThresholdSSB-r19               RSRP-Range                                                                    OPTIONAL, -- Need R

rsrp-ThresholdSSB-SUL                    RSRP-Range                                                                    OPTIONAL, -- Cond SUL

locationAndBandwidth-r19                 INTEGER (0..37949)                                                            OPTIONAL, -- Need R

absoluteFrequencyPointA-r19              ARFCN-ValueNR                                                                 OPTIONAL, -- Cond FDD

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB                                                  OPTIONAL, -- Need R

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480-v1700, kHz960-v1700, spare1}   OPTIONAL, -- Need R

p-Max-r19                                P-Max                                                                         OPTIONAL, -- Need R

sib1-restrictedSetConfig-r19 [RIL]: S030, NES            ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, spare1}          OPTIONAL  -- Need R

}

[Comments]:

[Apple] Agree.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# X202

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X202 | NES | 1 | Classify parameters of OD-SIB1 | R2-25xxxxx | Xiaomi (Li Zhao) |  | V003 | PropReject |

[Description]: According to the parameter list from RAN1, some parameters related to OD-SIB1 should be configured per od-sib1-Config instead of per SIB1-RequestConfig, e.g., sib1-rsrp-ThresholdSSB-r19, locationAndBandwidth-r19, absoluteFrequencyPointA-r19, ul-FrequencyBandList-r19, ul-SubCarrierSpacing-r19, etc.

[Proposed Change]: RAN2 to discuss to move those parameters not related to SIB1-RequestConfig from SIB1-RequestConfig to od-sib1-Config.

[Comments]:[Nokia] I’m not sure on this one. RAN1 xls seems to be bit vague on this. So far to use current asn.1 seems Ok anyway as the xls points out that all the parameters in frequenciInfoUL are per WUS config. So we would not do this change until it is confirmed with Ran

[Huawei]: Agree with Xiaomi. We understand that RRC parameters are up to RAN2 to specify but we did not see the motivation for deviating from the RAN1 parameter list, in particular what was the reasoning of the current parameter placement in OD-SIB1-Config-r19 and SIB1-RequestConfig-r19.

[vivo] Maybe it’s safer to keep them as it is now, like the sib1-rsrp-ThresholdSSB-r19 and ul-SubCarrierSpacing-r19, which is per Uplink configured (NUL or SUL)?

[Ericsson] It is this way dues to RAN2 SUL agreement. RAN1 did not consider SUL and hence this was not reflected in their parameter excel. There is no functional difference in RAN1 perepective with the existing order of parameters since all is there in the highest level IE.

[Samsung]: Agree with Ericsson. These parameters are UL carrier specific (SUL/NUL).

[Xiaomi] I think SUL related parameter is rsrp-ThresholdSSB-SUL, which is not reflected in RAN1 parameter list and OK to be configured within SIB1-RequestConfig as it relates to the selection of carrier when transmitting SIB1 request. However, there are many parameters not related to SIB1 request transmission, e.g., sib1-rsrp-ThresholdSSB-r19, locationAndBandwidth-r19, absoluteFrequencyPointA-r19, ul-FrequencyBandList-r19, ul-SubCarrierSpacing-r19, etc., that should be moved out according to RAN1 guidance. Also, there are some parameters instructed to be configured per SIB1 request configuration, e.g., ss-PBCH-BlockPower-r19, ssb-PositionsInBurst-r19 and sib1-TDD-UL-DL-ConfigurationCommon-r19 but are configured within od-sib1-Config. Thus, we think we need to discuss whether/how to follow the guidance from RAN1 parameter list.

[Apple] We agree with Samsung and Ericsson on NUL/SUL carrier specific parameters. To support SUL in NES cell, it is not sufficient to only introduce rsrp-ThresholdSSB-SUL because SUL can configure a separate RACH resource different from NUL. It is same as legacy OD-SIB: in SI-SchedulingInfo, it has si-RequestConfig and si-RequestConfigSUL with same type si-RequestConfig. Thus, we prefer to keep the current formulation.

[Rapporteur] There seems to be nothing broken with the current formulation. Note that RAN1 excel sheet does not indicate how signalling should be designed, and this is up to RAN2.

# X204

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X204 | NES | 2 | sib1-PDCCH-RestrictionToPRACH-r19 | R2-25xxxxx | Xiaomi (Haitao) |  | V014 | PropAgree |

[Description]: according to RAN1 agreement, this parameter should be optional. Currently it can only indicate TRUE.

[Proposed Change]: add OPTIONAL for this parameter.

[Comments]:

[Huawei]: Agree, this needs to be OPTIONAL, -- Need R

[Samsung]: Agree

[Apple] Agree

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# S031

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S031 | NES | 1 | prach-RootSequenceIndex-r19 |  | Anil Agiwal (Samsung) |  | V023 | PropAgree |

[Description]:  prach-RootSequenceIndex-r19should be mandatory similar to its configuration in RACH-ConfigCommon

[Proposed Change]: prach-RootSequenceIndex-r19 should be mandatory

[Comments]:

[Apple] Agree.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# H102

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H102 | NES | 2 | locationAndBandwidth-r19 field |  | Huawei (Marcin) |  | V013 | PropAgree |

[Description]:

This should be mandatory based on R1-2506622 and Agreement (RAN1#121):

The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ are mandatorily present in the UL-WUS configuration for both FDD and TDD system.

[Proposed Change]:

Remove   OPTIONAL, -- Need R

[Comments]:

[Samsung]: Agree

[Apple] Agree.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# H103

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H103 | NES | 2 | absoluteFrequencyPointA-r19 field |  | Huawei (Marcin) |  | V013 | PropReject |

[Description]:

This should be mandatory based on R1-2506622 and Agreement (RAN1#121):

The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ are mandatorily present in the UL-WUS configuration for both FDD and TDD system.

[Proposed Change]:

Remove   OPTIONAL, -- Cond FDD

[Comments]:

[Xiaomi] based on the RAN1 agreement and the parameter list, ‘absoluteFrequencyPointA’ is not mandatory and only configured in FDD？

Agreement

The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ are mandatorily present in the UL-WUS configuration for both FDD and TDD system.

[Apple] Agree with Xiaomi (i.e. only FDD is mandatory). @Huawei, RAN1 modified the agreement in RAN1#121 (i.e. they remove ‘absoluteFrequencyPointA’,) :

Agreement

The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ are mandatorily present in the UL-WUS configuration for both FDD and TDD system.

[Rapporteur] Agree with Xiaomi

# S030

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S030 | NES | 1 | sib1-restrictedSetConfig-r19 |  | Anil Agiwal (Samsung) |  | V023 | PropAgree |

[Description]:  sib1-restrictedSetConfig-r19should be mandatory similar to its configuration in RACH-ConfigCommon

[Proposed Change]: sib1-restrictedSetConfig-r19 should be mandatory

[Comments]:

[Apple] Agree.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# J003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J003 | NES | 1 | Field description on locationAndBandwidth |  | Sharp (LIU Lei) |  | V020 | PropAgree |

[Description]: The first PRB for SIB1 request should be dertermined by offsetToCarrier configured in OD-SIB1-Config, instead of the legacy IE.

[Proposed Change]: Remove the legacy IE decription as below:

locationAndBandwidth

Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting =275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)

[Comments]:

[Apple] Agree withi change.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting as follows:

# J004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J004 | NES | 1 | Field description on ul-FrequencyBandList |  | Sharp (LIU Lei) |  | V020 | PropAgree |

[Description]: According to RAN1, this refers to the IE within FrequencyInfoUL-SIB. The field name is changed a bit, then the field description should be aligned.

[Proposed Change]: Change frequencyBandList to ul-FrequencyBandList as below:

ul-FrequencyBandList

Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values for UL operation as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the  ul-FrequencyBandList field.

[Comments]:

[Apple] Agree withi change.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting as follows:

ul-FrequencyBandList

Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values for UL operation as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the ul-FrequencyBandList field.

# Z255

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z255 | NTN | 2 | Signalling to indicate ISA the same as serving cell | R2-25xxxx | ZTE (Zhihong) |  | v012 | ToDo |

[Description]: When a ISA is the same as serving cell, NW can only include intendedServiceAreaId without including the areaCoordinates, this help saving unnecessary signalling overhead. Also it is more flexible for NW to signal ISA which is a combination of ISA the same as serving cell and other ISAs.

[Proposed Change]: Make areaCoordinates as optional, and clarify UE understanding in field description when it is absent.

SIBXX information element

-- ASN1START

-- TAG-SIBXX-START

SIBXX-r19 ::=                     SEQUENCE {

intendedServiceAreaList-r19     IntendedServiceAreaList-r19      OPTIONAL,  -- Need R

lateNonCriticalExtension        OCTET STRING                     OPTIONAL,

...

}

IntendedServiceAreaList-r19 ::=   SEQUENCE (SIZE (1..maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19

IntendedServiceAreaInfo-r19 ::=   SEQUENCE {

intendedServiceAreaId-r19       MBS-IntendedAreaID-r19,

areaCoordinates-r19             CHOICE {

polygonArea-r19                   OCTET STRING,

circleArea-r19                    SEQUENCE {

center-r19                      ReferenceLocation-r17,

radius-r19                      INTEGER(0..65535)

}                         OPTIONAL

}

}

SIBXX field descriptions

areaCoordinates Indicates the ISA as shape of circle or polygon. Absence of this field indicates the ISA has the same coverage as serving cell.

[Comments]:

[Rapp] RAN2 agreed that for services that apply to the whole cell, legacy mechanisms apply, i.e., there is no ISA and no ISA ID needs to be defined.

# S026

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S026 | NTN | 1 | Missing FD of radius-r19 in SIBXX |  | Samsung (Shiyang) |  | v011 | Agreed |

[Description]: The FD of radius-r19 is missing which is needed to describe the circle area and the unit of radius.

[Proposed Change]:

Add a field description for radius-r19

Radius

Indicates the radius of circleArea. Each step represents 1m.

[Comments]:

[Rapp] Please refer to the RRC annex for drafing rules. There are some fields which do not need to be described since they are straightforward.

“The ASN.1 clause specifying the contents of a PDU type may be followed by a field description table where a further description of, e.g., the semantic properties of the fields may be included. The general format of this table is shown in the example below. The field description table is absent in case there are no fields for which further description needs to be provided e.g. because the PDU does not include any fields, or because an IE is defined for each field while there is nothing specific regarding the use of this IE that needs to be specified.”

# S053

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S053 | AIML | 1 | Interaction with NTN |  | Aby K Abraham |  | v026 | ToDo |

[Description]:

RAN2 to discuss if AI/ML for Phy is supported for NTN. If it is supported,allowedHARQ-mode should be applicable for SRBx

[Proposed Change]:

Discuss if AI/ML for Phy is applicable for NTN. If it is applicable, below change is needed.

LogicalChannelConfig field descriptions

allowedCG-List This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

allowedHARQ-mode Indicates the allowed HARQ mode of a HARQ process mapped to this logical channel. If the parameter is absent, there is no restriction for HARQ mode for the mapping. This field applies to SRB1, SRB2, SRB4, SRBx and DRBs.

[Comments]:

# N027

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N027 | AIML | 1 | Fieldname mismatch | N/A | Jerediah Fevold |  | vnnn | Agreed |

[Description]: Change the fieldname for ApplicabilitySetConfigId-r19 from applicabilitySetId-r19 to applicabilitySetConfigId-r19. Similar issues throughout remove the word “config” from the names, but there is no need for conciseness.

[Proposed Change]:

ApplicabilitySetConfigId-r19 ::=            INTEGER (0..maxNrofApplicabilitySetConfigs-1-r19) [RIL]: N027 AIML

ApplicabilityInfoReport-r19 ::=    SEQUENCE {

applicabilityInfoReportId-r19          CHOICE {

csi-ReportConfigId-r19                          CSI-ReportConfigId,

applicabilitySetConfigId-r19                     ApplicabilitySetConfigId-r19,

[Comments]:

[WI CR rapporteur-v022]: The intention was to use “config” only in field names in otherConfig, whereas field names in ApplicabilityConfigList were supposed to not contain “config”, so that we can distinguish in the procedural text the fields configured by the network vs. those sent by the UE. Accordingly, we agree with the proposed change for ApplicabilitySetConfigId, but not with the one for ApplicabilityInfoReport. We changed the status to “PropAgree”, with the intention to capture just the first proposed change in the CR.

# N111

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N111 | AIML | 2 | Excess hierarchy in Applicability Report | R2-2507181 | Jerediah Fevold |  | v030 | ToDo |

[Description]: The applicability report configuration is configured per SCell ID, but the full inference configurations already contain the SCell ID and the sets of inference related parameters could also include the SCell ID directly, as described in N112 and by the same TDoc referenced here. The extra hierarchy doesn’t only affect the ASN.1, but also the procedures which need to nest an additional loop. There isn’t a clear benefit to this.

[Proposed Change]: Remove a level of hierarchy by reporting the applicability of all configurations, full and sets of inference-related parameters, in a single list.

[Comments]:

# E041

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E041 | AIML | 2 | Associated ID validity | R2-25xxxx | Ericsson (Jens) |  | V022 | Rejected |

[Description]: According to current specifications, once an associated ID value is signaled from the network to the UE, the meaning of that value at the UE (or UE-side entity) corresponds to the same beam property/deployment forever. There is no way for the network to reset/reassign that value to other beam properties and inform the UE that should forget the old meaning of that value. We think it is unreasonable to assume that a given meaning holds for a given associated ID forever. It may happen that e.g. some hardware is replaced and the old hardware is not used again anywhere else in the network, or there is a failure/reset in the network entity storing/assigning associated IDs, after which the legacy associated ID meanings (assigned to deployment characteristics) cannot be recovered. Although such events may be very unlikely, they may still happen at a certain point in the future. Thus, there should be a way for the network and UE to make sure that they have the same understanding of an associated ID value. This is important for the UE (or UE-side entity) to be able to understand if a model exists and/or is trained for the same deployment assumed by the network for an associated ID.

[Proposed Change]: We will discuss this issue in more detail and present possible solutions in a Tdoc.

[Comments]:

# C100

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C100 | SBFD | 1 | Concern on ‘CFRA’ | R2-250xxxx | Jianxiang Li (CATT) |  | V005 | Agreed |

[Description]: For BFR, if there is a beam associated to a dedicated preamble/PRACH and above the threshold, using CFRA；Otherwise, using CBRA. If there is no suitable beam, this RO type can also be used for CBRA BFR. I’m wondering if the CFRA is required in ‘Indicates the second PRACH occasions of CFRA to be used by a SBFD aware UE’.

[Proposed Change]: remove ‘of CFRA’.

[Comments]: [ZTE] Agree with the intention. But ‘CFRA’ here is to explicitly reflect RAN2 agreement, so ‘CFRA’ should be kept here. Two alternatives of change: (1) RRC spec does not change, the RAN2 agreement‘SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA’ should be captured/reflected somewhere in MAC spec; (2) change the RRC field description to ‘Indicates the second PRACH occasions of CFRA or of the fallback CBRA to be used by a SBFD aware UE’

[LGE] In our understanding, it was intentionally added to consider the case that no CFRA resource is configured for BFR (e.g., only configuring RA prioritization parameters); in this case, the UE initiates the CBRA procedure and RO type should not be indicated by this field. However, given that the initial RO type selection procedure in MAC is now sepately described for the CFRA cases and CBRA cases, it would be clear in MAC spec enough even though ‘for CFRA’ is removed. In this sense, we are okay to remove the ‘for CFRA’ or ZTE’s alternative (2) to specify the fallback case from CFRA to CBRA.

[Rapp] Have same understanding as LGE also OK with ZTE alternative (2). However this "CFRA" is added specifically based on the RAN2 agreement for CFRA case. We need discussion/confirmation of RAN2 for expanding it to cover the fallback CBRA case. I will recommend "ToDo" status for this RIL. Since there might be alternative solutions, companies can discuss it in their contributions.

# Z401

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z401 | MIMO | 2 | The present condition for the pathlossOffsetPRACH-DCI-1-0 |  | ZTE (Wenting Li) |  | v008 | PropAgree |

[Description]:RAN1 spec is as below

-	1 bit if the UE is configured with higher layer parameter plOffsetInPrach_InDCI and at least one configured TCI state for the serving cell is configured with plOffset.

Ther are 2 conditons:

Condition 1: if the UE is configured with higher layer parameter plOffsetInPrach_InDCI

Condtion 2: at least one configured TCI state for the serving cell is configured with plOffset

Now in our CR, it seems that we intends to take the condtion 2 as the precondition of condition 1. If go to this way, to keep aligned, RAN1 spec can delete the condition 2.

Or we can delete the presence condition from our RAN2 spec.

[Proposed Change]: Delete the “This field can only be configured when at least one TCI state is configured with pathlossOffset”

pathlossOffsetPRACH-DCI-1-0

Enables the presence of a 1-bit DCI field in DCI format 1_0 for indicating the pathloss offset for PDCCH-ordered PRACH transmissions. This field can only be configured when at least one TCI state is configured with pathlossOffset (see TS 38.212 [17], clause 7.3.1).

[Comments]:

# Z402

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z402 | MIMO | 2 | The present condition for the prachAssociationDCI-1-0 |  | ZTE (Wenting Li) |  | v008 | PropReject |

[Description]:Similar to Z401, RAN1 spec is as below

1bit if the UE is provided with tag2-Id and SSB-MTC-AddtionalPCI, and the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, and the UE is provided with PrachAssociationIndicator_InDCI_format_1_0. This field is reserved if the cell indicated by Cell indicator field is a candidate cell.

Ther are 2 conditons:

Condition 1: the UE is provided with tag2-Id and SSB-MTC-AddtionalPCI…….

Condtion 2: the UE is provided with PrachAssociationIndicator_InDCI_format_1_0.

Now in our CR, it seems that we intends to take the condtion 1 as the precondition of condition 2. If go to this way, to keep aligned, RAN1 spec can delete the condition 1.

Or we can delete the presence condition from our RAN2 spec

[Proposed Change]: Delete the “This field can only be configured if twoTA-Without-MultiDCI-MultiTRP is configured.”

prachAssociationDCI-1-0

Enables the presence of 1-bit DCI field "PRACH association indicator" in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1). This field can only be configured if twoTA-Without-MultiDCI-MultiTRP is configured.

[Comments]:

[Ericsson(Lian)] Since this condition was explicitly discussed and agreed in RAN2, it is better to keep it as it is unless a major issue is found.

# Z352

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z352 | SBFD | 1 | Need code for sbfd-RSRP-ThresholdRO-Type |  | Yu Pan (ZTE) |  | V006 | Rejected |

[Description]: Currently the sbfd-RSRP-ThresholdRO-Type is Need S, but the absent behaviour is described in sbfd-RSRP-ThresholdRO-TypeUsage.

[Proposed Change]: two alternatives: (1) change sbfd-RSRP-ThresholdRO-Type from Need S to Need M; (2) add absent behaviour in the field description of sbfd-RSRP-ThresholdRO-Type

[Comments]:

[Rapp] With this restriction "( sbfd-RSRP-ThresholdRO-TypeUsage) is always configured together with sbfd-RSRP-ThresholdRO-Type ", there shall be no ambiguity how to interpret UE default behaviour by this " Need S".

# O000

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O000 | SBFD, MOB | 2 | Applicability of SBFD-RACH to LTM procedure |  | OPPO (Qianxi) |  | V003 | ToDo |

[Description]: R2 has not concluded the applicability of SBFD to LTM (other than inter-DU LTM), including the procedure for early UL sync, LTM execution (CSC-based) and CLTM execution.

[Proposed Change]: R2 discuss and conclude the applicability of SBFD to LTM (other than inter-DU LTM), including the procedure for early UL sync, LTM execution (CSC-based) and CLTM execution.

[Comments]:

[Rapp] RAN2 agreed in 131 meeting that " Support co-existence of SBFD with intra-DU LTM. Whether to support the co-existence between SBFD and other LTM cases is not discussed in the Rel-19 SBFD WI. ". I understand decision on whether to support the co-existence between SBFD and other LTM cases in Rel-19 needs to be made in the main session and impacts on other WG (e.g. RAN3) need to be considered. For SBFD WI, I will recommend PropReject on this RIL and I expect the interested companies can discuss it in their contribution for cross-WI issues (not SBFD WI).

[Rapp] The RIL status is updated to "ToDo" according to the instructions from the general RRC Rapporteur.

# L701

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L701 | SBFD | 1 | Whether to locate the rach-ConfigCommon and sbfd-RACH-DualConfig for the same feature combination in the same additionalRACH-Config. | R2-250xxxx | LGE (Hanseul Hong) |  | V008 | Agreed |

[Description]: In the RACH partitioning framework, when the 2-step RA resources and the 4-step RA resources are configured for the same feature combination, those two IEs are located in the same additionalRACH-Config IE, in order to simplify the UE implementation to identify the RA parameters associated with the same feature combination.

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

Since the SBFD RO (second PRACH occasion) defined in sbfd-RACH-DualConfig can also be associated with the feature combination and located in additionalRACH-Config IE, it should be discussed sbfd-RACH-DualConfig should be located in the same additionalRACH-Config with the non-SBFD RO (i.e., rach-ConfigCommon) for the same feature combination, in order to simplify the UE to determine the corresponding RA parameters (e.g., for the RO type switch case) as well.

[Proposed Change]: Discuss whether network needs to configure rach-ConfigCommon and sbfd-RACH-DualConfig in the same additionalRACH-Config, when the both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for a specific FeatureCombination. If so, following text can be added:

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config. If both rach-ConfigCommon and sbfd-RACH-DualConfig are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

[Comments]:

[Rapp] As this is not discussed before, I recommend "ToDo" and companies can discuss this in their contributions.

# C101

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C101 | SBFD | 1 | Reference |  | Jianxiang Li (CATT) |  | V005 | Agreed |

[Description]: reference is required for the first PRACH occasions or the second PRACH occasions.

[Proposed Change]: add ‘as specified in TS 38.213 [13]’

[Comments]:

[Rapp] Agree to add this reference.

# Z351

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z351 | SBFD | 1 | SBFD RACH configuration type, option 1 |  | Yu Pan (ZTE) |  | V006 | Agreed |

[Description]: Currently sbfd-RACH-SingleConfig is configured per UL BWP, not per RACH configuration in the UL BWP (i.e., sbfd-RACH-SingleConfig is not configured under AdditionalRACH-Config-r17). But if option 1 is enabled, all the RACH configuration under this UL BWP should enable option 1.

[Proposed Change]: In field description of sbfd-RACH-Config, add the underline wording: ‘Value sbfd-RACH-SingleConfig indicates the single RACH configuration for SBFD random access operation which is appliable to all RACH configurations in this BWP is enabled’

[Comments]:

[Rapp] Agree with the intention. Think adding "in this BWP" would be sufficient.

# E001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E001 | SBFD | 1 | Update FD for field sbfd-RACH-Config. |  | Ericsson (Min) |  | V009 | Agreed |

[Description]: wording/formulation of the FD can be improved

[Proposed Change]:

“value sbfd-RACH-DualConfig is used to configure dual RACH configurations and configure random access parameters in SBFD symbols by setting up one additional RACH configuration and can include all parameters in rach-ConfigCommon except rsrp-ThresholdSSB-SUL. If present and the UE is indicated to use SBFD random access operation for CFRA in the same BWP, the UE shall derive the SBFD RO locations based on this field.”

can be reformulated as

“Value sbfd-RACH-DualConfig indicates dual RACH configurations for SBFD random access operation are enabled wherein parameters for random access operation in the first PRACH occasions are configured in rach-ConfigCommon and parameters for random access operation in the second PRACH occasions are configured in sbfd-RACH-DualConfig. If present, the UE drvies PRACH occasions for CFRA based on this field”

[Comments]:

[Rapp] Understand the intention. I tried to improve the sentence in WI Rapp CR. Note the original FD includes TP e.g. "can include all parameters in rach-ConfigCommon except rsrp-ThresholdSSB-SUL" from RAN1.

# L702

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L702 | SBFD | 1 | SBFD RO in the field description of sbfd-RACH-Config |  | LGE (Hanseul Hong) |  | V008 | Agreed |

[Description]: In the current specification, SBFD RO is not used, but the second PRACH occasions is used.

[Proposed Change]: Change ‘SBFD RO’ to ‘second PRACH occasions’ in the field description of sbfd-RACH-Config. Further editorial change can be considered, e.g.,

sbfd-RACH-Config Indicated SBFD RACH configuration type, see clause 5 in TS 38.211 [16] and clause 8 in TS 38.213 [13]. Value sbfd-RACH-SingleConfig indicates the single RACH configuration for SBFD random access operation is enabled[RIL]: Z351, SBFD, and value sbfd-RACH-DualConfig is used to configure dual RACH configurations and configure random access parameters in SBFD symbols by setting up one additional RACH configuration and can include all parameters in rach-ConfigCommon except rsrp-ThresholdSSB-SUL. If present and the UE is indicated to use SBFD random access operation for CFRA in the same BWP, the UE shall derive locations of the second PRACH occasions based on this field.

[Comments]:

[Rapp] Agree/ implemented in WI Rapp CR.

# E002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E002 | SBFD | 1 | Definition for the term “SBFD aware UE” is missing. |  | Ericsson (Min) |  | V009 | Agreed |

[Description]: Definition for the term “SBFD aware UE” is missing.

[Proposed Change]:

Either add a definition for the term, or refer to RAN1 spec if there is definition, or just remove it.

[Comments]:

[Rapp] Add pointer to 38.300 where SBFD aware UE is defined.

# C102

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C102 | SBFD | 1 | RACH occasion type |  | Jianxiang Li (CATT) |  | V005 | Agreed |

[Description]: what is RACH occasion type is required to clarify here. It applies to other same issue in the CR.

[Proposed Change]: RACH occasion type (the first or the second RACH occasion as specified in TS 38.213 [13]

[Comments]:

[Rapp] add (the first or the second PRACH occasions as specified in TS 38.213 [13])

# C103

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C103 | SBFD | 1 | reference |  | Jianxiang Li (CATT) |  | V005 | Agreed |

[Description]: reference is required for the first PRACH occasions or the second PRACH occasions. It applies to other same issue in the CR.

[Proposed Change]: add ‘as specified in TS 38.213 [13]’

[Comments]:

[Rapp] added (the first or the second PRACH occasions as specified in TS 38.213 [13]) after the first appeared RACH occasion type.

# E004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E004 | SBFD | 1 | Unclear FD for sbfd-AdditionalRACH-Config |  | Ericsson (Min) |  | V009 | Agreed |

[Description]: Change FD for sbfd-AdditionalRACH-Config. The FD says

Indicates the configuration of dual RACH configurations by setting up one additional RACH configuration

[Proposed Change]:

Update the FD as “indicate the additional RACH configuration for random access operation in the second PRACH occasions. “

[Comments]:

[Rapp] Implemented in WI Rapp CR

# Z353

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z353 | SBFD | 1 | FD of sbfd-RACH-DualConfig-ValidRO-AcrossSymbolTypes |  | Yu Pan (ZTE) |  | V006 | Duplicate |

[Description]: In the FD of sbfd-RACH-DualConfig-ValidRO-AcrossSymbolTypes, the ‘RACH configuration option 2’ should be avoided in spec.

[Proposed Change]: Change the FD to ‘Indicates whether a configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots is valid for RACH configuration Option 2the corresponding sbfd-AdditionalRACH-Config’

[Comments]:

[Rapp] see E003

# E003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E003 | SBFD | 1 | Term “RACH configuration option 2” is undefined |  | Ericsson (Min) |  | V009 | Agreed |

[Description]: FD for field sbfd-RACH-DualConfig-ValidRO-AcrossSymbolTypes refers to “RACH configuration option 2”, this term is defined nowhere.

[Proposed Change]:

This term can be just removed from the text.

[Comments]:

[Rapp] Implemented.

# H350

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H350 | SBFD | 1 | Revision of value range | - | Huawei-Tao Cai |  | V004 | Agreed |

[Description]: According to RAN1 updated higher layer parameters list in R1-2506622, the range of parameter sbfd-Config2-PUSCH-RB-Offset-r19 shall be (0..maxNrofPhysicalResourceBlocks-1) not (0..maxNrofPhysicalResourceBlocks)

[Proposed Change]: Change the parameter range according to RAN1 revision.

[Comments]:

[Rapp] Implemented in the WI Rapp CR.

# H403

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H403 | MIMO | 2 | Support of LTM cell switch |  | Huawei (David) |  | v007 | ToDo |

[Description]: At LTM cell switch, the UE applies the LTM candidate TCI state indicated in the LTM cell switch command until a TCI state is indicated in the target cell. An LTM candidate TCI state cannot indicate pathlossoffset, so it can only indicate the UL TRP that has associated DL, but not the other UL TRP. In some scenario, this may result in insufficient transmission power for cell switch, resulting in failure.

[Proposed Change]: Add pathlossOffset to CandidateTCI-UL-State-r18:

CandidateTCI-State-r18 ::=           SEQUENCE {

tci-StateId-r18                      TCI-StateId,

qcl-Type1-r18                        LTM-QCL-Info-r18,

qcl-Type2-r18                        LTM-QCL-Info-r18                                                    OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r18           PathlossReferenceRS-Id-r17                                          OPTIONAL,   -- Cond Joint

tag-Id-ptr-r18                       ENUMERATED {n0,n1}                                                  OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                  Uplink-powerControlId-r17                                           OPTIONAL,   -- Cond Joint2

...,

[[

pathlossOffset-r19                  ENUMERATED {

dB-12, dB-8, dB-4, dB0, dB4, dB8,

dB12, dB16, dB20, dB24, dB28, dB32, dB36,

dB40, dB44, dB48, dB52, dB56, dB60}                     OPTIONAL    -- Need R

]]

}

CandidateTCI-UL-State-r18 ::=           SEQUENCE {

tci-UL-StateId-r18                      TCI-UL-StateId-r17,

referenceSignal-r18                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

},

pathlossReferenceRS-Id-r18              PathlossReferenceRS-Id-r17                                   OPTIONAL,   -- Need R

tag-Id-ptr-r18                          ENUMERATED {n0,n1}                                           OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                     Uplink-powerControlId-r17                                    OPTIONAL,   -- Cond separateTCI

...,

[[

pathlossOffset-r19                  ENUMERATED {

dB-12, dB-8, dB-4, dB0, dB4, dB8,

dB12, dB16, dB20, dB24, dB28, dB32, dB36,

dB40, dB44, dB48, dB52, dB56, dB60}                     OPTIONAL    -- Need R

]]

}

Remove the field pucch-Cell-r19 and update the description of CSI-ReportConfig:

[Comments]:

[ZTE]No conclusion yet, thus it’s better to wait for progress on this.

[Ericsson(Lian)] This seems to be a cross-WI issue and sould be discussed by LTM as well.

[Nokia (Andrew)] We also think this should not be discussed within the MIMO agenda item as it is a cross-WI issue.

# H130

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H130 | NES | 1 | SSB adaptation for deactivated SCell | R2-25xxxxx | Huawei (Lili) |  | V034 |  |

[Description]:

According to 38.321, when an SCell is deactivated, the MAC entity shall not monitor PDCCH on the SCell or for the SCell:

1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell;

The above PDCCH monitoring behaviour inherits from LTE R10. With this restriction, the SSB adaptation indicated by DCI cannot apply to deactivated SCell. Even though we think dynamically changing the SSB periodicity to a smaller value for fast SCell activation is a valid use case, we prefer not to change the PDCCH monitoring behaviour or to revert RAN2 previous agreement of not introducing new MAC CE. This fast SCell activation can be enabled by OD-SSB operation. Under OD-SSB discussion, it is allowed to use the OD-SSB MAC CE to dynamically change the OD-SSB configuration including SSB periodicity. Alternatively, the network can use RRC reconfiguration to update the SSB periodicity of a deactivated SCell.

[Proposed Change]:

[[

od-ssb-r19                      SetupRelease {OD-SSB-r19}                                       OPTIONAL,   -- Need M

adap-SSB-Config-r19             SetupRelease {Adap-SSB-Config-r19}                              OPTIONAL    -- Cond SSB-AdaptM

]]

| Conditional Presence | Explanation |
| --- | --- |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N: - in the masterCellGroup at - SCell addition, - reconfiguration with sync, - resume of an RRC connection. - in the secondaryCellGroup, when the SCG is not indicated as deactivated at: - SCG activation while the SCG was previously deactivated, - SCell addition, - reconfiguration with sync. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |
| SCellSIB20-Opt | This field is optionally present, Need M, if the field sCellSIB20 is configured. It is absent otherwise. |
| SCG-Opt | The field is optionally present, Need M, in an SpCellConfig for the PSCell. It is absent otherwise. |
| SSB-Adapt | The field is optionally present, Need M, for an activated SCell. It is absent otherwise. |

[Comments]:

# O007

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O007 | NES | 2 | Optionality of fields for offset |  | OPPO (Qianxi) |  | V004 | PropReject |

[Description]: For the two fields od-ssb-halfFrameIndex and od-ssb-SFN-Offset, since it says “If the field is absent, the UE applies the value 0.” And since R1 RRC list does not say it has to be mandatory present, we wonder whether it is necessary to restrict that “The field is mandatory present, when absoluteFrequencySSB of the serving cell is absent.”

[Proposed Change]: We are fine with either way, yet good to have R2 conclusion on it since it was not from R1 RRC parameter list directly.

[Comments]:

[Ericsson] there is RAN1 agreement though:

Agreement

- For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

  - For Case #1 (i.e., No always-on SSB on the cell),

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

    - Alt A: Same as for Case #1

    - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

[Apple] Agree with Ericsson.

[Rapporteur] The proposed change is not needed.

# L203

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L203 | NES | 2 | Missing scenarios for OD-SSB transmission. |  | LGE (Han Cha) |  | V016 | PropAgree |

[Description]: According to the current implementation of OD-SSB SFN offset, for example, following three cases cannot be supported illustrated in figure below. In the figure, each case has the same OD-SSB periodicity but has different OD-SSB SFN offset.

[Proposed Change]: Move od-ssb-SFN-Offset-r19 to OD-SSB-Config-r19 from OD-SSB-r19. The text proposal as follows:

OD-SSB-r19 ::=      SEQUENCE {

od-ssb-halfFrameIndex-r19               ENUMERATED {zero, one}                                             OPTIONAL,      -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19            ARFCN-ValueNR                                                      OPTIONAL,      -- Cond ODssbAOssb

od-ssb-SubcarrierSpacing-r19            SubcarrierSpacing                                                  OPTIONAL,      -- Cond ODssbOnly

od-ssb-PBCH-BlockPower-r19              INTEGER (-60..50)                                                  OPTIONAL,      -- Cond ODssbOnly

od-SSB-ConfigToAddModList-r19           SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-Config-r19       OPTIONAL,      -- Need N

od-SSB-ConfigToReleaseList-r19          SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-ConfigId-r19     OPTIONAL       -- Need N

}

[unchanged parts are omitted]

OD-SSB field descriptions

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS 38.213.

od-SSB-ConfigToAddModList List of OD-SSB configurations for this serving cell. Network configures maximum of one different OD-SSB frequency for the OD-SSB configurations which has than the serving cell SSB frequency.

od-ssb-halfFrameIndex Indicates whether OD-SSB is in the first half or the second half of the frame. If the field is absent, the UE applies the value 0.

od-ssb-PBCH-BlockPower Indicates average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7. If the field is absent, UE applies the value PBCH-BlockPower configured in IE ServingCellConfigCommon.

od-ssb-SubcarrierSpacing Indicates subcarrier spacing of OD-SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz If the field is absent, the UE applies the value subcarrierSpacing configured in IE BWP.

[unchanged parts are omitted]

#### –	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by od-ssb-ActivationStatus or by a MAC CE see TS 38.321 [3], clause 6.1.3.x.

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-SFN-Offset-r19                  INTEGER (0..15)                                                    OPTIONAL,      -- Cond ODssbAOssb

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated}                                                      OPTIONAL, -- Need S

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 },

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-NrofBursts-r19                  ENUMERATED {n5, n10, n15, n20, n25, n30, n40, n50, n75, n100, n150, n200}   OPTIONAL, -- Cond MACCE

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

OD-SSB-Config field descriptions

od-ssb-ActivationStatus Indicates the activation status of this OD-SSB pattern upon configuration. Only one OD-SSB pattern can be activated at one point of time.

od-ssb-NrofBursts Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is activated. Network only configures this field when od-ssb-ActivationStatus is absent. For FR1, the value range is {5, 10, 15, 20, 25, 30, 40, 50}. For FR2, the value range is {25, 30, 40, 50, 75, 100, 150, 200}.

od-ssb-Periodicity The SSB periodicity in ms.

od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, UE applies the value ssb-PositionsInBurst provided in ServingCellConfigCommon.

od-ssb-SFN-Offset Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (od-ssb-Periodicity) = 0. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

| Conditional Presence | Explanation |
| --- | --- |
| ODssbAOssb | The field is mandatory present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is optionally present otherwise. |
| MACCE | The field is optionally present, Need R, when od-ssb-ActivationStatus is absent. It is absent otherwise. |

[Comments]:

[Ericsson]

Yes, see N001 and response there

[Apple] Agree (same as N002).

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting.

# S028

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S028 | NES | 2 | OD-SSB-r19 |  | Anil Agiwal (Samsung) |  | V023 | Duplicate |

[Description]: The two parameters, od-ssb-SFN-Offset-r19 and od-ssb-halfFrameIndex-r19, shall be within OD-SSB-Config, and can be activated and adapted based on MAC CE, together with the periodicity.

RAN1 agreement and RAN1 spec are quoted as follow:

Agreement

•	For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

o	For Case #1 (i.e., No always-on SSB on the cell),

	Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

•	The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

•	If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

•	If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

•	The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

•	The value range of half frame index is 0 or 1.

o	For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

	Alt A: Same as for Case #1

	Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

TS 38.213 v19.0.0

-    the half frames for the transmission of the second SS/PBCH blocks are determined based on an indication by a MAC CE

-    the transmission of the second SS/PBCH blocks is in frames with SFN determined from (SFN+SFN_offset)⋅10 mod P=0, where P is the periodicity for the transmission of the second SS/PBCH blocks, and SFN_offset is the indicated SFN offset by the MAC CE from candidate values by od-ssb-sfn-Offset, if provided; else, SFN_offset=0. An index of a half frame with transmission of the second SS/PBCH blocks in a corresponding frame is indicated by the MAC CE from candidate values by od-ssb- halfFrameIndex, if provided; else the index is 0

[Proposed Change]: The two parameters, od-ssb-SFN-Offset-r19 and od-ssb-halfFrameIndex-r19, should be moved to OD-SSB-Config-r19

[Comments]:

[Apple] Agree (same as N002).

[Rapporteur] This is a duplicated issue. Please see N002

# N002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N002 | NES | 1 | Sfn offset and halfframeindex are now common for all od-ssb configs. This may be wrong |  | Jarkko Koskela (Nokia) |  | V009 | PropAgree |

[Description]: SFN offset and half frame index parameters are now common for all SSB configs. This is opposite to SSB adaptation configuration. It seems also counter intuitive that one cannot have OD_SSB specific time domain parameters as one can have different periodicity?.

[Proposed Change]: Consider moving these two parameters to Od-ssb-config-r19  We encourage people to check with their Ran1 delegates what is the intention from Ran1.

[Comments]: [OPPO] Same view.

[vivo] Agree with Nokia.

[LGE] Support Nokia’s proposal. For SFN offset for OD-SSB, we provide detailed comment in RIL:[L203], NES.

[Ericsson] Seems valid as per below RAN1 agreement

Agreement

- For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

  - For Case #1 (i.e., No always-on SSB on the cell),

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

    - Alt A: Same as for Case #1

    - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

[Apple] Agree as we mentioned under N001.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# L204

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L204 | NES | 2 | Correction on maximum value range for adap-ssb-Offset-r19 |  | LGE (Han Cha) |  | V016 | PropAgree |

[Description]: According to the consolidated parameters list received from RAN1 after RAN1#121, the value range of adap-ssb-Offset-r19 is INTEGER (0..15).

[Proposed Change]: The text proposal is as below:

Adap-SSB-BurstPeriodicity-r19 ::=      SEQUENCE {

adap-ssb-Periodicity-r19           ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,      -- Need S

adap-ssb-Offset-r19                INTEGER (0..15)                                        						  OPTIONAL,      -- Need N

adap-ssb-halfFrameIndex-r19        ENUMERATED { firsthalf, secondhalf }                                    OPTIONAL       -- Need N

}

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

[Comments]: Agree and see also our comment N003 for haflframeindex

[Apple] Agree.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# N003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N003 | NES | 1 | Haflframe index parameters for ssb adapatation has different values as for OD-SSB |  | Jarkko Koskela (Nokia) |  | V020 | PropAgree |

[Description]: Not sure but adap-ssb-halfFrameIndex has values “firsthalf, secondhalf”? but in the od-ssb for same parameter values are zero and one? It seems zero and one are more used generally in the other places as well.

[Proposed Change]: Changes values of adap-ssb-halfFrameIndex to “zero, one”

| SSBorAssociatedSSB2 | If the measObject is associated to an SCell with either SSB or OD-SSB, this field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If the measObject is associated to an SSB-less SCell, this field is optionally present, Need R, if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If ssb-ConfigMobility is not configured and associatedSSB is not configured for any cell, the field is absent, Need R. |
| --- | --- |

[Comments]:

[Apple] Agree. see copy from RAN1 excel (RAN1 suggests [0,1]:

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# V501

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V501 | NES | 1 | update field description for od-ssb-absoluteFrequency |  | vivo (Jianhui) |  | V015 | PropAgree |

[Description]:

For the following FD:

OD-SSB field descriptions

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS 38.213.

RAN1 once agreed that

Agreement for RAN1 #120bis

For a cell supporting on-demand SSB SCell operation,

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2

  - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB.

and now in TS 38.213, V19.0.0, the agreement is captured in related text as:

<irrelevant text omitted> the frequency location of the second SS/PBCH blocks is indicated by od-absoluteFrequencySSB, if provided; otherwise, by absoluteFrequencySSB

We understand that the correct comprehension for od-ssb-absoluteFrequency is that it can be the same or different from absoluteFrequencySSB. It is only when od-ssb-absoluteFrequency is the same as absoluteFrequencySSB that it can be absent.

However: 1) even if od-ssb-absoluteFrequency is the same as absoluteFrequencySSB, it can still be configured; 2) if absoluteFrequencySSB is not configured (SSB-less SCell), then od-ssb-absoluteFrequency must be present for OD-SSB configuration, and it has nothing to compare whether it is different from absoluteFrequencySSB or not.

Therefore, we think the current FD for od-ssb-absoluteFrequency needs to be modified as follows. Note that there are lots of field description in current RRC spec to use ‘If the field is absent, xxxxxxxx’ already.

[Proposed Change]:

OD-SSB field descriptions

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB. If the field is absent, the UE applies absoluteFrequencySSB indicated in FrequencyInfoDL for this serving cell, if configured. Additional restrictions as described in subclause 4.4. of TS 38.213.

[Comments]:

[Ericsson] agree with the intention, Text looks ok as well at first glance.

[Apple] We think the change may not be necessary because reasonable NW configuration is not to configure this field when it is same as absoluteFrequencySSB. But we are fine if majority prefer the change.

[vivo] The current FD needs modification anyway, as it says ‘Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB’, which implies the IE absoluteFrequencySSB is always configured. In this sense, OD-SSB case#1 is never supported from signalling perspective.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# H129

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H129 | NES | 1 | Field description of od-ssb-absoluteFrequency-r19 | R2-25xxxxx | Huawei (Lili) |  | V034 |  |

[Description]: In our understanding,

- When both absoluteFrequencySSB (for AO-SSB) and od-ssb-absoluteFrequency-r19 (for OD-SSB) are present, it refers to the case where AO-SSB and OD-SSB are inter-frequency.

- When absoluteFrequencySSB (for AO-SSB) is present, OD-SSB-r19 is present and od-ssb-absoluteFrequency-r19 is absent, it refers to the case where AO-SSB and OD-SSB are intra-frequency.

For clarity, and also for saving signalling overhead, we do not see the motivation to repeat the SSB frequency (as absoluteFrequencySSB) to od-ssb-absoluteFrequency-r19.

[Proposed Change]:

od-ssb-absoluteFrequency

Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS 38.213. This field is only included when the on-demand SSB has a different SSB frequency from the always-on SSB of the SCell.

[Comments]:

# C182

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C182 | NES | 1 | Ambiguity on SFN of which cell is referring to by the filed od-ssb-SFN-Offset |  | Rui (CATT) |  | V008 | PropAgree |

[Description]: In the field description of od-ssb-SFN-Offset, it is not clear SFN of which cell(e.g., SFN of pcell or  SFN scell) is referred,there is a need to clarify it is the SFN of the scell.

[Proposed Change]:

od-ssb-SFN-Offset

Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0, where SFN refers to the SFN of the NR serving cell. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

[Comments]:

[Ericsson] ok but could say “this serving cell”

[Apple] Prefer Ericson’s version/wording.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting as follows: “where SFN refers to the SFN of this serving cell”

# L205

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L205 | NES | 1 | Correction on the field description of adap-ssb-Offset |  | LGE (Han Cha) |  | V016 | PropAgree |

[Description]: The sentence for adap-ssb-Offset is included in the field description of adap-ssb-halfFrameIndex.

[Proposed Change]: Move the sentence from the field description of adap-ssb-halfFrameIndex to the field description of adap-ssb-Offset. The text proposal is as below:

SCellConfig field descriptions

adap-PosInDCI-ssbPeriodicityIndicationForScell The starting bit position of an information block of DCI format 2_9 for SSB burst periodicity switching of this serving cell (see TS 38.212 [17], subclause 7.3.1.3.10).

adap-ssb-halfFrameIndex Indicate whether SSB according to the adap-ssb-Periodicity is in the first half or the second half of the frame.

adap-ssb-Periodicity Additional SSB burst periodicity for the Scell. If the field is absent, the UE applies the value ms5.

adap-ssb-Offset Indicate SFN offset from the SFN which satisfies (SFN index *10) modulo (adap-ssb-Periodicity) = 0. The network configures this field according to adap-ssb-Periodicity such that the indicated system frame does not exceed the corresponding adaptive SSB periodicity.

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in an SCell in RRC_CONNECTED. This field is always configured when the network enables BFD relaxation for the UE in this SCell. This field is absent if failureDetectionSetN is present for the SCell.

preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated while this SCell is deactivated. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration. If the field is included for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the default DRX group shall belong to another Frequency Range. If drx-ConfigSecondaryGroup is configured, the field is optionally present. The network always includes the field if the field was previously configured for this SCell and the SCell remains in the secondary DRX group. Removal of an individual SCell from the secondary DRX group is supported by using an SCell release and addition. Otherwise, if drx-ConfigSecondaryGroup is not configured, the field is absent and the UE shall release the field. The UE shall also release the field if drx-ConfigSecondaryGroup is released without including sCellToAddModList.

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent and absoluteFrequencySSB is included, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. If the SCell is an SSB-less SCell (i.e., the IE absoluteFrequencySSB in ServingCellConfigCommon is absent), this field is absent.

[Comments]:

[Apple] Agree. See copy of RAN1 excel. They are indeed mis-placed.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# L206

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L206 | NES | 1 | Clarification on field description of adap-ssb-Periodicity-r19 |  | LGE (Han Cha) |  | V016 | PropAgree |

[Description]: When the network configures SSB burst periodicity for SSB adaptation, the network shall ensure that the SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity according to the RAN1 agreement. However, the current version does not reflect RAN1’s intension.

Related RAN1 working assumption and agreement are as follows:

RAN1#120bis Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst RAN1#121 Agreement When the SSB burst periodicity is switched from periodicity value P1 to periodicity value P2 based on DCI format 2_9 indication, - Alt 1: SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value. - the first SSB burst according to the periodicity value P2 is determined as the first SSB burst according to the SSB burst periodicity value P2 and associated SFN offset and half-frame index that is no earlier than slot m+d. SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

[Proposed Change]: Add clarification on the field description of adap-ssb-Periodicity. The text proposal is as below:

SCellConfig field descriptions

adap-PosInDCI-ssbPeriodicityIndicationForScell The starting bit position of an information block of DCI format 2_9 for SSB burst periodicity switching of this serving cell (see TS 38.212 [17], subclause 7.3.1.3.10).

adap-ssb-halfFrameIndex Indicate whether SSB according to the adap-ssb-Periodicity is in the first half or the second half of the frame. The network configures this field according to adap-ssb-Periodicity such that the indicated system frame does not exceed the corresponding adaptive SSB periodicity.

adap-ssb-Periodicity Additional SSB burst periodicity for the Scell. If the field is absent, the UE applies the value ms5. If the network configures adap-ssb-Periodicity, the network ensures that SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

adap-ssb-Offset Indicate SFN offset from the SFN which satisfies (SFN index *10) modulo (adap-ssb-Periodicity) = 0.

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in an SCell in RRC_CONNECTED. This field is always configured when the network enables BFD relaxation for the UE in this SCell. This field is absent if failureDetectionSetN is present for the SCell.

preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated while this SCell is deactivated. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration. If the field is included for an SCell configured with TRS for fast activation of the SCell, such TRS is not used for the corresponding SCell.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the default DRX group shall belong to another Frequency Range. If drx-ConfigSecondaryGroup is configured, the field is optionally present. The network always includes the field if the field was previously configured for this SCell and the SCell remains in the secondary DRX group. Removal of an individual SCell from the secondary DRX group is supported by using an SCell release and addition. Otherwise, if drx-ConfigSecondaryGroup is not configured, the field is absent and the UE shall release the field. The UE shall also release the field if drx-ConfigSecondaryGroup is released without including sCellToAddModList.

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent and absoluteFrequencySSB is included, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. If the SCell is an SSB-less SCell (i.e., the IE absoluteFrequencySSB in ServingCellConfigCommon is absent), this field is absent.

[Comments]:

[Rapporteur] The proposed change will be captured in the rapporteur CR to the next meeting as follows: “If configured, the network ensures that SSB transmission occasions with larger periodicity are subset of SSB transmission occasions with shorter periodicity.”

# H128

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H128 | NES | 1 | Conditional presence of od-ssb-absoluteFrequency-r19 | R2-25xxxxx | Huawei (Lili) |  | V034 |  |

[Description]: In our understanding, the absence/presence of absoluteFrequencySSB and OD-SSB-r19 indicate the different scenarios:

| absoluteFrequencySSB | OD-SSB-r19 |  |
| --- | --- | --- |
| absent | absent | SSB-less |
| absent | present | SSB-less + on-demand SSB (meaning that when RAN4 requirements are satisfied the cell can be SSB-less, otherwise the cell can activate on-demand SSB transmission) |
| present | absent | AO-SSB |
| present | present | OD-SSB + AO-SSB (absoluteFrequencySSB indicates AO-SSB frequency) |

Based on the above understanding, we think the conditional presence of od-ssb-absoluteFrequency-r19 is incorrect, because it ruled out the legacy SSB-less case. Regardless of whether absoluteFrequencySSB is present or absent, we think the can be optional. The conditional presence should be removed.

[Proposed Change]:

od-ssb-absoluteFrequency-r19            ARFCN-ValueNR                                                      OPTIONAL,      -- Need R

[Comments]:

# C183

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C183 | NES | 1 | Incorrect description for the condition ODssbOnly |  | Rui (CATT) |  | V008 | PropReject |

[Description]: The condition ODssbOnly is used for field od-ssb-SubcarrierSpacing and od-ssb-PBCH-BlockPower.For case 1, od-ssb-SubcarrierSpacing and od-ssb-PBCH-BlockPower should be mandatory.so the description of ODssbOnly needs to be modified.

[Proposed Change]:

| ODssbOnly | The field is mandatorily present when absoluteFrequencySSB of the serving cell is absent. It is absent otherwise. |
| --- | --- |

[Comments]: [OPPO] “If the field is absent, the UE applies the value subcarrierSpacing configured in IE BWP.”, “If the field is absent, UE applies the value PBCH-BlockPower configured in IE ServingCellConfigCommon.”, so seems there is already a way to handle SSB-less case rather than mandating the presence?

[Ericsson] agree with Oppo

[Apple] Agree with OPPO

[Rapporteur]: The proposed change is not needed.

# X556

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X556 | SONMDT | 2 | Missed cases when neigher of cho or cpc is fulfilled |  | Xiaomi (Shuai) |  | V001 | Rejected |

[Description]: When CHO with candidate SCGs are configured, and neither execution conditions for conditional handover or conditional PSCell change/addition was fulfilled, the current specs has not cover this scenario.

[Proposed Change]: Add “neither” for firstFulfilledConfig IE and fix the procedure text.

Cho-WithCandidateSCGInfo information element

-- ASN1START

-- TAG-CHO-WITHCANDIDATESCGINFO-START

Cho-WithCandidateSCGInfo-r19 ::=                    SEQUENCE{

firstFulfilledConfig-r19                       ENUMERATED {cho, cpc, neither}                             OPTIONAL,

timeBetweenFulfillment-r19                     TimeBetweenEvent-r17                              OPTIONAL,

timeBetweenLastFulfillmentAndEvent-r19         TimeBetweenEvent-r17                              OPTIONAL,

fulfilledConfigWhenChoOnly-r19                 ENUMERATED {cho, cpc, neither}                    OPTIONAL,

pCellId-r19              CHOICE {

cellGlobalId-r19         CGI-Info-Logging-r16,

pci-arfcn-r19            PCI-ARFCN-NR-r16

}                                                                                                OPTIONAL,

psCellId-r19             CHOICE {

cellGlobalId-r19         CGI-Info-Logging-r16,

pci-arfcn-r19            PCI-ARFCN-NR-r16

}                                                                                                OPTIONAL,

...

}

-- TAG-CHO-WITHCANDIDATESCGINFO-STOP

-- ASN1STOP

Cho-WithCandidateSCGInfo field descriptions

firstFulfilledConfig This field indicates if the execution conditions for conditional handover or conditional PSCell change/addition was fulfilled first or neither of them was fulfilled.

fulfilledConfigWhenChoOnly This field indicates if the execution conditions for conditional handover or conditional PSCell change/addition was fulfilled at the time of receiving a complementary conditional reconfiguration i.e., a conditional reconfiguration for a candidate PCell for which atleast one CHO with conditional SCG is already configured.

timeBetweenFulfillment This field logs the time between fulfilment of conditional handover and conditional PSCell change or addition execution conditions.

timeBetweenLastFulfillmentAndEvent This field logs the time between fulfilment of either conditional handover or conditional PSCell change or addition execution conditions and failure for RLF and SCG failure.

[Comments]:

[Rapporteur]: Rapporteur believes in case none of the conditions are fulfilled the field will be absent and it can be implicit indication of neither. Therefore RIL is rejected. Please note that this is different from fulfilledConfigWhenChoOnly since absent of fulfilledConfigWhenChoOnly does not mean neither.

# H310

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H310 | SONMDT | 1 | fulfilledConfigWhenChoOnly | R2-25xxxx | Jun Chen |  | V004 | Agreed |

[Description]: In section 6.3.2, in the IE Cho-WithCandidateSCGInfo, the IE fulfilledConfigWhenChoOnly-r19 has three values, and there should be a spare value. So this spare value can be added.

fulfilledConfigWhenChoOnly-r19                 ENUMERATED {cho, cpc, neither}                    OPTIONAL,

[Proposed Change]: Suggest to:

fulfilledConfigWhenChoOnly-r19                 ENUMERATED {cho, cpc, neither, spare}                    OPTIONAL,

[Comments]:

[Rapporteur]: Rapporteur wonders the use case of the spare value? Please bring a contribution for it for online discussion.

[Rapporteur]: RIL is agreed in RAN2#131bis

# H311

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H311 | SONMDT | 0 | a typo in atleast |  | Jun Chen |  | V004 | Agreed |

[Description]: In section 6.3.2, there is a typo in "atleast", i.e. there should be a space inside.

fulfilledConfigWhenChoOnly

This field indicates if the execution conditions for conditional handover or conditional PSCell change/addition was fulfilled at the time of receiving a complementary conditional reconfiguration i.e., a conditional reconfiguration for a candidate PCell for which atleast one CHO with conditional SCG is already configured.

[Proposed Change]: Suggest to change "atleast" into "at least".

[Comments]:

[Rapporteur]: should be moved to the Class 0 RILs, but agree and captured in the draft CR.

# Z309

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z309 | SONMDT | 1 | Definition of fulfilledConfigWhenChoOnly | R2-25xxxx | QI Tao |  | V012 | Agreed |

[Description]: “CHO with conditional SCG” should be updated to “CHO with candidate SCG”.

[Proposed Change]: update the terms as above.

fulfilledConfigWhenChoOnly

This field indicates if the execution conditions for conditional handover or conditional PSCell change/addition was fulfilled at the time of receiving a complementary conditional reconfiguration i.e., a conditional reconfiguration for a candidate PCell for which atleast one CHO with candidate SCG is already configured.

[Comments]:

[Rapporteur] Rapporture agrees with the RIL and implemented the change in the drafted CR according to the RIL.

# Z310

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z310 | SONMDT | 1 | Definition of timeBetweenLastFulfillmentAndEvent | R2-25xxxx | QI Tao |  | V012 | Agreed |

[Description]: The fulfilment of either cho or cpc should be the latest one if it is met multiple times previously.

[Proposed Change]: add “the latest”.

timeBetweenLastFulfillmentAndEvent

This field logs the time between the latest fulfilment of either conditional handover or conditional PSCell change or addition execution conditions and failure for RLF and or SCG failure.

[Rapporteur]

# Z354

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z354 | SBFD | 1 | Description of CLI-RSSI-MeasResourceId |  | Yu Pan (ZTE) |  | V006 | Agreed |

[Description]: In the description of CLI-RSSI-MeasResourceId, The IE CLI-RSSI-MeasResourceId should be used to identify a CLI-RSSI-MeasResource, not a CLI-RSSI-MeasResourceSet

[Proposed Change]: Change the description to ‘The IE CLI-RSSI-MeasResourceId is used to identify a CLI-RSSI-MeasResourceSet’

[Comments]:

[Rapp] Implemented.

# S001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S001 | MIMO | 2 | Wrong place of cri-TypeI-SinglePanelRI-Restriction-r19 |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: cri-TypeI-SinglePanelRI-Restriction-r19 is used for Rel-15 codebook type 'typeI-SinglePanel' as specified in TS 38.214 clause 5.2.1.4.2:

-	A CSI-ReportConfig can be configured with separate RI restrictions for each of the Ks CSI-RS resources, by higher layer parameter cri-typeI-SinglePanel-ri-restriction-r19 or cri-typeII-ri-restriction-r19, for codebookType set to 'typeI-SinglePanel' or 'typeII-r16', respectively.

However, currently cri-TypeI-SinglePanelRI-Restriction-r19 is placed under Rel-19 codebooktype typeI-SinglePanel-r19, which is wrong. According to RAN1, for CSI-CRI either Rel-15 single panel Type-I or Rel-16 Type-II codebook should be configured for all resources, in addition, the new resource-specific parameters are introduced, which are not intended for Rel-19 codebooktype typeI-SinglePanel-r19 or etypeII-r19, but applied with legacy Rel-15/16 codebook.

[Proposed Change]:

Move cri-TypeI-SinglePanelRI-Restriction-r19 to the block of Rel-15 codebooktype “typeI-SinglePanel” by adding an extention of the Rel-15 codebook block.

[Comments]:

[Nokia (Andrew)] We have the same understanding that the Rel-19 CRI reporting enhancements apply only to Rel-15 typeI-SinglePanel codebook and Rel-16 typeII-r16 codebook and do not apply to refined Rel-19 codebooks for 48/64/128 ports.

[ZTE(Wenting)] Agree with the intention, and the ASN.1 structure can be as follows

CodebookConfig-r19  ::=               SEQUENCE  {

codebookType                          CHOICE {/***********omit the unrelted part**********/} OPTIONAL,   -- Need R

cri-TypeI-SinglePanelRI-Restriction-r19   SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (8))		    OPTIONAL,    -- Need R

cri-TypeI-SinglePanelN1-N2-CBSR-r19        CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19               OPTIONAL,     -- Need R

cri-TypeII-RI-Restriction-r19             SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (4))         OPTIONAL,    -- Need R

cri-TypeII-N1-N2-CBSR-r19                 CRI-TypeII-N1-N2-CBSR-List-r19                          OPTIONAL     -- Need R

}

[Ericsson(Lian)] We can add the signaling as suggested by Wenting with some clarification on codebookConfig field description as well:

codebookConfig

Codebook configuration for Type-1 or Type-2 including codebook subset restriction. Network can only configure one of codebookConfig, codebookConfig-r16, codebookConfig-r17, codebookConfig-r18 or codebookConfig-r19 (including codebookType) in a CSI-ReportConfig. The network includes codebookConfig-v1730 only if codebookConfig-r17 is configured.

[Nokia (Andrew)] A couple of suggestions. 1) CodebookConfig-r19 should be a choice between the R19 codebook types, CRI-TypeI for R15 typeI-SinglePanel, and CRI-TypeII for typeII-r16. 2) The field description should also clarify the cases when CodebookConfig-r19 can be configured simultaneously with legacy codebooks.

CodebookConfig-r19  ::=               CHOICE  {

codebookType                          CHOICE {/***********omit the unrelted part**********/} ,

cri-TypeI-SinglePanel             SEQUENCE {

cri-TypeI-SinglePanelRI-Restriction-r19   SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (8))		    OPTIONAL,    -- Need R

cri-TypeI-SinglePanelN1-N2-CBSR-r19        CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19               OPTIONAL,     -- Need R

},

cri-TypeII                        SEQUENCE {

cri-TypeII-RI-Restriction-r19             SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (4))         OPTIONAL,    -- Need R

cri-TypeII-N1-N2-CBSR-r19                 CRI-TypeII-N1-N2-CBSR-List-r19                          OPTIONAL     -- Need R

}

}

codebookConfig

Codebook configuration for Type-1 or Type-2 including codebook subset restriction. Network can only configure one of codebookConfig, codebookConfig-r16, codebookConfig-r17, codebookConfig-r18 or codebookConfig-r19 (including codebookType) in a CSI-ReportConfig. The network includes codebookConfig-v1730 only if codebookConfig-r17 is configured. For codebookConfig-r19, the network configures cri-TypeI-SinglePanel when typeI-SinglePanel is configured in codebookConfig and configures cri-TypeII only when typeII-r16 is configured in codebookConfig-r16.

# S002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S002 | MIMO | 2 | Wrong place of cri-TypeI-SinglePanelN1-N2-CBSR-r19 |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: cri-TypeI-SinglePanelN1-N2-CBSR-r19 is used for Rel-15 codebook type 'typeI-SinglePanel' as specified in TS 38.214 clause 5.2.1.4.2:

-	A CSI-ReportConfig can be configured with separate Codebook Subset Restrictions for each of the Ks CSI-RS resources, by higher layer parameter cri-typeI-SinglePanel-CBSR-r19 or cri-typeII-CBSR-r19, for codebookType set to 'typeI-SinglePanel' or 'typeII-r16', respectively. For codebookType set to 'typeII-r16', cri-typeII-CBSR-r19 is configured as described in Clause 5.2.2.2.5, where only the bit values '00' or '11' of Table 5.2.2.2.5-6 are configurable.

However, currently cri-TypeI-SinglePanelN1-N2-CBSR-r19 is placed under Rel-19 codebooktype typeI-SinglePanel-r19, which is wrong. According to RAN1, for CSI-CRI either Rel-15 single panel Type-I or Rel-16 Type-II codebook should be configured for all resources, in addition, the new resource-specific parameters are introduced, which are not intended for Rel-19 codebooktype typeI-SinglePanel-r19 or etypeII-r19, but applied with legacy Rel-15/16 codebook.

[Proposed Change]:

Move cri-TypeI-SinglePanelN1-N2-CBSR-r19 to the block of Rel-15 codebooktype “typeI-SinglePanel” by adding an extention of the Rel-15 codebook block.

[Comments]:

[Nokia (Andrew)] We have the same understanding that the Rel-19 CRI reporting enhancements apply only to Rel-15 typeI-SinglePanel codebook and Rel-16 typeII-r16 codebook and do not apply to refined Rel-19 codebooks for 48/64/128 ports.

[ZTE(Wenting)] Same comments as in the S001

[Ericsson(Lian)] Same comments as in the S001

# Z405

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z405 | MIMO | 2 | Missed sub-elements for the ng-n1-n2-r19 |  | ZTE (Wenting Li) |  | v008 | PropReject |

[Description]: For the typeI-MultiPanel-r19, it supports 11 groups, but in the current Asn.1 only 7 groups are included, the 4 groups highlighted in red were missed

Supported values of (Ng, N1,N2):

48 ports: (2,4,3), (2,6,2),(2,12,1)

64 ports: (2,8,2), (2,16,1), (4,4,2), (2,4,4), (4,8,1)

128 ports: (4,4,4), (4,16,1), (4,8,2)

typeI-MultiPanel-r19                   SEQUENCE {

ri-Restriction-r19                        BIT STRING (SIZE (4)),

ng-n1-n2-r19                                  CHOICE {

two-four-three-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (192)),

two-six-two-TypeI-MultiPanel-Restriction-r19                 BIT STRING (SIZE (192)),

two-eight-two-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (256)),

four-four-two-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (128)),

two-four-four-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (256)),

four-four-four-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (256)),

four-eight-two-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (256))

}

}

[Proposed Change]: Change the reference clause as below

Supported values of (Ng, N1,N2):

typeI-MultiPanel-r19                   SEQUENCE {

ri-Restriction-r19                        BIT STRING (SIZE (4)),

ng-n1-n2-r19                                  CHOICE {

two-four-three-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (192)),

two-six-two-TypeI-MultiPanel-Restriction-r19                 BIT STRING (SIZE (192)),

two-twelve-one-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (192)),

two-eight-two-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (256)),

two-sixteen-one-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (256)),

four-four-two-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (128)),

two-four-four-TypeI-MultiPanel-Restriction-r19               BIT STRING (SIZE (256)),

four-eight-one-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (128)),

four-four-four-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (256)),

four-sixteen-one-TypeI-MultiPanel-Restriction-r19            BIT STRING (SIZE (256)),

four-eight-two-TypeI-MultiPanel-Restriction-r19              BIT STRING (SIZE (256))

}

}

[Comments]:

[Ericsson(Lian)] Those groups were identified as not present in RAN1 specifications either, so they were removed.

[Nokia (Andrew)] As reference: TS 38.214 Table 5.2.2.2.2a-1 for typeI-MultiPanel-r19.

Table 5.2.2.2.2a-1: Supported configurations of (Ng,N1,N2) and (O1,O2)

| Number of CSI-RS antenna ports, PCSI-RS | (Ng,N1,N2) | (O1,O2) |
| --- | --- | --- |
| 48 | (2,4,3) | (4,4) |
|  | (2,6,2) | (4,4) |
| 64 | (2,4,4) | (4,4) |
|  | (2,8,2) | (4,4) |
|  | (4,4,2) | (4,4) |
| 128 | (4,4,4) | (4,4) |
|  | (4,8,2) | (4,4) |

# Z406

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z406 | MIMO | 2 | Optimize the structure of the type2 |  | ZTE (Wenting Li) |  | v008 | PropAgree |

[Description]: The typeII-RI-Restriction-r19/ numberOfPMI-SubbandsPerCQI-Subband-r19 are included in all of 3 Type2 types, which can be moved out of the choice structure as the common part of Type2

typeII-RI-Restriction-r19                 BIT STRING (SIZE (4)),

numberOfPMI-SubbandsPerCQI-Subband-r19    INTEGER(1..2),

[Proposed Change]: Change the ASN.1 structure as below:

type2                                   SEQUENCE {

typeII                              CHOICE{

etypeII-r19                             SEQUENCE {

paramCombination-r19                      INTEGER (1..8),

n1-n2-r19		                           ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeII-CodebookSubsetRestriction-r19	   TypeII-X1-X2-CBSR-r19							OPTIONAL,    -- Need R

cri-TypeII-RI-Restriction-r19[RIL]: S003, MIMO             SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (4))         OPTIONAL,    -- Need R

cri-TypeII-N1-N2-CBSR-r19[RIL]: S0004, MIMO                 CRI-TypeII-N1-N2-CBSR-List-r19                 OPTIONAL     -- Need R

},

typeII-FePortSelection-r19              SEQUENCE {

paramCombination-r19                      INTEGER (1..7),

valueOfN-r19                              ENUMERATED {n2, n4}                          OPTIONAL  -- Need R

},

typeII-Doppler-r19                       SEQUENCE {

paramCombination-Doppler-r19              INTEGER (1..9),

td-dd-config-r19                          TD-DD-Config-r18,

predictionDelay-r19                       ENUMERATED {m0,n0,n1,n2},

n1-n2-r19		                          ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeII-CodebookSubsetRestriction-r19		TypeII-X1-X2-CBSR-r19							OPTIONAL    -- Need R

}

}

typeII-RI-Restriction-r19                 BIT STRING (SIZE (4)),

numberOfPMI-SubbandsPerCQI-Subband-r19    INTEGER(1..2)

}

[Comments]:

# Z407

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z407 | MIMO | 2 | Optional Present of the numberOfPMI-SubbandsPerCQI-Subband-r19 |  | ZTE (Wenting Li) |  | v008 | PropAgree |

[Description]: In the RAN1 parameter list table, it says “same as the legacy R in R16/17/18 typeII ”

| numberOfPMI-SubbandsPerCQI-subband-r19 | new | R value (same as the legacy R in R16/17/18 typeII ) |
| --- | --- | --- |

In the legacy field this field is optional present and thus this should also be optional

numberOfPMI-SubbandsPerCQI-Subband-r17 INTEGER(1..2) OPTIONAL, -- Need R

[Proposed Change]:

Add “OPTIONAL, -- Need R” to the  numberOfPMI-SubbandsPerCQI-Subband-r19

[Comments]:

# S003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S003 | MIMO | 2 | Wrong place of cri-TypeII-RI-Restriction-r19 |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: cri-TypeII-RI-Restriction-r19 is used for Rel-16 codebook type 'typeII-r16' as specified in TS 38.214 clause 5.2.1.4.2:

-	A CSI-ReportConfig can be configured with separate RI restrictions for each of the Ks CSI-RS resources, by higher layer parameter cri-typeI-SinglePanel-ri-restriction-r19 or cri-typeII-ri-restriction-r19, for codebookType set to 'typeI-SinglePanel' or 'typeII-r16', respectively.

However, currently cri-TypeII-RI-Restriction-r19 is placed under Rel-19 codebooktype etypeII-r19, which is wrong. According to RAN1, for CSI-CRI either Rel-15 single panel Type-I or Rel-16 Type-II codebook should be configured for all resources, in addition, the new resource-specific parameters are introduced, which are not intended for Rel-19 codebooktype typeI-SinglePanel-r19 or etypeII-r19, but applied with legacy Rel-15/16 codebook.

[Proposed Change]:

Move cri-TypeII-RI-Restriction-r19 to the block of Rel-16 codebooktype 'typeII-r16' by adding an extention of the Rel-16 codebook block.

[Comments]:

[Nokia (Andrew)] We have the same understanding that the Rel-19 CRI reporting enhancements apply only to Rel-15 typeI-SinglePanel codebook and Rel-16 typeII-r16 codebook and do not apply to refined Rel-19 codebooks for 48/64/128 ports.

[ZTE(Wenting)] Same comments as in the S001

[Ericsson(Lian)] Same comments as in the S001

# S004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S004 | MIMO | 2 | Wrong place of cri-TypeII-N1-N2-CBSR-r19 |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: cri-TypeII-N1-N2-CBSR-r19 is used for Rel-16 codebook type 'typeII-r16' as specified in TS 38.214 clause 5.2.1.4.2:

-	A CSI-ReportConfig can be configured with separate Codebook Subset Restrictions for each of the Ks CSI-RS resources, by higher layer parameter cri-typeI-SinglePanel-CBSR-r19 or cri-typeII-CBSR-r19, for codebookType set to 'typeI-SinglePanel' or 'typeII-r16', respectively. For codebookType set to 'typeII-r16', cri-typeII-CBSR-r19 is configured as described in Clause 5.2.2.2.5, where only the bit values '00' or '11' of Table 5.2.2.2.5-6 are configurable.

However, currently cri-TypeII-N1-N2-CBSR-r19 is placed under Rel-19 codebooktype etypeII-r19, which is wrong. According to RAN1, for CSI-CRI either Rel-15 single panel Type-I or Rel-16 Type-II codebook should be configured for all resources, in addition, the new resource-specific parameters are introduced, which are not intended for Rel-19 codebooktype typeI-SinglePanel-r19 or etypeII-r19, but applied with legacy Rel-15/16 codebook.

[Proposed Change]:

Move cri-TypeII-N1-N2-CBSR-r19 to the block of Rel-16 codebooktype 'typeII-r16' by adding an extention of the Rel-16 codebook block.

[Comments]:

[Nokia (Andrew)] We have the same understanding that the Rel-19 CRI reporting enhancements apply only to Rel-15 typeI-SinglePanel codebook and Rel-16 typeII-r16 codebook and do not apply to refined Rel-19 codebooks for 48/64/128 ports.

[ZTE(Wenting)] Same comments as in the S001

[Ericsson(Lian)] Same comments as in the S001

# S005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S005 | MIMO | 2 | Inconsistent value of paramCombination-r19 |  | Samsung (Shiyang) |  | V002 | PropReject |

[Description]: paramCombination-r19 is used for both codebook type etypeII-r19 and typeII-FePortSelection-r19, however the value range for etypeII-r19 is INTEGER (1..8), the value range for typeII-FePortSelection-r19 is INTEGER (1..7). For the same parameter, the value range should be consistent.

[Proposed Change]:

Change the value range of paramCombination-r19 for typeII-FePortSelection-r19 to INTEGER (1..8). This is aligned with RAN1 TS 38.214 clause 5.2.2.2.9a:

-	The values α, M and β are determined by the higher layer parameter paramCombination-r19, where the same mapping is used as in Table 5.2.2.2.7-1.

-	The UE is not expected to be configured with paramCombination-r19 equal to 8.

Note value 1..8 are listed in the legacy Table 5.2.2.2.7-1, and it has been specified by RAN1 clearly that 8 is not configured. So the RRC value range of paramCombination-r19 should be changed to INTEGER (1..8).

[Comments]:

[Ericsson(Lian)] The fact that RAN1 states that value 8 is not configured does not seem to contradict the ASN1 since value 8 cannot be configured for typeII-FePortSelection-r19.

# N051

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N051 | MIMO | 1 | Wrong reference in FD of codebookMode |  | Nokia (Andrew) |  | v004 | PropAgree |

[Description]: codebookMode is also applicable to typeI-SinglePanel-r19 codebooks (TS 38.214 clause 5.2.2.2.1a), yet the reference in the field description refers to clause 5.2.2.2.8 and 5.2.2.9 for typeII-CJT-r18 and typeII-CJT-PortSelection-r18 respectively; meanwhile codebookMode is also applicable to various typeI-SinglePanel and typeI-MultiPanel codebooks. The reference should just be generalized to clause 5.2.2.2.

[Proposed Change]:

codebookMode

CodebookMode as specified in TS 38.214 [19], clause 5.2.2.2.

[Comments]:

[ZTE(wenting)] We share the same view as Nokia

# S006

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S006 | MIMO | 1 | Missing information in FD of cri-TypeI-SinglePanelN1-N2-CBSR |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: FD of cri-TypeI-SinglePanelN1-N2-CBSR should be updated, to distinguish from FD of cri-TypeII-N1-N2-CBSR, currently the two have exactly the same FD.

[Proposed Change]:

cri-TypeI-SinglePanelN1-N2-CBSR

Number of antenna ports in first (n1) and second (n2) dimension and resource-specific codebook subset restriction for codebookType set to typeI-SinglePanel (see TS 38.214 [19] clause 5.2.1.4.2). Value no-cbsr means no codebook subset restriction is configured for the n1-n2 pair.

If a codebook subset restriction is configured for the n1-n2 pair, the number of elements in cbsr-list is up to the number of elements of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet(s) indicated by nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig in which the CodebookConfig is included. An element in the list corresponds to the element at the same position in nzp-CSI-RS-Resources.

[Comments]:

# S007

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S007 | MIMO | 1 | Missing information in FD of cri-TypeI-SinglePanelRI-Restriction |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: FD of cri-TypeI-SinglePanelRI-Restriction should be updated, currently it does not read correctly.

[Proposed Change]:

cri-TypeI-SinglePanelRI-Restriction

Resource-specific RI restriction for codebookType set to typeI-SinglePanel (see TS 38.214 [19], clause 5.2.1.4.2).

[Comments]:

[ZTE(Wenting)]We share the same view as Samsung

# S008

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S008 | MIMO | 1 | Missing information in FD of cri-TypeII-N1-N2-CBSR |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: FD of cri-TypeII-N1-N2-CBSR should be updated, to distinguish from FD of cri-TypeI-SinglePanelN1-N2-CBSR, currently the two have exactly the same FD.

[Proposed Change]:

cri-TypeII-N1-N2-CBSR

Number of antenna ports in first (n1) and second (n2) dimension and resource-specific codebook subset restriction for codebookType set to typeII-r16 (see TS 38.214 [19] clause 5.2.1.4.2). Value no-cbsr means no codebook subset restriction is configured for the n1-n2 pair.

If a codebook subset restriction is configured for the n1-n2 pair, the number of elements in cbsr-list is up to the number of elements of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet(s) indicated by nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig in which the CodebookConfig is included. An element in the list corresponds to the element at the same position in nzp-CSI-RS-Resources.

[Comments]:

# S009

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S009 | MIMO | 1 | Missing information in FD of cri-TypeII-ri-Restriction |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: FD of cri-TypeII-ri-Restriction should be updated, currently it does not read correctly.

[Proposed Change]:

cri-TypeII-ri-Restriction

Resource-specific RI restriction for codebookType set to typeII-r16 (see TS 38.214 [19], clause 5.2.1.4.2).

[Comments]:

[ZTE(Wenting)]We share the same view as Samsung

# N052

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N052 | MIMO | 1 | Wrong reference in FD of n1-n2 |  | Nokia (Andrew) |  | v004 | PropAgree |

[Description]: n1-n2 is also applicable to typeI-SinglePanel-r19, etypeII-r19, and typeII-Doppler-r19 codebooks, yet the reference in the field description refers to clause 5.2.2.2.1, which is for Rel-15 type-I-SinglePanel codebooks only. The reference should just be generalized to clause 5.2.2.2.

[Proposed Change]:

n1-n2

Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2).

[Comments]:

[ZTE(Wenting)] We think the n1-n2-r19 definition and the structure is quite different from the legacy n1-n2, to avoid the confusion, it’s better to use a different name, and add separate field escriptin for it. Maybe this RIL can be discussed together with our RILZ404

[Ericsson(Lian)] Since both field descriptions are not detailed in RAN2, it seems better to just keep the same field description for both cases.

# Z404

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z404 | MIMO | 2 | Change the name of n1-n2-r19 to distinghish it from the legacy n1-n2 |  | ZTE (Wenting Li) |  | v008 | PropReject |

[Description]: The n1-n2-r19 definition is quite different from the legacy n1-n2, to avoid the confusion, it’s better to use a different name, and add separate field descriptin for it

[Proposed Change]: Change the name from n1-n2-r19 to n1-n2-TypeI-r19 and add separate feiled description as follows

n1-n2-Type1-r19

Number of antenna ports in first (n1) and second (n2) dimension (see TS 38.214 [19] clause 5.2.2.2.1a).

Meanwhile, if this modification is accepted, use the same name of the FD of typeI-CodebookSubsetRestriction/ typeI-SoftScalingRank

[Comments]:

[Ericsson(Lian)] See resolution on N052.

# N053

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N053 | MIMO | 1 | Wrong reference in FD of ng-n1-n2 |  | Nokia (Andrew) |  | v004 | PropAgree |

[Description]: ng-n1-n2 is also applicable to typeI-MultiPanel-r19 codebooks, yet the reference in the field description refers to clause 5.2.2.2.2, which is for Rel-15 typeI-MultiPanel codebooks only. The reference should just be generalized to clause 5.2.2.2.

[Proposed Change]:

ng-n1-n2

Codebook subset restriction for Type I Multi-panel codebook (see TS 38.214 [19], clause 5.2.2.2).

[Comments]:

[ZTE(Wenting)] We agree with Nokia

# N054

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N054 | MIMO | 1 | Wrong reference in FD of numberOfPMI-SubbandsPerCQI-Subband |  | Nokia (Andrew) |  | v004 | PropAgree |

[Description]: numberOfPMI-SubbandsPerCQI-Subband is also applicable to etypeII-r19, typeII-FePortSelection-r19 and typeII-Doppler-r19 codebooks, yet the reference in the field description only refers to clauses for typeII-r16, typeII-CJT-r18, and typeII-CJT-PortSelection-r18. The reference should just be generalized to clause 5.2.2.2.

[Proposed Change]:

numberOfPMI-SubbandsPerCQI-Subband

Field indicates how PMI subbands are defined per CQI subband according to TS 38.214 [19], clause 5.2.2.2).

[Comments]:

[ZTE(Wenting)] We agree with Nokia

# N055

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N055 | MIMO | 1 | Wrong reference in FD of ri-Restriction |  | Nokia (Andrew) |  | v004 | PropAgree |

[Description]: ri-Restriction is also applicable to typeI-MultiPanel-r19 codebooks (TS 38.214 clause 5.2.2.2.2a), yet the reference in the field description refers to clause 5.2.2.2.2, which is for Rel-15 typeI-MultiPanel codebooks only. The reference should just be generalized to clause 5.2.2.2.

[Proposed Change]:

ri-Restriction

Restriction for RI for TypeI-MultiPanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2).

[Comments]:

[ZTE(wenting)] We share the same view as Nokia

# N056

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N056 | MIMO | 1 | Wrong reference in FD of typeI-SinglePanel-ri-Restriction |  | Nokia (Andrew) |  | v004 | PropAgree |

[Description]: typeI-SinglePanel-ri-Restriction is also applicable to typeI-SinglePanel-r19 codebooks (TS 38.214 clause 5.2.2.2.1a), yet the reference in the field description refers to clause 5.2.2.2.1, which is for Rel-15 typeI-SinglePanel codebooks only. The reference should just be generalized to clause 5.2.2.2.

[Proposed Change]:

typeI-SinglePanel-ri-Restriction

Restriction for RI for TypeI-SinglePanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2).

[Comments]:

[ZTE(Wenting)] Agree. Or we can add clause 5.2.2.2.1a

Restriction for RI for TypeI-SinglePanel-RI-Restriction (see TS 38.214 [19], clause 5.2.2.2.1 and clause 5.2.2.2.1a).

[Ericsson(Lian)] We can take Wenting’s suggestion into account.

# N057

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N057 | MIMO | 1 | Wrong reference in FD of valueOfN |  | Nokia (Andrew) |  | v004 | PropAgree |

[Description]: valueOfN is also applicable to typeII-FePortSelection-r19 codebooks (TS 38.214 clause 5.2.2.2.9a), yet the reference in the field description refers to clause 5.2.2.2.7, which is for Rel-17 typeII-PortSelection codebooks only. The reference should just be generalized to clause 5.2.2.2.

[Proposed Change]:

valueOfN

Field provides the value of parameter N as specified in TS 38.214 [19], clause 5.2.2.2. The field is present only when M=2 set by paramCombination, see TS 38.214 [19].

[Comments]:

# H351

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H351 | SBFD | 1 | Revision of value range | - | Huawei-Tao Cai |  | V004 | Agreed |

[Description]: According to RAN1 updated higher layer parameters list in R1-2506622, the range of parameter sbfd-Config2-PUSCH-RB-Offset-r19 shall be (0..maxNrofPhysicalResourceBlocks-1) not (0..maxNrofPhysicalResourceBlocks)

[Proposed Change]: Change the parameter range according to RAN1 revision.

[Comments]:

[Rapp] Implemented in the WI Rapp CR.

# K101

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K101 | MIMO | 1 | Field description update for nrofHARQ-Processes regarding mode-B CG |  | ASUSTeK (Xinra) |  | V005 | PropAgree |

[Description]: RAN2 has confirmed that the CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH), meaning that HARQ operation should not be used for transmitting the beam report. Therefore, configuring nrofHARQ-Processes for such CG is unnecessary. However, since this field is currently mandatory, it may cause confusion for the network regarding how to set the value. The best way for the UE is to ignore this field to avoid occupying HARQ processes in MAC layer.

[Proposed Change]:

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1. If the UE is configured with nrofHARQ-Processes-v1700, the UE shall ignore nrofHARQ-Processes (without suffix). The network sets the value of this field to 1 when cg-LTM-Configuration is configured. The UE shall ignore this field if it’s configured within ConfiguredGrantConfig for UE-initiated beam reporting in mode-B.

[Comments]:

# S017

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S017 | MIMO | 1 | Update description of CSI-AperiodicTriggerStateList to make it applicable to UEI-CSI |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]:

For mode-A UEI-CSI reporting, CSI trigger state indication in DCI is used as similar to AP-CSI reporting. Based on RAN1 agreement, the existing CSI request field in DCI for AP-CSI reporting is reused for UEI-CSI reporting and the existing mechanism for CSI trigger state is applied, but the CSI trigger state for UEI reporting is dedicated for UEI-CSI report, not shared with AP-CSI report.

However, the currently CSI-AperiodicTriggerStateList is only applied for AP-CSI trigger state as specified in RRC. The description should be updated.

[Proposed Change]:

#### –	CSI-AperiodicTriggerStateList

The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of CSI trigger states for aperiodic CSI and UE-initiated CSI reporting. Each codepoint of the DCI field "CSI request" is associated with one trigger state (see TS 38.321 [3], clause 6.1.3.13). Upon reception of the value associated with a trigger state, the UE will perform measurement of CSI-RS, CSI-IM and/or SSB (reference signals) and aperiodic/UE-initiated reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state.

[Comments]:

[Ofinno] We agree with the first proposed change by Samsung. However, for the second change, we suggest splitting the aperiodic CSI reporting and UE-initiated reporting into two sentences. Because in the case of UEIBR, the UE does not perform measurements of RSs upon/after reception of the triggering DCI. The UE performs measurements on RSs to detect the event before receiving the DCI. Thus, the second change is not correct for UEIBR case.

Our proposed change:

The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of CSI trigger states for aperiodic CSI and UE-initiated CSI reporting. Each codepoint of the DCI field "CSI request" is associated with one trigger state (see TS 38.321 [3], clause 6.1.3.13). Upon reception of the value associated with a trigger state for aperiodic CSI reporting, the UE will perform measurement of CSI-RS, CSI-IM and/or SSB (reference signals) and aperiodic CSI reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state. According to all entries in the associatedReportConfigInfoList for a trigger state for UE-initiated CSI reporting, the UE performs measurement of CSI-RS, CSI-IM and/or SSB (reference signals), and upon reception of the value associated with that trigger state, the UE will perform UE initiated-CSI reporting on L1.

# Z414

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z414 | MIMO | 2 | Missed parameter for the CSI-CJTC triggerningScheme |  | ZTE (Wenting Li) |  | v008 | PropReject |

[Description]:

Missed parameter for the CSI-CJTC triggerningScheme

| CSI-CJTC | CSI-AperiodicTriggerState | triggeringScheme | New | Configuring the triggering scheme either separate triggering or joint triggering | {separate, joint} |
| --- | --- | --- | --- | --- | --- |

[Proposed Change]:

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...,

[[

ap-CSI-MultiplexingMode-r17         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]],

[[

ltm-AssociatedReportConfigInfo-r18  LTM-CSI-ReportConfigId-r18                                    OPTIONAL  -- Need R

]],

[[

delayOffsetCompensation-r19         ENUMERATED {enabled}                                          OPTIONAL,  -- Need R

triggeringScheme-r19                ENUMERATED {separate, joint}                                  OPTIONAL  -- Need R

]]

}

[Comments]:

[Ericsson(Lian)] We agreed in RAN2#129-bis to not add this field.

# H352

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H352 | SBFD | 1 | missing "optional" tag | - | Huawei-Tao Cai |  | V004 | Agreed |

[Description]: There is no optionality tag for field resourcesForChannelCLI-r19 (which makes it mandatory). gNB has to configure resourcesForChannelCLI-r19) with at least an empty resourcesForChannelCLI-r19 which would be a waste of signalling (its elements resourceSetCLI-r19 and qcl-infoCLI-r19 are optional).

[Proposed Change]: Add "Optional Need R" for field resourcesForChannelCLI-r19.

[Comments]:

[Rapp] Implemented in the WI Rapp CR.

# S012

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S012 | MIMO | 2 | Missing field for resourcesForChannelCJTC-r19 |  | Samsung (Shiyang) |  | V002 | PropReject |

[Description]: for resourcesForChannelCJTC-r19, qcl-info is needed for each resource set which is similar to legacy resourcesForChannel. According to TS 38.214 clause 5.2.1.4.1:

For aperiodic CSI, a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-Dd', 'cjtc-F', 'cjtc-Dd-F' or 'cjtc-P' is expected to be configured with one CSI Resource Setting (given by higher layer parameter resourcesForChannelMeasurement). For reportQuantity set to 'cjtc-Dd', 'cjtc-F' or 'cjtc-Dd-F', the CSI Resource Setting is periodic with NTRP∈{2, 3,4} CSI-RS Resource Sets configured with higher layer parameter trs-Info. For reportQuantity set to 'cjtc-P', the CSI Resource Setting is configured with NTRP∈{2, 3,4} single-port CSI-RS Resources. Each of NTRP CSI-RS Resources or Resource Sets are configured with a TCI-state by higher layer signalling. The UE expects that all the CSI-RS resources are configured with the same bandwidth. A UE is not expected to be configured with interference measurement on CSI-IM and/or NZP-CSI-RS.

resourcesForChannel                 CHOICE {

nzp-CSI-RS                          SEQUENCE {

resourceSet                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

[Proposed Change]:

resourcesForChannelCJTC-r19     SEQUENCE {

nzp-CSI-RS2-r19                 SEQUENCE {

resourceSet2CJTC-r19                INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info2CJTC-r19                   SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL   -- Need R

},

nzp-CSI-RS3-r19                 SEQUENCE {

resourceSet3CJTC-r19                INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info3CJTC-r19                   SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL   -- Need R

}																						OPTIONAL,  -- Need R

nzp-CSI-RS4-r19                 SEQUENCE {

resourceSet4CJTC-r19                INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info4CJTC-r19                   SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL   -- Need R

}                                                                                     OPTIONAL  -- Need R

}                                                                                                    OPTIONAL -- Cond CJTC

[Comments]:

[ZTE(Wenting)] No strong view, We’d like to confirm whether optional with need R should also be added to the nzp-CSI-RS2-r19

[Ericsson(Lian)] This seems more something to confirm with RAN1, but it seems more productive to trigger this discussion directly in RAN1 so they can inform RAN2.

# S010

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S010 | MIMO | 1 | Wording in FD of delayOffsetCompensation |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: Wording in FD of delayOffsetCompensation should be updated.

[Proposed Change]:

delayOffsetCompensation

Indicates whether the UE should perform delay offset compensation based on the latest linked report with reportQuantity set to 'cjtc-Dd' when codebook type is set to typeII-CJT as specified in TS 38.214 [19] clause 5.2.1.4.2.

[Comments]:

# S011

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S011 | MIMO | 1 | FD of mr-SelectedResources not correct |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: FD of mr-SelectedResources is not correct. The parameter is applied for codebooktype Rel-15 'typeI-SinglePanel' and Rel-16 'typeII-r16', not for the Rel-19 codebook, according to TS 38.214 clause 5.2.1.4.2:

-	The CSI report contains M≤Ks CSIs, where the M sets of CSI parameters other than CRI, RI/LI (if applicable)/PMI/CQI, are independently calculated and indicated for each of the selected M CSI-RS resources. Subject to UE capability, a UE can be configured with M∈{1,…,  min4,Ks} and Ks≤8, if codebookType is set to 'typeI-SinglePanel', and with M∈{1,2} and Ks≤4, if codebookType is set to 'typeII-r16'.

-	If M=2 and codebookType is set to 'typeII-r16', each CSI-RS resource shall contain at most 16 ports and the higher layer parameter numberOfPMI-SubbandsPerCQI-Subband is set to '1'.

-	the CSI report contains M CRIs with the exception that, for aperiodic reporting and subject to UE capability, if CSI-ReportConfig is configured with higher layer parameter mrSelectedResources indicating MR<M CSI-RS resources to be selected for reporting, M-MR CRIs are reported. MR∈{1,2} if codebookType is set to 'typeI-SinglePanel' and MR∈{1}, if codebookType is set to 'typeII-r16'.

[Proposed Change]:

mr-SelectedResources

Indicates one or two resources to be selected among eight CSI-RS resources. This field is only configured for codebookType set to typeI-SinglePanel or typeII-r16. This field is used in clause 5.2.1.4.2 in TS 38.214 [19].

[Comments]:

[ZTE(Wenting)] We share the same view as Samsung, further more a comma is missed for the structure (as highlighted in yellow), and we are not sure whether the  firstSelectedResource-r19 should also be optional.

[[

resourcesForChannelCJTC-r19[RIL]: S012, MIMO     SEQUENCE {

resourceSet2CJTC-r19        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

resourceSet3CJTC-r19        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)             OPTIONAL,  -- Need R

resourceSet4CJTC-r19        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)             OPTIONAL  -- Need R

}                                                                                                    OPTIONAL -- Cond CJTC

mr-SelectedResources-r19         SEQUENCE {

firstSelectedResource-r19        INTEGER (1..8),

secondSelectedResource-r19       INTEGER (1..8)                                               OPTIONAL  -- Need R

}                                                                                                     OPTIONAL  -- Need R

]]

[Ericsson(Lian)] We can also correct the comma mentioned by Wenting. On the firstSelectedResource-r19, if this field would also be optional then it seems there would be no need to include the mr-SelectedResources-r19 field for configuration.

# Z355

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z355 | SBFD | 1 | FD of resourceSetCLI |  | Yu Pan (ZTE) |  | V006 | Agreed |

[Description]: In the FD of resourceSetCLI, ‘Entry number in cli-MeasurementResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on)’ the IE name is incorrect,and the sentence lacks verb

[Proposed Change]: Change the description to ‘This field refers to the entry number in cli-MeasurementResourceSetList cli-MeasResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig indicated by reportConfigId above (value 1 corresponds to the first entry, value 2 to the second entry, and so on)’

[Comments]:

[Rapp] Corrected the error. This "lacks of verb" is not critical: it is aligned with similar FDs.

# S052

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S052 | AIML | 2 | Qunatity configuration for Event triggered Logging of CSI measurements |  | Aby K Abraham |  | v026 | ToDo |

[Description]:

Eevent triggered CSI measurement logging can be performed based on L3 events. It is agreed to use CSI framework and not measurement framework and the event is captured as below in RRC CR.

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

threshold-r19                     CHOICE {

aboveThreshold-r19               MeasTriggerQuantity,

belowThreshold-r19               MeasTriggerQuantity

},

hysteresis                        Hysteresis,

timeToTrigger                     TimeToTrigger,

...

}

Now it is not clear how does the UE get the quantity configuration for the L3 filtering for deriving L3 measurements for the event evaluation for logging CSI measurements.

[Proposed Change]:

There can be two options for the quantity configuration for deriving L3 measureemnts for event evaluation for logging CSI measurements.

- Quantity configuration is provided in CSI measurement framework.

- UE uses the Quantity configuration from the measurement configuration, for e.g. the quantity configuration associated to measurement object through ServingCellMO. This means UE should be provided with the measurement configuration when CSI-LoggedMeasurementEventTriggerConfig is received.

According to the option chosen,below changes may be considered.

Option a. require below changes.

<Change 1>

#### –	CSI-MeasConfig

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also TS 38.214 [19], clause 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEASCONFIG-START

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...,

[[

reportTriggerSizeDCI-0-2-r16        INTEGER (0..6)                                                            OPTIONAL -- Need R

]],

[[

sCellActivationRS-ConfigToAddModList-r17  SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17   OPTIONAL, -- Need N

sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL  -- Need N

]],

[[

ltm-CSI-ReportConfigToAddModList-r18   SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL  -- Need N

]],

[[

csi-LoggedMeasurementConfigToAddModList-r19   SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LoggedMeasurementConfig-r19

OPTIONAL, -- Need N

csi-LoggedMeasurementConfigToReleaseList-r19  SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LoggedMeasurementConfigId-r19                                                                                                               OPTIONAL,  -- Need N

csi-LoggedMeasurementConfig-quantityConfig    QuantityConfigRS 														  OPTIONAL,  -- Need N

]]

}

-- TAG-CSI-MEASCONFIG-STOP

-- ASN1STOP

CSI-MeasConfig field descriptions

aperiodicTriggerStateList Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement (see TS 38.214 [19], clause 5.2.1).

csi-IM-ResourceSetToAddModList Pool of CSI-IM-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

csi-IM-ResourceToAddModList Pool of CSI-IM-Resource which can be referred to from CSI-IM-ResourceSet.

csi-LoggedMeasurementConfigToAddModList Configured CSI logged measurements for network-side data collection.

csi-ReportConfigToAddModList Configured CSI report settings as specified in TS 38.214 [19] clause 5.2.1.1.

csi-ResourceConfigToAddModList Configured CSI resource settings as specified in TS 38.214 [19] clause 5.2.1.2.

csi-SSB-ResourceSetToAddModList Pool of CSI-SSB-ResourceSet which can be referred to from CSI-ResourceConfig.

ltm-CSI-ReportConfigToAddModList Configured CSI report settings for LTM as specified in TS 38.214 [19].

nzp-CSI-RS-ResourceSetToAddModList Pool of NZP-CSI-RS-ResourceSet which can be referred to from CSI-ResourceConfig or from MAC CEs.

nzp-CSI-RS-ResourceToAddModList Pool of NZP-CSI-RS-Resource which can be referred to from NZP-CSI-RS-ResourceSet.

reportTriggerSize, reportTriggerSizeDCI-0-2 Size of CSI request field in DCI (bits) (see TS 38.214 [19], clause 5.2.1.5.1). The field reportTriggerSize applies to DCI format 0_1 and the field reportTriggerSizeDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 5.2.1.5.1).

scellActivationRS-ConfigToAddModList Configured RS for fast SCell activation as specified in TS 38.214 [19] clause 5.2.1.5.3.

csi-LoggedMeasurementConfig-quantityConfig Specifies L3 filter configurations for cell measurement results for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) for theevent evaluation for network side data collection.

}

<Change 2>

#### 5.5x.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

3> reset the associated information (e..g. timeToTrigger) of the CSI logged measurement configuration;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

<Change 3>

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution or event-triggered measurement logging for network-side data collection. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the altitude measurements.

<Change 4>

#### 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity, each beam measurement quantity, each sidelink measurement quantity as needed in clause 5.8.10, for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1, for each L2 U2N Relay UE measurement quantity according to 5.5.3.4, for evaluating the selected or detected NR sidelink U2N Relay UEs according to 5.8.15.3, for evaluating the SyncRef UE according to 5.8.5 and 5.8.6, for evaluating the NR sidelink U2U Relay/Remote UE threshold conditions according to 5.8.16.2 and 5.8.17.2, for evaluating the conditions for selection and reselection of NR sidelink U2U Relay UE according to 5.8.17.3, and for evaluating the detected NR sidelink U2U Relay UEs according to 5.8.17.4:

2>	filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation, for evaluating the SyncRef UE, or for evaluation for the event triggered CSI measurement logging by the following formula:

Fn = (1 – a)*Fn-1 + a*Mn

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR or for event-triggered measurement logging,the filterCoefficient of the RS from CSI measurement configuration; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

NOTE 4:	For CLI-RSSI measurement, it is up to UE implementation whether to reset filtering upon BWP switch.

NOTE 5:	For SSB measurements when multiple altitude range-based ssb-ToMeasure are configured, it is up to UE implementation whether to reset filtering upon entering a different altitude range.

NOTE 6:	Upon satellite switch with resynchronization, it is up to UE implementation to reset filtering for the serving cell.

Option b

<Change 1>

#### 5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

- reset the associated information (e..g. timeToTrigger) of the EventTriggeredConfig for event-triggered measurement logging for network-side data collection.

<Change 2>

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution or event-triggered measurement logging for network-side data collection. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the altitude measurements.

<Change 3>

#### 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity, each beam measurement quantity, each sidelink measurement quantity as needed in clause 5.8.10, for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1, for each L2 U2N Relay UE measurement quantity according to 5.5.3.4, for evaluating the selected or detected NR sidelink U2N Relay UEs according to 5.8.15.3, for evaluating the SyncRef UE according to 5.8.5 and 5.8.6, for evaluating the NR sidelink U2U Relay/Remote UE threshold conditions according to 5.8.16.2 and 5.8.17.2, for evaluating the conditions for selection and reselection of NR sidelink U2U Relay UE according to 5.8.17.3, and for evaluating the detected NR sidelink U2U Relay UEs according to 5.8.17.4:

2>	filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation, for evaluating the SyncRef UE or for evaluation for the event triggered CSI measurement logging by the following formula:

Fn = (1 – a)*Fn-1 + a*Mn

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

NOTE 4:	For CLI-RSSI measurement, it is up to UE implementation whether to reset filtering upon BWP switch.

NOTE 5:	For SSB measurements when multiple altitude range-based ssb-ToMeasure are configured, it is up to UE implementation whether to reset filtering upon entering a different altitude range.

NOTE 6:	Upon satellite switch with resynchronization, it is up to UE implementation to reset filtering for the serving cell.

<Change 4>

#### –	CSI-LoggedMeasurementConfig

The IE CSI-LoggedMeasurementConfig is used to configure a CSI logged measurement configuration. It defines a group of one or more CSI resources for which the UE logs the associated L1 radio measurements.

CSI-LoggedMeasurementConfig information element

-- ASN1START

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-START

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

loggingPeriodicity-r19                    ENUMERATED {n2, n3, n4, n5, spare4, spare3, spare2, spare1}            OPTIONAL,  -- Need M

csi-LoggedMeasurementEventTriggerConfig-r19         CSI-LoggedMeasurementEventTriggerConfig-r19                OPTIONAL,  -- Need R

...

}

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

threshold-r19                     CHOICE {

aboveThreshold-r19               MeasTriggerQuantity,

belowThreshold-r19               MeasTriggerQuantity

},

hysteresis                        Hysteresis,

timeToTrigger                     TimeToTrigger,

...

}

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-STOP

-- ASN1STOP

CSI-LoggedMeasurementConfig field descriptions

csi-LoggedMeasurementConfigId This field indicates the instance of CSI-LoggedMeasurementConfig.

csi-LoggedResourceConfig Resources in which the UE performs channel measurement whose associated measurement results are logged by the UE. The csi-LoggedResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources.

csi-LoggedMeasurementEventTriggerConfigThis field is used to configure the UE with event-triggered measurement logging. If this field is included and threshold is set to aboveThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is included and threshold is set to belowThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. If this field is not included, the UE starts the measurement logging according to csi-LoggedResourceConfig upon reception. This field is configured if the serving cell is configured with servingCellMO and a measurement identity associated to the measurement object indicated by servingCellMO..

loggingPeriodicity The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2nd occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3rd occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.

# B203

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B203 | AIML | 2 | Need code for loggingPeriodicity |  | Congchi Zhang |  | V011 | Agreed |

[Description]:

As described in procedure text, UE may either logging according to loggingPeriodicity or according to the resource periodicity. UE does not need to maintain the loggingPeriodicity if absent. “Need R” would be more appropriate.

[Proposed Change]:

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

loggingPeriodicity-r19                    ENUMERATED {n2, n3, n4, n5, spare4, spare3, spare2, spare1}            OPTIONAL,  -- Need R

csi-LoggedMeasurementEventTriggerConfig-r19         CSI-LoggedMeasurementEventTriggerConfig-r19                OPTIONAL,  -- Need R

...

}

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# N028

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N028 | AIML | 2 | Reuse of A1/A2 events for NW-side logging | R2-25xxxx | Jerediah Fevold |  | vnnn | Agreed |

[Description]: It was agreed to reuse A1 and A2 events. The implementation does not reflect that agreement. The proposed change below could be simplified by making the entire structure a choice which can only ever contain different types of events. That is, if we do not think this IE would be extended with anything else, then the outer level can be a CHOICE and the second set of extension markers could be removed.

[Proposed Change]:

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

eventId        CHOICE {

eventA1      SEQUENCE  {

a1-Threshold    MeasTriggerQuantity,

hysteresis      Hysteresis,

timeToTrigger   TimeToTrigger

},

eventA2     SEQUENCE  {

a2-Threshold    MeasTriggerQuantity,

hysteresis      Hysteresis,

timeToTrigger   TimeToTrigger

},

...

},

...

}

[Comments]:

[WI CR rapporteur-v022]: We think both the current solution and the proposed solution work. We suggest to discuss this issue in a Tdoc, addressing also other changes that are needed for the proposed solution.

# Z010

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z010 | AIML | 1 | UE measurement behavior when LoggingPeriodicity is configured |  | Fei |  | v027 | ToDo |

[Description]:

In RAN2#131bis meeting, it has been agreed to introduce the logging periodicity for logged NW side data collection, but RAN2 never discuss what is the logging behaviour if the logging periodicity is configured.

In our understanding, the basic intention of logging periodicity is to prevent UE from the logging interruption due to collision between the measurement resources for NW side data collection and measurement resources for other purposes, measurement gap, etc. Based on such intention, NW can provide a number of measurement resources in one logging period, and UE is able to select one of them that is not collided with any other measurements for logging..

Observation 1: The basic intention of logging periodicity is to prevent UE from the logging interruption due to the collision between measurement resources for NW side data collection and other measurement resources for different purposes, measurement gap, etc.

However, in the current TS 38.331, the logging periodicity mechanism has been defined as below：

loggingPeriodicity

The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2nd occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3rd occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.

Based on above description, the UE logging behavior ,if logging periodicity is configured, is shown in below:

It can be seen that, the loggingPeriodicity is just a configurable parameter that directly extends the periodicity of the NZP CSI resources by an integer multiple, UE is still required to perform the measurement on one specific resource for logging which deviates the basic intention of introducing the loggingPeriodicty in observation 1. In other words, NW can adjust the CSI resource periodicity directly instead of introducing an additional parameter (i.e. loggingPeriodicity) to do that, we do not see any values of the current loggingPeriodicity.

Observation 2: The current loggingPeriodicity is just to extend the periodicity of NZP CSI Resource by an integer multiple, and UE still needs to perform measurement on the certain measurement occasion which seems not helpful for the measurement resource collision case.

Therefore, we propose to change the UE measurement behavior when loggingPeridicity is configured, that is, to allow UE to select any one of measurement resources for logging those are contained in one logging period.

[Proposed Change]:

Discuss the true intention of loggingPeriodicity, if the intention is to prevent UE from logging interruption due to the measurement collision, then the propose change is as below:

LoggingPeriodicity

The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every two occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every three occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.

[Comments]:

# Z009

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z009 | AIML | Class 1 | The data logging is missing in general description of CSI-MeasConfig |  | Fei |  | V017 | Agreed |

[Description]:

In the current general field description of CSI-MeasConfig, the CSI measurement for logging in missing.

[Proposed Change]:

#### –	CSI-MeasConfig

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included , channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also TS 38.214 [19], clause 5.2, and the logging of channel state information for the serving cell in which CSI-MeasConfig is included as specified in 5.5.X.3

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”. Please note that we may make some additional small editorial changes when incorporating the proposed changes in the CR.

# H151

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H151 | MOB | 3 | Per cell configuration includes per CG items | R2-25xxxxx | Huawei (David) |  | V023 | Agreed |

[Description]: In Rel-18, LTM-CSI-ReportConfig is used for reports to be sent on the serving cell on which the LTM-CSI-ReportConfig is configured, so it is clear where to include each LTM-CSI-ReportConfig.

In Rel-19, LTM-CSI-ReportConfig is also used for reports via MAC CE, that may be sent on any serving cell, and conditional events, for which no report is sent. Consequently, in which serving cell the corresponding LTM-CSI-ReportConfig has no functional role. If there are 3 serving cells, PCell, SCell 1 and SCell 2, the network could configure 3 LTM-CSI-ReportConfig, one one each serving cell, and with the same ID value. Or it could put 2 in SCell 1 and 1 in SCell 2. Or the opposite.

In general, IEs that are unrelated to a particular serving cell should not be included in ServingCellConfig, they should be at a higher level.

[Proposed Change]: Event triggered reports and executions conditions are configured in CellGroupConfig.

In CellGroupConfig, add:

ltm-EventTrigeredReportToAddModList-r19 SEQUENCE (SIZE (1..maxNrofLTM-EventTriggeredReport-r19)) OF LTM-CSI-ReportConfig-r18                                                         OPTIONAL, -- Need N

ltm-EventTrigeredReportToAddModList-r19 SEQUENCE (SIZE (1..maxNrofLTM-EventTriggeredReport-r19)) OF LTM-CSI-ReportConfigId-r18                                                       OPTIONAL, -- Need N

ltm-CondEventToAddModList-19 SEQUENCE (SIZE (1..maxNrofLTM-CondEvent-r19)) OF LTM-CSI-ReportConfig-r18                                                         OPTIONAL, -- Need N

ltm-CondEventToAddModList-19 SEQUENCE (SIZE (1..maxNrofLTM-CondEvent-r19)) OF LTM-CSI-ReportConfigId-r18

This also makes it easier to refer to "the list of event triggered reports" in TS 38.321.

Note: it might also be clearer to define a separate type for event triggered report and a separate type for L1 conditional events.

[Comments]:

[Rapporteur] This RIL can be discussed based on contribution from proponent company

# H400

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H400 | MIMO | 2 | In which serving cell to include UE-initiated CSI reports is unclear |  | Huawei (David) |  | v007 | ToDo |

[Description]: CSI-MeasConfig is per serving cell. Since Rel-15, a CSI-ReportConfig :

- is included in the CSI-MeasConfig of the serving cell on which the reports is to be sent on PUCCH or on PUSCH

- can indicate a CSI-ResourceConfig from any serving cell, using the carrier field.

For UE-initiated CSI report:

- the carrier field still indicates the location of the CSI-ResourceConfig used for channel measurements

- pucch-Cell-r19 indicates on which cell the PUCCH indication is to be sent

- for mode-B, servCellIndex-r19 indicates on which cell the CG for PUSCH transmission is defined.

Therefore, in the CSI-MeasConfig of which serving cell the UE-initiated CSI report is configured plays no role i.e., if the UE has 3 serving cells, it is possible to configure a UE-initiated CSI report in one serving cell, another in another one, or swap the two, and it makes no difference, which allows several ways to configure the same thing, for no reason.

[Proposed Change]: Remove the field pucch-Cell-r19 and update the description of CSI-ReportConfig:

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). The IE CSI-ReportConfig is also used to configure UE initiated CSI reporting with UE-initiated report indication sent on PUCCH on the cell in which the CSI-ReportConfig is included. See TS 38.214 [19], clause 5.2.1.

[Comments]:

[ZTE(Wenting)] This issue is also related to the RIL Z408

Based on the RAN1 agreements, Cell A/B/C/D as follows can be totally different cells.

- Cell A: The cell on which the indicated TCI state used to determine the current beam RS

- Cell B: The cell of the current beam and the new beams (location of the CSI-ResourceConfig used for channel measurements)

- Cell C: The cell of the PUCCH

- Cell D: The cell of the PUSCH of mode B.

Then according to the RAN1 agreement as follow, the Cell D should be the Cell in which the CSI report is configured.

RAN1: Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured.

Thus the cell D can be implicitly indicated by the corresponding ServingCellConfig of the CSI-ReportConfig (e.g. ServingCellConfig-> CSI-MeasConfig->CSI-ReportConfig).

Thus to be aligned with RAN1’s agreement, we think the servingcell of the CSI-MeasConfig can be used to indicate the above Cell D. Thus we can move the servCellIndex-r19 for the cell B instead of the pucch-Cell-r19.

[Ericsson(Lian)] Seems better to discuss this via contribution to next meeting.

# N073

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N073 | AIML | 1 | Mismatch in ASN.1 and description of nrofReportedRS-v19xy |  | Sakira Hassan |  | V015 | Agreed |

[Description]: The value {n6, n8} provided for nrofReportedRS-v19xy doesn’t match with the description.

nrofReportedRS-v19xy                ENUMERATED {n6, n8}                                                     OPTIONAL,   -- Need R

nrofReportedRS The number (N) of measured RS resources to be reported per report setting in a non-group-based report. N <= N_max, where N_max is either 2 or 4 depending on UE capability. (see TS 38.214 [19], clause 5.2.1.4) When the field is absent the UE applies the value 1. Network does not configure nrofReportedRS-v19xy at the same time as nrofReportedRS (without suffix).

[Proposed Change]:

[Comments]:

[Ericsson-v022]: We agree with the issue. A proposed solution is:

nrofReportedRS

The number (N) of measured RS resources to be reported per report setting in a non-group-based report. N <= N_max, where N_max is 2. 4, 6 or 8 depending on UE capability.

(see TS 38.214 [19], clause 5.2.1.4) When the field is absent the UE applies the value 1. Network does not configure nrofReportedRS-v19xy at the same time as nrofReportedRS (without suffix).

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”, based on the solution from Ericsson.

# N021

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N021 | AIML | 2 | Remove the CHOICE hierarchy from the configuration for CSI prediction and beam prediction | R2-25xxxx | Jerediah Fevold |  | vnnn | Rejected |

[Description]: The extra hierarchy introduced by forcing a choice between predictionConfiguration-r19 and configurationForChannelMonitoring-r19 is unnecessary and does not have the advantage of grouping many parameters together. It should be obvious to the configuring entity that one or the other should be configured.

[Proposed Change]: Eliminate one level of hierarchy by removing the choice element predictionConfiguration from CSI-ReportConfig and bring predictionConfiguration-r19 and configurationForChannelMonitoring-r19 to the main level of CSI-ReportConfig.

[[

nrofReportedRS-v19xy                ENUMERATED {n6, n8}                                                     OPTIONAL,   -- Need R

reportQuantity-r19                  ReportQuantity-r19                                                      OPTIONAL,   -- Need R

csi-InferencePrediction-r19         ENUMERATED {true},

configurationForChannelPrediction-r19   SEQUENCE {

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

nrofReportedPredicted-RS-r19                ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                                        OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL,   -- Need R

...

} OPTIONAL,    -- Need R

configurationForChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}                                OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {n1, n2, n8, spare1}                             OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet))    OPTIONAL,   -- Need R

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}                     OPTIONAL,   -- Need R

...

} OPTIONAL    -- Need R

]]

[Comments]:

[WI CR rapporteur-v022]: We suggest to discuss the restructuring of this part in a Tdoc, since there are also other solutions that other companies raised in the past and in RIL H003.

# H003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H003 | AIML | 2 | Configuration for UE data collection | R2-25xxxx | Dawid |  | v017 | Rejected |

[Description]:

This is related to the following EN captured ubnder CSI-ReportConfig information element code:

Editor's Note: FFS whether/how to group the parameters (and whether/how to update the field descriptions) for prediction, monitoring, and UE-side data collection based on the beam management and CSI prediction use cases.

Currently “configurationForChannelPrediction-r19” is used for both inference and UE data collection. However, the notation “ForChannelPrediction” and “predictionConfiguration-r19” suggests that this parameter is only for inference. We think it is better to add a new parameter for data collection (e.g. configurationForUE-DataCollection-r19) and make “configurationForChannelPrediction-r19” applicable only for inference

[Proposed Change]:

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). See TS 38.214 [19], clause 5.2.1.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

reportFreqConfiguration                 SEQUENCE {

cqi-FormatIndicator                     ENUMERATED { widebandCQI, subbandCQI }                          OPTIONAL,   -- Need R

pmi-FormatIndicator                     ENUMERATED { widebandPMI, subbandPMI }                          OPTIONAL,   -- Need R

csi-ReportingBand                       CHOICE {

subbands3                               BIT STRING(SIZE(3)),

subbands4                               BIT STRING(SIZE(4)),

subbands5                               BIT STRING(SIZE(5)),

subbands6                               BIT STRING(SIZE(6)),

subbands7                               BIT STRING(SIZE(7)),

subbands8                               BIT STRING(SIZE(8)),

subbands9                               BIT STRING(SIZE(9)),

subbands10                              BIT STRING(SIZE(10)),

subbands11                              BIT STRING(SIZE(11)),

subbands12                              BIT STRING(SIZE(12)),

subbands13                              BIT STRING(SIZE(13)),

subbands14                              BIT STRING(SIZE(14)),

subbands15                              BIT STRING(SIZE(15)),

subbands16                              BIT STRING(SIZE(16)),

subbands17                              BIT STRING(SIZE(17)),

subbands18                              BIT STRING(SIZE(18)),

...,

subbands19-v1530                        BIT STRING(SIZE(19))

}   OPTIONAL    -- Need S

}                                                                                                           OPTIONAL,   -- Need R

timeRestrictionForChannelMeasurements           ENUMERATED {configured, notConfigured},

timeRestrictionForInterferenceMeasurements      ENUMERATED {configured, notConfigured},

codebookConfig                                  CodebookConfig                                              OPTIONAL,   -- Need R

dummy                                           ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

groupBasedBeamReporting                     CHOICE {

enabled                                     NULL,

disabled                                    SEQUENCE {

nrofReportedRS                          ENUMERATED {n1, n2, n3, n4}                                 OPTIONAL    -- Need S

}

},

cqi-Table                   ENUMERATED {table1, table2, table3, table4-r17}                                     OPTIONAL,   -- Need R

subbandSize                 ENUMERATED {value1, value2},

non-PMI-PortIndication      SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks OPTIONAL,   -- Need R

...,

[[

semiPersistentOnPUSCH-v1530         SEQUENCE {

reportSlotConfig-v1530              ENUMERATED {sl4, sl8, sl16}

}                                                                                                           OPTIONAL    -- Need R

]],

[[

semiPersistentOnPUSCH-v1610         SEQUENCE {

reportSlotOffsetListDCI-0-2-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL,    -- Need R

reportSlotOffsetListDCI-0-1-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL     -- Need R

}                                                                                                           OPTIONAL,    -- Need R

aperiodic-v1610                     SEQUENCE {

reportSlotOffsetListDCI-0-2-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL,    -- Need R

reportSlotOffsetListDCI-0-1-r16     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..32)   OPTIONAL     -- Need R

}                                                                                                           OPTIONAL,    -- Need R

reportQuantity-r16                  CHOICE {

cri-SINR-r16                         NULL,

ssb-Index-SINR-r16                   NULL

}                                                                                                           OPTIONAL,   -- Need R

codebookConfig-r16                          CodebookConfig-r16                                              OPTIONAL    -- Need R

]],

[[

cqi-BitsPerSubband-r17              ENUMERATED {bits4}                                                      OPTIONAL,   -- Need R

groupBasedBeamReporting-v1710       SEQUENCE {

nrofReportedGroups-r17              ENUMERATED {n1, n2, n3, n4}

}                                                                                                           OPTIONAL,   -- Need R

codebookConfig-r17                  CodebookConfig-r17                                                      OPTIONAL,   -- Need R

sharedCMR-r17                       ENUMERATED {enable}                                                     OPTIONAL,   -- Need R

csi-ReportMode-r17                  ENUMERATED {mode1, mode2}                                               OPTIONAL,   -- Need R

numberOfSingleTRP-CSI-Mode1-r17     ENUMERATED {n0, n1, n2}                                                 OPTIONAL,   -- Need R

reportQuantity-r17                  CHOICE {

cri-RSRP-Index-r17                  NULL,

ssb-Index-RSRP-Index-r17            NULL,

cri-SINR-Index-r17                  NULL,

ssb-Index-SINR-Index-r17            NULL

}                                                                                                           OPTIONAL    -- Need R

]],

[[

semiPersistentOnPUSCH-v1720         SEQUENCE {

reportSlotOffsetList-r17            SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-2-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-1-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL    -- Need R

}                                                                                                           OPTIONAL,   -- Need R

aperiodic-v1720                     SEQUENCE {

reportSlotOffsetList-r17            SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-2-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL,   -- Need R

reportSlotOffsetListDCI-0-1-r17     SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER(0..128)  OPTIONAL    -- Need R

}                                                                                                           OPTIONAL    -- Need R

]],

[[

codebookConfig-v1730                CodebookConfig-v1730                                                    OPTIONAL    -- Need R

]],

[[

groupBasedBeamReporting-v1800       SEQUENCE {

reportingMode-r18                   ENUMERATED {jointULDL, onlyUL}

}                                                                                                           OPTIONAL,   -- Need R

reportQuantity-r18                  TDCP-r18                                                                OPTIONAL,   -- Need R

codebookConfig-r18                  CodebookConfig-r18                                                      OPTIONAL,   -- Need R

csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18

OPTIONAL,   -- Need N

csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18

OPTIONAL    -- Need N

]],

[[

nrofReportedRS-v19xy                ENUMERATED {n6, n8}                                                     OPTIONAL,   -- Need R

reportQuantity-r19                  ReportQuantity-r19                                                      OPTIONAL,   -- Need R

predictionConfiguration-r19         CHOICE {

csi-InferencePrediction-r19         ENUMERATED {true},

configurationForChannelPrediction-r19   SEQUENCE {

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

nrofReportedPredicted-RS-r19                ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                                        OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL,   -- Need R

...

},

configurationForChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}                                OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {n1, n2, n8, spare1}                             OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet))    OPTIONAL,   -- Need R

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}                     OPTIONAL,   -- Need R

...

},

configurationForUE-DataCollection-r19   SEQUENCE {

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    		OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        		OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                       		OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                                OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1}   OPTIONAL,   -- Need R

...

}

}                                                                                                           OPTIONAL    -- Need R

]]

}

PortIndexFor8Ranks ::=              CHOICE {

portIndex8                          SEQUENCE{

rank1-8                             PortIndex8                                                      OPTIONAL,   -- Need R

rank2-8                             SEQUENCE(SIZE(2)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank3-8                             SEQUENCE(SIZE(3)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank4-8                             SEQUENCE(SIZE(4)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank5-8                             SEQUENCE(SIZE(5)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank6-8                             SEQUENCE(SIZE(6)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank7-8                             SEQUENCE(SIZE(7)) OF PortIndex8                                 OPTIONAL,   -- Need R

rank8-8                             SEQUENCE(SIZE(8)) OF PortIndex8                                 OPTIONAL    -- Need R

},

portIndex4                          SEQUENCE{

rank1-4                             PortIndex4                                                      OPTIONAL,   -- Need R

rank2-4                             SEQUENCE(SIZE(2)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank3-4                             SEQUENCE(SIZE(3)) OF PortIndex4                                 OPTIONAL,   -- Need R

rank4-4                             SEQUENCE(SIZE(4)) OF PortIndex4                                 OPTIONAL    -- Need R

},

portIndex2                          SEQUENCE{

rank1-2                             PortIndex2                                                      OPTIONAL,   -- Need R

rank2-2                             SEQUENCE(SIZE(2)) OF PortIndex2                                 OPTIONAL    -- Need R

},

portIndex1                          NULL

}

PortIndex8::=                       INTEGER (0..7)

PortIndex4::=                       INTEGER (0..3)

PortIndex2::=                       INTEGER (0..1)

TDCP-r18 ::=                        SEQUENCE {

delayDSetofLengthY-r18              SEQUENCE (SIZE (1.. maxNrofdelayD-r18)) OF DelayD,

phaseReporting-r18                  ENUMERATED {enable}                                                 OPTIONAL    -- Need R

}

DelayD ::=                          ENUMERATED { symb4, slot1, slot2, slot3, slot4, slot5, slot6, slot10 }

CSI-ReportSubConfig-r18 ::=         SEQUENCE {

reportSubConfigId-r18               CSI-ReportSubConfigId-r18,

reportSubConfigParams-r18           CHOICE {

a1-parameters                       SEQUENCE {

codebookSubConfig-r18               CodebookConfig                                              OPTIONAL,   -- Need R

portSubsetIndicator-r18             CHOICE {

p2                                  BIT STRING (SIZE (2)),

p4                                  BIT STRING (SIZE (4)),

p8                                  BIT STRING (SIZE (8)),

p12                                 BIT STRING (SIZE (12)),

p16                                 BIT STRING (SIZE (16)),

p24                                 BIT STRING (SIZE (24)),

p32                                 BIT STRING (SIZE (32))

}                                                                                               OPTIONAL,   -- Need R

non-PMI-PortIndication-r18          SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks

OPTIONAL   --  Need R

},

a2-parameters                       SEQUENCE {

nzp-CSI-RS-ResourceList-r18         SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18

}

}                                                                                                       OPTIONAL,   -- Need R

powerOffset-r18                     INTEGER(0..23)                                                      OPTIONAL    -- Need R

}

NZP-CSI-RS-ResourceIndex-r18 ::=    INTEGER (0..maxNrofNZP-CSI-RS-ResourcesPerSet-1-r18)

ReportQuantity-r19 ::=   CHOICE {

none-BM-r19                 NULL,

none-CSI-r19                NULL,

p-CRI-r19                   NULL,

p-SSB-Index-r19             NULL,

p-CRI-RSRP-r19              NULL,

p-SSB-Index-RSRP-r19        NULL,

rs-PAI-r19                  NULL,

sgcs-r19                    NULL

}

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

Editor's Note: FFS the value range of the fields nrofTimeInstance-r19, timeGap-r19, timeInstanceFor-RS-PAI-r19, and timeInstanceFor-SGCS-r19, based on RAN1 progress.

Editor's Note: FFS whether/how to group the parameters (and whether/how to update the field descriptions) for prediction, monitoring, and UE-side data collection based on the beam management and CSI prediction use cases.

[Comments]:

[WI CR rapporteur-v022]: We agree that the suggested change is a possibility to group the parameters and thus make the type of configuration clear. In the past we received input from some companies that we should avoid redundancy (the parameters for inference and those for data collection are the same), so we removed the separate CHOICE branch for UE-side data collection. Thus, we propose to discuss this issue in Tdocs, as for RIL N021, which is related.

# H008

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H008 | AIML | 2 | Configuration restrictions in predictionConfiguration-r19 |  | Dawid |  | V017 | Agreed |

[Description]:

Since the parameters under predictionConfiguration-r19 are for different purposes, they can only be configured for certain report quantities (i.e. when reportQuantity-r19 is present and set to specific values)

[Proposed Change]:

Add a field description for predictionConfiguration-r19 and capture the following configuration restrictions:

- configurationForChannelPrediction-r19 can only be configured when reportQuantity-r19 is present and indicates inference for BM (i.e., reportQuantity-r19 is set to 'p-CRI-r19', 'p-SSB-Index’-r19, 'p-CRI-RSRP-r19' or 'p-SSB-Index-RSRP-r19').

- configurationForChannelMonitoring-r19 can only be configured when reportQuantity-r19 is present and indicates monitoring for BM or monitoring for CSI prediction (i.e., reportQuantity-r19 is set to 'rs-PAI-r19' or 'sgcs-r19').

- configurationForUE-DataCollection-r19 can only be configured when reportQuantity-r19 is present and indicates UE-side data collection for BM (i.e., reportQuantity-r19 is set to 'none-BM-r19').

[Comments]:

[WR CI rapporteur-v022]: We agree with the intention of the proposal, but we can decide the exact wording once it is settled how to change the ASN.1 structure for inference/monitoring/UE-side data collection (in RILs H003, N021).

# S047

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S047 | AIML | 2 | Inference configuration | R2-25xxxx | Youn Heo |  | v026 | Rejected |

[Description]: configurations for prediction accuracy indicator (PAI) are quite different for BM and CSI.

In addition, BM and CSI PAI is distinguished and also inference configuration for BM is distinguished via reportQuantity-r19 (rs-PAI-r19 and csi-PAI-r19/sgcs-r19). Therefore, there is no reason to have a CHOICE structure under predictionConfiguration-r19.

It is noted that this issue is the similar issue as in N021 / H003 but add one more point about separating performance montoring for BM use case and CSI use case.

So, we propose to discuss together with N021 / H003.

[Proposed Change]:

csi-InferencePrediction-r19 ENUMERATED {true} OPTIONAL, -- Need R configurationForChannelPrediction-r19 SEQUENCE { resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofReportedPredicted-RS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {n1, n2, n4, n8} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL, -- Need R ... }, configurationForBM-PAI-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, nrofBestBeamForMonitoring-r19 ENUMERATED {n1, n2} OPTIONAL, -- Need R nrofTransmissionOccasion-r19 ENUMERATED {n1, n3, n7, n15} OPTIONAL, -- Need R timeInstanceFor-RS-PAI-r19 ENUMERATED {n1, n2, n8, spare1} OPTIONAL, -- Need R mappingToResourcesForChannelPrediction-r19 BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OPTIONAL, -- Need R timeInstanceFor-SGCS-r19 ENUMERATED {n1, spare3, spare2, spare1} OPTIONAL, -- Need R ... } configurationForCS-PAI-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, timeInstanceFor-SGCS-r19 ENUMERATED {n1, spare3, spare2, spare1} OPTIONAL, -- Need R ... }

[Comments]:

[WI CR rapporteur-v031]: We suggest to discuss this in a Tdoc, together with RILs N021/H003/A105.

# N022

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N022 | AIML | 2 | Move csi-InferencePrediction into configurationForChannelPrediction-r19 | N/A | Jerediah Fevold |  | vnnn | Rejected |

[Description]: The parameter csi-InferencePrediction-r19 is used to indicate if a CSI-ReportConfig is one configured for CSI prediction. Because the configuration for channel monitoring can only be linked to one AI/ML-enabled CSI-ReportConfig, and the referenced configuration will contain this parameter, it would be redundant to also configure it when configurationForChannelMonitoring-r19 is selected.

[Proposed Change]: Move csi-InferencePrediction-r19 inside of predictionConfiguration-r19.

configurationForChannelPrediction-r19   SEQUENCE {

csi-InferencePrediction-r19                 ENUMERATED {true},

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

nrofReportedPredicted-RS-r19                ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                                        OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL,   -- Need R

...

} OPTIONAL,    -- Need R

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropReject”. We agree that csi-InferencePrediction-r19 is configured only in an inference configuration and not in a performance monitoring configuration. However, we got input from several companies in past email discussions that they prefer to keep csi-InferencePrediction-r19 outside of configurationForChannelPrediction, since none of the other fields within configurationForChannelPrediction refers to the CSI prediction use case (they all refer to the BM case).

# N023

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N023 | AIML | 2 | Make csi-InferencePrediction-r19 an OPTIONAL parameter with Need R | N/A | Jerediah Fevold |  | vnnn | Rejected |

[Description]: csi-InferencePrediction-r19 should be OPTIONAL in CSI-ReportConfig. Otherwise, it is impossible to configure beam prediction.

[Proposed Change]: Add the OPTIONAL flag and Need R to csi-InferencePrediction-r19.

[Comments]:

[WI CR rapporteur-v020]: We changed the status from “ToDo” to “PropAgree”.

# C078

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C078 | AIML | 2 | Conditionally mandatory parameter |  | Tangxun |  | V003 | Rejected |

[Description]: in current spec, “refToPredictionConfig-r19” is a mandatory parameter for monitoring configuration. But according to RAN1 parameter list, i.e., “This field is mandatory present if the reportQuantity-r19 is set to ‘rspai-r19’”, it should be conditionally mandatory. In other words, we should add optional indication for this parameter.

[Proposed Change]: update the ASN.1 as below:

configurationForChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId                                         OPTIONAL,   -- Cond Rspai

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}                                OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {n1, n2, n8, spare1}                             OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet))    OPTIONAL,   -- Need R

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}                     OPTIONAL,   -- Need R

...

}

| Conditional Presence | Explanation |
| --- | --- |
| Rspai | It is mandatory present if the IE reportQuantity-r19 is set to ‘rs-PAI-r19’ |

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropReject”. We think the proposed solution is not fully correct, since refToPredictionConfig-r19 should be configured also if reportQuantity-r19 is set to ‘csi-PAI-r19’ (performance monitoring for CSI prediction). For simplicitly, we prefer to keep the original version, where refToPredictionConfig-r19 is mandatory within configurationForChannelMonitoring and, in turn, it is assumed that configurationForChannelMonitoring is configured only for performance monitoring configurations.

# Z411

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z411 | MIMO | 2 | Wrong structure for the reportQuantity-r19 |  | ZTE (Wenting Li) |  | v008 | PropReject |

[Description]: The sturcute is wrong

| reportQuantity-r19 | new |  | cri-RI-PMI-CQI for Type-I and Type-II cri-RI-LI-PMI-CQI for only Type-I |
| --- | --- | --- | --- |

reportQuantity-r19              CHOICE {

cjtc-Dd-r19                              NULL,

cjtc-F-r19                               NULL,

cjtc-P-r19                               NULL,

cjtc-Dd-F-r19                            NULL

}                                                                                                           OPTIONAL,   -- Need R

[Proposed Change]:

reportQuantity-r19              CHOICE {

cri-RI-PMI-CQI NULL,

cri-RI-LI-PMI-CQI NULL

}                                                                                                           OPTIONAL,   -- Need R

[Comments]: We can also discuss whether the legacy element can be reused.

reportQuantity CHOICE {

none             NULL,

cri-RI-PMI-CQI   NULL,

cri-RI-i1        NULL,

cri-RI-i1-CQI SEQUENCE {

pdsch-BundleSizeForCSI ENUMERATED {n2, n4} OPTIONAL -- Need S

},

cri-RI-CQI        NULL,

cri-RSRP          NULL,

ssb-Index-RSRP    NULL,

cri-RI-LI-PMI-CQI NULL

},

[Ericsson(Lian)] For the L1 parameter mentioned above, it was agreed indeed before to reuse the legacy field. The current reportQuantity is for CJTC.

# S013

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S013 | MIMO | 2 | linkedCJTC-Report-r19 can be merged to csi-ReportCJTC-r19 |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: linkedCJTC-Report-r19 can be merged to csi-ReportCJTC-r19 since they are all for CJTC.

[Proposed Change]:

Move linkedCJTC-Report-r19 into csi-ReportCJTC-r19, and move the FD into the table for CSI-ReportCJTC field descriptions

[Comments]:

# Z412

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z412 | MIMO | 2 | Remove ED for the CSI-ReportCJTC |  | ZTE (Wenting Li) |  | v008 | PropAgree |

[Description]:

Ran1 has updated the associatedSRS-ResourceSet in the R1-2506622

CSI-ReportCJTC-r19 ::=              SEQUENCE {

--Editor’s note: associatedSRS-ResourceSet can be updated based on further RAN1 discussion.

[Proposed Change]: Remove the Editor’s note

[Comments]:

# N059

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N059 | MIMO | 2 | Optionality of a1-Parameters-v19xy |  | Nokia (Andrew) |  | v010 |  |

[Description]: The field description for csi-ReportSubConfigToAddModList, csi-ReportSubConfigToAddModListExt states the following:

CSI-ReportConfig field descriptions

csi-ReportSubConfigToAddModList, csi-ReportSubConfigToAddModListExt List of CSI-ReportSubConfiguration(s) in a CSI report configuration to add or modify. No simultaneous configuration of portSubsetIndicator and a list of nzp-CSI-RS-resources in a same CSI report sub-configuration. The number of elements in a list is at least 2. If the network includes csi-ReportSubConfigToAddModListExt, it includes the same number of entries, and listed in the same order, as in csi-ReportSubConfigToAddModList.

For Rel-18, this part is redundant since it is impossible to configure portSubsetIndicator-r18 and nzp-CSI-RS-ResourceList-r18 simultaneously due to reportSubConfigParams being defined as a CHOICE between the a1-parameters and a2-parameters. Now that we have added CSI-ReportSubConfig-v19xy in Rel-19, this part is relevant so that the NW does not configure portSubsetIndicator-v19xy (in CSI-ReportSubConfig-v19xy) simultaneously with nzp-CSI-RS-ResourceList-r18 in (CSI-ReportSubConfig-r18).

On the other hand, a1-Parameters-v19xy is mandatory in CSI-ReportSubConfig-v19xy even though portSubsetIndicator-v19xy is optional. Assuming csi-ReportSubConfigToAddModListExt-r19 gets configured, this part and this part together imply that for CSI report subconfigurations where a2-Parameters are configured in the R18 subconfig, the NW must still configure the R19 subconfig with a1-Parameters-v19xy (but leaving the configuration empty).

This seems a bit counterintuitive even if it can work. Probably it is better to just make a1-Parameters-v19xy OPTIONAL and then update the field description to say that there cannot be simultaneous configuration of a1-Parameters and a2-Parameters.

[Proposed Change]:

csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18

OPTIONAL,   -- Need N

csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18

OPTIONAL    -- Need N

csi-ReportSubConfigToAddModListExt-r19 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-v19xy

OPTIONAL,   -- Need N

CSI-ReportSubConfig-r18 ::=         SEQUENCE {

reportSubConfigId-r18               CSI-ReportSubConfigId-r18,

reportSubConfigParams-r18           CHOICE {

a1-parameters                       SEQUENCE {

codebookSubConfig-r18               CodebookConfig                                              OPTIONAL,   -- Need R

portSubsetIndicator-r18             CHOICE {

p2                                  BIT STRING (SIZE (2)),

p4                                  BIT STRING (SIZE (4)),

p8                                  BIT STRING (SIZE (8)),

p12                                 BIT STRING (SIZE (12)),

p16                                 BIT STRING (SIZE (16)),

p24                                 BIT STRING (SIZE (24)),

p32                                 BIT STRING (SIZE (32))

}                                                                                               OPTIONAL,   -- Need R

non-PMI-PortIndication-r18          SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks

OPTIONAL   --  Need R

},

a2-parameters                       SEQUENCE {

nzp-CSI-RS-ResourceList-r18         SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18

}

}                                                                                                       OPTIONAL,   -- Need R

powerOffset-r18                     INTEGER(0..23)                                                      OPTIONAL    -- Need R

}

CSI-ReportSubConfig-v19xy ::=         SEQUENCE {

reportSubConfigParams-v19xy           SEQUENCE {

a1-Parameters-v19xy                     SEQUENCE {

portSubsetIndicator-v19xy             CHOICE {

p48                                 BIT STRING (SIZE (48)),

p64                                 BIT STRING (SIZE (64)),

p128                                BIT STRING (SIZE (128))

}                                                                                               OPTIONAL   -- Need R

}                                                                                                   OPTIONAL   -- Need R

},

...

}

CSI-ReportConfig field descriptions

csi-ReportSubConfigToAddModList, csi-ReportSubConfigToAddModListExt List of CSI-ReportSubConfiguration(s) in a CSI report configuration to add or modify. No simultaneous configuration of a1-parameters (including a1-Parameters-v19xy) and a2-parameters in a same CSI report sub-configuration. The number of elements in a list is at least 2. If the network includes csi-ReportSubConfigToAddModListExt, it includes the same number of entries, and listed in the same order, as in csi-ReportSubConfigToAddModList.

[Comments]:

# C250

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C250 | MIMO | 1 | Wrong place of field description of “When this field is set to event1, eventThreshold can only be configured with values from 14 to 113” |  | CATT (LeiWang) |  | V003 | PropAgree |

[Description]:

Besides eventTypeUE-IBR-r19, there are many other fields in IE CSI-ReportUE-IBR-r19. So the term “this field” in the “When this field is set to event1, eventThreshold can only be configured with values from 14 to 113” is a bit confusing and the corresponding description on the threshold limitation for event1 should be moved from the field description of csi-ReportUE-IBR to that of eventTypeUE-IBR-r19. And the start value should be 16 instead of 14 (mentioned in S014) according to the updated RAN1 parameter list.

[Proposed Change]:

csi-ReportUE-IBR

Configures parameters used for the UE initiated CSI reporting. When this field is configured, the UE ignores reportConfigType.

eventTypeUE-IBR

Indicates the event type for UE initiated CSI reporting and associated fields as specified in clause 5.2.1.5.4 of TS 38.214 [19]. When this field is set to event1, eventThreshold can only be configured with values from 16 to 113.

[Comments]:

# E032

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E032 | MIMO | 1 | Aligment of CSI-ReportUE-IBR-r19 |  | Ericsson (Lian) |  | V006 | PropAgree |

[Description]: RAN1 specifications refer to UE IBR as UE initiated CSI report. While this was already aligend in other parts of 38.331 it is missing for the ASN1 name of this IE and fields therein.

[Proposed Change]:

Change the IE name from csi-ReportUE-IBR-r19 to csi-ReportUE-Initiated-r19 and the following fields therein as well:

eventTypeUE-IBR to eventTypeUE-Initiated

nrofReportedRS-UE-IBR to nrofReportedRS-UE-Initated

[Comments]:

# S014

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S014 | MIMO | 2 | Rename eventThreshold-r19 for UEIR event1 |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: eventThreshold-r19 for UEIR event1 should be renamed to distinguish from eventThreshold-r19 for event2 and event7. Update the FD that only value 16..131 can be configured according to the latest RRC parameter list.

[Proposed Change]:

eventTypeUE-IBR-r19                      CHOICE {

event1-r19                                 SEQUENCE {

event1Threshold-r19 						   RSRP-Range

},

event2-r19                                 SEQUENCE {

eventThreshold-r19 						   INTEGER (0..31)

},

event7-r19                                 SEQUENCE {

eventThreshold-r19 						INTEGER (0..31),

valueOfQ-r19                            INTEGER (1..8)

}

},

csi-ReportUE-IBR

Configures parameters used for the UE initiated CSI reporting. When this field is configured, the UE ignores reportConfigType. When this field is set to event1, event1Threshold-r19 can only be configured with values from 16 to 113.

[Comments]:

[Ericsson(Lian)] Note that the field description update will be moved to eventTypeUE-IBR as suggested in C250.

# K102

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K102 | MIMO | 1 | Wrong parameter name pusch-ResourceOfModeB |  | ASUSTeK (Xinra) |  | V005 | PropReject |

[Description]: Parameter name pusch-ResourceOfModeB is not aligned with PHY layer parameter name and should be changed.

38.214:

-	on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

[Proposed Change]:

CSI-ReportUE-IBR-r19             ::=     SEQUENCE {

eventTypeUE-IBR-r19                      CHOICE {

event1-r19                                 SEQUENCE {

eventThreshold-r19[RIL]: S014, MIMO 						   RSRP-Range

},

event2-r19                                 SEQUENCE {

eventThreshold-r19 						   INTEGER (0..31)

},

event7-r19                                 SEQUENCE {

eventThreshold-r19 						INTEGER (0..31),

valueOfQ-r19                            INTEGER (1..8)

}

},

reportTransmissionMode-r19               CHOICE {

modeA-r19                                 NULL,

modeB-r19                                 SEQUENCE {

configuredPUSCHResourceOfModeB           SEQUENCE {

configuredGrantConfigIndex-r19              ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                                BWP-Id,

servCellIndex-r19							   ServCellIndex

},

minimumPucch-PuschOffset-r19[RIL]: N058, MIMO ENUMERATED { symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256, symb512}

--Editor’s note: minimumPucch-PuschOffset can be updated based on further RAN1 discussion.

}

},

configuredPUSCHResourceOfModeB Indicates Type-1 CG PUSCH resource for the UE initiated CSI reporting in mode-B.

[Comments]:

[Ericsson(Lian)] We should send an updated list to RAN1 with parameter names used in RAN2 so that RAN1 can update their specifications.

# K103

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K103 | MIMO | 2 | Redundant information in mode B CG configuration |  | ASUSTeK (Xinra) |  | V005 | ToDo |

[Description]: Since type Type 1 CG-PUSCH for UEI reporting mode-B is has been restricted to be on the same Serving Cell where the corresponding CSI-ReportConfig is configured by R1:

(38.214)

After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet […]. The UE sends the CSI report

-	on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or

-	on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

(end of 38.214)

Introducing servCellIndex as a RRC parameter for the Type 1 CG resource is redundant and may cause unnecessary flexibility conflict from network’s point of view. Besides, ul-BWP-id is also redundant. From RAN2 perspecitve, instead of using three parameters, a single parameter ConfiguredGrantConfigIndexMAC is sufficient to indicate Type-1 CG PUSCH resource for the PUSCH transmission in mode-B UE-initiated beam reporting.

configuredGrantConfigIndexMAC Indicates the index of the Configured Grant configurations within the MAC entity.

[Proposed Change]:

CSI-ReportUE-IBR-r19             ::=     SEQUENCE {

eventTypeUE-IBR-r19                      CHOICE {

event1-r19                                 SEQUENCE {

eventThreshold-r19[RIL]: S014, MIMO 						   RSRP-Range

},

event2-r19                                 SEQUENCE {

eventThreshold-r19 						   INTEGER (0..31)

},

event7-r19                                 SEQUENCE {

eventThreshold-r19 						INTEGER (0..31),

valueOfQ-r19                            INTEGER (1..8)

}

},

reportTransmissionMode-r19               CHOICE {

modeA-r19                                 NULL,

modeB-r19                                 SEQUENCE {

pusch-ResourceOfModeB-r19           SEQUENCE {

configuredGrantConfigIndex-r19              ConfiguredGrantConfigIndexMAC-r16

},

minimumPucch-PuschOffset-r19[RIL]: N058, MIMO ENUMERATED { symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256, symb512}

--Editor’s note: minimumPucch-PuschOffset can be updated based on further RAN1 discussion.

}

},

[Comments]: [Huawei] We don't agree that BWP is redundant. Moreover, we think it should be possible to configure CG in multiple BWP, otherwise, the network will have to configure multiple instance of the same even for reporting in different BWP, as only the active BWP can be used.

For mode-A, it is unclear on which serving cell the CSI-ReportConfig is to be included, so we suggest instead to remove the pucch-Cell parameter (see H400). Of course, we should inform RAN1 about this.

[ZTE(Wenting)]

It seems that there are 2 options:

Option 1: Use MAC level Index (ConfiguredGrantConfigIndexMAC)

Option 2: Use BWP level Index (ConfiguredGrantConfigIndex) together with BWP ID

For the option 1, at least we need to add some restriction on the serving cell index (type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig), thus we prefer the option 2 as in RIL Z408.

[Ericsson(Lian)] Seems better to discuss this via contribution to next meeting.

# H402

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H402 | MIMO | 2 | PUCCH/PUSCH resources can only be configured in one BWP |  | Huawei (David) |  | v007 | PropReject |

[Description]: For legacy CSI reports (periodic and semi-persistent) on PUCCH, it is possible to configure a PUCCH resource for each BWP, but for UE-initiated CSI report, only a single BWP can be used. It is unclear why it is so. For UE-initiated CSI reports on PUSCH, the CG can only be on a single BWP. It is also unclear why there is such a restriction.

[Proposed Change]:

CSI-ReportUE-IBR-r19 ::=               SEQUENCE {

eventTypeUE-IBR-r19                    CHOICE {

event1-r19                             SEQUENCE {

eventThreshold-r19                     RSRP-Range

},

event2-r19                             SEQUENCE {

eventThreshold-r19                     INTEGER (0..31)

},

event7-r19                             SEQUENCE {

eventThreshold-r19                     INTEGER (0..31),

valueOfQ-r19                           INTEGER (1..8)

}

},

reportTransmissionMode-r19               CHOICE {

modeA-r19                                NULL,

modeB-r19                                SEQUENCE {

pusch-ResourceOfModeB-r19                SEQUENCE {

configuredGrantPerBWP-r19           SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUSCH-CSI-CG-Resource,

ul-BWP-Id-r19                            BWP-Id,

servCellIndex-r19                        ServCellIndex

},

minimumPucch-PuschOffset-r19 ENUMERATED {symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256,

symb512}

--Editor’s note: minimumPucch-PuschOffset can be updated based on further RAN1 discussion.

},

},

nrofReportedRS-UE-IBR-r19                ENUMERATED {n1, n2, n3, n4},

tci-ServCellIndex-r19                    ServCellIndex                                                        OPTIONAL,    -- Need R

currentBeamReport-r19                    ENUMERATED {enabled}                                                 OPTIONAL,    -- Need R

conditionFulfillmentIndicator-r19        ENUMERATED {enabled}                                                 OPTIONAL,    -- Need R

eventCountWindow-r19                     SEQUENCE {

eventInstanceCount-r19                   INTEGER (2..16),

eventDetectionTimeWindow-r19             ENUMERATED {ms4, ms5, ms8, ms10, ms16, ms20, ms40, ms80, ms160, ms320, ms640, ms1280}

}                                                                                                             OPTIONAL,    -- Need R

pucch-Resource-r19                       SEQUENCE {

periodicityAndOffset                     CHOICE {

sym2                                     NULL,

sym6or7                                  NULL,

sl1                                      NULL,                       -- Recurs in every slot

sl2                                      INTEGER (0..1),

sl4                                      INTEGER (0..3),

sl5                                      INTEGER (0..4),

sl8                                      INTEGER (0..7),

sl10                                     INTEGER (0..9),

sl16                                     INTEGER (0..15),

sl20                                     INTEGER (0..19),

sl40                                     INTEGER (0..39),

sl80                                     INTEGER (0..79),

sl160                                    INTEGER (0..159),

sl320                                    INTEGER (0..319),

sl640                                    INTEGER (0..639)

},

resource                                 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource,

pucch-Cell-r19                           ENUMERATED {spCell, pucch-Scell}

}

}

PUSCH-CSI-CG-Resource ::=                  SEQUENCE {

uplinkBandwidthPartId                   BWP-Id,

configuredGrantConfigIndex-r19          ConfiguredGrantConfigIndex-r16

}

[Comments]:

[Ericsson(Lian)] RAN1 decided on the BWP limitation so it seems better to trigger this directly in RAN1 if needed.

# Z408

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z408 | MIMO | 2 | Redudant servCellIndex-r19 for the UEIBR mode B configuration |  | ZTE (Wenting Li) |  | v008 | ToDo |

[Description]:

Remove the “servCellIndex-r19” from the CSI-ReportConfig->reportTransmissionMode-r19->modeB-r19 in the current draft running CR.

[Proposed Change]: Remove the “servCellIndex-r19” from the CSI-ReportConfig->reportTransmissionMode-r19->modeB-r19 in the current draft running CR.

reportTransmissionMode-r19               CHOICE {

modeA-r19                                 NULL,

modeB-r19                                 SEQUENCE {

pusch-ResourceOfModeB-r19           SEQUENCE {

configuredGrantConfigIndex-r19              ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                                BWP-Id,

servCellIndex-r19							   ServCellIndex

},

[Comments]: We will provide a paper on this

# N058

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N058 | MIMO | 2 | minimumPucch-PuschOffset-r19 value definition inconsistency |  | Nokia (Andrew) |  | v004 | PropAgree |

[Description]: minimumPucch-PuschOffset-r19 is defined in symbols and parameterized as ENUMERATED { symb0, …, symb512 }, but typically RRC configurations referring to ‘symbols’ use the notation “n0, n1, …” or “symb0, sym1, …”. minimumPucch-PuschOffset-r19 should use the same for consistency.

[Proposed Change]: Change definition of minimumPucch-PuschOffset-r19 to one of the following:

minimumPucch-PuschOffset-r19 ENUMERATED { sym0, sym1, sym2, sym4, sym8, sym16, sym32, sym64, sym128, sym256, sym512}

minimumPucch-PuschOffset-r19 ENUMERATED { n0, n1, n2, n4, n8, n16, n32, n64, n128, n256, n512}

[Comments]: Note that FD for minimumPucch-PuschOffset-r19 will need to be updated accordingly.

# Z403

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z403 | MIMO | 2 | Change the reference chapter in the field description of the codebookMode in include R19 feature |  | ZTE (Wenting Li) |  | v008 | PropAgree |

[Description]: The codebookmode has been extended for the R19 new types, and the corresponding clauses have been added, e.g.5.2.2.2.1a, and there is no 5.2.2.9 in the current 38.214.

[Proposed Change]: Change the reference clause as below

codebookMode

CodebookMode as specified in TS 38.214 [19], clause 5.2.2.2 8 and 5.2.2.9.

[Comments]:

]:

# Z416

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z416 | MIMO | 2 | Delete the ED for the minimumPucch-PuschOffset-r19 |  | ZTE (Wenting Li) |  | v008 | PropAgree |

[Description]: Delete the ED for the minimumPucch-PuschOffset-r19 based on the R1-2506622

[Proposed Change]: Delete ED

reportTransmissionMode-r19               CHOICE {

modeA-r19                                 NULL,

modeB-r19                                 SEQUENCE {

pusch-ResourceOfModeB-r19           SEQUENCE {

configuredGrantConfigIndex-r19              ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                                BWP-Id,

servCellIndex-r19							   ServCellIndex

},

minimumPucch-PuschOffset-r19 ENUMERATED { symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256, symb512}

--Editor’s note: minimumPucch-PuschOffset can be updated based on further RAN1 discussion

[Comments]:

# C254

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C254 | MIMO | 2 | Re-structure of the conditionFulfillmentIndicator-r19 indication |  | CATT (LeiWang) |  | V003 | PropAgree |

[Description]:

According to the UE Initiated reporting in 5.2.1.5.4 of RAN1 spec 38.214, the conditionFulfillmentIndicator-r19 is only applied for the event 2 and event 7. That means this parameter is an event associated parameter, which is similar as the threshold associated the each event or the valueOfQ-r19 associated with event 7. So it is clearer to put it under the event configuration.

[Proposed Change]:

CSI-ReportUE-IBR-r19           ::=     SEQUENCE {

eventTypeUE-IBR-r19                      CHOICE {

event1-r19                                 SEQUENCE {

eventThreshold-r19 						   RSRP-Range

},

event2-r19                                 SEQUENCE {

eventThreshold-r19 						   INTEGER (0..31)

conditionFulfillmentIndicator-r19         ENUMERATED {enabled} 										    			  OPTIONAL,    -- Need R

},

event7-r19                                 SEQUENCE {

eventThreshold-r19 						INTEGER (0..31),

valueOfQ-r19                            INTEGER (1..8)

conditionFulfillmentIndicator-r19         ENUMERATED {enabled} 										    			  OPTIONAL,    -- Need R

}

},

reportTransmissionMode-r19               CHOICE {

modeA-r19                                 NULL,

modeB-r19                                 SEQUENCE {

pusch-ResourceOfModeB-r19           SEQUENCE {

configuredGrantConfigIndex-r19              ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                                BWP-Id,

servCellIndex-r19							   ServCellIndex

},

minimumPucch-PuschOffset-r19 ENUMERATED { symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256, symb512}

--Editor’s note: minimumPucch-PuschOffset can be updated based on further RAN1 discussion.

}

},

nrofReportedRS-UE-IBR-r19                       ENUMERATED {n1, n2, n3, n4},	tci-ServCellIndex-r19				      ServCellIndex                                                         OPTIONAL,    -- Need R

currentBeamReport-r19                    ENUMERATED {enabled}                                                 OPTIONAL,    -- Need R

[Comments]:

[ZTE(wenting)]Both ways can work, there is a typo for the proposed change part,i.e. the comma should be added to the eventThreshold-r19.

# H401

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H401 | MIMO | 2 | CSI-ReportUE-IBR-r19 has no extension markers |  | Huawei (David) |  | v007 | PropAgree |

[Description]: CSI-ReportUE-IBR-r19 has no extension markers, so if RAN1 wants to define new events, new reporting methods, or add any missing parameter or value, late in Rel-19 or in a later release, it will be necessary to add a CSI-ReportUE-IBR-vxy, that may appear after other additional parameters/values unrelated for UE-initiated CSI reports. This was already done but makes the specification less readable. To avoid this, it is better to add extension markers at the end, for each event/reporting mode (for event-specific extensions) and to add events or reporting options.

[Proposed Change]:

CSI-ReportUE-IBR-r19 ::=               SEQUENCE {

eventTypeUE-IBR-r19                    CHOICE {

event1-r19                             SEQUENCE {

eventThreshold-r19                     RSRP-Range,

...

},

event2-r19                             SEQUENCE {

eventThreshold-r19                     INTEGER (0..31),

...

},

event7-r19                             SEQUENCE {

eventThreshold-r19                     INTEGER (0..31),

valueOfQ-r19                           INTEGER (1..8),

...

},

...

},

reportTransmissionMode-r19               CHOICE {

modeA-r19                                NULL,

modeB-r19                                SEQUENCE {

pusch-ResourceOfModeB-r19                SEQUENCE {

configuredGrantConfigIndex-r19           ConfiguredGrantConfigIndex-r16,

ul-BWP-Id-r19                            BWP-Id,

servCellIndex-r19                        ServCellIndex

},

minimumPucch-PuschOffset-r19 ENUMERATED {symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256,

symb512},

...

--Editor’s note: minimumPucch-PuschOffset can be updated based on further RAN1 discussion.

},

...

},

nrofReportedRS-UE-IBR-r19                ENUMERATED {n1, n2, n3, n4},

tci-ServCellIndex-r19                    ServCellIndex                                                        OPTIONAL,    -- Need R

currentBeamReport-r19                    ENUMERATED {enabled}                                                 OPTIONAL,    -- Need R

conditionFulfillmentIndicator-r19        ENUMERATED {enabled}                                                 OPTIONAL,    -- Need R

eventCountWindow-r19                     SEQUENCE {

eventInstanceCount-r19                   INTEGER (2..16),

eventDetectionTimeWindow-r19             ENUMERATED {ms4, ms5, ms8, ms10, ms16, ms20, ms40, ms80, ms160, ms320, ms640, ms1280}

}                                                                                                             OPTIONAL,    -- Need R

pucch-Resource-r19                       SEQUENCE {

periodicityAndOffset                     CHOICE {

sym2                                     NULL,

sym6or7                                  NULL,

sl1                                      NULL,                       -- Recurs in every slot

sl2                                      INTEGER (0..1),

sl4                                      INTEGER (0..3),

sl5                                      INTEGER (0..4),

sl8                                      INTEGER (0..7),

sl10                                     INTEGER (0..9),

sl16                                     INTEGER (0..15),

sl20                                     INTEGER (0..19),

sl40                                     INTEGER (0..39),

sl80                                     INTEGER (0..79),

sl160                                    INTEGER (0..159),

sl320                                    INTEGER (0..319),

sl640                                    INTEGER (0..639)

},

resource                                 PUCCH-ResourceId,

ul-BWP-Id-r19                            BWP-Id,

pucch-Cell-r19                           ENUMERATED {spCell, pucch-Scell}

},

...

}

[Comments]:

[ZTE(Wenting)]We share the similar view, at least, the last “…” is useful.

# L001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L001 | AIML | 1 | Update sgcs-r19 to csi-pai-r19 |  | Soo Kim |  | V020 | Agreed |

[Description]: According to updated RAN1 higher layer parameter list (R1-2506622), UE-assisted performance monitoring quantity within reportQuantity-r19 is chaged from SGCS-r19 to csi-pai-r19.

[Proposed Change]: update the field name

| configurationForChannelMonitoring-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, nrofBestBeamForMonitoring-r19 ENUMERATED {n1, n2} OPTIONAL, -- Need R nrofTransmissionOccasion-r19 ENUMERATED {n1, n3, n7, n15} OPTIONAL, -- Need R timeInstanceFor-RS-PAI-r19 ENUMERATED {n1, n2, n8, spare1} OPTIONAL, -- Need R mappingToResourcesForChannelPrediction-r19 BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OPTIONAL, -- Need R timeInstanceFor-CSI-PAI-r19 ENUMERATED {n1, spare3, spare2, spare1} OPTIONAL, -- Need R ... } ..skip ReportQuantity-r19 ::= CHOICE { none-BM-r19 NULL, none-CSI-r19 NULL, p-CRI-r19 NULL, p-SSB-Index-r19 NULL, p-CRI-RSRP-r19 NULL, p-SSB-Index-RSRP-r19 NULL, rs-PAI-r19 NULL, csi-PAI-r19 NULL } ..skip |  |
| --- | --- |
| timeInstanceFor-RS-PAI Indicates the f-th time instance is used for the performance metric calculation. This field is present only if reportQuantity-r19 is set to 'rs-PAI-r19'. |
| timeInstanceFor-CSI-PAI Indicates the f-th doppler domain unit is used for the performance metric calculation for N4>1. This field is present only if reportQuantity-r19 is set to csi-PAI-r19'. |

[Comments]:

[WI CR rapporteur-v022]: Please note that the proposed changes for timeInstanceFor-SGCS are not aligned with the updated list of parameters from RAN1 in R1-2506622 (see part of the table below). We changed status from “ToDo” to “PropAgree” with the intention to capture in the CR only the changes referring to sgcs-r19.

| reportQuantity-r19 | new |  | none-CSI-r19' for UE-side data collection without CSI report reportQuantity-r19 is set to ‘SGCS-r19’ 'csi-pai-r19' for UE-assisted performance monitoring report | ‘none-CSI-r19', ‘SGCS-r19’, 'csi-pai-r19' |
| --- | --- | --- | --- | --- |
| timeinstanceforsgcs-r19 | new |  | Indicate the f-th doppler domain unit is used for the performance metric calculation for N4>1 | [1, …, N4] |

# N072

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N072 | AIML | 1 | Description of Assoicated ID indication for Set A and Set B |  | Sakira Hassan |  | V015 | Agreed |

[Description]: The descriptions of associatedIdForChannelMeasurement and associatedIdForChannelPrediction in the CSI-ReportConfig description box are not clarified well.

[Proposed Change]: Adopt the following changes:

CSI-ReportConfig field descriptions

associatedIdForChannelMeasurement Indicates the AssociatedId assigned to the resourcesForChannelMeasurement or to the resourcesForChannelPrediction. This field is absent if resourcesForChannelPrediction is not configured or if resourcesForChannelMeasurement is equal to or a subset of resourcesForChannelPrediction.

associatedIdForChannelPrediction Indicates the AssociatedId assigned to the resourcesForChannelMeasurement or to the resourcesForChannelPrediction. This field is absent if resourcesForChannelPrediction is not configured.

[Comments]:

[WI CR rapporteur-v022]: We agree that the field descriptions can be improved, but we think the proposed changes are not fully correct, so we suggest the following changes instead:

CSI-ReportConfig field descriptions

associatedIdForChannelMeasurement Indicates the associated ID for resourcesForChannelMeasurement. This field is absent if resourcesForChannelPrediction is not configured or if resourcesForChannelMeasurement is equal to or a subset of resourcesForChannelPrediction.

associatedIdForChannelPrediction Indicates the associated ID for resourcesForChannelPrediction. This field is absent if resourcesForChannelPrediction is not configured.

# Z413

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z413 | MIMO | 2 | Missed information in the FD of the linkedCJTC-Report |  | ZTE (Wenting Li) |  | v008 | PropAgree |

[Description]:

Missed information in the field description

[Proposed Change]:

linkedCJTC-Report

Linked CJTC report when codebook type is set to typeII-CJT-r18. This field is used in clause 5.2.1.4 in TS 38.214 [19].

[Comments]:

# H005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H005 | AIML | 1 | L1 parameters descriptions |  | Dawid |  | vnnn | Agreed |

[Description]:

The descriptions of nrofTimeInstance-r19 and timeGap-r19 parameters were updated by RAN1 in the latest higher layer parameters list in R1-2506622, as follows:

The descriptions in RRC need to be updated accordingly.

[Proposed Change]:

nrofTimeInstance

When reportQuantity-r19 is set to 'p-CRI-r19', 'p-SSB-Index-r19’, 'p-CRI-RSRP-r19' or 'p-SSB-Index-RSRP-r19', this field indicates the number of future time instance(s) N for prediction to be reported per report setting. When reportQuantity-r19 is set to 'none-BM-r19', this field indicates the number of expected future time instance(s) N of prediction per report setting. This field is not configured together with other reportQuantity-r19 settings. This field is present only if timeGap is configured.

timeGap

When reportQuantity-r19 is set to 'p-CRI-r19', 'p-SSB-Index-r19’, 'p-CRI-RSRP-r19' or 'p-SSB-Index-RSRP-r19':

- if nrofTimeInstance-r19 is set to 1, this field indicates the time gap between the reference time and the first future time instance for prediction,

- if nrofTimeInstance-r19 is set to >1, this field indicates the time gap between two consecutive future time instances for prediction

When reportQuantity-r19 is set to 'none-BM-r19':

- if nrofTimeInstance-r19 is set to 1, this field indicates the expected time gap between the reference time and the first future time instance of prediction,

- if nrofTimeInstance-r19 is set to >1, this field indicates the expected time gap between two consecutive future time instances of prediction.

This field is present only if resourcesForChannelPrediction-r19 and nrofTimeInstance-r19 are configured.

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# B204

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B204 | AIML | 1 | Description related to none-BM-r19 and none-CSI-r19 |  | Congchi Zhang |  | V011 | Rejected |

[Description]:

When ‘none-BM-r19’ or ‘none-CSI-r19’ is configured, UE is expected to perform and measurement and not report the result to gNB. Besides, And it would be good to clarify that ‘non-BM-r19’ and ‘none-CSI-r19’ are for UE-side data collection. In the procedure text there are many places quoted “UE-side data collection configuration” but there seems nowhere clarifies what is it.

[Proposed Change]:

resourcesForChannelPrediction

Indicates resources to be predicted based on measurements performed on resourcesForChannelMeasurement. The UE is not expected to measure the resources to be predicted, unless the reportQuantity-r19 is set to 'none-BM-r19' or ‘none-CSI-r19’. This field is present only if reportQuantity-r19 is set to 'p-CRI-r19', 'p-SSB-Index’-r19, 'p-CRI-RSRP-r19', 'p-SSB-Index-RSRP-r19', 'none-BM-r19', or ‘none-CSI-r19’. When reportQuantity-r19 is set to 'none-BM-r19' or ‘none-CSI-r19’, it implies the configuration is for UE-side data collection.

[Comments]:

[LGE-Soo-v020] Agree

[WI CR rapporteur-v022]: We are fine with the intention of the proposal. However, the proposed changes are not correct because resourcesForChannelPrediction is configured only for beam management and not for CSI prediction, so ‘none-CSI-r19’ should not appear in this field description. Perhaps alternative solutions can be provided.

[Lenovo-Congchi-v024]: Thanks repporteur for poingt it out. Another alternative could be making a general clarification in the field description of reportQuantity, since we already started clarifying reportQuantity-r19 there.

reportQuantity

The CSI related quantities to report. see TS 38.214 [19], clause 5.2.1. If the field reportQuantity-r16, reportQuantity-r17, reportQuantity-r18 or reportQuantity-r19 is present, UE shall ignore reportQuantity (without suffix). Network does not configure reportQuantity-r17 or reportQuantity-r18 together with reportQuantity-r16. Network does not configure reportQuantity-r19 together with reportQuantity-r16, reportQuantity-r17 or reportQuantity-r18. When reportQuantity-r19 is set to 'none-BM-r19' or ‘none-CSI-r19’, it implies the configuration is for UE-side data collection.

[WI CR rapporteur-v031]: We think it is somewhat strange to define just some values for reportQuantity-r19 in the field description (only those for UE-side data collection) and not the others. An alternative solution is covered in H008.

[WI CR rapporteur, 2025-10-24]: We changed the status to “Rejected”, in the sense that the alternative solution from H008 was included in the CR and also covers this RIL.

# N060

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N060 | MIMO | 1 | Reference to wrong version of nrofReportedRS in FD of conditionFulfillmentIndicator |  | Nokia (Andrew) |  | v010 |  |

[Description]: The field description of conditionFulfillmentIndicator states that it is only configured when the (Rel-15) field nrofReportedRS is n1. This should be the Rel-19 field nrofReportedRS-UE-IBR that was added for UE-IBR.

[Proposed Change]:

CSI-ReportUE-IBR field descriptions

conditionFulfillmentIndicator If configured, the UE includes an indication whether or not each reported RS fulfilled the event condition. This parameter is only configured if eventDetectionTimeWindow is configured and nrofReportedRS-UE-IBR is not set to n1.

Note: “nrofReportedRS-UE-Initated” should be used instead of “nrofReportedRS-UE-IBR” if E032 is agreed.

[Comments]:

# C251

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C251 | MIMO | 1 | Redundant field description in eventInstanceCount-r19 |  | CATT (LeiWang) |  | V003 | PropAgree |

[Description]:

The current field eventCountWindow-r19 includes two child fields, i.e., eventInstanceCount-r19 and eventDetectionTimeWindow-r19. And in the field description of eventInstanceCount-r19, we had captured the RAN1 intention that the eventInstanceCount-r19 is only configured if eventDetectionTimeWindow is configured. This sentence is similar funactionality as the conditional present condition. However, currently we capture this conditional present by the two mandatory present child fields in the eventCountWindow-r19. That means if the optional parent field eventCountWindow-r19 is configured, the two child fields are always present together. So it is redundant to further capture the sentence of “This field is only configured if eventDetectionTimeWindow is configured”

[Proposed Change]:

eventInstanceCount

Indicates the minimum number of event instances for one same new beam within a configured time window that the UE can initiate UEIBM report (see TS 38.214 [19], clause 5.2.1.5.4.1).

[Comments]:

# E033

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E033 | MIMO | 1 | Aligment of the term UE initiated CSI report in field description |  | Ericsson (Lian) |  | V006 | PropAgree |

[Description]: RAN1 specifications refer to UE IBR as UE initiated CSI report. While this was already aligend in other parts of 38.331 it is missing for the field description of pucch-Resource. There is also a typo in initiated wording in the field description.

[Proposed Change]:

Update the field description as follows:

pucch-Resource

Indicates the periodic PUCCH resource for the UE initiated CSI reporting indicator for both mode-A and mode-B UE initiated CSI reporting:

-	to request dynamically scheduled PUSCH to carry UE initiatedCSI report for mode-A;

-	to notify the network of a Type-1 CG PUSCH to carry UE initiatedCSI report for mode-B.

[Comments]: [Huawei] Agree. Similar change is needed for the field description of evenInstanceCount:

Indicates the minimum number of event instances for one same new beam within a configured time window that the UE can initiate CSI report (see TS 38.214 [19], clause 5.2.1.5.4.1). This field is only configured if eventDetectionTimeWindow is configured

# N029

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N029 | AIML | 1 | Variable name for maximum number of data collection candidate configs is inconsistent. | N/A | Jerediah Fevold |  | vnnn | Agreed |

[Description]: The variable for the maximum number of DataCollectionCandidateConfigId-r19 is inconsistent with the name of the ID it is counting.

[Proposed Change]:

DataCollectionCandidateConfigId-r19 ::=            INTEGER (0..maxNrofDataCollectionCandidateConfigs-1-r19)

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# O004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O004 | NES, SLRelay | 1 | Applicability of PO bundling to SL Relay | R2-25xxxxx | OPPO (Qianxi) |  | V002 | ToDo |

[Description]: It is not clear whether the paging adapation (i.e., PO bundling) feature can be applied to SL Relay UE (first/last Relay) and Remote UE.

[Proposed Change]: R2 discuss and conclude the applicability of paging adapation (i.e., PO bundling) feature to SL Relay UE (first/last Relay) and Remote UE or not.

[Comments]:

[Rapporteur] This requires further discussion in the next meeting.

# O005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O005 | NES, LPWUS | 1 | Applicability of PO bundling to LP-SS | R2-25xxxxx | OPPO (Qianxi) |  | V002 | Agred |

[Description]: It is not clear whether the paging adapation (i.e., PO bundling) feature can be enabled together with LP-SS feature or not.

[Proposed Change]: R2 discuss and conclude the applicability of co-configuring paging adaptation and LP-SS feature.

[Comments]:

[Apple] As paging adaptation and LP-WUS are both specified in Rel-19, we generally don’t pursue their feature combination in same release. Prefer to leave it to 6G.

[Rapporteur] This requires further discussion in the next meeting.

[Huawei] We think there are several possible solutions:

Option 1: gNB can configure both R19 LP-WUS and R19 paging adaptation, and when they are configured, R19 LP-WUS applies to legacy POs only, it does not apply to R19 POs.

Option 2: gNB can configure both R19 LP-WUS and R19 paging adaptation, and when they are configured, two LP-WUS configurations are provided, where one is for legacy POs, the other one is for R19 POs

Option 3: Do not support to configure R19 LP-WUS and R19 paging adaptation simultaneously in SIB1

The issue is also being discussed in RAN1, we prefer to wait for RAN1 conclusion.

[LPWUS WI Rapp]: This issue was discussed in LP-WUS session, and it was agreed as below, which have been implemented in the LP-WUS CR.

- RAN2 confirms that LP-WUS and paging adaptation can be configured and used together.

- Separate LP-WUS configuration for legacy UEs and UEs supporting paging adaptation. Introduce a separate lpwus-LoFrameOffsetList configuration for LP-WUS with paging adaptation.

# S034

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S034 | NES | 2 | pagingAdaptNAndPagingFrameOffset-r19 |  | Anil Agiwal (Samsung) |  | V023 | PropReject |

[Description]:  oneT, halfT, quarterT, oneEighthT and oneSixteenthT are not needed in pagingAdaptNAndPagingFrameOffset-r19. For paging adaptation, the key requirement is extension of gap between PFs

[Proposed Change]: Remove oneT, halfT, quarterT, oneEighthT and oneSixteenthT from pagingAdaptNAndPagingFrameOffset-r19

[Comments]:

[Apple] We think the restriction of value is not necessary. For example, 1/16 or larger value can be set when T<=16 frames. It can be left to NW implementation to set correct value.

[Rapporteur] This was brought up earlier and companies wanted to leave that to network implementation.

[Samsung]: It does not make sense to just increase number of POs without configuring T/32. As far as i remember we have not discussed this. The reason we allowed increased number of POs was for extending interval between PF to T/32.

# E034

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E034 | LPWUS | 2 | LP-WUS lastUsedCellOnly | R2-25xxx | Ericsson (Martin) |  | V006 | Rejected |

[Description]: RAN3 agreed to introduce lastUsedCellOnly for LP-WUS. RAN2 specifications need to be aligned with RAN3.

See draft minutes draft_RAN3 #129 Meeting Report_TDoc_Participants.zip and R3-255828:

[Last Used Cell]

- The anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

[Proposed Change]: The following changes need to be made:

- Add lastUsedCellOnly to SIB1

- Add noLastCellUpdate to RRCRelease message

Furthermore changes to 38.300 and 38.304 are needed. Further details are provided in Tdoc.

[Comments]:

[WI Rapp] The RAN3 agreement which said anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message could be applied to the scenario that LP-WUS is not only used in last used cell. It means that the LP-WUS can be used in any cell, i.e., don’t introduce lastUsedCellOnly for LP-WUS, also needs the agreement in RAN3.

The background for RAN3 discussion on this issue is, they think this part is related to the lastUsedCellOnly, but all RAN3 agreed that lastUsedCellOnly should be determined in RAN2.

Thus, the above RAN3 conclusion doesn't mean the “last used cell” was agreed in RAN3, and RAN3 has closed this issue.

Considering RAN2 has conclusion on this issue, there is no need to reopen it.

# H125

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H125 | NES | 1 | Optionality of R19 PEI configurations |  | Huawei (Lili) |  | V012 | PropReject |

[Description]: The R19 PEI configurations should be made optional.

[Proposed Change]:

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...,

[[

po-NumPerPEI-r19                          ENUMERATED {po1, po2, po4, po8}                        			OPTIONAL,  -- Need R

payloadSizeDCI-2-7-r19                    INTEGER (1..maxDCI-2-7-Size-r17)                        			OPTIONAL,  -- Need R

pei-FrameOffset-r19                       INTEGER (0..32)                        								OPTIONAL  -- Need R

]]

}

[Comments]: [OPPO] How to interpret the case where one specific parameter is absent (but the others are present?), considering [[]]] already provides optionality.

[Apple] Agree with OPPO.

[Rapporteur] This depends on whether it should be possible to configure PEI for Rel-19 NES UEs when PEI (Rel-17) is not configured. Based on the agreements below:

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

it should not be possible (although it was not explicitly captured) so the current implementation should be fine.

[Huawei] Even if the optionality is indicated by “[[ ]]” implicitly, the need code also needs to be provided. Basically all other fields added under “[[ ]]” are marked as OPTIONAL with a need code. If the concern is that these parameters are either all configured or all absent, then maybe we a can used a R19 field to group these parameters.

# C026

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C026 | NES, LPWUS | 2 | Co-existence of LP-WUS in idle/inactive and paging adaptation | R2-25xxxxx | Da Wang (CATT) |  | V004 | Duplicate |

[Description]: It is not clear whether LP-WUS in idle/inactive can be co-exist with Rel-19 paging adaptation mechanism in NES.

[Proposed Change]: R2 discuss whether and how LP-WUS in idle/inactive can be co-exist with Rel-19 paging adaptation mechanism in NES.

[Comments]:

# S056

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S056 | LPWUS | 1 | Incorrect value range of lpss-BinarySeqIndex-r19 |  |  |  | vnnn | Rejected |

[Description]: The value range of lpss-BinarySeqIndex-r19 is not aligned with the RAN1 agreed 38.211 CR (R1-2506651):

7.4.5.1.1	Generation of rOOK(n)

The sequence rOOK0,…,rOOK(NOOK-1) is defined by Tables 7.4.5.1.1-1 to 7.4.5.1.1-3 with the quantity MLPSS given by the higher-layer parameter XXX and the configuration index by the higher-layer parameter lpss-BinarySeqIndex.

Table 7.4.5.1.1-1: The sequence rOOK0⋯rOOK(NOOK-1) for MLPSS=1.

| Configuration | Length 6 | Length 8 |
| --- | --- | --- |
| 0 | [1 0 1 0 1 0] | [1 0 1 0 0 1 0 1] |
| 1 | [0 1 0 1 0 1] | [1 0 1 0 1 0 0 1] |
| 2 | [1 0 0 1 0 1] | [1 0 0 1 0 1 0 1] |
| 3 | [1 0 1 0 0 1] | [0 1 0 1 0 1 0 1] |

[Proposed Change]: The value range of lpss-BinarySeqIndex-r19 should be INTEGER (0..3)

[Comments]:

[WI Rapp]: Current RRC CR is captured according to RAN1 RRC parameters. I think it is better to ask RAN1 to update RRC parameter to RAN2 or update RAN1 specification.

# H051

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H051 | LPWUS | 1 | Need code for “lpss-OverlaidSeqRoot-r19” parameter | R2-25xxxxx | Kuang Yiru (Huawei) |  | V003 | Agreed |

[Description]: Based on the description for “OOK4-Only” conditional presence, the parameter “lpss-OverlaidSeqRoot-r19” is optional if M = 1. So, this needs to be Need R.

[Proposed Change]: Change to Need R for “lpss-OverlaidSeqRoot-r19” if M = 1.

[Comments]:

# H052

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H052 | LPWUS | 1 | Missing need code for “startSymbol2-r19” in “lpss-StartSymbol-r19” | R2-25xxxxx | Rama Kumar Mopidevi (Huawei) |  | V003 | Agreed |

[Description]: Need code missing for “startSymbol2-r19”.

[Proposed Change]: Add “Need R”.

[Comments]: if the startSymbol2-r19 is not present, the startSymbol2-r19 should be released.

# H053

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H053 | LPWUS | 1 | Impact on thresholds entry condition due to RAN4 definition on LR types | R2-25xxxxx | Rama Kumar Mopidevi (Huawei) |  | V003 | Rejected |

[Description]: RAN4 defined two LR types (i.e., Type 1 and Type 2) in RAN4#114bis and agreed two different requirements for these. Due to different requirements, the current thresholds for entry condition for LP-WUS monitoring and RRM measurement relaxation/serving cell measurement offloading should be extended to ensure that the NW can configure appropriate thresholds for different LR types.

[Proposed Change]: Create separate set of entry thresholds for each type of LR.

[Comments]:

# E036

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E036 | LPWUS | 2 | Type 1 and 2 LR | R2-25xxx | Ericsson (Martin) | See also H53 and H54. | V006 | Rejected |

[Description]: RAN4 agreed that the UE may implement two types of WUR, i.e. implementation supporting Noise Figure (NF) pluse Implementation Margin (IM) of 13.5 or 18 dB. See draft CR R4-2511904 and LS R4-2503003:

Agreement:

- For the FR1 requirements targeting at bands <2.5GHz

  - The IM+NF values are

    - Set 1: 18dB

    - Set 2: 13.5dB

[Proposed Change]:

In case LP-WUS is configured close the channel edge then LP-WUS UEs that only support IM+NF of 18 dB should not be allowed to use LP-WUS to prevent degradation in the paging performance. It should be possible to indicate in SIB1 that UE supporting only IM+NF of 18 dB is not supported in the cell.

Further details are provided in Tdoc.

[Comments]:

# H054

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H054 | LPWUS | 1 | Impact on thresholds exit condition due to RAN4 definition on LR types | R2-25xxxxx | Rama Kumar Mopidevi (Huawei) |  | V003 | Rejected |

[Description]: For the same reason given to H053, the current thresholds for exit condition for LP-WUS monitoring and RRM measurement relaxation/serving cell measurement offloading should be extended to ensure that the NW can configure appropriate thresholds for different LR types.

[Proposed Change]: Create separate set of exit thresholds for each type of LR.

[Comments]:

# C157

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C157 | MOB | 1 | There is no need to mandatorily provide the 2TA configuration in the IE EarlyUL-SyncConfig if tag2 is present in the SpCellConfig in ltm-CandidateConfig |  | Rui (CATT) |  | V005 | ToDo |

[Description]:

NW should have the flexibility to perform early UL sync on a specific TRP even though the mTRP configuration is present in the candidate configuration.

[Proposed Change]:

| Conditional Presence | Explanation |
| --- | --- |
| 2TA | This field is mandatoryoptional present if tag2 is present in the SpCellConfig in ltm-CandidateConfig. It is absent, Need R, otherwise. |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S. |
| TDD | This field is optionally present, Need R, for TDD LTM candidate cells. It is absent otherwise. |

[Comments]:

[Rapporteur] I guess it would be better to discuss this case online. No tdoc is needed anyway.

# J002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J002 | NES | 1 | Update field description of absoluteFrequencySSB |  | Sharp (LIU Lei) |  | V006 | ToDo |

[Description]: If od-ssb is configured, the field od-ssb-absoluteFrequency-r19 is mandatory present when absoluteFrequencySSB of the serving cell is absent, and it makes the UE can obtain timing on this SCell. Thus, the field description of absoluteFrequencySSB needs to be updated.

[Proposed Change]: The text proposal is as below:

absoluteFrequencySSB

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb is not configured for this serving cell, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference.

[Comments]:

[Ericsson] Since this very much is an issue in RRC we need to discuss also in RAN2. See E023, E024.

[vivo] Agree with the intention for OD-SSB case#1 that UE does not obtain timing reference from another cell when OD-SSB is configured, even if OD-SSB is configured but not activated.

[Apple] Agree with the issue. On the change, either this one or E023 is OK.

[Rapporteur] This requires further discussion in the next meeting.

[Samsung] The proposed change seems not correct. For case #1, How can UE obtain the timing based on OD-SSB if OD-SSB is not activated?

# E023

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E023 | NES | 1 | It is unclear what is the Case1 with respect of the SSBless Scell. | yes | Helka-Liina Määttänen (ER) |  | V019 | ToDo |

[Description]: In IE FrequencyInfoDL

absoluteFrequencySSB

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference.

Up until Release 19, a serving cell which is not associated with SSB is “SSB-less SCell”. This term is used in RRC but it is not definined. However, it seems to point to a case where UE obtains timing reference in a defined way as seen from field dercription of the absoluteFrequencySSB. The servingcellMO may or may not be configured for this case.

For Rel-19, Case 1, the scell is not associated with legacy SSB but only OD-SSB and absoluteFrequencySSB should be absent.

[Proposed Change]:

absoluteFrequencySSB

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb is absent in SCellConfig IE, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference.

[Apple] We agree with this change.

[Rapporteur] This requires further discussion in the next meeting.

[Samsung] The proposed change seems not correct. For case #1, How can UE obtain the timing based on OD-SSB if OD-SSB is not activated?

# V503

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V503 | NES | 1 | Relation between OD-SSB case 1 & SSB-less SCell |  | vivo (Jianhui) |  | V032 | ToDo |

[Description]:

Similar to the comments from J002 & E023, we agree with the intention to clarify the relation between OD-SSB case 1 & SSB-less SCell In IE FrequencyInfoDL.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference.

One more clarification is that the FD revision for the OD-SSB operation, if agreed, only applies to the UEs which support OD-SSB operation. However, as the IE absoluteFrequencySSB is only configured for CONNECTED UE, it’s natural that the NW does not configure od-ssb-config for a UE that does not support OD-SSB operation. Thus we think the proposed change in J002 is accurate and sufficient, with IE name correction as below.

[Proposed Change]:

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-config is not configured for this serving cell, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

[Comments]:

[Rapporteur] This requires further discussion in the next meeting.

# O400

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O400 | XR | 1 | Restriction on additionalPriority-r19 and priorityAdjustmentThreshold-r19 |  | Zhe Fu (OPPO) |  | V06 | PropRej |

[Description]: Currently, there are no restrictions on whether the enhanced LCP feature is applied only for DRB or not. Our understanding is that additionalPriority-r19 and priorityAdjustmentThreshold-r19 are only configured/applied for the logical channel associated with a DRB, as the motivation of the enhanced LCP is to prioritise the transmission of XR traffic with delay-sensitive data. Thus, we suggest reflecting such configuration restrictions for additionalPriority-r19 and priorityAdjustmentThreshold-r19.

[Proposed Change]: Select one of the following options to restrict additionalPriority-r19 and priorityAdjustmentThreshold-r19 to be configured only for a logical channel associated with a DRB.

Option 1: Add a condition for enhancedLCP-r19, as below:

enhancedLCP-r19                  SEQUENCE{

priorityAdjustmentThreshold-r19          INTEGER (1..64),

additionalPriority-r19                   INTEGER (1..16),

...

}                                                                                                        OPTIONAL     -- Cond DRB2

| Conditional presence | Explanation |
| --- | --- |
| DRB2 | This field is optionally present in case of DRB, need M. Otherwise, it is absent for SRBs and MRBs. |

Option 2: Add some description in the field descriptions of additionalPriority-r19 and priorityAdjustmentThreshold-r19, to clarify the two IEs are configured only for the logical channel associated with a DRB.

additionalPriority The additional logical channel priority that overrides the logical channel priority configured by the field priority when the logical channel priority adjustment condition is satisfied as specified in TS 38.321 [3]. For the same logical channel configuration, the value of the field shall be smaller than that of the field priority. This field can only be configured for the logical channel associated with a DRB.

priorityAdjustmentThreshold Remaining time threshold for determining whether the additional logical channel priority configured by additionalPriority is applied for the logical channel, as specified in TS 38.321 [3]. Value in number of milliseconds. This field can only be configured for the logical channel associated with a DRB.

[Comments]: [Rapp]My feeling is that this is not quite necessary. PDCP discard timer can only be configured for DRB. Then, isn’t it obvious that you cannot configure additional priority and the adjustment threshold for the other types of RBs. In addition, there are other previously introduced configurations that are only added for DRB, but without such restriction.

We can trust reasonable implementation

# H202

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H202 | GEN | 1 | Enhancing the readability of RRC procedure text | R2-250xxxx | Yinghao Guo (Huawei) |  | V09 | ToDo |

[Description]: Some chapters for RRC procedure text is too long, e.g., 17 pages for Reception of an RRCReconfiguration by the UE, 13 pages for Actions related to transmission of UEAssistanceInformation message. In these sections, for instance, if you read an if condition for a certain procedure, you need to look for the else if, sometimes, 2 or 3 pages away. The text is getting a bit hard to read because of this.

[Proposed Change]: The solution for this case can be to move the RRC procedure belong to a certain feature to a separate section and refer to these sections in the multiple instances in the main procedure

[Comments]:

# S055

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S055 | XR | 1 | Reference to ‘same LCH configuration’ |  | Milos (Samsung) |  | V10 | PropAgree |

[Description]: Current definition of additional priority states that “For the same logical channel configuration, the value of the field shall be smaller than that of the field priority”. It is unclear what “the same logical channel configuration” refers to. Does it refer to two configurations of the same logical channel, or to two sets of configurations of all LCHs at the UE (including e.g. LCG configuration)? Additional priority is set per logical channel, as part of per-LCH configuration; in other words, it should not concern itself with the set of configurations of all LCHs at the UE.

[Proposed Change]: Remove “For the same logical channel configuration” bit of the sentence.

[Comments]: [Rapp] Change to “Within the LogicalChannelConfig for a certain logical channel, if the field is present, the value of the field shall be smaller than that of the field priority”

# E054

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E054 | MOB | 1 | Need code of ltm-ExecutionCondition in LTM-Candidate |  | Tony (Ericsson) |  | V009 | ToDo |

[Description]: In RRC CR agreed in the last meeting (R2-2507729) it was agreed to have a new procedure for the handling of the execution condition for LTM. However, one of the proposal to have the setupRelese with need code “N” instead of “M”. However, according to the current procedural text agreed in R2-2507729 the setupRelease is not needed anymore as the UE will simply use the field ltm-ExecutionCondition simply to maintain a UE variable. For this reason, it would be better to have the field without any SetupRelease (and with need code Need N).

[Proposed Change]:

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...,

[[

ltm-NoSecurityChangeID-r19                     LTM-NoSecurityChangeId-r19                            OPTIONAL,    -- Need M

ltm-ExecutionCondition-r19                     LTM-ExecutionConditionList-r19         OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r19        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r19       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r19     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

ltm-CSI-ReportConfig-r19                       SetupRelease {LTM-CSI-ReportConfig-r18}               OPTIONAL,    -- Need M

ltm-CSI-IM-ResourceToAddModList-r19            SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceToReleaseList-r19           SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceSetToAddModList-r19         SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet

OPTIONAL,    -- Need N

ltm-CSI-IM-ResourceSetToReleaseList-r19        SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId

OPTIONAL     -- Need N

]]

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need S

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

LTM-NoSecurityChangeId-r19 ::= INTEGER (1..maxNrofLTM-Configs-plus1-r18)

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

[Comments]:

[MediaTek / Pasi]

Actually, the original discussion based on which this RIL was created was for LTM-Config -> ltm-ServingCellExecutionCondition-r19 field, not for LTM-Candidate -> ltm-ExecutionCondition-r19 field.

The change proposed by Ericsson to LTM-Candidate -> ltm-ExecutionCondition-r19 field seems incorrect, as the UE updates the UE variable based on this field only when LTM cell switch towards this candidate occurs (i.e., this candidate becomes the SpCell). It should be possible for the network to release the execution condition configured for a candidate, so LTM-Candidate -> ltm-ExecutionCondition-r19 field should be kept as SetupRelease.

[ZTE / Mengjie]

Agree with MediaTek.

# C158

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C158 | MOB | 1 | Add the field description for ltm-CSI-ReportConfig-r19 under the LTM-Candidate |  | Rui (CATT) |  | V005 | Rejected |

[Description]: suggest to add the field description for ltm-CSI-ReportConfig-r19 under the LTM-Candidate to clarify the following aspects,

-	It is used to configure CSI report setting for the candidate cell configured by the LTM-Candidate

-	UE ignores the associated RSs from other candidate cell when acquire CSI for this candidate cell.

-	If LTM-CSI-ReportConfig is configured under in an LTM-Candidate, the UE ignores the fields ltm-ReportConfigType and ltm-ReportContent.

[Proposed Change]:

[Comments]:

[Rapporteur] The understanding is not correct. When the LTM CSI report configuration is configured the UE shall not ignore the LTM CSI report configurations within the CSI-MeasConfig. This configuration are used for two different things. Also, how this field is used is already described in the RAN1 specification, so no need to further clarify in our spec.

# M202

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M202 | MOB | 1 | Need code for ltm-CSI-ReportConfig-r19 in LTM-Candidate IE |  | Pasi (MediaTek) |  | V008 | Agreed |

[Description]:

Need code "Need N" seems incorrect for ltm-CSI-ReportConfig-r19 in LTM-Candidate IE. We think it should be "Need M".

[Proposed Change]:

ltm-CSI-ReportConfig-r19     LTM-CSI-ReportConfig-r18                 OPTIONAL,     -- Need M

[Comments]:

[Rapporteur] This is a field which the UE used to send the first CSI report when an LTM cell switch is executed, but the understanding is that the UE shall not continue to use this configuration afterwards. We can double check what RAN1 specification captures or eventually we can ask them how we should interpret the use of this field.

[Huawei] This field is stored by the UE and to be used for every LTM cell switch to the LTM-Candidate, so Need N makes no sense. It can be Need M or Need R.

# B110

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B110 | MOB | 1 | CSI report configuration for early CSI acquisition after cell switch | R2-25xxxxx | Lenovo (Lianhai Wu) |  | V016 | Rejected |

[Description]: Based on the current spec, UE will keep the whole LTM-config IE after cell switch. However, LTM configu IE includes LTM-CSI-ReportConfig-r18 for early CSI acquisition. Early CSI acquisition is only for cell switch case. In addition, LTM-CSI-ReportConfig IE for early CSI acquisition is generated by source cell. The target cell does not know LTM-CSI-ReportConfigId of LTM-CSI-ReportConfig. Therefore, the target cell cannot release it. We propose that after CSI reporting at the target cell after or during cell switch triggered by LTM, LTM-CSI-ReportConfig IE of LTM-candidate IE can be released by UE.

[Proposed Change]: After CSI reporting at the target cell after or during cell switch triggered by LTM, LTM-CSI-ReportConfig IE of LTM-candidate IE can be released by UE. We will prepare one contribution to discuss this issue.

[Comments]:

[Rapporteur] Agree that after transmitting the CSI report due to the early CSI acquisition the UE shall not use  this field anymore. The field is need N so it is something that UE should use as a one-shot configuration and not use anymore. To me this already address the concern raised by this RIL (given also that UE actions are described in the RAN1 specification).

[Huawei] This field is the (only) ltm-CSI-ReportConfig-r19 in LTM-Candidate. It should be either Need M or Need R. If it is Need M, to release the field, the target cell must release the LTM-Candidate. If it is need R, to release the field, the target cell can reconfigure the LTM-Candidate with the field absent. The ltm-CSI-ReportConfigId in ltm-CSI-ReportConfig-r19 has no use.

# B111

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B111 | MOB | 1 | Maintenance of CSI resource in LTM-configu after cell switch | R2-25xxxxx | Lenovo (Lianhai Wu) |  | V016 | Rejected |

[Description]: Based on the current RRC CR, UE will keep the whole LTM-config IE after cell switch. CSI-RS/IM resources are included in LTM-config IE. Specifically, LTM configu IE includes periodical and SP CSI-RS resource sets and periodical and SP CSI-IM resource sets for L1 measurement or early CSI acquisition. Early CSI acquisition is only for cell switch case. Therefore, regarding the CSI resource for early aqiusition, after CSI reporting at the target cell after or during cell switch triggered by LTM, the configured periodic or SP CSI-RS/CSI-IM resource sets for the target cell can be released. Regarding CSI resource of the candidate cells for L1 measurment or early CSI acquisition, we suggest aligning with the maintenance of CSI resource of target cell for early CSI acquisition purpose. Target cell can reconfigure this CSI resource since the target cell is aware of CSI resource from other candiate cells.

[Proposed Change]:

Proposal x1: After CSI reporting at the target cell after or during cell switch triggered by LTM, the configured periodic or SP CSI-RS/CSI-IM resource sets for the target cell are released.

Proposal x2: After reconfiguration with sync that is triggered by LTM, the configured the periodic or SP CSI-RS/CSI-IM resource sets for all candidate cell(s), except the target cell, are released.

We will prepare one contribution to discuss this issue.

[Comments]:

[Rapporteur] See B111

[Huawei] In Rel-18, there is no release of periodic resources (SSB) after LTM cell switch, why should periodic CSI-RS resources be released after LTM cell switch in Rel-19? With respect to semi-persistent resources configured in LTM-Config, no matter for which candidate cell (they are mixing candidate cells anyway), they are deactivated at LTM cell switch, and it is up to the network (=target cell, now serving cell) to activate them via MAC CE or not, so there is no need to release them.

# C159

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C159 | MOB | 1 | Ambiguity of the ltm-ExecutionCondition field description on whether it is only used for MCG LTM |  | Rui (CATT) |  | V005 | Rejected |

[Description]:

CLTM is only supported on MCG LTM, However, “an ltm-Config associated with the MCG” used in the field description is not equal to MCG LTM.for example,a inter-CU SCG LTM configuration is also associated with the MCG.

[Proposed Change]:

LTM-Candidate field descriptions

ltm-ExecutionCondition This field can only be included in an ltm-Config associated with the MCG for LTM on the MCG.

[Comments]:

[MediaTek (Pasi)]

The field description talks about ltm-Config field (not LTM-Config IE), so it does not cover ltm-ConfigNRDC. Therefore, the current field description seems unambiguous.

Note that we have similar text also in clause 5.3.7.3: "1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG; and", which is unambiguous.

[Rapporteur] Probably the field description of ltm-ConfigNRDC needs to be modified. The field should contain LTM candidate configuration associated with the SCG and the MCG configuration (not LTM candidate configuration). Maybe we can do it this way?

[Huawei] 5.3.5.18.1 does not include any concept of "associated with the MCG", it says when something is for LTM on the MCG or for LTM on the SCG, so the wording proposed by CATT should be adopted to align with 5.3.5.18.1.

# O001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O001 | MOB, SLRelay | 1 | LTM, C-LTM applicability to intermediate Relay |  | OPPO (Qianxi) |  | V003 | Rejected |

[Description]: Based on the current spec, it is unclear whether LTM, CLTM based handover can be applied to the intermediate SL relay UEs.

[Proposed Change]: R2 to clarify whether LTM, CLTM based handover can be applied to the intermediate SL relay UEs or not.

[Comments]:

[Rapporteur] No tdoc expected on this as we can simply discuss online. Please note that this is not a Rel-19 issue but rather a Rel-18. My preference would be to not work on this coexistance, meaning that whatever situation we have in Rel-18 it applies also to Rel-19.

# V400

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V400 | MOB | 2 | Introduce new attemptLTM-Switch-r19 to cover the case when a UE only support intra-CU LTM fast recovery but not inter-CU fast recovery | R2-25xxxxx | vivo (Jing Liang) |  | V007 | Rejected |

[Description]: According to the last meeting agreement, a new UE capability for inter-CU LTM recovery is supported. Therefore, reusing the legacy attemptLTM-Switch-r18 cannot cover all cases.

For example, if a UE supports Rel-18 intra-CU LTM-based recovery but does NOT support Rel-19 inter-CU LTM-based recovery, the network will NOT configure attemptLTM-Switch-r18 to the UE while inter-CU LTM candidate cells are present. Then the UE cannot do intra-CU LTM-based recovery.

[Proposed Change]: A new indicator for the inter-CU LTM recovery should be introduced (e.g., attemptLTM-SwitchForInterCU-r19) instead of reusing the attemptLTM-Switch-r18. The procedure in section 5.3.7 for the LTM recovery should also be updated with the new indicator.

[Comments]:

[Rapporteur] This was already discussed during the RRC implementation. It seems there is no need to introduce an additional field.

[Huawei] If the UE does not support recovery for inter-CU LTM, the UE will simply not do it, and the network is aware of this, so there is no issue related to UE capability.

# H150

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H150 | MOB | 3 | ltm-ServingCellExecutionCondition as SetupRelease Need M contradicts with procedure text | R2-25xxxxx | Huawei (David) |  | V023 | Agreed |

[Description]: Normally, when a field is SetupRelease Need M, when the field is configured and in later reconfigurations, it is absent, the field is still kept and used. However, according to procedure text, upon LTM cell switch execution, the UE uses execution conditions from the selected LTM-Candidate, but ltm-ServingCellExecutionCondition remains configured.

Then, there are problems like:

- Z151 has a TP for 5.3.5.3, in order not to clear conditional measIds that are referred to by CLTM execution conditions, that TP mentions "LTM-ExecutionConditionList", but how to say which one it is? It is "the one that is in use", but this has actually no definition

- C153 is proposing that "release" affects the CLTM execution conditions in use, but normally, "release" of a SetupRelease Need M field is only for release of that field, it is not supposed to have actions on things coming from other fields

[Proposed Change]:

1) Change ltm-ServingCellExecutionCondition to a Need N field

2) Create a variable VarLTM-ExecutionConditions

3) Reception of ltm-ServingCellExecutionCondition and LTM cell switch override the variable contents

4) For the TP in Z151, use the variable

[Comments]:

[Rapporteur] This RIL can be discussed based on contribution from proponent company

# C160

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C160 | MOB | 1 | Ambiguity of the ltm-ServingCellExecutionCondition field description on whether it is only used for MCG LTM |  | Rui (CATT) |  | V005 | Rejected |

[Description]: similar issue as C159

[Proposed Change]:

[Rapporteur] See C159

[Huawei] 5.3.5.18.1 does not include any concept of "associated with the MCG", it says when something is for LTM on the MCG or for LTM on the SCG, so the wording proposed by CATT should be adopted to align with 5.3.5.18.1.

LTM-Config field descriptions

ltm-ServingCellExecutionCondition This field can can only be included in an ltm-Config associated with the MCG for LTM on the MCG.

[Comments]:

[MediaTek (Pasi)]

The field description talks about ltm-Config field (not LTM-Config IE), so it does not cover ltm-ConfigNRDC. Therefore, the current field description seems unambiguous.

Note that we have similar text also in clause 5.3.7.3: "1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG; and", which is unambiguous.

# M203

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M203 | MOB | 1 | Ambiguity of the Cond for attemptLTM-Switch |  | Pasi (MediaTek) |  | V008 | Agreed |

[Description]:

(Inspired by C159/C160)

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |

Based on clause 5.3.7.3, only LTM candidate configurations configured in ltm-Config for LTM on the MCG are considered for fast LTM recovery. The LTM candidate configurations configured in ltm-ConfigNRDC are not considered for fast LTM recovery.

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG; and

1> if at least one of the following conditions is fulfilled:

2>	the selected cell does not have the field ltm-NoSecurityChangeID configured and the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChange; or

2>	the cell selection is triggered by detecting radio link failure of the MCG and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChange; or

2>	the cell selection is triggered by detecting re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.x or 5.3.5.18.6 and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-NoSecurityChangeID configured within the LTM candidate configuration for which the re-configuration with sync failure is detected:

3>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

Since LTM candidate configurations configured in ltm-ConfigNRDC are also "associated with the MCG" (i.e., contain MCG configuration), the Cond for attemptLTM-Switch needs to be fixed to be in line with clause 5.3.7.3.

[Proposed Change]:

[Huawei] This text should be modified to say "for LTM on the MCG", which is the only thing defined in 5.3.5.18.1, there is no concept of "associated with the MCG".

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least one LTM candidate configuration in an ltm-Config associated with the MCG. Otherwise, the field absent. |

[Comments]:

# N102

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B111 | MOB | 1 | LTM Config NR-DC Clarification | R2-25xxxxx | Nokia(Srinivasan) |  | V017 | Agreed |

[Description]:

The field description for ltm-ConfigurationSCG says it cannot be included within LTM-Config. However LTM-Config IE does not have this field name within it. In our view this field is meant to provide LTM config for SCG LTM in case of inter SN SCG LTM. Hence better to reflect this in the field description.

LTM-ConfigNRDC

The IE LTM-ConfigNRDC is used to provide LTM configurations in NR-DC.

LTM-ConfigNRDC information element

-- ASN1START

-- TAG-LTM-CONFIGNRDC-START

LTM-ConfigNRDC-r19 ::=       SEQUENCE {

ltm-ConfigurationSCG-r19                   LTM-Config-r18                                                               OPTIONAL,    -- Need M

ltm-SK-CounterConfigToAddModList-r19       SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfigLTM-r19       OPTIONAL,    -- Need N

ltm-SK-CounterConfigToReleaseList-r19      SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF LTM-NoSecurityChangeId-r19    OPTIONAL,    -- Need N

...

}

-- TAG-LTM-CONFIGNRDC-STOP

-- ASN1STOP

LTM-ConfigNRDC field descriptions

ltm-ConfigurationSCG This field provides LTM configuration for Inter-CU SCG LTM. The network does not configure this field in an RRCReconfiguration message within ConditionalReconfiguration IE.

[Comments]:

[Rapporteur] We already captured in section 5.3.5.18.1 what this field is meant to carry and we should not repeat the same in the field description.

[Huawei] The addition is not needed. The existing sentence is misplaced, it should be part of the field description of ltm-ConfigNRDC in the RRCReconfiguration message.

# V401

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V401 | MOB | 2 | Missing RRC parameter used for LTM ‘CodebookConfig-LTM-r19’ and ‘cqi-Table’ for CSI acquisition | R2-25xxxxx | vivo (Jing Liang) |  | V007 | PropAgree |

[Description]: RRC parameters newly agreed by RAN1 for early CSI acquisition which include ‘CodebookConfig-LTM-r19’ and ‘cqi-Table’ are missed according to higher layers parameters list from RAN1 (LS in R1-2506626).

[Proposed Change]: Introduce two new RRC parameters ‘CodebookConfig-LTM-r19’ and ‘cqi-Table’ under the parent IE LTM-CSI-ReportConfig-r18.

[Comments]:

# H154

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H154 | MOB | 2 | It is unclear which fields are applicable to event triggered reports but not to execution conditions. |  | Huawei (David) |  | V018 | ToDo |

[Description]:

There are multiple fields that are specific to event-triggered reports, as opposed to LTM execution conditions. However, the field description of ltm-EventTriggeredReportContent looks like it is the only field in this case.

Also, the wording is obfuscated.

[Proposed Change]:

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha-r18                                        P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                         SEQUENCE {

eventId-r19                                CHOICE {

eventLTM2-r19                              SEQUENCE {

ltm2-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM3-r19                              SEQUENCE {

ltm3-Offset-r19                            MeasTriggerQuantityOffset,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM4-r19                              SEQUENCE {

ltm4-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM5-r19                              SEQUENCE {

ltm5-Threshold1-r19                        MeasTriggerQuantity,

ltm5-Threshold2-r19                        MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

...

},

eventTriggeredReportConfig                 SEQUENCE {

ltm-CandidateReportConfigList-r19          SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateReportConfig-r19

OPTIONAL, -- Need R

ltm-EventTriggeredReportContent-r19        LTM-EventTriggeredReportContent-r19            OPTIONAL, -- Need R

reportOnLeave-r19                          ENUMERATED {enabled}                           OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19       LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

...

}                                                                                                     OPTIONAL, -- Need R

servingSpecificOffset-r19                       MeasTriggerQuantityOffset                      OPTIONAL, -- Cond onlyLTM3

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900                                    OPTIONAL, -- Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18                               OPTIONAL, -- Need R

ltm-CondebookConfig-r19                        LTM-CodebookConfig-r19                                     OPTIONAL, -- Need R

ltm-cqi-Table-r19                              CQI-Table                                                  OPTIONAL  -- Need R

]]

}

LTM-CSI-ReportConfig field descriptions

eventId Type of LTM event for triggering event-triggered measurement report as specified in TS 38.321 [3].

eventTriggeredReportConfig If this field is included, the LTM-CSI-ReportConfig is for L1 measurement and event-triggered reporting as specified in TS 38.321 [3] clause 5.35. Otherwise, the LTM-CSI-ReportConfig is an execution condition for LTM as specified in TS 38.321 [3] clause 5.36.

hysteresis Hysteresis when evaluating the entering/leaving conditions for an LTM event.

ltm-CandidateReportConfigList List of report configurations for LTM candidate IDs. If the field is absent the UE shall measure all the LTM candidate cells associated to the field ltm-ResourcesForChannelMeasurement.

ltm-CodebookConfig Codebook configuration for LTM CSI report. Network can only set codebookType to typeI-SinglePanel in ltm-CSI-ReportConfig within a LTM-Candidate IE.

ltm-EventTriggeredPeriodicReport This field indicates when an LTM event is triggered, whether the event-triggered measurement report is sent periodically. If the field is absent, the event-triggered measurement report is sent once, as specified in TS 38.321 [3].

ltm-EventTriggeredReportContent This field indicates what to include in a measurement report when an LTM event is triggered.

ltm-ReportConfigType This field specifies how the UE shall report the measurement results for LTM either by gNB-scheduled measurement report or by event-triggered measurement report by MAC CE. The UE shall ignore this field if LTM-CSI-ReportConfig is configured in a LTM-Candidate IE.

ltm-ReportContent This field defines the content of the LTM L1 measurement report. The UE shall ignore this field if the field ltm-ReportConfigType is set to eventTriggered.

ltm-ResourcesForChannelMeasurement, ltm-ResourceForInterferenceMeasurements This field indicates the index of SSB or CSI-RS in the field LTM-CSI-ResourceConfig.

ltm2-Threshold, ltm4-Threshold, ltm5-Threshold1, ltm5-Threshold2 Thresholds defined in the entering/leaving conditions for different LTM events.

ltm3-Offset Offset for the entering/leaving condition for event LTM3. The actual value is field value * 0.5 dB.

reportOnLeave Indicates whether the event-triggered measurement report by MAC CE shall be triggered when leaving condition is satisfied, as specified in TS 38.321 [3].

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4). The UE shall ignore the offset provided by this field in case semiPersistentOnPUSCH is configured.

reportSlotOffsetList, reportSlotOffsetListDCI-0-1, reportSlotOffsetListDCI-0-2 Timing offset Y for semi persistent reporting using PUSCH and aperiodic reporting.

servingSpecificOffset Offset for event condition that is applicable for all the reference signals belonging to serving cell. If the field is absent, the value '0dB' is applied.

[Comments]:

# H153

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H153 | MOB | 2 | A field specific to eventLTM3 should be placed in eventLTM3 |  | Huawei (David) |  | V018 | ToDo |

[Description]:

A field that is specific to eventLTM3 should be defined in eventLTM3.

[Proposed Change]:

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha-r18                                        P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                         SEQUENCE {

eventId-r19                                CHOICE {

eventLTM2-r19                              SEQUENCE {

ltm2-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM3-r19                              SEQUENCE {

ltm3-Offset-r19                            MeasTriggerQuantityOffset,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

servingSpecificOffset-r19                  MeasTriggerQuantityOffset                      OPTIONAL, -- Need S

...

},

eventLTM4-r19                              SEQUENCE {

ltm4-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM5-r19                              SEQUENCE {

ltm5-Threshold1-r19                        MeasTriggerQuantity,

ltm5-Threshold2-r19                        MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

...

},

ltm-CandidateReportConfigList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateReportConfig-r19

OPTIONAL, -- Need R

ltm-EventTriggeredReportContent-r19                LTM-EventTriggeredReportContent-r19            OPTIONAL, -- Need R

reportOnLeave-r19                                  ENUMERATED {enabled}                           OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19               LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900                                    OPTIONAL, -- Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18                               OPTIONAL  -- Need R

]]

}

| Conditional Presence | Explanation |
| --- | --- |
| LTM2 | This field is mandatory in case the eventId is configured as eventLTM2. Otherwise, it is optionally present, need R. |
| notEventLTM2 | This field is not present when the eventId is configured as eventLTM2. Otherwise, it is optionally present, need S. |

[Comments]:

# H157

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H157 | MOB | 2 | The contents of LTM-CodebookConfig-r19 in R2-2507729 (CR endorsed at RAN2 131bis) is not an actual codebook config. |  | Huawei (David) |  | V021 | ToDo |

[Description]:

The endorsed CR includes:

ltm-CondebookConfig-r19                        LTM-CodebookConfig-r19                                     OPTIONAL, -- Need R

LTM-CodebookConfig-r19  ::=                CHOICE {

cri-RSRP                        NULL,

ssb-Index-RSRP                  NULL,

cri-RI-PMI-CQI                  NULL,

...

}

This is not at all a codebook configuration, this is a list of reporting quantities.

According to the RAN1 parameter list, it should be:

CHOICE { twoToThirtyTwoPorts                 CodebookConfig, moreThanThirtyTwoPorts          CodebookConfig-r19

...

}

[Proposed Change]:

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha-r18                                        P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                         SEQUENCE {

eventId-r19                                CHOICE {

eventLTM2-r19                              SEQUENCE {

ltm2-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM3-r19                              SEQUENCE {

ltm3-Offset-r19                            MeasTriggerQuantityOffset,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM4-r19                              SEQUENCE {

ltm4-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM5-r19                              SEQUENCE {

ltm5-Threshold1-r19                        MeasTriggerQuantity,

ltm5-Threshold2-r19                        MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

...

},

ltm-CandidateReportConfigList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateReportConfig-r19

OPTIONAL, -- Need R

ltm-EventTriggeredReportContent-r19                LTM-EventTriggeredReportContent-r19            OPTIONAL, -- Need R

reportOnLeave-r19                                  ENUMERATED {enabled}                           OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19               LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

servingSpecificOffset-r19                       MeasTriggerQuantityOffset                      OPTIONAL, -- Cond onlyLTM3

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900                                    OPTIONAL, -- Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18                               OPTIONAL, -- Need R

ltm-CondebookConfig-r19                        LTM-CodebookConfig-r19                                     OPTIONAL, -- Need R

ltm-cqi-Table-r19                              CQI-Table                                                  OPTIONAL  -- Need R

]]

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

LTM-EventTriggeredPeriodicReport-r19 ::= SEQUENCE {

reportInterval-r19                             ReportInterval-r19,

reportAmount-r19                               ENUMERATED {r2, r4, r8, r16, r32, r64, infinity, spare1},

...

}

LTM-EventTriggeredReportContent-r19 ::=     SEQUENCE {

maxNumberOfReportedBeams-r19                   INTEGER (1..16),

allowReportAnyBeam-r19                         ENUMERATED {enabled}                                       OPTIONAL, -- Need R

reportCurrentBeam-r19                          ENUMERATED {enabled}                                       OPTIONAL, -- Cond LTM2

...

}

LTM-CandidateReportConfig-r19 ::=            SEQUENCE {

ltm-CandidateReportConfigId-r19                LTM-CandidateId-r18,

candidateSpecificOffset-r19                    MeasTriggerQuantityOffset                              OPTIONAL, -- Cond notEventLTM2

...

}

LTM-ReportContent-v1900  ::=                SEQUENCE {

reportQuantity-r19                             ENUMERATED {cri-RSRP, ssb-index-RSRP, cri-RI-PMI-CQI, value1},

...

}

LTM-CodebookConfig-r19  ::=                CHOICE {

twoToThirtyTwoPorts             CodebookConfig,

moreThanThirtyTwoPorts          CodebookConfig-r19,

...

}

-- TAG-LTM-CSI-REPORTCONFIG-STOP

-- ASN1STOP

[Comments]:

# H158

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H158 | MOB | 2 | Sentence is difficult to read |  | Huawei (David) |  | V021 | ToDo |

[Description]:

The endorsed CR includes:

LTM-CSI-ReportConfig field descriptions

eventId Type of LTM event for triggering event-triggered measurement report as specified in TS 38.321 [3].

hysteresis Hysteresis when evaluating the entering/leaving conditions for an LTM event.

ltm-CandidateReportConfigList List of report configurations for LTM candidate IDs. If the field is absent the UE shall measure all the LTM candidate cells associated to the field ltm-ResourcesForChannelMeasurement.

ltm-CodebookConfig Codebook configuration for LTM CSI report. Network can only set codebookType to typeI-SinglePanel in ltm-CSI-ReportConfig within a LTM-Candidate IE.

The sentence is difficult to read (not clear that it refers to a field of CodebookConfig, could also be understood that the field can only be included in ltm-CSI-ReportConfig) and the restrictions is already captured in 38.214 clause 5.2.4a:

--- begin quote ---

The UE shall expect the following configuration provided by ltm-CSI-ReportConfig:

-	For the frequency granularity of the CSI report, the CQI format indicator is Wideband CQI.

-	For the frequency granularity of the CSI report, the PMI format indicator is Wideband PMI.

-	The codebook type is typeI-SinglePanel.

-	The reportQuantity is set to ‘cri-RI-PMI-CQI’.

--- end quote ---

Besides, if we want to capture such a restriction in 38.331, for consistency, the other restrictions should be captured too.

[Proposed Change]:

LTM-CSI-ReportConfig field descriptions

eventId Type of LTM event for triggering event-triggered measurement report as specified in TS 38.321 [3].

hysteresis Hysteresis when evaluating the entering/leaving conditions for an LTM event.

ltm-CandidateReportConfigList List of report configurations for LTM candidate IDs. If the field is absent the UE shall measure all the LTM candidate cells associated to the field ltm-ResourcesForChannelMeasurement.

ltm-CodebookConfig Codebook configuration for LTM CSI report.

[Comments]:

# H156

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H156 | MOB | 2 | ReportInterval-r1900 is not a new version of ReportInterval: it does not replace it anywhere and it does not match with the description of the ReportInterval IE. |  | Huawei (David) |  | V021 | ToDo |

[Description]:

ReportInterval is used in 4 IEs: ReportConfigInterRAT, ReportConfigNR, ReportConfigNR-SL and SL-ReportConfigList, but ReportInterval-r19 is used only in LTM-CSI-ReportConfig, and there is no place where ReportInterval-r19 replaces ReportInterval, as should be the case if this name is used according to A.3.1.2:

When an extension is introduced a suffix is added to the identifier of the concerned ASN.1 field and/or type. A suffix of the form "-rX" is used, with X indicating the release, for ASN.1 fields or types introduced in a later release (i.e. a release later than the original/first release of the protocol) as well as for ASN.1 fields or types for which a revision is introduced in a later release replacing a previous version, e.g., Foo-r9 for the Rel-9 version of the ASN.1 type Foo.

Therefore, this definition should not be added in the ReportInterval IE, the enumerated values should be directly included in LTM-CSI-ReportConfig.

[Proposed Change]:

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha-r18                                        P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                         SEQUENCE {

eventId-r19                                CHOICE {

eventLTM2-r19                              SEQUENCE {

ltm2-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM3-r19                              SEQUENCE {

ltm3-Offset-r19                            MeasTriggerQuantityOffset,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM4-r19                              SEQUENCE {

ltm4-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM5-r19                              SEQUENCE {

ltm5-Threshold1-r19                        MeasTriggerQuantity,

ltm5-Threshold2-r19                        MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

...

},

ltm-CandidateReportConfigList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateReportConfig-r19

OPTIONAL, -- Need R

ltm-EventTriggeredReportContent-r19                LTM-EventTriggeredReportContent-r19            OPTIONAL, -- Need R

reportOnLeave-r19                                  ENUMERATED {enabled}                           OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19               LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

candidateSpecificOffsetS-r19                       MeasTriggerQuantityOffset                      OPTIONAL, -- Cond onlyLTM3

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900                                    OPTIONAL, -- Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18                               OPTIONAL  -- Need R

]]

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

LTM-EventTriggeredPeriodicReport-r19 ::= SEQUENCE {

reportInterval-r19                             ENUMERATED {ms20, ms60, ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240,

ms20480, ms40960, min1, min6, min12, min30},

reportAmount-r19                               ENUMERATED {r2, r4, r8, r16, r32, r64, infinity, spare1},

...

}

LTM-EventTriggeredReportContent-r19 ::=     SEQUENCE {

maxNumberOfReportedBeams-r19                   INTEGER (1..16),

allowReportAnyBeam-r19                         ENUMERATED {enabled}                                       OPTIONAL, -- Need R

reportCurrentBeam-r19                          ENUMERATED {enabled}                                       OPTIONAL, -- Cond LTM2

...

}

LTM-CandidateReportConfig-r19 ::=            SEQUENCE {

ltm-CandidateReportConfigId-r19                LTM-CandidateId-r18,

candidateSpecificOffset-r19                    MeasTriggerQuantityOffset                              OPTIONAL, -- Cond notEventLTM2

...

}

LTM-ReportContent-v1900  ::=                SEQUENCE {

reportQuantity-r19                             ENUMERATED {cri-RSRP, cri-RI-PMI-CQI }           OPTIONAL, -- Need S,

...

}

-- TAG-LTM-CSI-REPORTCONFIG-STOP

-- ASN1STOP

LTM-EventTriggeredReportContent field descriptions

allowReportAnyBeam Indicates whether the UE can report the measurement results for the beams not satisfying the conditions of the events at least for the time duration configured by timeToTrigger as specified in TS 38.321 [3].

maxNumberOfReportedBeams This field defines maximum number of beams whose measurements results can be reported in the event-triggered measurement report by MAC CE as specified in TS 38.321 [3].

reportCurrentBeam Indicates whether the UE is required to report the measurement result of the current beam as specified in TS 38.321 [3].

reportInterval Value ms20 corresponds to 20 ms, value ms60 corresponds to 60 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

#### –	ReportInterval

The IE ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1) when reportType is set to either eventTriggered, periodical, cli-EventTriggered or cli-Periodical. Value ms120 corresponds to 120 ms, value ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

ReportInterval information element

-- ASN1START

-- TAG-REPORTINTERVAL-START

ReportInterval ::=                  ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,

min1,min6, min12, min30 }

-- TAG-REPORTINTERVAL-STOP

-- ASN1STOP

[Comments]:

# H155

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H155 | MOB | 2 | value ssb-index-RSRP should be the default when reportQuantity is not signalled |  | Huawei (David) |  | V021 | ToDo |

[Description]:

In Rel-18, there is no reportQuantity field and the UE will report ssb-Index-RSRP. In Rel-19, if the UE would be configured with a Rel-18 configuration plus the field reportQuantity set to ssb-Index-RSRP, its behaviour will be the same like the behaviour of the Rel-18 UE without this field. Therefore, there is no use to signal this value, it is sufficient to assume that, when the value is absent, ssb-Index-RSRP is assumed.

[Proposed Change]:

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha-r18                                        P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...,

eventTriggered-r19                         SEQUENCE {

eventId-r19                                CHOICE {

eventLTM2-r19                              SEQUENCE {

ltm2-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM3-r19                              SEQUENCE {

ltm3-Offset-r19                            MeasTriggerQuantityOffset,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM4-r19                              SEQUENCE {

ltm4-Threshold-r19                         MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

eventLTM5-r19                              SEQUENCE {

ltm5-Threshold1-r19                        MeasTriggerQuantity,

ltm5-Threshold2-r19                        MeasTriggerQuantity,

hysteresis-r19                             Hysteresis,

timeToTrigger-r19                          TimeToTrigger,

...

},

...

},

ltm-CandidateReportConfigList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateReportConfig-r19

OPTIONAL, -- Need R

ltm-EventTriggeredReportContent-r19                LTM-EventTriggeredReportContent-r19            OPTIONAL, -- Need R

reportOnLeave-r19                                  ENUMERATED {enabled}                           OPTIONAL, -- Need R

ltm-EventTriggeredPeriodicReport-r19               LTM-EventTriggeredPeriodicReport-r19           OPTIONAL, -- Need S

candidateSpecificOffsetS-r19                       MeasTriggerQuantityOffset                      OPTIONAL, -- Cond onlyLTM3

...

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...,

[[

ltm-ReportContent-v1900                        LTM-ReportContent-v1900                                    OPTIONAL, -- Need R

ltm-ResourceForInterferenceMeasurements-r19    LTM-CSI-ResourceConfigId-r18                               OPTIONAL  -- Need R

]]

}

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

LTM-EventTriggeredPeriodicReport-r19 ::= SEQUENCE {

reportInterval-r19                             ReportInterval-v1900,

reportAmount-r19                               ENUMERATED {r2, r4, r8, r16, r32, r64, infinity, spare1},

...

}

LTM-EventTriggeredReportContent-r19 ::=     SEQUENCE {

maxNumberOfReportedBeams-r19                   INTEGER (1..16),

allowReportAnyBeam-r19                         ENUMERATED {enabled}                                       OPTIONAL, -- Need R

reportCurrentBeam-r19                          ENUMERATED {enabled}                                       OPTIONAL, -- Cond LTM2

...

}

LTM-CandidateReportConfig-r19 ::=            SEQUENCE {

ltm-CandidateReportConfigId-r19                LTM-CandidateId-r18,

candidateSpecificOffset-r19                    MeasTriggerQuantityOffset                              OPTIONAL, -- Cond notEventLTM2

...

}

LTM-ReportContent-v1900  ::=                SEQUENCE {

reportQuantity-r19                             ENUMERATED {cri-RSRP, cri-RI-PMI-CQI }           OPTIONAL, -- Need S,

...

}

-- TAG-LTM-CSI-REPORTCONFIG-STOP

-- ASN1STOP

LTM-ReportContent field descriptions

nrOfReportedCells This field defines how many cells are reported within a single L1 measurement report instance.

nrOfReportedRS-PerCell This field defines how many RSs per cell are reported within a single L1 measurement report instance.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and the LTM-CSI-ResourceConfig IE associated to the LTM-CSI-ReportConfig IE includes resources for the current SpCell.

reportQuantity Indicates the report quantity for the CSI report. If the field is absent, the UE shall use ssb-Index-RSRP as report quantity.

[Comments]:

# C162

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C162 | MOB | 1 | Issue on the name and place of the candidateSpecificOffsetS field description |  | Rui (CATT) |  | V005 | Agreed |

[Description]:

Two issue to address,

- it is not suitable to used candidate in the the name of candidateSpecificOffsetS as it is used for serving cell

- the field description should be under LTM-CSI-ReportConfig but not LTM-CandidateReportConfig

[Proposed Change]:

LTM-CSI-ReportConfig field descriptions

eventId Type of LTM event for triggering event-triggered measurement report as specified in TS 38.321 [3].

hysteresis Hysteresis when evaluating the entering/leaving conditions for an LTM event.

ltm-CandidateReportConfigList List of report configurations for LTM candidate IDs. If the field is absent the UE shall measure all the LTM candidate cells associated to the field ltm-ResourcesForChannelMeasurement.

ltm-EventTriggeredPeriodicReport This field indicates when an LTM event is triggered, whether the event-triggered measurement report is sent periodically. If the field is absent, the event-triggered measurement report is sent once, as specified in TS 38.321 [3].

ltm-EventTriggeredReportContent This field indicates what to include in a measurement report when an LTM event is triggered. When this field is absent, the field ltm-ReportConfigType is set to eventTriggered, and the corresponding LTM-CSI-ReportConfigId is part of an LTM-ExecutionConditionList IE, when the associated LTM event is fulfilled, the UE triggers an LTM cell switch procedure instead of an event-triggered measurement report, as specified in TS 38.321 [3].

ltm-ReportConfigType This field specifies how the UE shall report the measurement results for LTM either by gNB-scheduled measurement report or by event-triggered measurement report by MAC CE. The UE shall ignore this field if LTM-CSI-ReportConfig is configured in a LTM-Candidate IE.

ltm-ReportContent This field defines the content of the LTM L1 measurement report. The UE shall ignore this field if the field ltm-ReportConfigType is set to eventTriggered.

ltm-ResourcesForChannelMeasurement, ltm-ResourceForInterferenceMeasurements This field indicates the index of SSB or CSI-RS in the field LTM-CSI-ResourceConfig.

ltm2-Threshold, ltm4-Threshold, ltm5-Threshold1, ltm5-Threshold2 Thresholds defined in the entering/leaving conditions for different LTM events.

ltm3-Offset Offset for the entering/leaving condition for event LTM3. The actual value is field value * 0.5 dB.

reportOnLeave Indicates whether the event-triggered measurement report by MAC CE shall be triggered when leaving condition is satisfied, as specified in TS 38.321 [3].

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4). The UE shall ignore the offset provided by this field in case semiPersistentOnPUSCH is configured.

reportSlotOffsetList, reportSlotOffsetListDCI-0-1, reportSlotOffsetListDCI-0-2 Timing offset Y for semi persistent reporting using PUSCH and aperiodic reporting.

servingSpecificOffset Offset for event condition that is applicable for all the reference signals belonging to serving cell. If the field is absent, the value '0dB' is applied.

LTM-CandidateReportConfig field descriptions

ltm-CandidateReportConfigId LTM candidate cell ID for which the UE is required to measure reference signal and perform LTM event evaluation as specified in TS 38.321 [3].

candidateSpecificOffset Offset for event condition that is applicable for all the reference signals belonging to the candidate cell with the candidate cell ID ltm-CandidateReportConfigId. If the field is absent, the value '0dB' is applied.

candidateSpecificOffsetS Offset for event condition that is applicable for all the reference signals belonging to serving cell. If the field is absent, the value '0dB' is applied.

[Comments]:

# C161

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C161 | MOB | 1 | Ambiguity of the reportQuantity field description on the wording “CSI report” |  | Rui (CATT) |  | V005 | Rejected |

[Description]:

It is not clear whether the wording “CSI report” means early CSI acquization.

[Proposed Change]:

LTM-ReportContent field descriptions

nrOfReportedCells This field defines how many cells are reported within a single L1 measurement report instance.

nrOfReportedRS-PerCell This field defines how many RSs per cell are reported within a single L1 measurement report instance.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and the LTM-CSI-ResourceConfig IE associated to the LTM-CSI-ReportConfig IE includes resources for the current SpCell.

reportQuantity Indicates the report quantity for the CSI report.

[Comments]:

[Rapporteur] In the early CSI acquisition the UE sends a CSI report. There is no problem with current text.

[Huawei]: This is the report quantity "for this IE", so "for the CSI report" is useless, then what remains is "reportQuantiy indicates the report quantity", which is also useless. Suggest deleting this description.

# Z162

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z162 | MOB | 1 | Clarification on the conditional presence of allowReportAnyBeam |  | ZTE (Mengjie Zhang) |  | V009 | Rejected |

[Description]: According to the current conditional presence of allowReportAnyBeam, the IE is is mandatory in case the eventId is configured as eventLTM2. However, if the eventLTM2 is configured as the L1 execution condition for CLTM, the field LTM-EventTriggeredReportContent can be absent, and there is no need to configure the child IE allowReportAnyBeam.

[Proposed Change]: To clarify that allowReportAnyBeam is mandatory in case the eventId is configured as eventLTM2 and the associated ltm-EventTriggeredReportContent is configured.

| Conditional Presence | Explanation |
| --- | --- |
| LTM2 | This field is mandatory in case the eventId is configured as eventLTM2 and the associated ltm-EventTriggeredReportContent is configured. Otherwise, it is optionally present, need R. |
| notEventLTM2 | This field is not present when the eventId is configured as eventLTM2. Otherwise, it is optionally present, need S. |
| onlyLTM3 | This fiels is optionally present, need S, when eventId is configured as eventLTM3. Otherwise, it is absent. |

[Comments]:

[Rapporteur] To me it does not make any sense to configure LTM2 as execution condition for LTM. To which candidate cell the UE will switch if the UE only evaluate the serving cell?

# V402

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V402 | MOB | 2 | Missing RRC parameter used for LTM ‘repetition’ for CSI acquisition |  | vivo (Jing Liang) |  | V007 | Agreed |

[Description]: RRC parameters newly agreed by RAN1 which is ‘repetition’ is missed according to higher layers parameters list from RAN1 (LS in R1-2506626).

[Proposed Change]: Introduce a new RRC parameters ‘repetition’ under the parent IE LTM-NZP-CSI-RS-ResourceSet.

[Comments]:

[Rapporteur] I already plan to implement the new RAN1 parameter in the next version of the RRC CR. No contribution needed

# C163

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C163 | MOB | 1 | Wrong fields in the LTM-CSI-IM-ResourceSet field descriptions |  | Rui (CATT) |  | V005 | Agreed |

[Description]:

There is no field ltm-CSI-IM-ResourceList in LTM-CSI-IM-ResourceSet, it should be ltm-CSI-IM-ResourceSetId and ltm-CandidateId

.

[Proposed Change]:

[Comments]:

# H200

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H200 | XR | 2 | Co-configuration of the fields dsr-ReporthingThresholdList and dsr-ReportNonDelayCriticalData |  | Yinghao Guo (Huawei) |  | V03 | PropAgree |

[Description]: Currently, a conditional presence tag has been added for the field dsr-ReportNonDelayCriticalData-r19 that it could only be configured when the field dsr-ReportingThresList-r19 is configured. It is better to create another field for multipleEntry DSR. Under this field, the dsr-ReportingThresList-r19 is mandatory present, and the field dsr-ReportNonDelayCriticalData-r19 is optionally present. Then, this can imply the relationship of the two fields with the current conditional presence tag.

[Proposed Change]: Create another field for multipleEntry DSR. Under this field, the dsr-ReportingThresList-r19 is mandatory present, and the field dsr-ReportNonDelayCriticalData-r19 is optionally present

[Comments]:

# V051

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V051 | XR | 1 | Restriction on ul-RateQueryConfigList-r19 |  | Vivo(Chenli) |  | V002 | PropRej |

[Description]: When both ul-RateControlConfigList and ul-RateQueryConfigList are configured, the QoS flow configured in the rate query configuration should be the subset of QoS flow configured for the rate control. Otherwise, the QoS flow in the rate control query MAC CE is useless. Thus, it is better to provide such restriction.

[Proposed Change]: In the field description of ul-RateQueryConfigList-r19, it is better to clarify this restriction, e.g.

ul-RateControlConfigList Includes the list of QoS flows for which the UL rate control is supported.

ul-RateQueryConfigList Includes the list of QoS flows for which the UL rate query is supported. The QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control.

[Comments]: [Rapp] Note that the issue was already proposed during the post meeting email discussion. And comment was received that this is not necessary

# Z201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z202 | XR | 1 | Field description of ul-RateQueryProhibitTimer |  | ZTE(Eswar) |  | V010 | Duplicate |

[Description]: The ul-RateQueryProhibitTimer timer is used to control the uplink date rate query procedure, not for control uplink date rate query MAC CE.

[Proposed Change]: In the field description of ul-RateQueryProhibitTimer, delete the “MAC CE”, e.g.

MAC-CellGroupConfig field descriptions

...

ul-RateQueryProhibitTimer This timer is used for uplink date rate query as specified in TS 38.321 [3]. Value s0 means 0 s, s0dot1 means 0.1 s and so on.

[Comments]: [Rapp] Samsung made the same comment in S039.

# S039

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S039 | XR | 1 | Correction on ul-RateQueryProhibitTimer |  | Vinay (Samsung) |  | V07 | PropAgree |

[Description]:

- FD has inconsistent description. It is not correct to use the term “data rate query MAC CE” as MAC spec only associates this timer with a specific “bit rate query” for a QoS flow.

- Typo (“date”)

ul-RateQueryProhibitTimer

This timer is used for uplink date rate query MAC CE as specified in TS 38.321 [3]. Value s0 means 0 s, s0dot1 means 0.1 s and so on.

[Proposed Change]: Corrected description and typo in FD as follows:

ul-RateQueryProhibitTimer This timer is used for uplink bit rate query as specified in TS 38.321 [3]. Value s0 means 0 s, s0dot1 means 0.1 s and so on.

[Comments]: [Rapp] OK, make sense

# C184

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C184 | NES | 1 | The needs of different ssb-ToMeasure configurations for OD-SSB |  | Rui (CATT) |  | V008 | PropReject |

[Description]: Since the value of od-ssb-PositionsInBurst-r19 can be changed in different od-ssb-Config, it seems there is a need to configure different ssb-ToMeasure configurations correspondingly.

[Proposed Change]:

[Comments]:

[Apple] Since od-ssb-PositionsInBurst-r19 is provided by NW and adapted via MAC-CE, we think ssb-ToMeasure is not necessary. Bascially, od-ssb-PositionsInBurst-r19 and ssb-ToMeasure provide similar information but the later (ssb-ToMeasure) is used when specific SSB’s beam bitmap is not known by the UE. Our understanding on the procedure (same procedure between OD-SSB and SSB adaptation):

- The UE first determines SMTC according to periodicity of OD-SSB indicated by MAC-CE.

- Then, within the SMTC window in 1), the UE applies the SNF-offset and HalfFrameIndex indicated by MAC-CE to identify the location of OD-SSB for seriving cell measurement.

[Rapporteur]:  The existing field ssb-ToMeasure is for the legacy always on SSB case and for e.g. mobility. The OD-SSB is used only for serving cell measurements of a configured SCell, and for those measurements the position in burst is defined, only for the OD-SSB occasions. RAN2 has agreed not to optimize any neighbourcell measurements due to OD-SSB hence the legacy parameter should not be changed.

# H126

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H126 | NES | 1 | Serving cell OD-SSB measurements for deactivated SCell | R2-25xxxxx | Huawei (Lili) |  | V012 | PropAgree |

[Description]: According to RAN4 agreements, for deactivated SCell, during FMW, the OD-SSB measurement is based on the OD-SSB periodicity regardless of the configured measCycleSCell. In fact, we think RAN2 should go through all the cases (deactivated SCell, activated SCell) and make sure the RAN2 spec is aligned with RAN4 agreements (regarding OD-SSB measurements, AO-SSB measurements, neighbour cell measurements).

[Proposed Change]:

measCycleSCell

The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on. This field is ignored for on-demand SSB measurements during fast measurement window.

[Comments]:

[Apple] We have same understanding as Huawei on RAN4 agreement. But we think maybe the first sentence has covered it because it has a reference to 38.133.

“The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14].”

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# S027

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S027 | NTN | 1 | Incorrect FD of smtc5list in MO |  | Samsung (Shiyang) |  | v011 | Agreed |

[Description]: The FD of smtc5list in MO is incorrect. smtc5list will not be configured in MO if it is not to indicate an additional SMTC periodicity.

[Proposed Change]:

Update the FD of smtc5list in MO as follows.

smtc4list, smtc5list

Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. smtc5list is configured to indicate an additional SMTC periodicity to the one indicated in smtc1.

[Comments]:

# E025

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E025 | NES | 1 | It is unclear what is the Case1 with respect of the SSBless Scell. | yes | Helka-Liina Määttänen (ER) |  | V019 | ToDo |

[Description]: In IE MeasObjectNR

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB2

ssbFrequency
Indicates the frequency of the SS associated to this MeasObjectNR. For operation with shared spectrum channel access, this field is a k*30 kHz shift from the sync raster where k = 0,1,2, and so on if the reportType within the corresponding ReportConfigNR is set to reportCGI (see TS 38.211 [16], clause 7.4.3.1). Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15], or TS 38.101-5 [75]).

| SSBorAssociatedSSB2 | If the measObject is associated to an SCell with SSB, this field is mandatory present if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If the measObject is associated to an SSB-less SCell, this field is optionally present, Need R, if ssb-ConfigMobility is configured or associatedSSB is configured in at least one cell. If ssb-ConfigMobility is not configured and associatedSSB is not configured for any cell, the field is absent, Need R. |
| --- | --- |

Up until Release 19, a serving cell which is not associated with SSB is “SSB-less SCell”. This term is used in RRC but it is not definined. However, it seems to point to a case where UE obtains timing reference in a defined way as seen from field dercription of the absoluteFrequencySSB. The servingcellMO may or may not be configured for this case.

For Rel-19, Case 1, the scell is not associated with legacy SSB but only OD-SSB and absoluteFrequencySSB should be absent. Presence of servingcell MO is unclear in this case.

If servingcellMO is present, the condition SSBorAssociatedSSB2  need to be updated as there would not be SS associated to this MO.

[Proposed Change]:

Case1 relation to SSB-less needs to be clarified and the condition needs to be updated accordingly.

[Comments]:

[Apple] We agree the issue raised by Ericsson. We can disuss how to resolve it in upcoing RAN2 meeting. Our current view is that the following spec impacts are needed:

[Rapporteur] This requires further discussion in the next meeting.

# N045

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N045 | SONMDT | 1 | Definition of distanceFromReference2 in MeasResults IE |  | Nokia (Mani) |  | V005 | Agreed |

[Description]: The definition of distanceFromReference2 in MeasResults IE is not accurate.

[Proposed Change]:

distanceFromReference2

This field indicates the distance between UE and the moving reference locations of associated neighbour cell if the conditional handover for NTN is based on condEventD2. Each step represents 50m. The actual measured distance shall be rounded down to the nearest lower step value. If the UE is within 50m of the moving reference location, the UE shall report a value of 0. The maximum value is 65535, which indicates a distance equal to or greater than 65535 multiplied by 50m.

[Comments]:

[Samsung] Same comment as above. We think that current text is better. Floor function is applicable for all real numbers. So it should cover the case within 50m also (though it is not practical for NTN)

[Rapporteur]: I have tried to improve the text based on the RIL and the comment from Samsung. If you are not happy with the current version, feel free to bring a contribution so we discuss it further. As of now I consider the RIL agreed given that I have made some changes according to the RIL.

# Z415

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z415 | MIMO | 2 | The definition of additionalOneSlotOffset Dopp is missed |  | ZTE (Wenting Li) |  | v008 | PropReject |

[Description]: Missed Parameter

| additionalOneSlotOffset Dopp | New | For Rel-19 Type-II based on Rel-18 Type-II Doppler: 1-slot offset (per NZP-CSI-RS-Resource Group) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet | {enabled} |
| --- | --- | --- | --- |

[Proposed Change]:

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]],

[[

additionalOneSlotOffset-r19             ENUMERATED{enabled}                             OPTIONAL,    -- Need R

additionalOneSlotOffset-r19             ENUMERATED{enabled}                             OPTIONAL,    -- Need R

additionalSlotOffset-r19         	    INTEGER(0..7)				                     OPTIONAL    -- Need R

]]

}

additionalOneSlotOffset See TS 38.214 [19], clause 5.2.2.3.1. This field is only configured for codebook typeII-Doppler-r19.

[Comments]:

[Ericsson(Lian)] It is implemented in NZP-CSI-RS-ResourceSet as described in L1 parameters (not in NZP-CSI-RS-Resource).

# N121

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N121 | MIMO | 1 | Applicable codebooks for additionalOneSlotOffset |  | Nokia (Andrew) |  | v010 |  |

[Description]: According to TS 38.214 clause 5.2.2.3.1, additionalOneSlotOffset can also be applied to Rel-15 typeI-SinglePanel and Rel-16 typeII codebooks, as shown below. Hence the field description should be updated.

“For an aperiodic NZP-CSI-RS-ResourceSet for channel measurement with Ks>1 resources, linked to a CSI-ReportConfig configured with higher layer parameter valueOfM, with reportQuantity set to 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI' and codebookType set to 'typeI-SinglePanel' or reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-r16', each of the Ks CSI-RS resources can be configured by higher layer parameter additionalOneSlotOffset-r19 with 0, 1, 2, 3, 4, 5, 6 or 7 additional slot offsets relative to the slot offset configured by the higher layer parameter aperiodicTriggeringOffset in the resource set.”

[Proposed Change]:

NZP-CSI-RS-Resource field descriptions

additionalOneSlotOffset See TS 38.214 [19], clause 5.2.2.3.1. This field is only configured for codebook typeI-SinglePanel, typeII-r16, typeI-SinglePanel-r19, typeI-MultiPanel-r19, etypeII-r19 and typeII-FePortSelection-r19.

[Comments]:

# Z409

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z409 | MIMO | 2 | Change the ASN.1 coding for the additionalOneSlotOffsetDoppler-r19 |  | ZTE (Wenting Li) |  | v008 | PropReject |

[Description]: The parameter in the corresponding RAN1 table and the running CR is as follows:

| NZP-CSI-RS-ResourceSet | additionalOneSlotOffset Dopp | For Rel-19 Type-II based on Rel-18 Type-II Doppler: 1-slot offset (per NZP-CSI-RS-Resource Group) relative to the slot offset configured by aperiodicTriggeringOffset in NZP-CSI-RS-ResourceSet | {enabled} |
| --- | --- | --- | --- |

[[ kdopp-r19 SEQUENCE { numberOfResourceGroups-r19 ENUMERATED {n4, n8, n12}, numberOfResourcesPerGroup-r19 ENUMERATED {n2, n3, n4} } OPTIONAL, -- Need R additionalOneSlotOffsetDoppler-r19 CHOICE { resourceGroup4 BIT STRING(SIZE(4)), resourceGroup8 BIT STRING(SIZE(8)), resourceGroup12 BIT STRING(SIZE(12)) } OPTIONAL -- Need R Editor’s note: FFS on how to define additionalOneSlotOffsetDoppler as a list. ]]

However, RAN1’s intention is not to define this parameter per Group, instead this parameter is for all of the groups. Thus there is no need to define this parameter as a list.

[Proposed Change]: No need to define additionalOneSlotOffsetDoppler-r19 as a list, instead it should be define as ENUMERATED {enabled} and applied for all groups.

additionalOneSlotOffsetDoppler-r19      ENUMERATED {enabled} OPTIONAL    -- Need R

[Comments]:

[Ericsson(Lian)] This was hanging for many meetings when RAN2 at the end decided to insert this parameter prer group. It seems better to raise this directly in RAN1 to get a clear direction in RAN2.

[Nokia (Andrew)] We also think this is defined incorrectly right now but are not completely sure whether ZTE’s proposal is the intended way either. Perhaps RAN2 can ask RAN1 to clarify.

# X205

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X205 | NES | 1 | How to configure od-ssb-PositionsInBurst | R2-25xxxxx | Xiaomi (Li Zhao) |  | V015 | PropAgree |

[Description]: according to RAN1 agreement and parameter list, this parameter should be mandatory present in case 1. For case 2, when OD-SSB and AO-SSB having same frequency, this parameter should be absent.

For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), if absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

Agreement (RAN1#120bis) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. • SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases o The case where center frequency of AO-SSB and OD-SSB are different o Case 1 • Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

[Proposed Change]:

| Conditional Presence | Explanation |
| --- | --- |
| ODssbOnly | The field is mandatory present, when absoluteFrequencySSB of the serving cell is absent. It is optionally present otherwise. |
| MACCE | The field is optionally present, Need R, when od-ssb-ActivationStatus is absent. It is absent otherwise. |

od-ssb-PositionsInBurst

Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, UE applies the value ssb-PositionsInBurst provided in ServingCellConfigCommon. This field is absent in case the od-ssb-absoluteFrequency is not configured.

[Comments]:

[Apple] Agree with the intention.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting as follows:

| Conditional Presence | Explanation |
| --- | --- |
| ODssbAOssb | The field is mandatory present, when absoluteFrequencySSB of the serving cell is absent. The field is absent when absoluteFrequencySSB of the serving cell is present and od-ssb-absoluteFrequency is absent. It is optionally present otherwise. |
| MACCE | The field is optionally present, Need R, when od-ssb-ActivationStatus is absent. It is absent otherwise. |

This is since it would be better if we follow the principle that configuration conditions are not split between the field description and the condition. There is no need to change the naming of the condition as it describes both.

[Samsung] Based on the description, it seems that the field can be absent when absoluteFrequencySSB of the serving cell is present and od-ssb-absoluteFrequency is present. Is this the correct understanding? Should not the field be present in this case?

# S035

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S035 | NES | 2 | Paging search space for Adapative POs |  | Anil Agiwal (Samsung) |  | V023 | PropAgree |

[Description]:

PagingSearchSpace can be zero or non zero. For paging search space zero, Ns can be 1 or 2 only.

Inorder to have larger value of Ns for adaptive POs, it should be possible to configure non zero paging search space for adaptive POs irrespective of paging search space configuration for legacy POs. There should be no restriction on legacy POs for adaptive PO configuration. So, Paging search space should be separately configured for paging adaptation.

[Proposed Change]: Include in PDCCH-ConfigCommon

pagingAdaptpagingSearchSpace                   SearchSpaceId                                           OPTIONAL,   -- Need S

[Comments]:

[Apple] Can this issue be concluded by RAN2 or RAN1?

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# C185

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C185 | NES | 1 | The needs of the new field commonSearchSpaceListExt-r19 in PDCCH-ConfigCommon |  | Rui (CATT) |  | V008 | PropReject |

[Description]: It seems there is no need to have this new filed as it is exactly same as the legacy commonSearchSpaceListExt-r18,so the legacy field can be reused.It is suggested to remove commonSearchSpaceListExt-r19

[Proposed Change]:

[Comments]:

[Rapporteur]: There is no explicit agreement regarding this parameter in RAN2, but the intention was to make it possible to configure a separate common search space for Rel-19 NES UEs. So, it should not be removed.

# Z256

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z256 | NTN | 2 | Missing linked SS in paging search space configuration | None | ZTE (Zhihong) |  | v012 | Rejected |

[Description]: SearchSpaceLinkingIdCE is only used to linked two SS, while searchSpaceId for the additional SS for pagingSearchSpace is still needed to be added in PDCCH-ConfigCommon

[Proposed Change]: Add pagingSearchSpaceExt in PDCCH-ConfigCommon per below:

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=              SEQUENCE {

controlResourceSetZero              ControlResourceSetZero                                  OPTIONAL,   -- Cond InitialBWP-Only

commonControlResourceSet            ControlResourceSet                                      OPTIONAL,   -- Need R

searchSpaceZero                     SearchSpaceZero                                         OPTIONAL,   -- Cond InitialBWP-Only

commonSearchSpaceList               SEQUENCE (SIZE(1..4)) OF SearchSpace                    OPTIONAL,   -- Need R

searchSpaceSIB1                     SearchSpaceId                                           OPTIONAL,   -- Need S

searchSpaceOtherSystemInformation   SearchSpaceId                                           OPTIONAL,   -- Need S

pagingSearchSpace                   SearchSpaceId                                           OPTIONAL,   -- Need S

ra-SearchSpace                      SearchSpaceId                                           OPTIONAL,   -- Need S

...,

[[

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                              SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT          SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}                                                                                           OPTIONAL    -- Cond OtherBWP

]],

[partailly omitted]

[[

applyIndicatedTCI-State-r18            ENUMERATED {first, second, both, none}                OPTIONAL,   -- Cond FollowUTCI

commonSearchSpaceListExt-r18           SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1800         OPTIONAL,   -- Need R

searchSpaceMulticastMCCH-r18           SearchSpaceId                                         OPTIONAL,   -- Need R

searchSpaceMulticastMTCH-r18           SearchSpaceId                                         OPTIONAL    -- Need S    ]],

[[

pagingSearchSpaceExt                   SearchSpaceId                                           OPTIONAL,   -- Cond PagingSearchSpace

]]

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

applyIndicatedTCI-State This field indicates, for PDCCH reception in CORESET #0, if UE applies the first, the second, both or none of the "indicated" DL only TCI or joint TCI as specified in TS 38.213 [13], clause 10.1. Value both is not configured if the CORESET is associated with searchSpaceZero for Type 0/0A/2 common search space and can be configured only if sfnSchemePDCCH is configured in the serving cell.

commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0. If the RedCap-specific initial downlink BWP does not contain the entire CORESET#0, the network configures the commonControlResourceSet in the RedCap-specific initial downlink BWP in SIB1 for (e)RedCap such that it does not have to be contained in the bandwidth of CORESET#0.

commonSearchSpaceList, commonSearchSpaceListExt, commonSearchSpaceListExt2 A list of additional common search spaces. If the network configures this field, it uses the SearchSpaceIds other than 0. If the field is included, it replaces any previous list, i.e. all the entries of the list are replaced and each of the SearchSpace entries is considered to be newly created and the conditions and Need codes for setup of the entry apply. If the network includes commonSearchSpaceListExt/commonSearchSpaceListExt2, it includes the same number of entries, and listed in the same order, as in commonSearchSpaceList.

controlResourceSetZero Parameters of the common CORESET#0 which can be used in any common or UE-specific search spaces. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) controlResourceSetZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions defined in TS 38.213 [13], clause 10 are satisfied.

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

firstPDCCH-MonitoringOccasionOfPO Indicates the first PDCCH monitoring occasion of each PO of the PF on this BWP, see TS 38.304 [20]. The field sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS120KHZoneSixteenthT can be applied for SCS 480kHz, corresponding to sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT, sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT and sCS480KHZquarterT-SCS120KHZoneSixteenthT in IE DownlinkConfigCommonSIB respectively.

followUnifiedTCI-State When set to enabled, for PDCCH reception in CORESET #0, the UE applies the "indicated" DL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5.

pagingSearchSpace, pagingSearchSpaceExt ID(s) of the search space(s) for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_INACTIVE while SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), or an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

...

| Conditional Presence | Explanation |
| --- | --- |
| FollowUTCI | The field is absent if the field followUnifiedTCI-State is present or if more than one value for the field coresetPoolIndex is configured in controlResourceSet for the same bandwidthpart. Otherwise, it is optionally present, Need R. |
| InitialBWP-Only | If SIB1 is broadcast the field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon except it is the RedCap-specific initial BWP not including CD-SSB and the entire CORESET#0 in which case the field is absent, Need R; it is absent in other BWPs and when sent in system information. If SIB1 is not broadcast and there is an SSB associated to the cell, the field is optionally present, Need M, in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in ServingCellConfigCommon (still with the same setting for all UEs). In other cases, the field is absent. |
| InitialBWP-Paging | This field is optionally present, Need R, if this BWP is the initialDownlinkBWP or initialDownlinkBWP-RedCap including CD-SSB and the entire CORESET#0, and pei-Config is configured in DownlinkConfigCommonSIB. Otherwise, this field is absent. |
| OtherBWP | This field is optionally present, Need R, if this BWP is not the initialDownlinkBWP and pagingSearchSpace is configured in this BWP. Otherwise this field is absent. |
| PagingSearchSpace | This field is optional present, need R, if pagingSearchSpace is present. Otherwise this field is absent. |

[Comments]:

# Z202

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z203 | XR | 1 | Field description of remaingTimeThresholdRLC-Polling and remainingTimeThresholdRLC-ReTx |  | ZTE(Eswar) |  | V003 | PropAgree |

[Description]: remaingTimeThresholdRLC-Polling is used to decide whether to trigger the remaining time based polling. If triggered, an indication is used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling.

remainingTimeThresholdRLC-ReTx is used to decide whether to trigger remaining time-based retransmission. If triggered, an indication is used by the PDCP entity to notify the RLC entity to trigger remaining time-based retransmission.

[Proposed Change]: Correct the field description of remaingTimeThresholdRLC-Polling and remainingTimeThresholdRLC-ReTx, e.g.

PDCP-Config field descriptions

...

remaingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to decide whether to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds.

remainingTimeThresholdRLC-ReTx Remaining time threshold used by the PDCP entity to decide whether to notify the RLC entity to trigger remaining time-based retransmission as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds.

[Comments]:[Rapp] OK

# V050

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V050 | XR | 1 | Coexistence of remaining time based RLC retransmission and polling | R2-25xxx | Vivo(Chenli) |  | V002 | ToDo |

[Description]: When both remaining time based RLC polling and remaining time based RLC retransmission are configured for the same Tx RLC entity, the threshold for enabling remaining time based RLC retransmission should be set as lower than that for enabling remaining time based RLC polling. Otherwise, the configuration for enabling remaining time based RLC polling will become useless since remaining time based RLC retransmission would have already been triggered before the UE polls the Rx RLC entity to request the STATUS report.

[Proposed Change]: In the field description of remainingTimeThresholdRLC-Polling-r19, it is better to clarify this restriction, e.g.

remaingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds. The network configures remaingTimeThresholdRLC-Polling to be lower than remainingTimeThresholdRLC-ReTx, if it is configured.

remainingTimeThresholdRLC-ReTx Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based retransmission as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds.

[Comments]: [Rapp] It is better to be further discussed with papers 1/ whether the two features of remaining time based polling and remaining time based RLC retransmission can be configured together 2/ if they are configured together, whether any restriction needs to be specified in the field description on the value of which field should be larger than the other one.

# E007

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E007 | LPWUS | 2 | Move lpwus-Config-r19 in SpCellConfig |  | Ericsson (Martin) |  | V002 | Rejected |

[Description]: LP-WUS is supported on PCell and/or PSCell and therefore lpwus-Config-r19 should be put in SpCellConfig.

[Proposed Change]:

PhysicalCellGroupConfig ::=         SEQUENCE {

…

[[

dcp-Config-r16                      SetupRelease { DCP-Config-r16 }                                 OPTIONAL,   -- Need M

…

]]

}

…

CellGroupConfig ::=                        SEQUENCE {

…

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

…

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

…

]],

[[

lpwus-Config-r19                      SetupRelease { LPWUS-Config-r19 }                                 OPTIONAL   -- Need M

]]

}

[Comments]: dcp-Config-r16 should also have been put there.

[WI Rapp]: RAN1 provides the corresponding WUS configuration in connected mode with per-CG granularity.

While these parameters should be included in this IE as: The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

Regarding the restriction on SpCell, it is described in the field description similar as others. Companies are invited to provide views on this issue.

# E043

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E043 | LPWUS | 2 | Exit condition for LP-WUS monitoring in connected mode | R2-25xxxxx | Ericsson (Martin) | See also H050 | V011 | ToDo |

[Description]: There is a possibility that the UE temporarily becomes unreachable when the LP-WUS quality rapidly changes and the gNB has deconfigured LP-WUS based on the measurement reporting of the UE. This can be avoided with an exit condition based on RLM/BFD measurements that the UE anyways has to perform. To use UAI signalling as proposed in H050 increases the signalling and is slower. When the exit condition is triggered and the UE stops using LP-WUS while the gNB continues using LP-WUS the UE remains reachable.

[Proposed Change]: The NW can configure an LP-WUS monitoring exit threshold based on RLM/BFD measurements via dedicated signalling.

[Comments]: Tdoc will be provided with the specification changes needed.

# S057

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S057 | LPWUS | 2 |  |  |  |  | vnnn | Agreed |

[Description]: The value range and location of lpwus-TCI-States-r19 is not aligned with the RAN1 agreement:

Agreement

For the RRC parameter LP-WUS_TCI_state(s)_CONNECTED,

  The parameter is configured per BWP on Pcell/Pscell and the configured CORESET#ID shall be within the set of CORESETs configured for the BWP.

  Value range: Integer (0 .. 15)

[Proposed Change]: The value range of lpwus-TCI-States-r19 should be INTEGER (0..15) and it is configured per BWP so the location of this parameter should be changed under BWP-DownlinkDedicated

[Comments]:

[WI Rapp]: This has been updated in R2-2507007 based on the latest RAN1 RRC parameter.

# C027

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C027 | LPWUS | 2 | Support of Option 1-2 with dual DRX group | R2-25xxxxx | Da Wang (CATT) |  | V012 | Agreed |

[Description]: It was agreed that the lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is smaller than or equal to that for the default DRX group. But this agreement has not been reflected in the running.

[Proposed Change]: R2 should capture the above agreement related to lpwus-PDCCH-MonitoringTimer and dual DRX group in RRC.

[Comments]:

# C030

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C030 | LPWUS | 1 | Configuration of Option 1-1 and Option 1-2 | No | Da Wang (CATT) |  | V012 | Rejected |

[Description]: For LP-WUS in RRC_CONNECTED mode, Option 1-1 and Option 1-2 can’t be configured simultaneously. This can be captured in RRC.

[Proposed Change]:

lpwus-PDCCH-MonitoringTimer Indicates the length of the timer for UE to monitor PDCCH after LP-WUS is detected for LP-WUS operation option 1-2 (see TS 38.321 [3], clause 5.7). LP-WUS operation option 1-1 and option 1-2 can’t be configured simultaneously.

[Comments]:

[WI Rapp]: There is no need to have such description for this parameter. Similar description has been captured for the concerned parameter.

# H055

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H055 | LPWUS | 1 | Replace “option 1-1” and “option 1-2” terminology with description | R2-25xxxxx | Kuang Yiru (Huawei) |  | V003 | Rejected |

[Description]: It’s not clear what option 1-1 and option 1-2 mean in RRC spec. These are already removed from MAC CR. Better to replace them with description.

[Proposed Change]:

- Describe option 1-1 as “LP-WUS operation in CONNECTED without lpwus-PDCCH-MonitoringTimer configured” and option 1-2 as “LP-WUS operation in CONNECTED with lpwus-PDCCH-MonitoringTimer configured”

- There are a few places in the spec where this needs to be changed. If RAN2 agrees to remove this, RAN1 spec(s) need to do the same thing.

[Comments]:

[WI Rapp]: This has been discussed together with MAC open issue and implemented in the latest LP-WUS RRC CR.

# O003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O003 | SBFD, NES | 2 | Applicability of NES related RACH parameters |  | OPPO (Qianxi) |  | V003 | ToDo |

[Description]: NES introduced parameters for additional RACH (addlRACH-Config-Adapt-r19) within RACH-ConfigCommon, it is unclear whether it is applicable to the case when SBFD RACH is configured, for both cases of sbfd-RACH-SingleConfig-r19 and sbfd-RACH-DualConfig-r19.

[Proposed Change]: R2 discuss and conclude the applicability of NES additional RACH (addlRACH-Config-Adapt-r19) within RACH-ConfigCommon, to the case when SBFD RACH is configured, for both cases of sbfd-RACH-SingleConfig-r19 and sbfd-RACH-DualConfig-r19.

[Comments]:

[Rapp] RAN2 has agreed that (in 130 meeting NES session) " RAN2 de-prioritizes the discussion of NES (including all NES scopes) and SBFD co-existence.". 
As SBFD related configuration sbfd-RACH-SingleConfig-r19 , sbfd-RACH-DualConfig-r19 and NES related configuration addlRACH-Config-Adapt-r19 are all optional fields, the network will, according to this agreement, not to configure SBFD and NES at the same time. I will recommend PropReject on this RIL from PoV of SBFD WI. If the proponent thinks still necessary to add specific restrictions, they can contribute to the cross-WI section.

[Rapp] The RIL status is updated to "ToDo" according to the instructions from the general RRC Rapporteur.

# H127

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H127 | NES | 1 | Which RA occasions to use when the configuration is absent in RACH-ConfigDedicated | R2-25xxxxx | Huawei (Lili) |  | V012 | ToDo |

[Description]: In legacy spec, for CFRA, if occasions is absent, the corresponding field in RACH-ConfigCommon applies. With the introduction of additional RA resources, both legacy RA occasions and additional RA occasions can be configured in the RACH-ConfigCommon, it needs to be made clear which RA occasion is used when the field is absent in RACH-ConfigDedicated. Considering RAN2 has agreed that RACH adaptation is not applied for L3 HO command, the simplest way could be clarifying that legacy RA occasions (i.e. RA occasions not configured in addlRACH-Config-Adaptation-r19) are used when the field occasions is absent in RACH-ConfigDedicated.

[Proposed Change]:

occasions

RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon (except the RA occasions configured in addlRACH-Config-Adaptation in RACH-ConfigCommon) in the first active UL BWP.

[Comments]:

[Apple] We agree with this change. From technique-wise, it is not clear whether the UE has received paging short message to activate additional RACH resource upon reception during HO. So, since it is not clear how it works, we prefer to not allow additional RACH during HO.

[Rapporteur]: RAN2 has agreed that RACH adaptation is not applied for L3 HO command, but the usecase for dedicated RACH Config is not limited to L3 HO, or? If not, there is no need to exclude additional RA resources here. This needs further discussion in the next meeting.

[Samsung]: All cases where RACH adaptation is not applied is clearly specified in MAC. No change in RRC is needed.

# C104

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C104 | SBFD | 1 | Concern on ‘for CFRA’ | R2-250xxxx | Jianxiang Li (CATT) |  | V005 | Agreed |

[Description]: If is RACH is triggered by RACH-ConfigDedicated, it may also be CBRA if the preamble index is set to 0b000000. In this case, the ra-OccasionType is also valid. Hence, it is suggested to remove “for CFRA”.

[Proposed Change]: remove ‘for CFRA’.

[Comments]: [ZTE] Same comment as C100

[LGE] In our understanding, it was intentionally added to consider the case that no CFRA resource is configured in RACH-ConfigDedicated IE; in this case, the UE initiates the CBRA procedure and RO type should not be indicated by this field as ZTE mentioned. However, given that the initial RO type selection procedure in MAC is now sepately described for the CFRA cases and CBRA cases, it would be clear in MAC spec enough even though ‘for CFRA’ is removed. In this sense, we are okay to remove the ‘for CFRA’ or ZTE’s alternative (2) to specify the fallback case from CFRA to CBRA.

[Rapp]: Same as for C100.

# C079

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C079 | AIML | 1 | srb-Identity-v19xy |  | Tangxun |  | V003 | Agreed |

[Description]: “srb-Identity-v19xy” is missing in the field description box

[Proposed Change]: update the field description as below:

srb-Identity, srb-Identity-v1700, srb-Identity-v1800, srb-Identity-v19xy Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only. Value 4 is applicable for SRB4 only. Value 5 is applicable for SRB5 only. Value x is applicable for SRBx only. If srb-Identity-v1700, srb-Identity-v1800 or srb-Identity-v19xy is received for an SRB, the UE shall ignore srb-Identity (i.e. without suffix) for this SRB.

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# L207

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L207 | NES | 1 | Optional configuration of msg1-FDM-r19 in RandomAccessAdaptConfig |  | LGE(Hanseul Hong) |  | V016 | PropReject |

[Description]: In RAN1#120, it is agreed to allow the separated configuration of number of Msg1-FDM, but it was remained as FFS for the case when there is no configuration of Msg1-FDM for additional PRACH resources.

Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

Regarding the FFS point for the case when there is no separated configuration of Msg1-FDM, there were several view on in RAN1#120bis meeting [R1-2503125], but it was not formally agree to mandatorily configure the Msg1-FDM for additional PRACH resources.

## Discussion point 3.1.3 (Msg1-FDM) Please provide your view on the FFS related to the Msg1-FDM. Company; Comment. ZTE, Sanechips; We are open to discuss the FFS related to the Msg1-FDM. Setting a default value equal to 1 is much simpler for us.. CATT; We prefer the Msg1-FDM always configure for the additional RACH resources which is same as legacy configuration.. Sharp; Either option is OK though we slightly prefer to use the same value with legacy RO configuration as a default value to have SSB mapping alignment.. Apple; Using the same value with legacy seems more reasonable.. Fujitsu; We think either option is workable, and we slightly prefer using the same value for legacy RO configuration.. LG; Use the corresponding value from legacy RO configuration when there is no configuration of Msg1-FDM for additional PRACH resources.. CEWiT; We think both options are workable, and we slightly prefer using the default value..

Therefore, separated msg1-FDM-r19 should be optionally configured, and further discuss the default value for the case when there is no separated Msg1-FDM, e.g., reuse the same value with legacy RO configuration.

[Proposed Change]: Make msg1-FrequencyStart-r19 in RandomAccessAdaptConfig as an optional field, with Need R.

RandomAccessAdaptConfig-r19 ::=    SEQUENCE {

prach-ConfigurationIndex-r19            INTEGER (0..255),

msg1-FDM-r19                           ENUMERATED {one, two, four, eight}, 												   OPTIONAL,   -- Need R

msg1-FrequencyStart-r19                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

prach-SubsetMask-Index-Adaptation-r19   ENUMERATED {one, two, three, four},

ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                                 OPTIONAL,   -- Need M

validityDurationForAddlRACH-Adaptation-r19                       ENUMERATED {n2, n4, n8, n16},

valueKforAssociationPatternPeriodsForPRACH-subsetMask-r19        ENUMERATED {n2, n4, n8, n16}                     OPTIONAL,   -- Need S

...

}

[Comments]:

[Apple] Since RAN1 didn’t finally agree the UE behavior when it is absent, we think it is sufficient to make it mandatory in RAN2. And it is just 2 bit, which is a small signaling overhead optimization.

[Rapporteur] The proposed change seems reasonable, but RAN2 should wait for an updated higher layer parameter list from RAN1 first.

# L208

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L208 | NES | 1 | Optional configuration of msg1-FrequencyStart-r19 in RandomAccessAdaptConfig |  | LGE (Hanseul Hong) |  | V016 | PropReject |

[Description]: According to RAN1#119, it is agreed that msg1-FrequencyStart can be separately configured for additional PRACH resources, but it is does not mean that separated msg1-FrequencyStart is always needed for the additional PRACH resource.

Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

Therefore, it would be better to optionally configure msg1-FrequencyStart for additional PRACH resources.

[Proposed Change]: Make msg1-FrequencyStart-r19 in RandomAccessAdaptConfig as an optional field, with Need R.

RandomAccessAdaptConfig-r19 ::=    SEQUENCE {

prach-ConfigurationIndex-r19            INTEGER (0..255),

msg1-FDM-r19                           ENUMERATED {one, two, four, eight},

msg1-FrequencyStart-r19                 INTEGER (0..maxNrofPhysicalResourceBlocks-1), 									   OPTIONAL,   -- Need R

prach-SubsetMask-Index-Adaptation-r19   ENUMERATED {one, two, three, four},

ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                                 OPTIONAL,   -- Need M

validityDurationForAddlRACH-Adaptation-r19                       ENUMERATED {n2, n4, n8, n16},

valueKforAssociationPatternPeriodsForPRACH-subsetMask-r19        ENUMERATED {n2, n4, n8, n16}                     OPTIONAL,   -- Need S

...

}

[Comments]:

[Apple] If it is absent, what is UE behavior? Similar to L207, we prefer to just make it mandatory at the late stage of Rel-19 since there is no RAN1 agreement on the behavior of absence.

[Rapporteur] RAN2 should wait for an updated higher layer parameter list from RAN1 first.

# L209

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L209 | NES | 1 | Optional configuration of prach-SubsetMask-Index-Adaptation-r19 in RandomAccessAdaptConfig |  | LGE (Hanseul Hong) |  | V016 | PropAgree |

[Description]: prach-SubsetMask-Index-Adaptation-r19 field is implemented in order to support the optional signaling of the subset of the additional PRACH resources, as agreed in RAN1#120.

Agreement For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources - The mask is applicable at unit of - Alt 1: PRACH association period - Alt 2: PRACH association pattern period - Alt 3: SFN level - The PRACH association period is determined based on valid additional ROs only. - The mask is applied after valid RO determination and SSB-RO mapping. - Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask. - This is applicable at least for adaptation for DCI 1_0 with P-RNTI - The DCI does not indicate PRACH mask selection - FFS: how the mask is identified - Option 1: The PRACH mask is from a PRACH mask table - Pre-defined table with N=[4 or 8 or 16] rows - The semi-static signalling indicates a PRACH mask index - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

In other words, the network should be able to configure DCI-based adaptation to for all the additional PRACH resources configured in RandomAccessAdaptConfig, without singaling PRACH mask for additional PRACH resources.

In this sense, in clause 8.1 of TS 38.213 v19.0.0, it is specified as prach-SubsetMask-Index-Adaptation ‘can be additionally provided,’ which implies that prach-SubsetMask-Index-Adaptation is not be always provided from the network.

For valid PRACH occasions associated with addl-RACH-Config-Adaptation in RACH-ConfigCommon, the UE can be additionally provided a PRACH mask index, by prach-SubsetMask-Index-Adaptation, that indicates one or more association periods per Kmask association pattern periods according to Table 8.1-0, where Kmask is provided by KforAPPForPRACHsubsetMask. Table 8.1-0: Mapping of mask index to association periods per Kmask association pattern periods Mask Index; Association periods (APs) per Kmask association pattern periods (APPs). 0; First half of APs in Kmask APPs. 1; First quarter of APs in Kmask APPs. 2; First eighth of APs in Kmask APPs. 3; First sixteenth of APs in Kmask APPs. Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI or a C-RNTI [5, TS 38.212]. For indication by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI.

On the other hand, according to current ASN.1 signaling of RandomAccessAdaptConfig, prach-SubsetMask-Index-Adaptation-r19 defined as mandatory field, which is not aligned with RAN1’s inctention in the above agreement and causes unnecessary network restriction. Therefore, it should be clarified that prach-SubsetMask-Index-Adaptation-r19 can be optionally provided.

[Proposed Change]: Make prach-SubsetMask-Index-Adaptation-r19 in RandomAccessAdaptConfig as an optional field, with Need R.

RandomAccessAdaptConfig-r19 ::=    SEQUENCE {

prach-ConfigurationIndex-r19            INTEGER (0..255),

msg1-FDM-r19                           ENUMERATED {one, two, four, eight},

msg1-FrequencyStart-r19                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

prach-SubsetMask-Index-Adaptation-r19   ENUMERATED {one, two, three, four},                   								   OPTIONAL,   -- Need R

ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                                 OPTIONAL,   -- Need M

validityDurationForAddlRACH-Adaptation-r19                       ENUMERATED {n2, n4, n8, n16},

valueKforAssociationPatternPeriodsForPRACH-subsetMask-r19        ENUMERATED {n2, n4, n8, n16}                     OPTIONAL,   -- Need S

...

}

[Comments]:

[Apple] Same understanding as LG and we should change it to “optional – need R” to align with 38.213.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# L210

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L210 | NES | 1 | Need code of ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RandomAccessAdaptConfig |  | LGE (Hanseul Hong) |  | V016 | PropAgree |

[Description]: In RAN1#120 and RAN1#120bis meetings, it is agreed to allow the separated configuration of number of SSB per RO and the number of CB preambles per SSB (i.e., ssb-perRACH-OccasionAndCB-PreamblesPerSSB IE), but it does not agree to mandatorily configure the separated configuration of ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

RAN#120 Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported RAN#120bis Agreement Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported - CB-PreamblesPerSSB

In addition, according to the current running MAC CR, it specifies that the UE selects the preamble corresponding to selected SSB based on the separated ssb-perRACH-OccasionAndCB-PreamblesPerSSB in addlRACH-Config-Adapt, if it is configured.

NOTE X: If an RO selected for preamble transmission is configured by addlRACH-Config-Adapt and ssb-perRACH-OccasionAndCB-PreamblesPerSSB is configured in addlRACH-Config-Adapt, UE selects preamble corresponding to selected SSB amongst the preambles determined according to ssb-perRACH-OccasionAndCB-PreamblesPerSSB in addlRACH-Config-Adapt.

In other words, ssb-perRACH-OccasionAndCB-PreamblesPerSSB may not be always configured in addlRACH-Config-Adapt, and if the separated ssb-perRACH-OccasionAndCB-PreamblesPerSSB is not configured, legacy ssb-perRACH-OccasionAndCB-PreamblesPerSSB may be used to determine preamble range for the selected SSB.

However, if Need M code is used for ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19, when the SIB1 is updated to modify the configuration of additional PRACH resources, ssb-perRACH-OccasionAndCB-PreamblesPerSSB value is maintained as a previous value. Therefore, ssb-perRACH-OccasionAndCB-PreamblesPerSSB should be always separately configured in addlRACH-Config-Adapt once it is provided in any SIB1 in previous value, since the UE will never release the value unless the configuration of additional PRACH resource is released.

Therefore, in order to support the optional configuration of ssb-perRACH-OccasionAndCB-PreamblesPerSSB, the need code should be modified to Need R

[Proposed Change]: Modify the Need code of ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19 to Need R from Need M.

RandomAccessAdaptConfig-r19 ::=    SEQUENCE {

prach-ConfigurationIndex-r19            INTEGER (0..255),

msg1-FDM-r19                           ENUMERATED {one, two, four, eight},

msg1-FrequencyStart-r19                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

prach-SubsetMask-Index-Adaptation-r19   ENUMERATED {one, two, three, four},

ssb-perRACH-OccasionAndCB-PreamblesPerSSB-r19   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                                 OPTIONAL,   -- Need R

validityDurationForAddlRACH-Adaptation-r19                       ENUMERATED {n2, n4, n8, n16},

valueKforAssociationPatternPeriodsForPRACH-subsetMask-r19        ENUMERATED {n2, n4, n8, n16}                     OPTIONAL,   -- Need S

...

}

[Comments]:

[Apple] We are not conviced. Note that legacy ssb-perRACH-OccasionAndCB-PreamblesPerSSB is type of “Optional –Need M”. The latency of SIB1 update is not a good justification because all IEs in SIB have similar issue and that is why we have modification period. We understand the intention of “need M” is to save signaling overhead since this IE has large payload size.

[Rapporteur]: The proposed change will be captured in the rapporteur CR to the next meeting

# N101

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B111 | MOB | 1 | Reference configuration for Inter-SN SCG LTM | R2-25xxxxx | Nokia(Srinivasan) |  | V017 | Agreed |

[Description]:

When Inter SN SCG LTM is configured the LTM config is delivered via MN RRC message. In this case RAN2 agreed that the reference configuration and candidate configurations will include all the parameters across both cell group. However the reference configuration definition still indicates it provides common configuration for cell group. This needs to be clarified.

[Proposed Change]: ( Highlighted in yellow).

–	ReferenceConfiguration

The IE ReferenceConfiguration is used provide a configuration that is common, within the same cell group, to all configured non-complete candidate configurations if included in the LTM Config IE within LTM-Config-NRDC. If the reference configuration is included for LTM-Config-NRDC this IE is used to provide configuration that is common across bothe cell groups, to all configured non-complete candidate configurations.

ReferenceConfiguration information element

-- ASN1START

-- TAG-REFERENCECONFIGURATION-START

ReferenceConfiguration-r18 ::= OCTET STRING (CONTAINING RRCReconfiguration)

-- TAG-REFERENCECONFIGURATION-STOP

-- ASN1STOP

[Comments]:

[Rapporteur] In the last meeting we have just agreed that there is no limitation in how the reference configuration is provided by the UE. On top of this, I would say that current statement is still correct as it clarify that the reference configuration is common to all the LTM candidate configuration within a cell group, which is actually true.

[Huawei] Even in Rel-18, this description is unclear, and which one to use for which candidate configuration should be covered by procedure text. Could be reworded:

The IE ReferenceConfiguration is used provide a configuration that is used for a set of non-complete candidate configurations.

# Z163

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z163 | MOB | 1 | Clarification on CondTriggerConfig field descriptions |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: The fields included in the CondTriggerConfig are applicable to not only conditional reconfiguration triggering condition, but also CLTM triggering condition based on L3 measurements. The CLTM case is missing in multiple field descriptions.

Note: We have defined CLTM in 3.2 Abbreviations, but it seems that the term is not used in other places in the current spec. I guess we can use “CLTM” here to clearly specify the case.

[Proposed Change]: To add “CLTM triggering condition based on L3 measurements” in the field description for related IEs.

CondTriggerConfig field descriptions

a3-Offset Offset value(s) to be used in NR conditional reconfiguration triggering condition or CLTM triggering condition based on L3 measurements for cond event a3. The actual value is field value * 0.5 dB.

a4-Threshold Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition or CLTM triggering condition based on L3 measurements for cond event a4.

a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition or CLTM triggering condition based on L3 measurements for cond event a5. In the same condeventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

condEventId Choice of NR conditional reconfiguration or CLTM event triggered criteria.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for condEventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for condEventD2. Each step represents 50m.

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

nesEvent Indicates the event is a NES-specific CHO event and the event is only considered to be satisfied if indication from lower layers is received indicating the applicability of NES-specific CHO event and the related entry condition(s) is fulfilled. This field can only be configured for condEventA3, condEventA4 or condEventA5. This field cannot be configured for CPAC.

referenceLocation1, referenceLocation2 The referenceLocation1 is associated to serving cell and referenceLocation2 is associated to candidate target cell.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation or CLTM evaluation based on L3 measurements.

[Comments]:

[Rapporteur] I guess that is still good to clarify that this procedure applies also to CLTM, so I am plannig to clarify this.

# Z164

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z164 | MOB | 1 | Clarification on reportType |  | ZTE (Mengjie Zhang) |  | V009 | Agreed |

[Description]: The condTriggerConfig can also be used for CLTM configuration.

Note: We have defined CLTM in 3.2 Abbreviations, but it seems that the term is not used in other places in the current spec. I guess we can use “CLTM” here to clearly specify the case.

[Proposed Change]: To add the CLTM case.

reportType

Type of the configured measurement report. In MR-DC, network does not configure report of type reportCGI using SRB3. The condTriggerConfig is used for CLTM, CHO, CPA or CPC configuration.

[Comments]:

# C164

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C164 | MOB | 1 | Suffix “-r19” should be used instead of “” for the ReportInterval-v19xy |  | Rui (CATT) |  | V005 | PropAgree |

[Description]:

As ReportInterval-v19xy includes all the values in the legacy ReportInterval, Suffix “-r19” should be used instead of “” for the ReportInterval-v19xy

.

[Proposed Change]:

ReportInterval information element

-- ASN1START

-- TAG-REPORTINTERVAL-START

ReportInterval ::=                  ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,

min1,min6, min12, min30 }

ReportInterval-v19xyr19 ::=              ENUMERATED {ms20, ms60, ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,

min1,min6, min12, min30 }

-- TAG-REPORTINTERVAL-STOP

-- ASN1STOP

[Comments]:

[Huawei] ReportInterval is used in 4 IEs: ReportConfigInterRAT, ReportConfigNR, ReportConfigNR-SL and SL-ReportConfigList, but ReportInterval-r19 is used only in LTM-CSI-ReportConfig, and there is no place where ReportInterval-r19 actually replaces ReportInterval, as should be the case if this name is used according to A.3.1.2:

When an extension is introduced a suffix is added to the identifier of the concerned ASN.1 field and/or type. A suffix of the form "-rX" is used, with X indicating the release, for ASN.1 fields or types introduced in a later release (i.e. a release later than the original/first release of the protocol) as well as for ASN.1 fields or types for which a revision is introduced in a later release replacing a previous version, e.g., Foo-r9 for the Rel-9 version of the ASN.1 type Foo.

Therefore, this definition should not be added in the ReportInterval IE, the enumerated values should be directly included in LTM-CSI-ReportConfig.

# S051

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S051 | AIML | 1 | RLC-Config and MAC-LogicalChannelConfig has to be mandatory present while configuring SRBx |  | Aby K Abraham |  | v026 | Agreed |

[Description]:

There is no default configuration defined for SRBx. So when the gNB is configuring SRBx, RLC-Config and MACLogicalChannelConfig need to be provided.

[Proposed Change]: Update the explanation of conditional presence for LCH-Setup as below:

RLC-BearerConfig information element

-- ASN1START

-- TAG-RLC-BEARERCONFIG-START

RLC-BearerConfig ::=                        SEQUENCE {

logicalChannelIdentity                      LogicalChannelIdentity,

servedRadioBearer                           CHOICE {

srb-Identity                                SRB-Identity,

drb-Identity                                DRB-Identity

}                                                                                               OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                              ENUMERATED {true}                                   OPTIONAL,   -- Need N

rlc-Config                                  RLC-Config                                          OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig                    LogicalChannelConfig                                OPTIONAL,   -- Cond LCH-Setup

...,

[[

rlc-Config-v1610                            RLC-Config-v1610                                    OPTIONAL    -- Need R

]],

[[

rlc-Config-v1700                            RLC-Config-v1700                                    OPTIONAL,   -- Need R

logicalChannelIdentityExt-r17               LogicalChannelIdentityExt-r17                       OPTIONAL,   -- Cond LCH-SetupModMRB

multicastRLC-BearerConfig-r17               MulticastRLC-BearerConfig-r17                       OPTIONAL,   -- Cond LCH-SetupOnlyMRB

servedRadioBearerSRB4-r17                   SRB-Identity-v1700                                  OPTIONAL    -- Cond LCH-SetupOnlySRB4

]],

[[

servedRadioBearerSRB5-r18                   SRB-Identity-v1800                                  OPTIONAL    -- Cond LCH-SetupOnlySRB5

]],

[[

servedRadioBearerSRBx-r19                   SRB-Identity-v19xy                                  OPTIONAL    -- Cond LCH-SetupOnlySRBx

]]

}

MulticastRLC-BearerConfig-r17 ::=           SEQUENCE {

servedMBS-RadioBearer-r17                   MRB-Identity-r17,

isPTM-Entity-r17                            ENUMERATED {true}                                   OPTIONAL    -- Need S

}

LogicalChannelIdentityExt-r17 ::=           INTEGER (320..65855)

-- TAG-RLC-BEARERCONFIG-STOP

-- ASN1STOP

RLC-BearerConfig field descriptions

isPTM-Entity If configured, indicates that the RLC entity is used for PTM reception. When the field is absent the RLC entity is used for PTP transmission/reception.

logicalChannelIdentity ID used commonly for the MAC logical channel and for the RLC bearer.

logicalChannelIdentityExt Extended logical channel ID used commonly for the MAC logical channel and for the RLC bearer for PTM reception. If this field is configured, the UE shall ignore logicalChannelIdentity.

reestablishRLC Indicates that RLC should be re-established. Network sets this to true at least whenever the security key used for the radio bearer associated with this RLC entity changes. For SRB2, multicast MRBs and DRBs, unless full configuration is used, it is also set to true during the resumption of the RRC connection or the first reconfiguration after reestablishment. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment, the network does not set this field to true. The network does not include this field if servedRadioBearer is set to drb-Identity and the RLC-BearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE. For DRBs, network doesn't include this field if the RLC-BearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE. Network doesn't include this field if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE which is received within a MCG RRCReconfiguration message via SRB1.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters. RLC mode reconfiguration can only be performed by DRB/multicast MRB release/addition or full configuration. The network may configure rlc-Config-v1610 only when rlc-Config (without suffix) is set to am.

servedMBS-RadioBearer Associates the RLC Bearer with a multicast MRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedMBS-RadioBearer.

servedRadioBearer, servedRadioBearerSRB4, servedRadioBearerSRB5, servedRadioBearerSRBx Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling restrictions (moreThanOneRLC in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so.

| Conditional Presence | Explanation |
| --- | --- |

| Conditional Presence | Explanation |
| --- | --- |
| LCH-Setup | This field is mandatory present upon creation of a new logical channel for a DRB or a multicast MRB or SRB4 or SRB5 or SRBx. This field is optionally present, Need S, upon creation of a new logical channel for an SRB except SRB4 and SRB5. It is optionally present, Need M, otherwise. |
| LCH-SetupModMRB | This field is optionally present upon creation of a new logical channel for PTM reception for a multicast MRB. If this field is included upon creation of a new logical channel for PTM reception for a multicast MRB, it shall be present when modifying this logical channel. The field is absent for logical channels configured for an SRB and a DRB. |
| LCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a DRB or an SRB (servedRadioBearer). It is absent, Need M otherwise. |
| LCH-SetupOnlyMRB | This field is mandatory present upon creation of a new logical channel for a multicast MRB and upon modification of MRB-Identity of the served MRB. It is absent, Need M otherwise. |
| LCH-SetupOnlySRB4 | This field is mandatory present upon creation of a new logical channel for SRB4 (servedRadioBearerSRB4). It is absent, Need M otherwise. |
| LCH-SetupOnlySRB5 | This field is mandatory present upon creation of a new logical channel for SRB5 (servedRadioBearerSRB5). It is absent, Need M otherwise. |
| LCH-SetupOnlySRBx | This field is mandatory present upon creation of a new logical channel for SRBx (servedRadioBearerSRBx). It is absent, Need M otherwise. |

[Comments]:

[WI CR rapporteur-v031]: We changed the status from “ToDo” to “PropAgree”.

# E012

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E012 | NTN | 2 | New RAN1 parameter on DL CE |  | Ericsson (Ignacio) |  | v001 | Agreed |

[Description]: RAN1 has updated its higher layer parameters list in . A new parameter for DL CE has been added: searchSpaceLinkingId-r19. Provided there exist an old parameter with the same name but different functionality, we suggest renaming it to searchSpaceLinkingId-CE-r19. Unlike RAN1’s proposal, this parameter should be included within SearchSpace IE within a new SearchSpaceExt-v19.

[Proposed Change]: Add the new RAN1 parameter with the following TP:

[Comments]:

searchSpaceLinkingId-CE-r19                  INTEGER (0.. maxNrofSearchSpacesLinks-1-r17)         OPTIONAL    -- Need R

SearchSpaceLinkingIdCE This parameter is used to link two search spaces of same type in the same BWP. If two search spaces have the same searchSpaceLinkingIdCE-r19 UE assumes these two search spaces are linked to PDCCH repetition. When PDCCH repetition is monitored in two linked search space (SS) sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets. The two linked SS sets have the same CSS set type other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero. The two linked SS sets have the same DCI formats to monitor. For intra-slot PDCCH repetition: The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration. The starting symbol of monitoring occasion of the second SS is located right after the ending symbol of monitoring occasion of the first SS. For linking monitoring occasions across the two SS sets that exist in the same slot: The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set.

[vivo] The field naming in the FD part should be SearchSpaceLinkingId-CE.

[Qualcomm] We prefer to discuss wording or if there is other way to link the two SS so please change to ToDO.

# Z356

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z356 | SBFD | 2 | MIMOParam-v19xy |  | Yu Pan (ZTE) |  | V006 | Agreed |

[Description]: Q1: RAN1’s parameter list only indicates the separate p0AlphaSet in single TRP scenario. And RAN1#119 has conclusion that ‘there is no RAN1 consensus to support mTRP scrnario for SBFD in Rel-19’ . So why to capture such list via MIMOParam-v19xy in SBFD WI?  Q2: For MIMOParam-v19xy, the son IE additionalUplinkPowerControlToAddModList-r19 does not have field description.

[Proposed Change]: Remove the MIMOParam-v19xy; If not, to add field description.

[Comments]:

[Rapp] added FD for additionalUplinkPowerControlToAddModList-r19. It is understood that the addition of MIMOParam-v19xy shall have no implication on single TRP vs mTRP.

# L703

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L703 | SBFD | 1 | ToReleaseList for additionalUplinkPowerControl |  | LGE (Hanseul Hong) |  | V008 | Agreed |

[Description]: In MIMOParam-v19xy, two paraemeters are defined for separated UL power control parameters in SBFD symbol.

MIMOParam-v19xy[RIL]:Z356, SBFD ::=                            SEQUENCE {

additionalUplinkPowerControlToAddModList-r19   SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlExt-v19xy  OPTIONAL, -- Need N

additionalUplinkPowerControlToReleaseList-r19  SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlId-r17     OPTIONAL, -- Need N

...

}

Note that Uplink-powerControlExt-v19xy does not contain separated Uplink Power Control ID field.

Uplink-powerControlExt-v19xy  ::=    SEQUENCE {

p0AlphaSetforPUSCH-SBFD-r19          P0AlphaSet-r17                                                       OPTIONAL, -- Need R

p0AlphaSetforPUCCH-SBFD-r19          P0AlphaSet-r17                                                       OPTIONAL, -- Need R

p0AlphaSetforSRS-SBFD-r19            P0AlphaSet-r17                                                       OPTIONAL  -- Need R

}

However, there are separated additionalUplinkPowerControlToReleaseList for additional UL power control indicating only the Uplink-powerControlId-r17, even though there is no Uplink Power Control ID for separated UL power control in SBFD symbol.

[Proposed Change]: There are two alternatives and Alt 1 seems more straightforward:

Alt 1) remove the release list for the separated UL power control in SBFD symbol, and follow the existing uplink-PowerControlToReleaseList-r17 defined in MIMOParam-r17

MIMOParam-v19xy[RIL]:Z356, SBFD ::=                            SEQUENCE {

additionalUplinkPowerControlToAddModList-r19   SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlExt-v19xy  OPTIONAL, -- Need N

...

}

Alt 2) define separated Uplink power control ID in Uplink-powerControlExt-v19xy to separately release the additional UL control prarameter in SBFD symbol.

Uplink-powerControlExt-v19xy  ::=    SEQUENCE {

ul-powercontrolId-r17                  Uplink-powerControlId-r17,

p0AlphaSetforPUSCH-SBFD-r19          P0AlphaSet-r17                                                       OPTIONAL, -- Need R

p0AlphaSetforPUCCH-SBFD-r19          P0AlphaSet-r17                                                       OPTIONAL, -- Need R

p0AlphaSetforSRS-SBFD-r19            P0AlphaSet-r17                                                       OPTIONAL  -- Need R

}

[Comments]:

[Rapp] Agree with the intention. To implement Alt 2) which is aligned with legacy implementation.

# E024

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E024 | NES, GEN | 1 | It is unclear what is the Case1 with respect of the SSBless Scell. | yes | Helka-Liina Määttänen (ER) |  | V019 | ToDo |

[Description]: In IE ServingCellConfig

servingCellMO

measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. If the serving cell is associated with SSB, the following relationship applies between the corresponding MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon/ServingCellConfigCommonSIB of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList.

If the serving cell is not associated with SSB (i.e. SSB-less SCell), the carrier frequency indicated by ssbFrequency of the corresponding MeasObjectNR, if configured, is within the frequency range indicated by any entry of the scs-SpecificCarrierList.

Up until Release 19, a serving cell which is not associated with SSB is “SSB-less SCell”. This term is used in RRC but it is not definined. However, it seems to point to a case where UE obtains timing reference in a defined way as seen from field dercription of the absoluteFrequencySSB. The servingcellMO may or may not be configured for this case.

For Rel-19, Case 1, the scell is not associated with legacy SSB but only OD-SSB and absoluteFrequencySSB should be absent. Presence of servingcell MO is unclear in this case.

[Proposed Change]:

field descriptions need to be updated for servingCellMO but needs RAN2 conclusion first.

Also definition for “SSB-less SCell” is needed but it it is unclear where this is discussed, hence GEN added.

[Comments]:

[Apple] We agree the issue raised by Ericsson. We can disuss how to resolve it in upcoing RAN2 meeting. Our current view is that the following spec impacts are needed:

We also think it is necessary to clarify the following aspects:

- “SSB-less SCell” need to be updated to: the serving cell is associated with neither AO-SSB nor OD-SSB.

- Clarify that ServingCellMO is mandatory present for OD-SSB case 1 (i.e. ssbFrequency is configured, and its value is the same as either the absoluteFrequencySSB or od-ssb-absoluteFrequency-r19 in SCellConfig).

[Rapporteur] This requires further discussion in the next meeting.

# J005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J005 | NES | 1 | Configuration limitation for servingCellMO-OD |  | Sharp (LIU Lei) |  | V020 | ToDo |

[Description]: For servingCellMO, there are some configuration limitations in the field description. The similar configuration limitation should be added for servingCellMO-OD.

[Proposed Change]: Considering csi-rs measurement for OD-SSB cell is still unclear, only add ssb frequency related part as below:

servingCellMO-OD

measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell instead of servingCellMO in IE ServingCellConfig. The following relationship applies between the corresponding MeasObjectNR and od-ssb of the serving cell: if ssbFrequency is configured, its value is the same as the od-ssb-absoluteFrequency.

[Comments]:

[Rapporteur] This requires further discussion in the next meeting.

# O006

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O006 | NES | 2 | How to handle the SSB-less case | R2-25xxxxx | OPPO (Qianxi) |  | V004 | ToDo |

[Description]: “This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell.” It is not clear how to handle the case where absoluteFrequencySSB is not provided, i.e., SSB-less case.

[Proposed Change]: Extend the condition of servingCellMO-OD as “This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than ssbFrequency of the servingCellMO” to cover SSB-less SCell.

[Comments]:

[Ericsson] agree, please see also E023, E024

[Apple] Agree with the proposed change on presence condition of servingCellMO-OD, which can cover the case that AO-SSB and OD-SSB are in different frequency. However, we want to clarify it is not intended to cover SSB-less case. We think SSB-less SCell case need to update to “the serving cell is associated with neither AO-SSB nor OD-SSB” (i.e. neither absoluteFrequencySSB nor od-ssb-absoluteFrequency-r19 in SCellConfig is configured).

[Rapporteur] This requires further discussion in the next meeting. Discussion is needed regarding the assumption of servingCellMO-OD and servingCellMO for Case1. In Case 2b, servingCellMO-OD is used for serving cell measurements based on OD-SSB. The intra neighbor cell and all CSI-RS measurements would be based on servingCellMO (at least in rapporteur’s understanding) Companies may have different views on CSI-RS measuremeents. For Case1, discussion is needed for all cases since companies have different undertanding on what Case1 is wrt SSB-less.

# C007

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C007 | NTN | 2 | The introduced SIBxx need to be added in the SI-SchedulingInfo | N | CATT (Da Wang) |  | v008 | Agreed |

[Description]: The introduced SIBxx need to be added in the SI-SchedulingInfo

[Proposed Change]:

SIB-TypeInfo-v1700 ::=              SEQUENCE {

sibType-r17                         CHOICE {

type1-r17                           ENUMERATED {sibType15, sibType16, sibType17, sibType18, sibType19, sibType20, sibType21,

sibType22-v1800, sibType23-v1800 ,sibType24-v1800, sibType25-v1800,

sibType17bis-v1820, sibTypexx-v1900, spare3, spare2, spare1,...},

type2-r17                           SEQUENCE {

posSibType-r17                      ENUMERATED {posSibType1-9, posSibType1-10, posSibType2-24, posSibType2-25,

posSibType6-4, posSibType6-5, posSibType6-6, posSibType2-17a-v1770,

posSibType2-18a-v1770, posSibType2-20a-v1770, posSibType1-11-v1800,

posSibType1-12-v1800, posSibType2-26-v1800, posSibType2-27-v1800,

posSibType6-7-v1800, posSibType7-1-v1800,...,

posSibType7-2-v1800, posSibType7-3-v1800, posSibType7-4-v1800},

encrypted-r17                       ENUMERATED { true }                                     OPTIONAL, -- Need R

gnss-id-r17                         GNSS-ID-r16                                             OPTIONAL, -- Need R

sbas-id-r17                         SBAS-ID-r16                                             OPTIONAL  -- Cond GNSS-ID-SBAS

}

},

valueTag-r17                            INTEGER (0..31)                                             OPTIONAL, -- Cond NonPosSIB

areaScope-r17                           ENUMERATED {true}                                           OPTIONAL  -- Need S

}

[Comments]:

# S015

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S015 | MIMO | 1 | FD of fourPortSRS-3Tx is in wrong place |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: fourPortSRS-3Tx is under SRS-ResourceSet, so the FD should be moved to the table for FD of SRS-ResourceSet, currently it is in the table for FD of SRS-Resource

[Proposed Change]:

Move the FD of fourPortSRS-3Tx into the table for SRS-ResourceSet field descriptions

[Comments]:

# Z410

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z410 | MIMO | 2 | Missed Reference for the FD of the srs-PortGrouping |  | ZTE (Wenting Li) |  | v008 | PropAgree |

[Description]: Missed Reference

[Proposed Change]:

srs-PortGrouping

If configured, it indicates that SRS port grouping is enabled. This field can be configured only if reportQuantity is set to cri-RI-CQI and the usage of the SRS resource set is set to antennaSwitching. (see TS 38.214 [19], clause 5.2.2.5).

[Comments

# S016

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S016 | MIMO | 1 | startingBitOfFormat2-3-r19xy should be startingBitOfFormat2-3-r19 |  | Samsung (Shiyang) |  | V002 | PropAgree |

[Description]: startingBitOfFormat2-3-r19xy should be startingBitOfFormat2-3-r19

[Proposed Change]:

Remove “xy” in the suffix of startingBitOfFormat2-3-r19xy, also remove in the FD

startingBitOfFormat2-3, startingBitOfFormat2-3-r19

The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands. The value 1 of the field corresponds to the first/left most bit of format2-3. The value 2 of the field corresponds to the second bit format2-3, and so on (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11.4). The network does not configure both startingBitOfFormat2-3 and startingBitOfFormat2-3-r19 simultaneously.

[Comments]:

# Z257

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z257 | NTN | 2 | Add conditional presence for offset and pci list in SSB-MTC5 | None | ZTE (Zhihong) |  | v012 | Rejected |

[Description]: Offset and pci-list is only present for SSB-MTC5 in SIB2 while it is absent for SSB-MTC included in MO, it shall be clear in the IE description.

[Proposed Change]: Add conditions for presence of pci-list and offset is optional presented when SSB-MTC5 is included in SIB2. SIB4 can be added if RAN2 agrees to extend the configuration for inter-frequency case.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

SSB-MTC2 ::=                        SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need M

periodicity                         ENUMERATED {sf5, sf10, sf20, sf40, sf80, spare3, spare2, spare1}

}

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

SSB-MTC5-r19 ::=             SEQUENCE {

pci-List-r19                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Cond SIB2

periodicity-r19              ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}             OPTIONAL,  -- Need M

offset-r19                   INTEGER (0..159)                                                               OPTIONAL   -- Cond SIB2

[partially omitted]

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

...

| Conditional Presence | Explanation |
| --- | --- |
| SIB2 | This field is optional present, need M, if it is included in SIB2 . Otherwise this field is absent. |

[Comments]:

[Rapp] In our understanding, RAN2 has agreed delta signaling between smtc4list and smtc5list only for SIB2. In the MO, the network already knows the UE capabilities, so it does not need to include both smtc4list and smtc5list. If the UE supports 2 periodicities and the network needs to configure 2 periodicities, it can simply include a full SMTC If companies are alright with this design, then this change would not be necessary.

# N122

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N122 | MIMO | 2 | Conditional presence and field description of pathlossOffset |  | Nokia (Andrew) |  | v011 |  |

[Description]: Currently the field description for pathlossOffset is the same under TCI-State and TCI-UL-State, i.e:

pathlossOffset

Indicates the pathloss offset applied to the UL only TCI or joint TCI state. Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

As identified in C252, C253, the description is not very precise because UL-only TCI states and joint TCI states are used in different scenarios. However, instead of the changes proposed in C252, C253, which require us to use different field descriptions for the same parameter, we think it is still possible to use a common field description (although modified from the existing one), while restricting the joint TCI case based on the conditional presence tag “JointTCI” (which is already used for the field ul-powerControl-r17).

| Conditional Presence | Explanation |
| --- | --- |
| JointTCI | This field is optionally present, Need R, if this serving cell is configured with unifiedTCI-StateType set to 'joint'. It is absent, Need R, otherwise. |

Furthermore, it would be good to clarify that pathlossOffset is applicable only to TCI states of UL-only TRPs, i.e. so it is not misunderstood that it can be applied to the DL (in case of joint-TCI) or to UL at TRPs where DL is also configured.

[Proposed Change]:

In TCI-State config make pathlossOffset conditional present based on JointTCI:

#### –	TCI-State

The IE TCI-State associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::=                       SEQUENCE {

tci-StateId                         TCI-StateId,

qcl-Type1                           QCL-Info,

qcl-Type2                           QCL-Info                                                    OPTIONAL,   -- Need R

...,

[[

additionalPCI-r17                   AdditionalPCIIndex-r17                                      OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r17          PathlossReferenceRS-Id-r17                                  OPTIONAL,   -- Cond JointTCI1

ul-powerControl-r17                 Uplink-powerControlId-r17                                   OPTIONAL    -- Cond JointTCI

]],

[[

tag-Id-ptr-r18                      ENUMERATED {n0,n1}                                          OPTIONAL    -- Cond 2TA

]],

[[

pathlossOffset-r19                  ENUMERATED {

dB-12, dB-8, dB-4, dB0, dB4, dB8,

dB12, dB16, dB20, dB24, dB28, dB32, dB36,

dB40, dB44, dB48, dB52, dB56, dB60}                 OPTIONAL    -- Cond JointTCI

]]

}

QCL-Info ::=                        SEQUENCE {

cell                                ServCellIndex                                               OPTIONAL,   -- Need R

bwp-Id                              BWP-Id                                                      OPTIONAL, -- Cond CSI-RS-Indicated

referenceSignal                     CHOICE {

csi-rs                              NZP-CSI-RS-ResourceId,

ssb                                 SSB-Index

},

qcl-Type                            ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

In TCI-State and TCI-UL-State config, generalize and clarify the field description of pathlossOffset:

pathlossOffset

Indicates the pathloss offset applied to the TCI state of an UL-only TRP (see TS 38.213 [13], clause 7). Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

[Comments]:

# C252

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C252 | MIMO | 1 | Wrong field description of pathlossOffset-r19 in the IE TCI-State |  | CATT (LeiWang) |  | V003 | PropAgree |

[Description]:

In the unified TCI state framework, the IE TCI-State is only used to configure the TCI state in the joint mode.

[Proposed Change]:

pathlossOffset

Indicates the pathloss offset applied to the joint TCI state. Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

[Comments]:

Nokia (Andrew): We have a different view on how to handle this issue. Refer to N122.

# C253

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C253 | MIMO | 1 | Wrong field description of pathlossOffset-r19 in the IE TCI-UL-State-r17 |  | CATT (LeiWang) |  | V003 | PropAgree |

[Description]:

In the unified TCI state framework, the IE TCI-UL-State-r17 is only used to configure the TCI state in the separate mode.

[Proposed Change]:

pathlossOffset

Indicates the pathloss offset applied to the UL TCI state. Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on.

[Comments]:

Nokia (Andrew): We have a different view on how to handle this issue. Refer to N122.

# C080

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C080 | AIML | 1 | TTT for stoping logging |  | Tangxun |  | V003 | Agreed |

[Description]: in current spec, “TimeToTrigger” is not only used in starting logging, but also used in stopping logging specified as below:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

But this has not been reflected in the description of TimeToTrigger.

[Proposed Change]: update the description as below:

–	TimeToTrigger

The IE TimeToTrigger specifies the value range used for time to trigger parameter, which concerns the time during which specific criteria for the event needs to be met in order to trigger a measurement report or start/stop logging of measurements for network-side data collection. Value ms0 corresponds to 0 ms and behaviour as specified in 7.1.2 applies, value ms40 corresponds to 40 ms, and so on.

[Comments]:

[Lenovo-Congchi-v011]: Agree

[WI CR rapporteur-v020]: We changed

# H453

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H453 | NR_SL_relay_multihop-Core | 1 | The timers T300/T301/T319 for multi-hop relay |  | Huawei (Jagdeep) |  | V12 | PropAgree |

[Description]: In the field description of the remote UE timers it should be clarified that for the case when the field is absent how the legacy t300 value should be used for multi hop scenario.

[Proposed Change]:

#### –	UE-TimersAndConstantsRemoteUE

The IE UE-TimersAndConstantsRemoteUE contains timers and constants used by the L2 U2N Remote UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstantsRemoteUE information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-START

UE-TimersAndConstantsRemoteUE-r17 ::= SEQUENCE {

t300-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t301-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t319-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

...

}

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-STOP

-- ASN1STOP

UE-TimersAndConstantsRemoteUE field descriptions

t300-RemoteUE Indicates the timer value of T300 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t300 applies to L2 U2N Remote UE for the single hop case. The effective T300 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components, is obtained by multiplying the base T300 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. If the field is absent, the timer value indicated in t300 multiplied by the Hop Count applies to L2 U2N Remote UE for the multihop hop case.

t301-RemoteUE Indicates the timer value of T301 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t301 applies to L2 U2N Remote UE for the single hop case. The effective T301 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components, is obtained by multiplying the base T301 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. If the field is absent, the timer value indicated in t301 multiplied by the Hop Count applies to L2 U2N Remote UE for the multihop hop case.

t319-RemoteUE Indicates the timer value of T319 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t319 applies to L2 U2N Remote UE for the single hop case. The effective T319 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T319 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. If the field is absent, the timer value indicated in t319 multiplied by the Hop Count applies to L2 U2N Remote UE for the multihop hop case.

[Comments]:

[Rapporteur]: Agree to clarify the field description of the remote UE timers as suggested above. Have changed the status from “ToDo” to “PropAgree”.

# O002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O002 | UECap, for NR_ENDC_RF_Ph4 | 2 | Merging of 46-4, 46-5 | R2-25xxxxx | OPPO (Qianxi) |  | V002 | ToDo |

[Description]: Clearly, 46-4 is a subset of 46-5, in the sense that 46-4 supports a smaller number of MPR reduction combination compared to 46-5. So, there is no need for UE to simultaneously report the support of 46-4 and 46-5.

[Proposed Change]: R2 implement 46-4 and 46-5 as two enumerate values of a same capability. If R2 cannot converge, ask R4 whether there is a reason that UE has to report the support of both 46-4 and 46-5 other than 46-5 only using a LS.

[Comments]:

# E038

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E038 | LPWUS | 1 or 2 | Clarify the minimumTimeGap-r19 capability signalling | R2-25xxx | Ericsson (Martin) |  | V006 | Agreed |

[Description]:

RAN1 agreed:

Agreement

The 3 capabilities for UE’s wake-up delay in idle/inactive mode are defined as follows:

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| 5/10/20 | 70 | 500 | 900 |
| 40 | 130 | 600 | 1000 |
| 80 | 250 | 800 | 1200 |
| 160 | 490 | 1200 | 1600 |

The MEGA 38.306 and 38.331 include:

-	minimumTimeGap-r19 indicates the minimum time gap between LP-WUS reception and UE to start PDCCH monitoring.

LPWUS-SupportedBandInfo-r19 ::=   SEQUENCE {

supportedBandIndicator-r19            FreqBandIndicatorNR,

-- R1 62-1: LP-WUS operation in IDLE/INACTIVE mode based on OOK signal

lpwus-OOK-r19                 ENUMERATED {supported}                                      OPTIONAL,

-- R1 62-1a: LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence

lpwus-OFDM-r19                ENUMERATED {supported}                                      OPTIONAL,

-- R1 62-1b: LP-SS based RRM measurement in IDLE/INACTIVE mode when LP-SS overlaid sequence is configured

lpwus-LP-SS-r19               ENUMERATED {supported}                                      OPTIONAL,

minimumTimeGap-r19                    SEQUENCE {

wakeUpDelay-SSB-Periodicity-LessThan20ms-r19     ENUMERATED {ms70, ms500, ms900}                             OPTIONAL,

wakeUpDelay-SSB-Periodicity-40ms-r19             ENUMERATED {ms130, ms600, ms1000}                           OPTIONAL,

wakeUpDelay-SSB-Periodicity-80ms-r19             ENUMERATED {ms250, ms800, ms1200}                           OPTIONAL,

wakeUpDelay-SSB-Periodicity-160ms-r19            ENUMERATED {ms490, ms1200, ms1600}                           OPTIONAL

}

}

It is not clear whether the UE is required to send at least one wake-up delay for one SSB periodicity, or a wake-up delay for each possible SSB periodicity. Furthermore, what is the implication when the UE did not signal a wake-up delay for the configured SSB periodicity in the cell?

[Proposed Change]:

Based on offline discussion a proposal will be provided in a Tdoc.

[Comments]:

[WI Rapp] This issue was discussed in capability CR, and implemented in capability mega CR (but with a different way).

# H201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H201 | XR, LPWUS, AIML,NTN | 2 | OtherConfig for XR, LPWUS, AIML, NTN UAI report |  | Yinghao Guo (Huawei) |  | v05 | PropAgree |

[Description]: There are multiple non-critical extensions of OtherConfig-v1900 created by XR, LPWUS, AIML,NTN

[Proposed Change]: Merge them into a single one.

[Comments]:

# V205

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V205 | NTN | 2 | FFS whether also a distance threshold is indicated. | Yes, R2-250xxxx | vivo (Stephen) |  | v005 | Duplicate X250 |

[Description]: We think a distance threshold is needed for the CONNECTED UE. There is no need for UE to report its location if the distance to the nearest location remains large.

[Proposed Change]: Introduce a distance threshold in Assisted-SSB-MTC-Config. The UE only sets the reference location bit to 1 when the UE is within the associated threshold range.

[Comments]:

[Xiaomi] We agree with the proposal to introduce distance threshold based closest reference location report.

# E042

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E042 | AIML | 2 | Necessity of dedicated flag to configure applicability reporting via UAI | R2-25xxxx | Ericsson (Andra) |  | V022 | Rejected |

[Description]: RAN2#130 agreed the following:

“2	Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.  Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)”

After adding the sets of inference related parameters for option B in otherConfig, it was not discussed how to combine this flag in ASN.1 with the configurations for option B.

[Proposed Change]: We will provide a more detailed discussion and solutions in a Tdoc. Some possible solutions are:

- Remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

- Keep the flag reportApplicabilityUAI-r19 within applicabilityReportConfig, as it is.

- Move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig.

[Comments]:

# C083

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C083 | AIML | 2 | AddModList for applicabilityConfigList-r19 | R2-250xxxx | Tangxun |  | V007 | Agreed |

[Description]: in current spec, “Need R” is used for “applicabilityConfigList-r19”, which means this will be a new list every time when UE receives it. Even if network only wants to modify the entry for one serving cell, it has to provide the whole list for all serving cells, otherwise the other entries for other cells will be deleted. Also considering the following procedural text:

2> if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

<other parts omitted>

3> for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4> set the applicabilityCellId to the serving cell index of the cell;

4> for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

A UE has to report applicability status for each serving cell upon receiving this applicabilityConfigList-r19. Then change based applicability reporting is actually not implemented.

[Proposed Change]: adopt AddModList structure for applicabilityConfigList-r19 as below:

applicabilityConfigToAddModList-r17                  applicabilityConfigToAddModList-r19                                    OPTIONAL,   -- Need N

applicabilityConfigToReleaseList-r17                 applicabilityConfigToReleaseList-r19                                   OPTIONAL,   -- Need N

To address C083 and C084 together, a tdoc is planned to provide a TP for both changes in ASN.1 part and procedural text part.

[Comments]:

[Huawei-Dawid-v017]: We raised the same issue during post-meeting RRC CR review and we agree with this proposal.

[WI CR rapporteur-v022]: We are fine with the proposed solution and also with the suggestion to discuss this in a Tdoc, to provide a complete TP.

# N112

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N112 | AIML | 2 | Excess hierarchy in configuration of inference-related parameters | R2-2507181 | Jerediah Fevold |  | v030 | ToDo |

[Description]: The sets of inference-related parameters are configured in lists per SCell ID, which introduces an additional level of hierarchy. The extra hierarchy doesn’t only affect the ASN.1, but also the procedures which need to nest an additional loop. There isn’t a clear benefit to this.

[Proposed Change]: Remove a level of hierarchy by configuring the SCell ID in the configuration of a set of inference-related parameters and configure all sets of inference-realted parameters in the same list.

[Comments]:

# B205

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B205 | AIML | 2 | applicabilityConfigCellId is mandatory |  | Congchi Zhang |  | V011 | Agreed |

[Description]:

Relevant to C084, the appiabilityConfigCellId of service cell index should dbe mandatory in that case, since the applicability config is per cell.

[Proposed Change]:

ApplicabilityConfig-r19 ::= SEQUENCE {

applicabilityConfigCellId-r19       ServCellIndex,

applicabilitySetConfigList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilitySets-r19)) OF ApplicabilitySetConfig-r19    OPTIONAL, -- Need R

...

}

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# C084

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C084 | AIML | 2 | AddModList for applicabilitySetConfigList-r19 | R2-250xxxx | Tangxun |  | V007 | Agreed |

[Description]: in current spec, “Need R” is used for “applicabilitySetConfigList-r19”, which means this will be a new list every time when UE receives it. Even if network only wants to modify one entry for one AI functionality, it has to provide the whole list for all functionalities, otherwise the other entries will be deleted. Also considering the following procedural text:

2> if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

<other parts omitted>

3> for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4> set the applicabilityCellId to the serving cell index of the cell;

4> for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

A UE has to report applicability status for each entry in a serving cell upon receiving this applicabilitySetConfigList-r19. Then change based applicability reporting is actually not implemented.

[Proposed Change]: adopt AddModList structure for applicabilitySetConfigList-r19 as below:

applicabilitySetConfigToAddModList-r17            applicabilitySetConfigToAddModList-r19                                    OPTIONAL,   -- Need N

applicabilitySetConfigToReleaseList-r17           applicabilitySetConfigToReleaseList-r19                                   OPTIONAL,   -- Need N

To address C083 and C084 together, a tdoc is planned to provide a TP for both changes in ASN.1 part and procedural text part.

[Comments]:

[Lenovo-Congchi-v011]: Tend to agree with CATT. Adopting add/mod/release seems more flexible. And the applicabilityConfigCellId should also be mandatory in this case. Relevant to B205.

[Xiaomi-Xing-012]: We also propose to use ToAddMod structure. By using the list structure, NW has to provide the full list upon modification or release for a certain entry. ToAddMod structure can be used to reduce the signalling upon modification or release for a certain entry.

[Huawei-Dawid-v017]: We raised the same issue during post-meeting RRC CR review and we agree with this proposal. A simple TP is provided below corresponding to both C083 and C084 (as we planned to submit this issue as well, but noticed it was already added. However, also procedural part will have to be corrected accordingly.

ApplicabilityReportConfig-r19 ::= SEQUENCE {

reportApplicabilityUAI-r19     ENUMERATED {true}                                                                    OPTIONAL, -- Need R

applicabilityConfigToAddModList-r19    SEQUENCE (SIZE (1..maxNrofServingCells)) OF ApplicabilityConfig-r19                  OPTIONAL, -- Need N

applicabilityConfigToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex                  OPTIONAL, -- Need N

...

}

ApplicabilityConfig-r19 ::= SEQUENCE {

applicabilityConfigCellId-r19       ServCellIndex                                                                      OPTIONAL, -- Need R

applicabilitySetConfigToAddModList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilitySets-r19)) OF ApplicabilitySetConfig-r19    OPTIONAL, -- Need N

applicabilitySetConfigToReleaseList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilitySets-r19)) OF ApplicabilitySetConfigId-r19    OPTIONAL, -- Need N

...

}

[WI CR rapporteur-v020]: We are fine with the proposed solution and also with the suggestion to discuss this in a Tdoc, to provide a complete TP.

# N030

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N030 | AIML | 2 | Applicability set config ID should not be optional in applicabilitySetConfig. | N/A | Jerediah Fevold |  | vnnn | Agreed |

[Description]: The field applicabilitySetConfigId-r19 in ApplicabilitySetConfig-r19 should not be OPTIONAL as it is required for reporting applicability. Other fields could also be checked, such as resourcesForChannelMeasurement-r19.

[Proposed Change]:

ApplicabilitySetConfig-r19 ::= SEQUENCE {

applicabilitySetConfigId-r19                ApplicabilitySetConfigId-r19                            ,   -- Need R [RIL]: N030 AIML

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”. The intention was to have as many fields as possible as optional, for potential cases in the future, where these fields are not required. However, since it is very unlikely that a configuration ID is not needed, the proposed change is fine.

# N035

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N035 | AIML | 1 | Applicability set is specific to two use cases but uses a generic name. | N/A | Jerediah Fevold |  | vnnn | Agreed |

[Description]: In case beam prediction is not the only use case which will support reporting applicability based on sets of inference-related parameters, we think it would be useful to name the IE carrying the parameters for beam prediction more specifically. The exact name is not important, but we have suggested adding CSI to the name below. Future extensions can be added as more lists of applicabilitySetConfig* if needed.

[Proposed Change]:

ApplicabilityConfig-r19 ::= SEQUENCE {

applicabilityConfigCellId-r19       ServCellIndex                                                                      OPTIONAL, -- Need R

applicabilitySetConfigCSI-List-r19      SEQUENCE (SIZE (1..maxNrofApplicabilitySetsCSI-r19)) OF ApplicabilitySetCSI-Config-r19    OPTIONAL, -- Need R

...

}

ApplicabilitySetCSI-Config-r19 ::= SEQUENCE {

applicabilitySetConfigId-r19                ApplicabilitySetConfigId-r19                            OPTIONAL,   -- Need R [RIL]: N030 AIML

resourcesForChannelMeasurement              CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# H010

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H010 | AIML | 1 | Report configuration type in ApplicabilitySetConfig-r19 | R2-25xxxx | Dawid |  | V017 | Rejected |

[Description]:

In our understanding, reportConfigType in ApplicabilitySetConfig-r19 needs to only indicate the type of the report, i.e. periodic, semi persistent on PUCCH/PUSCH or aperiodic. Additional information such as reportSlotConfig etc. is not required for the UE to determine applicability, so is not needed in this configuration.

[Proposed Change]:

ApplicabilitySetConfig-r19 ::= SEQUENCE {

applicabilitySetConfigId-r19                ApplicabilitySetConfigId-r19                            OPTIONAL,   -- Need R

resourcesForChannelMeasurement              CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

reportQuantity-r19                          ReportQuantity-r19                                      OPTIONAL,   -- Need R

reportConfigType-r19                   ENUMERATED {periodic, semiPersistentOnPUCCH, semiPersistentOnPUSCH, aperiodic }  OPTIONAL,   -- Need R

nrofReportedPredictedRS-r19                 ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                             OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1}          OPTIONAL,   -- Need R

...

}

[Comments]:

[WI CR rapporteur-v022]: We suggest to discuss this in Tdocs, since the interpretation reflected in the proposed changes is not obvious from the RAN1#121 agreement below, which assumes that the parameters are reused:

Agreement

For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused:

- For both BM-Case 1 and BM-Case 2:

  - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19

- For BM-Case 2:

  - TimeGap-r19, nroftimeinstance-r19,

- Note: this doesn’t imply the associated ID is always present

# E052

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E052 | AIML | 1 | Remove unnecessary reportQuantity values from otherConfig |  | Ericsson (Andra) |  | v006 | ToDo |

[Description]: Not all the values for reportQuantity-r19 included in otherConfig are needed for applicability reporting. Namely the values for UE-side data collection and monitoring are not needed, since they are not used for inference and they are thus not relevant to applicability reporting.

[Proposed Change]:

ApplicabilitySetConfig-r19 ::=          SEQUENCE {

applicabilitySetConfigId-r19            ApplicabilitySetConfigId-r19                                             OPTIONAL, -- Need R

resourcesForChannelMeasurement          CSI-ResourceConfigId                                                     OPTIONAL, -- Need R

resourcesForChannelPrediction-r19       CSI-ResourceConfigId                                                     OPTIONAL, -- Need R

associatedIdForChannelMeasurement-r19   AssociatedId-r19                                                         OPTIONAL, -- Need R

associatedIdForChannelPrediction-r19    AssociatedId-r19                                                         OPTIONAL, -- Need R

reportQuantity-r19                      CHOICE {

p-CRI-r19                   NULL,

p-SSB-Index-r19             NULL,

p-CRI-RSRP-r19              NULL,

p-SSB-Index-RSRP-r19

NULL

}

[Comments]:

# S041

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S041 | AIML | 2 | ToAddModList and To ReleaseList for candidate configuration | R2-25xxxx | Seung-Beom |  | v028 | Agreed |

[Description]: According to current ASN.1 structure, when NW adds/releases/modifies each serving cell, NW has to provide all configuration list. i.e., No delta signaling is supported.

For example, UE is configured with three DataCollectionCandidateConfig-r19 for 3 serving cell. When NW adds one more serving cell to UE (i.e., totally 4 serving cells), there is no way for NW to provide only one new DataCollectionCandidateConfig-r19 for the new cell. i.e., NW has to not only provide the new DataCollectionCandidateConfig-r19 but also the old three DataCollectionCandidateConfig-r19 again.

[Proposed Change]:

This can be implemented together with X004. So, we propose:

DataCollectionPreferenceConfig-r19 ::= SEQUENCE {

dataCollectionCandidateConfigList-r19   SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19   OPTIONAL, -- Need R

dataCollectionCandidateConfigToAddModList-r19   SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19   OPTIONAL, -- Need N

dataCollectionCandidateConfigToReleaseList-r19   SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need N

...

}

DataCollectionCandidateConfig-r19 ::= SEQUENCE {

dataCollectionServCellIndex-r19                  ServCellIndex,

dataCollectionCandidateConfigParameterList-r19   SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigParameters-r19  OPTIONAL, -- Need R

dataCollectionCandidateConfigParameterToAddModList-r19   SEQUENCE (SIZE (1.. maxCandidateConfig-r19)) OF DataCollectionCandidateConfigParameters-r19   OPTIONAL, -- Need N

dataCollectionCandidateConfigParameterToReleaseList-r19   SEQUENCE (SIZE (1.. maxCandidateConfig-r19)) OF DataCollectionCandidateConfigId-r19 OPTIONAL, -- Need N

...

}

DataCollectionCandidateConfigParameters-r19 ::= SEQUENCE {

dataCollectionCandidateConfigId-r19         DataCollectionCandidateConfigId-r19,

resourcesForChannelMeasurement              CSI-ResourceConfigId                                     OPTIONAL, -- Need R

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                     OPTIONAL, -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                         OPTIONAL, -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                         OPTIONAL, -- Need R

...

}

[Comments]:

[WI CR rapporteur-v031]: We suggest to discuss this in a Tdoc, together with X004, and include the impact on the procedural text.

# N113

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N113 | AIML | 2 | Excess hierarchy in Data Collection Candidate Config | R2-2507181 | Jerediah Fevold |  | v030 | ToDo |

[Description]: As with N111 and N112, the data collection candidates are configured in lists that are SCell ID specific. The extra hierarchy in the procedures and ASN.1 could be eliminated by moving the SCellIndex to the DataCollectionCandiateConfig.

[Proposed Change]: Remove a level of hierarchy by moving the SCell ID to the DataCollectionCandidateConfig and store all candidate configurations in a single list.

[Comments]:

# X004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X004 | AIML | 1 | candidate data collection configuration | R2-25xxxx | Xing |  | V012 | Agreed |

[Description]:

Currently, the inference related configuration and candidate data collection configuration is provided via a list in OtherConfig. However, with a list, NW has to provide the full list upon modification or release for a certain entry. ToAddMod structure can be used to reduce the signalling upon modification or release for a certain entry.

[Proposed Change]:

Change current structure of candidate data collection configuration to ToAddMod structure. UE behaviour should also be added.

[Comments]:

[WI CR rapporteur-v022]: Since this implies several changes, companies are encouraged to discuss this topic and provide TPs in Tdocs, as for the similar RILs C083/C084.

# C081

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C081 | AIML | 1 | availability of logged radio measurements data |  | Tangxun |  | V006 | Agreed |

[Description]: we notice that the availability indication of logged data in UAI has been removed from ASN.1, as it can’t be reported separately, i.e., implicitly indicated by full-buffer or low-power indication. But in the field description of loggedDataCollectionAssistanceConfig, the UE behaviour of reporting separate availability indication remains. So it should be removed to avoid further confusion.

[Proposed Change]: update the description as below:

loggedDataCollectionAssistanceConfig Configuration for the UE to report assistance information related to logging of radio measurements for network-side data collection.

[Comments]:

[WI CR rapporteur-v022]: We agree that the word “availability” can be removed, but we do not think we should remove the entire second sentence as suggested. Otherwise it is not clear what indications are configured. We propose the alternative change below and we changed the status from “ToDo” to “PropAgree”.

loggedDataCollectionAssistanceConfig Configuration for the UE to report assistance information related to logging of radio measurements for network-side data collection. When configured with loggedDataCollectionAssistanceConfig the UE reports that it has logged radio measurements for network-side data collection when the buffer reserved for logging of radio measurements for network-side data collection has become full and it reports when it determines that it has entered a low power state.

# C082

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C082 | AIML | 1 | availability of logged radio measurements data |  | Tangxun |  | V006 | Agreed |

[Description]: we notice that the availability indication of logged data in UAI has been removed from ASN.1, as it can’t be reported separately, i.e., implicitly indicated by full-buffer or low-power indication. But in the field description of loggedDataCollectionBufferThreshold, the UE behaviour of reporting separate availability indication remains. So it should be removed to avoid further confusion.

[Proposed Change]: update the description as below:

loggedDataCollectionBufferThreshold Buffer threshold for the UE to report assistance information related to logging of radio measurements for network-side data collection. Value kB16 means the threshold is set to 16 kB and so on.

[Comments]:

[WI CR rapporteur-v022]: As for C081, we agree that the word “availability” can be removed and the text can be improved, but we do not think we should remove the entire explanation, as suggested. We propose the alternative change below and we changed the status from “ToDo” to “PropAgree”.

loggedDataCollectionBufferThreshold Buffer threshold for the UE to report that it has logged radio measurements for network-side data collection, if the amount of data in the buffer reserved for logging of radio measurements for network-side data collection has become equal to or above this threshold.. Value kB16 means the threshold is set to 16 kB and so on.

# C029

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C029 | LPWUS | 1 | UAI configuration for UE’s preferred time offset for LP-WUS monitoring | No | Da Wang (CATT) |  | V012 | Agreed |

[Description]: UAI configuration for UE’s preferred time offset for LP-WUS monitoring is actually used to configure the report of preferred time offset for RRC_CONNECTED. But this is not reflected in RRC.

[Proposed Change]:

lpwus-OffsetPreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE’s preferred time offset for LP-WUS monitoring for RRC_CONNECTED mode.

[Comments]:

# H254

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H254 | NTN | 1 | Description of refLocList | R2-25xxxxx | Huawei (Lili) |  | V014 | Rejected |

[Description]: The description should also clarify that if the list is present, UE ignores the reference locations provided in SIB2 (to avoid any ambiguity if the information provided in dedicated signalling is different from that in SIB2).

[Proposed Change]:

refLocList

A list of reference locations for assisted SMTC configuration in RRC_CONNECTED state. If this field is absent when closestLocsToReport is signalled, the UE shall use the refLocList associated to smtc5list provided in SIB2, if available. If this field is present, UE ignores the refLocList provided in SIB2.

[Comments]:

# E049

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E049 | NR_SL_relay_multihop-Core | 1 | Wrong need code for sl-SRAP-ConfigRelay-ToAddModList sl-SRAP-ConfigRelay-ToReleaseList |  | Ericsson - Min |  | V017 | PropAgree |

[Description]:

-- ASN1START

-- TAG-SL-L2RELAYUE-CONFIG-START

SL-L2RelayUE-Config-r17 ::=        SEQUENCE {

sl-RemoteUE-ToAddModList-r17       SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17    OPTIONAL,    -- Need N

sl-RemoteUE-ToReleaseList-r17      SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16  OPTIONAL,    -- Need N

...,

[[

sl-U2U-RemoteUE-ToAddModList-r18   SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-U2U-RemoteUE-Config-r18   OPTIONAL,    -- Need N

sl-U2U-RemoteUE-ToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16   OPTIONAL     -- Need N

]]

}

SL-RemoteUE-ToAddMod-r17 ::=       SEQUENCE {

sl-L2IdentityRemote-r17            SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17            SL-SRAP-Config-r17                                                      OPTIONAL,    -- Need M

...,

[[

sl-SRAP-ConfigRelay-ToAddModList-r19     SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-Config-ToAddMod-r19      OPTIONAL,      -- Need R

sl-SRAP-ConfigRelay-ToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-ConfigId-r19               OPTIONAL      -- Need R

]]

}

The ToAddModList and the ToReleaseList have wrong need code. The need code should be corrected as Need N instead of Need R

In addition, the procedure texts on how to add/remove/mod the list are missing.

[Proposed Change]:

Proposed changes include

- Change need code from Need R to Need N

- Including procedure texts on add/remove/mod the list.

[Comments]:

[Rapporteur]: Agree to Change need code from Need R to Need N.

sl-SRAP-ConfigRelay-ToAddModList and sl-SRAP-ConfigRelay-ToReleaseList-r19 have been included in the procedure text but without hypen in between. The names in the procedure text will be changed to sl-SRAP-ConfigRelay-ToAddModList and sl-SRAP-ConfigRelay-ToReleaseList-r19 to align with the names in the ASN.1. Have changed the status from “ToDo” to “PropAgree”.

# H456

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H456 | NR_SL_relay_multihop-Core | 1 | The name of the sub IE sl-SRAP-ConfigRelay-r17 in SL-SRAP-Config-ToAddMod-r19 wrongly includes -r17 in its name |  | Huawei (Jagdeep) |  | V20 | PropAgree |

[Description]: The name of the sub IE sl-SRAP-ConfigRelay-r17 in SL-SRAP-Config-ToAddMod-r19 wrongly includes -r17 in its name which should be change to -r19

[Proposed Change]:

SL-SRAP-Config-ToAddMod-r19 ::=      SEQUENCE {

sl-SRAP-ConfigId-r19               SL-SRAP-ConfigId-r19,

sl-SRAP-ConfigRelay-r19            SL-SRAP-Config-r17

...

}

[Comments]:

[Rapporteur]: Agree to change the name of sl-SRAP-ConfigRelay-r17  to sl-SRAP-ConfigRelay-r19 as suggested above. Have changed the status from “ToDo” to “PropAgree”.

# H456

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H456 | NR_SL_relay_multihop-Core | 1 | The name of the sub IE sl-SRAP-ConfigRelay-r17 in SL-SRAP-Config-ToAddMod-r19 wrongly includes -r17 in its name |  | Huawei (Jagdeep) |  | V20 | PropAgree |

[Description]: The name of the sub IE sl-SRAP-ConfigRelay-r17 in SL-SRAP-Config-ToAddMod-r19 wrongly includes -r17 in its name which should be change to -r19

[Proposed Change]:

SL-SRAP-Config-ToAddMod-r19 ::=      SEQUENCE {

sl-SRAP-ConfigId-r19               SL-SRAP-ConfigId-r19,

sl-SRAP-ConfigRelay-r19            SL-SRAP-Config-r17

...

}

[Comments]:

[Rapporteur]: Agree to change the name of sl-SRAP-ConfigRelay-r17  to sl-SRAP-ConfigRelay-r19 as suggested above. Have changed the status from “ToDo” to “PropAgree”.

# J012

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J012 | NR_SL_relay_multihop-Core | 1 | Remove First U2N Relay UE in field description for SL-RelayUE-ConfigMH |  | Sharp (LIU Lei) |  | V006 | PropReject |

[Description]: First U2N Relay UE is immeterdiate U2N Relay UE, “First U2N Relay UE” in field description for SL-RelayUE-ConfigMH can be removed.

[Proposed Change]:

–	SL-RelayUE-ConfigMH

The IE SL-RelayUE-ConfigMH specifies the threshold configuration information for NR sidelink Last U2N Relay UE or Intermediate U2N Relay UE.

SL-RelayUE-ConfigMH information element

-- ASN1START

-- TAG-SL-RELAYUE-CONFIGMH-START

SL-RelayUE-ConfigMH-r19::=           SEQUENCE {

sd-RSRP-ThreshDiscConfigMH-r19       SL-RSRP-Range-r16,

sd-hystMaxRelayMH-r19                Hysteresis

}

-- TAG-SL-RELAYUE-CONFIGMH-STOP

-- ASN1STOP

SL-RelayUE-ConfigMH field descriptions

sd-RSRP-ThreshDiscConfigMH Indicates the threshold of SD-RSRP for an Last U2N Relay UE or Intermediate U2N Relay UE to evaluate AS layer conditions for discovery. The Last U2N relay UE applies the value of this field to evaluate AS layer conditions to decide whether to respond to the discovery solicitation message when performing the multi hop U2N Relay Discovery with Model B as specified in TS 23.304 [65]. The Intermediate U2N relay UE applies the value of this field to evaluate AS layer conditions to decide whether to forward the discovery solicitation message when performing the multi hop U2N Relay Discovery with Model B as specified in TS 23.304 [65].

[Comments]:

[Rapporteur]: It is essential to mention that this threshold is applicable to First U2N relay UE to avoid any confusion hence rapporteur recommends " PropReject " status for this RIL.

# V206

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V206 | NTN | 1 | - Need code for mbs-SessionAreaList-r19 should be Need S | Yes, R2-250xxxxx | vivo (Stephen) |  | v005 | Agreed |

[Description]: RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is applicable for reception within the entire cell area. So Need S is supposed to be used for mbs-SessionAreaList-r19

[Proposed Change]: Change Need R to Need S for mbs-SessionAreaList-r19

[Comments]:

[Rapp] In our understanding, based on RAN2 previous agreements, if the ISA (ID) is not provided for a certain service, then legacy mechanisms apply (it is intended for the whole area). Usually, legacy behaviour does not need an explanation, it is only the new functionality that needs to be clarified in new parameters. This should be discussed in the next meeting.

# V207

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V207 | NTN | 1 | Capture the following agreement. - RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is applicable for reception within the entire cell area, with legacy behavior applicable (FFS whether we capture this in the spec) | Yes, R2-250xxxxx | vivo (Stephen) |  | v003 | Duplicate V206 |

[Description]: The current specification only specifies the following cases: namely, cases where a service is not associated with an ISA entry, and cases where services are associated with an ISA entry with a specific area. The UE's behavior in the case where a service is associated with an empty ISA entry shall be further clarified.

[Proposed Change]: Capture the agreement in the FD of mbs-SessionAreaList.

mbs-SessionAreaList

Indicates the list of intended service areas associated with an MBS broadcast session in an NTN cell. If absent, UE considers the associated service can be received within the entire cell area.

[Comments]:

[Rapp] Both RILs are addressing the same issue and can be handled together in the contribution.

# X505

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X505 | SLRelay | 1 | Wrong IE name |  | Xiaomi (Xing Yang) |  | V005 | PropReject |

[Description]:

The pre-fix of ‘mh-indicationType-r19’ should be removed to align with legacy IE names. Otherwise, the procedure text shall differentiate the two IEs.

[Proposed Change]:

## 6.6	PC5 RRC messages

=================================NEXT CHANGE=======================================

### 6.6.2	Message definitions

#### –	NotificationMessageSidelink

The NotificationMessageSidelink message is used to send notification message from U2N Relay UE to the connected U2N Remote UE or from U2U Relay UE to the connected U2U Remote UE.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: U2N Relay UE to U2N Remote UE or U2N Parent UE to U2N Child UE or U2U Relay UE to U2U Remote UE

NotificationMessageSidelink message

-- ASN1START

-- TAG-NOTIFICATIONMESSAGESIDELINK-START

NotificationMessageSidelink-r17 ::=       SEQUENCE {

criticalExtensions                        CHOICE {

notificationMessageSidelink-r17           NotificationMessageSidelink-r17-IEs,

criticalExtensionsFuture                  SEQUENCE {}

}

}

NotificationMessageSidelink-r17-IEs ::=   SEQUENCE {

indicationType-r17                        ENUMERATED {

relayUE-Uu-RLF, relayUE-HO, relayUE-CellReselection,

relayUE-Uu-RRC-Failure

}                                     OPTIONAL,  -- Need N

lateNonCriticalExtension                  OCTET STRING                          OPTIONAL,

nonCriticalExtension                      NotificationMessageSidelink-v1800-IEs OPTIONAL

}

NotificationMessageSidelink-v1800-IEs ::= SEQUENCE {

sl-IndicationType-r18                     ENUMERATED {relayUE-PC5-RLF, spare1}  OPTIONAL,  -- Need N

sl-DestinationIdentityRemoteUE-r18        SL-DestinationIdentity-r16            OPTIONAL,  -- Need N

nonCriticalExtension                      NotificationMessageSidelink-v19xy-IEs OPTIONAL

}

NotificationMessageSidelink-v19xy-IEs ::= SEQUENCE {

indicationType-r19                     ENUMERATED {

relayUE-RelayReselection,

relayUE-CellSelection

}  								OPTIONAL,  -- Need N

nonCriticalExtension                      SEQUENCE {}                           OPTIONAL

}

-- TAG-NOTIFICATIONMESSAGESIDELINK -STOP

-- ASN1STOP

[Comments]:

[Rapporteur]: In Rapporteur understanding we will need a different name for the new cause value as it is used together with the legacy indicationType-r17                         hence rapporteur recommends " PropReject " status for this RIL.

# Z258

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z258 | NTN | 2 | Missing on demand SIBXX in SL-SIB-ReqInfo | No | ZTE (Zhihong) |  | v012 | Rejected |

[Description]: Current SL-SIB-ReqInfo allows request SIB19, SIB20 and SIB21which implies support of MBS and NTN for sidelink UEs. Based on which the ISA(s) enhancements could also be applicable for sidelink UE, therefore it is proposed to allow UE request the complete SIB sets for MBS service in NTN.

[Proposed Change]: Update the description as below:

SL-SIB-ReqInfo-r17 ::=                   ENUMERATED { sib1, sib2, sib3, sib4, sib5, sib6, sib7, sib8, sib9, sib10, sib11, sib12, sib13,

sib14, sib15, sib16, sib17, sib18, sib19, sib20, sib21, sibNotReq11, sibNotReq10,

sibNotReq9, sibNotReq8, sibNotReq7, sibNotReq6, sibNotReq5, sibNotReq4,

sibNotReq3, sibNotReq2, sibNotReq1, ..., sib17bis-v1820, sibxx-v1900 }

[Comments]:

[Rapp] RAN2 should clarify whether this scenario is supported.

[Qualcomm] This should be raised in sidelink work item.

# H006

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H006 | AIML | 2 | Missing imports |  |  |  | vnnn | Agreed |

[Description]:

CSI-LogMeasInfoCellList-r19 is missing from “IMPORTS” in section 7.4.

[Proposed Change]:

## 7.4	UE variables

NOTE:	To facilitate the specification of the UE behavioural requirements, UE variables are represented using ASN.1. Unless explicitly specified otherwise, it is however up to UE implementation how to store the variables. The optionality of the IEs in ASN.1 is used only to indicate that the values may not always be available.

#### –	NR-UE-Variables

This ASN.1 segment is the start of the NR UE variable definitions.

-- ASN1START

-- NR-UE-VARIABLES-START

NR-UE-Variables DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

AreaConfiguration-r17,

ARFCN-ValueNR,

CellIdentity,

EUTRA-PhysCellId,

maxCEFReport-r17,

maxCellReport,

MeasId,

MeasIdToAddModList,

MeasIdleCarrierEUTRA-r16,

MeasIdleCarrierNR-r16,

MeasResultIdleEUTRA-r16,

MeasResultIdleNR-r16,

MeasReselectionCarrierNR-r18,

MeasurementValidityDuration-r18,

MeasObjectToAddModList,

MeasConfigAppLayerId-r17,

MeasConfigAppLayer-r17,

maxNrofAppLayerMeas-r17,

AppLayerIdleInactiveConfig-r18,

PhysCellId,

RNTI-Value,

ReportConfigToAddModList,

RSRP-Range,

SL-MeasId-r16,

SL-MeasIdList-r16,

SL-MeasObjectList-r16,

SL-ReportConfigList-r16,

SL-QuantityConfig-r16,

Tx-PoolMeasList-r16,

QuantityConfig,

maxNrofCellMeas,

maxNrofMeasId,

maxFreqIdle-r16,

PhysCellIdUTRA-FDD-r16,

ValidityAreaList-r16,

CondReconfigToAddModList-r16,

ConnEstFailReport-r16,

LoggingDuration-r16,

LoggingInterval-r16,

LogMeasInfoList-r16,

LogMeasInfo-r16,

RA-Report-r16,

RLF-Report-r16,

TraceReference-r16,

WLAN-Identifiers-r16,

WLAN-NameList-r16,

BT-NameList-r16,

PLMN-Identity,

maxNrofRelayMeas-r17,

maxPLMN,

RA-ReportList-r16,

VisitedCellInfoList-r16,

AbsoluteTimeInfo-r16,

LoggedEventTriggerConfig-r16,

LoggedPeriodicalReportConfig-r16,

Sensor-NameList-r16,

SL-SourceIdentity-r17,

SuccessHO-Report-r17,

PLMN-IdentityList2-r16,

AreaConfiguration-r16,

maxNrofSL-MeasId-r16,

maxNrofFreqSL-r16,

maxNrofCLI-RSSI-Resources-r16,

maxNrofCLI-SRS-Resources-r16,

RSSI-ResourceId-r16,

SRS-ResourceId,

VisitedPSCellInfoList-r17,

SuccessPSCell-Report-r18,

maxNPN-r16,

SNPN-ConfigID-List-r18,

AreaConfiguration-v1800,

NID-r16,

SK-CounterConfig-r18,

ReferenceConfiguration-r18,

maxNrofLTM-Configs-plus1-r18,

maxSecurityCellSet-r18,

CSI-LogMeasInfoCellList-r19

FROM NR-RRC-Definitions;

-- NR-UE-VARIABLES-STOP

-- ASN1STOP

[Comments]:

[Lenovo-Congchi-v014]: Ok but need to be fixed already during CR implementation. Otherwise, the R19 ASN.1 will not pass syntax check.

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropAgree”.

# M204

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M204 | MOB | 1 | Ambiguity on UE variable VarLTM-ServingCellNoResetID for inter-CU LTM |  | Pasi (MediaTek) |  | V008 | Agreed |

[Description]:

Based on the procedural text in clause 5.3.5.18.6 (assuming X152 and X153 are agreed), the UE variable VarLTM-ServingCellNoResetID is used to determine need for L2 reset only when the LTM cell switch does not include security key change. It would make the specification more clear, if the description of the UE variable was clarified accordingly.

[Proposed Change]:

–		VarLTM-ServingCellNoResetID

The IE VarLTM-ServingCellNoResetID is used to store the ID associated with the serving cell based on which the UE determines whether a L2 reset is needed or not upon an LTM cell switch procedure which does not involve security key change.

VarLTM-ServingCellNoResetID UE variable

-- ASN1START

-- TAG-VARLTM-SERVINGCELLNORESETID-START

VarLTM-ServingCellNoResetID-r18 ::=     SEQUENCE {

ltm-ServingCellNoResetID-r18            INTEGER (1..maxNrofLTM-Configs-plus1-r18)                         OPTIONAL

}

-- TAG-VARLTM-SERVINGCELLNORESETID-STOP

-- ASN1STOP

[Comments]:

# E045

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E045 | NR_SL_relay_multihop-Core | 1 | Missing description/definition for the IE/field. relayUE-RRCState-r19 |  | Ericsson - Min |  | V012 | PropAgree |

[Description]:

cellAccessRelatedInfo-r17               CellAccessRelatedInfo,

sl-ServingCellInfo-r17                  SL-ServingCellInfo-r17,

...,

[[

sl-RelayIndication-r18                  SL-RelayIndicationMP-r18                  OPTIONAL

]],

[[

relayUE-RRCState-r19              		ENUMERATED {rrc-Connected, spare1}	  	  OPTIONAL

]]

}

Missing description/definition for the above IE/field.

[Proposed Change]:

Suggest to include description for the IE as

#### –	SL-RelayUE-RRCState

The IE SL-RelayUE-RRCState is used to indicate the RRC state of L2 U2N Relay UE In case of multi-hop L2 U2N Relay communication.

SL-RelayUE-RRCState information element

-- ASN1START

-- TAG-SL-RelayUE-RRCState-START

SL-RelayUE-RRCState-r19 ::=   ENUMERATED {rrc-Connected, spare1}

-- TAG-SL-RelayUE-RRCState-STOP

-- ASN1STOP

Also update the SL-AccessInfo-L2U2N-r17 as

SL-AccessInfo-L2U2N-r17 ::=             SEQUENCE {

cellAccessRelatedInfo-r17               CellAccessRelatedInfo,

sl-ServingCellInfo-r17                  SL-ServingCellInfo-r17,

...,

[[

sl-RelayIndication-r18                  SL-RelayIndicationMP-r18                  OPTIONAL

]],

[[

relayUE-RRCState-r19              		SL-RelayUE-RRCState-r19	  	  OPTIONAL

]]

}

[Comments]

[Comments]:

[Rapporteur]: Agree to add the definition for relayUE-RRCState-r19 as suggested above . Have changed the status from “ToDo” to “PropAgree”.

# K002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K002 | NR_SL_relay_multihop-Core | 1 | SIB/Paging information release due to SL RLF |  | ASUSTeK (Richard Kuo) |  | V007 | PropReject |

[Description]: According to clause 5.8.9.8.1 in the current RRC Spec, an Intermediate U2N Relay UE in RRC_IDLE/RRC_INACTIVE shall provide the SIB(s) /posSIB(s) required by a L2 U2N Remote UE and Paging related information to its parent UE. Thus, when the L2 Intermediate U2N Relay UE detects sidelink radio link failure with the L2 U2N Remote UE (or the child UE), the required SIB(s) and the related Paging information associated with the L2 U2N Remote UE in the parent UE should be released. This trigger is missing in 5.8.9.3 for Intermediate U2N Relay UE to initiate the Remote UE information for NR sidelink communication procedure.

[Proposed Change]:

#### 5.8.9.3   Sidelink radio link failure related actions

The UE shall:

1> upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1> upon T400 expiry for a specific destination; or

1> upon indication from MAC entity that HARQ-based Sidelink RLF for a specific destination has been detected; or

1> upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination; or

1> upon indication of consistent sidelink LBT failures for all RB sets for a specific destination from MAC entity:

2> consider sidelink radio link failure to be detected for this destination;

2> release the DRBs (if any) of this destination, according to clause 5.8.9.1a.1;

2> release the SRBs of this destination, according to clause 5.8.9.1a.3;

2> release the PC5 Relay RLC channels of this destination if configured, in according to clause 5.8.9.7.1;

2> discard the NR sidelink communication related configuration of this destination;

2> reset the sidelink specific MAC of this destination, except for end-to-end PC5 connection in L2 U2U Relay operation;

2> consider the PC5-RRC connection is released for the destination;

2> indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2> if UE is in RRC_CONNECTED:

3> if the UE is acting as L2 U2N Remote UE for the destination:

4> if MP is configured, and neither MCG transmission nor indirect path transmission is suspended:

5> initiate the indirect path failure information procedure as specified in 5.7.3c;

4> else (i.e., MP is not configured, or MP is configured and MCG transmission or indirect path transmission is suspended):

5> initiate the RRC connection re-establishment procedure as specified in 5.3.7;

3> else:

4> perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

2> else (i.e. the UE is in RRC_IDLE or RRC_INACTIVE):

3> if the UE is acting as L2 Intermediate U2N Relay UE for the destination and the destination is a child UE:

4> perform the Remote UE information for NR sidelink communication procedure with its parent UE, as specified in 5.8.9.8;

…

[Comments]:

[Rapporteur]: Similar to above RIL K002 . When PC5 RLF happens the remote UEs will perform RRCReestablishment and the gNB will eventually release the allocated resources in all the relays on the disrupted path for these remote UEs via the RRC Reconfiguration procedure. Hence hence Rapporteur recommends " PropReject " status for this RIL as such mechanism are not needed..

[Apple] This is for IDLE/INACTIVE Intermeidate relay UE, so RRC restablishment does not apply. I think the above change is needed, but it only covers RLF case and does not cover the PC5-link release case.

[Rapporteur]: PC5 RLF will only happen in the when the UEs are in RRC_CONNECTED State it will not happen when the UEs are in RRC_IDLE or inactive. So the PC5 RLF case duing IDLE / INACTIVE is not valid.

[RRC Rapp did not find this RIL in Review file]

# B206

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B206 | AIML | 1 | Transfer of applicability information in handover command preparatoin |  | Congchi Zhang |  | V011 | Agreed |

[Description]:

Relevant to the RAN2 discussion and the note below in 38.300

- NOTE 3:	UAI can be sent from the source gNB to the target gNB to exchange applicability reporting referring to the configurations from the source gNB.

The current UAI in handover preparation information message only conveys what has been reported by UE in the last UAI report. In the context of applicability reporting procedure, that means the target gNB may only be informed about the recently changed applicability information (i.e., delta instead of complete appliability information). It can be easily resolved by clarifying that the UAI in handover preparation information can also contain the complete applicability information not only the last reported ones.

[Proposed Change]:

ueAssistanceInformation

Includes for each UE assistance feature the information last reported by the UE, if any. It may also include any applicability information that has been reported by the UE.

[Comments]:

[WI CR rapporteur-v022]: We changed the status from “ToDo” to “PropReject”. We think the proposed changed is not aligned with the RAN2#128 agreement below, which was taken exactly so that no further enhancements are made for applicability reporting in case of handovers.

“5 Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.”

[Lenovo-Congchi-v024]: At the time when the agreement was made, companies including us assumed the UAI can convey the complete list, but seemed not correct when looking at the Stage 3 description (which happens somethimes). We still believe it’s something benefit from both NW and UE point of view, so the target gNB can configure properly considering the received complete (in)applicability info.

This issue was raised also by some other company before, and the same issue is raised in O300, while a different solution is provided. Although we believe allowing UAI to convey complete (in)applicability info would be the most straightforward way to fix, it could be worth some clarification in the next meeting. Would it be ok to at least mark it as ToDo instead? We can then prepare a Tdoc to list the possible fixes.

[Samsung-Beom-v028]: We agree the issue but can check if the proposed change is the best.

[WI CR rapporteur-v031]: Given the interest from several companies, we changed the status from “PropReject” to “ToDo” and we suggest that companies address this in a Tdoc.

# Z165

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z165 | MOB | 1 | No need to introduce ltm-Config-r19 in the CG-Config |  | ZTE (Mengjie Zhang) |  | V009 | ToDo |

[Description]: According to the latest RAN3 BLCR R2-256023, most IEs included in the ltm-Config have been introduced in XnAP messages for SCG LTM. Currently, only ltm-NoResetID and ltm-UE-MeasuredTA-ID are missed in the RAN3 interface messages, which are still under RAN3 discussion. Thus, we think there is no need to introduce the whole ltm-Config in the CG-Config, which may cause the redundant IE information transfer for SCG LTM. We can just introduce IEs currently missing in the RAN3 interface messages, e.g. ltm-NoResetID and ltm-UE-MeasuredTA-ID, if needed.

[Proposed Change]: To remove ltm-Config-r19 from the CG-Config, and just add the IEs (i.e. ltm-NoResetID and ltm-UE-MeasuredTA-ID) in the CG-Config message.

[Comments]:

# C165

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C165 | MOB | 1 | ltm-ReferenceConfigurationMCG in CG-ConfigInfo should contain the LTM reference configuration to be used at the SCG, but not MCG |  | Rui (CATT) |  | V005 | Agreed |

[Description]:

In the field description ltm-ReferenceConfigurationMCG in CG-ConfigInfo,it says “The field contains the LTM reference configuration to be used at the MCG”.In our understanding, it should be LTM reference configuration to be used at the SCG.

.

[Proposed Change]:

ltm-ReferenceConfigurationMCG

The field contains the LTM reference configuration to be used at the MCGSCG. This field is only used in NR-DC.

[Comments]:

[Rapporteur] I think that the proposal is correct as far as RAN3 has done the signalling correctly. My proposal is to wait one more meeting and then take a final decision on what to do with this aspect.

# S037

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S037 | MOB | 1 | Maintaining the LTM-Config and reference configuration received in INM for Inter-SN LTM | R2-25xxxxx | Samsung(Aby) |  | V014 | Agreed |

[Description]:

When the reference configuration or LTM configuration is included in CG-Config/CG-ConfigInfo, it makes sense for the receiver to maintain the received parameters. Otherwise the entire configuration needs to be included again in every CG-Config/CG-Config according to the clause 11.2.3.

For e.g. if MN sends a new DRX configuration or measurement gap configuration or sends a UAI to SN, entire ltm-config or ltm-referenceconfiguration  which was send previously needs to be again included in the CG-ConfigInfo even if there is no change for these configurations.

Since the ltm-config or ltm-referenceconfiguration are Need M parameters in air interface there is no need to resend them everytime a CG-Config/CG-ConfigInfo is send, They are very large IEs, potentially including thousands of bytes, since the reference configuration can  include entire RRCReconfiguration message, or for LTM-Config, many RRCReconfiguration messages of all the candidate cells.

To avoid this, Reference configiuration and LTM config can be included in the list in section 11.2.3	Mandatory information in inter-node RRC messages, to avoid the retransmission everytime a CG-Config/CG-ConfigInfo is send.

[Proposed Change]: Based on the above description, we suggest the following change:

11.2.3	Mandatory information in inter-node RRC messages

For the AS-Config transferred within the HandoverPreparationInformation:

-	The source node shall include all fields necessary to reflect the current AS configuration of the UE, except for the fields sourceSCG-NR-Config, sourceSCG-EUTRA-Config and sourceRB-SN-Config, which can be omitted in case the source MN did not receive the latest configuration from the source SN. For RRCReconfiguration included in the field rrcReconfiguration, ReconfigurationWithSync is included with only the mandatory subfields (e.g. newUE-Identity and t304) and ServingCellConfigCommon;

-	Need codes or conditions specified for subfields according to IEs defined in clause 6 do not apply. I.e. some fields shall be included regardless of the "Need" or "Cond" e.g. discardTimer;

-	Based on the received AS configuration, the target node can indicate the delta (difference) to the current AS configuration (as included in HandoverCommand)to the UE. The fields newUE-Identity and t304 included in ReconfigurationWithSync are not used for delta configuration purpose.

The candidateCellInfoListSN(-EUTRA) in CG-Config and the candidateCellInfoListMN(-EUTRA)/candidateCellInfoListSN(-EUTRA) in CG-ConfigInfo need not be included in procedures that do not involve a change of node.

For fields scg-CellGroupConfig, scg-CellGroupConfigEUTRA and scg-RB-Config in CG-Config (sent upon SN initiated SN change or other conditions as specified in field descriptions) and fields mcg-RB-Config, scg-RB-Config and sourceConfigSCG in CG-ConfigInfo (sent upon change of SN):

-	The source node shall include all fields necessary to reflect the current AS configuration of the UE, unless stated otherwise in the field description. For RRCReconfiguration included in the field scg-CellGroupConfig in CG-Config, ReconfigurationWithSync is included with only the mandatory subfields (e.g. newUE-Identity and t304) and ServingCellConfigCommon;

-	Need codes or conditions specified for subfields according to IEs defined in clause 6 do not apply;

-	Based on the received AS configuration, the target node can indicate the delta (difference) as compared to the current AS configuration to the UE. The fields newUE-Identity and t304 in ReconfigurationWithSync are always included by the target node, i.e. they are not used for delta configuration purpose to UE.

For fields in CG-Config and CG-ConfigInfo listed below, absence of the field means that the receiver maintains the values informed via the previous message. Note that every time there is a change in the configuration covered by a listed field, the MN or SN shall include the field and it shall provide the full configuration provided by that field unless stated otherwise. Otherwise, if there is no change, the field can be omitted:

-	configRestrictInfo;

-	gapPurpose;

-	measGapConfig (for which delta signaling applies);

-	measGapConfigFR2 (for which delta signaling applies);

-	measResultCellListSFTD;

-	measResultSFTD-EUTRA;

-	sftdFrequencyList-EUTRA;

-	sftdFrequencyList-NR;

-	ue-CapabilityInfo;

-	servFrequenciesMN-NR;

-	musim-GapConfigInfo-r18;

-	musim-CapRestrictionInfo-r18.

-  ltm-ReferenceConfigurationSCG

-  ltm-Config

-  ltm-ReferenceConfigurationMCG

For other fields in CG-Config and CG-ConfigInfo, the sender shall always signal the appropriate value even if same as indicated in the previous inter-node message, unless explicitly stated otherwise.

[Comments]:

[Rapporteur] We did not capture anything about this in Rel-18, but we can discuss if something is needed. Good if proponent companies has a contribution on it with a TP.

# S036

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S036 | MOB | 1 | Upate L1-MeasConfigNRDC with CSI-RS measurement related information. | R2-25xxxxx | Samsung(Aby) |  | V014 | ToDo |

[Description]:

In the capability CR, a number of L1 capabilities for CSI-RS measurements are added for LTM as below. This needs to be considered in RRC INM.

intraFreqL1-MeasConfigPeriodicCSI-RS-r19 Indicates whether UE supports intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s). This capability signalling comprises of the following parameters: - supportedMaxIntraFreqCellsConfig-r19 indicates the maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource; - supportedMaxIntraFreqCellsPerReport-r19 indicates the maximum number of candidate cells in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement; - supportedMaxReportBeamsPerReportedCell-r19 indicates the maximum number of candidate beams per candidate cell in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement; - supportedMaxReportBeamsReports-r19 indicates the maximum number of candidate beams in total across all cells© in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement; - supportedMaxAperiodic-LTM-CSI-ReportConfig-r19 indicates maximum number of aperiodic LTM-CSI-ReportConfig; - supportedMaxPeriodic-LTM-CSI-ReportConfig-r19 indicates maximum number of periodic LTM-CSI-ReportConfig; - supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r19 indicates maximum number of semi-persistent LTM-CSI-ReportConfig; UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18.

intraFreqL1-MeasConfigSP-CSI-RS-r19 Indicates whether UE supports intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s). This capability signalling comprises of the following parameters: - supportedMaxAperiodic-LTM-CSI-ReportConfig-r19 indicates maximum number of aperiodic LTM-CSI-ReportConfig using semi-persistent CSI-RS as measurement resource; - supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r19 indicates maximum number of semi-persistant LTM-CSI-ReportConfig using semi-persistent CSI-RS as measurement resource; UE supporting this feature shall also indicate support of intraFreqL1-MeasConfigPeriodicCSI-RS-r19. NOTE: The UE must support a non-zero value for at least one of supportedMaxAperiodic-LTM-CSI-ReportConfig-r19 and supportedMaxSemiPersistent-LTM-CSI-ReportConfig-r19.

maxCSI-RS-ResourceL1-Meas-r19 Indicates the max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for L1-RSRP measurement. If UE does not support this feature, there is no limitation on the number of CSI-RS resources for L1 measurement within a slot. A UE supporting this feature shall also indicate support of intraFreqL1-MeasConfigPeriodicCSI-RS-r19. NOTE: The CSI-RS resources of this feature

[Proposed Change]:

Based on the above description, we suggest the following change:

L1-MeasConfigNRDC-r18 ::= SEQUENCE {

maxL1-MeasNoGapSCG-r18                 INTEGER(0..maxNrofL1-MeasNoGap-r18)                               OPTIONAL,

maxL1-MeasWithGapSCG-r18               INTEGER(0..maxNrofL1-MeasWithGap-r18)                             OPTIONAL,

maxCellsL1-MeasNoGapSCG-r18            INTEGER(0..maxNrofCellsL1-MeasNoGap-r18)                          OPTIONAL,

maxCellsL1-MeasWithGapSCG-r18          INTEGER(0..maxNrofCellsL1-MeasWithGap-r18)                        OPTIONAL,

maxTotalCellsL1-MeasNoGapSCG-r18       INTEGER(0..maxNrofTotalCellsL1-MeasNoGap-r18)                     OPTIONAL,

maxSSBsL1-MeasNoGapSCG-r18             INTEGER(0..maxNrofSSBsL1-MeasNoGap-r18)                           OPTIONAL,

maxSSBsL1-MeasWithGapSCG-r18           INTEGER(0..maxNrofSSBsL1-MeasWithGap-r18)                         OPTIONAL,

maxTotalSSBsL1-MeasNoGapSCG-r18        INTEGER(0..maxNrofTotalSSBsL1-MeasNoGap-r18)                      OPTIONAL,

maxCellsL1-MeasIntraFreqSCG-r18        INTEGER(0..maxNrofSSBsL1-MeasIntraFreq-r18)                       OPTIONAL,

maxCellsL1-MeasInterFreqSCG-r18        INTEGER(0..maxNrofSSBsL1-MeasInterFreq-r18)                       OPTIONAL,

maxReportConfigsAperiodic-r18          INTEGER(0..maxNrofReportConfigsAperiodic-r18)                     OPTIONAL,

maxReportConfigsPeriodic-r18           INTEGER(0..maxNrofReportConfigsPeriodic-r18)                      OPTIONAL,

maxReportConfigsSemiPersistent-r18     INTEGER(0..maxNrofReportConfigsSemiPersistent-r18)                OPTIONAL,

...,

[[

maxSSBsL1-MeasNoGapSCGExt-r18          INTEGER(0..maxNrofSSBsL1-MeasNoGapExt-r18)                        OPTIONAL

]],

[[

maxCellsL1-CSIMeasIntraFreq-r19          INTEGER (1..maxNrofCellsL1-CSIMeasIntraFreq-r19)  OPTIONAL,

maxReportConfigsAperiodic-PeriodicCSI-RS-r19         INTEGER(0..maxNrofReportConfigsAperiodic-PeriodicCSI-RS-r19)                     OPTIONAL,

maxReportConfigsPeriodic-PeriodicCSI-RS-r19           INTEGER(0..maxNrofReportConfigsPeriodic-PeriodicCSI-RS-r19)                      OPTIONAL,

maxReportConfigsSP-PeriodicCSI-RS-r19     INTEGER(0..maxNrofReportConfigsSP-PeriodicCSI-RS-r19)                OPTIONAL,

maxReportConfigsAperiodic-SPCSI-RS-r19         INTEGER(0..maxNrofReportConfigsAperiodic-SPCSI-RS-r19)                     OPTIONAL,

maxReportConfigsSP-SPCSI-RS-r19         INTEGER(0..maxNrofReportConfigsSP-SPCSI-RS-r19)                     OPTIONAL,

maxTotalCSI-RS-L1-Meas-r18        INTEGER(0..maxNrofTotalCSI-RS-L1-Meas)                      OPTIONAL,

]]

}

L1-MeasConfigNRDC field descriptions

maxCellsL1-MeasInterFreqSCG Indicates the maximum number of RRC configured LTM candidate cells for intra- and inter-frequency L1 measurement.

maxCellsL1-MeasIntraFreqSCG Indicates the maximum number of RRC configured LTM candidate cells for intra-frequency L1 measurement.

maxCellsL1-MeasNoGapSCG Indicates the maximum number of neighbour cells UE can measure per frequency layer for intra-frequency or inter-frequency L1 measurements without measurement gaps.

maxCellsL1-MeasWithGapSCG Indicates the maximum number of neighbour cells UE can measure per frequency layer for inter-frequency L1 measurements with measurement gaps.

maxL1-MeasNoGapSCG Indicates the maximum number of frequency layers UE can measure for intra- and inter-frequency L1 measurements without measurement gaps.

maxL1-MeasWithGapSCG Indicates the maximum number of frequency layers UE can measure for inter-frequency L1 measurements with measurement gaps.

maxReportConfigsAperiodic Indicates the maximum number of aperiodic LTM CSI report configurations.

maxReportConfigsPeriodic Indicates the maximum number of periodic LTM CSI report configurations.

maxReportConfigsSemiPersistent Indicates the maximum number of semi-persistent LTM CSI report configurations.

maxSSBsL1-MeasNoGapSCG, maxSSBsL1-MeasNoGapSCGExt Indicates the maximum number of SSB resources UE can measure per frequency layer for intra-frequency or inter-frequency L1 measurements without measurement gaps. If the field maxSSBsL1-MeasNoGapSCGExt is included, the field maxSSBsL1-MeasNoGapSCG is not present.

maxSSBsL1-MeasWithGapSCG Indicates the maximum number of SSB resources UE can measure per frequency layer for inter-frequency L1 measurements with measurement gaps.

maxTotalCellsL1-MeasNoGapSCG Indicates the maximum total number of cells, including serving cells and neighboring cells, across all frequency layers of intra-frequency and inter-frequency L1 measurements, UE can measure without measurement gaps.

maxTotalSSBsL1-MeasNoGapSCG Indicates the maximum total number of SSB resources, including serving cells and neighboring cells, across all frequency layers of intra-frequency and inter-frequency L1 measurements, UE can measure without measurement gaps.

MaxIntraFreqCellsConfig Indicates the maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement using periodic CSI-RS resource.

MaxAperiodic-LTM-CSI-ReportConfig-usingPeriodicCSI-RS Indicates the maximum number of aperiodic LTM-CSI-ReportConfig using periodic CSI-RS resource

MaxPeriodic-LTM-CSI-ReportConfig Indicates the maximum number of periodic LTM-CSI-ReportConfig using periodic CSI-RS resource

MaxSP-LTM-CSI-ReportConfig-UsingPeriodicCSI-RS Indicates the maximum number of semi-persistent LTM-CSI-ReportConfig using periodic CSI-RS resource

MaxAperiodic-LTM-CSI-ReportConfig-usingSPCSI-RS Indicates the maximum number of aperiodic LTM-CSI-ReportConfig using semi-persistent CSI-RS.

MaxSP-LTM-CSI-ReportConfig-usingSPCSI-RS-r19 Indicates the maximum number of semi-persistant LTM-CSI-ReportConfig using semi-persistent CSI-RS.

MaxSP-LTM-CSI-ReportConfig-usingSPCSI-RS-r19 Indicates the Maximum total number of CSI-RS resources for L1 measurements for candidate cells and serving cells.

–	Multiplicity and type constraints definitions

-- ASN1START

-- TAG-NR-MULTIPLICITY-AND-CONSTRAINTS-START

maxMeasFreqsMN              INTEGER ::= 32  -- Maximum number of MN-configured measurement frequencies

maxMeasFreqsSN              INTEGER ::= 32  -- Maximum number of SN-configured measurement frequencies

maxMeasIdentitiesMN         INTEGER ::= 62  -- Maximum number of measurement identities that a UE can be configured with

maxCellPrep                 INTEGER ::= 32  -- Maximum number of cells prepared for handover

maxNrofL1-MeasNoGap-r18           INTEGER ::= 8  -- Maximum number of frequencies layers for L1 measurements UE can measure without gaps

maxNrofL1-MeasWithGap-r18         INTEGER ::= 8  -- Maximum number of frequencies layers for L1 measurements UE can measure with gaps

maxNrofCellsL1-MeasNoGap-r18      INTEGER ::= 8  -- Maximum number of neighboring cells for L1 measurements UE can measure without gaps

maxNrofCellsL1-MeasWithGap-r18    INTEGER ::= 8  -- Maximum number of neighboring cells for L1 measurements UE can measure with gaps

maxNrofTotalCellsL1-MeasNoGap-r18 INTEGER ::= 24 -- Maximum total number of cell across all frequencies layers UE can measure

maxNrofSSBsL1-MeasNoGap-r18       INTEGER ::= 8  -- Maximum number of SSB resources for L1 measurements without gaps

maxNrofSSBsL1-MeasNoGapExt-r18    INTEGER ::= 24  -- Maximum number of SSB resources for L1 measurements without gaps

maxNrofSSBsL1-MeasWithGap-r18     INTEGER ::= 8  -- Maximum number of SSB resources for L1 measurements with gaps

maxNrofTotalSSBsL1-MeasNoGap-r18  INTEGER ::= 64 -- Maximum total number of SSB resources for L1 measurements without gaps

maxNrofSSBsL1-MeasIntraFreq-r18   INTEGER ::= 8  -- Maximum number of RRC configured intra-frequency LTM candidate configurations

maxNrofSSBsL1-MeasInterFreq-r18   INTEGER ::= 8  -- Maximum number of RRC configured inter-frequency LTM candidate configurations

maxNrofReportConfigsAperiodic-r18 INTEGER ::= 4  -- Maximum number of aperiodic LTM CSI report configurations

maxNrofReportConfigsPeriodic-r18  INTEGER ::= 4  -- Maximum number of periodic LTM CSI report configurations

maxNrofReportConfigsSemiPersistent-r18  INTEGER ::= 4   -- Maximum number of semi-persistent LTM CSI report configurations

maxNrofCellsTA-Meas-r18           INTEGER ::= 8    -- Maximum number of LTM candidate cells for TA acquisition

maxNrofConfigJointTCI-States-r18  INTEGER ::= 128  -- Maximum number of joint LTM DL TCI states that can be configured

maxNrofConfigDL-TCI-States-r18    INTEGER ::= 128  -- Maximum number of separate LTM DL TCI states that can be configured

maxNrofConfigUL-TCI-States-r18    INTEGER ::= 64   -- Maximum number of separate LTM UL TCI states that can be configured

maxNrofCellsTCI-r18               INTEGER ::= 8    -- Maximum number of configured joint LTM TCI state(s) across candidate cells

maxNrofStoredConfigCells-r18      INTEGER ::= 16   -- Maximum number cells which can be stored by the UE

maxNrofConfigCells-r18            INTEGER ::= 4    -- Maximum number LTM candidate configurations on which UE can fast processing

maxNrofActivatedJointTCI-States-r18  INTEGER ::= 32  -- Maximum number of joint DL TCI states that can be activated via MAC CE

maxNrofActivatedDL-TCI-States-r18    INTEGER ::= 32  -- Maximum number of separate DL TCI states that can be activated via MAC CE

maxNrofActivatedUL-TCI-States-r18    INTEGER ::= 32  -- Maximum number of separate DL TCI states that can be activated via MAC CE

maxNrofCellsL1-CSIMeasIntraFreq-r19            INTEGER ::= 4    -- Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement  using  periodic CSI-RS resource

maxNrofReportConfigsAperiodic-PeriodicCSI-RS-r19 INTEGER ::= 4 -- Maximum number of aperiodic LTM-CSI-ReportConfig using periodic CSI-RS resource

maxNrofReportConfigsPeriodic-PeriodicCSI-RS-r19 INTEGER ::= 4 -- Maximum number of periodic LTM-CSI-ReportConfig using periodic CSI-RS resource

maxNrofReportConfigsSP-PeriodicCSI-RS-r19       INTEGER ::= 4 -- Maximum number of semi-persistant LTM-CSI-ReportConfig using periodic CSI-RS resource

maxNrofReportConfigsAperiodic-SPCSI-RS-r19 INTEGER ::= 4 -- Maximum number of aperiodic LTM-CSI-ReportConfig using semi-persistant CSI-RS resource

maxNrofReportConfigsSP-SPCSI-RS-r19       INTEGER ::= 4 -- Maximum number of semipersistant LTM-CSI-ReportConfig using semi-persistant CSI-RS resource

maxNrofTotalCSI-RS-L1-Meas-r19  INTEGER ::= 64 -- Maximum total number of CSI-RS resources for L1 measurements

-- TAG-NR-MULTIPLICITY-AND-CONSTRAINTS-STOP

-- ASN1STOP

[Comments]:

[Rapporteur] We can check what is needed once the capabilities are stable. Anyway good to bring this up. Good if proponent companies has a contribution on it with a TP

# E053

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E053 | NR_SL_relay_multihop-Core | 1 | The field sl-SRAP-ConfigRelay-ToReleaseList-r19 need to be removed |  | Ericsson - Min |  | V007 | ToDo |

[Description]:

-- ASN1START

-- TAG-SL-L2RELAYUE-CONFIG-START

SL-L2RelayUE-Config-r17 ::=        SEQUENCE {

sl-RemoteUE-ToAddModList-r17       SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17    OPTIONAL,    -- Need N

sl-RemoteUE-ToReleaseList-r17      SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16  OPTIONAL,    -- Need N

...,

[[

sl-U2U-RemoteUE-ToAddModList-r18   SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-U2U-RemoteUE-Config-r18   OPTIONAL,    -- Need N

sl-U2U-RemoteUE-ToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16   OPTIONAL     -- Need N

]]

}

SL-RemoteUE-ToAddMod-r17 ::=       SEQUENCE {

sl-L2IdentityRemote-r17            SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17            SL-SRAP-Config-r17                                                      OPTIONAL,    -- Need M

...,

[[

sl-SRAP-ConfigRelay-ToAddModList-r19     SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-Config-ToAddMod-r19      OPTIONAL,      -- Need R

sl-SRAP-ConfigRelay-ToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-ConfigId-r19               OPTIONAL      -- Need R

]]

}

After further check the relay UE release procedure, the relay UE release a remote UE with SL-DestinationIdentity. The indirectly connected childs associated with the remote UE associated will be released altogether.

##### 5.3.5.15.2	L2 U2N or U2U Remote UE Release

The L2 U2N Relay UE shall:

1>	if the release is triggered by reception of the sl-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5 RRC connection to a L2 U2N Remote UE with SL-DestinationIdentity:

4>	indicate upper layers to trigger PC5 unicast link release.

The L2 U2U Relay UE shall:

1>	if the release is triggered by reception of the sl-U2U-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-U2U-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5-RRC connection to a L2 U2U Remote UE with this SL-DestinationIdentity:

4>	release the configuration associated with the L2 U2U Remote UE.

Therefore, it means that sl-SRAP-ConfigRelay-ToReleaseList-r19 is not really needed.

Generally, it seems there is only marginal benefit of introducing sl-SRAP-ConfigRelay-ToAddModList-r19. Therefore, to simply the procedure and ASN.1,

It is better to Change the field sl-SRAP-ConfigRelay-ToAddModList-r19 to non ToAddMode list.

[Proposed Change]:

Proposed changes include

- Remove the field sl-SRAP-ConfigRelay-ToReleaseList-r19 in IE SL-RemoteUE-ToAddMod-r17

- Change the field sl-SRAP-ConfigRelay-ToAddModList-r19 to non ToAddMode list

[Comments]:

[Rapporteur]:

# V006

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V006 | LPWUS | 2 | Update the RRC parameters based on RAN1 progress. |  | Vivo(Chenli) |  | V010 | Agreed |

[Description]: Some of the RRC paramters from RAN1 are marked as ENUMERATED {ffs}, which should be updated based on the latest RAN1 RRC parameters in the LS in R2-2507007.

[Proposed Change]: Update the specification based on latest RAN1 parameters in R2-2507007.

[Comments]:

# V007

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V007 | LPWUS | 1 | Update the field description of lpwus-ActualDuration based on RAN1 progress. |  | Vivo(Chenli) |  | V010 | Agreed |

[Description]: The field description of lpwus-ActualDuration should be updated based on RAN1 LS in R2-2507007.

[Proposed Change]: Update the field description of lpwus-ActualDuration based on latest RAN1 parameters in R2-2507007.

[Comments]:

# V008

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V008 | LPWUS | 2 | Update the RRC parameters based on RAN1 progress. |  | Vivo(Chenli) |  | V010 | Agreed |

[Description]: Some of the RRC paramters from RAN1 are marked as ENUMERATED {ffs}, which should be updated based on the latest RAN1 RRC parameters in the LS in R2-2507007.

[Proposed Change]: Update the specification based on latest RAN1 parameters in R2-2507007.

[Comments]:

# V052

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V052 | XR | 1 | “Restriction” should be removed for MG cancellation |  | Vivo(Chenli) |  | V010 | ToDo |

[Description]: At the beginning of the discussion, MG cancellation is also applicable for scheduling restriction. But finally, it was not reflected in the other specifications and other WGs. So it should be also removed in the RRC specification.

[Proposed Change]: Remove the “restriction” in the field description of mg-CancellationDCI-0-1 1-2 and 1-3 in PDSCH-Config

[Comments]:

# V053

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V053 | XR | 1 | “Restriction” should be removed for MG cancellation |  | Vivo(Chenli) |  | V010 | ToDo |

[Description]: At the beginning of the discussion, MG cancellation is also applicable for scheduling restriction. But finally, it was not reflected in the other specifications and other WGs. So it should be also removed in the RRC specification.

[Proposed Change]: Remove the “restriction” in the field description of mg-CancellationDCI-0-1 1-2 and 1-3 in PUSCH-Config

[Comments]:

# V054

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V054 | XR | 1 | Update the term of “maxDSR-ReportingThres-r19” |  | Vivo(Chenli) |  | V010 | ToDo |

[Description]: maxDSR-ReportingThres-r19 is defined as the maximum number of reporting thresholds. Thus, in order to avoid the mis-understanding (e.g. max number of DSR), suggest to update it as “maxNrofReportingThresDSR” or something similar.

[Proposed Change]: Update “maxDSR-ReportingThres-r19” as “maxNrofReportingThresDSR” or something similar.

[Comments]:

# V080

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V080 | MIMO | 1 | Update the field description of “csi-ReportUE-IBR” |  | Vivo(Chenli) |  | V010 | ToDo |

[Description]: According to the Table 10.1.6.1-1 in TS38.133, in the field description of “csi-ReportUE-IBR”, when this field is set to event1, the value range of eventThreshold (i.e., “from 14 to 113”) should be revised as “from 16 to 113”.

[Proposed Change]: Update “When this field is set to event1, eventThreshold can only be configured with values from 14 to 113.” as “When this field is set to event1, eventThreshold can only be configured with values from 16 to 113.”.

[Comments]:

# V081

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V081 | MIMO | 1 | Update the field description of “fourPortSRS-3Tx” |  | Vivo(Chenli) |  | V010 | ToDo |

[Description]: According to RRC paramter from RAN1, this field is only applicable for all the 4-port resources in an SRS resource. But the current field description means it can be applicable for all SRS resources in the SRS resource set.

[Proposed Change]: Update the field description of “fourPortSRS-3Tx” as

Indicates whether port 1003 is disabled for all the 4-port resources in an SRS resource set in the SRS resource set with usage set to codebook or antennaSwitching, or whether 3Tx transmission is enabled for a configured SRS resource set with usage set to nonCodebook. This field can only be configured if nrofSRS-Ports for each SRS resources in the SRS resource set is set to ports4.

[Comments]:

# B002

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B002 | RedirToNTN | 1 | Incorrect and missing need code in the description of condition “SIB19” |  | Lenovo (Hyung-Nam) |  | V012 | ToDo |

[Description]: In the description of condition "SIB19" the Need code for the mandatory present case can be removed and Need code for the optionally present case is missing.

[Proposed Change]: Remove “Need R” from the mandatory present case and add “Need R” for the optionally present case as shown below.

| SIB19 | The field is mandatory present, Need R, if SIB19 is not configured in the cell; otherwise the field is optionally present, Need R. |
| --- | --- |

[Comments]:

# B003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B003 | SBFD | 1 | Incorrect constant in the sequence of srs-RSRP-MeasResourceSetToReleaseList-r19 |  | Lenovo (Hyung-Nam) |  | V012 | ToDo |

[Description]: The constant “maxNrofSRS-RSRP-MeasResourceSets-r19” is used in the sequence of srs-RSRP-MeasResourceSetToAddModList-r19. Therefore, to be aligned the constant “maxNrofSRS-RSRP-MeasResourceSets-r19” should be also used in the sequence of srs-RSRP-MeasResourceSetToReleaseList-r19 (instead of “maxNrofCLI-RSSI-MeasResourceSets-r19”).

[Proposed Change]: Replace “maxNrofCLI-RSSI-MeasResourceSets-r19” by “maxNrofSRS-RSRP-MeasResourceSets-r19”.

srs-RSRP-MeasResourceSetToAddModList-r19     SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResourceSets-r19)) OF SRS-RSRP-MeasResourceSet-r19

OPTIONAL, -- Need N

srs-RSRP-MeasResourceSetToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResourceSets-r19)) OF SRS-RSRP-MeasResourceSetId-r19

OPTIONAL, -- Need N

[Comments]:

# E056

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E056 | NTN | 2 | Ineffective use of reference locations | Yes | Ericsson (Philipp) |  | v014 | ToDo |

[Description]: It was agreed to extend the Rel-19 SMTC enhancements to inter-frequency, using as a baseline the Rel-19 agreed solutions for intra-frequency (for both location-based SMTC selection in RRC_IDLE/INACTIVE state and UE-assisted SMTC configuration in RRC_CONNECTED state). The latest CR implementation closely follows the agreement. However, this means signalling of a separate set of reference locations for the serving frequency and each neighbour frequency (for which location-based selection and UE-assisted configuration of SMTCs is relevant), as well as reporting of the N closest reference locations separately per frequency.

Considering the relevant inter-frequency scenarios as concluded in the previous offline discussion on this topic (see the summary in R2-2507763), the current CR implementation leads to very ineffective use of reference locations and has the following drawbacks:

- The UE cannot select among SMTCs associated to multiple frequencies, or report the N closest report reference locations, using a common set of reference locations (for multiple frequencies). Instead, the UE needs to select, or is configured, with a number of SMTC, e.g., N, for each frequency, potentially causing unnecessary effort for neighbour-cell measurement (see Scenario 1).

- A set of reference locations in system information is associated with SMTCs of a single frequency and cannot be reused for (e.g., additionally associated with) SMTCs of another frequency even if the reference locations for the two frequencies are identical (see Scenario 2). Signalling of several (perhaps identical) sets of reference locations is not only inefficient and reducing coverage (by significantly increasing SIB sizes), it may even block the transmission of SIB2/SIB4, due to lacking flexibility in SI scheduling.

[Proposed Change]: Both problems can be addressed by a solution where one common set of reference locations is provided (e.g., in SIB19), and each reference location is mapped to one or multiple smtc5list-r19 items in SIB2 or SIB4. The mapping may be explicitly indicated in the SIBs. This solution provides full flexibility and solves both above problems.

Alternatively, the second problem could be addressed by allowing the omission of refLocList-r19 for a certain frequency even if smtc5list-r19 if configured for that frequency. In this solution, the items of smtc5list-r19 for that frequency are implicitly associated with items of refLocList-r19 for another frequency. This is a simpler solution however addressing only the second problem, not the first one.

[Comments]:

# C032

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C032 | LPWUS | 1 | Missing description of lpwus-OffsetPreferenceConfig for otherConfig | No | CATT (Da Wang) |  | V016 | ToDo |

[Description]: The field description for otherConfig does not include newly defined field lpwus-OffsetPreferenceConfig.

[Proposed Change]: Add description of lpwus-OffsetPreferenceConfig in the field description of otherConfig.

otherConfig Contains configuration related to other configurations. When configured for the SCG, only fields drx-PreferenceConfig, maxBW-PreferenceConfig, maxBW-PreferenceConfigFR2-2, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig, maxMIMO-LayerPreferenceConfigFR2-2, minSchedulingOffsetPreferenceConfig, minSchedulingOffsetPreferenceConfigExt, rlm-RelaxationReportingConfig, bfd-RelaxationReportingConfig, btNameList, wlanNameList, sensorNameList, obtainCommonLocation, idc-AssistanceConfig, multiRx-PreferenceReportingConfigFR2, ul-TrafficInfoReportingConfig, n3c-RelayUE-InfoReportConfig, successPSCell-Config, sn-InitiatedPSCellChange and lpwus-OffsetPreferenceConfig can be included.

[Comments]:

# C033

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C033 | LPWUS | 2 | Changing field timer2 as one optional field | R2-25xxxx | CATT (Da Wang) |  | V016 | ToDo |

[Description]: The field timer2 is optinal present for lpwus-PDCCH-MonitoringTimer.

[Proposed Change]: There are two options:

Option 1: Change field timer2 as one optional field and Need S.

lpwus-PDCCH-MonitoringTimer-r19       SEQUENCE {

timer1-r19                    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, spare2, spare1}

},

timer2-r19                    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, spare2, spare1}

}                    OPTIONAL   -- Need S

}                                                         OPTIONAL    -- Cond Option1-2

}

Option 2: Adding one new condition for field timer2. The new condition, e.g., DRX-SecondaryGroup, means this field is mandatory present when the sencondary DRX Group is configured. It is absent otherwise, Need R. And the field description of lpwus-PDCCH-MonitoringTimer should be updated accordingly.

lpwus-PDCCH-MonitoringTimer-r19       SEQUENCE {

timer1-r19                    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, spare2, spare1}

},

timer2-r19                    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, spare2, spare1}

}              OPTIONAL    -- Cond DRX-SecondaryGroup

}                                                         OPTIONAL    -- Cond Option1-2

}

[Comments]:

# OF001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OF001 | MIMO | 1 | Update the field description of tci-ServCellIndex under CSI-ReportUE-Initiated |  | Ofinno (Hsin-Hsi) |  | V017 | ToDo |

[Description]: According to 38.214 (as quoted below), tci-ServCellIndex (CCofIndicatedTCI in the TS 38.214) is not only used for event 1 and event 2, where the indicated TCI state is used for current beam determination, but tci-ServCellIndex is also used for event 7, where the activated TCI state with the valueOfQ-th best quality is used for current beam determination.

5.2.1.5.4.1c UE Initiated CSI reporting for event 7 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal with the valueOfQ-th highest L1-RSRP out of the reference signals among the activated TCI states (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else

[Proposed Change]:

tci-ServCellIndex Indicates the serving cell on which the indicated TCI state used to determine the current beam RS is applied for event1 and event2. Indicates the serving cell on which the activated TCI state associated with the valueOfQ-th highest L1-RSRP used to determine the current beam RS is applied for event7. (see TS 38.214 [19], clause 5.2.1.5.4).

[Comments]:

# S059

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S059 | SONMDT | 1 | Setting noSuitableCellFound for LTM failure |  | Samsung (Aby) |  | V019 | ToDo |

[Description]:

In the current R19 spec, UE sets noSuitableCellFound for radio link failure or handover failure. This need to be extended for LTM cell switch failure, as it is used in conjunction with the reConnectCellId and timeUntilReconnection for optmisiation purposes.

#### 5.3.7.6	T311 expiry

Upon T311 expiry, the UE shall:

1>	if the procedure was initiated due to radio link failure or handover failure:

2>	set the noSuitableCellFound in the VarRLF-Report to true;

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

[Proposed Change]:

#### 5.3.7.6	T311 expiry

Upon T311 expiry, the UE shall:

1>	if the procedure was initiated due to radio link failure or handover failure or LTM cell switch failure:

2>	set the noSuitableCellFound in the VarRLF-Report to true;

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

[Comments]:

# S060

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S060 | SONMDT | 1 | Target cell need to be associated with ltm-Candidate |  | Samsung (Aby) |  | V020 | ToDo |

[Description]:

Based on the description in the LTM configuration section, ltm-config will be associated to the cell group or PCell/PSCell. It is the LTM-Candidate IE that is associated to the target PCell (or individual LTM candidate cells)

4>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with MCG ltm-Config and LTM-CSI-ReportConfig associated with the target PCell when connected to the source PCell:

5>	set the resultsSSB-Indexes in targetCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement results of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

[Proposed Change]:

4>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with MCG LTM-Candidate and LTM-CSI-ReportConfig associated with the target PCell when connected to the source PCell:

5>	set the resultsSSB-Indexes in targetCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement results of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

[Comments]: