3GPP TSG-RAN WG2 Meeting NR Adhoc-1801						R2-18xxxxx

Vancouver, Canada, 22 – 26 January 2018

Agenda item:	x.x

Source: 	Qualcomm Incorporated

Title: 	Summary for email discussion [100#32][NR] UE capabilities

Document for:		Discussion and decision

# Introduction

In this document we provide summary for the email discussion [100#32][NR] on UE capabilities.

# Phase 1 discussion

  - Discussion point 1: Whether the linking to BPC should be included in the MRDC BCs

It was discussed RAN2#100 if the linking information to BPC should be signalled per MR-DC band combination. A limitation if the linking information is not provided was clarified in the online discussion.

  - MN will not be able to determine when it selects a BPC how many carriers will be available in the other RAT.

The following figure describe the above limitation. In this example, the UE supports different LTE BPCs between EN-DC band combination 1 and 2. This is because the UE would have to use more resources (e.g. processing) in NR side in case of the EN-DC band combination 2 with respect to the EN-DC band combination 1. Now without any linking information (the red arrows), both LTE BPC#1 and #2 are available from eNB’s perspective. The eNB does not know at the time of selecting LTE BPC, how many NR carriers (or total aggregated bandwidth) the LTE BPC#1 or BPC#2 would allow. For example, the eNB may select LTE BPC#1 because it provides better performance for the same 1CC in LTE, but without knowing it only allows 1CC in NR.

Figure-1: BPCs dependent on EN-DC band combination

Companies are asked if the limitation explained above should be address by the standard.

Question 1:	Should this limitation be addressed?

- Yes

- No

| Company | Input | Comment |
| --- | --- | --- |
| Ericsson | No | We agree with the observation that from the ”EN-DC band combinations” and the ”LTE UE capability” the LTE eNB does not know, which of the two entries allows configuring two NR carriers. We had raised this issue a long time ago when companies in RAN2 suggested to maximize separation and abstraction among the RATs. It should be noted that the number of NR carriers is not the only thing that the eNB does not know: The number of NR MIMO layers, the NR modulation orders, the NR Beamforming schemes, the number or width of NR BWPs, ... are all hidden in the NR BPC. Nevertheless, the LTE BPC and the EN-DC BCs allow the eNB to choose an allowed configuration. If a network node wants to be smart, it may try to comprehend the BPC table of the other RAT to make an even better decision. |

Question 2:	If “yes” for Q1, which solution?

- Linking information to BPC is signalled per EN-DC band combination.

- For each LTE BPC, an indication of how many NR carriers (or total aggregated BW) are available is signalled

- Other solution (please explain)

| Company | Input | Comment |
| --- | --- | --- |
| Ericsson | none | Solution A would require including the fallback BCs which RAN2 agreed to avoid: In the example in Figure 1 the UE would have to include not only the top-level “Combination #2: LTE 1CC + NR 2CC” but also its fallback “Combination #1: LTE 1CC + NR 1CC”. And these two BCs would each contain explicit indexes to the same LTE BPC(s) but to different NR BPCs. While this solution adds significant overhead, it makes only a small sub-set of the NR capabilities visible to the LTE eNB. To consider other capabilities, the LTE eNB would anyway have to parse and comprehend the indexed NR BPC entries. And if the eNB wants to do this extra effort, it can do that also without the additional indexes, i.e., without increasing the amount of over-the-air signalling. Solution B would allow the eNB to determine the number of NR carriers using only the EN-DC BCs and the ”LTE” BPCs. But of course, all other baseband processing capabilities are still ”hidden” from the eNB so that it cannot make a good estimate of the achievable performance anyway. Furthermore, this scheme will result in increasing the total number of LTE BPC entries: In the example of Figure 1, the LTE BPC#2 offers ”1 LTE CC, 2 Layer, 64 QAM”. As shown, it could be combined with an NR BPC#2 offering ”2CC, 2 Layer, 64 QAM”. But it may also be compatible with an NR BPC#3 offering e.g. ”1 CC, 4 layer, 256 QAM”. In the currently agreed structure the LTE BPC#2 is compatible and linkable with both. But by including the number-of-NR-carriers into the LTE BPC, such a UE would have to split its LTE BPC#2 into two (one with 1-NR-CC, one with 2-NR-CCs). And finally, Solution B would be against the intention of separating the RAT-specific capabilities from each other, i.e., it would replicate the NR parameter number-of-NR-carriers in the LTE BPC. |

  - Discussion point 2: How to address the information from RAN4 about MIMO capability and intra-band non-contiguous CA in relation to carrier separation

RAN4 requested RAN2 to work on UE capability signalling to enable the following in their LS (R2-1712137/ R4-1711623).

- The UE support of NC CA is conditioned on the frequency separation (gap) between the CCs. This capability may be different for UL NC CA and DL NC CA.

- The MIMO capability for intra-band NC CA may depend on the CC frequency separation supported.

RAN4 also sent another LS indicating their preference to include some form of MIMO layer capability in CA band combination to address possible “constraints on how the number of RF chains can be assigned to different bands” (R4-1714257). Two options are mentioned in the LS (with the disclaimer “other options not precluded”).

- Option 1:	Signal the number of MIMO layers per band per CA band combination for the combinations that have constraints

- Option 2: Signal the maximum number of MIMO layers per CA band combination for the combinations that have constraints

It seems sensible to discuss the above all together to come up with a streamlined UE capability signalling structure.

Question 3:	Should these UE capabilities be implemented in UE capability signalling, and if yes how?

- Yes (please explain solution)

- No

| Company | Input | Comment |
| --- | --- | --- |
| Ericsson | OK (for the intra-B NC with max freq) No (for MIMO in BC) | In LTE, NC-Intra-Band CA is indicated by a BandCombination with two BandEntries with the same band number. We prefer using this signalling variant for UEs that “can support any CC placement”. The UEs that “support NC CA only when CC frequency separation is smaller than full width of the operating band” could instead use the intra-band contiguous CA signalling with a (new) bandwidth class. That class defines both the maximum total bandwidth from edge to edge as well as the total number of supported carriers and served carrier bandwidths. E.g., a UE supporting aggregation of two directly adjacent carriers indicates 1C as today. A UE supporting aggregation of two intra band NC carriers if the total edge-to-edge bandwidth is below a certain limit, indicates e.g. 1CX. RAN4 would define what "class CX” means, such as ”2 carriers, up to 20 MHz each, total bandwidth up to 80 MHz”. They could also define a ”class CY” meaning e.g. ”2 carriers, up to 20 MHz each, total bandwidth up to 160 MHz”. We assume that UEs supporting CY would also support CX and C. UEs supporting CX would also support C. Hence, UEs should omit these fallback BCs. A UE may add different BPCs for ”Class CX” and ”Class C” without duplicating the fallback BC. But since the intention is to operate these two carriers in both cases with a single RX chain, we don't expect UEs will typically do that and just list ”class CX” in the BPC. As agreed, the NW knows that this BPC applies both for BCs with class CX but also to those with class C. If the UE supports also the traditional intra-band non-contiguous CA without limited frequency separation, it will list it as 1A-1A in the BC table since it is not a super- or fallback- BC of 1C or 1CX. And, as suggested by RAN4, this would also allow UEs to indicate that “the MIMO capability for intra-band NC CA may depend on the CC frequency separation supported”: A UE could indicate BPCs with different number of MIMO layer for the 1A-1A case and the 1CX case. As discussed in the previous RAN2 meetings and as captured in RAN2's working assumption, we don't think that the BCs should contain the number of MIMO layers. |

# Phase 2 discussion

Building on top of the discussion (and hopefully conclusion) of the phase 1 discussion, the email discussion moderator would like to propose to further discuss the following aspects.

  - Discussion point 3: Inter-node coordination for BPCs in case of EN-DC

For EN-DC band combination coordination, the following was agreed in RAN2#100. It is left open in the meeting how the BPCs are coordinated in case of EN-DC.

- SCG-ConfigInfo (MN to SN): MN indicates which NR BCs the SN can select by field allowedBandCombinationNR that indicates a list of indices of all the EN-BCs in the MR DC capability container that includes the LTE BC it selected - SCG-Config (SN to MN for the case that the SN wants to request to use a different EN-BC): SN indicates which NR BC the SN wants to use by field requestedBandCombinationNR that indicates a list of indices of all the EN-BC in the MR DC capability container that includes the NR BC it wants to use.

Discussion on the same coordination issue for BPC was postponed in RAN2#100.

[Moderator’s note] Hoping that we can make an informed decision after phase 1 discussion.

| Company | Input | Comment |
| --- | --- | --- |

  - Discussion point 4: Decoupling of DL and UL bands for band combinations

3 options for the decoupling of DL and UL bands were discussed in the email discussion [99bis#28][NR] on UE capability ASN.1 (R2-1712677, also see the appendix of this document). It was suggested to further discuss between the option 1 and option 3.

No conclusion could be reached in RAN2#100, mainly due to lack of clarity on the content of the band combination UE capability signalling (e.g. whether MIMO capability is signalled).

[Moderator’s note] Hoping that we can make an informed decision after phase 1 discussion.

| Company | Input | Comment |
| --- | --- | --- |
| Ericsson | Option 1 | In our contribution R2-1713432 to RAN2-100 we provided an ASN.1 example for option 1 that can be summarized as follows: - each BandCombination comprises of (one or) several Band Entries. - each BandEntry includes an NR or EUTRA band number - each BandEntry contains one downlink bandwidth class - each BandEntry contains a list of uplink bandwidth classes. The lists of UL BWCs in all Band Entries of a BC must have the same length. Entries with the same “position” form a possible UL combination. - in NR and EUTRA Band Entries an uplink bandwidth class may be absent (NULL) indicating that the UE supports (in this band combination and in combination with the set of configured uplink carriers) only a downlink SCell on this band - in an NR Band Entry the downlink bandwidth class may be absent (NULL) indicating that the UE supports (in this band combination) only an Supplementary Uplink carrier on this band Besides being more signalling efficient (fewer “band numbers”, no explicit indexes) the structure is also easy to comprehend since things that belong together are signalled together (not in separate linked lists). |

# Summary

Based on this email discussion, the moderator would like to suggest the following.

xxxxxx

# Appendix (excerpt from Email Discussion on [99bis#28][NR] UE capability ASN.1)

2.1 ASN.1 structure of decoupling of DL and UL bands:

ASN.1 structure for decoupling of DL and UL bands were discussed by the email discussion [1] before RAN2#99bis and online discussion at RAN2#99bis. However it was not concluded. Please see the following part of RAN2#99bis Chairman Notes for remind:

R2-1710609	Email discussion report on [99#24][NR] Decoupling DL band and UL bands	Intel Corporation	discussion	Rel-15	NR_newRAT-Core

=>	Noted

R2-1710691	Further analysis on decoupling DL and UL bands	Intel Corporation	discussion	Rel-15	NR_newRAT-Core

-	DOCOMO think in the email most companies preferred approach 1 and think this could be a viable option. Think it also depends whether the some things such as MIMO capability is included in the BC.

-	Intel think we need to consider if option 3 has a problem with duplicating capabilities. The MIMO aspects needs to be discussed based on other email but think MIMO should be in the BC.

=>	Comeback to discussion after other capability discussion

-	Update from offline: Other discussions have not progressed enough to continue this discussion.

=>	Can be discussed as part of the UE capability email discussion.

Since key issues were already discussed by the previous email discussion and some dependency issues (e.g. whether MIMO capability is included in the BC) were already discussed at RAN2#99bis, it would be good to directly check companies’ preferred option out of the discussed options. Three options were copied from email discussion [1] below. Note compared to the one from [1], there is some modification in ASN.1 for clearer comparison.

Option1: The following example structure is a modified version from the proposed structure in [2]. In each band combination, there is one DL band combination and a set of UL band combinations (UL-BC-List IE in the example) that can be combined with the DL band combination. maxUL-Bandcombinations of UL BCs can be included. With UL-BC-List, it includes corresponding bwClassUL and UL related capabilities. In LTE case, multiple Timing advance capability can be an example of UL capability.

BandCombination_List ::= SEQUENCE (SIZE (1..maxDL-BandCombination)) OF BandCombination

BandCombination ::= SEQUENCE {

bandsDL = [BandX, BandY, BandZ],

bwClassDL = [C,A,A],

UL-BC-List = SEQUENCE (SIZE (1.. maxUL-BandCombinations)) OF UL-BandCombination

}

UL-BandCombination ::= SEQUENCE {

bwClassUL = [A,A,-],

}

Option2: Alternative structure is based on a proposal in [3]. Three band combinations IE groups (1) DL BC, 2) UL BC and 3) DL/UL BC are defined. DL BC includes UE capabilities which are defined per DL band/DL BC. UL BC includes UE capabilities which are defined per UL Band/UL BC. All remaining UE capabilities which are defined per DL/UL BC, are included in DL/UL BC. In each DL BC and UL BC, an Index is included and this index is used in DL/UL BC to link to the corresponding DL BC and UL BC. UL-BC-List is introduced to indicate multiple UL BCs per DL BC similar to option1.

BC_ParameterDL_List ::= SEQUENCE (SIZE (1..maxDL-BandCombination)) OF BC_ParameterDL

BC_ParameterUL_List ::= SEQUENCE (SIZE (1..maxUL-BandCombination)) OF BC_ParameterUL

BandCombination_List ::= SEQUENCE (SIZE (1..maxDL-BandCombination)) OF BandCombination

BC_ParameterDL ::= SEQUENCE {

bandsDL = [BandX, BandY, BandZ],

bwClassDL = [C,A,A]

}

BC_ParameterUL ::= SEQUENCE {

bandsUL = [BandX, BandY],

bwClassUL = [A,A]

}

BandCombination ::= SEQUENCE {

bc_ParameterDL_Index,	// Index = order

UL-BC-List = SEQUENCE (SIZE (1.. maxUL-BandCombinations)) OF bc_ParameterUL_Index

}

Option3: The combination between option1 and option2. It only has UL index (as the format of BIT STRING).

BC_ParameterUL_List ::= SEQUENCE (SIZE (1..maxUL-BandCombination)) OF BC_ParameterUL

BandCombination_List ::= SEQUENCE (SIZE (1..maxDL-BandCombination)) OF BandCombination

BC_ParameterUL ::= SEQUENCE {

bandsUL = [BandX, BandY],

bwClassUL = [A,A]

}

BandCombination ::= SEQUENCE {

bandsDL = [BandX, BandY, BandZ],

bwClassDL = [C,A,A],

ul-BC-List = BIT STRING (SIZE (1.. maxUL-BandCombinations))

}

Q1: Companies are encouraged to provide the input on the preferred option out of three options above.

| Option1 (List company name) | Option2 (List company name) | Option3 (List company name) | No strong preference (List company name) | Additional comments |
| --- | --- | --- | --- | --- |
| NTT DOCOMO |  |  |  | Given that RAN2 aims to take capability signalling away from the band combination, the advantage of Option 2/3 is diminishing? |
|  |  | Qualcomm Incorporated |  | We think using a list of indexes to UL band combination, as opposed to the BIT STRING, is better in term of overhead, because the UE may support a large number of UL band combinations, while the number of UL band combinations linked to a DL band combination is rather limited. |
|  |  | Huawei, HiSilicon |  | As in discussion [99#24] previously, Option 3 avoids the duplication of UL BC information. The only duplication should be if we have different DL capabilities (other than MIMO) for the same DL BC depending on the UL BC. |
|  |  | MediaTek |  | We believe bit string is the most efficient way to avoid UL BC across DL BC. |
| Ericsson |  |  |  | Options 2 and 3 may result in a lot of overhead: Example: A UE supports in total 10 bands and UL CA of any 3 out of those 10 bands. It might advertise up to 120 UL BCs (without any fallback BCs!). Hence, the ul-BC-List bit string has to be 128 bit. If this UE supports DL CA of any 5 bands, it advertises up to 252 DL band combinations (without any fallback BCs!). Hence, the 128-bit bit-string would be repeated up to 252 times, i.e., 4000 Byte. Furthermore, the band numbers (BandX, BandY) would be listed both in the DL and in the UL BCs |
| Nokia | Nokia | - |  | - Option 1 clearly provides the baseline gain from the reference signalling model in LTE. - An example (256 combinations of DL and 32 combinations of UL band parameters): Assume for each DL entry there are at least 2 or 3 UL combinations possible: Option 2: Size of Band Combination entry in bits (8 bits DL index + 5*3 UL index) 23 bits Option 3: Each Band Combination entry requires 8 bits + 5 bits 13 bits) It seems difficult how UE can partially fill in bitmap when asked to only report subset of band combinations. Essentially the bit string fixes a position for the UL-BC and then the order is fixed so partial reporting does not seem to be allowed. If the number of UL-BCs are very high then the bit string approach is attractive and the indexing approach starts becoming prohibitive. Conversely, for small number of UL-BCs, the difference between Option 2 and 3 is not much. One issue with Option 3 is that the bit map cannot reflect unambiguously when the network is requesting a subset of BCs, if a UL-BC is supported or not (a 0 is indicated in the bitmap and cannot clearly discriminate if the UE did not support the UL BC or did not simply fill it because the network asked it to be filtered away). We support Option 2 because it is easier to extend to changes in the future compared to Option 3. Note: FFS how fallback combinations are indicated |

[Observation1]: Based on the companies’ inputs, there is no clear majority companies’ preference. However option2 has only one supporting company out of seven companies, so RAN2 is asked to continue the discussion to select the preferred option between option1 and option3.

[Proposal1]: RAN2 is asked to continue the discussion to select the preferred option between option1 and option3 for decoupling of DL and UL bands.