3GPP TSG-RAN WG2#101	DRAFT R2-180xxxx

# Athens, Greece, 26th Feb. – 2nd Mar. 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0005 | rev | 6 | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update UE capabilities according to the following agreements (RAN2 NR AH and RAN2#101): - Linking MR-DC BCs to BPCs (R2-1800909) - BC structure with UL and DL decoupling (R2-181620) - Clarifications on BPC capabilities (R2-1801532) - UE capabilities on dynamic power sharing (R2-1801520) - L2/3 capabilities (R2-1801608) - I.044 (R2-1800955) - N.037, N.038, N.040, N.045, N.046, N.221, N.222 (R2-1800831) - 5.6, 5.7 to 5.7.1 (E.027), 6.3.3 (C.033, M.052, M.054, Z.078, I.078, I.083. H.277) in RIL 38.331 - Update UE capabilities according to the following agreements (RAN2#101) - Add UE capabilities on L1/RF/RRM features in accordance with RAN1/4 inputs in R1-1803513 and R4-1803564. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove “FFS if supportedBasebandProcessingCombination-MRDC is included here or BandCombinationList” in the ASN.1. - Fill in the blanks of BandCombinationList with UL and DL decoupling. - Updates supportedBW-PerCC and modulationOrder as per CC capability. - Add dynamicPowerSharing in MR-DC container. - Update L2/3 capabilities: - Relocate intraBandAsyncFDD into RF-Parameters in MR-DC container. - Add intraAndInterF-MeasAndReport and eventA-MeasAndReport into MeasParameters in UE-NR-Capability. - Add splitSRB-WithOneUL-Path and directSN-Addition into generalParameters in MR-DC container. - Add fdd-UE-MRDC-Capability, tdd-UE-MRDC-Capability, fdd-UE-NR-Capability, and tdd-UE-NR-Capability. - Remove “FFS utra, geran-cs, geran-ps and cdma2000-1XRTT” - Remove volteOverNR-PDCP from NR ASN.1 - Change MR-DC to eutra-nr in RAT-Type, add FreqBandList IE and replace requestedFreqBandList by FreqBandList in 5.6.1.4. - Update the following changes: - Change sentence to “if FreqBandList is received:” in 5.6.1.4. - Change maxSimultaneousBands to maxRequestedBands. - Add “MN” and “SN” into each basebandProcessingCombinationIndex in LinkedBasebandProcessingCombination. - Change maxServCell to maxNrofCC. - Change subCarrierSpacing to supportedSubCarrierSpacingList. - Clarify maxRateDRB-IP is removed. - Remove SupportedBandCombination ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination. - Update field description to remove E-UTRA in RAT-Type. - Update the following changes: - Add ENUMERATED {supported} for multipleSR-Configurations and multipleConfiguredGrantConfigurations. - Remove directSN-Addition and dataRateDRB-IP. - Add splitDRB-withUL-Both-MCG-SCG and srb3 into UE-MRDC-Capability. - Move tdm-Pattern from UE-EUTRA-Capability to UE-MRDC-Capability. - Add UECapabilityInformation message. - Implement RAN1/RAN4 type 3 parameters that is per band per band combination into the “BPC” structure. - Add explicit linking from the RF band combinations to BPCs. - Add additional SFTD related capability into UE-MRDC-Capability. - Add UE capabilities on L1/RF/RRM features |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

The First Change

5.6	UE capabilities

5.6.1	UE capability transfer

5.6.1.1	General

#### Editor’s Note: Targeted for completion in June 2018

#### 5.6.1.2	Initiation

Editor’s Note: Targeted for completion in June 2018.

5.6.1.3	Reception of the UECapabilityEnquiry by the UE

Editor’s Note: Targeted for completion in June 2018.

5.6.1.4	Compilation of band combinations supported by the UE

The UE shall:

1>	if includes FreqBandList is received:

2>	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message,  only consisting of bands included in FreqBandList, and prioritized in the order of FreqBandList, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

2>	for each band combination included in the candidate list:

3>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of candidates as specified in TS 38.306 [xx]:

4>	remove the band combination from the list of candidates;

2>	include all band combinations in the candidate list into supportedBandCombination;

1>	else:

2> include all band combinations supported by the UE into supportedBandCombination, excluding fallback band combinations with the same capabilities of another band combination included in the list of band combinations supported by the UE;

5.6.1.5	Compilation of baseband processing combinations supported by the UE

The UE shall:

1>	for each band combination included in supportedBandCombination:

2>	include the baseband processing combination supported for the band combination into supportedBasebandProcessingCombination, unless it is already included;

2>	if there are the fallback baseband processing combinations of this baseband processing combination as specified in TS 38.306 [xx] for which supported baseband capabilities are different from this baseband processing combination:

3>	include only these baseband processing combinations into supportedBasebandProcessingCombination;

The Next Change

## 6.2	RRC messages

### 6.2.1	General message structure

#### –	NR-RRC-Definitions

This ASN.1 segment is the start of the NR RRC PDU definitions.

-- ASN1START

-- TAG-NR-RRC-DEFINITIONSSTART

NR-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TAG-NR-RRC-DEFINITIONS-STOP

-- ASN1STOP

#### –	BCCH-BCH-Message

The BCCH-BCH-Message class is the set of RRC messages that may be sent from the network to the UE via BCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-BCH-MESSAGE-START

BCCH-BCH-Message ::= SEQUENCE {

message											BCCH-BCH-MessageType

}

BCCH-BCH-MessageType ::= CHOICE {

mib												MIB,

messageClassExtension	SEQUENCE {}

}

-- TAG-BCCH-BCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the downlink DCCH logical channel.

-- ASN1START

-- TAG-DL-DCCH-MESSAGE-START

DL-DCCH-Message ::= SEQUENCE {

message											DL-DCCH-MessageType

}

DL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

rrcReconfiguration							RRCReconfiguration,

spare15 NULL, spare14 NULL, spare13 NULL,

spare12 NULL, spare11 NULL, spare10 NULL,

spare9 NULL, spare8 NULL, spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-DL-DCCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::= SEQUENCE {

message											UL-DCCH-MessageType

}

UL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

measurementReport							MeasurementReport,

rrcReconfigurationComplete					RRCReconfigurationComplete,

ueCapabilityInformation						UECapabilityInformation,

spare13 NULL, spare12 NULL,

spare11 NULL, spare10 NULL, spare9 NULL,

spare8 NULL, spare7 NULL, spare6 NULL,

spare5 NULL, spare4 NULL, spare3 NULL,

spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

The Next Change

### 6.2.2	Message definitions

### << skip irrelevant part >>

#### –	UECapabilityInformation

The UECapabilityInformation message is used to transfer of UE radio access capabilities requested by the NG-RAN.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to NG-RAN

UECapabilityInformation message

-- ASN1START

-- TAG-UECAPABILITYINFORMATION-START

UECapabilityInformation ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

ueCapabilityInformation				UECapabilityInformation-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

UECapabilityInformation-IEs ::=	SEQUENCE {

ue-CapabilityRAT-ContainerList		UE-CapabilityRAT-ContainerList,

lateNonCriticalExtension           OCTET STRING                          	OPTIONAL,

nonCriticalExtension				SEQUENCE {}								OPTIONAL

}

-- TAG-UECAPABILITYINFORMATION-STOP

-- ASN1STOP

The Next Change

### 6.3.3	UE capability information elements

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=	SEQUENCE (SIZE (1..maxBandCombDL)) OF BandCombination

BandCombination ::= SEQUENCE {

bandAndDL-ParametersList			BandAndDL-ParametersList,

bandCombinationsUL					BIT STRING (SIZE (1.. maxBandCombUL)),

bandCombinationParametersList		SEQUENCE (SIZE (1..maxBandCombUL)) OF BandCombinationParameters 	OPTIONAL

}

BandAndDL-ParametersList ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandAndDL-Parameters

BandAndDL-Parameters ::= CHOICE {

bandAndDL-ParametersEUTRA	BandAndDL-ParametersEUTRA,

bandAndDL-ParametersNR		BandAndDL-ParametersNR

}

BandCombinationParameters ::= SEQUENCE {

ca-ParametersNR					CA-ParametersNR			OPTIONAL,

mrdc-Parameters					MRDC-Parameters			OPTIONAL

}

CA-ParametersNR ::=	SEQUENCE {

multipleTimingAdvances		ENUMERATED {supported}				OPTIONAL

-- R4 2-5: Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD)

simultaneousRxTxInterBandCA			ENUMERATED {supported}		OPTIONAL,

-- BCS related to R4 2-1 and Updated CA BW class in R4-1803374

supportedBandwidthCombinationSet	BIT STRING (SIZE (1..32))	OPTIONAL

}

MRDC-Parameters ::=	SEQUENCE {

singleUL-Transmission		ENUMERATED {supported}		OPTIONAL,

-- R4 1-10: Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective

ul-SharingEUTRA-NR					ENUMERATED {supported}		OPTIONAL,

-- R4 1-11: Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective

ul-SwitchingTimeEUTRA-NR			ENUMERATED {type1, type2}	OPTIONAL,

-- R4 2-4: Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD)

simultaneousRxTxInterBandENDC		ENUMERATED {supported}		OPTIONAL,

-- R4 2-6: Asynchronous FDD-FDD intra-band EN-DC

asyncIntraBandENDC					ENUMERATED {supported}		OPTIONAL,

basebandProcesingCombinationMRDC 	BasebandProcessingCombinationMRDC

}

BandAndDL-ParametersEUTRA ::= SEQUENCE {

bandEUTRA					FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA	CA-BandwidthClassEUTRA		OPTIONAL,

intraBandContiguousCC-InfoDL-EUTRA-List		SEQUENCE (SIZE (1..maxServCell)) OF IntraBandContiguousCC-InfoDL-EUTRA

}

BandAndDL-ParametersNR ::= SEQUENCE {

bandNR						FreqBandIndicatorNR,

ca-BandwidthClassDL			CA-BandwidthClassNR			OPTIONAL,

-- R4 2-3: Non-contiguous intra-band CA frequency separation class for FR2 as in the RAN4 LS R4-1803363

intraBandFreqSeparationDL	FreqSeparationClass					OPTIONAL,

intraBandContiguousCC-InfoDL-List		SEQUENCE (SIZE (1..maxServCell)) OF IntraBandContiguousCC-InfoDL

}

IntraBandContiguousCC-InfoDL ::=	SEQUENCE {

-- Related to RAN4 LS R2-1804078

maxNumberMIMO-LayersPDSCH			MIMO-LayersDL					OPTIONAL

}

IntraBandContiguousCC-InfoDL-EUTRA ::=	SEQUENCE {

-- Related to RAN4 LS R2-1804078

MIMO-CapabilityDL					ENUMERATED {twoLayers, fourLayers, eightLayers}					OPTIONAL

}

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

#### –	BandCombinationParametersUL-List

The IE BandCombinationParametersUL-List is used to contain list of NR and/or E-UTRA frequency UL band parameters combination for the supported NR CA and/or MR-DC band combinations included in supportedBandCombination in RF-Parameters and/or RF-Parameters-MRDC.

-- ASN1START

-- TAG-BANDCOMBINATIONPARAMETERSULLIST-START

BandCombinationParametersUL-List ::= SEQUENCE (SIZE (1..maxBandCombUL)) OF BandCombinationParametersUL

BandCombinationParametersUL ::= SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF BandParametersUL

BandParametersUL ::= CHOICE {

bandParametersUL-EUTRA		BandParametersUL-EUTRA,

bandParametersUL-NR			BandParametersUL-NR

}

BandParametersUL-EUTRA ::= SEQUENCE {

ca-BandwidthClassUL-EUTRA	CA-BandwidthClassEUTRA		OPTIONAL,

intraBandContiguousCC-InfoUL-EUTRA-List		SEQUENCE (SIZE (1..maxServCell)) OF IntraBandContiguousCC-InfoUL-EUTRA

}

BandParametersUL-NR ::= SEQUENCE {

ca-BandwidthClassUL			CA-BandwidthClassNR 				OPTIONAL,

-- R4 2-3: Non-contiguous intra-band CA frequency separation class for FR2 as in the RAN4 LS R4-1803363

intraBandFreqSeparationUL	FreqSeparationClass					OPTIONAL,

intraBandContiguousCC-InfoUL-List		SEQUENCE (SIZE (1..maxServCell)) OF IntraBandContiguousCC-InfoUL

}

IntraBandContiguousCC-InfoUL ::=	SEQUENCE {

-- Related to RAN4 LS R2-1804078

maxNumberMIMO-LayersCB-PUSCH		MIMO-LayersUL					OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH		MIMO-LayersUL					OPTIONAL

}

IntraBandContiguousCC-InfoUL-EUTRA ::=	SEQUENCE {

-- Related to RAN4 LS R2-1804078

MIMO-CapabilityUL						ENUMERATED {twoLayers, fourLayers}					OPTIONAL

}

-- TAG-BANDCOMBINATIONPARAMETERSULLIST-STOP

-- ASN1STOP

#### –

#### –	BasebandCombinationParametersUL-List

-- ASN1START

-- TAG-BASEBANDCOMBINATIONPARAMETERSULLIST-START

BasebandCombinationParametersUL-List ::= SEQUENCE (SIZE (1..maxBasebandProcCombUL)) OF BasebandCombinationParametersUL

BasebandCombinationParametersUL ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BasebandParametersPerBandUL

BasebandParametersPerBandUL ::= SEQUENCE {

ca-BandwidthClassUL				CA-BandwidthClassNR,

freqRange						ENUMERATED {fr1, fr2},

scalingFactor0dot75				ENUMERATED {supported}		OPTIONAL,	-- RAN1 confirmation is needed

basebandParametersPerCC-UL		SEQUENCE (SIZE (1..maxNrofCC)) OF BasebandParametersPerCC-UL

}

BasebandParametersPerCC-UL ::= SEQUENCE {

-- R4 2-2: Simultaneous reception or transmission with same or  different numerologies in CA

-- It is expressed by the combination of SCS whether simultaneous RxTx is supported or not.

supportedSubcarrierSpacingUL				SubcarrierSpacing,

-- R1 2-14: Codebook based PUSCH MIMO transmission. Absence of this field implies that CB-based PUSCH is not supported.

maxNumberMIMO-LayersCB-PUSCH		MIMO-LayersUL					OPTIONAL,

-- R1 2-15: Non-codebook based PUSCH MIMO transmission. Absence of this field implies that Non-CB-based PUSCH is not supported.

maxNumberMIMO-LayersNonCB-PUSCH		MIMO-LayersUL					OPTIONAL,

-- Accoding to the RAN4 LS R4-1803563, modulation order is added per CC granularity in BPC

-- FFS whether all of modulation order specified in the spec need to be signalled.

-- FFS how to address the requirements agreed by RAN4, e.g. mandaotry w/o capabiltiy for 64QAM. mandaotry with capabiltiy for DL 256QAM in FR1.

supportedModulationOrderUL					ModulationOrder					OPTIONAL,

-- R1 2-53: SRS resources

supportedSRS-Resources			SRS-Resources								OPTIONAL,

-- R1 2-55: SRS Tx switch

srs-TxSwitch					SRS-TxSwitch								OPTIONAL,

-- R1 2-57: Support low latency CSI feedback

lowLatencyCSI-Feedback				ENUMERATED {supported}					OPTIONAL,

-- R1 5-12 & 5-12a: Up to 2/7 PUSCHs per slot for different TBs

pusch-DifferentTB-PerSlot		SEQUENCE {

scs-15kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-30kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-60kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-120kHz						ENUMERATED {upto2, upto7}					OPTIONAL

}																			OPTIONAL,

-- R1 6-7: Two PUCCH group

twoPUCCH-Group					ENUMERATED {supported}						OPTIONAL,

-- R1 6-8: Different numerology across PUCCH groups

diffNumerologyAcrossPUCCH-Group	ENUMERATED {supported}						OPTIONAL,

-- R1 6-9: Different numerologies across carriers within the same PUCCH group

diffNumerologyWithinPUCCH-Group	ENUMERATED {supported}						OPTIONAL,

-- R1 6-10: Cross carrier scheduling

crossCarrierScheduling			ENUMERATED {supported}						OPTIONAL,

-- R1 6-11: Number of supported TAGs

supportedNumberTAG				ENUMERATED {n2, n3, n4}						OPTIONAL,

-- R1 6-18: Supplemental uplink with dynamic switch

dynamicSwitchSUL				ENUMERATED {supported}						OPTIONAL,

-- R1 6-19: Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS/PRACH on the other UL carrier in the same cell

-- Details on the channel/signal combination are to be described in TS 38.306

simultaneousTxSUL-NonSUL		ENUMERATED {supported}						OPTIONAL,

-- R1 6-22: UL search space sharing for CA

searchSpaceSharingCA-UL			ENUMERATED {supported})						OPTIONAL

}

-- TAG-BASEBANDCOMBINATIONPARAMETERSULLIST-STOP

-- ASN1STOP

#### –	BasebandProcessingCombinationMRDC

-- ASN1START

-- TAG-BASEBANDPROCESSINGCOMBINATIONMRDC-START

BasebandProcessingCombinationMRDC ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationLink

BasebandProcessingCombinationLink ::= SEQUENCE {

basebandProcessingCombinationIndexMN		BasebandProcessingCombinationIndex,

basebandProcessingCombinationLinkedIndexSN	SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationIndex

}

BasebandProcessingCombinationIndex ::= INTEGER (1..maxBasebandProcComb)

-- TAG-BASEBANDPROCESSINGCOMBINATIONMRDC-STOP

-- ASN1STOP

#### –	CA-BandwidthClassNR

-- ASN1START

-- TAG-CA-BANDWIDTHCLASS-START

-- Updated based on R4-1803374

CA-BandwidthClassNR ::= ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...}

-- TAG-CA-BANDWIDTHCLASS-STOP

-- ASN1STOP

#### –	CA-BandwidthClassEUTRA

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSEUTRA-START

CA-BandwidthClassEUTRA ::=	ENUMERATED {a, b, c, d, e, f, ...}

-- TAG-CA-BANDWIDTHCLASSEUTRA-STOP

-- ASN1STOP

#### –

#### –	FreqBandIndicatorEUTRA

-- ASN1START

-- TAG-FREQ-BAND-INDICATOR-EUTRA-START

FreqBandIndicatorEUTRA ::=	INTEGER (1..maxFBI-EUTRA)

-- TAG-FREQ-BAND-INDICATOR-EUTRA-STOP

-- ASN1STOP

#### –	FreqBandList

The IE FreqBandList is used to contain list of NR and/or E-UTRA frequency bands for which the UE is requested to provide its supported NR CA and/or MR-DC band combinations (i.e. within the UE capability containers for NR and MR-DC, as requested by E-UTRA).

FreqBandList information element

-- ASN1START

-- TAG-FREQBANDLIST-START

FreqBandList ::=	SEQUENCE (SIZE (1..maxRequestedBands)) OF FreqBandInformation

FreqBandInformation ::= CHOICE {

bandEUTRA				FreqBandIndicatorEUTRA,

bandNR					FreqBandIndicatorNR

}

-- TAG-FREQBANDLIST-STOP

-- ASN1STOP

#### –	FreqSeparationClass

The IE FreqSeparationClass is used for an intra-band non-contiguous CA band combination to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in a frequency band.

FreqSeparationClass information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASS-START

FreqSeparationClass ::=	ENUMERATED {c1, c2, c3, ...}

-- TAG-FREQSEPARATIONCLASS-STOP

-- ASN1STOP

#### –	MIMO-Layers

-- ASN1START

-- TAG-MIMO-Layers-START

MIMO-LayersDL ::=	ENUMERATED {twoLayers, fourLayers, eightLayers}

MIMO-LayersUL ::=	ENUMERATED {oneLayer, twoLayers, fourLayers}

-- TAG-MIMO-Layers-STOP

-- ASN1STOP

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, eutra-nr, spare2, spare1, ...}

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

#### –	SupportedBasebandProcessingCombination

-- ASN1START

-- TAG-SUPPORTEDBASEBANDPROCESSINGCOMBINATION-START

SupportedBasebandProcessingCombination ::= SEQUENCE (SIZE (1..maxBasebandProcCombDL)) OF BasebandProcessingCombination

BasebandProcessingCombination ::= SEQUENCE {

basebandParametersDL			SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BasebandParametersPerBandDL,

basebandParametersUL				BIT STRING (SIZE (1..maxBasebandProcCombUL))

}

BasebandParametersPerBandDL ::= SEQUENCE {

ca-BandwidthClassDL				CA-BandwidthClassNR		,

freqRange					ENUMERATED {fr1, fr2},

scalingFactor0dot75			ENUMERATED {supported}		OPTIONAL,	-- RAN1 confirmation is needed

basebandParametersPerCC-DL			SEQUENCE (SIZE (1..maxNrofCC)) OF BasebandParametersPerCC-DL

}

BasebandParametersPerCC-DL ::= SEQUENCE {

-- R4 2-2: Simultaneous reception or transmission with same or  different numerologies in CA

-- It is expressed by the combination of SCS whether simultaneous RxTx is supported or not.

supportedSubcarrierSpacingDL				SubcarrierSpacing,

-- R1 2-2: PDSCH beam switching

timeDurationForQCL					SEQUENCE {

scs-60kHz							ENUMERATED {s7, s14, s28}				OPTIONAL,

sch-120kHz							ENUMERATED {s14, s28}					OPTIONAL

}																			OPTIONAL,

-- R1 1-10: Support of SCell without SS/PBCH block

scellWithoutSSB					ENUMERATED {supported}						OPTIONAL,

-- R1 1-11: Support of CSI-RS RRM measurement for SCell without SS/PBCH block

csi-RS-MeasSCellWithoutSSB		ENUMERATED {supported}						OPTIONAL,

-- R1 2-3: PDSCH MIMO layers. Absence of this field implies support of one layer.

maxNumberMIMO-LayersPDSCH			MIMO-LayerDL							OPTIONAL,

-- Accoding to the RAN4 LS R4-1803563, modulation order is added per CC granularity in BPC

-- FFS whether all of modulation order specified in the spec need to be signalled.

-- FFS how to address the requirements agreed by RAN4, e.g. mandaotry w/o capabiltiy for 64QAM. mandaotry with capabiltiy for DL 256QAM in FR1.

supportedModulationOrderDL					ModulationOrder					OPTIONAL,

-- R1 2-15a: Association between CSI-RS and SRS

srs-AssocCSI-RS					ENUMERATED {supported}						OPTIONAL,

-- R1 3-1a: For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2

type1-3-CSS						ENUMERATED {supported}						OPTIONAL,

-- R1 3-5 & 3-5a: For type 1 with dedicated RRC configuration, type 3, and UE-SS,, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 (with a DCI gap)

pdcchMonitoringAnyOccasions		ENUMERATED {withoutDCI-gap, withDCI-gap}	OPTIONAL,

-- R1 5-1a: UE specific RRC configure UL/DL assignment

ue-SpecificUL-DL-Assignment		ENUMERATED {supported}						OPTIONAL,

-- R1 5-11 & 5-11a: Up to 2/7 unicast PDSCHs per slot for different TBs

pdsch-DifferentTB-PerSlot		SEQUENCE {

scs-15kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-30kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-60kHz						ENUMERATED {upto2, upto7}					OPTIONAL,

scs-120kHz						ENUMERATED {upto2, upto7}					OPTIONAL

}																			OPTIONAL,

-- R1 6-10: Cross carrier scheduling

crossCarrierScheduling			ENUMERATED {supported}						OPTIONAL,

-- R1 6-21: DL search space sharing for CA

searchSpaceSharingCA-DL			ENUMERATED {supported}						OPTIONAL

}

-- TAG-SUPPORTEDBASEBANDPROCESSINGCOMBINATION-STOP

-- ASN1STOP

#### –	ModulationOrder

-- ASN1START

-- TAG-MODULATION-ORDER-START

ModulationOrder ::=	ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

-- TAG-MODULATION-ORDER-STOP

-- ASN1STOP

#### –

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of containers, one for each RAT for which UE capabilities are transferred, if any.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-START

UE-CapabilityRAT-ContainerList ::=SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::= SEQUENCE {

rat-Type							RAT-Type,

ue-CapabilityRAT-Container			OCTET STRING

}

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For NR: the encoding of UE capabilities is defined in UE-NR-Capability. For EUTRA-NR: the encoding of UE capabilities is defined in UE-MRDC-Capability

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [yy].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=	SEQUENCE {

measParameters-MRDC					MeasParameters-MRDC					OPTIONAL,

rf-Parameters-MRDC					RF-Parameters-MRDC,

phy-Parameters-MRDC					Phy-Parameters-MRDC					OPTIONAL,

generalParameters-MRDC				GeneralParameters-MRDC-XDD-Diff		OPTIONAL,

fdd-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddXDD-Mode		OPTIONAL,

tdd-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddXDD-Mode		OPTIONAL,

fr1-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddFRX-Mode		OPTIONAL,

fr2-Add-UE-MRDC-Capabilities		UE-MRDC-CapabilityAddFRX-Mode		OPTIONAL,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-Parameters-MRDC-XDD-Diff		Phy-Parameters-MRDC-XDD-Diff		OPTIONAL,

measParameters-MRDC-XDD-Diff		MeasParameters-MRDC-XDD-Diff		OPTIONAL,

generalParameters-MRDC-XDD-Diff		GeneralParameters-MRDC-XDD-Diff		OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-Parameters-MRDC-FRX-Diff		Phy-Parameters-MRDC-FRX-Diff		OPTIONAL

}

RF-Parameters-MRDC ::= SEQUENCE {

supportedBandCombination	BandCombinationList,

bandCombinationParametersUL-List		BandCombinationParametersUL-List,

}

Phy-Parameters-MRDC ::= SEQUENCE {

phy-Parameters-MRDC-XDD-Diff		Phy-Parameters-MRDC-XDD-Diff		OPTIONAL,

phy-Parameters-MRDC-FRX-Diff		Phy-Parameters-MRDC-FRX-Diff		OPTIONAL

}

Phy-Parameters-MRDC-XDD-Diff ::=	SEQUENCE {

-- R1 8-1: Dynamic power sharing for LTE-NR DC

dynamicPowerSharing					ENUMERATED {supported}				OPTIONAL,

-- R1 6-13 & 8-2: Case 1 Single Tx UL LTE-NR DC

tdm-Pattern							ENUMERATED {supported}				OPTIONAL

}

Phy-Parameters-MRDC-FRX-Diff ::=	SEQUENCE {

-- R1 8-1: Dynamic power sharing for LTE-NR DC

dynamicPowerSharing					ENUMERATED {supported}				OPTIONAL,

-- R1 6-13 & 8-2: Case 1 Single Tx UL LTE-NR DC

tdm-Pattern							ENUMERATED {supported}				OPTIONAL

}

MeasParameters-MRDC ::= SEQUENCE {

measParameters-MRDC-Common			MeasParameters-MRDC-Common,

measParameters-MRDC-XDD-Diff		MeasParameters-MRDC-XDD-Diff			OPTIONAL

}

MeasParameters-MRDC-Common ::=	SEQUENCE {

-- R4 3-1: Independent measurement gap configurations for FR1 and FR2

independentGapConfig			ENUMERATED {supported}	OPTIONAL

}

MeasParameters-MRDC-XDD-Diff ::=	SEQUENCE {

-- R4 3-2: Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement

simultaneousRxDataSSB-DiffNumerology	ENUMERATED {supported}		OPTIONAL,

independentGapConfig		ENUMERATED {supported}	OPTIONAL,

sftd-MeasPSCell								ENUMERATED {supported}		OPTIONAL,

sftd-MeasNR-Cell						ENUMERATED {supported}		OPTIONAL

}

GeneralParameters-MRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path				ENUMERATED {supported}		OPTIONAL,

splitDRB-withUL-Both-MCG-SCG		ENUMERATED {supported}		OPTIONAL,

srb3									ENUMERATED {supported}		OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [yy].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters						OPTIONAL,

mac-Parameters					MAC-Parameters						OPTIONAL,

phy-Parameters					Phy-Parameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities			XDD-UE-NR-CapabilityAddXDD-Mode				OPTIONAL,

tdd-Add-UE-NR-Capabilities			XDD-UE-NR-CapabilityAddXDD-Mode				OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff			OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff			OPTIONAL,

measParametersXDD-Diff			MeasParametersXDD-Diff			OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff			OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff			OPTIONAL

}

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon				OPTIONAL,

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff				OPTIONAL,

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff				OPTIONAL,

phy-ParametersFR1				Phy-ParametersFR1					OPTIONAL,

phy-ParametersFR2				Phy-ParametersFR2					OPTIONAL,

supportedBasebandProcessingCombination		SupportedBasebandProcessingCombination,

basebandCombinationParametersUL-List		BasebandCombinationParametersUL-List

}

Phy-ParametersCommon ::=	SEQUENCE {

-- R1 1-9: CSI-RS based CFRA for HO

csi-RS-CFRA-ForHO					ENUMERATED {supported}						OPTIONAL,

-- R1 2-11: Downlink dynamic PRB bundling (DL)

dynamicPRB-BundlingDL				ENUMERATED {supported}						OPTIONAL,

-- R1 2-32a: Semi-persistent CSI report on PUCCH

sp-CSI-ReportPUCCH					ENUMERATED {supported}						OPTIONAL,

-- R1 2-32b: Semi-persistent CSI report on PUSCH

sp-CSI-ReportPUSCH					ENUMERATED {supported}						OPTIONAL,

-- R1 2-34: NZP-CSI-RS  based interference measurement

nzp-CSI-RS-IntefMgmt				ENUMERATED {supported}						OPTIONAL,

-- R1 2-42: Support Type II SP-CSI feedback on long PUCCH

type2-SP-CSI-Feedback-LongPUCCH		ENUMERATED {supported}						OPTIONAL,

-- R1 3-3: More than one CORESET per BWP (in addition to CORESET #0)

multipleCORESET						ENUMERATED {supported}						OPTIONAL,

-- R1 3-6: Dynamic SFI monitoring and dynamic UL/DL determination

dynamicSFI							ENUMERATED {supported}						OPTIONAL,

-- R1 3-7: Precoder-granularity of CORESET size

precoderGranularityCORESET			ENUMERATED {supported}						OPTIONAL,

-- R1 4-10: Dynamic HARQ-ACK codebook

dynamicHARQ-ACK-Codebook			ENUMERATED {supported}						OPTIONAL,

-- R1 4-11: Semi-static HARQ-ACK codebook

semiStaticHARQ-ACK-Codebook			ENUMERATED {supported}						OPTIONAL,

-- R1 4-12: HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group

spatialBundlingHARQ-ACK				ENUMERATED {supported}						OPTIONAL,

-- R1 4-21: Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI

dynamicBetaOffsetInd-HARQ-ACK-CSI	ENUMERATED {supported}						OPTIONAL,

-- R1 4-23: Repetitions for PUCCH format 1, 3,and 4 over multiple slots with K = 1, 2, 4, 8

pucch-Repetition-F1-3-4				ENUMERATED {supported}						OPTIONAL,

-- R1 5-2: RA type 0 for PUSCH

ra-Type0-PUSCH						ENUMERATED {supported}						OPTIONAL,

-- R1 5-3: Dynamic switching between RA type 0 and RA type 1 for PDSCH

dynamicSwitchRA-Type0-1-PDSCH		ENUMERATED {supported}						OPTIONAL,

-- R1 5-4: Dynamic switching between RA type 0 andRA type 1 for PUSCH

dynamicSwitchRA-Type0-1-PUSCH		ENUMERATED {supported}						OPTIONAL,

-- R1 5-6: PDSCH mapping type A with less than 7 OFDM symbols

pdsch-MappingTypeA					ENUMERATED {supported}						OPTIONAL,

-- R1 5-6a: PDSCH mapping type B

pdsch-MappingTypeB					ENUMERATED {supported}						OPTIONAL,

-- R1 5-7: Interleaving for VRB-to-PRB mapping for PDSCH

interleavingVRB-ToPRB-PDSCH			ENUMERATED {supported}						OPTIONAL,

-- R1 5-8: Interleaving for VRB-to-PRB mapping for PUSCH

interleavingVRB-ToPRB-PUSCH			ENUMERATED {supported}						OPTIONAL,

-- R1 5-10: Inter-slot frequency hopping for PUSCH

interSlotFreqHopping-PUSCH			ENUMERATED {supported}						OPTIONAL,

-- R1 5-13: Type 1 configured PUSCH repetitions within a slot

type1-PUSCH-RepetitionOneSlot		ENUMERATED {supported}						OPTIONAL,

-- R1 5-14: Type 1 configured PUSCH repetitions over multiple slots

type1-PUSCH-RepettitionMultiSlots	ENUMERATED {supported}						OPTIONAL,

-- R1 5-15: Type 2 configured PUSCH repetitions within a slot

type2-PUSCH-RepetitionOneSlot		ENUMERATED {supported}						OPTIONAL,

-- R1 5-16: Type 2 configured PUSCH repetitions over multiple slots

type1-PUSCH-RepettitionMultiSlots	ENUMERATED {supported}						OPTIONAL,

-- R1 5-17: PUSCH repetitions over multiple slots

pusch-RepetitionMultiSlots			ENUMERATED {supported}						OPTIONAL,

-- R1 5-17a: PDSCH repetitions over multiple slots

pdsch-RepetitionMultiSlots			ENUMERATED {supported}						OPTIONAL,

-- R1 5-18: DL SPS

downlinkSPS							ENUMERATED {supported}						OPTIONAL,

-- R1 5-19: Type 1 Configured UL grant

configuredUL-GrantType1				ENUMERATED {supported}						OPTIONAL,

-- R1 5-20: Type 2 Configured UL grant

configuredUL-GrantType2				ENUMERATED {supported}						OPTIONAL,

-- R1 5-21: Pre-emption indication for DL

pre-EmptIndication-DL				ENUMERATED {supported}						OPTIONAL,

-- R1 5-22 & 5-25: CBG-based re-transmission for DL/UL using CBGTI

cbg-TransIndication					BIT STRING (SIZE (2))						OPTIONAL,

-- R1 5-23: CBGFI for CBG-based re-transmission for DL

cbg-FlushIndication-DL				ENUMERATED {supported}						OPTIONAL,

-- R1 5-24: Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL

dynamicHARQ-ACK-CodeB-CBG-Retx-DL	ENUMERATED {supported}						OPTIONAL,

-- R1 5-26: Semi-static rate-matching resource set configuration for DL

rateMatchingResrcSetSemi-Static		ENUMERATED {supported}						OPTIONAL,

-- R1 5-27: Dynamic rate-matching resource set configuration for DL

rateMatchingResrcSetDynamic			ENUMERATED {supported}						OPTIONAL,

-- R1 5-28: Rate-matching around LTE CRS

rateMatchingLTE-CRS					ENUMERATED {supported}						OPTIONAL,

-- R4 1-8: BWP switching delay

bwp-SwitchingDelay					ENUMERATED {type1, type2}					OPTIONAL

}

Phy-ParametersXDD-Diff ::=	SEQUENCE {

-- R1 4-2: 2 PUCCH of format 0 or 2 in consecutive symbols

twoPUCCH-F0-2-ConsecSymbols			ENUMERATED {supported}						OPTIONAL,

-- R1 8-7: UL power control with 2 PUSCH closed loops

twoDifferentTPC-Loop-PUSCH			ENUMERATED {supported}						OPTIONAL,

-- R1 8-8: UL power control with 2 PUCCH closed loops

twoDifferentTPC-Loop-PUCCH			ENUMERATED {supported}						OPTIONAL

}

Phy-ParametersFRX-Diff ::=	SEQUENCE {

-- R1 2-6 & 2-16b: Support 1+2 DMRS (DL/UL)

oneFL-DMRS-TwoAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

-- R1 2-7 & 2-18: Supported 2 symbols front-loaded DMRS(DL/UL)

twoFL-DMRS							BIT STRING (SIZE (2))						OPTIONAL,

-- R1 2-8 & 2-18a: Supported 2 symbols front-loaded +2 symbols additional DMRS(DL/UL)

twoFL-DMRS-TwoAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

-- R1 2-9 & 2-19: Support 1+3 DMRS (DL/UL)

oneFL-DMRS-ThreeAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

-- R1 2-10: Support DMRS type (DL)

supportedDMRS-TypeDL				ENUMERATED {type1, type2}					OPTIONAL,

-- R1 2-17: Support DMRS type (UL)

supportedDMRS-TypeUL				ENUMERATED {type1, type2}					OPTIONAL,

-- R1 2-37: Support Semi-open loop CSI

semiOpenLoopCSI						ENUMERATED {supported}						OPTIONAL,

-- R1 2-38: CSI report without PMI

csi-ReportWithoutPMI				ENUMERATED {supported}						OPTIONAL,

-- R1 2-39: CSI report with CRI

csi-ReportWithCRI					ENUMERATED {supported}						OPTIONAL,

-- R1 2-39a: CSI report without CQI

csi-ReportWithoutCQI				ENUMERATED {supported}						OPTIONAL,

-- R1 2-44 & 2-47: 1 port of DL/UL PTRS

onePortsPTRS						BIT STRING (SIZE (2))						OPTIONAL,

-- R1 4-2: 2 PUCCH of format 0 or 2 in consecutive symbols

twoPUCCH-F0-2-ConsecSymbols			ENUMERATED {supported}						OPTIONAL,

-- R1 4-3: PUCCH format 2 over 1 – 2 OFDM symbols once per slot with FH

pucch-F2-WithFH						ENUMERATED {supported}						OPTIONAL,

-- R1 4-4: PUCCH format 3 over 4 – 14 OFDM symbols once per slot with FH

pucch-F3-WithFH						ENUMERATED {supported}						OPTIONAL,

-- R1 4-5: PUCCH format 4 over 4 – 14 OFDM symbols once per slot with FH

pucch-F4-WithFH						ENUMERATED {supported}						OPTIONAL,

-- R1 4-6: Non-frequency hopping for PUCCH formats 0 and 2

freqHoppingPUCCH-F0-2				ENUMERATED {notSupported}					OPTIONAL,

-- R1 4-7: Non-frequency hopping for PUCCH format 1, 3, and 4

freqHoppingPUCCH-F1-3-4				ENUMERATED {notSupported}					OPTIONAL,

-- R1 4-19: SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH)

mux-SR-HARQ-ACK-CSI-PUCCH			ENUMERATED {supported}						OPTIONAL,

-- R1 4-20: UCI code-block segmentation

uci-CodeBlockSegmentation			ENUMERATED {supported}						OPTIONAL,

-- R1 4-22: 1 long PUCCH format and 1 short PUCCH format in the same slot

onePUCCH-LongAndShortFormat			ENUMERATED {supported}						OPTIONAL,

-- R1 4-22a: 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2

twoPUCCH-AnyOthersInSlot			ENUMERATED {supported}						OPTIONAL,

-- R1 5-9: Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 before RRC connection

intraSlotFreqHopping-PUSCH			ENUMERATED {supported}						OPTIONAL,

-- R1 5-29: LBRM for PUSCH

pusch-LBRM							ENUMERATED {supported}						OPTIONAL,

-- R1 6-5a: PDCCH blind detection capability for CA

pdcch-BlindDetectionCA				ENUMERATED {supported}						OPTIONAL,

-- R1 8-3: TPC-PUSCH-RNTI

tpc-PUSCH-RNTI						ENUMERATED {supported}						OPTIONAL,

-- R1 8-4: TPC-PUCCH-RNTI

tpc-PUCCH-RNTI						ENUMERATED {supported}						OPTIONAL,

-- R1 8-5: TPC-SRS-RNTI

tpc-SRS-RNTI						ENUMERATED {supported}						OPTIONAL,

-- R1 8-6: Absolute TPC command mode

absoluteTPC-Command					ENUMERATED {supported}						OPTIONAL

-- R1 8-7: UL power control with 2 PUSCH closed loops

twoDifferentTPC-Loop-PUSCH			ENUMERATED {supported}						OPTIONAL,

-- R1 8-8: UL power control with 2 PUCCH closed loops

twoDifferentTPC-Loop-PUCCH			ENUMERATED {supported}						OPTIONAL,

-- R4 1-6: pi/2-BPSK for PUSCH

pusch-HalfPi-BPSK					ENUMERATED {supported}						OPTIONAL,

-- R4 1-7: pi/2-BPSK for PUCCH format 3/4

pucch-F3-4-HalfPi-BPSK				ENUMERATED {supported}						OPTIONAL,

-- R4 1-9: 1-symbol GP in unpaired spectrum

oneSymbolGP-TDD						ENUMERATED {supported}						OPTIONAL,

-- R4 2-7: Almost contiguous UL CP-OFDM

almostContiguousCP-OFDM-UL			ENUMERATED {supported}						OPTIONAL

}

Phy-ParametersFR1 ::=	SEQUENCE {

-- R1 3-2: Unicast PDCCH monitoring following Case 1-2

pdcchMonitoringSingleOccasion		ENUMERATED {supported}						OPTIONAL,

-- R4 1-1: 60kHz of subcarrier spacing for FR1

scs-60kHz							ENUMERATED {supported}						OPTIONAL,

-- R4 1-4: 256QAM for PDSCH in FR1

pdsch-256QAM-FR1					ENUMERATED {supported}						OPTIONAL

}

Phy-ParametersFR2 ::=	SEQUENCE {

-- R4 2-8: PA calibration gap

calibrationGapPA					ENUMERATED {supported}						OPTIONAL

}

RF-Parameters ::= SEQUENCE {

supportedBandListNR					SupportedBandListNR,

supportedBandCombination			BandCombinationList,

bandCombinationParametersUL-List	BandCombinationParametersUL-List

}

SupportedBandListNR ::=	SEQUENCE (SIZE (1..maxBands)) OF BandNR

BandNR ::=	SEQUENCE {

bandNR							FreqBandIndicatorNR,

-- Modified MPR behaviour as in RAN4 LS R2-1804077, which is needed for NSA as well as SA

modifiedMPR-Behaviour			BIT STRING (SIZE (8))						OPTIONAL,

-- R4 2-1: Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC

-- RAN4 agreed that 400 MHz is optional for FR2. The other values defined for FR1/fR2 in TS 38.101 are mandatory w/o capability bit.

maxChannelBW-PerCC				ENUMERATED {mhz400}							OPTIONAL,

mimo-ParametersPerBand			MIMO-ParametersPerBand						OPTIONAL,

-- R1 0-10: Extended CP

extendedCP						ENUMERATED {supported}						OPTIONAL,

-- R1 0-13: Phase coherence across non-contiguous UL symbols in slot in the transmission of one channel

phaseCoherenceUL				ENUMERATED {supported}						OPTIONAL,

-- R1 1-10: Support of SCell without SS/PBCH block

scellWithoutSSB					ENUMERATED {supported}						OPTIONAL,

-- R1 1-11: Support of CSI-RS RRM measurement for SCell without SS/PBCH block

csi-RS-MeasSCellWithoutSSB		ENUMERATED {supported}						OPTIONAL,

-- R1 2-15a: Association between CSI-RS and SRS

srs-AssocCSI-RS					ENUMERATED {supported}						OPTIONAL,

-- R1 3-1a: For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2

type1-3-CSS						ENUMERATED {supported}						OPTIONAL,

-- R1 3-4: More than one TCI state configurations per CORESET

multipleTCI						ENUMERATED {supported}						OPTIONAL,

-- R1 3-5 & 3-5a: For type 1 with dedicated RRC configuration, type 3, and UE-SS,, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 (with a DCI gap)

pdcchMonitoringAnyOccasions		ENUMERATED {withoutDCI-gap, withDCI-gap}	OPTIONAL,

-- R1 5-1a: UE specific RRC configure UL/DL assignment

ue-SpecificUL-DL-Assignment		ENUMERATED {supported}						OPTIONAL,

-- R1 5-11 & 5-11a: Up to 2/7 unicast PDSCHs per slot for different TBs

pdsch-DifferentTB-PerSlot		SEQUENCE {

scs-15kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-30kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-60kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-120kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

},																			OPTIONAL,

-- R1 5-12 & 5-12a: Up to 2/7 PUSCHs per slot for different TBs

pusch-DifferentTB-PerSlot		SEQUENCE {

scs-15kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-30kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-60kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

scs-120kHz						ENUMERATED {upto2, upto7}				OPTIONAL,

},																			OPTIONAL,

-- R1 6-2 & 6-3: Type A/B BWP adaptation (up to 2/4 BWPs) with same numerology

bwp-SameNumerology					ENUMERATED {upto2, upto4}				OPTIONAL,

-- R1 6-4: BWP adaptation (up to 4 BWPs) with different numerologies

bwp-DiffNumerology					ENUMERATED {upto4}						OPTIONAL,

-- R1 6-7: Two PUCCH group

twoPUCCH-Group					ENUMERATED {supported}						OPTIONAL,

-- R1 6-8: Different numerology across PUCCH groups

diffNumerologyAcrossPUCCH-Group	ENUMERATED {supported}						OPTIONAL,

-- R1 6-9: Different numerologies across carriers within the same PUCCH group

diffNumerologyWithinPUCCH-Group	ENUMERATED {supported}						OPTIONAL,

-- R1 6-10: Cross carrier scheduling

crossCarrierScheduling			ENUMERATED {supported}						OPTIONAL,

-- R1 6-11: Number of supported TAGs

supportedNumberTAG				ENUMERATED {n2, n3, n4}						OPTIONAL,

-- R1 6-19: Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS/PRACH on the other UL carrier in the same cell

-- Details on the channel/signal combination are to be described in TS 38.306

simultaneousTxSUL-NonSUL		ENUMERATED {supported}						OPTIONAL,

-- R1 6-21: DL search space sharing for CA

searchSpaceSharingCA-DL			ENUMERATED {supported}						OPTIONAL

-- R1 6-22: UL search space sharing for CA

searchSpaceSharingCA-UL			ENUMERATED {supported}						OPTIONAL

-- R4 1-4: 256QAM for PDSCH in FR2

pdsch-256QAM-FR2				ENUMERATED {supported}						OPTIONAL,

-- R4 1-5: 256QAM for PUSCH

pusch-256QAM					ENUMERATED {supported}						OPTIONAL

}

MIMO-ParametersPerBand ::= SEQUENCE {

-- R1 2-2: PDSCH beam switching

timeDurationForQCL					SEQUENCE {

scs-60kHz							ENUMERATED {s7, s14, s28}									OPTIONAL,

sch-120kHz							ENUMERATED {s14, s28}										OPTIONAL

}																									OPTIONAL,

-- R1 2-3: PDSCH MIMO layers. Absence of this field implies support of one layer.

maxNumberMIMO-LayersPDSCH			MIMO-LayersDL													OPTIONAL,

-- R1 2-14: Codebook based PUSCH MIMO transmission. Absence of this field implies that CB-based PUSCH is not supported.

maxNumberMIMO-LayersCB-PUSCH		MIMO-LayersUL													OPTIONAL,

-- R1 2-15: Non-codebook based PUSCH MIMO transmission. Absence of this field implies that Non-CB-based PUSCH is not supported.

maxNumberMIMO-LayersNonCB-PUSCH		MIMO-LayersUL													OPTIONAL,

-- R1 2-4: TCI states for PDSCH

maxNumberConfiguredTCIstates		ENUMERATED {n4, n8, n16, n32, n64}								OPTIONAL,

maxNumberActiveTCI-PerCC			ENUMERATED {n1, n2, n4, n8}										OPTIONAL,

-- R1 2-13: PUSCH transmission coherence

pusch-TransCoherence				ENUMERATED {nonCoherent, partialNonCoherent, fullCoherent}	OPTIONAL,

-- R1 2-20: Beam correspondence

beamCorrespondence					ENUMERATED {supported}											OPTIONAL,

-- R1 2-21: Periodic beam report on PUCCH

periodicBeamReport					ENUMERATED {supported}											OPTIONAL,

-- R1 2-22: Aperiodic beam report on PUSCH

apeioricBeamReport					ENUMERATED {supported}											OPTIONAL,

-- R1 2-23: Semi-persistent beam report on PUCCH

sp-BeamReportPUCCH					ENUMERATED {supported}											OPTIONAL,

-- R1 2-23a: Semi-persistent beam report on PUSCH

sp-BeamReportPUSCH					ENUMERATED {supported}											OPTIONAL,

-- R1 2-24: SSB/CSI-RS for beam management

beamManagementSSB-CSI-RS			BeamManagementSSB-CSI-RS										OPTIONAL,

-- R1 2-26: Receiving beam selection using CSI-RS resource repetition “ON”

maxNumberRxBeam						INTEGER (2..8)													OPTIONAL,

-- R1 2-27: Beam switching (including SSB and CSI-RS)

maxNumberRxTxBeamSwitchDL			SEQUENCE {

scs-15kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-30kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-60kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-120kHz							ENUMERATED {n4, n7, n14}									OPTIONAL,

scs-240kHz							ENUMERATED {n4, n7, n14}									OPTIONAL

}																									OPTIONAL,

-- R1 2-29: Non-group based beam reporting

maxNumberNonGroupBeamReporting		ENUMERATED {n1, n2, n4}											OPTIONAL,

-- R1 2-29a: Group based beam reporting

groupBeamReporting					ENUMERATED {supported}											OPTIONAL,

-- R1 2-30: UL beam management

uplinkBeamManagement				SEQUENCE {

maxNumberSRS-ResourcePerSet			ENUMERATED {n8, n16, n32},

maxNumberSRS-ResourceSet			INTEGER (1..8)

}																									OPTIONAL,

-- R1 2-31: Beam failure recovery

maxNumberCSI-RS-BFR					INTEGER (1..64)													OPTIONAL,

maxNumberSSB-BFR					INTEGER (1..64)													OPTIONAL,

maxNumberCSI-RS-SSB-BFR				INTEGER (1..256)												OPTIONAL,

-- R1 2-45 & 2-48: 2 ports of DL/UL PTRS

twoPortsPTRS						BIT STRING (SIZE (2))											OPTIONAL,

-- R1 2-53: SRS resources

supportedSRS-Resources			SRS-Resources														OPTIONAL,

-- R1 2-55: SRS Tx switch

srs-TxSwitch					SRS-TxSwitch														OPTIONAL,

-- R1 2-54a: Simultaneous SRS Tx

maxNumberSimultaneousSRS-PerCC		INTEGER (1..4)													OPTIONAL,

-- R1 2-57: Support low latency CSI feedback

lowLatencyCSI-Feedback				ENUMERATED {supported}											OPTIONAL

}

-- R1 2-24: SSB/CSI-RS for beam management

BeamManagementSSB-CSI-RS ::=	SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx	ENUMERATED {n8, n16, n32, n64},

maxNumberSSB-CSI-RS-ResourceTwoTx	ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density				ENUMERATED {one, three, oneAndThree}

}

-- R1 2-53: SRS resources

SRS-Resources ::=	SEQUENCE {

maxNumberAperiodicSRS-PerBWP					ENUMERATED {n1, n2, n4, n8, n16},

maxNumberAperiodicSRS-PerBWP-PerSlot			INTEGER (1..6),

maxNumberPeriodicSRS-PerBWP						ENUMERATED {n1, n2, n4, n8, n16},

maxNumberPeriodicSRS-PerBWP-PerSlot				INTEGER (1..6),

maxNumberSemiPersitentSRS-PerBWP				ENUMERATED {n0, n1, n2, n4, n8, n16},

maxNumberSP-SRS-PerBWP-PerSlot					INTEGER (0..6),

maxNumberSRS-Ports-PerResource					ENUMERATED {n1, n2, n4}

}

-- R1 2-55: SRS Tx switch

SRS-TxSwitch ::=	SEQUENCE {

supportedSRS-TxPortSwitch			ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4},

txSwitchImpactToRx					ENUMERATED {true}								OPTIONAL

}

PDCP-Parameters ::= SEQUENCE {

supportedROHC-Profiles	SEQUENCE {

profile0x0000		BOOLEAN,

profile0x0001		BOOLEAN,

profile0x0002		BOOLEAN,

profile0x0003		BOOLEAN,

profile0x0004		BOOLEAN,

profile0x0006		BOOLEAN,

profile0x0101		BOOLEAN,

profile0x0102		BOOLEAN,

profile0x0103		BOOLEAN,

profile0x0104		BOOLEAN

},

maxNumberROHC-ContextSessions	ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64, cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles			ENUMERATED {supported}	OPTIONAL,

continueROHC-Context			ENUMERATED {supported}	OPTIONAL,

outOfOrderDelivery				ENUMERATED {supported}	OPTIONAL,

shortSN							ENUMERATED {supported} 	OPTIONAL

}

RLC-Parameters ::= SEQUENCE {

am-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WIthLongSN					ENUMERATED {supported}	OPTIONAL

}

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon			MAC-ParametersCommon,	OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff	OPTIONAL

}

MAC-ParametersCommon ::=	SEQUENCE {

lcp-Restriction					ENUMERATED {supported}	OPTIONAL,

-- R1 4-24: PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource

pucch-SpatialRelInfoMAC-CE		ENUMERATED {supported}	OPTIONAL

}

MAC-ParametersXDD-Diff ::=	SEQUENCE {

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

multipleSR-Configurations		ENUMERATED {supported}	OPTIONAL,

-- If supported UE supports 8 SR configurations, otherwise 1 SR config is supported.

-- Confirmation is needed whether to align the number to what the configuration signalling can support.

multipleConfiguredGrantConfigurations	ENUMERATED {supported}	OPTIONAL

-- If supported UE supports 16 configured grant configurations, otherwise 1 ConfiguredGrant config is supported.

-- Confirmation is needed whether to align the number to what the configuration signalling can support, and to consider whether the 16 refers

-- to the configurations or the active ones only (as they are within the BWP).

}

MeasParameters ::= SEQUENCE {

measParametersCommon			MeasParametersCommon		OPTIONAL,

measParametersXDD-Diff			MeasParametersXDD-Diff		OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff		OPTIONAL

}

MeasParametersXDD-Diff ::=	SEQUENCE {

intraAndInterF-MeasAndReport	ENUMERATED {supported}	OPTIONAL,

eventA-MeasAndReport			ENUMERATED {supported}	OPTIONAL

-- Confirmation is needed on the need of capability/IOT signaling in LTE for support of the additional measurement gap configurations.

}

MeasParametersFRX-Diff ::=	SEQUENCE {

-- R1 1-3: SSB based SINR measurement

ss-SINR-Meas						ENUMERATED {supported}		OPTIONAL,

-- R1 1-5: CSI-RS based RRM measurement with associated SS-block

csi-RSRP-AndRSRQ-MeasWithSSB		ENUMERATED {supported}		OPTIONAL,

-- R1 1-5a: CSI-RS based RRM measurement without associated SS-block

csi-RSRP-AndRSRQ-MeasWithoutSSB		ENUMERATED {supported}		OPTIONAL,

-- R1 1-6: CSI-RS based SINR measurement

csi-SINR-Meas						ENUMERATED {supported}		OPTIONAL,

-- R1 1-7: CSI-RS based RLM

csi-RS-RLM							ENUMERATED {supported}		OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

### 6.3.4	Other information elements

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxBandComb								INTEGER ::=	ffsValue		-- Maximum number of band combinations

maxBandCombDL							INTEGER ::= ffsValue		-- Maximum number of DL band combinations

maxBandCombUL							INTEGER ::= ffsValue		-- Maximum number of UL band combinations

maxBasebandProcComb						INTEGER ::=	ffsValue		-- Maximum number of baseband processing combinations

maxBasebandProcCombDL					INTEGER ::= ffsValue		-- Maximum number of DL baseband processing combinations

maxBasebandProcCombUL					INTEGER ::= ffsValue		-- Maximum number of UL baseband processing combinations

maxNrofServingCells						INTEGER ::=	16		-- Max number of serving cells (SpCell + SCells) per cell group

maxNrofServingCells-1					INTEGER ::= 15		-- Max number of serving cells (SpCell + SCells) per cell group minus 1

maxNrofSCells							INTEGER ::=	15		-- Max number of secondary serving cells per cell group

maxNrofCellMeas							INTEGER ::=	ffsValue		-- Maximum number of entries in each of the cell lists in a measurement object

maxNrofSS-BlocksToAverage				INTEGER ::= ffsValue		-- Max number for the (max) number of SS blocks to average to determine cell

-- measurement

maxNrofCSI-RS-ResourcesToAverage		INTEGER ::= ffsValue		-- Max number for the (max) number of CSI-RS to average to determine cell

-- measurement

maxNrofDL-Allocations 					INTEGER ::= ffsValue

maxNrofSR-ConfigPerCellGroup			INTEGER ::= 8		-- Maximum number of SR configurations per cell group

maxLCG-ID								INTEGER ::= 7		-- Maximum value of LCG ID

mmaxLC-ID								INTEGER ::= ffsValue		-- Maximum value of Logical Channel ID

maxNrofTAGs								INTEGER ::=	4		-- Maximum number of Timing Advance Groups

maxNrofTAGs-1							INTEGER ::=	3		-- Maximum number of Timing Advance Groups minus 1

mmaxNrofBWPs					INTEGER ::= 4		-- Maximum number of BWPs per serving cell

mmaxNrofSymbols-1							INTEGER ::= 13		-- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots							INTEGER ::= 320		-- Maximum number of slots in a 10 ms period

maxNrofSlots-1							INTEGER ::= 319		-- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks			INTEGER ::= 275		-- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1			INTEGER ::= 274		-- Maximum number of PRBs

maxNrofControlResourceSets 				INTEGER ::= 12 	-- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1			INTEGER ::= 11  	-- Max number of CoReSets configurable on a serving cell minus 1

maxCoReSetDuration						INTEGER ::= 3		-- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces						INTEGER ::= 40		-- Max number of Search Spaces

maxNrofSearchSpaces-1					INTEGER ::= 339		-- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize					INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1				INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxINT-DCI-PayloadSize					INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1				INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns				INTEGER ::= 4		-- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1				INTEGER ::= 3		-- Max number of rate matching patterns that may be configured minus 1

maxNrofCSI-Reports						INTEGER ::= ffsValue 	-- Maximum number of report configurations

maxNrofCSI-RS-CellsRRM 					INTEGER ::= ffsValue 	-- Maximum number of FFS

maxNrofReportConfigIdsPerTrigger		INTEGER ::= 16	 	-- Maximum number of report configurations per reportTrigger

maxNrofCSI-ResourceConfigurations		INTEGER ::= ffsValue		-- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1		INTEGER ::= ffsValue		-- Maximum number of resource configurations minus 1

maxNrofCSI-ResourceSets					INTEGER ::= ffsValue		-- Maximum number of resource sets per resource configuration

maxNrofCSI-ResourceSets-1				INTEGER ::= ffsValue		-- Maximum number of resource sets per resource configuration minus 1

maxNrofFailureDetectionResources		INTEGER ::= ffsValue		-- Maximum number of failure detection resources

maxNrofNZP-CSI-RS-Resources-1			INTEGER ::= ffsValue		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-Resources				INTEGER ::= 3		-- Maximum number of Zero-Power (NZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1			INTEGER ::= 2		-- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1

maxNrofCSI-IM-Resources					INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1				INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-ResourcesPerSet			INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax in 38.214

maxNrofSSB-Resources-1					INTEGER ::= 63		-- Maximum number of SSB resources in a resource set minus 1

maxNrofCSI-RS-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of CSI-RS resources per resource set

maxNrofCSI-MeasId						INTEGER ::= ffsValue		-- Maximum number of link configurations

maxNrofCSI-MeasId-1						INTEGER ::= ffsValue		-- Maximum number of link configurations minus 1

maxNrofCSI-RS-ResourcesRRM				INTEGER ::= ffsValue		-- Maximum number of CSI-RS resources for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1			INTEGER ::= ffsValue		-- Maximum number of CSI-RS resources for an RRM measurement object minus 1

maxNrofObjectId							INTEGER ::= ffsValue		-- Maximum number of configured measurement objects

maxNrofPCI-Ranges						INTEGER ::= ffsValue		-- Maximum number of PCI ranges

maxReportConfigId					INTEGER ::= ffsValue		-- Maximum number of reporting configurations

maxNrofMeasId							INTEGER ::= ffsValue		-- Maximum number of configured measurements

mmaxNrofQuantityConfig					INTEGER	::= 2		-- Maximum number of quantity configurations

maxNrofSRS-ResourceSets					INTEGER ::= ffsValue		-- Maximum number of SRS resource sets.

maxNrofSRS-ResourceSets-1				INTEGER ::= ffsValue		-- Maximum number of SRS resource sets minus 1.

maxNrofSRS-Resources					INTEGER ::= ffsValue		-- Maximum number of SRS resources in an SRS resource set.

maxNrofSRS-Resources-1					INTEGER ::= ffsValue		-- Maximum number of SRS resources in an SRS resource set minus 1.

maxNrofSRS-TriggerStates-1 				INTEGER ::= 3		-- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxRAT-CapabilityContainers				INTEGER ::= ffsValue		-- Maximum number of interworking RAT containers (incl NR and MRDC)

maxRequestedBands						INTEGER ::= ffsValue		-- Maximum number of simultaneously requested bands

maxNrofCC								INTEGER ::=	ffsValue		-- Maximum number of carriers

maxSimultaneousBands					INTEGER ::= ffsValue		-- Maximum number of simultaneously aggregated bands

maxNrofSlotFormatCombinationsPerSet		INTEGER ::= ffsValue		-- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1	INTEGER ::= ffsValue		-- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofPUCCH-ResourceSets				INTEGER ::= 4		-- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1				INTEGER ::= 3		-- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-ResourcesPerSet-1			INTEGER ::= 7		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet minus 1.

maxNrofPUCCH-P0-PerSet					INTEGER ::= 8		-- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1	INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1.

maxNrofP0-PUSCH-AlphaSets				INTEGER ::= 30		-- Maximum number of P0-pusch-alpha-sets (see 38,213, section 7.1)

maxNrofP0-PUSCH-AlphaSets-1				INTEGER ::= 29		-- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, section 7.1)

maxNrofPUSCH-PathlossReferenceRSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1	INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1.

maxEARFCN								INTEGER ::= 262143	-- Highest value of extended E-UTRA EARFCN range

maxBands 								INTEGER ::= ffsValue

maxCellPrep		 						INTEGER ::= ffsValue

maxCellReport		 					INTEGER ::= ffsValue

maxCellSCG		 						INTEGER ::= ffsValue

maxDRB		 							INTEGER ::= ffsValue

maxFreq		 							INTEGER ::= ffsValue

maxLCH									INTEGER ::= ffsValue

maxNrofCSI-RS	 						INTEGER ::= ffsValue

maxNrofAggregatedCellsPerCellGroup 		INTEGER ::= ffsValue

maxNrofCandidateBeams 					INTEGER ::= ffsValue

maxNrofCSI-ReportConfig-1 				INTEGER ::= ffsValue

maxNrofCSI-ResrouceConfigurations 		INTEGER ::= ffsValue

maxNrofPCIsPerSMTC 						INTEGER ::= ffsValue

maxNrofQFIs 							INTEGER ::= ffsValue

maxNrofSR-Resoruces		 		INTEGER ::= ffsValue

maxNrofSlotFormatsPerCombination 		INTEGER ::= ffsValue

maxNrofSpatialRelationInfos 			INTEGER ::= ffsValue

maxNrofSRS-ResourcesPerSet 				INTEGER ::= ffsValue

maxNrofIndexesToReport 					INTEGER ::= ffsValue

maxNrofSSBs 							INTEGER ::= ffsValue

maxNrofTCI-StatesPDCCH 					INTEGER ::= ffsValue

maxNrofTCI-States						INTEGER ::= 64

maxNrofTCI-States-1						INTEGER ::= 63

maxNrofUL-Allocations 					INTEGER ::= ffsValue

maxQFI 									INTEGER ::= ffsValue

maxRA-CSIRS-Resources 					INTEGER ::= ffsValue

m

maxRA-SSB-Resources 						INTEGER ::= ffsValue

maxSCSs									INTEGER ::= ffsValue

maxSecondaryCellGroups 							INTEGER ::= ffsValue

ffsValue								INTEGER ::= 64

-- IE definitions introduced to not get warning at ASN.1 syntax check

CandidateRS-IndexInfoList ::=	ENUMERATED {ffsTypeAndValue}

CellIdentity ::=				ENUMERATED {ffsTypeAndValue}

CSI-RS-Index ::=					ENUMERATED {ffsTypeAndValue}

FilterCoefficient ::=			ENUMERATED {ffsTypeAndValue}

Hysteresis ::=					ENUMERATED {ffsTypeAndValue}

MeasObjectEUTRA ::=				ENUMERATED {ffsTypeAndValue}

MeasResultListEUTRA ::=			ENUMERATED {ffsTypeAndValue}

MeasResultSSTD ::=				ENUMERATED {ffsTypeAndValue}

PPDU-SessionID ::=				ENUMERATED {ffsTypeAndValue}

PhyCellNR ::=					ENUMERATED {ffsTypeAndValue}

PhysCellIdEUTRA ::=				ENUMERATED {ffsTypeAndValue}

PhysCellIdRange ::=				ENUMERATED {ffsTypeAndValue}

P-Max ::=						ENUMERATED {ffsTypeAndValue}

RA-Resources ::=				ENUMERATED {ffsTypeAndValue}

ReportConfigEUTRA ::=			ENUMERATED {ffsTypeAndValue}

RRC-TransactionIdentifier ::=	ENUMERATED {ffsTypeAndValue}

SchedulingRequestId ::=			ENUMERATED {ffsTypeAndValue}

ShortMAC-I ::=					ENUMERATED {ffsTypeAndValue}

SSB-Id ::=						ENUMERATED {ffsTypeAndValue}

TimeToTrigger ::=				ENUMERATED {ffsTypeAndValue}

UECapabilityInformation ::=	ENUMERATED {ffsTypeAndValue}

BW-PerCC ::=		ENUMERATED {ffsTypeAndValue}

FFS_Value ::=		ENUMERATED {ffsTypeAndValue}

FreqBandIndicatorNR ::=		ENUMERATED {ffsTypeAndValue}

MBSFN-SubframeConfigList ::=		ENUMERATED {ffsTypeAndValue}

NZP-CSI-RS-ResourceConfigId ::=		ENUMERATED {ffsTypeAndValue}

SlotFormatIndicator ::=		ENUMERATED {ffsTypeAndValue}

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

### –	End of NR-RRC-Definitions

-- ASN1START

END

-- ASN1STOP