Alternative 1:	Single IE for XDD and FRX differentiation.

UE-NR-Capability ::=	SEQUENCE {

phy-Parameters						Phy-Parameters,

fdd-Add-UE-NR-Capabilities			UE-NR-CapabilityAddXDD-FRX-Mode		OPTIONAL,

tdd-Add-UE-NR-Capabilities			UE-NR-CapabilityAddXDD-FRX-Mode		OPTIONAL,

fr1-Add-UE-NR-Capabilities			UE-NR-CapabilityAddXDD-FRX-Mode		OPTIONAL,

fr2-Add-UE-NR-Capabilities			UE-NR-CapabilityAddXDD-FRX-Mode		OPTIONAL,

fdd-FR1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-FRX-Mode		OPTIONAL,

tdd-FR1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-FRX-Mode		OPTIONAL,

tdd-FR2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-FRX-Mode		OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddXDD-FRX-Mode ::=	SEQUENCE {

phy-ParametersXDD-FRX-Diff			Phy-ParametersXDD-FRX-Diff			OPTIONAL,

...

}

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon					OPTIONAL,

phy-ParametersXDD-FRX-Diff		Phy-ParametersXDD-FRX-Diff				OPTIONAL,

}

Phy-ParametersXDD-FRX-Diff ::=	SEQUENCE {

-- FDD/TDD diff. needed

featureA						ENUMERATED {supported}					OPTIONAL,

-- FR1/FR2 diff. needed

featureB						ENUMERATED {supported}					OPTIONAL,

-- Both FDD/TDD and FR1/FR2 diff. needed

featureC						ENUMERATED {supported}					OPTIONAL

}

Alternative 2:	Separate IE for XDD and FRX differentiation

UE-NR-Capability ::=	SEQUENCE {

phy-Parameters						Phy-Parameters,

fdd-Add-UE-NR-Capabilities			UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities			UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities			UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities			UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff			OPTIONAL,

...

}

UE-NR-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff			OPTIONAL,

...

}

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon					OPTIONAL,

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff					OPTIONAL,

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff					OPTIONAL

}

Phy-ParametersXDD-Diff ::=	SEQUENCE {

-- FDD/TDD diff. needed

featureA						ENUMERATED {supported}					OPTIONAL,

-- Both FDD/TDD and FR1/FR2 diff. needed

featureC						ENUMERATED {supported}					OPTIONAL

}

Phy-ParametersFRX-Diff ::=	SEQUENCE {

-- FR1/FR2 diff. needed

featureB						ENUMERATED {supported}					OPTIONAL,

-- Both FDD/TDD and FR1/FR2 diff. needed

featureC						ENUMERATED {supported}					OPTIONAL

}