3GPP TSG RAN WG2 Meeting #102     			             	 						                         R2-18xxxxx Busan, Korea, 21 – 25 May 2018

Agenda item:	10.4.5.2

Source:	Nokia (Email rapporteur)

Title:		Email discussion report of [101bis#48][NR] UE capabilites structure

Document for:	Discussion, Decision

# Introduction

This document is to progress the following email discussion:

[101bis#48][NR] UE capabilites structure (Nokia)

Progress the following topics:

-	Name of feature sets

-	Update procedure text

-	Check if any impact in 38.306

-	Consider how to apply for the LTE side

-	Update the MR-DC capability container

-	Update the inter-node messages

-	Size of the lists

Intended outcome: Report and TPs to the next meeting

Deadline:  Thursday 2018-05-10

# Discussion

Plan is to progress the ASN.1 development based on:

R2-1806451	Baseline ASN.1 structure for UE capability signalling structure	Nokia	discussion	Rel-15	NR_newRAT-Core

=>	Agreed as baseline for further work.

There are 2 files prepared for the email discussion: The 1st file (many thanks to NTT Docomo) contains changes to the LTE standalone capability signalling exactly based on the changes made in R2-1806451. The 2nd file is a version that has compilation errors fixed (and based on R2-1806451).

1st file: DRAFT FeatureSet_36331-f10_v0.doc

2nd file: DRAFT FeatureSet_38331-f10_v0.doc

# List of open topics and company views

TOPIC 1: Companies had a view for alternative naming for “Feature Sets”.

QUESTION 1: What is the label companies prefer as an alternative?

| Company | View (QUESTION 1) |
| --- | --- |
| Nokia, Nokia Shanghai Bell | One example from our side (no strong view on exact naming, this can be changed in the end as well) Feature Sets  perBandofBandCombinationParameterSet Feature Set per CC  perCCParameterSet |

TOPIC 2: Application of “Feature Set” principle to LTE side. NTT Docomo provided a TS 36.331 where the concept is applied to LTE standalone (i.e. applying the changes as in TS 38.331 (in R2-1806451). Here are some additional remarks:

1) The feature set for LTE is added by backward compatible extension. So, there still remains BPC structure in the v15.1.0 extension. What is the preference to handle the obsoleted IEs from v15.1.0?
2) FD-MIMO parameters defined for BPC is renamed to use the feature set. It is still backward compatible merely to change the field/IE names
3) LTE features not used together with EN-DC are not included so far. These are, Prose, V2X, LTE DC. With this, there is clear distinction that this feature set is used only for EN-DC.

QUESTION 2: Please provide feedback on the 36.331 changes.

| Company | View (QUESTION 2) |
| --- | --- |
| Nokia, Nokia Shanghai Bell | - Okay for changes in TS 36.331 and aligned to the offline discussion version in TS 38.331 - We propose to remove the “obsoleted changes” i.e. BPC related changes from the LTE RRC (i.e. no need to be backward compatible here to ensure that the specification is cleaned up) - Okay to not include capabilities that are not relevant for EN-DC |

TOPIC 3: Dealing with the MR-DC capability container. The IE BandCombinationList is used in the UE-MRDC-Capability (within the RF-ParametersMRDC). Within the BandCombinationList, the modelling of BandParameterEUTRA needs to be agreed upon.

BandParameter ::=	CHOICE {

bandParameterEUTRA				BandParameterEUTRA,

bandParameterNR					BandParameterNR

}

Looking at the BandParameterNR, it seems reasonable to adapt the same model for the BandParameterEUTRA.

QUESTION 3: Is the following modelling acceptable?

BandParameterEUTRA	::=	SEQUENCE {

bandEUTRA-r15							FreqBandIndicator-r11,

featureSetListDL-EUTRA-r15				SEQUENCE (SIZE (1..maxNrofFeatureSetsPerBC)) OF FeatureSetDL-EUTRA-Id-r15,

featureSetListUL-EUTRA-r15				SEQUENCE (SIZE (1..maxNrofFeatureSetsPerBC)) OF FeatureSetUL-EUTRA-Id-r15

}

| Company | View (QUESTION 3) |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Okay to model BandParameterEUTRA like bandParameterNR |

TOPIC 4: Dimensioning of the DL/UL FeatureSet and FeatureSetperCC lists?

QUESTION 4: What would be a reasonable size of the lists?

| Company | View (QUESTION 4) |
| --- | --- |
| Nokia, Nokia Shanghai Bell | As a starting point (looking at LTE there are many more DL features than UL): - For DL Feature Sets and Feature Set per CC both 12 bit index size is enough (total 4095 unique combinations can be represented) - For UL Feature Sets and Feature Set per CC both 8 bit index size is enough (total 255 unique combinations can be represented) |

# Structuring of Text Proposal

Based on the outcome to Q1-Q4, the following is suggested to split the work across different technical specification impacts.

- Nokia will update procedure text and align inter-node messages for TS 38.331

- Intel will align TS 38.306

- Samsung will align TS 36.331

# Summary

# References

- R2-1806450: Offline discussion # 46 and #47 report,	NR_newRAT-Core

- R2-1806451: Baseline ASN.1 structure for UE capability signalling structure, 	NR_newRAT-Core