3GPP TSG-RAN WG2 Meeting #102                                                               R2-1809055

Busan, South Korea, May 21 – 25, 2018                                (Revision of R2-18090011)

Agenda item:	11.1.3

Source: 		Qualcomm Inc (Rapporteur)

Title: 	CP issues for architecture group 1

Document for:	Discussion

# 1. Introduction

This document includes a text proposal for TR 38.874 based on offline discussion.

# 2. Text Proposal for TR 38.874

The following changes to TR 38.874 are proposed:

********* Start of Change **********

# 8	Radio protocol aspects

Editor’s note:	Primary responsible WG for this clause is RAN2.

## 8.1	Packet Processing

## 8.2 	User-plane considerations for architecture group 1

### 8.2.1 	General

The following subsections describe various user plane aspects for architecture group 1 including placement of an adaptation layer, functions supported by the adaptation layer, support of multi-hop RLC, impacts on scheduler and QoS. The study will analyse described architecture options to identify trade-offs between these various aspects with the goal to recommend a single architecture for this group.

Figure 8.2 - 1: Protocol stack examples for UE-access using L2-relaying with adaptation layer for architecture 1a

Figure 8.2 - 2: Protocol stack example for UE-access using L2-relaying with adaptation layer for architecture 1b

### 8.2.2 	Adaptation Layer

The UE establishes RLC channels to the DU on the UE’s access IAB node in compliance with TS 38.300. Each of these RLC-channels is extended via a potentially modified form of F1-U, referred to as F1*-U, between the UE’s access DU and the IAB donor.

The information embedded in F1*-U is carried over RLC-channels across the backhaul links. Transport of F1*-U over the wireless backhaul is enabled by an adaptation layer, which is integrated with the RLC channel.

Within the IAB-donor, the baseline is to use native F1-U stack (see section 9).

Functions supported by the adaptation layer

In architecture 1a, information carried on the adaptation layer supports the following functions:

- Identification of the UE-bearer for the PDU,

- Routing across the wireless backhaul topology,

- QoS-enforcement by the scheduler on DL and UL on the wireless backhaul link,

- Mapping of UE user-plane PDUs to backhaul RLC channels,

- Others.

In architecture 1b, information carried on the adaptation layer supports the following functions:

- Routing across the wireless backhaul topology,

- QoS-enforcement by the scheduler on DL and UL on the wireless backhaul link,

- Mapping of UE user-plane PDUs to backhaul RLC channels

- Others.

Content carried on the adaptation layer header

The study will identify all information to be carried on the adaptation layer header. This may include:

- UE-bearer-specific Id

- UE-specific Id

- Route Id, IAB-node or IAB-donor address

- QoS information

- Potentially other information

Details of the information carried in the adaptation layer are FFS.

Processing of adaptation layer information

- The study will identify, which of the information on the adaptation layer is processed to support the above functions on each on-path IAB-node (hop-by-hop),

- and/or on the UE’s access-IAB-node and the IAB-donor (end-to-end).

Integration of adaptation layer into L2 Stack

The study will consider the following adaptation layer placements:

- integrated with MAC layer or above MAC layer (examples shown in Figure 8.2 - 1a, b),

- above RLC layer (examples shown in Figure 8.2-1c and Figure 8.2-2).

The figures show example protocol stacks and do not preclude other possibilities.

Adaptation header structure

The adaptation layer may consist of sublayers. It is perceivable, for example, that the GTP-U header becomes a part of the adaptation layer. Alternatively, a GTP-U/UDP/IP header stack may be part of the adaptation layer. The design of the adaption header may be architecture-dependent and is FFS.

Observations on adaptation layer placement

- The above-RLC adaptation layer can only support hop-by-hop ARQ. The above-MAC adaptation layer can support both hop-by-hop and end-to-end ARQ.

- Both adaptation layer placements can support aggregated routing, e.g. by inserting an IAB-node address into the adaptation header.

- Both adaptation layer placements can support per-UE-bearer QoS for a large number of UE-bearers.

  - For above-RLC adaptation layer, the LCID space has to be enhanced since each UE-bearer is mapped to an independent logical channel.

  - For above-MAC adaptation layer, UE-bearer-related info has to be carried on the adaptation header.

- Both adaptation layer placements can support aggregated QoS handling e.g. by inserting an aggregated QoS Id into the adaptation header.

  - Aggregated QoS handling reduces the number of queues. This is independent on where the adaptation layer is placed.

- For both adaptation layer placements, aggregation of routing and QoS handling allows proactive configuration of intermediate on-path IAB-nodes, i.e. configuration is independent of UE-bearer establishment/release.

- For both adaptation layer placements, RLC ARQ can be pre-processed on TX side.

Figure 8.2-3: Protocol stack examples for MT-access using L2-relaying with adaptation layer for architecture 1a

Figure 8.2-4: Protocol stack example for MT-access using L2-relaying with adaptation layer for architecture 1b

Adaptation layer for MT-access links

While the adaptation layer is included on RLC-channels that serve for backhauling it is not included on UE-access links. The MT on the IAB-node also sustains an access link, which is used by the IAB-node for its own traffic, e.g., for OAB support. This raises the question if the MT-access link should include an adaptation layer.

Figures 8.2-3 and 8.2-4 show examples of protocol stacks for MT-access. The adaptation layer for the MT-access link is shown dashed in these figures to indicate that it may or may not be included.

The following observations are made for and against the inclusion of the adaptation layer on the MT-access link:

- In case the adaptation layer is included on the MT-access link, the IAB-node’s access- and backhaul-traffic can be multiplexed onto the same RLC-channel(s). Consequently, the protocol stack for the MT-access link is different than for the UE-access link.

- In case the adaptation layer is not included on the MT-access link, the IAB-node’s access traffic and backhaul traffic have to use different RLC-channels. In this case, the same protocol stack can be used for MT-access as for UE access.

## Control-plane considerations for architecture group 1

### 8.3.1	Routing and QoS enforcement for CP signaling

…

### 8.3.2	CP signaling protocols

…

### 8.3.3	CP signaling security protection

…

### 8.3.4  CP alternatives for architecture 1a

In architecture 1a, the UE’s and the MT’s UP and RRC traffic can be protected via PDCP over the wireless backhaul. A mechanism has to be defined to also protect F1-AP traffic over the wireless backhaul.

The following three alternatives can be considered. Other alternatives are not precluded.

Figure 8.3.4- 1: Example for alternative 1 of architecture 1a. 1a: UE’s RRC, 1b: MT’s RRC, 1c: DU’s F1-AP

Alternative 1:

Figure 8.3.4-1 shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for alternative 1. In these examples, the adaptation layer is placed on top of RLC. On the IAB-node’s access link, the adaptation layer may or may not be included as discussed for U-plane in Section 8.2.2. The example does not preclude other options. This alternative has the following main features:

- The UE’s and the MT’s RRC are carried over SRB.

- On the UE’s or MT’s access link, the SRB uses an RLC-channel. On theMT’s access link, the RLC channel may or may not have an adaptation layer.

- On the wireless backhaul links, the SRB’s PDCP layer is carried over RLC-channels with adaptation layer. The adaptation layer placement in the RLC channel is the same for C-plane as for U-plane. The information carried on the adaptation layer may be different for SRB than for DRB.

- The DU’s F1-AP is encapsulated in RRC of the collocated MT. F1-AP is therefore protected by the PDCP of the underlying SRB.

- Within the IAB-donor, the baseline is to use native F1-C stack (see section 9).

Figure 8.3.4 - 2: Example for alternative 2 of architecture 1a. 2a: UE’s RRC, 2b: MT’s RRC, 2c: DU’s F1-AP

Alternative 2:

Figure 8.3.4 - 2 shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for alternative 2. In these examples, the adaptation layer resides on top of RLC. On the IAB-node’s access link, the adaptation layer may or may not be included as discussed for U-plane in Section 8.2.2. The example does not preclude other options. This alternative has the following main features:

- The UE’s and the MT’s RRC are carried over SRB.

- On the UE’s or MT’s access link, the SRB uses an RLC-channel. On the MT’s access link, the RLC channel may or may not have an adaptation layer.

- On the wireless backhaul link, the PDCP of the RRC’s SRB is encapsulated into F1-AP.

- The DU’s F1-AP is carried over an SRB of the collocated MT. F1-AP is protected by this SRB’s PDCP.

- On the wireless backhaul links, the PDCP of the F1-AP’s SRB is carried over RLC-channels with adaptation layer. The adaptation layer placement in the RLC channel is the same for C-plane as for U-plane. The information carried on the adaptation layer may be different for SRB than for DRB.

- Within the IAB-donor, the baseline is to use native F1-C stack (see section 9)

Figure 8.3.4 - 3: Example for alternative 3 of architecture 1a. 3a: UE’s RRC, 3b: MT’s RRC, 3c: DU’s F1-AP

Alternative 3:

Figure 8.3.4 - 3 shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for alternative 3. In these examples, the adaptation layer resides on top of RLC. On the IAB-node’s access link, the adaptation layer may or may not be included as discussed for U-plane in Section 8.2.2. The example does not preclude other options. This alternative has the following main features:

- The UE’s and the MT’s RRC are carried over SRB.

- On the UE’s or MT’s access link, the RRC’s SRB uses an RLC-channel. On the MT’s access link, the RLC channel may or may not have an adaptation layer.

- On the wireless backhaul links, the SRB’s PDCP layer is carried over RLC-channels with adaptation layer. The adaptation layer placement in the RLC channel is the same for C-plane as for U-plane. The information carried on the adaptation layer may be different for SRB than for DRB.

- The DU’s F1-AP is also carried over an SRB of the collocated MT. F1-AP is protected by this SRB’s PDCP.

- On the wireless backhaul links, the PDCP of the this SRB is also carried over RLC-channels with adaptation layer.

- Within the IAB-donor, the baseline is to use native F1-C stack (see section 9).

### 8.3.5 CP alternatives for architecture 1b

Figure 8.3.5 - 1: RRC and F1-AP for architecture 1b. 4a: UE’s RRC, 4b: MT’s RRC, 4c: DU’s F1-AP

Figure 8.3.5 - 1 shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for architecture 1b. In these examples, the adaptation layer carrying the DRB’s PDCP resides on top of RLC. On the IAB-node’s access link, the adaptation layer may or may not be included as discussed for U-plane in Section 8.2.2. The example does not preclude other options.

For architecture 1b, the UE’s or MT’s RRC is carried over SRB. On the wireless backhaul, this SRB’s PDCP is carried over native F1-C.

The DUs on IAB-node and IAB-donor use native F1-C stack.

- Over the wireless backlinks, the IP-layer of this native F1-C stack is provided by a PDU-session. This PDU-session is established between the MT collocated with the DU and a UPF.

- The PDU-session is carried by a DRB between the MT and the CU-UP. Between CU-UP and UPF, the PDU-session is carried via NG-U.

- IP transport between UPF and CU-CP is provided by the PDU-session’s DN. The baseline assumption is that this transport is protected.

Note: SA3 may evaluate requirements on the protection of F1-C transport across the DN between UPF and CU-CP.

- 8.4  User-plane considerations for architecture group 2

- …

********* End of Change **********