3GPP TSG-RAN WG2 Meeting #103bis                                                       R2-18xxxxx

Chengdu, China, Oct 8th – 12th, 2018

Agenda item:	11.1.2

Source: 		Qualcomm Incorporated (Rapporteur)

Title: 	Email discussion [103bis#32][NR - IAB] Unified Design

Document for:	Discussion

# 1. Introduction

This document contains email discussion:

[103bis#32][NR - IAB] Unified design (Qualcomm)

Intended outcome: attempt to extract a set of limited aspects to help reaching consensus on UP transport of unified design.  No new architecture options should be considered.

Intended outcome: Report to next meeting

Deadline:  Thursday 2018-11-01

This email discussion is based on online discussion:

F1-U termination

R2-1814369	Way forward on F1-U termination	Ericsson, AT&T, KDDI	discussion	Rel-15	FS_NR_IAB

Proposal 1. Further work on IAB for architecture group 1A should only consider solutions based on terminating GTP-U and NR user plane protocol in the IAB node.

-	Qualcomm thinks that we can put some of the GTP-U can be included in the adapt header.  Ericsson indicates that we also have the flow control.

-	Qualcomm thinks that we should consider this with the unified design and consider user plane transport as well.

-	Huawei and Intel thought this is RAN3.  Qualcomm explains that this is F1-U and adaptation layer design.

Proposal 2	If the previous proposal is agreed, the remaining work on architecture group 1A should focus on alternatives d) and e) in figure 8.2.2 – 1 in TR 38.874.

-	Qualcomm thinks that this is a WI level decision

=>	Placement of the adaptation layer and details of the adaptation layer should be analysed in view of the unified design

=>	Noted

# 2. Discussion

This discussion aims to identify design examples for the unified design. Each example specifies identifiers carried on F1*-U, where these identifiers are carried in the L2-header-stack, and how they are processed.

The design examples illustrate how the unified design could be realized, identify potential constraints, and may serve as guide for WI stage. Since these designs are solely examples there won’t be any down selection.

Companies should feel free to propose their favorite design example. We should end up with at least one design example, which rapporteur has already provided below. We may end up with a few. In case there are too many design examples, some consolidation will be done, e.g. combine those that only differ with respect to stage-3 aspects.

The discussion contains two phases:

Phase 1: Collecting design examples for unified architecture

Phase 2: Discussion of design example(s) identified

The TP will contain the design examples proposed (and potentially consolidated) and the comparison among them.

Each design example should address the following points:

- Characteristics: A few points on which of the present TR design aspects are applied to design. This should include how N:1 and 1:1 bearer mapping is supported, LCID space shortage, etc.

- F1*-U identifiers used and their placement on L2 header stack

- Downstream and upstream processing of F1-U and F1*-U identifiers and access identifiers by IAB-donor-DU and IAB-node.

It is important to include the processing of F1-U and F1*-U identifiers so that is becomes clear how the design works.

Please see example 1 below and use this template for further design proposals.

## 2.1  Phase 1: UP examples

Example 1 (Qualcomm)

Characteristics:

- IP termination point can be on IAB-donor-DU or IAB-node

- UE-bearers are N:1-mapped to RLC-channels

- RLC-channels are 1:1-mapped to LCHs

- LCID-space extension necessary to support 1:1 bearer mapping

F1*-U identifiers and their placement in L2 header stack:

- UE-bearer-ID above RLC

  - Needs to be available on L2 for packet processing

- IAB-node-address above RLC

  - Needs to be available on L2 for packet processing

  - IAB-node may have multiple addresses, or the address may contain a route-Id for the support of multiple independent routes.

- LCID with extended space on MAC-sub-header

Downstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 1-1: Downstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On wireline network, packet received from CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH-link, packet received from parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID |
| Packet processing | Node derives from packet header and lookup tables: - UE-bearer-ID from UE-bearer-ID - IAB-node-address based on UE-bearer-ID - Egress link type (i.e. if access or BH link) based on UE-bearer-ID - “Access” if UE of UE-bearer-ID is local - “BH” if UE of UE-bearer-ID is remote - If egress = “Access”, derive: - Egress link and UE-bearer based on UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID (N:1 bearer mapping). - LCID based on 1:1 mapping between RLC channel and LCH. | Node derives from packet header and lookup tables: - Ingress RLC channel through 1:1 mapping from LCID - Egress link type (i.e. if access or BH link) based on IAB-node-address - “Access” if address is local - “BH” if address is remote - If egress = “Access”, derive: - Egress link and UE-bearer from UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address - Egress RLC channel based on ingress RLC channel and IAB-node-address (mapping between BH RLC channels) - LCID via 1:1 mapping between RLC channel and LCH. |
| Egress packet | On BH link, packet transmitted to child BH-link holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID | On BH link, packet transmitted to child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID |

Upstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 1-2: Upstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet received from UE or MT holds: - LCID | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet received from UE or MT holds: - LCID |
| Packet processing | Node derives from packet header content and lookup tables: - Ingress RLC-channel based on LCID using 1:1 mapping between RLC channel and LCH. - Ingress link type (i.e. if access or BH link) based on ingress link and LCID. - “Access” if ingress link belongs to UE, or if ingress link belongs to child and LCID belongs to child-MT’s access channel. - “Backhaul” if ingress link belongs to child and andLCID belongs to child’s BH channel. - If ingress = “Access”, derive: - UE-bearer-ID from ingress link and LCID - If ingress = “BH”, derive: - UE-bearer-ID from UE-bearer-ID | Node derives from packet header content and lookup tables: - Ingress RLC-channel based on LCID using 1:1 mapping between RLC channel and LCH. - Ingress link type (i.e. if access or BH link) based on ingress link and LCID. - “Access” if ingress link belongs to UE, or if ingress link belongs to child and LCID belongs to child-MT’s access channel. - “Backhaul” if ingress link belongs to child and LCID belongs to child’s BH channel. - If ingress = “Access”, derive: - UE-bearer-ID from ingress link and LCID - IAB-node-address based on UE-bearer-ID - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID (N:1 bearer mapping) - If ingress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address - Egress RLC channel based on ingress RLC channel and IAB-node-address (mapping between BH RLC channels) - LCID via 1:1 mapping between RLC channel and LCH. |
| Egress packet | On wireline network, packet transmitted to CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH link, packet transmitted to parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID |

Example 2 (Huawei)

Characteristics:

- UE-bearers can be either N:1 mapped to RLC-channels, or 1:1 mapped to RLC-channels

- Two flavors of LCH based on type of RLC-channel(s) mapped to LCH:

  - N:1 mapped RLC-channels are 1:1 mapped to LCHs

  - 1:1 mapped RLC-channels are K:1 mapped to LCHs (K ≥ 1)

- LCH type explicitly or implicitly indicated (e.g. a set of LCIDs may be configured for N:1 mapping)

- LCID-space extension not needed

- IP termination for F1 on IAB-donor-DU

F1*-U identifiers and their placement in L2 header stack:

- UE-bearer-ID

  - Needs to be available on L2 for packet processing

- IAB-node-address

  - Needs to be available for routing

- LCID (reuse existing LCID space)

Processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Downstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 2-1: Downstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On wireline network, packet received from CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH-link, packet received from parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID |
| Packet processing | Node derives from packet header and lookup tables: - UE-bearer-ID from UE-bearer-ID - IAB-node-address based on UE-bearer-ID - Egress link type (i.e. if access or BH link) based on UE-bearer-ID - “Access” if UE of UE-bearer-ID is local - “BH” if UE of UE-bearer-ID is remote - If egress = “Access”, derive: - Egress link and UE-bearer based on UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID - If bearer mapping = N:1: - Egress LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress LCID based mapping between 1:1 mapped RLC channels and LCH | Node derives from packet header and lookup tables: - If LCD type = “N:1 bearer mapping”: - Ingress RLC channel based on LCID - If LCD type = “1:1 bearer mapping”: - Ingress RLC channel based on combination of UE-bearer-ID + LCID - Egress link type (i.e. if access or BH link) based on IAB-node-address - “Access” if address is local - “BH” if address is remote - If egress = “Access”, derive: - Egress link and UE-bearer from UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address (routing) - On selected egress link RLC-channel and LCID are selected as follows: - If bearer mapping = N:1: - Egress RLC-channel on UE-bearer-ID. - LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress RLC-channel based on ingress RLC-channel (mapping is implicit, since for 1:1 mapped bearers we can consider ingress and egress RLC channels to be the same RLC-channel) - LCID based mapping between 1:1 mapped RLC channels and LCH. |
| Egress packet | On BH link, packet transmitted to child BH-link holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID | On BH link, packet transmitted to child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID |

Upstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 2-2: Upstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - Donor-DU-address - LCID On access link, RLC packet received from UE or MT holds: - LCID | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet received from UE or MT holds: - LCID |
| Packet processing | Node derives from packet header content and lookup tables: - Ingress link type (i.e. if access or BH link) based on ingress link. - “Access” if ingress link belongs to local UE. - “Backhaul” if ingress link belongs to child and. Note: Bearers terminating at child-MT may be treated either as “Access” links or “Backhaul” links depending on whether Adapt is implemented for such bearers (can be finalized in WI stage). - If ingress = “Access”, derive: - UE-bearer-ID from ingress link and LCID - If ingress = “BH”, derive: - If LCD type = “N:1 bearer mapping”: - RLC channel based on LCID - If LCD type = “1:1 bearer mapping”: - RLC channel based on combination of UE-bearer-ID + LCID - UE-bearer-ID from UE-bearer-ID | Node derives from packet header and lookup tables: - If LCD type = “N:1 bearer mapping”: - Ingress RLC channel based on LCID - If LCD type = “1:1 bearer mapping”: - Ingress RLC channel based on combination of UE-bearer-ID + LCID - Egress link type (i.e. if access or BH link) based on IAB-node-address - “Access” if address is local - “BH” if address is remote - If egress = “Access”, derive: - Egress link and UE-bearer from UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address (routing) - On selected egress link RLC-channel and LCID are selected as follows: - If bearer mapping = N:1: - Egress RLC-channel on UE-bearer-ID. - LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress RLC-channel based on ingress RLC-channel (mapping is implicit, since for 1:1 mapped bearers we can consider ingress and egress RLC channels to be the same RLC-channel) - LCID based mapping between 1:1 mapped RLC channels and LCH. |
| Egress packet | On wireline network, packet transmitted to CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH link, packet transmitted to parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID |

Example 3 (LG)

Characteristics:

- Non-IP-based Adapt

- UE-bearers are N:1-mapped to RLC-channels

- No multiplexing between RLC-channels to LCHs same as legacy (i.e. one RLC channel is associated with only one LCH)

- LCID-space extension not needed (1:1 mapping is the special case of N:1 mapping. If IAB node can accommodate all UEs with 1:1 mapping, IAB node will provide 1:1 mapping. But if IAB node cannot support all UEs, IAB node would support N:1 mapping.)

- IP termination for F1 on IAB-donor-DU

F1*-U identifiers and their placement in L2 header stack:

- UE-bearer-ID above RLC

- IAB-node-address above RLC

Downstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 2-1: Downstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On wireline network, packet received from CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | F1*-U packet received from parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address |
| Packet processing | Node derives from F1-U-info and lookup tables: - IAB-node-address based on UE-bearer-ID - Egress link type (i.e. if access or BH link) based on UE-bearer-ID - “Access” if UE of UE-bearer-ID is local - “BH” if UE of UE-bearer-ID is remote - If egress = “Access”, derive: - UE-bearer based on UE-bearer-ID - If egress = “BH”, derive: - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID (N:1 bearer mapping). | Node derives from F1*-U info and lookup tables: - Egress link type (i.e. if access or BH link) based on IAB-node-address - “Access” if this IAB node is destination of the address - “BH” if this IAB node is not destination of the address - If egress = “Access”, derive: - UE-bearer from UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address - Egress RLC channel based on ingress RLC channel and IAB-node-address (mapping between BH RLC channels) |
| Egress packet | F1*-U packet transmitted to child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address | F1*-U packet transmitted to child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address |

Upstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 2-2: Upstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - Donor-DU-address | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address |
| Packet processing | Node derives from packet header content and lookup tables: - Ingress link type (i.e. if access or BH link) based on ingress link. - “Access” if ingress link belongs to UE, or if ingress link belongs to child-MT’s access channel. - “Backhaul” if ingress link belongs to child. - If ingress = “Access”, derive: - UE-bearer-ID from ingress link - If ingress = “BH”, derive: - UE-bearer-ID from UE-bearer-ID | Node derives from packet header content and lookup tables: - Ingress link type (i.e. if access or BH link) based on ingress link. - “Access” if ingress link belongs to UE, or if ingress link belongs to child-MT’s access channel. - “Backhaul” if ingress link belongs to child’s BH channel. - If ingress = “Access”, derive: - UE-bearer-ID from ingress link - IAB-node-address based on UE-bearer-ID - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID (N:1 bearer mapping) - If ingress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address |
| Egress packet | On wireline network, packet transmitted to CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH link, packet transmitted to parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address |

…

Example 4 (Samsung)

Characteristics:

- Non-IP-based Adapt

- LCID-space extension necessary to support 1:1 bearer mapping

- To support N:1 mapping, UE-bearers are N:1-mapped to RLC-channels and RLC-channels are 1:1-mapped to LCHs; LCID space extension is required to support 1:1 mapping

  - In one sub-option, the Adapt layer is always configured, even when not performing aggregation, to facilitate uniform packet processing for routing purposes;

  - In another sub-option, the Adapt layer is only enabled when N:1 mapping is needed

- The design only supports hop-by-hop ARQ for N:1 mapping; both hop-by-hop and end-to-end can be supported for 1:1 mapping

F1*-U identifiers and their placement in L2 header stack:

- UE-bearer-ID above RLC

- Option A: IAB-node-address above RLC (IAB-node-address explicitly included with UE bearer ID)

- Option B: UE-bearer-ID with a routing table (from which we get the next-hop information)

- LCID with extended space on MAC-sub-header

Processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 1: Processing for example 4 (Option A) (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | F1-U packet received from CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | F1*-U packet received from parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID |
| Packet processing | Node derives from F1-U-info and lookup tables: - IAB-node-address based on UE-bearer-ID - Egress link type (i.e. if access or BH link) based on UE-bearer-ID - “Access” if UE of UE-bearer-ID is local - “BH” if UE of UE-bearer-ID is remote - If egress = “Access”, derive: - UE-bearer based on UE-bearer-ID - If egress = “BH”, derive: - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID (N:1 bearer mapping). - LCID based on 1:1 mapping between RLC channel and LCH. | Node derives from F1*-U info and lookup tables: - Ingress RLC channel through 1:1 mapping from LCID - Egress link type (i.e. if access or BH link) based on IAB-node-address - “Access” if address is local - “BH” if address is remote - If egress = “Access”, derive: - UE-bearer from UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address - Egress RLC channel based on ingress RLC channel and IAB-node-address (mapping between BH RLC channels) - LCID via 1:1 mapping between RLC channel and LCH. |
| Egress packet | F1*-U packet transmitted to child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID | F1*-U packet transmitted to child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID |

Example 5 (Nokia)

Characteristics:

- IP termination for F1 on IAB-donor-DU

- Adaptation layer above MAC

- LCID space extension is not required

- UE-bearers can be either N:1 mapped to RLC-channels, or 1:1 mapped to RLC-channels

- GTP-U header is used for Multiplexing/De-multiplexing above RLC for N:1 mapping

- Adaptation header (UE-ID) together with LCID could be used for Multiplexing/De-multiplexing above MAC for 1:1

“F1*-U” identifiers and their placement in L2 header stack:

- IAB node address as part of adaptation header above MAC (for routing)

- UE-Id as part of adaptation header above MAC (for multiplexing RLC channels to logical channels), only for 1:1 mapping

- GTP-U header is used for Multiplexing/De-multiplexing above RLC for N:1 mapping

Protocol stack:

Processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 2-1: Downstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On wireline network, packet received from CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH-link, packet received from parent holds: - UE-bearer-ID (=GTP-U TEID) - IAB-node-address - UE-ID (optional) - LCID |
| Packet processing | Node derives from packet header and lookup tables: - UE-bearer-ID from UE-bearer-ID - IAB-node-address based on UE-bearer-ID - Egress link type (i.e. if access or BH link) based on UE-bearer-ID - “Access” if UE of UE-bearer-ID is local - “BH” if UE of UE-bearer-ID is remote - If egress = “Access”, derive: - UE-bearer based on UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID - If bearer mapping = N:1: - Egress LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress LCID based mapping between 1:1 mapped RLC channels and LCH | Node derives from packet header and lookup tables: - If LCH type = “N:1 bearer mapping”: - Ingress RLC channel based on LCID - If LCH type = “1:1 bearer mapping”: - Ingress RLC channel based on combination of UE- ID + LCID - Egress link type (i.e. if access or BH link) based on IAB-node-address - “Access” if address is local - “BH” if address is remote - If egress = “Access”, derive: - UE-bearer from UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address (routing) - On selected egress link RLC-channel and LCID are selected as follows: - If bearer mapping = N:1: - Egress RLC-channel on UE-bearer-ID. - LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress RLC-channel based on ingress RLC-channel (mapping is implicit, since for 1:1 mapped bearers we can consider ingress and egress RLC channels to be the same RLC-channel) - UE-ID and LCID based on mapping between 1:1 mapped RLC channels and LCH |
| Egress packet | On BH link, packet transmitted to child BH-link holds: - UE-bearer-ID (=GTP-U TEID) - IAB-node-address - UE-ID (optional) - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID | On BH link, packet transmitted to child holds: - UE-bearer-ID (=GTP-U TEID) - IAB-node-address - UE-ID (optional) - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID |

Upstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 2-2: Upstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On BH link, packet received from child holds: - UE-bearer-ID (=GTP-U TEID) - Donor-DU-address - UE-ID (optional) - LCID On access link, RLC packet received from UE holds: - LCID | On BH link, packet received from child holds: - UE-bearer-ID (=GTP-U TEID) - Donor-DU-address - UE-ID (optional) - LCID On access link, RLC packet received from UE holds: - LCID |
| Packet processing | Node derives from packet header content and lookup tables: - Ingress link type (i.e. if access or BH link) based on ingress link. - “Access” if ingress link belongs to local UE. - “Backhaul” if ingress link belongs to child and. - If ingress = “Access”, derive: - UE-bearer-ID from C-RNTI and LCID - UE-ID from C-RNTI - If ingress = “BH”, derive: - If LCH type = “N:1 bearer mapping”: - RLC channel based on LCID - If LCH type = “1:1 bearer mapping”: - RLC channel based on combination of UE-ID + LCID - UE-bearer-ID from UE-bearer-ID | Node derives from packet header and lookup tables: - If LCH type = “N:1 bearer mapping”: - Ingress RLC channel based on LCID - If LCH type = “1:1 bearer mapping”: - Ingress RLC channel based on combination of UE-ID + LCID - If egress = “BH”, derive: - Egress Donor-DU-address= Ingress Donor-DU-address - Egress link based on Donor-DU-address (routing) - On selected egress link RLC-channel and LCID are selected as follows: - If bearer mapping = N:1: - Egress RLC-channel on UE-bearer-ID. - LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress RLC-channel based on ingress RLC-channel (mapping is implicit, since for 1:1 mapped bearers we can consider ingress and egress RLC channels to be the same RLC-channel) - UE-ID and LCID based mapping between 1:1 mapped RLC channels and LCH. |
| Egress packet | On wireline network, packet transmitted to CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH link, packet transmitted to parent holds F1*-U-info with: - UE-bearer-ID (=GTP-U TEID) - IAB-node-address - UE-ID (optional) - LCID |

…

  - Phase 2: Discussion

- Does the above design example meet the requirements of the unified design? If you believe it doesn’t please provide reasons.

| Company | Answer |
| --- | --- |

- Is the above design example technically correct? Is something missing? Should something be changed? Is something unclear or not sufficiently well described?

| Company | Answer |
| --- | --- |

- Do you have a different design example in mind which should be captured here? Please describe this scenario, or describe how it would differ from the design example above.

| Company | Answer |
| --- | --- |

- Are there other aspects that should be considered for the discussion of the unified design? (Please note that this discussion only addresses UP and not CP)

| Company | Answer |
| --- | --- |

  - Summary

…

# 3. Text Proposal

********* Start of Change **********

# 8	Radio protocol aspects

…

### 8.x 	Examples of Unified Design for Architecture Group 1

Below, examples are provided for the unified design. Each example provides:

- Characteristics

- F1*-U identifiers and their placement in L2 header stack

- Variants, options, optimizations

- Downstream and upstream processing steps for IAB-node and IAB-donor-DU

Example 1

Characteristics:

- UE-bearers are N:1-mapped to RLC-channels.

- RLC-channels are 1:1-mapped to LCHs.

- 1:1 bearer mapping is supported via setting N=1. This requires LCID-space extension.

F1*-U identifiers and their placement in L2 header stack:

- UE-bearer-ID above RLC

  - Needs to be available on L2 at the MAC scheduler for UE-bearer-specific QoS support

- IAB-node-address/IAB-node-DU-address above RLC

  - Needs to be available on L2 for routing

- LCID with extended space on MAC-sub-header

Variants, options, optimizations:

- IAB-node/IAB-donor-DU may have multiple addresses, or the address may contain a route-Id for the support of multiple independent routes.

- MT-access link and BH link may be multiplexed onto common RLC channel. In this case, an access/backhaul multiplexing indicator is carried on the packet.

  - In downstream direction, this indicator is set by the MT’s parent node to differentiate access and backhaul traffic and evaluated by the MT.

  - In upstream direction, this indicator is set by the MT node to differentiate access and backhaul traffic and evaluated by the MT’s parent node.

- For 1:1 bearer mapping, end-to-end ARQ may be supported.

Downstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 1-1: Downstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On wireline network, packet received from CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH-link, packet received from parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID |
| Packet processing | Node derives from packet header and lookup tables: - UE-bearer-ID from UE-bearer-ID - IAB-node-address based on UE-bearer-ID - Egress link type (i.e. if access or BH link) based on UE-bearer-ID - “Access” if UE of UE-bearer-ID is local - “BH” if UE of UE-bearer-ID is remote - If egress = “Access”, derive: - Egress link and UE-bearer based on UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID (N:1 bearer mapping). - LCID based on 1:1 mapping between RLC channel and LCH. | Node derives from packet header and lookup tables: - Ingress RLC channel through 1:1 mapping from LCID - Egress link type (i.e. if access or BH link) based on IAB-node-address - “Access” if address is local - “BH” if address is remote - If egress = “Access”, derive: - Egress link and UE-bearer from UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address - Egress RLC channel based on ingress RLC channel and IAB-node-address (mapping between BH RLC channels) - LCID via 1:1 mapping between RLC channel and LCH. |
| Egress packet | On BH link, packet transmitted to child BH-link holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID | On BH link, packet transmitted to child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID |

Upstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 1-2: Upstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - IAB-donor-DU-address - LCID On access link, RLC packet received from UE or MT holds: - LCID | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - IAB-donor-DU-address - LCID On access link, RLC packet received from UE or MT holds: - LCID |
| Packet processing | Node derives from packet header content and lookup tables: - Ingress RLC-channel based on LCID using 1:1 mapping between RLC channel and LCH. - Ingress link type (i.e. if access or BH link) based on ingress link and LCID. - “Access” if ingress link belongs to UE, or if ingress link belongs to child and LCID belongs to child-MT’s access channel. - “Backhaul” if ingress link belongs to child and and LCID belongs to child’s BH channel. - If ingress = “Access”, derive: - UE-bearer-ID from ingress link and LCID - If ingress = “BH”, derive: - UE-bearer-ID from UE-bearer-ID | Node derives from packet header content and lookup tables: - Ingress RLC-channel based on LCID using 1:1 mapping between RLC channel and LCH. - Ingress link type (i.e. if access or BH link) based on ingress link and LCID. - “Access” if ingress link belongs to UE, or if ingress link belongs to child and LCID belongs to child-MT’s access channel. - “Backhaul” if ingress link belongs to child and LCID belongs to child’s BH channel. - If ingress = “Access”, derive: - UE-bearer-ID from ingress link and LCID - IAB-donor-DU-address based on UE-bearer-ID - Egress link based on IAB-donor-DU-address (routing) - Egress RLC-channel based on UE-bearer-ID (N:1 bearer mapping) - If ingress = “BH”, derive: - Egress IAB-donor-DU-address = Ingress IAB-donor-DU-address - Egress link based on IAB-donor-DU-address - Egress RLC channel based on ingress RLC channel and IAB-donor-DU-address (mapping between BH RLC channels) - LCID via 1:1 mapping between RLC channel and LCH. |
| Egress packet | On wireline network, packet transmitted to CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH link, packet transmitted to parent holds F1*-U-info with: - UE-bearer-ID - IAB-donor-DU-address - LCID |

Example 2

Characteristics:

- UE-bearers can be either N:1 mapped to RLC-channels, or 1:1 mapped to RLC-channels

- Two flavors of LCH based on type of RLC-channel(s) mapped to LCH:

  - N:1 mapped RLC-channels are 1:1 mapped to LCHs

  - 1:1 mapped RLC-channels are K:1 mapped to LCHs (K ≥ 1)

- LCH type explicitly or implicitly indicated (e.g. a set of LCIDs may be configured for N:1 mapping)

- LCID-space extension not needed

F1*-U identifiers and their placement in L2 header stack:

- UE-bearer-ID

  - Needs to be available on L2 at the MAC scheduler for UE-bearer-specific QoS support

- IAB-node-address/IAB-donor-DU-address

  - Needs to be available on L2 for routing

  - IAB-node/IAB-donor-DU may have multiple addresses, or the address may contain a route-Id for the support of multiple independent routes.

- LCID on MAC header reusing existing LCID field (no LCID-space extension needed)

Variants, options, optimizations:

- IAB-node/IAB-donor-DU may have multiple addresses, or the address may contain a route-Id for the support of multiple independent routes.

- MT-access link and BH link may be multiplexed onto common RLC channel. In this case, an access/backhaul multiplexing indicator is carried on the packet.

  - In downstream direction, this indicator is set by the MT’s parent node to differentiate access and backhaul traffic and evaluated by the MT.

  - In upstream direction, this indicator is set by the MT node to differentiate access and backhaul traffic and evaluated by the MT’s parent node.

- For 1:1 bearer mapping, end-to-end ARQ may be supported.

- For 1:1 mapping, the UE-bearer-Id may be replaced by UE-Id + LCID. This implies that all RLC-channels supporting the UE-bearer use the same LCID.

Downstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 2-1: Downstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On wireline network, packet received from CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH-link, packet received from parent holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID |
| Packet processing | Node derives from packet header and lookup tables: - UE-bearer-ID from UE-bearer-ID - IAB-node-address based on UE-bearer-ID - Egress link type (i.e. if access or BH link) based on UE-bearer-ID - “Access” if UE of UE-bearer-ID is local - “BH” if UE of UE-bearer-ID is remote - If egress = “Access”, derive: - Egress link and UE-bearer based on UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress link based on IAB-node-address (routing) - Egress RLC-channel based on UE-bearer-ID - If bearer mapping = N:1: - Egress LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress LCID based mapping between 1:1 mapped RLC channels and LCH | Node derives from packet header and lookup tables: - Determination of N:1 vs. 1:1 bearer mapping based on LCID. - If LCD type = “N:1 bearer mapping”: - Ingress RLC channel based on LCID - If LCD type = “1:1 bearer mapping”: - Ingress RLC channel based on combination of UE-bearer-ID + LCID - Egress link type (i.e. if access or BH link) based on IAB-node-address - “Access” if address is local - “BH” if address is remote - If egress = “Access”, derive: - Egress link and UE-bearer from UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-node-address = Ingress IAB-node-address - Egress link based on IAB-node-address (routing) - On selected egress link RLC-channel and LCID are selected as follows: - If bearer mapping = N:1: - Egress RLC-channel on UE-bearer-ID. - LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress RLC-channel based on ingress RLC-channel (mapping is implicit, since for 1:1 mapped bearers we can consider ingress and egress RLC channels to be the same RLC-channel) - LCID based mapping between 1:1 mapped RLC channels and LCH. |
| Egress packet | On BH link, packet transmitted to child BH-link holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID | On BH link, packet transmitted to child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet transmitted to UE or MT holds: - LCID |

Upstream processing of F1-U and F1*-U identifiers by IAB-donor-DU and IAB-node

Table 2-2: Upstream packet processing example 1 (red: ingress parameters; blue: egress parameters)

|  | IAB-donor DU | IAB-node |
| --- | --- | --- |
| Ingress packet | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - Donor-DU-address - LCID On access link, RLC packet received from UE or MT holds: - LCID | On BH link, packet received from child holds F1*-U-info with: - UE-bearer-ID - IAB-node-address - LCID On access link, RLC packet received from UE or MT holds: - LCID |
| Packet processing | Node derives from packet header content and lookup tables: - Ingress link type (i.e. if access or BH link) based on ingress link. - “Access” if ingress link belongs to local UE. - “Backhaul” if ingress link belongs to child and. Note: Bearers terminating at child-MT may be treated either as “Access” links or “Backhaul” links depending on whether Adapt is implemented for such bearers (can be finalized in WI stage). - If ingress = “Access”, derive: - UE-bearer-ID from ingress link and LCID - If ingress = “BH”, derive: - If LCD type = “N:1 bearer mapping”: - RLC channel based on LCID - If LCD type = “1:1 bearer mapping”: - RLC channel based on combination of UE-bearer-ID + LCID - UE-bearer-ID from UE-bearer-ID | Node derives from packet header and lookup tables: - Determination of N:1 vs. 1:1 bearer mapping based on LCID. - If LCD type = “N:1 bearer mapping”: - Ingress RLC channel based on LCID - If LCD type = “1:1 bearer mapping”: - Ingress RLC channel based on combination of UE-bearer-ID + LCID - Egress link type (i.e. if access or BH link) based on IAB-donor-DU-address - “Access” if address is local - “BH” if address is remote - If egress = “Access”, derive: - Egress link and UE-bearer from UE-bearer-ID - LCID based on UE-bearer-ID - If egress = “BH”, derive: - Egress IAB-donor-DU-address = Ingress IAB-donor-DU-address - Egress link based on IAB-donor-DU-address (routing) - On selected egress link RLC-channel and LCID are selected as follows: - If bearer mapping = N:1: - Egress RLC-channel on UE-bearer-ID. - LCID based on mapping between N:1 mapped RLC channel and LCH. - If bearer mapping = 1:1: - Egress RLC-channel based on ingress RLC-channel (mapping is implicit, since for 1:1 mapped bearers we can consider ingress and egress RLC channels to be the same RLC-channel) - LCID based mapping between 1:1 mapped RLC channels and LCH. |
| Egress packet | On wireline network, packet transmitted to CU holds F1-U-info with: - UE-bearer-ID (=GTP-U TEID) | On BH link, packet transmitted to parent holds F1*-U-info with: - UE-bearer-ID - IAB-donor-DU-address - LCID |