3GPP TSG-RAN WG2 Meeting #110-e	R2-200xxxx

# Online, June 01 – 12, 2020

Agenda item:	x.y.z

Source: 	Ericsson

Title: 	Email discussion report: [Post109bis-e][946][POS] Reference for additional path reporting (Ericsson)

Document for:		Discussion and Decision

# 1. 	Introduction

This document summarizes the following email discussion:

- [Post109bis-e][946][POS] Reference for additional path reporting (Ericsson)

Scope: Discuss the options for a time reference convention for additional path reporting and conclude a way forward, starting from the text proposal in Annex 1 of R2-2003997.

Intended outcome: Summary for next meeting

Deadline:  Long

To allow a discussion of the summary as well, companies are asked to provide comments no later than Tuesday May 19th, 10.00 UTC.

Section 2 lists the different options regarding the reference for additional path reporting as discussed in [1] and [2], and introduces the key issues of the way forward that companies are requested to provide comments to. This is also related to the text proposal in the Annex, based on [1].

## References

[1]	R2-2003997, "Email discussion report: [AT109bis-e][610][POS] LPP proposals (Ericsson).

[2]	R2-2003061, "Remaining issues with LPP", Huawei, HiSilicon.

[3]	R2-20xxxxx, "RAN2 Chairman’s Notes", RAN2#109bis-e.

# 2. 	Discussion

The current definition in LPP for the additional path reporting is ambiguous/unclear; in particular together with the additional measurement reporting capability. The figure below was originally provided in [2], but is here refined to also illustrate the third report representation brought forward during the email discussion reported in [1]. It illustrates the different path timing possibilities (for different resources of two exemplary TRPs):

Reference Path

Reference TRP

Path#1

Neighbouring TRP

Path#2

Path#2

Path#1

Path#1

Path#2

Path#2

Path#1

Path#1

Path#2

Path#2

Resource#0

Resource#1

Resource#2

Resource#0

Resource#1

Resource#2

RSTD

The black line illustrates the "main RSTD", nr-RSTD measurement.

The blue line illustrates the nr-RSTD-ResultDiff (provided in IE NR-DL-TDOA-AdditionalMeasurementElement).

The red line illustrates the nr-AdditionalPathList for the "main RSTD" (outside the NR-DL-TDOA-AdditionalMeasurementElement).

The green, orange and purple dashed curves are the candidates for the nr-AdditionalPathList for NR-DL-TDOA-AdditionalMeasurementElement according to [2] and the discussion in [1].

The proposal in [2] and the discussion in [1 ]is to clarify the definition of the time reference of the additional paths, with the following three options:

Option 1.  The additional path time reference is the first path of the resource (the reference path) illustrated in “orange” in the figure

Option 2.  The additional path time reference is the first path of the resource used to determine RSTD  illustrated in “green” in the figure.

Option 3. The additional measurements (blue – reference TRP and dashed blue – neighbour TRP) and additional path (dashed purple) time reference is the detected reference TRP path used to determine the RSTD value

A few things shall be noted:

- The time reference is only about reporting – the actual path timing measurements are the same for all options and it is possible in post-processing to go between the representations. For the reference cell with path timings tr00 and tr01 of resource 0 and tr10 and tr11 of resource 1, where resource 0 path timing tr00 is used to determine RSTD,

  - option 1 reports additional measurements (tr10-tr00) and additional paths (tr01-tr00) and (tr11-tr10)

  - option 2 reports additional measurements (tr10-tr00) and additional paths (tr01-tr00) and (tr11-tr00) – note that the same information as in option 1 is retrieved in post-processing by (tr11-tr00) - (tr10-tr00) = (tr11-tr10)

  - option 3 is the same as option 2 for the reference cell

- The resources are typically configured with beam sweeping and therefore corresponds to different DL-PRS transmission times. Therefore, definitions need to acknowledge this difference in transmission time.

- If there eventually will be RAN4 requirements for these relative RSTD and addional path timing reports, this can have an impact on how these are defined, if requirements are believed to be defined in relation to what has been reported, not measured.

Companies are asked to provide comments and a preference of option in the table below

| Company | Comments |
| --- | --- |

Summary:

The text proposal in the Annex of [1] is used as baseline for the discussion here, see Annex. Companies are asked to provide comments to the text proposal

| Company | Comments |
| --- | --- |

Summary:

# 3. 	Conclusion

# Annex 1, Text proposal to 3GPP TS 37.355 for Additional path representation

#### 6.5.10.4	NR-DL-TDOA Location Information Elements

#### –	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR-DL TDOA measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP in case RSTD measurements are reported. The first TRP in the list may or may not be the reference TRP indicated in the NR-DL-PRS-AssistanceData. Furthermore, the target device selects a reference resource per TRP, and compiles the measurements per TRP based on the selected reference resource.

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16						DL-PRS-IdInfo-r16,

nr-DL-TDOA-MeasList-r16							NR-DL-TDOA-MeasList-r16,

nr-AdditionalPathListRef-r16					NR-AdditionalPathList-r16		OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsRef-r16		NR-DL-TDOA-AdditionalMeasurements-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						INTEGER (0..ffs),	-- FFS on the value range

nr-AdditionalPathListNeighbor-r16		NR-AdditionalPathList-r16		OPTIONAL,

nr-TimingMeasQuality-r16				NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-DL-TDOA-AdditionalMeasurementsNeighbor-r16				NR-DL-TDOA-AdditionalMeasurements-r16,

...

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			INTEGER (0..ffs),	-- FFS on the value range to be decided in RAN4

dl-PRS-RSRP-ResultDiff-r16	INTEGER (FFS)		OPTIONAL, -- FFS on the value range	to be decided in RAN4

nr-AddMeasAdditionalPathList-r16		NR-AdditionalPathList-r16		OPTIONAL,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs per UE

-- ASN1STOP

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

nr-AdditionalPathListRef This field specifies one or more additional detected path timing values for the reference TRP, relative to the path timing used for determining the nr-RSTD value (the reference path timing). If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nr-AdditionalPathListNeighbor This field specifies one or more additional detected path timing values for the neighbour TRP, relative to the path timing used for determining the nr-RSTD value (the reference path timing). If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in FFS. Mapping of the measured quantity is defined as in FSS.

nr-TimingMeasQuality This field specifies the target device′s best estimate of the quality of the measurement.

nr-RSTD-ResultDiff This field specifies the relative time difference between the detected path timing of this DL-PRS resource relative to the path timing used for determining the nr-RSTD value, compensated for the difference in DL-PRS transmission timing.

nr-AddMeasAdditionalPathList This field specifies one or more additional detected path timing values of this DL-PRS resource, relative to the detected path timing of this DL-PRS resource (the reference path timing). If this field was requested but is not included, it means the UE did not detect any additional path timing values.

[…]

#### 6.5.12.4	NR-Multi-RTT Location Information Elements

#### –	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16	NR-Multi-RTT-MeasList-r16,

...

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-UE-RxTxTimeDiff-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range to be decided in RAN4

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16	OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-TimingMeasQuality-r16			NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-Multi-RTT-AdditionalMeasurements-r16		NR-Multi-RTT-AdditionalMeasurements-r16,

...

}

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-UE-RxTxTimeDiffAdditional-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range

nr-AddMeasAdditionalPathList-r16			NR-AdditionalPathList-r16		OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in FFS.

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP, relative to the path timing used for determining the nr-UE-RxTxTimeDiff valuevalue. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nr-AddMeasAdditionalPathList This field specifies one or more additional detected path timing values of this DL-PRS resource, relative to the detected path timing used for determining the nr-UE-RxTxTimeDiffAdditional value (the reference path timing). If this field was requested but is not included, it means the UE did not detect any additional path timing values.

[…]

#### –	NR-AdditionalPath

The IE NR-AdditionalPath is used by the target device to provide information about additional paths in association to the path timing measurements associated to NR positioning in the form of a relative time difference and a quality value. The additional path nr-relativeTimeDifference is the detected path timing relative to the reference path timing used for determining the positioning measurements, and each additional path can be associated with a quality value nr-path-Quality.

-- ASN1START

NR-AdditionalPath-r16 ::= SEQUENCE {

nr-relativeTimeDifference-r16	INTEGER (FFS),--FFS to be decided in RAN4

nr-path-Quality-r16				NR-TOAMeasQuality-r16				OPTIONAL,

...

}

-- ASN1STOP

NR-AdditionalPath field descriptions

nr-relativeTimeDifference This field specifies the additional detected path timing relative to the reference path timing. If the additional detected path timings and the reference path timing are associated to different DL-PRS transmission timings, the device subtracts the transnmission timing difference from the value. A positive value indicates that the particular path is later in time than the reference path timing; a negative value indicates that the particular path is earlier in time than the reference path timing.

nr-path-Quality This field specifies the target device′s best estimate of the quality of the detected timing of the additional path.