3GPP TSG-RAN WG2 Meeting #110-e	 R2-20xxxxx

Online, 01 – 12 June 2020

Title:	Draft LS on UE capability for CGI reporting

Response to:

Release:	Release 16

Work Item:	NR_newRAT-Core

Source:	vivo [TSG RAN WG2]

To:	RAN4

Cc:

Contact Person:

#### Name:	Kimba Dit Adamou, Boubacar

E-mail Address:	kimba@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has further discussed UE capability for CGI reporting and RAN2 has agreed to introduce new UE capabilities as follows:

- In 38.306

- eutra- CGI-Reporting-NEDC

Defines whether the UE supports acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NEDC is configured.

- eutra- CGI-Reporting-NRDC

Defines whether the UE supports acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NRDC is configured wherein MN and SN have different DRX cycles, and on-duration configured by MN does not contain on-duration configured by SN.

- nr- CGI-Reporting-NEDC

Defines whether the UE supports acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured.

- nr- CGI-Reporting-NRDC

Defines whether the UE supports acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN.

- In 36.306

- eutra-CGI-Reporting-NEDC-r16

This parameter defines whether the UE supports acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 36.331 [5] when the NE-DC is configured.

2. Actions:

To RAN4

ACTION: 	RAN2 respectfully asks RAN4 to take the RAN2 considerations on CGI reporting into account, and provide feedback, if any.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#111-e		17 – 28 August 2020			e-meeting