3GPP TSG RAN WG2#110-e	R2-200XXXX

Online meeting, 1st - 12th June, 2020

Title:	LS to RAN1 on physical layer related agreements

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

#### Name:	Jongyoul Lee

E-mail Address:	jongyool.lee@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 would like to inform the following physical layer related agreements:

Agreements on groupcast HARQ:

1: 	When TX UE enabled distance-based HARQ feedback by a SCI but RX UE’s location information is not available, RX UE sends HARQ feedback according to the decoding status of the MAC PDU.

2:	For sidelink groupcast option1, TX UE can enables HARQ feedback without the distance-based operation when range configuration for sidelink logical channel or zone_id is not (pre-)configured.

Agreements on mode 2 resource allocation:

1: 	RAN2 expects that RAN1 will discuss whether ReTX resources of a MAC PDU are reserved neither right on nor after new TX resource of the next MAC PDU for a configured sidelink grant reserved for a particular Sidelink process.

Agreements on resource pool:

1:	For SL mode2, UE considers mapping between LCH and HARQ feedback enable/disable attribute to select TX resource pool. If UE is configured with multiple TX resource pools with the same HARQ feedback enable/disable attribute, it’s up to UE implementation to select one out of them.

2:	Not to introduce LTE TX-carrier reselection based TX pool selection in Rel-16.

In addition, as in LTE sidelink, UE can create a selected sidelink grant for a Sidelink process in the resource reservation procedure for transmission of a single MAC PDU or transmissions of multiple MAC PDUs in NR sidelink.

During RAN2#110e, RAN2 discussed whether UE is allowed to reserve a retransmission opportunity of a MAC PDU beyond the next initial transmission opportunity of another MAC PDU for a Sidelink process in the resource reservation procedure for transmission of multiple MAC PDU. According to 38.321, UE cannot perform retransmission of the MAC PDU together with the initial transmission of another MAC PDU in parallel for the same selected sidelink grant because there is only one Sidelink process associated to the selected sidelink grant. Thus, some companies think the retransmission opportunity of a MAC PDU should be reserved neither right on nor after the next initial transmission opportunity for a particular Sidelink process.

2. Actions:

To RAN1 group

RAN2 respectfully asks RAN1 to take the above into account in their further work, and provide feedback on whether ReTX resources of a MAC PDU are reserved neither right on nor after new TX resource of the next MAC PDU for a configured sidelink grant reserved for a particular Sidelink process.

3. Date of Next TSG-RAN1 Meetings:

3GPPRAN2#111	17 - 28 August 2020   	online.

3GPPRAN2#112	2 – 13 November 2020   	online