3GPP TSG RAN WG2 Meeting #110-e	draft R2-2005817

Electronic meeting, June 01 - 12, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1665 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Release-16 UE capabilities based on RAN1, RAN4 feature lists and RAN2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation, NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav, NR_IAB-Core, NR_eMIMO-Core, NR_IIOT-Core, NR_2step_RACH-Core, 5G_V2X_NRSL-Core, NR_Mob_enh-Core, NR_pos-Core, NR_unlic-Core, LTE_NR_DC_CA_enh-Core, NR_SON_MDT-Core, NR_CLI_RIM, NG_RAN_PRN-Core, TEI16, NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2020-06-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Capture the Release-16 UE capabilities based on the RAN1 UE feature list (R1-2005097) and RAN4 UE feature list (R4-2009174) and the below endorsed CRs for other capabilities. All the entries that are not concluded from both RAN1 and RAN4 feature lists are not considered as part of this CR. List of changes - R1-2005097 RAN1 UE feature list - R4-2009174 RAN4 UE feature list - R2-2005857 UE capabilities for Rel-16 Power Saving WI - R2-2006000 UE capabilities for Rel-16 CLI (RAN1/RAN4) - R2-2005786 UE capabilities for RAN1 feature list NR_Mob_enh - R2-2005762 UE capabilities for RAN2 feature list NR_Mob_enh - R2-2005885 UE capabilities for RAN1 feature list NR_pos_Core - R2-2006296 UE capabilities for RAN2 feature list NR_IAB_Core - R2-2006409 UE capabilities for RAN2 feature list NR_IAB_Core - R2-2005807 UE capabilities for RAN2 feature list NG_RAN_PRN - R2-2006293 UE capabilities for RAN2 feature list NR_IIOT_Core - R2-2006360 Intraband EN_DC power class expansion for 29 dBm - R2-2006263 Introduction of UE capabilities for inter-frequency measurement without gap - R2-2006290 Introduction of mandatory gap patterns in Rel-16 - R2-2004583 UE capability for single entry PHR with P bit - R2-2006327 Introduction of signalling for high-speed train scenarios - R2-2006110 Introduction of NeedForGap capability for NR measurement - R2-2006366 Introduction of RAN2 UE capabilities for eDCCA - R2-2006053 Introduction of NR eURLLC capabilities - R2-2005973 Introduction of NR RAN2 V2X capabilities - R2-2005953 Introduction of NR RAN1/RAN4 V2X capabilities - R2-2005870 RAN2 UE capabilities for NR Shared Spectrum - R2-2006340 UE capabilities for NR MDT and SON - R2-2005900 Introduction of capability for on-demand SIB(s) procedure in CONNECTED - R2-2006203 Extension of CSI-RS capabilities per codebook type |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Release-16 capabilities from RAN1/RAN2/RAN4 are added based on the above list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN1, RAN2 and RAN4 related UE capabilities will not be captured in specifcations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3, 6.3.3, 6.4, 6.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS/TR .38.306, CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.8.3	Sidelink UE information for NR sidelink communication

5.8.3.1	General

Figure 5.8.3.1-1: Sidelink UE information for NR sidelink communication

The purpose of this procedure is to inform the network that the UE is interested or no longer interested to receive NR sidelink communication, as well as to request assignment or release of transmission resource for NR sidelink communication and to report parameters related to NR sidelink communication.

5.8.3.2	Initiation

A UE capable of NR sidelink communication that is in RRC_CONNECTED may initiate the procedure to indicate it is (interested in) receiving NR sidelink communication in several cases including upon successful connection establishment or resuming, upon change of interest, or upon change to a PCell providing SIB12 including sl-ConfigCommonNR. A UE capable of NR sidelink communication may initiate the procedure to request assignment of dedicated resources for NR sidelink communication transmission.

Upon initiating this procedure, the UE shall:

1>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

2>	ensure having a valid version of SIB12 for the PCell;

2>	if configured by upper layers to receive NR sidelink communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-RxInterestedFreqList; or if the frequency configured by upper layers to receive NR sidelink communication on has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink communication reception frequency of interest in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-RxInterestedFreqList:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it is no longer interested in NR sidelink communication reception in accordance with 5.8.3.3;

2>	if configured by upper layers to transmit NR sidelink communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell:

3>	if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or

3>	if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or

3>	if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqList; or if the information carried by the sl-TxResourceReqList has changed since the last transmission of the SidelinkUEInformationNR message:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink communication transmission resources required by the UE in accordance with 5.8.3.3;

2>	else:

3>	if the last transmission of the SidelinkUEInformationNR message included sl-TxResourceReqList:

4>	initiate transmission of the SidelinkUEInformationNR message to indicate it no longer requires NR sidelink communication transmission resources in accordance with 5.8.3.3.

5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

The UE shall set the contents of the SidelinkUEInformationNR message as follows:

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink communication or to request (configuration/ release) of NR sidelink communication transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure):

2>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

3>	if configured by upper layers to receive NR sidelink communication:

4>	include sl-RxInterestedFreqList and set it to the frequency for NR sidelink communication reception;

3>	if configured by upper layers to transmit NR sidelink communication:

4>	include sl-TxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink communication resource:

5>	set sl-DestinationIdentiy to the destination identity configured by upper layer for NR sidelink communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-RLC-ModeIndication to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB has been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-Failure as rlf for the associated destination for the NR sidelink communication transmission, if the sidelink RLF is detected;

5>	set sl-Failure as configFailure for the associated destination for the NR sidelink communication transmission, if RRCReconfigurationFailureSidelink is received as sidelink RRC reconfiguration failure;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-InterestedFreqList to indicate the frequency for NR sidelink communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-InterestedFreqList for NR sidelink communication transmission.

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE.

1>	The UE shall submit the SidelinkUEInformationNR message to lower layers for transmission.

Next Change

5.8.9.2	Sidelink UE capability transfer

5.8.9.2.1	General

This clause describes how the UE compiles and transfers its sidelink UE capability information for unicast to the initiating UE.

Figure 5.8.9.2.1-1: Sidelink UE capability transfer

5.8.9.2.2	Initiation

The UE may initiate the sidelink UE capability transfer procedure upon indication from upper layer when it needs (additional) UE radio access capability information.

5.8.9.2.3	Actions related to transmission of the UECapabilityEnquirySidelink by the UE

The initiating UE shall set the contents of UECapabilityEnquirySidelink message as follows:

1>	include in UE radio access capabilities for sidelink within ueCapabilityInformationSidelink;

NOTE:	It is up to initiating UE to decide whether ueCapabilityInformationSidelink should be included.

1>	set frequencyBandListFilterSidelink to include frequency bands for which the peer UE is requested to provide supported bands and band combinations;

1>	submit the UECapabilityEnquirySidelink message to lower layers for transmission.

5.8.9.2.4	Actions related to reception of the UECapabilityEnquirySidelink by the UE

The peer UE shall set the contents of UECapabilityInformationSidelink message as follows:

1>	include in UE radio access capabilities for sidelink within ueCapabilityInformationSidelink;

1>	compile a list of "candidate band combinations" only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilterSidelink (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on).

1>	Include into supportedBandCombinationListSidelink as many band combinations as possible from the list of "candidate band combinations", starting from the first entry;

1>	submit the UECapabilityInformationSidelink message to lower layers for transmission.

Next Change

#### –	SidelinkUEInformationNR

The SidelinkUEinformationNR message is used for the indication of NR sidelink UE information to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SidelinkUEInformationNR message

-- ASN1START

-- TAG-SIDELINKUEINFORMATIONNR-START

SidelinkUEInformationNR-r16::=         SEQUENCE {

criticalExtensions                  CHOICE {

sidelinkUEInformationNR-r16         SidelinkUEInformationNR-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SidelinkUEInformationNR-r16-IEs ::=     SEQUENCE {

sl-RxInterestedFreqList-r16            SL-InterestedFreqList-r16           OPTIONAL,

sl-TxResourceReqList-r16               SL-TxResourceReqList-r16            OPTIONAL,

lateNonCriticalExtension               OCTET STRING                        OPTIONAL,

nonCriticalExtension                   SEQUENCE {}                         OPTIONAL

}

SL-InterestedFreqList-r16 ::=          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

SL-TxResourceReqList-r16 ::=           SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-r16

SL-TxResourceReq-r16 ::=                SEQUENCE {

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16,

sl-CastType-r16                        ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-RLC-ModeIndicationList-r16          SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-RLC-ModeIndication-r16         OPTIONAL,

sl-QoS-InfoList-r16                    SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16          OPTIONAL,

sl-Failure-r16                         ENUMERATED {rlf, configFailure, spare2, spare1}                            OPTIONAL,

sl-TypeTxSyncList-r16                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16                OPTIONAL,

sl-TxInterestedFreqList-r16            SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)   OPTIONAL,

sl-CapabilityInformationSidelink-r16   OCTET STRING                                                               OPTIONAL

}

SL-QoS-Info-r16 ::=                    SEQUENCE {

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16,

sl-QoS-Profile-r16                    SL-QoS-Profile-r16                                                          OPTIONAL

}

SL-RLC-ModeIndication-r16 ::=          SEQUENCE {

sl-AM-Mode-r16                     SEQUENCE {

sl-AM-Mode-r16                     ENUMERATED {true},

sl-AM-QoS-InfoList-r16             SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16

}                                                                                                                 OPTIONAL,

sl-UM-Mode-r16                     SEQUENCE {

sl-UM-Mode-r16                     ENUMERATED {true},

sl-UM-QoS-InfoList-r16             SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16

}                                                                                                                 OPTIONAL

}

-- TAG-SIDELINKUEINFORMATIONNR-STOP

-- ASN1STOP

SidelinkUEinformationNR field descriptions

sl-RxInterestedFreqList Indicates the index of frequency on which the UE is interested to receive NR sidelink communication. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list.

sl-TxResourceReq Paramters to request the transmisison resouces for NR sidelink communication to the network in the Sidelink UE Information report.

SL-TxResourceReq field descriptions

sl-CapabilityInformationSidelink Includes the UECapabilityInformationSidelink message (which can be also included in ueCapabilityInformationSidelink-r16 in UECapabilityEnquirySidelink from peer UE) received from the peer UE.

sl-CastType Indicates the cast type for the correponding destination for which to request the resource.

sl-DestinationIdentity Indicates the destination for which the TX resource request and allocation from the network are concerned.

sl-Failure Indicates the sidelink RLF (value rlf) for the associated destination, when the sidelink RLF is detected. Indicates the sidelink AS configuration failure (value configFailure) for the associated destination, in case PC5-RRC AS configuration failure by receiving RRCReconfigurationFailureSidelink.

sl-QoS-InfoList Includes the QoS profile of the sidelink QoS flow as specified in TS 23.287 [55]

sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, which is unique for different destination and cast type.

sl-RLC-ModeIndication This field indicates the RLC mode and optionally the related QoS profiles for the sidelink radio bearer, which has not been configured by the network and is initiated by another UE in unicast. The RLC mode for one sidelink radio bearer is aligned between UE and NW by the sl-QoS-FlowIdentity.

sl-TxInterestedFreqList Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this relase, only one entry can be included in the list.

sl-TypeTxSyncList A list of synchronization reference used by the UE. The UE shall include the same number of entries, listed in the same order, as in sl-TxInterestedFreqList, i.e. one for each carrier freqeuncy included in sl-TxInterestedFreqList.

Next change

6.3.3	UE capability information elements

–	AccessStratumRelease

The IE AccessStratumRelease indicates the release supported by the UE.

AccessStratumRelease information element

-- ASN1START

-- TAG-ACCESSSTRATUMRELEASE-START

AccessStratumRelease ::= ENUMERATED {

rel15, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... }

-- TAG-ACCESSSTRATUMRELEASE-STOP

-- ASN1STOP

–	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=             SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombinationList-v1540 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540

BandCombinationList-v1550 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1550

BandCombinationList-v1560 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1560

BandCombinationList-v1570 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1570

BandCombinationList-v1580 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1580

BandCombinationList-v1590 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1590

BandCombinationList-v16xy ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v16xy

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

BandCombination-v1540::=            SEQUENCE {

bandList-v1540                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540,

ca-ParametersNR-v1540               CA-ParametersNR-v1540                       OPTIONAL

}

BandCombination-v1550 ::=           SEQUENCE {

ca-ParametersNR-v1550               CA-ParametersNR-v1550

}

BandCombination-v1560::=            SEQUENCE {

ne-DC-BC                                ENUMERATED {supported}                 OPTIONAL,

ca-ParametersNRDC                       CA-ParametersNRDC                      OPTIONAL,

ca-ParametersEUTRA-v1560                CA-ParametersEUTRA-v1560               OPTIONAL,

ca-ParametersNR-v1560                   CA-ParametersNR-v1560                  OPTIONAL

}

BandCombination-v1570 ::=           SEQUENCE {

ca-ParametersEUTRA-v1570            CA-ParametersEUTRA-v1570

}

BandCombination-v1580 ::=           SEQUENCE {

mrdc-Parameters-v1580               MRDC-Parameters-v1580

}

BandCombination-v1590::=            SEQUENCE {

supportedBandwidthCombinationSetIntraENDC  BIT STRING (SIZE (1..32))       OPTIONAL,

mrdc-Parameters-v1590                      MRDC-Parameters-v1590

}

BandCombination-v16xy ::=          SEQUENCE {

bandList-v16xy                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v16xy 	OPTIONAL,

ca-ParametersNR-v16xy               CA-ParametersNR-v16xy                  OPTIONAL,

ca-ParametersNRDC-v16xy				CA-ParametersNRDC-v16xy					OPTIONAL,

powerClass-v16xy					ENUMERATED {pc1dot5}					OPTIONAL

}

BandParameters ::=                      CHOICE {

eutra                               SEQUENCE {

bandEUTRA                           FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL,

ca-BandwidthClassUL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL

},

nr                                  SEQUENCE {

bandNR                              FreqBandIndicatorNR,

ca-BandwidthClassDL-NR              CA-BandwidthClassNR                    OPTIONAL,

ca-BandwidthClassUL-NR              CA-BandwidthClassNR                    OPTIONAL

}

}

BandParameters-v1540 ::=            SEQUENCE {

srs-CarrierSwitch                   CHOICE {

nr                                  SEQUENCE {

srs-SwitchingTimesListNR            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR

},

eutra                               SEQUENCE {

srs-SwitchingTimesListEUTRA         SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA

}

}                                                                              OPTIONAL,

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v16xy ::=         SEQUENCE {

srs-TxSwitch-v16xy                      SEQUENCE {

supportedSRS-TxPortSwitch-v16xy			ENUMERATED {t1r1-t1r2, t1r1-t1r2-t1r4, t1r1-t1r2-t2r2-t2r4, t1r1-t1r2-t2r2-t1r4-t2r4,

t1r1-t2r2, t1r1-t2r2-t4r4}

}                                                                              OPTIONAL,

intraFreqDAPS-Parameters-r16      SEQUENCE {

intraFreqDiffSCS-DAPS-r16                        ENUMERATED {supported}                 OPTIONAL,

intraFreqDAPS-r16                                ENUMERATED {supported}                 OPTIONAL,

intraFreqAsyncDAPS-r16                           ENUMERATED {supported}                 OPTIONAL,

intraFreqMultiUL-TransmissionDAPS-r16            ENUMERATED {supported}                 OPTIONAL,

intraFreqTwoTAGs-DAPS-r16                        ENUMERATED {supported}                 OPTIONAL,

intraFreqSemiStaticPowerSharingDAPS-Mode1-r16    ENUMERATED {supported}                 OPTIONAL,

intraFreqSemiStaticPowerSharingDAPS-Mode2-r16    ENUMERATED {supported}                 OPTIONAL,

intraFreqDynamicPowersharingDAPS-r16             ENUMERATED {short, long}               OPTIONAL

}

}

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

BandCombination field descriptions

BandCombinationList-v1540, BandCombinationList-v1550, BandCombinationList-v1560, BandCombinationList-v1570, BandCombinationList-v1580, BandCombinationList-v1590, BandCombinationList-r16 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix).

ca-ParametersNRDC If the field is included for a band combination in the NR capability container, the field indicates support of NR-DC. Otherwise, the field is absent.

ne-DC-BC If the field is included for a band combination in the MR-DC capability container, the field indicates support of NE-DC. Otherwise, the field is absent.

srs-SwitchingTimesListNR Indicates, for a particular pair of NR bands, the RF retuning time when switching between a NR carrier corresponding to this band entry and another (PUSCH-less) NR carrier corresponding to the band entry in the order indicated below: - For the first NR band, the UE shall include the same number of entries for NR bands as in bandList, i.e. first entry corresponds to first NR band in bandList and so on, - For the second NR band, the UE shall include one entry less, i.e. first entry corresponds to the second NR band in bandList and so on - And so on

srs-SwitchingTimesListEUTRA Indicates, for a particular pair of E-UTRA bands, the RF retuning time when switching between an E-UTRA carrier corresponding to this band entry and another (PUSCH-less) E-UTRA carrier corresponding to the band entry in the order indicated below: - For the first E-UTRA band, the UE shall include the same number of entries for E-UTRA bands as in bandList, i.e. first entry corresponds to first E-UTRA band in bandList and so on, - For the second E-UTRA band, the UE shall include one entry less, i.e. first entry corresponds to the second E-UTRA band in bandList and so on - And so on

–	CA-BandwidthClassEUTRA

The IE CA-BandwidthClassEUTRA indicates the E-UTRA CA bandwidth class as defined in TS 36.101 [22], table 5.6A-1.

CA-BandwidthClassEUTRA information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSEUTRA-START

CA-BandwidthClassEUTRA ::=          ENUMERATED {a, b, c, d, e, f, ...}

-- TAG-CA-BANDWIDTHCLASSEUTRA-STOP

-- ASN1STOP

–	CA-BandwidthClassNR

The IE CA-BandwidthClassNR indicates the NR CA bandwidth class as defined in TS 38.101-1 [15], table 5.3A.5-1 and TS 38.101-2 [39], table 5.3A.4-1.

CA-BandwidthClassNR information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSNR-START

CA-BandwidthClassNR ::=             ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...}

-- TAG-CA-BANDWIDTHCLASSNR-STOP

-- ASN1STOP

–	CA-ParametersEUTRA

The IE CA-ParametersEUTRA contains the E-UTRA part of band combination parameters for a given MR-DC band combination.

NOTE:	If additional E-UTRA band combination parameters are defined in TS 36.331 [10], which are supported for MR-DC, they will be defined here as well.

CA-ParametersEUTRA information element

-- ASN1START

-- TAG-CA-PARAMETERSEUTRA-START

CA-ParametersEUTRA ::=                          SEQUENCE {

multipleTimingAdvance                           ENUMERATED {supported}                          OPTIONAL,

simultaneousRx-Tx                               ENUMERATED {supported}                          OPTIONAL,

supportedNAICS-2CRS-AP                          BIT STRING (SIZE (1..8))                        OPTIONAL,

additionalRx-Tx-PerformanceReq                  ENUMERATED {supported}                          OPTIONAL,

ue-CA-PowerClass-N                              ENUMERATED {class2}                             OPTIONAL,

supportedBandwidthCombinationSetEUTRA-v1530     BIT STRING (SIZE (1..32))                       OPTIONAL,

...

}

CA-ParametersEUTRA-v1560 ::=                    SEQUENCE {

fd-MIMO-TotalWeightedLayers                             INTEGER (2..128)                                 OPTIONAL

}

CA-ParametersEUTRA-v1570 ::=                    SEQUENCE {

dl-1024QAM-TotalWeightedLayers                  INTEGER (0..10)                                 OPTIONAL

}

-- TAG-CA-PARAMETERSEUTRA-STOP

-- ASN1STOP

–	CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation related capabilities that are defined per band combination.

CA-ParametersNR information element

-- ASN1START

-- TAG-CA-PARAMETERSNR-START

CA-ParametersNR ::=                 SEQUENCE {

dummy                                         ENUMERATED {supported}      OPTIONAL,

parallelTxSRS-PUCCH-PUSCH                     ENUMERATED {supported}      OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH               ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

diffNumerologyAcrossPUCCH-Group               ENUMERATED {supported}      OPTIONAL,

diffNumerologyWithinPUCCH-GroupSmallerSCS     ENUMERATED {supported}      OPTIONAL,

supportedNumberTAG                            ENUMERATED {n2, n3, n4}     OPTIONAL,

...

}

CA-ParametersNR-v1540 ::=           SEQUENCE {

simultaneousSRS-AssocCSI-RS-AllCC                       INTEGER (5..32)         OPTIONAL,

csi-RS-IM-ReceptionForFeedbackPerBandComb               SEQUENCE {

maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC            INTEGER (1..64)         OPTIONAL,

totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC     INTEGER (2..256)        OPTIONAL

}                                                                               OPTIONAL,

simultaneousCSI-ReportsAllCC                            INTEGER (5..32)         OPTIONAL,

dualPA-Architecture                                     ENUMERATED {supported}  OPTIONAL

}

CA-ParametersNR-v1550 ::=           SEQUENCE {

dummy                               ENUMERATED {supported}      OPTIONAL

}

CA-ParametersNR-v1560 ::=           SEQUENCE {

diffNumerologyWithinPUCCH-GroupLargerSCS      ENUMERATED {supported}            OPTIONAL

}

CA-ParametersNR-v16xy ::=           SEQUENCE {

-- R1 9-3: Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA

parallelTxMsgA-SRS-PUCCH-PUSCH-r16                ENUMERATED {supported}            OPTIONAL,

-- R1 9-4: MsgA operation in a band combination including SUL

msgA-SUL-r16                                      ENUMERATED {supported}            OPTIONAL,

-- R1 10-9c: Joint search space group switching across multiple cells

jointSearchSpaceGroupSwitchingAcrossCells-r16		ENUMERATED {supported}				OPTIONAL,

-- R1 14-5: Half-duplex UE behaviour in TDD CA for same SCS

half-DuplexTDD-CA-SameSCS-r16					ENUMERATED {supported}				OPTIONAL,

-- R1 18-4	SCell dormancy within active time

scellDormancyWithinActiveTime-r16				ENUMERATED {supported}                  OPTIONAL,

-- R1 18-4a	SCell dormancy outside active time

scellDormancyOutsideActiveTime-r16		    	ENUMERATED {supported}                  OPTIONAL,

-- R1 18-6: Cross-carrier A-CSI RS triggering with different SCS

crossCarrierA-CSI-trigDiffSCS-r16				ENUMERATED {higherA-CSI-SCS,lowerA-CSI-SCS,both}		OPTIONAL,

-- R1 18-6a	Default QCL assumption for cross-carrier A-CSI-RS triggering

defaultQCL-CrossCarrierA-CSI-Trig-r16		    	ENUMERATED {supported}                  OPTIONAL,

-- R1 18-7: CA with non-aligned frame boundaries for inter-band CA

interCA-NonAlignedFrame-r16			ENUMERATED {supported}				OPTIONAL,

simul-SRS-Trans-InterBandCA-r16   INTEGER (1..2)                                  OPTIONAL,

daps-Parameters-r16                   SEQUENCE {

asyncDAPS-r16                           ENUMERATED {supported}                  OPTIONAL,

interFreqDAPS-r16                       ENUMERATED {supported}                  OPTIONAL,

interFreqDiffSCS-DAPS-r16               ENUMERATED {supported}                  OPTIONAL,

multiUL-TransmissionDAPS-r16            ENUMERATED {supported}                  OPTIONAL,

semiStaticPowerSharingDAPS-Mode1-r16    ENUMERATED {supported}                  OPTIONAL,

semiStaticPowerSharingDAPS-Mode2-r16    ENUMERATED {supported}                  OPTIONAL,

dynamicPowersharingDAPS-r16             ENUMERATED {short, long}                OPTIONAL,

ul-TransCancellationDAPS-r16            ENUMERATED {supported}                  OPTIONAL

}																				OPTIONAL,

codebookParametersPerBC-r16			CodebookParameters-v16xy					OPTIONAL

}

-- TAG-CA-PARAMETERSNR-STOP

-- ASN1STOP

–	CA-ParametersNRDC

The IE CA-ParametersNRDC contains dual connectivity related capabilities that are defined per band combination.

CA-ParametersNRDC information element

-- ASN1START

-- TAG-CA-PARAMETERS-NRDC-START

CA-ParametersNRDC ::=					SEQUENCE {

ca-ParametersNR-ForDC				CA-ParametersNR							OPTIONAL,

ca-ParametersNR-ForDC-v1540			CA-ParametersNR-v1540					OPTIONAL,

ca-ParametersNR-ForDC-v1550			CA-ParametersNR-v1550					OPTIONAL,

ca-ParametersNR-ForDC-v1560			CA-ParametersNR-v1560					OPTIONAL,

featureSetCombinationDC				FeatureSetCombinationId					OPTIONAL

}

CA-ParametersNRDC-v16xy ::=					SEQUENCE {

-- R1 18-1: Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode1-r16			ENUMERATED {supported}			OPTIONAL,

-- R1 18-1a: Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode2-r16			ENUMERATED {supported}			OPTIONAL,

-- R1 18-1b: Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-DynamicPwrSharing-r16		 	ENUMERATED {short, long}		OPTIONAL

}

-- TAG-CA-PARAMETERS-NRDC-STOP

-- ASN1STOP

CA-ParametersNRDC field descriptions

ca-ParametersNR-forDC (with and without suffix) If this field is present for a band combination, it reports the UE capabilities when NR-DC is configured with the band combination. If no version of this field (i.e., with and without suffix) is present for a band combination, the ca-ParametersNR field versions (with and without suffix) in BandCombination are applicable to the UE configured with NR-DC for the band combination.

featureSetCombinationDC If this field is present for a band combination, it reports the feature set combination supported for the band combination when NR-DC is configured. If this field is absent for a band combination, the featureSetCombination in BandCombination (without suffix) is applicable to the UE configured with NR-DC for the band combination.

–	CodebookParameters

The IE CodebookParameters is used to convey codebook related parameters.

CodebookParameters information element

-- ASN1START

-- TAG-CODEBOOKPARAMETERS-START

CodebookParameters ::=             SEQUENCE {

type1                                  SEQUENCE {

singlePanel                           SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode1andMode2},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

},

multiPanel                            SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode2, both},

nrofPanels                            ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

}                                                                                                               OPTIONAL

},

type2                                  SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                           INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction         ENUMERATED {supported}              OPTIONAL

}                                                                                                                   OPTIONAL,

type2-PortSelection                 SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                              INTEGER (2..4),

amplitudeScalingType                   ENUMERATED {wideband, widebandAndSubband}

}                                                                                                                   OPTIONAL

}

CodebookParameters-v16xy ::=		SEQUENCE {

supportedCSI-RS-ResourceListAlt-r16	SEQUENCE {

type1-SinglePanel-r16				SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)	OPTIONAL,

type1-MultiPanel-r16				SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)	OPTIONAL,

type2-r16							SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)	OPTIONAL,

type2-PortSelection-r16				SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)	OPTIONAL

}																															OPTIONAL

}

CodebookVariantsList-r16 ::=	SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-Resource

SupportedCSI-RS-Resource ::=     SEQUENCE {

maxNumberTxPortsPerResource      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand        INTEGER (1..64),

totalNumberTxPortsPerBand        INTEGER (2..256)

}

-- TAG-CODEBOOKPARAMETERS-STOP

-- ASN1STOP

CodebookParameters field descriptions

supportedCSI-RS-ResourceListAlt This field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type. The supported CSI-RS resource is indicated by an integer value which pinpoints SupportedCSI-RS-Resource defined in CodebookVariantsList. The value 0 corresponds to the first entry of CodebookVariantsList. The value 1 corresponds to the second entry of CodebookVariantsList, and so on. For each codebook type, the field shall be included in both codebookParametersPerBC and codebookParametersPerBand.

–	FeatureSetCombination

The IE FeatureSetCombination is a two-dimensional matrix of FeatureSet entries.

Each FeatureSetsPerBand contains a list of feature sets applicable to the carrier(s) of one band entry of the associated band combination. Across the associated bands, the UE shall support the combination of FeatureSets at the same position in the FeatureSetsPerBand. All FeatureSetsPerBand in one FeatureSetCombination must have the same number of entries.

The number of FeatureSetsPerBand in the FeatureSetCombination must be equal to the number of band entries in an associated band combination. The first FeatureSetPerBand applies to the first band entry of the band combination, and so on.

Each FeatureSet contains either a pair of NR or E-UTRA feature set IDs for UL and DL.

In case of NR, the actual feature sets for UL and DL are defined in the FeatureSets IE and referred to from here by their ID, i.e., their position in the featureSetsUplink / featureSetsDownlink list in the FeatureSet IE.

In case of E-UTRA, the feature sets referred to from this list are defined in TS 36.331 [10] and conveyed as part of the UE-EUTRA-Capability container.

The FeatureSetUplink and FeatureSetDownlink referred to from the FeatureSet comprise, among other information, a set of FeatureSetUplinkPerCC-Id:s and FeatureSetDownlinkPerCC-Id:s. The number of these per-CC IDs determines the number of carriers that the UE is able to aggregate contiguously in frequency domain in the corresponding band. The number of carriers supported by the UE is also restricted by the bandwidth class indicated in the associated BandCombination, if present.

In feature set combinations the UE shall exclude entries for fallback combinations with same capabilities, since the network may anyway assume that the UE supports those.

NOTE 1:	The UE may advertise fallback band-combinations in which it supports additional functionality explicitly in two ways: Either by setting FeatureSet IDs to zero (inter-band and intra-band non-contiguous fallback) and by reducing the number of FeatureSet-PerCC Ids in a Feature Set (intra-band contiguous fallback). Or by separate BandCombination entries with associated FeatureSetCombinations.

NOTE 2:	The UE may advertise a FeatureSetCombination containing only fallback band combinations. That means, in a FeatureSetCombination, each group of FeatureSets across the bands may contain at least one pair of FeatureSetUplinkId and FeatureSetDownlinkId which is set to 0/0.

NOTE 3:	The Network configures serving cell(s) and BWP(s) configuration to comply with capabilities derived from the combination of FeatureSets at the same position in the FeatureSetsPerBand, regardless of activated/deactivated serving cell(s) and BWP(s).

FeatureSetCombination information element

-- ASN1START

-- TAG-FEATURESETCOMBINATION-START

FeatureSetCombination ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::=          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=                  CHOICE {

eutra                           SEQUENCE {

downlinkSetEUTRA                FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA                  FeatureSetEUTRA-UplinkId

},

nr                              SEQUENCE {

downlinkSetNR                   FeatureSetDownlinkId,

uplinkSetNR                     FeatureSetUplinkId

}

}

-- TAG-FEATURESETCOMBINATION-STOP

-- ASN1STOP

–	FeatureSetCombinationId

The IE FeatureSetCombinationId identifies a FeatureSetCombination. The FeatureSetCombinationId of a FeatureSetCombination is the position of the FeatureSetCombination in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability). The FeatureSetCombinationId = 0 refers to the first entry in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability).

NOTE:	The FeatureSetCombinationId = 1024 is not used due to the maximum entry number of featureSetCombinations.

FeatureSetCombinationId information element

-- ASN1START

-- TAG-FEATURESETCOMBINATIONID-START

FeatureSetCombinationId ::=         INTEGER (0.. maxFeatureSetCombinations)

-- TAG-FEATURESETCOMBINATIONID-STOP

-- ASN1STOP

–	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=                  SEQUENCE {

featureSetListPerDownlinkCC             SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL               FreqSeparationClass                                                     OPTIONAL,

scalingFactor                           ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

crossCarrierScheduling-OtherSCS         ENUMERATED {supported}                                                  OPTIONAL,

scellWithoutSSB                         ENUMERATED {supported}                                                  OPTIONAL,

csi-RS-MeasSCellWithoutSSB              ENUMERATED {supported}                                                  OPTIONAL,

dummy1                                  ENUMERATED {supported}                                                  OPTIONAL,

type1-3-CSS                             ENUMERATED {supported}                                                  OPTIONAL,

pdcch-MonitoringAnyOccasions            ENUMERATED {withoutDCI-Gap, withDCI-Gap}                                OPTIONAL,

dummy2                                  ENUMERATED {supported}                                                  OPTIONAL,

ue-SpecificUL-DL-Assignment             ENUMERATED {supported}                                                  OPTIONAL,

searchSpaceSharingCA-DL                 ENUMERATED {supported}                                                  OPTIONAL,

timeDurationForQCL                      SEQUENCE {

scs-60kHz                           ENUMERATED {s7, s14, s28}                                               OPTIONAL,

scs-120kHz                          ENUMERATED {s14, s28}                                                   OPTIONAL

}                                                                                                           OPTIONAL,

pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-30kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-60kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-120kHz                              ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL

}                                                                                                           OPTIONAL,

dummy3                                  DummyA                                                                  OPTIONAL,

dummy4                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyB                        OPTIONAL,

dummy5                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyC                        OPTIONAL,

dummy6                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyD                        OPTIONAL,

dummy7                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyE                        OPTIONAL

}

FeatureSetDownlink-v1540 ::= SEQUENCE {

oneFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

additionalDMRS-DL-Alt                   ENUMERATED {supported}                       OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-DL       ENUMERATED {supported}                       OPTIONAL,

pdcch-MonitoringAnyOccasionsWithSpanGap SEQUENCE {

scs-15kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-30kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-60kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-120kHz                              ENUMERATED {set1, set2, set3}                OPTIONAL

}                                                                                    OPTIONAL,

pdsch-SeparationWithGap                 ENUMERATED {supported}                       OPTIONAL,

pdsch-ProcessingType2                   SEQUENCE {

scs-15kHz                               ProcessingParameters                         OPTIONAL,

scs-30kHz                               ProcessingParameters                         OPTIONAL,

scs-60kHz                               ProcessingParameters                         OPTIONAL

} OPTIONAL,

pdsch-ProcessingType2-Limited           SEQUENCE {

differentTB-PerSlot-SCS-30kHz           ENUMERATED {upto1, upto2, upto4, upto7}

} OPTIONAL,

dl-MCS-TableAlt-DynamicIndication       ENUMERATED {supported}                       OPTIONAL

}

FeatureSetDownlink-v16xy ::=           SEQUENCE {

-- R1 22-4e/4f/4g/4h: CBG based reception for DL with unicast PDSCH(s) per slot per CC with UE processing time Capability 1

cbgPDSCH-ProcessingType1-DifferentTB-PerSlot	SEQUENCE {

scs-15kHz			ENUMERATED {one-pdsch, upto2, upto4, upto7} OPTIONAL,

scs-30kHz			ENUMERATED {one-pdsch, upto2, upto4, upto7} OPTIONAL,

scs-60kHz			ENUMERATED {one-pdsch, upto2, upto4, upto7} OPTIONAL,

scs-120kHz			ENUMERATED {one-pdsch, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

-- R1 22-3e/3f/3g/3h: CBG based reception for DL with unicast PDSCH(s) per slot per CC with UE processing time Capability 2

cbgPDSCH-ProcessingType2-DifferentTB-PerSlot	SEQUENCE {

scs-15kHz			ENUMERATED {one-pdsch, upto2, upto4, upto7} OPTIONAL,

scs-30kHz			ENUMERATED {one-pdsch, upto2, upto4, upto7} OPTIONAL,

scs-60kHz			ENUMERATED {one-pdsch, upto2, upto4, upto7} OPTIONAL,

scs-120kHz			ENUMERATED {one-pdsch, upto2, upto4, upto7} OPTIONAL

} OPTIONAL

}

DummyA ::=      SEQUENCE {

maxNumberNZP-CSI-RS-PerCC                   INTEGER (1..32),

maxNumberPortsAcrossNZP-CSI-RS-PerCC        ENUMERATED {p2, p4, p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256},

maxNumberCS-IM-PerCC                        ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousCSI-RS-ActBWP-AllCC    ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26,

n28, n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52,

n54, n56, n58, n60, n62, n64},

totalNumberPortsSimultaneousCSI-RS-ActBWP-AllCC ENUMERATED {p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256}

}

DummyB ::=       SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode1AndMode2},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyC ::=        SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p8, p16, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode2, both},

supportedNumberPanels               ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyD ::=                 SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction          ENUMERATED {supported}                          OPTIONAL,

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyE ::=    SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

-- TAG-FEATURESETDOWNLINK-STOP

-- ASN1STOP

FeatureSetDownlink field descriptions

crossCarrierScheduling-OtherSCS The UE shall set this field to the same value as crossCarrierScheduling-OtherSCS in the associated FeatureSetUplink (if present).

featureSetListPerDownlinkCC Indicates which features the UE supports on the individual DL carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include at least as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list.

–	FeatureSetDownlinkId

The IE FeatureSetDownlinkId identifies a downlink feature set. The FeatureSetDownlinkId of a FeatureSetDownlink is the index position of the FeatureSetDownlink in the featureSetsDownlink list in the FeatureSets IE. The first element in that list is referred to by FeatureSetDownlinkId = 1. The FeatureSetDownlinkId=0 is not used by an actual FeatureSetDownlink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetDownlinkId information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKID-START

FeatureSetDownlinkId ::=            INTEGER (0..maxDownlinkFeatureSets)

-- TAG-FEATURESETDOWNLINKID-STOP

-- ASN1STOP

–	FeatureSetDownlinkPerCC

The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetDownlinkPerCC information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-START

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                                                  OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                                                           OPTIONAL,

supportedModulationOrderDL          ModulationOrder                                                         OPTIONAL

}

-- TAG-FEATURESETDOWNLINKPERCC-STOP

-- ASN1STOP

–	FeatureSetDownlinkPerCC-Id

The IE FeatureSetDownlinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetDownlinkPerCC-Id of a FeatureSetDownlinkPerCC is the index position of the FeatureSetDownlinkPerCC in the featureSetsDownlinkPerCC. The first element in the list is referred to by FeatureSetDownlinkPerCC-Id = 1, and so on.

FeatureSetDownlinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-ID-START

FeatureSetDownlinkPerCC-Id ::=      INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETDOWNLINKPERCC-ID-STOP

-- ASN1STOP

–	FeatureSetEUTRA-DownlinkId

The IE FeatureSetEUTRA-DownlinkId identifies a downlink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-DownlinkId = 1. The FeatureSetEUTRA-DownlinkId=0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-DownlinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRADOWNLINKID-START

FeatureSetEUTRA-DownlinkId ::=      INTEGER (0..maxEUTRA-DL-FeatureSets)

-- TAG-FEATURESETEUTRADOWNLINKID-STOP

-- ASN1STOP

–	FeatureSetEUTRA-UplinkId

The IE FeatureSetEUTRA-UplinkId identifies an uplink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-UplinkId = 1. The FeatureSetEUTRA-UplinkId =0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-UplinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRAUPLINKID-START

FeatureSetEUTRA-UplinkId ::=                    INTEGER (0..maxEUTRA-UL-FeatureSets)

-- TAG-FEATURESETEUTRAUPLINKID-STOP

-- ASN1STOP

–	FeatureSets

The IE FeatureSets is used to provide pools of downlink and uplink features sets. A FeatureSetCombination refers to the IDs of the feature set(s) that the UE supports in that FeatureSetCombination. The BandCombination entries in the BandCombinationList then indicate the ID of the FeatureSetCombination that the UE supports for that band combination.

The entries in the lists in this IE are identified by their index position. For example, the FeatureSetUplinkPerCC-Id = 4 identifies the 4th element in the featureSetsUplinkPerCC list.

NOTE:	When feature sets (per CC) IEs require extension in future versions of the specification, new versions of the FeatureSetDownlink, FeatureSetUplink, FeatureSets, FeatureSetDownlinkPerCC and/or FeatureSetUplinkPerCC will be created and instantiated in corresponding new lists in the FeatureSets IE. For example, if new capability bits are to be added to the FeatureSetDownlink, they will instead be defined in a new FeatureSetDownlink-rxy which will be instantiated in a new featureSetDownlinkList-rxy list. If a UE indicates in a FeatureSetCombination that it supports the FeatureSetDownlink with ID #5, it implies that it supports both the features in FeatureSetDownlink #5 and FeatureSetDownlink-rxy #5 (if present). The number of entries in the new list(s) shall be the same as in the original list(s).

FeatureSets information element

-- ASN1START

-- TAG-FEATURESETS-START

FeatureSets ::=    SEQUENCE {

featureSetsDownlink                 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink               OPTIONAL,

featureSetsDownlinkPerCC            SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC            OPTIONAL,

featureSetsUplink                   SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink                   OPTIONAL,

featureSetsUplinkPerCC              SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC              OPTIONAL,

...,

[[

featureSetsDownlink-v1540           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1540         OPTIONAL,

featureSetsUplink-v1540             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1540             OPTIONAL,

featureSetsUplinkPerCC-v1540        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1540        OPTIONAL

]],

[[

featureSetsDownlink-v16xy             	SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v16xy			OPTIONAL,

featureSetsUplink-v16xy             	SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v16xy	             OPTIONAL

]]

}

-- TAG-FEATURESETS-STOP

-- ASN1STOP

–	FeatureSetUplink

The IE FeatureSetUplink is used to indicate the features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetUplink information element

-- ASN1START

-- TAG-FEATURESETUPLINK-START

FeatureSetUplink ::=                SEQUENCE {

featureSetListPerUplinkCC           SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id,

scalingFactor                       ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

crossCarrierScheduling-OtherSCS     ENUMERATED {supported}                                                  OPTIONAL,

intraBandFreqSeparationUL           FreqSeparationClass                                                     OPTIONAL,

searchSpaceSharingCA-UL             ENUMERATED {supported}                                                  OPTIONAL,

dummy1                              DummyI                                                                  OPTIONAL,

supportedSRS-Resources              SRS-Resources                                                           OPTIONAL,

twoPUCCH-Group                      ENUMERATED {supported}                                                  OPTIONAL,

dynamicSwitchSUL                    ENUMERATED {supported}                                                  OPTIONAL,

simultaneousTxSUL-NonSUL            ENUMERATED {supported}                                                  OPTIONAL,

pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-30kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-60kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-120kHz                                ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL

}                                                                                                           OPTIONAL,

dummy2                               DummyF                                                                 OPTIONAL

}

FeatureSetUplink-v1540 ::=           SEQUENCE {

zeroSlotOffsetAperiodicSRS           ENUMERATED {supported}                     OPTIONAL,

pa-PhaseDiscontinuityImpacts         ENUMERATED {supported}                     OPTIONAL,

pusch-SeparationWithGap              ENUMERATED {supported}                     OPTIONAL,

pusch-ProcessingType2                SEQUENCE {

scs-15kHz                            ProcessingParameters                       OPTIONAL,

scs-30kHz                            ProcessingParameters                       OPTIONAL,

scs-60kHz                            ProcessingParameters                       OPTIONAL

}                                                                               OPTIONAL,

ul-MCS-TableAlt-DynamicIndication    ENUMERATED {supported}                     OPTIONAL

}

FeatureSetUplink-v16xy ::=           SEQUENCE {

-- R1 11-5: PUsCH repetition Type B

pusch-RepetitionTypeB-r16			SEQUENCE {

maxNumberPUSCH-Tx-r16				ENUMERATED {n2, n3, n4, n7, n8, n12},

hoppingScheme-r16					ENUMERATED {interSlotHopping, interRepetitionHopping, both}

}																					OPTIONAL,

-- R1 11-7: UL cancelation scheme for self-carrier

ul-CancellationSelfCarrier-r16			ENUMERATED {supported}						OPTIONAL,

-- R1 11-7a: UL cancelation scheme for cross-carrier

ul-CancellationCrossCarrier-r16			ENUMERATED {supported}						OPTIONAL,

-- R1 16-5c: The maximum number of SRS resources in one SRS resource set with usage set to ‘codebook’ for Mode 2

ul-FullPwrMode2-MaxSRS-ResInSet          ENUMERATED {n1, n2, n4}                    OPTIONAL,

-- R1 22-4a/4b/4c/4d: CBG based transmission for UL with unicast PUSCH(s) per slot per CC with UE processing time Capability 1

cbgPUSCH-ProcessingType1-DifferentTB-PerSlot	SEQUENCE {

scs-15kHz			ENUMERATED {one-pusch, upto2, upto4, upto7} OPTIONAL,

scs-30kHz			ENUMERATED {one-pusch, upto2, upto4, upto7} OPTIONAL,

scs-60kHz			ENUMERATED {one-pusch, upto2, upto4, upto7} OPTIONAL,

scs-120kHz			ENUMERATED {one-pusch, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

-- R1 22-3a/3b/3c/3d: CBG based transmission for UL with unicast PUSCH(s) per slot per CC with UE processing time Capability 2

cbgPUSCH-ProcessingType2-DifferentTB-PerSlot	SEQUENCE {

scs-15kHz			ENUMERATED {one-pusch, upto2, upto4, upto7} OPTIONAL,

scs-30kHz			ENUMERATED {one-pusch, upto2, upto4, upto7} OPTIONAL,

scs-60kHz			ENUMERATED {one-pusch, upto2, upto4, upto7} OPTIONAL,

scs-120kHz			ENUMERATED {one-pusch, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

supportedSRS-PosResources-r16              SRS-AllPosResources-r16                                          OPTIONAL

}

SRS-AllPosResources-r16 ::=                SEQUENCE {

srs-PosResources-r16                       SRS-PosResources-r16,

srs-PosResourceAP-r16                      SRS-PosResourceAP-r16                     OPTIONAL,

srs-PosResourceSP-r16                      SRS-PosResourceSP-r16                    OPTIONAL

}

SRS-PosResources-r16 ::=                       SEQUENCE {

maxNumberSRS-PosResourceSetPerBWP-r16                ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r16                  ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSRS-ResourcesPerBWP-PerSlot-r16             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maxNumberPeriodicSRS-PosResourcesPerBWP-r16          ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r16  ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-PosResourceAP-r16 ::=                SEQUENCE {

maxNumberAP-SRS-PosResourcesPerBWP-r16         ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberAP-SRS-PosResourcesPerBWP-PerSlot-r16 ENUMERATED (n1, n2, n3, n4, n5, n6, n8, n10, n12, n14)

}

SRS-PosResourceSP-r16 ::=                       SEQUENCE {

maxNumberSP-SRS-PosResourcesPerBWP-r16               ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSP-SRS-PosResourcesPerBWP-PerSlot-r16       ENUMERATED (n1, n2, n3, n4, n5, n6, n8, n10, n12, n14)

}

SRS-Resources ::=                           SEQUENCE {

maxNumberAperiodicSRS-PerBWP                ENUMERATED {n1, n2, n4, n8, n16},

maxNumberAperiodicSRS-PerBWP-PerSlot        INTEGER (1..6),

maxNumberPeriodicSRS-PerBWP                 ENUMERATED {n1, n2, n4, n8, n16},

maxNumberPeriodicSRS-PerBWP-PerSlot         INTEGER (1..6),

maxNumberSemiPersistentSRS-PerBWP           ENUMERATED {n1, n2, n4, n8, n16},

maxNumberSemiPersistentSRS-PerBWP-PerSlot   INTEGER (1..6),

maxNumberSRS-Ports-PerResource              ENUMERATED {n1, n2, n4}

}

DummyF ::=                                  SEQUENCE {

maxNumberPeriodicCSI-ReportPerBWP           INTEGER (1..4),

maxNumberAperiodicCSI-ReportPerBWP          INTEGER (1..4),

maxNumberSemiPersistentCSI-ReportPerBWP     INTEGER (0..4),

simultaneousCSI-ReportsAllCC                INTEGER (5..32)

}

-- TAG-FEATURESETUPLINK-STOP

-- ASN1STOP

FeatureSetUplink field descriptions

crossCarrierScheduling-OtherSCS The UE shall set this field to the same value as crossCarrierScheduling-OtherSCS in the associated FeatureSetDownlink (if present).

featureSetListPerUplinkCC Indicates which features the UE supports on the individual UL carriers of the feature set (and hence of a band entry that refers to the feature set). The UE shall hence include at least as many FeatureSetUplinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassUL, except if indicating additional functionality by reducing the number of FeatureSetUplinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUplinkPerCC-Id in this list.

–	FeatureSetUplinkId

The IE FeatureSetUplinkId identifies an uplink feature set. The FeatureSetUplinkId of a FeatureSetUplink is the index position of the FeatureSetUplink in the featureSetsUplink list in the FeatureSets IE. The first element in the list is referred to by FeatureSetUplinkId = 1, and so on. The FeatureSetUplinkId =0 is not used by an actual FeatureSetUplink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetUplinkId information element

-- ASN1START

-- TAG-FEATURESETUPLINKID-START

FeatureSetUplinkId ::=                  INTEGER (0..maxUplinkFeatureSets)

-- TAG-FEATURESETUPLINKID-STOP

-- ASN1STOP

–	FeatureSetUplinkPerCC

The IE FeatureSetUplinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetUplinkPerCC information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-START

FeatureSetUplinkPerCC ::=               SEQUENCE {

supportedSubcarrierSpacingUL            SubcarrierSpacing,

supportedBandwidthUL                    SupportedBandwidth,

channelBW-90mhz                         ENUMERATED {supported}                      OPTIONAL,

mimo-CB-PUSCH                           SEQUENCE {

maxNumberMIMO-LayersCB-PUSCH            MIMO-LayersUL                               OPTIONAL,

maxNumberSRS-ResourcePerSet             INTEGER (1..2)

}                                                                                   OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH         MIMO-LayersUL                               OPTIONAL,

supportedModulationOrderUL              ModulationOrder                             OPTIONAL

}

FeatureSetUplinkPerCC-v1540 ::=       SEQUENCE {

mimo-NonCB-PUSCH                      SEQUENCE {

maxNumberSRS-ResourcePerSet           INTEGER (1..4),

maxNumberSimultaneousSRS-ResourceTx   INTEGER (1..4)

} OPTIONAL

}

-- TAG-FEATURESETUPLINKPERCC-STOP

-- ASN1STOP

–	FeatureSetUplinkPerCC-Id

The IE FeatureSetUplinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetUplinkPerCC-Id of a FeatureSetUplinkPerCC is the index position of the FeatureSetUplinkPerCC in the featureSetsUplinkPerCC. The first element in the list is referred to by FeatureSetUplinkPerCC-Id = 1, and so on.

FeatureSetUplinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-ID-START

FeatureSetUplinkPerCC-Id ::=            INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETUPLINKPERCC-ID-STOP

-- ASN1STOP

–	FreqBandIndicatorEUTRA

-- ASN1START

-- TAG-FREQBANDINDICATOREUTRA-START

FreqBandIndicatorEUTRA ::=  INTEGER (1..maxBandsEUTRA)

-- TAG-FREQBANDINDICATOREUTRA-STOP

-- ASN1STOP

–	FreqBandList

The IE FreqBandList is used by the network to request NR CA and/or MR-DC band combinations for specific NR and/or E-UTRA frequency bands and/or up to a specific number of carriers and/or up to specific aggregated bandwidth. This is also used to request feature sets (for NR) and feature set combinations (for NR and MR-DC).

FreqBandList information element

-- ASN1START

-- TAG-FREQBANDLIST-START

FreqBandList ::=                SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation

FreqBandInformation ::=         CHOICE {

bandInformationEUTRA            FreqBandInformationEUTRA,

bandInformationNR               FreqBandInformationNR

}

FreqBandInformationEUTRA ::=    SEQUENCE {

bandEUTRA                       FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL,   -- Need N

ca-BandwidthClassUL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL    -- Need N

}

FreqBandInformationNR ::=       SEQUENCE {

bandNR                          FreqBandIndicatorNR,

maxBandwidthRequestedDL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxBandwidthRequestedUL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxCarriersRequestedDL          INTEGER (1..maxNrofServingCells)        OPTIONAL,   -- Need N

maxCarriersRequestedUL          INTEGER (1..maxNrofServingCells)        OPTIONAL    -- Need N

}

AggregatedBandwidth ::=         ENUMERATED {mhz50, mhz100, mhz150, mhz200, mhz250, mhz300, mhz350,

mhz400, mhz450, mhz500, mhz550, mhz600, mhz650, mhz700, mhz750, mhz800}

-- TAG-FREQBANDLIST-STOP

-- ASN1STOP

–	FreqSeparationClass

The IE FreqSeparationClass is used for an intra-band non-contiguous CA band combination to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in a frequency band.

FreqSeparationClass information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASS-START

FreqSeparationClass ::= ENUMERATED {c1, c2, c3, ...}

-- TAG-FREQSEPARATIONCLASS-STOP

-- ASN1STOP

#### –	HighSpeedParameters

The IE HighSpeedParameters is used to convey capabilities related to high speed scenarios.

HighSpeedParameters information element

-- ASN1START

-- TAG-HIGHSPEEDPARAMETERS-START

HighSpeedParameters-r16 ::= SEQUENCE {

measurementEnhancement-r16		ENUMERATED {supported}	OPTIONAL,

demodulationEnhancement-r16		ENUMERATED {supported}	OPTIONAL

}

-- TAG-HIGHSPEEDPARAMETERS-STOP

-- ASN1STOP

–	IMS-Parameters

The IE IMS-Parameters is used to convery capabilities related to IMS.

IMS-Parameters information element

-- ASN1START

-- TAG-IMS-PARAMETERS-START

IMS-Parameters ::=         SEQUENCE {

ims-ParametersCommon       IMS-ParametersCommon                  OPTIONAL,

ims-ParametersFRX-Diff     IMS-ParametersFRX-Diff                OPTIONAL,

...

}

IMS-ParametersCommon ::=   SEQUENCE {

voiceOverEUTRA-5GC         ENUMERATED {supported}                OPTIONAL,

...,

[[

voiceOverSCG-BearerEUTRA-5GC       ENUMERATED {supported}        OPTIONAL

]],

[[

voiceFallbackIndicationEPS-r16         ENUMERATED {supported}           OPTIONAL

]]

}

IMS-ParametersFRX-Diff ::= SEQUENCE {

voiceOverNR                ENUMERATED {supported}                OPTIONAL,

...

}

-- TAG-IMS-PARAMETERS-STOP

-- ASN1STOP

–	InterRAT-Parameters

The IE InterRAT-Parameters is used convey UE capabilities related to the other RATs.

InterRAT-Parameters information element

-- ASN1START

-- TAG-INTERRAT-PARAMETERS-START

InterRAT-Parameters ::=             SEQUENCE {

eutra                               EUTRA-Parameters                OPTIONAL,

...,

[[

utra-FDD-r16                        UTRA-FDD-Parameters-r16         OPTIONAL

]]

}

EUTRA-Parameters ::=                SEQUENCE {

supportedBandListEUTRA          SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA,

eutra-ParametersCommon              EUTRA-ParametersCommon                                      OPTIONAL,

eutra-ParametersXDD-Diff            EUTRA-ParametersXDD-Diff                                    OPTIONAL,

...

}

EUTRA-ParametersCommon ::=      SEQUENCE {

mfbi-EUTRA                          ENUMERATED {supported}          OPTIONAL,

modifiedMPR-BehaviorEUTRA           BIT STRING (SIZE (32))          OPTIONAL,

multiNS-Pmax-EUTRA                  ENUMERATED {supported}          OPTIONAL,

rs-SINR-MeasEUTRA                   ENUMERATED {supported}          OPTIONAL,

...,

[[

ne-DC                               ENUMERATED {supported}          OPTIONAL

]],

[[

nr-HO-ToEN-DC-r16                   ENUMERATED {supported}          OPTIONAL

]]

}

EUTRA-ParametersXDD-Diff ::=        SEQUENCE {

rsrqMeasWidebandEUTRA               ENUMERATED {supported}          OPTIONAL,

...

}

UTRA-FDD-Parameters-r16 ::=                SEQUENCE {

supportedBandListUTRA-FDD-r16              SEQUENCE (SIZE (1..maxBandsUTRA-FDD-r16)) OF SupportedBandUTRA-FDD-r16,

...

}

SupportedBandUTRA-FDD-r16 ::=           ENUMERATED {

bandI, bandII, bandIII, bandIV, bandV, bandVI,

bandVII, bandVIII, bandIX, bandX, bandXI,

bandXII, bandXIII, bandXIV, bandXV, bandXVI,

bandXVII, bandXVIII, bandXIX, bandXX,

bandXXI, bandXXII, bandXXIII, bandXXIV,

bandXXV, bandXXVI, bandXXVII, bandXXVIII,

bandXXIX, bandXXX, bandXXXI, bandXXXII}

-- TAG-INTERRAT-PARAMETERS-STOP

-- ASN1STOP

–	MAC-Parameters

The IE MAC-Parameters is used to convey capabilities related to MAC.

MAC-Parameters information element

-- ASN1START

-- TAG-MAC-PARAMETERS-START

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon            MAC-ParametersCommon        OPTIONAL,

mac-ParametersXDD-Diff          MAC-ParametersXDD-Diff      OPTIONAL

}

MAC-Parameters-v16xy ::- SEQUENCE {

mac-ParametersFRX-Diff-r16      MAC-ParametersFRX-Diff-r16  OPTIONAL

}

MAC-ParametersCommon ::=    SEQUENCE {

lcp-Restriction                 ENUMERATED {supported}      OPTIONAL,

dummy                           ENUMERATED {supported}      OPTIONAL,

lch-ToSCellRestriction          ENUMERATED {supported}      OPTIONAL,

...,

[[

recommendedBitRate              ENUMERATED {supported}      OPTIONAL,

recommendedBitRateQuery         ENUMERATED {supported}      OPTIONAL

]],

[[

recommendedBitRateMultiplier-r16 ENUMERATED {supported}     OPTIONAL,

lcid-ExtensionIAB-r16			ENUMERATED {supported}     OPTIONAL,

preEmptiveBSR-r16				ENUMERATED {supported}     OPTIONAL,

autonomousTransmission-r16				ENUMERATED {supported}     OPTIONAL,

lch-PriorityBasedPrioritization-r16		ENUMERATED {supported}     OPTIONAL,

lch-ToConfiguredGrantMapping-r16		ENUMERATED {supported}     OPTIONAL,

lch-ToGrantPriorityRestriction-r16		ENUMERATED {supported}     OPTIONAL,

singlePHR-P-r16                  ENUMERATED {supported}     OPTIONAL,

ul-LBT-FailureDetectionRecovery-r16 ENUMERATED {supported}  OPTIONAL

]]

}

MAC-ParametersFRX-Diff-r16 ::=  SEQUENCE {

directMCG-SCellActivation-r16       ENUMERATED {supported}      OPTIONAL,

directMCG-SCellActivationResume-r16 ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivation-r16       ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivationResume-r16 ENUMERATED {supported}      OPTIONAL,

-- R1 19-1: DRX Adaptation

drx-Adaptation-r16			SEQUENCE {

licensedBand-r16				MinTimeGap-r16					OPTIONAL,

unlicensedBand-r16				MinTimeGap-r16					OPTIONAL

}																OPTIONAL,

...

}

MAC-ParametersXDD-Diff ::=  SEQUENCE {

skipUplinkTxDynamic             ENUMERATED {supported}     OPTIONAL,

logicalChannelSR-DelayTimer     ENUMERATED {supported}     OPTIONAL,

longDRX-Cycle                   ENUMERATED {supported}     OPTIONAL,

shortDRX-Cycle                  ENUMERATED {supported}     OPTIONAL,

multipleSR-Configurations       ENUMERATED {supported}     OPTIONAL,

multipleConfiguredGrants    ENUMERATED {supported}         OPTIONAL,

...

}

MinTimeGap-r16 ::=			SEQUENCE {

scs-15kHz-r16				ENUMERATED {sl1, sl3}					OPTIONAL,

scs-30kHz-r16				ENUMERATED {sl1, sl6}					OPTIONAL,

scs-60kHz-r16				ENUMERATED {sl1, sl12}					OPTIONAL,

scs-120kHz-r16				ENUMERATED {sl2, sl24}					OPTIONAL

}

-- TAG-MAC-PARAMETERS-STOP

-- ASN1STOP

–	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]] ,

[[

condHandoverParametersCommon-r16                SEQUENCE {

condHandoverFDD-TDD-r16                          ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                          ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16				ENUMERATED {supported}					OPTIONAL,

supportedGapPattern-NRonly-r16			BIT STRING (SIZE (10))					OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16		ENUMERATED {supported}					OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16							ENUMERATED {supported}                  OPTIONAL,

multipleNS-And-Pmax-IAB-r16				ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=            SEQUENCE {

intraAndInterF-MeasAndReport        ENUMERATED {supported}                      OPTIONAL,

eventA-MeasAndReport                ENUMERATED {supported}                      OPTIONAL,

...,

[[

handoverInterF                      ENUMERATED {supported}                      OPTIONAL,

handoverLTE-EPC                     ENUMERATED {supported}                      OPTIONAL,

handoverLTE-5GC                     ENUMERATED {supported}                      OPTIONAL

]],

[[

sftd-MeasNR-Neigh                   ENUMERATED {supported}                      OPTIONAL,

sftd-MeasNR-Neigh-DRX               ENUMERATED {supported}                      OPTIONAL

]],

[[

condHandoverParametersXDD-Diff-r16          SEQUENCE {

condHandover-r16                             ENUMERATED {supported}                   OPTIONAL,

condHandoverFailure-r16                     ENUMERATED {supported}                   OPTIONAL,

condHandoverTwoTriggerEvents-r16            ENUMERATED {supported}                   OPTIONAL

}                                                                                OPTIONAL,

pcellT312-r16                       ENUMERATED {supported}                      OPTIONAL,

handoverIntraF-IAB-r16				ENUMERATED {supported}              OPTIONAL,

eutra-AutonomousGaps-r16            ENUMERATED {supported}                      OPTIONAL,

eutra-AutonomousGapsNEDC-r16		ENUMERATED {supported}						OPTIONAL,

eutra-AutonomousGapsNRDC-r16		ENUMERATED {supported}						OPTIONAL,

nr-AutonomousGaps-r16               ENUMERATED {supported}                      OPTIONAL,

nr-AutonomousGapsENDC-r16          ENUMERATED {supported}                      OPTIONAL,

nr-AutonomousGapsNEDC-r16			ENUMERATED {supported}						OPTIONAL,

nr-AutonomousGapsNRDC-r16			ENUMERATED {supported}						OPTIONAL,

handoverUTRA-FDD-r16                ENUMERATED {supported}                      OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB             ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                               ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                  ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                              ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                             ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                             ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology        ENUMERATED {supported}              OPTIONAL

]],

[[

nr-AutonomousGaps-r16                       ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-ENDC-r16                  ENUMERATED {supported}              OPTIONAL,

handoverUTRA-FDD-r16                        ENUMERATED {supported}              OPTIONAL,

cli-RSSI-Meas-r16                           ENUMERATED {supported}              OPTIONAL,

cli-SRS-RSRP-Meas-r16                            ENUMERATED {supported}              OPTIONAL,

condHandoverParametersFRX-Diff-r16          SEQUENCE {

condHandover-r16                              ENUMERATED {supported}           OPTIONAL,

condHandoverFailure-r16                      ENUMERATED {supported}           OPTIONAL,

condHandoverTwoTriggerEvents-r16             ENUMERATED {supported}           OPTIONAL

}                                                                         OPTIONAL,

pcellT312-r16                       ENUMERATED {supported}                OPTIONAL,

interFrequencyMeas-Nogap-r16				ENUMERATED {supported}				OPTIONAL,

simultaneousRxDataSSB-DiffNumerology-Inter-r16        ENUMERATED {supported}    OPTIONAL,

handoverIntraF-IAB-r16						ENUMERATED {supported}              OPTIONAL,

idleInactiveNR-MeasReport-r16               ENUMERATED {supported}              OPTIONAL

]]

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

–	MeasAndMobParametersMRDC

The IE MeasAndMobParametersMRDC is used to convey capability parameters related to RRM measurements and RRC mobility.

MeasAndMobParametersMRDC information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERSMRDC-START

MeasAndMobParametersMRDC ::=            SEQUENCE {

measAndMobParametersMRDC-Common         MeasAndMobParametersMRDC-Common                 OPTIONAL,

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff               OPTIONAL,

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff               OPTIONAL

}

MeasAndMobParametersMRDC-v1560 ::=      SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560      OPTIONAL

}

MeasAndMobParametersMRDC-v16xy ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v16xy      MeasAndMobParametersMRDC-Common-v16xy         OPTIONAL,

measAndMobParametersMRDC-XDD-Diff-v16xy    MeasAndMobParametersMRDC-XDD-Diff-v16xy       OPTIONAL,

measAndMobParametersMRDC-FRX-Diff-v16xy    MeasAndMobParametersMRDC-FRX-Diff-v16xy       OPTIONAL

}

MeasAndMobParametersMRDC-Common ::=     SEQUENCE {

independentGapConfig                    ENUMERATED {supported}                      OPTIONAL

}

MeasAndMobParametersMRDC-Common-v16xy ::=     SEQUENCE {

condPSCellChangeParametersCommon-r16                     SEQUENCE {

condPSCellChangeFDD-TDD-r16                              ENUMERATED {supported}                 OPTIONAL,

condPSCellChangeFR1-FR2-r16                              ENUMERATED {supported}                 OPTIONAL

}                                                                                      OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff ::=   SEQUENCE {

sftd-MeasPSCell                         ENUMERATED {supported}                      OPTIONAL,

sftd-MeasNR-Cell                        ENUMERATED {supported}                      OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff-v1560 ::=    SEQUENCE {

sftd-MeasPSCell-NEDC                           ENUMERATED {supported}              OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff-v16xy ::=    SEQUENCE {

condPSCellChangeParametersXDD-Diff-r16                     SEQUENCE {

condPSCellChange-r16                                      ENUMERATED {supported}                OPTIONAL,

condPSCellChangeTwoTriggerEvents-r16                     ENUMERATED {supported}                OPTIONAL

}                                                                                  OPTIONAL,

pscellT312-r16                                ENUMERATED {supported}               OPTIONAL

}

MeasAndMobParametersMRDC-FRX-Diff ::=          SEQUENCE {

simultaneousRxDataSSB-DiffNumerology           ENUMERATED {supported}              OPTIONAL

}

MeasAndMobParametersMRDC-FRX-Diff-v16xy ::=    SEQUENCE {

condPSCellChangeParametersFRX-Diff-r16                     SEQUENCE {

condPSCellChange-r16                                      ENUMERATED {supported}                OPTIONAL,

condPSCellChangeTwoTriggerEvents-r16                     ENUMERATED {supported}                OPTIONAL

}                                                                                  OPTIONAL,

pscellT312-r16                               ENUMERATED {supported}                OPTIONAL

}

-- TAG-MEASANDMOBPARAMETERSMRDC-STOP

-- ASN1STOP

–	MIMO-Layers

The IE MIMO-Layers is used to convey the number of supported MIMO layers.

MIMO-Layers information element

-- ASN1START

-- TAG-MIMO-LAYERS-START

MIMO-LayersDL ::=   ENUMERATED {twoLayers, fourLayers, eightLayers}

MIMO-LayersUL ::=   ENUMERATED {oneLayer, twoLayers, fourLayers}

-- TAG-MIMO-LAYERS-STOP

-- ASN1STOP

–	MIMO-ParametersPerBand

The IE MIMO-ParametersPerBand is used to convey MIMO related parameters specific for a certain band (not per feature set or band combination).

MIMO-ParametersPerBand information element

-- ASN1START

-- TAG-MIMO-PARAMETERSPERBAND-START

MIMO-ParametersPerBand ::=          SEQUENCE {

tci-StatePDSCH                      SEQUENCE {

maxNumberConfiguredTCIstatesPerCC   ENUMERATED {n4, n8, n16, n32, n64, n128}                                   OPTIONAL,

maxNumberActiveTCI-PerBWP           ENUMERATED {n1, n2, n4, n8}                                                OPTIONAL

}                                                                                                              OPTIONAL,

additionalActiveTCI-StatePDCCH              ENUMERATED {supported}                                             OPTIONAL,

pusch-TransCoherence                        ENUMERATED {nonCoherent, partialCoherent, fullCoherent}            OPTIONAL,

beamCorrespondenceWithoutUL-BeamSweeping    ENUMERATED {supported}                                             OPTIONAL,

periodicBeamReport                          ENUMERATED {supported}                                             OPTIONAL,

aperiodicBeamReport                         ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUCCH                          ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUSCH                          ENUMERATED {supported}                                             OPTIONAL,

dummy1                                      DummyG                                                             OPTIONAL,

maxNumberRxBeam                             INTEGER (2..8)                                                     OPTIONAL,

maxNumberRxTxBeamSwitchDL                   SEQUENCE {

scs-15kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-30kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-60kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-120kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-240kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL

}                                                                                                              OPTIONAL,

maxNumberNonGroupBeamReporting              ENUMERATED {n1, n2, n4}                                            OPTIONAL,

groupBeamReporting                          ENUMERATED {supported}                                             OPTIONAL,

uplinkBeamManagement                        SEQUENCE {

maxNumberSRS-ResourcePerSet-BM              ENUMERATED {n2, n4, n8, n16},

maxNumberSRS-ResourceSet                    INTEGER (1..8)

}                                                                                                              OPTIONAL,

maxNumberCSI-RS-BFD                 INTEGER (1..64)                                                            OPTIONAL,

maxNumberSSB-BFD                    INTEGER (1..64)                                                            OPTIONAL,

maxNumberCSI-RS-SSB-CBD             INTEGER (1..256)                                                           OPTIONAL,

dummy2                              ENUMERATED {supported}                                                     OPTIONAL,

twoPortsPTRS-UL                     ENUMERATED {supported}                                                     OPTIONAL,

dummy5                              SRS-Resources                                                              OPTIONAL,

dummy3                              INTEGER (1..4)                                                             OPTIONAL,

beamReportTiming                    SEQUENCE {

scs-15kHz                           ENUMERATED {sym2, sym4, sym8}                                              OPTIONAL,

scs-30kHz                           ENUMERATED {sym4, sym8, sym14, sym28}                                      OPTIONAL,

scs-60kHz                           ENUMERATED {sym8, sym14, sym28}                                            OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym56}                                           OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetDL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationDL                                               OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetUL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationUL                                               OPTIONAL

}                                                                                                              OPTIONAL,

dummy4                              DummyH                                                                     OPTIONAL,

aperiodicTRS                        ENUMERATED {supported}                                                     OPTIONAL,

...,

[[

dummy6                              ENUMERATED {true}                                                          OPTIONAL,

beamManagementSSB-CSI-RS            BeamManagementSSB-CSI-RS                                                   OPTIONAL,

beamSwitchTiming                    SEQUENCE {

scs-60kHz                           ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL

}                                                                                                              OPTIONAL,

codebookParameters                  CodebookParameters                                                         OPTIONAL,

csi-RS-IM-ReceptionForFeedback      CSI-RS-IM-ReceptionForFeedback                                             OPTIONAL,

csi-RS-ProcFrameworkForSRS          CSI-RS-ProcFrameworkForSRS                                                 OPTIONAL,

csi-ReportFramework                 CSI-ReportFramework                                                        OPTIONAL,

csi-RS-ForTracking                  CSI-RS-ForTracking                                                         OPTIONAL,

srs-AssocCSI-RS                     SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource  OPTIONAL,

spatialRelations                    SpatialRelations                                                           OPTIONAL

]],

[[

-- R1 16-2b-0: Support of default QCL assumption with two TCI states

defaultQCL-TwoTCI-r16				ENUMERATED {supported}                                                     OPTIONAL,

-- R1 16-7: Extension of the maximum number of configured aperiodic CSI report settings for all codebook types

extMaxAP-CSI-ReportSupport-r16		ENUMERATED {supported}                                                     OPTIONAL,

codebookParametersPerBand-r16		CodebookParameters-v16xy															OPTIONAL

]]

}

DummyG ::=                          SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n8, n16, n32, n64},

maxNumberSSB-CSI-RS-ResourceTwoTx   ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}

}

BeamManagementSSB-CSI-RS ::=        SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n0, n8, n16, n32, n64},

maxNumberCSI-RS-Resource            ENUMERATED {n0, n4, n8, n16, n32, n64},

maxNumberCSI-RS-ResourceTwoTx       ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}                                       OPTIONAL,

maxNumberAperiodicCSI-RS-Resource   ENUMERATED {n0, n1, n4, n8, n16, n32, n64}

}

DummyH ::=                          SEQUENCE {

burstLength                         INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..128)

}

CSI-RS-ForTracking ::=              SEQUENCE {

maxBurstLength                      INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..256)

}

CSI-RS-IM-ReceptionForFeedback ::=              SEQUENCE {

maxConfigNumberNZP-CSI-RS-PerCC                 INTEGER (1..64),

maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC      INTEGER (2..256),

maxConfigNumberCSI-IM-PerCC                     ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousNZP-CSI-RS-PerCC           INTEGER (1..64),

totalNumberPortsSimultaneousNZP-CSI-RS-PerCC    INTEGER (2..256)

}

CSI-RS-ProcFrameworkForSRS ::=                  SEQUENCE {

maxNumberPeriodicSRS-AssocCSI-RS-PerBWP         INTEGER (1..4),

maxNumberAperiodicSRS-AssocCSI-RS-PerBWP        INTEGER (1..4),

maxNumberSP-SRS-AssocCSI-RS-PerBWP              INTEGER (0..4),

simultaneousSRS-AssocCSI-RS-PerCC               INTEGER (1..8)

}

CSI-ReportFramework ::=                         SEQUENCE {

maxNumberPeriodicCSI-PerBWP-ForCSI-Report       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForCSI-Report      INTEGER (1..4),

maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report INTEGER (0..4),

maxNumberPeriodicCSI-PerBWP-ForBeamReport       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForBeamReport      INTEGER (1..4),

maxNumberAperiodicCSI-triggeringStatePerCC      ENUMERATED {n3, n7, n15, n31, n63, n128},

maxNumberSemiPersistentCSI-PerBWP-ForBeamReport INTEGER (0..4),

simultaneousCSI-ReportsPerCC                    INTEGER (1..8)

}

PTRS-DensityRecommendationDL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29)

}

PTRS-DensityRecommendationUL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29),

sampleDensity1                      INTEGER (1..276),

sampleDensity2                      INTEGER (1..276),

sampleDensity3                      INTEGER (1..276),

sampleDensity4                      INTEGER (1..276),

sampleDensity5                      INTEGER (1..276)

}

SpatialRelations ::=                    SEQUENCE {

maxNumberConfiguredSpatialRelations     ENUMERATED {n4, n8, n16, n32, n64, n96},

maxNumberActiveSpatialRelations         ENUMERATED {n1, n2, n4, n8, n14},

additionalActiveSpatialRelationPUCCH    ENUMERATED {supported}                              OPTIONAL,

maxNumberDL-RS-QCL-TypeD                ENUMERATED {n1, n2, n4, n8, n14}

}

DummyI ::=               SEQUENCE {

supportedSRS-TxPortSwitch           ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, tr-equal},

txSwitchImpactToRx                  ENUMERATED {true}                                       OPTIONAL

}

-- TAG-MIMO-PARAMETERSPERBAND-STOP

-- ASN1STOP

MIMO-ParametersPerBand field description

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework CSI related capabilities which the UE supports on each of the carriers operated on this band. For mixed FR1-FR2 band combinations these values may be further limited by the corresponding fields in Phy-ParametersFRX-Diff.

–	ModulationOrder

The IE ModulationOrder is used to convey the maximum supported modulation order.

ModulationOrder information element

-- ASN1START

-- TAG-MODULATIONORDER-START

ModulationOrder ::= ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

-- TAG-MODULATIONORDER-STOP

-- ASN1STOP

–	MRDC-Parameters

The IE MRDC-Parameters contains the band combination parameters specific to MR-DC for a given MR-DC band combination.

MRDC-Parameters information element

-- ASN1START

-- TAG-MRDC-PARAMETERS-START

MRDC-Parameters ::= SEQUENCE {

singleUL-Transmission               ENUMERATED {supported}              OPTIONAL,

dynamicPowerSharingENDC             ENUMERATED {supported}              OPTIONAL,

tdm-Pattern                         ENUMERATED {supported}              OPTIONAL,

ul-SharingEUTRA-NR                  ENUMERATED {tdm, fdm, both}         OPTIONAL,

ul-SwitchingTimeEUTRA-NR            ENUMERATED {type1, type2}           OPTIONAL,

simultaneousRxTxInterBandENDC       ENUMERATED {supported}              OPTIONAL,

asyncIntraBandENDC                  ENUMERATED {supported}              OPTIONAL,

...,

[[

dualPA-Architecture                 ENUMERATED {supported}              OPTIONAL,

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

ul-TimingAlignmentEUTRA-NR          ENUMERATED {required}               OPTIONAL

]],

[[

maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16    SEQUENCE{

eutra-TDD-Config0-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config1-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config2-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config3-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config4-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config5-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config6-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL

}        OPTIONAL,

-- R4 2-20: support co-located scenario only for inter-band EN-DC

interBandCoLocOnlySupportENDC-r16            ENUMERATED {type1, type2}           OPTIONAL

]]

}

MRDC-Parameters-v1580 ::= SEQUENCE {

dynamicPowerSharingNEDC             ENUMERATED {supported}              OPTIONAL

}

MRDC-Parameters-v1590 ::=	SEQUENCE {

interBandContiguousMRDC             ENUMERATED {supported}              OPTIONAL

}

-- TAG-MRDC-PARAMETERS-STOP

-- ASN1STOP

–	NRDC-Parameters

The IE NRDC-Parameters contains parameters specific to NR-DC, i.e., which are not applicable to NR SA.

NRDC-Parameters information element

-- ASN1START

-- TAG-NRDC-PARAMETERS-START

NRDC-Parameters ::=                 SEQUENCE {

measAndMobParametersNRDC            MeasAndMobParametersMRDC                    OPTIONAL,

generalParametersNRDC               GeneralParametersMRDC-XDD-Diff              OPTIONAL,

fdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

tdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

fr1-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

fr2-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

dummy                               SEQUENCE {}                                 OPTIONAL

}

NRDC-Parameters-v1570 ::=           SEQUENCE {

sfn-SyncNRDC                        ENUMERATED {supported}                      OPTIONAL

}

NRDC-Parameters-v16xy ::=           SEQUENCE {

measAndMobParametersNRDC-v16xy     MeasAndMobParametersMRDC-v16xy                OPTIONAL

}

-- TAG-NRDC-PARAMETERS-STOP

-- ASN1STOP

–	OLPC-SRS-Pos

The IE OLPC-SRS-Pos is used to convey OLPC SRS positioning related parameters specific for a certain band.

OLPC-SRS-Pos information element

-- ASN1START

-- TAG-OLPC-SRS-POS-START

OLPC-SRS-Pos-r16 ::=        SEQUENCE {

olpc-SRS-PosBasedOnPRS-Serving-r16         ENUMERATED {supported}                       OPTIONAL,

olpc-SRS-PosBasedOnSSB-Neigh-r16            ENUMERATED {supported}                       OPTIONAL,

olpc-SRS-PosBasedOnPRS-Neigh-r16            ENUMERATED {supported}                       OPTIONAL,

maxNumberPathLossEstimatePerServing-r16   NUMERATED {n1, n4, n8, n16}                OPTIONAL

}

--TAG-OLPC-SRS-POS-STOP

-- ASN1STOP

–	PDCP-Parameters

The IE PDCP-Parameters is used to convey capabilities related to PDCP.

PDCP-Parameters information element

-- ASN1START

-- TAG-PDCP-PARAMETERS-START

PDCP-Parameters ::=         SEQUENCE {

supportedROHC-Profiles      SEQUENCE {

profile0x0000               BOOLEAN,

profile0x0001               BOOLEAN,

profile0x0002               BOOLEAN,

profile0x0003               BOOLEAN,

profile0x0004               BOOLEAN,

profile0x0006               BOOLEAN,

profile0x0101               BOOLEAN,

profile0x0102               BOOLEAN,

profile0x0103               BOOLEAN,

profile0x0104               BOOLEAN

},

maxNumberROHC-ContextSessions       ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64,

cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles             ENUMERATED {supported}      OPTIONAL,

continueROHC-Context                ENUMERATED {supported}      OPTIONAL,

outOfOrderDelivery                  ENUMERATED {supported}      OPTIONAL,

shortSN                             ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationSRB                 ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationMCG-OrSCG-DRB       ENUMERATED {supported}      OPTIONAL,

...,

[[

drb-IAB-r16							ENUMERATED {supported}      OPTIONAL,

non-DRB-IAB-r16						ENUMERATED {supported}      OPTIONAL,

extendedDiscardTimer-r16       		ENUMERATED {supported}      OPTIONAL,

continueEHC-Context-r16				ENUMERATED {supported}      OPTIONAL,

ehc-r16								ENUMERATED {supported}      OPTIONAL,

maxNumberEHC-Contexts-r16			ENUMERATED {2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096, 8192, 16384, 32768, 65536}		OPTIONAL,

jointEHC-ROHC-Config-r16			ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationMoreThanTwoRLC-r16	ENUMERATED {supported}      OPTIONAL

]]

}

-- TAG-PDCP-PARAMETERS-STOP

-- ASN1STOP

–	PDCP-ParametersMRDC

The IE PDCP-ParametersMRDC is used to convey PDCP related capabilities for MR-DC.

PDCP-ParametersMRDC information element

-- ASN1START

-- TAG-PDCP-PARAMETERSMRDC-START

PDCP-ParametersMRDC ::=                 SEQUENCE {

pdcp-DuplicationSplitSRB                ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationSplitDRB                ENUMERATED {supported}      OPTIONAL

}

-- TAG-PDCP-PARAMETERSMRDC-STOP

-- ASN1STOP

–	Phy-Parameters

The IE Phy-Parameters is used to convey the physical layer capabilities.

Phy-Parameters information element

-- ASN1START

-- TAG-PHY-PARAMETERS-START

Phy-Parameters ::=                  SEQUENCE {

phy-ParametersCommon                Phy-ParametersCommon                        OPTIONAL,

phy-ParametersXDD-Diff              Phy-ParametersXDD-Diff                      OPTIONAL,

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                      OPTIONAL,

phy-ParametersFR1                   Phy-ParametersFR1                           OPTIONAL,

phy-ParametersFR2                   Phy-ParametersFR2                           OPTIONAL

}

Phy-ParametersCommon ::=            SEQUENCE {

csi-RS-CFRA-ForHO                   ENUMERATED {supported}                      OPTIONAL,

dynamicPRB-BundlingDL               ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUCCH                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUSCH                  ENUMERATED {supported}                      OPTIONAL,

nzp-CSI-RS-IntefMgmt                ENUMERATED {supported}                      OPTIONAL,

type2-SP-CSI-Feedback-LongPUCCH     ENUMERATED {supported}                      OPTIONAL,

precoderGranularityCORESET          ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-Codebook            ENUMERATED {supported}                      OPTIONAL,

semiStaticHARQ-ACK-Codebook         ENUMERATED {supported}                      OPTIONAL,

spatialBundlingHARQ-ACK             ENUMERATED {supported}                      OPTIONAL,

dynamicBetaOffsetInd-HARQ-ACK-CSI   ENUMERATED {supported}                      OPTIONAL,

pucch-Repetition-F1-3-4             ENUMERATED {supported}                      OPTIONAL,

ra-Type0-PUSCH                      ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PDSCH       ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PUSCH       ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeA                  ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeB                  ENUMERATED {supported}                      OPTIONAL,

interleavingVRB-ToPRB-PDSCH         ENUMERATED {supported}                      OPTIONAL,

interSlotFreqHopping-PUSCH          ENUMERATED {supported}                      OPTIONAL,

type1-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

type2-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

pusch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

pdsch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

downlinkSPS                         ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType1             ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType2             ENUMERATED {supported}                      OPTIONAL,

pre-EmptIndication-DL               ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-DL              ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-UL              ENUMERATED {supported}                      OPTIONAL,

cbg-FlushIndication-DL              ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-CodeB-CBG-Retx-DL   ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetSemi-Static     ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetDynamic         ENUMERATED {supported}                      OPTIONAL,

bwp-SwitchingDelay                  ENUMERATED {type1, type2}                   OPTIONAL,

...,

[[

dummy                               ENUMERATED {supported}                      OPTIONAL

]],

[[

maxNumberSearchSpaces               ENUMERATED {n10}                            OPTIONAL,

rateMatchingCtrlResrcSetDynamic     ENUMERATED {supported}                      OPTIONAL,

maxLayersMIMO-Indication            ENUMERATED {supported}                      OPTIONAL

]] ,

[[

-- R1 9-1: Basic channel structure and procedure of 2-step RACH

twoStepRACH-r16									ENUMERATED {supported}					OPTIONAL,

-- R1 11-1: Monitoring DCI format 1_2 and DCI format 0_2

dci-Format1-2And0-2-r16							ENUMERATED {supported}					OPTIONAL,

-- R1 11-1a: Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space

monitoringDCI-SameSearchSpace-r16				ENUMERATED {supported}					OPTIONAL,

-- R1 11-10: Type 2 configured grant release by DCI format 0_1

type2-CG-ReleaseDCI-0-1-r16						ENUMERATED {supported}					OPTIONAL,

-- R1 11-11: Type 2 configured grant release by DCI format 0_2

type2-CG-ReleaseDCI-0-2-r16						ENUMERATED {supported}					OPTIONAL,

-- R1 12-3: SPS release by DCI format 1_1

sps-ReleaseDCI-1-1-r16							ENUMERATED {supported}					OPTIONAL,

-- R1 12-3a: SPS release by DCI format 1_2

sps-ReleaseDCI-1-2-r16							ENUMERATED {supported}					OPTIONAL,

-- R1 14-8: CSI trigger states containing non-active BWP

csi-TriggerStateNon-ActiveBWP-r16				ENUMERATED {supported}					OPTIONAL,

-- R1 20-2: Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities

seperateSMTC-InterIAB-Support-r16	 			ENUMERATED {supported}                  OPTIONAL,

-- R1 20-3: Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors

seperateRACH-IAB-Support-r16	 				ENUMERATED {supported}                  OPTIONAL,

-- R1 20-5a: Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources

ul-flexibleDL-SlotFormatSemiStatic-IAB-r16		 	ENUMERATED {supported}                  OPTIONAL,

-- R1 20-5b: Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources

ul-flexibleDL-SlotFormatDynamics-IAB-r16	 	ENUMERATED {supported}                  OPTIONAL,

dft-S-OFDM-WaveformUL-IAB-r16		ENUMERATED {supported}                      OPTIONAL,

-- R1 20-6: Support DCI Format 2_5 based indication of soft resource availability to an IAB node

dci-25-AI-RNTI-Support-IAB-r16					 	ENUMERATED {supported}                  OPTIONAL,

-- R1 20-7: Support T_delta reception.

t-DeltaReceptionSupport-IAB-r16				 		ENUMERATED {supported}                  OPTIONAL,

-- R1 20-8: Support of Desired guard symbol reporting and provided guard symbok reception.

guardSymbolReportReception-IAB-r16				 	ENUMERATED {supported}                  OPTIONAL,

-- R1 18-8 HARQ-ACK codebook type and spatial bundling per PUCCH group

harqACK-CB-SpatialBundlingPUCCH-Group-r16				ENUMERATED {supported}                      OPTIONAL,

-- R1 19-2: Cross Slot Scheduling

crossSlotScheduling-r16							SEQUENCE {

licensedBand-r16									ENUMERATED {supported}                      OPTIONAL,

unlicensedBand-r16									ENUMERATED {supported}							OPTIONAL

}																							OPTIONAL,

maxNumberSRS-PosPathLossEstimateAllServingCells-r16          ENUMERATED {n1, n4, n8, n16}           OPTIONAL,

maxNumberSRS-PosSpatialRelationsAllServingCells-r16          ENUMERATED {n0, n1, n2, n4, n8, n16}   OPTIONAL,

extendedCG-Periodicities-r16		ENUMERATED {supported}                      OPTIONAL,

extendedSPS-Periodicities-r16		ENUMERATED {supported}                      OPTIONAL,

codebookVariantsList-r16			CodebookVariantsList-r16					OPTIONAL

]]

}

Phy-ParametersXDD-Diff ::=          SEQUENCE {

dynamicSFI                          ENUMERATED {supported}                      OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols         ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUSCH          ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUCCH          ENUMERATED {supported}                      OPTIONAL,

...,

[[

dl-SchedulingOffset-PDSCH-TypeA     ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB     ENUMERATED {supported}                      OPTIONAL,

ul-SchedulingOffset                 ENUMERATED {supported}                      OPTIONAL

]]

}

Phy-ParametersFRX-Diff ::=                  SEQUENCE {

dynamicSFI                                  ENUMERATED {supported}                      OPTIONAL,

dummy1                                      BIT STRING (SIZE (2))                       OPTIONAL,

twoFL-DMRS                                  BIT STRING (SIZE (2))                       OPTIONAL,

dummy2                                      BIT STRING (SIZE (2))                       OPTIONAL,

dummy3                                      BIT STRING (SIZE (2))                       OPTIONAL,

supportedDMRS-TypeDL                        ENUMERATED {type1, type1And2}               OPTIONAL,

supportedDMRS-TypeUL                        ENUMERATED {type1, type1And2}               OPTIONAL,

semiOpenLoopCSI                             ENUMERATED {supported}                      OPTIONAL,

csi-ReportWithoutPMI                        ENUMERATED {supported}                      OPTIONAL,

csi-ReportWithoutCQI                        ENUMERATED {supported}                      OPTIONAL,

onePortsPTRS                                BIT STRING (SIZE (2))                       OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols                 ENUMERATED {supported}                      OPTIONAL,

pucch-F2-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F3-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F4-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F0-2WithoutFH                         ENUMERATED {notSupported}                   OPTIONAL,

pucch-F1-3-4WithoutFH                       ENUMERATED {notSupported}                   OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot      ENUMERATED {supported}                      OPTIONAL,

uci-CodeBlockSegmentation                   ENUMERATED {supported}                      OPTIONAL,

onePUCCH-LongAndShortFormat                 ENUMERATED {supported}                      OPTIONAL,

twoPUCCH-AnyOthersInSlot                    ENUMERATED {supported}                      OPTIONAL,

intraSlotFreqHopping-PUSCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-LBRM                                  ENUMERATED {supported}                      OPTIONAL,

pdcch-BlindDetectionCA                      INTEGER (4..16)                             OPTIONAL,

tpc-PUSCH-RNTI                              ENUMERATED {supported}                      OPTIONAL,

tpc-PUCCH-RNTI                              ENUMERATED {supported}                      OPTIONAL,

tpc-SRS-RNTI                                ENUMERATED {supported}                      OPTIONAL,

absoluteTPC-Command                         ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUSCH                  ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUCCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-HalfPi-BPSK                           ENUMERATED {supported}                      OPTIONAL,

pucch-F3-4-HalfPi-BPSK                      ENUMERATED {supported}                      OPTIONAL,

almostContiguousCP-OFDM-UL                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-RS                                   ENUMERATED {supported}                      OPTIONAL,

sp-CSI-IM                                   ENUMERATED {supported}                      OPTIONAL,

tdd-MultiDL-UL-SwitchPerSlot                ENUMERATED {supported}                      OPTIONAL,

multipleCORESET                             ENUMERATED {supported}                      OPTIONAL,

...,

[[

csi-RS-IM-ReceptionForFeedback              CSI-RS-IM-ReceptionForFeedback              OPTIONAL,

csi-RS-ProcFrameworkForSRS                  CSI-RS-ProcFrameworkForSRS                  OPTIONAL,

csi-ReportFramework                         CSI-ReportFramework                         OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot       SEQUENCE {

sameSymbol                                  ENUMERATED {supported}                      OPTIONAL,

diffSymbol                                  ENUMERATED {supported}                      OPTIONAL

} OPTIONAL,

mux-SR-HARQ-ACK-PUCCH                       ENUMERATED {supported}                      OPTIONAL,

mux-MultipleGroupCtrlCH-Overlap             ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA             ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB             ENUMERATED {supported}                      OPTIONAL,

ul-SchedulingOffset                         ENUMERATED {supported}                      OPTIONAL,

dl-64QAM-MCS-TableAlt                       ENUMERATED {supported}                      OPTIONAL,

ul-64QAM-MCS-TableAlt                       ENUMERATED {supported}                      OPTIONAL,

cqi-TableAlt                                ENUMERATED {supported}                      OPTIONAL,

oneFL-DMRS-TwoAdditionalDMRS-UL             ENUMERATED {supported}                      OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-UL             ENUMERATED {supported}                      OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-UL           ENUMERATED {supported}                      OPTIONAL

]],

[[

pdcch-BlindDetectionNRDC                SEQUENCE {

pdcch-BlindDetectionMCG-UE              INTEGER (1..15),

pdcch-BlindDetectionSCG-UE              INTEGER (1..15)

}                                                                                       OPTIONAL,

mux-HARQ-ACK-PUSCH-DiffSymbol               ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 11-1b: Type 1 HARQ-ACK codebook support for relative TDRA for DL

type1-HARQ-ACK-Codebook-r16							ENUMERATED {supported}					OPTIONAL,

-- R1 11-8: Enhanced UL power control scheme

enhancedPowerControl-r16							ENUMERATED {supported}					OPTIONAL,

-- R1 16-1b-1: TCI state activation across multiple CCs

simultaneousTCI-ActMultipleCC-r16					ENUMERATED {supported}						OPTIONAL,

-- R1 16-1b-2: Spatial relation update across multiple CCs

simultaneousSpatialRelationMultipleCC-r16			ENUMERATED {supported}						OPTIONAL,

-- R1 16-1c: Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH

defaultSpatialRelationPathlossRS-r16				ENUMERATED {supported}						OPTIONAL,

-- R1 16-1d: Support of spatial relation update for AP-SRS via MAC CE

spatialRelationUpdateAP-SRS-r16					ENUMERATED {supported}						OPTIONAL,

cli-RSSI-FDM-DL-r16                         ENUMERATED {supported}                      OPTIONAL,

cli-SRS-RSRP-FDM-DL-r16                         ENUMERATED {supported}                      OPTIONAL,

-- R1 19-3: Maximum MIMO Layer Adaptation

maxLayersMIMO-Adaptation-r16					ENUMERATED {supported}						OPTIONAL,

]]

}

Phy-ParametersFR1 ::=                       SEQUENCE {

pdcch-MonitoringSingleOccasion              ENUMERATED {supported}                      OPTIONAL,

scs-60kHz                                   ENUMERATED {supported}                      OPTIONAL,

pdsch-256QAM-FR1                            ENUMERATED {supported}                      OPTIONAL,

pdsch-RE-MappingFR1-PerSymbol               ENUMERATED {n10, n20}                       OPTIONAL,

...,

[[

pdsch-RE-MappingFR1-PerSlot                 ENUMERATED {n16, n32, n48, n64, n80, n96, n112, n128,

n144, n160, n176, n192, n208, n224, n240, n256}         OPTIONAL

]]

}

Phy-ParametersFR2 ::=                       SEQUENCE {

dummy                                       ENUMERATED {supported}                                  OPTIONAL,

pdsch-RE-MappingFR2-PerSymbol               ENUMERATED {n6, n20}                                    OPTIONAL,

...,

[[

pCell-FR2                                   ENUMERATED {supported}                                  OPTIONAL,

pdsch-RE-MappingFR2-PerSlot                 ENUMERATED {n16, n32, n48, n64, n80, n96, n112, n128,

n144, n160, n176, n192, n208, n224, n240, n256}     OPTIONAL

]]

}

-- TAG-PHY-PARAMETERS-STOP

-- ASN1STOP

Phy-ParametersFRX-Diff field description

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework These fields are optionally present in fr1-fr2-Add-UE-NR-Capabilities in UE-NR-Capability. For a band combination comprised of FR1 and FR2 bands, these parameters, if present, limit the corresponding parameters in MIMO-ParametersPerBand.

–	Phy-ParametersMRDC

The IE Phy-ParametersMRDC is used to convey physical layer capabilities for MR-DC.

Phy-ParametersMRDC information element

-- ASN1START

-- TAG-PHY-PARAMETERSMRDC-START

Phy-ParametersMRDC ::=              SEQUENCE {

naics-Capability-List               SEQUENCE (SIZE (1..maxNrofNAICS-Entries)) OF NAICS-Capability-Entry         OPTIONAL,

...

}

NAICS-Capability-Entry ::=          SEQUENCE {

numberOfNAICS-CapableCC             INTEGER(1..5),

numberOfAggregatedPRB               ENUMERATED {n50, n75, n100, n125, n150, n175, n200, n225,

n250, n275, n300, n350, n400, n450, n500, spare},

...

}

-- TAG-PHY-PARAMETERSMRDC-STOP

-- ASN1STOP

PHY-ParametersMRDC field descriptions

naics-Capability-List Indicates that UE in MR-DC supports NAICS as defined in TS 36.331 [10].

–	ProcessingParameters

The IE ProcessingParameters is used to indicate PDSCH/PUSCH processing capabilities supported by the UE.

ProcessingParameters information element

-- ASN1START

-- TAG-PROCESSINGPARAMETERS-START

ProcessingParameters ::=        SEQUENCE {

fallback                        ENUMERATED {sc, cap1-only},

differentTB-PerSlot              SEQUENCE {

upto1                          NumberOfCarriers                    OPTIONAL,

upto2                          NumberOfCarriers                    OPTIONAL,

upto4                          NumberOfCarriers                    OPTIONAL,

upto7                          NumberOfCarriers                    OPTIONAL

} OPTIONAL

}

NumberOfCarriers ::=    INTEGER (1..16)

-- TAG-PROCESSINGPARAMETERS-STOP

-- ASN1STOP

–	PowSav-Parameters

The IE PowSav-Parameters is used to convey the capabilities supported by the UE for the power saving preferences.

PowSav-Parameters information element

-- ASN1START

-- TAG-POWSAV-PARAMETERS-START

PowSav-Parameters-r16 ::=         SEQUENCE {

powSav-ParametersCommon-r16               PowSav-ParametersCommon-r16                                        OPTIONAL,

powSav-ParametersFRX-Diff-r16             PowSav-ParametersFRX-Diff-r16                                      OPTIONAL,

...

}

PowSav-ParametersCommon-r16 ::=    SEQUENCE {

drx-Preference-r16                        ENUMERATED {supported}                                             OPTIONAL,

maxCC-Preference-r16                      ENUMERATED {supported}                                             OPTIONAL,

releasePreference-r16                     ENUMERATED {supported}                                             OPTIONAL,

-- R1 19-4a: UE assistance information

minSchedulingOffsetPreference-r16		  ENUMERATED {supported}                                             OPTIONAL,

...

}

PowSav-ParametersFRX-Diff-r16 ::=    SEQUENCE {

maxBW-Preference-r16                      ENUMERATED {supported}                                             OPTIONAL,

maxMIMO-LayerPreference-r16               ENUMERATED {supported}                                             OPTIONAL,

...

}

-- TAG-POWSAV-PARAMETERS-STOP

-- ASN1STOP

–	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, eutra-nr, eutra, utra-fdd-v16xy, ...}

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

–	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::=                   SEQUENCE {

supportedBandListNR                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550  BandCombinationList-v1550                   OPTIONAL

]],

[[

supportedBandCombinationList-v1560  BandCombinationList-v1560                   OPTIONAL

]],

[[

supportedBandCombinationList-v16xy  BandCombinationList-v16xy                   OPTIONAL,

]]

}

BandNR ::=                          SEQUENCE {

bandNR                              FreqBandIndicatorNR,

modifiedMPR-Behaviour               BIT STRING (SIZE (8))                           OPTIONAL,

mimo-ParametersPerBand              MIMO-ParametersPerBand                          OPTIONAL,

extendedCP                          ENUMERATED {supported}                          OPTIONAL,

multipleTCI                         ENUMERATED {supported}                          OPTIONAL,

bwp-WithoutRestriction              ENUMERATED {supported}                          OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                       OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                              OPTIONAL,

crossCarrierScheduling-SameSCS      ENUMERATED {supported}                          OPTIONAL,

pdsch-256QAM-FR2                    ENUMERATED {supported}                          OPTIONAL,

pusch-256QAM                        ENUMERATED {supported}                          OPTIONAL,

ue-PowerClass                       ENUMERATED {pc1, pc2, pc3, pc4}                 OPTIONAL,

rateMatchingLTE-CRS                 ENUMERATED {supported}                          OPTIONAL,

channelBWs-DL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

channelBWs-UL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

...,

[[

maxUplinkDutyCycle-PC2-FR1                  ENUMERATED {n60, n70, n80, n90, n100}   OPTIONAL

]],

[[

pucch-SpatialRelInfoMAC-CE          ENUMERATED {supported}                          OPTIONAL,

powerBoosting-pi2BPSK               ENUMERATED {supported}                          OPTIONAL

]],

[[

maxUplinkDutyCycle-FR2          ENUMERATED {n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}     OPTIONAL

]],

[[

channelBWs-DL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL,

channelBWs-UL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL

]],

[[

-- R1 10: NR-unlicensed

unlicensedParametersPerBand-r16		UnlicensedParametersPerBand-r16				OPTIONAL,

-- R1 11-7b: Independent cancellation of the overlapping PUSCHs in an intra-band UL CA

cancelOverlappingPUSCH-r16			ENUMERATED {supported}						OPTIONAL,

-- R1 14-1: Multiple LTE-CRS rate matching patterns

multipleRateMatchingEUTRA-CRS-r16	SEQUENCE {

maxNumberPatterns-r16					INTEGER (2..6),

maxNumberNon-OverlapPatterns-r16		INTEGER (1..3)

}																				OPTIONAL,

-- R1 14-1a: Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier

overlapRateMatchingEUTRA-CRS-r16	ENUMERATED {supported}						OPTIONAL,

-- R1 14-2: PDSCH Type B mapping of length 9 and 10 OFDM symbols

pdsch-MappingTypeB-Alt-r16			ENUMERATED {supported}						OPTIONAL,

-- R1 14-3: One slot periodic TRS configuration for FR1

oneShotPeriodicTRS-r16				ENUMERATED {supported}						OPTIONAL,

olpc-SRS-Pos-r16                      OLPC-SRS-Pos-r16                                     OPTIONAL,

spatialRelationsSRS-Pos-r16       SpatialRelationsSRS-Pos-r16                    OPTIONAL,

simul-SRS-Trans-IntraBandCA-r16   INTEGER (1..2)                                 OPTIONAL,

channelBW-DL-IAB-r16					CHOICE {

fr1-100mhz 		              	     SEQUENCE {

scs-15kHz                           ENUMERATED {supported}              OPTIONAL,

scs-30kHz                           ENUMERATED {supported}              OPTIONAL,

scs-60kHz                           ENUMERATED {supported}              OPTIONAL

},

fr2-200mhz                           SEQUENCE {

scs-60kHz                           ENUMERATED {supported}              OPTIONAL,

scs-120kHz                          ENUMERATED {supported}              OPTIONAL

}

}                                                                               OPTIONAL,

channelBW-UL-IAB-r16					CHOICE {

fr1-100mhz                           SEQUENCE {

scs-15kHz                           ENUMERATED {supported}              OPTIONAL,

scs-30kHz                           ENUMERATED {supported}              OPTIONAL,

scs-60kHz                           ENUMERATED {supported}              OPTIONAL

},

fr2-200mhz                           SEQUENCE {

scs-60kHz                           ENUMERATED {supported}              OPTIONAL,

scs-120kHz                          ENUMERATED {supported}              OPTIONAL

}

}																				OPTIONAL,

rasterShift7dot5-IAB-r16			ENUMERATED {supported}                       OPTIONAL,

ue-PowerClass-v16xy					ENUMERATED {pc1dot5}						OPTIONAL

]]

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

RF-Parameters field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationList A list of band combinations that the UE supports for NR (and NR-DC, if requested). The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

–	RF-ParametersMRDC

The IE RF-ParametersMRDC is used to convey RF related capabilities for MR-DC.

RF-ParametersMRDC information element

-- ASN1START

-- TAG-RF-PARAMETERSMRDC-START

RF-ParametersMRDC ::=                   SEQUENCE {

supportedBandCombinationList            BandCombinationList                 OPTIONAL,

appliedFreqBandListFilter               FreqBandList                        OPTIONAL,

...,

[[

srs-SwitchingTimeRequested              ENUMERATED {true}                   OPTIONAL,

supportedBandCombinationList-v1540      BandCombinationList-v1540           OPTIONAL

]],

[[

supportedBandCombinationList-v1550      BandCombinationList-v1550           OPTIONAL

]],

[[

supportedBandCombinationList-v1560      BandCombinationList-v1560           OPTIONAL,

supportedBandCombinationListNEDC-Only   BandCombinationList                 OPTIONAL

]],

[[

supportedBandCombinationList-v1570      BandCombinationList-v1570           OPTIONAL

]],

[[

supportedBandCombinationList-v1580      BandCombinationList-v1580           OPTIONAL

]],

[[

supportedBandCombinationList-v1590      BandCombinationList-v1590           OPTIONAL

]],

[[

supportedBandCombinationList-v16xy      BandCombinationList-v16xy           OPTIONAL

]]

}

-- TAG-RF-PARAMETERSMRDC-STOP

-- ASN1STOP

RF-ParametersMRDC field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter.

supportedBandCombinationList A list of band combinations that the UE supports for (NG)EN-DC and/or NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

supportedBandCombinationListNEDC-Only A list of band combinations that the UE supports only for NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

–	RLC-Parameters

The IE RLC-Parameters is used to convey capabilities related to RLC.

RLC-Parameters information element

-- ASN1START

-- TAG-RLC-PARAMETERS-START

RLC-Parameters ::= SEQUENCE {

am-WithShortSN                  ENUMERATED {supported}  OPTIONAL,

um-WithShortSN                  ENUMERATED {supported}  OPTIONAL,

um-WithLongSN                   ENUMERATED {supported}  OPTIONAL,

...,

[[

extendedT-PollRetransmit-r16       ENUMERATED {supported}  OPTIONAL,

extendedT-StatusProhibit-r16       ENUMERATED {supported}  OPTIONAL

]]

}

}

-- TAG-RLC-PARAMETERS-STOP

-- ASN1STOP

–	SDAP-Parameters

The IE SDAP-Parameters is used to convey capabilities related to SDAP.

SDAP-Parameters information element

-- ASN1START

-- TAG-SDAP-PARAMETERS-START

SDAP-Parameters ::= SEQUENCE {

as-ReflectiveQoS                ENUMERATED {true}       OPTIONAL,

...,

[[

sdap-QOS-IAB-r16			ENUMERATED {supported}  OPTIONAL,

sdapHeaderIAB-r16				ENUMERATED {supported}  OPTIONAL

]]

}

-- TAG-SDAP-PARAMETERS-STOP

-- ASN1STOP

#### –	SidelinkParameters

The IE SidelinkParameters is used to convey capabilities related to NR and E-UTRA sidelink communications.

SidelinkParameters information element

-- ASN1START

-- TAG-SIDELINKPARAMETERS-START

SidelinkParameters-r16 ::=	SEQUENCE {

sidelinkParametersNR-r16					SidelinkParametersNR-r16																		OPTIONAL,

sidelinkParametersEUTRA-r16					SidelinkParametersEUTRA-r16																		OPTIONAL

}

SidelinkParametersNR-r16 ::= SEQUENCE {

rlc-ParametersSidelink-r16                  RLC-ParametersSidelink-r16                                                        OPTIONAL,

mac-ParametersSidelink-r16                  MAC-ParametersSidelink-r16                                                        OPTIONAL,

fdd-Add-UE-Sidelink-Capabilities-r16        UE-SidelinkCapabilityAddXDD-Mode-r16                                             OPTIONAL,

tdd-Add-UE-Sidelink-Capabilities-r16        UE-SidelinkCapabilityAddXDD-Mode-r16                                             OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                                       OPTIONAL

}

SidelinkParametersEUTRA-r16 ::=			SEQUENCE {

sl-ParametersEUTRA1-r16					OCTET STRING														OPTIONAL,

sl-ParametersEUTRA2-r16					OCTET STRING														OPTIONAL,

sl-ParametersEUTRA3-r16					OCTET STRING														OPTIONAL,

supportedBandListSidelinkEUTRA-r16		SEQUENCE (SIZE (1..maxBandsEUTRA)) OF BandSidelinkEUTRA-r16		OPTIONAL,

...

}

RLC-ParametersSidelink-r16 ::= SEQUENCE {

am-WithLongSN-Sidelink-r16                   ENUMERATED {supported}  				OPTIONAL,

um-WithLongSN-Sidelink-r16                   ENUMERATED {supported}  				OPTIONAL,

...

}

MAC-ParametersSidelink-r16 ::= SEQUENCE {

mac-ParametersSidelinkCommon-r16            MAC-ParametersSidelinkCommon-r16        OPTIONAL,

mac-ParametersSidelinkXDD-Diff-r16          MAC-ParametersSidelinkXDD-Diff-r16      OPTIONAL,

...

}

UE-SidelinkCapabilityAddXDD-Mode-r16 ::=  SEQUENCE {

mac-ParametersSidelinkXDD-Diff-r16          MAC-ParametersSidelinkXDD-Diff-r16      OPTIONAL

}

MAC-ParametersSidelinkCommon-r16 ::=    SEQUENCE {

lcp-RestrictionSidelink-r16                 ENUMERATED {supported}     			    OPTIONAL,

multipleConfiguredGrantsSidelink-r16        ENUMERATED {supported}                  OPTIONAL,

...

}

MAC-ParametersSidelinkXDD-Diff-r16 ::=  SEQUENCE {

multipleSR-ConfigurationsSidelink-r16       ENUMERATED {supported}                  OPTIONAL,

logicalChannelSR-DelayTimerSidelink-r16     ENUMERATED {supported}                  OPTIONAL,

...

}

BandSidelinkEUTRA-r16 ::=			SEQUENCE {

freqBandSidelinkEUTRA-r16				FreqBandIndicatorEUTRA,

-- R1 15-7: Transmitting LTE sidelink mode 3 scheduled by NR Uu

gnb-ScheduledMode3SidelinkEUTRA-r16		SEQUENCE {

gnb-ScheduledMode3DelaySidelinkEUTRA-r16		ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1,

ms1dot25, ms1dot5, ms1dot75, ms2, ms2dot5, ms3, ms4,

ms5, ms6, ms8, ms10, ms20}

}																											OPTIONAL,

-- R1 15-9: Transmitting LTE sidelink mode 4 configured by NR Uu

gnb-ScheduledMode4SidelinkEUTRA-r16		ENUMERATED {supported}												OPTIONAL,

}

-- TAG-SIDELINKPARAMETERS-STOP

-- ASN1STOP

SidelinkParametersEUTRA field descriptions

sl-ParametersEUTRA1, sl-ParametersEUTRA2, sl-ParametersEUTRA3 This field includes IE of SL-Parameters-v1430 (where v2x-eNB-Scheduled-r14 and V2X-SupportedBandCombination-r14 shall not be included), SL-Parameters-v1530 (where V2X-SupportedBandCombination-r1530 shall not be included) and SL-Parameters-v1540 respectively defined in 36.331 [10]. It is used for reporting the per-UE capability for V2X sidelink communication.

–	SON-Parameters

The IE SON-Parameters contains SON related parameters.

SON-Parameters information element

-- ASN1START

-- TAG-SON-PARAMETERS-START

SON-Parameters-r16 ::= SEQUENCE {

rach-Report-r16			ENUMERATED {true}       OPTIONAL,

...

}

-- TAG-SON-PARAMETERS-STOP

-- ASN1STOP

–	SpatialRelationsSRS-Pos

The IE SpatialRelationsSRS-Pos is used to convey spatial relation for SRS for positioning related parameters.

SpatialRelationsSRS-Pos information element

-- ASN1START

-- TAG-SPATIALRELATIONSSRS-POS-START

SpatialRelationsSRS-Pos-r16 ::=                    SEQUENCE {

spatialRelation-SRS-PosBasedOnSSB-Serving-r16         ENUMERATED {supported}                       OPTIONAL,

spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16      ENUMERATED {supported}                       OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Serving-r16         ENUMERATED {supported}                       OPTIONAL,

spatialRelation-SRS-PosBasedOnSRS-r16                   ENUMERATED {supported}                       OPTIONAL,

spatialRelation-SRS-PosBasedOnSSB-Neigh-r16            ENUMERATED {supported}                       OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Neigh-r16            ENUMERATED {supported}                       OPTIONAL

}

--TAG-SPATIALRELATIONSSRS-POS-STOP

-- ASN1STOP

–	SRS-SwitchingTimeNR

The IE SRS-SwitchingTimeNR is used to indicate the SRS carrier switching time supported by the UE for one NR band pair.

SRS-SwitchingTimeNR information element

-- ASN1START

-- TAG-SRS-SWITCHINGTIMENR-START

SRS-SwitchingTimeNR ::= SEQUENCE {

switchingTimeDL         ENUMERATED {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}  OPTIONAL,

switchingTimeUL         ENUMERATED {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}  OPTIONAL

}

-- TAG-SRS-SWITCHINGTIMENR-STOP

-- ASN1STOP

–	SRS-SwitchingTimeEUTRA

The IE SRS-SwitchingTimeEUTRA is used to indicate the SRS carrier switching time supported by the UE for one E-UTRA band pair.

SRS-SwitchingTimeEUTRA information element

-- ASN1START

-- TAG-SRS-SWITCHINGTIMEEUTRA-START

SRS-SwitchingTimeEUTRA ::= SEQUENCE {

switchingTimeDL            ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7}

OPTIONAL,

switchingTimeUL            ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7}

OPTIONAL

}

-- TAG-SRS-SWITCHINGTIMEEUTRA-STOP

-- ASN1STOP

–	SupportedBandwidth

The IE SupportedBandwidth is used to indicate the maximum channel bandwidth supported by the UE on one carrier of a band of a band combination.

SupportedBandwidth information element

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth ::=      CHOICE {

fr1                         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2                         ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

–	UE-BasedPerfMeas-Parameters

The IE UE-BasedPerfMeas-Parameters contains UE-based performance measurement parameters.

UE-BasedPerfMeas-Parameters information element

-- ASN1START

-- TAG-UE-BASEDPERFMEAS-PARAMETERS-START

UE-BasedPerfMeas-Parameters-r16 ::= SEQUENCE {

barometerMeasReport-r16		ENUMERATED {true}       OPTIONAL,

immMeasBT-r16				ENUMERATED {true}       OPTIONAL,

immMeasWLAN-r16				ENUMERATED {true}       OPTIONAL,

loggedMeasBT-r16				ENUMERATED {true}       OPTIONAL,

loggedMeasurements-r16		ENUMERATED {true}       OPTIONAL,

loggedMeasWLAN-r16			ENUMERATED {true}       OPTIONAL,

orientationMeasReport-r16	ENUMERATED {true}       OPTIONAL,

speedMeasReport-r16			ENUMERATED {true}       OPTIONAL,

gnss-Location-r16			ENUMERATED {true}       OPTIONAL,

ulPDCP-Delay-r16				ENUMERATED {true}       OPTIONAL,

...

}

-- TAG-UE-BASEDPERFMEAS-PARAMETERS-STOP

-- ASN1STOP

–	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of radio access technology specific capability containers.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-CONTAINERLIST-START

UE-CapabilityRAT-ContainerList ::=    SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::=        SEQUENCE {

rat-Type                              RAT-Type,

ue-CapabilityRAT-Container            OCTET STRING

}

-- TAG-UE-CAPABILITYRAT-CONTAINERLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For rat-Type set to nr: the encoding of UE capabilities is defined in UE-NR-Capability. For rat-Type set to eutra-nr: the encoding of UE capabilities is defined in UE-MRDC-Capability. For rat-Type set to eutra: the encoding of UE capabilities is defined in UE-EUTRA-Capability specified in TS 36.331 [10]. For rat-Type set to utra-fdd: the octet string contains the INTER RAT HANDOVER INFO message defined in TS 25.331 [45].

–	UE-CapabilityRAT-RequestList

The IE UE-CapabilityRAT-RequestList is used to request UE capabilities for one or more RATs from the UE.

UE-CapabilityRAT-RequestList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-START

UE-CapabilityRAT-RequestList ::=        SEQUENCE (SIZE (1..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request

UE-CapabilityRAT-Request ::=            SEQUENCE {

rat-Type                                RAT-Type,

capabilityRequestFilter                 OCTET STRING                    OPTIONAL,   -- Need N

...

}

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-Request field descriptions

capabilityRequestFilter Information by which the network requests the UE to filter the UE capabilities. For rat-Type set to nr or eutra-nr: the encoding of the capabilityRequestFilter is defined in UE-CapabilityRequestFilterNR. For rat-Type set to eutra: the encoding of the capabilityRequestFilter is defined by UECapabilityEnquiry message defined in TS36.331 [10], in which RAT-Type in UE-CapabilityRequest includes only 'eutra'.

rat-Type The RAT type for which the NW requests UE capabilities.

–	UE-CapabilityRequestFilterCommon

The IE UE-CapabilityRequestFilterCommon is used to request filtered UE capabilities. The filter is common for all capability containers that are requested.

UE-CapabilityRequestFilterCommon information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-START

UE-CapabilityRequestFilterCommon ::=            SEQUENCE {

mrdc-Request                                SEQUENCE {

omitEN-DC                                   ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNR-DC                                ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNE-DC                                ENUMERATED {true}                      OPTIONAL     -- Need N

}                                                                                  OPTIONAL,        -- Need N

...,

[[

codebookTypeRequest-r16						SEQUENCE {

type1-SinglePanel-r16							ENUMERATED {true}						OPTIONAL,		-- Need N

type1-MultiPanel-r16							ENUMERATED {true}						OPTIONAL,		-- Need N

type2-r16										ENUMERATED {true}						OPTIONAL,		-- Need N

type2-PortSelection-r16						ENUMERATED {true}						OPTIONAL			-- Need N

}																						OPTIONAL			-- Need N

]]

}

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-STOP

-- ASN1STOP

UE-CapabilityRequestFilterCommon field descriptions

codebookTypeRequest Only if this field is present, the UE includes SupportedCSI-RS-Resource supported for the codebook type(s) requested within this field (i.e. type I single/multi-panel, type II and type II port selection) into codebookVariantsList, codebookParametersPerBand and codebookParametersPerBC. If this field is present and none of the codebook types is requested within this field (i.e. empty field), the UE includes SupportedCSI-RS-Resource supported for all codebook types into codebookVariantsList, codebookParametersPerBand and codebookParametersPerBC.

includeNE-DC Only if this field is present, the UE supporting NE-DC shall indicate support for NE-DC in band combinations and include feature set combinations which are applicable to NE-DC. Band combinations supporting both NE-DC and (NG)EN-DC shall be included in supportedBandCombinationList, band combinations supporting only NE-DC shall be included in supportedBandCombinationListNEDC-Only.

includeNR-DC Only if this field is present, the UE supporting NR-DC shall indicate support for NR-DC in band combinations and include feature set combinations which are applicable to NR-DC.

omitEN-DC Only if this field is present, the UE shall omit band combinations and feature set combinations which are only applicable to (NG)EN-DC.

–	UE-CapabilityRequestFilterNR

The IE UE-CapabilityRequestFilterNR is used to request filtered UE capabilities.

UE-CapabilityRequestFilterNR information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERNR-START

UE-CapabilityRequestFilterNR ::=            SEQUENCE {

frequencyBandListFilter                     FreqBandList                          OPTIONAL,   -- Need N

nonCriticalExtension                        UE-CapabilityRequestFilterNR-v1540    OPTIONAL

}

UE-CapabilityRequestFilterNR-v1540 ::=      SEQUENCE {

srs-SwitchingTimeRequest                    ENUMERATED {true}                     OPTIONAL,  -- Need N

nonCriticalExtension                        SEQUENCE {}                           OPTIONAL

}

-- TAG-UE-CAPABILITYREQUESTFILTERNR-STOP

-- ASN1STOP

–	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [26].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=              SEQUENCE {

measAndMobParametersMRDC            MeasAndMobParametersMRDC                                                        OPTIONAL,

phy-ParametersMRDC-v1530            Phy-ParametersMRDC                                                              OPTIONAL,

rf-ParametersMRDC                   RF-ParametersMRDC,

generalParametersMRDC               GeneralParametersMRDC-XDD-Diff                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

tdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

fr1-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

fr2-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

featureSetCombinations              SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

pdcp-ParametersMRDC-v1530           PDCP-ParametersMRDC                                                             OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                                                    OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1560                                                        OPTIONAL

}

UE-MRDC-Capability-v1560 ::=        SEQUENCE {

receivedFilters                     OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                         OPTIONAL,

measAndMobParametersMRDC-v1560      MeasAndMobParametersMRDC-v1560                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

tdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v16xy                                                                     OPTIONAL

}

UE-MRDC-Capability-v16xy ::=        SEQUENCE {

measAndMobParametersMRDC-v16xy      MeasAndMobParametersMRDC-v16xy                                                  OPTIONAL,

generalParametersMRDC-v16xy         GeneralParametersMRDC-v16xy                                                  OPTIONAL,

pdcp-ParametersMRDC-v16xy           PDCP-ParametersMRDC-v16xy															OPTIONAL,

measAndMobParametersMRDC-v16xy		MeasAndMobParametersMRDC-v16xy														OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff                                           OPTIONAL,

generalParametersMRDC-XDD-Diff          GeneralParametersMRDC-XDD-Diff                                              OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode-v1560 ::=    SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560                                  OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff

}

GeneralParametersMRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path             ENUMERATED {supported}                                                          OPTIONAL,

splitDRB-withUL-Both-MCG-SCG        ENUMERATED {supported}                                                          OPTIONAL,

srb3                                ENUMERATED {supported}                                                          OPTIONAL,

v2x-EUTRA                           ENUMERATED {supported}                                                          OPTIONAL,

...

}

GeneralParametersMRDC-v16xy ::= SEQUENCE {

f1c-OverEUTRA-r16                	ENUMERATED {supported}                                                          OPTIONAL

}

PDCP-ParametersMRDC-v16xy ::= SEQUENCE {

scg-DRB-NR-IAB-r16                	ENUMERATED {supported}                                                          OPTIONAL

}

MeasAndMobParametersMRDC-v16xy ::= SEQUENCE {

interNR-MeasEUTRA-IAB-r16           ENUMERATED {supported}                                                  				OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

UE-MRDC-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList and supportedBandCombinationListNEDC-Only in UE-MRDC-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

–	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        OPTIONAL,

mac-Parameters                  MAC-Parameters                                                        OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                                                  OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

featureSets                     FeatureSets                                                           OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

lateNonCriticalExtension        OCTET STRING                                                          OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530                                                OPTIONAL

}

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

dummy                                    ENUMERATED {supported}                                       OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                                          OPTIONAL,

inactiveState                            ENUMERATED {supported}                                       OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                                       OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                                               OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                        OPTIONAL,

ims-Parameters                          IMS-Parameters                                                OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                   OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                        OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1560                                       OPTIONAL

}

UE-NR-Capability-v1560 ::=               SEQUENCE {

nrdc-Parameters                         NRDC-Parameters                                               OPTIONAL,

receivedFilters                         OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)       OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1570                                        OPTIONAL

}

UE-NR-Capability-v1570 ::=               SEQUENCE {

nrdc-Parameters-v1570                   NRDC-Parameters-v1570                                         OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v16xy                                        OPTIONAL

}

UE-NR-Capability-v16xy ::=               SEQUENCE {

inDeviceCoexInd-r16                     ENUMERATED {supported}                                        OPTIONAL,

dl-DedicatedMessageSegmentation-r16     ENUMERATED {supported}                                        OPTIONAL,

nrdc-Parameters-v16xy                   NRDC-Parameters-v16xy                                         OPTIONAL,

powSav-Parameters-r16                   PowSav-Parameters-r16                                         OPTIONAL,

fr1-Add-UE-NR-Capabilities-v16xy        UE-NR-CapabilityAddFRX-Mode-v16xy                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v16xy        UE-NR-CapabilityAddFRX-Mode-v16xy                             OPTIONAL,

bh-RLF-Indication-r16					ENUMERATED {supported}                                        OPTIONAL,

directSN-AdditionFirstRRC-IAB-r16		ENUMERATED {supported}                                        OPTIONAL,

bap-Parameters-r16						BAP-Parameters-r16													OPTIONAL,

referenceTimeProvision-r16				ENUMERATED {supported}                                        OPTIONAL,

sidelinkParameters-r16					SidelinkParameters-r16												OPTIONAL,

highSpeedParameters-r16					HighSpeedParameters-r16												OPTIONAL,

mac-Parameters-v16xy                    MAC-Parameters-v16xy                                          OPTIONAL,

mcgRLF-RecoveryViaSCG-r16               ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredMCG-SCells-r16          ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredSCG-r16                 ENUMERATED {supported}                                        OPTIONAL,

resumeWithSCG-Config-r16                ENUMERATED {supported}                                        OPTIONAL,

ue-BasedPerfMeas-Parameters-r16         UE-BasedPerfMeas-Parameters-r16                               OPTIONAL,

son-Parameters-r16						SON-Parameters-r16				                              OPTIONAL,

onDemandSIB-Connected-r16               ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                   OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=         SEQUENCE {

phy-ParametersXDD-Diff                  Phy-ParametersXDD-Diff                                        OPTIONAL,

mac-ParametersXDD-Diff                  MAC-ParametersXDD-Diff                                        OPTIONAL,

measAndMobParametersXDD-Diff            MeasAndMobParametersXDD-Diff                                  OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::= SEQUENCE {

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                                            OPTIONAL,

measAndMobParametersFRX-Diff        MeasAndMobParametersFRX-Diff                                      OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                       OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v16xy ::=    SEQUENCE {

powSav-ParametersFRX-Diff-r16            PowSav-ParametersFRX-Diff-r16                                OPTIONAL,

mac-ParametersFRX-Diff-r16               MAC-ParametersFRX-Diff-r16                                   OPTIONAL

}

BAP-Parameters-r16 ::=                   SEQUENCE {

flowControlBH-RLC-ChannelBased-r16		ENUMERATED {supported}                                       OPTIONAL,

flowControlRouting-ID-Based-r16			ENUMERATED {supported}                                       OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList in UE-NR-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

–	UnlicensedParametersPerBand

The IE UnlicensedParametersPerBand is used to convey unlicensed operation related parameters specific for a certain unlicensed band (not per feature set or band combination).

UnlicensedParametersPerBand information element

-- ASN1START

-- TAG-UNLICENSEDPARAMETERSPERBAND-START

UnlicensedParametersPerBand-r16 ::=           SEQUENCE {

-- R1 10-2g: SSB-based BFD/CBD for dynamic channel access mode

ssb-BFD-CBD-dynamicChannelAccess-r16				ENUMERATED {supported}				OPTIONAL,

-- R1 10-2h: SSB-based BFD/CBD for semi-static channel access mode

ssb-BFD-CBD-semi-staticChannelAccess-r16    			ENUMERATED {supported}				OPTIONAL,

-- R1 10-2i: CSI-RS-based BFD/CBD for NR-U

csi-RS-BFD-CBD-r16									ENUMERATED {supported}				OPTIONAL,

-- R1 10-10: RSSI and channel occupancy measurement and reporting

rssi-ChannelOccupancyReporting-r16					ENUMERATED {supported}				OPTIONAL,

-- R1 10-11:SRS starting position at any OFDM symbol in a slot

srs-StartAnyOFDM-Symbol-r16							ENUMERATED {supported}				OPTIONAL,

-- R1 10-20: Support search space set configuration with freqMonitorLocation-r16

searchSpaceFreqMonitorLocation-r16					INTEGER (1..5)						OPTIONAL,

-- R1 10-20a: Support coreset configuration with rb-Offset

coreset-RB-Offset-r16								ENUMERATED {supported}				OPTIONAL,

-- R1 10-23:CGI reading on unlicensed cell for ANR functionality

cgi-Acquisition-r16									ENUMERATED {supported}				OPTIONAL,

-- R1 10-25: Enable configured UL transmissions when DCI 2_0 is configured but not detected

configuredUL-Tx-r16									ENUMERATED {supported}				OPTIONAL,

-- R1 10-8: Type B PDSCH length {3, 5, 6, 8, 9, 10, 11, 12, 13} without DMRS shift due to CRS collision

typeB-PDSCH-length-r16								ENUMERATED {supported}				OPTIONAL,

-- R1 10-9: Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring

searchSpaceSetGroupSwitchingwithDCI-r16				ENUMERATED {supported}				OPTIONAL,

-- R1 10-9b: Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring FFS:per band or per UE

searchSpaceSetGroupSwitchingwithoutDCI-r16			ENUMERATED {supported}				OPTIONAL,

-- R1 10-9d: Support Search space set group switching capability 2

searchSpaceSetGroupSwitchingcapability2-r16			ENUMERATED {supported}				OPTIONAL,

-- R1 10-14: Non-numerical PDSCH to HARQ-ACK timing

non-numericalPDSCH-HARQ-timing-r16					ENUMERATED {supported}				OPTIONAL,

-- R1 10-15: Enhanced dynamic HARQ codebook

enhancedDynamicHARQ-codebook-r16					ENUMERATED {supported}				OPTIONAL,

-- R1 10-16: One-shot HARQ ACK feedback

oneShotHARQ-feedback-r16							ENUMERATED {supported}				OPTIONAL,

-- R1 10-17: Multi-PUSCH UL grant

multiPUSCH-UL-grant-r16								ENUMERATED {supported}				OPTIONAL,

-- R1 10-26: CSI-RS based RLM for NR-U

csi-RS-RLM-r16										ENUMERATED {supported}				OPTIONAL,

-- R1 10-26a: CSI-RS based RRM for NR-U

csi-RS-RRM-r16										ENUMERATED {supported}				OPTIONAL,

-- R1 10-3: PRB interlace mapping for PUSCH

pusch-PRB-interlace-r16								ENUMERATED {supported}				OPTIONAL,

-- R1 10-3a: PRB interlace mapping for PUCCH

pucch-F0-F1-PRB-Interlace-r16						ENUMERATED {supported}				OPTIONAL,

-- R1 10-12: OCC for PRB interlace mapping for PF2 and PF3

occ-PRB-PF2-PF3-r16									ENUMERATED {supported}				OPTIONAL,

-- R1 10-13a: Extended CP range of more than one symbol for CG-PUSCH

extCP-rangeCG-PUSCH-r16								ENUMERATED {supported}				OPTIONAL,

-- R1 10-18: Configured grant with retransmission in CG resources

configuredGrantWithReTx-r16							ENUMERATED {supported}				OPTIONAL,

-- R1 10-24: CG-UCI multiplexing with HARQ ACK

mux-CG-UCI-HARQ-ACK-r16								ENUMERATED {supported}				OPTIONAL,

-- R1 10-28: Configured grant with Rel-16 enhanced resource configuration

cg-resourceConfig-r16								ENUMERATED {supported}				OPTIONAL,

}

-- TAG-UNLICENSEDPARAMETERSPERBAND-STOP

-- ASN1STOP

<< skip unrelated part >>

6.4	RRC multiplicity and type constraint values

–	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

ffsValue                                INTEGER ::= 65536   -- Placehold for all FFS values, to be removed

maxNrofFFS-r16                          INTEGER ::= 65536   -- Maximum number of FFS

maxAI-DCI-PayloadSize-r16               INTEGER ::= 128      --Maximum size of the DCI payload scrambled with ai-RNTI

maxAI-DCI-PayloadSize-r16-1             INTEGER ::= 127      --Maximum size of the DCI payload scrambled with ai-RNTI minus 1

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxBandsUTRA-FDD-r16                    INTEGER ::= 64      -- Maximum number of bands listed in UTRA-FDD UE caps

maxBT-IdReport-r16                      INTEGER ::= 32      -- Maximum number of Bluetooth IDs to report

maxBT-Name-r16                          INTEGER ::= 4       -- Maximum number of Bluetooth name

maxCBR-Config-r16                       INTEGER ::= 8       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control

maxCBR-Config-1-r16                     INTEGER ::= 7

maxCBR-Level-r16                        INTEGER ::= 16      -- Maximum nuber of CBR levels

maxCBR-Level-1-r16                      INTEGER ::= 15

maxCellBlack                            INTEGER ::= 16      -- Maximum number of NR blacklisted cell ranges in SIB3, SIB4

maxCellHistory-r16                      INTEGER ::= 16      -- Maximum number of visited cells reported

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 65535   -- Maximum number of cells per carrier for idle/inactive measurements is FFS

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxCellWhite                            INTEGER ::= 16      -- Maximum number of NR whitelisted cell ranges in SIB3, SIB4

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellBlack                      INTEGER ::= 16      -- Maximum number of E-UTRA blacklisted physical cell identity ranges

-- in SIB5

maxEUTRA-NS-Pmax                        INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxLogMeasReport-r16                    INTEGER ::= 520     -- Maximum number of entries for logged measurements

maxMultiBands                           INTEGER ::= 8       -- Maximum number of additional frequency bands that a cell belongs to

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

maxNR-NS-Pmax                           INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFreqIdle-r16                         INTEGER ::= 8       -- Maximum number of carrier frequencies for idle/inactive measurements

maxNrofServingCells                     INTEGER ::= 32      -- Max number of serving cells (SpCells + SCells)

maxNrofServingCells-1                   INTEGER ::= 31      -- Max number of serving cells (SpCell + SCells) per cell group

maxNrofAggregatedCellsPerCellGroup      INTEGER ::= 16

maxNrofDUCells-r16                      INTEGER ::= 512     -- Max number of cells configured on the collocated IAB-DU

maxNrofAssociatedDUCellsPerMT-r16       INTEGER ::= 65535   -- FFS

maxNrofAvailabilityCombinationsPerSet-r16   INTEGER ::= 512 -- Max number of AvailabilityCombinationId used in the DCI format 2_5

maxNrofAvailabilityCombinationsPerSet-r16-1 INTEGER ::= 511 -- Max number of AvailabilityCombinationId used in the DCI format 2_5 minus 1

maxNrofSCells                           INTEGER ::= 31      -- Max number of secondary serving cells per cell group

maxNrofCellMeas                         INTEGER ::= 32      -- Maximum number of entries in each of the cell lists in a measurement

-- object

maxNrofCG-SL-r16                        INTEGER ::= 8       -- Max number of configured sidelink grant

maxNrofSS-BlocksToAverage               INTEGER ::= 16      -- Max number for the (max) number of SS blocks to average to determine cell

-- measurement

maxNrofCondCells-r16                    INTEGER ::= 8       -- Max number of conditional candidate SpCells

maxNrofCSI-RS-ResourcesToAverage        INTEGER ::= 16      -- Max number for the (max) number of CSI-RS to average to determine cell

-- measurement

maxNrofDL-Allocations                   INTEGER ::= 16      -- Maximum number of PDSCH time domain resource allocations

maxNrofSR-ConfigPerCellGroup            INTEGER ::= 8       -- Maximum number of SR configurations per cell group

maxLCG-ID                               INTEGER ::= 7       -- Maximum value of LCG ID

maxLC-ID                                INTEGER ::= 32      -- Maximum value of Logical Channel ID

maxLC-ID-Iab-r16                        INTEGER ::= ffsValue -- Maximum value of BH Logical Channel ID extension

maxLTE-CRS-Patterns-r16                 INTEGER ::= 3       -- Maximum number of additional LTE CRS rate matching patterns

maxNrofTAGs                             INTEGER ::= 4       -- Maximum number of Timing Advance Groups

maxNrofTAGs-1                           INTEGER ::= 3       -- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs                             INTEGER ::= 4       -- Maximum number of BWPs per serving cell

maxNrofCombIDC                          INTEGER ::= 128     -- Maximum number of reported MR-DC combinations for IDC

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed

-- from 0..13)

maxNrofSlots                            INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks           INTEGER ::= 275     -- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1      INTEGER ::= 276     -- Maximum number of PRBs plus 1

maxNrofControlResourceSets-1            INTEGER ::= 11      -- Max number of CoReSets configurable on a serving cell minus 1

maxNrofControlResourceSets-1-r16        INTEGER ::= 15      -- Max number of CoReSets configurable on a serving cell extended in minus 1

maxNrofCoresetPools-r16                 INTEGER ::= 2       -- Maximum number of CORESET pools

maxCoReSetDuration                      INTEGER ::= 3       -- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces-1                   INTEGER ::= 39      -- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize                  INTEGER ::= 128     -- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1                INTEGER ::= 127     -- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxINT-DCI-PayloadSize                  INTEGER ::= 126     -- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1                INTEGER ::= 125     -- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns                INTEGER ::= 4       -- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1              INTEGER ::= 3       -- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup        INTEGER ::= 8       -- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1       INTEGER ::= 47      -- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers            INTEGER ::= 128     -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger  INTEGER ::= 16      -- Maximum number of report configurations per trigger state for aperiodic

-- reporting

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1           INTEGER ::= 191     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per cell

maxNrofNZP-CSI-RS-ResourceSets-1        INTEGER ::= 63      -- Maximum number of NZP CSI-RS resources per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig    INTEGER ::= 128     -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources              INTEGER ::= 32      -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1            INTEGER ::= 31      -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1         INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets           INTEGER ::= 16

maxNrofCSI-IM-Resources                 INTEGER ::= 32      -- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1               INTEGER ::= 31      -- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax

-- in 38.214.

maxNrofCSI-IM-ResourcesPerSet           INTEGER ::= 8       -- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax

-- in 38.214

maxNrofCSI-IM-ResourceSets              INTEGER ::= 64      -- Maximum number of NZP CSI-IM resources per cell

maxNrofCSI-IM-ResourceSets-1            INTEGER ::= 63      -- Maximum number of NZP CSI-IM resources per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig     INTEGER ::= 16      -- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet           INTEGER ::= 64      -- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets             INTEGER ::= 64      -- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1           INTEGER ::= 63      -- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

maxNrofFailureDetectionResources        INTEGER ::= 10      -- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1      INTEGER ::= 9       -- Maximum number of failure detection resources minus 1

maxNrofFreqSL-r16                       INTEGER ::= 8       -- Maximum number of carrier frequncy for for NR sidelink communication

maxNrofSL-BWPs-r16                      INTEGER ::= 4       -- Maximum number of BWP for for NR sidelink communication

maxFreqSL-EUTRA-r16                     INTEGER ::= 8       -- Maximum number of EUTRA anchor carrier frequncy for NR sidelink

-- communication

maxNrofSL-MeasId-r16                    INTEGER ::= 84      -- Maximum number of sidelink measurement identity (RSRP)

maxNrofSL-ObjectId-r16                  INTEGER ::= 64      -- Maximum number of sidelink measurement objects (RSRP)

maxNrofSL-ReportConfigId-r16            INTEGER ::= 64      -- Maximum number of sidelink measurement reporting configuration(RSRP)

maxNrofSL-PoolToMeasureEUTRA-r16        INTEGER ::= 8       -- Maximum number of resoure pool for V2X sidelink measurement to measure

-- for each measurement object (for CBR)

maxNrofSL-PoolToMeasureNR-r16           INTEGER ::= 8       -- Maximum number of resoure pool for NR sidelink measurement to measure for

-- each measurement object (for CBR)

maxFreqSL-NR-r16                        INTEGER ::= 8       -- Maximum number of NR anchor carrier frequncy for NR sidelink

-- communication

maxNrofSL-QFIs-r16                      INTEGER ::= 2048    -- Maximum number of QoS flow for NR sidelink communication per UE

maxNrofSL-QFIsPerDest-r16               INTEGER ::= 64      -- Maximum number of QoS flow per destination for NR sidelink communication

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPCI-Ranges                       INTEGER ::= 8       -- Maximum number of PCI ranges

maxPLMN                                 INTEGER ::= 12      -- Maximum number of PLMNs broadcast and reported by UE at establisghment

maxNrofCSI-RS-ResourcesRRM              INTEGER ::= 96      -- Maximum number of CSI-RS resources for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1            INTEGER ::= 95      -- Maximum number of CSI-RS resources for an RRM measurement object minus 1

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxNrofQuantityConfig                   INTEGER ::= 2       -- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM                  INTEGER ::= 96      -- Maximum number of cells with CSI-RS resources for an RRM measurement

-- object

maxNrofSL-Dest-r16                      INTEGER ::= 32      -- Maximum number of destination for NR sidelink communication

maxNrofSL-Dest-1-r16                    INTEGER ::= 31      -- Highest index of destination for NR sidelink communication

maxNrofSLRB-r16                         INTEGER ::= 512     -- Maximum number of radio bearer for NR sidelink communication per UE

maxSL-LCID-r16                          INTEGER ::= 512     -- Maximum number of RLC bearer for NR sidelink communication per UE

maxSL-SyncConfig-r16                    INTEGER ::= 16      -- Maximum number of sidelink Sync configurations

maxNrofRXPool-r16                       INTEGER ::= 16      -- Maximum number of Rx resource poolfor NR sidelink communication

maxNrofTXPool-r16                       INTEGER ::= 8       -- Maximum number of Tx resourcepoolfor NR sidelink communication

maxNrofPoolID-r16                       INTEGER ::= 16      -- Maximum index of resource pool for NR sidelink communication

maxNrofSRS-PathlossReferenceRS-r16-1    INTEGER ::= ffsValue --

maxNrofSRS-ResourceSets                 INTEGER ::= 16      -- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1               INTEGER ::= 15      -- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-PosResourceSets-r16          INTEGER ::= 16      -- Maximum number of SRS Positioning resource sets in a BWP.

maxNrofSRS-PosResourceSets-1-r16        INTEGER ::= 15      -- Maximum number of SRS Positioning resource sets in a BWP minus 1.

maxNrofSRS-Resources                    INTEGER ::= 64      -- Maximum number of SRS resources.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources in an SRS resource set minus 1.

maxNrofSRS-PosResources-r16             INTEGER ::= 64      -- Maximum number of SRS Positioning resources.

maxNrofSRS-PosResources-1-r16           INTEGER ::= 63      -- Maximum number of SRS Positioning resources in an SRS Positioning

-- resource set minus 1.

maxNrofSRS-ResourcesPerSet              INTEGER ::= 16      -- Maximum number of SRS resources in an SRS resource set

maxNrofSRS-TriggerStates-1              INTEGER ::= 3       -- Maximum number of SRS trigger states minus 1, i.e., the largest code

-- point.

maxNrofSRS-TriggerStates-2              INTEGER ::= 2       -- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers             INTEGER ::= 8       -- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands                    INTEGER ::= 32      -- Maximum number of simultaneously aggregated bands

maxNrofSlotFormatCombinationsPerSet     INTEGER ::= 512     -- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1   INTEGER ::= 511     -- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofTrafficPattern-r16               INTEGER ::= 8       -- Maximum number of Traffic Pattern for NR sidelink communication.

maxNrofPUCCH-Resources                  INTEGER ::= 128

maxNrofPUCCH-Resources-1                INTEGER ::= 127

maxNrofPUCCH-ResourceSets               INTEGER ::= 4       -- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1             INTEGER ::= 3       -- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet            INTEGER ::= 32      -- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-P0-PerSet                  INTEGER ::= 8       -- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUCCH power

-- control minus 1.

maxNrofPUCCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1 extended.

maxNrofPUCCH-ResourceGroups-r16         INTEGER ::= 4       -- Maximum number of PUCCH resources groups.

maxNrofPUCCH-ResourcesPerGroup-r16      INTEGER ::= ffsValue -- Maximum number of PUCCH resources in a PUCCH group.

maxNrofPUCCH-ResourcesPerGroup-1-r16    INTEGER ::= ffsValue -- Maximum number of PUCCH resources in a PUCCH group minus 1.

maxNrofServingCells-r16                 INTEGER ::= ffsValue -- Maximum number of serving cells in simultaneousTCI-UpdateList.

maxNrofP0-PUSCH-AlphaSets               INTEGER ::= 30      -- Maximum number of P0-pusch-alpha-sets (see 38,213, clause 7.1)

maxNrofP0-PUSCH-AlphaSets-1             INTEGER ::= 29      -- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, clause 7.1)

maxNrofPUSCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUSCH power

-- control minus 1.

maxNrofPUSCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended

maxNrofPUSCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- minus 1

maxNrofNAICS-Entries                    INTEGER ::= 8       -- Maximum number of supported NAICS capability set

maxBands                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

maxBandsMRDC                            INTEGER ::= 1280

maxBandsEUTRA                           INTEGER ::= 256

maxCellReport                           INTEGER ::= 8

maxDRB                                  INTEGER ::= 29      -- Maximum number of DRBs (that can be added in DRB-ToAddModLIst).

maxFreq                                 INTEGER ::= 8       -- Max number of frequencies.

maxFreqIDC-r16                          INTEGER ::= 128     -- Max number of frequencies for IDC indication.

maxCombIDC-r16                          INTEGER ::= 128     -- Max number of reported UL CA for IDC indication.

maxFreqIDC-MRDC                         INTEGER ::= 32      -- Maximum number of candidate NR frequencies for MR-DC IDC indication

maxNrofCandidateBeams                   INTEGER ::= 16      -- Max number of PRACH-ResourceDedicatedBFR that in BFR config.

maxNrofCandidateBeams-r16               INTEGER ::= 64      -- Max number of candidate beam resources in BFR config.

maxNrofCandidateBeamsExt-r16            INTEGER ::= 9999    -- FFS

maxNrofPCIsPerSMTC                      INTEGER ::= 64      -- Maximun number of PCIs per SMTC.

maxNrofQFIs                             INTEGER ::= 64

maxNrofResourceAvailabilityPerCombination-r16 INTEGER ::= 64  -- FFS

maxNrOfSemiPersistentPUSCH-Triggers     INTEGER ::= 64      -- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources                     INTEGER ::= 8       -- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination        INTEGER ::= 256

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

maxNrofIndexesToReport                  INTEGER ::= 32

maxNrofIndexesToReport2                 INTEGER ::= 64

maxNrofSSBs-r16                         INTEGER ::= 64      -- Maximum number of SSB resources in a resource set.

maxNrofSSBs-1                           INTEGER ::= 63      -- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI                          INTEGER ::= 8       -- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH                  INTEGER ::= 64

maxNrofTCI-States                       INTEGER ::= 128     -- Maximum number of TCI states.

maxNrofTCI-States-1                     INTEGER ::= 127     -- Maximum number of TCI states minus 1.

maxNrofUL-Allocations                   INTEGER ::= 16      -- Maximum number of PUSCH time domain resource allocations.

maxQFI                                  INTEGER ::= 63

maxRA-CSIRS-Resources                   INTEGER ::= 96

maxRA-OccasionsPerCSIRS                 INTEGER ::= 64      -- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1                       INTEGER ::= 511     -- Maximum number of RA occasions in the system

maxRA-SSB-Resources                     INTEGER ::= 64

maxSCSs                                 INTEGER ::= 5

maxSecondaryCellGroups                  INTEGER ::= 3

maxNrofServingCellsEUTRA                INTEGER ::= 32

maxMBSFN-Allocations                    INTEGER ::= 8

maxNrofMultiBands                       INTEGER ::= 8

maxCellSFTD                             INTEGER ::= 3       -- Maximum number of cells for SFTD reporting

maxReportConfigId                       INTEGER ::= 64

maxNrofCodebooks                        INTEGER ::= 16      -- Maximum number of codebooks suppoted by the UE

maxNrofCSI-RS-Resources                 INTEGER ::= 7       -- Maximum number of codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-r16			INTEGER ::= 512		-- Maximum number of alternative codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-1-r16		INTEGER ::= 511		-- Maximum number of alternative codebook resources supported by the UE minus 1

maxNrofSRI-PUSCH-Mappings               INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1             INTEGER ::= 15

maxSIB                                  INTEGER::= 32       -- Maximum number of SIBs

maxSI-Message                           INTEGER::= 32       -- Maximum number of SI messages

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

maxAccessCat-1                          INTEGER ::= 63      -- Maximum number of Access Categories minus 1

maxBarringInfoSet                       INTEGER ::= 8       -- Maximum number of Access Categories

maxCellEUTRA                            INTEGER ::= 8       -- Maximum number of E-UTRA cells in SIB list

maxEUTRA-Carrier                        INTEGER ::= 8       -- Maximum number of E-UTRA carriers in SIB list

maxPLMNIdentities                       INTEGER ::= 8       -- Maximum number of PLMN identites in RAN area configurations

maxDownlinkFeatureSets                  INTEGER ::= 1024    -- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets                    INTEGER ::= 1024    -- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand                   INTEGER ::= 128     -- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets                    INTEGER ::= 1024    -- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations               INTEGER ::= 1024    -- (for MR-DC/NR)Total number of Feature set combinations (size of the

-- pool)

maxInterRAT-RSTD-Freq                   INTEGER ::= 3

maxHRNN-Len-r16                         INTEGER ::= ffsValue -- Maximum length of HRNNs, value is FFS

maxNPN-r16                              INTEGER ::= 12      -- Maximum number of NPNs broadcast and reported by UE at establishment

maxNrOfMinSchedulingOffsetValues-r16    INTEGER ::= 2       -- Maximum number of min. scheduling offset (K0/K2) configurations

maxK0-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K2)

maxDCI-2-6-Size-r16                     INTEGER ::= 140     -- Maximum size of DCI format 2-6

maxDCI-2-6-Size-1-r16                   INTEGER ::= 139     -- Maximum DCI format 2-6 size minus 1

maxNrofUL-Allocations-r16               INTEGER ::= 64      -- Maximum number of PUSCH time domain resource allocations

maxNrofP0-PUSCH-Set-r16                 INTEGER ::= 2       -- Maximum number of P0 PUSCH set(s)

maxCI-DCI-PayloadSize-r16               INTEGER ::= 126     -- Maximum number of the DCI size for CI

maxCI-DCI-PayloadSize-r16-1             INTEGER ::= 125     -- Maximum number of the DCI size for CI minus 1

maxWLAN-Id-Report-r16                   INTEGER ::= 32      -- Maximum number of WLAN IDs to report

maxWLAN-Name-r16                        INTEGER ::= 4       -- Maximum number of WLAN name

maxRAReport-r16                         INTEGER ::= 8       -- Maximum number of RA procedures information to be included in the

-- RA report

maxTxConfig-r16                         INTEGER ::= 64

maxTxConfig-1-r16                       INTEGER ::= 63

maxPSSCH-TxConfig-r16                   INTEGER ::= 16      -- Maximum number of PSSCH TX configurations

maxNrofCLI-RSSI-Resources-r16           INTEGER ::= 64      -- Maximum number of CLI-RSSI resources for UE

maxNrofCLI-RSSI-Resources-r16-1         INTEGER ::= 63      -- Maximum number of CLI-RSSI resources for UE minus 1

maxNrofSRS-Resources-r16                INTEGER ::= 32      -- Maximum number of SRS resources for CLI measurement for UE

maxCLI-Report-r16                       INTEGER ::= 8

maxNrofConfiguredGrantConfig-r16        INTEGER ::= 12      -- Maximum number of configured grant configurations per BWP

maxNrofConfiguredGrantConfig-r16-1      INTEGER ::= 11      -- Maximum number of configured grant configurations per BWP minus 1

maxNrofConfiguredGrantConfigMAC-r16     INTEGER ::= 32      -- Maximum number of configured grant configurations per MAC entity

maxNrofConfiguredGrantConfigMAC-r16-1   INTEGER ::= 31      -- Maximum number of configured grant configurations per MAC entity minus 1

maxNrofSPS-Config-r16                   INTEGER ::= 8       -- Maximum number of SPS configurations per BWP

maxNrofSPS-Config-r16-1                 INTEGER ::= 7       -- Maximum number of SPS configurations per BWP minus 1

maxNrofDormancyGroups                   INTEGER ::= 5       --

maxNrofPUCCH-ResourceGroups-1-r16       INTEGER ::= 3       --

maxNrofServingCellsTCI-r16              INTEGER ::= ffsValue    --

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

–	End of NR-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

Next Change

### 6.6.1	General message structure

#### –	PC5-RRC-Definitions

This ASN.1 segment is the start of the PC5 RRC PDU definitions.

-- ASN1START

-- TAG-PC5-RRC-DEFINITIONS-START

PC5-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

RRC-TransactionIdentifier,

SN-FieldLengthAM,

SN-FieldLengthUM,

LogicalChannelIdentity,

maxNrofSLRB-r16,

maxNrofSL-QFIs-r16,

maxNrofSL-QFIsPerDest-r16,

RSRP-Range,

SL-MeasConfig-r16,

SL-MeasId-r16,

FreqBandList

FROM NR-RRC-Definitions;

-- TAG-PC5-RRC-DEFINITIONS-STOP

-- ASN1STOP

Next Change

#### –	UECapabilityEnquirySidelink

The UECapabilityEnquirySidelink message is used to request UE sidelink capabilities. It is only applied to unicast of NR sidelink communication.

Signalling radio bearer: Sidelink SRB for PC5-RRC

RLC-SAP: AM

Logical channel: SCCH

Direction: UE to UE

UECapabilityEnquirySidelink information element

-- ASN1START

-- TAG-UECAPABILITYENQUIRYSIDELINK-START

UECapabilityEnquirySidelink ::=         SEQUENCE {

rrc-TransactionIdentifier-r16           RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

ueCapabilityEnquirySidelink-r16         UECapabilityEnquirySidelink-IEs-r16,

criticalExtensionsFuture                SEQUENCE {}

}

}

UECapabilityEnquirySidelink-IEs-r16 ::= SEQUENCE {

ueCapabilityRequestFilterSidelink-r16   UE-CapabilityRequestFilterSidelink                                      OPTIONAL, -- Need N

ueCapabilityInformationSidelink-r16     OCTET STRING                                                            OPTIONAL,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                              OPTIONAL

}

-- TAG-UECAPABILITYENQUIRYSIDELINK-STOP

-- ASN1STOP

UECapabilityEnquirySidelink-IEs field descriptions

ueCapabilityInformationSidelink This filed indicates the UECapabilityInformationSidelink message to provide the UE sidelink capability, which can be optionally sent together with UECapabilityEnquirySidelink.

#### –	UE-CapabilityRequestFilterSidelink

The IE UE-CapabilityRequestFilterSidelink is used to request filtered UE capabilities.

UE-CapabilityRequestFilterSidelink information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERSIDELINK-START

UE-CapabilityRequestFilterSidelink ::=      SEQUENCE {

frequencyBandListFilterSidelink             FreqBandList                          OPTIONAL,   -- Need N

nonCriticalExtension                        SEQUENCE {}    OPTIONAL

}

-- TAG-UE-CAPABILITYREQUESTFILTERSIDELINK-STOP

-- ASN1STOP

#### –	UECapabilityInformationSidelink

The IE UECapabilityInformationSidelink message is used to transfer UE radio access capabilities. It is only applied to unicast of NR sidelink communication.

Signalling radio bearer: Sidelink SRB for PC5-RRC

RLC-SAP: AM

Logical channel: SCCH

Direction: UE to UE

UECapabilityInformationSidelink information element

-- ASN1START

-- TAG-UECAPABILITYINFORMATIONSIDELINK-START

UECapabilityInformationSidelink ::=         SEQUENCE {

rrc-TransactionIdentifier-r16               RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

ueCapabilityInformationSidelink-r16         UECapabilityInformationSidelink-IEs-r16,

criticalExtensionsFuture                    SEQUENCE {}

}

}

UECapabilityInformationSidelink-IEs-r16 ::= SEQUENCE {

accessStratumReleaseSidelink-r16			AccessStratumReleaseSidelink-r16,

pdcp-ParametersSidelink-r16                 PDCP-ParametersSidelink-r16                                             OPTIONAL,

rlc-ParametersSidelink-r16                  RLC-ParametersSidelink-r16                                              OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        SEQUENCE{}                                                              OPTIONAL

}

AccessStratumReleaseSidelink-r16 ::= ENUMERATED {

rel16, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... }

PDCP-ParametersSidelink-r16 ::= SEQUENCE {

outOfOrderDeliverySidelink-r16              ENUMERATED {supported}      OPTIONAL,

...

}

RLC-ParametersSidelink-r16 ::= SEQUENCE {

am-WithLongSN-Sidelink-r16                  ENUMERATED {supported}  OPTIONAL,

um-WithLongSN-Sidelink-r16                  ENUMERATED {supported}  OPTIONAL,

...

}

-- TAG-UECAPABILITYINFORMATIONSIDELINK-STOP

-- ASN1STOP

End of changes