3GPP TSG-RAN WG2 #113-e	R2-21xxxxx

E-meeting, 25 January – 05 February 2021

Agenda item:	8.4.2

Source: 	Samsung

Title: 	Report from email discussion [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung)

Document for:	Discussion and decision

# Introduction

This submission contains the report from the following post-RAN2#112-e email discussion:

- [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung)

Scope: Starting from previous outcomes, determine which issues to address. If time permits can have a round of discussion on candidate solutions, e.g. which solution to address identified issue. Details also as proposed in [Post112-e][030], Intermediate deadlines by Rapporteur.

Intended outcome: Report, including at least an agreeable list of issues to focus on going forward, in the order of priority/support, pave the way for meeting discussions and agreements.

Deadline: Long

As a reminder, the following has been agreed as one of the objectives of the Rel-17 IAB WI:

Topology, routing and transport enhancements [RAN2-led, RAN3]:

- Specifications of enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation

Starting from guidelines provided by the Chair in the post-RAN2#111-e email discussion [Post111-e][902][eIAB], and the submissions on the relevant issues to RAN2#111-e (none of them were treated as there were no TUs allocated to Rel-17 IAB WI at RAN2#111-e, but were a very useful and valid starting point nonetheless), a discussion aimed at agreeing a baseline and discussing main issues was held and is captured in R2-2011061. A follow-up, more detailed discussion was then held during RAN2#112-e ([AT112-e][030][eIAB], captured in R2-2011142), aiming at discussing initial solutions.

This document aims to revisit the fundamental issues in more detail, capture and quantify support for different identified issues, and map the solutions already identified in R2-2011142 to a comprehensive list of issues.

# Phase-1: collecting issues

In Tables 1/2/3 below, rapporteur has captured Fairness/Latency/Congestion issues based on R2-2011142 and R2-2011061, and various offline discussions already held. Companies are invited to add further issues. At this stage there is no need to dispute an issue, or indicate support for an issue (although naturally adding your own issue which is not initially in the rapporteur’s list will indicate some level of support for that specific issue).

You are however welcome to share clarifications on certain issues in the final column (this is especially important if you were one of the original proposers of an issue/related solutions, and what is captured is not complete or correct in your view).

Please note there is currently no mapping of solutions to these issues (this will happen in Phase-3) – at this stage suffice it to say that there are fewer issues than solutions (as is expected – this does not mean that down-selection of solutions has taken place). Also, several of Topology and Latency issues overlap, so they are only captured in one place.

## Topology-wide fairness

| Issue no. | Issue description | Why cannot the issue be solved with Rel-16 baseline and/or implementation? | Why is the issue worth pursuing? | Any comments? |
| --- | --- | --- | --- | --- |
| IF-1 | The scheduler of an IAB node may not have all the information needed (e.g. link quality across multiple hops) to make appropriate scheduling decisions (e.g. proportional fair) for different bearers/RLC channels across multiple child-IAB nodes | The scheduler of an IAB node does not have the information on links further downstream | Improves scheduling efficiency and increases fairness |  |
| IF-2 | Workload on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced | IAB nodes do not have the QoS info of the bearers themselves, while CU has no info of the BH links load | Helps achieve similar latency for UEs with same QoS requirement |  |
| IF-3 | IAB node cannot prioritize bearers with e.g. higher number of hops | The scheduler of an IAB node does not have a latency reference (e.g. total number of hops, remaining number of hops) for the packets being scheduled | Helps provide same QoS experience regardless of hop number |  |
| IF-4 | IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers | The scheduler has no knowledge of number of aggregated bearers on a BH channel | Helps provide same QoS experience regardless of bearer mapping |  |
| IF-5 | IAB node cannot offer per-bearer fairness | The scheduler does not have QoS information of individual bearers, and feedback from CU to access node (via NR UP protocol) is not visible to intermediate nodes | Helps provide same QoS experience regardless of bearer mapping |  |
| … |  |  |  |  |

## Multi-hop latency

| Issue no. | Issue description | Why cannot the issue be solved with Rel-16 baseline and/or implementation? | Why is the issue worth pursuing? | Any comments? |
| --- | --- | --- | --- | --- |
| IL-1 | IAB node cannot help ensure that overall or remaining PDB is met for a packet | IAB node does not have information on the remaining PDB or overall PDB | Helps to meet required PDB |  |
| IL-2 | IAB node may need to report joint buffer status for LCHs with rather differing QoS requirements | IAB node’s scheduling granularity is limited by the Rel-16 number of LCGs | Helps to meet required PDB |  |
| IL-3 | Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors | Buffer size calculation for pre-emptive BSR is left to implementation | Helps ensure more consistent scheduling across the network(s) |  |
| IL-4 | IAB node cannot determine the one-hop latency for the access link | Latency between donor CU and IAB-node-DU, including possible multiple hops in between, is unknown to the IAB node | Helps to meet required PDB |  |
| IL-5 | The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free | The CU does not have information on the buffer status of IAB nodes or status of individual links | Helps ensure more efficient scheduling and meet required PDB |  |
| IL-6 | The CU is unable to configure routing based on actual latency per BH RLC channel | The CU does not have information (e.g. measurement reports) on one-hop latency per BH RLC channel | Helps to meet required PDB |  |
| IL-7 | IAB node cannot reliably/efficiently discard packets | The only PDB information known to the IAB node is one-hop PDB (used for scheduling) – the CU cannot currently provide e.g. a recommended discard PDB | Helps ensure more efficient scheduling and meet required PDB |  |
| … |  |  |  |  |

## Congestion mitigation

| Issue no. | Issue description | Why cannot the issue be solved with Rel-16 baseline and/or implementation? | Why is the issue worth pursuing? | Any comments? |
| --- | --- | --- | --- | --- |
| IC-1 | Long-term congestion on a single link cannot be alleviated using existing mechanisms | Rel-16 HbH flow control cannot handle long-term congestion | Helps alleviate and prevent congestion |  |
| IC-2 | Congestion in a chain (local part of the network) cannot be alleviated using existing mechanisms | Rel-16 HbH flow control only handles localized congestion | Helps alleviate and prevent congestion |  |
| IC-3 | Child node keeps requesting UL resources and/or allocating UL resources to its descendant nodes, even if the parent node is experiencing upstream congestion | There is no UL HbH flow control in Rel-16 | It can bring enhancements on top of existing UL scheduling procedures and potentially alleviate upstream congestion |  |
| IC-4 | IAB node does not know if the parent node has recovered (or is likely to recover) from BH RLF | IAB node has no knowledge of the progress of recovery from BH RLF – only that initial recovery has failed | Minimizes unnecessary topology changes, helps prevent congestion |  |
| IC-5 | IAB node may send flow control feedback multiple times | Triggering of flow control feedback is done based on a simple threshold rule | Keeps overhead in check |  |
| IC-6 | Child node stays with the same parent node, even if the parent node is experiencing downstream congestion, which could cause congestion to worsen further downstream | Child node is unaware of congestion at parent node | Helps alleviate and prevent congestion |  |
| IC-7 | CU cannot update the routing path that is experiencing congestion | CU has no knowledge of local congestion conditions | Minimizes unnecessary topology changes, helps prevent congestion | vivo: Another reason why we think this issue should be pursued is that it also helps to maintain the centralized management role of CU, so that the routing behaviors under within the same topology can be well-controlled. |
| … |  |  |  |  |

# Phase-2: consolidating the list of issues, collecting support for individual issues

# Phase-3: mapping of previously proposed solutions to the consolidated list of issues

# Conclusions