3GPP TSG-RAN WG2 #113 electronic	R2-20xxxxx

Electronic Meeting, Jan 25 – Feb 05, 2021

Agenda Item:	x.x.x.x

Source:	Ericsson

Title:	Summary of e-mail discussion on UE indication when it no longer experiences overheating

Document for:	Discussion, Decision

1	Introduction

This contribution summarizes the following e-mail discussion:

[Post112-e][067][NR TEI16] UE indication when it no longer experiences overheating (Ericsson)

Scope: Based on R2-2010543, find solution, prepare for decisions next meeting

Intended outcome: Report

Deadline: Long

2	Discussion

The general overheating framework is that the UE indicates that it no longer experiences overheating by including an overheating assistance IE without any fields inside. This behaviour is handled explicitly e.g. as in NR SA:

(From TS 38.331)

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in OverheatingAssistance IE;

(From TS 38.331)

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in OverheatingAssistance IE;

However, it is not clear whether a similar principle is applicable to the SCG in EN-DC case, i.e. whether the SN should always be informed that there is no longer a preference for the SCG configuration concerning overheating. This was discussed in RAN2#112-e and no conclusion was reached:

R2-2010543	UE indication when it no longer experiences overheating	Ericsson	discussion	NR_newRAT-Core

-	[029] Rapporteur: Continue to discuss R2-2010543 in phase 2, including:

- Whether UE needs to inform network overheating of SCG is resolved? And whether absence of overheatingAssistanceForSCG field is sufficient for this purpose?

- Continue to discuss Proposal 2 to ensure all companies have the same understanding on inter-node operation.

[029] Noted

[029] Discuss by email to next meeting

[AT112-e][067][NR TEI16] UE indication when it no longer experiences overheating (Ericsson)

Scope: Based on R2-2010543, find solution, prepare for decisions next meeting

Intended outcome: Report

Deadline: Long

Furthermore, given the discussion in RAN2#112-e, it could be good to confirm also for NR-DC case whether companies have the same understanding on the SN aspects mentioned to EN-DC. Therefore, NR-DC is also discussed in this document.

2.1	EN-DC

In Rel-16, the UE in EN-DC may send an overheating indication concerning the SCG by including overheatingAssistanceForSCG within the OverheatingAssistance IE. If the UE would indicate that it no longer has a preference for the SCG concerning overheating, it could indicate it by including overheatingAssistanceForSCG without any fields therein, as done for the overall overheating framework (see the excerpt above). It should also be noted that the overall handling of 36.331 and 38.331 procedures may also result in an empty overheating assistance IE (see Annex A).

The current UE behaviour for EN-DC in Rel-16 can already result in the report of overheating including overheatingAssistanceForSCG without any fields therein.

Nevertheless, this behaviour is not consistently used since not always the UE will indicate to the SCG that it no longer has a preference for SCG concerning overheating. Such misalignment can be exemplified in the EN-DC cases below:

- UE indicates a preference for MCG and SCG configuration concerning overheating, but later it wants to express solely a preference for the MCG configuration concerning overheating while it no longer has a preference for SCG concerning overheating;

- UE indicates a preference for MCG and SCG configuration concerning overheating, but later it wants to express solely a preference for the SCG configuration concerning overheating while it no longer has a preference for MCG concerning overheating;

- UE indicates a preference for MCG and SCG configuration concerning overheating, but later it no longer experiences overheating (i.e. it would no longer have any preference for MCG or SCG concerning overheating);

For case A, the UE will, when it no longer has preferences for the SCG concerning overheating, send overheatingAssistanceForSCG (without any fields therein) within OverheatingAssistance IE.

For case B, it should result in sending OverheatingAssistance IE containing only overheatingAssistanceForSCG (which would contain the UE preferences for SCG regarding overheating), i.e. without including the fields that indicate preference for MCG.

But for case C, the UE will not include overheatingAssistanceForSCG within OverheatingAssistance IE, it will actually send OverheatingAssistance without any fields therein. Hence, it is not clear how the SN will be informed that the UE no longer has preferences for the SCG concerning overheating in case C, since the MN will not receive an overheatingAssistanceForSCG.

It is not clear in all cases for overheating in EN-DC how the SN is informed that the UE no longer has preferences for the SCG concerning overheating.

It should also be noted that if the SN is not informed that the UE no longer has preferences for the SCG concerning overheating (or it no longer experiences overheating at all), the SN will have no means to understand that there is no further restriction concerning overheating and may keep the UE with a downgraded SCG configuration.

If the SN is not informed that the UE no longer has preferences for the SCG concerning overheating, the SN may keep the UE with a downgraded SCG configuration.

To address the issue above concerning overheating for the SCG, the following solutions 1 and 2 were outlined in [2]. Another possibility (solution 3) is also described below.

- Solution 1:

- As long as the MN receives overheatingAssistanceForSCG IE from UE, MN should store and include overheatingAssistanceForSCG IE in every CG-ConfigInfo sent to SN.

- If the SN receives a CG-ConfigInfo which does not include overheatingAssistanceForSCG IE, the SN considers the UE no longer has preferences for the SCG concerning overheating (e.g. the MN sends CG-ConfigInfo without including overheatingAssistancForSCG because the MN has received from the UE an OverheatingAssistance IE without overheatingAssistancForSCG).

- Solution 2:

- When the MN receives OverheatingAssistance IE including overheatingAssistanceForSCG from UE. MN will forward it to SN, but the MN does not store it. This means for follow-up X2 messages, MN will not include overheatingAssistanceForSCG in CG-ConfigInfo unless it receives a new one from UE. This also implies that for the SN to be aware that the UE no longer has preferences for the SCG concerning overheating, whenever the UE no longer has such preferences, it should report to the MN overheatingAssistanceForSCG with no fields therein.

- Solution 3:

- When the MN receives OverheatingAssistance IE including overheatingAssistancForSCG from UE. MN will forward it to SN, but the MN does not store it. This means for follow-up X2 messages, MN will not include overheatingAssistanceForSCG in CG-ConfigInfo unless it receives a new one from UE. Furthermore, when the MN receives from the UE an OverheatingAssistance IE without any fields therein (i.e. the UE no longer experiences overheating), the MN includes a new field (to be introduced in 38.331) to indicate to the SN that the UE no longer has a preference for the SCG concerning overheating.

- Companies are also invited to provide further solutions.

Note that all the solutions would require further changes in 38.331(and in 36.331 in case of solution 2), i.e. both network behaviour and UE behaviour may need to be corrected. For instance, in case of solution 2, it should be noted that not always the SN currently gets the UE report of overheatingAssistanceForSCG as explained above, hence, if solution 2 is adopted the UE behaviour would have to be corrected to always include the SCG field overheatingAssistanceForSCG, when configured to provide overheating report to both MN and SN in EN-DC. The changes associated to each solution are provided in Annex B.

Depending on the solution adopted, both network behaviour and UE behaviour may need to be corrected.

Question 1: Which solution is preferred by companies?

| Company | Solution 1/2/3 | Comments |
| --- | --- | --- |
| Huawei, HiSilicon (Yiru) | Solution 2 or Solution 4 | Solution 1 is not preferred due to extra signaling overhead, Solution 2 is preferred compared with Solution 3, since the same logic is used, i.e. the SCG restriction concerning overheating is not needed anymore if the CG-ConfigInfo includes an empty overheatingAssistanceSCG-r16. But this solution introduces the NBC changes to UE. Besides, we provide a Solution 4: - When the MN receives OverheatingAssistance IE including overheatingAssistancForSCG from UE. MN will forward it to SN, but the MN does not store it. This means for follow-up X2 messages, MN will not include overheatingAssistanceForSCG in CG-ConfigInfo unless it receives a new one from UE. Furthermore, when the MN receives from the UE an OverheatingAssistance IE without any fields therein (i.e. the UE no longer experiences overheating), the MN generates a new CG-ConfigInfo including an overheatingAssistanceSCG-r16 with all subfields absent (an empty IE) to indicate to the SN that the UE no longer has a preference for the SCG concerning overheating. In this soluiton, the same logic between MN and SN for no preference on SCG configuration is used, and the impacts on the spec and UE behavior are minimized. There is another ASN.1 issue we just found, the IE overheatingAssistanceForSCG-r16 is not optional, OverheatingAssistance-v1610 ::= SEQUENCE { overheatingAssistanceForSCG-r16 OCTET STRING } which is obviously not aligned with the current procedural text: 3> do not include reducedUE-Category, reducedMaxCCs and overheatingAssistanceForSCG (if configured to provide overheating assistance indication for NR SCG) in OverheatingAssistance IE; It was optional field in the agreed CR R2-2006276 but the “optional“ was removed in Draft_36331-g10_v2 version. If Solution 2 is preferred, currrent ASN.1 seems OK. If Solution 4 is preferred, “optional“ needs to be added. As it impacts ASN.1 change, we are open to hear companies‘ views. |
| QCOM | Solution-2 | Solution-1 is inefficient Solution-3 seems as a work-around to avoid changes at the UE Solution-2 is preferred as it unified the overheating procedural behaivor for NR cells. |
| MediaTek | Solution-2 (with further clarifiocan) | Solution 2 seems more reasonable although it may have some NBC concerns. We however prefer to have unified rule that empty of the fields in IE OverheatingAssistance (within IE OverheatingAssistanceForSCG) implies that 1) UE does not suffer for overheating or 2) UE has no preference to limit SCG configuration even in overheating scenario. To clarify, we think Solution-2 implies that (in case overheating is not detected anymore) - UE to report LTE UE assitance information and include overheatingAssistanceForSCG which containes empty NR IE OverheatingAssistance - Empty implies that IE OverheatingAssistance exist without any optional sub-fields. Furthermore, we believe that the draft TP in Annex B for solution 2 should be further updated. The propsoed change in 36.331 should be 2> else (if the UE no longer experiences an overheating condition): 3> do not include reducedUE-Category, and reducedMaxCCs and overheatingAssistanceForSCG (if configured to provide overheating assistance indication for NR SCG) in OverheatingAssistance IE; 3> if configured to provide overheating assistance indication for NR SCG: 4> include overheatingAssistanceForSCG in the OverheatingAssistance IE; 4> set overheatingAssistanceForSCG in accordance with clause 5.7.4.3a as specified in TS 38.331 [82]; And the last else part in 38.331 should be 2> else (if the UE no longer experiences an overheating condition): 3> do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in OverheatingAssistance IE; |
| Ericsson | Solution 2 | We also would think it is better to keep the overheating principle consistent – this should also avoid problems in the future without having to handle two different mechanisms. |
| ZTE (LiuJing) | Solution 2 | We also prefer solution 2 as it makes the principle consistent for both Uu and X2 interfaces. Regarding Solution 4, we think it works from network implementation point of view. However, the overheatingAssistanceSCG-r16 field is defined as a container(OCTECT STRING), which means it is not supposed to be generated by MN, so solution 4 may be not spec compliant. In addition, for Solution 2, the proposed changes from MediaTek make sense to us. |

2.1	NR-DC

Given the discussion for EN-DC, it can be good to clarify whether companies have the same understanding on the overheating framework for NR-DC.

For NR-DC case, the UE sends an overheating report solely for the MN and based on this report the MN may trigger further coordination with the SN according to the fields below:

[[

p-maxNR-FR1-MCG-r16               P-Max                                                           OPTIONAL,

powerCoordination-FR2-r16         SEQUENCE {

p-maxNR-FR2-MCG-r16                P-Max                                                      OPTIONAL,

p-maxNR-FR2-SCG-r16                P-Max                                                      OPTIONAL,

p-maxUE-FR2-r16                    P-Max                                                      OPTIONAL

}                                                                                                 OPTIONAL,

nrdc-PC-mode-FR1-r16    ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}                OPTIONAL,

nrdc-PC-mode-FR2-r16    ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}                OPTIONAL,

maxMeasSRS-ResourceSCG-r16       INTEGER(0..maxNrofCLI-SRS-Resources-r16)                         OPTIONAL,

maxMeasCLI-ResourceSCG-r16       INTEGER(0..maxNrofCLI-RSSI-Resources-r16)                        OPTIONAL,

maxNumberEHC-ContextsSN-r16      INTEGER(0..65536)                                                OPTIONAL,

allowedReducedConfigForOverheating-r16      OverheatingAssistance                                 OPTIONAL,

maxToffset-r16                   T-Offset-r16                                                     OPTIONAL

]]

CG-ConfigInfo field descriptions

alignedDRX-Indication This field is signalled upon MN triggered CGI reporting by the UE that requires aligned DRX configurations between the MCG and the SCG (i.e. same DRX cycle and on-duration configured by MN completely contains on-duration configured by SN).

allowedBC-ListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select the SCG band combination. Each entry refers to: - a band combination numbered according to supportedBandCombinationList and supportedBandCombinationList-UplinkTxSwitch in the UE-MRDC-Capability (in case of (NG)EN-DC), or according to supportedBandCombinationList and supportedBandCombinationListNEDC-Only in the UE-MRDC-Capability (in case of NE-DC), or according to supportedBandCombinationList in the UE-NR-Capability (in case of NR-DC), - and the Feature Sets allowed for each band entry. All MR-DC band combinations indicated by this field comprise the MCG band combination, which is a superset of the MCG band(s) selected by MN.

allowedReducedConfigForOverheating Indicates the reduced configuration that the SCG is allowed to configure. reducedMaxCCs in allowedReducedConfigForOverheating indicates the maximum number of downlink/uplink PSCell/SCells that the SCG is allowed to configure. This field is used in (NG)EN-DC and NR-DC. reducedMaxBW-FR1 and reducedMaxBW-FR2 in allowedReducedConfigForOverheating indicates the maximum aggregated bandwidth across all downlink/uplink carriers of FR1 and FR2, respectively that the SCG is allowed to configure. This field is only used in NR-DC. reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in allowedReducedConfigForOverheating indicates the maximum number of downlink/uplink MIMO layers of each serving cell operating on FR1 and FR2, respectively that the SCG is allowed to configure. This field is only used in NR-DC.

Therefore, in NR-DC case, there is no overheating report sent from the MN to the SN, the MN simply may restrict the SN configuration using the fields above. Hence, the MN should always notify the SN about overheating restrictions towards the SCG with allowedReducedConfigForOverheating, which basically means that absence of such indication would imply that there is no restriction towards the SCG.

For NR-DC, there is no overheating report sent from the MN to the SN and thus the MN should notify the SN about overheating restrictions towards the SCG with allowedReducedConfigForOverheating.

Note, however, that the MN signals allowedReducedConfigForOverheating to the SN inside configRestrictInfo, and according to 38.331, the latter field is not always transmitted, i.e. absence of configRestrictInfo does not mean that there is no restriction towards the SN, the restrictions should be kept by the SN until a new

For the other fields in CG-Config and CG-ConfigInfo, the sender shall always signal the appropriate value even if same as indicated in the previous RRC INM, unless explicitly stated otherwise. As an exception to this general rule, the absence of the below listed fields means that the receiver maintains the values informed via the previous message. Note that every time there is a change in the configuration covered by a listed field, the MN shall include the field and it shall provide the full configuration provided by that field. Otherwise, if there is no change, the field can be omitted:

-	configRestrictInfo;

For the other fields in CG-Config and CG-ConfigInfo, the sender shall always signal the appropriate value even if same as indicated in the previous RRC INM, unless explicitly stated otherwise. As an exception to this general rule, the absence of the below listed fields means that the receiver maintains the values informed via the previous message. Note that every time there is a change in the configuration covered by a listed field, the MN shall include the field and it shall provide the full configuration provided by that field. Otherwise, if there is no change, the field can be omitted:

-	configRestrictInfo;

configRestrictInfo is provided to the SN.

Therefore, the overheating behavior for NR-DC should result in the understanding below:

- Once the MN sends CG-ConfigInfo>configRestrictInfo> allowedReducedConfigForOverheating to the SN, the SN should account for restrictions in the SCG concerning overheating. As long as the MN does not send any further CG-ConfigInfo to the SN containing the field configRestrictInfo, such restrictions concerning the SCG should still apply, including for overheating.

- If the SN receives a CG-ConfigInfo>configRestrictInfo which does not include allowedReducedConfigForOverheating, the SN considers the UE no longer has preferences for the SCG concerning overheating (e.g. the MN sends CG-ConfigInfo>configRestrictInfo without including allowedReducedConfigForOverheating because the MN has received from the UE an OverheatingAssistance IE without any fields therein).

The above is basically the principle of the overall restrictions signalled in configRestrictInfo. Hence, there is no change needed to account for it. If companies are aligned on this aspect, it may be sufficient to simply confirm this behaviour in meeting notes.

For overheating in NR-DC, the field allowedReducedConfigForOverheating should work in the same way as any other restrictions signaled within CG-ConfigInfo>configRestrictInfo.

Question 2: Do companies agree with the understanding above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | OK to simply confirm this behaviour in meeting notes. |
| QCOM | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| ZTE | Yes |  |

3	Conclusion

In the previous sections we made the following observations:

Observation 1	The current UE behaviour for EN-DC in Rel-16 can already result in the report of overheating including overheatingAssistanceForSCG without any fields therein.

Observation 2	It is not clear in all cases for overheating in EN-DC how the SN is informed that the UE no longer has preferences for the SCG concerning overheating.

Observation 3	If the SN is not informed that the UE no longer has preferences for the SCG concerning overheating, the SN may keep the UE with a downgraded SCG configuration.

Observation 4	Depending on the solution adopted, both network behaviour and UE behaviour may need to be corrected.

Observation 5	For NR-DC, there is no overheating report sent from the MN to the SN and thus the MN should notify the SN about overheating restrictions towards the SCG with allowedReducedConfigForOverheating.

Observation 6	For overheating in NR-DC, the field allowedReducedConfigForOverheating should work in the same way as any other restrictions signaled within CG-ConfigInfo>configRestrictInfo.

Based on the discussion in the previous sections we propose the following:

References

R2-2010543, UE indication when it no longer experiences overheating, Ericsson, RAN2#112-e, November 2-13

R2-2011176, [AT112-e][029][NR TEI16] Misc Corrections II, ZTE Corporation, RAN2#112-e, November 2-13

5	Annex

5.1	Annex A

UE behaviour for EN-DC in case it no longer experiences overheating for SCG in Rel-16

(From TS 36.331)

The UE shall set the contents of the UEAssistanceInformation message for overheating assistance indication:

1>	if configured to provide overheating assistance indication:

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce its DL category and UL category:

4>	include reducedUE-Category in the OverheatingAssistance IE;

4>	set reducedUE-CategoryDL to the number to which the UE prefers to temporarily reduce its DL category;

4>	set reducedUE-CategoryUL to the number to which the UE prefers to temporarily reduce its UL category;

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if configured to provide overheating assistance indication for NR SCG:

4>	include overheatingAssistanceForSCG in the OverheatingAssistance IE;

4>	set overheatingAssistanceForSCG in accordance with clause 5.7.4.3a as specified in TS 38.331 [82];

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedUE-Category, reducedMaxCCs and overheatingAssistanceForSCG (if configured to provide overheating assistance indication for NR SCG) in OverheatingAssistance IE;

(From TS 36.331)

The UE shall set the contents of the UEAssistanceInformation message for overheating assistance indication:

1>	if configured to provide overheating assistance indication:

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce its DL category and UL category:

4>	include reducedUE-Category in the OverheatingAssistance IE;

4>	set reducedUE-CategoryDL to the number to which the UE prefers to temporarily reduce its DL category;

4>	set reducedUE-CategoryUL to the number to which the UE prefers to temporarily reduce its UL category;

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if configured to provide overheating assistance indication for NR SCG:

4>	include overheatingAssistanceForSCG in the OverheatingAssistance IE;

4>	set overheatingAssistanceForSCG in accordance with clause 5.7.4.3a as specified in TS 38.331 [82];

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedUE-Category, reducedMaxCCs and overheatingAssistanceForSCG (if configured to provide overheating assistance indication for NR SCG) in OverheatingAssistance IE;

.

As depicted in the highlighted procedure above, the overheatingAssistanceForSCG may be included, but if there are no preferences for the SCG regarding overheating, the procedures in 5.7.4.3a in 38.331 will result in an overheatingAssistanceForSCG without any fields therein.

(From TS 38.331)

5.7.4.3a	Setting the contents of OverheatingAssistance IE

The UE shall set the contents of OverheatingAssistance IE if initiated to provide overheating assistance indication for SCG in (NG)EN-DC according to clause 5.6.10.3 as specified in TS 36.331 [10]:

1>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers for SCG:

2>	include reducedMaxCCs in the OverheatingAssistance IE;

2>	set reducedCCsDL to the number of maximum SCells of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedCCsUL to the number of maximum SCells of the SCG the UE prefers to be temporarily configured in uplink;

1>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1 for SCG:

2>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

2>	set reducedBW-FR1-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1 of the SCG;

2>	set reducedBW-FR1-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1 of the SCG;

1>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2 for SCG:

2>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

2>	set reducedBW-FR2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2 of the SCG;

2>	set reducedBW-FR2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2 of the SCG;

1>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1 for SCG:

2>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

2>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 of the SCG the UE prefers to be temporarily configured in uplink;

1>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2 for SCG:

2>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

2>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 of the SCG the UE prefers to be temporarily configured in uplink;

(From TS 38.331)

#### 5.7.4.3a	Setting the contents of OverheatingAssistance IE

The UE shall set the contents of OverheatingAssistance IE if initiated to provide overheating assistance indication for SCG in (NG)EN-DC according to clause 5.6.10.3 as specified in TS 36.331 [10]:

1>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers for SCG:

2>	include reducedMaxCCs in the OverheatingAssistance IE;

2>	set reducedCCsDL to the number of maximum SCells of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedCCsUL to the number of maximum SCells of the SCG the UE prefers to be temporarily configured in uplink;

1>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1 for SCG:

2>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

2>	set reducedBW-FR1-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1 of the SCG;

2>	set reducedBW-FR1-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1 of the SCG;

1>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2 for SCG:

2>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

2>	set reducedBW-FR2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2 of the SCG;

2>	set reducedBW-FR2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2 of the SCG;

1>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1 for SCG:

2>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

2>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 of the SCG the UE prefers to be temporarily configured in uplink;

1>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2 for SCG:

2>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

2>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 of the SCG the UE prefers to be temporarily configured in downlink;

2>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 of the SCG the UE prefers to be temporarily configured in uplink;

5.2	Annex B

Solution 1: Proposed changes for 38.331

CG-ConfigInfo field descriptions

overheatingAssistanceSCG Contains the UE's preference on reduced configuration for NR SCG to address overheating. This field is only used in (NG)EN-DC. The absence of this field indicates that the UE does not have a preference on reduced configuration for NR SCG to address overheating.

Solution 2: Proposed changes for 36.331

5.6.10.3	Actions related to transmission of UEAssistanceInformation message

******************* omitted unchanged parts ********************

The UE shall set the contents of the UEAssistanceInformation message for overheating assistance indication:

1>	if configured to provide overheating assistance indication:

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce its DL category and UL category:

4>	include reducedUE-Category in the OverheatingAssistance IE;

4>	set reducedUE-CategoryDL to the number to which the UE prefers to temporarily reduce its DL category;

4>	set reducedUE-CategoryUL to the number to which the UE prefers to temporarily reduce its UL category;

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if configured to provide overheating assistance indication for NR SCG:

4>	include overheatingAssistanceForSCG in the OverheatingAssistance IE;

4>	set overheatingAssistanceForSCG in accordance with clause 5.7.4.3a as specified in TS 38.331 [82];

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedUE-Category, reducedMaxCCs (if configured to provide overheating assistance indication for NR SCG) in OverheatingAssistance IE;

3>	include overheatingAssistanceForSCG in the OverheatingAssistance IE;

3>	set overheatingAssistanceForSCG in accordance with clause 5.7.4.3a as specified in TS 38.331 [82];

******************* omitted unchanged parts ********************

Solution 2: Proposed changes for 38.331

5.7.4.3a	Setting the contents of OverheatingAssistance IE

1>	if the UE experiences internal overheating:

2>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers for SCG:

3>	include reducedMaxCCs in the OverheatingAssistance IE;

3>	set reducedCCsDL to the number of maximum SCells of the SCG the UE prefers to be temporarily configured in downlink;

3>	set reducedCCsUL to the number of maximum SCells of the SCG the UE prefers to be temporarily configured in uplink;

2>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1 for SCG:

3>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

3>	set reducedBW-FR1-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1 of the SCG;

3>	set reducedBW-FR1-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1 of the SCG;

2>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2 for SCG:

3>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

3>	set reducedBW-FR2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2 of the SCG;

3>	set reducedBW-FR2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2 of the SCG;

2>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1 for SCG:

3>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

3>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 of the SCG the UE prefers to be temporarily configured in downlink;

3>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 of the SCG the UE prefers to be temporarily configured in uplink;

2>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2 for SCG:

3>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

3>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 of the SCG the UE prefers to be temporarily configured in downlink;

3>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 of the SCG the UE prefers to be temporarily configured in uplink;

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedUE-Category, reducedMaxCCs in the OverheatingAssistance IE;

Solution 3: Proposed changes for 38.331

–	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB. It can also be used by a CU to request a DU to perform certain actions, e.g. to request the DU to perform a new lower layer configuration.

Direction: Secondary gNB or eNB to master gNB or eNB, alternatively CU to DU.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

******************* omitted unchanged parts ********************

CG-Config-v1620-IEs ::=             SEQUENCE {

ueAssistanceInformationSCG-r16      OCTET STRING (CONTAINING UEAssistanceInformation)  OPTIONAL,

nonCriticalExtension                CG-Config-v16xy-IEs                                        OPTIONAL

}

CG-Config-v16xy-IEs ::=             SEQUENCE {

overheatingAbsenceSCG-r16            ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                        OPTIONAL

}

******************* omitted unchanged parts ********************

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

overheatingAbsenceSCG This field indicates that the UE no longer has a preference for NR SCG to address overheating.