3GPP TSG-RAN WG2 Meeting#113-e	Draft_R2-201xxxx

Online, 25th Jan. - 5th Feb. 2021

Agenda Item:	x.x.x.x

Source:	Huawei

Title:	[Draft] Summary of email discussion [351] (N)RSRP reference for TA validation for PUR

Document for:	Discussion and Decision

# Introduction

In RAN2#112-e, the following CR on the (N)RSRP reference for the first TA validation for PUR was discussed in offline [AT112-e][304].

R2-2009730	Clarification on the reference (N)RSRP for the first TA validation for PUR	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4480	-	F	NB_IOTenh3-Core, LTE_eMTC5-Core

- QC thinks the added description is clear from the procedure text. Huawei and Ericsson think the clarification is useful for the first TA validation. Ericsson thinks the wording could be improved.

Postponed

[AT112-e][304][NBIOT/eMTC R16] Clarification on the reference (N)RSRP for the first TA validation for PUR (Huawei)

Scope: Improve the wording of the change.

Intended outcome: Agreed CR in R2-2010909

Deadline: Tuesday 10th 1200 UTC

- Rapporteur reports there are different understandings and suggests an email discussion.

- QC agrees with this summary and think we should ensure a common understanding.

During the offline discussion, 4 cases were raised for the potential (N)RSRP reference update but there was no consensus on whether (N)RSRP reference should be updated in all the 4 cases. The following email discussion was assigned to further discuss the 4 cases and try to reach a common understanding:

[Post112-e][351][NBIOT/eMTC R16] (N)RSRP reference for the TA validation for PUR (Huawei)

Scope: To come to common understanding of the different cases

Intended outcome: Report and possibly CR to the next meeting

Deadline: Tuesday Jan 12 1100 UTC

# Discussion

For (N)RSRP based TA validation in PUR, whether TA should be considered as (re-)validated and the (N)RSRP reference should be updated in the following cases was discussed but there was no consensus:

- Case 1: Upon reception of RRC release message including pur-Config(-NB) in RRC_CONNECTED mode;

- Case 2: Upon reception of RRC release message in response to uplink transmission using PUR;

- Case 3: Upon reception of Timing Advance Command MAC CE;

- Case 4: Upon reception of (N)PDCCH indicates timing advance adjustment as specified in TS 36.212.

For each case, companies are invited to provide comments on whether TA should be considered as (re-)validated and the (N)RSRP reference should be updated. Based on the reply, companies are further invited to comment if anything needs to be clarified in the specification.

Case 1: Upon reception of RRC release message including pur-Config(-NB) in RRC_CONNECTED mode

Question 1a. For case 1, do you think TA should be considered as (re-)validated and the (N)RSRP reference updated? If yes, please also indicate whether it depends on the following cases raised in the offline discussion:

- whether it depends on pur-Config explicitly included (need ON)

- whether it depends on what is included in pur-Config. e,g, pur-RSRP-ChangeThreshold-r16 (Need ON) and/or pur-TimeAlignmentTimer-r16 (Need OR)

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | Yes | Thanks for the CR and all the discussion during offline in last meeting. Now we agree with QC’s comments mentioned in last meeting, e.g., if RSRP-based TA validation is configured (pur-RSRP-ChangeThreshold is set to “setup”), the RSRP reference should be updated by the UE after every TA validation. According to the current specification, the straightforward understanding (#1) could be that TA validation only means the actions in 5.3.3.19 which is only invoked when UE initiates transmission using PUR. But during the offline discussion, we can see other understanding (#2) that TA validation also means (re)acquiring valid TA or guaranteeing the validity of TA, e.g., TA value validation. With the following reason, we prefer the #2 understanding: - If the TA value is (re-)validated but RSRP reference is not updated, we can see the risk of inaccurate TA validation during the future PUR initiation. This may cause incorrect judgement on whether PUR can be initiated. In other words, when the UE performs the actions in 5.3.3.19 when it later initiates transmission using PUR, the stored RSRP reference doesn't correspond to the latest TA value. This would cause the calculated RSRP change cannot reflect the actual change of TA value. Therefore, we think a high level rule should be that, the RSRP reference value should be updated accordingly when the first time and every subsequent TA value validation (e.g., every time the TA timer (re)starts or the TA value is updated). Moreover, although pur-Config and pur-RSRP-ChangeThreshold are “need ON” type IE, because they use the definition of SetupRelease{}, they can still be set to release, which is equivalent to the case that the IE is absent for the IE type of “need OR”. With the above comments, the explicit description for our thinking on such high level rule can be as following: - If the pur-TimeAlignmentTimer IE is included in pur-Config(-NB) and the pur-TimeAlignmentTimer is not running and/or if the pur-RSRP-ChangeThreshold IE is set to “setup” in pur-Config(-NB) and there has no stored (N)RSRP reference, this can be seen as the first time TA value validation and the current serving cell RSRP value should be stored as RSRP reference. - If pur-TimeAlignmentTimer is already running, reception of TAC MAC CE (this triggers restart of pur-TimeAlignmentTimer) can be seen as another TA value validation and RSRP reference should also be updated. - If the pur-TimeAlignmentTimer is already running and the pur-TimeAlignmentTimer IE is included in pur-Config(-NB), as this triggers restart of pur-TimeAlignmentTimer, this also can be seen as another TA value validation and RSRP reference should also be updated. - If there already has stored (N)RSRP reference and only the pur-RSRP-ChangeThreshold IE is set to “setup” in pur-Config(-NB), we think this is not TA value validation. Therefore, the stored (N)RSRP reference should be maintained, e.g., not updated. (With reference to relaxed monitoring, the SrxlevRef would not be updated when s-SearchDeltaP is (re)provided in SIB). - If there already has stored (N)RSRP reference, and the pur-RSRP-ChangeThreshold IE is set to “release” in pur-Config(-NB), it can be seen as RSRP-based TA validation is not configured and the RSRP reference can be released, if stored. |
| Qualcomm | Yes, see comments | Thank you ZTE for the nice explanation. With respect to the ZTE response above: - While pur-TimeAlignmentTimer based and (N)RSRP-based TA validation are two independent TA validation procedures for PUR, we tend to agree with “If the pur-TimeAlignmentTimer is already running and the pur-TimeAlignmentTimer IE is included in pur-Config(-NB), as this triggers restart of pur-TimeAlignmentTimer, this also can be seen as another TA value validation and RSRP reference should also be updated”. - We disagree with “If there already has stored (N)RSRP reference and only the pur-RSRP-ChangeThreshold IE is set to “setup” in pur-Config(-NB), we think this is not TA value validation. Therefore, the stored (N)RSRP reference should be maintained, e.g., not updated.” So, in our view, upon reception of RRC release message including pur-Config(-NB) - When (N)RSRP based TA validation and/or pur-TimeAlignmentTimer is explicitly configured by current signalling: the (N)RSRP reference should be updated. - In the case of PUR reconfiguration where (N)RSRP based TA validation was previously configured but pur-RSRP-ChangeThreshold-r16 is not explicitly included in the current signalling: because of need ON, this means the (N)-RSRP based TA validation is still configured and previous threshold values still apply. On whether this should constitute a TA revalidation, while both options can work, we can consider this as the case of not revalidated, which means the network has the option to explicitly signal the value again if TA revalidation is desired. - If the pur-Config(-NB) explicitly releases (N)RSRP based TA validation by the current signalling, the discussion is moot. |
| Huawei, HiSilicon | Yes for some cases | We have similar understanding with the high level principle mentioned by ZTE. But we think one case has been missed: If only pur-RSRP-ChangeThreshold has been configured at the beginning, and then PUR reconfiguration is received by the UE in which pur-TimeAlignmentTimer IE is included in pur-Config(-NB), in this case, pur-TimeAlignmentTimer is started instead of re-started. We think RSRP reference should be updated for this case also. Based on above, we think RSRP reference should be updated when: - pur-RSRP-ChangeThreshold is configured for the first time (the case discussed in CR R2-2009730) - When PUR TA timer is (re-)started |

Question 1b. According to your reply to Question 1a, for case 1, is there anything to be clarified in current specification?

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | Yes | Based on the above comments for Q1a, we think the issue “it is not specified what is the reference value for the first TA validation” mentioned in CR [R2-2009730] exists and therefore clarification is needed. Based on the proposed change in CR[R2-2009730], we have the following example change (highlight yellow texts): pur-RSRP-ChangeThreshold Indicates the threshold(s) of change in serving cell RSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB and so on. When pur-RSRP-ChangeThreshold is set to setup, if decreaseThresh is absent the value of increaseThresh is also used for decreaseThresh. If there is no stored RSRP reference, tThe RSRP of the serving cell at the time the field is received is used as the reference for the first TA validation, see clause 5.3.3.19.. |
| Qualcomm | See Q2 |  |
| Huawei, HiSilicon | Yes | Agree with the change proposed by ZTE. |

Case 2: Upon reception of RRC release message in response to uplink transmission using PUR

Question 2a. For case 2, do you think TA should be considered as (re-)validated and the (N)RSRP reference updated?

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | - | In this case, if PUR TA timer is not restarted, RSRP reference also doesn’t need to be updated. But if PUR TA timer is restarted due to some conditions, RSRP reference needs to be updated. |
| Qualcomm | Depends | If the RRC release message includes pur-Config(-NB), the case is same as discussed in Q1 above. If the RRC release message does not include pur-Config(-NB), TA can be considered as not revalidated for the purpose of (N)RSRP based TA validation. |
| Huawei, HiSilicon | Depends | Agree with ZTE, if RSRP based TA validation has been configured and PUR TA timer is (re-)started, the RSRP reference should be updated. |

Question 2b. According to your reply to Question 2a, for case 2, is there anything to be clarified in current specification?

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | No |  |
| Qualcomm | Yes | From Q1 and Q2 response, one common criterion for considering the TA is revalidated is when pur-RSRP-ChangeThreshold-r16 is included and set to setup, and/or pur-TimeAlignmentTimer is included. |
| Huawei, HiSilicon | Yes | Changes are needed but have already been covered by our TP in Q1a. |

Case 3: Upon reception of Timing Advance Command MAC CE

Question 3a. For case 3, do you think TA should be considered as (re-)validated and the (N)RSRP reference updated?

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | Yes | According to our high level comments for Q1a, upon reception of TAC MAC CE, the TA value can be considered revalidated and therefore the RSRP reference should also be updated to the current serving cell RSRP value (if the pur-RSRP-ChangeThreshold is previously set to “setup”). We think this needs some clarification in MAC spec. |
| Qualcomm | Yes | Following is currently captured in MAC spec: when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5]: - apply the Timing Advance Command or the timing advance adjustment; So, reception of TA command MAC CE (Q3) OR reception of PDCCH indicates timing advance adjustment as specified in TS 36.212 (Q4) revalidates the TA value. Then for the (N)RSRP-based TA validation, “since the last TA validation” should be based on the comparison against “serving cell (N)RSRP” at such time. |
| Huawei, HiSilicon | Yes | In this case, TA value is updated at the UE and the (N)RSRP reference should be updated. |

Question 3b. According to your reply to Question 3a, for case 3, is there anything to be clarified in current specification?

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | Yes | According to our comments for Q1a and Q3a, we have the following example change: #### TS 36.321 #### 5.4.7.2 Maintenance of PUR Uplink Time Alignment …… - when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5]: - apply the Timing Advance Command or the timing advance adjustment; - start or restart the pur-TimeAlignmentTimer, if configured. - MAC entity indicates to upper layers the UE shall set the stored RSRP reference value to the current RSRP value of the serving cell.. |
| Qualcomm | Yes | The MAC spec text shown above covers both Q3 and Q4 and clearly indicates that TA is indeed revalidated. However, currently the TA revalidation due to MAC CE is based on MAC spec/layer, whereas (N)RSRP based validation is captured in RRC. There should be some kind of indication from MAC to RRC, for the case of (N)RSRP based TA validation is configured, to indicate that TA has been revalidated, so that RRC can update the reference. Such indication (from MAC to RRC) could be added in MAC spec in the same section shown above. So, we agree with ZTE’s intention with some wording suggestion: - when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5]: - apply the Timing Advance Command or the timing advance adjustment; - start or restart the pur-TimeAlignmentTimer, if configured. - indicate to upper layers that the Timing Advance value has been updated. Then RRC spec should also specify, among other things, handling of such MAC indication. See Q5. |
| Huawei, HiSilicon | Yes | We agree with the intention and also agree with Qualcomm that the following is enough as in RRC we already have “since the last TA validation, the serving cell (N)RSRP…”: - indicates to upper layers that the validated timing advance has been updated. |

Case 4: Upon reception of (N)PDCCH indicates timing advance adjustment as specified in TS 36.212

Question 4a. For case 4, do you think TA should be considered as (re-)validated and the (N)RSRP reference be updated?

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | Yes | This case is similar as case 3 in which TA value also can be considered revalidated and therefore the RSRP reference should also be updated. |
| Qualcomm | Yes | See Q3 |
| Huawei, HiSilicon | Yes | Same as case 3. |

Question 4b. According to your reply to Question 4a, for case 4, is there anything to be clarified in current specification?

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | Yes | See our comments for Q3b. |
| Qualcomm | Yes | See Q3 |
| Huawei, HiSilicon | Yes | See Q3b. |

Other clarification needed in the specification

Question 5 Regarding TA validation and (N)RSRP reference update, apart from the changes commented in above questions for the 4 cases, is there any other change needed in current specification?

| Company name | Opinion | Comments |
| --- | --- | --- |
| ZTE | No |  |
| Qualcomm | Yes | The MAC changes are shown above in Q3b. For RRC, based on above discussion, there are following conditions to be clarified. (It is better to clarify the full procedure in the procedural text than partly in a field description.) #### 5.3.3.19 Timing alignment validation for transmission using PUR If pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is configured, the UE shall store, or update if already stored, its serving cell reference (N)RSRP value when any of the following conditions is fulfilled: - upon reception of indication from lower layers that Timing Advance value has been updated; or - upon reception of RRCConnectionRelease message when pur-TimeAlignmentTimer is included; or - upon reception of RRCConnectionRelease message when pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is included and set to setup. The UE shall consider the timing alignment value for transmission using PUR to be valid when all of the following conditions are fulfilled: 1> if pur-TimeAlignmentTimer is configured: 2> pur-TimeAlignmentTimer is running as confirmed by lower layers; 1> if pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is configured: 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not increased by more than increaseThresh; and 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not decreased by more than decreaseThresh; |
| Huawei, HiSilicon | No | Above TP from Qualcomm could be an alternative for RRC if the case for pur-RSRP-ChangeThreshold reconfiguration is excluded in the following bullet: - upon reception of RRCConnectionRelease message when pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is included and set to setup. We think the RRC change proposed by ZTE and Huawei (update the field description only) is simpler, but we are also fine to go with majority view. |
| Qualcomm2 | See comments | It seems the main contention above is whether to update the (N)RSRP reference at the UE upon reconfiguration of the change threshold by the network. We fail to understand why network would provide a new change threshold but would not want the UE to compare that threshold with the newer (N)RSRP value but with the older one (which was stored at the time of first change threshold configuration). We should avoid the inconsistency. Why should the UE update the (N)RSRP reference threshold when it receives (arguably unrelated) PUR TAT value by RRC, but not the actual change threshold value by RRC itself which is in fact intended to be directly compared with the (N)RSRP reference? In fact, we would be ok to NOT update the (N)RSRP reference upon reception of PUR TAT value, because they are not directly related. Therefore, the text update in field description alone is not sufficient in RRC. After thinking a bit further, it seems, we can have a cleaner approach as below for RRC. (The suggested MAC changes are as shown in Q3b.) #### 5.3.3.19 Timing alignment validation for transmission using PUR The UE shall consider the timing alignment value for transmission using PUR to be valid when all of the following conditions are fulfilled: 1> if pur-TimeAlignmentTimer is configured: 2> pur-TimeAlignmentTimer is running as confirmed by lower layers; 1> if pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is configured: 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not increased by more than increaseThresh; and 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not decreased by more than decreaseThresh; .. #### 5.3.3.3d UE actions upon receiving PUR indications from lower layers The UE shall: 1> if repetition adjustment is indicated by lower layers: 2> update numRepetitions (npusch-NumRepetitionsIndex in NB-IoT) in previously stored pur-Config in accordance with the received indication; Upon reception of indication from lower layers that Timing Advance value has been updated, the UE shall: 1> if the UE is configured with pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT): 2> store, or update if already stored, the current serving cell (N)RSRP value as serving cell reference (N)RSRP value (see 5.3.3.19); For CP transmission using PUR, upon indication from lower layers that transmission using PUR is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. Upon reception of PUR fallback or PUR failure indication from lower layers, the procedure ends. NOTE: For transmission using PUR, further UE actions upon reception of PUR fallback or PUR failure indication from lower layers (see TS 36.321 [6]) is left up to implementation. .. #### 5.3.8.3 Reception of the RRCConnectionRelease by the UE The UE shall: <<skip>> 1> if the RRCConnectionRelease message includes the pur-Config: 2> if pur-Config is set to setup: 3> store or replace the PUR configuration provided by the pur-Config; 3> if pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is included and set to setup: 4> store, or update if already stored, the current serving cell (N)RSRP value as serving cell reference (N)RSRP value (see 5.3.3.19); 3> if pur-TimeAlignmentTimer is included in the received pur-Config: 4> configure lower layers in accordance with pur-TimeAlignmentTimer; 3> else: 4> if pur-TimeAlignmentTimer is configured, indicate to lower layers that pur-TimeAlignmentTimer is released; 3> start maintenance of PUR occasions as specified in 5.3.3.20; 2> else: 3> if pur-TimeAlignmentTimer is configured, indicate to lower layers that pur-TimeAlignmentTimer is released; 3> release pur-Config, if configured; 3> discard previously stored pur-Config; <<skip>> |

# Conclusion

This offline discussion focused on (N)RSRP based TA validation and (N)RSRP reference update for PUR:

TBD

# References

# Contact delegates

| Delegate | Company name | Email |
| --- | --- | --- |
| Baokun Shan | Huawei, HiSilicon | baokun.shan@huawei.com |
| Umesh Phuyal | Qualcomm | uphuyal@qti.qualcomm.com |