3GPP TSG-RAN WG2 #113 electronic	R2-210xxxx

Online, Jan 25 – Feb 5, 2021

Agenda Item:	6.4.3

Source:	LG (rapporteur)

Title:	Summary of email discussion [POST113-e][705][V2X] MAC impacts from the latest RAN1 decisions (LG)

Document for:	Discussion, Decision

Introduction

This is to kick off the following email discussion:

- [POST113-e][705][V2X/SL] MAC impacts from the latest RAN1 decisions (LG)

Scope: Discuss MAC impacts from the latest RAN1 decisions.

Intended outcome: Report, agreeable corresponding CRs and response LS (if needed)

Deadline: Long

Discussion

RAN1 agreement in RAN1#104-e (i.e. latest RAN1 decision)

Update on 1/28: Agreements: - The following value, OH, are used for the calculation of SL max data rate. - 0.25 for FR2 in SL - RAN1 sends an LS to RAN2 to inform the agreed overhead value for SL max data rate and also to fix the typo as below. - Qmis the maximum supported modulation order between 6 or 8 given by higher layer parameter sl-Tx-256QAM and sl-Rx-256QAM, OH for FR1? Check on 1/29 2/1 Agreements: - The following value, OH, are used for the calculation of SL max data rate. - 0.217 for FR1 in SL Update on 1/28: to check on 1/29 Agreements: Clarify in Clause 16.5.1.2 and 16.5.2.2 in TS 38.213 that an UL transmission resulting in DL/SL HARQ-ACK information multiplexed in PUSCH may be scheduled by DCI format 0_2. Agreements: The parameter pdsch-HARQ-ACK-Codebook is always used for reporting SL HARQ-ACK information. Agreements: Clarify that when DCI format 3_0 does not include the PSFCH-to-HARQ_feedback timing indicator field, the feedback slot is determined by sl-PSFCH-ToPUCCH-CG-Type1-r16 for CG type-1 and sl-PSFCH-ToPUCCH-r16 otherwise. Last proposal? 2/1 Agreements: - Send an LS to RAN2 describing that: - Per RAN1 agreements, the parameter sl-N1PUCCH-AN-r16 should be used for SL CG Type 2 (only for PSCCH/PSSCH transmissions without a corresponding PDCCH). - In RAN1’s understanding, the parameter cannot be configured for SL CG type-2 and asking RAN2 whether they can provide a solution that would be compatible with the existing RAN1 agreements. LS & CR? 2/2 Agreement: - Send an LS to RAN2 with the following agreement and asking them to make the appropriate changes to their specifications. Agreement: - The parameter pdsch-HARQ-ACK-Codebook is always used for reporting SL HARQ-ACK information.. Update on 1/28: Agreements: - UE does not expect the case when a PUSCH with no UCI overlaps with two non-overlapping PUCCHs each of which contains SL HARQ-ACK and Uu UCI. - No spec change is needed. Agreements: - PUCCH transmission for the response of MsgB and Msg4 is prioritized over SL transmission(s). Agreements: - For prioritization between SL PSFCH or S-SSB reception and PUCCH carrying SL HARQ reporting, - The PUCCH transmission has higher priority than a SL PSFCH or S-SSB reception if a priority value of the PUCCH is smaller than a priority value of the SL PSFCH or S-SSB reception. - If the priority value of the PUCCH transmission is larger than the priority value of the SL PSFCH or S-SSB reception, the SL PSFCH or S-SSB reception has higher priority. Proposal 1? 1/29 2/1 2/2 Update on 2/2: Agreement: In terms of prioritization between PUCCH with SL HARQ-ACK and PUCCH with UCI, the principle specified in 9.2.5.0 are used only. Update on 1/28: to check on 1/29 Update on 1/31: Agreement: - Alt 1 shown below is agreed.

After checking the latest RAN1 agreements above, Rapporteur thinks that there is no additional points (to be captured on MAC spec) other than those covered by on-going email discussion [1][2]. Just to be clear whether this understanding is correct, I would like to collect other companies’ views.

Question 2.1: Do you think that there are any MAC impacts from the RAN1 decision? If yes, please provide it.

| Company | Answer (yes or no) | Comments |
| --- | --- | --- |

Proposal:

Reference

[1] Email discussion of [POST113-e][707][V2X/SL] Spec update to level 3 logical slots (OPPO)

[2] Email discussion of [POST113-e][708][V2X/SL] How to handle DG for retransmissions? (OPPO)