3GPP TSG-RAN2 Meeting #113bis-e	R2-210xxxx

eMeeting, 12th – 20th April, 2021

Agenda Item:	5.4.3 UE capabilities and Capability Coordination

Source: 	Ericsson

Title:  	Summary of [Post113-e][051][NR15] DL scheduling slot offset

Document for:	Discussion and Decision

Introduction

The DL scheduling offset was discussed during RAN2#113-e [1,2], but no agreements were reached. It was decided to continue the discussion until RAN2#113bis-e:

[Post113-e][051][NR15] DL scheduling slot offset (Ericsson)

Scope: Continue discussion from [AT113-e][012] R2-2101731

Intended outcome: Report with Agreeable proposals

Deadline: Long

The deadline of the email discussion is Wednesday, 24th of March 2021, 11 UTC.

Contact information

| Company | Email |
| --- | --- |
| Ericsson | martin.van.der.zee@ericsson.com |
| ZTE | Li.wenting@zte.com.cn |

Background

K0>0 for System Information and Random Access

During the offline discussion #012 during RAN2#113-e [2] the use of K0>0 for System Information (SI) and Random Access (RA) was mentioned. It is the understanding of the rapporteur that the use of K0>0 for SI is not possible, i.e. the network does not know if the UE has IOT-tested K0>0 in case of SI, and there can always be legacy UEs that did not IOT-test this feature. The network can also not use K0>0 for RA when the UE comes from Idle mode and the network does not know the UE capabilities or during Contention Based Random Access (CBRA) when the network does not know which UE responds. But in case of Contention Free Random Access (CFRA)  in connected mode the network could use K0>0 with PDCCH/PDSCH transmissions when the UE has IOT-tested it.

The network can use K0>0 for Paging, when the (IOT) capability is signalled in the Paging message to the gNB. In case the UE(s) that are paged in the Paging Occasion (PO) have IOT-tested K0>0, then the network can safely use K0>0 in that PO. In case one or more UEs in the PO did not indicate to have IOT-tested K0>0, then the network cannot use K0>0 for that PO.

PDSCH configuration in 38.331

Up to 16 K0 values between 0-32 can be configured via pdsch-TimeDomainAllocationList in SIB1 for the initial BWP used for Paging, System Information and Random Access in PDSCH-ConfigCommon IE:

pdsch-TimeDomainAllocationList  PDSCH-TimeDomainResourceAllocationList  OPTIONAL,   -- Need R

PDSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation

PDSCH-TimeDomainResourceAllocation ::=   SEQUENCE {

k0                   INTEGER(0..32)                     OPTIONAL,   -- Need S

…

maxNrofDL-Allocations INTEGER ::= 16  -- Maximum number of PDSCH time domain resource allocations

The pdsch-TimeDomainAllocationList provides the possible K0 values the NW may use for the time between PDCCH and following PDSCH transmission. The actual value that is used in a particular PDCCH transmission, from the possible list in SIB1, is dynamically indicated in the PDCCH.

So for SI and CBRA the network would only use K0 is 0 values in the PDCCH transmissions. But for CFRA in connected mode the network could use a K0 > 0, if configured in the list and if the UE has indicated to have IOT-tested it. The intention is to enable similar flexibility for Paging by introducing the capabilities in the Paging message.

Discussion

Introduction of DL scheduling offset capabilities in Paging message

The proposal is to add dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB to the UERadioPagingInformation message so that the gNB can know if the UE has IOT-tested K0>0 for Paging:

UERadioPagingInformation ::= SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

ueRadioPagingInformation            UERadioPagingInformation-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-vxyz-IEs                       OPTIONAL

}

UERadioPagingInformation-vxyz-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1-r15     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1-r15     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-r15     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2-r15     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1-r15     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1-r15     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-r15     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2-r15     ENUMERATED {supported}          OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                 OPTIONAL

}

Issue 1: Do companies agree to add SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB to the UERadioPagingInformation message?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | We are not sure why these capabilities were not added from the beginning in REL-15, but perhaps the use of K0 > 0 was not considered in much details during that time period. @Nokia: - This proposal is not specifically for time multiplexing pattern2, nor specifically for FR2, i.e. the proposal is for both FR1 and FR2. This proposal is also not specifically for K0 = 1 but for values larger than 0 (in PDSCH-TimeDomainResourceAllocationList up to 32 slots can be configured for K0). We understand that the default table B for FR2 in 38.214 (Table 5.1.2.1.1-4) includes value K0 = 1, and therefore legacy UE supports K0 = 1 for FR2. This was discussed in previous RAN2 meeting, and companies think that the operator also for that case has to check that there is no IOT issue with legacy UEs. So we have come to understanding that the network can only use K0 > 0 when the UE explicitly indicates via the IOT capability bits that it has IOT-tested K0 > 0. - Thanks for spotting, i.e. I overlooked that. Please see updated ASN.1 above where the TDDx and FRx capabilities are explicitly signalled. |
| Huawei, HiSilicon | Yes | We share the view from rapporteur in the Background clause. We introduce IOT bit for paging, if the UE has IOT-tested K0>0 for paging, the network can safely use K0>0 for scheduling of paging. But for SI and RA (except for CFRA in connected mode), no specific IOT bit is introduced, so the network CANNOT use K0>0 for SI and RA in this case. In addition to the introduction of IOT capability for paging, it would be good to clarify (e.g. capture it in meeting minutes): The network CANNOT use K0>0 before knows that the UE has IOT-tested K0>0. |
| MediaTek | Yes |  |
| Nokia | Yes, but | - We just wanted to be sure that this is about pattern2 where the option to use K0=1 is supposed to be needed/mandatory if network would use the default Type0 CSS. The use case would be for FR2? - The capabilities are differentiated for FRx and xDD, but there seems to be only 1 bit allocated in the Radio Paging Information IE. Can you please explain the usage of this single bit in network for all the combinations? |
| ZTE | Yes | We share the same view as Ericsson and Huawei. |

Configuration of K0 > 0 in pdsch-TimeDomainAllocationList

It is the understanding of the rapporteur that a UE implementation supports the configuration of possible K0 values that the UE implementation might not have IOT-tested. In case the UE has not IOT-tested K0 > 0 then the network will not use K0 > 0 in PDCCH transmissions to that UE:

Issue 2: Do companies confirm that a UE that does not support dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB capability does support pdsch-TimeDomainAllocationList configuration including K0 values larger than 0?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | Companies are kindly invited to make a distinction between support/IOT-tested and configuration in this case. We agree that a UE may set dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB capability to false. But in our understanding any UE implementation should support the configuration of K0 > 0. @MDT & @Nokia: We agree that in general the NW should not configure a feature that the UE does not support. But we think that this case is a bit different, i.e. the NW configures the possible values the UE may use for both UEs supporting K > 0 and UEs not supporting K > 0. For UEs that do not support the feature, i.e. that have not IOT-tested K > 0, the NW will not use K0 > 0. |
| Huawei, HiSilicon | Yes | We understand UE can support the pdsch-TimeDomainAllocationList configuration including K0>0, but if the UE does not have IOT-tested K0>0 for paging, the network can only use K0=0 for scheduling of paging, even though the pdsch-TimeDomainAllocationList configuration includes K0>0. |
| MediaTek | No | We believe that the general principle is that NW does not configure a feature that is NOT supported (or NOT IOT tested). That’s the main reason to have UE capability reporting. We are also wondering why NW want to provide a list of possible K0 value but only use K0=0 in real scheduling. |
| Nokia | Yes, but | But we were also wondering about the scenario from MTK, would it really be the case that the network never uses anything else than K0=0 but still publishes a list on SIB1 indicating other values? How is that supposed to be used? |
| ZTE | Yes, but | We think the network can configure pdsch-TimeDomainAllocationList with K0>0, and then only use k0=0 for the UE that not IOT tested with K0>0. But, It depends on UE vendors understanding/implementation on whether the UE that not IOT tested with K0>0 can support K0>0 configuration (though K0>0 would not be used for such kinds of UE). |

Summary of email discussion

TBD

Conclusions

TBD

References

- R2-2101731, DL scheduling slot offset capability, Ericsson, Qualcomm, DISC, Rel-15, RAN2#113-e

- R2-2102374, Summary of [012][NR15] UE Capabilites IV, Huawei, Report, RAN2#113-e