# 3GPP TSG-RAN WG2 #113-e						draft R2-2102055

# e-meeting, 25th January – 5th February 2021			revised from R2_2102036

# Title:	[DRAFT] LS on UE location aspects in NTN

# Release:	Release 17

# Work Item:	NR_NTN_solutions-Core, 5GSAT_ARCH

Source:	Thales [to be RAN2]

To:	SA2, RAN3

Cc:	SA3, SA3-LI,

Contact Person:

Name:	Nicolas Chuberre

Tel. Number:

E-mail Address:	nicolas.chuberre@thalesaleniaspace.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

As part of the WI NR-NTN-solutions, RAN2 has been discussing how to meet SA3-LI and SA2 requirements with regards to regulatory services (including e.g. lawful intercept).

To specify the necessary NG-RAN and UE functions, RAN2 would like to point your attention to the following.

Core Network selection by RAN is assisted via information which currently includes:

- User Location Information (ULI) as determined by the RAN, which includes TAC and cell ID of the serving cell;

- Mobility measurements requested by RAN and reported by the UE, including e.g. neighbor cells (both terrestrial and NTN), inter-RAT measurements, WLAN measurements (as described in TSs 38.300 and 38.331);

- UE position, obtained by GNSS, based on measurements provided by the UE (as defined in TS 38.305).

- Question 1: RAN2 would like to ask RAN3, SA3-LI and SA2 to confirm whether the current functionality identified above is sufficient for use in Non-Terrestrial Networks.

2. Actions:

To SA WG2, RAN WG3, SA WG3 and SA WG3-LI.

ACTION: 	RAN2 respectfully asks SA2 and RAN WG3 to take the above into account and provide feedback on the above question.

3. Date of Next RAN2 Meetings:

RAN2#113-bis-e	12th – 20th April 2021	Electronic meeting

RAN2#114-e	19th – 27th May 2021	Electronic meeting