3GPP TSG-RAN WG2 Meeting #113bis-e 	R2-210xxxx

Online, 12th - 20th, April, 2021

Agenda Item:	9.1.3

Source: 	Huawei

Title:  	Summary of [Post113-e][351][NBIOT R17] Paging carrier selection

Document for:	Discussion and Decision

# Introduction

New Rel-17 work item on additional enhancements for NB-IoT and eMTC was approved at RAN#86-e and revised at RAN#88-e [1]. One of the objectives in the WID is to introduce carrier specific configuration:

- Introduce support for NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.). [NB-IoT] [RAN2, RAN3]

The following agreements were made for paging carrier selection improvements in the previous RAN2 meetings [2]:

- Paging carrier selection Improvements based on CE level is considered - Paging carrier selection Improvements based on DRX cycle may be considered - whether DRX cycle is considered as part of CE level (Rmax) or can be also considered separately - Paging carrier selection Improvements solely based on WUS or GWUS is not considered - FFS service based - Select between one of the options: - Option 1: UE selects a paging carrier based on a rule configured by the network - Option 2: NW configures a specific paging carrier - Working assumption: For both options, when coverage changes, mechanism that requires UE to report the update of coverage is not introduced.

The following email discussion is to further discuss the details and pros/cons of the two options for paging carrier selection:

- [Post113-e][351][NBIOT/eMTC R17] Paging carrier selection (Huawei)

Scope: Details and pros and cons of the 2 options.

Intended outcome: Report to the next meeting.

Deadline: Friday March 26 1100 UTC

# Discussion

This document focuses on the details and pros / cons of the following 2 options:

    - Option 1: UE selects a paging carrier based on a rule configured by the network

    - Option 2: NW configures a specific paging carrier

In offline summary R2-2102155 [3], for the pros/cons of the 2 options, the following aspects were mentioned:

- Whether coverage based paging carrier selection is restricted to the last known cell

- Coverage information that can be used to determine paging carrier

- Power boosting on different paging carrier

- Load balancing

- Combination with DRX cycle based paging carrier selection

- Fallback scenario

- Complexity / Standard impacts

## Last known cell or not

In offline summary R2-2102155 [3], one essential aspect is whether coverage based paging carrier selection is restricted only to the last know cell or not.

Question 1: Whether the coverage based paging carrier selection is restricted only to the last know cell or not and whether this is different for the two options?

| Company name | Yes/No | Comments |
| --- | --- | --- |

## Coverage information used to determine the paging carrier

In offline summary R2-2102155 [3], it was commented that in option 2, the eNB can consider a number of different aspects when assigning a paging carrier to the UE, e.g.:

- UE capability

- DRX cycle support

- Coverage information

- What sort of service the UE typically requires

- Carrier power boost information

- Carrier specific interference levels (that may be dynamic)

- UE differentiation information (e.g. battery operated, traffic profile, …)

- Load balancing between UEs in the cell (Rel-13 vs 14/15/16 vs ≥17)

Taking all these criteria into consideration is not possible for option 1, thus, it is a pro of option 2 (con of option 1).

Question 2: Do you agree that the eNB can consider a number of different aspects when assigning a paging carrier to the UE in option 2 and this can be consider as a pro of option 2 (con of option 1)?

| Company name | Yes/No | Comments |
| --- | --- | --- |

## Power boosting on different paging carriers

In offline summary R2-2102155 [3], it was commented that different paging carriers may have different power level due to power boosting. In this case, for the same coverage, two paging carriers with different power level may have different Rmax.

Question 3: Do you agree that different power level on different paging carrier should be considered when selecting paging carrier? If yes, which option can achieve this better?

| Company name | Yes/No | Option | Comments |
| --- | --- | --- | --- |

## Load balancing

In offline summary R2-2102155 [3], for load balancing among paging carriers, companies have different views on the two options. Some companies think that Option 2 is more flexible but some companies think that in Option 2 it is difficult for eNB to equally distribute UEs with same CEL to different paging carriers via dedicated signalling.

Question 4: Which option can achieve load balancing between different paging carriers easier /better? Why?

- Option 1

- Option 2

- Equal

| Company name | Option | Why? |
| --- | --- | --- |

## Combination with DRX cycle based paging carrier selection

In offline summary R2-2102155 [3], the combination between coverage based and DRX cycle based paging carrier selection were commented. For both options, there were solutions to support DRX cycle based paging carrier selection:

- For Option 1, 2-level carrier selecting rule can be considered, e.g. the UE selects a set of paging carriers which can satisfy its coverage requirement and then selection a paging carrier based on DRX cycle.

- For Option 2, the eNB can take UE specific DRX cycle into consideration when assigning the paging carrier to the UE.

Question 5: Which option can achieve DRX cycle based paging carrier selection easier/better? Why?

- Option 1

- Option 2

- Equal

| Company name | Option | Why? |
| --- | --- | --- |

## Fallback scenario

In offline summary R2-2102155 [3], fallback scenario (when coverage/serving cell changes) need to be considered for both options.

Question 6a: Which option can address fallback scenario caused by serving cell change easier/better? Why?

- Option 1

- Option 2

- Equal

| Company name | Option | Why? |
| --- | --- | --- |

Question 6b: Which option can address fallback scenario caused by coverage change easier/better? Why?

- Option 1

- Option 2

- Equal

| Company name | Option | Why? |
| --- | --- | --- |

## Complexity / Standard impacts

In offline summary R2-2102155 [3], complexity / standard impacts was discussed.

Question 7: Which option is simpler and / or has smaller standard impacts? Companies are encouraged to list potential complexity and standard impacts

- Option 1

- Option 2

- Equal

| Company name | Option | Potential complexity and standard impacts |
| --- | --- | --- |

## Other

Please provides other pros/cons for both options, if any.

| Company name | Option | Comments |
| --- | --- | --- |

# Conclusion

This paper focused on coverage based paging carrier selection improvements. Corresponding proposals are listed as follows:

TBD

# References

- RP-201306, “Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88-e, Online, June 29 - July 3, 2020

- R2-2102164, “RAN2 agreements for Rel-17 NB-IoT and LTE-MTC”, Ericsson, RAN2#113e, Online, Jan 25 - Feb 05, 2021

- R2-2102155, “Summary of [AT113-e][305][NBIOT R17] Paging carrier selection improvements”, Huawei, RAN2#113e, Online, Jan 25 - Feb 05, 2021

# Contact delegates

| Delegate | Company name | Email |
| --- | --- | --- |
| Baokun Shan | Huawei, HiSilicon | baokun.shan@huawei.com |