3GPP TSG-RAN WG2 Meeting #113bis-e	R2-21xxxxx

# Online, 12 – 20 April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on parameters of SL configured grant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO(rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2020-03-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - RAN2 agreed that “The equation to define CG resource slot should be defined based on Level_3 logical slots i.e. logical slots within one resource pool” in RAN2#113e meeting. The two parameters i.e. sl-TimeOffsetCG-Type1 and sl-TimeReferenceSFN-Type1 should be redefined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To field description of parameter sl-TimeOffsetCG-Type1 and sl-TimeReferenceSFN-Type1 are clarified based on level 3 logical slots. Impact analysis Impacted 5G NR SL: NR SA Impacted functionality: SL mode 1 operation Inter-operability: If one UE is implemented according to this CR while the other UE is not, there maybe resource collision among TX UEs. If UE implements according to the CR and the network does not, the CG resource allocation is misaligned between TX UE and network. If the network implements according to the CR and the UE does not, the CG resource allocation is misaligned between TX UE and network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF THE 1st CHANGE

#### –	SL-ConfiguredGrantConfig

The IE SL-ConfiguredGrantConfig specifies the configured grant configuration information for NR sidelink communication.

SL-ConfiguredGrantConfig information element

-- ASN1START

-- TAG-SL-CONFIGUREDGRANTCONFIG-START

SL-ConfiguredGrantConfig-r16 ::=           SEQUENCE {

sl-ConfigIndexCG-r16                       SL-ConfigIndexCG-r16,

sl-PeriodCG-r16                            SL-PeriodCG-r16                                                       OPTIONAL, -- Need M

sl-NrOfHARQ-Processes-r16                  INTEGER (1..16)                                                       OPTIONAL, -- Need M

sl-HARQ-ProcID-offset-r16                  INTEGER (1..16)                                                       OPTIONAL, -- Need M

sl-CG-MaxTransNumList-r16                  SL-CG-MaxTransNumList-r16                                             OPTIONAL, -- Need M

rrc-ConfiguredSidelinkGrant-r16            SEQUENCE {

sl-TimeResourceCG-Type1-r16                INTEGER (0..496)                                                  OPTIONAL, -- Need M

sl-StartSubchannelCG-Type1-r16             INTEGER (0..26)                                                   OPTIONAL, -- Need M

sl-FreqResourceCG-Type1-r16                INTEGER (0..6929)                                                 OPTIONAL, -- Need M

sl-TimeOffsetCG-Type1-r16                  INTEGER (0..7999)                                                 OPTIONAL, -- Need R

sl-N1PUCCH-AN-r16                          PUCCH-ResourceId                                                  OPTIONAL, -- Need M

sl-PSFCH-ToPUCCH-CG-Type1-r16              INTEGER (0..15)                                                   OPTIONAL, -- Need M

sl-ResourcePoolID-r16                      SL-ResourcePoolID-r16                                             OPTIONAL, -- Need M

sl-TimeReferenceSFN-Type1-r16              ENUMERATED {sfn512}                                               OPTIONAL  -- Need S

}                                                                                                                OPTIONAL, -- Need M

...

}

SL-ConfigIndexCG-r16 ::=          INTEGER (1..maxNrofCG-SL-r16)

SL-CG-MaxTransNumList-r16 ::=     SEQUENCE (SIZE (1..8)) OF SL-CG-MaxTransNum-r16

SL-CG-MaxTransNum-r16 ::=                  SEQUENCE {

sl-Priority-r16                            INTEGER (1..8),

sl-MaxTransNum-r16                         INTEGER (1..32)

}

SL-PeriodCG-r16 ::=            CHOICE{

sl-PeriodCG1-r16               ENUMERATED {ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000, spare6,

spare5, spare4, spare3, spare2, spare1},

sl-PeriodCG2-r16               INTEGER (1..99)

}

-- TAG-SL-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

SL-ConfiguredGrantConfig field descriptions

sl-ConfigIndexCG This field indicates the ID to identify configured grant for sidelink.

sl-CG-MaxTransNumList This field indicates the maximum number of times that a TB can be transmitted using the resources provided by the configured grant. sl-Priority corresponds to the logical channel priority.

sl-FreqResourceCG-Type1 Indicates the frequency resource location of sidelink configured grant type 1. An index giving valid combinations of one or two starting sub-channel and length (joinly encoded) as resource indicator (RIV), as defined in TS 38.214 [19].

sl-N1PUCCH-AN This field indicates the HARQ resource for PUCCH for sidelink configured grant type 1. The actual PUCCH-Resource is configured in sl-PUCCH-Config and referred to by its ID.

sl-NrOfHARQ-Processes This field indicates the number of HARQ processes configured for a specific configured grant. It applies for both Type 1 and Type 2.

sl-PeriodCG This field indicates the period of sidelink configured grant in the unit of ms.

sl-PSFCH-ToPUCCH-CG-Type1 This field, for configured grant type 1, indicates slot offset between the PSFCH associated with the last PSSCH resource of each period and the PUCCH occasion used for reporting sidelink HARQ.

sl-ResourcePoolID Indicates the resource pool in which the configured sidelink grant Type 1 is applied.

sl-StartSubchannelCG-Type1 This field indicates the starting sub-channel of sidelink configured grant Type 1. An index giving valid sub-channel index.

sl-TimeOffsetCG-Type1 This field indicates the slot offset related to logical slot defined by sl-TimeReferenceSFN-Type1, as specified in TS 38.321 [3].

sl-TimeReferenceSFN-Type1 Indicates SFN used for determination of the offset of a resource in time domain. If it is present, the UE uses the 1st logical slot after closest SFN with the indicated number preceding the reception of the sidelink configured grant configuration Type 1, see TS 38.321 [3], clause 5.8.3. If it is not present, the reference SFN is 0.

sl-TimeResourceCG-Type1 This field indicates the time resource location of sidelink configured grant Type 1. An index giving valid combinations of up to two slot positions (jointly encoded) as time resource indicator (TRIV), as defined in TS 38.212 [17].

END OF THE 1st CHANGE