3GPP TSG-RAN WG2 Meeting #114-e	draftR2-2106602

Electronic Meeting, May 19th - 27th 2021

Source: 	Document Rapporteur (Ericsson)

Title:	RAN2 agreements for Rel-17 NB-IoT and LTE-MTC

Agenda Item:	9.1.1

Document for:	Endorsement

# 1	Introduction

This document lists RAN2 agreements made for Rel-17 “Additional enhancements for NB-IoT and LTE-M” work item (WI code NB_IOTenh4_LTE_eMTC6; WID in RP-201306), until and including the RAN2#114-e, May 19th – 27th 2021.

NOTE:	Email agreements for CRs/specific topics are not listed in this document.

# 2	Additional enhancements for NB-IoT and LTE-MTC

## 2.1	Organizational

RAN2#111-e agreements: Will maintain a document similar to the one used in R16 for capturing agreements. The endorsed report can be provided in R2-2008309. [Post111-e][350][NBIOT/eMTC R17] Capture the agreements (Ericsson) - Scope: Capture the agreements. - Intended outcome: endorsed report in R2-2008309 - Deadline: Friday, 2020-09-04 13:00 UTC RAN2#112-e agreements: None (except for this document, see clause 5) RAN2#113-e agreements: None (except for this document, see clause 5) RAN2#113bis-e agreements: None (except for this document, see clause 5) RAN2#114-e agreements: None (except for this document, see clause 5)

## 2.2	NB-IoT neighbour cell measurements and corresponding measurement triggering before RLF

RAN2#111-e agreements: Study current RLF procedure to understand the time taken to select the cell for access. Start and end points FFS. Support neighbour cell measurements in RRC_CONNECTED, at least for intra-frequency. - FFS inter-frequency - FFS whether measurements are done on the anchor carrier - FFS how neighbour cell measurement is triggered - FFS how to perform neighbour cell measurements Working assumption: Neighbour cell measurement results are not reported to the network in RRC_CONNECTED. - FFS whether and when other information can be sent RAN2#112-e agreements: Enhancements to the random-access procedure are not considered. The solution includes reduction of the time between declaration of RLF and the start of the random-access procedure (points C and D) FFS whether the solution includes reduction of the time between out-of-sync detection and declaration of RLF (points B and C) RAN2#113-e agreements: Neighbour cells measurement (detection and measurements) are performed only on the anchor carrier. The solution is optional R2-2102165 LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state LS out Rel-17 To: RAN4. NB_IOTenh4_LTE_eMTC6-Core RAN2#113bis-e agreements: None RAN2#114-e agreements: The criteria to start measurements is based on a combination of serving cell quality threshold (option b) and variance of the serving cell quality (option c) Configuration of the criteria to start the measurements is supported. FFS whether any further information needs to be provided by NW FFS whether any assistance information from UE is needed. FFS if/how to support ‘early’ RLF.

## 2.3	NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration

RAN2#111-e agreements: Paging carrier selection Improvements based on CE level is considered Paging carrier selection Improvements based on DRX cycle may be considered - whether DRX cycle is considered as part of CE level (Rmax) or can be also considered separately Enhancements for NPRACH Carrier selection carrier may be considered Paging carrier selection Improvements solely based on WUS or GWUS is not considered FFS service based RAN2#112-e agreements: None RAN2#113-e agreements: Select between one of the options: - Option 1: UE selects a paging carrier based on a rule configured by the network - Option 2: NW configures a specific paging carrier Working assumption: For both options, when coverage changes, mechanism that requires UE to report the update of coverage is not introduced. RAN2#113bis-e agreements: None RAN2#114-e agreements: Rel-17 paging carriers and the legacy paging carriers should be exclusive. RAN2 assumes S1AP/NGAP update is not needed. Carrier selection criteria does not include power boosting or service FFS: For option 1, whether DRX can be part of the carrier selection criteria Rel-17 paging carrier configuration is provided in broadcast signalling. Select between the following sub-options: Option 1c: Network enables UE to select a Rel-17 paging carrier by providing the coverage information (CEL/Rmax) for the carrier selection to the UE in dedicated signalling Option 2a: NW indicates the carrier to use explicitly via dedicated signalling based on information determined within the NW. - FFS for both options whether there is a report from the UE to suggest a carrier or provide a metric report Working assumption: UE metric for determining carrier suitability and selection is based on measured NRSRP. FFS whether to use a hysteresis/longer averaging/timer For option 1, upon cell change, FFS: Alt 1: based on previously determined CEL and broadcasted paging carrier configuration in the new cell. Alt 2: UE needs to perform fallback mechanism. For option 2, upon cell change, UE needs to perform fallback mechanism. Whenever the R17 coverage based carrier criteria is met, UE uses the R17 coverage based carrier, otherwise UE should use the fallback mechanism For both options, fall back carrier is legacy paging carrier based on UE_ID

## 2.4	Other

### NB-IoT 16-QAM for unicast in UL and DL

RAN2#113bis-e agreements: Working assumption: For the UE supporting 16-QAM, the L2 buffer size is 12000 bytes. Working assumption: Support of 16-QAM has separate UE capabilities for DL and UL RAN2#114-e agreements: None

### LTE-MTC 14 HARQ processes in DL for HD-FDD Cat M1 UEs

RAN2#113bis-e agreements: 14 HARQ activation is configured by dedicated RRC signalling. Working assumption: No change to current L2 buffer size requirement RAN2#114-e agreements: None

### LTE-MTC Max DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only

RAN2#113bis-e agreements: DL TBS of 1736 bits is configured by dedicated RRC signalling. FFS: Whether to update L2 buffer size requirement RAN2#114-e agreements: None

# 3	References: List of approved outgoing Rel-17 LSs

| Tdoc | Meeting | Title | WI | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-2102165 | RAN2#113-e | LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state | NB_IOTenh4_LTE_eMTC6-Core | RAN4 |  |

# 4	References: Reports from the breakout session

| Meeting | Date | Title | Tdoc |
| --- | --- | --- | --- |
| RAN2#111-e, Online | 17-28 August 2020 | Report NB-IoT breakout session | R2-2008127 |
| RAN2#112-e, Online | 2-13 November 2020 | Report NB-IoT breakout session | R2-2010707 |
| RAN2#113-e, Online | Jan 25 - Feb 5 2021 | Report NB-IoT breakout session | R2-2101957 |
| RAN2#113bis-e, Online | 12 - 20 April 2021 | Report NB-IoT breakout session | R2-2104307 |
| RAN2#114-e, Online | 19 – 27 May 2021 | Report NB-IoT breakout session | R2-2106477 |

# 5	Change history

| Tdoc | Title | Comment |
| --- | --- | --- |
| R2-2008309 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#111-e |
| R2-2010911 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#112-e |
| R2-2102164 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#113-e |
| R2-2104451 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#113bis-e |
| R2-2106602 | RAN2 agreements for Rel-17 additional enhancements for NB-IoT & LTE-MTC | Post RAN2#114-e |