3GPP TSG RAN WG1 #114-e			R2-210xxxx

e-Meeting, May 19th – 27th, 2021

Title:	LS on On-demand PRS

Response to:

Release:	Rel-17

Work Item:	NR_pos_enh

Source:	RAN2

To:	RAN3

Cc:	RAN1

Contact Person:

#### Name: Ritesh Shreevastav

E-mail Address: ritesh.shreevastav[at]ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

During RAN2 #113bis-e and #114-e, the following agreements regarding On-Demand PRS in Rel-17 were reached.

RAN2 #113bis-e

Agreements:

UE-initiated on-demand PRS request is enabled by enhancing LPP RequestAssistanceData.  FFS how much control the network has over the UE request.

The UE-initiated mechanism is enabled by the UE request triggering a request from the LMF, and the actual PRS changes are requested by the LMF irrespective of whether the procedure is UE- or LMF-initiated.

Put the stage 2 description for UE-initiated and LMF-initiated PRS request under the same framework.

RAN2 #114-e

Agreement:

The network can signal predefined PRS configurations to the UE and the UE can select one to request.  FFS if the UE can request a configuration with different parameters and exactly which parameters are flexible.

Define a new LPP assistance data IE which can contain a set of possible on-demand DL-PRS configurations, where each on-demand DL-PRS configuration has an associated identifier.

This new LPP assistance data IE can be included in an LPP Provide Assistance Data message and/or a new posSIB.

The procedure(s) for on-demand DL-PRS should support at least the following functionality (up to RAN3 what is in NRPPa vs. OAM, etc.):

-	Providing the requested on-demand DL-PRS configuration information from an LMF to the gNB (e.g., explicit parameter or identifier of a predefined DL-PRS configuration), and confirmation of the request by the gNB

-	Provision of (possible/allowed) on-demand DL-PRS configurations that the gNB can support from a gNB to an LMF

-	TRP capability transfer (e.g., whether the RAN node supports the reconfiguration of DL-PRS, etc.)

The agreements in RAN2#114-e applies to both UE-initiated and LMF-initiated on-demand PRS.

2. Actions:

To RAN3

ACTION: 	RAN2 respectfully requests RAN3 to take the above RAN2 agreements into account.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #115-e	16 – 27 August 2021	E-Meeting

TSG-RAN WG2 Meeting #116-e	1 – 12 November 2021	E-Meeting