3GPP TSG-RAN WG2 Meeting #116 electronic	               R2-21xxx

Electronic meeting, 1st-12th November 2021

Agenda Item:	8.4.3

Source: 	Huawei, HiSilicon

Title:	Summary of [Post115-e][088][eIAB] inter-CU routing open issues

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of email discussion.

- [Post115-e][088][eIAB] inter-CU routing open issues (Huawei)

Scope: Address the listed open points for inter-CU routing:

- What’s the BAP address added in BAP header in the first topology (i.e. the BAP address of ingress data at the boundary node);

- How to differentiate the concatenated traffic and non-concatenated traffic;

- How to determine whether a data should be delivered to upper layer (for downstream);

- How to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology).

Intended outcome: Report

Deadline: Long

1st round deadline: initial comments collection. October 12th, 0900 UTC

2nd round deadline: one week for proposal checking and TP/Running CR discussion if possible. October 21th, 0900 UTC

RAN2 related agreements:

- For inter-donor-DU re-routing, support the “previous routing ID to new routing ID” BAP header rewriting. - For intra-CU cases, Support inter-donor-DU re-routing at least in the scenarios of NR-DC among donor-DUs, inter-donor-DU recovery and inter-donor-DU migration. - Support inter-CU re-routing, i.e. IAB-node re-routes the data to its original donor-CU via the alternative BAP path over the topology in target CU. - As baseline, support the 1:1 and N:1 mapping from “previous routing ID” to “new routing ID” for BAP header rewriting at the boundary node, in inter-CU routing. - As baseline, support the 1:1 and N:1 mapping from “ingress BH link + ingress BH RLC ID” to “egress BH link + egress BH RLC ID” for bearer mapping at the boundary node, in inter-CU routing.

RAN3 related agreements:

- One common inter-donor topology transport mechanism should be defined for all scenarios where traffic between a donor and an IAB DU traverses the network under another donor; FFS whether it is possible to achieve a common signaling design for all scenarios - RAN3 prefers that the boundary node processes access traffic in the same manner as the non-boundary access IAB-node. - RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic. - FFS: In addition to BAP header rewriting, performs routing and bearer mapping in the same manner as the non-boundary intermediate IAB-node. - RAN3 assumes that the boundary node has only one BAP address in each topology. - RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers. - For DL traffic, the configurations of BAP routing entry and BAP-routing-ID mapping at the boundary node need to indicate the ingress topology they refer to. For UL traffic, they need to indicate the egress topology they refer to. The indications may be implicit.

# Discussion

- 2.1 Terminologies clarification

Terminologies to be used in this email discussion:

Inter-CU topology redundancy        Inter-CU partial migration

Pleases note the terminology clarification is only for the scope of email discussion, not for the specification usage.

Boundary IAB node: IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a parent DU.

Alternative modelling:

Boundary IAB node (upstream): IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a parent DU. (IAB-2 in the figure) (same as above)

Boundary IAB node (downstream): IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a child DU. (IAB-3 in the figure)

First topology: the topology fragment before the boundary node for a traffic. (For example, the 1st topology is IAB-2 and IAB-4 for upstream; donor-DU2, IAB-3 and IAB-2 for downstream [blue nodes].)

Second topology: the topology fragment after the boundary node for a traffic. (For example, the 2nd topology is IAB-2, IAB-3 and donor-DU2 for upstream; IAB-2 and IAB-4 for downstream [green nodes].)

Concatenated traffic: the traffic routing across two topologies which belong to different CUs.  [Traffic that is carried across the blue topology and processed on the BAP layer of the boundary node.]

(For example the brown line and purple line in above figure). Please note this terminology includes both descendant node traffic and access traffic. You can understand this as the traffic from/to neighbor CU2’s topology, which needs to be handled by boundary node.

Non-concatenated traffic: [Traffic that is carried across the green topology only and processed on the BAP layer of the boundary node.] the traffic routing across one topology which belong to one CU. (For example the orange line and red line in above figure, as supported in R16). Please note this terminology includes both descendant node traffic and access traffic. You can understand this as the traffic from/to CU1’s topology, which needs to be handled by boundary node.

Inter-topology routing: the BAP routing across two topologies which belong to different CUs, i.e. the “inter-CU routing” used before.

Inter-topology re-routing: BAP  re-routes the data (finally terminated to its original donor-CU) via the alternative BAP path over the topology in target CU, i.e. the “inter-CU re-routing” used before.

- 2.2 Unified BAP operations

The goal of this section is to design the solution for BAP operations to cover inter-topology routing, inter-topology re-routing and inter-donor-DU re-routing. We strive to comprehensive solution for those 3 cases and strive to similar solution for upstream and downstream, if possible.

### 2.2.1 Assumption and modelling clarification

Before discussing the solution, the traffic/data needs to be handled by the BAP of boundary IAB-node is clarified below. (As agreed by R3, boundary node’s access traffic will be handled same as R16, which is not discussed below)

For upstream at boundary node:

- Case 1: Data from CU1 topology to be forwarded to CU1 topology; [i.e. non-concatenated traffic’s intra-topology routing]

- Case 2: Data from CU1 topology to be forwarded via CU2 topology; [i.e. concatenated traffic’s inter-topology routing]

- Case 3: Data from CU1 topology originally to be forwarded to CU1 topology, but re-routed to CU2 topology due to RLF. [i.e. non-concatenated traffic’s inter-topology re-routing]

- Case 4: Data from CU1 topology originally to be forwarded to CU2 topology, but re-routed to CU1 topology due to RLF. [i.e. concatenated traffic’s inter-topology re-routing]

- Case 5: Data from CU1 topology originally to be forwarded to donor-DU1 of CU1 topology, but re-routed to donor-DU5 of CU1 topology due to RLF. [i.e. intra-topology inter-donor-DU re-routing]

- Case X: Data from upper layers to be transmitted on CU1 topology.

- Case Y: Data from upper layers to be transmitted on CU2 topology.

For downstream at boundary node:

- Case 6: Data from CU1 topology to be delivered to upper layer;

- Case 7: Data from CU1 routed via CU2 topology to be delivered to upper layer;

- Case 8: Data from CU1 topology to be directly forwarded to next hop node in the CU1 topology;

- Case 9: Data from CU1 routed via CU2 topology to be BAP header rewritten and then forwarded to next hop node in the CU1 topology;

For the alternative modelling the possible cases are:

For upstream at boundary node (IAB-2):

Case 1-5: same as above

For downstream at boundary node (IAB-3):

- Case 6: Does not exist (downstream boundary node only handles data from its own CU);

- Case 7: Data from CU2 topology to be delivered to upper layer;

- Case 8: Data from CU2 topology to be directly forwarded to next hop node in the CU2 topology;

- Case 9: Data from CU2 topology to be BAP header rewritten and then forwarded to next hop node in CU1 topology;

It is important to clarify the assumption to ensure that companies on the same page on the BAP operation basic modelling when discussing the solutions later.

Below is the straight forward basic BAP operation modelling assumed by rapporteur:

Basic BAP operation modelling: Header rewriting for concatenated traffic=>Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing.

Below is the straightforward basic BAP operation modelling assumed for the alternative modelling:

Basic BAP operation modelling: Checking routing table for both non-concatenated and concatenated traffic => Header rewriting for concatenated traffic or for re-routing

NOTE: For clarification on “Checking routing table specific for concatenated or non-concatenated traffic”, please see the Question 9 below.

Clarification 1: Similar to the R16 local re-routing, for inter-topology re-routing and inter-donor-DU re-routing, BAP should “Check routing table” first before “Header rewriting for re-routing”, to try the primary path first and then do the re-routing if no primary path available.

- 5.2.1.3 Routing For a BAP Data PDU to be transmitted, BAP entity shall: - if the BAP Data PDU corresponds to a BAP SDU received from the upper layer, and - if the BH Routing Configuration has not been (re)configured by F1AP after the last (re)configuration of defaultUL-BH-RLC-Channel by RRC: - select the egress link on which the egress BH RLC channel corresponding to defaultUL-BH-RLC-Channel is configured as specified in TS 38.331 [3] for non-F1-U packets; - else if there is an entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, whose BAP path identity is the same as the PATH field, and whose egress link corresponding to the Next Hop BAP Address is available: - select the egress link corresponding to the Next Hop BAP Address of the entry; NOTE 1: An egress link is not considered to be available if the link is in BH RLF. - else if there is at least one entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available: - select an entry from the BH Routing Configuration whose BAP address is the same as the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available; - select the egress link corresponding to the Next Hop BAP Address of the entry selected above;

Question 1: Do you confirm/agree the pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on previous BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF or type2 indication, etc.), similar to R16?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | Yes | BAP header re-writing should always be based on routing, i.e., re-writing is done only if (re)routing requires header re-writing. |
| Kyocera | Yes | We assume the re-routing is performed after the routing fails. |
| CATT | Yes | BAP header rewriting should be performed after R16 routing and rerouting mechanism can not work. If there is no available next hop found based on previous BAP address and routing ID, BAP layer will perform BAP rewriting for rerouting. |
| QCOM | Yes | In other words: Routing  link not available  use other link  header rewriting |
| Lenovo |  | Does it mean that only path ID change does not trigger BAP header rewriting? If yes, we agree with the criteria. [Rapp] Yes as in R16. |
| Ericsson | Yes, but load balancing should also be considered as precondition/criteria. | BH RLF, or type-2 RLF reception can be used as triggering criteria for re-routing, but RAN3 also agreed that load balancing is an use case for inter-donor re-routing (besides robustness). Hence, the boundary IAB node can be configured to re-route certain upstream traffics via the CU2. It should also be clarified that in this question we are discussing the precondition/criteria for the BAP header rewriting in the upstream. In the downstream, the only precondition for the BAP header rewriting is that the traffic should come from the CU2. [Rapp]: Traditionally, we only call the RLF case as “re-routing” while the CU configured case as “routing”, e.g. for local balancing. The re-routing is triggered by IAB-node, while load balancing purpose routing is fully control/configured by CU. |
| LG | Yes | We assume BAP header rewriting is performed only when the R16 local re-routing mechanism does not work. |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes | If the primary link is not available, then the backup link should be used for re-routing. |
| Samsung | Yes but… | We assume the above re-routing is referring to the BH RLF case. However, we agree with Ericsson that the case for re-routing due to the congestion should also be covered. During BL CR discussion for 38.340, we discussed how the unavailable BH link may be the one with failure or congestion. It seems that the re-routing based on congestion needs some further discussion. So, we suggest the following rewording: the pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on previous BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF or type2 indication, FFS due to congestion, etc.), similar to R16 [Rapp]: etc. in the question includes any other trigger for local rerouting. |
| ZTE | Yes | Re-routing is performed if there is no available next hop found. Then the node re-writes the BAP header based on the routing table. |
| Canon | Yes |  |
| Fujitsu | Yes | Re-routing is the last step, and the criteria should be similar to R16. |
| Futurewei | Yes | We agree that this is the logical approach |
| Intel | Yes |  |

Summary:

There is clear majority to confirm the question.

Proposal 1: The pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on previous BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF, congestion or type2 indication, etc.), similar to R16.

Clarification 2: Similar to the legacy BAP modeling, the “Determination of delivery to upper layer” at RX should be before “Checking routing table” at TX.

With the above clarification 1/2, there is one more FFS point on the basic BAP operation modeling, which may be the key point of this discussion.

Modelling FFS: For concatenated traffic’s inter-topology routing, FFS on whether checking “Header rewriting for concatenated traffic” is performed first or “Determination to deliver to upper layer” is performed first.

The below Question 4/5 depends on this modeling FFS, and it will be discussed/down-selection in Question 7 in the end.

Therefore, before going into the details, rapporteur asks companies to share views on the above assumptions and modelling.

Question 2: Companies are asked for comments on the above data cases (case 1-9) and/or on the BAP basic operation modeling, if any? (Mainly on the basic modeling and its clarification 1/2, while the modelling FFS will be asked and down-selected later in Q7)

| Companies | Comments, if any |
| --- | --- |
| Nokia | As indicated above, for the alternative boundary node modelling, - cases 1-5 are valid, case 6 does not exist and for cases 7-9 data is always from CU2 (since the downstream boundary node belongs to topology of CU2) - BAP operation is much simpler and basically the same as for Rel16: check whether data is delivered to upper layers, if not check routing table and perform header re-writing if needed |
| Kyocera | We agree that the Rx operation, i.e., to determine the delivery to upper layer, is performed before the Tx operation, as same with Rel-16. Just small clarifications: In Case 5, “donor-DU5” is not depicted in the figure, but we assume it’s another donor-DU of CU1. [Rapp]: Yes In Case 9, we assume the data is forwarded to the next hop node in “CU1 topology” [Rapp]: Yes, updated. |
| CATT | We agree that the BAP basic operation modeling and the 9 cased. |
| QCOM | You are missing two cases for upstream traffic: - Case X: Data from upper layers to be forwarded to CU1 topology. - Case Y: Data from upper layers to be forwarded to CU2 topology. These are the equivalent cases 6 and 7 for downstream traffic. [QC1]: We don’t see why this is straightforward since it has the header rewriting step twice. We propose an alternative approach: Routing  Header rewriting. Both of these alternatives should be discussed here. |
| Lenovo | For the basic modeling, the last step “Looking-up routing table again for re-routing” may be not needed. Since in the step of “Header rewriting for re-routing”, the egress link has been selected already. |
| Ericsson | We are fine with the Rel.16 legacy modeling in which the RX side of the BAP entity checks whether to forward the traffic to upper layers or not. Regarding the cases the following should be clarified: Case 1 / 2: Clarify differences between case 1 / 2 and case 3 / 4. Is the only difference between them the criteria for the BAP header rewriting? i.e. in case 3 and 4 the triggering criteria is RLF, whereas in case 1 and 2 is some NW configuration on the traffic, e.g. some traffic to be routed to CU1 directly, and some other traffic to be routed via CU2. [Rapp]: We are not touching that details on the difference between case 1/2 and case 3/4 yet. We also would like to change “forwarded to CU2 topology” -> “forwarded via CU2 topology” since the traffic is still terminated in CU1 even though it goes via CU2. [Rapp]: OK Case 7 / 9: Clarify what “from” means. In our understanding it should be for case 7 “Data from CU1 routed via CU2 and to be delivered to upper layers”, and for case 9 “Data from CU1 routed via CU2 and to be forwarded to next hop after BAP header rewriting”. [Rapp]: Yes Case X/Y: Does these cases really exist? The boundary IAB node is still anchored to CU1 so from the upper layers perspective all the traffic is terminated in the CU1. So from the upper layers perspective what is the difference between case X/Y and the other cases 1-2-3-4-5? [Rapp]: No intention to discuss X/Y. But those should be the access traffic, while other cases are from descendent node traffic. |
| LG | We are ok with 9 data cases, but have one comment on BAP basic operation modeling. We wonder if iterated routing table look-up is needed as the last step in the suggested BAP basic operation modeling. Actually, when the new BAP routing ID for header rewriting is selected, the BAP entity, at the same time, should check whether the next hop of the rewritten BAP header is available or not. So, we think that the last step may be merged to the step of Header rewriting for re-routing in the BAP basic operation modeling. |
| vivo | For clarification 2: agree. For data cases: Case 4 needs to be clarified, the rest of the cases are ok to us. Take the brown line (IAB4->IAB2->IAB3->Donor-DU2->CU2) in the given illustration (the left figure, inter-CU topology redundancy) as an example, we wonder if Case 4 describes the following situation: an RLF occurred over the BH link between IAB3 and IAB2, the boundary node (IAB2) therefore re-routes the data (was destined to IAB3) to IAB1 and ultimately deliver to CU1. [Rapp] Yes. If the above assumption is the intention, we are confused how can the boundary node choose the correct BAP routing ID (as there should not be any pre-configured entry for this concatenated traffic in the BH routing configuration, instead the corresponding entry is only available for the Header rewriting configuration). So we think for Case 4 the transmission of concatenated data may be stopped until the BH link recovers. In other words, Case 4 may not exist? [Rapp]: To clarify: To achieve case 4, CU should configure the header rewriting for its original concatenated path (IAB4->IAB2->IAB3->Donor-DU2), also configure the header rewriting in case of inter-topology rerouting due to RLF, and also the routing entry based on the new routing ID. Companies are welcome to comment on whether to support case 4 at all. For BAP basic operation modeling: we share similar view with Lenovo and LG that the last step can simultaneously happen with the ‘selection of re-routed BAP routing ID’, which implies that the boundary node should check the available status of the backup BH link(s) and choose the feasible one to be re-written in BAP header. |
| Samsung | Regarding Clarification 1 and the following statement therein: ‘BAP should “Check routing table” first before “Header rewriting for re-routing” ‘ – as mentioned in our response to Q1, we agree that rerouting should only be done if routing according to existing table is not possible (e.g., BH RLF, or congestion). However, it is not clear to us what is meant by BAP checking routing table before header rewriting and what kind of modelling this implies. Header rewriting is done based on information/instruction from the network and once relevant information is received, it needs to be actioned. The statement above implies that BAP layer at the node has some sort of decision-making capacity? [Rapp]: For local re-routing including inter-donor-DU and inter-topology re-routing, header rewriting is need (yes, configured by CU). In clarification 1, BAP should first check the routing table to see “if routing according to existing table is not possible”, then header rewriting is performed in case R16 BAP address based routing is not possible. The rewriting and re-routing is still based on the CU configuration. |
| ZTE | We basically agree with the basic modeling and the 9 cases proposed by rapporteur. For the case 3,4,5, the re-routing based on RLF is mentioned. However, the re-routing based on congestion is missing. It is suggested to clarify it. For BAP basic operation modeling, we think the header re-writting for concatenated traffic and header re-writting for re-routing may be separate steps. When performing the header re-writting for concatenated traffic, header re-writting table for concatenated traffic can be used to replace the previous BAP routing ID. Then it check the legacy routing table (which containing the routing ID and next hop BAP address) for the availability of egress link based on legacy procedure. Suppose the re-rotuing is necessary, the BAP routing ID is re-written based on the selected routing entry in the legacy routing table. It is suggested to consider these steps separately. [Rapp]: Yes, that’s the assumption of the basic modeling. |
| Canon | For downstream at boundary node, case 7 may be divided into two cases: - Case 7a: Non-concatenated traffic data from CU2 topology to be delivered to upper layer (i.e. CU2 sending data to the boundary node); - Case 7b: Concatenated traffic data from CU2 topology to be delivered to upper layer (i.e. CU1 sending data to the boundary node through CU2’s topology). [Rapp]: Yes case 7a in your comment is the Case Y or X. But that’s the legacy R16 kind of traffic. While case 6 is: Non-concatenated traffic data from CU1 topology to be delivered to upper layer (i.e. CU1 sending data to the boundary node through its own topology). |
| Fujitsu | We agree with the 9 cases and the two more cases proposed by QC. We think the BAP basic operation modelling needs more discussion. In some complicated scenario, e.g., RLF from SCG in the inter-CU topology redundancy scenario, some condition can be judged to choose which new BAP routing ID should be used (or which BAP header rewriting configuration to be chosen) for header re-writing, instead of doing BAP header rewriting twice. |
| Futurewei | We are fine with the use cases discussed above for both modeling proposed by e-mail discussion’s rapporteur, and also the alternative modeling proposed by Nokia. Regarding basic BAP modeling, we think it is preferable for clarity to define routing and header re-writing in separate sections in the spec. Therefore, we prefer to define a solution where routing and header re-writing are each done only once (rather than potentially twice as proposed by the rapporteur). We think there are potentially two approaches to achieve this: - Perform BAP header re-writing (if needed) on egress link after routing decision, but before RLC bearer mapping. This seems to be the approach proposed both by QCM and Nokia. - Perform BAP header re-writing (if needed) on the ingress link of the following IAB node (child node in downstream direction, parent node in upstream direction). Generally, the first approach seems like it might be somewhat simpler to capture in the spec. However, we are open to discuss both. Regarding alternative modeling proposed by Nokia, we think that not withstanding any assumptions or preferences on the part of RAN3, RAN2 has the responsibility to define BAP layer functionality. Accordingly, RAN2 should not assume that RAN3 has fully discussed all of the implications to BAP processing for a particular architectural modeling. Therefore, we believe that RAN2 can discuss both modeling approaches and their implications for the BAP spec and decide on the simplest approach. Of course, RAN2 should liaise with RAN3 on their considerations and agreements, as appropriate. |
| Intel | In general, we are ok with the baseline that Rel-16 legacy modeling of handling traffic to upper layer is checked at the RX side of the BAP entity. For the 9 cases summarized above, three comments from our side: 1) with the change “to”->”via” from Ericsson on case 2, case 4 also be updated as below: “Data from CU1 topology originally to be forwarded via CU2 topology, but re-routed to CU1 topology due to RLF.” 2) We are wondering whether Case X is overlapped with Case 1, as data from upper layer in case X is also generated in CU1’s topology. This also indicates that Case 1 includes both traffic descendant node traffic and access traffic, which aligns with the terminology defined in the beginning. 3) For topology redundancy scenario, as captured in RAN3 agreement: “WA: boundary and descendant nodes may have a different F1-termination node.” The descendant IAB-nodes of the boundary IAB-node may terminate at a different IAB-donor CU. Hence, the boundary IAB-node may also handle traffic from IAB-donor CU2 e.g. partial migration of IAB-node 5). For the completeness, we propose to add two new cases in downstream: - “Case 10: Data from CU2 topology to be delivered to upper layer;” - “Case 11: Data from CU2 routed via CU2 topology and then forwarded to next hop node in the CU2 topology; An example figure is shown as below: |

Summary:

There is clear majority to confirm the clarification 2. Below proposal will be implied by the proposal given in Q7.

Proposal: Similar to the legacy BAP modeling, RAN2 confirm that the “Determination of delivery to upper layer” at RX side should be before “Checking routing table” at TX side.

Upstream header rewriting based re-routing issue, as Lenovo, LG, vivo pointed out:

- For the order about “Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing.” for local rerouting, another alternative is: Once local rerouting is triggered, select the available next hop node directly and then perform the header rewriting for re-routing. Also, the header rewriting entry configured for the previous routing ID and the availability of the new next hop should be checked together.

Therefore, following proposal is given to clarify that: 1) both rewriting entry configured and availability of egress link should be checked; 2) FFS on whether egress link selection or header rewriting is first.

Proposal 2: Once the “BAP header rewriting based re-routing” is triggered, BAP routes the data to the available egress link, if there is matched entry in the Header rewriting table for re-routing. FFS egress link selection is performed before or after header rewriting (can be discussed in running CR).

Since majority as fine to support case 1-9, we can confirm below proposal to support case 4:

Proposal 3: For upstream at the boundary node, it is supported to local re-route the data to CU1 topology, due to RLF/congestion/etc., which was from CU1 topology and originally to be forwarded via CU2 topology.

ZTE, Fujitsu, Futurewei commented on, mainly for case 4, whether the header rewriting should be performed twice. This is also related to whether to use “routing/next hop selection first and then header rewriting” as option 2 below.

For upstream case 4:

Option 1: Basic BAP operation modelling proposed by rapporteur: “Header rewriting for concatenated traffic=>Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing.”

Option 2: Modelling to avoid performing the header rewriting twice: “Checking routing table as in legacy R16=> if failed, consider the “BAP header rewriting based (re-)routing” is triggered and find/select the available alternative egress link=>perform the header rewriting accordingly, if possible.”

Rapporteur propose to use separate header rewriting steps at the baseline (as mentioned by ZTE), since no other companies comment on this assumption to the basic modelling. Some enhancement to merge those can be discussed after we have concluded on at least one workable basic modelling. Even if option1 is considered in the specification, option 2 is also allowed by implementation.

Proposal 4: For upstream at the boundary node, it is FFS on whether to merge the BAP header rewriting operations/steps for inter-topology routing and inter-topology re-routing.

Intel mentioned the more than 2 boundary nodes scenario based on the RAN3 WA. Rapporteur understand this is not confirmed by RAN3 yet due to its low priority and the limited TU left for R17. We can consider the impact to our BAP operations later once RAN3 confirm to support this complicated scenario.

### 2.2.2 Solution discussion

Following discussion is to address the open issues from last RAN2 meeting for inter-topology routing:

- What’s the BAP address added in BAP header in the first topology (i.e. the BAP address of ingress data at the boundary node);

- How to differentiate the concatenated traffic and non-concatenated traffic;

- How to determine whether a data should be delivered to upper layer (for downstream);

- How to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology).

#### Question 3: How to differentiate the concatenated traffic and non-concatenated traffic for downstream

NOTE: for upstream case, this can be automatically identified by the solution in Q4, since only concatenated traffic performs BAP header rewriting.

Motivation 1 for this question: based on the RAN3 agreement below, BAP operations is different for concatenated traffic and non-concatenated traffic, at least for downstream, including the Determination to deliver to upper layer and Header rewriting for concatenated traffic only.

- RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic.

Motivation 2 for this question:

- There is no assumption on the CU coordination to avoid the routing ID/BAP address/path ID collision, used in the two ingress topologies at the boundary node for downstream. Namely that the downstream ingress BAP data at the boundary node from two topologies may have the same routing ID/BAP address/path ID.

- So, any solution based on routing ID/BAP address/Path ID in below Question 4/5 only works if the boundary node can identify whether the data is concatenated traffic or non-concatenated traffic first (i.e. from which topology). Then, the new behavior of the solution in Question 4/5 only applies to the concatenated traffic rather than non-concatenated traffic.

  - For example, in the BAP address based solution in Q5, the boundary node needs to determine whether the downstream traffic is concatenated traffic or non-concatenated traffic, so that it knows which BAP address (i.e. CU1 configured or CU2 configured) to be used for determination to deliver to upper layer. Note that R3 agreed the two BAP addresses of boundary node may be same.

Based on the above motivations, the boundary node needs to differentiate the concatenated traffic and non-concatenated traffic for downstream (i.e. identify the ingress link).

One example way: For topology redundancy scenario, it is clear that data received from MCG is the non-concatenated while from SCG is the concatenated traffic. For partial migration case, boundary node only has the MCG with all downstream traffic are concatenated traffic. For sure, this assumes the IAB-node knows it is the boundary IAB node (e.g. by the configuration with two BAP address or the configuration of BAP header rewriting table for inter-topology routing).

Question 3: Do you agree that the ingress link is able to identify/differentiate the concatenated and non-concatenated traffic at the boundary node for downstream?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | With the alternative boundary node modelling, IAB-nodes in the other topology are allocated “pseudo” BAP address (or Routing ID). For “pseudo” Routing IDs, the boundary node is configured with the actual Routing IDs (of the other topology) to be used in BAP header re-writing. Thus, the boundary node can identify the concatenated traffic based the “pseudo” Routing ID. The same applies to both downstream and upstream. Thus, with the alternative boundary node modelling, the BAP operation is same as in Rel16: - based on Routing ID, the BAP PDU is routed to next hop node. - the only new thing is that If the next hop node (based on the pseudo Routing ID) is in the other topology, the BAP performs header re-writing. |
| Kyocera | No | We’re still not sure if it’s always possible to identify/differentiate the concatenated traffic and non-concatenated traffic based on the ingress link, considering various deployment scenarios, e.g., combination with CP/UP separation etc. We would prefer a unified solution for both downstream and upstream. So, we think the two cases should be discussed together. |
| CATT | No | We think it is not reliable to identify/differentiate the concatenated and non-concatenated traffic by ingress link. [Rapp]: As clarified, this is to just differentiate the traffic from CU1 topology and CU2 topology. |
| QCOM | Question is not clear | We believe the question was aiming for something else. The ingress link certainly knows if it resides in the blue topology or the green topology. So it either receives only blue or only green traffic. For this reason, it does not have to differentiate between green and blue traffic. Do you mean: The ingress link is the differentiator between blue and green ingress traffic? [Rapp]: Yes, that’s the intention of this question. |
| Lenovo | Yes |  |
| Ericsson | Yes | It seems quite straightforward to assume that one ingress link can only be associated to either the concatenated traffic or the non-concatenated traffic. |
| LG | Yes | Considering that one link can be used to connect one topology, we think the ingress link can identify/differentiate the concatenated and non-concatenated traffic, i.e., the ingress link is the differentiator between blue and green ingress traffic. |
| Huawei, HiSilicon | Yes | This is to first determine the ingress topology. Then for the traffic from inter-topology, below questions will further differentiate the traffic to be header rewritten. |
| vivo | Yes | To address Kyocera’s concern: In case of CP/UP separation, the boundary node is able to differentiate the F1-C traffic delivered via SCG from the concatenated traffic because the F1-C traffic is received over NR(Uu) instead of BH ingress link. |
| Samsung | Yes | We share the rapporteur’s understanding. |
| ZTE | Yes | For the downstream traffic, we are basically fine with the analysis by rapporteur. |
| Canon | No | We think it is not possible to differentiate the concatenated traffic and non-concatenated traffic based on the ingress link. An additional mechanism is required. [Rapp]: please check the definition of “concatenated traffic” in this paper. We also prefer a unified solution for both downstream and upstream. |
| Fujitsu | Yes | The principle of the example proposed by rapporteur can be used. However, the scenario for topology redundancy is not accurate. For topology redundancy, the F1-terminating donor initiates traffic offload, and both MN and SN can be F1-termniating donor. That is to say, data received from the ingress link towards the F1-terminationg node is the non-concatenated while from the ingress link towards the non-F1-terminating node is the concatenated traffic. Based on RAN3 agreement “If IAB node establishes NRDC before F1-C, the IAB node can implicitly derive whether MN or SN is the F1-terminating donor, e.g., based on who provides the default BAP configuration”, the boundary node is able to derive the relation of ingress link and the F1-terminating node. [Rapp]: Agree. This example will be explained/updated in details based on the RAN3 new agreement to considering the SN as F1-terminating node case. |
| Futurewei |  | Although we agree that it seems possible to differentiate concatenated from non-concatenated downstream traffic by the parent node of the ingress link, we tend to agree with comments from Kyocera and Canon in that it would be preferable to define a unified solution that would apply equally to both upstream and downstream traffic. |
| Intel | Yes if only considers traffic of IAB-donor CU1; No if also considers traffic of IAB-donor CU2. | For concatenate traffic and non-concatenate traffic to/from IAB-donor CU1, we agree that the ingress link can successfully differentiate whether the packets are routing via CU1’s topology or CU2’s topology. However, the boundary IAB-node cannot differentiate “concatenate traffic of CU1 via CU2” and “non-concatenate traffic of CU2”. As we commented in Q2, in both topology redundancy scenario and partial inter-CU migration scenario, it is also possible that IAB-donor CU2 has some traffic routing through the boundary IAB-node (i.e. IAB-node 2) to its own descendant IAB-node in the topology of IAB-donor CU2 or to IAB-node 2 directly. It is possible that those traffic streams share the same ingress link for downstream traffic. Hence, for this scenario, the concatenate traffic of CU1 via CU2 and non-concatenate traffic of CU2 may not be differentiated by ingress link. [Rapp] “non-concatenate traffic of CU2” will not arrive at boundary node, since its F1-terminatign CU is CU1. The scenario, as shown in your figure in Q2, can be discussed later based on new R3 formal agreement. |

Summary:

Yes: 10 companies (QCOM?, Lenovo, Ericsson, LG, Huawei, vivo, Samsung, ZTE, Fujitsu, Intel)

No: 3 companies (Kyocera, CATT?, Canon)

Another modelling: Nokia

[Rapp]: New modelling will be discussed in Q7 in the end.

Also, Kyocera, Canon, Futurewei want a unified solution for UL and DL. That would be discussed in Q7 in the end.

Intel’s scenario can be discussed later after RAN3 confirm the WA on support this complicated case.

Rapporteur propose below based on the majority. NOTE: This propose will also be merged into the full picture proposals in Q7.

Proposal 5: For downstream, the boundary node is able to identify/differentiate the traffic routed via inter-topology and the traffic routed via intra-topology, based on the ingress link.

#### Question 4: How to determine whether the BAP header of a data should be rewritten

NOTE: Somehow Question4 and Question5 are related and address similar issue, and somehow rely on the Q3/6.

The issue: For concatenated traffic, boundary node need to know whether any BAP data needs to perform the BAP header rewriting for inter-topology routing purpose.

Option 1: Routing ID based approach

- Option 1a: by checking the routing ID in BAP header with the configured BAP Header rewriting table (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if routing ID in header matches any “previous routing ID” in the rewriting table, the BAP header should be rewritten.

  - This kind of “specific routing ID” is naturally configured by the first column of the BAP Header rewriting table.

  - This is not mutual exclusion with using “pseudo BAP address”.

- Option 1b: (assuming “Determination to deliver to upper layer” first in the modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 1a is used.

Option 2: BAP address based approach

- Option 2a: by checking the BAP address in BAP header with the configured specific BAP address(es) set (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if the BAP address in the header belongs to the configured BAP address(es) set (e.g. the pseudo BAP addresses), the BAP header should be rewritten.

  - This requires separate configuration on the specific BAP address set.

- Option 2b: (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 2a is used.

Option 3: Path ID based approach

- Option 3a: by checking the path ID in BAP header with the configured specific path ID(s) set (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if the path ID in the header belongs to the configured path ID(s) set, the BAP header should be rewritten.

  - This requires separate configuration on the specific path ID set.

- Option 3b: (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 3a is used.

Option 4: Routing ID based approach with alternative boundary node modelling

- for both upstream and downstream, by checking the Routing ID in BAP header with the configured routing table determine the Next Hop IAB-Node (same as Rel16)

  - if the Next Hop Node is in the other topology (e.g., indicated in the routing table or in a separate header re-writing table), perform BAP header re-writing (new in Rel17)

    - replace the “previous routing ID” in the BAP header with the “new routing ID”.

This alternative uses “pseudo” BAP Routing IDs.

Option 5: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., for BAP header rewriting.

Question 4: Which option do you prefer to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology) for concatenated traffic in inter-topology routing at the boundary node?

| Companies | Option? | Comments Please clarify if you have different solutions for upstream and downstream. You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | Option 4 | Option 4 with alternative boundary node modelling uses Rel16 approach for both upstream and downstream in the corresponding boundary nodes. Header rewriting is performed only if Routing ID based routing indicates that Next Hop Node is in the other topology. |
| Kyocera | Option 5 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used to determine whether the data should be performed with BAP header rewriting, as well as whether the data should be forwarded to the upper layer. We assume it’s common solution for both upstream and downstream. We wonder if RAN3’s assumption, “1a: RAN3 assumes that the boundary node has only one BAP address in each topology”, meant Option 2a is no longer applicable. |
| CATT | Option 1a | The procedure of “Header rewriting for concatenated traffic first” seems more fluently. |
| QCOM | Option 1b | In Rel-16, it is first determined whether traffic is to be delivered to upper layer. Otherwise, it is routed. In Rel-17, we can follow same approach as Rel-16 where the difference is the routing may also incur header rewriting. This deprioritizes options 1a, 2a and 3a. Options 1b, 2b and 3b are the same for downstream. For upstream, option 1b is more generic since BAP routing ID includes both BAP address and BAP path ID. |
| Lenovo | Option 1a | For option 2 and 3, they are both the subset of option 1, then option 1 has more flexibility for configuration. And since the DESTINATION BAP address used for intra-topology routing packets may be the same as the DESTINATION BAP address used for the packets to be delivered to the upper layer, then BAP header rewriting needs to be performed firstly for the intra-topology routing packets. |
| Ericsson | Option 1a for the TX side, and Option 1b for the RX side. | For RX: We do not see why receiving operations should be changed compared with legacy. First the receiving part of the BAP entity determines whether to pass the incoming BAP PDU to upper layers. For TX: If the RX side of the BAP entity determines that the BAP PDU should be passed to the transmitting part of the BAP entity, then the solution in option 1a should follow, i.e. the UE checks if for the BAP routing ID in the header there is any matching entry “previous BAP routing ID”. If there is a matching entry, the BAP header is rewritten with the corresponding “New Routing ID”. Regarding the pseudo-BAP addresses, we believe that those should be used irrespectively of the solution adopted for the BAP header rewriting. Pseudo-BAP addresses are needed because the CU2 does not need to know the real BAP addresses when injecting packets into the CU2 topology, given that anyhow the boundary node will do BAP header rewriting for the packets coming via CU2. |
| LG | Option 1b | We prefer to check first whether the received packet is to be delivered to upper layer or not as in Rel-16 and then BAP header rewriting should be determined based on BAP routing ID. |
| Huawei, HiSilicon | Option 1a is preferred, Option 1b is also acceptable | Agree with QCOM and Lenovo. |
| vivo | Option 2b | The traffic can be differentiated based on the (pseudo) BAP address [Option 2], while the BAP header can be re-written based on the BAP routing ID configured in the Header rewriting Configuration [Option 1]. Since Q4 asks when (instead of how) should the IAB-node determine to re-write, we would like to go for Option 2. |
| Samsung | Option 1b for downlink | We have some concerns about upstream. The above assumption seems to indicate that the header rewriting table is referring to the mapping between “previous BAP routing ID” to “new routing ID”. This may result in the situation as below: In the above figure, the topology of CU1 is as below: CU1 ----- DU1 ----- IAB1 ----- IAB2 (boundary node) ----- IAB4. The traffic from IAB4 to DU1 can be set the same BAP routing ID (e.g., 1). However, different traffic may be sent from IAB4 to IAB2 via different ingress BH RLC CHs (for example, via ingress BH RLC CH 1 and ingress BH RLC CH2). If the header rewriting is based on BAP routing ID only, it means that all traffic with BAP routing ID =1 should be forwarded to the topology of CU2. However, the case of offloading traffic via one ingress BH RLC CH cannot be realized. In other words, the upstream header rewritten based on “previous BAP routing ID” results in that the traffic of one routing path shall be offloaded to topology of CU2 in total, and it cannot support the offloading of partial traffic over one routing path. To overcome this, we suggest that: For upstream, the BAP header rewriting is determined by the “previous BAP routing ID” and optional “ingress BH RLC CH”. |
| ZTE | Option 1a | The boundary node first checks BAP-routing-ID mapping table regardless of UL or DL packet. If the routing ID of the packet matches any “previous routing ID” in the rewriting table, the BAP header should be rewritten. Option 1 can provides more flexible configuration in comparison with option 2 and option 3 because routing ID includes BAP address and path ID and option 2 and 3 can be regarded as different variants of option 1. |
| Canon | Option 3a (or Option 5) | As a first step, the boundary node checks if the traffic is concatenated or not. In this respect, we prefer option 3a, which proposes to rely on dedicated path identifiers, to be specifically used for identifying concatenated traffic (even though Option 5 requires the use of a reserved bit within the BAP header, we think this option is acceptable for concatenated traffic identification). Then in a second step, the header is rewritten in a boundary node for concatenated traffic. Here it is required to have identified the ingress topology. In a third step, it is checked whether the traffic should be delivered to upper layer. |
| Fujitsu | Option 1b | Since option 1b refers to option 1a for upstream, the contents in the parentheses in option 1a should be removed. For upstream, after determination not to deliver to upper layer, then check the routing ID in BAP header with the configured BAP Header rewriting table. |
| Futurewei | Option 4 (if alternative modeling is agreed) or Option 5 | Option 4 seems to have by far the least spec impact and works as long as alternative modeling approach proposed by Nokia is acceptable. Option 5 seems futureproof and could be applied in both upstream and downstream directions. Furthermore, there is no significant issues regarding collision of BAP routing IDs if reused between topologies. Option 1 does not seem to have any advantages compared to Option 4 for the alternative modeling approach. On the other hand, it seems there would be potential for confusion due to BAP routing ID collisions between the two topologies, as discussed in section 2.3. Options 2 & 3 are special cases of Option 1, so we don’t see any obvious advantage of considering these compared to Option 1. |
| Intel | Option 1b in RX, then option 1a in TX afterwards | We agree that the same approach as Rel-16 could be considered, i.e. determine first whether the packet to be delivered to the upper layer or not. However, we share the same view with Ericsson. From our understanding, option 1b does not indicate the exact BAP rewriting action is performed. The actual BAP rewriting action is performed after packet differentiation at TX side of BAP entity. If only CU1’s traffic is considered in this discussion, TX of BAP entity will perform BAP header rewriting according to the configured BAP address mapping table. If also consider non-concatenate CU2’s traffic at boundary IAB-node, assuming “Determination to deliver to upper layer” is performed first, two potential types of traffics remaining at the boundary IAB-node, i.e. CU1’s traffic to descendant IAB-nodes via CU2 and CU2’s traffic to descendant IAB-nodes. Before BAP header rewriting, the TX of BAP entity should first identify CU1’s traffic via CU2 and CU2’s traffic. CU2’s non-concatenate traffic does not need to perform BAP header rewriting. |

Summary:

Majority options:

- Option 1a: CATT, Lenovo, Huawei, ZTE

- Option 1b: QCOM, LG, Huawei, Samsung, Fujitsu

  - [Rapp]: Additional info in the rewriting table proposed by Samsung can be discussed in the future.

- Option 1a for the TX side, and Option 1b for the RX side: Ericsson, Intel

  - [Rapp]: This can be considered as option 1b. Ericsson and Intel assume the RX will determine whether to deliver to upper layer first and the rest will be forwarded to TX for header rewriting. So, as to “how to determine whether to rewrite at RX”, it is based on 1b; as to “how to perform rewriting at TX” it is 1a. The question is for “how to determine”, so rapporteur considers this as support of 1b. Also see the proposal as clarification.

Options with minority support:

- Option 2b: vivo

- Option 3a: Canon

  - [Rapp]: As commented by other companies, option 3 can be considered as variant of option 1.

- Option 4: Nokia, Futurewei

- Option 5: Kyocera, Futurewei

To make progress, RAN2 should first try to converge on the options with majority views. It means we can first give proposals based on option 1a and 1b for the two different modellings. NOTE: This proposal will also be merged into the full picture proposals in Q7.

Proposal 6a: Option 1a is proposed for modelling A: [Second majority]

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is determined as to be header rewritten and forwarded to the TX side, if the routing ID in header matches any “previous routing ID” in the header rewriting table.

  - At BAP TX side, perform the header rewriting based on the configured header rewriting table.

- For upstream at the boundary node, for any received data from lower layer:

  - At BAP RX side, no data will be delivered to upper layer, based on the solution in Q6b.

  - At BAP TX side, the data is determined as to be header rewritten and perform the header rewriting accordingly, if the routing ID in header matches any “previous routing ID” in the rewriting table.

Proposal 6b: Option 1b is proposed for modelling B: [Majority]

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is determined as to be header rewritten and forwarded to the TX side, if the data is determined as NOT to be delivered to upper layer, based on the solution in Q5.

  - At BAP TX side, perform the header rewriting based on the configured header rewriting table.

- For upstream at the boundary node, same as Proposal 6a.

#### Question 5: How to determine whether a data should be delivered to upper layer (for downstream)

NOTE: Somehow Question4 and Question5 are related and address similar issue, and somehow rely on the Q3/6.

NOTE: For upstream, see the issue discussed in Q6.

The issue: For downstream, the boundary node needs to know whether any received BAP data should be delivered to upper layer.

Clarification on one RAN3 assumption: “RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers.”

- It means ONLY the data with matched BAP address can be delivered to upper layer. But, it does NOT means any data with matched BAP address should be delivered to upper layer. Somehow, the downstream traffic terminated at the boundary node has to add the BAP address in header same as the boundary node’s BAP address.

- Namely that FFS whether path ID part is additionally used to determine whether a BAP data should be delivered to upper layer.

- Anyway, this is RAN3 assumption, RAN2 should have our final conclusion.

Option 1: Routing ID based approach

- Option 1a: by checking the routing ID in BAP header with the configured BAP Header rewriting table (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if routing ID in BAP header does NOT match any “previous routing ID” in the rewriting table (i.e. not to be header rewritten) , the data should be delivered to upper layer.

  - This kind of “specific routing ID” is naturally configured by the first column of the BAP Header rewriting table.

- Option 1b: by checking the routing ID in BAP header with some configured routing ID set (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the routing ID in the BAP header belongs to the configured routing ID set, the data should be delivered to upper layer.

  - This requires separate configuration on the specific routing ID set

Option 2: BAP address based approach

- Option 2a: (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if it is determined NOT to be BAP header rewritten based on the solution in Q4, the data should be delivered to upper layer.

- Option 2b: by checking the BAP address in BAP header with the configured specific BAP address(es)  (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the BAP address in the header is same as the boundary node BAP address configured by CU2, the data should be delivered to upper layer.

Option 3: Path ID based approach

- Option 3a: (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if it is determined NOT to be BAP header rewritten based on the solution in Q4, the data should be delivered to upper layer.

- Option 3b: by checking the path ID in BAP header with the configured specific path ID(s) set (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the path ID in the header belongs to the configured path ID(s) set, the data should be delivered to upper layer.

  - This requires separate configuration on the specific path ID set

Option 4: Routing ID based approach with alternative boundary node modelling

- same as Rel16: by checking the BAP address in BAP header determine if the data should be delivered to upper layers

Option 5: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., it’s not the data to be forwarded to the upper layer. Otherwise, the data is forwarded to the upper layer. Note: DESTINATION field in BAP header is set to the BAP address of boundary node, for both the data on concatenated traffic and the data to be forwarded to the upper layer. Note that this flag is the same as one in Question 4.

Question 5: Which option do you prefer to determine whether a data should be delivered to upper layer for downstream in inter-topology routing at the boundary node?

| Companies | Option? | Comments You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | Option 4 | With the alternative boundary node modelling header re-writing always happens on the transmitting side of the topology boundary. Therefore, each IAB-node (including the boundary node) always receives BAP PDUs with BAP Routing IDs from ID space of its own topology. Therefore, normal Rel16 operation can be used for determining whether the BAP PDU should be delivered to upper layers: check whether BAP address in the BAP header is your own BAP address. |
| Kyocera | Option 5 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used, as commented in Q4. |
| CATT | Option 1a | [Rapp] I move the wording updates in option 1a here, proposed by CATT (i.e. adding the highlight part) - “For downstream concatenated traffic, if routing ID in BAP header does NOT match any “previous routing ID” in the rewriting table (i.e. not to be header rewritten) and DESTINATION field of this BAP PDU matches the BAP address of this node, the data should be delivered to upper layer.” |
| QCOM | Option 2b | In Rel-16, it is first determined whether traffic is to be delivered to upper layer. Otherwise, it is routed. In Rel-17, we can follow same approach as Rel-16 where the difference is the routing may also incur header rewriting. This deprioritizes options 1a, 2a and 3a. Following the RAN3 agreement, the BAP address is used to identify packets that have to be passed to upper layers => Option 2b |
| Lenovo | Option 2a | Option 2 can reuse the mechanism in R16. And since the DESTINATION BAP address used for intra-topology routing packets may be the same as the DESTINATION BAP address used for the packets to be delivered to the upper layer, then BAP header rewriting needs to be performed firstly for the intra-topology routing packets. [Rapp] This can be differentiated first by the ingress link, see Q3. |
| Ericsson | Option 2b | Option 2b is as in Rel.16, i.e. if the BAP destination in the BAP header matches the BAP address of this node the BAP PDU is delivered to upper layers, otherwise the packet is passed to the TX part of the BAP entity. Then the TX part of the BAP entity will be in charge of doing the BAP header rewriting. |
| LG | Option 1b with comments | It is preferred to check first whether the received packet is to be delivered to upper layer or not as in Rel-16, but only BAP address is used for this purpose in Rel-16. We think that, at least inter-topology scenario, BAP address only approach may not be sufficient and additional information would be needed to determine whether this packet is terminated at the boundary node or needs to be forwarded to another topology. In this case, we think option 1b can be considered. In our view, path ID can be additionally used to determine whether a BAP data should be delivered to upper layer as addressed by rapporteur above. For example, a specific routing ID set can be configured and those have 5 separate routing IDs which have same BAP address but separate path ID. One of them can be used for delivering to upper layer and others can be used for forwarding to another topology. Having said that, if the BAP address only is sufficient to determine whether a data should be delivered to upper layer, option 2b is also acceptable. |
| Huawei, HiSilicon | Option 1a is preferred; Option 2b is also acceptable. | Option 1a/2a/3a seem similar in this issue (i.e. data not to be header rewritten will be delivered to upper layer). Option 1b requires additional “specific routing ID configuration”. |
| vivo | Option 2b | BAP address is used to indicate that whether the packet has arrived at its destination, as specified in Rel-16, so it’s straight-forward to follow the same principle. |
| Samsung | Option 2b | As in Rel-16 baseline. |
| ZTE | Option 1a | For DL, we think the destination BAP address of the DL packet routed via the CU2 topology can be the BAP address of the boundary node, which is allocated by CU2. The boundary node first checks BAP-routing-ID mapping table upon receiving a DL concatenated packet. If the routing ID of the packet matches any “previous routing ID” in the rewriting table, the BAP header should be rewritten. Otherwise, the DL packets without BAP header re-writting are delivered to the upper layer. |
| Canon | Option 3a (or Option 5) | As a first step, the boundary node checks if the traffic is concatenated or not. In this respect, we prefer option 3a, which proposes to rely on dedicated path identifiers, to be specifically used for identifying concatenated traffic (even though Option 5 requires the use of a reserved bit within the BAP header, we think this option is acceptable for concatenated traffic identification). Then in a second step, the header is rewritten in a boundary node for concatenated traffic. Here it is required to have identified the ingress topology. In a third step, it is checked whether the traffic should be delivered to upper layer. |
| Fujitsu | Option 2b | This is the most straightforward and efficient way. Otherwise, the boundary node will have to check the BAP Header rewriting table every time when a packet is received downstream, which increases the processing delay. |
| Futurewei | Option 4 (if alternative modeling is agreed) Or Option 5 | Option 2b seems to be similar to legacy behavior and is therefore attractive. However, 2b is different than legacy behavior in that IAB node has potentially two BAP addresses (one for topology 1 and the other for topology 2). Then the spec needs to specify which of these BAP addresses needs to be checked for different parent nodes, which is different behavior compared to legacy. For Option 2b we generally need to check for different IAB node address depending on which topology the packet was received from (see response to Q 6b below) which adds further complications to the spec. Our understanding is that Option 4 essentially avoids this additional specification impact as each boundary node only belongs to a single topology. In this respect Option 4 would be identical to legacy behavior. Otherwise, Option 4 seems similar to 2b. Hence, we have some preference for Option 4 compared to Option 2b, as there would be less spec impact. Options 1b and 3b do not seem to have any significant advantage compared to 2b. Options 1a, 2a, 3a are not clear to us. It is not clear how downstream concatenated traffic is identified in these options if the BAP routing ID does not match any configuration. [Rapp] This is based on the ingress link in Q3. Option 5 also seems feasible, as the concatenation flag is absent in the BAP header, then legacy processing should apply to the packet. The existing reception procedure could be easily modified to take this into account (adding a single if clause). |
| Intel | Option 2b | We prefer to use the similar approach in Rel-16, i.e. comparing DESTINATION field of the BAP header with the BAP address of the boundary IAB-node in the corresponding topology, i.e. BAP address of CU1’s non-concatenate traffic BAP header compares with CU1 assigned boundary IAB-node’s BAP address; BAP address of CU2’s concatenated traffic BAP header compares with CU2 assigned boundary IAB-node’s BAP address. [Rapp] That’s the correct interpretation of option 2b. |

Summary:

NOTE: 1a/2a/3a are actually the same.

Option 1a: CATT, Huawei, ZTE

Option 2a: Lenovo

Option 3a: Canon

Option 1b: LG

Option 2b: QCOM, Ericsson, LG, Huawei, vivo, Samsung, Fujitsu, Intel

Option 4: Nokia, Futurewei

Option 5: Kyocera, Futurewei

To make progress, RAN2 should first try to converge on the options with majority views. It means we can first give proposals based on option 1a and 2b for the two different modellings. NOTE: This propose will also be merged into the full picture proposals in Q7.

Proposal 7a: Option 1a is proposed for modelling A: [Second majority]

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is delivered to upper layer, if it is determined NOT to be BAP header rewritten based on the solution in Q4.

Proposal 7b: Option 2b is proposed for modelling B: [Majority]

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is delivered to upper layer, if the BAP address in the header is same as the boundary node BAP address configured by CU2.

#### Question 6: The BAP address of ingress data at the boundary node

Issue 1: BAP address collision in the first topology, if the BAP address in header is set to the real destination

- There is no assumption on the CU coordination for BAP address allocation. So, the BAP address of two nodes in the two topologies may be same.

- For concatenated traffic, the BAP address in header used in the first topology should not be the real destination node, otherwise it causes the BAP address collision with non-concatenated traffic.

Question 6a: In the first topology, do you confirm that the “BAP address in the header of concatenated traffic” should not be collided with the “BAP address in the header of non-concatenated traffic”, which relies on the CU implementation to configure suitable BAP routing ID to be added? (This does not imply any specification impact.)

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia |  | This is not an issue with the alternative boundary node modelling since each node only receives BAP PDUs with BAP addresses from its own topology (real for destinations in own topology or pseudo for destinations in the other topology), i.e., the collision of BAP addresses does not happen with the alternative. |
| Kyocera | Yes | We agree that for concatenated traffic, the DESTINATIN field in BAP header is not the real destination. Otherwise, the boundary node would think it’s non-concatenated traffic. |
| CATT | Yes | It is depended on the implementation. |
| QCOM | No | From Q3, we know how to distinguish b/w concatenated and non-concatenated traffic. Assuming two packets carry same BAP address, the packet of the concatenated traffic will be header re-written while the packet of the non-concatenated traffic will not be header re-written. What’s the problem? [Rapp]: This is the issue in the first topology, not an issue after rewriting. See the above statement “- For concatenated traffic, the BAP address in header used in the first topology should not be the real destination node, otherwise it causes the BAP address collision with non-concatenated traffic.” |
| Lenovo | No | BAP address collision may not cause any problem since the BAP header rewriting will be performed by the boundary IAB node. And we only need to guarantee BAP routing ID in the header of concatenated traffic” not being collided with the “BAP routing ID in the header of non-concatenated traffic”. |
| Ericsson | Yes | We believe that this simplifies the BAP specification effort, because it would immediately differentiate the concatenated traffic from the non-concatenated traffic. Additionally, we believe that besides the possibility of address collision, the real destinations may not be used because those may collide with some other addressed of IAB nodes under the CU2 which obviously should be avoided. Hence, the specification needs to at least assume that the BAP destination of traffics coming via the CU2 may not be the real destination. |
| LG | see comments | The question is not clear and we have same doubt as QC. Having said that, based on the rapporteur’s further comments, if the question is to ask whether concatenated traffic in the first topology has the BAP address of the boundary node, not real destination in the second topology, to avoid BAP address collision with non-concatenated traffic. We can say “Yes”. However, if others, we are not sure. [Rapp]: Yes, that’s the intention. We can check the phase 2 proposal later. |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes | We agree that the concatenated traffic should first carry the boundary node’s (pseudo) BAP address instead of the real destination address, and then the boundary node can be able to replace the old BAP routing ID with the new one (that corresponds to the real destination node). |
| Samsung | Yes for downstream | Let’s take the following example: - Downstream: 1st topology contains donor-DU2, IAB-3 and IAB-2 The non-concatenated traffic is the traffic belonging to CU2 only. Concatenated traffic is routed to different destination node from the non-concatenated traffic. Thus, the statement in Q6a is right. - Upstream: 1st topology contains IAB-2 and IAB4 The non-concatenated traffic is the traffic routed to CU1. Concatenated traffic is routed to CU2. However, in 1st topology, the BAP address of both non-concatenated traffic and concatenated traffic is set by the CU1. There is no collision issue. We understand that the concatenated and non-concatenated traffic in 1st topology can use either the same or different BAP address. Specifically, “same BAP address” means both traffic flows in 1st topology are set with the same BAP address belonging to one donor DU of CU1, while “different BAP address” means both traffic flows in 1st topology are set with different BAP addresses belonging to different donor DUs of CU1. In summary, for upstream, we didn’t see any collision issue, and the statement in Q6a is not needed. [Rapp]: With same BAP address, it may cause confusion once IAB4 with DC triggers local re-routing based on BAP-address-only. Our proposal is (assuming the traffic routing across topology belonging to CU2 only is non-concatenated): For downstream, in the first topology (e.g., donor-DU2, IAB-3 and IAB-2 in above figure) , “BAP address in the header of concatenated traffic” should not be collided with the “BAP address in the header of non-concatenated traffic” |
| ZTE | No | This issue depends on the conclusion of Q4&Q5. If the operation of the boundary node is: “Header rewriting for concatenated traffic” -> “Determination to deliver to upper layer” We cannot see any problem if the “BAP address in the header of concatenated traffic” is collided with the “BAP address in the header of non-concatenated traffic”. Besides, we think the “routing ID in the header of concatenated traffic” should not be collided with the “routing ID in the header of non-concatenated traffic”. Otherwise, the boundary node would re-write the routing ID of the non-concatenated traffic. Anyway, this relies on CU1/CU2 implementation because they are aware of the potential routing ID collision. For example, for UL, CU1 determines the migrated F1-U tunnel and non-migrated F1-U tunnel, if the routing IDs of theses 2 F1-U tunnels are the same, CU1 can modify the routing ID for the migrated F1-U tunnel beforehand. |
| Canon | Yes | It depends on the selected option. A solution based on header rewriting should avoid the BAP addresses to collide. |
| Fujitsu | See comment | We agree with the first part, i.e., BAP address in the header of concatenated traffic should not be collided with the BAP address in the header of non-concatenated traffic. We support to use a pseudo/virtual BAP address to avoid collision. The pseudo/virtual BAP address configuration and procedure should be standardized (RAN3 is involved). |
| Futurewei | Not necessarily | We think this may depend on the selected option. For example, with Option 5 as proposed by Kyocera, a flag in the BAP header is used to differentiate concatenated from non-concatenated traffic. Hence, in this case there should not be any ambiguity which could lead to confusing concatenated and non-concatenated traffic, even if they share a common routing ID. Similarly, as pointed out by Nokia above, for Option 4 the scope of BAP addresses is limited to a single topology (as a boundary node belongs to only one topology). Hence, in this case there should also be no issue with address collisions either. |
| Intel | Yes with comments | From our understanding, the intention of this question is to differentiate “the concatenated traffic via CU2” with “non-concatenated traffic from CU2”, so that the traffic could terminates at the right destination. This is the scenario referring to the case 10 and 11 we commented in Q2. For downstream traffic, the destination BAP address in the BAP header of CU1’s concatenated traffic via CU2 is added at IAB-donor DU2. Hence, IAB-donor DU2 should avoid the collision between CU1’s concatenated traffic via CU2 and CU2’s non-concatenated traffic. If this is the case, we propose with following changes for clarification: the “BAP address in the header of concatenated traffic” (e.g. CU1’s traffic via CU2) should not be collided with the “BAP address in the header of non-concatenated traffic” (e.g. CU2’s traffic) |

Summary:

No proposal is needed for now.

Issue 2: Ambiguity of determination to deliver to upper layer at the boundary node, if the BAP address in header is set to the BAP address of boundary node.

One CU implementation example in the above question could be adding the boundary node’s BAP address in the header for concatenated traffic. While it may cause below issue:

- For upstream, if the BAP address in header of the ingress data is same as the boundary node’s BAP address, the R16 BAP RX operation will erroneously deliver it to upper layer, while the concatenated traffic should be forwarded to the other topology. Extra solution is needed.

- 5.2.2 Receiving operation Upon receiving a BAP Data PDU from lower layer (i.e. ingress BH RLC channel), the receiving part of the BAP entity shall: - if DESTINATION field of this BAP Data PDU matches the BAP address of this node: - remove the BAP header of this BAP Data PDU and deliver the BAP SDU to upper layers; - else: - deliver the BAP Data Packet to the transmitting part of the collocated BAP entity.

- For downstream, if the BAP address in header is same as the boundary node’s BAP address, extra solution is required to further differentiate the traffic to be header rewritten from the traffic to be delivered to upper layer. (see Q4/5)

To address the above issue 1/2 (mainly for issue 2), following options can be considered:

Option 1: Routing ID based approach

- As one CU implementation example, the BAP address in header can be set to the BAP address of boundary node for concatenated traffic.

- At the boundary node, BAP RX will treat/consider the received BAP data with specific routing ID separately as concatenated traffic (e.g. with the routing ID in header matched the BAP header rewetting table). Then, regardless the BAP address in header, BAP RX will not deliver the data to upper layer the as long as its routing ID belongs to some specific routing ID set (so that the traffic terminated at donor-DU or descendant node will not be erroneously delivered to upper layer).

- This is somehow from the summary proposal in RAN3#113 comeback #1304 “1c: RAN3 assumes that for inter-topology traffic, any BAP routing ID in the ingress topology represents an alias for the BAP routing ID for the egress topology”.

Option 2: BAP address based approach

- Only the traffic terminated at the boundary node adds the BAP address of the boundary node, while the traffic terminated at donor-DU or descendant node adds some other “pseudo BAP address” (different with the boundary node’s BAP address) for concatenated traffic.

Option 3: Path ID based approach

- As one CU implementation example, the BAP address in header can be set to the BAP address of boundary node for concatenated traffic.

- At the boundary node, BAP RX will treat/consider the received BAP data with specific path ID separately as concatenated traffic. Then, regardless the BAP address in header, BAP RX will not deliver the data to upper layer the as long as its path ID belongs to some configured specific path ID set (so that the traffic terminated at donor-DU or descendant node will not be erroneously delivered to upper layer).

Option 4: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., it’s not the data to be forwarded to the upper layer. Otherwise, the data is forwarded to the upper layer. Note: DESTINATION field in BAP header is set to the BAP address of boundary node, for both the data on concatenated traffic and the data to be forwarded to the upper layer. Note that this flag is the same as one in Question 4.

Option 5: to add if any

Question 6b: Which option do you prefer to address the ambiguity of determination to deliver to upper layer at the boundary node (i.e. issue 2 above)?

| Companies | Option? | Comments Please clarify if you have different solutions for upstream and downstream. You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | 2 | With the alternative boundary node modelling there is no ambiguity, same as in Rel16 where BAP address is used to determine when to deliver to upper layers (option 2). |
| Kyocera | Option 4 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used, as commented in Q4. |
| CATT | 1 |  |
| QCOM | 2 | In upstream, there is no delivery to upper layers at the boundary node so Q6b does not apply. [Rapp] Please see the above statement: “- For upstream, if the BAP address in header of the ingress data is same as the boundary node’s BAP address, the R16 BAP RX operation will erroneously deliver it to upper layer, while the concatenated traffic should be forwarded to the other topology. Extra solution is needed.” In downstream, this is addressed in Q5. Determination to deliver to upper layer is done first based on BAP address matching. Traffic not to be delivered to upper layer will carry a different BAP address than that configured to the boundary node in the ingress topology. |
| Lenovo | Option 1 | If the BAP header rewriting will be performed firstly for the inter-topology routing packets before“Determination to deliver to upper layer”. Then, there will no further ambiguity for the above issue. |
| Ericsson | Option 2 | As already expressed in Q4/5, the RX operations can be kept same as in legacy. The boundary IAB node will have two BAP addresses, i.e. one BAP address assigned by CU1 and one by CU2. If the incoming BAP PDU has a BAP destination that matches one of these two addresses, the packet is delivered to upper layers, otherwise it is passed to the TX part of the BAP entity for BAP header rewriting (if the BAP PDU came via the CU2). Regarding the upstream, we agree with QC that this issue will never occur in upstream. The descendant nodes will always set the BAP destination in the BAP header as the BAP address of the donor DU1, even if their traffic is re-routed by the boundary IAB node through the CU2. Hence it will never happen that the boundary IAB node passes upstream data to upper layers, unless the BAP address of the donor DU1 is the same as the BAP address of the boundary IAB node, which would obviously be an erroneous NW configuration. |
| LG | 3 or 1 | For this issue, option 1 and option 3 can be interpreted as same solution. For example, when option 1 is applied, if a specific routing ID set can be configured and those have 5 separate routing IDs which have same BAP address but separate path ID. One of them can be used for delivering to upper layer and others can be used for forwarding to another topology. With this configuration, option 1 is same with option 3. |
| Huawei, HiSilicon | 1 | Option 1 will make the UL and DL operations unified, where the only required configuration is the header rewriting table (i.e. the first column is the specific routing ID set). |
| vivo | Option 2 | See comments in Q4/5/6a. |
| Samsung | Option 2 (with rewording for downstream, while new wording is needed for upstream) | For upstream, the ambiguity comes from the case where the BAP address of donor DU of CU1 is the same as the BAP address of boundary node assigned by CU2. To avoid this, the IAB-DU of boundary IAB node should not use the BAP address assigned by CU2 to match the BAP address of the received packet when determining whether to deliver to upper layers. For downstream, the ambiguity can be resolved by setting “pseudo BAP address” different from BAP address of boundary node assigned by CU2. The above option 2 does not differentiate upstream and downstream, which means that the upstream traffic in the 1st topology (e.g., traffic from IAB4) should also set “pseudo BAP address”, which is not true in our understanding. Instead, it should set the BAP address of Donor DU1, which is the legacy method. Thus, our proposal is: - In upstream, the delivery to upper layer is avoided by not taking the BAP address of boundary IAB node assigned by CU2 into account - In downstream, Option 2 with rewording as “Only the traffic terminated at the boundary node adds the BAP address of the boundary node assigned by CU2, while the traffic terminated at donor-DU or descendant node adds some other “pseudo BAP address” (different with the boundary node’s BAP address assigned by CU2) for concatenated traffic” |
| ZTE | 1 | We think option 1 could solve the ambiguity issue if the header re-writting for concatenated traffic is performed initially. |
| Canon | Option 3 (and Option 1) | Option 1 seems to be pretty similar to Option 3. We support using specific values for path ID to discriminate concatenated from non-concatenated traffic. Then, regardless the BAP address in header, BAP RX will not deliver the data to the upper layer as long as its path ID belongs to some configured specific path ID set. |
| Fujitsu | 2 | Option 1 is unclear. If “the BAP address in header can be set to the BAP address of boundary node for concatenated traffic”, it becomes option 3. We think Option 1 should be removed. We support option 2. |
| Futurewei | Option 2 or Option 4 | Regarding Option 2, the rapporteur is technically correct, the Destination address for upstream traffic should be different that the BAP address of the boundary IAB node. However, we are not sure that any “extra solution” is needed to avoid this case. Even in Rel. 16 we would expect the Destination address for upstream traffic to correspond to a IAB DU, and as BAP addresses should normally be unique within the topology, the upstream Destination address should be different than the BAP address for the boundary IAB node (or any other IAB node within the topology). There is of course the possibility that the upstream Destination address (assigned in topology 1) could collide with the boundary IAB node address assigned in topology 2. The alternative modeling solution proposed by Nokia does not have this issue, as each boundary node belongs to only one topology. This issue could also be addressed by Option 2b in Question 5, but with a bit more standards impact. In this case the receiving IAB node should only match the Destination address of a packet to the BAP address assigned to the IAB node in the same topology from which the packet was received (this may be what SS is proposing above). However, this would seem a bit more cumbersome to capture in the spec than if we split the boundary node functionality across the two topologies (as proposed by Nokia in their alternative). |
| Intel | Option 2 | We prefer to remain Rel-16 approach, that is, “Only the traffic terminated at the boundary node adds the BAP address of the boundary node. Also, the packets will deliver to upper layer if BAP Address matches”. |

Summary:

Option 1: CATT, Lenovo, LG, Huawei, ZTE,

Option 2: Nokia, QCOM, Ericsson, vivo, Samsung, Fujitsu, Futurewei, Intel

Option 3: LG, Canon

Option 4: Kyocera, Futurewei,

To make progress, RAN2 should first try to converge on the options with majority views. It means we can first give proposals based on option 1 and 2 for the two different modellings. NOTE: This proposal will also be merged into the full picture proposals in Q7.

Proposal 8a: Option 1 is proposed for modelling A: [Second majority]

- At the boundary node, BAP RX side will treat/consider the received BAP data with specific routing ID separately. Regardless the BAP address in header, BAP RX side will not deliver the data to upper layer the as long as its routing ID belongs to some specific routing ID set.

Samsung raised one issue for option 2: For upstream, the ambiguity comes from the case where the BAP address of donor DU of CU1 is the same as the BAP address of boundary node assigned by CU2.

Rapporteur understand there is also another ambiguous case, where the BAP address of donor-DU of CU2 is same as the BAP address of boundary node assigned by CU1. But, this can be avoided by CU1 implementation when allocating boundary node BAP address and configuring the header to added.

Rapporteur understand the same issue occurs to downstream, where the “pseudo BAP address” may be same as the BAP address of boundary node assigned by CU1. But, this can be differentiated by the ingress link.

Therefore, as proposed by Samsung, the boudnary node can only check the CU1 assigned BAP address for upstream. This can be the reivised option 2’ as below.

Please note that, in the R16 specification, the intermediate node RX side will delivery data to upper layer once with matched BAP address for upstream. This was to keep aligned with downstream RX operation, even though there is no data matched in reality. For R17, boundary node can directly skip any UL RX checking BAP address for delivery to upper layer, which also needs some clarification/impacts in the spec. Companies are welcome to discuss this during running CR implementation if option 2’ is agreed.

Proposal 8b: Option 2’ is proposed for modelling B: [Majority]

- For upstream at the boundary node:

  - BAP RX side will NOT deliver data to upper layer, if the BAP address in header does not match the boundary node’s BAP address configured by CU1.

- For downstream at the boundary node:

  - For the data received from inter-topology, only the traffic terminated at the boundary node adds the BAP address of the boundary node configured by CU2, while the traffic terminated at descendant node adds some other “pseudo BAP address” (different with the boundary node’s BAP address configured by CU2).

  - For the data received form intra-topology, BAP RX side will only deliver data to upper layer if the BAP address in header matches the boundary node’s BAP address configured by CU1.

- Question 7: The full picture of BAP operations at boundary node

Basic BAP operation modelling: Header rewriting for concatenated traffic=>Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing. Modelling FFS: For concatenated traffic’s inter-topology routing, FFS on whether checking “Header rewriting for concatenated traffic” is performed first or “Determination to deliver to upper layer” is performed first.

Based on the solution to above questions and the BAP operation modeling above you prefer to, the full picture of BAP operations at the boundary node can be discussed/selected.

Approach A: “Header rewriting for concatenated traffic” is performed first

- 1. Determine and perform Header rewriting for concatenated traffic, based on the solution in Q4;

- 2. If not to be header rewritten, determination to deliver to upper layer;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID.

- 5. Mapping to BH RLC Channel

Approach B: “Determination to deliver to upper layer” is performed first

- 1. Determination to deliver to upper layer, based on the solution in Q5 and Q6;

- 2. If not to be delivered to upper layer in downstream or based on the solution in Q4, determine and perform Header rewriting for concatenated traffic, using the solution in Q4;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID.

- 5. Mapping to BH RLC Channel

Approach C: “Alternative boundary node modelling”

- 1. Determination to deliver to upper layer, based on the BAP address (same as Rel16);

- 2. If not to be delivered to upper layer, in both downstream and upstream, check the routing table to determine the next hop node (same as Rel16);

- 3. If the next hop node is not available perform re-routing to determine the new next hop node (same as Rel16);

- 4. If the next hop node is in the other topology (=concatenated traffic) or inter-donor-DU re-routing happens, perform Header rewriting; (new in Rel17)

- 5. Mapping to BH RLC Channel

Approach D: “Flag in BAP header approach”

- 1. Determination to deliver to upper layer, based on the BAP address and the flag with “0” (backward compatible with Rel-16);

- 2. If not to be delivered to upper layer, in both downstream and upstream, determine and perform Header rewriting for concatenated traffic, based on the flag with “1”;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID;

- 5. Mapping to BH RLC Channel.

Question 7: Please indicate the modeling and options you prefer to, and explain how the BAP operation works in tails to address all the previous issues?

| Company | Modell-ing FFS (A or B?) | BAP operation options |  |  | Comments Please explain how the whole BAP procedures works in details, with the option you select in Q3-7. |
| --- | --- | --- | --- | --- | --- |
|  |  | Opt. to Q4 | Opt. to Q5 | Opt. to Q6 |  |
| Nokia | C | 4 | 4 | 2 | With the alternative boundary node modelling the natural choice is Approach C where only step 4 is new. All the other steps are the same as in Rel16. |
| Kyocera | B | 5 | 5 | 4 | We prefer to define the additional flag in each BAP header, i.e., to use one of existing “R” bits. This flag is set to “1” for concatenated traffic; otherwise, it’s “0” as same with Rel-16. We also think the DESTINATIN field in BAP header is set to the BAP address of boundary node for both the concatenated traffic and the data to be delivered to the upper layer. For detailed procedure: The Rx part can determine the data delivered to the upper layer, if the DESTINATION field and the additional flag in BAP header are the boundary node’s BAP address and the additional flag is “0” respectively. Otherwise, the data is passed to the Tx part. The Tx part should first determine whether the data is non-concatenated traffic or concatenated traffic, according to the additional flag, i.e., “0” or “1” respectively. For non-concatenated traffic, Rel-16 routing is performed. For concatenated traffic, BAP header rewriting operation is performed first. Then, the routing is performed based on the proper routing table, i.e., for concatenated traffic. If the egress link is not available during the routing operation, the re-routing is triggered. In this case, the BAP header rewriting operation is performed before the re-routing operation. |
| CATT | A | 1 | 1 | 1 |  |
| QCOM | B | 1b | 2b | 2 | First, in RX part and DL, identify traffic for upper layers, which carries the boundary-node’s BAP address of the ingress link. All other traffic is forwarded to TX part. For UL, all traffic is passed to TX part. Then, in TX part, for DL, “concatenated traffic”, i.e., traffic from blue ingress link, will be header rewritten and then routed; “non-concatenated traffic”, i.e, traffic from green ingress link, will be routed without header rewriting. For UL, header rewriting and routing is one step for all traffic. Traffic, whose header is rewritten, will be forwarded to blue egress link. Traffic, whose header is not rewritten, will be forwarded to green egress link. |
| Lenovo | A | 1a | 2a | 1 | In the first topology, different sets of BAP routing ID are used for concatenated traffic and non-concatenated traffic. Upon receiving the packets at the boundary IAB node, the BAP entity firstly performs BAP header rewriting for some specific packets. After BAP header rewriting, only the DL access traffic of the boundary IAB node has its own BAP address and then be delivered to the upper layer. While for the other packets, they can be routed to the next hop based on the routing table or based on the rerouting mechanism. |
| Ericsson | B | 1a at TX side 1b at RX side | 2b | 2 | We should clarify that this question mainly applies to downstream. For upstream, we agree with QC. The step “Determination to deliver to upper layer” is not applicable to the upstream, because all the upstream traffic has as BAP destination the BAP address of DU1, hence the boundary IAB node will never deliver upstream traffic to upper layers. On the other hand, in upstream there is another step that is not present in the downstream, i.e. the determination of whether the egress link is available or not (RLF), and the determination of whether the ingress traffic should be load-balanced via CU2. |
| LG | B | 1b | 1b | 3 or 1 | For DL, when the boundary node receives a packet: 1) The BAP entity determines whether the received packet should be delivered to upper layer or not based on BAP address and path ID, if necessary. Path ID may be considered together only for concatenated traffic. (e.g., If BAP address is the BAP address of the boundary node, but the path ID is not for the boundary node, the BAP entity considers this packet is not for delivering to upper layer.) 2) Then the BAP entity recognizes concatenated traffic based on the ingress link of the received packet and rewrites the BAP header of the received packet if it is concatenated traffic. 3) After that, the BAP entity checks routing table for concatenated or non-concatenated traffic. 4) When header rewriting is needed after routing table checking, e.g., inter-topology or inter-donor-DU re-routing and so on, the BAP entity selects the new BAP routing ID for header rewriting if there are configured BAP routing ID for header rewriting and the next hop link of the rewritten BAP routing ID is available at the same time. (i.e., Don’t need to check routing table again after rewriting BAP header.) For UL, the most of steps are same as above. The difference would be that the BAP entity recognizes concatenated traffic based on the egress link of the packet. The boundary node may not need to check whether the received packet should be delivered to upper layer and all UL traffics can move to the step for routing table check. |
| Huawei, HiSilicon | A | 1a | 1a | 1 | For DL: Based on the ingress link, the non-concatenated traffic can be identified and will be handled same as R16. For concatenate traffic, if the routing ID matches the “previous routing ID” in header rewriting table, RX side do the rewriting and forward to TX side; otherwise, RX side deliver to the upper layer. At TX side, check the routing table for the data after header rewriting. For UL: If the routing ID matches the “previous routing ID” in header rewriting table, RX side do the rewriting. [RX will not deliver to upper layer if the BAP address does not match (as in R16), or if the routing ID belongs to the “previous routing ID” (new in R17). ]. TX side check the routing table for the data after header rewriting. For non-contacted traffic, legacy BAP operation can handle it same as R16. |
| vivo | B | 2b | 2b | 2 | In general: the UL concatenated traffic firstly carries the boundary node’s (pseudo) BAP address instead of the real destination address, and then the boundary node can be able to replace the old BAP routing ID with the new one (that corresponds to the real destination node). The DL concatenated traffic is differentiated via the ingress link by the boundary node and be re-written based on the BAP routing ID (same as for UL). We also agree that all non-concatenated traffic in UL will not be delivered to the upper layer of the boundary/intermediate nodes since the destination is set to the Donor-DU within the first topology, while for the concatenated UL traffic, the boundary node only needs to re-write the BAP header. So in short, there will be no determination of this step for all UL traffic. |
| Samsung | B | 1b for dowsntream, see comment for upstream | 2b | 2 with rewording for dowsntream and new wording for upstream | We can take modeling approach B as the baseline. In this approach, we notice that two header rewriting processes may be performed when rerouting is triggered. How to achieve this may need some further discussion. Opt. to Q4: - For downstream, option 1b, while for upstream, the BAP header rewritten is determined by the “previous BAP routing ID” and optional “ingress BH RLC CH” Opt. to Q6 (we assume referring to Q6b) - In upstream, the delivery to upper layer is avoided by not taking the BAP address of boundary IAB node assigned by CU2 into account - In downstream, Option 2 with rewording as “Only the traffic terminated at the boundary node adds the BAP address of the boundary node assigned by CU2, while the traffic terminated at donor-DU or descendant node adds some other “pseudo BAP address” (different with the boundary node’s BAP address assigned by CU2) for concatenated traffic” |
| ZTE | A | 1a | 1a | 1 | We basically support approach A. However, for step 3, it assums that the separate routing table(entry) specific for concatenated or non-concatenated traffic is configured. However, it is not still under discussion (e.g. Q9) without conclusion. It is suggested to modify it as follows: “3. Checking routing table to determine the egress link; |
| Canon | Based on A | 3a | 3a | 3 | - 1. Determination of concatenated or non-concatenated traffic based on the solution in Q4/5 (dedicated path identifier); - 2. The header is rewritten in a boundary node for concatenated traffic. Here it is required to have identified the ingress topology; - 3. It is checked whether the traffic should be delivered to upper layer, as in R16 specifications; - 4. Checking routing table for routing; - 5. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID; - 6. Mapping to BH RLC Channel. |
| Fujitsu | See comment | 1b | 2b | 2 | We agree with the essential part of approach B: “Determination to deliver to upper layer” is performed first”. For the detailed steps, as we answered in Q2, we think the BAP basic operation modelling needs more discussion. In some complicated scenario, e.g., RLF from SCG in the inter-CU topology redundancy scenario, some condition can be judged to choose which new BAP routing ID should be used (or which BAP header rewriting configuration to be chosen) for header re-writing, instead of doing BAP header rewriting twice. |
| Futurewei | B | 4 or 5 | 4 or 5 | 2 or 4 | Option 4 has least impact on spec and is not impacted by most of the configuration ambiguities discussed in section 2.3. However, Option 4 requires alternative modeling with separate boundary nodes. If this is not agreeable, then Option 5 would seem to have the next least spec impact. |
| Intel | B if only considers CU1’s traffic | 1b | 2b | 2 | When considering CU1’s traffic only, we agree comments from QC and Ericsson. If also considers CU2’s non-concatenate traffic, following procedure should be considered at the boundary IAB-node: Approach B’: “Determination to deliver to upper layer” is performed first - 1. Determination to deliver to upper layer, based on the solution in Q5 and Q6; - 2. If not to be delivered to upper layer in downstream or based on the solution in Q4, determine whether it’s CU1’s concatenated traffic via CU2 or CU1’s non-concatenated traffic or CU2’s non-concatenated traffic. - 3. perform Header rewriting for concatenated traffic, using the solution in Q4; - 4. Checking routing table specific for concatenated or non-concatenated traffic; - 5. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID. 6. Mapping to BH RLC Channel |

Summary:

Approach A: “Header rewriting for concatenated traffic” is performed first

- Supported by 5 companies: CATT, Lenovo, Huawei, HiSilicon, ZTE, Canon

  - CATT: 1a, 1, 1

  - Lenovo: 1a, 2a, 1

  - Huawei, ZTE: 1a, 1a, 1

Approach B: “Determination to deliver to upper layer” is performed first

- Supported by 9 companies: Kyocera, QCOM, Ericsson, LG, vivo, Samsung, Fujitsu (partially), Futurewei, Intel

  - QCOM, Ericsson, Samsung, Fujitsu, Intel: 1b, 2b, 2;

  - LG: 1b, 1b, 1;

  - Vivo: 2b, 2b, 2

Approach C: “Alternative boundary node modelling”

- Supported by Nokia and Futurewei(?).

- This requires RAN3 to revise their assumption on the boundary node.

Approach D: “Flag in BAP header approach”

- Supported by Kyocera and Futurewei.

- Actually, this can be considered as one solution for the BAP operations solution for Q4/5, rather than a total new modelling, since Kyocera also selected “Modelling B” in Q7.

Approach E: “Routing before Header rewriting for concatenated traffic”

- This is mentioned by QCOM in Q2.

- This can be considered as further detailed modelling under approach B (seems so, since QCOM also select modeling B.)

Rapporteur’s WF: In addition to the modeling A/B collecting companies’ view in phase 1, three novel modellings C/D/E are raised by one or two companies for each. In the very late phase of R17 on this fundamental issue, it is not possible to conclude on the details solution for those new modelling without collecting companies’ view in this email discussion. Rapporteur understand that concluding on one basic workable modeling with majority support is more practical than investing all fancy modeling to find the optimized one.

Therefore, rapporteur propose to make progress in RAN2 to down-select between modelling A and B.

That means approach C can be considered once RAN3 revise their assumption, and approach E can be considered if RAN2 find modeling A/B needs to be revised.

As confirmed in previous question, below is the assumption to make the full picture proposals

Basic BAP operation modelling: Header rewriting for concatenated traffic=>Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing. Similar to the legacy BAP modeling, RAN2 confirm the “Determination of delivery to upper layer” at RX should be before “Checking routing table” at TX.

Proposal 9: [To discuss] [about 9 vs. 5] RAN2 follow the majority to adopt modelling B and associated approach B in below:

Approach A: Checking “Header rewriting for concatenated traffic” is performed first, using option 1a in Q4, option 1a in Q5, Option 1 in Q6b.

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is determined as to be header rewritten and forwarded to the TX side, if the routing ID in header matches any “previous routing ID” in the rewriting table. Otherwise, the data is delivered to upper layer.

  - At BAP TX side, perform the header rewriting based on the configured rewriting table, and then perform routing and mapping to BH RLC CH.

- For upstream at the boundary node, for any received data from lower layer:

  - At BAP RX side, data will NOT be delivered to upper layer and will be forwarded to TX side, if routing ID in header matches any “previous routing ID” in the rewriting table or the BAP address in header does not match the boundary node BAP address (as in legacy).

  - At BAP TX side, the data is determined as to be header rewritten and perform the header rewriting accordingly, if routing ID in header matches any “previous routing ID” in the rewriting table; and then perform routing and mapping to BH RLC CH.

Approach B: Determination to deliver to upper layer” is performed first, using option 1b in Q4, option 2b in Q5, Option 2’ in Q6b.

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is delivered to upper layer, if the BAP address in the header is same as the boundary node BAP address configured by CU2; otherwise, the data is determined as to be header rewritten and forwarded to the TX side.

    - This requires that traffic not terminated at the boundary should not use the BAP address in header same as the boundary node BAP address configured by CU2.

  - At BAP TX side, perform the header rewriting based on the configured rewriting table, and then perform routing and mapping to BH RLC CH.

- For upstream at the boundary node, for any received data from lower layer:

  - At BAP RX side, data will NOT be delivered to upper layer and will be forwarded to TX side, if the BAP address in header does not match the boundary node BAP address configured by CU1

    - This requires that traffic terminated at donor-DU should not use the BAP address in header same as the boundary node BAP address configured by CU1.

  - At BAP TX side, the data is determined as to be header rewritten and perform the header rewriting accordingly, if routing ID in header matches any “previous routing ID” in the rewriting table; and then perform routing and mapping to BH RLC CH.

#### Question 8: Bearer mapping leftover

Based on the last meeting agreement, the legacy bearer mapping operation seems sufficient by considering the ingress to egress BH RLC channel mapping at the boundary node.

Question 8: Do you see any other essential issue in details for BAP operation on bearer mapping at the boundary node?

| Companies | Comments, if any |
| --- | --- |
| Nokia | No issues |
| CATT | No issues |
| QCOM | Yes. The bearer mapping configuration specifies mappings based on prior-hop and next-hop BAP addresses. Since BAP addresses are only unique within the scope of each topology, the bearer mapping configuration needs to indicate the ingress topology for the ingress link and the egress topology for the egress link. [Rapp] discussed in Q10c. |
| Lenovo | No issues. |
| Ericsson | No issues |
| LG | No issues so far. |
| vivo | No issues so far. This can be done based on the pre-configuration between ingress BH RLC CH and egress BH RLC CH, the same as in Rel-16. |
| Samsung | Support for 1:N remapping. If we only support 1:1 and N:1 mapping, it means that the packets conveyed via one ingress BH RLC CH in the first topology should be transmitted via the same egress BH RLC CH towards the same egress link in the second topology. In other words, this restricts the traffic mapping in the first topology to be the same as that in the second topology. How could this always be ensured, since the first topology and second topology are managed by different CUs? With 1:1 and N:1 mapping as the baseline, we understand the intention is to reuse the bearer mapping configuration defined in Rel-16. However, such restrictions limit the flexibility of the offloading, which may result in the offload failure, especially for the case of inter-donor migration. Specifically, due to the restriction of 1:1 and N:1 mapping, some traffic may not be accepted at the CU2 topology in inter-donor migration case even though CU2 could accept it as long as 1:N mapping is supported. |
| ZTE | No issues |
| Canon | For ingress to egress BH RLC channel mapping at the boundary node, it is required to indicate the IAB topology associated with a next IAB node, and the IAB topology associated with a prior IAB node. [Rapp]: Somehow related to Q10c. |
| Fujitsu | Current bearer and BH RLC channel mappings are configured independently by each donor-CU. If inter-donor path redundancy is supported there may be issues if there is no cooperation between donors. Use uplink as example, if multiple bearers are mapped to separate egress BH RLC CHs at boundary node by the non-F1-terminating donor, and they are mapped to one ingress BH RLC CH at boundary node by the F1-terminationg donor, it is impossible for the boundary node to do 1:N BH RLC channel mapping. Cooperation between two donors may be necessary. |

No proposal can be given for now. Any other issues can be based on contribution submitted to next RAN2 meeting.

- 2.3 Configuration assumptions

Inter-CU topology redundancy        Inter-CU partial migration

### 2.3.1 Ingress/egress topology ambiguity

For DL traffic, the configurations of BAP routing entry and BAP-routing-ID mapping at the boundary node need to indicate the ingress topology they refer to. For UL traffic, they need to indicate the egress topology they refer to. The indications may be implicit.

Based on the above RAN3 agreement, routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for concatenated or non-concatenated traffic (i.e. the topology).

The motivation is:

- The two parent nodes in different topology of the boundary node may have the same BAP address (e.g. IAB-1 and IAB-3), which makes the “Next Hop BAP Address” of routing entry ambiguous for concatenated and non-concatenated traffic, in upstream.

- In upstream, the routing IDs (the one of non-concatenated traffic and the new one after header rewriting of concatenated traffic) may be same for the data to be routed to the two different egress topologies. It means there may be two entries with same routing ID, if the routing tables are merged as one.

Question 9: Do you agree that the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for concatenated or non-concatenated traffic (e.g. separate routing tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | Not necessary | It is not necessarily indicated in the routing table, it can also be indicated in the header re-writing table of the boundary node. This works for the alternative boundary node modelling for both up and downstream. For upstream, (where the alternative boundary node is the same as boundary node assumed by RAN3) we assume that IAB-3 has a pseudo BAP address in the green topology (IAB-3 itself does not know the pseudo address). Then the routing is performed normally and the next hop BAP address in IAB-2 is either IAB-1 BAP address (real) or IAB-3 BAP address (pseudo). The egress link is determined based on the next hop node. The routing table then can indicate that IAB-3 address is pseudo (i.e., in different topology) and there can be an additional entry in the routing table giving the actual Routing ID to be used in header re-writing (or there can be a separate table for rewriting). For the alternative boundary node modelling, the same approach applies for downstream where the boundary node is IAB-3. |
| Kyocera | Yes | We assume the boundary IAB-node is configured with separate routing tables, i.e., one managed by a CU and another managed by another CU. Of course, the signalling details, e.g., how to configure these tables, is up to RAN3. |
| CATT |  | Whatever rewriting table need be configured in boundary IAB-node, it can be indicated in the header re-writing table of the boundary node. |
| QCOM |  | For downstream, if routing is done before header rewriting, the routing configuration needs to indicate the ingress topology. In upstream, routing and header rewriting cannot be separated. Header rewriting itself indicates the egress topology. |
| Lenovo | Yes |  |
| Ericsson |  | It is not clear what is meant with “routing table configuration” at the boundary node. Is that the legacy BH Routing configuration mapping the BAP routing IDs to next hop? Or the new table mapping previous routing ID to new routing ID? [Rapp]: Yes, it is the legacy BH Routing configuration mapping. Obviously, the legacy BH Routing Configuration provided by CU1 should be configured separately, from the new rewriting table mapping the previous BAP routing ID to the new BAP Routing IDs. However, we believe that only one BH Routing Configuration is needed, since in downstream all the traffic towards the descendant nodes pertains the CU1 BH routing configuration, and for the upstream the header rewriting itself indicates the egress topology (as mentioned by Qualcomm). |
| LG | Yes | The routing table or entry may need to be clearly identified whether it is for concatenated or non-concatenated traffic to perform properly. One routing table or separate routing table can be discussed later. |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes |  |
| Samsung |  | For the downstream, only one routing table is needed for both concatenated and non-concatenated traffic. However, before performing routing, the boundary node should determine the header rewriting according to the ingress cell group (MCG or SCG). For upstream, two routing tables or separate routing entries are need for topology of CU1 and topology of CU2, respectively. Whether the header rewriting is applied or not determines which routing table/entry is used for routing. Thus, the routing table/routing entry is configured to differentiate the applicable topology, and header rewriting determines the applicable topology for the specific packet. |
| ZTE | No | If the boundary node firstly checks BAP-routing-ID mapping table upon receiving an UL/DL packet, it can perform routing based on the following rule. - For UL non-concatenated traffic, it refers to the old routing table to determine the next hop. - For DL non-concatenated traffic, it refers to the old routing table to determine the next hop. - For DL concatenated traffic, it refers to the old routing table to determine the next hop. - For UL concatenated traffic, it refers to new routing table to determine the next hop. Here the old routing table is configured by CU1, and the new routing table is configured by CU2 for the UL packet forwarding in CU2 topology. Based on the above analysis, the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for UL concatenated traffic. For the DL, it seems not necessary to differentiate it. |
| Canon | No |  |
| Fujitsu | No | For upstream, the boundary node knows the traffic is concatenated or non-concatenated based on solution in Q4. It then knows which parent link it will forward this packet to (after possible BAP header rewriting). For downstream, there is no such need for separate routing tables/entries, since after BAP header rewriting, the new BAP routing ID will be in the same domain of non-concatenated traffic. |
| Futurewei | Yes | Generally, we think that the specifics of how to configure the routing table is the scope or RAN3, as this would be captured in RAN3 specs. However, we do not have any objection to having some indication in the routing table to indicate if a particular entry is for concatenated or non-concatenated traffic. We believe that all proposed solutions would probably need such differentiation. |
| Intel | No | As we discussed in Q4, the ingress link can be used to differentiate CU1’s concatenated traffic via CU2 and CU1’s non-concatenated traffic. BAP header rewriting will be performed before checking routing table. Hence, there’s no need to continue differentiate concatenate and non-concatenated traffic in the routing table. If the questions also refers to CU2’s traffic, we believe the routing tables at the boundary IAB-node should be configured separated by each IAB-donor CU. |

Summary:

Yes: Kyocera, Lenovo, LG, Huawei, vivo, Samsung (for UL), ZTE (for UL), Futurewei,

No: Canon, Fujitsu, Intel

Another modelling: Nokia

No clear answer: CATT, QCOM, Ericsson

For upstream, QCOM, Ericsson, Fujitsu consider no need to check the routing table for concatenated traffic, since the rewriting itself indicate the egress topology.

- [Rapp]: Skipping UL routing function for concatenated traffic changes R16 modelling that egress link selection is based on routing.

Other companies preferring “NO” did not give the solution to address the issue for upstream raised by rapporteur and the RAN3 agreement.

Therefore, rapporteur propose not to make too much modelling change compare to R16 and follow the majority and RAN3 agreement, at least for now.

Proposal 10: [8 vs. 3+] At least for upstream, the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for intra-topology or inter-topology (e.g. separate routing tables/entries).

### 2.3.2 Upstream and downstream ambiguity

Since there is no assumption on the routing ID coordination between CUs, the routing ID used in the different topologies may be same. Also, R16 assumes the routing table configuration does not differentiate the upstream and downstream.

For concatenated traffic, one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 2nd topology for upstream. There may be routing ID collision in the routing table configuration at the boundary node (i.e. one upstream routing ID may be same as one downstream routing ID).

Therefore, the routing table should be separate for upstream and downstream for concatenated traffic. Otherwise, there will be two entries with same routing IDs (i.e. one upstream routing ID may be same as one downstream routing ID.)

Question 10a: Do you agree that, for concatenated traffic, the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate routing tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | This is one benefit of the alternative boundary node modelling: there is no ambiguity. The boundary node only routes BAP Routing IDs of one topology, therefore, there is no ambiguity. The header re-writing is performed after determining the next hop node, i.e., there is no routing after the header re-writing. |
| Kyocera | Yes | Unless the routing ID coordination between CUs is assumed in RAN3, we agree that it may be necessary to differentiate between upstream or downstream, in the routing table(s). However, we assume it means BAP layer needs to identify the directions of each packet, which is different from Rel-16 principle. We assume it would need an additional behaviour in BAP routing, e.g., to identify whether each packet is on upstream or downstream traffic. |
| CATT | Yes |  |
| QCOM | See comment | If routing is done before header rewriting, the routing configuration needs to indicate the ingress topology for DL traffic. This is not needed for UL traffic. If header rewriting is done before routing, there is no need for UL routing entry, which implies that the DL routing entry does not have to carry a DL indicator. |
| Lenovo | Yes |  |
| Ericsson | No | Assuming that the issue is about “one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 1st topology for upstream.”, we wonder how likely is that this situation really occurs. This can only happen in case one routing ID assigned by CU1 for downstream traffic at the boundary IAB node, is exactly the same as one routing ID assigned by CU2 for upstream traffic at the boundary IAB node. For this to happen, it has to occur that the BAP address of the donor DU2 is exactly the same as the real BAP address of a descendant node, and also the associated PATH ID has to be the same. This seems to be a very unlikely situation, and specifying separate tables/entries for downstream/upstream for such a corner case looks like an overkill. [Rapp]: See my reply in the comment box. The 2nd topology for UL and DL are different topologies. |
| LG | No | We don’t think the following assumption is correct in the description above. “For concatenated traffic, one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 2nd topology for upstream.” In one topology, i.e., 2nd topology, why is one routing ID used for both upstream and downstream. If CU does, this is bad configuration. If one of 2nd topology above is intended to 1st topology, i.e., one of them is typo, we are still doubt whether separate for upstream and downstream is needed and think that separate routing tables/entries for concatenated or non-concatenated traffic may be sufficient as answered in Q9 above. This may be also related to which BAP address added in BAP header in the first topology. [Rapp]: See my reply in the comment box. The 2nd topology for UL and DL are different topologies. [2nd round views based on the further clarification above by the rapporteur] The IAB node 4 in green topology should be configured by the donor-CU1 and BAP routing mapping of a UL packet toward donor-DU2 (blue topology)in the IAB node 4 would be also configured by the donor-CU1. This means that even though “one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 2nd topology for upstream”, we think that separate routing tables/entries for concatenated or non-concatenated traffic would be sufficient because the IAB node 4 may be configured with the specific BAP routing ID and/or path ID to be used for the packet which is transmitted to the donor-DU2 via the boundary node. In other words, this would be related to which BAP address added in BAP header in the first topology as already mentioned above. We are not sure why the routing table should be separate for upstream and downstream for concatenated traffic. |
| Huawei, HiSilicon | Yes |  |
| vivo | No | If the BAP header of the DL concatenated traffic is set to the real boundary node’s address, and in UL it’s set to the pseudo BAP address, then it seems rare to have the same BAP routing ID as described in the above statement. So we think it may not be necessary to have a separate routing table for DL/UL respectively. |
| Samsung |  | According to Modeling approach B, we consider the routing is performed after header rewriting. Moreover, the header rewriting determines the topology used for data transmission. Thus, as long as the routing table/routing entry is configured to differentiate the applicable topology, there is no ambiguity issue. How to achieve this is a RAN3 issue. So, we propose that: Routing table/entry can be configured as topology-specific. Signaling design to achieve this depends on RAN3. |
| ZTE | No | We prefer to configure an additional routing table to the boundary node for the UL concatenated traffic forwarding in CU2 topology. For the DL concatenated traffic, DL non concatenated traffic and UL non concatenated traffic, the same routing table may be configured. |
| Canon | No |  |
| Fujitsu | No | There are ways to avoid the ambiguity. One example is that for upstream, the boundary node knows the traffic is concatenated or non-concatenated based on solution in Q4. It then knows which parent link it will forward this packet to (after possible BAP header rewriting). In this way, the upstream routing entry for the concatenated traffic is not needed. There will be no duplicate routing IDs in the routing table. In any case, there is no need to indicate whether a routing ID is for downstream or upstream in the routing table. If there are duplicate routing IDs in the table, the next-hop BAP address which is on the egress link of the current BAP entity may be chosen. |
| Futurewei | No | Although routing table configuration is strictly speaking in RAN3’s scope to decide, we are generally not in favor of having multiple routing tables as this would unnecessarily complicate the spec. Some solutions avoid Routing ID ambiguity altogether. For example, the alternative modeling proposed by Nokia does not suffer from this issue, as the scope of routing IDs is limited to a single topology. This is one of the main benefits of this alternative modeling. Also, we disagree with Kyocera’s comment above. In fact, if we go with their proposal to differentiate concatenated and non-concatenated traffic via a flag in the BAP header, then the presence (or absence) of this flag provides another field that can be used to index the routing table. In other words, entries in the routing table can be indexed by Routing ID + the concatenation flag, rather than just the Routing ID alone. Whether this is captured in the spec as a separate Routing table configuration, or an additional IE in the existing Routing table is stage 3 detail for RAN3 to decide. |
| Intel | No | If only considers CU1’s traffic (including non-concatenated and concatenated traffic via CU2), as IAB-DU2’s F1 interface is terminated at IAB-donor CU1, the routing table configuration is still determined and configured by IAB-donor CU1. Additionally, as commented in Q7, the BAP header rewriting is performed before checking the routing table configured by each IAB-donor CU. It is up to CU1’s implementation to avoid the conflict of different flows, which is the same as Rel-16. If also considers CU2’s non-concatenated traffic (e.g. IAB-DU2 also has non-concatenated access traffic of CU2), two IAB-donor CUs should configure the routing table separately. By knowing the ingress link and the corresponding topology, the boundary IAB-node can follow the routing table configured by the corresponding IAB-donor CU. |

Summary:

Yes: Kyocera, CATT, Lenovo, Huawei, ZTE(?),

No: LG, vivo, Canon, Fujitsu, Futurewei, Intel.

No (due to misunderstanding the issue?): Ericsson

No clear answer: QCOM, Samsung

Another modelling: Nokia

As in Q9, QCOM, Fujitsu mention one solution that no UL routing table checking for concatenated traffic, please see the reply in Q9.

There is no strong support on this proposal, it is proposed to come back to this issue later.

Proposal 11a: It is FFS on whether the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate routing tables/entries), after more progress in RAN2 and RAN3.

The same issue occurs for the BAP header rewriting table. For concatenate traffic, one previous routing ID for upstream may be same as one previous routing ID for downstream. If the upstream and downstream have the merged one BAP header rewriting table, there may be two entries in the table with the same Previous Routing ID.

Question 10b: Do you agree that, for concatenated traffic, the BAP header rewriting configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | Again with the alternative boundary node modelling this is not an issue: routing is performed first based on (real or pseudo) Routing ID of boundary node’s own topology and header re-writing is performed only for traffic to other topology (both for upstream and downstream). Therefore, there is no need to separate up or downstream entries. |
| Kyocera | Yes | As same with our comment in Question 10a, we agree that it may be necessary to differentiate between upstream or downstream, in the BAP header rewriting table(s), unless the routing ID coordination between CUs is assumed in RAN3. |
| CATT | Yes |  |
| QCOM | Not enough | This is fine for UL traffic. For DL traffic, there is still ambiguity since it may arrive from both topologies in case of a dual connected boundary node. [Rapp]: This seems the motivation that we have Q3. |
| Lenovo | Yes |  |
| Ericsson | No | Similar to Q10a, this seems to be a corner case. For this to happen, it needs to occur that the previous routing ID of the upstream is the same as the previous routing ID of the downstream. Hence, one condition should be that the BAP address of the donor DU1 (upstream) is the same as the pseudo BAP address assigned by the CU2 for a descendant node (downstream). Additionally, also the PATH ID has to be the same. This seems to be a very unlikely situation, and specifying separate tables/entries for downstream/upstream BH routing configuration for such a corner case looks like an overkill. |
| LG | No | same comments in Q10a |
| Huawei, HiSilicon | Yes |  |
| vivo | No | same comments in Q10a |
| Samsung |  | We agree that the header rewriting table/entry needs to differentiate the topology. How to achieve this is RAN3 issue. So, we propose Header rewriting table/entry can be configured as topology-specific. Signaling design to achieve this depends on RAN3. |
| ZTE | Yes | We think it is necessary to differentiate the UL and DL BAP routing ID in the BAP header re-writting table. |
| Canon | No |  |
| Fujitsu | Yes |  |
| Futurewei | Not necessarily | For example, the alternative modeling proposed by Nokia does not have such ambiguity as header rewriting for upstream and downstream directions occur in different boundary nodes. However, we agree that other proposed solutions could have this problem. In these cases further differentiation of configuration would be needed as mentioned by SS above. How to achieve this is within the scope of RAN3. |
| Intel | No | As we commented in Q10a, the routing table and BAP header rewriting table should be managed and configured by IAB-donor CU separately. |

Summary:

Yes: Kyocera, CATT, Lenovo, Huawei, Samsung, ZTE, Fujitsu

No: Ericsson, LG, vivo, Canon, Intel.

No clear answer: QCOM

Another modelling: Nokia, Futurewei,

There is no strong support on this proposal, it is proposed to come back to this issue later.

Proposal 11b: It is FFS on whether the header rewriting configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate routing tables/entries), after more progress in RAN2 and RAN3.

The similar issue occurs for the bearer mapping table. The parent node of the boundary node in CU2’s topology (IAB-3) may have the same BAP address with the child node of the boundary node (IAB-4). In that case, one entry of the BH RLC CH mapping table can be “BAP add1+ BH RLC ID1=>BAP add1 + BH RLC ID2”. It is ambiguous on whether it is “RLC1 of IAB-3=>RLC2 of IAB-4” as downstream or “RLC1 of IAB-4=>RLC2 of IAB-3” as upstream.

Question 10c: Do you agree that, for concatenated traffic, the BH RLC Channel Mapping Configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | Not an issue with the alternative boundary node modelling where the BAP addresses are unique (as in Rel16) (either real or pseudo but unique) and ingress BH RLC channel can be mapped to egress BH RLC channel with no ambiguity. |
| Kyocera | Yes | We think it’s necessary, unless the BAP address coordination between the CUs is assumed in RAN3. |
| CATT | Yes |  |
| QCOM | Yes | The bearer mapping includes prior hop and next hop BAP addresses. For DL, the prior hop BAP address needs to indicate to which topology it belongs. For UL, the next hop BAP address needs to indicate to which topology it belongs. There are two options: Option 1: Prior/next hop BAP address in BH RLC CH configuration include an optional topology indicator. Option 2: BH RLC CH configuration includes one DL vs. UL indicator and one topology indicator, referring to ingress for DL and to egress for UL. |
| Lenovo | Yes |  |
| Ericsson | Too early to decide | This question assumes that the legacy BH RLC channel mapping information is reused as it is, including the prio/next hop BAP address and the ingress/egress BH RLC CH ID. Therefore, this question implies that the prior hop BAP address and the next hop BAP addresses are exchanged across the two topologies as well as the ingress BH RLC channel IDs and supposedly their associated QoS requirements. Therefore, some level of coordination would be required. However, RAN2/3 has not discussed yet whether coordination between nodes is needed and whether information exchange on prior/next hops, ingress/egress BH RLC CH is needed. That should be discussed first, before addressing possible upstream/downstream collisions on the BH RLC channel mapping configuration. |
| LG | FFS | We think that this case is different from Q10a/Q10b and something additional would be needed. So, we can put this FFS. |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes |  |
| Samsung |  | We agree that the bearer mapping table/entry needs to differentiate the topology. How to achieve this is RAN3 issue. So, we propose Bearer mapping table/entry can be configured as topology-specific. Signaling design to achieve this depends on RAN3. |
| ZTE | Yes, but | The boundary node can distinguish UL/DL packet according to the prior hop, e.g. the prior hop is its child node or parent node. So the issue raised by moderator does not exist. However, there is another BH RLC channel mapping ambiguity issue. We take the following figure as an example. Suppose the UL concatenated traffic and UL concatenated traffic from IAB node 4 are associated with different routing ID while the bearer mapping table is in the following form, Prior-hop BAP address; Ingress BH RLC channel ID; Next-hop BAP address; Egress BH RLC channel ID. A4; 1; A1; 1. A4; 1; A2; 2. If BAP address of IAB node 1 and IAB node 2 are the same as Ax, the bearer mapping table would be as follows: Prior-hop BAP address; Ingress BH RLC channel ID; Next-hop BAP address; Egress BH RLC channel ID. A4; 1; Ax; 1. A4; 1; Ax; 2. In this case, for UL concatenated traffic, even if the boundary node know the next-hop is IAB-node 2, it cannot figure out whether egress BH RLC channel 1 or 2 should be used. [Rapp] Egress link will be determined first by routing. This issue should be address by Q9. |
| Canon | No | However, for ingress to egress BH RLC channel mapping at the boundary node, it is required to indicate the IAB topology associated with a next IAB node, and the IAB topology associated with a prior IAB node. |
| Fujitsu | Yes |  |
| Futurewei | Not necessarily | As mentioned by Nokia above, the alternative modeling may not have this issue. For other proposed solution it may be necessary to include an indication of the topology of the BAP address, as indicated by other companies above. |
| Intel | No | As we commented in previous questions, we think the BH RLC Channel Mapping configuration can be topology specific. The packets will look into the BH RLC channel mapping configuration after BAP header rewriting. |

Summary:

Yes: Kyocera, CATT, QCOM, Lenovo, Huawei, vivo, Samsung, ZTE, Fujitsu

No: Canon, Intel.

- [Rapp] Canon, Intel seem refer the “topology specific bearer mapping configuration”. So for the boundary node concatenated traffic, topology-wise is the same thing as direction-wise.

No clear answer (Or FFS): Ericsson, LG

Another modelling: Nokia, Futurewei,

Based on the majority view, below proposal can be the RAN2 assumption for now.

Proposal 11c: The BH RLC Channel Mapping Configuration (or its entry) at the boundary node should be able to indicate the ingress topology, i.e. whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate tables/entries). Signaling details can be discussed later by RAN2 and RAN3.

### 2.3.3 Header rewriting configuration

In the basic BAP operation modelling, BAP may perform the BAP header rewriting twice or for two purposes: one for header rewriting for inter-topology routing and one for header rewriting for re-routing (e.g. inter-topology re-routing or inter-donor-DU re-routing).

Question 11: Do you agree that, the BAP header rewriting configuration for inter-topology routing and the BAP header rewriting configuration for re-routing at the boundary node should be two separate rewriting tables?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | With the alternative boundary node modelling routing (and potential re-routing) is performed first to determine the next hop node and only if that next hop requires header re-writing (inter-topology or inter-donor-DU), then perform the re-writing. Thus, the header re-writing ONLY happens once. |
| Kyocera | FFS | We agree the BAP header rewriting may be performed twice, i.e., one for inter-CU routing and another for re-routing. But we still wonder if one BAP header rewriting configuration can handle the two cases, which will depend on the outcome of Question 9, while we tend to assume it’s clearer to have the separate tables. |
| CATT | No | A unified rewriting table is better. |
| QCOm | Yes | We need two routing tables. Let’s assume, we had only one header rewriting table. Case 4: UL traffic is forwarded from green to blue topology: - UL packet includes virtual BAP routing ID for blue donor-DU in green topology. - Header rewriting table includes entry: Virtual BAP routing ID for blue donor DU  Real BAP routing ID of blue donor DU. - If the blue parent link has RLF, the boundary node needs to apply header rewriting for re-routing. This could be done in a consecutive rewriting step. In this case, the table has to include an entry: Real BAP routing ID for blue donor DU  Real BAP routing ID of green donor DU. This entry cannot be in the same table since the ingress BAP routing IDs refer to a different topologies and may accidently collide. Alternatively, the first header rewriting step is not executed. In this case, the header rewriting table needs to include an entyr: Virtual BAP routing ID for blue donor DU  Real BAP routing ID of green donor DU. This implies that the BAP header rewriting table has two conflicting entries: Virtual BAP routing ID for blue donor DU  Real BAP routing ID of blue donor DU. Virtual BAP routing ID for blue donor DU  Real BAP routing ID of green donor DU. Analogue reasoning can be applied for Case 3. Consequence: The boundary node needs to perform two consecutive steps using two different tables: - Header rewriting + routing based on header rewriting table 1 - If egress link is not available, perform header rewriting based on table 2. |
| Lenovo | No | Rerouting is the local behavior of intermediate IAB node, there is no need to introduce BAP header rewriting table for local rerouting. |
| Ericsson | No | We should aim at minimizing specification differences between inter-topology routing and inter-donor DU re-routing, unless there is a clear issue. For example, it is not clear in the example from Qualcomm why the IAB node should first perform a BAP header rewriting due to inter-topology routing and then overwrite the new header again to route the packet back to CU1. Shouldn´t the IAB node first check the availability of the link towards the blue topology before performing any inter-topology routing? |
| LG | No | We think that it is reasonable to have BAP header rewriting table for inter-topology routing, but BAP header rewriting table for local re-routing is not needed. One additional field in the routing table is sufficient for BAP header rewriting by local re-routing. |
| Huawei, HiSilicon | Yes | Agree with QCOM on the analyses of case4. |

| vivo | No | We think the two tables can be merged into a single one (such as Header Rewriting Configuration), and we also agree that the node should first check the availability of the chosen link before selecting the re-written BAP routing ID. |
| --- | --- | --- |
| Samsung | Yes |  |
| ZTE | No | We think it is necessary to have a header re-writing for concatenated traffic for inter-topology routing. However, for the inter-topology traffic re-routing, the legacy routing table containing the routing ID and next hop BAP address can be reused. For example, the legacy routing table may be checked to determine the availability of egress link. Suppose the re-rotuing is necessary, the BAP routing ID is re-written based on the selected routing entry in the legacy routing table. |
| Canon | No |  |
| Fujitsu | Yes |  |
| Futurewei | No | We think there are potentially several approaches to address rewriting without introducing separate routing tables. |
| Intel | Yes | We prefer two separate BAP rewriting tables to be configured at the boundary IAB-node. Considering the BAP rewriting table for inter-donor DU rerouting, both original BAP address and new BAP address are real BAP addresses allocated by IAB-donor CU. However, in the BAP rewriting table for inter-donor CU migration, the original BAP address is the “pseudo BAP address” which is allocated by another CU’s topology. It is possible that this “pseudo BAP address” may be conflict with the real BAP address in the first topology. Also, the boundary IAB-node will only use either one of them to update the BAP header. It would be simpler to use separate BAP rewriting table for different rerouting purpose. |

Summary:

Yes: QCOM, Huawei, Samsung, Fujitsu, Intel

No: Nokia, CATT, Lenovo, Ericsson, LG, vivo, ZTE, Canon, Futurewei

FFS: Kyocera

There is no clear majority view. This can be discussed later after we finalize whether we will perform header rewriting twice.

- 2.4 Phase 2 proposals checking

### 2.4.1 Proposals on re-routing

Proposal 1: The pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on previous BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF, congestion or type2 indication, etc.), similar to R16.

Question Ph2-1: Is the proposal 1 above agreeable/acceptable?

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Acceptable | We’d like to clarify that this proposal applies to both upstream and downstream. Our understanding is that the behavior can be triggered by BH RLC/type 2 indication in UL direction, and by congestion (DL HbH flow control feedback at least for now) in DL direction. |
| Nokia | Agree |  |
| QCOM | See comment | P1 is fine for the non-boundary node. For inter-topology traffic at boundary node, if routing is done after header rewriting, the availability of the next hop is determined by the REWRITTEN header and not the PREVIOUS header. Further, “similar to Rel16” is misleading. We should ether say “as in Rel16” or leave it out. |
| Kyocera | Agree |  |
| Fujitsu | See comment | After some thinking, we think this proposal is not so straightforward. For a dual-connected boundary node, if RLF happens at one parent link, there may be inter-donor-DU re-routing (case 3 and case 4 in 2.2.1). There may be separate BAP header rewriting configurations for inter-topology routing and re-routing at this boundary node. The simplest condition is that if RLF happens in topology redundancy scenario, the BAP header rewriting for re-routing is triggered, i.e., one header rewriting table is chosen. P1 does not address this scenario. |
| Samsung | Agree with comments | P1 is meant to refer to boundary node. However, P1 is only valid under the assumption that the header rewriting operation has been performed first (this could be what QC are saying as well). The current proposal uses “previous BAP routing ID”, which may cause a misunderstanding that the BAP header rewriting for re-routing is performed before header rewriting operation. So we propose the following rewording: The pre-condition/criteria of “BAP header rewriting for re-routing” at the boundary node is that there is no available next hop found based on previous BAP routing ID (which could be either a new BAP routing ID after header rewriting, or previous routing ID if header rewriting is not applicable) and based on BAP address in the routing table (e.g. due to BH RLF, congestion or type2 indication, etc.), similar to R16. |
| Ericsson | Agree | Not clear the concern from QC and Samsung. We assume that the proposal applies to the upstream, right? Hence in the upstream those conditions (BH RLF, type-2 RLF, congestion) should be fulfilled before applying the header rewriting. Hence, when determining whether the egress link is available or not, it should be used the previous routing ID, not the new routing ID. In the downstream, there are no conditions for the BAP header rewriting, i.e. all packets coming from the target topology should be subject to BAP header rewriting. |

Proposal 2: Once the “BAP header rewriting based re-routing” is triggered, BAP routes the data to the available egress link, if there is matched entry in the Header rewriting table for re-routing. FFS egress link selection is performed before or after header rewriting (can be discussed in running CR).

Question Ph2-2: Is the proposal 2 above agreeable/acceptable?

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Agree |  |
| Nokia |  | We may not need a separate Header rewriting table. We would prefer to amend the routing table with the required information, see TP in Annex A. |
| QC | Acceptable | On Nokia’s comment: Header rewriting for re-routing is only supported on UL, where there are at most 2 parent nodes available. Therefore, if one link is “not available” as next hop, the node can only select the 2nd link. No “routing” is needed anymore. |
| Kyocera | Agree |  |
| Fujitsu | Agree |  |
| Samsung | Agree |  |
| Ericsson | Agree |  |

Proposal 3: For upstream at the boundary node, it is supported to local re-route the data to CU1 topology, due to RLF/congestion/etc., which was from CU1 topology and originally to be forwarded via CU2 topology.

Question Ph2-3: Is the proposal 3 above agreeable/acceptable?

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Acceptable | We are fine to go with the majority view. |
| Nokia | Agree | We think that the routing and possible rerouting should always be done first and only perform the header rewriting if the selected egress link requires header rewriting, With that Case 4 is no special case. |
| QCOM | Agree |  |
| Kyocera | Agree |  |
| Fujitsu | Agree | This is a normal inter-donor-DU re-routing case. |
| Samsung | Agree |  |
| Ericsson | To be discussed | What is the reason to call it “local routing” if anyhow the upstream traffic is transmitted to the original source CU? If the link towards CU2 is not available (due to RLF, BH RLF, congestion, etc.) we should not have any BAP header rewriting. It seems unnecessary to first perform local routing to CU2, and then another local routing back to CU1. Hence in this case, we should not have any local routing. Simply, if the upstream link towards CU2 is not available, the upstream traffic should be transmitted to the CU1 straight away, i.e. no local routing needed. |

Proposal 4: For upstream at the boundary node, it is FFS on whether to merge the BAP header rewriting operations/steps for inter-topology routing and inter-topology re-routing.

Question Ph2-4: Is the proposal 4 above agreeable/acceptable?

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Agree |  |
| Nokia | Disagree | Only one header rewriting step after routing and rerouting. As we have indicated the routing and rerouting should be performed first and header rewriting only if the selected egress link requires it. With Approach C, there are no FFSs, see TP in Annex A. |
| QC | See comment | It is not clear what “merge” refers to. We propose rewording: “… it is FFS on whether the BAP header rewriting operations due to inter-topology routing and inter-topology re-routing are done in one step”. |
| Kyocera | Agree |  |
| Fujitsu | Agree | We support the merge of the BAP header rewriting operations/steps for inter-topology routing and inter-topology re-routing. Fine with FFS. |
| Samsung | Agree |  |

### 2.4.2 Proposals on inter-topology routing

Proposal 5: For downstream, the boundary node is able to identify/differentiate the traffic routed via inter-topology and the traffic routed via intra-topology, based on the ingress link.

Question Ph2-5: Is the proposal 5 above agreeable/acceptable?

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Agree |  |
| Nokia |  | With Approach C, for downstream the boundary node is the parent node of the inter-topology link and the question is obsolete. |
| QCOM | Proposal needs clarification | The proposal only applies to descendent node traffic since traffic for the boundary node itself is delivered to upper layers without being routed, neither intra- nor inter-topology. We need to capture this since it leads to misunderstandings in the proposals below. We therefore propose the following rewording: Proposal 5: For downstream, the boundary node is able to identify/differentiate the descendent-node traffic to be routed via inter-topology and the traffic routed via vs. intra-topology, based on the ingress link. |
| Kyocera | - | We still prefer the unified solution for both upstream and downstream, as much as possible, e.g., using “Flag in BAP header approach”. |
| Fujitsu | Agree |  |
| Samsung | Agree |  |

Proposal 6a: Option 1a is proposed for modelling A: [Second majority]

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is determined as to be header rewritten and forwarded to the TX side, if the routing ID in header matches any “previous routing ID” in the header rewriting table.

  - At BAP TX side, perform the header rewriting based on the configured header rewriting table.

- For upstream at the boundary node, for any received data from lower layer:

  - At BAP RX side, no data will be delivered to upper layer, based on the solution in Q6b.

  - At BAP TX side, the data is determined as to be header rewritten and perform the header rewriting accordingly, if the routing ID in header matches any “previous routing ID” in the rewriting table.

Proposal 6b: Option 1b is proposed for modelling B: [Majority]

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is determined as to be header rewritten and forwarded to the TX side, if the data is determined as NOT to be delivered to upper layer, based on the solution in Q5.

  - At BAP TX side, perform the header rewriting based on the configured header rewriting table.

- For upstream at the boundary node, same as Proposal 6a.

Question Ph2-6: Are you fine to compromise to agree either P6a or P6b?

| Companies | OK to compromise as either 6a or 6b? | Prefer 6a or 6b? | Comments |
| --- | --- | --- | --- |
| vivo | OK | 6b | This follows the legacy procedure. |
| Nokia | NOK |  | Approach C is much simpler since Rel16 behavior can be used, see Annex A. |
| QC | See comment | See comment | Based on proposal 5, “inter-topology data” refers to traffic that is routed to the other topology. This excludes data to be delivered to upper layers which are not routed. If you leave out data for upper layers in P6b, it becomes the same as 6a. Apart from that, 6a seems to be to introduce a different mechanism to deliver packets to upper layers for the two ingress links of a DC boundary node. - If packets arrive in topology 1: Delivery to upper layers based on BAP address match. - If packets arrive in topology 2: Delivery to upper layers if BAP RT ID was NOT found in BAP-rewriting table. This doesn’t make sense. What is the motivation? |
| Kyocera | - | - | We still think “Flag in BAP header approach” is beneficial. But if RAN2 agrees to choose one from 6a/6b, we think 6b is better. |
| Fujitsu | OK | 6b | 6b is more aligned with R16 operation. |
| Samsung | OK | 6b | Our strong preference remains Proposal 6b, with the additional clarification that for upstream, the BAP header rewriting is determined by the “previous BAP routing ID” and optional “ingress BH RLC CH” (which should be added into P6b, as FFS). We would like to additionally suggest to discuss P9 first. |
| Ericsson | See comments | 6b | If there is no clear majority, we believe that the solution that requires the least spec impact should be selected, which is option 6b, because in 6b the legacy RX operations are not impacted. |

Proposal 7a: Option 1a is proposed for modelling A: [Second majority]

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is delivered to upper layer, if it is determined NOT to be BAP header rewritten based on the solution in Q4.

Proposal 7b: Option 2b is proposed for modelling B: [Majority]

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is delivered to upper layer, if the BAP address in the header is same as the boundary node BAP address configured by CU2.

Question Ph2-7: Are you fine to compromise to agree either P7a or P7b?

| Companies | OK to compromise as either 7a or 7b? | Prefer 7a or 7b? | Comments |
| --- | --- | --- | --- |
| vivo | Ok | 7b |  |
| Nokia | NOK |  | With Approach C Rel16 operation is used, no changes are needed on BAP RX side. |
| QC | No | 7b | Again, based on proposal 5, “inter-topology data” refers to traffic that is routed to the other topology. This excludes data to be delivered to upper layers which are not routed. As above, 7a seems to be to introduce a different mechanism to deliver packets to upper layers for the two ingress links of a DC boundary node. - If packets arrive in topology 1: Delivery to upper layers based on BAP address match. - If packets arrive in topology 2: Delivery to upper layers if BAP RT ID was NOT found in BAP-rewriting table. This doesn’t make sense. What is the motivation? |
| Kyocera | OK | 7b | We still think 7b + “Flag in BAP header approach” works well and a unified solution for both upstream and downstream. |
| Fujitsu | OK | 7b | 7b is aligned with R16 operation. |
| Samsung | OK | 7b | P7b remains our strong preference. But, similar to our answer to the previous question, we suggest to discuss P9 first. |
| Ericsson | See comments | 7b | If there is no clear majority, we believe that the solution that requires the least spec impact should be selected. With option 7b, the legacy RX operations are not impacted, which makes option 7b the preferred solution. |

Proposal 8a: Option 1 is proposed for modelling A: [Second majority]

- At the boundary node, BAP RX side will treat/consider the received BAP data with specific routing ID separately. Regardless the BAP address in header, BAP RX side will not deliver the data to upper layer the as long as its routing ID belongs to some specific routing ID set.

Proposal 8b: Option 2’ is proposed for modelling B: [Majority]

- For upstream at the boundary node:

  - BAP RX side will NOT deliver data to upper layer, if the BAP address in header does not match the boundary node’s BAP address configured by CU1.

- For downstream at the boundary node:

  - For the data received from inter-topology, only the traffic terminated at the boundary node adds the BAP address of the boundary node configured by CU2, while the traffic terminated at descendant node adds some other “pseudo BAP address” (different with the boundary node’s BAP address configured by CU2).

  - For the data received form intra-topology, BAP RX side will only deliver data to upper layer if the BAP address in header matches the boundary node’s BAP address configured by CU1.

Question Ph2-8: Are you fine to compromise to agree either P8a or P8b?

| Companies | OK to compromise as either 8a or 8b? | Prefer 8a or 8b? | Comments |
| --- | --- | --- | --- |
| vivo | ok | 8b |  |
| Nokia | NOK |  | With Approach C Rel16 operation is used, no changes are needed. Header rewriting is done always by tx-side of the inter-topology link, therefore, BAP receiver only sees Routing IDs of its own topology, see TP in Annex A. |
| QCOM | No | 8b | We don’t like 8a since it introduces a different behavior to pass packets to upper layers for each ingress topology. This does not make sense. |
| Kyocera | - | - | We still think “Flag in BAP header approach” is beneficial. But if RAN2 agrees to choose one from 8a/8b, we think 8b is better. |
| Fujitsu | OK | 8b | 8b is aligned with R16 and current RAN3 agreement. |
| Samsung | OK | 8b | P8b is our strong preference (revised original Option 2). For upstream in P8b, we propose to delete “if the BAP address in header does not match …”. The reason is that if the upstream traffic at boundary IAB node belongs to CU1’s topology, then the BAP address in header is Donor DU1’s address, which is definitely different from the BAP address of boundary node assigned by CU1. And, similar to our answer to the previous two questions, we suggest to discuss P9 first. |
| Ericsson | See comments | 8b, but for upstream… | Similar to questions above, option 8b requires the least changes in the spec. With option 8b, the legacy RX operations are not impacted However, for the upstream, we wonder if it really exists the case in which the boundary IAB node is the destination node of the upstream traffic (unless the boundary IAB node is an IAB-donor node which does not make sense). |

Proposal 9: [To discuss] [about 9 vs. 5] RAN2 follow the majority to adopt modelling B and associated approach B in below:

Approach A: Checking “Header rewriting for concatenated traffic” is performed first, using option 1a in Q4, option 1a in Q5, Option 1 in Q6b.

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is determined as to be header rewritten and forwarded to the TX side, if the routing ID in header matches any “previous routing ID” in the rewriting table. Otherwise, the data is delivered to upper layer.

  - At BAP TX side, perform the header rewriting based on the configured rewriting table, and then perform routing and mapping to BH RLC CH.

- For upstream at the boundary node, for any received data from lower layer:

  - At BAP RX side, data will NOT be delivered to upper layer and will be forwarded to TX side, if routing ID in header matches any “previous routing ID” in the rewriting table or the BAP address in header does not match the boundary node BAP address (as in legacy).

  - At BAP TX side, the data is determined as to be header rewritten and perform the header rewriting accordingly, if routing ID in header matches any “previous routing ID” in the rewriting table; and then perform routing and mapping to BH RLC CH.

Approach B: Determination to deliver to upper layer” is performed first, using option 1b in Q4, option 2b in Q5, Option 2’ in Q6b.

- For downstream at the boundary node, based on the proposal 5 in Q3, for any received data from inter-topology identified by the ingress link:

  - At BAP RX side, the data is delivered to upper layer, if the BAP address in the header is same as the boundary node BAP address configured by CU2; otherwise, the data is determined as to be header rewritten and forwarded to the TX side.

    - This requires that traffic not terminated at the boundary should not use the BAP address in header same as the boundary node BAP address configured by CU2.

  - At BAP TX side, perform the header rewriting based on the configured rewriting table, and then perform routing and mapping to BH RLC CH.

- For upstream at the boundary node, for any received data from lower layer:

  - At BAP RX side, data will NOT be delivered to upper layer and will be forwarded to TX side, if the BAP address in header does not match the boundary node BAP address configured by CU1

    - This requires that traffic terminated at donor-DU should not use the BAP address in header same as the boundary node BAP address configured by CU1.

  - At BAP TX side, the data is determined as to be header rewritten and perform the header rewriting accordingly, if routing ID in header matches any “previous routing ID” in the rewriting table; and then perform routing and mapping to BH RLC CH.

Question Ph2-9x: Are you fine to compromise that RAN2 only down select between approach A and B as working assumption (while other approach C/D/E can be revisited if need in the future).

| Companies | OK to compromise as either A or B? | Comments |
| --- | --- | --- |
| Vivo | ok |  |
| Nokia | No | By selecting Approach C all issues are solved and no FFSs are left, see Annex A. |
| QCOM | No | As discussed above. |
| Kyocera | - | We still think “Flag in BAP header approach” is beneficial. |
| Fujitsu | OK | Support Approach B. |
| Samsung | OK | Support Approach B. |
| Ericsson | OK |  |

Question Ph2-9y: Are you fine to compromise to agree either approach A or B [This is to converge on the wording/details of approach A and B]?

| Companies | OK to compromise as either approach A or B? | Prefer approach A or B? | Comments |
| --- | --- | --- | --- |
| Vivo | ok | B |  |
| Nokia | No |  | Approach C much simpler and faster to specify, see Annex A. |
| QCOM | No | B | Approach A is not sensible and it has not be motivated. |
| Kyocera | - | - | If RAN2 agrees only Approach A/B are viable solutions, we prefer B. |
| Fujitsu | OK | B |  |
| Samsung | OK | B | We suggest to delete “This requires …” for upstream, since in reality the received traffic at the boundary node in upstream always belongs to CU1’s topology, and will not be delivered to the upper layers, which is the same as legacy case. In addition, the description of Approach B indicates that the TX side operation is same for both upstream and downstream. In order to simplify the description, we propose the following text: - At BAP RX side, the data is delivered to upper layer, if the BAP address in the header of received packet is same as the boundary node BAP address configured by CU of the ingress topology (DL: ingress topology can be either CU1’s topology or CU2’s topology; UL: ingress topology is always CU1’s topology); Otherwise, the data is passed on to the TX side - At BAP TX side, perform the header rewriting based on the configured rewriting table if routing ID in header matches any “previous routing ID” in the rewriting table, and then perform routing and mapping to BH RLC CH. |
| Ericsson | No | B, but for upstream… | Whether to compromise or not depends also on the specification impact of the solution. Approach B has much less impact, given that the legacy RX operations are basically not impacted. Regarding upstream, as also mentioned by Samsung and in our reply to P8, we do not see any case in which the boundary node should pass ingress traffic to higher layers, unless the boundary node is an IAB-donor which does not make sense. |

### 2.4.3 Proposals on configurations

Proposal 10: [8 vs. 3+] At least for upstream, the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for intra-topology or inter-topology (e.g. separate routing tables/entries).

Question Ph2-10: Is the proposal 10 above agreeable/acceptable?

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Agree |  |
| Nokia |  | By combining the routing table and header rewriting table, each entry in the routing table indicates whether the Next Hop node is in the same topology or different topology (requiring header rewriting), see Annex A. |
| QCOM | See rewording | Let’s phrase it simpler: P10’: The boundary node must know which topology the BAP-routing-ID and the next-hop BAP address in the routing configuration refers to. After we have agreed on P10, we can discuss how the boundary node can figure this out from a routing configuration is receives. |
| Kyocera | Agree |  |
| Fujitsu | Disagree | We still prefer to select egress link by looking up the BAP header rewriting configuration. It will avoid adding one new routing table or new indication in routing table. No need to change R16 F1AP signaling for this aspect. |
| Samsung | See rewording | This proposal is technically agreeable. However, it seems to imply that the indication of intra-topology or inter-topology should be given when configuring the routing table, which is related to signaling design. In addition, together with P11a, there is an impression of indicating the upstream and downstream direction when configuring the routing table. We want to avoid this since 1) in Rel-16, a routing table is applicable for both DL and UL, 2) the differentiation of upstream and downstream is not future-proof in case multiple connectivity is supported for the boundary IAB node in future. Combining P10 and P11a, We understand that the rapporteur aims to differentiate four cases: 1) inter-topo + upstream, 2) inter-topo + downstream, 3) intra-topo + upstream, and 4) intra-topo + downstream. The essential goal is to differentiate the applicable egress topology, i.e., CU1 topology or CU2 topology. In other words, as long as the egress topology is determined, the applicable routing table can be determined. Thus, we propose the following changes: Proposal 10: [8 vs. 3+] At least for upstream, the routing table configuration (or its entry) at the boundary node should be able to achieve differentiation of egress topology indicate whether it is for intra-topology or inter-topology (e.g. separate routing tables/entries). Signalling design depends on further RAN2 and RAN3 progress. |
| Ericsson | FFS | Before agreeing on ambiguity/collisions issues in the BH routing table, BH Rewriting table, RLC BH mapping, etc we would like to better understand the scenarios/situations in which this may occur. This topic has also dependency on the BAP modeling discussed above, so we suggest first agreeing on that, and then discuss other issues. |

Proposal 11a: It is FFS on whether the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate routing tables/entries), after more progress in RAN2 and RAN3.

Question Ph2-11a: Is the proposal 11a above agreeable/acceptable? (P11a is only intended to keep it as FFS)

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Agree | We can discuss the issue later. |
| Nokia |  | One of the benefit of Approach C is that routing table configuration remains the same for both upstream and downstream, see Annex A. |
| QCOM |  | May already be addressed after we have handled P10’. |
| Kyocera | Agree |  |
| Fujitsu | Disagree | There is no need to indicate whether a routing ID is for downstream or upstream in the routing table. If there are duplicate routing IDs in the table, the next-hop BAP address which is on the egress link of the current BAP entity may be chosen. That is, based on whether the TX part is at the MT BAP entity or DU BAP entity, the boundary node will choose the corresponding next hop. For example, if there are duplicate routing IDs, one for downstream and one for upstream, they will have different next hop BAP addresses, one on the egress link of MT BAP entity, another on the egress link of DU BAP entity. There is no room for confusion. |
| Samsung | See comments | Already addressed by our rewording of P10. |

Proposal 11b: It is FFS on whether the header rewriting configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate routing tables/entries), after more progress in RAN2 and RAN3.

Question Ph2-11b: Is the proposal 11b above agreeable/acceptable? (P11b is only intended to keep it as FFS)

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Agree |  |
| Nokia | Disagree | One of the benefit of Approach C is that also the header rewriting (e.g., as part of routing table) configuration remains the same for both upstream and downstream, see Annex A. |
| QCOM | See rewording | We can make more progress, by stating: P11b’: The boundary node must know to which topology each BAP routing ID in the header rewriting configuration refers to. After we have agreed on P11b’, we can discuss how the boundary node can figure this out from the configuration received. |
| Kyocera | Agree |  |
| Fujitsu | Agree |  |
| Samsung | See rewording | Proposal 11b: It is FFS on whether the header rewriting configuration (or its entry) at the boundary node should be able to achieve differentiation of the ingress topology indicate whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate routing tables/entries), after more progress in RAN2 and RAN3. |

Proposal 11c: The BH RLC Channel Mapping Configuration (or its entry) at the boundary node should be able to indicate the ingress topology, i.e. whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate tables/entries). Signaling details can be discussed later by RAN2 and RAN3.

Question Ph2-11c: Is the proposal 11c above agreeable/acceptable?

| Companies | Agree/Acceptable? | Comments |
| --- | --- | --- |
| vivo | Agree |  |
| Nokia | Disagree | With Approach C, no changes are needed for BH RLC channel mapping configuration. |
| QCOM | See rewording | We can make more progress, by stating: P11c’: The boundary node must know to which topology each BH RLC channel in the BH RLC Channel Mapping Configuration refers to. After we have agreed on P11c’, we can discuss how the boundary node can figure this out from the configuration received. |
| Kyocera | Agree |  |
| Fujitsu | Agree |  |
| Samsung | See rewording | Proposal 11c: The BH RLC Channel Mapping Configuration (or its entry) at the boundary node should be able to achieve differentiation between the ingress and egress topology indicate the ingress topology, i.e. whether it is for upstream toward inter-topology or downstream from inter-topology (e.g. separate tables/entries). Signaling details can be discussed later by RAN2 and RAN3. |
| Ericsson | FFS | Before agreeing on ambiguity/collisions issues in the BH routing table, BH Rewriting table, RLC BH mapping, etc we would like to better understand the scenarios/situations in which this may occur. This topic has also dependency on the BAP modeling discussed above, so we suggest first agreeing on that, and then discuss other issues. |

# Conclusion and proposals

Based on the above summary, following proposals are given.

# Reference

- R2-2107251	Summary of [Post114-e][075][eIAB] Open Issues on Re-routing	Huawei, HiSilicon

- R2-2107170	BAP-layer traffic processing at the boundary node	Qualcomm

- R2-2107253	Inter-donor routing, local rerouting and RLF indication for R17-IAB	Huawei,

- R2-2107445	Inter-donor CU Topology migration	 Intel

- R2-2107648	Open issues on (re-)routing	Fujitsu

- R2-2108026	Topology adaptation enhancements	Samsung

- R2-2108141	Discussion on inter-donor topology redundancy	ZTE, Sanechips

- R2-2108422	Boundary IAB node behaviour for partial and full inter-donor migration	Ericsson

- R2-2108482	Solutions for Inter-Donor Routing and Bearer Mapping 	Futurewei Technologies

# Annex A 38.340 Text proposal for Approach C

With Approach C, there are two boundary nodes for each inter-topology link, one for upstream and one for downstream. Therefore, all the nodes are in one topology only, also the boundary nodes, which simplifies the configuration. The boundary node is always at the transmitting side of the inter-topology link:

-	Boundary IAB node (upstream): IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a parent DU. (IAB-2 in the figure) (same as RAN3)

-	Boundary IAB node (downstream): IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a child DU. (IAB-3 in the figure)

This modelling change simplifies the BAP (and F1AP) spec significantly:

- Only Routing section requires changes; other sections work as in Rel16.

- Same spec works for both upstream and downstream, same as in Rel16.

- Inter-donor-DU rerouting works for both intra and inter-topology.

- Even multiple boundary nodes, i.e., where the routing path goes from one topology to another and back several times, is also supported when configured by CUs.

- Straight forward reuse of Rel16, no FFSs

- 5.2.1.3 Routing The BAP entity performs routing based on: - the BH Routing Configuration derived from an F1AP message as specified in TS 38.473 [5]. Each entry of the BH Routing Configuration contains: - a BAP Routing ID consisting of a BAP address and a BAP path identity, which is indicated by BAP Routing ID IE, - a Next Hop BAP Address which is indicated by Next-Hop BAP Address IE, - optionally a New BAP Routing ID for inter-topology routing, and - optionally a list of Alternative BAP addresses. Each upstream destination BAP address is optionally mapped to a list of Alterative BAP addresses for inter-donor-DU rerouting. For a BAP Data PDU to be transmitted, BAP entity shall: - if the BAP Data PDU corresponds to a BAP SDU received from the upper layer, and - if the BH Routing Configuration has not been (re)configured by F1AP after the last (re)configuration of defaultUL-BH-RLC-Channel by RRC: - select the egress link on which the egress BH RLC channel corresponding to defaultUL-BH-RLC-Channel is configured as specified in TS 38.331 [3] for non-F1-U packets; - else if there is an entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, whose BAP path identity is the same as the PATH field, and whose egress link corresponding to the Next Hop BAP Address is available: - select the egress link corresponding to the Next Hop BAP Address of the entry; - if the selected entry contains New BAP Routing ID: - replace the DESTINATION field and the PATH field in the header of this BAP PDU such that the DESTINATION field is set to the BAP address of New Routing ID of the selected entry, and the PATH field is set to the BAP path identity of New Routing ID of the selected entry NOTE 1: An egress link is not considered to be available if the link is in BH RLF. NOTE 2: For each combination of a BAP address and a BAP path identity, there should be at most one entry in the BH Routing Configuration. There could be multiple entries of the same BAP address in the BH Routing Configuration. - else if there is at least one entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available: - select an entry from the BH Routing Configuration whose BAP address is the same as the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available; - select the egress link corresponding to the Next Hop BAP Address of the entry selected above; - if the selected entry contains New BAP Routing ID: - replace the DESTINATION field and the PATH field in the header of this BAP PDU such that the DESTINATION field is set to the BAP address of New Routing ID of the selected entry, and the PATH field is set to the BAP path identity of New Routing ID of the selected entry. - else if at least one Alternative BAP address is configured for the destination BAP address and there is at least one entry in the BH Routing Configuration whose BAP address matches one of the Alternative BAP addresses, and whose egress link corresponding to the Next Hop BAP Address is available: - select an entry from the BH Routing Configuration whose BAP address is the same as one of the configured Alternative BAP addresses, and whose egress link corresponding to the Next Hop BAP Address is available; - if the selected entry contains New BAP Routing ID: - replace the DESTINATION field and the PATH field in the header of this BAP PDU such that the DESTINATION field is set to the BAP address of New Routing ID of the selected entry, and the PATH field is set to the BAP path identity of New Routing ID of the selected entry; - else: - replace the DESTINATION field and the PATH field in the header of this BAP PDU such that the DESTINATION field is set to the BAP address of Routing ID of the selected entry, and the PATH field is set to the BAP path identity of Routing ID of the selected entry. - select the egress link corresponding to the Next Hop BAP Address of the entry selected above;