3GPP TSG-RAN WG2 Meeting #116 electronic	               R2-21xxx

Electronic meeting, 1st-12th November 2021

Agenda Item:	8.4.3

Source: 	Huawei, HiSilicon

Title:	Summary of [Post115-e][088][eIAB] inter-CU routing open issues

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of email discussion.

- [Post115-e][088][eIAB] inter-CU routing open issues (Huawei)

Scope: Address the listed open points for inter-CU routing:

- What’s the BAP address added in BAP header in the first topology (i.e. the BAP address of ingress data at the boundary node);

- How to differentiate the concatenated traffic and non-concatenated traffic;

- How to determine whether a data should be delivered to upper layer (for downstream);

- How to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology).

Intended outcome: Report

Deadline: Long

1st round deadline: initial comments collection. October 12th, 0900 UTC

2nd round deadline: one week for proposal checking and TP/Running CR discussion if possible. October 21th, 0900 UTC

RAN2 related agreements:

- For inter-donor-DU re-routing, support the “previous routing ID to new routing ID” BAP header rewriting. - For intra-CU cases, Support inter-donor-DU re-routing at least in the scenarios of NR-DC among donor-DUs, inter-donor-DU recovery and inter-donor-DU migration. - Support inter-CU re-routing, i.e. IAB-node re-routes the data to its original donor-CU via the alternative BAP path over the topology in target CU. - As baseline, support the 1:1 and N:1 mapping from “previous routing ID” to “new routing ID” for BAP header rewriting at the boundary node, in inter-CU routing. - As baseline, support the 1:1 and N:1 mapping from “ingress BH link + ingress BH RLC ID” to “egress BH link + egress BH RLC ID” for bearer mapping at the boundary node, in inter-CU routing.

RAN3 related agreements:

- One common inter-donor topology transport mechanism should be defined for all scenarios where traffic between a donor and an IAB DU traverses the network under another donor; FFS whether it is possible to achieve a common signaling design for all scenarios - RAN3 prefers that the boundary node processes access traffic in the same manner as the non-boundary access IAB-node. - RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic. - FFS: In addition to BAP header rewriting, performs routing and bearer mapping in the same manner as the non-boundary intermediate IAB-node. - RAN3 assumes that the boundary node has only one BAP address in each topology. - RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers. - For DL traffic, the configurations of BAP routing entry and BAP-routing-ID mapping at the boundary node need to indicate the ingress topology they refer to. For UL traffic, they need to indicate the egress topology they refer to. The indications may be implicit.

# Discussion

- 2.1 Terminologies clarification

Terminologies to be used in this email discussion:

Inter-CU topology redundancy        Inter-CU partial migration

Pleases note the terminology clarification is only for the scope of email discussion, not for the specification usage.

Boundary IAB node: IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a parent DU.

Alternative modelling:

Boundary IAB node (upstream): IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a parent DU. (IAB-2 in the figure) (same as above)

Boundary IAB node (downstream): IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a child DU. (IAB-3 in the figure)

First topology: the topology fragment before the boundary node for a traffic. (For example, the 1st topology is IAB-2 and IAB-4 for upstream; donor-DU2, IAB-3 and IAB-2 for downstream [blue nodes].)

Second topology: the topology fragment after the boundary node for a traffic. (For example, the 2nd topology is IAB-2, IAB-3 and donor-DU2 for upstream; IAB-2 and IAB-4 for downstream [green nodes].)

Concatenated traffic: the traffic routing across two topologies which belong to different CUs. (For example the brown line and purple line in above figure). Please note this terminology includes both descendant node traffic and access traffic. You can understand this as the traffic from/to neighbor CU2’s topology, which needs to be handled by boundary node.

Non-concatenated traffic: the traffic routing across one topology which belong to one CU. (For example the orange line and red line in above figure, as supported in R16). Please note this terminology includes both descendant node traffic and access traffic. You can understand this as the traffic from/to CU1’s topology, which needs to be handled by boundary node.

Inter-topology routing: the BAP routing across two topologies which belong to different CUs, i.e. the “inter-CU routing” used before.

Inter-topology re-routing: BAP re-routes the data (finally terminated to its original donor-CU) via the alternative BAP path over the topology in target CU, i.e. the “inter-CU re-routing” used before.

- 2.2 Unified BAP operations

The goal of this section is to design the solution for BAP operations to cover inter-topology routing, inter-topology re-routing and inter-donor-DU re-routing. We strive to comprehensive solution for those 3 cases and strive to similar solution for upstream and downstream, if possible.

### 2.2.1 Assumption and modelling clarification

Before discussing the solution, the traffic/data needs to be handled by the BAP of boundary IAB-node is clarified below. (As agreed by R3, boundary node’s access traffic will be handled same as R16, which is not discussed below)

For upstream at boundary node:

- Case 1: Data from CU1 topology to be forwarded to CU1 topology; [i.e. non-concatenated traffic’s intra-topology routing]

- Case 2: Data from CU1 topology to be forwarded to CU2 topology; [i.e. concatenated traffic’s inter-topology routing]

- Case 3: Data from CU1 topology originally to be forwarded to CU1 topology, but re-routed to CU2 topology due to RLF. [i.e. non-concatenated traffic’s inter-topology re-routing]

- Case 4: Data from CU1 topology originally to be forwarded to CU2 topology, but re-routed to CU1 topology due to RLF. [i.e. concatenated traffic’s inter-topology re-routing]

- Case 5: Data from CU1 topology originally to be forwarded to donor-DU1 of CU1 topology, but re-routed to donor-DU5 of CU1 topology due to RLF. [i.e. intra-topology inter-donor-DU re-routing]

For downstream at boundary node:

- Case 6: Data from CU1 topology to be delivered to upper layer;

- Case 7: Data from CU2 topology to be delivered to upper layer;

- Case 8: Data from CU1 topology to be directly forwarded to next hop node in the CU1 topology;

- Case 9: Data from CU2 topology to be BAP header rewritten and then forwarded to next hop node in the CU1 topology;

For the alternative modelling the possible cases are:

For upstream at boundary node (IAB-2):

Case 1-5: same as above

For downstream at boundary node (IAB-3):

- Case 6: Does not exist (downstream boundary node only handles data from its own CU);

- Case 7: Data from CU2 topology to be delivered to upper layer;

- Case 8: Data from CU2 topology to be directly forwarded to next hop node in the CU2 topology;

- Case 9: Data from CU2 topology to be BAP header rewritten and then forwarded to next hop node in CU1 topology;

It is important to clarify the assumption to ensure that companies on the same page on the BAP operation basic modelling when discussing the solutions later.

Below is the straight forward basic BAP operation modelling assumed by rapporteur:

Basic BAP operation modelling: Header rewriting for concatenated traffic=>Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing.

Below is the straightforward basic BAP operation modelling assumed for the alternative modelling:

Basic BAP operation modelling: Checking routing table for both non-concatenated and concatenated traffic => Header rewriting for concatenated traffic or for re-routing

NOTE: For clarification on “Checking routing table specific for concatenated or non-concatenated traffic”, please see the Question 9 below.

Clarification 1: Similar to the R16 local re-routing, for inter-topology re-routing and inter-donor-DU re-routing, BAP should “Check routing table” first before “Header rewriting for re-routing”, to try the primary path first and then do the re-routing if no primary path available.

- 5.2.1.3 Routing For a BAP Data PDU to be transmitted, BAP entity shall: - if the BAP Data PDU corresponds to a BAP SDU received from the upper layer, and - if the BH Routing Configuration has not been (re)configured by F1AP after the last (re)configuration of defaultUL-BH-RLC-Channel by RRC: - select the egress link on which the egress BH RLC channel corresponding to defaultUL-BH-RLC-Channel is configured as specified in TS 38.331 [3] for non-F1-U packets; - else if there is an entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, whose BAP path identity is the same as the PATH field, and whose egress link corresponding to the Next Hop BAP Address is available: - select the egress link corresponding to the Next Hop BAP Address of the entry; NOTE 1: An egress link is not considered to be available if the link is in BH RLF. - else if there is at least one entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available: - select an entry from the BH Routing Configuration whose BAP address is the same as the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available; - select the egress link corresponding to the Next Hop BAP Address of the entry selected above;

Question 1: Do you confirm/agree the pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on previous BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF or type2 indication, etc.), similar to R16?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | Yes | BAP header re-writing should always be based on routing, i.e., re-writing is done only if (re)routing requires header re-writing. |
| Kyocera | Yes | We assume the re-routing is performed after the routing fails. |
| CATT | Yes | BAP header rewriting should be performed after R16 routing and rerouting mechanism can not work. If there is no available next hop found based on previous BAP address and routing ID, BAP layer will perform BAP rewriting for rerouting. |

Clarification 2: Similar to the legacy BAP modeling, the “Determination of delivery to upper layer” at RX should be before “Checking routing table” at TX.

With the above clarification 1/2, there is one more FFS point on the basic BAP operation modeling, which may be the key point of this discussion.

Modelling FFS: For concatenated traffic’s inter-topology routing, FFS on whether checking “Header rewriting for concatenated traffic” is performed first or “Determination to deliver to upper layer” is performed first.

The below Question 4/5 depends on this modeling FFS, and it will be discussed/down-selection in Question 7 in the end.

Therefore, before going into the details, rapporteur asks companies to share views on the above assumptions and modelling.

Question 2: Companies are asked for comments on the above data cases (case 1-9) and/or on the BAP basic operation modeling, if any? (Mainly on the basic modeling and its clarification 1/2, while the modelling FFS will be asked and down-selected later in Q7)

| Companies | Comments, if any |
| --- | --- |
| Nokia | As indicated above, for the alternative boundary node modelling, - cases 1-5 are valid, case 6 does not exist and for cases 7-9 data is always from CU2 (since the downstream boundary node belongs to topology of CU2) - BAP operation is much simpler and basically the same as for Rel16: check whether data is delivered to upper layers, if not check routing table and perform header re-writing if needed |
| Kyocera | We agree that the Rx operation, i.e., to determine the delivery to upper layer, is performed before the Tx operation, as same with Rel-16. Just small clarifications: In Case 5, “donor-DU5” is not depicted in the figure, but we assume it’s another donor-DU of CU1. [Rapp]: Yes In Case 9, we assume the data is forwarded to the next hop node in “CU1 topology” [Rapp]: Yes, updated. |
| CATT | We agree that the BAP basic operation modeling and the 9 cased. |

### 2.2.2 Solution discussion

Following discussion is to address the open issues from last RAN2 meeting for inter-topology routing:

- What’s the BAP address added in BAP header in the first topology (i.e. the BAP address of ingress data at the boundary node);

- How to differentiate the concatenated traffic and non-concatenated traffic;

- How to determine whether a data should be delivered to upper layer (for downstream);

- How to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology).

#### Question 3: How to differentiate the concatenated traffic and non-concatenated traffic for downstream

NOTE: for upstream case, this can be automatically identified by the solution in Q4, since only concatenated traffic performs BAP header rewriting.

Motivation 1 for this question: based on the RAN3 agreement below, BAP operations is different for concatenated traffic and non-concatenated traffic, at least for downstream, including the Determination to deliver to upper layer and Header rewriting for concatenated traffic only.

- RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic.

Motivation 2 for this question:

- There is no assumption on the CU coordination to avoid the routing ID/BAP address/path ID collision, used in the two ingress topologies at the boundary node for downstream. Namely that the downstream ingress BAP data at the boundary node from two topologies may have the same routing ID/BAP address/path ID.

- So, any solution based on routing ID/BAP address/Path ID in below Question 4/5 only works if the boundary node can identify whether the data is concatenated traffic or non-concatenated traffic first (i.e. from which topology). Then, the new behavior of the solution in Question 4/5 only applies to the concatenated traffic rather than non-concatenated traffic.

  - For example, in the BAP address based solution in Q5, the boundary node needs to determine whether the downstream traffic is concatenated traffic or non-concatenated traffic, so that it knows which BAP address (i.e. CU1 configured or CU2 configured) to be used for determination to deliver to upper layer. Note that R3 agreed the two BAP addresses of boundary node may be same.

Based on the above motivations, the boundary node needs to differentiate the concatenated traffic and non-concatenated traffic for downstream (i.e. identify the ingress link).

One example way: For topology redundancy scenario, it is clear that data received from MCG is the non-concatenated while from SCG is the concatenated traffic. For partial migration case, boundary node only has the MCG with all downstream traffic are concatenated traffic. For sure, this assumes the IAB-node knows it is the boundary IAB node (e.g. by the configuration with two BAP address or the configuration of BAP header rewriting table for inter-topology routing).

Question 3: Do you agree that the ingress link is able to identify/differentiate the concatenated and non-concatenated traffic at the boundary node for downstream?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | With the alternative boundary node modelling, IAB-nodes in the other topology are allocated “pseudo” BAP address (or Routing ID). For “pseudo” Routing IDs, the boundary node is configured with the actual Routing IDs (of the other topology) to be used in BAP header re-writing. Thus, the boundary node can identify the concatenated traffic based the “pseudo” Routing ID. The same applies to both downstream and upstream. Thus, with the alternative boundary node modelling, the BAP operation is same as in Rel16: - based on Routing ID, the BAP PDU is routed to next hop node. - the only new thing is that If the next hop node (based on the pseudo Routing ID) is in the other topology, the BAP performs header re-writing. |
| Kyocera | No | We’re still not sure if it’s always possible to identify/differentiate the concatenated traffic and non-concatenated traffic based on the ingress link, considering various deployment scenarios, e.g., combination with CP/UP separation etc. We would prefer a unified solution for both downstream and upstream. So, we think the two cases should be discussed together. |
| CATT | No | We think it is not reliable to identify/differentiate the concatenated and non-concatenated traffic by ingress link. |

#### Question 4: How to determine whether the BAP header of a data should be rewritten

NOTE: Somehow Question4 and Question5 are related and address similar issue, and somehow rely on the Q3/6.

The issue: For concatenated traffic, boundary node need to know whether any BAP data needs to perform the BAP header rewriting for inter-topology routing purpose.

Option 1: Routing ID based approach

- Option 1a: by checking the routing ID in BAP header with the configured BAP Header rewriting table (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if routing ID in header matches any “previous routing ID” in the rewriting table, the BAP header should be rewritten.

  - This kind of “specific routing ID” is naturally configured by the first column of the BAP Header rewriting table.

  - This is not mutual exclusion with using “pseudo BAP address”.

- Option 1b: (assuming “Determination to deliver to upper layer” first in the modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 1a is used.

Option 2: BAP address based approach

- Option 2a: by checking the BAP address in BAP header with the configured specific BAP address(es) set (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if the BAP address in the header belongs to the configured BAP address(es) set (e.g. the pseudo BAP addresses), the BAP header should be rewritten.

  - This requires separate configuration on the specific BAP address set.

- Option 2b: (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 2a is used.

Option 3: Path ID based approach

- Option 3a: by checking the path ID in BAP header with the configured specific path ID(s) set (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if the path ID in the header belongs to the configured path ID(s) set, the BAP header should be rewritten.

  - This requires separate configuration on the specific path ID set.

- Option 3b: (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 3a is used.

Option 4: Routing ID based approach with alternative boundary node modelling

- for both upstream and downstream, by checking the Routing ID in BAP header with the configured routing table determine the Next Hop IAB-Node (same as Rel16)

  - if the Next Hop Node is in the other topology (e.g., indicated in the routing table or in a separate header re-writing table), perform BAP header re-writing (new in Rel17)

    - replace the “previous routing ID” in the BAP header with the “new routing ID”.

This alternative uses “pseudo” BAP Routing IDs.

Option 5: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., for BAP header rewriting.

Question 4: Which option do you prefer to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology) for concatenated traffic in inter-topology routing at the boundary node?

| Companies | Option? | Comments Please clarify if you have different solutions for upstream and downstream. You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | Option 4 | Option 4 with alternative boundary node modelling uses Rel16 approach for both upstream and downstream in the corresponding boundary nodes. Header rewriting is performed only if Routing ID based routing indicates that Next Hop Node is in the other topology. |
| Kyocera | Option 5 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used to determine whether the data should be performed with BAP header rewriting, as well as whether the data should be forwarded to the upper layer. We assume it’s common solution for both upstream and downstream. We wonder if RAN3’s assumption, “1a: RAN3 assumes that the boundary node has only one BAP address in each topology”, meant Option 2a is no longer applicable. |
| CATT | Option 1a | The procedure of “Header rewriting for concatenated traffic first” seems more fluently. |

#### Question 5: How to determine whether a data should be delivered to upper layer (for downstream)

NOTE: Somehow Question4 and Question5 are related and address similar issue, and somehow rely on the Q3/6.

NOTE: For upstream, see the issue discussed in Q6.

The issue: For downstream, the boundary node needs to know whether any received BAP data should be delivered to upper layer.

Clarification on one RAN3 assumption: “RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers.”

- It means ONLY the data with matched BAP address can be delivered to upper layer. But, it does NOT means any data with matched BAP address should be delivered to upper layer. Somehow, the downstream traffic terminated at the boundary node has to add the BAP address in header same as the boundary node’s BAP address.

- Namely that FFS whether path ID part is additionally used to determine whether a BAP data should be delivered to upper layer.

- Anyway, this is RAN3 assumption, RAN2 should have our final conclusion.

Option 1: Routing ID based approach

- Option 1a: by checking the routing ID in BAP header with the configured BAP Header rewriting table (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if routing ID in BAP header does NOT match any “previous routing ID” in the rewriting table (i.e. not to be header rewritten) and DESTINATION field of this BAP PDU matches the BAP address of this node, the data should be delivered to upper layer.

  - This kind of “specific routing ID” is naturally configured by the first column of the BAP Header rewriting table.

- Option 1b: by checking the routing ID in BAP header with some configured routing ID set (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the routing ID in the BAP header belongs to the configured routing ID set, the data should be delivered to upper layer.

  - This requires separate configuration on the specific routing ID set

Option 2: BAP address based approach

- Option 2a: (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if it is determined NOT to be BAP header rewritten based on the solution in Q4, the data should be delivered to upper layer.

- Option 2b: by checking the BAP address in BAP header with the configured specific BAP address(es)  (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the BAP address in the header is same as the boundary node BAP address configured by CU2, the data should be delivered to upper layer.

Option 3: Path ID based approach

- Option 3a: (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if it is determined NOT to be BAP header rewritten based on the solution in Q4, the data should be delivered to upper layer.

- Option 3b: by checking the path ID in BAP header with the configured specific path ID(s) set (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the path ID in the header belongs to the configured path ID(s) set, the data should be delivered to upper layer.

  - This requires separate configuration on the specific path ID set

Option 4: Routing ID based approach with alternative boundary node modelling

- same as Rel16: by checking the BAP address in BAP header determine if the data should be delivered to upper layers

Option 5: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., it’s not the data to be forwarded to the upper layer. Otherwise, the data is forwarded to the upper layer. Note: DESTINATION field in BAP header is set to the BAP address of boundary node, for both the data on concatenated traffic and the data to be forwarded to the upper layer. Note that this flag is the same as one in Question 4.

Question 5: Which option do you prefer to determine whether a data should be delivered to upper layer for downstream in inter-topology routing at the boundary node?

| Companies | Option? | Comments You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | Option 4 | With the alternative boundary node modelling header re-writing always happens on the transmitting side of the topology boundary. Therefore, each IAB-node (including the boundary node) always receives BAP PDUs with BAP Routing IDs from ID space of its own topology. Therefore, normal Rel16 operation can be used for determining whether the BAP PDU should be delivered to upper layers: check whether BAP address in the BAP header is your own BAP address. |
| Kyocera | Option 5 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used, as commented in Q4. |
| CATT | Option 1a |  |

#### Question 6: The BAP address of ingress data at the boundary node

Issue 1: BAP address collision in the first topology, if the BAP address in header is set to the real destination

- There is no assumption on the CU coordination for BAP address allocation. So, the BAP address of two nodes in the two topologies may be same.

- For concatenated traffic, the BAP address in header used in the first topology should not be the real destination node, otherwise it causes the BAP address collision with non-concatenated traffic.

Question 6a: In the first topology, do you confirm that the “BAP address in the header of concatenated traffic” should not be collided with the “BAP address in the header of non-concatenated traffic”, which relies on the CU implementation to configure suitable BAP routing ID to be added? (This does not imply any specification impact.)

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia |  | This is not an issue with the alternative boundary node modelling since each node only receives BAP PDUs with BAP addresses from its own topology (real for destinations in own topology or pseudo for destinations in the other topology), i.e., the collision of BAP addresses does not happen with the alternative. |
| Kyocera | Yes | We agree that for concatenated traffic, the DESTINATIN field in BAP header is not the real destination. Otherwise, the boundary node would think it’s non-concatenated traffic. |
| CATT | Yes | It is depended on the implementation. |

Issue 2: Ambiguity of determination to deliver to upper layer at the boundary node, if the BAP address in header is set to the BAP address of boundary node.

One CU implementation example in the above question could be adding the boundary node’s BAP address in the header for concatenated traffic. While it may cause below issue:

- For upstream, if the BAP address in header of the ingress data is same as the boundary node’s BAP address, the R16 BAP RX operation will erroneously deliver it to upper layer, while the concatenated traffic should be forwarded to the other topology. Extra solution is needed.

- 5.2.2 Receiving operation Upon receiving a BAP Data PDU from lower layer (i.e. ingress BH RLC channel), the receiving part of the BAP entity shall: - if DESTINATION field of this BAP Data PDU matches the BAP address of this node: - remove the BAP header of this BAP Data PDU and deliver the BAP SDU to upper layers; - else: - deliver the BAP Data Packet to the transmitting part of the collocated BAP entity.

- For downstream, if the BAP address in header is same as the boundary node’s BAP address, extra solution is required to further differentiate the traffic to be header rewritten from the traffic to be delivered to upper layer. (see Q4/5)

To address the above issue 1/2 (mainly for issue 2), following options can be considered:

Option 1: Routing ID based approach

- As one CU implementation example, the BAP address in header can be set to the BAP address of boundary node for concatenated traffic.

- At the boundary node, BAP RX will treat/consider the received BAP data with specific routing ID separately as concatenated traffic (e.g. with the routing ID in header matched the BAP header rewetting table). Then, regardless the BAP address in header, BAP RX will not deliver the data to upper layer the as long as its routing ID belongs to some specific routing ID set (so that the traffic terminated at donor-DU or descendant node will not be erroneously delivered to upper layer).

- This is somehow from the summary proposal in RAN3#113 comeback #1304 “1c: RAN3 assumes that for inter-topology traffic, any BAP routing ID in the ingress topology represents an alias for the BAP routing ID for the egress topology”.

Option 2: BAP address based approach

- Only the traffic terminated at the boundary node adds the BAP address of the boundary node, while the traffic terminated at donor-DU or descendant node adds some other “pseudo BAP address” (different with the boundary node’s BAP address) for concatenated traffic.

Option 3: Path ID based approach

- As one CU implementation example, the BAP address in header can be set to the BAP address of boundary node for concatenated traffic.

- At the boundary node, BAP RX will treat/consider the received BAP data with specific path ID separately as concatenated traffic. Then, regardless the BAP address in header, BAP RX will not deliver the data to upper layer the as long as its path ID belongs to some configured specific path ID set (so that the traffic terminated at donor-DU or descendant node will not be erroneously delivered to upper layer).

Option 4: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., it’s not the data to be forwarded to the upper layer. Otherwise, the data is forwarded to the upper layer. Note: DESTINATION field in BAP header is set to the BAP address of boundary node, for both the data on concatenated traffic and the data to be forwarded to the upper layer. Note that this flag is the same as one in Question 4.

Option 5: to add if any

Question 6b: Which option do you prefer to address the ambiguity of determination to deliver to upper layer at the boundary node (i.e. issue 2 above)?

| Companies | Option? | Comments Please clarify if you have different solutions for upstream and downstream. You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | 2 | With the alternative boundary node modelling there is no ambiguity, same as in Rel16 where BAP address is used to determine when to deliver to upper layers (option 2). |
| Kyocera | Option 4 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used, as commented in Q4. |
| CATT | 1 |  |

#### Question 7: The full picture of BAP operations at boundary node

Basic BAP operation modelling: Header rewriting for concatenated traffic=>Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing. Modelling FFS: For concatenated traffic’s inter-topology routing, FFS on whether checking “Header rewriting for concatenated traffic” is performed first or “Determination to deliver to upper layer” is performed first.

Based on the solution to above questions and the BAP operation modeling above you prefer to, the full picture of BAP operations at the boundary node can be discussed/selected.

Approach A: “Header rewriting for concatenated traffic” is performed first

- 1. Determine and perform Header rewriting for concatenated traffic, based on the solution in Q4;

- 2. If not to be header rewritten, determination to deliver to upper layer;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID.

- 5. Mapping to BH RLC Channel

Approach B: “Determination to deliver to upper layer” is performed first

- 1. Determination to deliver to upper layer, based on the solution in Q5 and Q6;

- 2. If not to be delivered to upper layer in downstream or based on the solution in Q4, determine and perform Header rewriting for concatenated traffic, using the solution in Q4;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID.

- 5. Mapping to BH RLC Channel

Approach C: “Alternative boundary node modelling”

- 1. Determination to deliver to upper layer, based on the BAP address (same as Rel16);

- 2. If not to be delivered to upper layer, in both downstream and upstream, check the routing table to determine the next hop node (same as Rel16);

- 3. If the next hop node is not available perform re-routing to determine the new next hop node (same as Rel16);

- 4. If the next hop node is in the other topology (=concatenated traffic) or inter-donor-DU re-routing happens, perform Header rewriting; (new in Rel17)

- 5. Mapping to BH RLC Channel

Approach D: “Flag in BAP header approach”

- 1. Determination to deliver to upper layer, based on the BAP address and the flag with “0” (backward compatible with Rel-16);

- 2. If not to be delivered to upper layer, in both downstream and upstream, determine and perform Header rewriting for concatenated traffic, based on the flag with “1”;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID;

- 5. Mapping to BH RLC Channel.

Question 7: Please indicate the modeling and options you prefer to, and explain how the BAP operation works in tails to address all the previous issues?

| Company | Modell-ing FFS (A or B?) | BAP operation options |  |  | Comments Please explain how the whole BAP procedures works in details, with the option you select in Q3-7. |
| --- | --- | --- | --- | --- | --- |
|  |  | Opt. to Q4 | Opt. to Q5 | Opt. to Q6 |  |
| Nokia | C | 4 | 4 | 2 | With the alternative boundary node modelling the natural choice is Approach C where only step 4 is new. All the other steps are the same as in Rel16. |
| Kyocera | B | 5 | 5 | 4 | We prefer to define the additional flag in each BAP header, i.e., to use one of existing “R” bits. This flag is set to “1” for concatenated traffic; otherwise, it’s “0” as same with Rel-16. We also think the DESTINATIN field in BAP header is set to the BAP address of boundary node for both the concatenated traffic and the data to be delivered to the upper layer. For detailed procedure: The Rx part can determine the data delivered to the upper layer, if the DESTINATION field and the additional flag in BAP header are the boundary node’s BAP address and the additional flag is “0” respectively. Otherwise, the data is passed to the Tx part. The Tx part should first determine whether the data is non-concatenated traffic or concatenated traffic, according to the additional flag, i.e., “0” or “1” respectively. For non-concatenated traffic, Rel-16 routing is performed. For concatenated traffic, BAP header rewriting operation is performed first. Then, the routing is performed based on the proper routing table, i.e., for concatenated traffic. If the egress link is not available during the routing operation, the re-routing is triggered. In this case, the BAP header rewriting operation is performed before the re-routing operation. |
| CATT | A | 1 | 1 | 1 |  |

#### Question 8: Bearer mapping leftover

Based on the last meeting agreement, the legacy bearer mapping operation seems sufficient by considering the ingress to egress BH RLC channel mapping at the boundary node.

Question 8: Do you see any other essential issue in details for BAP operation on bearer mapping at the boundary node?

| Companies | Comments, if any |
| --- | --- |
| Nokia | No issues |
| CATT | No issues |

- 2.3 Configuration assumptions

Inter-CU topology redundancy        Inter-CU partial migration

### 2.3.1 Ingress/egress topology ambiguity

For DL traffic, the configurations of BAP routing entry and BAP-routing-ID mapping at the boundary node need to indicate the ingress topology they refer to. For UL traffic, they need to indicate the egress topology they refer to. The indications may be implicit.

Based on the above RAN3 agreement, routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for concatenated or non-concatenated traffic (i.e. the topology).

The motivation is:

- The two parent nodes in different topology of the boundary node may have the same BAP address (e.g. IAB-1 and IAB-3), which makes the “Next Hop BAP Address” of routing entry ambiguous for concatenated and non-concatenated traffic, in upstream.

- In upstream, the routing IDs (the one of non-concatenated traffic and the new one after header rewriting of concatenated traffic) may be same for the data to be routed to the two different egress topologies. It means there may be two entries with same routing ID, if the routing tables are merged as one.

Question 9: Do you agree that the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for concatenated or non-concatenated traffic (e.g. separate routing tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | Not necessary | It is not necessarily indicated in the routing table, it can also be indicated in the header re-writing table of the boundary node. This works for the alternative boundary node modelling for both up and downstream. For upstream, (where the alternative boundary node is the same as boundary node assumed by RAN3) we assume that IAB-3 has a pseudo BAP address in the green topology (IAB-3 itself does not know the pseudo address). Then the routing is performed normally and the next hop BAP address in IAB-2 is either IAB-1 BAP address (real) or IAB-3 BAP address (pseudo). The egress link is determined based on the next hop node. The routing table then can indicate that IAB-3 address is pseudo (i.e., in different topology) and there can be an additional entry in the routing table giving the actual Routing ID to be used in header re-writing (or there can be a separate table for rewriting). For the alternative boundary node modelling, the same approach applies for downstream where the boundary node is IAB-3. |
| Kyocera | Yes | We assume the boundary IAB-node is configured with separate routing tables, i.e., one managed by a CU and another managed by another CU. Of course, the signalling details, e.g., how to configure these tables, is up to RAN3. |
| CATT |  | Whatever rewriting table need be configured in boundary IAB-node, it can be indicated in the header re-writing table of the boundary node. |

### 2.3.2 Upstream and downstream ambiguity

Since there is no assumption on the routing ID coordination between CUs, the routing ID used in the different topologies may be same. Also, R16 assumes the routing table configuration does not differentiate the upstream and downstream.

For concatenated traffic, one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 2nd topology for upstream. There may be routing ID collision in the routing table configuration at the boundary node (i.e. one upstream routing ID may be same as one downstream routing ID).

Therefore, the routing table should be separate for upstream and downstream for concatenated traffic. Otherwise, there will be two entries with same routing IDs (i.e. one upstream routing ID may be same as one downstream routing ID.)

Question 10a: Do you agree that, for concatenated traffic, the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate routing tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | This is one benefit of the alternative boundary node modelling: there is no ambiguity. The boundary node only routes BAP Routing IDs of one topology, therefore, there is no ambiguity. The header re-writing is performed after determining the next hop node, i.e., there is no routing after the header re-writing. |
| Kyocera | Yes | Unless the routing ID coordination between CUs is assumed in RAN3, we agree that it may be necessary to differentiate between upstream or downstream, in the routing table(s). However, we assume it means BAP layer needs to identify the directions of each packet, which is different from Rel-16 principle. We assume it would need an additional behaviour in BAP routing, e.g., to identify whether each packet is on upstream or downstream traffic. |
| CATT | Yes |  |

The same issue occurs for the BAP header rewriting table. For concatenate traffic, one previous routing ID for upstream may be same as one previous routing ID for downstream. If the upstream and downstream have the merged one BAP header rewriting table, there may be two entries in the table with the same Previous Routing ID.

Question 10b: Do you agree that, for concatenated traffic, the BAP header rewriting configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | Again with the alternative boundary node modelling this is not an issue: routing is performed first based on (real or pseudo) Routing ID of boundary node’s own topology and header re-writing is performed only for traffic to other topology (both for upstream and downstream). Therefore, there is no need to separate up or downstream entries. |
| Kyocera | Yes | As same with our comment in Question 10a, we agree that it may be necessary to differentiate between upstream or downstream, in the BAP header rewriting table(s), unless the routing ID coordination between CUs is assumed in RAN3. |
| CATT | Yes |  |

The similar issue occurs for the bearer mapping table. The parent node of the boundary node in CU2’s topology (IAB-3) may have the same BAP address with the child node of the boundary node (IAB-4). In that case, one entry of the BH RLC CH mapping table can be “BAP add1+ BH RLC ID1=>BAP add1 + BH RLC ID2”. It is ambiguous on whether it is “RLC1 of IAB-3=>RLC2 of IAB-4” as downstream or “RLC1 of IAB-4=>RLC2 of IAB-3” as upstream.

Question 10c: Do you agree that, for concatenated traffic, the BH RLC Channel Mapping Configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | Not an issue with the alternative boundary node modelling where the BAP addresses are unique (as in Rel16) (either real or pseudo but unique) and ingress BH RLC channel can be mapped to egress BH RLC channel with no ambiguity. |
| Kyocera | Yes | We think it’s necessary, unless the BAP address coordination between the CUs is assumed in RAN3. |
| CATT | Yes |  |

### 2.3.3 Header rewriting configuration

In the basic BAP operation modelling, BAP may perform the BAP header rewriting twice or for two purposes: one for header rewriting for inter-topology routing and one for header rewriting for re-routing (e.g. inter-topology re-routing or inter-donor-DU re-routing).

Question 11: Do you agree that, the BAP header rewriting configuration for inter-topology routing and the BAP header rewriting configuration for re-routing at the boundary node should be two separate rewriting tables?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | With the alternative boundary node modelling routing (and potential re-routing) is performed first to determine the next hop node and only if that next hop requires header re-writing (inter-topology or inter-donor-DU), then perform the re-writing. Thus, the header re-writing ONLY happens once. |
| Kyocera | FFS | We agree the BAP header rewriting may be performed twice, i.e., one for inter-CU routing and another for re-routing. But we still wonder if one BAP header rewriting configuration can handle the two cases, which will depend on the outcome of Question 9, while we tend to assume it’s clearer to have the separate tables. |
| CATT | No | A unified rewriting table is better. |

# Conclusion and proposals

Based on the above summary, following proposals are given.

# Reference

- R2-2107251	Summary of [Post114-e][075][eIAB] Open Issues on Re-routing	Huawei, HiSilicon

- R2-2107170	BAP-layer traffic processing at the boundary node	Qualcomm

- R2-2107253	Inter-donor routing, local rerouting and RLF indication for R17-IAB	Huawei,

- R2-2107445	Inter-donor CU Topology migration	 Intel

- R2-2107648	Open issues on (re-)routing	Fujitsu

- R2-2108026	Topology adaptation enhancements	Samsung

- R2-2108141	Discussion on inter-donor topology redundancy	ZTE, Sanechips

- R2-2108422	Boundary IAB node behaviour for partial and full inter-donor migration	Ericsson

- R2-2108482	Solutions for Inter-Donor Routing and Bearer Mapping 	Futurewei Technologies