3GPP TSG-RAN WG2 Meeting #116 electronic	               R2-21xxx

Electronic meeting, 1st-12th November 2021

Agenda Item:	8.4.3

Source: 	Huawei, HiSilicon

Title:	Summary of [Post115-e][088][eIAB] inter-CU routing open issues

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of email discussion.

- [Post115-e][088][eIAB] inter-CU routing open issues (Huawei)

Scope: Address the listed open points for inter-CU routing:

- What’s the BAP address added in BAP header in the first topology (i.e. the BAP address of ingress data at the boundary node);

- How to differentiate the concatenated traffic and non-concatenated traffic;

- How to determine whether a data should be delivered to upper layer (for downstream);

- How to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology).

Intended outcome: Report

Deadline: Long

1st round deadline: initial comments collection. October 12th, 0900 UTC

2nd round deadline: one week for proposal checking and TP/Running CR discussion if possible. October 21th, 0900 UTC

RAN2 related agreements:

- For inter-donor-DU re-routing, support the “previous routing ID to new routing ID” BAP header rewriting. - For intra-CU cases, Support inter-donor-DU re-routing at least in the scenarios of NR-DC among donor-DUs, inter-donor-DU recovery and inter-donor-DU migration. - Support inter-CU re-routing, i.e. IAB-node re-routes the data to its original donor-CU via the alternative BAP path over the topology in target CU. - As baseline, support the 1:1 and N:1 mapping from “previous routing ID” to “new routing ID” for BAP header rewriting at the boundary node, in inter-CU routing. - As baseline, support the 1:1 and N:1 mapping from “ingress BH link + ingress BH RLC ID” to “egress BH link + egress BH RLC ID” for bearer mapping at the boundary node, in inter-CU routing.

RAN3 related agreements:

- One common inter-donor topology transport mechanism should be defined for all scenarios where traffic between a donor and an IAB DU traverses the network under another donor; FFS whether it is possible to achieve a common signaling design for all scenarios - RAN3 prefers that the boundary node processes access traffic in the same manner as the non-boundary access IAB-node. - RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic. - FFS: In addition to BAP header rewriting, performs routing and bearer mapping in the same manner as the non-boundary intermediate IAB-node. - RAN3 assumes that the boundary node has only one BAP address in each topology. - RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers. - For DL traffic, the configurations of BAP routing entry and BAP-routing-ID mapping at the boundary node need to indicate the ingress topology they refer to. For UL traffic, they need to indicate the egress topology they refer to. The indications may be implicit.

# Discussion

- 2.1 Terminologies clarification

Terminologies to be used in this email discussion:

Inter-CU topology redundancy        Inter-CU partial migration

Pleases note the terminology clarification is only for the scope of email discussion, not for the specification usage.

Boundary IAB node: IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a parent DU.

Alternative modelling:

Boundary IAB node (upstream): IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a parent DU. (IAB-2 in the figure) (same as above)

Boundary IAB node (downstream): IAB-node, whose IAB-DU is terminated to a different IAB-donor-CU than a child DU. (IAB-3 in the figure)

First topology: the topology fragment before the boundary node for a traffic. (For example, the 1st topology is IAB-2 and IAB-4 for upstream; donor-DU2, IAB-3 and IAB-2 for downstream [blue nodes].)

Second topology: the topology fragment after the boundary node for a traffic. (For example, the 2nd topology is IAB-2, IAB-3 and donor-DU2 for upstream; IAB-2 and IAB-4 for downstream [green nodes].)

Concatenated traffic: the traffic routing across two topologies which belong to different CUs.  [Traffic that is carried across the blue topology and processed on the BAP layer of the boundary node.]

(For example the brown line and purple line in above figure). Please note this terminology includes both descendant node traffic and access traffic. You can understand this as the traffic from/to neighbor CU2’s topology, which needs to be handled by boundary node.

Non-concatenated traffic: [Traffic that is carried across the green topology only and processed on the BAP layer of the boundary node.] the traffic routing across one topology which belong to one CU. (For example the orange line and red line in above figure, as supported in R16). Please note this terminology includes both descendant node traffic and access traffic. You can understand this as the traffic from/to CU1’s topology, which needs to be handled by boundary node.

Inter-topology routing: the BAP routing across two topologies which belong to different CUs, i.e. the “inter-CU routing” used before.

Inter-topology re-routing: BAP  re-routes the data (finally terminated to its original donor-CU) via the alternative BAP path over the topology in target CU, i.e. the “inter-CU re-routing” used before.

- 2.2 Unified BAP operations

The goal of this section is to design the solution for BAP operations to cover inter-topology routing, inter-topology re-routing and inter-donor-DU re-routing. We strive to comprehensive solution for those 3 cases and strive to similar solution for upstream and downstream, if possible.

### 2.2.1 Assumption and modelling clarification

Before discussing the solution, the traffic/data needs to be handled by the BAP of boundary IAB-node is clarified below. (As agreed by R3, boundary node’s access traffic will be handled same as R16, which is not discussed below)

For upstream at boundary node:

- Case 1: Data from CU1 topology to be forwarded to CU1 topology; [i.e. non-concatenated traffic’s intra-topology routing]

- Case 2: Data from CU1 topology to be forwarded via CU2 topology; [i.e. concatenated traffic’s inter-topology routing]

- Case 3: Data from CU1 topology originally to be forwarded to CU1 topology, but re-routed to CU2 topology due to RLF. [i.e. non-concatenated traffic’s inter-topology re-routing]

- Case 4: Data from CU1 topology originally to be forwarded to CU2 topology, but re-routed to CU1 topology due to RLF. [i.e. concatenated traffic’s inter-topology re-routing]

- Case 5: Data from CU1 topology originally to be forwarded to donor-DU1 of CU1 topology, but re-routed to donor-DU5 of CU1 topology due to RLF. [i.e. intra-topology inter-donor-DU re-routing]

- Case X: Data from upper layers to be transmitted on CU1 topology.

- Case Y: Data from upper layers to be transmitted on CU2 topology.

For downstream at boundary node:

- Case 6: Data from CU1 topology to be delivered to upper layer;

- Case 7: Data from CU1 routed via CU2 topology to be delivered to upper layer;

- Case 8: Data from CU1 topology to be directly forwarded to next hop node in the CU1 topology;

- Case 9: Data from CU1 routed via CU2 topology to be BAP header rewritten and then forwarded to next hop node in the CU1 topology;

For the alternative modelling the possible cases are:

For upstream at boundary node (IAB-2):

Case 1-5: same as above

For downstream at boundary node (IAB-3):

- Case 6: Does not exist (downstream boundary node only handles data from its own CU);

- Case 7: Data from CU2 topology to be delivered to upper layer;

- Case 8: Data from CU2 topology to be directly forwarded to next hop node in the CU2 topology;

- Case 9: Data from CU2 topology to be BAP header rewritten and then forwarded to next hop node in CU1 topology;

It is important to clarify the assumption to ensure that companies on the same page on the BAP operation basic modelling when discussing the solutions later.

Below is the straight forward basic BAP operation modelling assumed by rapporteur:

Basic BAP operation modelling: Header rewriting for concatenated traffic=>Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing.

Below is the straightforward basic BAP operation modelling assumed for the alternative modelling:

Basic BAP operation modelling: Checking routing table for both non-concatenated and concatenated traffic => Header rewriting for concatenated traffic or for re-routing

NOTE: For clarification on “Checking routing table specific for concatenated or non-concatenated traffic”, please see the Question 9 below.

Clarification 1: Similar to the R16 local re-routing, for inter-topology re-routing and inter-donor-DU re-routing, BAP should “Check routing table” first before “Header rewriting for re-routing”, to try the primary path first and then do the re-routing if no primary path available.

- 5.2.1.3 Routing For a BAP Data PDU to be transmitted, BAP entity shall: - if the BAP Data PDU corresponds to a BAP SDU received from the upper layer, and - if the BH Routing Configuration has not been (re)configured by F1AP after the last (re)configuration of defaultUL-BH-RLC-Channel by RRC: - select the egress link on which the egress BH RLC channel corresponding to defaultUL-BH-RLC-Channel is configured as specified in TS 38.331 [3] for non-F1-U packets; - else if there is an entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, whose BAP path identity is the same as the PATH field, and whose egress link corresponding to the Next Hop BAP Address is available: - select the egress link corresponding to the Next Hop BAP Address of the entry; NOTE 1: An egress link is not considered to be available if the link is in BH RLF. - else if there is at least one entry in the BH Routing Configuration whose BAP address matches the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available: - select an entry from the BH Routing Configuration whose BAP address is the same as the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available; - select the egress link corresponding to the Next Hop BAP Address of the entry selected above;

Question 1: Do you confirm/agree the pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on previous BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF or type2 indication, etc.), similar to R16?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | Yes | BAP header re-writing should always be based on routing, i.e., re-writing is done only if (re)routing requires header re-writing. |
| Kyocera | Yes | We assume the re-routing is performed after the routing fails. |
| CATT | Yes | BAP header rewriting should be performed after R16 routing and rerouting mechanism can not work. If there is no available next hop found based on previous BAP address and routing ID, BAP layer will perform BAP rewriting for rerouting. |
| QCOM | Yes | In other words: Routing  link not available  use other link  header rewriting |
| Lenovo |  | Does it mean that only path ID change does not trigger BAP header rewriting? If yes, we agree with the criteria. [Rapp] Yes as in R16. |
| Ericsson | Yes, but load balancing should also be considered as precondition/criteria. | BH RLF, or type-2 RLF reception can be used as triggering criteria for re-routing, but RAN3 also agreed that load balancing is an use case for inter-donor re-routing (besides robustness). Hence, the boundary IAB node can be configured to re-route certain upstream traffics via the CU2. It should also be clarified that in this question we are discussing the precondition/criteria for the BAP header rewriting in the upstream. In the downstream, the only precondition for the BAP header rewriting is that the traffic should come from the CU2. [Rapp]: Traditionally, we only call the RLF case as “re-routing” while the CU configured case as “routing”, e.g. for local balancing. The re-routing is triggered by IAB-node, while load balancing purpose routing is fully control/configured by CU. |
| LG | Yes | We assume BAP header rewriting is performed only when the R16 local re-routing mechanism does not work. |
| Huawei, HiSilicon | Yes |  |

Clarification 2: Similar to the legacy BAP modeling, the “Determination of delivery to upper layer” at RX should be before “Checking routing table” at TX.

With the above clarification 1/2, there is one more FFS point on the basic BAP operation modeling, which may be the key point of this discussion.

Modelling FFS: For concatenated traffic’s inter-topology routing, FFS on whether checking “Header rewriting for concatenated traffic” is performed first or “Determination to deliver to upper layer” is performed first.

The below Question 4/5 depends on this modeling FFS, and it will be discussed/down-selection in Question 7 in the end.

Therefore, before going into the details, rapporteur asks companies to share views on the above assumptions and modelling.

Question 2: Companies are asked for comments on the above data cases (case 1-9) and/or on the BAP basic operation modeling, if any? (Mainly on the basic modeling and its clarification 1/2, while the modelling FFS will be asked and down-selected later in Q7)

| Companies | Comments, if any |
| --- | --- |
| Nokia | As indicated above, for the alternative boundary node modelling, - cases 1-5 are valid, case 6 does not exist and for cases 7-9 data is always from CU2 (since the downstream boundary node belongs to topology of CU2) - BAP operation is much simpler and basically the same as for Rel16: check whether data is delivered to upper layers, if not check routing table and perform header re-writing if needed |
| Kyocera | We agree that the Rx operation, i.e., to determine the delivery to upper layer, is performed before the Tx operation, as same with Rel-16. Just small clarifications: In Case 5, “donor-DU5” is not depicted in the figure, but we assume it’s another donor-DU of CU1. [Rapp]: Yes In Case 9, we assume the data is forwarded to the next hop node in “CU1 topology” [Rapp]: Yes, updated. |
| CATT | We agree that the BAP basic operation modeling and the 9 cased. |
| QCOM | You are missing two cases for upstream traffic: - Case X: Data from upper layers to be forwarded to CU1 topology. - Case Y: Data from upper layers to be forwarded to CU2 topology. These are the equivalent cases 6 and 7 for downstream traffic. [QC1]: We don’t see why this is straightforward since it has the header rewriting step twice. We propose an alternative approach: Routing  Header rewriting. Both of these alternatives should be discussed here. |
| Lenovo | For the basic modeling, the last step “Looking-up routing table again for re-routing” may be not needed. Since in the step of “Header rewriting for re-routing”, the egress link has been selected already. |
| Ericsson | We are fine with the Rel.16 legacy modeling in which the RX side of the BAP entity checks whether to forward the traffic to upper layers or not. Regarding the cases the following should be clarified: Case 1 / 2: Clarify differences between case 1 / 2 and case 3 / 4. Is the only difference between them the criteria for the BAP header rewriting? i.e. in case 3 and 4 the triggering criteria is RLF, whereas in case 1 and 2 is some NW configuration on the traffic, e.g. some traffic to be routed to CU1 directly, and some other traffic to be routed via CU2. [Rapp]: We are not touching that details on the difference between case 1/2 and case 3/4 yet. We also would like to change “forwarded to CU2 topology” -> “forwarded via CU2 topology” since the traffic is still terminated in CU1 even though it goes via CU2. [Rapp]: OK Case 7 / 9: Clarify what “from” means. In our understanding it should be for case 7 “Data from CU1 routed via CU2 and to be delivered to upper layers”, and for case 9 “Data from CU1 routed via CU2 and to be forwarded to next hop after BAP header rewriting”. [Rapp]: Yes Case X/Y: Does these cases really exist? The boundary IAB node is still anchored to CU1 so from the upper layers perspective all the traffic is terminated in the CU1. So from the upper layers perspective what is the difference between case X/Y and the other cases 1-2-3-4-5? [Rapp]: No intention to discuss X/Y. But those should be the access traffic, while other cases are from descendent node traffic. |
| LG | We are ok with 9 data cases, but have one comment on BAP basic operation modeling. We wonder if iterated routing table look-up is needed as the last step in the suggested BAP basic operation modeling. Actually, when the new BAP routing ID for header rewriting is selected, the BAP entity, at the same time, should check whether the next hop of the rewritten BAP header is available or not. So, we think that the last step may be merged to the step of Header rewriting for re-routing in the BAP basic operation modeling. |

### 2.2.2 Solution discussion

Following discussion is to address the open issues from last RAN2 meeting for inter-topology routing:

- What’s the BAP address added in BAP header in the first topology (i.e. the BAP address of ingress data at the boundary node);

- How to differentiate the concatenated traffic and non-concatenated traffic;

- How to determine whether a data should be delivered to upper layer (for downstream);

- How to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology).

#### Question 3: How to differentiate the concatenated traffic and non-concatenated traffic for downstream

NOTE: for upstream case, this can be automatically identified by the solution in Q4, since only concatenated traffic performs BAP header rewriting.

Motivation 1 for this question: based on the RAN3 agreement below, BAP operations is different for concatenated traffic and non-concatenated traffic, at least for downstream, including the Determination to deliver to upper layer and Header rewriting for concatenated traffic only.

- RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic.

Motivation 2 for this question:

- There is no assumption on the CU coordination to avoid the routing ID/BAP address/path ID collision, used in the two ingress topologies at the boundary node for downstream. Namely that the downstream ingress BAP data at the boundary node from two topologies may have the same routing ID/BAP address/path ID.

- So, any solution based on routing ID/BAP address/Path ID in below Question 4/5 only works if the boundary node can identify whether the data is concatenated traffic or non-concatenated traffic first (i.e. from which topology). Then, the new behavior of the solution in Question 4/5 only applies to the concatenated traffic rather than non-concatenated traffic.

  - For example, in the BAP address based solution in Q5, the boundary node needs to determine whether the downstream traffic is concatenated traffic or non-concatenated traffic, so that it knows which BAP address (i.e. CU1 configured or CU2 configured) to be used for determination to deliver to upper layer. Note that R3 agreed the two BAP addresses of boundary node may be same.

Based on the above motivations, the boundary node needs to differentiate the concatenated traffic and non-concatenated traffic for downstream (i.e. identify the ingress link).

One example way: For topology redundancy scenario, it is clear that data received from MCG is the non-concatenated while from SCG is the concatenated traffic. For partial migration case, boundary node only has the MCG with all downstream traffic are concatenated traffic. For sure, this assumes the IAB-node knows it is the boundary IAB node (e.g. by the configuration with two BAP address or the configuration of BAP header rewriting table for inter-topology routing).

Question 3: Do you agree that the ingress link is able to identify/differentiate the concatenated and non-concatenated traffic at the boundary node for downstream?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | With the alternative boundary node modelling, IAB-nodes in the other topology are allocated “pseudo” BAP address (or Routing ID). For “pseudo” Routing IDs, the boundary node is configured with the actual Routing IDs (of the other topology) to be used in BAP header re-writing. Thus, the boundary node can identify the concatenated traffic based the “pseudo” Routing ID. The same applies to both downstream and upstream. Thus, with the alternative boundary node modelling, the BAP operation is same as in Rel16: - based on Routing ID, the BAP PDU is routed to next hop node. - the only new thing is that If the next hop node (based on the pseudo Routing ID) is in the other topology, the BAP performs header re-writing. |
| Kyocera | No | We’re still not sure if it’s always possible to identify/differentiate the concatenated traffic and non-concatenated traffic based on the ingress link, considering various deployment scenarios, e.g., combination with CP/UP separation etc. We would prefer a unified solution for both downstream and upstream. So, we think the two cases should be discussed together. |
| CATT | No | We think it is not reliable to identify/differentiate the concatenated and non-concatenated traffic by ingress link. [Rapp]: As clarified, this is to just differentiate the traffic from CU1 topology and CU2 topology. |
| QCOM | Question is not clear | We believe the question was aiming for something else. The ingress link certainly knows if it resides in the blue topology or the green topology. So it either receives only blue or only green traffic. For this reason, it does not have to differentiate between green and blue traffic. Do you mean: The ingress link is the differentiator between blue and green ingress traffic? [Rapp]: Yes, that’s the intention of this question. |
| Lenovo | Yes |  |
| Ericsson | Yes | It seems quite straightforward to assume that one ingress link can only be associated to either the concatenated traffic or the non-concatenated traffic. |
| LG | Yes | Considering that one link can be used to connect one topology, we think the ingress link can identify/differentiate the concatenated and non-concatenated traffic, i.e., the ingress link is the differentiator between blue and green ingress traffic. |
| Huawei, HiSilicon | Yes | This is to first determine the ingress topology. Then for the traffic from inter-topology, below questions will further differentiate the traffic to be header rewritten. |

#### Question 4: How to determine whether the BAP header of a data should be rewritten

NOTE: Somehow Question4 and Question5 are related and address similar issue, and somehow rely on the Q3/6.

The issue: For concatenated traffic, boundary node need to know whether any BAP data needs to perform the BAP header rewriting for inter-topology routing purpose.

Option 1: Routing ID based approach

- Option 1a: by checking the routing ID in BAP header with the configured BAP Header rewriting table (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if routing ID in header matches any “previous routing ID” in the rewriting table, the BAP header should be rewritten.

  - This kind of “specific routing ID” is naturally configured by the first column of the BAP Header rewriting table.

  - This is not mutual exclusion with using “pseudo BAP address”.

- Option 1b: (assuming “Determination to deliver to upper layer” first in the modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 1a is used.

Option 2: BAP address based approach

- Option 2a: by checking the BAP address in BAP header with the configured specific BAP address(es) set (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if the BAP address in the header belongs to the configured BAP address(es) set (e.g. the pseudo BAP addresses), the BAP header should be rewritten.

  - This requires separate configuration on the specific BAP address set.

- Option 2b: (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 2a is used.

Option 3: Path ID based approach

- Option 3a: by checking the path ID in BAP header with the configured specific path ID(s) set (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For both upstream and downstream, if the path ID in the header belongs to the configured path ID(s) set, the BAP header should be rewritten.

  - This requires separate configuration on the specific path ID set.

- Option 3b: (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream, the BAP header should be rewritten, if the data is determined as NOT to be delivered to upper layer for concatenated traffic, based on the solution in Q5.

  - For upstream, option 3a is used.

Option 4: Routing ID based approach with alternative boundary node modelling

- for both upstream and downstream, by checking the Routing ID in BAP header with the configured routing table determine the Next Hop IAB-Node (same as Rel16)

  - if the Next Hop Node is in the other topology (e.g., indicated in the routing table or in a separate header re-writing table), perform BAP header re-writing (new in Rel17)

    - replace the “previous routing ID” in the BAP header with the “new routing ID”.

This alternative uses “pseudo” BAP Routing IDs.

Option 5: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., for BAP header rewriting.

Question 4: Which option do you prefer to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology) for concatenated traffic in inter-topology routing at the boundary node?

| Companies | Option? | Comments Please clarify if you have different solutions for upstream and downstream. You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | Option 4 | Option 4 with alternative boundary node modelling uses Rel16 approach for both upstream and downstream in the corresponding boundary nodes. Header rewriting is performed only if Routing ID based routing indicates that Next Hop Node is in the other topology. |
| Kyocera | Option 5 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used to determine whether the data should be performed with BAP header rewriting, as well as whether the data should be forwarded to the upper layer. We assume it’s common solution for both upstream and downstream. We wonder if RAN3’s assumption, “1a: RAN3 assumes that the boundary node has only one BAP address in each topology”, meant Option 2a is no longer applicable. |
| CATT | Option 1a | The procedure of “Header rewriting for concatenated traffic first” seems more fluently. |
| QCOM | Option 1b | In Rel-16, it is first determined whether traffic is to be delivered to upper layer. Otherwise, it is routed. In Rel-17, we can follow same approach as Rel-16 where the difference is the routing may also incur header rewriting. This deprioritizes options 1a, 2a and 3a. Options 1b, 2b and 3b are the same for downstream. For upstream, option 1b is more generic since BAP routing ID includes both BAP address and BAP path ID. |
| Lenovo | Option 1a | For option 2 and 3, they are both the subset of option 1, then option 1 has more flexibility for configuration. And since the DESTINATION BAP address used for intra-topology routing packets may be the same as the DESTINATION BAP address used for the packets to be delivered to the upper layer, then BAP header rewriting needs to be performed firstly for the intra-topology routing packets. |
| Ericsson | Option 1a for the TX side, and Option 1b for the RX side. | For RX: We do not see why receiving operations should be changed compared with legacy. First the receiving part of the BAP entity determines whether to pass the incoming BAP PDU to upper layers. For TX: If the RX side of the BAP entity determines that the BAP PDU should be passed to the transmitting part of the BAP entity, then the solution in option 1a should follow, i.e. the UE checks if for the BAP routing ID in the header there is any matching entry “previous BAP routing ID”. If there is a matching entry, the BAP header is rewritten with the corresponding “New Routing ID”. Regarding the pseudo-BAP addresses, we believe that those should be used irrespectively of the solution adopted for the BAP header rewriting. Pseudo-BAP addresses are needed because the CU2 does not need to know the real BAP addresses when injecting packets into the CU2 topology, given that anyhow the boundary node will do BAP header rewriting for the packets coming via CU2. |
| LG | Option 1b | We prefer to check first whether the received packet is to be delivered to upper layer or not as in Rel-16 and then BAP header rewriting should be determined based on BAP routing ID. |
| Huawei, HiSilicon | Option 1a is preferred, Option 1b is also acceptable | Agree with QCOM and Lenovo. |

#### Question 5: How to determine whether a data should be delivered to upper layer (for downstream)

NOTE: Somehow Question4 and Question5 are related and address similar issue, and somehow rely on the Q3/6.

NOTE: For upstream, see the issue discussed in Q6.

The issue: For downstream, the boundary node needs to know whether any received BAP data should be delivered to upper layer.

Clarification on one RAN3 assumption: “RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers.”

- It means ONLY the data with matched BAP address can be delivered to upper layer. But, it does NOT means any data with matched BAP address should be delivered to upper layer. Somehow, the downstream traffic terminated at the boundary node has to add the BAP address in header same as the boundary node’s BAP address.

- Namely that FFS whether path ID part is additionally used to determine whether a BAP data should be delivered to upper layer.

- Anyway, this is RAN3 assumption, RAN2 should have our final conclusion.

Option 1: Routing ID based approach

- Option 1a: by checking the routing ID in BAP header with the configured BAP Header rewriting table (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if routing ID in BAP header does NOT match any “previous routing ID” in the rewriting table (i.e. not to be header rewritten) , the data should be delivered to upper layer.

  - This kind of “specific routing ID” is naturally configured by the first column of the BAP Header rewriting table.

- Option 1b: by checking the routing ID in BAP header with some configured routing ID set (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the routing ID in the BAP header belongs to the configured routing ID set, the data should be delivered to upper layer.

  - This requires separate configuration on the specific routing ID set

Option 2: BAP address based approach

- Option 2a: (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if it is determined NOT to be BAP header rewritten based on the solution in Q4, the data should be delivered to upper layer.

- Option 2b: by checking the BAP address in BAP header with the configured specific BAP address(es)  (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the BAP address in the header is same as the boundary node BAP address configured by CU2, the data should be delivered to upper layer.

Option 3: Path ID based approach

- Option 3a: (assuming “Header rewriting for concatenated traffic” first in the Modelling FFS)

  - For downstream concatenated traffic, if it is determined NOT to be BAP header rewritten based on the solution in Q4, the data should be delivered to upper layer.

- Option 3b: by checking the path ID in BAP header with the configured specific path ID(s) set (assuming “Determination to deliver to upper layer” first in the Modelling FFS)

  - For downstream concatenated traffic, if the path ID in the header belongs to the configured path ID(s) set, the data should be delivered to upper layer.

  - This requires separate configuration on the specific path ID set

Option 4: Routing ID based approach with alternative boundary node modelling

- same as Rel16: by checking the BAP address in BAP header determine if the data should be delivered to upper layers

Option 5: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., it’s not the data to be forwarded to the upper layer. Otherwise, the data is forwarded to the upper layer. Note: DESTINATION field in BAP header is set to the BAP address of boundary node, for both the data on concatenated traffic and the data to be forwarded to the upper layer. Note that this flag is the same as one in Question 4.

Question 5: Which option do you prefer to determine whether a data should be delivered to upper layer for downstream in inter-topology routing at the boundary node?

| Companies | Option? | Comments You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | Option 4 | With the alternative boundary node modelling header re-writing always happens on the transmitting side of the topology boundary. Therefore, each IAB-node (including the boundary node) always receives BAP PDUs with BAP Routing IDs from ID space of its own topology. Therefore, normal Rel16 operation can be used for determining whether the BAP PDU should be delivered to upper layers: check whether BAP address in the BAP header is your own BAP address. |
| Kyocera | Option 5 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used, as commented in Q4. |
| CATT | Option 1a | [Rapp] I move the wording updates in option 1a here, proposed by CATT (i.e. adding the highlight part) - “For downstream concatenated traffic, if routing ID in BAP header does NOT match any “previous routing ID” in the rewriting table (i.e. not to be header rewritten) and DESTINATION field of this BAP PDU matches the BAP address of this node, the data should be delivered to upper layer.” |
| QCOM | Option 2b | In Rel-16, it is first determined whether traffic is to be delivered to upper layer. Otherwise, it is routed. In Rel-17, we can follow same approach as Rel-16 where the difference is the routing may also incur header rewriting. This deprioritizes options 1a, 2a and 3a. Following the RAN3 agreement, the BAP address is used to identify packets that have to be passed to upper layers => Option 2b |
| Lenovo | Option 2a | Option 2 can reuse the mechanism in R16. And since the DESTINATION BAP address used for intra-topology routing packets may be the same as the DESTINATION BAP address used for the packets to be delivered to the upper layer, then BAP header rewriting needs to be performed firstly for the intra-topology routing packets. |
| Ericsson | Option 2b | Option 2b is as in Rel.16, i.e. if the BAP destination in the BAP header matches the BAP address of this node the BAP PDU is delivered to upper layers, otherwise the packet is passed to the TX part of the BAP entity. Then the TX part of the BAP entity will be in charge of doing the BAP header rewriting. |
| LG | Option 1b with comments | It is preferred to check first whether the received packet is to be delivered to upper layer or not as in Rel-16, but only BAP address is used for this purpose in Rel-16. We think that, at least inter-topology scenario, BAP address only approach may not be sufficient and additional information would be needed to determine whether this packet is terminated at the boundary node or needs to be forwarded to another topology. In this case, we think option 1b can be considered. In our view, path ID can be additionally used to determine whether a BAP data should be delivered to upper layer as addressed by rapporteur above. For example, a specific routing ID set can be configured and those have 5 separate routing IDs which have same BAP address but separate path ID. One of them can be used for delivering to upper layer and others can be used for forwarding to another topology. Having said that, if the BAP address only is sufficient to determine whether a data should be delivered to upper layer, option 2b is also acceptable. |
| Huawei, HiSilicon | Option 1a is preferred; Option 2b is also acceptable. | Option 1a/2a/3a seem similar in this issue (i.e. data not to be header rewritten will be delivered to upper layer). Option 1b requires additional “specific routing ID configuration”. |

#### Question 6: The BAP address of ingress data at the boundary node

Issue 1: BAP address collision in the first topology, if the BAP address in header is set to the real destination

- There is no assumption on the CU coordination for BAP address allocation. So, the BAP address of two nodes in the two topologies may be same.

- For concatenated traffic, the BAP address in header used in the first topology should not be the real destination node, otherwise it causes the BAP address collision with non-concatenated traffic.

Question 6a: In the first topology, do you confirm that the “BAP address in the header of concatenated traffic” should not be collided with the “BAP address in the header of non-concatenated traffic”, which relies on the CU implementation to configure suitable BAP routing ID to be added? (This does not imply any specification impact.)

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia |  | This is not an issue with the alternative boundary node modelling since each node only receives BAP PDUs with BAP addresses from its own topology (real for destinations in own topology or pseudo for destinations in the other topology), i.e., the collision of BAP addresses does not happen with the alternative. |
| Kyocera | Yes | We agree that for concatenated traffic, the DESTINATIN field in BAP header is not the real destination. Otherwise, the boundary node would think it’s non-concatenated traffic. |
| CATT | Yes | It is depended on the implementation. |
| QCOM | No | From Q3, we know how to distinguish b/w concatenated and non-concatenated traffic. Assuming two packets carry same BAP address, the packet of the concatenated traffic will be header re-written while the packet of the non-concatenated traffic will not be header re-written. What’s the problem? [Rapp]: This is the issue in the first topology, not an issue after rewriting. See the above statement “- For concatenated traffic, the BAP address in header used in the first topology should not be the real destination node, otherwise it causes the BAP address collision with non-concatenated traffic.” |
| Lenovo | No | BAP address collision may not cause any problem since the BAP header rewriting will be performed by the boundary IAB node. And we only need to guarantee BAP routing ID in the header of concatenated traffic” not being collided with the “BAP routing ID in the header of non-concatenated traffic”. |
| Ericsson | Yes | We believe that this simplifies the BAP specification effort, because it would immediately differentiate the concatenated traffic from the non-concatenated traffic. Additionally, we believe that besides the possibility of address collision, the real destinations may not be used because those may collide with some other addressed of IAB nodes under the CU2 which obviously should be avoided. Hence, the specification needs to at least assume that the BAP destination of traffics coming via the CU2 may not be the real destination. |
| LG | see comments | The question is not clear and we have same doubt as QC. Having said that, based on the rapporteur’s further comments, if the question is to ask whether concatenated traffic in the first topology has the BAP address of the boundary node, not real destination in the second topology, to avoid BAP address collision with non-concatenated traffic. We can say “Yes”. However, if others, we are not sure. [Rapp]: Yes, that’s the intention. We can check the phase 2 proposal later. |
| Huawei, HiSilicon | Yes |  |

Issue 2: Ambiguity of determination to deliver to upper layer at the boundary node, if the BAP address in header is set to the BAP address of boundary node.

One CU implementation example in the above question could be adding the boundary node’s BAP address in the header for concatenated traffic. While it may cause below issue:

- For upstream, if the BAP address in header of the ingress data is same as the boundary node’s BAP address, the R16 BAP RX operation will erroneously deliver it to upper layer, while the concatenated traffic should be forwarded to the other topology. Extra solution is needed.

- 5.2.2 Receiving operation Upon receiving a BAP Data PDU from lower layer (i.e. ingress BH RLC channel), the receiving part of the BAP entity shall: - if DESTINATION field of this BAP Data PDU matches the BAP address of this node: - remove the BAP header of this BAP Data PDU and deliver the BAP SDU to upper layers; - else: - deliver the BAP Data Packet to the transmitting part of the collocated BAP entity.

- For downstream, if the BAP address in header is same as the boundary node’s BAP address, extra solution is required to further differentiate the traffic to be header rewritten from the traffic to be delivered to upper layer. (see Q4/5)

To address the above issue 1/2 (mainly for issue 2), following options can be considered:

Option 1: Routing ID based approach

- As one CU implementation example, the BAP address in header can be set to the BAP address of boundary node for concatenated traffic.

- At the boundary node, BAP RX will treat/consider the received BAP data with specific routing ID separately as concatenated traffic (e.g. with the routing ID in header matched the BAP header rewetting table). Then, regardless the BAP address in header, BAP RX will not deliver the data to upper layer the as long as its routing ID belongs to some specific routing ID set (so that the traffic terminated at donor-DU or descendant node will not be erroneously delivered to upper layer).

- This is somehow from the summary proposal in RAN3#113 comeback #1304 “1c: RAN3 assumes that for inter-topology traffic, any BAP routing ID in the ingress topology represents an alias for the BAP routing ID for the egress topology”.

Option 2: BAP address based approach

- Only the traffic terminated at the boundary node adds the BAP address of the boundary node, while the traffic terminated at donor-DU or descendant node adds some other “pseudo BAP address” (different with the boundary node’s BAP address) for concatenated traffic.

Option 3: Path ID based approach

- As one CU implementation example, the BAP address in header can be set to the BAP address of boundary node for concatenated traffic.

- At the boundary node, BAP RX will treat/consider the received BAP data with specific path ID separately as concatenated traffic. Then, regardless the BAP address in header, BAP RX will not deliver the data to upper layer the as long as its path ID belongs to some configured specific path ID set (so that the traffic terminated at donor-DU or descendant node will not be erroneously delivered to upper layer).

Option 4: Flag in BAP header approach

  - For both upstream and downstream, the additional flag is shown in each BAP header, which uses one of the existing “R” bits. If the flag is set to “1” then the data is the concatenated traffic, i.e., it’s not the data to be forwarded to the upper layer. Otherwise, the data is forwarded to the upper layer. Note: DESTINATION field in BAP header is set to the BAP address of boundary node, for both the data on concatenated traffic and the data to be forwarded to the upper layer. Note that this flag is the same as one in Question 4.

Option 5: to add if any

Question 6b: Which option do you prefer to address the ambiguity of determination to deliver to upper layer at the boundary node (i.e. issue 2 above)?

| Companies | Option? | Comments Please clarify if you have different solutions for upstream and downstream. You can also add more options (e.g. option 1c, option 4, …) |
| --- | --- | --- |
| Nokia | 2 | With the alternative boundary node modelling there is no ambiguity, same as in Rel16 where BAP address is used to determine when to deliver to upper layers (option 2). |
| Kyocera | Option 4 | We think the additional flag in BAP header, i.e., using one of existing “R” bits, is used, as commented in Q4. |
| CATT | 1 |  |
| QCOM | 2 | In upstream, there is no delivery to upper layers at the boundary node so Q6b does not apply. [Rapp] Please see the above statement: “- For upstream, if the BAP address in header of the ingress data is same as the boundary node’s BAP address, the R16 BAP RX operation will erroneously deliver it to upper layer, while the concatenated traffic should be forwarded to the other topology. Extra solution is needed.” In downstream, this is addressed in Q5. Determination to deliver to upper layer is done first based on BAP address matching. Traffic not to be delivered to upper layer will carry a different BAP address than that configured to the boundary node in the ingress topology. |
| Lenovo | Option 1 | If the BAP header rewriting will be performed firstly for the inter-topology routing packets before“Determination to deliver to upper layer”. Then, there will no further ambiguity for the above issue. |
| Ericsson | Option 2 | As already expressed in Q4/5, the RX operations can be kept same as in legacy. The boundary IAB node will have two BAP addresses, i.e. one BAP address assigned by CU1 and one by CU2. If the incoming BAP PDU has a BAP destination that matches one of these two addresses, the packet is delivered to upper layers, otherwise it is passed to the TX part of the BAP entity for BAP header rewriting (if the BAP PDU came via the CU2). Regarding the upstream, we agree with QC that this issue will never occur in upstream. The descendant nodes will always set the BAP destination in the BAP header as the BAP address of the donor DU1, even if their traffic is re-routed by the boundary IAB node through the CU2. Hence it will never happen that the boundary IAB node passes upstream data to upper layers, unless the BAP address of the donor DU1 is the same as the BAP address of the boundary IAB node, which would obviously be an erroneous NW configuration. |
| LG | 3 or 1 | For this issue, option 1 and option 3 can be interpreted as same solution. For example, when option 1 is applied, if a specific routing ID set can be configured and those have 5 separate routing IDs which have same BAP address but separate path ID. One of them can be used for delivering to upper layer and others can be used for forwarding to another topology. With this configuration, option 1 is same with option 3. |
| Huawei, HiSilicon | 1 | Option 1 will make the UL and DL operations unified, where the only required configuration is the header rewriting table (i.e. the first column is the specific routing ID set). |

#### Question 7: The full picture of BAP operations at boundary node

Basic BAP operation modelling: Header rewriting for concatenated traffic=>Checking routing table specific for concatenated or non-concatenated traffic => Header rewriting for re-routing=> Looking-up routing table again for re-routing. Modelling FFS: For concatenated traffic’s inter-topology routing, FFS on whether checking “Header rewriting for concatenated traffic” is performed first or “Determination to deliver to upper layer” is performed first.

Based on the solution to above questions and the BAP operation modeling above you prefer to, the full picture of BAP operations at the boundary node can be discussed/selected.

Approach A: “Header rewriting for concatenated traffic” is performed first

- 1. Determine and perform Header rewriting for concatenated traffic, based on the solution in Q4;

- 2. If not to be header rewritten, determination to deliver to upper layer;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID.

- 5. Mapping to BH RLC Channel

Approach B: “Determination to deliver to upper layer” is performed first

- 1. Determination to deliver to upper layer, based on the solution in Q5 and Q6;

- 2. If not to be delivered to upper layer in downstream or based on the solution in Q4, determine and perform Header rewriting for concatenated traffic, using the solution in Q4;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID.

- 5. Mapping to BH RLC Channel

Approach C: “Alternative boundary node modelling”

- 1. Determination to deliver to upper layer, based on the BAP address (same as Rel16);

- 2. If not to be delivered to upper layer, in both downstream and upstream, check the routing table to determine the next hop node (same as Rel16);

- 3. If the next hop node is not available perform re-routing to determine the new next hop node (same as Rel16);

- 4. If the next hop node is in the other topology (=concatenated traffic) or inter-donor-DU re-routing happens, perform Header rewriting; (new in Rel17)

- 5. Mapping to BH RLC Channel

Approach D: “Flag in BAP header approach”

- 1. Determination to deliver to upper layer, based on the BAP address and the flag with “0” (backward compatible with Rel-16);

- 2. If not to be delivered to upper layer, in both downstream and upstream, determine and perform Header rewriting for concatenated traffic, based on the flag with “1”;

- 3. Checking routing table specific for concatenated or non-concatenated traffic;

- 4. If inter-topology or inter-donor-DU re-routing is triggered, perform header rewriting for re-routing and select the next hop by looking-up the routing table with the new routing ID;

- 5. Mapping to BH RLC Channel.

Question 7: Please indicate the modeling and options you prefer to, and explain how the BAP operation works in tails to address all the previous issues?

| Company | Modell-ing FFS (A or B?) | BAP operation options |  |  | Comments Please explain how the whole BAP procedures works in details, with the option you select in Q3-7. |
| --- | --- | --- | --- | --- | --- |
|  |  | Opt. to Q4 | Opt. to Q5 | Opt. to Q6 |  |
| Nokia | C | 4 | 4 | 2 | With the alternative boundary node modelling the natural choice is Approach C where only step 4 is new. All the other steps are the same as in Rel16. |
| Kyocera | B | 5 | 5 | 4 | We prefer to define the additional flag in each BAP header, i.e., to use one of existing “R” bits. This flag is set to “1” for concatenated traffic; otherwise, it’s “0” as same with Rel-16. We also think the DESTINATIN field in BAP header is set to the BAP address of boundary node for both the concatenated traffic and the data to be delivered to the upper layer. For detailed procedure: The Rx part can determine the data delivered to the upper layer, if the DESTINATION field and the additional flag in BAP header are the boundary node’s BAP address and the additional flag is “0” respectively. Otherwise, the data is passed to the Tx part. The Tx part should first determine whether the data is non-concatenated traffic or concatenated traffic, according to the additional flag, i.e., “0” or “1” respectively. For non-concatenated traffic, Rel-16 routing is performed. For concatenated traffic, BAP header rewriting operation is performed first. Then, the routing is performed based on the proper routing table, i.e., for concatenated traffic. If the egress link is not available during the routing operation, the re-routing is triggered. In this case, the BAP header rewriting operation is performed before the re-routing operation. |
| CATT | A | 1 | 1 | 1 |  |
| QCOM | B | 1b | 2b | 2 | First, in RX part and DL, identify traffic for upper layers, which carries the boundary-node’s BAP address of the ingress link. All other traffic is forwarded to TX part. For UL, all traffic is passed to TX part. Then, in TX part, for DL, “concatenated traffic”, i.e., traffic from blue ingress link, will be header rewritten and then routed; “non-concatenated traffic”, i.e, traffic from green ingress link, will be routed without header rewriting. For UL, header rewriting and routing is one step for all traffic. Traffic, whose header is rewritten, will be forwarded to blue egress link. Traffic, whose header is not rewritten, will be forwarded to green egress link. |
| Lenovo | A | 1a | 2a | 1 | In the first topology, different sets of BAP routing ID are used for concatenated traffic and non-concatenated traffic. Upon receiving the packets at the boundary IAB node, the BAP entity firstly performs BAP header rewriting for some specific packets. After BAP header rewriting, only the DL access traffic of the boundary IAB node has its own BAP address and then be delivered to the upper layer. While for the other packets, they can be routed to the next hop based on the routing table or based on the rerouting mechanism. |
| Ericsson | B | 1a at TX side 1b at RX side | 2b | 2 | We should clarify that this question mainly applies to downstream. For upstream, we agree with QC. The step “Determination to deliver to upper layer” is not applicable to the upstream, because all the upstream traffic has as BAP destination the BAP address of DU1, hence the boundary IAB node will never deliver upstream traffic to upper layers. On the other hand, in upstream there is another step that is not present in the downstream, i.e. the determination of whether the egress link is available or not (RLF), and the determination of whether the ingress traffic should be load-balanced via CU2. |
| LG | B | 1b | 1b | 3 or 1 | For DL, when the boundary node receives a packet: 1) The BAP entity determines whether the received packet should be delivered to upper layer or not based on BAP address and path ID, if necessary. Path ID may be considered together only for concatenated traffic. (e.g., If BAP address is the BAP address of the boundary node, but the path ID is not for the boundary node, the BAP entity considers this packet is not for delivering to upper layer.) 2) Then the BAP entity recognizes concatenated traffic based on the ingress link of the received packet and rewrites the BAP header of the received packet if it is concatenated traffic. 3) After that, the BAP entity checks routing table for concatenated or non-concatenated traffic. 4) When header rewriting is needed after routing table checking, e.g., inter-topology or inter-donor-DU re-routing and so on, the BAP entity selects the new BAP routing ID for header rewriting if there are configured BAP routing ID for header rewriting and the next hop link of the rewritten BAP routing ID is available at the same time. (i.e., Don’t need to check routing table again after rewriting BAP header.) For UL, the most of steps are same as above. The difference would be that the BAP entity recognizes concatenated traffic based on the egress link of the packet. The boundary node may not need to check whether the received packet should be delivered to upper layer and all UL traffics can move to the step for routing table check. |
| Huawei, HiSilicon | A | 1a | 1a | 1 | For DL: Based on the ingress link, the non-concatenated traffic can be identified and will be handled same as R16. For concatenate traffic, if the routing ID matches the “previous routing ID” in header rewriting table, RX side do the rewriting and forward to TX side; otherwise, RX side deliver to the upper layer. At TX side, check the routing table for the data after header rewriting. For UL: If the routing ID matches the “previous routing ID” in header rewriting table, RX side do the rewriting. [RX will not deliver to upper layer if the BAP address does not match (as in R16), or if the routing ID belongs to the “previous routing ID” (new in R17). ]. TX side check the routing table for the data after header rewriting. For non-contacted traffic, legacy BAP operation can handle it same as R16. |

#### Question 8: Bearer mapping leftover

Based on the last meeting agreement, the legacy bearer mapping operation seems sufficient by considering the ingress to egress BH RLC channel mapping at the boundary node.

Question 8: Do you see any other essential issue in details for BAP operation on bearer mapping at the boundary node?

| Companies | Comments, if any |
| --- | --- |
| Nokia | No issues |
| CATT | No issues |
| QCOM | Yes. The bearer mapping configuration specifies mappings based on prior-hop and next-hop BAP addresses. Since BAP addresses are only unique within the scope of each topology, the bearer mapping configuration needs to indicate the ingress topology for the ingress link and the egress topology for the egress link. [Rapp] discussed in Q10c. |
| Lenovo | No issues. |
| Ericsson | No issues |
| LG | No issues so far. |

- 2.3 Configuration assumptions

Inter-CU topology redundancy        Inter-CU partial migration

### 2.3.1 Ingress/egress topology ambiguity

For DL traffic, the configurations of BAP routing entry and BAP-routing-ID mapping at the boundary node need to indicate the ingress topology they refer to. For UL traffic, they need to indicate the egress topology they refer to. The indications may be implicit.

Based on the above RAN3 agreement, routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for concatenated or non-concatenated traffic (i.e. the topology).

The motivation is:

- The two parent nodes in different topology of the boundary node may have the same BAP address (e.g. IAB-1 and IAB-3), which makes the “Next Hop BAP Address” of routing entry ambiguous for concatenated and non-concatenated traffic, in upstream.

- In upstream, the routing IDs (the one of non-concatenated traffic and the new one after header rewriting of concatenated traffic) may be same for the data to be routed to the two different egress topologies. It means there may be two entries with same routing ID, if the routing tables are merged as one.

Question 9: Do you agree that the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for concatenated or non-concatenated traffic (e.g. separate routing tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | Not necessary | It is not necessarily indicated in the routing table, it can also be indicated in the header re-writing table of the boundary node. This works for the alternative boundary node modelling for both up and downstream. For upstream, (where the alternative boundary node is the same as boundary node assumed by RAN3) we assume that IAB-3 has a pseudo BAP address in the green topology (IAB-3 itself does not know the pseudo address). Then the routing is performed normally and the next hop BAP address in IAB-2 is either IAB-1 BAP address (real) or IAB-3 BAP address (pseudo). The egress link is determined based on the next hop node. The routing table then can indicate that IAB-3 address is pseudo (i.e., in different topology) and there can be an additional entry in the routing table giving the actual Routing ID to be used in header re-writing (or there can be a separate table for rewriting). For the alternative boundary node modelling, the same approach applies for downstream where the boundary node is IAB-3. |
| Kyocera | Yes | We assume the boundary IAB-node is configured with separate routing tables, i.e., one managed by a CU and another managed by another CU. Of course, the signalling details, e.g., how to configure these tables, is up to RAN3. |
| CATT |  | Whatever rewriting table need be configured in boundary IAB-node, it can be indicated in the header re-writing table of the boundary node. |
| QCOM |  | For downstream, if routing is done before header rewriting, the routing configuration needs to indicate the ingress topology. In upstream, routing and header rewriting cannot be separated. Header rewriting itself indicates the egress topology. |
| Lenovo | Yes |  |
| Ericsson |  | It is not clear what is meant with “routing table configuration” at the boundary node. Is that the legacy BH Routing configuration mapping the BAP routing IDs to next hop? Or the new table mapping previous routing ID to new routing ID? [Rapp]: Yes, it is the legacy BH Routing configuration mapping. Obviously, the legacy BH Routing Configuration provided by CU1 should be configured separately, from the new rewriting table mapping the previous BAP routing ID to the new BAP Routing IDs. However, we believe that only one BH Routing Configuration is needed, since in downstream all the traffic towards the descendant nodes pertains the CU1 BH routing configuration, and for the upstream the header rewriting itself indicates the egress topology (as mentioned by Qualcomm). |
| LG | Yes | The routing table or entry may need to be clearly identified whether it is for concatenated or non-concatenated traffic to perform properly. One routing table or separate routing table can be discussed later. |
| Huawei, HiSilicon | Yes |  |

### 2.3.2 Upstream and downstream ambiguity

Since there is no assumption on the routing ID coordination between CUs, the routing ID used in the different topologies may be same. Also, R16 assumes the routing table configuration does not differentiate the upstream and downstream.

For concatenated traffic, one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 2nd topology for upstream. There may be routing ID collision in the routing table configuration at the boundary node (i.e. one upstream routing ID may be same as one downstream routing ID).

Therefore, the routing table should be separate for upstream and downstream for concatenated traffic. Otherwise, there will be two entries with same routing IDs (i.e. one upstream routing ID may be same as one downstream routing ID.)

Question 10a: Do you agree that, for concatenated traffic, the routing table configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate routing tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | This is one benefit of the alternative boundary node modelling: there is no ambiguity. The boundary node only routes BAP Routing IDs of one topology, therefore, there is no ambiguity. The header re-writing is performed after determining the next hop node, i.e., there is no routing after the header re-writing. |
| Kyocera | Yes | Unless the routing ID coordination between CUs is assumed in RAN3, we agree that it may be necessary to differentiate between upstream or downstream, in the routing table(s). However, we assume it means BAP layer needs to identify the directions of each packet, which is different from Rel-16 principle. We assume it would need an additional behaviour in BAP routing, e.g., to identify whether each packet is on upstream or downstream traffic. |
| CATT | Yes |  |
| QCOM | See comment | If routing is done before header rewriting, the routing configuration needs to indicate the ingress topology for DL traffic. This is not needed for UL traffic. If header rewriting is done before routing, there is no need for UL routing entry, which implies that the DL routing entry does not have to carry a DL indicator. |
| Lenovo | Yes |  |
| Ericsson | No | Assuming that the issue is about “one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 1st topology for upstream.”, we wonder how likely is that this situation really occurs. This can only happen in case one routing ID assigned by CU1 for downstream traffic at the boundary IAB node, is exactly the same as one routing ID assigned by CU2 for upstream traffic at the boundary IAB node. For this to happen, it has to occur that the BAP address of the donor DU2 is exactly the same as the real BAP address of a descendant node, and also the associated PATH ID has to be the same. This seems to be a very unlikely situation, and specifying separate tables/entries for downstream/upstream for such a corner case looks like an overkill. [Rapp]: See my reply in the comment box. The 2nd topology for UL and DL are different topologies. |
| LG | No | We don’t think the following assumption is correct in the description above. “For concatenated traffic, one routing ID used in the 2nd topology for downstream may be same as one routing ID used in 2nd topology for upstream.” In one topology, i.e., 2nd topology, why is one routing ID used for both upstream and downstream. If CU does, this is bad configuration. If one of 2nd topology above is intended to 1st topology, i.e., one of them is typo, we are still doubt whether separate for upstream and downstream is needed and think that separate routing tables/entries for concatenated or non-concatenated traffic may be sufficient as answered in Q9 above. This may be also related to which BAP address added in BAP header in the first topology. [Rapp]: See my reply in the comment box. The 2nd topology for UL and DL are different topologies. |
| Huawei, HiSilicon | Yes |  |

The same issue occurs for the BAP header rewriting table. For concatenate traffic, one previous routing ID for upstream may be same as one previous routing ID for downstream. If the upstream and downstream have the merged one BAP header rewriting table, there may be two entries in the table with the same Previous Routing ID.

Question 10b: Do you agree that, for concatenated traffic, the BAP header rewriting configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | Again with the alternative boundary node modelling this is not an issue: routing is performed first based on (real or pseudo) Routing ID of boundary node’s own topology and header re-writing is performed only for traffic to other topology (both for upstream and downstream). Therefore, there is no need to separate up or downstream entries. |
| Kyocera | Yes | As same with our comment in Question 10a, we agree that it may be necessary to differentiate between upstream or downstream, in the BAP header rewriting table(s), unless the routing ID coordination between CUs is assumed in RAN3. |
| CATT | Yes |  |
| QCOM | Not enough | This is fine for UL traffic. For DL traffic, there is still ambiguity since it may arrive from both topologies in case of a dual connected boundary node. [Rapp]: This seems the motivation that we have Q3. |
| Lenovo | Yes |  |
| Ericsson | No | Similar to Q10a, this seems to be a corner case. For this to happen, it needs to occur that the previous routing ID of the upstream is the same as the previous routing ID of the downstream. Hence, one condition should be that the BAP address of the donor DU1 (upstream) is the same as the pseudo BAP address assigned by the CU2 for a descendant node (downstream). Additionally, also the PATH ID has to be the same. This seems to be a very unlikely situation, and specifying separate tables/entries for downstream/upstream BH routing configuration for such a corner case looks like an overkill. |
| LG | No | same comments in Q10a |
| Huawei, HiSilicon | Yes |  |

The similar issue occurs for the bearer mapping table. The parent node of the boundary node in CU2’s topology (IAB-3) may have the same BAP address with the child node of the boundary node (IAB-4). In that case, one entry of the BH RLC CH mapping table can be “BAP add1+ BH RLC ID1=>BAP add1 + BH RLC ID2”. It is ambiguous on whether it is “RLC1 of IAB-3=>RLC2 of IAB-4” as downstream or “RLC1 of IAB-4=>RLC2 of IAB-3” as upstream.

Question 10c: Do you agree that, for concatenated traffic, the BH RLC Channel Mapping Configuration (or its entry) at the boundary node should be able to indicate whether it is for upstream or downstream (e.g. separate tables/entries)?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | Not an issue with the alternative boundary node modelling where the BAP addresses are unique (as in Rel16) (either real or pseudo but unique) and ingress BH RLC channel can be mapped to egress BH RLC channel with no ambiguity. |
| Kyocera | Yes | We think it’s necessary, unless the BAP address coordination between the CUs is assumed in RAN3. |
| CATT | Yes |  |
| QCOM | Yes | The bearer mapping includes prior hop and next hop BAP addresses. For DL, the prior hop BAP address needs to indicate to which topology it belongs. For UL, the next hop BAP address needs to indicate to which topology it belongs. There are two options: Option 1: Prior/next hop BAP address in BH RLC CH configuration include an optional topology indicator. Option 2: BH RLC CH configuration includes one DL vs. UL indicator and one topology indicator, referring to ingress for DL and to egress for UL. |
| Lenovo | Yes |  |
| Ericsson | Too early to decide | This question assumes that the legacy BH RLC channel mapping information is reused as it is, including the prio/next hop BAP address and the ingress/egress BH RLC CH ID. Therefore, this question implies that the prior hop BAP address and the next hop BAP addresses are exchanged across the two topologies as well as the ingress BH RLC channel IDs and supposedly their associated QoS requirements. Therefore, some level of coordination would be required. However, RAN2/3 has not discussed yet whether coordination between nodes is needed and whether information exchange on prior/next hops, ingress/egress BH RLC CH is needed. That should be discussed first, before addressing possible upstream/downstream collisions on the BH RLC channel mapping configuration. |
| LG | FFS | We think that this case is different from Q10a/Q10b and something additional would be needed. So, we can put this FFS. |
| Huawei, HiSilicon | Yes |  |

### 2.3.3 Header rewriting configuration

In the basic BAP operation modelling, BAP may perform the BAP header rewriting twice or for two purposes: one for header rewriting for inter-topology routing and one for header rewriting for re-routing (e.g. inter-topology re-routing or inter-donor-DU re-routing).

Question 11: Do you agree that, the BAP header rewriting configuration for inter-topology routing and the BAP header rewriting configuration for re-routing at the boundary node should be two separate rewriting tables?

| Companies | Yes or No? | Comments |
| --- | --- | --- |
| Nokia | No | With the alternative boundary node modelling routing (and potential re-routing) is performed first to determine the next hop node and only if that next hop requires header re-writing (inter-topology or inter-donor-DU), then perform the re-writing. Thus, the header re-writing ONLY happens once. |
| Kyocera | FFS | We agree the BAP header rewriting may be performed twice, i.e., one for inter-CU routing and another for re-routing. But we still wonder if one BAP header rewriting configuration can handle the two cases, which will depend on the outcome of Question 9, while we tend to assume it’s clearer to have the separate tables. |
| CATT | No | A unified rewriting table is better. |
| QCOm | Yes | We need two routing tables. Let’s assume, we had only one header rewriting table. Case 4: UL traffic is forwarded from green to blue topology: - UL packet includes virtual BAP routing ID for blue donor-DU in green topology. - Header rewriting table includes entry: Virtual BAP routing ID for blue donor DU  Real BAP routing ID of blue donor DU. - If the blue parent link has RLF, the boundary node needs to apply header rewriting for re-routing. This could be done in a consecutive rewriting step. In this case, the table has to include an entry: Real BAP routing ID for blue donor DU  Real BAP routing ID of green donor DU. This entry cannot be in the same table since the ingress BAP routing IDs refer to a different topologies and may accidently collide. Alternatively, the first header rewriting step is not executed. In this case, the header rewriting table needs to include an entyr: Virtual BAP routing ID for blue donor DU  Real BAP routing ID of green donor DU. This implies that the BAP header rewriting table has two conflicting entries: Virtual BAP routing ID for blue donor DU  Real BAP routing ID of blue donor DU. Virtual BAP routing ID for blue donor DU  Real BAP routing ID of green donor DU. Analogue reasoning can be applied for Case 3. Consequence: The boundary node needs to perform two consecutive steps using two different tables: - Header rewriting + routing based on header rewriting table 1 - If egress link is not available, perform header rewriting based on table 2. |
| Lenovo | No | Rerouting is the local behavior of intermediate IAB node, there is no need to introduce BAP header rewriting table for local rerouting. |
| Ericsson | No | We should aim at minimizing specification differences between inter-topology routing and inter-donor DU re-routing, unless there is a clear issue. For example, it is not clear in the example from Qualcomm why the IAB node should first perform a BAP header rewriting due to inter-topology routing and then overwrite the new header again to route the packet back to CU1. Shouldn´t the IAB node first check the availability of the link towards the blue topology before performing any inter-topology routing? |
| LG | No | We think that it is reasonable to have BAP header rewriting table for inter-topology routing, but BAP header rewriting table for local re-routing is not needed. One additional field in the routing table is sufficient for BAP header rewriting by local re-routing. |
| Huawei, HiSilicon | Yes | Agree with QCOM on the analyses of case4. |

# Conclusion and proposals

Based on the above summary, following proposals are given.

# Reference

- R2-2107251	Summary of [Post114-e][075][eIAB] Open Issues on Re-routing	Huawei, HiSilicon

- R2-2107170	BAP-layer traffic processing at the boundary node	Qualcomm

- R2-2107253	Inter-donor routing, local rerouting and RLF indication for R17-IAB	Huawei,

- R2-2107445	Inter-donor CU Topology migration	 Intel

- R2-2107648	Open issues on (re-)routing	Fujitsu

- R2-2108026	Topology adaptation enhancements	Samsung

- R2-2108141	Discussion on inter-donor topology redundancy	ZTE, Sanechips

- R2-2108422	Boundary IAB node behaviour for partial and full inter-donor migration	Ericsson

- R2-2108482	Solutions for Inter-Donor Routing and Bearer Mapping 	Futurewei Technologies