3GPP TSG-RAN WG2 Meeting #116 electronic	R2-21xxxxx

Electronic Meeting, Nov 1 – 12, 2021

Agenda item:	8.21.2.1

Source:	CMCC (Email Rapporteur)

Title:	Report for [Post115-e][090][TEI17] Mobility-state-based cell reselection for NR High Speed railway Dedicated Network (CMCC)

Document for:	Discussion

# Introduction

RAN2#115-e meeting arranged the following email discussion to check the HSDN CRs. Here some questions are listed to check companies’ views.

- [Post115-e][090][TEI17] Mobility-state-based cell reselection for NR High Speed railway Dedicated Network (CMCC)

Scope: Check and progress CRs to agreeable status.

Intended outcome: Agreeable CRs

Deadline: Long

Please add company contact details into the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| CMCC | Ningyu Chen | chenningyu@chinamobile.com |
| Ericsson | Mattias Bergström | Mattias.a.bergstrom@ericsson.com |
| MediaTek | Chun-Fan (Felix) Tsai | Chun-fan.tsai@mediatek.com |

# Discussion

## UE Capability

In LTE spec 36.306, the HSDN capability is missing, i.e., there is no HSDN capability for UE either with or without signalling in LTE. But from rapporteur point of view, a capability for HSDN cell reselection without capability signalling seems needed.

Question 1: Companies are invited to share views on whether a capability for NR HSDN is needed?

- Option 1: Capability without RRC signalling

- Option 2: Capability with RRC signalling

- Option 3: No capability

| Company | Which option? | Comments |
| --- | --- | --- |
| CMCC | Option 1 | HSDN feature only impact on SIB reception and cell reselection priority handling in idle/inactive mode. And no impact on connected mode. So HSDN capability without RRC signalling seems ok. |
| Ericsson | Option 1 | Agree with CMCC. |
| MediaTek | Option 1 | As this is idle/inactive mode behavior and all configuration in SIB, it is reasonable to have optional feature without UE capability. |

If the answer to Question 1 is option 1, companies are invited to check the following wording for the optional HSDN capability without signalling for 38.306CR?

- 5.x	High speed dedicated network features

Definitions for feature

HSDN cell reselection It is optional for UE to support HSDN cell reselection priority handling in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21] and TS 38.331 [9].

Question 2: Is the wording above for HSDN capability agreeable? Comments are welcome

| Companies | Comments |
| --- | --- |
| Ericsson | It is fine. But another possibility is to place this description under "5.4 Other". |
| MediaTek | Yes |

If a capability for NR is needed, then next question is whether we need to also add the missing capability back for LTE 36.306, and in which release to add it.

Question 3: Do you agree the LTE HSDN capability is also needed in 36.306? And which release the 36.306 CR should be?

| Company | Yes/No | Which release? And further comments if any. |
| --- | --- | --- |
| CMCC | Yes | If companies agree to add the missing LTE HSDN capability for 36.306, we would prefer to add it from Rel-15, since HSDN was introduced in R15 LTE. In this case, shadow 36.306CRs for R15, R16 and R17 are also needed. |
| Ericsson | No strong view |  |
| MediaTek | Yes | Same view as CMCC |

  - CRs for 38.331, 38.304 and 36.331

Draft CRs for 38.331 and 38.304 are generally copy/paste from LTE specification. Companies are invited to check on the details.

Draft CRs for 36.331 is also provided to address inter-RAT cell reselection from LTE to NR, as commented by Lenovo in last meeting (details can be found in the Annex).

Comments or corrections can be added directly to the draft CRs.

In case any essential issue is recognized, companies can also raise it here for discussion.

| Company | Comments |
| --- | --- |
| MediaTek | Editorial comment on 36.306 new section – We don’t use table format in 36.306. It could just be a paragraph. |
| Ericsson | The "hsdn-Indication" in SIB4 states: hsdn-Indication This field indicates whether there are deployed HSDN cells or not on the the DL carrier frequency indicated by dl-CarrierFreq. It seems that this indication is actually not used anywhere in the specification. Also the field only says whether there are HSDN cells on the frequency or not. In my understanding this says whether there are zero or at least one. But that information is perhaps not so interesting to the UE? Instead we assume that the intention behind this indication was (when it was added to LTE) to indicate whether all cells on a frequency are HSDN-cells. So we should perhaps change the field description to: hsdn-Indication This field indicates, if present, that all cells on the DL carrier frequency indicated by dl-CarrierFreq are HSDN cells. |

# Conclusions

TBD

# Annex: Agreements and comments in RAN2#115e

- MobState cell reselection for HSDN

R2-2108501	Mobility-state-based cell reselection to support NR High Speed railway Dedicated Network (HSDN)	CMCC, CATT, Ericsson, Huawei, ZTE, OPPO, vivo	discussion	Rel-17	TEI17

DISCUSSION

-	QC support this. Is there R4 impact? CMCC indicate that same as LTE there is no impact to R4.

-	LG support to resue this solution for NR. Can this feature be used for FR2 cells? CMCC has no strong opinion on the applicability for FR2. Think it is general.

-	Lenovo wonder for Inter-RAT cell reselection should be counted? There was earlier discussed and decided to be left to UE impl. LG think this can be left for UE impl, no need to specify for this.

-	Nokia think this is copy-paste of LTE solution to NR, and for LTE inter_RAT cell reselection is not counted, but this may not be very relevant, IRAT cell reselection should be rare.

-	Nokia have comments on the CRs.

-	Lenovo wonder about the direction of IRAT reselection. CMCC think it may be considered to support both direction but that would require a LTE RRC CR.

-	Apple wonder whether we need a UE capability. CMCC think in LTE there is no signalled capability. NR could use the same way. CMCC think we may need to check whehterh a CR is needed, in any case no signalling is needed.

- Introduce 1 bit indication in SIB1 to indicate the NR HSDN cell, same as LTE.

- HSDN neighbouring cell list with PCI(s) can be provided in SIB for intra-frequency, inter-frequency, and inter-RAT cell reselection, same as LTE.

- the number of equivalent cells can be indicated for MSE in SIB2, same as LTE.

- When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities), same as LTE.

- When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than network configured priorities), same as LTE

- Discuss and check the CRs in a post meeting email discussion, CRs for next meeting