3GPP TSG-RAN WG2#116-e	R2-21xxxxx

Online, November 1 - 12, 2021

Source: 	Huawei

Title: 	[Post115-e][219][R17 DCCA] UE-initiated SCG activation (Huawei)

Agenda Item:	x.x.x.x.

Document for:	Discussion and decision

# 1	Introduction

This document is the summary of the following email discussion:

[Post115-e][219][R17 DCCA] UE-initiated SCG activation  (Huawei)

Scope: Discuss the details of UE-initiated SCG activation and whether we need it. Should clarify technical aspects.

Intended outcome: report

Deadline:  October 21 UTC 0900

Q0: What is your name and email address?

| Company | Name and email address |
| --- | --- |
| OPPO | Shukun Wang (wangshukun@oppo.com) |
| Lenovo, Motorola Mobility | Congchi Zhang (zhangcc16@lenovo.com) |
| Apple | Naveen Palle ( naveen.palle@apple.com) |
| Ericsson | Stefan Wager (Stefan.wager@ericsson.com) |

# 2	Discussion

## 2.1	UL data arrival

RAN2 agreed to support UE-triggered SCG activation and the following solutions were suggested:

1)	While the SCG is deactivated, all uplink data are only sent to the MCG and the network can use network-triggered SCG activation based on observed traffic or BSR on MCG.

This means that before SCG deactivation or at SCG deactivation, all DRBs with uplink are (re)configured with an MCG RLC bearer, which can be used for uplink.

FFS whether the network needs to explicitly deactivate all uplink transmissions via the SCG at SCG deactivation using existing signalling (e.g. deactivate PDCP duplication, set primary path to MCG) or this is done as a result of generic UP handling at SCG deactivation (like in case of SCG RLF).

2)	In case of UL data arrival on SCG bearers, the UE sends an indication to the MCG and then the network can use network-triggered SCG activation. (FFS indication details).

3)	In case of UL data arrival on SCG bearers, the UE initiates random access towards the SCG (or uses SR on PUCCH) and resumes UL data transmission, using the received UL grants (in RAR or PDCCH).

This means that the UE configuration is not modified, and in the case of SR/PDCCH, the previously activated TCI states are used for PDCCH reception.

In order to allow the SN to respond to RA/SR quickly, the SN informs the MN of SCG activation and does not wait for confirmation from the MN to allocate grants (within RAR or DCI) to the UE and to send the UL data on MN-terminated bearers to the MN.

FFS: can the MN/SN modify the UE configuration before uplink data transmission? If yes, with or without data loss.

Solution 3) relies on the possibility for the UE to send RACH to the SCG to activate the SCG:

-	on the UE side, without any information at SCG activation

-	on the network side, without the possibility for the MN or the SN to reject the SCG activation, and without the possibility to do any RRC reconfiguration at SCG activation.

SN modification procedure as agreed by RAN3 for SCG activation/deactivation would need modifications to support this, so it depends on RAN3.

If RAN3 would support this, solutions 1 and 2 could also be optimized. For example:

- 	1a), the MN sends a network-triggered SCG activation indication to the UE, without any RRC reconfiguration, and notifies the SCG activation to the SN. Upon reception of the network-triggered SCG activation indication:

-	If RACH-less SCG activation is possible, the UE starts monitoring SCG PDCCH as soon as it can and the SN can allocate grants to the UE as soon as it receives the MN notification.

-	If RACH-less SCG activation is not possible, the UE will send a RACH to the SCG as soon as it can.

-	1b), the same as 1a) but the UE initiates SCG activation as soon as it has UL data for the DRB, without waiting for the network-triggered SCG activation indication. This behaviour could be configured per DRB (in case the network prefers to use the SCG for any UL data on a DRB).

-	2a), the UE sends an indication to the MN and

-	if RACH-less SCG activation is possible, the UE starts monitoring SCG PDCCH as soon as it can and the SN can allocate grants to the UE as soon as it receives the MN notification.

-	If RACH-less SCG activation is not possible, the UE will send a RACH to the SCG as soon as it can.

Below is a tentative comparison of the solutions, in terms of activation delay, specification impact and issues or limitations.

| Option | 1: Use MCG RLC bearers for uplink and network-triggered activation |  | 2: The UE sends an indication and the network can use network-triggered activation |  |
| --- | --- | --- | --- | --- |
| Delay components | - UE to MCG data/BSR - MN to SN message - SN to MN message - MN to UE SCG activation indication - RACH-less case: UE delay to start monitoring SCG PDCCH with indicated or previously activated TCI states - RACH case: delay to send RACH towards SCG and get initial grant |  | - UE to MCG activation request - MN to SN message - SN to MN message - MN to UE SCG activation indication - RACH-less case: delay to start monitoring SCG PDCCH with indicated or previously activated TCI states - RACH case: delay to send RACH towards SCG and get initial grant |  |
| Specification impact | None besides capturing the restriction. |  | Define the UL data arrival notification towards the MCG. |  |
| Issues / limitations | SCG activation delay is impacted by MN-SN round-trip delay. |  |  |  |
|  | The network must allocate an MCG leg for all DRBs |  | SCG activation delay delays initial UL data |  |
| Option | 1a: the UE transmits UL data via MCG and the MN activates the SCG | 1b: the UE transmits UL data via MCG and activates the SCG | 2a: the UE notifies UL data arrival to the MN and activates the SCG | 3: the UE activates the SCG |
| Delay components | - UE to MCG data/BSR - MN to UE SCG activation indication - RACH-less case: Max (UE delay to start monitoring SCG PDCCH with previously activated TCI states, MN to SN message) - RACH case: delay to send RACH towards SCG and get initial grant | - RACH-less case: Max(UE to MCG data/BSR + MN to SN message, UE delay to start monitoring SCG PDCCH with previously activated TCI states) - RACH case: delay to send RACH towards SCG and get initial grant | - UE to MCG activation request - RACH-less case: Max (UE delay to start monitoring SCG PDCCH with previously activated TCI states, MN to SN message) - RACH case: delay to send RACH towards SCG and get initial grant | Using RACH: delay to send RACH towards SCG and get initial grant Using SR: delay to send SR and start monitoring SCG PDCCH with previously activated TCI states |
| Specification impact | Modify MN-SN behaviour/interactions to support SCG activation without the possibility for the SN or the MN to reject the SCG activation or do any RRC reconfiguration at SCG activation |  |  |  |
|  | Support SCG activation indication without SN-provided information. | Trigger SCG activation upon UL data arrival. |  |  |
|  |  | Optional: whether to trigger activation for UL data is configurable per DRB | Define the UL data arrival notification towards the MCG. | The MN handles UL PDCP PDUs (for MN-terminated DRBs) from the SN even before SCG activation. |
| Issues and limitations |  |  | SCG activation delay also delays initial UL data |  |
|  | RACH-less only works when RACH-less activation without indication of PDCCH TCI state is possible (not feasible without BFD) |  |  |  |
|  | In RACH-less case, SCG activation delay is affected by one-way MN-SN delay, unless UE delay to start SCG PDCCH monitoring is larger. |  |  | RACH-less requires SCG SR resources (may reduce capacity) |
|  | RACH can only use CBRA (risk of collisions, some more delay) or permanently reserved resources for CFRA (may reduce capacity) |  |  |  |
|  | Reconfigurations may be required while the SCG is deactivated in order to be ready for SCG activation (increases signalling overhead and UE power consumption) |  |  |  |

A number of observations can be made:

-	2 could also be used for split bearers (e.g. if the network prefers the UE to use the SCG for UL data).

-	There are similarities between 1b and 3:

-	UL data arrival triggers SCG activation in both cases.

-	SCG activation is the same in RACH-case in 1b and 3 with RACH (except that initial UL data are transmitted via MCG in 1b)

-	on the UE side, SCG activation is similar in RACH-less case in 1b and 3 with SR (except that SR and/or BSR and/or UL data are sent to the MCG in 1b and SR is sent to the SCG in 3).

-	3 could also be used for split bearers, but this would delay SCG

-	1a, 1b, 2a and 3 are only feasible if RAN3 defines new procedures to support SCG activation without the possibility for MN/SN to reject activation or modify the configuration at activation, otherwise only 1 and 2 are feasible.

In addition, if such procedures would be defined, it would be necessary to decide whether they can always be used when the SCG is deactivated or it is a network choice and it is up to the SN and the MN to negotiate this together with the UE configuration.

Therefore, supporting such solutions may not be straightforward.

Important note: fast MCG link recovery via RACH to the SCG is discussed in the next section. It may or may not be supported regardless of which of the solution(s) for UL data is/are supported.

Q1: Do companies agree with the above descriptions, comparisons of candidate solutions and observations for UE-initiated SCG activation for UL data transmission? (please add any solution if needed)

| Company | Comments |
| --- | --- |
| OPPO | It is ok to us. |
| Lenovo, Motorola Mobility | For solution 1a, 1b, 2a, when MN receives the SCG activation indication from UE and sends the message to SN, it is still possible for SN to reject the SCG activation, right? For example, - SN sends a SN modification reject message over Xn to MN - SN does not provide any UL grant or not responding to RACH request from UE For solution 3 UE activates SCG, it is also possible for MN to reject the SCG activation, for example: - UE sends RACH request to SN - SN informs MN using SN modification required message - MN rejects the SN modification required message. - SN does not send RACH Response message to UE Thus, we don’t think the following assumption is really a precondition: - on the network side, without the possibility for the MN or the SN to reject the SCG activation, and without the possibility to do any RRC reconfiguration at SCG activation. For UE behaviour in this case, following legacy, upon not receiving RAR or UL grant, UE might try again sending RACH request or SR to SN, which is non idea. However, we believe such situation can be avoided/resolved by a reasonable NW implementation, e.g., reconfiguring UE as soon as possible. |
| Apple | Agree with the observations. We do not think UE self-activation of SCG along with sending info to the NW is needed (1a, 1b, 2a, 2b), but more details below. We understand that some level of monitoring the PDCCH on SCG is needed for 3. But just solutions 1 and 2 which are NW triggered (not 1a, 1b, 2a, 2b, where UE activates SCG without NW trigger) are good to pursue. |
| Ericsson | We agree with Apple that the combination of UE self-activation and sending info to the network is needed, but to our understanding that concerns only 1b and 2a. If we understand 1a correctly, the UE always waits for the network to activate the SCG? |

Q2: Do companies want to support in Rel-17 SCG activation without the possibility for the MN/SN to reject or modify the UE configuration at SCG activation?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | It is possible to reallocate radio resource and bearer type due to load in SCG side and MCG side. |
| Lenovo, Motorola Mobility | No, and we don’t think this is an issue | We believe reasonable NW implementation shall not reject, but we don’t think it’s needed to put such restriction in the spec. In addition, please see our comment in Q1, we don’t believe “NW cannot reject” is the precondition for solution 1a, 1b, 2a, 3 |
| Apple | We do not oppose. | Do not see why not. The re-configuration can happen later as well. In our understanding, modification of SCG at activation is always possible with NW triggered activation. But for the cases of UE initiated activation, we can agree that for rel-17, modification is not allowed for R-17. |
| Ericsson | No | The network should be in control of the procedure and there may be a need for RRC reconfigurations during SCG activation/deactivation. |

Q3: If SCG activation without the possibility for the MN/SN to reject or modify the UE configuration at SCG activation is supported in Rel-17, which solution do companies prefer to support for UL data arrival on SCG bearer? on split bearer? (could be different)

| Company | Solution(s) | Comments |
| --- | --- | --- |
| Apple | We prefer solution 1 (not 1a, 1b), which is also simplest in terms of spec impact. Next preference is solution 2 (not 2a, 2b) with UAI, which is useful for SCG bearer. | With both sol 1 and sol 2, and with the assumption that the UE does NOT activate SCG until the NW triggers it (NW responds to UE with sol 2, NW triggers with Sol 1), many aspects are resolved, and would be simplest for Rel-17. So do not prefer Sol 1a, 2b, 2a. We do not need to address the SR/PUCCH aspects. We can evaluate others in Rel-18 except for MCG recovery. |

Q4: If SCG activation without the possibility for the MN/SN to reject or modify the UE configuration at SCG activation is NOT supported in Rel-17 (i.e. 1a, 1b, 2a and 3 are not possible), which solution do companies want to support for UL data arrival on SCG bearer? on split bearer? (could be different)

| Company | Solution(s) | Comments |
| --- | --- | --- |
| OPPO | 1a | Except SCG activation indication from UE to network, the rest part of procedure is same as DL data arrival case. |
| Lenovo, Motorola Mobility | 3 | Please see our comment in Q1, we don’t believe “NW cannot reject” is the precondition for solution 1a, 1b, 2a, 3. Among those solutions, we think 3 is the simplest and shall be supported as a start point. For 1a, 1b, 2a, some careful analysis about the signalling flow is needed. |
| Apple | Sol 1 and Sol 2 | We anyway do not prefer UE self-activation of SCG after sending something to MCG (1a, 1b, 2a). |
| Ericsson | 1 or 1a | This causes least specification complexity, as it builds on legacy BSR reporting for indicating arrival of UL data and network triggered SCG activation, which is same as for arrival of DL data. |

## 2.2	MCG failure recovery via the deactivated SCG

Another proposal is specific to MCG failure recovery, using the deactivated SCG to send the MCGFailureInformation message. In this case, communication via the MCG is not possible so the UE needs to initiate transmission towards the SCG, using either random access or SR.

In this scenario, upon transmission of the MCGFailureInformation message using the SCG RLC bearer associated the SRB1 or SRB3, the UE starts a timer and expects a reconfiguration quickly. In this scenario, it may not be so useful to resume operation on SCG RLC bearers associated with DRBs before the reconfiguration, it is sufficient to resume the SCG DRB associated with SRB1 or SRB3.

Supporting UE-initiated activation of the SCG only for SRB1/SRB3 to send one RRC message and receive an MCG RRC reconfiguration, while MCG transmission is interrupted, is a rather different procedure as compared with full SCG activation simultaneously with MCG operation as in option 3) above, so it should be considered separately and compared with RRC re-establishment.

Below is a tentative comparison of the solutions, in terms of delay until resume of MCG operation, specification impact and issues or limitations.

|  | 1) RRC reestablishment | 2) RACH/SR towards the SCG and resume of SRB1/SRB3 |
| --- | --- | --- |
| Delay | - Cell selection - CBRA with RestablishmentRequest - Context fetch - RRC re-establishment - first RRC reconfiguration | - RA/SR with MCGFailureInformation - Handover preparation - RRC reconfiguration |
| Specification impact | None | The procedure should be configured to the UE, need to ensure support by MN and SN (could be similar to support and configuration of MCG recovery in Rel-16). If CFRA is used, preallocation of resources. If SR is used, SR resources need to be allocated. |
| Issues / limitations | - The network must allocate an MCG leg for all DRBs On the optional optimization: - only works when RACH-less activation without indication of PDCCH TCI state is possible | If CFRA is used, this uses permanently allocated RACH resources, which reduces capacity. If SR is used: - need to reserve PUCCH resources - only works when RACH-less activation without indication of PDCCH TCI state is possible |

Q5: Do companies agree with the above descriptions and comparisons of candidate solutions for MCG RLF while the SCG is deactivated? (please add any solution if needed)

| Company | Comments |
| --- | --- |
| OPPO | For solution 2, CB RACH can be used. And in MSG3, the SCG activation indication can be transmitted. Except this, RAN2 should also consider how to handle the case that SCG deactivation command is received while T316 is running. |
| Lenovo, Motorola Mobility | The description looks fine, agree with rapporteur for MCG link recovery only activating SRB1 or SBR3 is enough, but not sure if it is needed to define a “half” activated SCG, or it could be simpler to fully activate the SCG from spec point of view. |
| Apple | Agree, and also we do not see the necessity to have a CFRA for RLF recovery. MCG failure should not be a common case to have NW assign dedicated resources for this. |
| Ericsson | We don’t see how the issues/limitations listed for RRC reestablishment are related to the RRC reestablishment? They seem more related to SCG deactivation as such? Only issue/limitation we see is the longer interruption time compared to recovery via the deactivated SCG. For the recovery via deactivated SCG, a possible issue is the added step of SCG activation than needs to be executed before the MCGFailureInformation message can be transmitted. This extra step may decrease reliability of the MCG recovery procedure. |

Q6: Which solution do companies prefer for MCG RLC while the SCG is deactivated?

| Company | Solution(s) | Comments |
| --- | --- | --- |
| OPPO | 2 | CB RACH can be used and MCGFailureInformation is included in MSG3. |
| Lenovo, Motorola Mobility | 2 |  |
| Apple | Sol-2 (a simplified version of this) | UE would RACH to provide the MCG Failure RRC message using SRB1/SRB3, without any changes to DRBs. We already agreed that DRBs/PDCP entities associated with these are not suspended. So no special action is needed, just a change in spec to have the UE trigger sending the SRB message, which would trigger a RACH (assuming SR is not considered valid in SCG deactivated state). The additional aspect would be for the UE to monitor the PDCCH (which would be new for the SCG deactivated state). We think not allowing MCG recovery would be not making use of the available SCG. |
| Ericsson | 1 | While we are in general supportive of MCGFailureInformation, we think that if SCG activation is only supported via MCG (solution 1 in Q1), then for simplicity it is better to trigger RRC reestablishment upon MCG failure when SCG is deactivated. Also the extra step of SCG activation before transmission will increase the delay and decrease the reliability of the MCG failure recovery procedure. |

# 3	Conclusion