3GPP TSG-RAN2 Meeting #116 electronic	R2-21XXXX

# Online, 1 - 12 November, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | - | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-17 paging with service indication for MUSIM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the email discussion “[Post115-e][236][MUSIM] Paging with service indication”, the following were proposed: Proposal 1: Introduce paging cause by using the ”nonCriticalExtension” in the Paging record. Proposal 2: No need to study solution proposals based on extending legacy Paging record. Proposal 3: FFS if B.1 (parallel list with 1 paging cause value “voice”) or B.2 (parallel list with 2 paging cause values “voice, other”) is the preferred ASN.1 coding approach. Proposal 4: The solution proposal to introduce paging cause in NR will be used for LTE. Proposal 5: No need to send an LS to SA2 asking to consider a NAS solution to introduce paging cause in LTE. Proposal 6: For paging reception in RRC_IDLE, UE forwards the paging cause to NAS. It’s up to NAS whether to accept or reject the paging. Proposal 7: For the AS-NAS interaction for paging reception in RRC_INACTIVE, FFS Option 2 or Option 3 (i.e. up to UE implementation) is the preferred solution. Proposal 8: The AS-NAS interaction principles for NR are applied to LTE. Proposal 9: Introduction of paging cause impacts 38.331 and 36.331 specs; FFS if it impacts stage 2 specs (38.300 and 36.300) Relevant NR proposals from the above should be added to the RRC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Section 5.3.2.3, - For paging reception in RRC_IDLE, pagingCause (if present) will be forwarded to upper layers; - For paging reception in RRC_INACTIVE, Editor note was added as there was no consensus on solution proposal to use for AS-NAS interaction; In Section 6.2.2, - ASN.1 code was added to introduce paging cause; - Editor note was added as there was no consensus on the number of pagingcause values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R17 MUSIM paging with service indication is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.3 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.3.2.3	Reception of the Paging message by the UE

Upon receiving the Paging message, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	forward the ue-Identity, accessType (if present) and pagingCause (if present) to the upper layers;

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

3>	if the UE is configured by upper layers with Access Identity 1:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess;

3>	else if the UE is configured by upper layers with Access Identity 2:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess;

3>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

Editor’s note: For RAN paging reception in RRC_INACTIVE for Multi-SIM UEs, whether RRC informs the upper layers an indication about the RAN paging and the RRC layer resumes the RRC connection based on a request from the upper layer OR it’s up to UE implementation needs to be discussed.

2>	else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	forward the ue-Identity, accessType (if present) and pagingCause (if present) to the upper layers;

3> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

START OF CHANGE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v17xy-IEs                                              OPTIONAL

}

Paging-v17xy-IEs ::=                SEQUENCE {

pagingRecordList-v17xy              PagingRecordList-v17xy                                                  OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}                                                             OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v17xy ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v17xy

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v17xy ::=              SEQUENCE {

pagingCause-r17                     ENUMERATED {FFS}        OPTIONAL    -- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

Editor’s note: Whether to have one cause value or two cause values for “pagingCause-r17” needs to be discussed.

PagingRecord field descriptions

accessType Indicates whether the Paging message is originated due to the PDU sessions from the non-3GPP access.

pagingRecordList If the network includes pagingRecordList-v17xy, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix).

pagingCause Indicates whether the Paging message is originated for voice. Applicable to Multi-SIM UEs.

END OF CHANGE