3GPP TSG-RAN WG2 Meeting #116-e	R2-21xxxxx

Electronic, 1st – 12th Nov, 2021

Agenda item:	8.5.4

Source: 	Huawei, HiSilicon (Rapporteur)

Title: 	Summary of [Post115-e][513][IIoT] QoS survival time

Document for:	Discussion and decision

# Introduction

This is the summary of the following email discussion:

- [Post115-e][513][IIoT] QoS survival time (Huawei)

Scope: Discuss the following issues:

•     Details about Survival Time State Triggering based on HARQ NACK (and exiting mechanism), for example (MAC Procedures, PDCP behaviour, RRC configuration, PDCP pre- configuration and what is configured.   Discuss any potential issues (e.g. with pre-allocation of radio resources – CG resource wastage and availability)

•     Discuss any other solutions that may still have good amount of support

Intended outcome: agreeable baseline CR

Deadline: Long

| Company | Name | E-mail |
| --- | --- | --- |
| Huawei, HiSilicon | Tao Cai | tao.cai@huawei.com |
| CATT | Pierre Bertrand | pierrebertrand@catt.cn |
| Nokia | Ping-Heng Wallace Kuo | Ping-Heng.Kuo@nokia.com |
| vivo | Kimba Dit Adamou, Boubacar | kimba@vivo.com |
| ZTE | Ting Lu | lu.ting@zte.com.cn |
| Qualcomm | Sherif ElAzzouni | selazzou@qti.qualcomm.com |
| Ericsson | Zhenhua Zou | zhenhua.zou@ericsson.com |
| Samsung | Milos Tesanovic | m.tesanovic@samsung.com |
| Lenovo, Motorola Mobility | Joachim Löhr | jlohr@lenovo.com |
| Intel | Yujian Zhang | yujian.zhang@intel.com |
| InterDigital | Dylan Watts | dylan.watts@interdigital.com |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| Sequans | Olivier Marco | omarco at sequans.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| OPPO | Zhe Fu | fuzhe@OPPO.com |
| Fujitsu | Ohta, Yoshiaki | ohta.yoshiaki@fujitsu.com |
| TCL | Hejun Wang | hejun.wang@tcl.com |
| LG | SunYoung LEE | ssunyoung.lee@lge.com |
| Apple | Ralf Rossbach | rrossbach@apple.com |

# Overall description

The target of this email discussion is to design a workable Survival Time solution with details on triggering Survival Time state, the related behaviour inside Survival Time state and on exiting Survival Time state, also to figure out potential spec impacts on MAC, PDCP, RRC, etc.

In RAN2#115-e meeting, the following agreements for RAN enhancements based on QoS parameters have been made:

Agreements

- RAN2 does not assume that physical HARQ-NACK messages are always available, i.e. RAN2 will not mandate explicit HARQ-NACK feedback

- Given the application message size range under study, RAN2 will not optimize the ST design based on case of segmentation of message into multiple TBs. (This does not preclude the use of RLC segmentation; instead, it rules out optimizations for the case with RLC segmentation)

- Following entry into the Survival Time state, PDCP duplication for ST configuration is activated.  The gNB pre-configures which RLC entities can be activated for duplication when entering ST state.  FFS the number of supported RLC entities.

- RAN2 will at least continue working and discussing the HARQ NACK solution.  Details are FFS.

Based on these agreements, the instructions from session chair on the scope, and the recent discussions around “Rel-17 IIoT URLLC scope” in RAN#93-e, this email discussion will be focused on the Survival Time solution with HARQ-NACK based triggering, as well as on how other solutions could work with HARQ-NACK based triggering.

The following aspects will be discussed and companies can provide their views on the questions/issues listed in Phase 1. In Phase 2, companies’ inputs are requested on the specification implementations (e.g. on the baseline CR).

- On Survival Time state triggering:

- How can the UE identify the corresponding DRB(s) that should enter the Survival Time state based on the HARQ process ID of the retransmission grant?

- What could be the value of N, fixed to “one” or to be configured by the network?

- How to provide in time the additional resources to be used during Survival Time state? Also how to prevent the additional resources to be wasted outside Survival Time state.

- How to ensure UE and gNB have the common understanding on what RLC entities (legs) will be activated following entry into Survival Time state? (assuming Rel-16 PDCP duplication activation is the baseline behaviour)

  - The details on the interaction between MAC layer and PDCP layer, e.g., how MAC indicates PDCP layer to activate/deactivate PDCP duplication based on HARQ-NACK feedback.

  - What number of legs to configure, also consider the case PDCP duplication is already activated.

- On how to exit Survival Time state.

An exemplary high-level framework for Survival Time state triggering based on HARQ-NACK is illustrated in the following Fig.1.

Fig.1 High-level framework for Survival Time (ST) state triggering based on HARQ-NACK

# Phase 1

## On retransmission grant as trigger for entering Survival Time state

According to the previous discussions, it is assumed that at least the retransmission grant can be taken as implicit HARQ-NACK. In order that the retransmission grant can be used to trigger Survival Time state, the mapping relation between the service with Survival Time requirements and the received retransmission grant needs to be clarified.

In RAN2#113-e, we have agreed that “Only periodic traffic is considered for survival time work in Rel-17”. According to Rel-16 IIoT specifications, CG configuration(s) can be mapped to specific logical channel(s) to serve periodic deterministic traffic. When CG is used for data transmission from a DRB with Survival Time support, the UE can easily determine the corresponding DRB that should enter Survival Time state when receiving a retransmission grant for a CG. Besides, the network can also be aware of which DRB should enter Survival Time state, since the network clearly knows the transmission on the previous CG occasion fails to be decoded.

Further, following the second agreement of last meeting, for a single TB, the UE and the gNB would have the common understanding which DRB should enter Survival Time state.

- Given the application message size range under study, RAN2 will not optimize the ST design based on case of segmentation of message into multiple TBs. (This does not preclude the use of RLC segmentation; instead, it rules out optimizations for the case with RLC segmentation)

Q1: Would your company agree that CG resources will be used for service with Survival Time requirements, such that the mapping relation between the service and the retransmission grant is commonly known to both gNB and UE, and CG retransmission scheduling (addressed by CS-RNTI) can be used for Survival Time state triggering?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| CATT | Agree | This is indeed the most likely mapping for most stringent traffic types. |
| Nokia | Agree but… | Since we are dealing with low-latency periodic traffics, it makes sense to serve these traffics on configured grant based on Rel-16 framework. However, we are a bit concerned about the sentence in the question: “…the mapping relation between the service and the retransmission grant is commonly known to both gNB and UE…” In our understanding this is not necessarily “commonly known”, but the UE has to be explicitly configured by the gNB to realize a mapping relationship between a DRB and a LCH, a CG configuration, or a HARQ PID. RAN2 should further discuss what MAC parameter should be linked to the DRB for this purpose. |
| vivo | Agree | Based on TSCAI from AMF, RAN is able to allocate CG resources for IIOT service via mapping logical channel(s) for the service with Survival Time requirements to CG configuration(s). Thus, gNB and UE have a common understanding of the mapping relationship between service with Survival Time requirements and the CG resources. Even with multiple LCHs mapped to the same CG configuration, NW can always predict which services share certain CG resource given only periodic services are considered in Rel-17. Therefore, for CG, it is fine to consider CG retransmission scheduling addressed by CS-RNTI as implicit NACK, which is used for triggering ST state. |
| ZTE | Agree, but | We agree that CG resources will be used for service with Survival Time requirements and CG retransmission scheduling (addressed by CS-RNTI) can be used for Survival Time state triggering. However, as the CG retransmission scheduling may be lost, in such case gNB and UE may not have consistent understanding on the transmission status and UE may not be able to enter Survival Time state timely. Therefore, only relying on CG retransmission scheduling to trigger Survival Time state would be not enough. |
| Qualcomm | Agree | Given that we assume that a DRB configured with survival time would have periodic deterministic traffic and is scheduled on a dedicated CG. Let us denote this CG as CG1. The MAC can identify this re-tx grant and map it to a DRB with an ST flow by verifying that: - Re-tx DCI is scrambled by CS-RNTI - HARQ Process ID corresponds to CG1 - NDI is not toggled - The CG timer of CG1 is running. Given that the MAC can easily verify all this information by decoding the re-tx DCI, we expect the UE to quickly identify this “implicit” HARQ-NACK with no issues. |
| Ericsson | Agree with the first part, but disagree with the second part | On the first part (CG resources will be used for service with Survival Time requirements), additionally agree with Nokia that the mapping relation must be explicitly configured by the gNB. On the second part (CG retransmission scheduling (addressed by CS-RNTI) can be used for Survival Time state triggering), it greatly restricts gNB implementation flexibility: - The retransmission of the previously failed message is not essential to meet the survival time requirement, i.e., what matters is the subsequent message. Network may decide not to retransmit the previously failed message to allocate more radio resources for the subsequence message. This implementation possibility is ruled out by this retransmission grant based approach. - The gNB may dimension to use one initial transmission and one retransmission to meet the PDB/PER of the message (e.g., in the case of 2 milliseconds of delay budget). This implementation possibility is ruled out by this retransmission grant based approach. In UCE, the gNB may prefer letting CG retransmission timer to expire to triggering retransmission rather than sending an explicit retransmission grant. Additionally, the gNB may not be aware of the HARQ process ID, if the CG-UCI was not decoded by the gNB. |
| Samsung | Agree | Same understanding as vivo and Qualcomm. [We do not think that it is necessary to limit (by normative means) the mapping of ST traffic to CG only, but this is covered in questions that follow.] |
| Lenovo/MotM | Agree | Using CG resources for services/LCHs with ST requirements was from the beginning the common understanding |
| Intel | Agree |  |
| Huawei, HiSilicon | Agree | Same understanding as vivo and Qualcomm. Since we have agreed that “Only periodic traffic is considered for survival time work in Rel-17”, CG can be assumed to serve traffic with survival time support in Rel-17. |
| InterDigital | Agree | We have the same understanding as vivo and Qualcomm |
| Futurewei | Agree |  |
| Sequans | Agree with the first part, but disagree with the second part | Same view as Ericsson. Also retransmission grant means sending a DCI on the carrier that just failed. We don't think this is the best choice for an "emergency" mechanism. Sending a DCI on a different carrier (as it is anyway assumed there is one to save the situation) seems to make more sense. |
| Xiaomi | Agree |  |
| OPPO | Agree | It is a feasible way to support the service with Survival Time requirements. |
| Fujitsu | Agree | Similar view with Qualcomm, meaning that the UE just follows PDCCH that it receives from the gNB. |
| TCL | Agree, but … | We agree with the first part that “CG resources will be used for services with Survival Time requirements”. However, we share the same view with Nokia that mapping relation between the service and the retransmission grant should be explicitly configured. On the second part (CG retransmission scheduling (addressed by CS-RNTI) can be used for Survival Time state triggering), it is not enough for the CG retransmission scheduling may be lost. Thus additional mechanism is necessary as complementary, e.g. timer expiration. |
| LG | Agree with the first part, but disagree with the second part | We agree that the CG resource is used for a serving a traffic with a Survival Time requirement by associating the logical channel with the CG resource. However, we don’t think that the retransmission grant can be used for triggering ST mechanism because: - The deprioritized MAC PDU may not be known to the network. Note that this is the reason why we have autonomous transmission. Thus, the network may not provide retransmission grant timely. - Even though RAN2 agreed not to optimize ST mechanism in unlicensed band, it should work at least. In unlicensed band, as the UE selects the HARQ process for the CG resource by itself, the network may not know the HARQ process to which the retransmission grant should be provided. To resolve this, 1) the network may need assign only one or two HARQ processes for the CG, or 2) the network should provide retransmission grants for all possible HARQ processes. Method 1 is not flexible from scheduling/transmission perspective while method 2 is not desirable from resource efficiency perspective. In our view, a unified ST mechanism is preferred which could be applicable to both of licensed and unlicensed, prioritized and deprioritized transmission. In this regard, retransmission grant seems not a proper criterion for entering ST mode. |
| Apple | Agree but | Same understanding as Nokia. In addition, a UE may have multiple LCHs mapped to a CG where only some of them belong to a DRB configured for Survival Time. Therefore, it is not sufficient to simply check on the CG, the UE / gNB may as well need to verify whether the MAC PDU contains an SDU from a DRB configured with Survival Time support. Lastly, we do not think the agreement that “only periodic traffic is considered for survival time work in Rel-17” implies to tailor the solution to CGs only. Entering survival time state should be possible in response to data sent on a DG as well, as covered in Q3 below. |

Q2: Which option would your company support to specify (or not specify) the details on how UE identifies the corresponding DRB that should enter Survival Time state, according to the mapping relationship (as in Q1)?

Option 1: The details can be left to UE implementation.

Option 2: Specify the details in the spec, e.g. how the UE identifies the corresponding DRB according to the mapping relationship between CG configuration(s) and logical channel(s).

Option 3: Others. Please elaborate.

| Company | Option | Comments |
| --- | --- | --- |
| CATT | 2 | Survival Time is triggered in the UE by receiving a dynamic grant for a retransmission of a MAC PDU carrying an LCH associated with a DRB configured to support Survival Time. See also Q4. |
| Nokia | Option 2 with possibility of HARQ PID | We do not think it can be left to UE implementation, as the UE does not know which DRB has survival time requirement. Note that TSCAI is provided to the gNB rather than the UE. Moreover, in addition to LCH and CG as mentioned in Option 2, we believe the mapping relationship could be built between the DRB and HARQ PID as well. For instance, the UE may check the HARQ PID of the retransmission grant to decide if a DRB should enter survival time state. |
| vivo | Option 1 | MAC layer needs to record which LCHs are multiplexed in a certain MAC PDU. As a result, MAC can identify which DRB with ST requirements suffers transmission failure when retransmission grant is received. From our view, the behaviors are internal operations of UE, which can be left to UE implementation. |
| ZTE | Option 2 | According to the previous discussion and agreements, we assume the Survival Time related configuration would be provided to the UE which includes pre-configured PDCP duplication and RLC entities that can be activated for duplication. Based on this, UE can know which DRB is with Survival Time support (Also see Q4). Moreover, UE needs to map retransmission grant with a DRB and then determine whether a corresponding DRB should enter Survival Time state. For this part, the interaction between lower layer (i.e. MAC layer) and PDCP layer is also needed (Also see Q7). |
| Qualcomm | Option 3 | At the DRB level, this can be configurable by the network that a DRB has survival time constraint. At the CG level, the “entering survival state” behaviour can be a property of CG configuration. This does not need specification as long as the UE can identify a CG mapped to a DRB with survival time as in Q1. |
| Ericsson | Option 3: no need to specify. | Postpone this to stage-3 CR discussion. Since PDCP duplication is agreed as the baseline for survival time enhancement, what the UE can do is to activate/de-activate the RLC entity for PDCP duplication. There is a one-to-one mapping from the logical channel to the DRB (i.e., from the logical channel ID and the RRC configuration, the UE knows which DRB this logical channel belongs to). Therefore, if there is a mapping relation between CG and logical channel, then the UE know the corresponding DRB. |
| Samsung | Option 2 | It is true (as Qualcomm mention) that ST behaviour can be a property of a CG configuration; however, this assumes a one-to-one mapping between DRBs and CGs (no multiplexing). |
| Lenovo/MotM | Option 2 | We agree that part of the UE behaviour for entering ST state should be specified. The mapping between DRB/LCH and CG as well as the DRB configuration w.r.t PDCP duplication details when entering ST state is configured by NW. |
| Intel | Option 1 | As the evaluation of survival time entry is based on HARQ NACK, and UE clearly knows which DRBs are included in the UL MAC PDU associated with the HARQ NACK, we think this can be left to UE implementation. What needs to be specified can be something in high level like “When HARQ NACK is received, for DRBs configured with survival time and associated with the HARQ NACK, …” |
| Huawei, HiSilicon | Option 1 or Option 2 | We may specify that the UE to identify the DRB, e.g. based on HARQ-NACK information, mapping relationship between CG configurations and LCHs, the information about DRB with survival time support, etc. Nevertheless, all these “linking” behaviours are internal operations of UE, which can be left to UE implementation. |
| InterDigital | Option 1 or Option 2 | Depending on the mapping between DRBs and CGs, a UE may implicitly identify the corresponding DRB that enters ST state. |
| Futurewei | Option 1 | With CG resources configured for DRB with ST requirements and CG retransmission scheduling used as the ST trigger, the UE should be able to implicitly identify the DRB having the ST requirements. |
| Sequans | Option 3 | We don't support the "activate PDCP duplication upon entering ST state" solution, for reasons explained later. In our view PDCP duplication should have been preactivated, and it is the opportunistic resource allocation on additional legs during "ST state" that provide the required reliability boost. Such opportunistic resource allocation can make use of existing LCH mapping restrictions, e.g. CG restricted to given LCH, or DG restricted with maxPUSCH-Duration and/or allowedPHY-PriorityIndex. |
| Xiaomi | Option 2 | As the UE could be configured with multiple mapping(s) between CG and DRB, the UE is not able to know which DRB needs to fulfil the survival time requirement. Some indication from the gNB is needed to identify which DRB needs to fulfil the survival time requirement. |
| OPPO | Option 3: no need to specify. | The DRBs can be configured by the network with the constraint on ST requirements. The mapping rule between the DRBs with the ST constraint and the CGs can be configured by the network, using e.g. allowedCG-List-r16. The UE can know the corresponding DRB by the configurations mentioned above. Thus, there is no need to specify anything. |
| Fujitsu | Option 2 or 3 | Option 1 seems not work because TSCAI is not provided to the UE. So the Options 2 or 3 would be taken into account. However, we would first discuss what configurations would be provided to DRB. If PDCP pre-configuration is provided to the UE for ST mode handling, then the corresponding RLC is also known to the UE and the LCH(s) can also be identified, which further can identify the associated CG resource(s). This means that ST-related configuration can identify the corresponding DRB. |
| TCL | Option 3 | Agree with other companies that we should specify how UE identifies the corresponding DRB which should enter Survival Time state. And the mapping relation between DRB/LCH and CG can be used for the UE to identify DRB in Survival Time state. However, besides PDCP duplication, other solutions, e.g. L1/L2 adaption, are not precluded in the discussions of previous meetings for UE to improving transmission reliability for UE enters Survival Time state. In another hand, as is mentioned in Q3, with DG taken into consideration, the UE and the gNB may have different understanding which DRB will enter Survival Time state when receiving a retransmission grant. Thus We suggest to postpone this discussion until the problems mentioned above are discussed. |
| LG | Option 2 | Before deciding whether to specifying it or not, RAN2 should first discuss for which RBs the PDCP Duplication should be ON. Method 1. All of the RBs mapped to the CG for which the retransmission grant is received. Method 2. Only the RBs included in the MAC PDU transmitted on the CG for which the retransmission grant is received. As the network does not know which RB is included in the MAC PDU, it should assume all RBs mapped to the CG are activated in ST mode, i.e., method 1, based on the mapping between the CG and the logical channel. In addition, from the UE perspective, method 1 is also simple because the UE does not need to look at the content of the MAC PDU again when the retransmission grant is received. Thus, we prefer method 1. Without specifying which RBs to activate, the UE may be confused whether to take method 1 or 2. Thus, we have to specify. |
| Apple | Option 2 or 1 | We assume the network may configure a DRB with an attribute for survival time. UE implementation can identify whether a HARQ-NACK was received for a MAC PDU carrying an SDU from a DRB configured with survival time support (and related pre-configuration of resources to be used for PDCP duplication). However, since the UE and the network need to have on a common understanding of the survival time status, some high-level details may need to be specified. |

If DG is used for a DRB with Survival Time support, the network may have no prior knowledge that packets from which specific logical channel(s) will be multiplexed into the MAC PDU generated for the DG. Even though several LCP restrictions are defined in spec, it is still difficult for the network to allocate a dynamic grant to serve only one specific logical channel. There is a risk that the UE and the gNB have different understanding which DRB will enter Survival Time state when receiving a retransmission grant. A confirmation could be done on this case for the following specification work.

Q3: Would your company agree not to consider DG is used for DRB with Survival Time support and retransmission grant for DG (addressed by C-RNTI) is not considered as Survival Time state trigger in Rel-17?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| CATT | Disagree | Although we agree that the most common case is for CGs, we don’t see the need to bring a restriction on DGs. In some scenarios DGs could be used on top of CG to address e.g. packet jitter and the UE may serve only few (e.g. 2) LCHs so that there would be no ambiguity regarding which LCH was carried in the failed DG (e.g. using maxPUSCH-Duration and/or allowedPHY-PriorityIndex). |
| Nokia | Disagree | In the end how the data is conveyed may be up to implementation, so we do not think it is necessary to have such restrictions. |
| Vivo | Agree | We agree with the rapporteur that gNB has no way to restrict which LCH(s) can be mapped to a DG, hence UE and the gNB may have different understanding on which DRB should enter Survival Time state when receiving a retransmission grant. To avoid such confusion and complexity, we can focus on the cases where CG is used for DRB with Survival Time in Rel-17. |
| ZTE | Disagree | We have similar understanding as CATT that in the case that initial transmission is on CG, DG can be used for retransmission grant. We don’t see any potential ambiguity between gNB and UE about mapping between DRB and retransmission grant. |
| Qualcomm | Agree | Although we agree with Nokia and CATT that Survival time is a QoS parameter that doesn’t necessarily need to be tied to a specific grant type, we think that given the time remaining in Rel-17 and the current scope of periodic deterministic traffic focusing on CG and avoiding complications coming from DG is the straightforward option. For example, the DG does not have the same LCH restrictions as CG, so the UE would first need to determine whether a DG is carrying a survival time payload or not. RAN2 would need to look at how to solve the problem in DG as well, which would probably need too many rounds of discussion given the available time. |
| Ericsson | Agree | It is agreed that only periodic traffic is considered for survival time work in the Rel-17, and thus it is a typical configuration by the network to use CG instead of DG. |
| Samsung | Disagree | Same view as CATT and Nokia. Imposing this limitation would not make much difference in our view to the standardisation workload, but it would impose unnecessary restrictions. |
| Lenovo/MotM | Disagree | We don’t see a need to have this restriction even though the most common case is that CG resources are used for DRBs with Survival Time requirements. |
| Intel | Agree | Agree with rapporteur. |
| Huawei, HiSilicon | Agree | We can focus on the case where CG is used for DRB with Survival Time support in Rel-17. |
| InterDigital | Disagree | Same view as CATT and Nokia. |
| Futurewei | Agree | We should at least ensure a design based on CG can be finished within the remaining time in Rel-17. |
| Sequans | Disagree | The gNB may want to override a CG with a DG. Or even only use DGs. The gNB anyway knows it expect e.g. 1 packet on LCH x every transmission interval y, and can act whenever it is not received, whatever the resource used, CG or DG. So we don't need such restriction. |
| Xiaomi | Agree | Given the limited remaining time in Rel-17, we prefer to focus on the CG. |
| OPPO | Agree | As we agreed, only periodic traffic is considered for survival time work in Rel-17. Typically, it is CG resources are used for periodic traffic. Thus, we prefer to focus on CG resources for ST requirements. |
| Fujitsu | Disagree | Similar view as Nokia. |
| TCL | Disagree | Even though the most common cases are for CG, we still need DGs to handle some scenarios, e.g. when CG based initial transmission failed, we may need DG to perform retransmission. With the DGs as supplementary to CGs, we will not have to configure CGs for such retransmissions which would lead to resource inefficiency for the transmission failure seldom happens. With the consideration above, we hold the view that the restriction is not necessary. |
| LG | Agree | It is sufficient to focus on CG because we have assumed periodic traffic in Rel-17 for ST mechanism. |
| Apple | Disagree | While the support of multiple configured grants is an optional capability IoT terminals may have very tailored feature sets. Some UEs may just support a small number of CGs where parallel support of DG and CG is required to serve the data from all DRBs. To restrict survival time to only CGs seems an unnecessary limitation. |

In order to distinguish which DRB(s) is with Survival Time support, it is thought that one parameter can be configured for a DRB to indicate its requirement for Survival Time support by RRC, as also suggested in [5][6].

Q4: Would your company agree that a RRC parameter is configured for a DRB with Survival Time support?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| CATT | Agree | Survival Time support is configured at DRB level, and a new parameter survivalTimeSupport can be added in PDCP-Config along with PDCP duplication configuration. |
| Nokia | Agree | Based on RAN2’ agreement: - Following entry into the Survival Time state, PDCP duplication for ST configuration is activated. The gNB pre-configures which RLC entities can be activated for duplication when entering ST state. FFS the number of supported RLC entities. We believe at least in the pdcp-Config of the DRB some configuration parameters are needed, so the UE knows how it should behave for this DRB when it is in or out of the survival time state. We think whether this DRB has survival time requirement can be implied via the presence of this “state-dependent adaptive behaviour” parameter, so if this parameter is absent then it basically means the DRB has no survival time requirement. Any other “explicit” configuration is probably not needed. |
| Vivo | Agree | We agree that a new RRC parameter should be introduced to configure the Survival Time requirement for a DRB. In addition, it was specified in TS23.501 that when Survival Time information is provided for a TSN stream, then it should not be aggregated with other TSN streams into a single QoS flow, or if they are aggregated, then the Survival Time parameter shall not be provided. Follow the same light, we think that RAN should restrict that only one QoS flow can be mapped into the DRB with Survival Time requirement configured. |
| ZTE | Agree | RAN2 has agreed to support Survival Time related configuration, e.g., pre-configured PDCP duplication and RLC entities which can be activated for duplication when entering ST. We assume such configuration is provided for each DRB with Survival Time support and can be done with RRC parameter. |
| Qualcomm | Agree | One option is to configure some RRC parameter at the DRB level to indicate the presence of an ST parameter. Rel-17 should keep this feature such that all DRBs that map into a CG should have the same ST configuration so this can also be a CG property. |
| Ericsson | Disagree | We are not against adding RRC parameters and it is obvious that RRC parameters are needed. In the end, what matters is UE actions when a parameter is configured. However, the UE actions are not clear yet and thus we prefer postponing this to the running CR discussion. |
| Samsung | Agree | No need to postpone this to the running CR discussion (as proposed by Ericsson) – this question is sufficiently high-level that we can agree now. Spec details can be ironed out later. |
| Lenovo/MotM | Agree | PDCP/RLC duplication configuration used by UE during ST state is according to RAN2 agreements preconfigured. |
| Intel | Agree |  |
| Huawei, HiSilicon | Agree | At least a RRC parameter is needed. The related details, e.g. it is configured for a DRB or a CG configuration, can be left for Stage-3 discussion. |
| InterDigital | Agree |  |
| Futurewei | Agree | Agree on the need for such an RRC parameter. Details can be discussed in stage 3. |
| Sequans | Disagree | Same view as Ericsson. Not sure what is the point to agree that now. |
| Xiaomi | Agree |  |
| OPPO | Agree | At least a new RRC parameter should be introduced to configure the DRB with the ST requirement/constraint. |
| Fujitsu | Agree | Exact parameters can be discussed in Stage-3 and RRC running CR discussions. |
| TCL | Agree, but… | We agree that the new RRC parameter configured for a DRB is needed for the Survival Time support is configured at DRB level. Furthermore, for the mechanisms we discussed to improve the transmission reliability for UEs in Survival Time state, e.g. PDCP duplication, are not restricted to Survival Time issue, we hold the view that explicit configuration is needed. However, we have the same concern with Ericsson, the UE actions should be specified when this parameter is configured. |
| LG | Disagree | This is a stage-3 level discussion which can be easily finalized if the UE behaviour of PDCP Duplication is clearly defined. |
| Apple | Agree |  |

In [2], it was discussed whether the triggering condition for entering Survival Time state shall be configurable number N(≥1) of consecutive HARQ-NACKs. Compared with simply fixing N=1 in spec, configurable N (≥1) would bring more spec impacts while would be able to prevent “too early” entering into Survival Time state in certain scenarios. With configurable N (≥1), the UE may need to use a counter to count the consecutive transmission failures. It will further involve the initialization, counting, resetting of the counter. Besides, further discussion is needed on where the counter is maintained, e.g. in HARQ entity, in MAC entity, in RLC entity, or in PDCP entity.

Q5: Which option would your company support for N HARQ-NACKs as Survival Time state trigger?

Option 1: fix N=1.

Option 2: N is configurable and can be 1 or larger than 1.

| Company | Option | Comments |
| --- | --- | --- |
| CATT | Option 1, but see comment | For a DRB configured with moreThanOneRLC: Survival Time is triggered in the UE by receiving one dynamic grant for a retransmission of a MAC PDU carrying an LCH associated with a DRB configured to support Survival Time. For a DRB configured with moreThanTwoRLC-DRB: Survival Time is triggered in the UE by receiving, for each logical channel with PDCP duplication activated associated with this DRB, one dynamic grant for a retransmission of the last MAC PDU carrying that logical channel. This latter behaviour in case PDCP duplication is already activated for a subset of RLC entities could be interpreted as Option 2. |
| Nokia | Option 1 | In our understanding, regardless of what survival time requirement is, N=1 is able to solve all problematic scenarios. Even the UE may enter the survival time state too early, it is much better to be safe than sorry in critical applications in IIoT/TSC use cases. Option 2 requires the UE to implement a counter, which adds complexity. Furthermore, Option 1 is less sensitive to situations where the UE erroneously decode the PDCCH. |
| vivo | Option 2 | For some use cases (e.g. transfer interval = survival time = 2ms), force UE to enter ST state after only 1 retransmission grant is received will cause unnecessary PDCP duplication activation and resource waste. Thus, the N should be configurable. |
| ZTE | Option 2 | Previously we already have had much discussion on the necessity of N greater than 1. Simply to say, considering the different service characteristics and in order to avoid unnecessary resource waste, we should support N greater than 1. Moreover, even N is restricted to 1, we think all the mentioned specification work, e.g., counting and resetting counting are still needed. There is not much difference between the specification work for N=1 or N greater than 1. We don't think setting of N is related to the number of RLC entities. No matter how many RLC entities are there, N can be 1 or greater and is counted per each RLC entity. Therefore, we disagree with above CATT view that counting once retransmission grant for several RLC entities can be equivalent to counting N greater than 1. In the case of several RLC entities, we have such similar understanding as CATT that only when receiving the indications about receiving retransmission grant from all the RLC entities, the ST state needs to be entered. |
| Qualcomm | Option 1 (but open to option 2) | We think N=1 can solve the most critical cases of short survival times. However, we understand the concern of some companies of entering survival time too often or a ping-pong effect where the UE keeps entering and exiting a survival state. Thus, we are open to both options if a use-case was indeed made for N>1. We do not think managing a counter of HARQ-NACK N at the MAC would add much complexity. |
| Ericsson | Option 1 | We agree that in the cases of long survival time and PDB (e.g., two milliseconds), gNB may additionally dimension one retransmission to meet the PDB. However, we are more concerned on the complexity of maintaining the counter not only in the specification but also in the network implementation, e.g., PDCCH decoding error mentioned by Nokia. |
| Samsung | Option 2 | With Option 1 we may be entering ST state too frequently. Additionally, as vivo points out, it’s wasteful to force the use of N=1 for all relevant applications. |
| Lenovo/MotM | Option 1 | In our understanding Option 1 is sufficient. Option 2 will introduce additional complexity. We have similar as Nokia that it’s better to have N being fixed to 1 from reliability point of view and also from the perspective of avoiding mismatch between UE and NW. |
| Intel | Option 2 if RAN2 agrees to support N HARQ-NACK solution | As in our reply in section 3.6, we prefer to use HARQ-NACK and Tx side timer / elapsed time jointly as survival time state trigger. If RAN2 prefers to use HARQ-NACK only, we prefer Option 2 as it can supports slightly longer survival time which allows one or more HARQ retransmissions. |
| Huawei, HiSilicon | Option 1 | Similar view with Nokia and Ericsson. N=1 can handle the most stringent use cases. For cases which can tolerate several HARQ retransmissions, the network will have enough time to react (e.g. with conventional tools) to avoid survival time expiry. Considering ST state won’t occur frequently, the “too early triggering of ST state” and related “waste of resource” might be secondary issues. Besides, maintaining a counter for N>1 would incur extra complexity to the UE, as well as to the network. |
| InterDigital | Option 1; open to Option 2 | As Nokia mentions, it is better to be safe and enter ST state early. |
| Futurewei | Option 1 | But open to Option 2. |
| Sequans | None or Option 1 | We don't support using HARQ-NACK, but if it is agreed Option 1 is simpler. |
| Xiaomi | Option 2 | Option 2 is more aligned with survival time requirement provided by SA. However the survival time requirement is to count the number of consecutive packet loss of multiple application packets. It seems that Option 2 cannot count the number of consecutive packet loss of multiple APPLICATION packets |
| OPPO | Option 1 | We also think N=1 is sufficient to handle the most critical cases of survival time. For Option 2, we have some concerns on the complexity of UE maintaining the counter. But, we also understand companies concern on Option 1 of entering ST too often. Thus, we are open to Option2 if the majority wants. |
| Fujitsu | Option 2 | Option 2 can cover Option 1. |
| TCL | Option 2 | Generally, we agree with the view that N=1 can solve most critical cases of short Survival Times. However, for the services can tolerate more than one packets losses, with N = 1, the service shall enter Survival Time and trigger PDCP duplication too often which shall lead to unnecessary resource waste. Thus we prefer option 2 that the N is configurable which is more flexible. For the need of counter, we don’t think it’s a problem. The complexity introduced is acceptable. |
| LG | Option 1 | We could focus on N=1 for now. |
| Apple | Option 2 | We would like a configuration option to cover a range of scenarios as well as prevent the UE from entering ST state unnecessarily which is also not an efficient use of resources. |

Q6: In case RAN2 agrees to Option 2 as above, which entity shall handle the counting of N? Please comment on any detail needs to be considered to make each option work properly.

Option 1: MAC entity

Option 2: PDCP entity

Option 3: Others, e.g. RLC entity

| Company | Option | Comments |
| --- | --- | --- |
| CATT | 1 | For the case of a DRB configured with moreThanTwoRLC-DRB, the counting of HARQ-NACKs over the logical channels with PDCP duplication activated should be done in MAC. |
| Nokia | 1 | HARQ is a MAC/PHY mechanism, certainly it is the most straightforward solution for MAC to do the counting of NACK it has received. |
| vivo | Option 2 | For service with stringent Survival Time requirements, PDCP duplication may be activated even before entering the ST state, assuming PDCP duplication with more legs will be activated after UE entering ST state. In this case, before entering ST state, UE has to monitor retransmission grant for all the activated legs to perform counting of N. If the counting is performed under PDCP, the interaction between different legs may be needed, which should be avoided. Thus, it is more suitable to handle the counting of N at PDCP entity. |
| ZTE | Option 1 and with comments | The counting of N (retransmission grant) for each RLC entity would be in MAC entity. But for the complete Survival Time function, we think PDCP entity should also be involved, e.g., when counting of N reaches the threshold, an indication about reception of retransmission grant needs to be sent from MAC layer to PDCP layer (Also see Q7). With above assumption, we disagree with CATT that counting N over several LCHs is also be in MAC entity. Counting like this may require interaction between different HARQ processes that seems not a general way. Instead, as PDCP entity itself needs to maintain RLC entities, we think it’s more reasonable to let PDCP entity process the indications from different HARQ processes/RLC entities. PDCP entity can finally decide whether to trigger ST state based on the combined analysis of the transmission status on all the RLC entities. |
| Qualcomm | 1 | To minimize interaction between MAC and PDCP, MAC should do the counting and only communicate with PDCP through a trigger signal. |
| Ericsson | Option 1 | As the baseline, we should start with the layer in which this command is received. |
| Samsung | Option 1 | Agree with Ericsson. |
| Lenovo/MotM | Option 1 |  |
| Intel | Option 1 | Our understanding is that the question is related to in which specification to define the trigger behavior in general, not specific regarding counting of N. There is coupling between MAC and PDCP regarding the triggering of survival state, and we prefer to define the triggering operation in MAC since it is the first receiver layer that is handling the HARQ NACK and is aware of the LCH/DRB configured with survival time handling. |
| Huawei, HiSilicon | Option 1 | Agree with Nokia and Ericsson |
| InterDigital | Option 1 | Agree with Nokia. |
| Futurewei | Option 1 |  |
| Xiaomi | Option 1 or 2 | We think both Option 1 and 2 are feasible, as the count is anyway within the UE. |
| OPPO | Option 1 |  |
| Fujitsu | Option 1 | MAC deals with MAC. |
| TCL | Option 1, but … | Agree with ZTE and Qualcomm. HARQ operation mainly in MAC and PHY, hence the counting of N for each RLC should be in MAC entity. However, the Survival Time is configured at DRB level, the PDCP shall be involved, e.g. the interaction between the MAC and the PDCP. |
| LG | Option 1 |  |
| Apple | Option 1/3 | To handle the counting in MAC seems preferable. We would like to avoid frequent interaction between MAC and PDCP when several HARQ-NACK occasions are to be supervised. However, while the configuration of N and respective trigger actions need to be specified (which has a direct link to MAC), the detailed procedure and the exact sublayer handling the counting can be left to UE implementation. |

## 3.2 On receiving N HARQ-NACKs

In Rel-16 MAC spec, when Duplication Activation/Deactivation MAC CE or Duplication RLC Activation/Deactivation MAC CE is received, the MAC will indicate the activation/deactivation information for a DRB or RLC entity(s) to the PDCP. Similarly for Survival Time solution, if the lower layer entity identifies that one DRB is to enter Survival Time state, the interaction between lower layer and PDCP layer is necessary in order to aid the PDCP adapt legs for robust transmission. It is assumed such interaction in Rel-17 would be similar as that in Rel-16, the following question is for confirmation.

Q7: Would your company agree to specify the interaction between lower layer (i.e. MAC layer) and PDCP layer for Survival Time state triggering?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| CATT | Agree but | We do not see any impact to R16 PDCP specification |
| Nokia | Not sure | Cross-layer interaction is definitely needed, but not sure if specification is needed for the time being. We can discuss it when RAN2 is working on Stage-3 details. |
| Vivo | Agree | MAC shall provide NACK information to assist PDCP layer to perform the counting of N. |
| ZTE | Agree | As mentioned in Q6, we assume PDCP entity must be involved, especially in the case of multiple legs. We think to let MAC entity count N over several LCHs may need interaction between HARQ processes and is not a general way. Instead, it’s more reasonable to let PDCP entity analyse the transmission status on all the RLC entities and finally determine whether to enter ST state. So the interaction between lower layer (i.e. MAC layer) and PDCP layer and also how to enter ST state based on the process of all the interactions in PDCP layer needs to be specified. Afterward, the process after entering ST state and triggering PDCP duplication would follow legacy way. |
| Qualcomm | Agree | We do not see needed changes to PDCP. In fact, this is similar to legacy MAC CE activation of PDCP duplication, except that the MAC now can also trigger activation by receiving a different L1 signal (implicit HARQ NACK), but the same mechanism should hold. |
| Ericsson | Agree |  |
| Samsung | Agree | Same understanding as Qualcomm. But are happy to delay this as per Nokia’s suggestion since this is fairly low-level, spec drafting issue. |
| Lenovo/MotM | Agree | Some level of layer interaction between MAC and PDCP needs to be specified in our understanding. |
| Intel | Agree |  |
| Huawei, HiSilicon | Agree | PDCP entity is responsible for activating PDCP duplication, while HARQ-NACK is perceived by MAC entity. Interaction between MAC layer and PDCP layer seems needed, but we are also fine to discuss the details later. |
| InterDigital | Agree |  |
| Futurewei | - | UE should be able to reuse the interaction between MAC and PDCP for the legacy MAC CE based activation. Except the trigger for the interaction being different for ST, not sure anything new is needed. |
| Sequans | Disagree | We don't support the "activate PDCP duplication upon entering ST state" solution, for reasons explained later. |
| Xiaomi | Agree |  |
| OPPO | Agree | The interaction between MAC and PDCP is needed to some extent. But, it can be discussed later since it is stage-3 details. |
| Fujitsu | Disagree | Interaction is needed, but not sure if such an internal interaction within device is clearly specified in specifications. |
| TCL | Agree | As many companies have mentioned in Q6, more than one layers, e.g. MAC and PDCP, are involved in the Survival Time function, Thus interactions between different layers are needed. Agree with Nokia to postpone the cross-layer interaction discussion. |
| LG | Agree |  |
| Apple | Agree | Interaction between MAC and PDCP is needed, but whether the mechanism is similar to Rel-16 or whether an extended mechanism is suitable can still be discussed. |

We have agreed that PDCP duplication activation is used for Survival Time state, obviously a DRB configured with Survival Time support should be configured with PDCP duplication.

- Following entry into the Survival Time state, PDCP duplication for ST configuration is activated.  The gNB pre-configures which RLC entities can be activated for duplication when entering ST state.  FFS the number of supported RLC entities.

Further, according to Rel-16 specs, if PDCP duplication is configured for the DRB, the network can configure the initial duplication state for the DRB, e.g. through the PDCP-duplication parameter in moreThanOneRLC IE if only two legs are configured, or through the duplicationState parameter in moreThanTwoRLC-DRB IE if more than two legs are configured. It is understandable that the behaviour, following entry into the Survival Time state, shall also be specified when the initial duplication state is set as “true”, i.e., the PDCP duplication state is already activated for the associated RLC entity. In any case, the network and the UE shall have the common understanding about the duplication state adopted for the DRB in Survival Time state. As commented by some companies [2], for the DRB entering Survival Time state, it is reasonable to activate all legs to guarantee the robustness of next transmission. Some companies however think other options can be adopted, for example, the network can explicitly indicate PDCP duplication state when the DRB enters Survival Time state [15]. It is rapporteur’s understanding that the below Option 2 is a generic solution while the implementation of Option 1 (Survival Time state duplication state could be fixed in the spec) would be simpler.

Q8: Which option would your company support on the duplication state adopted in Survival Time state for a DRB?

Option 1: Activate all configured legs, following entry into Survival Time state.

Option 2: Network indicates by RRC, e.g. a bitmap, the PDCP duplication state that the UE should apply upon entry of Survival Time state, the UE changes the duplication state accordingly.

Option 3: Others. Please elaborate.

| Company | Option | Comments |
| --- | --- | --- |
| CATT | 1 | Given Survival Time is the most critical time a DRB can experience, why would NW configure a DRB with N legs for duplication, but would only use a subset of those during Survival Time? We don’t see a valid justification for the added flexibility (and complexity) of option 2. |
| Nokia | 2 | We do not see a valid justification to limit gNB flexibility like in Option 1. We would like to point out that, from a gNB vendor point of view, configuring N>2 RLC entities for a DRB does NOT mean that the gNB wants the UE to use all of these N RLC entities for duplication in the most critical situation. The gNB may simply decide to configure additional RLC entities for higher flexibility in terms of resource allocation. Besides, if we allow the UE to activate all RLC entities, then it basically mean the gNB has to provision resources for all these legs – which is extremely wasteful if this is not needed. Option 2 basically covers all possibilities including Option 1, so it provides the best implementation flexibility for the gNB to decide what the UE should do when a DRB enters the survival time state. This is a win-win situation for all companies with different reliability boosting mechanisms in mind. It is worth noting that, it should be up the gNB to decide the way to improve the reliability of the associated radio link, and “increasing the number of copies” is NOT the only possible way to boost the reliability. In fact, activating more RLC entities may lead to potential performance degradation instead due to power limitation of the UE. In many cases, the gNB may not want to the UE to increase the number of copies even if survival time state is triggered, e.g. - The gNB does not want the UE to generate more interference (e.g. cross-link interference) toward other gNB/UEs. - The gNB wants to utilize some radio resource for other UEs or traffics by deactivating some of the legs while the UE is in survival time state. - The gNB think the desirable reliability can be achieved simply by switching the leg (e.g. from FR2 to FR1) - The gNB think the desirable reliability can be achieved simply by switching the associated CG configuration (e.g. from LCH1 to LCH2, where LCH1 and LCH2 have different LCH mapping restrictions) All in all, since the gNB has knowledge about the UE and the overall network status, this is should be up to gNB implementation to decide what UE should do in survival time state. Option 1 essentially eliminates gNB’s flexibility in such context, which should not be considered. |
| vivo | Option 1 | When entering the ST state, UE should do its best to improve the reliability of the subsequent packets. The most straightforward way is to activate all configured legs for duplication transmission. |
| ZTE | Option 3 | In our understanding, network provides the Survival time related configuration (but it may not be a bitmap type). And then UE can decide how many legs and which legs (among the pre-configured set of RLC entities) are used for PDCP duplication after entering ST state, e.g., based on the link quality of the associated serving cell(s) which is well known by the UE. We don’t think it means that UE change the duplication state. |
| Qualcomm | 1 | We prefer to keep the behaviour simple by activating all configured legs to increase reliability in survival state to a new (higher) target. We do not see much value in the associated dynamicity of which RLC legs are activated at the cost of the complexity of adding multiple configurations then signalling to the UE how to navigate those configurations. |
| Ericsson | Agree on the need to de-activate some RLC entities, but prefer another approach, i.e., Option 3 | We share the concern by Nokia, in particular on UL UE power limitation and network implementation flexibility. On the other hand, the bitmap can only be (re)-configured by RRC which is a slow process and inflexible for the gNB. Even though further enhancements by MAC CE are possible, it may be challenging to finish in this release. We prefer the below approach in which the bitmap is effectively achieved by sending the corresponding CG type 2 activation/de-activation command: - a CG type 2 activation command to activate one RLC entity for PDCP duplication; a CG type 2 de-activation command to de-activate one RLC entity for PDCP duplication; - The network configures by RRC which CG command (with a certain index) is linked to which RLC entity. |
| Samsung | Option 1 | We understand Nokia’s concerns about potentially limiting NW behaviour if Option 1 is adopted; however our understanding is similar to CATT’s – that this limitation is not a major restriction, since Option 1 is the likely/expected behaviour. |
| Lenovo/MotM | Option 1 | We don’t have a strong opinion here. However, Option 1 seems to be the most straightforward option and the potential drawbacks addressed by Nokia can be also considered by configuring the number of RLC legs and CG resources (power splitting issue). |
| Intel | Option 1 | Option 1 is simpler and provides maximum reliability. In addition, survival time triggering is a rare event, therefore resource overhead of option 1 is not an issue. |
| Huawei, HiSilicon | Option 1 or Option 2 | We have sympathy for Nokia’s proposal yet we prefer Option 1 as much easier for implementation. Further, if legacy activation/deactivation MAC CE is agreed to be used by gNB (as in next question), gNB may “tailor” PDCP duplication state for the UE even outside ST state. In this way, “deactivating unneeded RLC entities” of Option 2 can be already done beforehand. |
| InterDigital | Option 1 | Similar to others, we believe that a simpler approach is best and is the most likely used configuration even if Option 2 were agreed upon. |
| Futurewei | Option 1 | The extra (and normally de-activated) legs are configured for the purpose of meeting ST requirements. Therefore, option 1 is simple and straightforward. |
| Sequans | Option 3 | We don't support the "activate PDCP duplication upon entering ST state" solution, for reasons explained later. In our view, the ST requirements could already be achieved by e.g. alternating allocations on different carriers (as described in our contribution [19]). Still, it seems not enough for companies that push for "activate PDCP duplication upon entering ST state" solution. Problem Requirement for this "solution" (as we understand it): upon failure of packet N transmission, ensure that packet N+1 is transmitted (on additional carrier(s) ) with PDCP duplication. Our proposed solution: - PDCP duplication should have been preactivated, and it is the opportunistic resource allocation on additional legs during "ST state" that provide the required reliability boost. - Existing LCH mapping restrictions are used, e.g. gNB can activate a CG with specific LCH mapping on additional leg whenever needed - The PDCP discard timer can be set to the transmission interval duration to ensure latest packet will be sent. Obviously, it is possible to have 2 packets being queued at the same time in an additional leg (due to arrival jitter), but this can be easily handled by gNB scheduling a first UL grant with twice the TB size. If really companies want a single packet, it could be easily specified that RLC SDUs from a given LCH are discarded by arrival of a new RLC SDU (i.e. discard old RLC SDUs such that only N=1 RLC SDU mac is buffered at a given time). Benefit: - No spec change - PDCP processing (including duplication) remains out of PHY control loop, no additional timing constraints, i.e. it is certain that the packet would have already been duplicated - No need to rely on a DCI sent on a carrier that just failed |
| Xiaomi | Option 1 and 2 | We think both Option 1 and 2 can have its own use cases to satisfy different reliability requirement. We can accept to leave some flexibility for gNB configuration. |
| OPPO | Option 1 | If the UE is already triggered to ensure ST requirement, the most important thing is to increase the reliability. From this perspective, Option 1 is the simple and most straightforward option. |
| TCL | Option 2, but not object to option 1 | To activate all configured legs in option 1 is simple for the standard. However, the channel quality of the RLC entity changes which leads to different transmission reliabilities of different legs. The option 1 is not able to handle such case. As Nokia mentioned, the option 2 can handle the changeable radio environments of different legs with L1/L2 adaption achieved on top of duplication. So we share the same view with Nokia to prefer option 2. But not object to option 1, for simplicity. |
| LG | Option 2 | We think Option1 can be achieved by Option 2 and don’t see any need to limit the configuration to option 1. If a DRB is configured with PDCP Duplication for ST mode, there can be some common RLC legs which are activated in both of non-ST mode and ST mode. However, it doesn’t necessarily mean that all legs should be activated upon entering into ST mode but some additional/different RLC legs may need to be activated in ST mode. |
| Apple | Option 2 | We think the proposal from Nokia offers flexibility by adapting the configuration in a more fine-granular manner, while providing a benefit to efficiently utilize radio resources. Furthermore, this solution allows for potential extensions in the future, for example, through additional LCP restrictions if required. Nevertheless, support of survival time should not force the UE having to support PDCP duplication over more than two RLC entities as well. We assume the UE capability for 'more than two' legs remains independent and there will be a separate optional capability for survival time. |

The following question is to confirm that for the DRB with Survival Time support, whether or not the network is allowed to control the duplication state for the DRB via legacy activation/deactivation MAC CE.

Q9: For the DRB configured with Survival Time support, would your company agree that the network can still control the duplication state for the DRB via legacy activation/deactivation MAC CE (Please note this question is also relevant to “using legacy activation/deactivation MAC CE to exit Survival Time state”)?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| CATT | Agree | Specifically to exit Survival Time state. |
| Nokia | Agree | The gNB should be able to change the duplication state by using the legacy MAC CE in any case. If really there is any problem, the gNB can simply refrain from sending such MAC CE by implementation. |
| Vivo | Agree | A UE in ST state needs to exit ST state after packets are delivered successfully to improve the resource efficiency. In our view, a natural way to exit ST state is that NW deactivates one or more duplication legs via legacy A/D MAC CE. Following the same light, we see no reason to prevent the NW from activating duplication via legacy A/D MAC CE. If DRB with ST support is configured with two legs, R15 A/D MAC CE should be used. If DRB with ST support is configured with more than two legs, R16 A/D MAC CE should be used. We think there is no spec impact. |
| ZTE | Agree | The legacy R16 process, e.g., network-controlled PDCP duplication is naturally supported. But in R17, it will also support that UE autonomously activates/deactivates the PDCP duplication. |
| Qualcomm | Agree | It is imperative that the network also has control and can explicitly instruct the UE to activate or deactivate duplication. |
| Ericsson | Agree |  |
| Samsung | Agree |  |
| Lenovo/MotM | Agree |  |
| Intel | Agree | There is no reason to restrict network behavior. |
| Huawei, HiSilicon | Agree |  |
| InterDigital | Agree |  |
| Futurewei | Agree |  |
| Xiaomi | Agree |  |
| OPPO | Agree |  |
| TCL | Agree, with comments | We agree to reuse legacy PDCP Duplication activation/deactivation mechanism, i.e. NW controlled the duplication state via activation/deactivation MAC CE, for the DRB configured with Survival Time support. We also agree with ZTE to not preclude UE autonomously control PDCP duplication state in the future discussions. |
| LG | Agree |  |
| Apple | Agree | The network should be able to adjust the duplication state at any time. While this option allows for flexibility it may not be the default mechanism to exit survival time. |

If the network can still control the duplication state for a DRB configured with Survival Time support, e.g. via MAC CE, besides the “Survival Time state duplication state” indication, it is possible that more legs have already been activated when the DRB enters Survival Time state. The following question is to confirm the UE behaviour, if the UE is asked to deactivate duplication of some legs after entering Survival Time state for the DRB. It is rapporteur’s understanding that it is important to avoid misunderstanding between the network and the UE on the UE behaviour after entering Survival Time state.

Q10: Following entry into the Survival Time state, if more legs have already been activated than what are indicated by the “to be adopted duplication state in Survival Time state” for the DRB, which option would your company support on the intended UE behaviour?

Option 1: Follow the network indicated duplication state which shall be adopted in Survival Time state for the DRB.

Option 2: Keep the current duplication activation state for the DRB.

Option 3: Others. Please elaborate.

| Company | Option | Comments |
| --- | --- | --- |
| CATT | None | Per our answer to Q8, we disagree that Survival Time trigger can result in less activated legs than what NW has already activated. It could be that NW has already activated, e.g. via MAC CE, duplication for all legs. In such case, triggering Survival Time has no impact on the duplication configuration of that DRB. |
| Nokia | 1 | Basically we think this is a corner case, because the gNB would simply avoid such situation by implementation. For instance, the NW could simply not to send reTX grant to trigger survival time state in such cases (if this is really a problem). |
| vivo | Option 3: this issue may not exit | This issue does not exist if all the configured legs are activated for duplication transmission when entering the ST state. Please see our answer to Q8. |
| ZTE | Option 3 | We understand that the legs “to be adopted duplication state in Survival Time state” mentioned in the question are just the pre-configured set of RLC entities. As commenting in Q8, we assume it may be the case that not all the pre-configured RLC entities would be adopted, e.g., UE can select by itself the legs (among the pre-configured ones) for PDCP duplication after entering ST state. We tend to agree it may be possible that more legs have already been activated than the ones selected by UE for PDCP duplication after entering ST state. In this case, we assume the number of activated RLC entities can be kept. But for the legs that are selected by UE and have already been activated, their resource configuration should be updated according to that in Survival time configuration. Such process may be similar, but not same as Option 2. |
| Qualcomm | 3/None | Agree with CATT and vivo. If survival state targets the maximum reliability by activating all possible RLC legs. This is the behaviour that should be activated in survival state irrespective from how many/which legs the network has already configured. We also view this case as a bit unrealistic that the network would configure a ST behaviour that does not target maximum reliability then override this outside of survival state to a higher reliability target and still enter the survival state which now targets a lower reliability target by configuration. |
| Ericsson | Option 1 | This seems like a corner case. |
| Samsung | Option 3 | This is not a likely or even realistic issue (that we would have fewer activated legs than what NW had already activated). |
| Lenovo/MotM |  | This seems like a strange configuration/set up when UE upon entering ST state uses less RLC legs than what it is currently using for PDCP duplication. As already mentioned by other companies if UE uses always all configured RLC legs during ST state, such issue is not really existing. |
| Intel | Option 3 | It is not clear to us whether the question is related to UE behavior when receiving Duplication RLC Activation/Deactivation MAC CE. Anyway, our preference is that UE just follows the latest MAC CE as well as triggering behavior. For example, if UE enters survival time state, UE just turns on PDCP duplication for associated RLC legs. If UE later on receives Duplication RLC Activation/Deactivation MAC CE, UE just follows the MAC CE. |
| Huawei, HiSilicon | Option 1 with comments | If Option 2 for Q8 is finally agreed, we think Option 1 here shall be adopted. |
| InterDigital | Option 3 | This may not be an issue depending on the outcome of Q8. |
| Futurewei | Option 1 | Also agree that this is an unlike scenario. |
| Sequans | Option 3 | Automatically handled by our proposal with opportunistic resource allocation on additional legs. |
| Xiaomi | None | This seems to be an unlikely scenario. |
| OPPO | 3/None | Agree with companies it is a strange case. |
| TCL | Option 3/1 | When enter into Survival Time state, the transmission reliability should be improved, thus the legs indicated to be activated should be more than legs have been activated. The issue mentioned in the Question is contradictory to this, it should not be exist. However, if such issue happens, we think the UE should follow the NW indication, as is mentioned in option 1. |
| LG | Option 1 | The network can handle which legs to activate. The UE needs to simply follow the network’s decision. |
| Apple | Option 1 (with comments) | How this option will play out together with the ST framework remains to be seen but we tend to have a similar understanding as Huawei. The details can be discussed during stage-3. |

During the online discussion of RAN2#115-e meeting, some companies raised the issue “packets already sent to RLC” when discussing the activation of pre-configured PDCP duplication configuration:

IV. Following entry into the Survival Time state, the pre-configured PDCP duplication configuration is activated. (16/18) - Sequans points out that there may be packets already sent to RLC and we need to deal with duplication in that case. Vivo, Apple and LG agrees.

This issue has been discussed also in [2]. When the transmission of first packet (packet A) fails, if the next packet (packet B) arrives before receiving HARQ-NACK, the UE may already process the packet B and delivered to RLC. The PDCP duplication is thus activated “to no avail”, for the transmission of packet B. If the transmission of packet B fails, it already violates Survival Time requirements. This issue is related with the timing of HARQ-NACK sent by the gNB. Some companies questioned the uncertainty on the arrival of HARQ-NACK, as it’s related to gNB scheduling, it may arrive later than the allowed range of survival time. Rapporteur understands that these timeline issues could be complicated, yet any sensible network deployment should ensure that, when serving a QoS flow with Survival Time support, the scheduling of HARQ-NACK signalling should be set to meet the stringent timing requirement. On the UE side, when it is serving the periodic traffic and also is expecting a possible HARQ-NACK, UE might need to avoid “too early” processing of packets.

Q11: Which option would your company support, regarding the abovementioned timeline issues with UE packet processing and HARQ-NACK scheduling?

Option 1: Not to specify extra behaviour, it is up to gNB/UE implementation to handle.

Option 2: Specify UE behaviour to handle the case that it receives HARQ-NACK for which the packet has already been sent to RLC. Please provide the details.

Option 3: Others. Please elaborate.

| Company | Option | Comments |
| --- | --- | --- |
| CATT | 1 | - Time to activate PDCP duplication: The timeline of the HARQ-NACK solution is illustrated below for the worst-case scenario of 500µs periodicity. Latency values are the shortest possible R16 RAN1 latencies assuming extreme configurations (PDCCH and PUSCH configured every 2 symbols, etc) and PDSCH/PUSCH processing capability #2, and for the 60kHz numerology: Comparing timelines (5) and (6), (5) is for decoding the dynamic Re-Tx grant and preparing the associated PUSCH retransmission on the dynamic grant, while (6) is for decoding the HARQ-NACK, activating the duplication and preparing the PUSCH for the next CG. And as can be observed, (6) is 1.8 times larger than (5), although the only additional processing is the duplication activation. Thus, based on these numbers, it can be assessed that the HARQ-NACK solution provides enough time to activate duplication before the next CG opportunity. - Preprocessing Preprocessing is an implementation technique allowed since R15, primarily introduced to cope with the extreme data rates of NR. It allows UE producing PDCP and RLC SDUs in advance, even before receiving an associated UL grant, thus facilitating UE implementation. However, for the URLLC traffic we agreed to focus on, each packet is quite small (≤ 50 bytes) hence is not expected to bring heavy load (e.g. ciphering) on PDCP. Moreover, if preprocessing is an attractive technique for eMBB traffic, on the contrary it is not appropriate to URLLC traffic for which the UE is expected to wait for the last minute before the transmission opportunity to cope with late data arrival, thus minimizing the e2e latency. More specifically, the TSN application is expected to deliver the packets to the UE modem at deterministic and periodic times, with the requirement that e2e latency < transfer interval, meaning that each packet must have been transmitted before the next packet arrives. Therefore, in order to secure the e2e latency, the CGOs carrying the packets should not be configured too far away from the expected reception time at the UE modem interface, meaning too early PDCP preprocessing is impractical. Furthermore, as commented by the Rapporteur, another cause resulting in the issue discussed here could be that the gNB latency in sending back the HARQ-NACK is too loose to be received before UE PDCP has processed packet # (n+1). However, this would contradict the round-trip time assumption from above figure, where, from the time the HARQ-NACK is received, the remaining time for decoding the HARQ-NACK, activating the duplication and preparing the PUSCH for the next CG is 1.8 times larger than the necessary time, according to RAN1 timelines. Therefore, this would reflect a wrong gNB implementation for this usecase. |
| Nokia | 1 | This issue is the main reason why we think reactive method can be difficult in practice, and we think proactive method is much easier because HARQ timing is not easy to track and fulfil in some cases. But since most companies believe it can work, we think there is no need to further complicate the specification. |
| vivo | Option 2 | There is always a small gap between the packet arrival time and the CG occasion, to ensure the UE has time to process the received packet for transmission. In some cases, a longer gap is needed for the UE servings several DRBs with ST requirements, to handle the case more than one packets arrived at the same time and ensure the last packet treated can be mapped on the corresponding CG. As the ST packet is quite small (≤ 50 bytes) , the processing of the packet will be very fast. Hence, even the ST packets arrivals every close before the CG, there is a risk that the ST packet is pre-processed before the retransmisison grant is received. In the current specification, PDCP always performs pre-processing at the reception of a PDCP SDU from upper layers. Thus, it is very likely the abovementioned timeline issue happens and UE fails to meet the ST requirements. We suggest RAN2 to discuss how to handle it. |
| ZTE | Option2 | The mentioned issue is that, due to scheduling of HARQ-NACK of the current packet (#n) is failed or delayed, especially when N is greater than 1, the indication to PDCP entity about reception of retransmission grant would be delayed and further the activation of PDCP duplication is delayed, even later than the next packet (#n+1). Besides, even for one packet, if the first few retransmission grant (s) are lost, it will also cause the delay of indication to PDCP entity and also the delay of PDCP duplication for the remaining retransmission of the current packet [Issue#1]. Moreover, for the latter case in [Issue#1], even the HARQ-NACK is finally received, if the AN-PDB of current packet has already been exceeded, the (delayed) PDCP duplication for the remaining retransmission of the current packet would be unnecessary. That may cause unnecessary resources waste [Issue#2]. CATT has some analysis and think the HARQ-NACK solution provides enough time to activate duplication before the next CG opportunity. We can agree that the former case in [Issue#1] would occur not much. But due to the following further reasons, we think both of the issues cannot be completely avoided: - The availability of network resources (PDCCH, PUSCH resources) is uncertain. Even the IIoT service may have higher QoS, when there are several UEs with higher QoS service, the prioritized scheduling for each UE or each services cannot be always guaranteed. - In the case of N greater than 1, when the previous one or few retransmission grant(s) are lost, the start of counting retransmission grant must be delayed. Therefore, we also cannot agree with Rapporteur that “sensible network deployment should ensure that the scheduling of HARQ-NACK signalling should be set to meet the stringent timing requirement”. We also don’t agree to re-discuss the proactive method (too much potential resource waste). We only need to figure out scheme to resolve or alleviate the remaining issues of HARQ-NACK option. The details can be found in the comments for Q17 and Q18. |
| Qualcomm | 1 | We view the PDCP activation in survival time as a straightforward procedure, whereby a HARQ-NACK for PDU #N is received by the UE, processed, and sent to PDCP in-order-to activate duplication starting PDU #N+1. We assume that both the gNB and the UE know the timelines required to perform this operation in a near-deterministic way, i.e., - The gNB knows that the HARQ-NACK needs to be sent within a certain time limit. - The UE knows that it cannot process the PDCP PDU too early for a DRB that may enter survival mode anytime. Thus, a good implementation can ensure that the gNB can send the HARQ-NACK DCI as quickly as possible and a UE should delay processing the PDCP PDU as closely as possible to the designated CG. These guidelines would not be easy to capture in the spec., since we do not specify how PDUs move between internal buffers, thus, we think they best be left for implementation. |
| Ericsson | Option 1 | UE would infer from RRC configurations that the DRB associated with this PDCP entity has survival time requirement and so a smart UE implementation (in addition to gNB scheduling/configuration) should restrain from delivering early PDCP PDUs to lower layers. |
| Samsung | Option 1 | Agree with Ericsson and CATT that pre-processing (early delivery) can be avoided. |
| Lenovo/MotM | Option 1 | We share CATT’s view that pre-processing is one implementation option which was introduced in Rel-15 in order to address the limited processing time a UE has for very high data rates. Basically, pre-processing shall ensure that UE can immediately feed PHY with e.g. (at least) part of a MAC PDU, in order to start the encoding, i.e. CB segment is encoded. Therefore, from that perspective we don’t think that UE performs pre-processing for the traffic flows we discuss here, i.e. URLLC traffic with small TB sizes. We also share the view from CATT and Nokia that gNB should provide the HARQ NACK (retransmission grant) in a timely manner such that UE has sufficient time to activate duplication. A potential option which basically avoids the HARQ timing and pre-processing issues, would be that UE always generates PDCP duplicates for all configured RLC legs even though the RLC legs are not activated. For cases when UE receives a retransmission grant (NACK) and correspondingly enters ST state UE transmits the already generated PDCP duplicates which are pending in the RLC layer for initial transmission. In case UE is not entering the ST state, i.e. MAC PDU was successfully received by the NW, UE just discards the duplicates from the RLC buffer. It should be noted though that such solution would introduce additional complexity and standardization efforts. |
| Intel | Option 1 | Our understanding is that gNB implementation should configure CG and survival time solution properly and send HARQ-NACK in suitable timing. |
| Huawei, HiSilicon | Option 1 | We think gNB/UE implementation can handle the issue. |
| InterDigital | Option 1 | This is another example showing that relying solely on HARQ-NACK ST state activation can lead to failure to activate ST state in time. gNB/UE implementation should ensure PDCP PDU processing is done as late as possible. However, a Tx-side timer can also safeguard against late arriving (or never arriving) HARQ-NACK. |
| Futurewei | Option 1 |  |
| Sequans | Option 3 | We don't agree with the rapporteur that "This issue is related with the timing of HARQ-NACK sent by the gNB.". In our view, this issue is related to RAN2 agreeing with a solution while not considering the basic requirements that needs to be fulfilled. The requirement (as we understood it) is that when entering ST state due to failure of packet N, the packet N+1 should be duplicated so that reliability is enhanced. RAN2 agreed "PDCP duplication for ST configuration is activated". And most companies want this to be similar as existing mechanism. We also do not want to introduce a new kind of PDCP duplication activation. However, upon PDCP duplication activation, only packets newly received by PDCP will be duplicated. As agreed in RAN2 NR AH#1801 and implemented in specification: - Upon packet duplication activation, only PDCP SDUs/PDUs not submitted to lower layers are duplicated. If packet N+1 was already processed at the time HARQ NACK indicating failure of packet N, it will not be duplicated. Hence, the solution does not meet the above basic requirement, which should be a (big) problem. For us "relying on gNB/UE implementation" boils down to the fact that the feature will not be specified in a way that the above requirement can be met, i.e. it will not be testable and will be basically useless. Some comments to previous answers from CATT: "Thus, based on these numbers, it can be assessed that the HARQ-NACK solution provides enough time to activate duplication before the next CG opportunity."  Maybe, but not related to the problem. Thanks for the timing diagram, as it appears clearly that most of the time new packet might arrive at PDCP before ReTx DG of previous packet. "Preprocessing is an implementation technique allowed since R15, primarily introduced to cope with the extreme data rates of NR. It allows UE producing PDCP and RLC SDUs in advance, even before receiving an associated UL grant, thus facilitating UE implementation."  This is also not relevant. PDCP PDUs are built and sent (without timing restriction) to RLC already from LTE Rel-8 specification. So, this is both not related to the pre-processing discussions from NR (which were more related to RLC and MAC) and not an implementation technique. It is just the way it is specified, for very good reasons (PDCP is supposed to handle non real-time processing and should not be in PHY control loop). " Moreover, if preprocessing is an attractive technique for eMBB traffic, on the contrary it is not appropriate to URLLC traffic for which the UE is expected to wait for the last minute before the transmission opportunity to cope with late data arrival, thus minimizing the e2e latency."  We really don't see why UE should wait for the last minute to build PDCP PDU in URLLC use case. How does it help in case of late data arrival? Maybe there is a confusion with building of MAC PDU (?). Note that even if we say a smart UE can wait last minute to build the PDCP PDU, it does not change the fact that there would not be such requirement and final UE behaviour would be completely up to UE implementation. " Furthermore, as commented by the Rapporteur, another cause resulting in the issue discussed here could be that the gNB latency in sending back the HARQ-NACK is too loose to be received before UE PDCP has processed packet # (n+1). "  The issue is not related to HARQ-NACK arriving late. The deadline of DCI arrival for that feature should be CG minus PDCCH processing/PUSCH preparation time, and should not be related at all to the time instant " UE PDCP has processed packet # (n+1). " which is not known and not specified. PDCP processing should be out of L1 control loop as it has always been, which is possible without spec change as we proposed earlier. |
| Xiaomi | Option 1 |  |
| OPPO | Option 1 |  |
| TCL | Option 1 | We can use preprocessing technique to handle the issue for now, i.e. left to implementation. |
| LG | Option 2 | The pre-processing is a general way of handling the protocol data in NR from Rel-15, for which a big protocol change comes together. By not considering the existence of UL resource in upper layer, the protocol gets simpler as well as faster. Thus, we don’t think it is only targeted to eMBB and not applicable to URLLC. In our view, the pre-processing is a kind of UE ability, which doesn’t depend on each DRB characteristics. Thus, it is strange to assume that pre-processing is currently performed some DRBs, e.g., eMBB DRB, while not being allowed for other DRBs, e.g., URLLC DRB. In addition, the retransmission grant can be received after the new PDCP data is already processed and delivered to the lower layer. One may argue that the network would configure the CG resource properly so that CG occasion would occur right after the new data arrival, hence, there is enough time to activate PDCP Duplication for the new PDCP data before transmitting it on the next CG. However, the pre-processing can be different per UE implementation and cannot be assumed the best to all UEs. It means that there could be sufficient time between the new PDCP data arrival and the CG occasion and it is possible that the retransmission grant is received after the new PDCP data arrival. Thus, we don’t think pre-processing issue can be simply ignored or left up to another UE implementation. The only way of avoid the issue of pre-processing in ST mode would be to have a restriction that pre-processing is not allowed for a DRB which is configured with ST requirement and it needs to be specified. In our view, it loses simple/fast UE processing depending on the DRBs, which also complicates the UE implementation when having both of DRBs having or not having the ST Requirement simultaneously. |
| Apple | Option 1 | This issue illustrates that PDCP duplication is not as straight-forward a solution as it may seem. Pre-processing has been part of the NR design since Rel-15 and adding restrictions to the data path would come as a burden to implement the survival time feature for the UE. To capture the constraints and conditions in the specification can lead to more complex scenarios, thus it seems best to leave this up to UE implementation. |

There are some observations/proposals [11] discussing the possible impact of handover process on the Survival Time mechanism. Rapporteur suggests that we design the Rel-17 Survival Time solution in a “minimum but working” manner and avoid involving further complexity, the following question is thus for confirmation.

Q12: Would your company agree that RAN2 not to consider the interaction between Survival Time solution and handover procedure in Rel-17?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| CATT | Agree | The service area of the usecases we agreed to focus on are 50 m x 10 m x 10 m. We don't see any need to address HO-related issues. |
| Nokia | Not sure | We think it depends on RAN3. Because in our understanding, if we want to address survival time issue with mobility considerations, then at least RAN3 needs to confirm that survival time related information can be exchanged between serving and target gNBs. |
| vivo | Agree | In RAN2#114, RAN2 has agreed to focus on the top 3 rows of Table 5.2-1 from TS 22.104 (transfer interval = survival time = 0.5/1/2ms). For these three use cases, the handover interruption time is much longer than the ST requirement. Although DAPS has been specified to provide 0 handover interruption, DAPS can not be configured with PDCP duplication, i.e. the current ST solution cannot work with DAPS. With this in mind, it is our view that there is no way to guarantee the ST requirement during handover. Hence, we also suggest not to consider the interaction between ST solution and handover procedure in Rel-17. |
| ZTE | Agree | Even we think it’s still valid to consider interaction between Survival Time solution and handover procedure at least for UE in cell edge, in order to focus on the main part of Survival Time support, we are fine to leave this issue to the future releases. |
| Qualcomm | Agree |  |
| Ericsson | Agree | The Ran2 impact is not clear |
| Samsung | Agree but… | As Nokia and Ericsson point out, the RAN2 impact is unclear and it is possible RAN3 input may be needed. |
| Lenovo/MotM | Agree |  |
| Intel | Agree |  |
| Huawei, HiSilicon | Agree |  |
| InterDigital | Agree |  |
| Futurewei | Agree |  |
| Xiaomi | Agree |  |
| OPPO | Agree |  |
| TCL | Agree, but … | Agree with Nokia, RAN2 impact is not clear. RAN3 is involved in the issue, we can send LS to RAN3 about this issue, then decide whether to consider the mobility enhancement. |
| LG | Agree |  |
| Apple | Agree |  |

## On gNB to guarantee the needed resources

It has been discussed [2] that it shall be guaranteed the radio resources are available for the legs activated, following entry into the Survival Time state, meanwhile the “pre-configured” resources shall not be wasted outside Survival Time state. Assuming the network and the UE have the common understanding that a DRB is inside or outside the Survival Time state, it could be gNB scheduling/network implementation to pre-configure the needed resources for the to-be-activated legs. If the resources can be used only after the configured legs are activated, the resources can be used elsewhere (e.g. by other UEs) when the configured legs are not activated.  When the configured legs are activated, the network should deactivate the usage of these resources for other UEs. The relevant solutions are proposed also in [5][21], the difference between solutions could be on how to (or whether) specify certain details.

Q13: Which option would your company support, to provide radio resources for the activated legs following entry into the Survival Time state?

Option 1: The resource pre-allocation and (de)activation are up to network implementation, including enabling the resource usage for UE entering Survival Time state, and disabling the resource usage for other UEs.

Option 2: To specify that CG resources are pre-configured and their activation are conditional on entering Survival Time state.

Option 3: Other solutions. Please provide the details.

| Company | Option | Comments |
| --- | --- | --- |
| CATT | 2 | Dedicated CG resources can be configured to ST-UE for the duplication paths and - Specification restricts the ST-UE from using the CG out of Survival Time, or - The initial state of CG type-1 is set to “deactivated” and UE activates it autonomously when activating duplication for the associated RLC entity. We would prefer the latter option as it is cleaner and could also be generalized to any duplication activation scenario (not specific to a Survival Time DRB), thus “grouping” the Duplication activation MAC CE and the CG type-2 activation into one single command. This would also allow implicitly activating multiple CG configurations associated (via LCP restriction e.g. associated LCH or carrier) with the RLC entity that has been duplication-activated. |
| Nokia | 2 (and possibly Option 3) | We think Option 2 makes some sense, but we should also consider the behaviour where the gNB subsequently deactivates the corresponding duplication leg using Rel-16 MAC CE – in this case the associated CG should be deactivated upon reception of this MAC CE, in order to avoid resource wastage when the DRB exits the survival time state. On the other hand, we are also concerned about the situation where the message cannot be completely transmitted within survival time interval as short as 0.5ms due to MAC CE allocation in the same PUSCH during LCP. We think RAN2 may further discuss how to avoid this issue (e.g. MAC CE on the resource for survival time support should be limited to make sure the message can be completely transmitted). |
| vivo | Option 1 | We share the same understanding with the rapporteur that the network and the UE have a common understanding of whether a DRB is in the Survival Time state or not. With this assumption, we think the activation/deactivation of pre-configured CG resources can be left to network implementation. For activated pre-configured CG resources configured to UE A for duplication, if the corresponding DRB of UE A is not in ST state, the network can reallocate the radio resource of the CG to other UEs since no data from UE A is expected to send on the CG resources(assuming no other LCHs are mapped to the pre-configured CG resources). On the other hand, when the corresponding DRB of UE A enters ST state, the network will stop the radio resource reallocation and UE A will start to use the CG resources for duplication. The above operation on activation/deactivation of pre-configured CG resources requires no enhancement of specification. |
| ZTE | Option 2 | RAN2 has agreed that gNB implementation solutions on their own are not sufficient. That’s also why RAN2 work/study UE-based reactive solutions to address survival time. So we don’t support Option 1. We also prefer Option2. We have similar understanding that, in Option 2, initial state of CG type-1 is set to “deactivated” and UE activates it autonomously when activating duplication for the associated RLC entity. But we think this is restricted to DRB with Survival Time support. Generalizing this to other duplication activation scenario is out of scope. |
| Qualcomm | 3/1 | First, we think that this “it shall be guaranteed the radio resources are available for the legs activated, following entry into the Survival Time state, meanwhile the “pre-configured” resources shall not be wasted outside Survival Time state” is not a hard rule that needs to be followed. It may be thought of as a guideline/good practice, but we think if the network wants to pre-configure resources and effectively waste them or overbook them for the same UE and let internal prioritization take care of this efficient resource use, there shouldn’t be a rule against that. That being said, if we want to apply this efficient radio-resource usage, the gNB can configure a type 1 CG and add LCH restrictions to guarantee it is not used outside of survival time. Since the gNB effectively triggers survival state using HARQ-NACK, it can simply reallocate the CG to other traffic/UEs outside of survival time knowing that the ST DRB will not transmit anything. In case of type 2 CG, we think option 1 would be sufficient to activate and deactivate CG instance in and out of survival state, respectively. While the idea of option 2 is appealing, we think standardizing would have significant RAN2/RAN1 impact, thus we prefer working within the existing framework of type 1/type 2 CGs. |
| Ericsson | Option 1 but | We have a clarification that it does not necessarily need to be pre-configured. Our view is that The resource allocation and (de)activation are up to network implementation Option 2 does not work. If the resources for PDCP duplication are pre-configured, then these resources are not adaptive to the channel conditions and, thus, may fail to meet the survival time requirement. |
| Samsung | Option 2 | This has already been agreed (in a sense), as pointed out by ZTE. Not sure about Ericsson’s concerns about Option 2 not being adaptive to the channel conditions. |
| Lenovo/MotM | Option 2 |  |
| Intel | Option 2 |  |
| Huawei, HiSilicon | Option 1 | We believe leaving this to network implementation is sufficient. |
| InterDigital | Option 2 | We agree with ZTE. As far as how the network uses unused pre-configured ST resources outside of ST state, that is up to network implementation. |
| Futurewei | Option 1 |  |
| Sequans | Option 1 but | In our view PDCP duplication should have been preactivated, and it is the opportunistic resource allocation on additional legs during "ST state" that provide the required reliability boost. |
| Xiaomi | Option 1 |  |
| OPPO | Option 1 | We share a similar understanding that the network and the UE can have the common understanding that a DRB is inside or outside the Survival Time state. With this being in mind, the current LCH-to-CG mapping restriction can be used to ensure the pre-allocated resource is used properly. This operation can be performed by gNB implementation. |
| TCL | Option 2 with comments | Pre-configured CGs and conditional activation on entering Survival Time state can resolve most common cases. But, with CG transmission failure(s) and retransmission grant considered, if the pre-configured CGs would cover such cases, resource waste would be a concern for the CG transmission failures rarely occurs. Therefore, DGs should be considered as supplementary to pre-configured CGs. |
| LG | Option 2 | Both option 1 and option 2 would work. However, option 1 requires more signalling overhead to activate all relevant CGs and CG Confirmation MAC CE. Thus, we prefer option 2. |
| Apple | Option 2 |  |

## On alternative Survival Time state trigger

Besides retransmission grant, CG activation command as Survival Time state triggering is proposed as alternative Survival Time state trigger. When a CG configuration being mapped to a logical channel belonging to a DRB with Survival Time support is activated via DCI, the DRB shall enter Survival Time state and activate backup legs for duplication transmission. For the past several meetings, pros and cons on this trigger are thoroughly discussed and compared against the retransmission grant.

Q14: Which option would your company agree to, on using CG activation command as a trigger for entering Survival Time state?

Option 1: Using CG activation command as Survival Time state trigger, instead of retransmission grant.

Option 2: Using CG activation command as Survival Time state trigger, together with retransmission grant.

Option 3: Not to use CG activation command as Survival Time state trigger.

| Company | Option | Comments |
| --- | --- | --- |
| CATT | 3 | The CG activation grant method requires sending such activation grant for each CG configured to serve the RLC entity, while the HARQ-NACK solution requires only one HARQ-NACK signal per RLC entity. Moreover, each such CG type 2 activation cmd will trigger a configured uplink grant confirmation, including a Multiple Entry Configured Grant Confirmation MAC CE. Such MAC CE will be multiplexed with the duplicated data in the next CG transmission, thus uselessly enlarging the payload. It is answered in [16] that the CG type 2 activation cmd would be modified in the specifications to not trigger a configured grant confirmation when used in this usecase. We actually prefer the similar implicit activation mechanism we describe in Q13 because we think it is also more logical that duplication activation triggers associated CG configuration(s) activation rather than the other way around, for example, a CC can be configured to be served by two CG configurations, and activating the duplication in that CC can implicitly activate both CGs. In [16] it is questioned why the network would configure one CC to be served by two configured grants. We believe we should not restrict implementations and such multi-CG allocation can be e.g. staggered CG configurations to optimize the latency wrt packet arrival jitter or periodicities not exactly addressed by current spec. It also makes more sense, time-line-wise, that Duplication activation triggers CG activation rather than the other way around because duplication activation at PDCP is more urgent (occurs first in the processing sequence) than the CG activation. |
| Nokia | 1 or 3 (see comments) | We have some sympathy to Option 1, but considering the time availability in Rel-17, we think Option 3 may be a more suitable way forward to make sure this WI can complete on time. |
| Vivo | Option 3 | Option1 brings some restrictions, e.g. cannot be applied to Type 1 CG. Option2 needs to specify two solutions for Survival Time state trigger, we see no big benefit to justify the redundancy functions. We prefer a more flexible solution, i.e. only using retransmission grant as Survival Time state trigger. |
| ZTE | Option 3 | For relying on “CG activation grant”, it has similar “not quickly enough” issue and “lost” risk in bad radio condition as those gNB-based options. Therefore, it may cause that PDCP duplication cannot be activated in time as expected and not preferred. |
| Qualcomm | 3 | We do not think that CG activation grant on CC2 should play a role in PDCP activation: - Our biggest issue is that this solution mandates that CC2 only activates duplication on a type 2 CG. We are not convinced that there is a good reason to make that unnecessary binding between PDCP duplication and CC2 CG type 2 grant activation. As we mentioned in Q13, there is a way to activate duplication while using CG type 1 on CC2. The network may want to schedule dynamic grants, pre-allocate resources, pre-emptively activate grants, or any other configuration that does not rely on CG type 2. - L1 signalling between secondary CC and duplication from primary CC is complex and not preferable. - UE may not be monitoring this CC2 due to dormancy issue HARQ-NACK mechanism that is well discussed between companies is a more straightforward cleaner solution given the time frame. We do not see a good incentive to adopt this alternate proposal. |
| Ericsson | Option 1 | The main benefit of option 1 is that resources for PDCP duplication in the survival time are, at the same time, allocated by CG type-2 activation command, and thus adaptive to the channel conditions. CG type-1 is not suitable for this survival time operation, as the resources are pre-configured. To answer CATT’s comments above - We don’t agree with CATT that there is a need to have staggered CG configurations (which is used for unknown arrival time). In the context of survival time, UE traffic is periodic and gNB has TSC-AI that contains packet arrival time. Additionally, survival time requirement roots in factory automation in which the traffic periodicity can be tuned to match the typical values supported by NR. Also, RAN2 agrees to focus on transfer interval = survival time = 0.5/1/2ms which is supported by NR. - The issue with “duplication activation triggers associated CG configuration(s)” is that the resources (e.g., MCS, time/frequency domain resource allocation) to transmit the transport block have to be pre-configured and not adaptive to the channel conditions. We have addressed QC’s comment in our previous paper R2-2108099 [16], section 5, copied below. Both reTx grant-based triggering and CG activation-based triggering require UE to be ready to use PDCP duplication in the secondary CC. If the UE is in dormancy in the secondary CC, then it is not possible either to activate PDCP duplication with reTx grant-based triggering since network needs to activate that CC which introduces delay. For CG activation-based triggering, the L1 signal on the secondary CC is to activate the RLC entity for PDCP duplication in the secondary CC. It is indeed the case for reTx grant-based triggering approach, since it uses the DCI command (retx-grant) in one CC to activate PDCP duplication in another CC.. |
| Samsung | Option 3 |  |
| Lenovo/MotM | Option 3 |  |
| Intel | Option 3 | We think using retransmission grant is sufficient and can handle both type 1 and type 2 CG. Using CG activation command can only work for type 2 CG. |
| Huawei, HiSilicon | Option 3 | Same view as CATT |
| InterDigital | Option 3 | Adding a new trigger based on the reception of a gNB transmission does not address the remaining issue of misdetection of a gNB transmission triggering ST state. |
| Futurewei | Option 3 |  |
| Sequans | If such trigger is needed, Option 1 | As indicated earlier, we think PDCP duplication should have been preactivated, and it is the opportunistic resource allocation on additional legs during "ST state" that provide the required reliability boost. So no need for a trigger to enter "ST state". Still if we have to choose one, Option 1 is better as it uses a different carrier while retransmission DCI uses the carrier that just failed (even if DCI is more protected, we assume there would be some higher risk of missing it in TDD). |
| Xiaomi | Option 3 |  |
| OPPO | Option 3 |  |
| TCL | Option 3 | We don’t see much benefit of using CG activation command as Survival Timer state trigger. Retransmission grant is flexible and suitable. |
| LG | Option 3 |  |
| Apple | See comment | Using a CG activation command allows for more adaptivity in an additional option that does not necessarily require PDCP duplication. |

Besides, during the email discussion [2], some companies also support autonomous activation of PDCP duplication on the part of UE. However it is not clear how UE can autonomously activate PDCP duplication based on inputs other than retransmission grant or CG activation grant, i.e., it is not clear how “UE autonomous activation of PDCP duplication” can be a standalone option without any element of retransmission grant option and CG activation grant option.  Further it is not clear how the network can be aware of Survival Time state triggering for a DRB and enable the resource usage for activated legs meanwhile disable the resource usage by other UEs timely. Based on recent RAN2/RAN discussion, rapporteur considers that it might be good to deprioritize this option.

Q15: Would your company agree to deprioritize autonomous activation of PDCP duplication on the part of UE in Rel-17?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| CATT | Agree the intention but… | We think the HARQ-NACK solution discussed above could be viewed as an autonomous activation since UE does not receive an explicit activation command. Similar, for example, to UE autonomous transmission of a deprioritized CG in another CG. So we agree the question re-formulated as in above Rapporteur’s text: deprioritize autonomous activation of PDCP duplication based on inputs other than retransmission grant or CG activation grant. |
| Nokia | Agree | We think much more discussion is needed to figure out how it could work. Considering that we are approaching the end of Rel-17, we agree with the rapporteur that this should be de-prioritized. |
| Vivo | See comments | It is not clear to us what is the exact meaning of the autonomous activation of PDCP duplication. Does UE need any information from NW to decide when to activate PDCP duplication? |
| ZTE | Disagree | We cannot clearly understand the question. Per our understanding, reception of one or several retransmission grant (s) is just the trigger for entering the Survival Time State. After entering the Survival Time state, UE should autonomously activate the PDCP duplication. There is no saying that autonomous activation exists independently. We also generally agree with CATT that the HARQ-NACK solution discussed above could be viewed as an autonomous activation since UE does not receive an explicit activation command. For the question how the network can be aware of Survival Time state triggering, we think after the network fails to decode the transmission on the CG occasion, the network can know it’s time to enable the resource usage for activated legs meanwhile disable the resource usage by other Ues. |
| Qualcomm | Agree with CATT | Autonomous PDCP duplication is a general term so far that can encompass the solution(s) being discussed. Thus, we can reword the proposal to focus more on HARQ-NACK triggered PDCP duplication. |
| Ericsson | Agree | Agree with Rapporteur and we understand “autonomous activation of PDCP duplication” as UE activates PDCP duplication without any indication from gNB. |
| Samsung | Disagree | Same concerns as ZTE about the phrasing of the question. |
| Lenovo/MotM |  | Not sure that we understand the question correctly. In our understanding UE activating PDCP duplication upon reception of a HARQ NACK is some UE autonomous mechanism. |
| Intel | Agree | Same understanding as Ericsson. |
| Huawei, HiSilicon | Agree | We agree with CATT’s re-formulation of the question. |
| InterDigital | Disagree | Similar concerns as ZTE. Furthermore, UE should be able to autonomously activate PDCP duplication in the event it has not received any HARQ feedback (e.g. possibly due to misdetection). In such a case, the following may occur: - HARQ-NACK was transmitted. The UE doesn’t receive it, and it behaves as though it received HARQ-NACK. No misunderstanding between UE and gNB. - HARQ-ACK was transmitted. The UE doesn’t receive it and it behaves as though it received HARQ-NACK. Some misunderstanding between UE and gNB. The UE unnecessarily enters ST state. However, if the other resources were not re-scheduled to other UEs, there is no problem. If the resources were re-scheduled to other UEs, then interference may be an issue, but it shouldn’t be a big problem for the gNB to handle it. - HARQ-NACK/ACK were transmitted and received by the UE. No misunderstanding. |
| Futurewei | Agree |  |
| Sequans | Agree |  |
| Xiaomi | Agree |  |
| OPPO |  | Not sure we understand this question correctly. Similar view as companies, HARQ-NACK solution we discussed can also be classified as an autonomous activation since UE does not directly receive a PDCP activation command. |
| TCL | Disagree | Have same concern with CATT and ZTE. In our understanding, UE enters Survival Time state upon receiving HARK-NACK could be viewed as an autonomous activation. Furthermore, the solution UE should be able to autonomously activate PDCP duplication after entering Survival Time state is not precluded. For the question that how the NW can aware of Survival Time state triggering, we share the same view with ZTE, it is not a big concern. For example, when the NW determine to send HARQ-NACK(s) to UE indicate the UE to enter Survival Time state, the NW knows the UE would enter Survival Time state too, i.e. the NW have the common awareness of the UE entering Survival Time state. |
| LG | Agree | We see some benefit of UE autonomous PDCP Duplication. However, it can be considered in later release due to lack of time in Rel-17. |
| Apple | Disagree | Similar view as vivo and ZTE. |

## On exiting Survival Time state

The issue on “how to exit the Survival Time state” has been discussed in [2]. There is a majority of support to leave the exiting behaviour to network control, yet there are also some support to define UE autonomous exiting method based on e.g. timer or counter. Considering Survival Time state is low probability event, how to trigger the exiting wouldn’t become a time critical or resource critical issue. The network can control the exiting by deactivate PDCP duplication or reduce the number of activated legs through legacy MAC CE.

Q16: Which option would your company support on exiting the Survival Time state?

Option 1: It is up to network control and not to consider further optimization. (Please note this option is relevant to the discussion for Q9.)

Option 2: To specify UE’s behaviour for autonomous exiting, e.g. via a timer-based method or a counter-based method. Please provide the details.

Option 3: Other solutions. Please provide the details.

| Company | Option | Comments |
| --- | --- | --- |
| CATT | 1 | Unlike for triggering Survival Time, there is no hard latency requirement for exiting it. Therefore, we think that Option 1, which is simple and direct, is sufficient. |
| Nokia | 1 | Exiting survival time state is not so urgent, and we can simply rely on the legacy MAC CE from the gNB. We are open to discuss Option 2 if any clear benefit is shown. |
| vivo | Option 1 | Different from entering the ST state, it is not urgent for UE to exit the ST state. We think it can be left to network control, i.e. network can deactivate one or more legs for duplication to indicates UE to exit the ST state. No further optimization is needed. |
| ZTE | Option 2 | With Option 2, UE can exit ST state faster and also the additional signalling exchange can be avoided. Therefore, even we think Option 1 is feasible, we still prefer Option 2. |
| Qualcomm | No strong opinion | Option 1 will always exist as a baseline. Option 2 can also be adopted if a strong need is shown by companies. |
| Ericsson | Option 1 | In this context, de-activating PDCP duplication is not time-critical. |
| Samsung | Option 2 | Option 2 is needed for additional flexibility and increased responsiveness. |
| Lenovo/MotM | Option 1 | We think that Option 1 is anyway supported by the current specification, i.e. MAC CE. Even though we have some sympathy for option 2, we think that Option 1 is sufficient. |
| Intel | Option 2 | Our preference is that UE only uses survival time state (tuning on PDCP duplication) for one burst and then exits survival time state. That is, if burst n fails, UE enters survival time state to turn on PDCP duplication for improved reliability. After transmitting burst n+1, UE exists survival time state and returns to normal configuration. |
| Huawei, HiSilicon | Option 1 or Option 2 | Same view as Qualcomm |
| InterDigital | Option 1 |  |
| Futurewei | Option 1 |  |
| Sequans | Option 3 | As indicated earlier, we think PDCP duplication should have been preactivated, and it is the opportunistic resource allocation on additional legs during "ST state" that provide the required reliability boost. So no need for a trigger to exit "ST state". |
| Xiaomi | Option 1 |  |
| OPPO | No strong opinion | Option 1 can be a baseline. Option 2 can provide more flexibility. |
| TCL | Option 1 or option 2 | Agree with other companies that exiting Survival Time state is not time sensitive operation, we can rely on NW control by reusing legacy MAC CE. Thus we agree with option 1, it’s simpler. For option 2, it shall avoid additional signalling exchange. In another aspect, besides PDCP duplication, other reliability boosting solutions are not precluded, e.g. L1/L2 adaptations. With UE autonomous solutions on exiting the Survival Time state, the UE is able to adjust the L1/L2 configurations quickly based on the radio environments change, which shall avoid entering and exiting Survival Time state frequently. |
| LG | Option 1 |  |
| Apple | Option 2 | Our view is that the UE uses PDCP duplication for a short time only, and option 2 allows for a faster return to normal operation. From a resource efficiency point of view RAN2 should avoid mandating a separate signalling every time the UE exists survival time. |

## 3.6 On combining HARQ-NACK option and Tx-side timer option

During the online discussion of last RAN2 meeting, there are some companies propose to continue consider Tx-side timer based option.

VII. UE-based reactive solution based on Tx-side timer are deprioritized in R17. (12/18) - ZTE doesn’t think it is an issue of prioritization or de-prioritization. HARQ NACK option is not always reliable. ZTE would like to consider combing the two options, use the timer and HARQ NACK? InterDigital agrees with ZE, Vivo, and oppo agrees. Intel prefers a solution that looks at both. - Ericsson points out that there is a technical issue with tx-side timer that it requires feedback for each message and thus resource inefficient. - LG thinks that tx-side timer is not clear yet and different companies.

Based on the above RAN2 online discussion and also RAN#93-e discussion around other ST solutions, it is rapporteur’s understanding that Tx-side timer option is to be discussed as in combination with HARQ-NACK option. The following questions are used to understand how Tx-side timer option can help to solve any existing issues for HARQ-NACK option, preferably taking into account all the design in above sections. It would be helpful also to clarify the issues identified previously on Tx-side timer option can be avoided/solved in this combined solution.

Q17: Has your company identified any residual issues of HARQ-NACK option that can be solved by the combination of HARQ-NACK option and Tx-side timer option?

| Company | Issue x (x=1,2,…) still exists for HARQ-NACK option | Motivation on why combining HARQ-NACK option and Tx-side timer option would solve issue x |
| --- | --- | --- |
| vivo | Issue1: the duration of HARQ-NACK counting for a PDCP PDU need to be define | For each PDCP PDU from a DRB configured with ST requirements , a duration(i.e. Tx-sider timer) needs to be defined for UE to perform HARQ-NACK counting. The Tx-sider timer is started when a PDCP PDU is delivered to lower layer. While the Tx_sider timer is running: if N retransmisson grants are received, UE should enter ST state; if a new transmission grant is received, the Tx_sider timer should be stop. When the Tx-sider timer expires, the PDCP PDU is considered as transmitted successfully, and the counter should be reset. The duration of the Tx-sider timer should be smaller than the transfer interval, otherwise if the Nth HARQ-NACK is received very close before the Tx-sider timer expiring, the UE may fail to activate the PDCP duplication before the PDCP delivers the next PDU to low layer. In addition, to overcome the possible missing of retransmisson grant, some enhancements in gNB maybe be needed, e.g. N+1 retransmisson grants are sent to UE before Tx_sider timer expires, which can be left to gNB implementation. Currently, there is no existing timer can be used as Tx-sider timer. Hence, a new Tx-side timer should to be introduced. |
| ZTE | Issue1: we think this issue is similar as the one mentioned by vivo. Due to that scheduling of HARQ-NACK of the current packet (#n) is failed or delayed, especially when N is greater than 1, the indication to PDCP entity would be delayed and further the activation of PDCP duplication is delayed, even later than the next packet (#n+1). Besides, even for one packet, if the first few retransmission grant(s) are lost, it will also cause the delay of indication to PDCP entity and also the delay of PDCP duplication for the remaining retransmission of the current packet. Issue2: For the latter case in [Issue#1], even the HARQ-NACK is finally received, if the AN-PDB of current packet has already been exceeded, the (delayed) PDCP duplication for the remaining retransmission of the current packet would be unnecessary and waste. | Below we give the combined HARQ-NACK option and Tx-side timer solutions to alleviate the Issue1 and resolve the Issue2. For resolving the Issue1, previously we suggest in the case that all the retransmission grants are lost, UE can directly enter ST state and activate PDCP duplication when the configured Tx-side timer expires. But considering that no introduction of explicit HARQ-ACK and in the case without implicit HARQ-ACK (without initial transmission grant), UE cannot distinguish this abnormal case from normal case of successful transmission and may trigger (much) unnecessary activation of PDCP duplication, we no longer pursue such process. Instead, we propose Solution1 to alleviate the Issue1 in the more occurred case that one or a few retransmission grant(s) are lost but finally at least one retransmission grant can be received. As we assume the case that all the retransmission grants are lost would be very rare, we think this Solution1 would be acceptable. Solution1: The main intention of Solution1 is to use minimum N when a configured Tx-side timer expires. In detail, the UE is configured with a timer with length equal or less than AN PDB. The time length can correspond to the value of N. The timer can be maintained for HARQ process in MAC layer. When counting of N reaches the threshold, if the timer doesn’t expire, MAC entity can send indication about reception of retransmission grant to PDCP entity. On the other hand, if N doesn’t reaches the threshold (or even no any retransmission grant is received) but the timer expires, when MAC entity receives a retransmission grant later, MAC entity would send indication to PDCP entity even though the N may still not reach the threshold (e.g., the indication would be sent on reception of the first retransmission grant after expiration of timer). Similar as vivo’s comment, if initial transmission grant is received as implicit HARQ-ACK, the timer can be stopped and no indication is sent. Solution2: Solution2 is on top of Solution1. The main intention of Solution2 is to avoid unnecessary PDCP duplication for the remaining retransmission of the current packet and to trigger PDCP duplication from the next packet. In detail, different indications can be sent to PDCP entity in different subcases: - When counting of N reaches the threshold, if the timer doesn’t expire, MAC entity send indication1 about reception of retransmission grant and PDCP duplication from current packet to PDCP entity. - If N doesn’t reaches the threshold but the timer expires, when MAC entity receives a retransmission grant later, MAC entity send indication2 about reception of retransmission grant and PDCP duplication from next packet to PDCP entity. |
| Intel | Issue 3: missed HARQ NACK The issue is similar to what is mentioned by vivo and ZTE to some extent. | In N HARQ NACK solution, a counter mechanism is required to keep track of N-1 received NACKs and survival time state is then triggered upon receiving the Nth NACK. For such solution, it may be possible to miss or lose intermediate NACKs from the gNB, which could result in survival time state not being triggered when required. In addition, in case of TDD, DL and UL are in the same frequency and could have similar channel condition, therefore NACK might be lost if the decoding for UL transmission fails. When combining HARQ-NACK and Tx-side timer operation, the UE can enter survival time state only upon receiving the Nth NACK which coincides with time duration where there isn’t time to complete HARQ retransmission within duration x. This Tx-side timer solution thus has equivalent functionality as N-NACK solution, but at the same time it will be more robust towards loss of any of the N-1 NACKs. It is only critical for the UE to receive the last NACK (where duration x has passed) from the gNB which shall trigger entering the survival time state. |
| InterDigital | Issue 3: missed HARQ NACK | When the ReTx grant is misdetected, the UE may assume that the gNB properly decoded the transmission. The UE then neglects to enter ST state in a timely manner. Using a Tx-side timer in addition to the reTx grant based solution can safeguard against this issue. In this case, the UE can be triggered to enter ST state upon reception of reTx grant or upon a timer expiring. The timer can be restarted whenever the UE determines HARQ-ACK. |
| TCL | Issue 1: the loss of HARQ-NACK transmitted by NW to UE | Agree with the issue 1 mentioned by ZTE. If the UE maintains TX-timer, the HARQ-NACK delay or failed from the NW can be resolved that the UE enters Survival Time state when there is no HARQ-NACK received before the Tx-Timer expiration. In this way, the tx-timer can be deemed as supplementary to the HARQ-NACK solution. |

Q18-1: Would your company agree with the provided motivation of combining two options, for issue 1 in the table of Q17?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| vivo | Proponent |  |
| ZTE | Agree | Re-summary: Solution1: The main intention of Solution1 is to use minimum N when a configured Tx-side timer expires. If N doesn’t reaches the threshold (or even no any retransmission grant is received) but the timer expires, when MAC entity receives a retransmission grant later, MAC entity would send indication to PDCP entity even though the N may still not reach the threshold (e.g., the indication would be sent on reception of the first retransmission grant after expiration of timer). |
| Samsung | Agree | Agree to look into this after the key issues in previous sections have been agreed/resolved. |
| Intel | Agree | In general, issues raised by vivo, ZTE and Intel are related to the missing / delayed HARQ NACK. As the motivation for survival time discussion is to support high reliability, we think the missing HARQ NACK issue should be considered. |
| OPPO | Agree | It is more suitable to the case of the counting number N >1. |
| TCL | Agree | Adopt Tx-timer as supplementary to HARQ-NACK solution. |
| Apple | Agree | Similar view as Samsung, Intel, and Interdigital. |

Q18-2: Would your company agree with the provided motivation of combining two options, for issue 2 in the table of Q17?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| ZTE | Agree | Re-summary: Solution2: Solution2 is on top of Solution1. The main intention of Solution2 is to avoid unnecessary PDCP duplication for the remaining retransmission of the current packet and to trigger PDCP duplication from the next packet. In detail, different indications (PDCP duplication from current packet or from next packet) can be sent to PDCP entity in different subcases. |

Q18-3: Would your company agree with the provided motivation of combining two options, for issue 3 in the table of Q17?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Apple | Agree | RAN2 should investigate this after key issues in previous sections have been agreed/resolved. |

Q18-x: Would your company agree with the provided motivation of combining two options, for issue x in the table of Q17?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Q19: Overall, would your company agree to adopt Tx-side timer option, to be combined with HARQ-NACK option?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| vivo | Agree | In addition, Tx-side timer can work alone. The case is very useful when the UE has both URLLC and eMBB traffic simultaneously. When a URLLC TB transmitted via CG is successful, the corresponding HARQ is scheduled via DG for new transmission(assuming there are a lot of eMBB data to transmit). The new transmission grant is used as trigger to stop the running Tx-side timer. The solution avoid the reliability issue of HARQ-NACK option. |
| ZTE | Agree |  |
| Qualcomm | Disagree | We have not identified any issues or reasoning that would justify the deployment of a Tx-side timer, furthermore, it still seems from previous discussions that the proponents are not unified on what the timer is doing (start, stop, reset conditions) and what issues it’s solving. The solutions proposed so far still seem to assume that PDCP duplication is triggered after receiving 1 or N HARQ retransmission grants. Unclear what the attached timer is doing here except acting as a somewhat redundant implicit ACK, similar to CG timer. We do not agree that the timer solves the issue of a lost retransmission grant, since a HARQ-ACK is not available, a lost retransmission grant is indistinguishable from a successful transmission, there would not be a UE side solution for that. |
| Intel | Agree |  |
| InterDigital | Agree |  |
| OPPO | Agree |  |
| TCL | Agree | Agree with vivo. |
| Apple | Agree | The Tx-side timer could be configurable by the network if a scenario requires it. |

## 3.7 Other issues

Q20: Are there any other issues your company thinks as necessary to discuss, in order to complete the design of Survival Time solution?

| Company | Issue y (y=1, 2…) | Motivation on why issue y shall be solved, in order to complete the design of Survival Time solution. |
| --- | --- | --- |
| Nokia | 1 | Behaviour clarification for DC-based duplication: When duplication is configured in Dual-Connectivity, and is already activated before the DRB enters the survival time state (i.e. at least 2 RLC entities across the two MAC entities are already activated), does the DRB enter the survival time state based on HARQ-NACK indication from one of the MAC entity or both MAC entities ? We think this should be a trivial issue but the behaviour anyway should be clarified in the specification. |
| Nokia | 2 | Providing status update to RAN3: According to the WI objective: - RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3] Since RAN3 is also involved in this objective, we think RAN2 should provide an LS to RAN3 once some mature progress has been made for the solution, so RAN3 is able to assess if there is any need of specification change from their point of view. |
| vivo | Issue 1: Distinguish PDCP data PDU and PDCP control PDU | According to the TS 22.104, the ST requirement is only applied to application messages, i.e. PDCP data PDU. Thus, the transmission failure of PDCP control PDU（e.g. EHC feedback） should not trigger the corresponding DRB to enter ST state. |
| vivo | Issue 2: Counter reset | RAN2 should consider when and how to perform counter reset. |
| Sequans | 5 | As explained earlier, we should step back and confirm whether the following "upon failure of packet N transmission, ensure that packet N+1 is transmitted (on additional carrier(s) ) with PDCP duplication." is what we want to achieve with this feature, and whether it is ok to just leave it to UE implementation (as it seems the majority view). |
| Xiaomi | 6 | The survival time requirement from SA2 is to count the number of consecutive packet loss of the application layer packet. We are not sure how the current solution is used to count the number of consecutive packet loss of the application layer packet. |
| Apple | Issue 7: Reliability add-on | Some cases exist where additional protection may be required. For example, consider the cases below: a) the UE is not capable to support PDCP duplication with more than two legs (while more legs would be needed to achieve the desired level of protection); or b) PDCP duplication is already activated with the maximum number of RLC entities. To target these use cases, RAN2 should discuss whether additional methods are needed to protect the transmission of packets in survival time. |
| Apple | Issue 8: Zero survival time | For services where the survival time is zero, loss of a single message causes the IIoT application to enter a down state. In such cases every application layer message must be delivered. The current mechanism only targets PDCP duplication of the next PDCP PDU, which cannot mitigate this case. Obviously one option is to apply the PDCP duplication for such DRBs all the time. However, this also means a waste of resources, plus, packets might still encounter retransmissions. To enhance the protection of the retransmission one possible approach is to apply a different level of reliability to the initial transmission and the retransmission. |

Q21-1: Would your company agree with the proponent of issue 1?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Q21-2: Would your company agree with the proponent of issue 2?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Q21-y: Would your company agree with the proponent of issue y?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

# Phase 2 review on agreeable CR

# Reference

- R2-2108834, Report for Rel-17 Small data and URLLC/IIoT, Session Chair (InterDigital).

- R2-2107173, Report from email discussion [Post114-e][511][URLLC/IIoT] QoS Solutions (Samsung), Samsung.

- R2-2107154, Discussion on two-level PERs for survival time handling,	Huawei, HiSilicon.

- R2-2107174, Entering, operating in, and exiting the Survival Time state,	Samsung Electronics GmbH.

- R2-2107203, UE-based reactive solution for survival time,	CATT.

- R2-2107558, RAN Enhancement to support Survival Time,	QUALCOMM Europe Inc. – Italy.

- R2-2107611, Reliability enhancements for CG/SPS,	Apple.

- R2-2107612, Further considerations on survival time for new QoS,	Apple.

- R2-2107658, L1/L2 configuration adaptation,	Fujitsu.

- R2-2107738, Consideration on RAN enhancement based on new QoS,	OPPO	.

- R2-2107742, Remaining issues on enhanced QoS,	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd.

- R2-2107802, Consideration on reactive solution for survival time, 	vivo.

- R2-2107806, Further discussions on RAN enhancements based on Survival Time,	III.

- R2-2107895, Discuss on the mechanism to guarantee the survival time,	Lenovo, Motorola Mobility.

- R2-2108023, Analysis of Potential RAN Enhancements for Survival Time,	Nokia, Nokia Shanghai Bell.

- R2-2108099, RAN enhancements based on new QoS related parameters,	Ericsson.

- R2-2108169, Discussion on RAN enhancement to support new QoS, 	China Telecommunications.

- R2-2108435, UE-based Survival time handling,	Intel Corporation.

- R2-2108457, ST handling with alternating CC allocations,	Sequans Communications.

- R2-2108459, Discussion on avoiding prematurely entering Survival Time state,	Futurewei Technologies.

- R2-2108516, Discussion on the RAN support for new QoS parameters,	CMCC.

- R2-2108666, Enhancements based on new QoS requirements,	InterDigital.

- R2-2108786, Progress of QoS,	LG Electronics UK.

- R2-2108795, Clarification on the survival time requirement	, Xiaomi Communications.