- 3GPP TSG RAN WG2 Meeting #116-e      	                          R2-210xxxx
E-Conference, 1st – 12th Nov. 2021

Agenda item:	8.7.3.1

Source:	Qualcomm Incorporated

Title:	Summary of email discussion#611 - Discovery shared/dedicated pool issue (Qualcomm)

WID/SID:	FS_NR_SL_relay – Release 17

Document for:	Discussion and Decision

# Introduction

This is for below email discussion:

- [Post115-e][611][Relay] Discovery shared/dedicated pool issue (Qualcomm)

Scope: Clarify from the UE perspective the terminology on the network configuration of dedicated discovery pool vs. shared pool for communication and discovery, and determine whether to support option 2 from discussion [AT115-e][617], in which the network configures both shared and dedicated pools.

Intended outcome: Report to next meeting

Deadline:  Long

The discussion is split into two phases:

- Phase 1: Companies are invited to provide views on the questions provided until the deadline: October 14th, 1200 UTC

- Phase 2: Rapporteur submits summary based on provided inputs and comments from companies until October 20th, 1200 UTC.

# Discussion

In existing TS 38.331 [1], TX and RX resource pools can be configured in NR SIB12, dedicated RRC message and pre-configuration, as illustrated in Figure.1, Figure.2 and Figure.3.

Figure.1 Rel-16 resource pool configuration in SIB

Figure.2 Rel-16 resource pool configuration in dedicated RRC message

Figure.3 Rel-16 resource pool configuration in pre-configuration

To align RAN2 understanding, Rapporteur would like to discuss the following 3 possible cases separately based on Rel-16 resource pools configuration framework:

- Only shared pools are configured

- Only dedicated pools are configured (if support)

- Both shared pools and dedicated pools are configured (if support)

## Only shared pools configured

It is obvious that shared pool will reuse Rel-16 resource pool configuration. Because shared pool was agreed as baseline, Rapporteur would like to first discuss the case when only shared is configured.

According to companies’ contributions in RAN2#115-e [1-9], there are two different understandings on shared pool configuration for discovery:

- Alt-1: All the configured TX pools (in SIB/RRC/Pre-config) can be used for discovery and SL communication [3-9]

  - No ASN.1 change compared with Rel-16 resource pool configured

- Alt-2: Extra indication is introduced to indicate whether each configured TX pool (in SIB/RRC/Pre-config) is shared for discovery and SL communication, or for SL communication only [9].

  - Extra indication (e.g. SLcommOnly) is required to be introduced within SL-ResourcePoolConfigm, e.g., the TX pool only for SL communication if SLcommOnly is set:

Q1: If only shared TX pools are configured in SIB/RRC/Pre-config, which understanding on shared pool configuration for discovery do you prefer?

- Alt-1: All the configured TX pools (in SIB/RRC/Pre-config) can be used for discovery and SL communication

- Alt-2: Extra indication is introduced to indicate whether each configured TX pool (in SIB/RRC/Pre-config) can be used for discovery and SL communication, or for SL communication only.

| Company | Alt-1 or Alt-2? | Comments |
| --- | --- | --- |
| OPPO | Alt-1 | The purpose of having shared Tx pools is to simplify network configuration and avoid resource fragmentation. Thus, if still need to have one bit to differentiate whether the Tx pools can be shared, the gain would be offset. |
| Ericsson | Alt-1 | We don’t see the need to have communication only resource pool, which would require extra design efforts. Also, the benefits are unclear. |
| InterDigital | Alt-1 | We see no benefit to Alt-2. |
| Qualcomm | Alt-1 | We have only up to 8 TX pools (i.e., maxNrofTXPool-r16=8) can be configured. There is no need to put further restriction on legacy TX pool in Alt-2 (i.e., whether SL communication only), which may decrease capability of the system. For Alt-1, our understanding is: - For Mode 1, it is up to gNB implementation to decide whether the shared pool is used for discovery or SL communication in one time instance - For Mode 2, UE follows legacy LCP procedure to decide whether the shared pool is used for discovery or SL communication in one time instance |

## Only dedicated pools configured?

Although RAN2 agreed to support dedicated pool besides shared pool as illustrated below, there is no explicit agreement whether only dedicated pool can be configured in SIB/RRC/Pre-configuration.

Proposal 6 [discussion]: RAN2 agrees dedicated discovery resource pool is supported besides shared resource pool configuration, whether it is configured is based on network implementation. And PHY layer parameters and design shall reuse the Rel-16 legacy resource pool design (including resource allocation design).

In RAN2#115-e [1], there were below two understandings for the UEs which are interested in discovery only:

- Understanding 1: these UEs can be configured with only dedicated pool for discovery (i.e., legacy resource pool is NOT configured).

- Understanding 2: these UEs should always be configured with legacy resource pool and optionally configured with dedicated pool for discovery.

Rapporteur’s understanding is:

- Dedicated RRC message: it seems to be valid that only dedicated pools can be configured for the UEs which are only interested in discovery (but without interest in SL communication)

- SIB: it seems there is no clear use scenario where SIB only configures dedicated pools because we need to support Rel-16 legacy V2X UE anyways

- Pre-configuration: it seems to be valid that only dedicated pools are pre-configured for the UEs which are only capable of, or interested in discovery

Because the answers may depend on how dedicated pools are configured, Rapporteur would like to confirm case by case. Please note that this question is NOT intended to discuss whether shared pool and dedicated pool can be configured simultaneously.

Q2: For RRC_IDLE/RRC_INACTIVE UEs which are interested in discovery only, do you agree that they can be configured with only dedicated pools for discovery in SIB?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | No | The gNB may not be aware of which UEs are only interested in discovery. Also, a UE may move from a cell to another cell. The situation for a cell will change from time to time. there is no point to limit a cell to only signal dedicated resource pool for discovery in the SIB. |
| InterDigital | No | We agree with rapporteur that we need to support legacy V2X UEs |
| Qualcomm | No | Legacy resource pools (i.e., shared pools) are required to be configured in SIB to support Rel-16 legacy V2X UEs. |

Q3: For RRC_CONNECTED UEs which are interested in discovery only, do you agree that they can be configured with only dedicated pools for discovery in dedicated RRC message?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | No | In general, we don’t see the need for the dedicated resource pool for discovery, which may decrease resource utilization issue. If there are few UEs interested in discovery only, the dedicated resource pool will be mostly likely to be under utilized. |
| InterDigital | No | This should be a network choice. |
| Qualcomm | Yes | If the concerned UE is only interested in discovery, RRC can only configure dedicated pools for this UE, to save power consumption. |

Q4: For OOC UEs which are interested in discovery only, do you agree that they can be pre-configured with only dedicated pools for discovery via pre-configuration?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | No | In general, we don’t see the need for the dedicated resource pool for discovery, which may decrease resource utilization issue. If there are few UEs interested in discovery only, the dedicated resource pool will be mostly likely to be under utilized. |
| InterDigital | No | This should also be a network choice. |
| Qualcomm | Yes | If the concerned UE is only interested in discovery, pre-configuration can only configure dedicated pools for this UE, to save power consumption. |

## Both shared and dedicated pools configured?

In offline discussion#617 [1], this topic was controversial if the UE is interested in both discovery and communication. Rapporteur understand that one reason of the chaos is because different aspects were mixed during the discussion of [617]. Thus, Rapporteur would like to first make clear the assumptions and the issue.

- Assumption: focus is the UEs which are interested in both discovery and SL communication.

- Issues:

- Whether both shared and dedicated pools can be configured simultaneously.

  - It is common understanding that the UE can be configured with only shared pools.

  - The controversial part is whether dedicated pools can also be configured besides shared pools. Some companies had concern that power saving gain of dedicated pool can’t be achieved if both are configured.

- Assume both types of pools for discovery can be configured simultaneously, whether shared pool can be used for both discovery and SL communication, or SL communication only.

  - Some companies had understanding if both are configured simultaneously, shared pool can only be used for SL communication.

For above two issues, Rapporteur would like to discuss separately for SIB, RRC and pre-configuration.

### SIB configuration

First, we discuss configuration in SIB for RRC_IDLE/INACTIVE UE. Please note that the serving cell may have sidelink communication only UEs, discovery only UEs, and UEs which are interested in both discovery and SL communication. To avoid misunderstanding, Rapporteur provided below example ASN.1 change for companies to provide response. Please note that it is just an example for accurate illustration of different options, and whether exceptional pool can be used for dedicated pool is out of scoping.

Figure.4 Example of both shared and dedicated pools configured in SIB (if support)

Then, according to offline discussion [AT115-e][617], we have the following options which may be configured together with shared pools (i.e., sl-RxPool and sl-TxPoolSelectedNormal):

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can NOT be configured simultaneously.

Q5: For RRC_IDLE/RRC_INACTIVE UEs, which option(s) do you think can be configured for discovery in SIB besides shared pool?

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can NOT be configured simultaneously with TX shared pool.

- Others (please describe if any)

| Company | Option a) or b) or c)? | Comments |
| --- | --- | --- |
| OPPO | a) | The benefit to introduce dedicated pools for discovery only is to consider the power consumption for those UEs who are only interested in discovery transmission/reception. To ensure the benefit, shared pools should only be used to transmit sidelink data. |
| Ericsson | b) | If dedicated and shared resource pools can be configured together, option b would be better. The other options put too much restriction to the gNB. |
| InterDigital | b) | The purpose of configuring both dedicated and shared pool is to have flexibility on where to transmit the discovery. So option b) is the only one providing this flexibility. |
| Qualcomm | b) | For IDLE/INACTIVE UEs, camping cell is not aware of whether they are discovery only, or SL communication only, or supporting both. Thus, we think it makes sense that NW is allowed to provide both shared pool and dedicated pool simultaneously in SIB for IDLE/INACTIVE UEs. Thus, we don’t prefer c), which puts too much restriction to gNB. Between a) and b), we prefer b) because 2 issues of a) are: - The dedicated pool may be configured sparsely. As a consequence, discovery will take longer if only the dedicated pool can be used for discovery. - For mode 2, it may bring extra (and unnecessary) spec impact on LCP restriction, e.g., TX shared pools need to be precluded in LCP procedure (similar to Rel-16 SL HARQ A/N enable) when UE sends discovery message In option b), we prefer it is up to UE implementation in Mode 2 to select shared pool or dedicated pool to transmit discovery message. |

### Dedicated RRC configuration

Then, we discuss configuration in dedicated RRC message for RRC_CONNECTED UE. Please note that the serving cell should know whether the concerned CONNECTED UE supports only sidelink communication, or discovery only, or both discovery and SL communication. To avoid misunderstanding, Rapporteur provided below example ASN.1 change for companies to provide response. Please note that it is just an example for accurate illustration of different options, and whether exceptional pool can be used for dedicated pool is out of scoping.

Figure.5 Example of both shared and dedicated pools configured in RRC (if support)

Then, according to offline discussion [AT115-e][617], we have the following options which may be configured together with shared pool (i.e., sl-RxPool, sl-TxPoolSelectedNormal and sl-TxPoolScheduling):

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal and sl-TxPoolScheduling) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal and sl-TxPoolScheduling) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can NOT be configured simultaneously.

Q6: For RRC_CONNECTED UEs, which option(s) do you think can be configured in dedicated RRC message for discovery besides shared pool?

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal and sl-TxPoolScheduling) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can be configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal and sl-TxPoolScheduling) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool, sl-DiscTxPoolSelectedNormal and sl-DiscTxPoolScheduling) can NOT be configured simultaneously with TX shared pool.

- Others (please describe if any)

| Company | Option a) or b) or c)? | Comments |
| --- | --- | --- |
| OPPO | a | See comment in Q5 |
| Ericsson | b) | If dedicated and shared resource pools can be configured together, option b would be better. The other options put too much restriction to the gNB. |
| InterDigital | b) | Same as Q5, because there should be no limitations placed on what the network is allowed to configured in dedicated RRC. |
| Qualcomm | b) | For CONNECTED UE, gNB is aware of whether it is discovery only, or SL communication only, or supporting both. So, we agree with some company that it seems no strong motivation, but we also agree with Ericsson and IDT that RAN2 generally leave the option to NW (i.e., not prohibit it in spec and up to NW whether to configure it. Thus, we don’t prefer c), which puts too much restriction to gNB. Between a) and b), we also prefer b) because of the 2 issues of a) analyzed in our comments to Q5. Similar to Q5, in this case, it is up to gNB implementation to select dedicated pool or shared for discovery in Mode 1, and it is up to UE implementation in Mode 2. |

### Pre-configuration

Finally, we discuss pre-configuration for OOC UE. To avoid misunderstanding, Rapporteur provided below example ASN.1 change for companies to provide response. Please note that it is just an example for accurate illustration of different options, and whether exceptional pool can be used for dedicated pool is out of scoping.

Figure.6 Example of both shared and dedicated pools in pre-configuration (if support)

Then, according to offline discussion [AT115-e][617], we have the following options which may be configured together with shared pool (i.e., sl-RxPool and sl-TxPoolSelectedNormal):

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be configured simultaneously. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can NOT be configured simultaneously.

Q7: For OOC UEs, which option(s) do you think can be pre-configured for discovery besides shared pool?

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be pre-configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can only be used for SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can be pre-configured simultaneously with TX shared pool. And TX shared pool (i.e., sl-TxPoolSelectedNormal) can be used for both discovery and SL communication.

- Dedicated pools for discovery only (i.e., sl-DiscRxPool and sl-DiscTxPoolSelectedNormal) can NOT be pre-configured simultaneously with TX shared pool.

- Others (please describe if any)

| Company | Option a) or b) or c)? | Comments |
| --- | --- | --- |
| OPPO | a) | See comment in Q5 |
| Ericsson | b) |  |
| InterDigital | b) | Same as Q5, because there should be no limitations placed on what the network is allowed to configured in preconfiguration. |
| Qualcomm | b) | We tend to agree with some company that it seems no strong motivation, but we also agree with Ericsson and IDT that RAN2 generally leave the option to NW (i.e., not prohibit it in spec and up to NW whether to configure it). Thus, we don’t prefer c), which puts too much restriction to gNB. Between a) and b), we also prefer b) because of the 2 issues of a) analyzed in our comments to Q5. In option b), we prefer it is up to UE implementation in Mode 2 to select shared pool or dedicated pool to transmit discovery message. |

### Selection between shared pool and dedicated pool (if allowed)

If you pick option b) for Q5 or Q6 or Q7, it means you think:

- Both dedicated pool and shared pool can be configured simultaneously (at least in some cases), and

- Both can be used for discovery.

Thus, a followed question is how to select between the two (i.e., shared pool and dedicated pool) to carry discovery traffic. This question was discussed in RAN2#115-e, and it was agreed that it is up to gNB for mode 1, but 3 options were captured in Chair Notes to down select for mode 2:

Agreements:

FFS if the network can configure shared and dedicated pool simultaneously.

For mode 1, if agreed that both shared and dedicated resource pools can be configured, it is up to gNB which one the UE should use to transmit discovery message.

For mode 2, if agreed that both shared and dedicated resource pools can be configured, downselect from the following options:

- Left to UE implementation

- Dedicated pool should be prioritised

- Shared pool should be prioritised

Q8: If you pick option b) for either Q5 or Q6 or Q7, which option do you prefer on selection between shared pool and dedicated pool to carry discovery traffic in Mode 2?

- Left to UE implementation

- Dedicated pool should be prioritized.

- Shared pool should be prioritized

| Company | Option a) or b) or c)? | Comments |
| --- | --- | --- |
| Ericsson | a | It is sufficient to leave to UE implementation. |
| InterDigital | b) or c) with comments | For the network to have the full advantage of configuring both pools, not only should the UE prioritize one pool over another, but there should be conditions under which the UE should be allowed to transmit on the non-prioritized pool (e.g. based on congestion on one pool). Otherwise, the network would need to account for the case (for example) where all UEs choose the dedicated pool, resulting in significant resource fragmentation. |
| Qualcomm | a | Please note that a) is the outcome of legacy LCP procedure specified in 38.321 without further spec impact: - RAN2 has agreed to use the same LCH priority value 1 for sidelink discovery message - According to existing clause 5.22.1.4.1.2 of TS 38.321, the UE selects the destination associated with the highest SL LCH priority for a new transmission. And we have a NOTE: “if multiple Destinations have the logical channels satisfying all conditions above with the same highest priority, which Destination is selected among them is up to UE implementation.” - Then, when UE sends discovery in Mode 2, it should: - First check all logical channels with LCH priority 1 in configured shared pools and dedicated pools. - If both shared pool and dedicated pool have available logical channels with LCH priority 1, UE can select either one based on its implementation according to the NOTE in clause 5.22.1.4.1.2 of TS 38.321. From technique perspective, benefit of a) is that latency of discovery delivery can be reduced because UE is allowed to select a resource pool which comes earlier. Please note that NW is not aware of when discovery delivery is triggered. Thus, if priority is configured like b) or c), UE may be forced to wait another resource pool for discovery, even if one resource pool is already available. |

# Conclusion

Based on companies’ inputs, Rapporteur made below proposals:

# References

[1] R2-2108949, [AT115-e][617][Relay] Continuation of discussion on discovery (CATT), CATT

[2] R2-2106994, Leftover Issues for Sidelink Discovery, CATT

[3] R2-2107089, Remaining issues on relay discovery, Qualcomm

[4] R2-2107313, Leftover aspects of Relay discovery, Intel

[5] R2-2108152, Relay Discovery transmission for stage 3, LG

[6] R2-2107212, Discussion on remaining issue of relay discovery, OPPO

[7] R2-2108143	Further discussion on Relay discovery, ZTE, Sanechips

[8] R2-2107468, Left issues for SL discovery, Ericsson

[9] R2-2108626, Remaining issue on relay discovery, Huawei, HiSilicon