3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2xxxxxx

Online, November, 2021

Agenda item:	8.15

Source: 	LG Electronics Inc. (Rapporteur)

Title: 		[POST115-e][712][V2X/SL] 38.321 running CR (LG)

Document for:	Discussion and decision

# Introduction

This is to kick off the offline discussion on [POST115-e][712][V2X/SL] 38.321 running CR (LG):

- [POST115-e][712][V2X/SL] 38.321 running CR (LG)

Scope: Prepare the running CR with the agreements made up to now. Rapporteur can provide stage 3 open issue list to help further discussion.

Intended outcome: 38.321 running CR to be endorsed. Open issue list for stage 3 discussion.

Deadline: Long email discussion

Two word files are provided for this email thread: one for discussion on stage 3 open issues and one for the actual running CR to 38.321 with corresponding RAN2 agreements in word memos. Please directly provide your comment on the running CR desirably with word memos, if your comment is not related to the open issue listed below. Open issues to be discussed directly on Running CR are not included in this document.

# Issues for a running CR to 38.321

#### Issue 5.22.1.4.1.3-1: Priority order of SL DRX Command MAC CE

RAN2 previously agreed in RAN2#113-e that SL DRX Command MAC CE is introduced for SL DRX operation in unicast. RAN2 should discuss the priority order of the Sidelink DRX Command MAC CE.

Priority order of Sidelink MAC CE is specified in 38.321 as follows:

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	data from SCCH;

-	Sidelink CSI Reporting MAC CE;

-	data from any STCH.

Thus, one of the options is to have the same priority order of Sidelink CSI Reporting MAC CE.

Q5.22.1.4.1.3-1: Which option do you prefer for priority order of Sidelink DRX Command MAC CE?

- Same priority order of Sidelink CSI Reporting MAC CE

- Between data from SCCH and Sidelink CSI Reporting MAC CE

- Between Sidelink CSI Reporting MAC CE and data from any STCH

- Any other

| Company | Option | Comment |
| --- | --- | --- |

#### Issue 5.x.1-1: Starting point of sl-drx-HARQ-RTT-Timer in case of HARQ feedback disabled

RAN2 has not yet discussed when the Rx UE starts the sl-drx-HARQ-RTT-Timer in the HARQ feedback disabled case.

Thus, RAN2 should discuss when the RX UE should start the sl-drx-HARQ-RTT-Timer in case of HARQ feedback disabled.

Rapporteur think that for the case where the TX UE schedules more than one sidelink resource through the SCI, it is reasonable to assume that a blind retransmission will not be scheduled before the UE finishes all the transmission opportunities scheduled through the SCI.

Q5.x.1-1: Which option do you prefer for starting point of sl-drx-HARQ-RTT-Timer in case of HARQ feedback disabled?

- started at the first slot after the end of last PSSCH resource scheduled through SCI

- started at the first slot after the fist slot of SCI reception

- Any other?

| Company | Option | Comment |
| --- | --- | --- |

#### Issue 6.1.3.x-1: Priority value of SL DRX Command MAC CE

RAN2 should discuss and decide the priority value of the Sidelink DRX Command MAC CE. One of the options is to have the same priority vaue (e.g., fixed value “1”) of Sidelink CSI Reporting MAC CE.

Q6.1.3.x-1: Can Sidelink DRX Command MAC CE's priority value be set to a fixed value (i.e., “1”)?

- Yes

- No

- Any other?

| Company | Yes or no | Comment |
| --- | --- | --- |

# Conclusion and recommendation

…