3GPP TSG-WG2 Meeting #116-e	R2-2111405

# eMeeting, November 1st - 12th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | Draft | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NB-IoT/eMTC support for Non-Terrestrial Networks [Stage 2 Running CR] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Eutelsat |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of specific NTN vocabulary, architecture and protocols aspects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT/eMTC for Non-Terrestrial Networks is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | <Will be populated later on> |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-0: creation based on RAN2#115-e agreements Rev-1: update based on RAN2#116-e agreements |  |  |  |  |  |  |  |  |

Start of changes

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

Editor’s Note: where indicated, related definitions of the present section may need to be aligned with NR NTN stage 2 CR or provided by the relevant 3GPP WG.

For the purposes of the present document, the following terms and definitions apply.

Access Control: the process that checks whether a UE is allowed to access and to be granted services in a closed cell.

Aerial UE communication: functionality enabling Aerial UE function as defined in 23.17.

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Carrier frequency: center frequency of the cell.

Cell: combination of downlink and optionally uplink resources. The linking between the carrier frequency of the downlink resources and the carrier frequency of the uplink resources is indicated in the system information transmitted on the downlink resources.

Cell Group: in dual connectivity, a group of serving cells associated with either the MeNB or the SeNB.

CHO candidate cell: a candidate cell for CHO, for which UE has been configured with a CHO configuration.

Conditional Handover (CHO): a handover procedure that is executed only when execution condition(s) are met.

Control plane CIoT 5GS Optimisation: Enables support of efficient transport of user data (IP, Ethernet and Unstructured) or SMS messages over control plane via the AMF without triggering user-plane resource establishment, as defined in TS 24.501 [91]. In the context of this specification, a NB-IoT UE that only supports Control plane CIoT 5GS Optimisation is a UE that does not support User plane CIoT 5GS Optimisation and NG-U data transfer but may support other CIoT 5GS Optimisations.

Control plane CIoT EPS optimisation: Enables support of efficient transport of user data (IP, non-IP or SMS) over control plane via the MME without triggering data radio bearer establishment, as defined in TS 24.301 [20]. In the context of this specification, a NB-IoT UE that only supports Control plane CIoT EPS optimisation is a UE that does not support User plane CIoT EPS optimisation and S1-U data transfer but may support other CIoT EPS optimisations.

CSG Cell: a cell broadcasting a CSG indicator set to true and a specific CSG identity.

CSG ID Validation: the process that checks whether the CSG ID received via handover messages is the same as the one broadcast by the target E-UTRAN.

CSG member cell: a cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN and for which the CSG whitelist of the UE includes an entry comprising cell's CSG ID and the respective PLMN identity.

DAPS Handover: a handover procedure that maintains the source eNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target eNB.

DCN-ID: DCN identity identifies a specific dedicated core network (DCN).

Dual Connectivity: mode of operation of a UE in RRC_CONNECTED, configured with a Master Cell Group and a Secondary Cell Group.

Early Data Forwarding: data forwarding that is initiated before the UE executes the handover.

en-gNB: as defined in TS 37.340 [76].

E-RAB: an E-RAB uniquely identifies the concatenation of an S1 Bearer and the corresponding Data Radio Bearer. When an E-RAB exists, there is a one-to-one mapping between this E-RAB and an EPS bearer of the Non Access Stratum as defined in [17].

Feeder link: [to be aligned to NR NTN stage 2 CR].

Frequency layer: set of cells with the same carrier frequency.

FeMBMS: further enhanced multimedia broadcast multicast service.

FeMBMS/Unicast-mixed cell: cell supporting MBMS transmission and unicast transmission as SCell.

Geosynchronous Orbit: Earth-centred orbit at approximately 35,786 kilometres in altitude above Earth's surface and synchronised with Earth's rotation. A geostationary orbit is a non-inclined geosynchronous orbit, i.e. in the Earth’s equator plane.

Handover: procedure that changes the serving cell of a UE in RRC_CONNECTED.

Hybrid cell: a cell broadcasting a CSG indicator set to false and a specific CSG identity. This cell is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs.

Late Data Forwarding: data forwarding that is initiated after the source eNB knows that the UE has successfully accessed a target eNB.

Local Home Network: as defined in TS 23.401 [17].

LTE bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in the eNB only to use eNB radio resources only.

LWA bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in both the eNB and the WLAN to use both eNB and WLAN resources.

LWAAP PDU: in LTE-WLAN Aggregation, a PDU with DRB ID generated by LWAAP entity for transmission over WLAN.

Make-Before-Break HO/SeNB change: maintaining source eNB/SeNB connection after reception of RRC message for handover or change of SeNB before the initial uplink transmission to the target eNB during handover or change of SeNB.

Master Cell Group: in dual connectivity, a group of serving cells associated with the MeNB, comprising of the PCell and optionally one or more SCells.

Master eNB: in dual connectivity, the eNB which terminates at least S1-MME.

MBMS-dedicated cell: cell dedicated to MBMS transmission.

MBMS/Unicast-mixed cell: cell supporting both unicast and MBMS transmissions.

MCG bearer: in dual connectivity, a bearer whose radio protocols are only located in the MeNB to use MeNB resources only.

Membership Verification: the process that checks whether a UE is a member or non-member of a hybrid cell.

Multi-Connectivity: Mode of operation whereby a multiple Rx/Tx UE in the connected mode is configured to utilise radio resources amongst E-UTRA and/or NR provided by multiple distinct schedulers connected via non-ideal backhaul.

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 200 kHz.

NB-IoT UE: a UE that uses NB-IoT.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

Non-anchor carrier: in NB-IoT, a carrier where the UE does not assume that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Non-geosynchronous orbit: Earth-centred orbit with an orbital period that does not match Earth's rotation on its axis. This includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO).

Non-terrestrial networks: [to be aligned to NR NTN stage 2 CR].

NR: NR radio access

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [93], between two or more nearby UEs, using NR technology but not traversing any network node.

NTN-gateway: [to be aligned to NR NTN stage 2 CR].

NTN payload: [to be aligned to NR NTN stage 2 CR].

PLMN ID Check: the process that checks whether a PLMN ID is the RPLMN identity or an EPLMN identity of the UE.

Power saving mode: mode configured and controlled by NAS that allows the UE to reduce its power consumption, as defined in TS 24.301 [20], TS 23.401 [17], TS 23.682 [57].

Primary PUCCH group: a group of serving cells including PCell whose PUCCH signalling is associated with the PUCCH on PCell.

Primary Timing Advance Group: Timing Advance Group containing the PCell. In this specification, Primary Timing Advance Group refers also to Timing Advance Group containing the PSCell unless explicitly stated otherwise.

ProSe-enabled Public Safety UE: a UE that the HPLMN has configured to be authorized for Public Safety use, and which is ProSe-enabled and supports ProSe procedures and capabilities specific to Public Safety. The UE may, but need not, have a USIM with one of the special access classes {12, 13, 14}.

ProSe Per-Packet Priority: a scalar value associated with a protocol data unit that defines the priority handling to be applied for transmission of that protocol data unit.

ProSe UE-to-Network Relay: a UE that provides functionality to support connectivity to the network for Remote UE(s).

ProSe UE-to-Network Relay Selection: Process of identifying a potential ProSe UE-to Network Relay, which can be used for connectivity services (e.g. to communicate with a PDN).

ProSe UE-to-Network Relay Reselection: process of changing previously selected ProSe UE-to-Network Relay and identifying potential a new ProSe UE-to-Network Relay, which can be be used for connectivity services (e.g. to communicate with PDN).

Public Safety ProSe Carrier: carrier frequency for public safety sidelink communication and public safety sidelink discovery.

PUCCH group: either primary PUCCH group or a secondary PUCCH group.

PUCCH SCell: a Secondary Cell configured with PUCCH.

RACH-less HO/SeNB change: skipping random access procedure during handover or change of SeNB.

Receive Only Mode: See TS 23.246 [48].

Remote UE: a ProSe-enabled Public Safety UE, that communicates with a PDN via a ProSe UE-to-Network Relay.

Satellite: a space-borne vehicle orbiting the Earth and carrying the NTN payload.

SCG bearer: in dual connectivity, a bearer whose radio protocols are only located in the SeNB to use SeNB resources.

Secondary Cell Group: in dual connectivity, a group of serving cells associated with the SeNB, comprising of PSCell and optionally one or more SCells.

Secondary eNB: in dual connectivity, the eNB that is providing additional radio resources for the UE but is not the Master eNB.

Secondary PUCCH group: a group of SCells whose PUCCH signalling is associated with the PUCCH on the PUCCH SCell.

Secondary Timing Advance Group: Timing Advance Group containing neither the PCell nor PSCell.

Service link: wireless link between the NTN payload and the UE.

Short Processing Time: For 1 ms TTI length, the operation with short processing time in UL data transmission and DL data reception.

Short TTI: TTI length based on a slot or a subslot.

Sidelink: UE to UE interface for sidelink communication, V2X sidelink communication and sidelink discovery. The Sidelink corresponds to the PC5 interface as defined in TS 23.303 [62].

Sidelink Control period: period over which resources are allocated in a cell for sidelink control information and sidelink data transmissions. The Sidelink Control period corresponds to the PSCCH period as defined in TS 36.213 [6].

Sidelink communication: AS functionality enabling ProSe Direct Communication as defined in TS 23.303 [62], between two or more nearby UEs, using E-UTRA technology but not traversing any network node. In this version, the terminology "sidelink communication" without "V2X" prefix only concerns PS unless specifically stated otherwise.

Sidelink discovery: AS functionality enabling ProSe Direct Discovery as defined in TS 23.303 [62], using E-UTRA technology but not traversing any network node.

Split bearer: in dual connectivity, a bearer whose radio protocols are located in both the MeNB and the SeNB to use both MeNB and SeNB resources.

Split LWA bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in both the eNB and the WLAN to use both eNB and WLAN radio resources.

Switched LWA bearer: in LTE-WLAN Aggregation, a bearer whose radio protocols are located in both the eNB and the WLAN but uses WLAN radio resources only.

Timing Advance Group: a group of serving cells that is configured by RRC and that, for the cells with an UL configured, use the same timing reference cell and the same Timing Advance value.

User plane CIoT 5GS Optimisation: Enables support for change from 5GMM-IDLE mode to 5GMM-CONNECTED mode without the need for using the Service Request procedure, as defined in TS 24.501 [91].

User plane CIoT EPS optimisation: Enables support for change from EMM-IDLE mode to EMM-CONNECTED mode without the need for using the Service Request procedure, as defined in TS 24.301 [20].

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [72], between nearby UEs, using E-UTRA technology but not traversing any network node.

WLAN Termination: the logical node that terminates the Xw interface on the WLAN side.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1xCSFB	Circuit Switched Fallback to 1xRTT

5GC	5G Core Network

ABS	Almost Blank Subframe

AC	Access Category

ACK	Acknowledgement

ACLR	Adjacent Channel Leakage Ratio

AM	Acknowledged Mode

AMBR	Aggregate Maximum Bit Rate

ANDSF	Access Network Discovery and Selection Function

ANR	Automatic Neighbour Relation

ARP	Allocation and Retention Priority

ARQ	Automatic Repeat Request

AS	Access Stratum

AUL	Autonomous Uplink

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BL	Bandwidth reduced Low complexity

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

BSR	Buffer Status Report

C/I	Carrier-to-Interference Power Ratio

CA	Carrier Aggregation

CAZAC	Constant Amplitude Zero Auto-Correlation

CBC	Cell Broadcast Center

CC	Component Carrier

CG	Cell Group

CHO	Conditional Handover

CIF	Carrier Indicator Field

CIoT	Cellular Internet of Things

CMAS	Commercial Mobile Alert Service

CMC	Connection Mobility Control

C-plane	Control Plane

C-RNTI	Cell RNTI

CoMP	Coordinated Multi Point

CP	Cyclic Prefix

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRE	Cell Range Extension

CRS	Cell-specific Reference Signal

CSA	Common Subframe Allocation

CSG	Closed Subscriber Group

CSI	Channel State Information

CSI-IM	CSI interference measurement

CSI-RS	CSI reference signal

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCN	Dedicated Core Network

DeNB	Donor eNB

DFTS	DFT Spread OFDM

DL	Downlink

DMTC	Discovery Signal Measurement Timing Configuration

DRB	Data Radio Bearer

DRS	Discovery Reference Signal

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

DTX	Discontinuous Transmission

DwPTS	Downlink Pilot Time Slot

E-CID	Enhanced Cell-ID (positioning method)

E-RAB	E-UTRAN Radio Access Bearer

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EAB	Extended Access Barring

ECGI	E-UTRAN Cell Global Identifier

ECM	EPS Connection Management

EDT	Early Data Transmission

EHC	Ethernet Header Compression

eHRPD	enhanced High Rate Packet Data

eIMTA	Enhanced Interference Management and Traffic Adaptation

EMM	EPS Mobility Management

eNB	E-UTRAN NodeB

EPC	Evolved Packet Core

EPDCCH	Enhanced Physical Downlink Control Channel

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

G-RNTI	Group RNTI

GBR	Guaranteed Bit Rate

GERAN	GSM EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GP	Guard Period

GRE	Generic Routing Encapsulation

GSM	Global System for Mobile communication

GSO	Geosynchronous Orbit

GUMMEI	Globally Unique MME Identifier

GUTI	Globally Unique Temporary Identifier

GWCN	GateWay Core Network

GWUS	Group Wake Up Signal

H-SFN	Hyper System Frame Number

HARQ	Hybrid ARQ

(H)eNB	eNB or HeNB

HO	Handover

HPLMN	Home Public Land Mobile Network

HRPD	High Rate Packet Data

HSDPA	High Speed Downlink Packet Access

ICIC	Inter-Cell Interference Coordination

IDC	In-Device Coexistence

IP	Internet Protocol

ISM	Industrial, Scientific and Medical

KPAS	Korean Public Alert System

L-GW	Local Gateway

LAA	Licensed-Assisted Access

LB	Load Balancing

LBT	Listen Before Talk

LCG	Logical Channel Group

LCR	Low Chip Rate

LCS	LoCation Service

LEO	Low Earth Orbit

LHN	Local Home Network

LHN ID	Local Home Network ID

LIPA	Local IP Access

LMU	Location Measurement Unit

LPPa	LTE Positioning Protocol Annex

LTE	Long Term Evolution

LWA	LTE-WLAN Aggregation

LWAAP	LTE-WLAN Aggregation Adaptation Protocol

LWIP	LTE WLAN Radio Level Integration with IPsec Tunnel

LWIP-SeGW	LWIP Security Gateway

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBR	Maximum Bit Rate

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCCH	Multicast Control Channel

MCE	Multi-cell/multicast Coordination Entity

MCG	Master Cell Group

MCH	Multicast Channel

MCS	Modulation and Coding Scheme

MDT	Minimization of Drive Tests

MeNB	Master eNB

MEO	Medium Earth Orbit

MGW	Media Gateway

MIB	Master Information Block

MIMO	Multiple Input Multiple Output

MME	Mobility Management Entity

MMTEL	Multimedia telephony

MO-EDT	Mobile Originated Early Data Transmission

MPDCCH	MTC Physical Downlink Control Channel

MSA	MCH Subframe Allocation

MSI	MCH Scheduling Information

MSP	MCH Scheduling Period

MT-EDT	Mobile Terminated Early Data Transmission

MTC	Machine-Type Communications

MTCH	Multicast Traffic Channel

MTSI	Multimedia Telephony Service for IMS

N2	Reference point between the NG-RAN and the AMF

NACK	Negative Acknowledgement

NAS	Non-Access Stratum

NB-IoT	Narrow Band Internet of Things

NCC	Next Hop Chaining Counter

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NG-RAN	NG Radio Access Network

NGSO	Non-Geosynchronous Orbit

NH	Next Hop key

NNSF	NAS Node Selection Function

NPBCH	Narrowband Physical Broadcast channel

NPDCCH	Narrowband Physical Downlink Control channel

NPDSCH	Narrowband Physical Downlink Shared channel

NPRACH	Narrowband Physical Random Access channel

NPUSCH	Narrowband Physical Uplink Shared channel

NPRS	Narrowband Positioning Reference Signal

NPSS	Narrowband Primary Synchronization Signal

NR	NR Radio Access

NRT	Neighbour Relation Table

NSSS	Narrowband Secondary Synchronization Signal

NTN	Non-Terrestrial Network

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OPI	Offload Preference Indicator

OTDOA	Observed Time Difference Of Arrival (positioning method)

P-GW	PDN Gateway

P-RNTI	Paging RNTI

PA	Power Amplifier

PAPR	Peak-to-Average Power Ratio

PBCH	Physical Broadcast CHannel

PBR	Prioritised Bit Rate

PCC	Primary Component Carrier

PCCH	Paging Control Channel

PCell	Primary Cell

PCFICH	Physical Control Format Indicator CHannel

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control CHannel

PDCP	Packet Data Convergence Protocol

PDN	Packet Data Network

PDSCH	Physical Downlink Shared CHannel

PDU	Protocol Data Unit

PHICH	Physical Hybrid ARQ Indicator CHannel

PHY	Physical layer

PLMN	Public Land Mobile Network

PMCH	Physical Multicast CHannel

PMK	Pairwise Master Key

PPPP	ProSe Per-Packet Priority

PPPR	ProSe Per-Packet Reliability

PRACH	Physical Random Access CHannel

PRB	Physical Resource Block

ProSe	Proximity based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSC	Packet Scheduling

PSCCH	Physical Sidelink Control CHannel

PSCell	Primary SCell

PSDCH	Physical Sidelink Discovery CHannel

PSK	Pre-Shared Key

PSM	Power Saving Mode

PSSCH	Physical Sidelink Shared CHannel

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control CHannel

PUR	Preconfigured Uplink Resource

PUR-RNTI	Preconfigured Uplink Resource RNTI

PUSCH	Physical Uplink Shared CHannel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QCI	QoS Class Identifier

QoE	Quality of Experience

QoS	Quality of Service

R-PDCCH	Relay Physical Downlink Control CHannel

RA-RNTI	Random Access RNTI

RAC	Radio Admission Control

RACH	Random Access Channel

RANAC	RAN-based Notification Area code

RAT	Radio Access Technology

RB	Radio Bearer

RBC	Radio Bearer Control

RCLWI	RAN Controlled LTE-WLAN Interworking

RF	Radio Frequency

RIBS	Radio-interface based synchronization

RIM	RAN Information Management

RLC	Radio Link Control

RMTC	RSSI Measurement Timing Configuration

RN	Relay Node

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNC	Radio Network Controller

RNL	Radio Network Layer

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

ROM	Receive Only Mode

RRC	Radio Resource Control

RRM	Radio Resource Management

RTT	Round Trip Time

RU	Resource Unit

S-GW	Serving Gateway

S-RSRP	Sidelink Reference Signal Received Power

S1-MME	S1 for the control plane

SAE	System Architecture Evolution

SAP	Service Access Point

SBCCH	Sidelink Broadcast Control Channel

SC-FDMA	Single Carrier – Frequency Division Multiple Access

SC-MCCH	Single Cell Multicast Control Channel

SC-MTCH	Single Cell Multicast Transport Channel

SC-N-RNTI	Single Cell Notification RNTI

SC-PTM	Single Cell Point To Multiploint

SC-RNTI	Single Cell RNTI

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCH	Synchronization Channel

SCTP	Stream Control Transmission Protocol

SD-RSRP	Sidelink Discovery Reference Signal Received Power

SDAP	Service Data Adaptation Protocol

SDF	Service Data Flow

SDMA	Spatial Division Multiple Access

SDU	Service Data Unit

SeGW	Security Gateway

SeNB	Secondary eNB

SFN	System Frame Number

SI	System Information

SI-RNTI	System Information RNTI

S1-U	S1 for the user plane

SIB	System Information Block

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

SL-BCH	Sidelink Broadcast Channel

SL-DCH	Sidelink Discovery Channel

SL-RNTI	Sidelink RNTI

SL-SCH	Sidelink Shared Channel

SPDCCH	Short PDCCH

SPID	Subscriber Profile ID for RAT/Frequency Priority

SPT	Short Processing Time

SPUCCH	Short PUCCH

SR	Scheduling Request

SRB	Signalling Radio Bearer

sTAG	Secondary Timing Advance Group

STCH	Sidelink Traffic Channel

SU	Scheduling Unit

TA	Tracking Area

TAC	Tracking Area Code

TAG	Timing Advance Group

TB	Transport Block

TCP	Transmission Control Protocol

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TFT	Traffic Flow Template

TM	Transparent Mode

TMGI	Temporary Mobile Group Identity

TNL	Transport Network Layer

TTI	Transmission Time Interval

U-plane	User plane

UAC	Unified Access Control

UDC	Uplink Data Compression

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UMTS	Universal Mobile Telecommunication System

UpPTS	Uplink Pilot Time Slot

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

V2I	Vehicle-to-Infrastructure

V2N	Vehicle-to-Network

V2P	Vehicle-to-Pedestrian

V2V	Vehicle-to-Vehicle

V2X	Vehicle-to-Everything

VRB	Virtual Resource Block

WLAN	Wireless Local Area Network

WT	WLAN Termination

WUS	Wake Up Signal

X2-C	X2-Control plane

X2 GW	X2 GateWay

X2-U	X2-User plane

Xw-C	Xw-Control plane

Xw-U	Xw-User plane

Next change

## 4.x	Non-Terrestrial Networks

Editor’s Note: the present section needs to be updated by RAN3.

Non-Geosynchronous Orbit (NGSO) includes Low-Earth Orbit and Medium Earth Orbit.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

Next change

## 23.x	Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks

### 23.x.1	General

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks (see sub-clause 4.X) is only applicable to E-UTRA connected to EPC.

In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification.

To accommodate long propagation delays in NTN, increased timer values and window sizes, or delayed starting times, are specified for the physical layer and for higher layers.

### 23.x.2	Synchronisation

For the serving cell, the network broadcast ephemeris information and common Timing Advance parameters for the serving cell for the UEs autonomous Timing Advance and frequency shift pre-compensation.

Editor’s note: Updates to serving cell ephemeris information are not bound to the BCCH modification period.

Before performing random access, the UE shall autonomously pre-compensate the Timing Advance for the long propagation delay as well as the frequency doppler shift. The pre-compensation is done by a common Timing Advance, UE being aware of its own position, and identifying the satellite position through the satellite ephemeris.

In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation, but the UE is not expected to perform GNSS acquisition.

The UEs may be configured to report information about the UE specific Timing Advance pre-compensation at initial access or in connected mode. Triggers for UE specific Timing Advance information reporting in connected mode can be event-based.

Editor’s note: triggers other than event-based may be defined for UE specific Timing Advance information reporting in connected mode, depending on further NR NTN agreements.

Editor’s Note: It is feasible to use the legacy barring bit to block legacy UEs, and it is possible to have a new bit that assumes the functionality of the old bit. It is FFS if it is needed to use the barring bit or whether other mechanism can be assumed (new band etc).

### 23.x.3	Support of discontinuous coverage

As a satellite moves on a specified orbit, for example in case of a NGSO satellite, the satellite beam(s) coverage area may move and cover different portions of a geographical area due to the orbital movement of the satellite. As a consequence, a UE located in the concerned geographical area may experience a situation of discontinuous coverage, due to e.g. a sparse satellites constellation deployment.

Editor’s Note: the above "discontinuous coverage" description may be further completed for describing differences between Quasi-Earth-fixed and moving beams/cells. Part or all of it could be further moved to section 3.1 if felt more relevant.

Editor’s Note: discontinuous coverage without excessive UE power consumption and without excessive failures / recovery actions is supported, at least for Idle mode. The requirement is applicable for all reference scenarios (GSO, MEO and LEO).

Editor’s Note: Satellite assistance information will be used by the UE for predicting coverage discontinuity. The details of the assistance information are FFS. FFS whether any applicable agreements made in NR-NTN can be reused.

Editor’s Note: The details of UEs actions when predicted to be out of coverage is FFS, e.g. stopping unnecessary cell search in the Idle mode, and FFS to what extent this needs to be specified.

Editor’s Note: It is FFS to what extent it needs to be specified the details of UE’s prediction of discontinuous coverage and its ability to detect when it is back in coverage.

Editor’s Note: Satellite Ephemeris Parameters (not same as for L1 pre-compensation, for the constellation, not just single satellite) is needed for the UE for predicting coverage discontinuity. Other info, e.g. beam info, elevation angle, reference location or corresponding is FFS.

Editor’s Note: Providing the start-time of (incoming) satellite’s coverage and end-time of serving satellite’s coverage is needed for Quasi-Earth Fixed satellites.

Editor’s Note: From RAN2 point of view, the existing power saving mechanisms e.g. DRX, PSM, eDRX, relaxed monitoring, and WUS can be reused in IoT-NTN. Minor enhancements in existing power saving mechanisms to support discontinuous coverage is FFS.

### 23.x.4	Mobility Management

#### 23.x.4.1	Mobility Management in ECM-IDLE

The principles described in clause 10.1.1 apply in NTN unless specified otherwise hereafter.

The network may broadcast more than one TAC per PLMN in a cell in order to reduce the signalling load at cell edge in NTN, in particular for Earth-moving cell coverage. The AS layer indicates all received TACs for the selected PLMN to the NAS layer.

NOTE:	Whether the network broadcasts a single TAC or more than one TAC per PLMN is up to network implementation.

Editor’s note: FFS if Satellite assistance information for neighbour cell(s) is provided to UE for cell selection/reselection (justification would be needed).

Editor’s note: When the network stops broadcasting a TAC, the UE needs to know it. FFS how this is done.

Editor’s note: RAN2 assumes that Satellite assistance information, e.g. for cell selection reselection, for serving cell is provided to UE.

For quasi-Earth-fixed cells, timing information on when the cell is going to stop serving the area may be broadcast by the network. This may be used by the UE to start measurements on neighbour cells before the broadcast stop time of the serving cell, while the exact start of the measurements is up to UE implementation.

Editor’s note: FFS details and to what extent this needs to be covered in the TS.

#### 23.x.4.2	Mobility Management in ECM-CONNECTED

Radio link failure and RRC connection re-establishment are supported in NTN. The principles described in clause 10.1.6 apply unless specified otherwise.

Editor’s note: No enhancement to R16 RLF and RRC connection Re-establishment procedures are introduced in R17 (this does not include handling of UL synchronisation loss which is FFS and does not include non continuous coverage).

Conditional handover is supported for BL UEs and UEs in enhanced coverage.

Editor’s note: FFS which CE Mode(s) to apply. No enhancement to R16 CHO are introduced in R17.

End of changes

The following appendix shall be removed from the final CR.

# T1	Appendix: RAN2 agreements for the LTE_NBIOT_eMTC_NTN WI

## T1.1 Support of Non continuous coverage

RAN2#116-e Agreements:

- Satellite Ephemeris Parameters (not same as for L1 pre-compensation, for the constellation, not just single satellite) is needed for the UE for predicting coverage discontinuity. Other info, e.g. beam info, elevation angle, reference location or corresponding is FFS.

- Providing the start-time of (incoming) satellite’s coverage and end-time of serving satellite’s coverage is needed for Quasi-Earth Fixed satellites.

- From RAN2 point of view, the existing power saving mechanisms e.g. DRX, PSM, eDRX, relaxed monitoring, and WUS can be reused in IoT-NTN. Minor enhancements in existing power saving mechanisms to support discontinuous coverage is FFS.

RAN2#115-e Agreements:

- RAN2 confirms that the following will be supported: discontinuous coverage without excessive UE power consumption and without excessive failures / recovery actions. It is expected that this need to be taken into account at least for Idle mode. The requirement is applicable for all reference scenarios (GEO, MEO and LEO).

- Sattelite assistance information will be used by the UE for predicting coverage discontinuity. The details of the assistance information is FFS. FFS whether any applicable agreements made in NR-NTN can be reused.

- The details of UEs actions when predicted to be out of coverage is FFS, e.g. stopping unnecessary cell search in the Idle mode, and FFS to what extent this need to be specified.

- It is FFS to what extent it need to be specified the details of UE’s prediction of discontinuous coverage and its ability to detect when it is back in coverage.

- RAN2 sends an LS to SA2 and CT1 (cc: RAN3) for the possible alignment work in their specification due to the support of discontinuous coverage.

## T1.2 User plane impacts

RAN2#116-e Agreements:

- The estimate of UE-eNB RTT is equal to the sum of UE’s TA and K_mac, where the UE’s TA is given by T_TA=(N_TA+N_(TA,UE-specific) 〖+N〗_(TA,common) 〖+N〗_(TA,offset) )×T_s, and K_mac value is broadcasted by network.

- RAN2 confirm that the start of mac-ContentionResolutionTimer is delayed by UE-eNB RTT in IoT NTN.

- Any enhancements on (N)PRACH resource selection in IoT NTN will not be pursued in Rel-17.

- An offset equal to UE-eNB RTT is added to the formula used for calculating the (UL) HARQ RTT timer in IoT NTN.

- Support UE-specific TA reporting using MAC CE in Msg3/Msg5 for IoT NTN.

- For IoT NTN, UE specific TA reporting during RACH procedure (MSG3/MSG5) in RRC IDLE is enabled/disabled by SI, similar with NR NTN.

- Support TA reporting in RRC connected mode in IoT NTN.

- UE-specific TA report uses MAC CE.

- Support event-triggered for TA reporting in connected mode. Wait for NR NTN agreements for other triggers.

- On how to extend RLC t-Reordering in IoT NTN, wait for NR NTN agreements and see if they can be reused.

- Don’t change the L2 buffer requirement for IoT NTN (assume the network may need to limit the bit rate in order to not exceed L2 buffer).

- The PDCP discardTimer should be extended to support eMTC over NTN.

- If PDCP discardTimer is agreed to be extended to support eMTC over NTN, how to extend the timer value can wait for the conclusion for RLC t-reordering timer.

- The ra window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset)

RAN2#115-e Agreements:

- Start of ra-ResponseWindow is delayed by an offset. Postpone discussion on the offset value until further agreements regarding RACH are made in RAN1.

- If the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN.

- Start of mac-ContentionResolutionTimer is delayed by an offset, (assumed equal to UE-eNB RTT). This can be revisited if RAN1 decides something that requires to change this.

- If the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the mac-ContentionResolutionTimer for IoT NTN.

- From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to eNB implementation to ensure sufficient time on UE side for the Msg3 transmission for IoT NTN.

- RAN2 assumes that TA information (FFS what) reporting by the UE on network enabling will be needed in IoT NTN. Expect RAN1 need to progress on this, and can maybe reuse NR NTN progress. FFS in which message this is provided.

- UE-eNB RTT is taken into account when calculating the (UL) HARQ RTT timer.

- RAN2 assumes that sr-ProhibitTimer need to be extended. Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

- From RAN2’s perspective, delayed start of pur-ResponseWindowTimer with UE-eNB RTT can be supported. This can be revised if RAN1 finds issues to support PUR that are not small.

- pur-ResponseWindowSize is not extended for IoT NTN.

- SPS is supported without modification for IoT NTN.

- RAN2 confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN.

- Do not extend the PDCP discardTimer for NB-IoT over NTN.

- FFS whether to extend the PDCP discardTimer for eMTC over NTN.

- Do not extend PDCP t-Reordering for IoT NTN.

## T1.3 Control plane impacts - TA and Mobility related

RAN2#116-e Agreements:

- The AS layer indicates to NAS layer all of the received TACs for the selected PLMN.

- For quasi-earth fixed cell, UE should start measurements on neighbour cells before the broadcast stop time of the serving cell, i.e the time when the serving cell stops covering the current area, and the exact time to start measurements (inter and intra-frequency) is up to UE implementation. FFS to what extent this need to be covered in the TS.

- Location-assisted cell reselection (e.g. as for NR NTN) is not supported for IoT NTN in rel 17.

- The use of hard TAC or soft TAC is up to network implementation in earth-fixed and earth-moving cells.

- Relaxed monitoring further enhancements are not considered for IoT NTN in rel-17.

RAN2#115-e Agreements:

- Cell selection / reselection procedures for NB-IoT and LTE-M in TN is the baseline in NB-IoT/LTE-M NTN.

- RAN2 assumes that Satellite assistance information, e.g. for cell selection reselection, for serving cell is provided to UE.

- Wait for the progress in RAN1 before discussion on whether satellite assistance information is broadcast in a separate information block.

- The timing information on when a cell is going to stop serving the area is broadcast at least for the quasi-earth fixed case. FFS details.

- The network may broadcast more than one TAC per PLMN in a cell, which is up to network implementation.

- The UE determines the Tracking Area based on the broadcast information (the use of other information is not excluded).

- When the network stops broadcasting a TAC, the UE needs to know it. FFS how this is done.

- UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area.

- Rel-16 LTE CHO mechanism is supported for LTE-M devices in IoT NTN. FFS which CE Mode(s) to apply

- No procedural update is required to support connected mode mobility for LTE-M.

- Rel-16 RLF / connection re-establishment mechanisms are supported in IoT NTN assuming that minor adjustments to UE specific timers and constants would be sufficient.

- FFS if Satellite assistance information for neighbour cell(s) is provided to UE for cell selection/reselection (justification would be needed).

- The value range for parameter t304 is not extended with larger values.

- Send an LS to RAN4 to inform that RRM impacts for supporting CHO should be taken into consideration.

- Postpone the discussion on whether specific timers and constants for RLF and RRC connection re-establishment procedures require extended value range and/or new behaviour till next meeting.

- System information update notification procedure is not used to inform TAC updates, at least for TAC additions (FFS removals)

## T1.5 Control plane – Other

RAN2#116-e Agreements:

- The serving cell ephemeris information (used for L1 pre-compensation) is signalled in a new SIB, which is NTN specific.

- Update to serving cell ephemeris information does not affect the system information value tag and does not trigger System information modification procedure. How to trigger re-read of this information is FFS. FFS if the UE shall reacquire the new SIB when SI update is triggered.

- Updates to serving cell ephemeris information are not bound to the BCCH modification period.

- The timing information on when a serving cell is going to stop serving the area is broadcast in the same SIB as the ephemeris information.

- Broadcast of the timing information on when a serving cell is going to stop serving the area is only applicable to quasi earth fixed cell (not to moving cell).

- No enhancement to R16 RLF and RRC connection Re-establishment procedures are introduced in R17.  (this does not include handling of UL synchronisation loss which is FFS and does not include non continuous coverage).

- No extension to timers and constants is required for RLF and RRC connection Re-establishment.

- No need to extend the 10 s delay for actions upon reception of RRCConnectionRelease in NB-IoT.

- It is feasible to use the legacy barring bit to block legacy UEs, and it is possible to have a new bit that assumes the functionality of the old bit. It is FFS if it is needed to use the barring bit or whether other mechanism can be assumed (new band etc).

- No enhancement to R16 CHO are introduced in R17.

RAN2#115-e Agreements:

None

## Contact Information

| Contact Name | Company | Email |  |
| --- | --- | --- | --- |
| Rene Faurie | Eutelsat | rfaurie-LS AT sfr DOT fr |  |