3GPP TSG-RAN WG2 Meeting #116 electronic	                          R2-210xxxx

1th November – 12th November 2021

Agenda item:	9.1.3

Source:	ZTE (email discussion rapporteur)

Title:	Report of [Post116-e][311][NBIOT/eMTC R17] NB-IoT carrier selection

Document for:	Discussion and Decision

Introduction

This document is the report of the offline email discussion “[Post116-e][311][NBIOT/eMTC R17] NB-IoT carrier selection”, as indicated below:

[Post116-e][311][NBIOT/eMTC R17] NB-IoT carrier selection (ZTE)

Scope: open issues and solution details

Intended outcome: report to the next meeting

Deadline: long (December 17th, 0900 UTC)

Contact information

Please provide your contact information when feedback:

| Company | Contact Name | Email |
| --- | --- | --- |
| ZTE | Ting Lu | lu.ting@zte.com.cn |

Discussion

In RAN2#116 e-meeting, RAN2 has achieved the following agreements:

Agreements: - DRX is not used a criterion that needs to be explicitly considered for paging carrier selection. - Option 1c with Alt2 (fallback when cell change) is supported.

Based on the previous meeting discussion, the description of Option 1c with Alt2 is as following:

- Option 1c: Network enables UE to select a Rel-17 paging carrier by providing the “coverage level” information (Rmax/CEL) for the carrier selection to the UE in dedicated signaling.

    - Alt2: fallback to legacy carrier selection scheme when cell change.

In this email discussion, the details of Option 1c are discussed.

The following procedure flow for Option 1c is copied from [R2-2110191] for reference:

Information assigned via dedicated signalling

According to the discussion in the previous meetings, for Option 1c, the eNB would provide a coverage level related information to the UE during RRC connection release procedure. As mentioned in [R2-2110110], such information can be a certain Rmax/NPDCCH repetitions for decoding NPDCCH. Moreover, such information already can be estimated by the eNB based on UE’s service quality in connected mode and sent to MME/AMF (in UEPagingCoverageInformation-NB-> npdcch-NumRepetitionPaging) for subsequent PAGING.

Rmax in dedicated signalling

Q1-01: Do companies agree that eNB would provide a Rmax information, e.g., npdcch-NumRepetitionPaging, to the UE when UE is released to idle?

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Yes | We think it can also be allowed to provide Rmax information in Msg4 in EDT or PUR procedure. Moreover, we have assumption that the Rmax/npdcch-NumRepetitionPaging in RRC connection release message have same value range as that for Rmax/npdcch-NumRepetitionPaging in R17 paging carrier list in SIB. |

Conclusion:

Enable/disable indication in dedicated signalling

In [R2-2110149], company suggests that for UE in extended coverage, it can be disabled CEL-based paging carrier selection via RRC release message.

Q1-02: Do companies agree that UE can be enabled/disabled CEL-based paging carrier selection when UE is released to idle?

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Yes | The provision of Rmax information, e.g., npdcch-NumRepetitionPaging in RRC connection release message can act as an implicit enable indication. Without Rmax information in RRC connection release message, the UE is disabled CEL-based paging carrier selection. That means it should still use legacy paging carrier selection scheme. |

Conclusion:

R17 carrier list configuration in SIB

According to the discussion in previous meetings, for Option 1c, the possible contents in the R17 carrier list configuration in SIB has been mentioned as following:

- Each R17 paging carrier can be configured with a npdcch-NumRepetitionPaging. One or more R17 paging carriers can be configured with the same npdcch-NumRepetitionPaging that means these paging carriers are corresponding to a same coverage level. Different coverage levels can be supported with different npdcch-NumRepetitionPaging.

- A common nB value (which may be different from (e.g., larger than) the cell nB) can be configured to all R17 paging carriers with the same npdcch-NumRepetitionPaging. (That means this parameter is coverage specific).

- A common coverage specific DRX cycle can be configured to all R17 paging carriers with the same npdcch-NumRepetitionPaging. Or the simplest, it's no need to configure coverage specific DRX cycle and then the cell-specific default paging cycle can still be applied.

- A common coverage specific ue-SpecificDRX-CycleMin (which may be different from (e.g., smaller than) the cell specific ue-SpecificDRX-CycleMin) can be configured to all R17 paging carriers with the same npdcch-NumRepetitionPaging.

Rmax for R17 paging carrier in SIB

Q2-01: Do companies agree that one or more R17 paging carriers can be configured with a same Rmax parameter, e.g., npdcch-NumRepetitionPaging, that means these paging carriers are corresponding to a same coverage level?

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Yes | We assume this is just for Stage-2 agreement. For stage-3, it can be further discussed. One possible way is to always configure npdcch-NumRepetitionPaging for each R17 coverage-based paging carrier and mention one or more R17 paging carriers can be configured with a same npdcch-NumRepetitionPaging value. Or another way is to configure a R17 paging carrier list for each configured npdcch-NumRepetitionPaging value. |

Conclusion:

Different CELs for R17 paging carrier

During the email discussion, some companies think different coverage levels can be allowed. However, as some companies think that it’s likely to be very few coverage-based paging carriers in a cell, they think the realistic assumption seems to be that there are only a few different coverage levels and generally only very few paging carriers, e.g., 1, for each given coverage level.

In [R2-2110149], company also suggest that for simple UE and network implementations, division of paging carriers into two subsets of carriers each mapped to normal and maximum coverage is considered as basis for paging carrier selection. And, optionally the Rel-17 paging carriers can be further divided into two sub-groups based on CEL.

Q2-02: Do companies agree that different coverage levels (e.g., more than one coverage levels) in R17 paging carrier list would be allowed? If answer is “Yes”, please indicate preferred number.

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Yes | With trade-off between flexibility and complexity, we think 3 or 4 Rmax values can be allowed to configure in Rel-17 paging carrier list in SIB to achieve 3 or 4 coverage levels, e.g., 3 or 4 sub-groups of Rel-17 paging carrier. |

Conclusion:

Value range for Rmax in SIB

In [R2-2110149], company suggest for UE in deep coverage, it can be disabled CEL-based paging carrier selection via RRC release message. That means it should still use legacy paging carrier selection scheme. For UE in good or normal coverage, it can be enabled CEL-based paging carrier selection via RRC release message.

Per rapporteur’s understanding, company may have assumption that network can only configure R17 paging carriers for good or normal coverage. That may further means the value range for Rmax, e.g., npdcch-NumRepetitionPaging can be less than r2048.

Q2-03: Companies are invited to give your preference for the value range of the configured npdcch-NumRepetitionPaging for R17 paging carrier (list):

- Alt1: Same as legacy, e.g., npdcch-NumRepetitionPaging-r17   ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}

- Alt2: With limited values, e.g., npdcch-NumRepetitionPaging-r17   ENUMERATED {r1, r8, r32, r64}, the possible values can be discussed later

- Alt3: Other

| Company | Preferred alternative | Additional comment(s) |
| --- | --- | --- |
| ZTE | Alt1 | We are fine with either Alt1 or Alt2 but prefer Alt1. For Alt1, a large value range can be provided. This is beneficial to configuration flexibility. Maybe some restriction (e.g., at most 3 or 4 coverage levels are supported) can be mentioned. For Alt2, there is implicit assumption that for UE in deep coverage (e.g., the required repetition number is larger than r64), they cannot use R17 paging carrier. This is acceptable but a bit restricted. |

Conclusion:

nB for R17 paging carrier

In [R2-2110475], company gives explanation that in NB-IoT, the low values of the paging occasion density (nB) were motivated by the potentially large number of NPDCCH repetitions and the desire to avoid PO overlapping between different UEs. Thus, with R17 carriers being configured with lower number of NPDDCH repetition, it could make sense to allow for larger paging occasion density (nB) on the R17 paging carriers. This is also common understanding from some other companies. According to the previous discussion, different alternatives have been mentioned, e.g., nB configuration on carrier basis or commonly for carriers with the same number of NPDCCH repetitions.

Q2-04: Companies are invited to give your preference for nB configuration for R17 paging carrier (list):

- Alt1: coverage specific nB, which means a common nB value is configured to the R17 paging carriers with same npdcch-NumRepetitionPaging

- Alt2: carrier specific nB, which can be configured differently for each R17 paging carrier.

- Alt3: no nB configuration. That means cell specific nB would be applied.

| Company | Preferred alternative | Additional comment(s) |
| --- | --- | --- |
| ZTE | Alt1 |  |

Conclusion:

DRX cycle for R17 paging carrier

In previous discussion, some companies have suggested to allow carrier specific DRX cycle configuration as the number of NPDCCH /NPDSCH repetitions will be lower and this could provide more paging flexibility to the NW.

However, as mentioned in [R2-2110475], there are also arguments that this would mean that some UEs (depending on the value of their UE specific DRX if configured) would monitor paging and perform measurements more often and thus impact the power consumption.

Q2-05: Companies are invited to give your preference for DRX cycle configuration for R17 paging carrier (list):

- Alt1: coverage specific DRX cycle, which means a common DRX cycle value is configured to the R17 paging carriers with same npdcch-NumRepetitionPaging

- Alt2: carrier specific DRX cycle, which can be configured differently for each R17 paging carrier.

- Alt3: no DRX cycle configuration. That means cell specific default DRX cycle would be applied.

| Company | Preferred alternative | Additional comment(s) |
| --- | --- | --- |
| ZTE | Alt1 | RAN2 has agreed that “DRX is not used a criterion that needs to be explicitly considered for paging carrier selection”. Therefore, we think it’s no need to configured carrier specific DRX cycle. For configuration flexibility, we still a bit prefer to support coverage specific DRX cycle. Although NW would configure this value to correspond to the related coverage level, as NW cannot predict the traffic frequency of the UEs that selects this coverage level, we assume generally NW would not configure this value too much smaller than the cell specific default DRX cycle. For Alt1, we assume no additional UE behavior needs to be specified as UE can uniquely determine this DRX cycle as long as a paging carrier is selected based on the assigned Rmax. This coverage specific DRX cycle can be used as default DRX cycle instead of the previous cell specific default paging DRX cycle. |

Conclusion:

ue-SpecificDRX-CycleMin for R17 paging carrier

As mentioned in [R2-2110475], the reason for the introduction of cell specific parameter ue-SpecificDRX-CycleMin is to avoid CSS overlapping for a given UE if the large numbers of NPDCCH repetitions is needed. Therefore, it seems reasonable to allow smaller ue-SpecificDRX-CycleMin on the R17 paging carriers. In addition, as the ue-SpecificDRX-CycleMin is directly linked to the number of NPDCCH repetitions, there is no reason to have different values for R17 paging carriers with the same number of NPDCCH repetitions.

Q2-06: Companies are invited to give your preference for ue-SpecificDRX-CycleMin configuration for R17 paging carrier (list):

- Alt1: coverage specific ue-SpecificDRX-CycleMin, which means a common ue-SpecificDRX-CycleMin value is configured to the R17 paging carriers with same npdcch-NumRepetitionPaging.

- Alt2: carrier specific D ue-SpecificDRX-CycleMin, which can be configured differently for each R17 paging carrier.

- Alt3: no ue-SpecificDRX-CycleMin configuration. That means cell specific ue-SpecificDRX-CycleMin would be applied.

| Company | Preferred alternative | Additional comment(s) |
| --- | --- | --- |
| ZTE | Alt1 | As the npdcch-NumRepetitionPaging could be configured with relatively small value for the carriers corresponding to a certain coverage level, in order to avoid CSS overlapping, we need to introduce the corresponding coverage specific ue-SpecificDRX-CycleMin. This is out of the reason similar to that for introducing cell specific ue-SpecificDRX-CycleMin. |

Conclusion:

PagingWeight

In [R2-2110191], company think if UE selected more than one carrier based on coverage, paging weight can be used for network to perform load balancing when determining one from these selected carriers.

In [R2-2110695], company think that it is highly unlikely there would be more than one paging carrier for a specific coverage level lower than maximum coverage level in the cell. For that reason, if a coverage-based paging carrier is more appropriate for a UE then it should always select that carrier. Therefore, company suggest that paging Weight is not used for selecting coverage-based paging carrier.

Q2-07: Do companies agree that paging weight can still be used in coverage-based paging carrier selection?

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Yes | We firstly think assuming only one paging carrier for a specific coverage level is too restricted. For Option 1c, we assume the legacy calculation of the PF, i_s, PNB, wg, and the (final determination of) NB-IoT paging carrier can be reused (as much as possible). Compared with the legacy scheme, the main difference is that a new carrier list (CEL-based paging carrier list) would be used instead of the legacy carrier list. Therefore, we see no clear motivation to exclude the use of this paging weight parameter for R17 paging carrier selection. The related changes may be very similar as that for GWUS support. For example, one of the possible change might be as following : - W: Total weight of all NB-IoT paging carriers, i.e. W = W(0) + W(1) + … + W(Nn-1). If UE monitors GWUS according to clause 7.5.1, Total weight of all NB-IoT paging carriers configured with GWUS. If UE selects paging carrier based coverage enhanced level and UE does not monitor GWUS according to clause 7.5.1, W is the total weight of all NB-IoT paging carriers configured within PCCH-Config-NB-r17 and with the value of npdcch-NumRepetitionPaging-r17 that is equal to the npdcch-NumRepetitionPaging-r17 assigned to UE; If UE selects paging carrier based coverage enhanced level and UE monitors GWUS according to clause 7.5.1, W is the total weight of all NB-IoT paging carriers configured within PCCH-Config-NB-r17 and with the value of npdcch-NumRepetitionPaging-r17 that is equal to the npdcch-NumRepetitionPaging-r17 configured to UE, and configured with GWUS. |

Conclusion:

Mixed operation

In [R2-2110695], company mentioned that in legacy signaling, up to two downlink non-anchor carrier lists can be broadcasted if cell supports mixed operation mode, e.g., dl-ConfigList-r14 list and dl-ConfigListMixed-r15. Only a UE that supports mixedOperationMode is required to use the dl-ConfigListMixed-r15 in addition to the dl-ConfigList-r14.

Company further think that coverage-based paging carrier selection should be possible to use by a UE without the need to support mixedOperationMode. And company suggest that coverage-based paging carrier configuration shall be separate for non-mixed operation mode and for mixed operation mode. Only a UE supporting mixedOperationMode and coverage-based paging carrier selection is required to use mixed operation mode coverage-based configuration.

Q2-08: Do companies agree that coverage-based paging carrier configuration shall be separate for non-mixed operation mode and for mixed operation mode?

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Yes | We agree that coverage-based paging carrier selection should be possible to use by a UE without the need to support mixedOperationMode. Therefore, the R17 UEs with different mixed operation capabilities may finally maintain different CEL-based paging carrier lists. But this does not necessarily mean that two carrier lists configuration need to be introduced in R17. |

Conclusion:

Which SIB

Q2-09: Companies are invited to provide your choice on the following alternatives for providing the configuration of R17 carrier list?

- Alt1: Extension in SIB22-NB

- Alt2: A new SIB

- Alt3: Other

| Company | Preferred alternative | Additional comment(s) |
| --- | --- | --- |
| ZTE | Alt1 | Generally, as we assume the total number of paging carriers for R17 and R17 earlier in SIB configuration would not exceed 16, we think it’s feasible and enough to extend SIB22-NB to include R17 paging carrier list configuration. |

Conclusion:

ASN.1 example

In [R2-2110110], one example for R17 carrier list configuration is given as following:

Alt1:

SystemInformationBlockType22-NB information element -- ASN1START SystemInformationBlockType22-NB-r14 ::= SEQUENCE { dl-ConfigList-r14 DL-ConfigCommonList-NB-r14 OPTIONAL, -- Need OR ul-ConfigList-r14 UL-ConfigCommonList-NB-r14 OPTIONAL, -- Need OR // omit the unchanged part// } DL-ConfigCommonList-NB-r14 ::= SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigCommon-NB-r14 UL-ConfigCommonList-NB-r14 ::= SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 UL-ConfigCommonListTDD-NB-r15 ::= SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommonTDD-NB-r15 DL-ConfigCommon-NB-r14 ::= SEQUENCE { dl-CarrierConfig-r14 DL-CarrierConfigCommon-NB-r14, pcch-Config-r14 PCCH-Config-NB-r14 OPTIONAL, -- Need OR ..., [[ wus-Config-r15 WUS-ConfigPerCarrier-NB-r15 OPTIONAL -- Cond WUS ]], [[ gwus-Config-r16 WUS-ConfigPerCarrier-NB-r15 OPTIONAL -- Cond GWUS ]], [[ pcch-Config-r17 PCCH-Config-NB-r17 OPTIONAL -- Cond pcch-Config-r14 ]] } PCCH-Config-NB-r14 ::= SEQUENCE { npdcch-NumRepetitionPaging-r14 ENUMERATED { r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1} OPTIONAL, -- Need OP pagingWeight-r14 PagingWeight-NB-r14 DEFAULT w1, ... } PCCH-Config-NB-r17 ::= SEQUENCE { npdcch-NumRepetitionPaging-r17 ENUMERATED { r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1} OPTIONAL, -- Need OP pagingWeight-r17 PagingWeight-NB-r14 DEFAULT w1, defaultPagingCyclePerRmax-r17 ENUMERATED {rf32, rf64, rf128, rf256, rf512, rf1024} OPTIONAL, -- Need OR nB-PerRmax-r17 ENUMERATED { fourT, twoT, oneT, halfT, quarterT, one8thT, one16thT, one32ndT, one64thT, one128thT, one256thT, one512thT, one1024thT, spare3, spare2, spare1} OPTIONAL, -- Need OR ... } PagingWeight-NB-r14 ::= ENUMERATED {w1, w2, w3, w4, w5, w6, w7, w8, w9, w10, w11, w12, w13, w14, w15, w16} // omit the unchanged part// -- ASN1STOP SystemInformationBlockType22-NB field descriptions. …………... pcch-Config Configure the PCCH parameters for the non-anchor DL carrier. If pcch-Config-r17 is configured in a cell, the UE supporting Coverage Enhanced Level based paging carrier selection will only select the carrier with pcch-Config-r17 configured for paging. The UE not supporting Coverage Enhanced Level based paging carrier selection does not select the carrier with pcch-Config-r17 configured for paging.. Conditional presence; Explanation. pcch-Config-r14; This field is optionally present, Need OR, if the field pcch-Config-r14 is absent. Otherwise the field is not present..

In [R2-2110695], another example for R17 carrier list configuration is given as following:

Alt2:

-- ASN1START

SystemInformationBlockType22-NB-r14 ::= SEQUENCE {

dl-ConfigList-r14 DL-ConfigCommonList-NB-r14 OPTIONAL, -- Need OR

ul-ConfigList-r14 UL-ConfigCommonList-NB-r14 OPTIONAL, -- Need OR

pagingWeightAnchor-r14 PagingWeight-NB-r14 OPTIONAL, -- Cond pcch-config

nprach-ProbabilityAnchorList-r14 NPRACH-ProbabilityAnchorList-NB-r14 OPTIONAL, -- Cond nprach-config

lateNonCriticalExtension OCTET STRING OPTIONAL, ...,

[[ mixedOperationModeConfig-r15 SEQUENCE {

dl-ConfigListMixed-r15 DL-ConfigCommonList-NB-r14 OPTIONAL, -- Cond dl-ConfigList

ul-ConfigListMixed-r15 UL-ConfigCommonList-NB-r14 OPTIONAL, -- Cond ul-ConfigList

pagingDistribution-r15 ENUMERATED {true} OPTIONAL, -- Need OR

nprach-Distribution-r15 ENUMERATED {true} OPTIONAL -- Need OR

} OPTIONAL, -- Need OR

ul-ConfigList-r15 UL-ConfigCommonListTDD-NB-r15 OPTIONAL -- Cond TDD

]],

[[ 	dl-ConfigListWithCoverage-v17xy	DL-ConfigCommonList-NB-v17xy OPTIONAL, -- Need OR

dl-ConfigListMixedWithCoverage-v17xy	DL-ConfigCommonList-NB-v17xy OPTIONAL -- Need OR

]]

}

--- other existing IEs omitted ----

DL-ConfigCommonList-NB-v17xy ::= SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigCommon-NB-r17

--- other existing IEs omitted ----

DL-ConfigCommon-NB-r17 ::= SEQUENCE {

dl-CarrierConfig-r17 DL-CarrierConfigCommon-NB-r14 OPTIONAL, -- Need OR

pcch-Config-r17 PCCH-Config-NB-r17 OPTIONAL, -- Need OR

wus-Config-r15 WUS-ConfigPerCarrier-NB-r15 OPTIONAL, -- Cond WUS

gwus-Config-r16 WUS-ConfigPerCarrier-NB-r15 OPTIONAL, -- Cond GWUS

...

}

PCCH-Config-NB-r17 ::= SEQUENCE {

carrierSpecificPagingCycle-r17 ENUMERATED {rf32, rf64, rf128, rf256, rf512, rf1024, spare2,

spare1}, OPTIONAL -- Need OP

carrierSpecific-nB-r17 ENUMERATED {fourT, twoT, oneT, halfT, quarterT, one8thT, one16thT,

one32ndT, one64thT, one128thT, one256thT, one512thT,

one1024thT, spare3, spare2, spare1}, OPTIONAL -- Need OP

npdcch-NumRepetitionPaging-r17 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512,

r1024, r2048, spare4, spare3, spare2, spare1},

carrierSpecificCoverageLevel-r17 RSRP-Range,

...

}

Q2-10: Companies are invited to give your preference on the following ASN.1 example alternatives. The choice might be used as the start point for further stage-3 discussion. No matter what the answer is, companies can further provide some high level suggestions on the ASN.1:

- Alt1

- Alt2

- Alt3 (Other)

| Company | Preferred alternative | Additional comment(s) |
| --- | --- | --- |
| ZTE | Alt1 | We think Alt1 has less signaling overhead. And with Alt1, the dl-ConfigList-r14 and dl-ConfigListMixed-r15 can be used as legacy. For Alt2, we worry about that, if new carrier lists are introduced, there may have impact on the description for determining the npdcch-CarrierIndex in NPRACH resource. In Alt1, the coverage specific ue-SpecificDRX-CycleMin and coverage specific NRSRP criteria/threshold (see our comments for Q3-02) are missing in PCCH-Config-NB-r17. In Alt1, the parameter npdcch-NumRepetitionPaging, default Paging Cycle and nB are currently configured per carrier, we assume additional restriction would be mentioned if we agree they should be coverage specific. Based on Alt1, one example for configuring coverage specific parameters can be as following: SystemInformationBlockType22-NB-r14 ::= SEQUENCE { // omit the unchanged part// ul-ConfigList-r15 UL-ConfigCommonListTDD-NB-r15 OPTIONAL -- Cond TDD ]], [[ PCCH-ConfigList-NB-r17 ::= SEQUENCE (SIZE(1..maxRmaxNumber-NB-r17)) OF PCCH-Config-NB-r17 OPTIONAL, -- Need OR ]] } // omit the unchanged part// DL-ConfigCommon-NB-r14 ::= SEQUENCE { // omit the unchanged part// [[ gwus-Config-r16 WUS-ConfigPerCarrier-NB-r15 OPTIONAL -- Cond GWUS ]], [[ PCCH-ConfigIndex-r17 INTEGER(1..maxRmaxNumber-NB-r17) OPTIONAL -- Cond PCCH-Config-r14 ]] } PCCH-Config-NB-r14 ::= SEQUENCE { // omit the unchanged part// } PCCH-Config-NB-r17 ::= SEQUENCE { npdcch-NumRepetitionPaging-r17 ENUMERATED { r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1} OPTIONAL, -- Need OP pagingWeight-r17 PagingWeight-NB-r14 DEFAULT w1, defaultPagingCycle-r17 ENUMERATED {rf32, rf64, rf128, rf256, rf512, rf1024} OPTIONAL, -- Need OR ue-SpecificDRX-CycleMin-r17 ENUMERATED {rf32, rf64, rf128, rf256, rf512, rf1024} OPTIONAL, -- Need OR nB-r17 ENUMERATED { fourT, twoT, oneT, halfT, quarterT, one8thT, one16thT, one32ndT, one64thT, one128thT, one256thT, one512thT, one1024thT, spare3, spare2, spare1} OPTIONAL, -- Need OR rsrpThreshold-r17 RSRP-Range, ... } // omit the unchanged part// |

Conclusion:

Determining suitability of assigned Rmax and paging carrier selection

NRSRP threshold

RAN2 has agreed UE metric for determining carrier suitability and selection is based on NRSRP.

In [R2-2110149], company suggest each group of Rel-17 paging carriers can be configured with a NRSRP threshold value to enable coverage-based paging carrier selection for fallback scenarios (e.g., when coverage level changes). In the fallback scenario, UE selects the paging carrier based on the estimated RSRP value from idle mode measurements against the configured threshold value. During email discussion [AT116-e][304] ([R2-2111394]), some other companies mentioned similar thoughts, e.g., the UE checks its serving NRSRP vs the threshold corresponding to its assigned CEL, and if suitable, selects a carrier in the list of carrier corresponding to its assigned CEL.

Per rapporteur’s understanding on the mentioned contributions and previous discussion, a simple way for determining suitability of assigned Rmax and selecting R17 paging carrier might be summarized as following:

- NRSRP criteria/threshold can be provided by network and one Rmax (npdcch-NumRepetitionPaging) maps to one and only one NRSRP criteria/threshold.

- UE compares its serving cell NRSRP with the NRSRP criteria/threshold which is corresponding to UE’s assigned Rmax (npdcch-NumRepetitionPaging):

  - If UE’s serving cell NRSRP is equal to or higher (better) than the NRSRP criteria/threshold, the UE can determine its coverage doesn’t change worse. UE can keep using the assigned Rmax and select a R17 carrier in the carrier list corresponding to this assigned Rmax.

  - Otherwise (e.g., UE’s serving cell NRSRP is below the NRSRP criteria/threshold), the UE fallback to legacy carrier selection scheme.

Q3-01: Do companies agree to take the above way as baseline for determining suitability of assigned Rmax and paging carrier selection? If the answer is “No”, please indicate the main difference in your thinking.

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Yes | Fine with such basic way and further details can be discussed. A hysteresis or filter parameter may be additionally applied when performing comparison, as mentioned in below discussion. |

Conclusion:

During previous email discussion, some companies have mentioned such NRSRP criteria/threshold can be provided in SIB, e.g., corresponding to a coverage level (Rmax/npdcch-NumRepetitionPaging).

In [R2-2111394], company has mentioned another way. They do not think there is a need to assign a NRSRP value (threshold) per R17 paging carrier in system information broadcast, however such value should be provided to the UE when it is released to idle along with an hysteresis mechanism so that the UE can determine whether its coverage situation has changed since it was released to idle. e.g., if the UE measures an NRSRP value lower than the configured value for longer than hysteresis, it should consider itself in worse coverage than what network assumed during release and performs fallback.

Per rapporteur’s understanding, if NRSRP criteria/threshold is provided in RRC release message, it is UE specific and one-to-one mapping to the assigned Rmax. UE can directly compare its serving cell NRSRP with this NRSRP criteria/threshold. If NRSRP criteria/threshold is provided in SIB, UE needs to firstly determine a NRSRP criteria/threshold in SIB which is corresponding to UE’s assigned Rmax.

Q3-02: Companies are invited to indicate your preference on how to provide NRSRP criteria/threshold:

- Alt1: NRSRP criteria/threshold is provided in SIB and is coverage specific, which means a common NRSRP criteria/threshold value is configured to the R17 paging carriers with same npdcch-NumRepetitionPaging

- Alt2: NRSRP criteria/threshold is provided when UE is released to idle

- Alt3: Other

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Alt1 | We think Alt1 is enough and simple. For Alt2, for same assigned Rmax, NRSRP criteria/threshold can be different for different UEs. It’s a bit more flexible but no clear benefit can be seen. And signaling overhead would increase. |

Conclusion:

Hysteresis/longer averaging/timer for UE metric

RAN2 has agreed to use a hysteresis/longer averaging/timer for UE metric based on NRSRP. Details are FFS.

In [R2-2111394], company mentions that UE can determine whether its coverage situation has changed since it was released to idle. e.g., if the UE measures an NRSRP value lower than the configured value for longer than hysteresis. In [R2-2110694], company also mentions that hysteresis is needed to avoid frequent switching between paging carriers. This is so that UE does not switch paging carrier just based on one or two NRSRP measurements. In [R2-2110475], company mentions that the UE switches to the R17 carrier if the NRSRP is better than the configured threshold during 5 mn or one eDRX cycle if longer. Per rapporteur’s understanding, companies mentions a similar way to use a “hysteresis”. But this may be more like a filtering scheme, e.g., to compare NRSRP with the configured threshold over a period.

Q3-03: Do companies agree that a fixed (e.g., one eDRX cycle) or a configurable “hysteresis”, e.g., a timer period can be applied when UE compares its serving cell NRSRP with the NRSRP criteria/threshold?

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Yes | We agree it’s beneficial to let the suitability checking based on NRSRP continue for a time period to ensure a stable evaluation result. Such time period can be fixed. It’s simple. It’s also acceptable to us to configure such time period along with the NRSRP criteria/threshold in SIB. |

Conclusion:

In [R2-2110694], company mentions another kind of hysteresis, e.g., a hysteresis similar to that applied to serving cell that can prevent frequent switching between different paging carriers. But such hysteresis will cause the UE to ‘stick’ to the selected paging carrier for longer period. Company further indicate negative impact on paging performance due to hysteresis can be minimized by ensuring there is reasonable headroom e.g., the coverage-based paging carrier can still be usable when the serving cell is 6dB lower than the required coverage level to select the paging carrier. Larger the headroom, the lower the benefit of coverage-based paging carrier.

Per rapporteur’s understanding, here company suggest an “offset” (headroom) to the NRSRP criteria/threshold.

Q3-04: Do companies agree to further apply an “offset” (headroom) to the configured NRSRP criteria/threshold?

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | No | We cannot see clear benefit of such “offset” (headroom). If network can assume such “offset”, network can directly apply this offset to the previous “absolute” NRSRP criteria/threshold when configure it. E.g., network may originally wanted to set NRSRP criteria/threshold to -110dB, if it can assume an offset 6dB, network can directly configure NRSRP criteria/threshold to -116dB. Moreover, we don’t think a hysteresis similar to that applied to serving cell is needed. The possible way of such hysteresis may be to apply two different NRSRP criteria/thresholds for switching from R17 carrier to legacy carrier and switching from legacy carrier to R17 carrier. We think the intention of reducing ping-pong may be not so clear. In this R17 paging carrier selection, the main point should be to keep consistent paging carrier selection between UE and network. Therefore, before each PO, it’s more important to guarantee UE can try to keep using the R17 carrier determined by the assigned Rmax (as network would start paging on this carrier). We think an “absolute” NRSRP criteria/threshold is enough and a conservative configuration for this NRSRP criteria/threshold in SIB would be helpful. The only issue we can see may be that during paging retransmission, e.g., UE already fallback to a legacy paging carrier, if later UE is quickly back to R17 paging carrier because radio quality change better, it seems UE may miss the paging. However, as we can assume network may send paging on both R17 paging carrier and legacy paging carrier when it retransmit the paging (a possible network implementation strategy to handle CEL change case), this issue can be naturally resolved. |

Conclusion:

In [R2-2110475], company further mentions an adaptation of the cell reselection principle, e.g., the UE does not switch paging carrier if it has stayed less than [xx] seconds on the carrier. Also, it seems preferable that the UE does not switch carrier within a PTW as this may have impact on the paging escalation mechanism.

Q3-05: Do companies agree to introduce a “restriction” that the UE does not switch paging carrier if it has stayed less than [xx] seconds on the carrier or within a PTW?

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | Maybe No? | As our comments for Q3-04, before each PO, UE would perform suitability checking of assigned Rmax and paging carrier selection. It’s more important to guarantee UE can try to keep using the R17 carrier determined by the assigned Rmax (as network would start paging on this carrier). A conservative configuration for NRSRP criteria/threshold in SIB would be helpful, e.g., as long as the coverage is not too bad, UE could continue to use the selected R17 paging carrier. We guess the proposed scheme in [R2-2110475] may be to let UE wait for a time period before performing suitability checking? We cannot see the necessity but worry about the complexity. Or maybe this can be purely left to UE implementation. |

Conclusion:

UE report

RAN2 still FFS whether to introduce new UE report and/or whether to mandate support of existing Msg5 reporting.

In [R2-2110694], per rapporteur’s understanding, company suggests UE can consider the measured NRSRP over many paging occasions (over longer period than 10s of seconds?) and determine an initial threshold for coverage-based paging carrier selection. UE can further determine a coverage-based paging carrier compatible with the measurement (and the initial threshold?) and report the index of this paging carrier to the network. Furthermore, network indicates to the UE whether it is allowed to use this coverage-based paging carrier that the UE indicates to the network.

In [R2-2110475], company mentions that UE report of its ‘preferred’ paging carrier does not bring any benefit compared to the reporting of the coverage status of the UE as any UE selection will be based on either NRSRP or estimated Rmax. In addition, the eNB may want to be a bit conservative or take other information into account (e.g. interferences on the downlink carriers) in the configuration of the selection criteria. Therefore, company think it’s no need to introduce new UE reporting to assist in the configuration of the paging carrier selection criteria.

In [R2-2109912] and [R2-2111113], companies think Msg5 based reporting (e.g., NRSRP and NRSRQ of serving cell measurement result in Msg5) can be reused or can be made mandatory.

Q4: Do companies agree not to introduce UE report for a preferred/selected coverage-based paging carrier? If Yes, that means at least existing Msg5 reporting can be re-used. Companies can further indicate whether to make the report in Msg5 mandatory.

| Company | Yes/No | Additional comment(s) |
| --- | --- | --- |
| ZTE | No | We have sympathy with the analysis in [R2-2110475] and cannot see the obvious benefit of such UE report for a preferred/selected coverage-based paging carrier. We think suitable NRSRP criteria/threshold configuration in SIB and network implementation strategy for paging retransmission can be helpful enough. We are fine to make the legacy Msg5 report mandatory. But just note this is no use in EDT and PUR procedure (we assume Msg4 in EDT or PUR procedure can also be used to assign Rmax information to UE). |

Conclusion:

Other aspects

Q5: Any other details need to be addressed/clarified?

| Company | Additional comment(s) |
| --- | --- |

Conclusion:

Conclusion

TBD

References

[1] R2-2109911, Report of email discussion [302] [NBIOT/eMTC R17] Carrier Selection, Ericsson, RAN2#116e

[2] R2-2109912, Analysis of Rmax based solution and carrier-based solution, Ericsson, RAN2#116e

[3] R2-2110110, Option1c for CEL-based paging carrier selection, ZTE Corporation, Sanechips, RAN2#116e

[4] R2-2110148, Paging strategy impacts for coverage based paging carrier selection, Nokia, Nokia Shanghai Bells, RAN2#116e

[5] R2-2110149, Network configuration for paging carrier selection based on coverage level, Nokia, Nokia Shanghai Bells, RAN2#116e

[6] R2-2110191, Further discussion on enhanced paging carrier selection, NEC Corporation, RAN2#116e

[7] R2-2110475, Discussion on coverage based paging carrier, Huawei, HiSilicon, RAN2#116e

[8] R2-2110694, Further consideration on open issues for coverage-based paging carrier selection, Qualcomm Incorporated, RAN2#116e

[9] R2-2110695, Signalling for coverage-based paging carrier selection, Qualcomm Incorporated, RAN2#116e

[10] R2-2111113, Discussion on details of paging carrier selection options, MediaTek Inc., RAN2#116e

[11] R2-2111394, Report of [AT116-e][304][NBIOT/eMTC] NB-IoT carrier selection, ZTE Corporation, RAN2#116e