3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-2xxxxx

Electronic Meeting, Jan 17-25, 2022

Agenda item:	xx

Source:	CMCC

Title:	Report for open issues for L2M

Document for:	Discussion and Decision

# Introduction

- [Post116-e][879][SON/MDT]  Running R17 38.314 (CMCC)

Scope: building the whole running CR, including the agreed changes and agreements from this meeting.

Including the definition of the measurement of excess packet delay for NR

Intended outcome: running CR

Phase 1: Comments on the open issues

Deadline for Phase 1 : Dec 10, 0900UTC

Phase 2: Check 38.314 running CR

Deadline for Phase 2: December 17th, 0900 UTC

Contact list

| Company and Name | Email |
| --- | --- |
| Huawei, HiSilicon (Jun Chen) | jun.chen@huawei.com |

# Agreements in RAN2#116-e

R2-2111534	38.314 CR to PRB Usage for MIMO	CMCC	CR	Rel-17	38.314	16.4.0	0019	-	B	NR_ENDC_SON_MDT_enh-Core

=>	CR is agreed in principle

=>	Will be merged into big R17 38.314 CR when available.

R2-2110642	Discussion on L2M	Huawei, CMCC, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core	R2-2108567

=>	Introduce packet “reliability” measurement for D1, i.e. reuse the LTE metric.

R2-2111300	Report of email discussion [AT116e][852][SON/MDT] Packet “reliability” measurement for D1 (Huawei)

Agreements

1 The new delay measurement can be called excess packet delay for NR.

2 FFS: the definition of the measurement of excess packet delay for NR is:

-	it represents the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs received. The delay for each packet is calculated from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant)

3 The network can collect the measurement excess packet delay for NR from the UE.

4 LTE excess packet delay reporting can be used as a baseline, and details can be further discussed.

# Discussion

According to the agreement, the 38.314 running CR will capture two measurements:

1) PRB usage in R2-2111534, which is already stable and will be merged to the running CR.

2) excess packet delay for NR, which still has one open FFS on the definition.

Rapporteur suggest to further discuss on the FFS during Phase 1 and then draft running CR during Phase 2.

There are mainly two options for the definition of excess packet delay for NR, details can be found in [1,2]:

Option a (Align with NR D1 delay): It represents the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs received. The delay for each packet is calculated from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant).

Option b (Reuse LTE definition): It represents the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs transmitted (from UE’s PDCP to UE’s RLC). The delay from packet arrival at PDCP upper SAP until the packet starts to be delivered to RLC.

In order to fully understand the two options, some background information is provided.

For option a, it was seen as an enhancement of D1 measurement (UL PDCP Packet Average Delay per DRB per UE) and URLLC services are the main use cases. D1 measurement was introduced in Rel-16 NR SON/MDT, and it is for QoS verfication of MDT and the QOS monitoring.

For option b, the LTE excess packet delay was introduced in Rel-13 (LTE_eMDT2), and the measurement is for MMTEL voice and video traffic. The basic idea is that UL PDCP queuing delay measurement should be performed per QCI per UE and should reflect the packet delay observed at UE’s PDCP layer only (from packet arrival at PDCP upper SAP until the packet starts to be delivered to RLC).

Q1: Companies are invited to share views on which option is preferred?

| Company | Option a/b | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Option a | As desribed above, option b was introdued because of MMTEL voice and video traffic. However, we think the new delay measurement is mainly about URLLC services. In addition, D1 is part of the RAN part of UL paceket delay, and we see the benefits of collecting more information about D1 so that the network can have more means to identify delay problems. The delay definition in option b is different from D1, and it may help address delay problem for MMTEL voice and video traffic, but we do not think it will bring much help regarding URLLC services. |

Q2: In case companies have any further comments on excess packet delay for NR, please share comments here.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | If option a is to be selected, we think there are the following differences compared with LTE excess packet delay reporting (we commented on it in the report R2-2111300): (1) LTE mechanism uses per QCI for reporting, and here per DRB can be considered. And this may also impact the configuration/reporting procedure (not much) (2) In LTE mechanism, the delayThreshold-r13 has the values ms30, ms40… In NR, we think these values can be re-designed, because it is expected that the UE will experience less transmission time in NR than in LTE, and the threshold values can be even smaller if considering toB cases, e.g. 0.25ms, 0.5ms, 1ms (3) In TS 36.314, Table 4.2.1.1.1-1: EXCESS DELAY RATIO measurement report mapping captures the mapping information between reported value and measured quantity value. We are generally fine with the table as the granularity is plentiful. However, for toB cases, it might need finer granularity, e.g. 99.999%, but we are open for it for now |

# Conclusions

TBD

# Reference

- R2-2110642	Discussion on L2M	Huawei, CMCC, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core	R2-2108567

- R2-2111300	Report of email discussion [AT116e][852][SON/MDT] Packet “reliability” measurement for D1 (Huawei)