3GPP TSG-RAN WG2 Meeting #116-bis-e                                                    R2-22xxxxx

E-meeting, January 17 – 25, 2022

Agenda item:	8.4.1

Source: 		Qualcomm Incorporated (Rapporteur)

Title: 	 [Post116bis-e][079][eIAB] Open Issues (Qualcomm)

Document for:	Discussion

# 1	Introduction

This document captures:

[Post116bis-e][079][eIAB] Open Issues (Qualcomm)

Scope: Determine if Company input by Pre117-e discussions shall be used, and how many / which Pre-discussions shall be done. Capture Open Issues not captured in the CR email discussions and suggest how to treat. [After finalization, Merge open issues from other discussions into a WI OI list (OI for which company input is invited in some way shall be listed in the WI-list).

Intended outcome: Open Issues list, and organization of Pre117-e Company input discussions for the WI.

Deadline: Short.

The deadline is Friday Jan 28 0800 UTC.

# Discussion

All issues identified in the other Post116bis-e discussions need to be addressed and are not separately captured here.

## Update of ST2

We need to update ST2 38300 and 37400. This discussion should include latest agreements as well as RAN3’s BL CR on 38.300 in R3-221230. Further, all editor notes need to be addressed.

This could be done via email discussion.

Q1: Are there other aspects related to ST2 TSs to be considered? Do you agree that email discussion is sufficient?

| Company | Comment |
| --- | --- |
| Samsung | Agree that email discussion is sufficient. |
| Intel | Agree |

## MAC

Please provide comments related to MAC in the MAC-related thread.

The rapporteur noticed that the following MAC CE requested by RAN1 in R2-2200095 still needs to be handled:

“Signaling from an IAB-node/IAB-donor to a child node indicating beams of an the child IAB-DU in the direction of which simultaneous operation is restricted.”

This could be done via email discussion.

Q2: Any comments on the MAC CE related to RAN1 agreement above? Do you agree that email discussion would be sufficient?

| Company | Comment |
| --- | --- |
| Samsung | Please note that there is a dedicated (short) MAC discussion which focuses on update to CR but also on collecting open issues (as acknowledged by the rapporteur above). This present discussion [079] should then collate those issues as well. But in general we agree that the only open issues to do with MAC are new MAC CE(s) requested by RAN1, and this could be handled via an email discussion. There is no need for separate contributions to the meeting on this matter. One additional outstanding issue is padding BSR but this is minor and could also be handled via a single email discussion dedicated to open MAC issues. |
| Intel | Agree with Samsung. |

## BAP

Please provide comments related to BAP in the BAP-related thread.

We need to further address the FFS from the agreement of this meeting:

Referring to previous agreement “Will have rewriting mapping configuration(s) Old routing ID to New routing ID that limits the possible rewriting (for all cases of re-writing)”: It is FFS whether for upstream there would be a configuration optimization such that the “New Routing ID” is the same for all entries (a.k.a. default routing ID)

The following options for the optimization of rewriting mappings for UL inter-donor-DU re-routing have been proposed in prior meetings/discussions:

Option a: No optimization, i.e., inter-donor-DU re-routing uses configurations of (Ingress BAP routing ID, Egress BAP routing ID)-pairs. For this option, we need to resolve the ambiguity between re-routing and inter-topology routing for a boundary node as discussed during [AT116bis-e][049][eIAB].

Option b: Rewriting mapping for inter-donor-DU re-routing is based on a default egress BAP routing ID(s) configured for each parent link.

Option c: Rewriting mapping for inter-donor-DU re-routing is based on the BAP routing IDs included in the routing entries configured for each parent.

Option d: Others.

This could be done via email discussion.

Q3: Any comments on this topic? Do you agree that email discussion would be sufficient?

| Company | Comment |
| --- | --- |
| Samsung | Just to reiterate our view that using the default routing ID does not violate any previous agreement and that our answer to the ‘FFS’ is yes. Perhaps this topic should be driven by company contributions? We’d be very happy to hear other views. |
| Intel | Agree. |

## RLF indication

The rapporteur believes that we have exhausted this topic. The type-2/3 RLF indications were supposed to provide performance optimization during BH RLF recovery. This performance optimization is already small since BH RLF recovery is expected to be a rare and short event.

So far, the only purpose of this indication we agreed to capture on ST2 was local UL re-routing. This, obviously, only applies if the child node is dual connected. We further watered this down by not making it mandatory even if available. We could not agree to propagate this type-2 indication, which means that it cannot benefit any dual-connected descendent node further down the tree. We further could not agree to capture any other purposes of the type-2 indication in the spec.

The remaining open issues only relate to corner cases and further optimizations, e.g., partial re-routing, adding information on the type-2 indication, etc. The rapporteur does not believe that any further discussion would lead to convergence on the remaining matters or would add benefit to an already critically slimmed down feature.

For these reasons, we can stop ST2 discussions on type-2/3 RLF indication. Remaining issues can be included in St3 discussions (BAP and RRC).

Q4: Any comments on this topic?

| Company | Comment |
| --- | --- |
| Samsung | OK to stop ST2 discussions but do not agree all the remaining issues are ‘corner cases’. However we are ok to discuss them in BAP and RRC discussions, so long as this does not mean we have excluded certain specific outstanding issues. Alternatively, the rapporteur could please share a list of such proposed exclusions, and we could then discuss this list? Otherwise the proposal seems a bit vague to us. |
| Intel | Agree. |

## RAN3 efforts

RAN3 agreed to proceed with solution 1 for latency reduction of intra-donor topology adaptation. RAN3 informed RAN2 about this solution in LS in R2-2106948. RAN2 replied with potential concerns in LS in R2-2109108.

Related to solution 1, RAN3 further agreed in this meeting:

CHO combined with solution#1 is not addressed by RAN3 unless requested by RAN2.

RAN2 should discuss RAN2-related aspects of RAN3’s solution 1.

This could be done via email discussion.

Q5: Any comments on this topic? Other RAN3 topics to be discussed in RAN2? Do you agree that email discussion would be sufficient?

| Company | Comment |
| --- | --- |
| Samsung | Agree that email discussion is sufficient. |
| Intel | For solution 1, RAN3 has also agreed following agreements: WA: Upon migration/HO failure case, the buffered RRC message is still transferred to child node. Agree to confirm solution 1: An IAB-DU buffers an RRC message for a child IAB-MT based on an indication in the F1AP message carrying this RRC message. From RAN2 point of view, there’s still open issues on how to handle the received RRC messages upon migration failure. This is because, except bap-config, the received RRC messages would carry other RRC configurations. It is important for RAN2 to discuss how to handle such messages if partial information is expired. Since RAN2 didn’t have enough time to discuss this topic during previous meetings, we think this topic should be discussed based on company’s contribution to have a clearer view on how the solution is solved in RAN2. |

## Other issues

Please indicate if there are any other issues which need discussion and have not been captured above, e.g., related to:

- CP-UP separation

- UE capabilities

- Topology adaptation

- Others

Q6: Any aspects missed? Do you believe that these aspects can be discussed via email discussion?

| Company | Comment |
| --- | --- |
| Samsung | We think the list above captures well the key overarching topics. Is the proposal to have a single email discussion for all of them? Perhaps we could use company contributions instead, but agree on a specific list of topics first? |
| Intel | As captured in Chair’s Note, following aspects of UE capabilities need to be discussed and solved: - FFS UE capability for Rel-17 intra-donor DU local-rerouting and inter-donor DU re-routing. - FFS whether need to differentiate the capability between “inter-donor CU partial migration” and “inter-donor CU routing for topology redundancy” - FFS the feature group for BAP header rewriting based inter-donor CU routing - FFS the feature group for local rerouting |

# 3	Summary

…