3GPP TSG-RAN WG2 Meeting #116bis-e     			          	                         R2-22xxxx

Electronic Meeting, 17 – 25 January 2022

Agenda item:	8.8.1

Source:	CMCC

Title:	List of open issues for RAN slicing WI

WID/SID:	FS_NR_slice

Document for:	Discussion and Decision

Introduction

This document aims at summarizing critical open issues that need to be addressed for RAN slicing WI by RAN2, including open points captured in the running CRs for RAN2 specifications.

As per chairman guidelines, this discussion shall propose the pre-discussions for next meeting.

- Each open issue should be associated with suggested treatment/handling.

  - Company input into Pre117-e-offline (i.e. no company tdocs)

  - Company tdocs invited.

  - CR rapporteur handled issue (CR rapporteur will propose resolution as input to next meeting).

  - Other, e.g. immature area, reference to dependency, unclear status etc.

Rapporteur has provided suggested treatment for each OI with colored index.

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company and Name | Contact E-mail |
| --- | --- |
| CMCC, Ningyu Chen | chenningyu@chinamobile.com |
| Qualcomm, Peng Cheng |  |
| Huawei, HiSilicon, Jun Chen | jun.chen@huawei.com |

List of RRC open issues (as captured in the RRC running CR [1])

| Issue | Relevant section in TS 38.331 |  | Suggested handling |
| --- | --- | --- | --- |
| OI 1.1: RAN2 assumes that for purpose of UE checking supported slices on the highest ranked cell at TA/RA boundary, gNB can provide in SIB the slice group that supported by these neighbour cells. If this conflicts with SA2, RAN2 will align with SA2. FFS if the slice group is mapped by the mapping relationship in current RA or not. FFS PCI list and/or TAC per slice group are provided. FFS what is the UE behaviour if gNB doesn’t provide supported slice group info on the best ranked cell. | Chairman notes |  | Company tdocs with additional details are invited. |
| OI 1.2: FFS if RA-prioritization will be different for 2-step and 4-step RACH | 6.3.1 |  | Company input into Pre117-e-offline |
| OI 1.3: Whether to introduce a T320-like timer for slice-based cell reselection priorities in dedicated signalling, and if needed, there are two options: Option 1: introduce a new T320-like timer which is independent from the current T320 timer. Option 2: re-use the current T320 timer. | 5.3.8.3 |  | Company input into Pre117-e-offline |
| OI 1.4: FFS in which SIB to broadcast slice info for the purpose of inter-frequency reselection, SIB4 or new SIB. | 6.3.1 |  | Company input into Pre117-e-offline |
| Q 1.5: Whether to support dedicated RACH resources and RACH prioritization parameters in dedicated signalling. | 6.2.2 | Company input into Pre117-e-offline |  |

Q1: Do you agree with above essential open issue list and the suggested handling? Is there any other essential open issues need to be handled for close of this WI?

| Company | Comments |
| --- | --- |
| Qualcomm | We agree with Rapporteur’s plan, and have below comments: First, it is not clear whether we will have Pre117-e-offline. We think it is necessary to complete all remaining issues (at least for some issues with majority view but no time to treat online). It is better that Rapporteur can confirm we have such offline. Then, for OI 1.1, it should also be treated in Pre1170-e-offline (if any), right? |
| Huawei, HiSilicon | We think that the slice group definitions are still open, e.g. RAN2 discussed the following proposal but no agreements were made: Proposal 1.2: The working assumption is that the maximum of slice group is 16. FFS whether it is to be updated. We note that OI 3.7 is for the above open issues and it can be also put to above open issue list table or we add a note for OI 3.7 (e.g. the issue is also valid for RRC CR). We share similar views as Qualcomm that Pre117-e-offline should be confirmed, and we do think that the offline is necessary and important for progressing on these open issues. |

List of MAC open issues (as captured in the MAC running CR [2])

| Issue | Relevant section in TS 38.321 | Suggested handling |
| --- | --- | --- |
| OI 2.1: FFS whether RA prioritization and RA partitioning will work independently or RA prioritization and RA partitioning should configure/work simultaneously for a specific slice group. | 5.1.1a | Company input into Pre117-e-offline (i.e. no company tdocs) |
| OI 2.2: The parameters should be aligned with RRC spec, e.g., ra-PrioritizationForSlicing, ra-PrioritizationForSlicingTwoStep, enableRA-PrioritizationForSlicing, ra-Prioritization, RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA for Slicing. | 5.1.1a | To be updated by CR rapporteur to align with RRC CR. |
| OI 2.3: FFS on the impact of RA fallback from 2-step Slicing RA to 4-step Slicing RA or 4-step common RA. | 5.1.3a, 5.1.4a, 5.1.5 | To be updated later by CR rapporteur to align with common RACH decision. |

Q2: Do you agree with above essential open issue list and the suggested handling? Is there any other essential open issues need to be handled for close of this WI?

| Company | Comments |
| --- | --- |
| Qualcomm | We agree Rapporteur’s plan. It seems one issue on EN of 38.321 is missed: Editor’s Note: To be updated to align with common RACH decision and general MAC CR, if needed. We think it can be handled by CR rapporteur. |

List of 38.304 open issues (as captured in the 38.304 running CR [3])

| Issue | Source or Relevant section in TS 38.304 | Suggested handling |
| --- | --- | --- |
| OI 3.1: Option A without formula: Solution 4, all NAS-prioritised slices with frequency priorities as well as legacy frequency priorities are considered, without iteration, without formula | Chairman notes | Top priority Company tdocs with additional details are invited. |
| OI 3.2: In case prioritised slice is not supported in the highest ranked cell on the target frequency, what’s the UE behaviour, e.g., uses legacy frequency priority or recalculate frequency priority? | Email discussion [Post116-e][242][Slicing] | Company tdocs with additional details are invited. |
| OI 3.3: Whether additional exit condition needed for fallback to legacy cell reselection. | Email discussion [Post116-e][242][Slicing] | Proposed in comments of [242] email discussion and can be discussed based on company contributions. |
| OI 3.4: After the UE fallbacks to legacy cell reselection, the next trigger of slice-based cell reselection. | Email discussion [Post116-e][242][Slicing] | Proposed in comments of [242] email discussion and can be discussed based on company contributions. |
| OI 3.5: If the UE is configured with slice based dedicated priority, but the UE cannot find a suitable cell, whether and how to fallback to legacy cell reselection. | Email discussion [Post116-e][242][Slicing] | Proposed in comments of [242] email discussion and can be discussed based on company contributions. |
| OI 3.6: Whether the inter-RAT frequency should be considered in slice-based cell reselection. | Email discussion [Post116-e][242][Slicing] | Proposed in comments of [242] email discussion and can be discussed based on company contributions. |
| OI 3.7: The definition of slice group is FFS. A group which is associated with one or multiple slices. And a slice is associated to none or one slice group. FFS associated to multiple slice groups | 3.1 | Depending on SA2 further agreements. |
| OI 3.8: Slice specific cell reselection parameters. | 5.2.4.7.0 | CR rapporteurs to update aligned with RRC spec. |
| OI 3.9: Whether to confirm the granularities of the slice groups for cell reselection are per TA. | Chairman notes | Company input into Pre117-e-offline (i.e. no company tdocs) |

Q3: Do you agree with above essential open issue list and the suggested handling? Is there any other essential open issues need to be handled for close of this WI?

| Company | Comments |
| --- | --- |
| Qualcomm | We agree with Rapporteur’s plan, but have below comments: - For OI 3.1 and OI 3.2, it is suggested to include it in Pre117-e-offline. And it is better if two or more candidate CRs can be identified in offline for further down-select during online discussion. - Also suggest to include 3.3-3.6 in Pre117-e-offline because they are detailed issues and it is better to identify candidate solutions or get a majority in offline if possible |
| Huawei, HiSilicon | As we commented in section 1.1, OI 3.7 is also valid for RRC CR. In addition, we think that the size of slice group id can be an open issue. Regarding which of WGs should solve OI 3.7, we think that RAN2 can discuss and solve the issues, and then RAN2 informs SA2. |

List of 38.300 open issues (as captured in the stage-2 running CR [4])

| Issue | Relevant section in TS 38.300 | Suggested handling |
| --- | --- | --- |
| OI 4.1: Details of slice grouping and how it is provided to the UE are FFS, depends on SA2 | 16.3.3, 16.3.X | CR rapporteur to update based on further RAN2 agreements or SA2 further agreements. |

Q4: Do you agree with above essential open issue list and the suggested handling? Is there any other essential open issues need to be handled for close of this WI?

| Company | Comments |
| --- | --- |

References

- R2-2200973 Running RRC CR for RAN slicing (Huawei)

- R2-2201536 Running MAC CR for RAN slicing (OPPO)

- R2-2200044 Running 38.304 CR for RAN slicing (Ericsson)

- R2-2111400 Running Stage-2 CRs for RAN slicing (Nokia)