3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-220xxxx

Online, January, 2022

Agenda item:	8.15.2

Source: 	OPPO

Title: 	Summary of [705]

Document for:	Discussion and Decision

Introduction

This document is for the following discussion

[POST116bis-e][705][V2X/SL] Open issues on SL DRX (OPPO)

Scope: 1st phase: Make an open issue lists with the proposed candidate options or rapporteur suggestion. Open issue lists can include pre-identified issues (e.g. FFS, not decided or skipped from previous offline/email discussion) and new issues raised in company contributions at RAN2#116bis. For new issues that have not discussed before, rapporteur can collect companies’ inputs (e.g. whether it is essential issue that need to be considered and closed in Rel-17) and based on that, determine whether to be included in the open issue list or not. Note open issue lists also include UE capability issues raised in the company contributions.

2nd phase: email discussion on the identified open issues with collecting companies’ inputs on the candidate options or rapporteur’s suggestion.

Intended outcome:  Open issue list with the proposed candidate options or rapporteur’s suggestion from 1st phase (in R2-2201805). Discussion summary for the identified open issues from 2nd phase.

Deadline: 1st phase (1/21 – 1/28 UTC), 2nd phase (2/9 – 2/14 UTC)

Discussion

Based on the Chairman guidance on categorization

- Each open issue should be associated with suggested treatment/handling.

  - Company input into Pre117-e-offline (i.e. no company tdocs)

  - Company tdocs invited.

  - CR rapporteur handled issue

  - Other, e.g. immature area, reference to dependency, unclear status etc.

The issues in this section is of category-1 (where some issues explicitly mention running-CR dependency can be handled as 3 jointly)

In each section, the issues are grouped as either old issues or new issues, and for new issues, companies can input on the need to discuss based on the following guidance. Furthermore, companies can also input if believe a specific old issue should be categorized into new issue (in order to doubt the necessity to discuss it), please be free to input as well

For new issues that have not discussed before, rapporteur can collect companies’ inputs (e.g. whether it is essential issue that need to be considered and closed in Rel-17) and based on that, determine whether to be included in the open issue list or not.

  - Unicast-Specific Issues

    - Common issues

Left issue on what DRX pattern to use for UC-based DCR message, to address the following FFS point

7:	The default SL DRX configuration for BC/GC can be used for the DCR message. FFS for UC (at least for the initial message).

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200373 | OPPO | Proposal 1 Apply the same DRX scheme for UC-based DCR message as for BC-based DCR message, i.e., the default SL DRX configuration for BC/GC. |  |

Q2.1.1-1 (old issue): Do you agree the default SL DRX configuration for BC/GC can be used for both BC-based and UC-based DCR message?

Left issue on whether DRX is applicable to message between DCR message and RRCReconfigurationSidleink message, to address the following skipped proposal at R2#116

Proposal 25: RAN2 further discuss that whether SL DRX should be applied for the PC5-S messages which are sent after the DCR message and before SL unicast DRX configuration is applied.

- Skipped.

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200318 | CATT | Proposal 14: It is slightly preferred that the SL DRX is not applied to SL UC messages after DCR and before the SL DRX configuration is applied. |  |
| R2-2200373 | OPPO | Proposal 2 The PC5-S/PC5-RRC signalling after DCR and before UC DRX configuration is exchanged in a non-DRX manner. |  |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 12: RAN2 discuss the SL DRX configuration used during unicast establishment procedure, with following options l Option 1: preconfigured SL DRX configuration for [DCR~DCA], per-PQI SL DRX configuration after DCA and until dedicated SL DRX configuration is completed l Option 2: preconfigured SL DRX configuration after DCR and until dedicated SL DRX configuration is completed l Option 3: UE is always awake, i.e., no DRX, after the unicast link has been established and until dedicated SL DRX configuration is completed |  |
| R2-2200483 | HW | Proposal 14: RAN2 to adopt Option 2 to transfer other PC5-S message (SMC, DCA, etc.), PC5-RRC message related with UE capability interaction (i.e. UECapabilityEnquirySidelink message and UECapabilityInformationSidelink message), and the first RRCReconfigurationSidelink message (incl. DRX configuration): - Option 1: Using the same BC/GC DRX configuration for DCR message transmission to transmit these message. - Option 2: From RX UE perspective, DRX is deactivated after receiving DCR message and activated when receiving the first RRCReconfigurationSidelink message (incl. DRX configuration) |  |
| R2-2200528 | Intel Corporation | Proposal 2: It is proposed to not apply SL DRX for the PC5-S/PC5-RRC messages which are sent after the DCR message and before SL unicast DRX configuration is applied. |  |
| R2-2200544 | LG Electronics France | Proposal 2: The common default SL DRX configuration for BC/GC can be used until receiving RRCReconfigurationSidelink for the initial SL DRX configuration between TX UE and RX UE in unicast. For example, the messages for DCR, DCA, capability exchange, and initial SL DRX configuration can be transmitted on the default SL DRX configuration. |  |
| R2-2200938 | Ericsson | Proposal 21 Apply the common default SL DRX configuration for GC/BC also to the other initial signalling sent after the DCR message and before the SL unicast DRX configuration is applied. |  |
| R2-2201523 | Lenovo, Motorola Mobility | SL DRX Configuration during Unicast establishment procedure Proposal 5: RAN2 agree the SL DRX configuration used during unicast establishment procedure, with following option:  Option 1: preconfigured SL DRX configuration for [DCR~DCA], per-PQI SL DRX configuration after DCA and until dedicated SL DRX configuration is complete |  |

Q2.1.1-2 (old issue): Which option do you prefer for messages delivery between PC5-S DCR message and PC5-RRC RRCReconfigurationSidelink message including DRX configuration

Option-1: DRX is not applied

Option-2: DRX is applied, using default SL DRX configuration for BC/GC, i.e., the same as the one used for DCR message

Left issue on the content of assistance information of desired DRX configuration, to address the following skipped proposals in R2#116

Proposal 14: RAN2 to further discuss whether the drx-inactivity timer should be included in the RX UE’s desired SL DRX configuration.

Proposal 15: RAN2 to further discuss whether the HARQ RTT timer should be included in the RX UE’s desired SL DRX configuration.

Proposal 16: RAN2 to further discuss whether the HARQ retransmission timer should be included in the RX UE’s desired SL DRX configuration.

Proposal 19: RAN2 to further discuss when the Rx UE rejects the SL DRX configuration included in the RRCReconfigurationSidelink, which PC5-RRC signaling should be sent from Rx UE to Tx.

- Proposal 14, 15, 16 and 19 are skipped.

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200318 | CATT | Proposal 5: The inactivity timer, HARQ RTT timer and retransmission timer could be included in the desired SL DRX configuration to help gNB or Tx UE to determine the SL DRX configuration. |  |
| R2-2200344 | NEC Corporation | Proposal 1 From signaling design point of view, include drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer to the assistance information signaling. |  |
| R2-2200344 | NEC Corporation | Proposal 2 Whether to indicate RX UE’s desired drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer is up to RX UE’s implementation. |  |
| R2-2200373 | OPPO | Proposal 3 Inactivity timer/retransmission timer/RTT timer are not included in the RX UE’s desired SL DRX configuration. |  |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 6: Assistance information from Rx UE includes information with respect to a shift (drx-StartOffset) of the DRX Cycle with respect to the current start of the DRX cycle i.e., no other DRX configuration parameter from the Rx UE is provided. | For shift of the DRX cycle, this is the only paper proposing it, moderator suggest not to prioritize this issue. |
| R2-2200791 | Xiaomi | Proposal 9: RX UE provides undesired SL DRX configuration to TX UE in assistant information, i.e. its activated configured SL/UL grant resource allocation. |  |
| R2-2201152 | InterDigital | Proposal 4: Drx-inactivity timer, HARQ RTT timer, and HARQ retransmission timer are not included in the RX UE’s desired SL DRX configuration. |  |
| R2-2200528 | Intel Corporation | Proposal 1: The drx-inactivity timer, SL HARQ RTT and HARQ retransmission timer shall not be included as part of the RX UE’s SL DRX desired configuration and how to configure them is up to the TX UE (or its serving gNB). |  |

Q2.1.1-3a (old issue): Whether inactivity timer, HARQ RTT timer and re-transmission timer are included in assistance information from Rx UE to Tx UE? (companies can express preference for each timer respectively)

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200373 | OPPO | Proposal 4 For on-duration timer length and cycle value, value range (e.g., the minimum value and the maximum value for a parameter) can be used to express the desired SL DRX configuration. |  |
| R2-2200791 | Xiaomi | Proposal 8: Multiple sets of preferred SL DRX configuration could be included in assistance information, each set of SL DRX configuration corresponds to Uu DRX or SL DRX on other destinations. |  |
| R2-2200893 | vivo | Proposal 8 For the content of SL assistance information, agree one set of preferred SL DRX timers configuration included in UEAssistanceInformationSidelink. |  |

Q2.1.1-3b (new issue): In assistance information from Rx UE to Tx UE, for each DRX setting (cycle, timer and etc.), do you think a single value is enough or multiple values are needed (detailed signalling format, whether multiple setting combination, or value range of each parameter, can be left to RRC running-CR discussion) (companies can express preference for each DRX setting respectively)

Left issue to consolidate the initiation condition for Rx-UE to send assistance information

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 4: Assistance information from a Rx UE for SL DRX configuration is triggered when 1) Tx UE capability indicate Tx UE support SL DRX; 2) DRX configuration received from Tx UE is not suitable. | For 2), moderator understand as long as the previously sent assistance information has not changed, there is no need to re-send it. |

In the current running-CR, it is described as

5.8.9.X.2	Initiation

For sidelink unicast, a UE capable of sidelink DRX may send this assistance information to its peer UE when the previously transmitted sidelink DRX assistance information has changed.

Moderator understands that on top of the existing initiation condition, further condition can be considered, e.g., 1) Tx capability indicate Tx-UE support SL DRX, 2) the assistance information has not been sent before.

Q2.1.1-4 (new issue): On top of the existing RRC running-CR, any additional initiation condition needed for the delivery of assistance information?

Condition-1: peer-UE is capable of sidelink DRX

Condition-2: the assistance information has not been sent previously

Condition-3: serving cell is capable of sidelink DRX

Left issue to consolidate the Tx-UE behaviour to send DRX configuration

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 3: Assistance from Rx to Tx can be sent at any point. Tx does not have to wait for Rx assistance to decide and signal a DRX configuration to the Rx UE. | These two are different views. |
| R2-2200544 | LG Electronics France | Proposal 3: When initial SL DRX configuration, TX UE should wait for assistance information from RX UE for a certain period after capability exchange with RX UE. |  |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 7 Considering that how to configure SL DRX is based on TX UE implementation, RAN2 shall design a mechanism to ensure that the TX UE will try its best to configure suitable SL DRX for the RX UE. | These two are different views Moderator understand assistance information has to be taken into account otherwise it goes against the motivation to introduce it. Seems the current RRC running CR did not put a restriction in the following section 5.8.9.X.3 Actions related to reception of UEAssistanceInformationSidelink message For sidelink unicast, when a UE is in RRC_CONNECTED, it may report this assistance information received from its peer UE to the network. For sidelink unicast, when a UE in IDLE/INACTIVE or OOC has obtained this assistance information from its peer UE, it may derive the value of the inactivity timer based on its implementation. |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 5: If Rx assistance is available at the Tx UE, it can be considered by the Tx UE. |  |

Q2.1.1-5a (new issue): After capability exchange, is there a need to define a time restriction for Tx-UE to send DRX related configuration to RX-UE?

Q2.1.1-5b (new issue): Upon reception of UEAssistanceInformationSidelink, do you agree to capture Tx-UE behaviour on taking it into account for DRX configuration derivation (e.g., “it may derive the value of DRX settings based on its implementation by taking assistance information into account”, detailed wording can be left to MAC running-CR discussion)

Left issue to consolidate Rx-UE behaviour to reject a DRX configuration, firstly, condition to reject DRX configuration

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200483 | HW | Proposal 2: RAN2 to discuss the following triggering conditions of SL DRX configuration failure/rejection in RX UE: - The received SL DRX does not match the desired SL DRX of the RX UE - The received SL DRX does not match the configured SL DRX(s) for other SL connection(s) of the RX UE - The received SL DRX does not match the SL DRX configuration(s) configured for its RX UE(s) - The received SL DRX does not match the power saving demand of the RX UE. | Moderator understand the first one seems straightforward, i.e., “The received SL DRX does not match the desired SL DRX of the RX UE” |

Q2.1.1-6 (new issue): Is there a need to capture in spec the condition for Rx-UE to reject a DRX configuration?

Option-1: No

Option-2: Yes, condition of “the received SL DRX does not match the desired SL DRX of the RX UE sent in assistance information”

Secondly, what message to use to reject a DRX configuration

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200318 | CATT | Proposal 3: RRCReconfigurationFailureSidelink is used by Rx UE to reject the Tx UE’s SL DRX configuration. |  |
| R2-2200318 | CATT | Proposal 4: If SL DRX configuration is rejected by Rx UE, the Rx UE can send RRCReconfigurationFailureSidelink message to Tx UE, and it is unnecessary to introduce additional cause value in the RRCReconfigurationFailureSidelink message. |  |
| R2-2200344 | NEC Corporation | Proposal 4 When the Rx UE rejects the SL DRX configuration included in the RRCReconfigurationSidelink, RRCReconfigurationCompleteSidelink with a new DRX rejection indication should be sent from Rx UE to Tx UE. |  |
| R2-2200373 | OPPO | Proposal 5 Use RRCReconfigurationCompleteSidelink message to indicate the SL DRX rejection from Rx UE. |  |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 8: Reuse RRCReconfigurationFailureSidelink to indicate SL DRX configuration failure |  |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 9: A failure cause is added in RRCReconfigurationFailureSidelink to differentiate whether the radio configuration is failed or SL DRX configuration is failed. Similar information |  |
| R2-2200791 | Xiaomi | Proposal 2: If there is configuration error for the sidelink configuration carried in RRCReconfigurationSidelink, UE response with RRCReconfigurationFailureSidelink, otherwise, UE response with RRCReconfigurationCompleteSidelink. |  |
| R2-2200791 | Xiaomi | Proposal 3: Introduce new indication for RX UE to inform TX UE the sidelink DRX configuration accept or reject on sidelink. |  |
| R2-2200791 | Xiaomi | Proposal 4: Indication of DRX configuration accept or reject is carried in RRCReconfigurationCompleteSidelink. |  |
| R2-2200791 | Xiaomi | Proposal 5: TX UE checks the indication of DRX configuration accept or reject in RRCReconfigurationCompleteSidelink to determine whether the sidelink DRX configuration carried in corresponding RRCRecofigurationSidelink is applied or not by RX UE |  |
| R2-2200791 | Xiaomi | Proposal 6: TX UE considers the Sidelink configuration other than DRX carried in corresponding RRCRecofigurationSidelink applied by RX UE upon reception of RRCReconfigurationCompleteSidelink. |  |
| R2-2200938 | Ericsson | Proposal 3 RX UE replies RRCReconfigurationFailureSidelink if the SL DRX configuration is rejected, with a new rejection cause included. |  |
| R2-2200938 | Ericsson | Proposal 4 In case the RX UE has rejected the SL DRX configuration, the RX UE shall reject the whole RRC reconfiguration as in Uu. |  |
| R2-2201523 | Lenovo, Motorola Mobility | SL DRX configuration rejection Proposal 1: Reuse RRCReconfigurationFailureSidelink to indicate SL DRX configuration failure |  |
| R2-2201523 | Lenovo, Motorola Mobility | Proposal 2: A failure cause is added in RRCReconfigurationFailureSidelink to differentiate whether the radio configuration has failed, or SL DRX configuration has failed. |  |
| R2-2200544 | LG Electronics France | Proposal 4: RX UE can continue to use the prior SL DRX configuration until receiving a new SL DRX configuration after transmitting assistance information/rejection message. | Moderator understand it would be business as usual if RRCReconfigurationFailureSidelink is used, and can clarify in case RRCReconfigurationCompleteSidelink is used |

Q2.1.1-7(old issue): In order for Rx-UE to reject a DRX configuration, which message to use, RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink?

Q2.1.1-7a (old issue): In case RRCReconfigurationFailureSidelink is adopted, do you agree to introduce an indication for the DRX configuration rejection RRCReconfigurationFailureSidelink?

Q2.1.1-7b (old issue): In case RRCReconfigurationCompleteSidelink is adopted, do you agree to introduce an indication for the DRX configuration rejection RRCReconfigurationCompleteSidelink?

Q2.1.1-8 (new issue): In case RRCReconfigurationCompleteSidelink is adopted, after rejecting the DRX configuration, should the Rx-UE use the prior SL DRX configuration until receiving a new SL DRX configuration?

Given the tool to reject the undesired DRX-configuration by Rx-UE, left issue on the necessity of additional tool to avoid Tx-UE implementation keeping sending the undesired DRX configuration

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 8 If RX UE sends SL DRX assistance information, but the TX UE does not configure acceptable SL DRX for the RX UE and no SL DRX is used before, the RX UE can use desired SL DRX configuration included in the assistance information, or use default SL DRX instead of using no SL DRX. | I.e., if the DRX configuration is not desired, RX UE would start using desired configuration by itself. Moderator understand the feasibility of this scheme relates to whether all DRX parameters are included in assistance information or not. |
| R2-2200544 | LG Electronics France | Proposal 5: If RX UE receives SL DRX configuration unable to comply despite transmitting assistance information or rejection messages, the RX UE should be allowed unicast session release. | I.e., if the DRX configuration is not desired, RX UE may disconnect. |
| R2-2200544 | LG Electronics France | Proposal 6: If RX UE does not receive any new SL DRX configuration from TX UE despite transmitting assistance information or rejection messages, the RX UE should be allowed unicast session release. |  |
| R2-2200544 | LG Electronics France | Proposal 7: RX UE needs a timer after transmitting assistance information or rejection message to TX UE. The timer is used to determine whether RX UE finally complies with the SL DRX configuration. | I.e., the timer used as a “deadline” for Tx-UE to send desired DRX configuration |
| R2-2200893 | vivo | Proposal 4 The SL DRX negotiation procedure between SL TX UE and SL RX UE can be either one-shot or multiple-shot. |  |
| R2-2200893 | vivo | Proposal 5 When SL TX UE is RRC IDLE/IANCTIVE/OOC, it’s up to SL TX UE to select one shot or multiple shots for the SL DRX negotiation procedure between SL TX UE and SL RX UE. |  |
| R2-2200893 | vivo | Proposal 5 When SL TX UE is RRC IDLE/IANCTIVE/OOC, it’s up to SL TX UE to select one shot or multiple shots for the SL DRX negotiation procedure between SL TX UE and SL RX UE. |  |
| R2-2200893 | vivo | Proposal 6 When SL TX UE is RRC IDLE/IANCTIVE/OOC, if multiple-shot SL DRX negotiation is executed, RAN2 to discuss some mechanism (e.g., timer or counter) to avoid endless negotiation between SL TX UE and SL RX UE. | I.e., the timer + counter for Tx-UE to send desired DRX configuration. Moderator suggest to focus on the timer since the counter proposal is from a single paper, and anyway counter cannot work alone without timer. |
| R2-2200893 | vivo | Proposal 7 When SL TX UE is RRC CONNECTED, it’s up to SL TX UE’s serving gNB to select one shot or multiple shots for the SL DRX negotiation procedure between SL TX UE and SL RX UE. No specification impact is foreseen. |  |

Q2.1.1-9a (new issue): Is there a need to introduce a restriction for Tx-UE to send desired DRX configuration to Rx-UE after Rx-UE reject the DRX configuration

Option-1: No

Option-2 (new issue): Yes, a timer is needed (e.g., the timer starts upon Rx-UE reject the DRX)

Q2.1.1-9b (new issue): If option-2 (or any similar timer-based solution) is selected in the question above, what should be the result upon the expiry of this timer?

Option-1: Rx-UE starts using desired DRX configuration autonomously;

Option-2: Rx UE release the unicast link with Tx UE (e.g., using PC5-S message PROSE DIRECT LINK RELEASE REQUEST)

    - Issues related to network involvement

Left issue on gNB capability w.r.t SL-DRX

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200544 | LG Electronics France | Proposal 15: If gNB does not have DRX capability, TX UE keeps the controllability of configuring SL DRX configuration for RX UE. |  |
| R2-2200790 | Xiaomi | Proposal 6: UE triggers SUI to report SL DRX information, i.e. received assistance information and SL DRX, if SL DRX configuration in SIB is present and UE did not report SL DRX information. |  |
| R2-2200893 | vivo | Proposal 1 The UE shall only report SUI carrying the SL DRX configuration or SL assistance information to its serving gNB if its serving gNB is SL-DRX capable. |  |
| R2-2200893 | vivo | Proposal 2 Introduce 1-bit indication in SIB12 to indicate gNB’ support of SL DRX. | Moderator understand there is at least other ways to implicitly indicate it, e.g., by the existence of SL-DRX configuration for GC/BC in SIB12, or rely on configuration in dedicated RRC signalling for RRC_CONNECTED UE to control the report of DRX related information using SUI. |

Q2.1.2-1a (new issue): Do you agree that it is possible that gNB, which provides SL configuration to in-coverage UE,  may or may not support SL-DRX?

Q2.1.2-1b (new issue): If yes to 1a above, how for gNB to notify its capability of SL-DRX support to UE?

Option-1: using indication in SIB12 explicitly

Option-2: using indication in SIB12 implicitly

Option-3: using indication in dedicated RRC signaling

Left issue on what additional report to gNB is needed besides the following agreed one(s)

Agreements on TX-UE centric or RX-UE centric DRX configuration determination

1: 	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, support signalling exchange including both 1) Signaling-1: signalling from RX-UE to TX-UE, and 2) Signaling-2: signalling from TX-UE to RX-UE.

[…]

3:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may report the information received in signaling-1 (Rx->Tx) to the serving network.

[…]

5:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Rx-UE is in-coverage and in RRC_CONNECTED state, Rx-UE report the DRX configuration received in signalling-2 (Tx->Rx) to the serving network.

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200791 | Xiaomi | Proposal 7: CONNECTED TX UE indicate RX UE’s reject or reception of sidelink DRX to gNB. | Report for reject, based on 116b agreement, moderator understand it is for mode-1 case only. |
| R2-2200938 | Ericsson | Proposal 5 A RRC_CONNECTED Tx UE informs its serving gNB of the rejection of SL DRX configuration. |  |
| R2-2200544 | LG Electronics France | Proposal 11: When TX UE in RRC_IDLE/INACTIVE or OoC, performing SL DRX, becomes RRC_CONNECTED, if the serving gNB of TX UE configures to provide, TX UE reports SL DRX configuration for RX UE and stored assistant information from RX UE. | Report by Tx-UE on DRX configuration, based on 116b agreement, moderator understand it is for mode-2 case only Report by Tx-UE on assistance information, based on 116b agreement, moderator understand it is for mode-1 case only |
| R2-2200544 | LG Electronics France | Proposal 14: RX UE reports the latest SL DRX configuration received from TX UE to its serving gNB if the serving gNB configures to provide but not provided yet. | Report by Rx-UE on DRX configuration |
| R2-2201135 | Apple | Proposal 10 If mode 2 TX UE self-determines the SL DRX configuration for unicast link, Mode-2 TX UE in RRC_CONNECTED may inform its serving gNB about its decided SL-DRX configuration by including it in UE Assistance information. | Report by Tx-UE on DRX configuration, moderator understand it is reported only for the DRX configuration accepted by Rx-UE |
| R2-2200544 | LG Electronics France | Proposal 15: If gNB does not have DRX capability, TX UE keeps the controllability of configuring SL DRX configuration for RX UE. | i.e., even in mode-1, it behave as for mode-2 for DRX setting |

Q2.1.2-2a (new issue): At least for gNB which is capable of SL-DRX, do you agree that Tx-UE report assistance information only in case of mode-1?

Q2.1.2-2b (new issue): At least for gNB which is capable of SL-DRX, do you agree that Tx-UE report DRX configuration reject information only in case of mode-1?

Q2.1.2-2c (new issue): At least for gNB which is capable of SL-DRX, do you agree that Tx-UE report DRX configuration accepted by Rx-UE only in case of mode-2?

Q2.1.2-2d (new issue): If yes to 2a above, do you agree to rely on the gNB capability notification, as output of Q2.1.2-1b above, to disable Tx/Rx-UE report (including all DRX related report by Tx-UE, i.e., assistance information, DRX reject information, DRX configuration information, and report by Rx-UE, i.e., DRX configuration information for UC and QoS information for GC/BC), if gNB is not capable of SL-DRX?

Q2.1.2-2e (new issue): If yes to 2a above, do you agree to rely on the gNB capability notification, as output of Q2.1.2-1b above, to rely on Tx-UE itself (as for mode-2) to determines SL DRX for RX UE?

Left issue on signalling content

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2201582 | Samsung Research America | [Proposal 2]: For UC, list of source UE id (as TX UE id), the destination UE id (as RX UE id), SL DRX cycle length, SL DRX start offset and SL DRX on-duration timer are included in the report. |  |
| R2-2201582 | Samsung Research America | [Proposal 3]: For UC, SL DRX inactivity timer, SL DRX HARQ RTT, and SL DRX HARQ retransmission timer are not needed in the report. | Moderator see the point that for Rx-UE it is not feasible for gNB to know the status of inactivity / RTT / Retx timer, while it may be possible for gNB of Tx-UE (mode-2) |

Q2.1.2-3a (new issue): For DRX configuration report by Rx-UE, which DRX parameter(s) should be included?

Parameter-1: SL DRX cycle length

Parameter-2: SL DRX start offset

Parameter-3: SL DRX on-duration timer length

Parameter-4: SL DRX inactivity timer length

Parameter-5: SL DRX HARQ RTT timer length

Parameter-6: SL DRX HARQ retransmission timer length

Q2.1.2-3b (new issue): If one answer Yes to Q2.1.2-2d, for DRX configuration report by Tx-UE, which DRX parameter(s) should be included?

Parameter-1: SL DRX cycle length

Parameter-2: SL DRX start offset

Parameter-3: SL DRX on-duration timer length

Parameter-4: SL DRX inactivity timer length

Parameter-5: SL DRX HARQ RTT timer length

Parameter-6: SL DRX HARQ retransmission timer length

Left issue on the usage of DRX command MAC CE for mode-1 UE

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200373 | OPPO | Proposal 13 RAN2 to discuss the following options on SL DRX command MAC CE in mode-1: 1) gNB takes in charge of sending SL DRX command MAC CE; 2) Tx UE determines on sending SL DRX command MAC CE by itself and reports this to gNB, and 3) do not use SL DRX command MAC CE in mode-1. | The reason is to check how for NW and UE to sync on DRX active time considering the usage of DRX command MAC CE by Tx-UE. |

Q2.1.2-4 (new issue): For Tx-UE in mode-1, whether SL DRX command MAC CE can be used?

Option-1: No

Option-2: Yes, and Tx-UE can only use it based on network indication (for which new signalling is needed)

Option-3: Yes, and Tx-UE has to notify network on the usage (for which new signalling is needed)

    - Companies Input on section 2.1 for Phase-1

Please share your view on the open issue list above (even companies can input using multiple rows for different questions/issues)

| Company | Concerned question | Comment |
| --- | --- | --- |

  - Groupcast/Broadcast-Specific Issues

Left issue on LCP impact due to Tx profile

| Tdoc | Company | Proposals and Moderator’s remark | Moderator’s recommendation |
| --- | --- | --- | --- |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 1 It is suggested to send a LS to SA2 to request them consider the TX profile issue when UE selects the default Destination Layer-2 IDs for the initial signalling. | Moderator understand the root issue is due to a single L2 ID map to different service type with different Tx profiles |
| R2-2200938 | Ericsson | Proposal 34 Define rules for UE to determine which profile shall be applied in case UE has data with different profiles (e.g., belonging to different services types, etc.) for transmission, e.g., select the profile according to the service with the highest priority. | Moderator understand the root issue is due to a single L2 ID map to different service type with different Tx profiles |

Given the following agreement from 116b, moderator understand it is a special case of “a single L2 ID mapped to multiple DRX pattern”, i.e., One associated DRX pattern is non-DRX, and the straightforward solution is to ignore such associated Tx profile since that does not affect the DRX pattern selection.

10:	Working assumption (down-selection for DRX cycle and on-duration for GC/BC when multiple QoS profiles are associated with the same DST L2 id) is confirmed as an agreement.

11:	TX/RX UE determines the DRX cycle applied for groupcast/broadcast transmissions associated with a specific L2 destination ID as the minimum DRX cycle configured for any of the QoS profiles associated with that L2 destination ID.

Q2.2-1 (new issue): Do you agree if a same L2 ID associates with both DRX-based Tx profile and non-DRX based Tx profile, the DRX setting are decided based on the DRX-based Tx profile only.

Left issue on Tx profile report

| Tdoc | Company | Proposals and Moderator’s remark | Moderator’s recommendation |
| --- | --- | --- | --- |
| R2-2200483 | HW | Proposal 18: For Rel-17 TX UE, UE reports TX profile information associated with DST L2 ID in SUI message, to assist the alignment of the Uu DRX of TX UE and SL DRX of RX UE, and to assist the SL transmission of TX UE limited by active time of SL DRX. |  |

When Tx profile is introduced in LTE, the eNB awareness of the mapping between TX profiles and Destination L2 IDs is discussed by RAN2 and the following RAN2 view is concluded and sent to SA2(R2-1815665).

RAN2 view is that the eNB should be provided by the operator or V2X service provider with a mapping between TX profiles and Destination L2 IDs, e.g. as part of the UE subscription profile or via network implementation signalling.

And that RAN2 view is confirmed by SA2 and captured in their spec (23.285):

When the network scheduled operation mode is used, following additional principle applies:

-When the eNB receives a request for PC5 resource from a UE, the eNB may deduce the Tx Profile from the Destination L2 ID.

NOTE 1:	The mapping from Destination L2 ID to Tx Profile is configured in the eNB. The eNB can determine the Tx Profiles that the UE needs to use for transmitting the packets thus utilising the resources available appropriately (i.e. handling of sidelink grant), see TS 36.321 [26] for details.

Moderator understand the same principle and be applied in NR, i.e., gNB is aware of the mapping between L2 ID and Tx profile, no signalling from UE to gNB is needed for reporting Tx profile.

Q2.2-2 (new issue): How for gNB to be aware of the mapping between L2 ID and Tx profile in NR?

Option-1: Reuse the LTE solution, i.e., no spec effort by RAN2;

Option-2: Rely on UE to report mapping, in SUI message, i.e., spec effort by RAN2

    - Companies Input on section 2.2 for Phase-1

Please share your view on the open issue list above (even companies can input using multiple rows for different questions/issues)

| Company | Concerned question | Comment |
| --- | --- | --- |

  - Common Issues for all cast types

    - RTT/Re-tx timer related

Left issue on Re-tx timer start or not upon PSFCH-ACK dropping, i.e., related to the following FFS from R2#116bis

17:	For unicast, sl-drx-RetransmissionTimer is started after expiring sl-drx-HARQ-RTT-Timer when the PSFCH (NACK) transmission is dropped. FFS for ACK transmission dropping.

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200318 | CATT | Proposal 8: For sidelink unicast, sl-drx-RetransmissionTimer is started after the sl-drx-HARQ-RTT-Timer expires regardless of whether the unsent PSFCH is ACK or NACK. |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 20: For unicast, sl-drx-RetransmissionTimer is started after expiring sl-drx-HARQ-RTT-Timer when the PSFCH (NACK) transmission is dropped |  |
| R2-2200374 | OPPO | Proposal 8 For P22 of [716], for ACK/NACK FB case, sl-drx-RetransmissionTimer is started after expiry of sl-drx-HARQ-RTT-Timer only if the dropped PSFCH transmission is NACK. |  |
| R2-2200483 | HW | Proposal 4: If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization), and HARQ RTT timer expires, the RX UE starts retransmission timer regardless of whether the data corresponding to the unsent PSFCH was decoded successfully or not. |  |

Based on the online discussion result, moderator observe the majority view is clear.

Q2.3.1-1 (old issue): For unicast, do you agree sl-drx-RetransmissionTimer is NOT started after expiry of sl-drx-HARQ-RTT-Timer when the PSFCH of ACK transmission is dropped?

RTT timer start position if FB-disabled

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200318 | CATT | Proposal 9：when PSFCH is not configured and SL HARQ feedback is disabled，the SL HARQ RTT, if configured and resource assignment information (SCI-based RTT timer) is not present, is started at the first slot after the end of last PSSCH resource. | FB disabled + no Re-tx resource in SCI + PSFCH not configured |
| R2-2200318 | CATT | Proposal 10: when PSFCH is configured and SL HARQ feedback is configured to be disabled, the SL HARQ RTT, if configured and resource assignment information (SCI-based RTT timer) is not present, is started at the first slot after the end of last PSSCH resource. | FB disabled + no Re-tx resource in SCI + PSFCH configured |
| R2-2200483 | HW | Proposal 7: For HARQ feedback enabled case, when SCI indicates a retransmission resource, the value of HARQ RTT timer should be derived by n-k, where n is the time gap between the current transmission resource and the next reserved retransmission resource, and k is time gap between the current transmission resource and the SL HARQ feedback resource. | FB-enabled + Re-tx resource in SCI |
| R2-2200535 | LG Electronics France | Proposal 5. If resource assignment information exists in the SCI, the Rx UE can start the SL HARQ RTT timer for each PSSCH resource scheduled in the SCI, and the SL HARQ RTT timer can be running until the next retransmission resource. | Re-tx resource in SCI |
| R2-2200535 | LG Electronics France | Proposal 6. If there is no resource assignment information in the SCI, the Rx UE uses the SL DRX HARQ RTT timer (zero value or non-zero value) configured by the gNB. And Rx UE can start the SL HARQ RTT timer in the first slot after the corresponding PSSCH resource. | No Rx-tx resource in SCI |
| R2-2201152 | InterDigital | Proposal 1: For HARQ feedback disabled or PSFCH not configured, the RX UE starts the HARQ RTT timer upon reception of SCI. | Feedback disabled + PSFCH not configured |

moderator understand the existing agreement so far on RTT timer starting position is valid at least for the case where SCI does not indicate re-tx resource and FB is enabled, so the following question is to confirm the validity of the agreement for other cases.

22:	For transmissions with HARQ feedback, the RX UE starts the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission.

23:	If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.

Q2.3.1-2a (old issue): For resource pool with PSFCH, whether the above agreement (RTT timer starts at end of PSFCH) holds for FB disabled case?

Q2.3.1-2b (old issue): For resource pool with PSFCH, whether the above agreement (RTT timer starts at end of PSFCH) holds for the case where SCI indicating re-tx resource?

Q2.3.1-2c (old issue): For resource pool without PSFCH, do you agree RTT timer starts in the slot following the end of PSSCH resource (for both SCI indicating re-tx resource and not indicating re-tx resource)?

Left issue on applicable scenario for RTT timer and Re-tx timer

| Tdoc | Company | Proposals | Moderator‘s remark and recommendation |
| --- | --- | --- | --- |
| R2-2201152 | InterDigital | Proposal 3: Two separate HARQ retransmission timers are configured and used by the UE for HARQ enabled transmissions versus HARQ disabled/PSFCH not configured. |  |
| R2-2201152 | InterDigital | Proposal 2: Two separate HARQ RTT timers are configured and used by the UE for HARQ enabled transmissions versus HARQ disabled/PSFCH not configured. | Moderator understand the agreement below has covered the intention 9: HARQ RTT is supported for both HARQ enabled and HARQ disabled cases by allowing HARQ RTT timer to be set to different values. FFS on the specific values that can be used for HARQ disabled case. While the only uncovered case is PSFCH-not-configured |
| R2-2200938 | Ericsson | Proposal 15 RAN2 to discuss how to set the HARQ RTT timer value in a SL DRX configuration for it to work for both HARQ-disabled case and HARQ-enabled case but the timer value cannot be deduced from SCI. | Considering now the running-CR is implemented in a way that the DRX configuration is set independent from the resource pool configuration, moderator understand the agreement below means that two values can be configured for FB-enable/disable case respectively, and UEs can use correspondingly 9: HARQ RTT is supported for both HARQ enabled and HARQ disabled cases by allowing HARQ RTT timer to be set to different values. FFS on the specific values that can be used for HARQ disabled case. |
| R2-2200373 | OPPO | Proposal 14 Allow different RTT timer setting for 1) resource pool with PSFCH and FB enabled case, 2) resource pool with PSFCH and FB disabled case, and 3) resource pool without PSFCH. |  |

Given the existing agreement, moderator understand RTT timer is also necessary for FB disabled case, so the question is just whether to differentiate between pool with and without PSFCH

HARQ RTT is supported for both HARQ enabled and HARQ disabled cases by allowing HARQ RTT timer to be set to different values.  FFS on the specific values that can be used for HARQ disabled case.

Q2.3.1-3a (old issue): For resource pool where PSFCH is not configured, in case SCI does not indicate re-transmission resource, how to decide the RTT timer length?

Option-1: use a same RTT timer length value, i.e., a same value for FB-disabled case regardless whether PSFCH is configured or not

Option-2: use different RTT timer length value, e.g., fix the length of RTT timer length for pool without PSFCH to be zero

Q2.3.1-3b (new issue): For Re-tx timer, do you think a single value is sufficient to cover all cases (FB-enable/disable, PSFCH configured/not-configured), or is there a need to use different values for different cases?

Option-1: single value is sufficient

Option-2: multiple values are needed (if this option is selected, plz indicate which scenario(s) have to be differentiated by configured different values)

    - Retransmission grant dropping due to DRX inactive time

Left issue on whether retransmission grant can be used for initial transmission, in case the initial transmission grant was dropped with no MAC PDU generated

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200483 | HW | Proposal 10: UE obtains MAC PDU for a SL grant for a retransmission, if UE obtains no MAC PDU for the corresponding prior SL grant. | Moderator understand the current spec allows it for mode-1 CG resource already, but not for mode-1 DG and mode-2 grant yet. 1> if the sidelink grant is a configured sidelink grant and no MAC PDU has been obtained in a sl-PeriodCG of the configured sidelink grant: |

Q2.3.2-1a (old issue): For mode-1 DG, if the initial transmission occasion was dropped due to no Rx-UE in DRX active time, do you agree TX-UE can use re-transmission occasion for initial transmission?

Q2.3.2-1b (old issue): For mode-2 grant, if the initial transmission occasion was dropped due to no Rx-UE in DRX active time, do you agree TX-UE can use re-transmission occasion for initial transmission?

Left issue on reporting A/N for re-transmission grant dropping

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200374 | OPPO | Proposal 2 For P8/9 of [716], for both initial transmission and re-transmission, if a mode-1 SL grant is dropped due to not in SL active time of any destination that has data to be sent, UE sends ACK to gNB by following the current MAC specification. |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 11: when mode 1 SL grant is not in SL active time of any destination that has data to be sent to, for initial transmission or retransmission and the mode 1 grant is dropped, UE sends ACK to gNB |  |
| R2-2200790 | Xiaomi | Proposal 4: UE set NACK in PUCCH if SL grant for retransmission was dropped. |  |
| R2-2200938 | Ericsson | Proposal 32 The TX UE sends NACK to the serving gNB if the mode 1 SL grant is dropped and sl-PUCCH-Config is configured. |  |
| R2-2201061 | ZTE Corporation, Sanechips | Proposal1: For both initial transmission and retransmission, it is suggested for UE to indicate NACK to gNB if mode1 SL grant is not in SL active time of any destination. |  |
| R2-2201135 | Apple | Proposal 3 when mode 1 SL grant is not in SL active time of any destination that has data to be sent, UE sends NACK to gNB if initial transmission or retransmission is dropped. |  |

One point raised by HW is that there might be a difference considering HARQ buffer is empty or not, where the former case (empty HARQ buffer) is the same as for initial transmission (for which ACK was agreed), since UE can make use of the grant for initial transmission (at least for CG, see Q2.3.2-1a/b above)

Q2.3.2-2a (old issue): For mode-1 re-transmission grant, if the MAC PDU has been generated (i.e., the initial transmission has been performed), and the re-transmission grant is dropped due to no Rx-UE in active time, whether Tx-UE should report ACK or NACK to network via PUCCH?

Q2.3.2-2b (old issue): For mode-1 re-transmission grant, if the MAC PDU has NOT been generated (i.e., the initial transmission has NOT been performed), and the re-transmission grant is dropped due to no Rx-UE in active time, whether Tx-UE should report ACK or NACK to network via PUCCH?

    - DRX vs. Resource selection

Left issue on how to specify the behaviour for MAC layer to provide active-time to PHY layer

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200374 | OPPO | Proposal 6 For P15 of [716], MAC layer provides active-time to PHY layer for resource set determination, where the generation of active-time is by UE implementation. |  |
| R2-2200535 | LG Electronics France | Proposal 4. The MAC layer can provide the RX UE’s active time where SL DRX timers are running now or will be running in future (on-duration timer, Inactivity timer, retransmission timer) to the physical layer. |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 15: The format of active time content of RX UE provided by the MAC layer to the physical layer is up to UE implementation |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 16: TX UE can select TX resources within RX UE’s active time consider timers that are running, and timers that can be predicted to running, for both single MAC PDU and multiple MAC PDU cases, and for both initial transmission and retransmission cases. |  |
| R2-2200938 | Ericsson | Proposal 6 When providing active time to the Physical layer, the MAC layer prefilters destinations, to minimize the possibility that transmission to a destination cannot be made due to no overlapping between the SL grant and the active time of the destination. |  |
| R2-2200938 | Ericsson | Proposal 8 For a given destination, what type of active time (i.e., current or future active time) to be included in the active time provided by the MAC layer to the PHY layer is up to Tx UE implementation. |  |
| R2-2201478 | ITL | Proposal 1: MAC layer provide PHY layer with the Rx UE’s current active time and the other DRX related information (e.g., inactivity timer) to decide the Rx UE’s future active time |  |
| R2-2201150 | InterDigital | Proposal 1: RX UE active time for resource selection of an initial transmission resource includes current or future/expected time in which on duration (for all cast types) or inactivity timer (for unicast/groupcast only) at the RX UE are running. |  |
| R2-2201135 | Apple | Proposal 7 How MAC layer provide “active time” to PHY layer should be specified. |  |
| R2-2201061 | ZTE Corporation, Sanechips | Proposal2: For initial transmission for single MAC PDU, the TX UE can select TX resource within RX UE’s active time. How to identify the RX UE’s active time can be up to UE implementation. |  |

This issue was discussed in Post-116 [716], which the following result

Rapporteur Summary: Out of 18 companies

Option-a: 4 (RX UE’s active time where SL DRX timers are running now.)

Option-b: 3 (RX UE’s active time where on duration timer will be running in future.)

Option-c: 0 (RX UE’s active time where inactivity timer will be running in future.)

Option-d: 0 (RX UE’s active time where retransmission timer will be running in future.)

Option-e: 14 (UE implementation)

I.e., clear majority on leave it to UE-implementation instead of specifying all the details.

So moderator suggest a WF as follows.

1) Use normative text to capture that active-time is to be provided by MAC layer to PHY layer

2) Leave the details to UE implementation, including cast-type / destination selection, which timer to define the active-time, which can rely on NOTE

3) further details up to MAC running-CR discussion.

Examples can be as follows (based on 0550):

2>	if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check:

3> if one or multiple SL DRX is configured:

4> indicate SL DRX Active time of UE receiving SL-SCH data to the physical layer.

NOTE 3C:	How the MAC entity determines active time is left to UE implementation.

Q2.3.3-1a (old issue): Do you support to capture the “MAC layer provides active-time to PHY layer” as normative text?

Q2.3.3-1b (old issue): Do you agree to leave cast-type / destination selection, DRX timer selection within the active-time derivation to UE implementation (including the possibility to capture using a NOTE)

Left issue on impact on resource selection due to DRX impact

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200374 | OPPO | Proposal 7 For P16/17 of [716], MAC layer selects resources for (re)transmission based on the resource set reported by PHY layer taking into account of the DRX information, but the detailed selection behaviour is up to UE implementation. |  |
| R2-2200483 | HW | Proposal 11: A retransmission resource can be selected if it is in active time updated according to its prior transmission, or if it can be indicated by a prior SCI. |  |
| R2-2200535 | LG Electronics France | Proposal 1. For single MAC PDU transmission, the TX UE shall select initial transmission resource and retransmission resources in the RX UE’s active time where SL DRX timers are running now or will be running in future (on-duration timer, inactivity timer, retransmission timer). |  |
| R2-2200535 | LG Electronics France | Proposal 2. Resource selection for single MAC PDU transmission can be equally applied to the resource selection for the initial period for the multiple MAC PDUs transmission of the TX UE. |  |
| R2-2200535 | LG Electronics France | Proposal 3. Assuming that the announced periodic transmissions of the Tx UE is considered as the SL DRX active time of the Rx UE, the Tx UE can perform resource selection from the resources (announced periodic transmissions) of the non-initial period for multiple MAC PDU transmission. | Delta part due to the reservation period field |
| R2-2200938 | Ericsson | Proposal 10 For initial transmission and retransmission, both in case of single MAC PDU and multiple MAC PDUs, it is simplest for the TX UE to select resources based on the existing MAC procedures. RAN2 considers other options if their benefits are justified. |  |
| R2-2201061 | ZTE Corporation, Sanechips | Proposal2: For initial transmission for single MAC PDU, the TX UE can select TX resource within RX UE’s active time. How to identify the RX UE’s active time can be up to UE implementation. |  |
| R2-2201478 | ITL | Proposal 2: For the initial transmission, MAC layer should select the resource within the current Rx UE’s active time which would include both SL DRX timers are running now and on-duration timer will be running in future |  |
| R2-2201478 | ITL | Proposal 3: For the retransmission, MAC layer could select the resource within the current & future Rx UE’s active time |  |
| R2-2201150 | InterDigital | Proposal 2: For one-shot unicast/groupcast transmissions, the TX UE selects a resource for the initial transmission from the set of resources in the RX UE’s active time. |  |
| R2-2201150 | InterDigital | Proposal 3: For one-shot unicast/groupcast transmissions, the TX UE selects at least N retransmissions within the RX UE’s active time. Proposal 4: The minimum number of retransmission resources (N) that should be selected from the RX UE’s active time is configured per priority and CBR. | Single paper to propose a minimum number of retransmission resource (N), moderator suggest not to prioritize it for now |
| R2-2201150 | InterDigital | Proposal 5: For one-shot broadcast transmissions, the TX UE selects resources for the initial transmission and all retransmissions within the RX UE’s active time. Proposal 6: For multi-shot transmissions, the TX UE selects the resources for the initial transmission of the first TB of the multi-shot transmission from the active time of the RX UE(s). Same rules for the retransmission resources as for one-shot are applied. | Delta part due to no inactivity/re-tx timer for BC |
| R2-2201150 | InterDigital | Proposal 7: MAC Layer selects resources associated with the active time of at least the highest priority L2 destination ID with data available for transmission and having DRX configured. |  |
| R2-2200938 | Ericsson | Proposal 20 For groupcast, the TX UE can only select the resources for the initial transmission associated with the time in which the on-duration timer at the TX UE is running. | Delta part due to GC |
| R2-2200894 | vivo | Proposal 4: RAN2 to agree that MAC layer should prioritize to select resources in subset 1 for initial transmission and retransmission. |  |
| R2-2200894 | vivo | Proposal 5: RAN2 to discuss whether/when MAC layer can select resources in subset 2 for initial transmission and/or retransmission. |  |

By reading all proposals, there are quite some points for which paper(s) proposed (for which moderator understand proponent looks for normative text finally). These points are summarized in the following table.

NOTE that we have the following agreement

TX UE shall select initial transmission resource only in the RX UE’s active time where SL DRX timers are running now or will be running in future (at least on-duration timer). Further details of active time can be considered later. FFS on spec impact.

|  | Broadcast | Groupcast | Unicast |
| --- | --- | --- | --- |
| Initial-transmission, Single-shot, and Initial-transmission of initial period, Multi-short | On-duration timer ?? + on-duration timer to be running in the future | On-duration timer + inactivity timer + retransmission timer already running ?? + on-duration timer to be running in the future ?? + Inactivity timer to be running in the future ?? + Re-transmission timer to be running in the future | On-duration timer already running ?? + on-duration timer to be running in the future ?? + Inactivity timer to be running in the future ?? + Re-transmission timer to be running in the future |
| Initial-transmission of non-initial period, Multi-short | On-duration timer ?? + on-duration timer to be running in the future ?? + active time due to reservation period field | On-duration timer + inactivity timer + retransmission timer already running ?? + on-duration timer to be running in the future ?? + Inactivity timer to be running in the future ?? + Re-transmission timer to be running in the future ?? + active time due to reservation period field | On-duration timer already running ?? + on-duration timer to be running in the future ?? + Inactivity timer to be running in the future ?? + Re-transmission timer to be running in the future ?? + active time due to reservation period field |
| Re-transmission, Single-shot, and Re-transmission of initial period, Multi-short | On-duration timer ?? + on-duration timer to be running in the future ?? + Retransmission timer to be running in the future | On-duration timer + inactivity timer + retransmission timer already running ?? + on-duration timer to be running in the future ?? + Inactivity timer to be running in the future ?? + Re-transmission timer to be running in the future | On-duration timer already running ?? + on-duration timer to be running in the future ?? + Inactivity timer to be running in the future ?? + Re-transmission timer to be running in the future |
| Re-transmission of non-initial period, Multi-short | On-duration timer ?? + on-duration timer to be running in the future ?? + Retransmission timer to be running in the future ?? + active time due to reservation period field | On-duration timer + inactivity timer + retransmission timer already running ?? + on-duration timer to be running in the future ?? + Inactivity timer to be running in the future ?? + Re-transmission timer to be running in the future ?? + active time due to reservation period field | On-duration timer already running ?? + on-duration timer to be running in the future ?? + Inactivity timer to be running in the future ?? + Re-transmission timer to be running in the future ?? + active time due to reservation period field |

Where the bullet with ?? are the part that may have to be debated based on moderator observation, and considering the discussion in post-116 [716] as follows

Proposal 16: RAN2 should further discuss the options below for the Tx UE’s behaviour to select an initial transmission resource for single MAC PDU transmission.

a)	(9/19)For initial transmission for single MAC PDU, the TX UE can select TX resource within RX UE’s active time where SL DRX timers are running now.

b)	(9/19) For initial transmission for single MAC PDU, the TX UE can select TX resource within RX UE’s active time where on duration timer will be running in future.

c)	(6/19) For initial transmission for single MAC PDU, the TX UE can select TX resource within RX UE’s active time where inactivity timer will be running in future.

d)	(2/19) For initial transmission for single MAC PDU, the TX UE can select TX resource within RX UE’s active time where retransmission timer will be running in future.

e)	(6/19) select resources according to the existing procedure in the MAC

Proposal 17: RAN2 should further discuss the options below for the Tx UE’s behaviour to select a retransmission resource for single MAC PDU transmission.

a)	(9/19) For retransmission for single MAC PDU, the TX UE can select TX resources within RX UE’s active time where SL DRX timers are running now.

b)	(9/19) For retransmission for single MAC PDU, the TX UE can select TX resources within RX UE’s active time where on duration timer will be running in future.

c)	(9/19) For retransmission for single MAC PDU, the TX UE can select TX resources within RX UE’s active time where inactivity timer will be running in future.

d)	(8/19) For retransmission for single MAC PDU, the TX UE can select TX resources within RX UE’s active time where retransmission timer will be running in future.

e)	 (6/19) select resources according to the existing procedure in the MAC.

Moderator understand it is hard to conclude on all details one-by-one given the controversial status and the limited, so would like to suggest a WF that

1) Use normative text to capture that MAC layer will select initial and re-transmission resource considering SL DRX timer that are running and will be running in the future.

2) Leave the details to decide “SL DRX timer that are running and will be running in the future” to UE implementation, including further difference between cast types selection, between destination selection, between initial/re-transmission, between single and multi-shot, which can rely on NOTE

3) further details up to MAC running-CR discussion.

Examples can be as follows (based on 0550):

<for initial transmission>

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] considering SL DRX timer that are running and will be running in the future, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

NOTE 3C:	How the MAC entity determines SL DRX timer that are running and will be running in the future is left to UE implementation.

Q2.3.3-2a (old issue): Do you support to capture the select resource “considering SL DRX timer that are running and will be running in the future” as normative text?

Q2.3.3-2b (old issue): Do you agree to leave further difference between cast types selection, between destination selection, between initial/re-transmission, between single and multi-shot to UE implementation (including the possibility to capture using a NOTE)

Left issue on impact on resource re-selection of retransmission resources due to DRX re-transmission timer.

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200545 | SHARP Corporation | Proposal 1: For resource re-selection of the pre-emption check in SL DRX, the re-selected resource is not earlier than the pre-empted resource in time domain. |  |
| R2-2200545 | SHARP Corporation | Proposal 2: For resource re-selection of the pre-emption check in SL DRX, the time gap between the re-selected resource and the pre-empted resource is not larger than the duration of SL HARQ Retransmission timer. |  |
| R2-2200894 | vivo | Proposal 9: RAN2 further discuss, when pre-emption is allowed, Tx UE does not reselect a resource earlier than the pre-empted resource. |  |
| R2-2200894 | vivo | Proposal 10: RAN2 kindly asks RAN1 to consider the issue of the miss-reception on the resource reselection if the resource revaluation and/or pre-emption are enabled. Details is up to RAN1. |  |
| R2-2200894 | vivo | Proposal 11: If Proposal 10 is agreed, RAN2 sends an LS to RAN1 for feedback (see Annex). |  |
| R2-2200894 | vivo | Proposal 12: Relying on the SCI-based resource reservation, RAN2 studies the determination mechanism for HARQ RTT timer by setting the warm-up window. |  |
| R2-2201150 | InterDigital | Proposal 9: A TX UE which performs re-selection of retransmission resources due to pre-emption ensures that the newly selected re-transmission resource does not occur earlier in time than the pre-empted resource when communicating to an RX UE in DRX |  |
| R2-2201150 | InterDigital | Proposal 10: If RAN2 agrees to the above proposal, send LS to inform RAN1. |  |

Moderator understand it is the result of introducing SCI based RTT timer, and the resource reselection operation is done at MAC layer after receiving set-A reported by PHY layer.

Q2.3.3-3 (old issue): Do you agree that for resource reselection due to pre-emption, the reselected resource should be not earlier than the pre-empted resource in time domain?

Left issue on the need of resource (re)selection trigger considering DRX impact

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200318 | CATT | Proposal 12: For mode 2 Tx UE, resource (re)selection needs to be triggered when there is no SL grant can be used in SL DRX active time for the destination which has SL data available for transmission. And the (re) selected SL grant shall be in SL DRX active time. |  |
| R2-2200483 | HW | Proposal 12: If the current reserved resources do not fall into the SL DRX active time of any destination, or if there is no SL grant in the SL active time of the destination that has data to be sent, resource (re)selection is triggered. |  |
| R2-2200545 | SHARP Corporation | Proposal 3: If SL DRX is configured or re-configured, e.g. leading to the change of active time, MAC entity shall trigger the TX resource (re-)selection. | Single paper for the proposal, moderator suggest not to prioritize for now |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 6: Resource (re)selection triggers to PHY are a) when new data becomes available for transmission and on-duration timer is running; b) when Inactivity timer is (re)started and c) when CSI request is sent to the Rx UE. | Single paper for the proposal, moderator suggest not to prioritize for now |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 7: MAC can trigger resource selection with PHY in the next start of on-duration timer i.e., in the next DRX cycle period when PDB still allows it, if the remaining active time is less than T1. | Single paper for the proposal, moderator suggest not to prioritize for now |
| R2-2200938 | Ericsson | Proposal 9 The MAC layer triggers resource reselection if the MAC layer cannot find sufficient resources in the reported set of resources to be aligned with the active time of the desired destination. |  |
| R2-2201150 | InterDigital | Proposal 8: A TX UE triggers resource (re)selection if there are no selected sidelink grants which fall in the active time of a L2 destination ID having data available for transmission. |  |

This issue has been discussed in At-116 [706], with the following minutes

[Proposal 8] RAN2 to choose among below options for triggering resource (re)selection:

Option 1: If the current reserved resources do not fall into the SL DRX active time of any destination. (10/18)

Option 2: If there is no SL grant in the SL DRX active time of the destination that has data to be sent. (13/18)

Option 3: If the MAC layer cannot find resources in the reported set of resources to be aligned with the active time of any desired Destination. (6/18)

Option 4: No trigger needed. (3/18)

- Skipped.

Moderator suggest to focus on the option-1/2 to make final conclusion

Q2.3.3-4 (old issue): Do you agree to introduce additional resource reselection trigger as follows?

Option 1: If the current reserved resources do not fall into the SL DRX active time of any destination.

Option 2: If there is no SL grant in the SL DRX active time of the destination that has data to be sent.

    - Capability

      - Capability for SL-DRX

Left issue on UE capability for SL-DRX

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200373 | OPPO | Proposal 6 For R17 SL Broadcast and Groupcast, support DTX as conditionally mandatory per-UE capability without capability bit in PC5-RRC, and FFS whether to define DRX capability as mandatory or optional per-UE capability without capability bit in PC5-RRC. |  |
| R2-2200373 | OPPO | Proposal 7 For R17 SL unicast, for the capability of DCR message delivery, follow the same conclude for broadcast and groupcast. |  |
| R2-2200373 | OPPO | Proposal 8 For R17 SL unicast, define DTX/DRX capability for SL unicast data transmission as optional per-UE capability with capability bits in PC5-RRC, with no FR1-FR2 or FDD-TDD differentiation. FFS whether separate capability is needed for DTX and DRX. |  |
| R2-2200373 | OPPO | Proposal 9 For R17 SL unicast, define DTX/DRX capability for SL unicast data transmission as optional per-UE capability with capability bits in Uu-RRC. Follow the conclusion in Proposal 7 above on whether to define separate capability bit for DTX and DRX. |  |
| R2-2200373 | OPPO | Proposal 10 For R17 SL broadcast and groupcast, if Proposal 9 concludes that DRX capability being optional, define per-UE DRX capability bit for SL broadcast and groupcast in Uu-RRC. FFS whether to define separate capability bit for broadcast and groupcast. FFS whether to define capability bit for DTX in Uu-RRC. |  |
| R2-2200373 | OPPO | Proposal 11 For R17 SL, RAN2 discuss whether to define capability of SL-related RTT timer and Re-transmission timer for PDCCH monitoring as conditionally mandatory or optional per-UE capability with capability bit. with no FR1-FR2 or FDD-TDD differentiation. |  |

Firstly, question on whether to define different capability for cast-types, DTX for Tx-UE and DRX for Rx-UE.

Q2.3.4-1a (new issue): Do you prefer to define separate capability for different cast types (except for UC-based DCR message, which is up to Q2.3.4-1c below)?

Option-1: a single capability covering all cast types

Option-2: separate capability for Unicast and for Broadcast + Groupcast

Option-3: separate capability for each cast type

Q2.3.4-1b (new issue): Do you prefer to define separate capability for Tx and Rx for DRX?

Option-1: single capability covering both Tx and Rx side

Option-2: separate capability for Tx and Rx side

Q2.3.4-1c: For UC-based DCR message, do you agree to follow the conclusion of BC related capability?

Secondly, question on the detailed attributive for each capability (regardless of whether combined or separate capability is defined, which will depend on the output of Q2.3.4-1a/b above)

|  | UC | GC | BC |
| --- | --- | --- | --- |
| DTX | Optional per-UE capability with capability bits in PC5-RRC, with no FR1-FR2 or FDD-TDD differentiation with capability bits in PC5-RRC, with no FR1-FR2 or FDD-TDD differentiation | Conditionally mandatory per-UE capability Without capability bit in PC5-RRC With capability bit in Uu-RRC with no FR1-FR2 or FDD-TDD differentiation | Conditionally mandatory per-UE capability Without capability bit in PC5-RRC With capability bit in Uu-RRC with no FR1-FR2 or FDD-TDD differentiation |
| DRX | Optional per-UE capability with capability bits in PC5-RRC, with no FR1-FR2 or FDD-TDD differentiation with capability bits in PC5-RRC, with no FR1-FR2 or FDD-TDD differentiation | Conditionally mandatory per-UE capability Without capability bit in PC5-RRC With capability bit in Uu-RRC with no FR1-FR2 or FDD-TDD differentiation | Conditionally mandatory per-UE capability Without capability bit in PC5-RRC With capability bit in Uu-RRC with no FR1-FR2 or FDD-TDD differentiation |

Q2.3.4-1d (new issue): for DTX + UC case, any aspect in the table that you disagree?

Q2.3.4-1e (new issue): for DTX + GC case, any aspect in the table that you disagree?

Q2.3.4-1f (new issue): for DTX + BC case, any aspect in the table that you disagree?

Q2.3.4-1g (new issue): for DRX + UC case, any aspect in the table that you disagree?

Q2.3.4-1h (new issue): for DRX + GC case, any aspect in the table that you disagree?

Q2.3.4-1i (new issue): for DRX + BC case, any aspect in the table that you disagree?

      - Capability for Uu-DRX

Left issue on UE capability for SL-DRX

| Tdoc | Company | Proposals | Moderator‘s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200373 | OPPO | Proposal 11 For R17 SL, RAN2 discuss whether to define capability of SL-related RTT timer and Re-transmission timer for PDCCH monitoring as conditionally mandatory or optional per-UE capability with capability bit. With no FR1-FR2 or FDD-TDD differentiation. |  |

Q2.3.4-2a (new issue): Do you agree to introduce capability of SL-related RTT timer and Re-transmission timer for PDCCH monitoring?

Q2.3.4-2b (new issue): if yes to 2a above, do you disagree any component of the attributive of this capability (conditionally mandatory, per-UE, without FR1/2 diff, and without FDD/TDD diff)?

    - Companies Input on section 2.3 for Phase-1

Please share your view on the open issue list above (even companies can input using multiple rows for different questions/issues)

| Company | Concerned question | Comment |
| --- | --- | --- |
| Xiaomi |  | There seems to be a missing open issue regarding whether drx-RetransmissionTimerSL is started after expiring drx-HARQ-RTT-TimerSL when the PUCCH(ACK) transmission is dropped. Note we already agreed the PUCCH(NACK) drop case in this meeting as following, drx-RetransmissionTimerSL is started after expiring drx-HARQ-RTT-TimerSL when the PUCCH (NACK) transmission is dropped. [OPPO] I though the Q2.3.1-1 can cover the FFS point. |
| Xiaomi | Q2.3.1-1 | According to the quoted contributions, 2 companies support to start RTX timer and 1 company prefer not to start, while 1 company didn’t explicitly express the view. I think it’s better to list the two options rather than propose not to start directly. |
| Xiaomi | Q2.3.1-3a | I’m confused with the question. What’s the meaning of ‘same’ or ‘different’ here. In the question, the scenario is resource pool is not configured with PSFCH, so it’s always FB-disabled. I only see two kinds of methods, derived from SCI or configured value. Certainly the configured value can’t be the same as the one derived SCI. [OPPO] The Q is limited to configured RTT case only. See if the reformulated Q is comprehensive now. |
| Xiaomi |  | Proposal 11 in R2-2200791 is not included in the open issue list. Proposal 11: If RX UE is not able to monitor SL in the rest time of on-duration or inactivity timer running, e.g. due to UL/SL transmission or lack of reception resource, RX UE sends indication to TX UE. TX UE stop DRX timers associated with RX UE. The proposal intends to resolve the active time misalignment between TX and RX UE. RX UE may not be able to monitor SL if there is SL/UL transmission during active time. But TX UE would still assume RX UE active according to timer running, which may result in RX UE missing data reception. We think this can be included in the open issue list. [OPPO] sorry for missing that, now added into 2.4.2 section below, suggest not to trigger it as an essential issue. |

  - Others

Based on the Chairman guidance on categorization

- Each open issue should be associated with suggested treatment/handling.

  - Company input into Pre117-e-offline (i.e. no company tdocs)

  - Company tdocs invited.

  - CR rapporteur handled issue

  - Other, e.g. immature area, reference to dependency, unclear status etc.

The issues in this section is of category-4.

    - Proposals concluded in 116bis

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 5 For a mode 2 RRC-CONNECTED TX UE, whether TX UE or serving cell of the Tx UE decides the SL DRX configuration of RX UE should be decided and indicated by the network. | 6: For unicast and TX UE in RRC CONNECTED and Mode 2 RA, TX UE determines SL DRX for RX UE. |
| R2-2200318 | CATT | Proposal 1: For SL UC, Tx UE in OOC can determine the SL DRX configuration without relying on pre-configuration. | 3: For IDLE/INACTIVE/OOC UE, It is up to TX UE implementation to set sl-DRX-ConfigUC-PC5. |
| R2-2200318 | CATT | Proposal 2: For SL UC, Tx UE in IDLE/INACTIVE can determine the SL DRX configuration without relying on SIB. | 3: For IDLE/INACTIVE/OOC UE, It is up to TX UE implementation to set sl-DRX-ConfigUC-PC5. |
| R2-2200374 | OPPO | Proposal 4 For P11/12 of [716], RAN2 should discuss the feasible solution to perform the down selection of DRX cycle and on-duration timer before confirming the WA. |  |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 7: An Assistance Information REQ msg from Tx to Rx UE for seeking assistance information is not supported. | 14: The SL DRX assistance information request from Tx UE to Rx UE is not supported in the current release. |
| R2-2200544 | LG Electronics France | Proposal 13: Unicast-specific pre-configuration for SL DRX is not used in the OoC case. |  |
| R2-2200318 | CATT | Proposal 11: SL specific drx-HARQ-RTT-TimerSL is supported in case PSFCH is configured in resource pool and sl-PUCCH-Config is not configured. | 6: drx-HARQ-RTT-TimerSL is supported in case PSFCH is configured in resource pool and sl-PUCCH-Config is not configured. NW can set value as zero or any other value. |
| R2-2200791 | Xiaomi | Proposal 1: Unicast sidelink DRX configuration is per direction for IC. |  |
| R2-2200938 | Ericsson | Proposal 1 Support applying DRX configuration per both directions in addition to per-direction DRX operation. |  |
| R2-2200938 | Ericsson | Proposal 2 DRX operation option (i.e., DRX operation for both direction or per-direction DRX operation) is configurable. |  |
| R2-2200938 | Ericsson | Proposal 11 The priority order of SL DRX Command MAC CE is between SL CSI Reporting MAC CE and data from any STCH. | 1: The priority order of Sidelink DRX Command MAC CE is between Sidelink CSI Reporting MAC CE and data from any STCH. |
| R2-2200938 | Ericsson | Proposal 12 Allow a UE to transmit a SL DRX Command MAC CE alone in a MAC PDU. | 3: For the same pair of L2 SRC/DST ID, the SL DRX command MAC CE can be transmitted alone or with data in the MAC PDU. |
| R2-2200938 | Ericsson | Proposal 13 Do not introduce a specific SR resource or SR configuration for requesting SL resource for transmitting SL DRX Command CE. | 5: RAN2 does not define a separate SR configuration for SL DRX Command MAC CE. |
| R2-2201135 | Apple | Proposal 1 Capture “SL DRX Command MAC CE has a lower priority than SL-SCI reporting MAC CE in LCP“ in MAC specification w/o changing the priority order list in clause 5.22.1.4.1. | 1: The priority order of Sidelink DRX Command MAC CE is between Sidelink CSI Reporting MAC CE and data from any STCH. |
| R2-2201135 | Apple | Proposal 8 RRC_CONNECTED RX UE reports SL-DRX configuration to its serving gNB after RX UE accepting the received sidelink DRX configuration. | 6: UE reports sidelink DRX configuration to its serving gNB, upon accepting sidelink DRX configuration information from the peer UE. |
| R2-2201135 | Apple | Proposal 9 RRC_CONNECTED mode 2 TX UE determining SL DRX configuration w/o gNB involvement should be supported. | 6: For unicast and TX UE in RRC CONNECTED and Mode 2 RA, TX UE determines SL DRX for RX UE. |
| R2-2201523 | Lenovo, Motorola Mobility | Discussion for proposals of [POST116-e][715] Proposal 4: If the RX UE determines the acceptance or rejection of SL DRX configuration for UC received from peer UE, the Rx UE UE reports sidelink DRX configuration to its serving gNB after UE accepting the received sidelink DRX configuration | 6: UE reports sidelink DRX configuration to its serving gNB, upon accepting sidelink DRX configuration information from the peer UE. |
| R2-2200938 | Ericsson | Proposal 22 For unicast, when a TX UE is in RRC_CONNECTED, the serving gNB of the TX UE determines the SL DRX configurations for the RX UE, regardless of whether Mode 1 scheduling or Mode 2 resource allocation is adopted. Proposal 23 For unicast, the RX UE determines by itself whether to accept or reject the SL DRX configurations of the RX UE received from the TX UE. |  |
| R2-2200893 | vivo | Proposal 3 UE reports SL DRX configuration to its serving gNB, only if the UE accepts and successfully completes the SL DRX configuration receiving from its peer UE. |  |
| R2-2200484 | Huawei, HiSilicon | Proposal 1: For the case when PUCCH resource is not scheduled, the UE does not start the drx-HARQ-RTT-TimerSL for the corresponding SL HARQ process. | 6: drx-HARQ-RTT-TimerSL is supported in case PSFCH is configured in resource pool and sl-PUCCH-Config is not configured. NW can set value as zero or any other value. |
| R2-2200484 | Huawei, HiSilicon | Proposal 2: Correct “sl-PUCCH-Config is configured or not” to “PUCCH resource is scheduled or not”. | 6: drx-HARQ-RTT-TimerSL is supported in case PSFCH is configured in resource pool and sl-PUCCH-Config is not configured. NW can set value as zero or any other value. |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 10: support drx-HARQ-RTT-TimerSL in case PSFCH is configured in resource pool and sl-PUCCH-Config is not configured |  |
| R2-2200790 | Xiaomi | Proposal 3: UE set ACK in PUCCH if SL grant for initial transmission was dropped. |  |
| R2-2200938 | Ericsson | Proposal 14 When HARQ feedback is disabled and the resource assignment information is not present in SCI, the HARQ RTT timer should be set to be zero. | 7: UE uses configured sl-drx-HARQ-RTT-Timer value when the resource assignment information for the next re-transmission does not exist in the SCI regardless of whether HARQ feedback is enabled or disabled. |
| R2-2200938 | Ericsson | Proposal 30 SL-specific drx-HARQ-RTT-Timer is not needed when sl-PUCCH-Config is not configured. |  |
| R2-2200938 | Ericsson | Proposal 7 Upon reception of a SL grant, the MAC layer selects a suitable destination based on active time of each destination so that the utilization of the granted SL resources is maximized. | 14: Tx UE should select a destination associated with an Rx UE that is in SL active time for the SL transmission occasion in SL LCP. |
| R2-2201135 | Apple | Proposal 2 drx-HARQ-RTT-TimerSL is not supported if sl-PUCCH-Config is not configured. |  |
| R2-2200318 | CATT | Proposal 7: drx-RetransmissionTimerSL is started after drx-HARQ-RTT-TimerSL expires regardless of whether the unsent PUCCH is ACK or NACK. | 15: drx-RetransmissionTimerSL is started after expiring drx-HARQ-RTT-TimerSL when the PUCCH (NACK) transmission is dropped. |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 17: Confirm drx-RetransmissionTimerSL is started after expiring drx-HARQ-RTT-TimerSL when the PUCCH (NACK) transmission is dropped | 15: drx-RetransmissionTimerSL is started after expiring drx-HARQ-RTT-TimerSL when the PUCCH (NACK) transmission is dropped. |
| R2-2200790 | Xiaomi | Proposal 2: If PUCCH was dropped regardlss NACK or ACK, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. |  |
| R2-2200938 | Ericsson | Proposal 31 In case PUCCH is dropped due to UL/SL prioritization, drx-RetransmissionTimerSL is started after expiring drx-HARQ-RTT-TimerSL when the unsent PUCCH is NACK. |  |
| R2-2201582 | Samsung Research America | [Proposal 1]: SUI is used to report SL DRX configurations to the gNB. | 1: UE uses SUI to report sidelink DRX configuration or sidelink assistance information to its serving gNB. |
| R2-2200938 | Ericsson | Proposal 26 Existing Uu RRC message could be used by the RX UE to report the received SL DRX configurations. |  |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 9 If the RRC CONNECTED UE is configured with sidelink DRX for SL groupcast/broadcast, it shall report the related SL DRX configuration to the serving cell, then the serving cell can decide whether to update Uu DRX. |  |
| R2-2200319 | CATT | Proposal 1: TX/RX UE selects the maximum length among the on-duration timer associated with the QoS profile(s) of selected DRX cycle. |  |
| R2-2200319 | CATT | Proposal 2: When UE fails to decode the MAC PDU in GC, it will not trigger the inactivity timer, the retransmission timer will be initialized and keep UE in active time as legacy behavior. |  |
| R2-2200319 | CATT | Proposal 3: Rel-17 Tx UE in RRC_CONNECTED state should report its SL DRX configuration of SL GC/BC to network. |  |
| R2-2200319 | CATT | Proposal 4: For Rel-17 Tx UE using SL GC/BC, it can notify which L2 destination ID will use SL DRX and the detailed sidelink DRX configuration to gNB via sidelinkUEInformationNR. |  |
| R2-2200483 | HW | Proposal 9: the sl-drx-startoffset can be determined with the following equation: n=DST L2 ID MODE N where N is the total number of sl-drx-startoffset values, and n is an index in the N sl-drx-startoffset values |  |
| R2-2200528 | Intel Corporation | Proposal 3: RAN2 is proposed to confirm the working assumption on down-selection for DRX cycle and on-duration timer, i.e. down selection of DRX cycle and on-duration timer for GC/BC is needed in case multiple QoS profiles are associated with the same L2 DST ID. |  |
| R2-2200528 | Intel Corporation | Proposal 4: For down-selection of on-DurationTimer and DRX cycle, the UE selects the longest timer duration/cycle value to ensure that missed transmissions during DRX sleep are minimized. |  |
| R2-2200535 | LG Electronics France | Proposal 7. RAN2 should not force a down-selection of one SL DRX cycle/one SL DRX onduration timer that may degrade the performance of power-saving of the UE or may cause a problem of not satisfying the QoS requirements of some SL GC/BC services. |  |
| R2-2200535 | LG Electronics France | Proposal 8. As a compromised solution, RAN2 should support both UE behaviour that can down-select one SL DRX cycle among multiple SL DRX cycles and UE behaviour that can select the shortest “N” SL DRX cycles according to its (pre)configuration. |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 13: TX/RX UE selects the length of the on-duration timer associated with the same QoS profile of selected DRX cycle |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 14: Inactivity timer is not (re)started in case of MAC PDU decoding failure (i.e., only L1 DST ID is available) for GC |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 18: #113-e meeting's agreement also apply to GC NACK only case: If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization or ACK) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 19: sl-drx-RetransmissionTimer is started if PSFCH (NACK) transmission is dropped (due to UL/SL prioritization) in GC NACK only |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 21: sl-drx-StartOffset (ms) = DST L2 ID MOD (number of offset values) |  |
| R2-2200790 | Xiaomi | Proposal 1: UE reports the SL DRX information, e.g. QoS profile, of the broadcast and groupcast destination, which UE is interested in reception. |  |
| R2-2200791 | Xiaomi | Proposal 12: Confirm the WA pf down-selection for DRX cycle and on-duration for GC/BC when multiple QoS profiles are associated with the same DST L2 ID. |  |
| R2-2200791 | Xiaomi | Proposal 13: Select the length of the on-duration timer associated with the QoS profile, which is associated with the smallest DRX cycle. |  |
| R2-2200791 | Xiaomi | Proposal 14: In NACK only HARQ feedback, UE does not start the HARQ RTT timer, if PSFCH is not transmitted. |  |
| R2-2200894 | vivo | Proposal 6: Due to many uncertain specification efforts required in Option-1, RAN2 confirms Option-5 to determine the sl-drx-startoffset. |  |
| R2-2200894 | vivo | Proposal 7: Introduce the equation to set sl-drx-SlotOffset as follows: - sl-drx-SlotOffset (ms)= 1/32 ◊ (DST L2 ID MOD 32) |  |
| R2-2200894 | vivo | Proposal 8: TX/RX UE selects the length of the on-duration timer associated with the same QoS profile of selected DRX cycle. |  |
| R2-2200938 | Ericsson | Proposal 17 For GC and BC, no need to down-select one DRX cycle from available DRX cycles. |  |
| R2-2200938 | Ericsson | Proposal 18 For GC and BC, no need to down-select one DRX on-duration timer from available DRX on-duration timers. |  |
| R2-2200938 | Ericsson | Proposal 19 For GC and BC, determine the sl-drx-startoffset using Option-1, i.e., derive an index to the N sl-drx-startoffset values based on DST L2 ID. |  |
| R2-2200938 | Ericsson | Proposal 27 For groupcast or broadcast, the TX UE may report assistance information (e.g., SidelinkUEInformationNR) to its serving gNB regarding traffic type (e.g., associated L2 ID or PQI). |  |
| R2-2200938 | Ericsson | Proposal 28 For groupcast or broadcast, the RX UE may report assistance information (e.g., SidelinkUEInformationNR) to its serving gNB regarding SL DRX configurations adopted for its GC/BC reception. |  |
| R2-2200938 | Ericsson | Proposal 29 For groupcast or broadcast, no additional mechanism is needed in order to achieve alignment of Uu DRX and SL DRX. |  |
| R2-2201061 | ZTE Corporation, Sanechips | Proposal3: for GC NACK only,if the RX UE does not transmit PSFCH for a HARQ enabled transmission, RX UE also need to start the HARQ RTT timer if RX UE does not transmit the feedback. |  |
| R2-2201135 | Apple | Proposal 5 To solve the down-selection among multiple SL_DRX configurations for the same GC/BC L2 destination ID: a): onDurations of different PQI set to identical value; b): DRX cycle can be chosen from the set [T, 2T…, 2nT] as the busiest cycle T used for the PQI corresponding to the most stringent latency requirements. |  |
| R2-2201135 | Apple | Proposal 6 UE behavior for decoding failure is to not (re)start inactivity timer. No further optimization is needed. |  |
| R2-2201135 | Apple | Proposal 11 For mode 1 RRC_CONNECTED UE engaged with SL broadcast/groupcast, if alignment is desired, in principle, gNB should align the Uu DRX configuration to match the SL DRX configuration. FFS exceptions. |  |
| R2-2201152 | InterDigital | Proposal 5: RAN2 confirms the working assumption of the need of down-selection for DRX cycle and on-duration for GC/BC when multiple QoS profiles are associated with the same L2 ID. |  |
| R2-2201152 | InterDigital | Proposal 6: The TX/RX UE determines the DRX cycle applied for groupcast/broadcast transmissions associated with a specific L2 destination ID as the minimum DRX cycle configured for any of the QoS profiles associated with that L2 destination ID |  |
| R2-2201152 | InterDigital | Proposal 7: The TX/RX UE determines the on duration applied for groupcast/broadcast transmissions associated with a specific L2 destination ID as the maximum on duration configured for any of the QoS profiles associated with that L2 destination ID |  |
| R2-2201582 | Samsung Research America | [ [Proposal 4]: For GC/BC, SL DRX configuration is not needed in the report. |  |
| R2-2201585 | Samsung Research America | [Proposal 1]: For GC/BC, RAN2 is asked to confirm working assumption (down-selection for DRX cycle and on-duration for GC/BC when multiple QoS profiles are associated with the same DST L2 id) as an agreement. |  |
| R2-2201585 | Samsung Research America | [Proposal 2]: For GC/BC, it is proposed to down-select to the shortest DRX cycle length and the longest on-duration length if down-selection is needed. |  |
| R2-2200938 | Ericsson | Proposal 35 A RX UE may determine SL DRX to be disabled when a new service becomes available and the TX profile of the new service doesn’t allow SL DRX to be applied. | Moderator understand this issue is concluded by the following agreement: For GC/BC only communication, a Rel-17 RX UE determines SL DRX is used if all service types/L2 ids of interest have an associated TX profile corresponding to support of SL DRX. A Rel-17 RX UE enables SL DRX operation for a service type/L2 id with the associated TX profile. |

    - Other proposals

| Tdoc | Company | Proposals | Moderator’s remark and recommendation |
| --- | --- | --- | --- |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 3 The TX UE can reconfigure SL DRX configuration for the RX UE by its own when the timer T310/T311 is running. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 4 RAN2 is suggested to discuss following two options during handover procedure for TX UE: Option 1: The TX UE can reconfigure SL DRX configuration for the RX UE by its own when the timer T304 is running. Option 2: The target cell provides the SL DRX configuration for the RX UE to the TX UE via source cell during the handover procedure. Then the TX UE delivers the received SL DRX configuration from target cell to the RX UE after TX UE completes handover to the target cell. | Moderator assume we can rely on legacy handling during HO as for non-DRX SL configuration |
| R2-2200318 | CATT | Proposal 6: For sidelink unicast, the desired DRX configuration should be sent based on each L 2 destination ID. | Moderator assume it is already this manner |
| R2-2200535 | LG Electronics France | Proposal 9. If the SL DRX command MAC CE is triggered, but a UE determines that the grant will not be allocated within the current SL DRX cycle in consideration of processing time, etc., the UE should be able to cancel the triggered BSR (or pending SR). If a grant for the SL DRX command MAC CE is allocated in the next SL DRX cycle, the grant should be discarded and the transmission of the SL DRX command MAC CE should be cancelled. | Moderator understand DRX command MAC CE is not DRX cycle specific, so no issue here |
| R2-2200544 | LG Electronics France | Proposal 8: The serving cell of TX UE reports assistant information received from TX UE to the target cell for TX UE’s handover. | Given SUI message forwarding will happen during HO preparation, moderator assume the info included SUI would be forwarded as well |
| R2-2200544 | LG Electronics France | Proposal 9: When TX UE performs handover to the target cell, the target cell gives Uu DRX and SL DRX configuration for the TX UE through serving gNB. | Moderator understand it is the same procedure as for non-DRX configuration |
| R2-2200544 | LG Electronics France | Proposal 10: After TX UE completes handover to the target cell, the TX UE delivers SL DRX configuration received from target gNB through serving gNB to the RX UE. | Moderator understand it is the same procedure as for non-DRX configuration |
| R2-2200544 | LG Electronics France | Proposal 12: When RRC_IDLE/INACTIVE or OoC TX UE supporting SL DRX becomes RRC_CONNECTED, if the serving gNB of TX UE doesn’t configure to provide SL DRX related information, TX UE can keep performing priori SL DRX. | Moderator understand it is the same procedure as for non-DRX configuration |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 9: For unicast transmissions, one additional condition is required for the destination selection, that Tx UE considers only those SL LCH(s) for the selection of the Destination whose corresponding DRX ActiveTime matches with the allocated SL resources, e.g. SL resources allocated by gNB are within the DRX ActiveTime of the SL LCH(s). | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2201135 | Apple | Proposal 12 For mode 1 RRC_CONNECTED UE engaged with SL unicast, if alignment is desired, TX UE incorporated the alignment requirement as QoS latency requirements and shared with the peer RX UE during the SL DRX negotiation procedure. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200938 | Ericsson | Proposal 24 For unicast and Tx UE is in RRC_CONNECTED, alignment between Uu DRX of the Tx UE and SL DRX of the Rx UE is up to the serving gNB of the TX UE. Proposal 25 Alignment between Uu DRX of the Rx UE and SL DRX of the Rx UE is up to the serving gNB of the RX UE. | Since it is mainly about gNB implementation, suggest not to prioritize for now |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 4: In groupcast communication, a new transmission may not be made when drx-HARQ-retransmission timer is running. | Considering it has been discussed at 116, moderator suggest not to prioritize this issue For GC: - Option1: Initial transmission is allowed during the time when on-duration and inactivity timer run. - Option2: Initial transmission is allowed during any active time. Option 1: Qualcomm, Lenovo, IDT, Huawei, Ericsson (5) Option 2: LG, OPPO, Nokia, Intel, Apple, MediaTek, NEC, ZTE, Fraunhofer, ASUSTek (10) |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 5: Latest DRX configuration is sent to PHY for resource selection triggers for a certain destination. | Moderator understand the conclusion is to send active time instead of DRX configuration to PHY |
| R2-2200318 | CATT | Proposal 13: For mode 1 Tx UE, SL SR/BSR needs to be triggered when there is no SL grant in SL DRX active time for the destination which has SL data available for transmission. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 1: Sensing procedure is designed from a Tx UE’s perspective (as in legacy) even when a DRX configuration is in use. | Moderator understand it is more of R1 scope on how to design sensing operation. |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 2: Sensing does not affect the current MAC timers for DRX i.e. SL-on-duration timer start should be according to DRX cycle configuration agreed among the peer device(s) | Moderator understand it is more of R1 scope on how to design sensing operation. |
| R2-2200483 | HW | Proposal 20: RAN2 to confirm that SL-DRX can be reused for L2 relay-related ProSe discovery without additional specification effort. |  |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 1: UE does not (re)start Uu inactivity timer when receive a new SL scheduling for broadcast transmission. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200786 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN2 to support SL sync search optimization for power saving at SL DRX UEs. Proposal 2: A UE may perform full SL sync search only when the current SyncRef UE is potentially no longer suitable or when there may potentially be a significantly more suitable SyncRef UE in the vicinity. Proposal 3: PSBCH message conveys an indication to perform reselection of synchronization reference with full SL sync search. Proposal 4: RAN2 to consider PSCCH/PSSCH transmissions for conveying SL sync related information such as SLSS ID and InCoverage indicator. Proposal 5: RAN2 to consider a group specific SyncRef UE for SL DRX UE group. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200790 | Xiaomi | Proposal 5: TX UE indicate dropped grant to gNB by HARQ process ID included in the DCI, which scheduled dropped SL grant. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200791 | Xiaomi | Proposal 10: RAN2 to specify TX UE’s behaviour regarding sidelink DRX maintenance, i.e. TX UE shall maintain DRX timer running associate with RX UE to determine RX UE’s active time. | Moderator suggest to rely on MAC running-CR discussion for the detailed discussion. |
| R2-2200894 | vivo | Proposal 1: LCP enhancements for ensuring a TX UE transmits data in the active time of an RX UE are not applied when the DRX is not operated on related Destination(s). | Moderator share the view yet understand the current agreement was made for DRX case so aligned with the intention, suggest to discuss the details in running-CR discussion. |
| R2-2201061 | ZTE Corporation, Sanechips | Proposal4: There is no need to consider the case that no SL slots are available in UE’s active time. |  |
| R2-2201458 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN2 to discuss whether/how to allow for lower priority logical channels to be used for the selection of destination. Proposal 2: The decision on whether to prioritise a lower priority logical channel can be based on QoS parameters, remaining active time, or whether more resources are provided in the sidelink grant. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2201152 | InterDigital | Proposal 8: RAN2 concludes that DRX for L2 SL Relay is not addressed in Rel17 since the non-relay case was prioritized in Rel17. |  |
| R2-2201624 | Qualcomm Finland RFFE Oy | Proposal 2: Random selection may be used for resource selection triggered by an SL CSI report when there is no resource candidate in a dedicated resource pool selected for SL CSI report. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2201135 | Apple | Proposal 13 Uu/SL DRX alignment shall be on a best-effort basis and shall not sacrifice Uu radio performance. |  |
| R2-2200483 | HW | Proposal 5: For SL groupcast, initial transmission is only allowed during the time when onduration timer or inactivity timer is running, and retransmission of a SL process is only allowed during the time when onduration timer, inactivity timer, or the retransmission timer of this SL process is running. | Considering it has been discussed at 116, moderator suggest not to prioritize this issue For GC: - Option1: Initial transmission is allowed during the time when on-duration and inactivity timer run. - Option2: Initial transmission is allowed during any active time. Option 1: Qualcomm, Lenovo, IDT, Huawei, Ericsson (5) Option 2: LG, OPPO, Nokia, Intel, Apple, MediaTek, NEC, ZTE, Fraunhofer, ASUSTek (10) |
| R2-2200483 | HW | Proposal 3: RAN2 to discuss the following two methods to avoid the packet loss in RX UE caused by SL HARQ feedback disabled: - Option 1: When, in mode 1, the RX UE receives the indication that the TX UE will request retransmission resource for a HARQ feedback disabled SL process, or when, in mode 2, RX UE receives a SCI indicating both HARQ feedback disabled and the subsequent blind retransmission resource(s), RX UE starts the corresponding retransmission timer in SL DRX upon HARQ RTT timer expiry regardless of whether or not the data is decoded successfully. - Option 2: If RX UE receives a SCI indicating HARQ feedback disabled, RX UE starts retransmission timer in SL DRX upon HARQ RTT timer expiry regardless of whether or not the data is decoded successfully. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 2 Differentiating sidelink resource pool for SL DRX-capable UE and SL DRX-incapable UE should be considered. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200344 | NEC Corporation | Proposal 3 When TX UE doesn’t receive drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer from assistance information, TX UE considers that RX UE is ok with any drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer. | Moderator understand this is straightforward if they are not included in the assistance information. |
| R2-2200483 | HW | Proposal 1: Apart from desired SL DRX configuration, the SL DRX assistance information can also include the Uu DRX configuration of RX UE, configured SL DRX configurations for other SL connections of the RX UE, the SL DRX configurations configured for its RX UE(s), and the power saving requirement of the RX UE. | Considering the related proposal has been discussed in Post-115 [716] Q5.1-1, where only minority support option-3/4, moderator think no need to prioritize the issues for now. |
| R2-2200264 | ZTE Corporation, Sanechips | Proposal 6 All the current SL DRX configuration of the other PC5-S connections can be included in the assistance information. | Considering the related proposal has been discussed in Post-115 [716] Q5.1-1, where only minority support option-3/4, moderator think no need to prioritize the issues for now. |
| R2-2200318 | CATT | Proposal 15: The DRX timers, at least including on-duration timer, need to be extended if the logic SL slot is not sufficient in the active time calculated by physical slot. | Moderator suggest not to prioritize this issue given there are negative voice on this direction, + there are left issues on whether / how to define the threshold, and how to handle the case that the delayed subframe entering into the next cycle |
| R2-2200483 | HW | Proposal 6: When the SL DRX timers are calculated in the unit of physical slot, the following solutions can be considered to overcome the SL service transmission performance degradation problem: - Option 1: allow to extend the SL DRX timer when the number of “available slots” in the original running time is smaller than a threshold or the number of “unavailable slots” in the original running time is larger than a threshold. - Option 2: if the start time of onduration/inactivity/retransmission timer does not lie within available slot, delay the start time to the nearest available slot. | Moderator suggest not to prioritize this issue given there are negative voice on this direction, + there are left issues on whether / how to define the threshold, and how to handle the case that the delayed subframe entering into the next cycle |
| R2-2201585 | Samsung Research America | [Proposal 3]: For GC/BC, SL-QoS-Profile-r16 is reused to map between SL DRX cycle length and QoS profile. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2201585 | Samsung Research America | [Proposal 4]: For GC/BC, index indicating the order of SL-QoS-Profile in Rel-16 can be used. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200319 | CATT | Proposal 5: For GC with DRX configuration, Tx UE will transmit the initial transmission at any active time (including the time for on duration timer, inactivity timer and retransmission timer). | Considering it has been discussed at 116, moderator suggest not to prioritize this issue For GC: - Option1: Initial transmission is allowed during the time when on-duration and inactivity timer run. - Option2: Initial transmission is allowed during any active time. Option 1: Qualcomm, Lenovo, IDT, Huawei, Ericsson (5) Option 2: LG, OPPO, Nokia, Intel, Apple, MediaTek, NEC, ZTE, Fraunhofer, ASUSTek (10) |
| R2-2200415 | Lenovo, Motorola Mobility | Proposal 11: Solutions to avoid resource congestion, half duplex issue at the beginning of active time need to be discussed. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2201523 | Lenovo, Motorola Mobility | Proposal 3: Solutions to avoid resource congestion, half duplex issue at the beginning of active time need to be discussed. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200373 | OPPO | Proposal 17 RAN2 discuss how for network to perform mode-1 scheduling for SL GC considering inactivity timer for transmission (i.e., inactivity timer for UE-A => UE-B direction) can be (re)started upon reception of new data with the same destination ID (i.e., due to new data of UE-B => UE-A direction). | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200762 | Lenovo, Motorola Mobility | Proposal 3: RAN2 to discuss how to ensure that Rx UE doesn’t start the sl-drxInactivityTimer for SL CG allocations and mode 2 multi-shot transmission. | Since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200544 | LG Electronics France | Proposal 1: TX profile should include the following information at least: - Release identification - SL DRX ON/OFF | Moderator understand this is already concluded in last RAN2 meeting. |
| R2-2200483 | HW | Proposal 15: In Rel-17, TX profile is used to identify being SL DRX compatible or incompatible. | Moderator understand this is already concluded in last RAN2 meeting. |
| R2-2200483 | HW | Proposal 16: RAN2 to confirm TX profile to be provided with L2 ID. | Moderator suggest to deprioritize the issues already captured in SA2 spec |
| R2-2200483 | HW | Proposal 17: TX profile mechanism can be reused directly for DCR message. | Moderator share the view, yet understand the existing agreement already align with the intention. |
| R2-2200938 | Ericsson | Proposal 33 RAN2 to adopt a new term e.g., “communication profile” to replace the term “TX profile”. | Moderator suggest to deprioritize the terminology issue |
| R2-2200938 | Ericsson | Proposal 20 For groupcast, the TX UE can only select the resources for the initial transmission associated with the time in which the on-duration timer at the TX UE is running. | Considering it has been discussed at 116, moderator suggest not to prioritize this issue For GC: - Option1: Initial transmission is allowed during the time when on-duration and inactivity timer run. - Option2: Initial transmission is allowed during any active time. Option 1: Qualcomm, Lenovo, IDT, Huawei, Ericsson (5) Option 2: LG, OPPO, Nokia, Intel, Apple, MediaTek, NEC, ZTE, Fraunhofer, ASUSTek (10) |
| R2-2200894 | vivo | Proposal 2: RAN2 to discuss the timing when MAC layer should indicate active time to PHY layer, e.g. : - When DRX configuration is changed - When resource (re)selection is triggered - When current active time is changed | Moderator understand the 2nd one is aligned with the agreement so far, while for the first and third one, since just a single paper on this direction, suggest not to prioritize for now |
| R2-2200894 | vivo | Proposal 3: MAC layer indicates the ratio threshold that needs to be satisfied for candidate resources provided by PHY to be within indicated active time, to ensure there are enough resources applicable for MAC layer to select. | Moderator understand it is more of R1 scope to discuss. |
| R2-2200483 | HW | Proposal 13: The default SL BC/GC DRX configuration should always be configured by NW. | Moderator understand this can be discussed in RRC Running CR |
| R2-2200938 | Ericsson | Proposal 36 Upon arrival of a new service whose TX profile doesn’t allow SL DRX to be applied, UE may inform this to its neighbour UEs which have unicast connections to the UE and the gNB if the UE is in RRC CONNECTED. | Since this is the single paper proposing this, moderator suggest not to prioritize it for now. |
| R2-2200791 | Xiaomi | Proposal 11: If RX UE is not able to monitor SL in the rest time of on-duration or inactivity timer running, e.g. due to UL/SL transmission or lack of reception resource, RX UE sends indication to TX UE. TX UE stop DRX timers associated with RX UE. | Since this is the single paper proposing this, moderator suggest not to prioritize it for now. |

Conclusions

The contributions submitted to AI 8.15.2 are summarized above, with moderator comments. Moderator recommendations are as follows.

Reference

- R2-2200007	Summary of [POST116-e][718][V2X SL] SL DRX configuration (Ericsson)	Ericsson	discussion

- R2-2200045	Summary of [POST116-e][715][V2X/SL] RRC open issues	Huawei, HiSilicon (Rapporteur)	discussion

- R2-2200051	Summary of [POST116-e][716][SL] MAC open issues	LG Electronics Inc. (Rapporteur)	discussion

- R2-2200264	Discussion on remaining issues of SL DRX	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

- R2-2200318	Leftover Issues for Sidelink Unicast DRX	CATT	discussion	Rel-17	NR_SL_enh-Core

- R2-2200319	Leftover issues for Sidelink GCBC DRX	CATT	discussion	Rel-17	NR_SL_enh-Core

- R2-2200344	Further discussions on leftover issues of sidelink DRX configuration	NEC Corporation	discussion

- R2-2200345	Further discussions on sidelink MAC open issues	NEC Corporation	discussion

- R2-2200373	Discussion on DRX left issues	OPPO	discussion	Rel-17	NR_SL_enh-Core

- R2-2200374	Discussion on DRX left issues from [716] [718]	OPPO	discussion	Rel-17	NR_SL_enh-Core

- R2-2200415	SL DRX CP aspects	Lenovo, Motorola Mobility	discussion	NR_SL_enh-Core	Revised

- R2-2200483	Remaining issues for sidelink DRX	Huawei, HiSilicon	discussion	Rel-17	NR_SL_enh-Core

- R2-2200484	Remaining issues of SL communication impact on Uu DRX	Huawei, HiSilicon	discussion	Rel-17	NR_SL_enh-Core

- R2-2200528	Leftover aspects on SL DRX	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

- R2-2200530	On SL DRX and candidate resource selection	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

- R2-2200535	Discussion on remaining issues for SL DRX	LG Electronics France	discussion	Rel-17	NR_SL_enh-Core

- R2-2200536	Consideration on sidelink DRX for unicast	LG Electronics France	discussion	Rel-17	NR_SL_enh-Core	Withdrawn

- R2-2200544	Consideration on sidelink DRX for unicast	LG Electronics France	discussion	Rel-17

- R2-2200545	Discussion on resource (re-)selection in SL DRX	SHARP Corporation	discussion	NR_SL_enh-Core

- R2-2200749	Discussion on remaining issues regarding Sidelink DRX	ASUSTeK	discussion	Rel-17	NR_SL_enh-Core

- R2-2200762	Remaining MAC issues for SL DRX	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2200786	NR Sidelink Synchronization Reference Search Optimization at UE for Power Saving	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh-Core

- R2-2200790	Discussion on Uu impact	Xiaomi	discussion

- R2-2200791	Discussion on Sidelink DRX open issues	Xiaomi	discussion

- R2-2200893	RRC remaining issues on SL DRX	vivo	discussion	Rel-17

- R2-2200894	MAC remaining issues on SL DRX	vivo	discussion	Rel-17

- R2-2200938	Remaining aspects of SL DRX	Ericsson	discussion	Rel-17	NR_SL_enh-Core

- R2-2201061	Discussion on remaining issues of SL DRX timers	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

- R2-2201135	Discussion on remaining issues on SL-DRX	Apple	discussion	Rel-17	NR_SL_enh-Core

- R2-2201150	Resource Selection Considering DRX	InterDigital	discussion	Rel-17	NR_SL_enh-Core

- R2-2201151	Consideration of the Active Time for Periodic Transmissions	InterDigital, Ericsson, ZTE, AsusTek, Huawei, HiSilicon, Lenovo, Motorola  Mobility, Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_enh-Core

- Revised in R2-2201635

- R2-2201635	Consideration of the Active Time for Periodic Transmissions	InterDigital, Ericsson, ZTE, AsusTek, Huawei, HiSilicon, Lenovo, Motorola  Mobility, Nokia, Nokia Shanghai Bell, Samsung	discussion	Rel-17	NR_SL_enh-Core

- R2-2201152	Remaining Aspects on SL DRX	InterDigital	discussion	Rel-17	NR_SL_enh-Core

- R2-2201458	SL data transmission considering SL DRX active time	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh-Core	R2-2110747

- R2-2201478	Resource selection considering SL DRX 	ITL	discussion

- R2-2201523	SL DRX CP aspects	Lenovo, Motorola Mobility	discussion	NR_SL_enh-Core	R2-2200415

- R2-2201582	UE report on SL DRX for Uu DRX alignment	Samsung Research America	discussion

- R2-2201585	Remaining details for GC/BC	Samsung Research America	discussion

- R2-2201624	Discussion on Remaining Design Aspects for SL DRX	Qualcomm Finland RFFE Oy	discussion