3GPP TSG-RAN WG2 Meeting #117-e		R2-220xxxx

Electronic, 21st Feb. – 3rd Mar. 2022

Title:	[Draft] Reply LS to RAN4 on RLM/BFD relaxation for ePowSav

Response to:	R2-2202168_R4-2202769, R2-2203982_R4-2206790

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	vivo [to be RAN2]

To:	RAN4

Cc:

- Contact Person:

- Name:	                Chenli

E-mail Address:	 Chenli5g@vivo.com

- 1. Overall Description:

RAN2 would like to thank RAN4 for the LS on signalings for enabling RLM and BFD relaxation in R17 UE power saving. Based on further RAN2 progress, RAN2 discussed signaling for RLM/BFD relaxation, and achieved the following conclusions:

- The configuration for RLM relaxation feature and BFD relaxation feature are provided in SpCellConfig and SpCellConfig/ScellConfig, respectively. - Low mobility criterion is configured in NR Pcell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC. - Introduce optional parameter(s) of offset on Qx (Qin for RLM relaxation and FFS for BFD relaxation) for good serving cell quality criterion in dedicated signaling. If the offset is absent, a pre-defined value is used (e.g. FFS [0dB]). FFS on stage-3 details (i.e. value range of parameters). - Serving cell quality criterion is configured provided in PCell/PScell for RLM relaxation and in PCell/PScell for BFD relaxation (regarding Scell waiting for R4 input).

Besides, RAN2 also discussed the mechanism for RLM/BFD relaxation, and achieved the following conclusions:

Working Agreement: - UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. - The feature is configured by RRC dedicated signalling, this is the only enable disable function that is supported.

- 2. Actions:

- To RAN WG4

- RAN2 kindly request RAN4 to take the above information into account during the following work, and study/specify the corresponding requirements/approaches for RLM/BFD relaxation, and provide feedback, if any.

- 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #118-e		16th – 27th May 2022				e-Meeting

TSG RAN WG2 Meeting #119		TBD								TBD