Page 4
Draft prETS 300 ???: Month YYYY


3GPP TSG-RAN WG2 Meeting #117-e
R2-220xxxx
E-meeting, February 21 - March 3, 2022
Title:
DRAFT LS Reply to CT1 on Small Data Transmission
Response to:
R2-2203859/ C1-221891) NAS's trigger for resume for SDT
Release:
Release-17
Work Item:
NR_SmallData_INACTIVE-Core
Source:
Apple [To be RAN2]
To:
CT1
Cc:

Contact Person:


Name:
Fangli XU
E-mail Address:
fangli_xu@apple.com            
Send any reply LS to:
3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org
Attachments:
R2-2204234 (38.300 stage-2 CR: Introduction of SDT)

1. Overall Description:

RAN2 thanks CT1 for their LS in C1-221891 on the topic of NAS's trigger for resume for SDT.
RAN2 has made further agreements related to AS/NAS interaction in RAN2#117 meeting as follows: 
	1) If UE detects an SDT failure of ongoing SDT session for the transfer of NAS message, RRC informs NAS about the failure for NAS message transfer. 
2) For non-SDT data arrival indication, ResumeCause value is included in UAI as an optional IE (and hence is provided to the network if upper layers provide it to the AS). If NAS provides the RRC establishment cause, the UE shall include it in resume cause.


 RAN2 has agreed the following overall framework for SDT:  

· 
Small Data Transmission (SDT) is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state).
· 
During an SDT session, the UE is kept in RRC_INACTIVE state, whilst exchanging UL and DL packets over radio bearers configured for SDT; when NAS triggers UL data/signalling to be transmitted over radio bearers configured for SDT, the AS responds to the trigger from NAS layer only after the SDT session is completed or when SDT failure is detected. The AS does not indicate to upper layers whether the suspended RRC connection has been resumed before that. 
·  If the SDT procedure failure is detected, the UE transitions to RRC_IDLE and AS informs NAS about the resume failure
The agreed RAN2 stage-2 CR for SDT in R2-2204234 is attached for reference. 
2. Actions:

To CT1 group:

Action: RAN2 respectfully asks CT1 to take the above information into account. 
3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #118-e   16-27 May 2022
Electronic Meeting

TSG-RAN2 Meeting#119-e    22-26 Aug 2022
TBD

1/1


