3GPP TSG-RAN2 Meeting #118-e								R2-2206712

# Electronic, 9th – 20th May, 2022

Source: 		Email discussion Rapporteur (Huawei, HiSilicon)

Title: 	Summary for [Post118-e][502] UP open issues and CR to 38.321 (Huawei)

Agenda item:	6.6.2

Document for: 	Discussion and Decision

General

This document contains the list of comments made during the review of the MAC CR for SDT

Contacts

| Name | Company | Email address |
| --- | --- | --- |
| SeungJune Yi | LG Electronics | seungjune.yi@lge.com |

Comments

| Companies | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| LG01 | 5.2 3> when the Contention Resolution is considered successful for Random Access procedure when the CG-SDT procedure is ongoing: Double “when” is misleading | Change the second “when” to “while”. 3> when the Contention Resolution is considered successful for Random Access procedure while the CG-SDT procedure is ongoing: |  |
| LG02 | 5.2 1> when instruction from the upper layer has been received for indicating the expiry of the cg-SDT-TimeAlignmentTimer: The text is not aligned with RRC text. instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running; | Align the text between RRC and MAC. Either using “expiring” or “stopping”. If we choose to keep the RRC text, the MAC spec should keep the original text, i.e. “stopping”. |  |
| LG03 | 5.8.2 1> if, after initial transmission for CG-SDT with CCCH message has been performed according to clause 5.4.1, PDCCH addressed to the MAC entity's C-RNTI has not been received, and the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for initial transmission for CG-SDT with CCCH message (i.e., retransmission of initial transmission of CG-SDT); 3> if this is the initial transmission of CG-SDT with CCCH message after the CG-SDT procedure is initiated as in clause 5.27: (i.e., SSB selection for initial transmission for CG-SDT) 3> else if PDCCH addressed to C-RNTI has been received after the initial transmission of CG-SDT with CCCH message: (i.e., SSB for subsequent new transmission for CG-SDT) Three highlighted texts are not aligned. | Remove “SSB selection for” from the second text and remove “SSB for” from the third text. |  |
| LG04 | 5.27.2 The MAC entity shall: 1> store the RSRP of the downlink pathloss reference derived based on the measObject configured for the Serving Cell as in TS 38.331 [5]. It is not clear when the MAC entity stores the RSRP. | Add the timing when the MAC entity stores the RSRP. The MAC entity shall, upon reception of configuration for CG-SDT: 1> store the RSRP of the downlink pathloss reference derived based on the measObject configured for the Serving Cell as specified in TS 38.331 [5]. |  |