3GPP TSG-RAN WG2 Meeting #119-e		R2-22xxxxx

- Online, August 17-29, 2022

Title:	LS on L1/L2-based inter-cell mobility

Response to:

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN2

To:	RAN1, RAN3, RAN4

Cc:

      - Contact Person:	Li-Chuan TSENG

      - E-mail Address:	li-chuan.tseng@mediatek.com

1. Overall Description:

Regarding L1/L2-based inter-cell mobility (Objective #1) in Rel-18 NR further mobility enhancement, RAN2 made the following agreements and assumptions:

- Assumption: HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. FFS if TRS tracking after HO and CSI RS measurement should also be included, i.e. the time to use a high-performance beam (can be clarified further). - Assumption: To reduce HO interruption time, investigate e.g. solutions to reduce the time for UE reconfiguration (already in the WID), downlink and uplink synchronization after handover decision (other parts of dynamic switch not precluded). - Confirm to Support L1/L2-based inter-cell mobility for inter-DU scenario (as well as intra-DU scenarios). - The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different. - R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery. - ICBM is one scenario considered for L1L2 mobility, but is not the only one, and is not a prerequisite for using L1L2 mobility. - RAN2 to consider preparation of target cell configurations capable of dynamic switching without need for full configuration. - Measurement delay can/may be considered in this work - Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS) - R2 will initially focus on PCell mobility. - R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - a) the target PCell/target SCell(s) is not a current serving cell (CA  CA scenario with PCell change) - b) FFS the target PCell is a current SCell - c) FFS the target SCell is the current PCell. - DC scenarios are FFS (e.g. PSCell mobility may be a low hanging fruit FFS). - Current options on the table: to configure a L1/L2 inter-cell mobility candidate cell: - a. One RRCReconfiguration message for candidate target cell - b. One CellGroupConfig IE for each candidate target cell - c. One SpCellConfig IE for each candidate target cell - Will send an LS to RAN1 and RAN3 on the progress of this meeting.

Regarding the selective activation cell of groups and CHO with CPAC, RAN2 made the following agreements:

- The selective activation of cell groups should correspond to support of subsequent conditional changes (CPC) after a cell group change (normal or conditional). CPA FFS. - Initial focus on SCG - There is interest to support delta configuration, to reduce the signalling overhead (FFS if some other objective should be achieved) - FFS how many subsequent conditional changes are targeted (and what is the impact of such assumption). - FFS whether there is a security issue: e.g. to determine vertical or horizontal key derivation, e.g. security parameters re-used as part of subsequent CG switch (for the case when UE goes back to a previous cell, maybe in another SN), and FFS on the procedure/method with which the UE derives the SN security, e.g. based on a prior MN config (without RRC CPC config at the time of SN switch). - Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17. - CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported. - FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential.

2. Actions

To RAN1, RAN3, and RAN4 groups

ACTION: 	RAN2 kindly ask RAN1/3/4 to work on enhancements in the scope of RAN1/3/4 work in the above agreements. RAN2 believes that the parts in red for L1/L2-based inter-cell mobility are the most relevant for RAN1/4 work, and the parts in red for the selective activation cell of groups are the most relevant for RAN3 work.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #119bis-e		10 – 19 October 2022			Online

TSG RAN WG2 Meeting #120		11 – 14 November 2022		EU