3GPP TSG-RAN WG2 #119bis-e	R2-22xxxxx

Online, 10 – 19 October, 2022

Agenda Item:	8.9.4

Source: 	LG Electronics Inc.

Title: 	[Post119-e][408][Relay] Path operations in multi-path relaying (LG)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [Post119-e][408][Relay] Path operations in multi-path relaying (LG)

Scope: Discuss issues with path management in the multi-path relaying objective:

- The set of supported path addition/modification/removal procedures, for both scenarios 1 and 2

  - Baseline expectation is agnostic to scenarios 1 and 2

  - Can be discussed if some procedures can be omitted for scenario 2

- Need for a concept of a “primary path” and steps towards its definition if needed

- L2 functions in relation to the path concept, e.g., consequences of the bearer mapping agreement and PDCP duplication (for both scenarios 1 and 2)

Intended outcome: Report to next meeting

Deadline: Long

Companies are requested to provide their views on the issues and questions listed in this document.

# Discussion

### Issue 1: Path addition/modification/release

RAN3 made some agreements related to path addition/modification/release as follows:

- Addition of direct/indirect path are supported as follows:

  - Add direct path, after the establishment of the indirect path.

  - Add indirect path, after the establishment of the direct path.

  - This does not imply the exclusion of any other path addition possibility.

- RAN3 will study the signaling impact on the direct or indirect path change under the same gNB for a UE connected via multi-path. The other mobility scenarios can be further considered based on RAN2 decision.

- The following use cases are not supported in Rel-18.

  - Configure two indirect paths

  - More than two paths

  - Inter-gNB multi-path support

Considering RAN3 agreements and contributions to RAN2#119-e, RAN2 could discuss which cases can be supported or not supported for scenario 1 and 2 as shown in Question 1-1 and 1-2.

Question 1-1: Do you agree all of the following MP cases in Scenario 1? If not, which case should be excluded in Scenario 1?

- The remote UE configured only on the direct path adds the indirect path under the same gNB;

- The remote UE configured only on the indirect path adds the direct path under the same gNB;

- The remote UE configured with multi-path releases the indirect path;

- The remote UE configured with multi-path releases the direct path;

- The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

- The remote UE configured with multi-path keeps the serving relay UE for the indirect path and the serving cell of the remote UE for the direct path while the serving relay UE changes the serving cell of the relay UE under the same gNB;

- The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

| Company | Supported case(s) | Excluded case(s) | Comment |
| --- | --- | --- | --- |
| LG Electronics | All | None |  |
| OPPO | A,B,C,D | F | F is a group-HO case, to us it is out of WID scope. While we are open to hear the others’ view on the support of E and G. |
| Apple | A,B,C,D | E,F,G | We think the multi-path support in Rel-18 work should focus on basic path addition/release scenarios. It is better to leave mobility related scenarios to the service continuity objective to discuss. |
| vivo | All | None | At this SI phase all potential valid scenarios should be considered. Potential down-prioritization may be further considered during normative stage. |
| Xiaomi | All except F | F | Not sure how F can work. In L2 relay, the serving cell of remote UE for the indirect path is the same as relay UE’s serving cell. If relay UE changes serving cell, serving cell of remote UE for the indirect path would also change. |
| Ericsson | A, C, G | B, D, E, F | Given limited time in R18, it is beneficial to simplify the design efforts for SL multipath, by focusing on the direct path. The direct path should be always the primary path if it is feasible. Scenario B is more like an optimization scenario, remote UE can trigger a path switch from the indirect path to the direct path when remote UE has changed from OOC to IC. After that, remote UE can add the indirect path if needed. Scenario D should be excluded owing to Scenario B. Similarly for Scenario E, remote UE can remove the indirect path, and triggers handover if the Uu connection is bad. Scenario F is a group handover, it is out of the WID |
| Qualcomm | All except F | F | Case F requires group HO for Remtoe UE and Relay UE, which is out of scope. Case G needs to be clarified a new relay with/without serving cell change. Case E and G should be discussed later or in normative work, and reuse service continuity mechanism defined for intra-gNB case. |
| CMCC | A,B,C | D,E,F,G | Considering limited time, Multi-path in Rel-18 work scope should be focused on basic path addition/release scenarios. Cases E,F,G can be considered in future release. For D, we understand that direct path is always primary path for MP, so, case D is excluded. |
| Nokia | A, B, C, D | F | A, B, C, D are the basic scenarios. We think RAN2 should focus on them in Rel-18. Mobility scenarios E and G may be supported if they require minimal additional specifications (e.g., G may be supported if I2I path switch is supported, and a non-optimized version of scenario E can be performed with D+B). F seems to us the most complex case that can easily lead to group HO and thus it may be better to exclude it from Rel-18. |
| Huawei, HiSilicon | A+C+G with higher priority; B+D+E with secondary priority | F | If A is supported, the C should be supported, and G would be a useful case for mobility. Similarly if B is supported, then D should be supported, and E would be the corresponding mobility scenario. F is the case not supported even in Rel-17 U2N relay, it could be excluded in Rel-18 due to limited time. |
| ZTE | A, B, C, D | E, F, F | It is suggested to focus on the most fundamental add or release one path operation. For the modification of one of the path can be deprioritized and even excluded in R18. |
| Samsung | A,B,C,D,E,G | F | Scenario F is group handover which is out of the WID. With A,B,C,D, E&G will not cause big impact so it can be supported. |

Question 1-2: Do you agree all of the MP cases of Q1-1 in Scenario 2? If not, which case should be excluded in Scenario 2?

| Company | Supported case(s) | Excluded case(s) | Comment |
| --- | --- | --- | --- |
| LG Electronics | A to F | G | We assume that the remote UE does not reselect the pre-configured relay UE in Scenario 2. |
| OPPO | A,C | B,D,E,F,G | For Scenario-2, we assume the direct path is always present, i.e., there is no use case for the remote-UE to connect via indirect path only. And there is no need to consider mobility for scenario-2. |
| Apple | A,C | B,D,E,F,G | Same vias as OPPO. For scenario 2, it is try to double the UL throughput by aggregate two in-coverage UEs, so we assume direct path is always present. |
| vivo | A to G | None | For scenario 2, Case G would occur when a remote UE has static relationship with more than one relay UEs. |
| Xiaomi | All except F and G | F, G | Similar concern on F as above question. Regarding G, remote UE would not change relay UE, since the relation is static. |
| Ericsson | A, C, G | B, D, E, F | Similar comments as for Q1-1. We think G can be also supported in scenario 2, since remote UE may be preconfigured with multiple relay candidates, the Uu connection of the relay UE may become bad (e.g., RLF is triggered). In this case, it would be ok to reuse the similar relay reselection procedure as for Scenario 1 to select a different relay UE. It is also important to note that, we need to have same supported cases between scenario 1 and 2, to simplify the RAN2 design efforts, and to reuse as much as possible functions of scenario-1 for scenario-2. |
| Qualcomm | All except F | F | Case F requires group HO for Remtoe UE and Relay UE, which is out of scope. The relay change for scenario 2 should not be excluded, the relay reselection is out of 3GPP, can be left to implementation or pre-configuration. We shouldn’t limit the mobility function for scenario 2 as long as the solution for scenario 1 can be applied for scenario 2 with UE implementation possibility. |
| CMCC | A,C | B,D,E,F,G | For scenario2, the direct path is always available. Case B and D can be excluded. For cases E,F,G can be left to further enhancement in future release, as we commented in Q1-1. |
| Nokia | A, C | B, D, F, G | Our assumption is that direct path is always present, and thus B and D are not relevant. We think that indirect path switch (G) is out of scope of 3GPP. As in case of scenario 1), RAN2 should focus on the basic scenarios (A and C), and only support E if it can be supported without extra specifications. |
| Huawei, HiSilicon | A+C | F, B, D, E, G? | In scenario 2, the indirect link cannot provide initial access, thus B+D+E is not practical. For G, we are open to relay UE change in scenario 2. |
| ZTE | A, B, C, D | E, F, G | RAN2 has confirmed that UE aggregation may improve the throughput and reliability/robustness for UE at the edge of a cell. So it is likely that the UE is initially connected to the network via indirect path. Based on this observation, case B and D may also be supported. |
| Samsung | A, B, C, D, E | F, G | F is group handover which is out of WID. G can be deprioritized since we assume the connection relationship between remote UE and relay UE is static. |

### Issue 2: Primary path for control plane

RAN3 made an agreement related to primary path:

- RAN3 waits for the RAN2 progress on whether and how to define the Primary path in multi-path support.

In RAN2, some companies submitted proposals related to the primary path as follows:

| Company | Proposals |
| --- | --- |
| Spreadtrum Communications | Proposal 3: The primary and secondary path should be introduced in multi-path relaying. Proposal 4: The primary and secondary path configuration can be as following: - For scenario 1: the Uu link configured as primary path and the relay link configured as secondary path, or the relay link configured as primary path and the Uu link configured as secondary path. - For scenario 2: the Uu link configured as primary path and the link of non-standardized UE-UE connection configured as secondary path. |
| Lenovo | Proposal 4: RAN2 is suggested to discuss which option can be selected. - Option 1: Two paths can be considered as MCG and SCG, respectively. - Option 2: A primary path and a secondary path are specified. But no concept of MCG and SCG is specified. - Option 3: No concept of a primary path and a secondary path is needed. |
| Ericsson | Proposal 2 Introduce the terms ‘Primary path’ and ‘Secondary path’ for the multi-path scenario. Proposal 3 Only support that the direct path is configured as the primary path and the indirect path is configured as the secondary path and the other case where the primary path is the indirect path and the secondary path is the direct path is down-prioritized in Rel-18. Proposal 4 For a multipath UE, RAN2 only studies the scenario where the UE first establishes a direct path and later adds an indirect path as the second path and the scenario where the UE first establishes an indirect path and later adds a direct path is down-prioritized in Rel-18. |
| InterDigital | Proposal 3: The concept of a primary path, which can be either the direct or relayed path, is supported in multi-path, both for SRB and DRB Proposal 4: The network controls the amount of data routed by the UE to each of the primary and secondary paths, for each split bearer |
| OPPO | Proposal 11 For Scenario-1 of multi-path Relay, RAN2 not purse the concept of primary / secondary path / bearer before the motivation / definition is clarified. Proposal 23 For Scenario-2 of multi-path Relay, RAN2 not purse the concept of primary / secondary path / bearer before the motivation / definition is clarified. |
| Qualcomm | Proposal 3: Terminologies “Primary path” and “Secondary path” are introduced for multi-path operation of Uu direct and L2 based Relay indirect path towards the same gNB. Proposal 4: The Primary path is defined to the path over which the UE establishes an RRC connection. |

Considering RAN3 agreement and contributions to RAN2#119-e, RAN2 could discuss whether to introduce the concept of a primary path and which aspects can be considered as the characteristics of the primary path for scenario 1 and 2 as shown in Question 1-1 and 1-2. The rapporteur thinks that the considered characteristics of the primary path can be used to define the primary path and specify the primary path, if needed.

Considering contributions to RAN2#119-e, the rapporteur observed that some companies propose the concept of the primary path mainly for control plane while others address the primary RLC entity with the concept of the primary path i.e. mainly for layer 2. Accordingly, the issue 2 focuses on control plane aspects of the primary path while the issue 3 will address the primary RLC entity for other layer 2 aspects.

Question 2-1: Do you agree to introduce the concept of a primary path in Scenario 1 from CP perspective? If yes, which part(s) of the following CP aspects can be considered as the characteristics of the primary path?

- The primary path is the path where the remote UE has initially established an RRC connection.

- The primary path is the path where the remote UE has re-established an RRC connection.

- The primary path is the path that gNB indicated for the remote UE during mobility.

- The primary path is the path configured on PCell of the remote UE.

  - If the primary path is the indirect path, the PCell of the remote UE is same as the PCell of the relay UE.

  - Note: The direct path and indirect path can be served by the same cell (i.e. PCell) or by different cells as agreed in RAN2#119-e.

- The primary path is the path that gNB indicates as the primary path.

- The primary path is the path used as the AS security anchor.

- The primary path is the path where the remote UE acquires system information.

- The primary path is the path where the remote UE exchanges NAS messages.

- The primary path is the path where SRB is transmitted if duplicated SRB 0/1/2 is not configured

- The primary path is the path based on whose status UE performs RRC connection control, e.g. reestablishment.

- Anything else?

| Company | Yes/No | Considered characteristics | Comment |
| --- | --- | --- | --- |
| LG Electronics | Yes, the concept of a primary path can be supported. | A to F | Further study on G and H |
| OPPO | No |  | We would like to understand the consequence if we do not import a new term of “primary path”, if one can always simply refers to Uu or PC5 interface (if there is a need to refer to – and based the following, there seems not so far). - For A, B, G, H, what procedure cannot work if without such term? Maybe proponent can clarify more.. - For C, E, we may want to discuss firstly on the definition of the primary path before discussing how to indicate it.. - For D, we agree there is a need for UE to know which cell (the one of direct or indirect path) is the P-cell, yet we believe it can be done easily without introducing a new term, e.g., the cell via which the UE setup the RRC, or which is indicated by ReconfigurationWithSync would be the PCell - For F, since R18 focus on same gNB case, there is no need to worry about AS-security-input, which (i.e., target PCI) is only used for inter-gNB HO key derivation. |
| Apple | Yes | A,B,C,D,E | In general, we think the primary path is where the most AS control plane signalling transport (e.g., Uu RRC) is used. For F, we are not sure the security matters as we only focus on intra-gNB case. |
| vivo | Yes | A, B, C,D,E | We think the primary path may be useful for some legacy mechanisms, e.g. PCell decision, RLF declaration, HO case, and so on. From the perspective of Uu RRC signalling transmission, we also agree that split SRB can be configured with a primary leg like legacy PDCP behaviours. For F, the security issue seems not to be considered in intra-gNB scenario as the PDCP anchor will not be changed. For G, following Rel-17 SL relay it’s up to Remote UE’s implementation whether it may acquire system information via direct Uu path, we think it’s not appropriate to use this Case G for defining the primary path. For H, as legacy NAS messages are transmitted by SRB2 which relies on dedicated RRC configuration by gNB and thous this Case H should not be considered for primary path concept. |
| Xiaomi | Yes | E, G, I, J | Similar as DC, the primary path is used to provide SRB. Also based on the primary path link status, UE can decide whether to trigger RRC connection reestablishment. |
| Ericsson | Yes | A, B, D (update), E, I, J | We have similar comments as other companies for scenarios F, G, H C is already covered in E, mobility need not be handled separately |
| Qualcomm | Yes | A,B,G | Our paper R2-2207137 also proposes to introduce primary path in P3 and P4, so copy into the table. Introducing primary path will save our efforts on many CP issues, e.g. inactive state handling, SI reception, inter-gNB HO, RLF and Re-establishment handling. All of the CP issues, we can take existing procedure defined for DC as baseline. And additionally the solution defined to intra-gNB can be easily extended to inter-gNB in future release. For C, it is related to mobility procedure, we need to firstly look at what the mobility procedure looks like. For D, we understand the primary path refers to direct path or indirect path, not to which cell. For direct path, there could be primary cell and other serving cells if CA configured to Remote UE; for indirect path, there could be primary cell and other serving cells if CA configured to Relay UE And there is no PCell concept from Remote UE perspective on indirect path, even for Rel-17. So we do need to introduce PCell of Remote UE on indirect path in MP operation. For E, if primary path can be flexibly configured by gNB, then it is equivalent to no primary path defined? For F, since this is the same gNB, we don’t quite understand security anchor. For a bearer mapped on secondary path, there still AS security and no primary path involved? For H, SRB2 can be carried on primary path or secondary path or both depends on bearer type configuration. |
| CMCC | Yes | A,B,C,D,E,I,J |  |
| Nokia | Yes | A, B, (C), D, E, I, J | We think that C is covered by E |
| Huawei, HiSilicon | See comments | I | For the cases listed, - A/B/C/D/F/G/J all mean Pcell to us, then we would like first to understand what’s the difference between so called primary path here and existing concept of PCell. - E has no definiation of primary path but just say it can be indicated by network, we think this can be decided after concluding on the need/definition of the so called primary path. - I seems meant to be the primary RLC entity, which we agree to support for split SRB in the similar way as legacy split bearer. |
| ZTE | Yes | E | The concept of primary path is different from primary cell. According to the legacy spec, the primary path and secondary path is mainly used for the data split/ duplication operation in DC scenario and it is configured per RB. With the data split as an example, if the total amount of PDCP data volume and RLC data volume pending for initial transmission is smaller than ul-DataSplitThreshold, the primary path is used. Otherwise, both primary path and secondary path may be used. In addition, the BSR report is usually associated with the primary path. We think the primary path may also be used for the support of data split/duplication operation and the gNB based configuration for primary path is preferred. |
| Samsung | Yes | A, B, E, I | C is covered by E so a specific case for mobility seems to be unnecessary. For D, it implies that the direct path is always primary path since the concept of PCell is only applicable for direct path. For indirect path, can the PCell of relay UE be regarded as PCell of remote UE? In addition, D seems to be covered by A and B. For F, Rel-18 only considers intra-gNB case so it cannot differentiate primary path. For G, H, in our understanding, one of the intentions of having primary path for CP is to select a path with enough reliability. System information/NAS message has no connection towards the reliability. J seems to be the scheme applicable for primary path when it is decided, rather than a way to determine the primary path. |

Question 2-2: Do you agree to introduce the concept of a primary path in Scenario 2? If yes, which part(s) of the above aspects can be considered as the characteristics of the primary path?

| Company | Yes/No | Considered characteristics | Comment |
| --- | --- | --- | --- |
| LG Electronics | Yes, the concept of a primary path can be supported. | A to F | Further study on G and H |
| OPPO | No |  | Please refer to Q2-1 |
| Apple | Yes | A to E |  |
| vivo | Yes | A to E |  |
| Xiaomi | Yes | E, G, I, J | Common design is preferred for both scenario 1 and 2 |
| Ericsson | Yes | A, B, D (update), E, I, J | We have similar comments as other companies for scenarios F, G, H Same comment for C |
| Qualcomm | Yes | A,B,G | Same comments as Question 2-1 |
| CMCC | Yes | A,B,C,D,E,I,J |  |
| Nokia | Yes, but comments |  | RAN2 should use agreements for Scenario 1) as a baseline and check if they can be applied in this scenario as well |
| Huawei, HiSilicon | NO for A/B/C/D/F/G/J | I | Different from scenario1, it is clear only direct link can provide initial access, then in the cases A/B/C/D/F/G/J, it could be only indirect link, then we do not get the point to have this concept for A/B/C/D/F/G/J. |
| ZTE | Yes | E | Same comments as for Question 2-1 |
| Samsung | Yes | A, B, E, I | Same as scenario 1 |

Some companies think that the direct path is configured as the primary path and the indirect path is configured as the secondary path and the other case where the primary path is the indirect path and the secondary path is the direct path is down-prioritized in Rel-18.

Question 3-1: Which option do you support in Scenario 1 when both paths are configured (if yes in Q2)?

- Option 3A: The primary path is always configured on the direct path.

- Option 3B: The primary path is always configured on the indirect path.

- Option 3C: The primary path can be configured on either the direct or the indirect path.

- Option 3D: Option 3B is excluded. Option 3A and 3C can be further studied.

| Company | Considered characteristics | Comment |
| --- | --- | --- |
| LG Electronics | 3D |  |
| OPPO | None |  |
| Apple | 3C |  |
| vivo | 3C |  |
| Xiaomi | 3C |  |
| Ericsson | 3A | It is simpler for RAN2 to design, to only support the primary path is on the direct path. |
| Qualcomm | Option 3C | We need to consider the scenario that a Remote UE in out-of-coverage accesses to a gNB over indirect path and after a while the Remote UE moves into coverage of direct path, then the gNB can configure Uu direct path as secondary path to enable multi-path operation. And we don’t see any issue to take indirect path as primary path. In addition, we prefer to remove the wording “configured” in each option. “configured” implies network indicates which we are not agreed. |
| CMCC | 3A |  |
| Nokia | 3C |  |
| Huawei, HiSilicon | See comments | For the primary RLC entity of split SRB (i.e. I in Question 2-1), 3D is preferred. |
| ZTE | 3C | According to the legacy DC operation, either MCG or SCG may be configured as primary path for data split/duplication operation. It is suggested to adopt similar design for the multi-path SL relay. Actually it leaves more flexibility for gNB to configure either direct or indirect path as primary path. |
| Samsung | 3C |  |

Question 3-2: Which option do you support in Scenario 2 when both paths are configured (if yes in Q2)??

- Option 3A: The primary path is always configured on the direct path.

- Option 3B: The primary path is always configured on the indirect path.

- Option 3C: The primary path can be configured on either the direct or the indirect path.

- Option 3D: Option 3B is excluded. Option 3A and 3C can be further studied.

| Company | Considered characteristics | Comment |
| --- | --- | --- |
| LG Electronics | 3D |  |
| OPPO | None |  |
| Apple | 3A | For scenario 2, as UEs are always in-coverage then 3A is the most straight-forward choice |
| vivo | 3C | This can be common part for Scenario 1 and Scenario 2. |
| Xiaomi | 3C | Common design is preferred. |
| Ericsson | 3A | Again, scenarios 1 and 2 should have as much commonality as possible. It is sufficient to adopt 3A in this release. |
| Qualcomm | 3C | Same comments as Question 3-1. |
| CMCC | 3A | For scenario 2, the direct path is always present. |
| Nokia | 3D | RAN2 should further study if using indirect path is feasible if primary path may be the indirect path in case of scenario 1) (See Q3-1) |
| Huawei, HiSilicon | See comments | For the primary RLC entity of split SRB (i.e. I in Question 2-1), 3D is preferred. |
| ZTE | 3C | Same comments as Question 3-1. |
| Samsung | 3C | Same as scenario 1 |

Some companies wonder whether the primary path can support both SRBs and DRBs while the secondary path can be support DRBs only. In other words, it can be discussed whether SRBs can be configured on either only the primary path or both paths. Note that whether to support SRB0 on a specific path is related to Question 2-1 and 2-2. Thus, SRB0 is not subject to the question 4-1 and 4-2.

Question 4-1: Which option do you support in Scenario 1, if yes in Q2 (note: SRB0 is not subject to this question)?

- Option 4A: SRB1, SRB2, SRB3 and SRB4 can be configured on the primary path only.

- Option 4B: SRB1, SRB2, SRB3 and SRB4 can be configured on both the primary path and the secondary path.

- Option 4C: Some SRB(s) can be configured on the primary path only while other SRB(s) can be configured on both the primary path and the secondary path.

| Company | Supported option | Comment |
| --- | --- | --- |
| LG Electronics | Option 4C | Which SRB can be configured only on the primary path can be further discussed. |
| OPPO | None | Yet we would like to highlight we are supportive that SRB1/2 can be configured to deliver PDCP PDU via direct path and/or indirect path. (we do not see the reason to introduce new terms of P-path though) |
| Apple | 4C |  |
| vivo | 4C | In general, we think SRB3 and SRB4 should not be supported according to the definition in TS 38.331 as below. SRB3 is not supported in multiple path because only SA architecture is considered. SRB4 is not supported due to cross-WI feature. - SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel; - SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 can only be configured by the network after AS security activation. Moreover, SRB1 and SRB2 can be configured on primary path only or, both the primary path and the secondary path, where the latter is mainly used for split bearer operation. |
| Xiaomi | 4C | Similar as DC, SRB 1/2 can be carried on both path by duplicated SRB. SRB0 can only be carried on primary path. |
| Ericsson | 4C, but see comments | Agree with Vivo, SRB3/4 are not applicable here |
| Qualcomm | 4B except SRB3 FFS for SRB4 | From specification, we don’t need to restrict the path for SRB1 and SRB2, can leave to gNB implementation how to configure. There should be no SRB3 since no SN node. For SRB4, RAN2/RAN3 are discussing whether SRB4 can be split bearer in DC case. This can be revisited based on QoE progress. |
| CMCC | 4C | We share understanding with vivo. |
| Nokia | 4C |  |
| Huawei, HiSilicon | See comments | If we are discussing primary RLC entity, we think this related to the conclusion on Question 3, if the primary RLC entity can be configured to either indirect or direct, then it is fine to make different configuration for different SRB. Otherwise, the SRBs should have the same configuration. |
| ZTE | 4B | Agree with QC that whether primary path only or both path can be used can be up to gNB implementation for SRB1 and SRB2. SRB3 is not applicable for multi-path relay in this release. SRB4 is FFS. |
| Samsung | 4C | Same view as vivo for SRB3 and SRB4 |

Question 4-2: Which option do you support in Scenario 2, if yes in Q2 (note: SRB0 is not subject to this question)??

- Option 4A: SRB1, SRB2, SRB3 and SRB4 can be configured on the primary path only.

- Option 4B: SRB1, SRB2, SRB3 and SRB4 can be configured on both the primary path and the secondary path.

- Option 4C: Some SRB(s) can be configured on the primary path only while other SRB(s) can be configured on both the primary path and the secondary path.

| Company | Supported option | Comment |
| --- | --- | --- |
| LG Electronics | Option 4C | Which SRB can be configured only on the primary path can be further discussed. |
| OPPO | None | Yet we would like to highlight we are supportive that SRB1/2 can be configured to deliver PDCP PDU via direct path only. (we do not see the reason to introduce new terms of P-path though) |
| Apple | 4A | We assume primary path is always the direct path in scenario 2 |
| vivo | 4C | Same comment as in Question 4-1 |
| Xiaomi | 4C | Common design is preferred. |
| Ericsson | 4C | Same comment as in Question 4-1 |
| Qualcomm | 4B except SRB3 FFS for SRB4 | Same comment as in Question 4-1 |
| CMCC | 4A | The primary path is always configured on direct path for scenario2. |
| Nokia | Comments | RAN2 should use agreements for Scenario 1) as a baseline and check if they can be applied in this scenario as well (e.g., if it is feasible to use indirect path for SRBs without major specification work) |
| Huawei, HiSilicon | See comments | Same comments to scenario 1 and 2. |
| ZTE | 4B | Same comments as Question 4-1. |
| Samsung | 4C | Same as scenario 1 |

RAN2 agreed to support the MP split bearer which is a bearer mapped to both paths, based on the existing split bearer framework. It can be further discussed whether the MP split bearer can support SRB as well as DRB. In 38.331, it is specified that split SRB is supported for all the MR-DC options in both SRB1 and SRB2 (split SRB is not supported for SRB0 and SRB3).

Question 5-1: Do you think that the MP split bearer can be supported for SRB in Scenario 1? If yes, which SRB can be configured as the MP split bearer?

| Company | Yes/No | Which SRB for MP split bearer | Comment |
| --- | --- | --- | --- |
| LG Electronics | Yes | SRB1 and SRB2 | Same as for MR-DC |
| OPPO | Yes | SRB1 and SRB2 |  |
| Apple | Yes | SRB1/2 |  |
| vivo | Yes | SRB1 and SRB2 | Same as for MR-DC |
| Xiaomi | Yes | SRB1/2 |  |
| Ericsson | Yes | SRB1 and SRB2 |  |
| Qualcomm | Yes | SRB1 and SRB2 FFS for SRB4 | Same as for MR-DC so far. But RAN2/RAN3 are discussing whether SRB4 can be split bearer in DC case. This can be revisited based on QoE progress. |
| CMCC | Yes | SRB1/2 |  |
| Nokia | Yes | SRB1 and SRB2 |  |
| Huawei, HiSilicon | Yes | SRB1 and SRB2 FFS for SRB4 |  |
| ZTE | Yes | SRB1 and SRB2, FFS for SRB4 |  |
| Samsung | Yes | SRB1 and SRB2 |  |

Question 5-2: Do you think that the MP split bearer can be supported for SRB in Scenario 2? If yes, which SRB can be configured as the MP split bearer?

| Company | Yes/No | Which SRB for MP split bearer | Comment |
| --- | --- | --- | --- |
| LG Electronics | Yes | SRB1 and SRB2 | Same as for MR-DC |
| OPPO | No |  | We believe in scenario-2, all SRBs just need to be carried via direct path. |
| Apple | No |  | Agree with OPPO |
| vivo | Yes | SRB1 and SRB2 | Same as for MR-DC |
| Xiaomi | Yes | SRB1/2 |  |
| Ericsson | Yes | SRB1 and SRB2 |  |
| Qualcomm | Yes | SRB1 and SRB2 FFS for SRB4 | Same comments as Question 5-1 |
| CMCC | No |  | For scenario 2, SRBs are only configured on direct path. |
| Nokia | Comments |  | RAN2 should use agreements for Scenario 1) as a baseline and check if they can be applied in this scenario as well (e.g., if it is feasible to use indirect path for SRBs without major specification work) |
| Huawei, HiSilicon | Yes | SRB1 and SRB2 FFS for SRB4 |  |
| ZTE | Yes | SRB1 and SRB2, FFS for SRB4 |  |
| Samsung | Yes | SRB1 and SRB2 |  |

In addition, considering the existing split bearer framework, it is obvious that the primary RLC entity and the split secondary RLC entity are supported for each MP split bearer. If the MP split bearer can be supported for SRB, it can be further discussed whether the primary RLC entity of the MP split bearer is always configured on the primary path of the control plane or can be configured on any path.

Question 6-1: Which option do you support in Scenario 1, if yes in Q5

- Option 6A: The primary RLC entity of the MP split bearer for SRB is always configured on the primary path of the control plane

- Option 6B: The primary RLC entity of the MP split bearer for SRB can be configured on either the primary path or the secondary path.

- Option 6C: The primary RLC entity of the MP split bearer for SRB can be configured on the primary path of the corresponding SRB.

| Company | Supported option | Comment |
| --- | --- | --- |
| LG Electronics | 6B | At least upon detection of RLF on the primary path, the primary RLC entity of the MP split bearer for SRB can be reconfigured on the secondary path. |
| OPPO | 6A | As in legacy where the P-path (here the term is the one used in 38.323, i.e., the RLC entity used for PDCP to deliver PDU to) of SRB is always at MCG, we believe it is safer to fix it to direct path as well. |
| Apple | 6A |  |
| vivo | 6A | In this option, whether the primary RLC entity is on direct path or indirect path can be aligned with the definition of the primary path of the control plane by resolving Q2 and Q3 firstly. |
| Xiaomi | 6A |  |
| Ericsson | 6A |  |
| Qualcomm |  | This should be discussed in normative work after we have clear view on the other issues for SRB. |
| CMCC | 6A |  |
| Nokia | 6A with Comments | The rapporteur's view is that "it is obvious that the primary RLC entity and the split secondary RLC entity are supported for each MP split bearer". Even if there is the primary path for SRB (see Q2-1), we are not sure why we need to define primary RLC entity further. Note that there is no dedicated RLC entity for the remote UE in the indirect path (RLC is hop-by-hop). |
| Huawei, HiSilicon | Slightly prefer 6B |  |
| ZTE | 6C | According to the current spec, the primary path are defined per SRB. It should be noted that SRB1 and SRB2 of IAB-MT may be configured with different primary paths (either MCG or SCG). Then the primary path indicates the cell group ID and LCID of the primary RLC entity. So we propose 6C to capture this. According to TS 38.311: “primaryPath: Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 [5], clause 5.2.1 for UL data transmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs, except for the split SRB2 of the IAB-MT, and, when the SCG is deactivated, for DRBs. The NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW always indicates logicalChannel if CA based PDCP duplication is configured in the cell group indicated by cellGroup of this field.” |
| Samsung | 6A |  |

Question 6-2: Which option do you support in Scenario 2, if yes in Q5

- Option 6A: The primary RLC entity of the MP split bearer for SRB is always configured on the primary path of the control plane

- Option 6B: The primary RLC entity of the MP split bearer for SRB can be configured on either the primary path or the secondary path.

- Option 6C: The primary RLC entity of the MP split bearer for SRB can be configured on the primary path of the corresponding SRB.

| Company | Supported option | Comment |
| --- | --- | --- |
| LG Electronics | 6B | At least upon detection of RLF on the primary path, the primary RLC entity of the MP split bearer for SRB can be reconfigured on the secondary path. |
| OPPO | None | No need to support split bearer for SRB in scenario-2. |
| Apple | None |  |
| vivo | 6A | Same comments as in Question 6-1. |
| Xiaomi | 6A |  |
| Ericsson | 6A |  |
| Qualcomm |  | Same comments as in Question 6-1. |
| CMCC | None |  |
| Nokia | Comments | RAN2 should use agreements for Scenario 1) as a baseline and check if they can be applied in this scenario as well (e.g., if it is feasible to use indirect path for SRBs without major specification work) |
| Huawei, HiSilicon | Slightly prefer 6B |  |
| ZTE | 6C | Same comments as in Question 6-1. |
| Samsung | 6A | Same as scenario 1 |

### Issue 3: Layer 2

In RAN2, some companies submitted proposals related to MP split bearers and/or PDCP duplication possibly with the primary path as follows:

| Company | Proposals |
| --- | --- |
| ZTE, Sanechips | Proposal 8: Suppose the data split/duplication is supported for multi-path delivery, it is FFS whether primary path and secondary path should be configured for remote/anchor UE. |
| Samsung | Proposal 8-2: RAN2 is kindly asked to discuss the primary path configuration for PDCP duplication. |
| InterDigital | Proposal 3: The concept of a primary path, which can be either the direct or relayed path, is supported in multi-path, both for SRB and DRB Proposal 4: The network controls the amount of data routed by the UE to each of the primary and secondary paths, for each split bearer |
| CATT | Proposal 4: PDCP duplication via direct link and indirect link should be supported to enhance reliability. Proposal 6: If PDCP duplication and/or dual connectivity of direct link and indirect link are introduced, it should reuse the legacy Uu mechanism as much as possible. |
| China Telecom | Proposal 2: The data duplication and split functions could be supported for multi-path scenario, and the current PDCP duplication and split mechanisms could be used as a baseline. |
| Lenovo | Proposal 6: PDCP Duplication can be supported in Multi-path case. Proposal 7: The study to support PDCP duplication in Rel-18 multi-path case can start from the two legs case. Proposal 8: Legacy split bearer can be supported in Multi-path case. |
| Samsung | Proposal 3-1: RAN2 is kindly asked to address the following 5 aspects in Rel-18 sidelink relay enhancement: - Configurations of multi-path - Data transmission over two paths - Dynamic path activation/deactivation - Handling of path failure - PDCP duplication support |
| LG Electronics | Proposal 8: Support PDCP duplication over direct path and indirect path |

In TS 38.323, if the transmitting PDCP entity is associated with at least two RLC entities, the UE can submit the PDCP PDU to either the primary RLC entity or the split secondary RLC entity.

Considering the existing split bearer framework, it is obvious that the primary RLC entity and the split secondary RLC entity are supported for each MP split bearer. Assuming that the MP split bearer can be supported for DRB, it can be discussed whether the primary RLC entity of the MP split bearer for DRB is always configured on the primary path of the control plane or can be configured on any path.

Question 7-1: Which option do you support in Scenario 1, if yes in Q2?

- Option 7A: The primary RLC entity of the MP split bearer for DRB is always configured on the primary path of the control plane

- Option 7B: The primary RLC entity of the MP split bearer for DRB can be configured on either the primary path or the secondary path.

- Option 7C: The primary RLC entity of the MP split bearer for DRB can be configured on the primary path of the corresponding DRB.

| Company | Supported option | Comment |
| --- | --- | --- |
| LG Electronics | 7B |  |
| OPPO | 7B |  |
| Apple | 7B | We may also need discuss whether there is a primary path concept for UP first. I assume no for Scenario 1, but better to check. |
| vivo | 7B | In MR-DC, the primary RLC entity of split DRB indicated by dedicated RRC singalling can be either on MCG path or SCG path. Similarily, in multi path operation, this can be configured on either direct path or indirect path. |
| Xiaomi | 7B |  |
| Ericsson | 7B |  |
| Qualcomm | 7B |  |
| CMCC | 7B |  |
| Nokia | 7A with comments | For split operation perspective, we assume that PDCP data PDU from a DRB can be transmitted either over the primary or the secondary path. However, we wonder why we need to define primary RLC entity further because we only consider the same gNB case for MP split bearer and primary path can be used for indicating a specific RLC entity to which PDCP control/data PDU is submitted. Note that there is no dedicated RLC entity for the remote UE in the indirect path (RLC is hop-by-hop). |
| Huawei, HiSilicon | 7B |  |
| ZTE | 7C | Once again, we think that primary path is different from MCG/PCell, it is configured per RBs. For different DRBs, it may be associated with different primary paths. For example DRB1 may be configured with direct path as primary path while the DRB2 may be configured with indirect path as primary path. Based on this understanding, the primary RLC entity of the MP split bearer for a given DRB should be configured on the primary path of the corresponding DRB and it may be actually via direct path or indirect path associated with the primary path. |
| Samsung | 7B |  |

Question 7-2: Which option do you support in Scenario 2, if yes in Q2?

- Option 7A: The primary RLC entity of the MP split bearer for DRB is always configured on the primary path of the control plane

- Option 7B: The primary RLC entity of the MP split bearer for DRB can be configured on either the primary path or the secondary path.

- Option 7C: The primary RLC entity of the MP split bearer for DRB can be configured on the primary path of the corresponding DRB.

| Company | Supported option | Comment |
| --- | --- | --- |
| LG Electronics | 7B |  |
| OPPO | 7A | In Scenario-2, we believe it is a safer way to always rely on direct-path. |
| Apple | 7A | For Scenario 2, the primary path for both UP and CP are direct path. |
| vivo | 7B |  |
| Xiaomi | 7B |  |
| Ericsson | 7B | It is preferred to have the same for both scenario 1 and scenario 2 |
| Qualcomm | 7B |  |
| CMCC | 7A |  |
| Nokia | Comments | RAN2 should use agreements for Scenario 1) as a baseline and check if they can be applied in this scenario as well |
| Huawei, HiSilicon | 7B |  |
| ZTE | 7C | Same comments as in Question 7-1. |
| Samsung | 7B | Same as scenario 1 |

In addition, some companies proposed to support PDCP duplication for reliable transmission. The PDCP duplication can be based on the existing Uu framework.

Question 8-1: Do you agree to support PDCP duplication for the MP split bearer in Scenario 1 based on the existing framework?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes |  |
| OPPO | Yes |  |
| Apple | Yes |  |
| vivo | Yes |  |
| Xiaomi | Yes |  |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |

Question 8-2: Do you agree to support PDCP duplication for the MP split bearer in Scenario 2 based on the existing framework?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes |  |
| OPPO | Yes |  |
| Apple | Yes |  |
| vivo | Yes |  |
| Xiaomi | Yes |  |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| CMCC | Yes |  |
| Nokia | Comment | RAN2 should use agreements for Scenario 1) as a baseline and check if they can be applied in this scenario as well |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |

RAN2#119-e agreed that for a MP split bearer in scenario 1, one PDCP entity at the remote UE is configured with one direct Uu RLC channel and one indirect PC5 RLC channel. For upstream, a PDCP entity delivers to a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side. For downstream, a PDCP entity receives from a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side. Note that it is understood that the SRAP entity is also configured for the indirect path in scenario 1 considering that the indirect path corresponds to the Rel-17 U2N RLC channel.

Some companies think that the adaptation layer is also needed to support the scenario 2 as in Figure 1 (e.g. for data routing with bearer mapping/identification) while others think that the adaptation layer is not needed (e.g. as in Figure 2 or 3).

Figure 1. UP protocol stack for multi-path in R2-2208429, CMCC

Figure 2. UP protocol stack for UE aggregation (DC-like) in R2-2208429, CMCC

Figure 3. UP protocol stack for UE aggregation (DAPS-like) in R2-2208429, CMCC

Question 9A: Do you think that that an adaptation layer is needed for Scenario 2? Provide the reason(s) why the adaptation layer is needed for Scenario 2.

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes | Adaptation layer is needed to support data routing. For example, the relay UE and the remote UE configure multiple PDCP/RLC entities for multiple indirect/split bearers. So, when a PDCP entity of the remote UE submits a PDCP PDU to lower non-standard layers of the remote UE, the lower non-standard layers of the relay UE cannot identify which RLC entity of the relay UE needs to process the PDCP PDU received from the remote UE. |
| OPPO | Yes | As clarified by LG, we cannot ensure the ‘blackbox’ lower layer (let’s imagine a simple wired connection..) has a way for remote/relay UE to differ between different bearers/PDCP-entity. |
| Apple | Yes | First, the SRAP layer supports the many-to-one mapping. It is at least unclear now that whether only one-to-one mapping is sufficient for scenario 2. So, it is safe to keep SRAP layer. Second, even for the same remote UE, we do not know how different DRBs can be distinguished in the “ideal UE-o-UE” link, because LCID concept is not applicable in this non-3GPP link. So, it is still safe to keep an adaptation layer. |
| vivo | Not necessarily | In Scenario 2, RAN2 needs to study and confirm whether the valid mapping relationship between Remote UE and Relay UE is many-to-one or one-to-one. If there is only one-to-one mapping between the remote UE bearer and relay UE Uu bearer, there is no need for an adaptation layer. Furthermore, the relationship between the remote UE and the relay UE is static or pre-configured. One-to-one mapping can be guaranteed w/o introduction of adaptation layer. |
| Xiaomi | Yes | We see the necessity of adaptation layer in two aspects. One is bearer mapping between remote UE and relay UE. The other is the UE addressing, in case multiple remote UEs are connected to the same relay UE. |
| Ericsson | At least No for the first hop (i.e., between remote UE and relay UE) | The first hop is a non-standardized interface; therefore, support of the adaptation layer would incur additional design efforts for RAN2. We can just fully leave it to UE implementation. But for the second hop (i.e., between relay UE and the gNB), it would be good to support the adaptation layer to simplify the design efforts for scenario 2 by reusing the solutions of scenario 1. |
| Qualcomm | Yes | For ideal connection between the two UEs, we assume one Relay can serve multiple Remote UEs, and multiple bearers can be configured for each Remote UE. Then the functions of data transfer, determination of UE ID and BEARER ID, determination of egress link is needed for E2E packets transmission. |
| CMCC | No | The typical scenario 2 we expect is anchor UE (refers to remote UE) connects to more than one aggregated UE (refers to relay UE), in this scenario, which is determined by the purely requirement of scenario 2 is to boost UL throughput, as illustrated in R2-2208429. Nevertheless, the functionality of adaption layer is indicating the remote UE ID and bearer ID in case of multi remote UEs are connected to one relay UE and multi bearers mapping to one PC5 bearers or UU bearers. Hence, for scenario 2, relay UEs can be configured RLC entity (logical channel) for the target RB and identify which RLC entity of the relay UE needs to process the PDCP PDU received from the remote UE, not rely on adaption layer any more. We agree with comments of Ericsson that SRAP at the PC5 interface is not needed any more. |
| Nokia | Comments | The interface between the Remote UE and the Relay UE is not known and thus it is unclear if adding a layer on top of that interface is feasible. We should leave it to UE implementation (from 3GPP perspective). The need for the adaptation layer on Uu interface (between the Relay UE and gNB) requires further discussion. |
| Huawei, HiSilicon | No if a relay UE can only serve one remote UE and only 1:1 bearer mapping is supported. | We understand the SRAP based routing adopted in scenario 1 can work in scenario 2. However, if the only use case in scenario 2 is that one relay UE serve only one remote UE, and the bearer mapping between E2E bearer and relay UE’s Uu RLC channel is fixed to 1:1, then all the SRAP functions are not needed in scenario 2, in this case for simplicity/transmission efficiency SRAP layer is not needed. So RAN2 needs first figure out and agree the target use case. |
| ZTE | No for Uu | If we consider 1:1 bearer mapping between remote UE’s RB and Uu RLC channel of relay UE, it is not necessary to add adaptation subheader for Uu interface. With regard to the UE-UE internal connection, in order to support the interoperability of UEs from different vendors, it is better to add the adaptation subheader for the UE-UE interface which may be used by the relay UE (manufactured by vendor 1) to identify the associated RBs of the received packet from remote UE (manufactured by vendor 2) and then map this packet to the corresponding Uu RLC channel. |
| Samsung | Yes for the hop between relay UE and the gNB | The hop between remote UE and relay UE can be realized by implementation. The hop between relay UE and gNB needs the SRAP since in UL, gNB needs differentiate the data of remote UE and its own data, while in DL, relay UE needs differentiate the data of remote UE and its own data. |

Question 9B: Do you think that the adaptation layer of Scenario 2 can be based on Rel-17 SRAP? What kind of change do you expect compared to Rel-17 SRAP?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG Electronics | Yes | We think that the adaptation layer of Scenario 2 can be designed based on the SRAP specification. However, the adaptation layer of Scenario 2 does not need to understand/process SL data units. If the same SRAP is used for both scenario 1 and 2, the SRAP entity in Scenario 1 and 2 needs to additionally understand the type of the lower layers e.g. based on the header of a SRAP PDU (e.g. 3GPP SL or non-standard). Since the adaptation layer of Scenario 2 does not need to differentiate the type of the lower layers, the adaptation layer of Scenario 2 can be something new. |
| OPPO | Yes | Our observation is that some proposals said the R17 SRAP is too powerful and since some functions of R17 SRAP is not needed, and thus we can do something different in scenario-2 – to us it means if we simply reuse, there is no problem. |
| Apple | Yes | We support to reuse R17 SRAP as a baseline. FFS details |
| vivo |  | See comment to Q9A |
| Xiaomi | As baseline | R17 SRAP can be baseline. |
| Ericsson | Yes, only for the second hop | For the first hop, we don’t think the adaptation layer is needed, this can be just fully left up to UE implementation. For the second hop, we should reuse the R17 SRAP. |
| Qualcomm | Yes | The functions of data transfer, determination of UE ID and BEARER ID, determination of egress link is needed for E2E packets transmission. Since there is no logical channel on ideal connection, then the SRAP layer function of determination of egress link and determination of egress RLC channel should be disabled. It can be further discussed UEs in scenario 2 just ignores the egress RLC channel related configuration or the gNB does not configure the configuration. |
| CMCC | SRAP is NOT needed | See comments in Q9A. |
| Nokia | Comments | Before agreeing on the need and the function of the adaptation layer, it cannot be decided if Rel-17 SRAP is suitable or not. Of course, if Rel-17 SRAP is suitable, then reusing it is sensible decision. |
| Huawei, HiSilicon | See comments | If RAN2 concludes the adaptation layer is required for scenario 2, then the same protocol layer can be adopted for both scenario 1 and 2. |
| ZTE | See comments | If the adaptation layer of UE-UE interface is supported for the purpose of UE interoperability, the SRAP may be reused as baseline. |
| Samsung | Yes |  |

# Conclusion and recommendation

In conclusion, Rapporteur recommends agreeing the following proposals:

# Agreements on MP

RAN2#119-e

- RAN2 anticipate benefits from multi-path in the following areas:

  - Relay and direct multi-path operation (including both scenarios 1 and 2) can provide efficient path switching between direct path and indirect path

  - The remote UE in multi-path operation can provide enhanced user data throughput and reliability compared to a single link

  - gNB can offload the direct connection of the remote UE in congestion to indirect connection via the relay UE (e.g. at different intra/inter-frequency cells)

- RAN2 can confirm the justifiable benefits that multi-path with relay and UE aggregation can improve the throughput and reliability/robustness, e.g., for UE at the edge of a cell, and UE with limited UL transmission power.

- The terms “relay UE” and “remote UE” are used for scenarios 1 and 2.  FFS if we would use additional terms specific to scenario 2.

- Confirm the remote UE in Scenario 1 and the remote UE in Scenario 2 as follows:

  - Scenario 1: the remote UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay,

  - Scenario 2: the remote UE is connected to the same gNB using one direct path and one indirect path via 2) via another UE (where the UE-UE inter-connection is assumed to be ideal).

- RAN2 assumes that the relation between remote UE and relay UE in scenario 2 is pre-configured or static and how the relation is pre-configured or static is out of the 3GPP scope.

- RAN2 deprioritizes discussion on authorization and association mechanism between remote UE and relay UE in scenario 2.

- Support the following cell deployment scenarios for multi-path relaying in Rel-18:

  - Scenario C1: The relay UE and remote UE are served by a same cell.

  - Scenario C2: The relay UE and remote UE are served by different intra-frequency cells of a same gNB

  - Scenario C3: The relay UE and remote UE are served by different inter-frequency cells of a same gNB

- Support the following sidelink scenarios for multi-path:

  - Scenario S1: SL TX/RX and Uu share the same carrier at the remote UE.

  - Scenario S2: SL TX/RX and Uu use different carriers at the remote UE.

  - Scenario S3: SL TX/RX and Uu share the same carrier at the relay UE.

  - Scenario S4: SL TX/RX and Uu use different carriers at the relay UE.

- Support direct bearer (bearer mapped to direct path on Uu), indirect bearer (bearer mapped to indirect path via relay UE), and MP split bearer (bearer mapped to both paths, based on the existing split bearer framework).

- For a MP split bearer in scenario 1, one PDCP entity at the remote UE is configured with one direct Uu RLC channel and one indirect PC5 RLC channel.

  - For upstream, a PDCP entity delivers to a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side.

  - For downstream, a PDCP entity receives from a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side.

- FFS if we need to take decisions on the mapping of protocol entities in scenario 2.

RAN3#117-e

- From RAN3 perspective, multi-path scenario should be supported in Rel-18.

- Both intra-DU and inter-DU cases will be supported under the same gNB.

- RAN3 waits for the RAN2 progress on how to define control plane and user plane scenarios for multi-path support.

- RAN3 waits for the RAN2 progress on whether and how to define the Primary path in multi-path support.

- Addition of direct/indirect path are supported as follows:

  - Add direct path, after the establishment of the indirect path.

  - Add indirect path, after the establishment of the direct path.

  - This does not imply the exclusion of any other path addition possibility.

- RAN3 will study the signaling impact on the direct or indirect path change under the same gNB for a UE connected via multi-path. The other mobility scenarios can be further considered based on RAN2 decision.

- The following use cases are not supported in Rel-18.

  - Configure two indirect paths

  - More than two paths

  - Inter-gNB multi-path support