3GPP TSG RAN2 #120								R2-22xxxxx

Toulouse, France, 14th – 18th November, 2022

Agenda Item:		XX.YY

Source:			Huawei (email rapporteur)

Title:			Report of [Post119bis-e][877][R18 SON/MDT] RACH enhancement (Huawei)

Document for:		Discussion and Decision

1   Introduction

This is the email report of [Post119bis-e][877]:

[Post119bis-e][877][R18 SON/MDT] RACH enhancement (Huawei)

Focus on P2/7/8 in R2-2210793. Discussion can be used to collect companies’ opinions on these topics.

Intended outcome: Report

Deadline: Nov 3rd

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |

2   Discussion

2.1   Enhancements of the RA report based on some features

In the previous report [16], P2 was made.

Summary proposal 2: RAN2 discuss enhancement of the RA report based on the specific features such as Msg3 repetition, SDT operation, Slicing, Redcap, SCG Activation/Deactivation.

It can be seen that 5 features are mentioned in the above proposal, and they are from some contributions at RAN2#119b-e meeting. So it is proposed to check these features one by one.

2.1.1   Msg3 repetition

At RAN2#119b-e meeting, some proposals are related to Msg3 repetition, and here is a summary:

| Tdoc | Proposals |
| --- | --- |
| [12], ZTE | Proposal 5: UE includes indication to indicate whether RSRP of selected beam is above rsrp-ThresholdMsg3 or not per RA attempt. Proposal 6: Include Msg3 repetition number configured and applied for the RA procedure. |
| [8], NEC | Proposal 3: RAN2 also considers to store and report RA related information with regarding the following RACH enhancement in Rel-17  Msg3 repetition |
| [5], Samsung | Proposal 3: Include RACH information related to features involving RA partitioning (SDT, slicing, msg3 repetition and Redcap) in RACH report. |

It can be seen that the contribution [12] provided some specific proposals, so it is suggested to discuss them.

Q1: Do companies agree with P5 and P6 in [12]? Please provide your comments in the comment column if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

2.1.2   SDT operation

At RAN2#119b-e meeting, some proposals are related to SDT operation, and here is a summary:

| Tdoc | Proposals |
| --- | --- |
| [5], Samsung | Proposal 3: Include RACH information related to features involving RA partitioning (SDT, slicing, msg3 repetition and Redcap) in RACH report. |
| [10], Ericsson | Proposal 4 UE includes RA and SDT information in RA report when an SDT operation fails. |

It can be seen that the contribution [10] provided a specific proposal, so it is suggested to discuss it.

Q2: Do companies agree with P4 in [10]? Please provide your comments in the comment column if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

2.1.3   Slicing

At RAN2#119b-e meeting, some proposals are related to Slicing, and here is a summary:

| Tdoc | Proposals |
| --- | --- |
| [5], Samsung | Proposal 2: When the applicable feature is slicing, include NSAG Id and NAS provided NSAG priority of the relevant NSAGs in RACH report. Proposal 3: Include RACH information related to features involving RA partitioning (SDT, slicing, msg3 repetition and Redcap) in RACH report. |

It can be seen that the contribution [5] provided a specific proposal, so it is suggested to discuss it.

Q3: Do companies agree with P2 in [5]? Please provide your comments in the comment column if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

2.1.4   Redcap

At RAN2#119b-e meeting, some proposals are related to Redcap, and here is a summary:

| Tdoc | Proposals |
| --- | --- |
| [5], Samsung | Proposal 3: Include RACH information related to features involving RA partitioning (SDT, slicing, msg3 repetition and Redcap) in RACH report. |

It can be seen that the contribution [5] just mentioned the Redcap, but no specific proposals are provided. So it is suggested to collect companies’ views on possible enhancements.

Q4: For enhancements of the RA report based on Redcap, what are the companies’ views on possible enhancements?

| Company | Comments |
| --- | --- |

2.1.5   SCG Activation/Deactivation

At RAN2#119b-e meeting, some proposals are related to SCG Activation/Deactivation, and here is a summary:

| Tdoc | Proposals |
| --- | --- |
| [8], NEC | Proposal 3: RAN2 also considers to store and report RA related information with regarding the following RACH enhancement in Rel-17  SCG activation/deactivation |

It can be seen that the contribution [8] just mentioned the SCG Activation/Deactivation, but no specific proposals are provided. So it is suggested to collect companies’ views on possible enhancements.

Q5: For enhancements of the RA report based on SCG Activation/Deactivation, what are the companies’ views on possible enhancements?

| Company | Comments |
| --- | --- |

2.2   SgNB RACH report for MR-DC scenario

In the previous report [16], P7 and P8 were made.

Summary proposal 7: For NE-DC, the UE collects SN RA report container (for LTE) and reports to the NR MN. Additionally, the UE also includes the PSCell identity for the stored SN RA report (FFS on the format). Summary proposal 8: For EN-DC and NG-EN-DC, there are the following options: - the NR SN fetching the list of NR RA reports via SRB3 can be considered for the SN RACH report in the (NG) EN-DC scenario - the UE collects SN RA report container (for NR) and reports to the LTE MN, and additionally the UE also includes the PSCell identity for the stored SN RA report (FFS on the format).

In addition, RAN3 agreed on the LS [17], which is likely to be treated at RAN2#120. In the LS, RAN3 mentions the following (while other content is about RACH report retrieval):

- RAN3 has supported SN RA Report for EN-DC, (NG)EN-DC, and NR-DC scenarios in Rel-17. No further work will be triggered in RAN3.

- RAN3 believes that if RAN2 decides to support SN RA Report for EN-DC and (NG)EN-DC, the UE should report the PSCell identity outside the RACH report to help an eNB forward the report to the correct node without the need to decode the RACH report.

It is the rapporteur’s understanding that the LS [17] can be considered here.

Q6: Do companies agree with P7 in [16]? Please provide your comments in the comment column if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

Q7: Do companies agree with P8 in [16]? Please provide your comments in the comment column if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

3   Conclusion

[To be updated]

4   References (RAN2#119b-e Tdocs for AI 8.13.6 RACH enhancement)

[1] R2_119bis-e_Skeleton_v2

[2] R2-2209567	Discussion on RACH report enhancement for RACH partitioning	vivo	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

[3] R2-2209572	RACH enhancement for SON	CATT	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

[4] R2-2209766	SON enhancements for RACH partitioning	Apple	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

[5] R2-2209825	SON/MDT Enhancements for RACH	Samsung R&D Institute India	discussion

[6] R2-2209898	Discussion on RACH enhancement	Huawei, HiSilicon	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

[7] R2-2209986	RACH report enhancements for RACH partition	Spreadtrum Communications	discussion	Rel-18

[8] R2-2209999	Discussion on RACH enhancements	NEC	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

[9] R2-2210030	Discussion on the SON/MDT enhancement for RACH report	Beijing Xiaomi Software Tech	discussion	Rel-18

[10] R2-2210179	RACH report enhancements	Ericsson	discussion	NR_ENDC_SON_MDT_enh2-Core

[11] R2-2210271	RACH report related enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

[12] R2-2210291	Consideration on RACH enhancements	ZTE Corporation, Sanechips	discussion	Rel-18

[13] R2-2210511	SONMDT enhancement for RACH Enhancement.	CMCC	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

[14] R2-2210574	Discussion on RACH partitioning	China Telecom Corporation Ltd.	discussion

[15] RAN2-119-e-SONMDT-HU _2022-10-19 0535 UTC

[16] R2-2210793, Pre-meeting summary of 8.13.6 (Huawei), Huawei (Summary rapporteur)

[17] R3-226053, Reply LS on SN RACH report status in R17, Source: RAN3, To: RAN2