3GPP TSG-RAN WG2 Meeting #120 Electronic	R2-220xxxx

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.21.1

Source:	Qualcomm (Rapporteur)

Title:	[Post120][052][NR17] higher granularity per-FR gap capability

WID/SID:	TEI17

Document for:	Discussion and Decision

# 1	Introduction

This document is kick off the post meeting discussion [052]:

Per-FR Gap

R2-2212388	Capability for per-FR gaps		Ericsson	discussion

R2-2211620	Discussion on per-FR gap 	Intel Corporation	discussion	Rel-17	TEI17

R2-2211363	More granular per-FR gaps	Nokia, Nokia Shanghai Bell	discussion	Rel-17	TEI17

R2-2212526	Higher granularity for per-FR gap capability discussion	Qualcomm Incorporated	discussion	Rel-17	TEI17

[Post120][052][NR17] higher granularity per-FR gap capability (Qualcomm)

Scope: Based on R2-2212527, R2-2212528, Review and update if needed, for agreement. Include also determination whether inter-node signalling is needed, and if so update CRs to include inter-node signaling.

Intended outcome: Tech Endorsed 38.331 38.306 CRs (for TSG RAN)

Deadline: Short

# 2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Qualcomm (Rapporteur) | Mouaffac | mambriss@qti.qualcomm.com |

# 3	Discussion

The intention behind this discussion is to:

- Check the draft CRs and provide feedback:

  - Modify the cover page of the CR to include (NG)EN-DC architecture.

  - Modify the capability CR to ensure independentGapConfig (legacy capability) and independentGapConfig-maxCC-r17 (new capability) are mutually exclusive.

- Check if there is a need to enhance the inter-node messaging to ensure proper coordination between MN and SN when this feature is supported.

One item still not agreed on, is the starting/ending range value for the N1/N2/N3. Some companies prefer it to start from [0..31], other from [1..32].

Question 1: please provide your preference for the N1/N2/N3 range:

Option-1: range is [0..31]

Option-2: range is [1..32]

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Selected Option | Please provide the technical Arguments behind your preference |
| Qualcomm Inc | 1 | This will allow the UE to provide value “0” to indicate that independentGapConfig is not supported when configured cells are: - all FR1 cells (N1 = 0) - or FR2 cells (N2 = 0) - or mix of FR1 and FR2 cells (N3 = 0) Subsequently when UE provides a N1/N2/N3 values > 0, then independentGapConfig will be supported when configured cells are: - all FR1 cells and number of serving cells >= N1 in this case, per 38.133 UE is expected to support gapless measurement on FR2 - all FR2 cells and number of serving cells >= N2 in this case, per 38.133 UE is expected to support gapless measurement on FR1 - FR1+FR2 serving cells >= N3 2 independent gap configurations is supported on FR1 and FR2 cells. |

Summary 1: TBD.

Proposal 1: TBD.

Question 2: is there a need to enhance the current inter-node messaging to ensure proper coordination exists between the MN and SN when this feature is supported?

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Please provide the technical Arguments that supports your claim |
| Qualcomm |  | It seems a minor introduce of 2 indications in both directions (MNSN) may be needed. |

Summary 2: TBD.

Proposal 2: TBD.

Question 3:do companies agree with the suggested inter-node messaging by ZTE (please check draft CR)

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Please provide the technical Arguments that supports your claim |

Summary 3: TBD.

Proposal 3: TBD.

# 4	Conclusion

TBD.