3GPP TSG-RAN WG2 Meeting #121				  								       R2-23xxxxx

Athens , GR, 27th Feb- 3rd Mar, 2023

Agenda item:	8.10.3

Source: 	vivo

Title: 	[Post120][651][IDC]Further details of TDM solution (vivo)

Document for:		Discussion

# 1.	Introduction

This paper is to trigger the following email discussion of IDC TDM solutions:

[Post120][651][IDC]  Further details of TDM solution (vivo)

Scope:

- Details of periodic pattern, e.g. values (applied use case), ASN.1

- Signalling details of TDM, e.g. how to configure, how to report..

- Details of autonomous denial (LTE as baseline, ASN.1 and procedure)

Intended outcome: Report to next meeting (with Text proposal)

Deadline:  Long

Deadline for comments: Long - Kick off: Jan 9th, Deadline for company inputs Jan, 20th.   Inactive Period January 23 to 27.  Comments on rapporteur summary Jan. 30th to February 3rd

## 1.1	Contacts

Contact person for each participating company:

| Company | Name | Email Address |
| --- | --- | --- |
| Vivo | Xiaodong Yang | Yangxiaodong5g@vivo.com |
| xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| Qualcomm | Sherif Elazzouni | selazzou@qti.qualcomm.com |

# 2.	Discussion

The objective related to the IDC TDM solution is quoted as follows:

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi). - Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4). Note: The TDM solution is considered complementary to the FDM solution. Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

The RAN2 agreements related to the IDC TDM solutions are quoted as follows:

RAN2#119 meeting agreements: The use cases (e.g. BT voice, BT eSCO and WLAN beacon) as described in 3GPP TR 36.816 for LTE TDM solutions are considered for developing the Rel-18 IDC TDM solution in RAN2. Rel-18 IDC TDM solution(s) targets at resolving the adjacent channel interference issue and the intermodulation distortion interference issue, as LTE. As the baseline, the UE reports the TDM assistance information for IDC affected frequency list , as LTE. Note, this does not exclude MUSIM gap like solution.

RAN2#120 meeting agreements: Periodic pattern is supported; FFS on the values; Option 3 (i.e. UL and/or DL transmission occasion(s) solution) is not supported in Rel-18. The periodic pattern reported by the UE includes cycle, start offset and active duration. FFS, whether multiple patterns are supported. FFS on per CG pattern. RAN2 confirms the understanding that in Rel-17 NR RRC, the values from periodic pattern in MUSIM-gap is a subset of the DRX parameters. NR DRX values can be treated as a starting point for assistance information reported by UE. FFS, on exact values. RAN2 reconfirms the previous RAN2 agreement that the aperiodic traffics as described in 3GPP TR 36.816 are considered for developing the Rel-18 IDC TDM solution in RAN2. Autonomous denial solution is supported in Rel-18 IDC, RAN2 will not introduce other solution on aperiodic use case (i.e. no report from UE on this aperiodic issue). Not agreed: the aperiodic gap in the MUSIM-gap solution is supported in Rel-18 IDC.

## 2.1 Periodic pattern

In RAN2#120 meeting, we have achieved below agreements regarding periodic pattern:

Periodic pattern is supported; FFS on the values;

The periodic pattern reported by the UE includes cycle, start offset and active duration. FFS, whether multiple patterns are supported. FFS on per CG pattern.

NR DRX values can be treated as a starting point for assistance information reported by UE. FFS, on exact values.

To reach a common understanding for periodic pattern parameters, i.e. cycle, start offset and active duration, firstly we would like to use ASN.1 to clarify the field description, then we further discuss the open issues of periodic pattern:

- FFS the exact values;

- FFS whether multiple patterns are supported.

- FFS on per CG pattern

The ASN.1 framework and field descriptions are as follows.

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

<skipped>

UEAssistanceInformation-v1610-IEs ::= SEQUENCE {

idc-Assistance-r16                  IDC-Assistance-r16                  OPTIONAL,

<skipped>

nonCriticalExtension                UEAssistanceInformation-v1700-IEs   OPTIONAL

}

UEAssistanceInformation-v1700-IEs ::= SEQUENCE {

ul-GapFR2-Preference-r17              UL-GapFR2-Preference-r17              OPTIONAL,

musim-Assistance-r17                  MUSIM-Assistance-r17                  OPTIONAL,

overheatingAssistance-r17             OverheatingAssistance-r17             OPTIONAL,

maxBW-PreferenceFR2-2-r17             MaxBW-PreferenceFR2-2-r17             OPTIONAL,

maxMIMO-LayerPreferenceFR2-2-r17      MaxMIMO-LayerPreferenceFR2-2-r17      OPTIONAL,

minSchedulingOffsetPreferenceExt-r17  MinSchedulingOffsetPreferenceExt-r17  OPTIONAL,

rlm-MeasRelaxationState-r17           BOOLEAN                               OPTIONAL,

bfd-MeasRelaxationState-r17           BIT STRING (SIZE (1..maxNrofServingCells)) OPTIONAL,

nonSDT-DataIndication-r17             SEQUENCE {

resumeCause-r17                       ResumeCause                       OPTIONAL

}                                                                           OPTIONAL,

scg-DeactivationPreference            ENUMERATED { scgDeactivationPreferred, noPreference }    OPTIONAL,

uplinkData-r17                        ENUMERATED { true }                   OPTIONAL,

rrm-MeasRelaxationFulfilment-r17      BOOLEAN                               OPTIONAL,

propagationDelayDifference-r17        PropagationDelayDifference-r17        OPTIONAL,

nonCriticalExtension                  UEAssistanceInformation-v17xy-IEs    OPTIONAL

}

UEAssistanceInformation-v17xy-IEs ::= SEQUENCE {

idc-Assistance-r18                IDC-Assistance-r18                  OPTIONAL,    nonCriticalExtension                  SEQUENCE {}                           OPTIONAL

}

IDC-Assistance-r16 ::=                  SEQUENCE {

affectedCarrierFreqList-r16             AffectedCarrierFreqList-r16               OPTIONAL,

affectedCarrierFreqCombList-r16         AffectedCarrierFreqCombList-r16           OPTIONAL,

...

}

IDC-Assistance-r18 ::=                  SEQUENCE {

tdm-AssistanceInfo-r18				TDM-AssistanceInfo-r18						OPTIONAL,

...

}

TDM-AssistanceInfo-r18 ::=	CHOICE {

periodicPatternInfo-r11				SEQUENCE {

cycleLength-r18					ENUMERATED {FFS},

startOffset-r18					INTEGER (FFS)	OPTIONAL,

activeDuration-r18				ENUMERATED {FFS}

},

...

}

AffectedCarrierFreqList-r16 ::= SEQUENCE (SIZE (1.. maxFreqIDC-r16)) OF AffectedCarrierFreq-r16

AffectedCarrierFreq-r16 ::=     SEQUENCE {

carrierFreq-r16                 ARFCN-ValueNR,

interferenceDirection-r16       ENUMERATED {nr, other, both, spare}

}

AffectedCarrierFreqCombList-r16 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqComb-r16

AffectedCarrierFreqComb-r16 ::=     SEQUENCE {

affectedCarrierFreqComb-r16         SEQUENCE (SIZE (2..maxNrofServingCells)) OF  ARFCN-ValueNR    OPTIONAL,

victimSystemType-r16                VictimSystemType-r16

}

VictimSystemType-r16 ::=    SEQUENCE {

gps-r16                     ENUMERATED {true}        OPTIONAL,

glonass-r16                 ENUMERATED {true}        OPTIONAL,

bds-r16                     ENUMERATED {true}        OPTIONAL,

galileo-r16                 ENUMERATED {true}        OPTIONAL,

navIC-r16                   ENUMERATED {true}        OPTIONAL,

wlan-r16                    ENUMERATED {true}        OPTIONAL,

bluetooth-r16               ENUMERATED {true}        OPTIONAL,

...

}

<skipped>

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

affectedCarrierFreqList Indicates a list of NR carrier frequencies that are affected by IDC problem.

affectedCarrierFreqCombList Indicates a list of NR carrier frequencie combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA.

activeDuration Indicates the desired active duration of periodic pattern that the NR is recommended to configure. FFS Value.

cycleLength Indicates the desired cycle length of periodic pattern that the NR is recommended to configure. FFS Value.

startOffset Indicates the desired starting offset of periodic pattern that the NR is recommended to configure. The UE shall set the value of startOffset smaller than the value of cycleLength.

victimSystemType Indicate the list of victim system types to which IDC interference is caused from NR when configured with UL CA. Value gps, glonass, bds, galileo and navIC indicates the type of GNSS. Value wlan indicates WLAN and value bluetooth indicates Bluetooth.

#### Question 1: Do you agree that ASN.1 framework and field description for the periodic pattern reported by the UE?

| Company | Answers (Yes/No) | Comments |
| --- | --- | --- |
| Xiaomi | Yes | We think that the ASN.1 framework provided by the rapporteur is a good starting point. The field type and the exact values can be discussed in the subsequent questions of the email discussion. |
| Qualcomm | As a start that can be extended | Agree that those fields are necessary for reporting and were already available in LTE. However, we should not preclude adding information that can be helpful to the NW such as the frequency information where the IDC problem happens. Also, agree with Xiaomi that field type and values can be further worked out. |

We would like to discuss the exact values of periodic pattern parameters. The use cases (e.g. BT voice, BT eSCO and WLAN beacon) as described in 3GPP TR 36.816 for LTE TDM solutions are considered for developing the Rel-18 IDC TDM solution in RAN2.

NR DRX values can be treated as a starting point for assistance information reported by UE. According to TS38.331, NR DRX long cycle values are as below, in the range of [10, 10240]ms.

-- ASN1START -- TAG-DRX-CONFIG-START DRX-Config ::= SEQUENCE { <Skipped> drx-LongCycleStartOffset CHOICE { ms10 INTEGER(0..9), ms20 INTEGER(0..19), ms32 INTEGER(0..31), ms40 INTEGER(0..39), ms60 INTEGER(0..59), ms64 INTEGER(0..63), ms70 INTEGER(0..69), ms80 INTEGER(0..79), ms128 INTEGER(0..127), ms160 INTEGER(0..159), ms256 INTEGER(0..255), ms320 INTEGER(0..319), ms512 INTEGER(0..511), ms640 INTEGER(0..639), ms1024 INTEGER(0..1023), ms1280 INTEGER(0..1279), ms2048 INTEGER(0..2047), ms2560 INTEGER(0..2559), ms5120 INTEGER(0..5119), ms10240 INTEGER(0..10239) }, shortDRX SEQUENCE { drx-ShortCycle ENUMERATED { ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }, drx-ShortCycleTimer INTEGER (1..16) } OPTIONAL, -- Need R drx-SlotOffset INTEGER (0..31) } <Skipped> } -- TAG-DRX-CONFIG-STOP -- ASN1STOP

#### Question 2: What’s the values of cycle, start offset and active duration in the periodic pattern recommended by the UE?

Option 1:NR values

Option 2: other values. Please specify.

| Company | Answers (Option 1/2) | Comments |
| --- | --- | --- |
| Xiaomi | Option 1 | Option 1 is aligned with the RAN2 agreement. It is probably better to clarify that the NR values of long DRX configuration is used, since the it is unclear how (and in which use case) the short DRX recommended by the UE can resolve the IDC issue. |
| Qualcomm | Option 2 | Disagree with Xiaomi that long DRX configurations only are needed. As mentioned in the rappoterur summary and previously agreed, BT eSCO should be supported which can need a gap <1ms over a ~4ms cycle so those numbers are not covered by the long cycle (to the extent that we will attempt to solve this problem using DRX). On the other hand tailoring values for use cases would be an overkill so we see no issue in including short+long cycle values. We would also like to add the value “infinity” to enable a one-shot gap request. Due to lack of coordination and synchronization between NR and other RATs, the UE can resort to requesting one-shot gap when an important IDC event is anticipated. In these cases, having a periodic pattern is not useful. For example in a BT scenario, it is very hard for NR and BT to coexist in time periodically due to misalignment between cycles so it’s likely that the UE can apply some combination of autonomous denial and aperiodic request. |

In NR DRX-Config IE, the slot offset with 1/32ms granularity is configured for DRX values. We may discuss whether such a slot offset with 1/32ms granularity is required for periodic pattern.

#### Question 3: Whether the slot offset with 1/32ms granularity for cycle, startOffset and active duration is required?

| Company | Answers (Yes/No) | Comments |
| --- | --- | --- |
| Xiaomi | Yes At least for start offset and active duration | For cycle, there is no 1/32ms granularity for DRX cycle in the current RRC specification. We are open to discuss extra granularity for DRX cycle, since some uses cases (e.g. BT eSCO) may have very short interval (e.g. less than 1ms) or non-integer interval (e.g. 3.75ms). According to the current RAN4 specification of 38.133, introducing finer DRX cycle does not impact the RM requirement, as only 160/320ms DRX cycle are used to differentiate the UE RM requirements. However, we should limit the configuration of finer values only for DRX, so as to control the workload in RAN4, as RAN4 does not have sufficient TU reserved to define the RM requirements for extra measurement gap values. For start offset and active duration, we think that 1/32ms granularity should be reused as the current specification, and this can make the perodic pattern more accurate, as also indicated by the above use cases.. |
| Qualcomm | Yes | Agree with Xiaomi |

In RAN2#120, one open issue is whether multiple periodic patterns are supported, some companis clarified that multiple patterns are from MUSIM gaps. Please note that, if multiple periodic patterns are supported, the ASN.1 in Q1 will need an update.

#### Question 4: Whether multiple periodic patterns are supported? If yes, whether the assistance information enhancement is needed from UE reporting?

| Company | Answers (Yes/No) | Comments |
| --- | --- | --- |
| Xiaomi | No strong view | We see some benefits of introducing multiple periodic patterns from UE reporting. Since it is possible that the UE could be using several RATs simultaneously (e.g. WiFi for data transmission, BT for ear pod and NR for phone call), the periodic traffic patterns in different RAT(s) could be different and could also cause different interferences (e.g. 2.4GHz/ 5GHz / 6GHz) in different NR bands, providing multiple periodic patterns can help resovling the IDC issues in more complicated cases. However the proponents are recommended to provide the details on how the multiple periodic patterns are reported (e.g. per-UE patterns or others). The maximum number (e.g. 4) of multiple periodic patterns should also be limited to reduce the complexity at the gNB and the UE. After the reception of the assistance information, we consider that the gNB can reuse the Rel-17 muliple DRX configurations (i.e. up-to 2 DRX groups) and measurement gap configurations (e.g. up-to 4 (3 MUSIM periodic measurement gap and 1 legacy measurement gap)) . No extra standard effort is required. |
| Qualcomm | Yes | Since the most problematic bands (N40/N41) are the ones that see most interference from WiFi different channels (possibly with different configurations) and/or Bluetooth, it is very possible that a single UE can see multiple IDC patterns. In this case it is beneficial to the UE to report multiple DRX patterns. For this case, we can use 4 patterns as suggested by Xiaomi and following the MUSIM precedent, since many companies wanted to support MUSIM reporting. The UE can report up to 4 patterns in the same way it is suggested that one pattern is reported. We would like to emphasize that the UE reporting an issue via requested “DRX” cycle does not mean th gNB has to solve the problem via DRX configuration only as this is not always possible since the UE supports a single DRX configuration per-cell group. On the contrary, the gNB can use these reports for information and utilize any implementation tool at it’s disposal to solve the problem, e.g., avoid scheduling the UE at the problematic slots, balance several IDC issues in one DRX configuration, choose to ignore some IDC issues while solving the most severe ones, or enabling autonomous denials; thus we think there is no extra standard work required beyond UAI signalling details and no modifications in DRX operations are needed. If deployment complexity is an issue then we can stick to reporting one pattern with multiple patterns support configurable by the NW. |

One open issue is whether per CG pattern is supported. In the rapporteur’s understanding, this is regarding the IDC handling for MR-DC, including how to configure IDC for MCG and SCG, How to report the recommended periodic pattern for MCG and SCG.

#### Question 5: Whether per CG pattern is supported? If yes, whether and whats the enhancemens are needed from UE side and network side?

| Company | Answers (Yes/No) | Comments |
| --- | --- | --- |
| Xiaomi | Yes | This is required for EN-DC, since the NR periodic pattern reported to the eNB of LTE MCG is supposed to be used by the gNB of the NR SCG. It is probably more accurate to say that the periodic pattern reported is for NR affected frequencies. For the NR-DC, this per-CG pattern is not essential as the periodic pattern is reported together with the affected frequency list. The UE is not able to know whether a non-serving NR frequency reported in the affected frequency list will be configured for MCG or SCG. |
| Qualcomm | Yes | For MR-DC deployments, SN can configure and receive reporting via SRB3 without much MN involvement. This issue is especially important in EN-DC where the IDC problem is reported and solved fully within the NR SN. Since we did agreed not to do any LTE enhancements, it makes sense to enable SN reporting within NR. Modifications would be to allow configuration and reporting to happen in SN with the details identical to MN, and allow this signalling to happen over SRB3 when available. |

Regarding the signaling procedure, there are two open issues:

- How to configure the UE to report the TDM assistance information;

In LTE IDC solution, the UE is allowed to provide Time Domain Multiplexing (TDM) based assistance information when it is configured to provide IDC indications and upon change of IDC problem information, i.e. the UE may be configured to provide IDC assistance information, no matter for FDM or TDM.

- How does the UE provide the TDM assistance information.

In LTE IDC solution, when detecting the IDC issue, the UE provides the TDM assistance information for the affected frequencies.

Taking LTE solution as baseline, we have the text proposal for TS38.331 as below:

### 5.3.5 RRC reconfiguration #### 5.3.5.9 Other configuration The UE shall: 1> if the received otherConfig includes the idc-AssistanceConfig: 2> if idc-AssistanceConfig is set to setup: 3> consider itself to be configured to provide IDC assistance information in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide IDC assistance information; ### 5.7.4 UE Assistance Information #### 5.7.4.2 Initiation <skipped> A UE capable of providing IDC assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting IDC problem if the UE did not transmit an IDC assistance information since it was configured to provide IDC indications, or upon change of IDC problem information. <skipped> Upon initiating the procedure, the UE shall: 1> if configured to provide IDC assistance information: 2> if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information: 3> if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or 3> if on one or more supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself: 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information; 2> else if the current IDC assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information; NOTE 1: The term "IDC problems" refers to interference issues applicable across several subframes/slots where not necessarily all the subframes/slots are affected. NOTE 2: For the frequencies on which a serving cell or serving cells is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds. For frequencies on which a SCell or SCells is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell or SCells would result in interference issues that the UE would not be able to solve by itself. For a non-serving frequency, reporting IDC problems indicates an anticipation that if the non-serving frequency or frequencies became a serving frequency or serving frequencies then this would result in interference issues that the UE would not be able to solve by itself. #### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message The UE shall set the contents of the UEAssistanceInformation message as follows: <skipped> 1> if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3: 2> if there is at least one carrier frequency included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself: 3> include the field affectedCarrierFreqList with an entry for each affected carrier frequency included in candidateServingFreqListNR; 3> for each carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly; 3> include Time Domain Multiplexing (TDM) based assistance information, unless the UE has no Time Doman Multiplexing based assistance information that could be used to resolve the IDC problems: 4> if the UE has periodic pattern related assistance information that could be used to resolve the IDC problems, 5> include cycleLength, startOffset and activeDuration; 4> use the MCG as timing reference if TDM based assistance information regarding the SCG is included(FFS); 2> if there is at least one supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself: 3> include victimSystemType for each UL CA combination included in affectedCarrierFreqCombList; 3> if the UE sets victimSystemType to wlan or bluetooth: 4> include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems; 3> else: 4> optionally include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems; NOTE 1: When sending an UEAssistanceInformation message to inform the IDC problems, the UE includes all IDC assistance information (rather than providing e.g. the changed part(s) of the IDC assistance information). NOTE 2: Upon not anymore experiencing a particular IDC problem that the UE previously reported, the UE provides an IDC indication with the modified contents of the UEAssistanceInformation message (e.g. by not including the IDC assistance information in the idc-Assistance field).

#### Question 6: Do you agree the above signaling procedure of TDM?

| Company | Answers (Yes/No) | Comments |
| --- | --- | --- |
| Xiaomi | Partially yes | We think the procedural texts provided by the rapporteur can be a baseline for NR 38.331. We think that the EN-DC case may require some modification in the LTE specification, so that the UE can also provide the NR periodic pattern of the NR affected frequencies via the LTE MCG. |
| Qualcomm | Postpone | Text can be worked out afte most details are agreed. We see no concerns with rappoteurs suggestion as a starting point, but prefer concluding most critical issue before we assess procedural text. |

## 2.2 Autonomous denial

According to TS36.331, the LTE autonomous denial solution is quoted as follows:

Procedure: #### 5.3.10.9 Other configuration The UE shall: 1> if the received otherConfig includes the idc-Config: 2> if idc-Indication is included (i.e. set to setup): 3> consider itself to be configured to provide IDC indications in accordance with 5.6.9; 3> if idc-Indication-UL-CA is included (i.e. set to setup): 4> consider itself to be configured to indicate UL CA related information in IDC indications in accordance with 5.6.9; 3> if idc-HardwareSharingIndication is included (i.e. set to setup): 4> consider itself to be configured to indicate IDC hardware sharing problem indications in IDC indications in accordance with 5.6.9; 3> if idc-Indication-MRDC is included (i.e. set to setup): 4> consider itself to be configured to provide IDC indications for MR-DC in accordance with 5.6.9; 2> else: 3> consider itself not to be configured to provide IDC indications; 2> if autonomousDenialParameters is included: 3> consider itself to be allowed to deny any transmission in a particular UL subframe if during the number of subframes indicated by autonomousDenialValidity, preceeding and including this particular subframe, it autonomously denied fewer UL subframes than indicated by autonomousDenialSubframes; 2> else: 3> consider itself not to be allowed to deny any UL transmission;

ASN.1: IDC-Config-r11 ::= SEQUENCE { idc-Indication-r11 ENUMERATED {setup} OPTIONAL, -- Need OR autonomousDenialParameters-r11 SEQUENCE { autonomousDenialSubframes-r11 ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1}, autonomousDenialValidity-r11 ENUMERATED { sf200, sf500, sf1000, sf2000, spare4, spare3, spare2, spare1} } OPTIONAL, -- Need OR ..., [[ idc-Indication-UL-CA-r11 ENUMERATED {setup} OPTIONAL -- Cond idc-Ind ]], [[ idc-HardwareSharingIndication-r13 ENUMERATED {setup} OPTIONAL -- Need OR ]], [[ idc-Indication-MRDC-r15 CHOICE{ release NULL, setup CandidateServingFreqListNR-r15 } OPTIONAL -- Cond idc-Ind ]] }

Firstly, we discuss the time unit for NR autonomous denial. In LTE solution, the time unit is one subframe. The UE may autonomously deny fewer UL subframes than indicated by autonomousDenialSubframes. For NR, The UE may autonomously deny fewer UL slots.

Based on the time unit, we need futher confirm the values for Validity period and number of time units.

#### Question 7: What’s the values of Validity period and number of Subframes?

Option 1: Subframe as time unit,  reuse the values of LTE solution.

autonomousDenialParameters-r18 SEQUENCE { autonomousDenialSubframes-r18 ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1}, autonomousDenialValidity-r18 ENUMERATED { sf200, sf500, sf1000, sf2000, spare4, spare3, spare2, spare1} } OPTIONAL, -- Need R

Option 2: Slot as time unit. Please specify the values.

| Company | Answers (Option 1/2) | Comments |
| --- | --- | --- |
| Xiaomi | No strong view | We think both Option 1 and Option 2 can work. Option 2 is more adaptive to the NR framework, since slot is the transmission unit in NR. If we choose Option 1, some clarifications are needed on how many transmissions can be dropped within a NR subframe. If we choose Option 2, new values and parameters need to be defined. From our understanding, Option 1 is slightly simpler considering the specification change. |
| Qualcomm | Option 2 | We think slot as time unit would be simpler. The NW and UE can count any subslot dropping as a slot. We think subframe would be more complicated since NR does not utilize the subframe concept as much as LTE. More importantly, due to the flexible numerology of NR, it would be clearer to operate with slot time units since a dropped subframe can involve a different number of slots depending on numerology. We think the rest of LTE framework can be adopted while working on the appropriate values. |

By using the LTE autonomous denial solution as the baseline, the text proposal for the autonomous denial solution for NR is as follows.

Please note that the time unit uses subframe temporarily. the time unit and values could be further updated according to Q7.

### 5.3.5 RRC reconfiguration #### 5.3.5.9 Other configuration The UE shall: 1> if the received otherConfig includes the idc-AssistanceConfig: 2> if idc-AssistanceConfig is set to setup: 3> consider itself to be configured to provide IDC assistance information in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide IDC assistance information; 2> if autonomousDenialParameters is included: 3> consider itself to be allowed to deny any transmission in a particular subframe if during the number of subframes indicated by autonomousDenialValidity, preceeding and including this particular subframe, it autonomously denied fewer subframes than indicated by autonomousDenialSubframes;

ASN.1: IDC-AssistanceConfig-r16 ::= SEQUENCE { candidateServingFreqListNR-r16 CandidateServingFreqListNR-r16 OPTIONAL, -- Need R ... [[autonomousDenialParameters-r18 SEQUENCE { autonomousDenialSubframes-r18 ENUMERATED {FFS}, autonomousDenialValidity-r18 ENUMERATED {FFS} } OPTIONAL -- Need R ]], }

#### Question 8: Do you agree the ASN.1 and signalling procedure of autonomous denial?

| Company | Answers (Yes/No) | Comments |
| --- | --- | --- |
| Xiaomi | Partially yes | We think that the the ASN.1 and signalling procedure of autonomous denial provided by the rapporteur can be considered as the baseline. As also indicated in our answer for Question 7, either subframe-granularity or the slot-granularity may require some clarifications in the procedural text. If we choose Option 1, some clarifications are needed on how many transmissions can be dropped within a NR subframe. If we choose Option 2, new values and parameters need to be defined. |
| Qualcomm | No | This can be worked out after agreeing in Q7, but following the LTE mechanism by including some variation of the proposed two fields should be agreeable. |

## 2.3 RAN4 Impact

Autonomous denial solution needs new RAN4 performance requirements. From the rapporteur’s understanding, A LS to RAN4 could be needed, to inform RAN4 the agreements of TDM solution, such as the values of autonomous denial parameters.

#### Question 9: Whether an LS to RAN4 is needed?

| Company | Answers (Yes/No) | Comments |
| --- | --- | --- |
| Xiaomi | Yes | As indicated in the WID and workplan, we need to inform RAN4 at least of the autonomous denial parameters. Furthermore, we consider that we could also inform RAN4 that the finer values (if agreed) for the periodic pattern is used for the DRX configuration, and no extra measurement gap configuration is required. |
| Qualcomm | Yes | Agree with Xiaomi |

# 3.	Conclusion

After collecting companies’ feedbacks, the discussion on the IDC TDM solutions is summarized as follows:

TBD

# 4. Text proposal

To be updated according to conclusion.

# 4.	Reference

[1] R2-2211978	Summary of [Post119-e][651][IDC] Comparison of TDM solutions (Xiaomi)	Xiaomi	discussion	Rel-18	NR_IDC_enh-Core