3GPP TSG-RAN WG2 Meeting #121-bis                                    	R2-230xxxx

E-Meeting: April 17-26, 2023

Agenda item:	8.6.3.1

Source:	Qualcomm Incorporated

Title:	Report of [POST121][105][IoT NTN Enh] Neighbour cell assistance information

Document for:	Discussion and Decision

# Introduction

This document provides the report of the following email discussion.

- [POST121][105][IoT NTN Enh] Neighbour cell assistance information (Qualcomm)

Scope: Discuss the possible content of neighbour cell assistance information and whether this should be included in an existing or new SIB

Intended outcome: report of the email discussion

Deadline:  Long

# Discussion

Content of the neighbor cell assistance information

RAN4 has sent LS in [1] indicating that following information is helpful for UE to perform neighbor cell measurements.

Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.

Accordingly, the Rel-18 WID is updated to include the signaling details for neighbor satellite information.

In addition, the common TA parameters should also be provided as optional parameters as clarified by RAN4 in [1]. There are also discussions on time and location-based enhancements for neighbor cell measurements and cell reselection procedure, for which, additional information like beam information or reference location will need to be provided for the neighbor cell.

- In addition to ephemeris and optional epoch time of the satellite associated with a neighbor cell, what other parameters can be optionally broadcast as neighbor cell assistance information?

- Validity duration

- Common TA parameters

- For moving cell, footprint information

- For fixed cell, satellite start time and stop time

- Reference location and distance threshold/radius

- Others

- Note: List of frequen4cies/cells, satellite IDs are addressed in Q2 and Q3.

| Company | Options | Comments |
| --- | --- | --- |
| MediaTek | 1,2,4,5 | Validity duration can be optional, as it can be absent when it is same as the serving cell. For moving cell, (5) reference location and distance threshold/radius is similar to (3) footprint information. Hence, it seems that Option 5 cover for both earth moving cell and earth fixed cell and thus option 3 is not needed. |
| Qualcomm | 1,2, (3 or 5),4 | All these information are helpful and network can optionally provide to UEs. |
| Lenovo | 1,2,4 for all cells, and: 5 for fixed cells 3 or 5 for moving cells | We have not decided on what to be indicated for the moving neighbour cell and how it is represented. Both 3 and 5 are possible and we think it is better to have it open for now. |
| OPPO | 1 | For 2) common TA parameters, it is not needed for IoT NTN as there is no SMTC in IoT NTN and therefore no need for SMTC adjustment. For 4), it is sufficient to have it for serving cell and no need for neighbour cells. For 5), they are not needed for Idle mode UEs, but reference location may be needed for connected mode UEs, e.g. for conditional D1 event for CHO. |
| Apple | 1, 2, 4 5 for moving cells | Between 3 and 5 for moving cell, 5 seems simpler than 3. |

In NR, the list of neighbor cells is provided which created long discussion on associating neighbor cell and corresponding satellite. In IoT NTN case, the other option is to provide list of satellite and include the frequency and list of cells under each satellite information. The absence of ephemeris can also be defined as it is possible that the neighbor cell is also associated with the serving satellite. An example is shown below.

SatelliteInfoList-r18 ::=	SEQUENCE (SIZE (1..maxSat-r17)) OF SatelliteInfo-r18

SatelliteInfo-r18::=			SEQUENCE {

satelliteId-r18				INTEGER (0..255),

ephemerisInfo-r18			CHOICE {

orbitalParameters-r18			EphemerisOrbitalParameters-r18,

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}			OPTIONAL	-- Need OR

}				OPTIONAL,	-- Need OP

carrierFreqList-r18			CarrierFreqList-r18	OPTIONAL,	-- Need OP

nta-CommonParameters-r18			SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

} 				OPTIONAL,	-- Need OP

ul-SyncValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900} OPTIONAL,	-- Need OP

footprintInfo-r18			SEQUENCE {

referencePoint-r18			SEQUENCE {

longitude-r18				INTEGER (-131072..131071),

latitude-r18				INTEGER (-131072..131071)

} OPTIONAL,	-- Need OR

radius-r18					INTEGER (1..256)					OPTIONAL	-- Need OR

}				OPTIONAL,	-- Need OP

serviceInfo-r18				SEQUENCE {

t-ServiceStart-r18				TimeOffsetUTC-r17				OPTIONAL,	-- Need OR

t-ServiceStop-r18				TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}				OPTIONAL	-- Need OP

}

CarrierFreqList-r18 ::=		SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqInfo-r18

CarrierFreqInfo-r18 ::=	SEQUENCE {

FreqNeighCellList-r18				SEQUENCE (SIZE (1..maxCellInter)) OF PhysCellId			OPTIONAL,		-- Need OR

...

}

- How should the list of frequencies and list of cells associated with a satellite be included?

- Similar to NR, list of neighbor cells and associate each cell with the frequency and satellite information.

- Similar to SIB32, list of satellites and include list of frequencies/cells for each satellite. Absence of ephemeris could mean serving satellite.

| Company | Options | Comments |
| --- | --- | --- |
| MediaTek | Option 2 | We think Option 2 is easier to maintain and understand. |
| Qualcomm | Option 2 | The corresponding cell list can be provided for each satellite. We agree it would be simpler. |
| Lenovo | Option 2 |  |
| OPPO | Option 2 |  |
| Apple | No strong views | We can go with the majority view. |

It may not be possible to broadcast all neighbor satellites information due to TBS size limit. When configuring measurement in connected mode, it is possible that network may want the UE to perform measurement of the cell associated with the satellite whose information is not being broadcast. In this case network can provide all necessary satellite information via dedicated information. However, it is possible that the network wants UE to perform the measurement of the cell associated with the satellite whose information is already being broadcast, in this case signaling overhead can be reduced by just providing satellite ID.

- Do you agree to include satellite ID (as done in SIB32) such that this could be provided to UE via SIB3/4/5 or via RRC for connected mode measurements?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Mediatek | No strong view | Okay to go with the majority. |
| Qualcomm | Yes | This is nice to have. |
| Lenovo | Yes | OK to have |
| OPPO | No | We are not sure what neighbour cell’s information needs to be signalled to the UE for connected mode measurement since in LTE we don’t have PDD report and SMTC configuration. It would be good that proponents can provide detailed information and also the reason why this information is needed for connected mode measurement. |
| Apple | Yes | This indeed can give UE more information whether to check IoT NTN SIB to acquire the corresponding ephemeris data. |

Existing SIB vs new SIB

The other debate now is where to signal additional information such as ephemeris, epoch time and possibly many other parameters such as common TA, list of frequencies/cells, service time, reference location etc. that apply ONLY to the Rel-18 UEs as legacy UEs (i.e., Rel-17 UEs) won’t understand this information. One simple option is to extend the existing SIB, i.e., SIB31 to include addition neighbor cell information and the other option is to introduce new SIB to carry the neighbor cell information. There are also other options raised, such as repurposing the SIB32 for neighbor cell measurement or extending SIB3/4/5.

More information on each option is provided in the table.

| Option#1: extend SIB31 | Option#2: New SIBxx (i.e., another SIB32 version for measurement purpose) | Option#3: extend existing SIB3/4/5 | Option#4: Repurpose SIB32 |
| --- | --- | --- | --- |
| 1. Additional burden for legacy UEs (Rel-17 UEs) to acquire the larger TBS SIB with contents that must be discarded. 2. SIB31 is essential, and UE needs to acquire it and maintain it, however the neighbor cell information is not. 3. Larger the TBS, larger the number of repetitions required. This can negatively impact initial access delay. | 1. Not essential for initial access. Up to UE when to acquire and how to maintain. 2. Not subject to SIB update notification procedure 3. Network will have flexibility in scheduling and new SIB can be scheduled with larger periodicity unlike SIB31 4. Legacy UE does not need to acquire 5. it is possible for network to schedule SI message including essential SIBs together for initial access as SIB31 TBS size remains unchanged. - SIB2 + SIB31 | For each inter-frequency or intra-frequency cell, satellite information needs to be provided. This could lead to more signaling overhead and complexity. | 1. This does not work as legacy UEs will assume the discontinuous coverage is enabled, that will be wrong design. 2. The UEs not supporting discontinuous coverage, will not even try to acquire SIB32. |

- Where (in which SIB) the neighbor cell/satellite information should be broadcast?

- Extend SIB31.

- New SIBxx.

- Extend SIB3/4/5.

- Repurpose SIB32

| Company | Options | Comments |
| --- | --- | --- |
| Mediatek | Option 2 | A new SIB is preferred in order to obtain the maximum possible space for neighbor cell information. Option (4) will not work and Option (1) and Option (3) will incur additional overhead in signalling. |
| Qualcomm | Option 2 | It is better not to make SIB31 worse. Option 2 can accommodate more satellite information. |
| Lenovo | Option 2 |  |
| OPPO | Option 2 |  |
| Apple | Option 2 |  |

- If answer to Question 4 is (1) i.e., extend SIB31, how to address the following issues?

Issue#1: SI TBS size limit is 936 bits for eMTC and 680 bits for NB-IoT and SIB31 is essential. Legacy Rel-17 UEs will have burden to maintain valid SIB31 with most of the content being discarded from SIB31.

Issue#2: As size of SIB31 grows larger, it may not be possible to schedule SIB31 in a SI message with essential SIBs such as SIB2. This can add network constraint in SIB31 scheduling with shorter periodicity to reduce initial access delay. Otherwise, UE will suffer with larger initial access delay to initiate random access.

| Company | Comments on issue#1 | Comments on Issue#2 |
| --- | --- | --- |
| Mediatek | The TBS size limit for NB-IoT is 680 bits. |  |
| Qualcomm | Thank you for correction. To be more accurate, it is 936 bits for eMTC. |  |

- Any other issues to cover?

| Company | Comments |
| --- | --- |

# Conclusion

No table of contents entries found.

# Reference

[1] R2-2211171, “LS on information for neighbor/target cell in IoT NTN”, Toulouse, 14th - 18th November, 2022.