3GPP TSG-RAN WG2 #121-bis-e	R2-xxxxxxx

E-Meeting, Apr 17 – Apr 26, 2023

Source: 	Qualcomm Incorporated

Title: 	Report of [Post121][044][eMob] SCG Selective Activation in NR-DC Signalling interaction

Agenda Item:	8.4.3

Document for:	Discussion and decision

# Introduction

This document is a summary of the following email discussion.

- [Post121][044][eMob] SCG Selective Activation in NR-DC Signalling interaction (QC)

Scope: Progress proposals on stage2ish detailed level for the signalling, expected outcome agreeable signalling charts with text, and/or parts text only.

Intended outcome: Report

Deadline: Long, Friday, Mar 31, 2023

Please try to provide your comments by the end of Wednesday, Mar 29, 2023. This will give some time to prepare the signalling flows based on agreed proposals by the final deadline Friday, Mar 31, 2023.

Please provide your comments for the revised Question 4 in Section 3 by the final deadline on Friday, Mar 31, 2023.

Companies are invited to indicate the person providing input in the table below:

| Company | Name / Email |
| --- | --- |
| Huawei, HiSilicon | David Lecompte / david.lecompte@huawei.com |
| Lenovo | Congchi Zhang, zhangcc16@lenovo.com |
| OPPO | youxin@oppo.com |
| vivo | liangjing@vivo.com |
| CATT | zhourui@catt.cn |
| Ericsson | cecilia.eklof@ericsson.com |
| Futurewei | Jialinzou88@yahoo.com |
| ZTE | Mengjie Zhang / zhang.mengjie@zte.com.cn |
| NEC | Hisashi Futaki / hisashi.futaki @ nec.com |
| Xiaomi | Yi Xiong / xiongyi3@xiaomi.com |
| NTT DOCOMO | souki.watanabe.gf@nttdocomo.com |
| ITRI | NellenHuang@itri.org.tw |
| Nokia | Srinivasan.selvaganapathy@nokia.com |
| MediaTek | Felix Tsai / chun-fan.tsai@mediatek.com |
| Intel | Tangxun/ xun.tang@intel.com |
| Qualcomm | Punyaslok Purkayastha / punyaslo@qti.qualcomm.com |

# Discussion

In the last RAN2 #121 meeting [1], it was agreed to support the following scenarios for SCG Selective Activation:

- SN initiated intra-SN SCG selective activation.

- MN initiated inter-SN SCG selective activation.

- SN initiated inter-SN SCG selective activation.

In the following subsections, we discuss proposals related to these scenarios.

Once a set of proposals are agreed, we will create signalling flows with descriptions based on the agreed proposals, and on proposals agreed in the previous RAN2 meetings.

## MN initiated inter-SN SCG selective activation

Figure 1 shows a signalling flow for the procedure. In the figure the UE is assumed to be in DC operation when the procedure is initiated. We describe the steps in the signalling flow and proposals related to the steps made by companies in their RAN2 #121 meeting contributions [1].

Steps 1-3:

Based on UE measurement reports, source MN initiates the procedure by transmitting SN Addition Request to the candidate SNs for configuring the candidate target PSCells.

Figure 1: Source MN initiated procedure for SCG selective activation

In this case, similar as in Rel-17 MN initiated CPA/CPC, a proposal by some companies [27], [16], [8], is that source MN can generate all the execution conditions in the procedure. This proposal is aligned with the legacy principle that the node which initiates the preparation phase of the procedure should generate the execution conditions.

We consider later another option where candidate SNs can generate some of the execution conditions in the procedure.

In legacy MN initiated PSCell change (Rel-17 inter-SN CPC, Rel-15 SN/PSCell change), only A4 events were defined since the procedures were understood to be used for load balancing purposes only. Some companies [18], [16], have proposed to consider A3/A5 events for Rel-18 MN initiated SCG Selective Activation, where the candidate target PSCell is compared with the current serving PSCell. The reasons provided were: (a) in principle, for PSCell mobility, the candidate target PSCell should be compared with the current serving PSCell, (b) the Rel-18 procedure is also for subsequent PSCell changes after a first PSCell change, and (c) preventing an unnecessary PSCell change if the serving PSCell quality is good, which can happen if only A4 is used.

Question 1: Do companies agree on the following proposals regarding execution conditions for the MN initiated inter-SN SCG Selective Activation?

- Source MN can generate all the execution conditions in the procedure (similar as in Rel-17 MN initiated inter-SN CPC).

- A3/A5 events, where the candidate target PSCell is compared with the current serving PSCell, should be supported.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | Since Rel-15, the SN is responsible for mobility due to coverage while the MN only considers SN change for load balancing. This principle was followed also for CPC in Rel-17, i.e. only A4 is used by the MN for CPAC. One reason for the decision is that the SN always takes care of mobility for coverage, so it would be redundant to introduce mobility for coverage by the MN, and this may also require extra MN-SN coordination to avoid conflicts. Therefore, we expect that: - for initial CPAC, the MN initially configures conditions using A4 (for load balancing) - for subsequent CPC, execution conditions are configured by (candidate) SN(s) (for coverage) |
| Lenovo | (a) Yes (b) Maybe | a) In case of MN initiated SCG selective activation, we can follow the legacy and MN generates the execution conditions. As a baseline A4 is possible, although it is in principle for load balancing purposes only. b) For A3/A5 events, the comparison is between the candidate PSCell and the current serving PSCell. Considering the serving PSCell during SCG selective activation could change, it implies that every candidate PSCell could be configured with a set of execution conditions each is associated with a possible serving PSCell. In this sense, since MN know all possible candidate PSCells, it’s easier if MN can configure the A3/A5 events. Otherwise, if we let candidate SN configure the A3/A5 events, that could imply the candidate SN should be aware of the existence of candidate PSCells belonging to other SNs, which seems a bit complicated. On the other hand, we understand that when the target SN becomes the new source SN, it can update the execution conditions as part of SN initiated SCG selective activation. |
| OPPO | No | For MN initiated inter-SN CPC, it is usually triggered for load balancing purpose and A4 event is sufficient to find a PScell. If A3/A5 is introduced, we have the following concerns: - In legacy A3/A5 events, if the execution conditions are generated by MN, UE evaluates the event by comparing the measurements with PCell. If A3/A5 events is considered, the measurement configuration should be provided by SN. Otherwise, we need to introduce new measurement event to differentiate whether the event refer to Pcell or Pscell. [QC (Rapp)] Agree that a new measurement event may be needed, or the existing definition of A3/A5 may need to be enhanced to cover this case. - For MN initiated inter-SN SCG Selective Activation, we have agreed that CPA configuration can be used for CPC, which means at least A4 event should be provided to UE for CPA scenario. If A3/A5 is introduced, extra interaction between SN and MN may be caused. |
| vivo | Yes for a), No for b) | a) is quite straightforward to us as it is aligned with the legacy principle. For b), although we share the same view that there are benefits for A3/A5 events, we understand they can be anyway used in SN-initiated inter-SN procedures, so no need to support A3/A5 by MN with extra effort compared with Rel-17. |
| CATT |  | For (a), it depends on which execution conditions configuration solution is chosen. For (b),for legacy A3/A5, it compares candidate PSCells with the source PSCell, , the execution condition may be invalid as the source PSCell will be changed after each SCG change.so directly resuing legacy A3/A5 could be problematic. [QC (Rapp)] This may not be an issue. MN can configure the A3/A5 events for comparing the new source PSCell, i.e., the source PSCell after the cell change, with the candidate PSCells for the next CPC. |
| Ericsson | - Yes - Yes | MN initiated is only useful if A3/A5 events are supported. A4 cannot be used since it doesn’t take the serving cell into account. In particular, this is problematic at intra-frequency cell, as the UE may change cell to a cell that is worse than the serving cell. The UE is not aware of the load situation, so the use case for load balancing is not clear. |
| Futurewei | OK for a) No for b) | Regarding triggering event, the principle in R17 can also be held here as a simple solution. But it is also doable to have candidate provide their event triggering conditions for the measurement on each candidate. |
| ZTE | No | For (a), the MN initiated inter-SN CPC is usually used for load balancing. However, the subsequent conditional PSCell mobility triggered by the UE is mainly due to the coverage problem. Thus, we think the MN initiated inter-SN SCG Selective Activation just means the preparation procedure is initiated by the MN. The MN can generate the execution conditions for the candidate PSCells suggested by the MN, while the candidate SN (including the source SN if it’s configured as a candidate SN as well) can also generate the execution conditions for the candidate PSCells suggested by the candidate SN, i.e. used for the subsequent CPC when the candidate SN becomes the serving SN. For (b), if it’s allowed to let the candidate SN generate the execution conditions (i.e. A3/A5 events, based on candidate SCG MeasConfig), then we think no much need to support A3/A5 events for MN initiated CPC. Besides, it may be difficult for the MN to generate the suitable A3/A5 thresholds for the serving PSCell’s neighbour cells since the MN may not know such neighbour cells. |
| NEC | (a) Yes (b) No | (a) Is straightforward and sufficient. (b) We assume that the selective SCG activation is to remove a following RRCReconfiguration after first/previous PSCell change. For MN-initiated case, there is no difference with respect to the execution condition. |
| Xiaomi | (a) Yes (b) with comments | For (a), the same principle of R17 MN initiated inter-SN CPC/CPA should also be reused for MN initiated SCG selective activation in R18. For (b), in legacy, A4 is configured for load balancing purposes only. So, event A4 used for subsequent CPC is not a clear use case in SCG selective activation. In the legacy R17 principle, A3/A5 for PSCell change can only be configured by SN, which is associated with SCG measurement configuration. If (b) is yes, to support MN configure A3/A5 for subsequent CPC, some extra effort is needed. If (b) is no, MN initiated SCG selective activation can only be configured for CPA. To support a CPA configuration can be used for CPC with different events, a conditional reconfiguration including MN and SN involvement is needed. |
| NTT DOCOMO | (a)Yes (b)see comments | (b) event A3/A5 is considered useful as a condition for CPC associated with UE moving, but it is necessary to consider that the serving cell changes in subsequent CPCs in selective activation. |
| ITRI | No for a), Yes for b) | For a), we think the MN is responsible for determining whether to accept an execution condition (which may be generated by other nodes), rather than generating all execution conditions in the procedure. For b), we think the MN can only uses event A4 as execution conditions for initial CPAC. For subsequent CPC, the execution conditions can be updated. In other words, A3/A4/A5 events can be used after the condition update for subsequent CPC. |
| Nokia | Yes. See comments | 1 a) MN generating the measurement-configurations and execution condition will simplify the changes need for having different execution conditions depending on the new serving cell. For example, MN can generate the execution condition for initial serving cell change similar to CPAC and it can include the changes needed in execution condition for different-cells as variation of measurement-config and measurement-ID as part of MCG change. [QC (Rapp)] Ok. 1 b) As there are some benefits for MN generated execution condition for SAPC, it is also good to support A3/A5 with comparison of target-cells against current serving cell. |
| MediaTek | Yes for a), No for b) | For a), follow legacy procedure (at least for A4) is okay. For b), we understand there is more control flexibility. But as there is also SN-initiated intra/inter-SN scenario, which can be based on event A3/A5. So, it seems no strong need to support A3/A5 for MN-initiated case. |
| Intel | Yes for both a) and b) | Even if it’s MN initiated CPC, considering subsequent CPC is supported without RRC reconfiguration, A3/A5 should be supported as the neighbour cell quality should be compared with current PSCell. |
| Qualcomm | Yes for a), b) | a): Source MN generating all the execution conditions seems to be a viable option. b): We support b) on principle. |

In the previous RAN2 meetings (RAN2 #121 [1], RAN2 #120 [28], RAN2 #119-bis-e [29]) it was agreed to support delta configuration signalling for the procedure, as also some related signalling details. One question that was left open was what configuration should be the reference SCG configuration. Several options were proposed by companies in their contributions:

- The reference SCG configuration is the initial source SCG configuration [27], [11], [15], [16].

- The reference SCG configuration is provided by the source SN (this option includes Option (1)) [19].

- The reference SCG configuration is the current serving SCG configuration [2], [14], [16].

- Follow similar design as LTM [9]. We may then need to wait for LTM to develop the principles for the design.

- The reference SCG configuration is a separate configuration determined by the MN [11].

Question 2: Which of the following option(s) on the configuration that should be the reference SCG configuration is preferred by companies? Please indicate in your comments if there is any other option you would like to discuss.

- The reference SCG configuration is the initial source SCG configuration.

- The reference SCG configuration is provided by the source SN (this option includes Option (1)).

- The reference SCG configuration is the current serving SCG configuration.

- Follow similar design as LTM.

- The reference SCG configuration is a separate configuration determined by the MN.

- The reference SCG configuration is provided by a candidate SN

- The reference SCG configuration is the serving SCG configuration indicated by MN

- Up to network implementation

Companies are requested to consider this question for SN initiated inter-SN SCG selective activation also and provide response.

| Company | Option(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | 4) 5) 6) | Determined by the MN does not say where the MN is taking it from. It could be from the source SN or from a candidate target SN. [QC (Rapp)] Just wondering how this works for the case when MN takes it from a candidate SN. In this case, does MN take a reference SCG configuration from a candidate SN (say SN1) and provide it to a candidate SN (say SN2) of a subsequent CPC, so that SN2 can provide a delta SCG configuration? Note: the initial step could be CPA, in this case there is no source SN. Note2: Maybe we should also discuss whether there is only one reference configuration or multiple reference configuration (e.g. per C-SN). |
| Lenovo | (1) or (4) | Comparing (1) and (5), following CPC principle, when preparing delta configuration for SCG selective activation, MN will anyway provide the source SCG configuration to target SN and target SN will then take the source SCG configuration as the reference. In this sense, the initial source SCG configuration when preparing the SCG selective activation is a natural reference, and there seems no need for network to provide/point another set of configurations as reference, which may lead to extra signalling and complex. In addition, in our view, the UE and target SN only need to know a reference configuration for the sake of delta configuration, and the UE/target SN do not care if the reference configuration is the initial source SCG config or a separate configuration. In other word, no matter we use the initial source SCG or a separate SCG configuration as reference, the behaviours of UE/target SN in these two cases are the same. In short, (1) seems simpler and following legacy principle should be taken as the baseline, (5) requires unnecessary complexity at MN, and it is a bit strange that MN generates reference configuration for SCG operation. (3) implies a candidate PSCell will be configured with a set of delta configurations each is associated with a possible serving cell. It seems a bit complex and additional signalling overhead if we use (1). |
| OPPO | (4) | For (1)(2)(3), they are not applicable to CPA case. For (5)(6), they can be considered as further details of (4). We can wait for the progress of LTM and then check whether there is something specific needs to be studied for selective activation of SCG. |
| vivo | 4) 7) | To us, (1) is the most simple and straightforward one, however, using the initial source SCG configuration may make the delta configuration less efficient after a number of PSCell changes in subsequent CPAC procedure, when the reference configuration is supposed to be updated. Therefore, we think it would be easier by introducing (7) the reference SCG configuration is the serving SCG configuration indicated by MN, i.e. it is up to MN to indicate a serving SCG configuration to serve as reference configuration, it can be (1) (at the beginning), or (3) (after one PSCell change) for which the network think is suitable to be used in a certain situation. For (4), as in LTM session, there are a lot of discussions for how to determine the reference configuration and even for the case when there is no reference configuration. So, we think it would be also easy if we follow the similar design in LTM. For (5), it is not clear whether delta configuration can apply to “CPA” selective activation, so the necessity for (5) can be FFS. |
| CATT | (4) or (5) | On (4), It is beneficial for avoiding duplication of effort to reuse the referenece config design in LTM as much as possible, but we still need to check whether all the referenece config design in LTM is applicable for selective activation when the referenece config design for LTM is completed. On (5), separate reference configuration has already been agreed for LTM, so it is OK to use it as the start point for selective activation. |
| Ericsson | 8) | 3) is a bit unclear. Is it the current configuration at CPC execution? [QC (Rapp)] Yes, that is our understanding based on reading the relevant contributions. 2) could be used for CPC, but 6) could be applicable for CPA, i.e. different for different use cases. The network can decide what is included in the reference configuration and signal it to the UE. Should the reference configuration only contain an SCG configuration or should it contain both MCG and SCG configuration? Both MN- and SN- initiated inter-SN CPC (rel-17 CPC) and CPA contain both MCG and SCG configuration. This could be up to network implementation to decide what is included in the reference configuration. [QC (Rapp)] The point that the reference configuration may contain both MCG and SCG configurations is a valid one. |
| Futurewei | Modified-(6)+ or (4) | Reference configuration is w.r.t. delta configuration for subsequent cell change. The assumption is initial pre-configuration already configured the candidate SN(s). After a CPC is completed, if a candidate SN is maintained and delta configuration for the candidate SN is required, we prefer the reference configuration to be: 6)+ the current configuration of a candidate SN which is originally provided by the SN. And the delta configuration should also be provided by the maintained candidate SN. We are OK with (4) in case a separate reference configuration is decided for LTM. |
| ZTE | (2)(4)(5)(6) | We think it would be more flexible to let any one of source SN or candidate SN provide the reference SCG configuration. Assuming there is only one reference SCG configuration, a possible procedure to provide the reference configuration could be as follows: - First, the MN determines which SN (source or candidate) to provide the reference SCG configuration and sends the request to the selected SN. The SN generates the reference SCG configuration and sends it to the MN. - Second, the MN sends the reference SCG configuration to other candidate SNs, to generate candidate SCG configurations. - Third, the MN includes the reference configuration as a separate configuration or includes an indication to indicate the reference configuration (e.g. the reference is the initial source configuration,or the reference is a specific candidate SCG configuration) into the RRCReconfiguration message with candidate SCG configurations to the UE. - Fourth, based on the NW indication, the UE stores the configured/indicated reference configuration as a separate configuration, i.e. the UE handling is similar to LTM. Besides, we want to further clarify whether the reference configuration can also include the MCG part configuration. Since the inter-SN CPC candidate configuration may include both MCG part and SCG part, it’s possible to provide a complete reference configuration including both MCG part and SCG part for the UE. [QC (Rapp)] Agree that the complete reference configuration can include the MCG part and the SCG part (mentioned also by Ericsson). |
| NEC | (1) or (4) | We assume (4) also includes (1). Although the design for LTM is not completely stable (e.g. signalling details), the same principle can be applied. On thing to be confirmed is that the MN provides the reference SCG configuration in this case. |
| Xiaomi | (4) (5) | First, we can follow similar design as LTM. And we need to check whether the principle for the reference configuration in LTM can be full reused for SCG selective activation. In accordance with the design in LTM, the reference SCG configuration is a separate configuration determined by the MN. And MN can acquire the separate configuration from the source SN or the target SN, or generated by MN. |
| NTT DOCOMO | See comments | For (4), For reference configuration/delta configuration specifications, we agree with using a common mechanism as much as possible, but since LTM supports different scenarios and the discussion is still in progress, it is better to wait for the progress of the discussion. For (5), providing a configuration other than the initial/source configuration by the MN may cause complexity. |
| ITRI | (5)(6) | We think the reference configuration should be managed separately by the MN. For example, the MN is responsible for determining whether the reference SCG configuration provided by a candidate SN is acceptable or not. We are OK with options 5 and 6. |
| Nokia | 5) and 1) | To minimise the signaling overhead for delta-configurations and also changes during selective activation, NW generated reference configuration is preferred. If such option is not possible or supported at NW, initial SCG configuration can be considered as reference configuration for switching the configuration during selective activation. This option (1) requires the UE to switch to initial configuration and apply the target delta configuration. This two level switching may increase interruption time, if the UE cannot replace the parts which are different between the current and reference configuration. [QC (Rapp)] Ok, this seems to be a drawback with Option (1). We don’t think how option 6 will work in case if the SAPC is prepared simultaneously with multiple target cells. In that case how the ‘delta configurations’ with respect to the candidate SN can be used. More details on option 6 is needed. Not clear on how option 8 can resolve the need for reconfiguration during cell-change. Eventhough use of full-configuration is network implementation option, it can be done already. And RAN2 agrees that there will be reference configuration for selective activation. This option is not inline with the agreement. |
| MediaTek | (4) | We suggest the clarify the UU aspect on delta configuration first. Our view is to reuse the same principle as in LTM to avoid duplicate work. Then which node to determine/provide/generate the reference configuration (e.g. 5, 6) is further details. We also need to clarify the following open issues <1> Whether there is only one reference configuration or multiple reference configuration (as pointed out by HW). In our view, one is enough. More than one may largely increase the SPEC complexity. <2> Whether the reference configuration include MCG part? (as pointed out by Ericsson). In our view, not really necessary, but open for this. |
| Intel | (4) | The design of reference configuration should be as common as possible for both LTM and subsequent CPC to avoid duplicate work. |
| Qualcomm | (1), (2), (4), (5) | (5) seems feasible for the CPA case. |

Steps 4-6:

As in Rel-17 MN initiated inter-SN CPC, source MN should prepare the final RRC reconfiguration message containing the configuration for SCG Selective Activation to be provided to the UE. There could be easy agreement on this.

Question 3: Do companies agree that, for MN initiated inter-SN SCG Selective Activation, source MN prepares the RRC reconfiguration message containing the configuration for SCG selective activation to be provided to the UE?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Lenovo | Yes |  |
| OPPO | Yes |  |
| vivo | Yes |  |
| CATT | Yes |  |
| Ericsson | Yes |  |
| Futurewei | Yes |  |
| ZTE | Yes |  |
| NEC | Yes |  |
| Xiaomi | Yes |  |
| NTT DOCOMO | Yes |  |
| ITRI | Yes |  |
| Nokia | Yes | In either of the above options, NW is aware of the reference-configuration. So it can generate the final configuration. |
| MediaTek | Yes |  |
| Intel | Yes |  |
| Qualcomm | Yes |  |

We now discuss what should be the contents of the RRC reconfiguration message at a high level, without discussing details regarding the message structure. Based on company contributions, it seems that at least the following information should be included:

- The reference SCG configuration, or an indication regarding which configuration should be considered as the reference. The indication can be used, for example, in the case the current serving SCG configuration is to be used as the reference.

- The complete/full set of all candidate target PSCells configured for the procedure. After cell change to a candidate target PSCell:

  - The set of candidate target PSCells to be evaluated for the next conditional PSCell change.

  - The execution condition(s) associated with the candidate target PSCells in (a).

  - The SCG configurations associated with the candidate target PSCells in (a). For each candidate target PSCell, an indication whether the associated SCG configuration is a delta with respect to the reference SCG configuration or is a full configuration. The signalling of such an indication from a candidate SN to the MN was agreed in the last RAN2 #121 meeting [1].

(2) a) means that the full set of candidate target PSCells do not need to be evaluated for a subsequent conditional PSCell change, i.e., there is an applicable subset configured by the network that should be evaluated.

The complete/full set of all candidate target PSCells may include the initial serving PSCell since the UE may come back to it later.

Question 4: Do companies agree that the following information should be included in the RRC reconfiguration message to the UE containing the configuration for SCG selective activation?

- The reference SCG configuration, or an indication regarding which configuration should be considered as the reference.

- The complete/full set of all candidate target PSCells configured for the procedure. After cell change to a candidate target PSCell:

  - The set of candidate target PSCells to be evaluated for the next conditional PSCell change.

  - The execution condition(s) associated with the candidate target PSCells in (a).

  - The SCG configurations associated with the candidate target PSCells in (a). For each candidate target PSCell, an indication whether the associated SCG configuration is a delta with respect to the reference SCG configuration or is a full configuration.

Companies are requested to consider this question for SN initiated inter-SN SCG selective activation also and provide response.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | (1) is ok, (2) with comments | In Rel-17, the SCG configuration for each candidate target PSCell is included in an MN message, because CPC execution can also modify the MN configuration, and differently for each candidate PSCell. We expect that the same applies for Rel-18. In Rel-17, the UE looks inside the MN message to be applied at CPC execution in order to determine the candidate target PSCell. It could be the same, but we are ok to also indicate separately the candidate target PSCell (e.g., frequency and PCI), that could be helpful for the UE to avoid looking into every configuration that may actually be never executed. [QC (Rapp)] Agree that indicating the candidate target PSCells separately saves some UE processing. |
| Lenovo | (1) No, but depends on Q2 (2) see comments | (1) as explained in Q2, if UE by default take the initial SCG configuration as the reference, there is no need to provide a separate reference SCG or an indication. It is also signalling saving. (2) We suppose the key point is the RRC reconfiguration for SCG selective activation shall for each candidate PSCell include the related execution conditions and SCG configurations, wherein the SCG configuration could be delta or full configuration. Note that legacy RRC reconfiguration message already include a fullConfig flag, we don’t foresee any new indicator for full configuration is needed. [QC (Rapp)] A point to note is that the execution conditions for a candidate PSCell may depend on the serving PSCell as well, e.g., if A3/A5 events (please see Q1 and Q6) are configured. We thought that the new indicator might be needed since in the legacy case if the fullConfig flag was not present it meant that a delta config with respect to the source config is provided, whereas in the current case a delta config with respect to a reference config is provided. |
| OPPO | - Agree - See comments | - For both MN initiated and SN initiated SCG selective activation, we understand the configuration basically contains the following parts: - Reference SCG configuration - Condition configuration - Candidate cell configuration - Regarding to (2), we agree with the motivation is for measurement/evaluation overhead reduction. While we understand the candidate cell list can always be add/modified by NW due to UE movement, that is, the “set of candidate target PSCells to be evaluated” should also be updated if the full set of candidates is modified. Then, additional signalling overhead as well as spec complexity may be introduced. |
| vivo | (1) yes (2) see comments | For (1), we think the reference configuration or the related indication should anyway be needed in the RRC reconfiguration message, unless a specified reference configuration is used which we do not support. For (2), the question should be clarified, as Huawei mentioned, why it is limited to ‘After cell change to a candidate target PSCell’. In our understanding, for the initial RRC reconfiguration message when configuring the CPAC, 2(a)(b)(c) are all needed. But after cell change to a candidate target PSCell: 2(a) is possible but do not have to be RRC signalling (e.g. MAC CE) 2(b) is OK if there is a need to update the execution condition 2(c) is not needed as the indication for delta/full configuration should already be provided in the initial RRC reconfiguration message when configuring the CPAC. |
| CATT | See comments | We agree with (1) and (2), but for a, b, c in (2), maybe more discussion is needed for evaluate the benefit compared with the effort. |
| Ericsson | - Yes | In principle, it looks fine. Maybe not all information in 2) need to be included when the UE changes PSCell. In some cases there is a need to update, but not always. The network can always update the configurations if needed though. How much that needs to be included in the candidate configuration also depends on what is included in the reference configuration. The configurations as in c) does not have to be included, the UE already has these configurations in many cases (provided earlier). Also, in most cases the target configuration contains both an MCG and an SCG configuration. |
| Futurewei | (1) may be not (2) is fine | (1) maybe not needed in case taking current configuration as the reference. (2) in principle is needed. |
| ZTE | - Yes - See comments | For (1), agree to explicitly include the reference configuration or related indication. For (2), generally agree for each candidate PSCell, to provide the execution condition, candidate PSCell configuration and a set of candidate target PSCells to be evaluated for the next conditional PSCell change. But the detailed signaling can be further discussed. For (2)-c, regarding the indication on delta configuration, we want to firstly clarify whether the indication is included in the candidate cell configuration (e.g. the current fullConfig can be reused) or outside of the candidate cell configuration (e.g. an new indication to be introduced). In our understanding, we think the current fullConfig IE included in the candidate cell configuration can be reused. |
| NEC | (1) Yes (2) partly | For (2) “a”, we do not see strong need for that (i.e. subset concept), because we do not expect so many candidate cells far away from the original source cell, which may result in unused lots of signalling due to uncertainty of UE mobility. This can be considered in future (if needed). For (2) “c”, basically Yes, but the indication may not necessarily be explicitly sent and the signalling (ASN.1) detail can be left to Stage 3 discussion later. For SN-initiated, we assume (almost) the same way can be applied. |
| Xiaomi | (1) Yes (2) with comments | We agree with (1). A separate reference configuration which is configured by network should be included in the RRC reconfiguration message for SCG selective activation. For (2) We support candidate cell configuration and associated conditions can be included in the RRC reconfiguration for SCG selective activation. For (2)a, the confusing thing is why only a part of candidate target PSCell can be evaluated for the next conditional PSCell change and which candidate target PSCells don’t need to be evaluated. If all candidate target cells are configured for SCG selective activation, all candidate target cells should be evaluated. Does it mean only partial candidate target PSCell is configured for selective activation and other candidate target PSCell is configured for other use case (e.g. legacy CPAC). If it is yes, we think RAN2 need to discuss whether the coexistence between legacy CPAC and SCG selective activation can be supported. Ok for (2) b For (2) c, legacy fullConfig has been included in RRC reconfiguration and a new indication is not needed. |
| NTT DOCOMO | (1)(2)see comments | For(1) This probably depends on the conclusion of Q2: an option that does not use separate configuration could make the Reference configuration indication unnecessary. |
| ITRI | (1) yes (2) see comments | We think the initial RRC message should contains at least: candidate cells, condition configurations of candidate cells, (an indication of) reference configuration, and candidate cell configurations. In case the above information has been provided in the initial RRC message, whether 2(a), 2(b), and 2(c) should be provided via RRC after cell change can be further studied. |
| Nokia | See comments | OK for 1). For 2) OK for a. For b. further discussions needed on accommodating the cell specific execution conditions and its impact to evaluations. For C. The set of configurations prepared already contain the complete set of target-cells. So the first part is redundant. By default the target configuration is delta configuration with respect to initial-serving-cell, it can be considered for explicit-configuration. Full-config flag already exist in the configuration that can be used without explicit new parameter. |
| MediaTek | (1) ok with comment (2) is confusing | For (1), we understand of course the UE should be provided with SCG reference configuration. The indication is kind of unclear. Maybe the absent of reference configuration should imply that the UE use current configuration. For (2), It is not so clear that we are talking about reference configuration or candidate configuration. Inside reference configuration, we think NO need to have candidate target PSCell list configuration. Inside candidate configuration, there could be candidate target PSCell list configuration that NW could control the UE to add/modify/release the candidate target PSCell. |
| Intel | (1) yes (2) yes for b and c | For (1), similar to LTM, reference configuration can be configured. For (2), the execution condition and whether the configuration of candidate cell is delta on top of reference or full config are also needed. But for 2(a), we don’t think an applicable subset is beneficial for efficiency. |

Steps 7-9:

A key aspect of the Rel-18 procedure is that UE should not release the conditional configurations after a PSCell change.  This was agreed in the previous RAN2 meeting (RAN2 #119-bis-e [2]). It was left FFS whether UE keeps all configurations or only those indicated by the network.

Several company contributions [2], [5], [7], [16], [18], [23], [25], [26], [27], have indicated support for network indication on the conditional (CPC) configurations UE should release after a PSCell change. UE keeps the other conditional (CPC) configurations. Some company contributions [27] have proposed that the network indication can be provided in the RRC reconfiguration message containing the configuration for SCG selective activation.

Question 5: Do companies agree on the following proposals on whether UE keeps or releases conditional (CPC) configurations after a conditional PSCell change?

- After UE performs a conditional PSCell change to a candidate target PSCell, UE releases those conditional (CPC) configurations that are indicated by the network. UE keeps the other CPC configurations.

- The network indication(s) can be provided in the RRC reconfiguration message containing the configuration for SCG selective activation.

Companies are requested to consider this question for SN initiated inter-SN SCG selective activation also and provide response.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | See comments | We prefer that PSCell change is handled the same, regardless whether it is Rel-18 CPC execution or a non-conditional RRC message. At any time, the network can use RRC signalling to add, modify or release CPC configurations (the list of CPC configuration will probably be a ToAddModList). The simplest would be that, when the UE applies an RRC message - whether for Rel-18 CPC execution or normal reconfiguration -, the UE keeps each previously configured Rel-18 CPC configuration that the RRC message does not release by explicit signalling. [QC (Rapp)] Ok. |
| Lenovo | See comment | The proposals need further clarification. Do the proposals mean that when NW provides conditional reconfiguration for a candidate PSCell, it also indicates if UE should keep it after subsequent CPC? Ok if yes. [QC (Rapp)] Please see the response to Vivo’s comment. The indication(s) are provided in the RRC reconfiguration message containing the SCG selective activation configuration. |
| OPPO | See comments | It relies on whether to support the coexistence between legacy CPAC and SCG selective activation. [QC (Rapp)] Though the coexistence question needs to be addressed, we can perhaps consider this later. |
| vivo | See comments | It should be clarified whether it means the indication is sent by network after each successful CPAC. If yes, then we think it is unnecessary because the purpose of SCG selective activation is to reduce the signalling between UE and gNB. [QC (Rapp)] The indication is not meant to be sent by the network after each successful CPAC. As part (2) of the question states, the indication(s) are provided in the initial RRC reconfiguration message containing the SCG selective activation configuration. This message is sent by MN to UE immediately after preparation, i.e., Step 5 of the signalling flow of Figure 1. The basic principle can be as easy as: The UE should release the Rel-18 CPC configuration(s) whenever indicated by NW, otherwise the UE may keep the CPC configuration(s) after a conditional PSCell change. [QC (Rapp)] Ok. |
| CATT | See comments | Same concern as vivo, does it imply In (1) that after conditional PSCell change ,network indicate UE to release those conditional (CPC) configurations? [QC (Rapp)] Please see the response to Vivo’s comments. We understand the network can add/modify/release the candidate SCG configuration at any time the network expects e.g. when the network detects that some SCGs have good or poor radio link quality based on RRM measurement. UE will keep/add/modify/release the candidate SCG configuration based on the network configuration, otherwise, UE will keep all stored candidate SCG configurations after each SCG change. |
| Ericsson | See comments | The legacy behaviour is to release the conditional reconfigurations at reconfigurationWithSync. We had expected that the network would indicate which configurations to keep, but it could be fine to indicate which ones to release, but then a rel-18 indication is needed to separate the configurations from the legacy conditional reconfigurations. In case of conditional reconfiguration, the indication should be included in the message containing the conditional reconfiguration (of SCG selective activation). In case of legacy PSCell change, the indication should be included in the PSCell change command. [QC (Rapp)] Ok. Agree that a Rel-18 indication is needed to separate the Rel-18 and legacy conditional configurations. |
| Futurewei | See comments | In principle, the release of candidate(s) should be only instructed by the network. The network instruction via RRC can be issued any time before or after the execution of a pscell change. On the other hand, the candidate release should not be preconfigured, i.e., the UE should perform candidate release upon reception of RRC instruction. After received release instruction, the UE should not wait to perform the candidate release after pscell change executed. It is not practical for network to predict which candidate cell will be the new pscell, therefore, should not pre-configure which candidate should be releases after a pscell change. The key point is after a PSCell change execution, the following candidate(s)’ release should be instructed by network also after the PSCell change execution based on the latest mobility situation. [QC (Rapp)] If we understand the comment correctly, what is being proposed is that the network should transmit an RRC message after every CPC execution to explicitly release some of the CPC configurations at the UE. However, this would result in significant signalling overhead and seems unnecessary. |
| ZTE | See comments | After a PSCell change (including normal and conditional case), the UE can simply keep all stored candidate PSCell configurations. If the NW wants to release some candidate PSCells, the NW can explicitly release them via ToReleaseList in RRCReconfiguration message. [QC (Rapp)] The question Q5 is asking whether the network release indication can be provided in the RRC reconfiguration message containing the SCG selective activation configuration. Please see the response to Vivo’s comments. |
| NEC | See comments | Indication can be optional in a way that the network sends the indication for candidate PSCell(s) to be kept after CPC, i.e. default is to release with assuming most UEs moves in one direction most of time (not coming back). |
| Xiaomi | See comments | Does it mean network pre-configure UE to keep/release candidate configurations and UE will keep/release these candidate configurations which is indicated by network after PSCell change. If yes, we think RAN2 need to discuss whether the coexistence between legacy CPAC and SCG selective activation can be supported (i.e. UE can be configured with multiple candidate configurations simultaneously, one part for legacy CPAC, the other for SCG selective activation) first. [QC (Rapp)] Yes, the question is asking whether network pre-configures UE to release indicated candidate configurations after PSCell change. If CPAC and SCG selective activation cannot be configured simultaneously, P5 is not needed. [QC (Rapp)] If CPAC and SCG selective activation cannot be configured simultaneously, the question still seems relevant. E.g., suppose only SCG selective configuration is configured, it was agreed (RAN2 #119-bis-e) that UE keeps the configurations, but it was left FFS whether UE releases some configurations based on network indication. The question is trying to address this FFS. |
| NTT DOCOMO | see comments | In Legacy CPAC, the candidate configuration is not maintained after CPAC is executed. Therefore, we should first discuss whether and how to discriminate between selective activation and legacy CPAC. For example, when selective activation is initiated, the NW may add an indication to the RRC reconfiguration that indicates to maintain the configuration on some basis. The indication to discard should be determined after discussing the mechanism for maintaining. |
| ITRI | See comments | If the proposals mean that the indication(s) are included in the initial RRC reconfiguration message containing the configuration for SCG selective activation, i.e. network need not send the indication(s) to UE after each CPAC execution, we agree. [QC (Rapp)] Yes, the proposals mean that the indication(s) are included in the initial RRC reconfiguration message. |
| Nokia | 2 | The Network can decide based on the hold-time required at different target cells and the signalling overhead to reconfigure the target cells for every cell change. If NW decides to keep the configuration, the evaluation for selective activation can be skipped for these cells depending on the current serving cell. |
| MediaTek | See comments | As commented in previous question, “Inside candidate configuration, there could be candidate target PSCell list configuration that NW could control the UE to add/modify/release the candidate target PSCell.” Therefore, after applying the candidate configuration, the list will be updated as NW controlled. |
| Intel | See comments | We can generally agree the network can use RRC signalling to add, modify or release CPC configurations. The detail can be FFS at this stage. [QC (Rapp)] The question is asking whether the indication(s) to add or release can be provided in the initial RRC reconfiguration message itself, so that the network can avoid sending the indication(s) in RRC signalling later, e.g., at PSCell changes. |
| Qualcomm | Yes for both (1) and (2) |  |

Steps 10-13:

After CPC is triggered for a candidate target PSCell, UE sends RRC reconfiguration complete to the source MN including candidate target PSCell ID (this is similar as in Rel-17 CPC). The source MN informs the source SN, the candidate SN of the triggered candidate target PSCell, and the other candidate SNs, and may include other information that is FFS.

### Execution conditions generated by candidate SN

Another option to consider is that a candidate SN can generate some of the execution conditions. This aspect is discussed in [19], [27], [16], [8]. After UE performs a cell change to a PSCell that belongs to a candidate SN, it seems reasonable that the candidate SN generates the execution conditions for the subsequent conditional PSCell change (CPC). The candidate SN can set the parameters of the execution conditions, e.g., event trigger thresholds, times to trigger, more accurately. The candidate SN may make use of the UE measurements provided by the source MN to it and can configure only the neighbour cells as the candidate target cells for CPC evaluation.

Question 6: Regarding the node that generates the execution conditions for the procedure, do companies agree that another option is that the candidate SN generates the execution conditions for the subsequent CPC?

Companies are requested to consider this question for SN initiated inter-SN SCG selective activation also and provide response.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | It should be C-SN’s responsibility to decide the condition/when the UE should leave this PSCell. [QC (Rapp)] Ok. |
| Lenovo | Maybe not, but depends on Q1 | Similar discussion has taken place in Rel17 CPC. We prefer to follow the legacy principle and let MN to generate the execution conditions. One problem for the target SN generating the execution conditions is that a target SN does not necessarily know the existence of candidate PSCells belonging to other candidate SNs, not sure if the target SN can configure accurately A3/A5 events considering different possible serving PSCells in this case. [QC (Rapp)] The problem stated seems to be a valid one. It can be solved by the MN providing to a target SN the candidate PSCells belonging to the other candidate SNs, since the MN has this information. On the other hand, we understand that when the target SN becomes the new source SN, it can update the execution conditions as part of SN initiated SCG selective activation. |
| OPPO | See comments | See our reply on Q1: - For MN initiated case, the execution condition is generated by MN. - For SN initiated case, the execution condition is generated by SN. If A3/A5 events are introduced for MN-initiated CPC, the execution conditions may need to be generated by SN. |
| vivo | No | Agree with Lenovo that similar discussion happens before and we can just follow the legacy design. Moreover, if a candidate SN generates the execution conditions, the execution conditions would anyway need to be sent to the source/serving SN for check. If the source/serving SN is not satisfied with the execute conditions generated, extra exchange between source/serving and candidate SN occurs. So the principle for SN-initiated inter-SN CPAC should be that it is the serving SN rather than the candidate SN to decide the condition of when the UE should leave the serving PSCell. When the CPAC is finished and the target SN becomes new serving SN, of course it can update the execution conditions if necessary. [QC (Rapp)] Please see the response to CATT on the intent of the question. |
| CATT | See comments | Need to clarify the intention of Q6,e.g. whether the C-SN generates the execution conditions for the subsequent conditional PSCell change when initially preparing the preconfiguration for selective activation, or after PSCell change? [QC (Rapp)] The intent of the question is to check whether the candidate SN generates the execution conditions for the subsequent CPC during initial preparation for SCG selective activation. For MN-initiated inter-SN SCG selective activation, it is more suitable for MN to generate the execution conditions based on MN-configured measConfig. For SN initiated inter-SN SCG selective activation, the execution conditions need to be configured by each candidate target SN based on measConfig of each candidate PSCell as serving PSCell. |
| Ericsson | Yes for SN-initiated | Yes for SN-initiated CPC. For MN-initiated CPC, the MN decides the execution conditions. |
| Futurewei | See comments | Based on the question, the assumption is that a PSCell change is completed, and a candidate cell becomes a new serving PSCell. In this case, we prefer to follow the R17 principle. We have similar view as vivo: - For MN initiated case, the execution condition is generated by MN. - For SN initiated case, the execution condition is generated by SN. The subsequent cell changes follow the same configuration unless new configuration is received by the UE. If the new PSCell initiates a new CPC, SN initiated rule should be followed. |
| ZTE | Yes | We think it makes sense to let candidate SN generate the execution conditions (based on the candidate SCG MeasConfig) for the subsequent CPC, i.e. used when the candidate SN becomes the serving SN. |
| NEC | See comments | For MN-initiated, we think no need for this, as commented in Q1. The execution condition configured by the MN can be applied for subsequent mobility as well. For SN-initiated, there can be two ways to make the feature useful. One way is as proposed here in Q6. The other way is to consider a kind of swapping for “serving PSCell” in the execution condition. The initial PSCell is the serving PSCell at the time of selective SCG activation preparation. In the subsequent mobility, “PSCell” selected at first/previous PSCell change can be considered as “serving PSCell” in the execution condition configured by the original source SN (i.e. reusing the condition). [QC (Rapp)] There seem to be issues with the “swapping” proposal. The execution conditions will likely change when the serving PSCell is different, e.g., the event threshold parameter can be different. Thus, reusing may not be the right thing to do. [NEC] Thanks for feedback. We understand this way in Q6 is more feasible than swapping. |
| Xiaomi | See comments | For MN-initiated, the MN generates the execution conditions. For SN-initiated, if it is needed, target SN can update execution conditions when it becomes the source SN. |
| NTT DOCOMO | Yes |  |
| ITRI | Yes | After a CPAC is executed, a candidate SN becomes the serving SN. We think it is reasonable that the execution conditions of subsequent CPC can be generated (or updated) by the candidate SN. |
| Nokia | See comments | The question needs to be clarified. The execution conditions for the initial switching from current SCG needs to be decided by source cell only. Here we don’t see option for candidate generating the execution condition. Candidate can provide the change in execution conditions for the case when it later becomes serving cell only. This can be included as part of Target cell delta configuration itself for the SN initiated scenario. But here the target SN may prepare the modified execution conditions considering all the candidate cells of SAPC as seen from the measurement-report included in the source-RRC-container. Whether this part needs to be optimised requires further discussion. [QC (Rapp)] Agree with the comment. The question is not for the initial switching, where we agree it needs to be decided by the source SN. The question is for a subsequent CPC, i.e., a CPC after the first. |
| MediaTek | See Comment | For MN initiated case, the execution condition is generated by MN. For SN initiated case, it is okay for candidate SN to generates the execution conditions. |
| Intel | Yes for SN-initiated | For SN-initiated CPC, the execution condition can be associated to MeasIds of SN MeasConfig. |
| Qualcomm | Yes | For the MN initiated case, this can be one of the options, i.e., source MN generates the execution conditions for the first CPC and candidate SN generates the execution conditions for a subsequent CPC. |

## SN initiated inter-SN SCG selective activation

Figure 2 shows a signalling flow for the procedure assuming UE is initially in DC operation.

As for the MN initiated case, the source SN can generate all the execution conditions for the SN initiated procedure [27]. In such a case, the source SN includes the execution conditions in the SN Change Required message to the source MN.

Question 7: Do companies agree that source SN can generate all the execution conditions for the procedure (similar as in Rel-17 SN initiated inter-SN CPC)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | The execution conditions generated by the source SN are only applicable when the PSCell is the current PSCell, because they are references to the UE configuration in the current PSCell (now or after the UE goes back to this PSCell). If the UE moves to another SN, the UE should apply execution conditions generated by that other SN, because they refer to measurements configured by that SN. Therefore, the common solution will be “candidate SN generating the execution conditions” |
| Lenovo | Yes, with comment | Since source SN knows the possible candidate PSCells belonging to other candidate SNs, it is possible that source SN generates all the execution conditions considering all possible serving PSCells. [QC (Rapp)] As mentioned by Huawei, one issue with source SN generating all execution conditions is that after UE moves to another SN it would apply the measurement config provided by that other SN and the execution conditions would then have to refer to the measurement config of the other SN. On the other hand, after UE switches to a different target SN, the target SN (which becomes the new source SN) can update the execution conditions, if it wants. |
| OPPO | See comments | It depends on whether the CPC configuration can be used for CPA. If yes, MN needs to be involved for the generation of execution conditions for subsequent CPA. |
| vivo | Yes | We prefer the source node generates the execution conditions for CPC without the candidate node involvement as in legacy. When the CPAC is finished and the target SN becomes new serving SN, of course it can update the execution conditions if necessary. |
| CATT | No | Source SN only can generate the execution conditions for the first PSCell change, i.e. the serving PSCell is the source SN, after the PSCell change, the execution condition is not applicable. And based on legacy CPC, the execution condition is based on the measConfig generated by the source SN, if the same principle is followed each candidate target SN generates the execution conditions based on measConfig of each candidate PSCell as serving PSCell. This solution will introduce complexity and signalling overhead. |
| Ericsson | See comments | Yes for the first PSCell change. For subsequent, there might be a need for the candidate PSCell to update. |
| Futurewei | No | We have similar view as Huawei and CATT. Due to the limited vision of the source SN, it is better to let the candidate SN(s) to determine the event triggering condition. Anyway, the triggering condition is for UE access to the corresponding candidate SN/cell. |
| ZTE | No | Since the execution condition generated by the source SN is based on the source SCG MeasConfig, the execution condition may become invalid when the UE moves to other candidate SN. Thus, we think that the source SN can generate the execution conditions (based on source SCG MeasConfig) for the subsequent CPC used when the current serving PSCell belongs to the source SN. And the candidate SN can generate the execution conditions (based on candidate SCG MeasConfig) for the subsequent CPC used when the current serving PSCell belongs to the candidate SN. |
| NEC | See comments | We assume the execution condition configured by the source SN can be reused for the subsequent mobility, as in MN-initiated case. However, if “all” means that the execution condition per candidate PSCell are configured by the source SN, we do not think they are necessary. Instead of that, other way (e.g. discussed in Q6) may be considered. |
| Xiaomi | Yes with comments | Initial source SN can generate all execution conditions for all candidate PSCells. After SN change, the new source SN can update these execution conditions and add new candidate PSCells, if needed. |
| NTT DOCOMO | See comments | As mentioned by Ericsson, after first PSCell change, there are cases where execution condition needs to be updated. |
| ITRI | No | We share the same view as Huawei and CATT. |
| Nokia | See comments | Our comments to Q6 is also applicable here. Some further clarifications on the SAPC scenario and execution conditions. - The first switching from serving SCG to all other target cells will be similar to CPAC , so the execution condition for this part needs to generated by MN or source SN depending on the supported scenario. - Update of initial execution conditions depending on the actual serving cell is another issue to be resolved for selective activation. As the selective activation intend to reduce the reconfiguration after cell-switch, inclusion of these changes related to execution condition also requires pre-configuration. This part naturally should be done by Target SN. How the combining of different execution condition changes accommodated in first RRC message carrying CPAC configuration requires further discussion. |
| MediaTek | See comment | In our view, the current execution condition could be generated by source SN (as legacy). Inside candidate configuration, there could be execution condition configuration to update execution condition. This execution condition is prepared by candidate SN. Once the UE applying the candidate configuration, candidate SN becomes source SN, and the execution condition is based on source SN. |
| Intel | See comments | As mentioned by other companies, source SN can decide on the execution condition for the first PSCell change. If SN is changed, the latest source SN has to update the execution condition to associate it to the new SN MeasConfig. |
| Qualcomm | No | We think that source SN can generate the execution conditions for the first CPC and candidate SN for the subsequent CPC. |

The possibility that execution conditions for the procedure may be updated upon a PSCell change has been discussed in [2], [18]. These contributions propose that, upon PSCell change (Step 12 in Figure 2), the source MN provides to the candidate SN (corresponding to the triggered candidate target PSCell) the execution conditions and related SCG measConfigs of the CPC configurations. The target SN may respond with updated execution conditions and SCG measConfigs.

Figure 2: Source SN initiated inter-SN SCG selective activation

Question 8: Do companies agree on the following proposals?

- The execution conditions for the procedure may be updated by the network upon a PSCell change.

- Upon PSCell change, the source MN provides to the target SN the execution conditions and the related SCG measConfigs of the CPC configurations, and the target SN may respond with updated execution conditions and SCG measConfigs for the CPC configurations.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Not sure | The proposal is apparently to have an RRC reconfiguration after each CPC execution, in order to provide the execution conditions for subsequent CPC. [QC (Rapp)] Sorry for any confusion. The intent of the question is whether the initial set of execution conditions provided to the UE can be updated after a subsequent CPC. In this case, the message could also include the CPC configurations for subsequent CPC, i.e. the network only uses Rel-17 CPC configurations (so it works with any UE supporting CPC), even if, on the network side, the behaviour can be different from Rel-17 (e.g. not tell the candidate SN to release the CPC configurations). If we want to avoid RRC reconfiguration after each CPC execution, it means the execution conditions to apply after CPC execution should be provided together with the CPC configuration (i.e. before the first CPC execution). |
| Lenovo | Yes with comment | We assume the proposal is essentially saying when a target SN becomes the new source SN, it can update the execution conditions as it wants. |
| OPPO | Yes |  |
| vivo | Yes for 1) FFS for 2) | For 1) it is OK as the execution conditions can be updated whenever needed. For 2), In legacy CPC, the serving SN decides the execution conditions configured to UE and may update them when necessary. So, it seems natural to make the target SN know the already configured execution conditions and the related source SCG measConfig upon PSCell change. However, we understand this kind of procedure can cause a lot of unnecessary signalling overhead between UE and network if such coordination happens a lot e.g. upon each successful PSCell change in subsequent CPAC procedure, if the SCG measConfig configured by target SN (new serving SN) is different from the source SCG measConfig, the mismatch happens between the CPC configured to UE (before this PSCell change) and the newly configured SCG measConfig. Then RRC reconfiguration is needed to update the CPC execution conditions. We think solutions should be considered to reduce the signalling overhead to update the execution conditions. |
| CATT | No | Considering the intention of the objective, we prefer there is no RRC reconfiguration after each PSCell change for execution conditions update. The above mentioned solution need NW to update the configuration each PSCell addition/change. And after each PSCell addition/change whether the UE need to evaluate each candidate SCG based on the execution condition configured before the PSCell change/addition is not clear, due to the new serving PSCell may reconfigure the execution condition and measConfig. [QC (Rapp)] The proposal is not to make update of the execution conditions mandatory after each PSCell change. Proposal (1) states that the execution conditions “may be updated” and it is up to the network to send updated execution conditions after a PSCell change, if needed. |
| Ericsson | - Yes - Maybe | - At a PSCell change, the new serving SN should be informed of other configured candidate PSCells. Additional signalling to the UE after the PSCell change should be avoided. |
| Futurewei | Yes for 1) 2) FFS | In principle yes for 1). 2) is a MN initiated reconfiguration. The procedure should be MN notify the SNs for addition/maintaining/release. In response, the newly added/maintained candidate SNs provide their config/update on the execution condition. Then the MN reconfigure the updated execution conditions to the UE. After an execution, it is possible new source SN may also initiate a reconfiguration and performing similar procedure for sequential cell changes. |
| ZTE | - Maybe - No | For (1), if the execution conditions for the subsequent CPC is not pre-configured by the NW with candidate PSCell configurations, and if the initial execution conditions are generated by the source SN, then the target SN can provide the new/updated execution conditions after a PSCell change, i.e. when the candidate SN becomes the serving SN. [QC (Rapp)] The question is for the scenario where the execution conditions for the subsequent CPC are pre-configured but may need to be updated upon a PSCell change. For (2), in our understanding, the MN does not need to inform the execution conditions and the related SCG MeasConfigs of the CPC configurations to the target SN. If the target SN knows which candidate PSCells need to be evaluated for the next CPC, the target SN can directly generate the new execution conditions for the candidate PSCells (including updated SCG MeasConfig) and send them to the MN. And then the MN provides the new execution condition to the UE via RRCReconfigurtaion message, i.e. similar to the legacy CPC modification procedure. |
| NEC | Not sure | We understand this way may be feasible, but given that RRCReconfiguration message is sent to the UE upon/after PSCell change, the benefit of the feature would be reduced. [QC (Rapp)] Ok, please see the response to CATT’s comment. |
| Xiaomi | Yes for 1) FFS for 2) | For 1) network can update any conditional configurations including execution conditions whenever needed. For 2), if the execution conditions has been changed upon PSCell change, MN can inform SN. MN doesn’t always need to provide the execution conditions and the related SCG measConfigs to the target SN. |
| NTT DOCOMO | (1)(2)see comments | It is conceivable that an update of the Execution condition may be necessary, but further discussion is needed on this procedure, because RRC reconfiguration after CPC execution may reduce the gain of selective activation. |
| ITRI | Yes for (1) FFS for (2) | After a CPAC is executed, a candidate SN becomes the serving SN. We think it makes sense that the execution conditions can be updated by the current serving SN. For (2), we think the detailed procedure can be further discussed. |
| Nokia | No | For 1) As the main motivation for SAPC is to continue the evaluation and execution for consecutive cell-changes without reconfiguration, RAN2 should aim to avoid the RRC-Reconfiguration affecting the ongoing evaluation. So modification of binding of the measurement-ID for candidate configuration will not impact the continuation of evaluation. The measurement objects relevant for SAPC evaluation after cell change should be available at the time of execution itself without UE waiting for RRC-Reconfiguration. For 2) Change of measurement object after cell-change will lead to delay in evaluation that is not desirable from WI objective. |
| MediaTek | (1) Yes (2) Not sure | As commented in Q7 “Inside candidate configuration, there could be execution condition configuration to update execution condition.” So we think that it should be possible the execution condition after PSCell change. For (2), we think this behavior is done in preparation phase (i.e. not upon PSCell change while execution condition met). [QC (Rapp)] Ok. However, the question is not for the preparation phase but upon a later PSCell change. [MTK2] If (2) in PSCell Change execution phase, it maybe also fine. However, does this imply that there may be additional RRC Reconfiguration message to update the execution condition every time the UE perform selective SCG activation. This seems reduce the benefit to have this kind of subsequent CPC, as normal RRC Reconfiguration is very like to be needed. |
| Intel | 1) yes 2)FFS | If the execution condition is associated to SN MeasConfig, when SN changes, it’s reasonable to update execution condition. If additional RRC configuration to UE is to be avoided, it’s better to only support execution condition associated to MN MeasConfig. And another option is the execution conditions are associated to SN MeasConfig within reference configuration (i.e., this par is common for all candidate PSCells), in this way the execution conditions can be maintained unchanged for subsequent PSCell change. |
| Qualcomm | - Yes - FFS | 1): We think that the execution conditions may be updated by the network upon a PSCell change. 2): On the information provided by the source MN to the candidate SNs, this can be discussed later. |

# Revised Q4 on content of the initial RRC reconfiguration containing the configuration for SCG selective activation

Based on feedback from companies above, it seems revision to Question 4 is needed as it was unclear and confusing to most companies. Question 4 was regarding what should be the content of the initial RRC reconfiguration message containing the configuration for SCG selective activation. Based on company contributions and on the feedback from companies above, it seems at least the following could be included in the initial RRC reconfiguration:

- The reference configuration, or an indication regarding which configuration should be considered as the reference. The reference configuration could include both the MCG and SCG parts.

- The complete/full set of all candidate target PSCells configured for the procedure.

- The SCG configurations (and MCG configurations) associated with the candidate target PSCells in (2).

- In case A4 execution conditions are configured for the procedure, e.g., for MN initiated SCG selective activation and when MN generates all the execution conditions, the execution condition(s) associated with the candidate target PSCells in (2) are included.

- In case A3/A5 execution conditions are configured, e.g., for the case of SN initiated SCG selective activation and when source SN generates the execution conditions for the first conditional PSCell change (CPC) and candidate SN for the subsequent CPC. For each cell in (2) when it becomes a serving PSCell:

  - The set of candidate target PSCells to be evaluated for the next CPC.

  - The execution condition(s) associated with the candidate target PSCells in (a).

(5) a) means that the full set of candidate target PSCells do not need to be evaluated for a subsequent CPC, i.e., there is an applicable subset configured by the network that should be evaluated.

The complete/full set in (2) includes the initial serving PSCell.

Question 4 (Revised version): Do companies agree that the following information should be included in the initial RRC reconfiguration message containing the configuration for SCG selective activation?

- The reference configuration, or an indication regarding which configuration should be considered as the reference. The reference configuration could include both the MCG and SCG parts.

- The complete/full set of all candidate target PSCells configured for the procedure.

- The SCG configurations (and MCG configurations) associated with the candidate target PSCells in (2).

- In case A4 execution conditions are configured for the procedure, e.g., for MN initiated SCG selective activation and when MN generates all the execution conditions, the execution condition(s) associated with the candidate target PSCells in (2) are included in the message.

- In case A3/A5 execution conditions are configured, e.g., for the case of SN initiated SCG selective activation and when source SN generates the execution conditions for the first CPC and candidate SN for the subsequent CPC. For each cell in (2) when it becomes a serving PSCell, include the following:

  - The set of candidate target PSCells to be evaluated for the next CPC.

  - The execution condition(s) associated with the candidate target PSCells in (a).

Companies are requested to consider this question for SN initiated inter-SN SCG selective activation also and provide response.

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Partial | For initial RRC reconfiguration message, we understand that it is the message to setup selective SCG activation in the beginning. For (1) we disagree the part about indication, this part is still not clear, we suggest to have some FFS. For (2) and (3), we suggest to reworking with some clarification FFS. Overall suggestion on (1) to (3) are - The reference configuration including at least SCG configuration (FFS other configuration). FFS how many numbers of reference configuration is supported. FFS the UE behavior while no reference configuration. - The list of all candidate target PSCells. FFS whether it is included in candidate configuration (as legacy CPC) or outside the candidate configuration. - The candidate configuration associated with each candidate target PSCells in (2). FFS the RRC model for the candidate configuration. (Note: MTK view is that the candidate configuration is a complete RRC Reconfiguration message contains at least SCG part) For (4) and (5), it seems that the proposal mixed the UU aspect (what execution condition to be configured in initial RRC reconfiguration message) and the inter-node behavior (which node to generates the execution condition). We suggest the following bullet (4) in initial RRC reconfiguration message which covers some aspect from original bullet (4) and (5). - The execution conditions associated with each candidate target PSCells in (2). For MN-initiated procedure, execution conditions based on event A4 is supported, FFS whether A3/A5 is supported. For SN-initiated procedure, execution conditions based on event A3/A5 is supported. For (5), I still don’t completely understand. Is the proposal to have a table indicate which candidate PSCell to be evaluated after swich to one candidate PSCell ? I will suggest to put this FFS for now. |
| NEC | (1) Yes, with comments (2) Yes, with comments (3) Yes (4) Yes (5) No | (1) we support a single reference configuration for SCG (and MCG, if needed), respectively. Thus, we assume “indication” here is to indicate either source SCG (and MCG) configuration or different SCG (and MCG) configuration. I.e., we do not consider multiple different configurations at a time. (2) “complete/full set” does/should not necessarily mean to exclude additional PSCell(s) configured later. We understand the intention is to differentiate from “subset” in (5) a. (5) we understand the last part of the first sentence is still based on the assumption that this approach is agreed. Then for (5) a, we still do not see strong need for this, because we do not expect so many candidate cells far away from the original source cell, which may result in unused lots of signalling due to uncertainty of UE mobility. Also, this is complicated for the UE. |

# Conclusion

Based on the input from all companies, we have the following proposals.

# References

- Chairman’s meeting notes, RAN2 #121.

- R2-2300317	Discussion on NR-DC with selective activation cell of groups	vivo

- R2-2300352	Discussion on selective activation of cell groups and sequential LTM	Futurewei

- R2-2300384	Discussion on selective activation of SCGs for NR-DC	OPPO

- R2-2300404	Discussion on selective activation of cell groups	Intel Corporation

- R2-2300465	Further discussion on selective activation of cell groups	Vodafone

- R2-2300649	Discussion on NR-DC with SCG selective activation	 Spreadtrum Communications

- R2-2300752	Execution condition in selective SCG activation	Apple

- R2-2300817	Discussion on selective SCG activation	MediaTek Inc.

- R2-2300921	Discussion on NR-DC with selective activation of the cell groups.	DENSO CORPORATION

- R2-2300924	Further analysis on configuration and signalling aspects for SAPC	Nokia, Nokia Shanghai Bell

- R2-2300949	On SCG selective activation	 Lenovo

- R2-2301007	Discussion on NR-DC with selective activation cell of groups	KDDI Corporation

- R2-2301060	Subsequent change of SCGs and selective activation 	InterDigital Inc.

- R2-2301156	Discussions on selective SCG activation	NEC

- R2-2301218	Discussion on selective activation of the cell groups	ZTE Corporation, Sanechips

- R2-2301255	Discussion on NR-DC with selective activation of cell groups	CMCC

- R2-2301340	NR-DC with selective activation	Ericsson

- R2-2301360	NR-DC selective activation of SCG	Huawei, HiSilicon

- R2-2301564	Considerations on Subsequent CPAC after SCG Change	Samsung

- R2-2301597	Discussion on Selective Activation of Cell Groups in NR-DC	Transsion Holdings

- R2-2301740	Selective Cell Group Activation	LG Electronics

- R2-2301820	Discussion of selective activation	Sharp

- R2-2301842	Discussion on scenarios for selective activation of the cell groups	China Telecom

- R2-2301395	Discussion on NR-DC with selective activation of the cell groups	Xiaomi

- R2-2300094	Discussion on Selective Activation of Cell Groups in NR-DC	CATT

- R2-2300281	SCG Selective Activation in NR-DC	Qualcomm Incorporated

- Chairman’s meeting notes, RAN2 #120.

- Chairman’s meeting notes, RAN2 #119-bis-e.